JOINT WMO TECHNICAL PROGRESS REPORT ON THE GLOBAL DATA PROCESSING AND FORECASTING SYSTEM AND NUMERICAL WEATHER PREDICTION RESEARCH ACTIVITIES FOR 2015

Korea Meteorological Administration

Republic of Korea

- Summary of highlights

- Major changes in the operational NWP suites in 2015

  - Clear Sky Radiance (CSR) data of Water Vapour channels from geostationary satellite are newly introduced (see 4.2.1)

  - New tropical cyclone bogus scheme is adopted (see 4.2.1)

  - New surface model (JULES) is applied for surface analysis process (see 4.2.1)

  - WRF based regional NWP model KWRF (10kmL40) is retired from operation (see 4.3.1)

  - Soil moisture input data are more frequently updated from once a day to 4 times a day (see 4.3.1)

  - Ancillary data set update in regional model : use new 4km MODIS LAI climatology (see 4.3.2)

  - Ensemble MOS (EMOS) based global EPS is newly implemented (see 4.3.4)

  - The Asian Dust model is upgraded to the ADAM2-Haze to deal with both Asian Dust and haze (see 4.6.4)

- Operation of a new Local Ensemble Prediction System (see 4.3.5)

  - KMA has been running the short-range ensemble prediction system (LENS) operationally since October 2015 to provide guidance for early warning by providing a probabilistic prediction of high impact weather. The system was designed to forecast 69 hours at every 12 hours based on enhanced KMA global ensemble prediction system (EPSG) via downscaling of initial perturbation. The LENS consists of one control member and 12 perturbation members.

- Operation of a new Seasonal Prediction System (GloSea5) (see 4.8)

  - New seasonal forecasting system of KMA is implemented operationally in Jan. 2014. This system, named the global seasonal forecasting system version5 (GloSea5), is a joint seasonal forecast system between KMA and Met-Office, UK and shares model results of each centre.

Table 1.1 Main operational atmospheric and application models in KMA as of Dec. 2015

| Purpose | Model (domain) |  |  | Resolution | Forecast range |
| --- | --- | --- | --- | --- | --- |
| Long-range forecast | GloSea5 (Global) |  |  | N216L85 | Seasonal (7 months) |
| Medium-range forecast | UM(Global) | Deterministic |  | N512 L70 (25km) | 288 hours |
|  |  | Ensemble |  | N320 L70 M24 (40km) | 288 hours |
| Short-range regional forecast | UM (East Asia) |  |  | 12km L70 | 87 hours |
| Short-range local forecast | UM (Korea) |  | Deterministic | 1.5km L70 | 36 hours |
|  |  |  | Ensemble | 3.0kmL70 M12 | 69 hours |
| Very short-range forecast | KLAPS (Korea) |  |  | 5km L40 | 12 hours |
| Ocean wave | WaveWatch3 (Global) |  |  | 55km (1/2°) | 288 hours |
|  | WaveWatch3 (Regional) |  |  | 8km (1/12°) | 87 hours |
|  | WaveWatch3 (Coastal) |  |  | 1km (1/120°) | 72 hours |
| Tide/Storm surge | POM (Regional) |  |  | 8km (1/12°) | 87 hours |
| Asian dust | ADAM2 (Regional) |  |  | 25km L47 | 72 hours |
| Tropical cyclone | DBAR (Regional) |  |  | 35km | 72 hours |

- Equipment in use

The supercomputer Cray XE6 is dedicated to the operation of short-, medium-range numerical weather prediction as well as long-range forecast and climate simulations.

- Name of the model: Cray XE6

- CPUs and performance

- Number of nodes: 3,760

- Number of cores: 90,240

- Core performance: AMD 2.1GHz 12-cores processor

- Peak performance: 758 T Flops (379 [operational purpose] + 379 [back-up & research purpose])

- Operating system:  SUSE Linux 11

- Memory

- Memory per node :  32 GB

- Memory total     :  120 TB

- Disk storage

- Lustre file system: 4 PB

- Backup file system (VTL, TAPE LTO-4 media): 6.5PB

- Data and Products from GTS in use

The number of observation data used for the operational NWP models has been gradually increased.  The ATMS, CrIS, and several Clear Sky Radiances from Geostationary Satellite were newly introduced for the operational global NWP suite. The impact of various satellite radiance data was successfully verified and so it was also added at the end of June. The ratios of assimilated observation against received observation are presented in Table 3.1.

Table 3.1 Types and extent of observations received through GTS/FTP and assimilated in KMA’s operational global data assimilation system

|  | In-situ observation |  |  | Indirect assimilation |  | Direct assimilation |  |  |  |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface | SONDE | Aircraft | Scatwind | AMV | CSR | ATOVS | AIRS | IASI | GPS-RO |  |
| Received | 103876 | 6612 | 536731 | 1665300 | 2497681 | 55102 | 3420510 | 198428 | 262260 | 1719 | 8748219 |
| Assimilated | 97911 | 2447 | 87541 | 49978 | 142587 | 11365 | 126083 | 27298 | 28444 | 1660 | 575314 |
| Ratio(%) | 94 | 37 | 16 | 3 | 6 | 21 | 4 | 14 | 11 | 97 | 7 |

- Surface : SYNOP, Ship, Buoy

- SONDE : TEMP, PILOT, Wind Profiler, Drop-sonde

- Aircraft : ACARS, AMDAR, AIREP

- Scatwind : ASCAT

- AMV : MTSAT, GOES, METEOSAT7, MSG, COMS, MODIS, AVHRR

- CSR : COMS

- In preparation: CSR of SEVIRI, METEOSAT7, and GOES-E/W, ATMS & CrIS

- Forecasting System

4.1	System run schedule and forecast ranges

Short- and medium-range forecast

- Global Data Assimilation and Prediction System (GDAPS; UM N512L70) is used for 12-day forecast (00/12UTC) and 87-hour forecast (06/18UTC) with 2 hour 25 minute observation data cut-off. GDAPS is used for short-range weather forecasts, weekly forecast as well as for the provision of lateral boundary conditions of the two short-range regional NWP systems for the East Asia domain.

- Regional Data Assimilation and Prediction System (RDAPS; UM 12kmL70) is operated 4 times daily (00/06/12/18UTC) for 87-hour forecast.

- Local Data Assimilation and Prediction System (LDAPS; UM 1.5kmL70) runs 4 times daily (00/06/12/18UTC) to produce 36-hour forecast with 3-hourly 3DVAR cycle, and KLAPS (5km L40), a local very short-range forecasting system runs every hour analysing weather conditions around the Korean peninsula.

- For typhoons originating in the western North Pacific, four track forecasts are obtained from Double Fourier Series BARotropic typhoon prediction model (DBAR), RDAPS (UM 12kmL70), GDAPS (UM N512L70), and global EPS (UM N320L70M24).

Long-range forecast

- Seasonal forecast is run every day including forecast and hindcast (re-forecast). 9 members of hindcast and 4 members of forecast are implemented for a day. And once a week (every Tuesday), Ensemble products for the long range forecast are generated. (see section 4.8 for details.)

  - Medium range forecasting system (4-10 days)

4.2.1	Data assimilation, objective analysis and initialization

4.2.1.1 In operation

The table 4.1 shows the major characteristics of KMA’s operational global data assimilation in 2015.

Table 4.1 Configuration of operational data assimilation for the global NWP suite in 2015

| Analysis resolution | N320L70 (horizontal resolution : ~40km) |
| --- | --- |
| Inner loop resolution | None |
| Analysis domain top | 80km |
| Analysis method | Hybrid Ensemble 4DVAR (from 24 members of Ensemble Prediction System) |
| Observations used | SYNOP, Ship, Buoy, METAR, ASCAT, Sonde, Pilot, Windprofiler Airep, ACARS(Amdar), AMV-Meteosat7, AMV-Meteosat10, AMV-GOES, AMV-MTSAT, AMV-MODIS, AMV-AVHRR, AMV-COMS, ATOVS(global, EARS, AP-RARS, SA-RARS), IASI, AIRS, GPSRO, CSR-COMS, CSR-Meteosat7, CSR-Seviri, CSR-GOES-W/E, ATMS, CrIS |
| Data Base | ODB(Observation Data Base) from ECMWF |
| Pre-process | OPS(Observation Processing System): Quality control and reformation of observation data for data assimilation |

KMA’s global data assimilation system has been upgraded every year since KMA started operation of the Unified Model system (2010) introduced from the UK Met Office. The followings show major improvements in global data assimilation system in 2015.

- Hybrid data assimilation system based on 4D-VAR will be upgraded to use 49 members from the ensemble forecast system in June 2016.

- Stability of PF model in 4D-VAR was enhanced with introduction of new version of data assimilation enhancing the stability of 4D-VAR calculation, especially in polar region.

- Clear Sky Radiance (CSR) data of Water Vapour channels from geostationary satellite were newly introduced.

- The radiances data of ATMS and CrIS from Suomi-NPP satellite started to be used in operation

- In 2015, TC (Tropical Cyclone) bogus system was improved (The improved TC bogus system was applied in the operational model in June 2015). There were two key points.

- More strict condition to turn on TC bogus scheme was applied. TC bogus programs were changed not to be activated when TC intensity is weak (i.e. central pressure of observed TC is over 990hPa or higher than background value).

- More generous QC for bogus data was applied. QC programs were modified to use all bogus point values.

Table 4.2 Configuration of surface analysis in operation for the global NWP suite

| Land Surface Model(LSM) | JULES |
| --- | --- |
| Analysis method | Extended Kalman Filter (EKF) for soil moisture contents |

KMA’s global data assimilation system has adopted JULES as LSM and Extended Kalman Filter (EKF) for surface analysis of soil moisture contents.

- EKF uses both of background error of model and observed error of soil moisture contents and gives optimized soil moisture and soil temperature in analysis.

- JULES model was applied to surface analysis process. JULES is being used to get H-matrix by ensemble which consists of 4 layered soil moisture and soil temperature and skin temperature.

4.2.1.2 Research performed in this field

      - Impact test of COMS high-resolution AMV

The higher resolution AMV of COMS (about 64 km resolution) has been tested in the KMA global system. In the preliminary results, the forecasting errors were reduced compared with the operation cycle (using 96 km resolution data) in winter season, but slightly increased in summer season. For better use of the higher resolution AMV, the blacklisting strategy and error profile were tuned : two ways of QC were compared as follows.

- Case1 : strict and complex QC – observation data was selected by considering both QI and error statistics together

- Case2 : loose and simple QC – all AMV with QI > 80 were used

As the results with more strict QC (Case1), the forecasting errors were decreased especially in the Asian region showing better background fit to observation (sonde and CSR as well as AMV).  In the experiment with the simple QC (Case2), the background fit to observation became worse especially in the mid-level atmosphere, and the forecast impact was negative due to the grown possibility to assimilate bad quality AMV.

4.2.2	Model

4.2.2.1 In operation

Table 4.3 Key model parameters of global model (GDAPS [UM N512L70]) for medium-range forecast

| Dynamics |  |
| --- | --- |
| Basic equation | Non-hydrostatic finite difference model with full equation. |
| Prognostic variables | Horizontal and vertical wind components, potential temperature, pressure, density, specific humidity, specific cloud water. |
| Integration domain | Global |
| Horizontal grid | Spherical latitude-longitude gird with Arakawa C-grid staggering of variables. Resolution : 0.234° latitude and 0.352° longitude (~25km) |
| Vertical grid | 70 levels (surface~80km). Hybrid-η vertical coordinate with Charney-Phillips grid staggering of variables. |
| Time integration | Two time-level semi-Lagrangian advection with a pressure correction semi-implicit time stepping method using a Helmholtz solver to include non-hydrostatic terms. Model time step = 600 sec. |
| Forecast range | 288 hours. |
| Physics |  |
| Horizontal diffusion | Second-order diffusion of winds, specific humidity and potential temperature. |
| Vertical diffusion | Second-order diffusion of winds only between 500 and 150 hPa in the tropics (equatorward of 30°). |
| Cloud | Prognostic cloud fraction and condensate cloud scheme (PC2, Wilson et al, 2008). |
| Precipitation | Wilson and Ballard (1999) single-moment bulk microphysics scheme, coupled with the PC2 cloud scheme. Prognostic rain Abel and Shipway (2007) rain fall speeds |
| Convection | Modified mass-flux convection scheme with convective available potential energy (CAPE) closure, momentum transports and convective anvils based on Gregory and Rowntree (1990). |
| Radiation | Edwards-Slingo (1996) radiation scheme with non-spherical ice spectral files. 6 absorption bands in the SW, and 9 bands in the LW. |
| Boundary Layer | First order non-local boundary layer scheme of based on Lock et al. (2000) |
| Gravity wave drag | Orographic scheme including a flow blocking scheme which represents the effects of sub-grid orography. Non-orographic spectral scheme which represents the effect of gravity waves in the stratosphere and mesosphere. |
| Land surface | Joint UK Land Environment Simulator (JULES) 4 layer soil model using van Genuchten (1980) soil hydrology |

4.2.2.2 Research performed in this field

- Dynamics test

- Model dynamic core of Unified Model upgrade test : New Dynamics → ENDGame

- Model horizontal resolution test : N512 (25km) → N768 (17km)

- Numerical noise test associated with orography in high altitude area

- Physics test

- Upgrade aerosols climatology : Direct radiative effect of aerosol species

- Refinement of surface / soil ancillaries

- Upgrade leaf area index (LAI)

4.2.3	Operationally available Numerical Weather Prediction Products

Operational global products routinely available on model’s original horizontal resolution in GRIB-II format are as follow

- Pressure level data : 3-hourly up to +84 hours, 6 hourly for +90~+288 hours

  - 3-dimensional wind components : 26 levels (1000~0.4 hPa)

  - Geopotential height : 26 levels (1000~0.4 hPa)

  - Temperature : 26 levels (1000~0.4 hPa)

  - Relative humidity w.r.t. ice/water : 19 levels (1000~10 hPa)

  - Wet bulb temperature : 850/700/500 hPa

- Single level or soil layer data

  - 99 single level or soil layer variables are also operationally available as 3-hourly data, which are instant values or accumulated/time-averaged/maximum/minimum values.

- Graphical products

  - Weather charts of basic products in the domain of Korea, East Asia and Northern hemisphere

  - Guidance charts for significant weather phenomena such as stability parameters, visibility etc.

  - Guidance at stations within Korea such as the Meteogram and the Skew T-Log P diagram

  - Thumbnail views for the accumulated precipitation

4.2.4	Operational techniques for application of NWP products (MOS, PPM, KF, Expert Systems, etc.)

4.2.4.1 In operation

The application techniques for both the medium- and short-range forecasting systems are described in section 4.3.4.1.

4.2.4.2 Research performed in this field

Nothing to report.

4.2.5	Ensemble Prediction System (EPS)

4.2.5.1 In operation

KMA medium range global ensemble forecast system which is based on the UK Met Office Global and Regional Ensemble Prediction System (MOGREPS) has been running operationally since March 2011. After the new KMA ensemble prediction system based on MOGREPS, the version of UM and physic are updated and sea surface temperature (SST) perturbations in ETKF are added. The updated KMA hybrid 4-dVAR Ensemble Prediction System has been run from 29 June 2013. It runs 4 times a day up to 12 days at 00 and 12 UTC to support weekly forecasts and runs up to 9 hours at 06 and 18UTC to provide ensemble background error statistics to hybrid ensemble-4dVar data assimilation system.

KMA participates in the WMO THORPEX TIGGE program and provides operational global ensemble products to the TIGGE archive centres. The standard verification scores of EPS are regularly exchanged through the EPS verification website hosted by JMA.

Table 4.3 Configuration of the global ensemble prediction system for medium-range as of 2015.

|  | Ensemble Prediction System for Global (EPSG) |
| --- | --- |
| Period | 29. June. 2013 ~ |
| Horizontal resolution | N320 (0.5625x0.375 degrees) |
| Vertical resolution (model top) | 70 levels (80km) |
| Forecast model | Deterministic Global Data Assimilation and Prediction System(GDAPS) |
| Model version and physics | UM vn7.9 / PS28(GA3.1) |
| Data assimilation for control analysis | The hybrid Ensemble-4DVAR using Global deterministic forecast for background field |
| Forecast length | 12 days at 00, 12 UTC / 9hours at 06, 18 UTC |
| Number of runs per day (time) | 4 (00, 06, 12, 18UTC) |
| Initial perturbation method (area) | ETKF(Ensemble Transform Kalman Filter) |
| Initial SST perturbation | ETKF |
| Ensemble size | 24 members (1 control + 23 perturbed members) per run |
| Perturbation of physics | Random Paramter(RP), Stochastic Kinetic Energy Backscattering(SKEB2) |
| Ancillary | CAP7.7(aerosol, ozone, soil, vegtation, etc) |

4.2.5.2 Research performed in this field

Upgrade for new operational system in 2016

•	 Model dynamic core of Unified Model upgrade test : New Dynamics → ENDGame

•	 Model horizontal resolution test : N320 (40km) → N400 (32km)

•	 Number of ensemble members : 1+23 → 1+24+24

The upgrade in dynamic core (ENDGame), model version, physics, and surface perturbations for new KMA EPS showed a slight improvement in RMSEs and CRPSS of geopotential height at 500hPa level (Fig. 4.1).

Fig. 4.1 The root mean squared errors and continuous rank probability skill score (CRPSS) of 500 hPa geopotential height over the northern hemisphere.

The errors and spread in the low level temperature and upper level wind are not significant (figure not shown). The upgraded global ensemble system showed improved precipitation forecast in convective area over the Tropical region. Upgraded new system reproduced well the rainfall amount over the western Pacific area and narrow band shape over the eastern Pacific area compared with GPM observation (Fig. 4.2).

Fig. 4.2 The simulated monthly accumulated rainfall amounts for the upgraded system (PS03), current system (PS01), and GPM observed rainfall amount. Right panel shows the biases and differences.

4.2.5.3 Operationally available EPS Products

The following products from global EPS are available:

- Mean and Spread for MSLP, geopotential height at 500hPa, and temperature at 850hPa

- Spread and Spaghetti for geopotential height at 500hPa

- Stamp map for geopotential height at 500hPa, MSLP and precipitation

- Probability of precipitation for several thresholds

- Ensemble meteogram of temperature, total cloud amount, precipitation, wind speed at 10m, and surface air temperature for 25 major cities

- Ensemble meteogram of surface air temperature corrected bias for 20 cities

- Extreme Forecast Index (EFI) for daily rainfall, wind gust, maximum and minimum temperature

- Probability of Tropical Cyclone genesis and existence in monitoring area

  - Short-range forecasting system (0-72 hrs)

4.3.1	Data assimilation, objective analysis and initialization

4.3.1.1 In operation

4DVAR has been used as a main data assimilation system for the limited area model based on the Unified Model which covers the East Asia domain since it was introduced to the operational suite in 2011. Most of the observation species assimilated in the global NWP system are also available for the regional system. KMA’s other operational regional NWP model, WRF 10kmL40, was retired in order to focus more resources on UM-based regional and local NWP system.

Table 4.4 Configuration of operational data assimilation for the short-range NWP suite as of 2015

|  | RDAPS (UM 12kmL70) |
| --- | --- |
| Data assimilation | 4DVAR |
| Initial condition | From regional 4DVAR |
| Boundary condition | From the Global UM |
| Number of vertical layers | 70 |
| Domain top | 80km |
| Horizontal grids | 540 X 432 |
| Horizontal resolution of analysis | 12km |
| Inner loop resolution | 36km 70 layers |
| Initialization | - |
| Domain | East Asia |

In 2015, several key elements in KMA's regional 4DVAR system and model were modified to provide better forecast. In spite of general improvement of performance that regional model showed since KMA introduce Unified Model from UKMO, regional model sometimes produced wrong forecast for the specific severe weather case. The wrong forecast cases were reviewed and typhoon bogus system, surface analysis and satellite data thinning were changed based on review results. Typhoon bogus points were optimally relocated and QC criteria for bogus data were changed. Also, thinning ranges for ATOVS, IASI and AIRS were changed based on sensitivity test results. Regarding surface analysis, soil moisture input data were more frequently updated from once a day to 4 times a day. With these improvements, model performance was improved about 4.9 % maximally and typhoon forecasts for Nakri and Halong case were greatly improved. These improvements were applied operationally from Jun. 2015.

4.3.1.2 Research performed in this field

Nothing to report

4.3.2	Model

4.3.2.1 In operation

KWRF (based on WRF) is dropped from operation in 2015 to focus on RDAPS which showed an appropriate performance in short-range forecast. KMA has been operating RDAPS based on the Unified Model with 12-km resolution and the implementation of 4DVAR since in May 2010. The model with 12km horizontal resolution, 70 vertical layers (top~80km) provides 87-hour prediction 4 times a day. Table 4.5 shows configuration of RDAPS.

- Major changes in the operational regional model :

- Use 8 types of aerosol climatology in radiation scheme

- Ancillary data set update : use new 4km MODIS LAI climatology

- Improvement of heat and moisture fluxes

- Improve the basic geographic height and soil data in ancillary data set

Table 4.5 Key model parameters of regional models (UM 12kmL70) for short-range forecast

|  | RDAPS (UM 12km L70) |
| --- | --- |
| Base Model | UM(Unified Model) |
| Governing Equation | Complete equation (Non-hydrostatic) |
| Horizontal Resolution | 12km(0.11x0.11)/540x432 |
| Number of vertical layers | 70 |
| Model time step | 200sec |
| Forecast Length | 3days(87hours) |
| Initial condition | From regional 4DVAR |
| Time integration | Semi-implicit Semi-Lagrangian scheme |
| Radiation Process | Edwards-Slingo general 2-stream scheme |
| Surface Process | Joint UK Land Environment Simulator (JULES) 4 layer soil model using van Genuchten (1980) soil hydrology |
| PBL Process | First order non-local boundary layer scheme of based on Lock et al. (2000) |
| Convection Process | Mass flux convection with CAPE closure |
| Microphysics | Mixed-phase precipitation |
| Gravity Wave Drag | G.W. drag due to orography (GWDO) |
| Surface B. C. | Surface Analysis + Climatology |
| Operation Frequency | Four times daily / 6hour D.A. cycle |

4.3.2.2 Research performed in this field

To correspond with the new version of global model, GDAPS, the dynamic core and physics processes of RDAPS needed to be updated and test its performance. The verification results of RDAPS with new dynamic and physics systems, showed a comparable result with the operational RDAPS.

4.3.3	Operationally available NWP products

Operational RDAPS (Regional Data Assimilation and Prediction System) products routinely available on model’s original horizontal resolution in GRIB-II format are as follow:

- Pressure level data : 3-hourly up to +87 hours

  - 3-dimensional wind components : 24 levels (1000~50hPa)

  - Geopotential height : 24 levels (1000~50hPa)

  - Temperature : 24 levels (1000~50hPa)

  - Relative humidity w.r.t. ice/water : : 24 levels (1000~50hPa)

  - Wet bulb temperature : 850/700/500hPa

  - Ertel Potential Vorticity at theta surface : 330/315/300K

- Single level or soil layer data

  - 101 single level or soil layer variables are also operationally available as 3-hourly data, which are instant values or accumulated/time-averaged/maximum/minimum values.

- Graphical products

- Same as in the section 4.2.3

4.3.4 Operational techniques for application of NWP products

4.3.4.1 In operation

For medium-range forecast, MOS, PPM and Kalman filter scheme based on GDAPS has been used to provide temperature guidance for 19 major cities. Ensemble MOS (EMOS) based global EPS (UM N320L70M24) was newly implemented in 2014. EMOS provides daily Max/Min temperature and cloud cover up to 12 days for synoptic stations and airports. EMOS products were displayed in form of EPS meteograms.

For non-observational location, SSPS (site-specific processing system) has been operated. SSPS is one of packages of UKPP in MetOffice and extract guidance of specific location with NWP grid data. SSPS in KMA provides mountain guidance with RDAPS prediction.

Table 4.6 Operational statistical techniques for application of NWP

| NWP Model | Element | Techniques | Basis | Projection | Interval |
| --- | --- | --- | --- | --- | --- |
| RDAPS (UM 12km) | 2m temperature | MOS, KF | 00, 12 UTC | +6 ~ +87h | 3h |
|  | Daily max/min temperature | MOS | 00, 12 UTC | +3day | 1d |
|  | Total cloud cover | MOS, PPM | 00, 12 UTC | +6 ~ +87h | 3h |
|  | Probability of precipitation over 0.1mm | MOS | 00, 12 UTC | +6 ~ +87h | 3h |
|  | Type of precipitation | MOS, PPM | 00, 12 UTC | +6 ~ +87h | 3h |
|  | 2m relative humidity | MOS | 00, 12 UTC | +6 ~ +87h | 3h |
|  | 10m wind direction/speed | MOS | 00, 12 UTC | +6 ~ +87h | 3h |
| GDAPS (UM N512) | Daily max/min temperature | MOS,KF | 00, 12 UTC | +12day | 1d |
|  | Type of precipitation | PPM | 00, 12 UTC | +12day | 12h |
|  | Total cloud cover | PPM | 00, 12 UTC | +12day | 12h |
| EPSG (N320) | Daily max/min temperature | EMOS | 00, 12 UTC | +12day | 1d |
|  | Total cloud cover | EMOS | 00, 12 UTC | +12day | 12h |

4.3.4.2 Research performed in this field

Though a statistical approach outperforms the use of direct model output and is competitive with subjective official forecasts, well-trained MOS prediction is not always superior to the dynamic model. MOS tends to remove the typical bias in the training data as MOS is trained in statistical way. A statistical blending technique was developed to get additional improvement. The screen temperature and relative humidity predictions from three dynamic models and theirs MOS techniques were blended by a weighted average method. The screen temperature after the blending showed a 10% additional improvement compared with the full MOS prediction. In order to remove jumpiness according to model initial time, the current blended temperature prediction was combined with the previous 12 hour blended temperature prediction. This double-blending approach showed slight improvements in overall scores.

4.3.5	Ensemble Prediction System

4.3.5.1 In operation

KMA has been running the short-range ensemble prediction system (LENS) operationally since October 2015 to provide guidance for early warning by providing a probabilistic prediction of high impact weather. The system was designed to forecast 69 hours at every 12 hours based on enhanced KMA global ensemble prediction system (EPSG) via downscaling of initial perturbation. The LENS consists of one control member and 12 perturbation members. It showed overall better performance in precipitation forecast than the deterministic high resolution local area model. (Fig.4.4)

Fig. 4.3 The model domain of short-range ensemble prediction system (LENS)

Fig. 4.4 The equitable threat score with respect to precipitation threshold value for the deterministic high resolution model (LDPS) and LENS during the one month period of July 2015.

4.3.5.2 Research performed in this field

Nothing to report

4.3.5.3 Operationally available EPS Products

The following products from the LENS are available:

- Mean and Spread for MSLP and 1.5m temperature

- Probability of precipitation, snow, windgust, and visibility for several thresholds

- Ensemble meteogram of temperature, total cloud amount, precipitation, wind speed at 10m, and surface air temperature for 25 major cities

- Probability matched precipitation amounts

  - Short-range Local Forecasting system (0-24 hrs)

    - Data assimilation, objective analysis and initialization

4.4.1.1 In operation

All observations within 1 hour 30 minutes time range from analysis time are assimilated by incremental 3DVAR. The analysis increments are computed on the 3km horizontal resolution inner grid by minimizing the cost function, which measures the differences from the model forecasts and observations. The rain rate data derived from radar reflectivity and rain gauge observations with 10 minute interval is added to be assimilated via latent heat nudging. The incremental analysis update(IAU) is applied to remove noises in analysis increments.

4.4.1.2 Research performed in this field

Impact of ground-based GNSS data assimilation is assessed using 1.5km high resolution model. KMA receives raw signal data from several GNSS networks of around 100 stations over Korea peninsular and generates Zenith Total Delay (ZTD) observation of 15 stations in near-real time. The assimilation of ground-based GNSS data gives a positive impact on the lower tropospheric humidity field in early forecast times and significant improvements for heavy rainfall cases in the summer. It will be implemented operationally before the end of 2016. To make better use of the ground-based GNSS data, new bias correction method has been investigated based on long-term monitoring results. An initial trial shows a slightly positive impact on quantitative precipitation forecasts.

4.4.2	Model

4.4.2.1 In operation

KMA started running LDAPS (UM 1.5kmL70) operationally on 15 May 2012 to produce detailed short-range weather prediction for the Korean peninsula. The model with 1.5km horizontal resolution, 70 vertical layers (top~40km) provides 36-hour prediction 4 times a day. Table 4.7 shows configuration of the LDAPS model.

- Major changes in the operational local model(LDAPS) :

- Update aerosol type in radiation scheme

- Add graupel as the prognostic variables in LS precipitation scheme

- Improve the size distribution of ice particles and rain in LS precipitation scheme

- Use the murk aerosol information to calculate droplet number in LS precipitation scheme

- Include fractional snow cover for sublimation and melting process in land surface scheme

Table 4.7 Key model parameters LDAPS (UM 1.5kmL70) for short-range local forecast

| Horizontal grid | Variable grid (Inner: 1.5km, 0.0135X0.0135) |
| --- | --- |
| Vertical grid | 70 levels (surface~39km). |
| Time step | 50 sec |
| Forecast range | 36 hours. |
| I.C / Data assimilation | 3DVAR ( FGAT, IAU) |
| Boundary condition | From the global model |
| Surface B. C. | Surface analysis + Climatology |
| Operation Frequency | 8 times daily / 3-hourly D.A. cycle |
| Time integration | Semi-implicit Semi-Lagrangian scheme |
| Radiation Process | Edwards-Slingo general 2-stream scheme |
| Surface Process | Joint UK Land Environment Simulator (JULES) 4 layer soil model using van Genuchten (1980) soil hydrology |
| PBL Process | First order non-local boundary layer scheme of based on Lock et al. (2000) |
| Microphysics | Mixed-phase precipitation |
| Gravity Wave Drag | G.W. drag due to orography (GWDO) |

4.4.2.2 Research performed in this field

By applying the extended-domain with the variable grid of LDAPS, the sensitivity test was performed for the improvement of synoptic-scale simulation and the precipitation prediction. The extended-domain LDAPS which had more lager forecast domains with the same data assimilation system and physics processes as the operational LDAPS showed better performances in simulations of precipitation and regional forecast over the Korean peninsula. The extended-domain of the variable grid in LDAPS is planned for the future release of operational LDAPS.

4.4.3	Operationally available NWP products

Operational LDAPS products routinely available on model’s original horizontal resolution in GRIB-II format are as follow

- Pressure level data : 1-hourly up to +36 hours

  - 3-dimensional wind components : 24 levels (1000~50hPa)

  - Geopotential height : 24 levels (1000~50hPa)

  - Temperature : 24 levels (1000~50hPa)

  - Relative humidity w.r.t. ice/water : : 24 levels (1000~50hPa)

- Single level or soil layer data

- 78 single level or soil layer variables are also operationally available as 1-hourly data, which are instant values or accumulated/time-averaged/maximum/minimum values.

- Graphical products

- Same as in the section 4.2.3

4.4.4 Operational techniques for application of NWP products

4.4.4.1 In operation

Nothing to report

4.4.4.2 Research performed in this field

Nothing to report

4.4.5	Ensemble Prediction System

4.4.5.1 In operation

Nothing to report

4.4.5.2 Research performed in this field

Nothing to report

4.4.5.3 Operationally available EPS Products

None

  - Nowcasting and Very Short-range Forecasting Systems (0-6 hrs)

4.5.1	Nowcasting System

4.5.1.1 In operation

The Lagrangian extrapolation based nowcasting system at KMA provides sky condition, precipitation, and lightning forecasts up to 6 hour with 15 minute interval for sky conditions and 10 minute interval for precipitation and lighting data.

Table 4.8 Nowcasting forecast products

| Type | Forecast products | interval |
| --- | --- | --- |
| Satellite | Moisture image | 15 min |
|  | Ultraviolet image |  |
|  | Cloud amount |  |
| Radar | Precipitation amount | 10 min |
|  | Precipitation types |  |
| Lightning | Distribution of lightning |  |
|  | Probability of lightning |  |

4.5.1.2 Research performed in this field

Nothing to report

4.5.2	Models for Very Short-range Forecasting Systems

4.5.2.1 In operation

The operational very short range forecast system is called Korea Local Analysis and Prediction System (KLAPS), and runs every hour with 5-km horizontal resolution. The lateral boundary conditions are provided from the 15-km resolution forecasts which 4 times a day.

The KLAPS has a cloud analysis and a diabatic initialization scheme for the very quick precipitation spin-up within a 1-h integration of numerical weather prediction model. The KLAPS also produces 9 current weather information (temperature, sky condition, precipitation amount, relative humidity, wind direction, speed, and precipitation types, and lightning) and 4 prediction elements (precipitation amount, sky condition, precipitation type, probability of lightning) at selected grid points.

Table 4.9 Operational configuration of KLAPS as of 2015

|  | Domain 1 | Domain 2 |
| --- | --- | --- |
| Horizontal resolution | 15km | 5km |
| Number of grid-points | 251 x 251 | 235 x 283 |
| Domain distance(km) | 10,800 x 10,800 | 3,150 x 3,150 |
| Vertical layers/Model Top | 40 sigma layers / 50 hPa |  |
| Time step for integration | 60s | 20s |
| Cumulus Parameterization | New Kain-Fritsch | None |
| Boundary layer | YSU PBL |  |
| Microphysics | WDM6 |  |
| Radiation | Dudhia & RRTM |  |
| Land-surface | 5 layer thermal diffusion scheme |  |
| Lateral Boundary data | GDAPS(UM N512L70) forecast data |  |

4.5.2.2 Research performed in this field

Nothing to report

  - Specialized Numerical Predictions

4.6.1	Tropical Cyclone Prediction System

4.6.1.1 In operation

Tropical Cyclone Centre location, maximum wind speed (Vmax), and minimum central pressure (Pmin) of the tropical cyclones are predicted from GDAPS(UM N512L70), RDAPS(UM 12kmL70), EPS(UM N320L70M24) and centre location are predicted from DBAR(Double Fourier Series BARotropic Typhoon Model) systems. Configuration of DBAR is described in Table 4.10.

Table 4.10 Key model parameters and operational specification of DBAR

| Basic equation | Depth integrated shallow water equation |
| --- | --- |
| Prognostic variables | Depth integrated Pressure, wind (U, V) |
| Integration domain | Global |
| Horizontal grid | Spectral model with equivalent grid spacing of 0.3516° latitude and longitude (1024 x 513) |
| Initial condition | GDAPS analysis (850, 700, 600, 500, 400, 300, 250, 200hPa mean) |
| TC vortex initialization | GFDL type bogussing |
| Forecast range | 72 hours, 4 times/day (00/06/12/18UTC) |

4.6.1.2 Research performed in this field

- TRUM (Typhoon Regional Unified Model) :

The UM-based typhoon prediction model TRUM is operationally tested in real time at the National Typhoon Centre. The dynamical initialization and GFDL-type bogussing are applied.

Table 4.11 Configuration of TRUM as of 2015

| Base Model | UM (Unified Model) version8.2 |
| --- | --- |
| Horizontal Resolution | 6km x 6km (1200 x 1000) |
| Number of Vertical Layers | 70 levels (surface~80km) |
| Time Step | 150s |
| TC vortex initialization | Dynamical initialization and GFDL-type bogussing |
| Surface Process | Joint UK Land Environment Simulator (JULES) 4 layer soil model using van Genuchten (1980) soil hydrology |
| PBL Process | First order non-local boundary layer scheme of based on Lock et al. (2000) |
| Cumulus parameterization | None |
| Gravity Wave Drag | G.W. drag due to orography (GWDO) |
| Initial and Lateral Boundary data | GDAPS(UM N512L70) forecast data |
| Forecast length | 72 hours, 4 times/day (00/06/12/18 UTC) |

4.6.2	Ocean Wind Wave Prediction System

4.6.2.1 In operation

The current operational ocean wind wave prediction systems are the global (GoWW3, 1/2°), the regional (ReWW3, 1/12°), the coastal (CoWW3, 1/120°) systems which employs the 3rd generation community wave model WAVEWATCH-III version 2.22. The operational cycle of 3 systems are two times per day (00, 12UTC) and the lead forecast times are 12 days for global, 87hours for regional and 72hours for coastal system. The mesh size of 1km with covering 5 regional forecast offices’ marine forecast area domains nested inside the regional system. The directional wave spectra at boundaries of coastal domain are provided from 1/12° ReWW3.

Table 4.12 Configuration of operational ocean wind wave prediction systems as of 2015

|  | GoWW3 | ReWW3 | CoWW3 (5 Domains) |
| --- | --- | --- | --- |
| Code | WAVEWATCH-III v. 2.22 |  |  |
| Coordinate | Spherical Coordinate |  |  |
| Domain | 70°S-70°N 0°E-358.75°E | 20°N-50°N 115°E-150°E | Daejeon: 36.0°-39.0°N, 123.0°-127.0°E (481x361) Gwangju: 33.0°-36.5°N, 124.0°-128.0°E (481x421) Busan: 33.5°-37.0°N, 127.0°-131.0°E (481x421) Kangwon: 36.5°-40.0°N, 127.0°-132.5°E (661x421) Jeju: 31.0°-35.5°N, 123.0°-129.0°E (721x541) |
| Spatial Resolution /Dimensions | 1/2° (720 x 281) | 1/12° (421 x 361) | 1/120° |
| Spectral Resolution | Frequency: 25 Direction: 24 | Frequency no. 25, Direction no.: 36 (10° interval) |  |
| Time Step | 720 sec | 300 sec | 30 sec |
| Forecast Length | 288 hours | 87 hours | 72 hours |
| Number of Runs | 2 times / day (00, 12 UTC) |  |  |
| Initial & Boundary Data | 12-hour forecast from the previous run |  | 12-hour forecast from the previous run ReWW3 boundary |
| Forcing Data | GDAPS (UM N512L70) Sea SFC Winds (6-hour interval) | RDAPS(UM 12kmL70) Sea SFC Winds (3-hour interval) |  |

Table 4.13 WAVEWATCH-III Grid pre-processor input information chosen for GoWW3

| Frequency increment factor and first frequency (Hz) | 1.1 0.04118 25 24 |
| --- | --- |
| Time steps (global / CFL for x-y / for k-theta / source) | 1440 / 720 / 1440 / 720 sec |
| Input Source Terms | Tolman and Chalikov / SIN2 SWELLF = 0.1, ZWND = 10., STABSH = 1.4 |
| Nonlinear interactions | Discrete Interaction Approximation (DIA) |
| Dissipation Source Terms & Bottom Friction | Tolman and Chalikov / JONSWAP SBT1 GAMMA = -0.019 |
| Propagation schemes | Ultimate-Quickest with averaging |
| Grid Definition | - Latitude-longitude grid (LLG): 720 (0.0) 281 (-70.0) - Bottom depth: 15m - Sub-grid information: none |

4.6.2.2 Research performed in this field

Next version of operational KMA wind wave prediction system are tested based on WAVEWATCH-III version 4.18. The main focus of new version are replaced by WAM cycle-4 scheme (ST4) for source term package.

4.6.3	Tide and Storm Surge Prediction System

4.6.3.1 In operation

The current operational Tide and Storm Surge Model are the regional (RTSM, 1/12°) and the coastal (CTSM, 1/120°) systems based on POM (Princeton Ocean Model) (Blumberg and Mellor, 1987). In this model, the level of storm surge calculated by the difference between tide level and sea level change caused by meteorological effects. Those systems have same operational cycle and resolutions of ocean wind wave prediction system

Table 4.14 Configuration of operational Tide and Storm Surge models as of 2015

|  | RTSM | CTSM(5 Domains) |
| --- | --- | --- |
| Model | 2-D Ocean Circulation Model (POM 2D) |  |
| Coordinate system | Spherical coordinate |  |
| Model domain | 115 E-150 E, 20 N-52 N | Daejeon: 36.0°-39.0°N, 123.0°-127.0°E (481x361) Gwangju: 33.0°-36.5°N, 124.0°-128.0°E (481x421) Busan: 33.5°-37.0°N, 127.0°-131.0°E (481x421) Kangwon: 36.5°-40.0°N, 127.0°-132.5°E (661x421) Jeju: 31.0°-35.5°N, 123.0°-129.0°E (721x541) |
| Horizontal resolution | 1/12 by 1/12 , (421 385) | 1/120° |
| T | 200 sec | 200 sec |
| Prediction time | 87 hour (00,12UTC) | 72 hours (00,12UTC) |
| Initial field | Hot start |  |
| Input data | RDAPS sea surface wind and MSLP, 8 tidal constituents |  |

4.6.3.2 Research performed in this field

Nothing to report

4.6.4	Asian Dust Prediction System

4.6.4.1 In operation

The UM-ADAM2(N512) model is upgraded to the ADAM2-Haze to deal with both Asian Dust and haze case. The configuration of ADAM2-Haze model is same with that of the UM-ADAM2 except for using the emission inventory data to predict the haze. Anthropogenic and biogenic emissions are implemented for haze initial condition. The ADAM2-Haze uses 2006 INTEX-Band 2008 CAPSS emissions as emission inventories for East Asia and Korea region. The ADAM2-Haze applies the data assimilation technique to improve the initial state of PM10 concentration. OI(Optimal Interpolation) is applied with surface PM10 data of more than 1500 observation sites operated in China and Korea.

- Major changes in the operational Asian dust model :

  - Prediction of haze

  - Data assimilation of surface PM10 concentrations

Table 4.15 Configuration of Asian dust prediction system as of 2015

| Model | ADAM2-Haze(N512) |
| --- | --- |
| Meteorological model | Operational global model (UM N512L70) |
| Model domain | East Asia |
| Horizontal/Vertical resolution | Horizontally 25km (with 340 x 220 grids) Vertically 49 layers (up to 100 hPa) |
| Prediction range | 72 hours (00/06/12/18UTC) |
| Dust size range | 0.2 ~ 74 (㎛ in diameter) |
| Dust size bins | 11 bins |
| Dust density | 2600 kg m-3 |
| Dust particle size distribution | Log normal distribution |
| Vegetation data | SPOT NDVI (1km resolution) |
| Domestic emission inventory | 2008 CAPSS |
| Foreign emission inventory | 2006 INTEX-B |
| Data Assimilation Method | Optimal Interpolation |
| Assimilated data | surface PM10 data |

4.5.4.2 Research performed in this field

72-hour forecasts for the wintertime in 2015 uses to evaluate the performance of the ADAM2-Haze model quantitatively. The results shows that the mean bias and RMSE are 4.8 and 25.3 ㎍/㎥, respectively, when compared with the surface PM10 concentrations observed in more than 30 stations of Korea.

  - Extended Range Forecasts (ERF) (10 days to 30 days)

4.7.1	Models

4.7.1.1 In operation

Nothing to report

4.7.1.2 Research performed in the field

Nothing to report

4.7.2	Operationally available NWP model and EPS LRF products

See 4.8.2

  - Long Range Forecasts (LRF) (30 days up to two years)

4.8.1	In operation

A new seasonal forecasting system of KMA was implemented operationally in Jan. 2014. This system, named the global seasonal forecasting system version5 (GloSea5), is a joint seasonal forecast system between KMA and Met-Office, UK and shares model results of each centre.

GloSea5 is the ensemble prediction system for seasonal forecasting and it is a fully coupled model with atmosphere, ocean, land surface and sea-ice components. The component models are coupled through the OASIS coupler. The atmospheric model is based on Met Office Unified Model(UM) and land surface model is JULES and their resolution is 0.833 × 0.556 degrees. The ocean model is based on NEMO, Sea ice model is CICE and the grid spacing is 1/4 degrees. The stochastic physics scheme - SKEB2  is used to generate ensemble spread.

- Initialisation

For the forecasting, the initial atmospheric conditions are generated by the KMA NWP 4DVAR(00Z) and the initial conditions of ocean and sea-ice are taken from Met-Office daily.

Two members run out to 235 days (for seasonal forecast) and two members run out to 63 days(for intra-seasonal forecast).  The first 60 days of all forecast members (four members) are used for intra-seasonal products.

ECMWF EAR-Interim data are used to initialize the atmosphere and land surface in the hindcast members. Also, UKMO reanalysis for ocean and sea-ice are used for hindcast. The hindcast period is 14 years from 1996 to 2009 and the hindcasts are initialized 4 times a month (1, 9, 17 and 25 in each calendar month). Hindcast set consists of three members per start date per year.

Table 4.15 Configuration of seasonal prediction system (GloSea5) as of 2015

| GloSea5 Model Configuration |  |
| --- | --- |
| Atmosphere | Met Office Unified Model (GA 3.0) N216L85 : 0.83x0.56 deg. 85layers |
| Ocean | Nucleus for European Modelling of the Ocean (GO 3.0), Orca Tri-polar grid at 0.25, 75layers |
| Land-Surface | Joint UK Land Environment Simulator(GL 3.0) |
| Sea-Ice | The Los Alamos Sea Ice Model(GSI 3.1) |
| Coupler | OASIS3 |

Fig. 4.5 GloSea5 operational schedule (forecast and hindcast) and composition of ensembles

4.8.2	Operationally available EPS LRF products

KMA LRF products are interpreted by forecasters and gridded data supplied for the WMO lead center and APCC Multi-Model Ensemble. Every week, LRF products are generated as anomalies relative to a model climatology and are calculated as weekly mean fields, monthly mean fields and 3-monthly mean fields respectively. Probabilities are produced for MSLP, 2m Temperature and precipitation and applied to the 1month, 3month and seasonal outlook.

- Verification of prognostic products

  - Annual verification summary

The summary of annual verification statistics for GDAPS is calculated by comparing the model forecast to the analysis and radiosonde observation (see Table 5.1 and 5.2). Table 5.3 present annual verification statistics of global ensemble prediction system (EPSG) against analysis. All verification scores are only for prediction from 1200 UTC initials.

Table 5.1 Annual scores against analysis in 2015

| VERIFICATION AGAINST ANALYSIS in 2015 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 24 hour | 72 hour | 120 hour |
| Northern Hemisphere | 500-hPa height RMSE (m) | 7.35 | 21.88 | 43.27 |
|  | Wind RMSE 250 hPa (m/s) | 3.87 | 8.15 | 13.09 |
| Southern Hemisphere | 500-hPa height RMSE (m) | 9.20 | 27.11 | 53.08 |
|  | Wind RMSE 250 hPa (m/s) | 4.14 | 8.82 | 14.14 |
| Tropics | Wind RMSE 850 hPa (m/s) | 1.96 | 3.03 | 3.77 |
|  | Wind RMSE 250 hPa (m/s) | 3.60 | 5.89 | 7.56 |

Table 5.2 Annual scores against radiosondes measurements in 2015

| VERIFICATION AGAINST RADIOSONDES in 2015 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 24 hour | 72 hour | 120 hour |
| Asia | 500-hPa height RMSE (m) | 12.15 | 20.62 | 33.53 |
|  | Wind 850 hPa (m/s) | 3.75 | 5.02 | 6.42 |
|  | Wind 250 hPa (m/s) | 5.57 | 8.69 | 12.20 |
| Australia New Zealand | 500-hPa height RMSE (m) | 12.78 | 22.73 | 39.37 |
|  | Wind 850 hPa (m/s) | 3.94 | 5.41 | 7.00 |
|  | Wind 250 hPa (m/s) | 5.33 | 8.61 | 12.52 |
| Europe | 500-hPa height RMSE (m) | 10.63 | 23.44 | 46.06 |
|  | Wind 850 hPa (m/s) | 3.59 | 5.03 | 6.85 |
|  | Wind 250 hPa (m/s) | 5.07 | 9.57 | 15.35 |
| North America | 500-hPa height RMSE (m) | 9.52 | 21.37 | 39.85 |
|  | Wind 850 hPa (m/s) | 3.97 | 5.55 | 7.49 |
|  | Wind 250 hPa (m/s) | 5.69 | 9.93 | 14.78 |
| Northern Hemisphere | 500-hPa height RMSE (m) | 11.64 | 23.13 | 42.47 |
|  | Wind 850 hPa (m/s) | 3.81 | 5.24 | 7.01 |
|  | Wind 250 hPa (m/s) | 5.34 | 9.14 | 13.82 |
| Southern Hemisphere | 500-hPa height RMSE (m) | 12.92 | 24.22 | 42.49 |
|  | Wind 850 hPa (m/s) | 4.36 | 5.65 | 7.30 |
|  | Wind 250 hPa (m/s) | 6.00 | 9.32 | 13.61 |
| Tropics | Wind 850 hPa (m/s) | 3.59 | 4.16 | 4.65 |
|  | Wind 250 hPa (m/s) | 5.10 | 6.80 | 8.62 |

Table 5.3 Annual scores against Global Ensemble Prediction System (EPSG) analysis in 2015

| EPS VERIFICATION AGAINST ANALYSIS in 2015 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 72 hour | 120 hour | 192 hour |
| Northern Hemisphere | 500-hPa height | Ensemble mean RMSE (m) | 24.67 | 44.65 | 70.4 |
|  |  | Spread/EM error (%) | 116.3 | 104.7 | 96.63 |
|  |  | Brier Skill Score +1.0 standard deviation | 0.80 | 0.61 | 0.33 |
|  | 850-hPa temperature | Ensemble mean RMSE (m) | 1.7 | 2.46 | 3.45 |
|  |  | Spread/EM error (%) | 95.29 | 95.93 | 93.04 |
|  |  | Brier Skill Score +1.0 standard deviation | 0.64 | 0.48 | 0.25 |

- Plans for the future (next 4 years)

  - Development of the GDPFS

6.1.1	Major changes in the operational DPFS which are expected in the next year

- Medium-range Forecasting System

  - A new dynamical core (ENDGame) of the Unified Model will be implemented and used for the operational NWP model with higher resolution (~17km) on the new supercomputer.

  - Improve large scale precipitation by including the effect of aerosol in the global model

  - Enhance observation data utilization : COMS satellite wind and European geostationary satellite clear sky radiation data

  - FY-3C/MWHS, Megha-Tropique/SAPHIR, GCOM-W/AMSR2 will be used in operation.

  - Variational Bias Correction scheme will be adopted.

- Short-range Forecasting System

  - Refinement of surface / soil ancillaries : new soil hydraulic properties (wilting and critical points), new soil thermal conductivity

  - Upgrade leaf area index (LAI) and model topography

  - Upgrade regional data assimilation and observation processing software

- Short-range Local Forecasting System

  - Development of extended domain Local Prediction System

  - Develop 4dVar and incorporate more local observation such as radar reflectivity

- Long-range Forecasting System

  - New configuration of GloSea5 (GC2) will be implemented.

6.1.2	Major changes in the operational DPFS which are envisaged within the next 4 years

- Introduction of New HPC(supercomputer)

  - A new supercomputer Cray XC40, which will substitute the current Cray XE6 will be introduced in KMA next year.

- NWP Models

  - Extended medium-range (2 weeks ~ 1 month) weather prediction system based on atmosphere-ocean couple model with ensemble approach will be developed and looked for feasibility.

  - Higher resolution (~10km) will be implemented on the operational NWP model.

  - Constant SST is being used for forecasting period currently. And the change of SST for medium-range will be applied in operation.

- Data Assimilation

  - 4dEnVar, which uses ensemble forecast system to keep the flow-dependency even without PF model, will be developed with the aim of its operational application.

  - COMS follow-on geostationary satellite will be tested in NWP system.

  - Planned research Activities in NWP, Nowcasting and Long-range Forecasting

6.2.1	Planned Research Activities in NWP

- Development of a new global NWP system

  - An R&D project for development of a new global NWP system in Korea started in 2011. KIAPS (Korea Institute of Atmospheric Prediction Systems) developed core modules for the new NWP system – dynamical core, physics parameterization package, observation pre-processing / data assimilation and their framework, etc. – and the prototype global NWP system will be developed by 2016, targeting the development of the first operational version to be finished by 2019.

  - Diurnal and semi-diurnal variation of NWP model will be investigated using operational model.

  - The evaluation and diagnostic tool which will be used to analyse main reason of degradation on global model according to leading time, is being developed.

  - Stratospheric bias of analysis field and its evolution depending on forecasting time will be investigated to enhance the performance of medium range forecast.

6.2.2	Planned Research Activities in Nowcasting

Nothing to report

6.2.3	Planned Research Activities in Long-range Forecasting

Nothing to report

- References

- http://www.wmolc.org/~GPC_Seoul : WMO LRF MME web site – contains information on KMA’s LRF system and produces

- http://web.kma.go.kr/eng/biz/forecast_02.jsp : Brief introduction of KMA’s NWP system

- http://super.kma.go.kr/eng/index.jsp : National Center for Meteorological Supercomputer web site – contains information on KMA’s supercomputer

- http://www.kiaps.org : KIAPS(Korea Institute of Atmospheric Prediction Systems) web site

ENDGame : Even Newer Dynamics for General atmospheric modelling of environment

ADAM : Asian Dust and Aerosol Model

Asian Dust and Haze Forecast Model

Intercontinental Chemical Transport Experiment

Clean Air Policy Support System