ANNEX

2.	MANUAL ON CODES: TABLE-DRIVEN CODE FORMS

- 2017-2.2.1(CM-I)/New GRIB2 Code table 4.2 entries [FT2017-2, Validation]

ADD:

in GRIB2 Code table 4.2 (FT2017-2),

Discipline 0 (Meteorological Products), Category 1 (Moisture)

| Number | Parameter | Unit |
| --- | --- | --- |
| 87 | Stratiform precipitation rate | kg m-2 s-1 |

Discipline 10 (Oceanographic Products), Category 1 (Currents)

| Number | Parameter | Unit |
| --- | --- | --- |
| 4 | Rip current occurrence probability | % |

Code tables 4.16/4.244 and PDT4.35 (Validation),

Code table 4.16 – Quality value associated with parameter

Code figure	Meaning

0	Confidence index (see Note 2)

1	Quality indicator (see Note 3 and Code table 4.244)

2	Correlation of product with used calibration product (see Note 4)

3	Standard deviation (see Note 5)

4	Random error (see Note 5)

5-191	Reserved

192-254	Reserved for local use

255	Missing

Notes:

(1)	When a non-missing value is used from this code table, the original data value is a quality value associated with the parameter defined by octets 10 and 11 of the product definition template.

(2)	The original data value is a non-dimensional number from 0 to 1, where 0 indicates no confidence and 1 indicates maximal confidence.

(3)	The original data value is defined by Code table 4.244.

(4)	The original data value is a non-dimensional number without units.

(5)	The original data value is in the same units as the parameter defined by octets 10 and 11 of the product definition template.

Code table 4.244 – Quality indicator

Code figure	Meaning

0	No quality information available

1	Failed

2	Passed

3-191	Reserved

192-254	Reserved for local use

255	Missing

Product definition template 4.35 – satellite product with or without associated quality values

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Observation generating process identifier (defined by originating centres)

14	Quality value associated with parameter (see Code Table 4.16)

15	Number of contributing spectral bands (NB)

16–   Repeat the following 11 octets for each contributing band (nb = 1, NB)

(16+11(nb–1))–(17+11(nb–1))	Satellite series of band nb (code table defined by originating/generating centre)

(18+11(nb–1))–(19+11(nb–1))	Satellite numbers of band nb (code table defined by originating/ generating centre)

(20+11(nb–1))–(21+11(nb–1))	Instrument types of band nb (code table defined by originating/generating centre)

(22+11(nb–1))	Scale factor of central wave number of band nb

(23+11(nb–1))–(26+11(nb–1))	Scaled value of central wave number of band nb (units: m–1)

Note:	For “satellite series of band nb”, “satellite numbers of band nb” and “instrument types of band nb”, it is recommended to encode the values as per BUFR Code tables 0 02 020, 0 01 007 (Common Code table C–5) and 0 02 019 (Common Code table C–8), respectively.

- 2017-2.2.2(CM-I)/New GRIB2 Code table 4.2 entry [FT2017-2]

ADD:

in GRIB2 Code table 4.2,

Discipline 0 (Meteorological Products), Category 1 (Moisture)

| Number | Parameter | Unit | Description |
| --- | --- | --- | --- |
| 121 | Fraction of snow cover | Proportion | Fraction (0-1) of the cell / grid-box occupied by snow. |

- 2017-2.2.3(CM-I)/New GRIB2 Code table 4.2 entries [FT2017-2]

ADD:

in GRIB2 Code table 4.2,

| Parameter | Product Discipline | Parameter Category | Parameter number | Units |
| --- | --- | --- | --- | --- |
| Highest freezing level | 0 | 19 (Physical atmospheric properties) | 32 | m |
| Visibility through liquid fog | 0 | 19 (Physical atmospheric properties) | 33 | m |
| Visibility through ice fog | 0 | 19 (Physical atmospheric properties) | 34 | m |
| Visibility through blowing snow | 0 | 19 (Physical atmospheric properties) | 35 | m |
| Categorical convective precipitation | 0 | 1 (Moisture) | 88 | Code table 4.222 |

- 2017-2.4.1(CM-I)/New BUFR entries for GPM precipitation data [FT2017-2]

ADD:

in BUFR Table D,

|  |  | (Global Precipitation Measurement (GPM) precipitation data) |  |
| --- | --- | --- | --- |
| 3 40 015 | 0 01 007 | Satellite identifier |  |
|  | 0 02 019 | Satellite instruments |  |
|  | 3 01 011 | Year, Month, Day |  |
|  | 3 01 012 | Hour, Minute |  |
|  | 0 04 007 | Seconds within a minute (microsecond accuracy) |  |
|  | 2 01 133 | Increase bit width |  |
|  | 0 05 041 | Scan line number |  |
|  | 2 01 000 | Cancel increase bit width |  |
|  | 2 01 130 | Increase bit width |  |
|  | 0 05 043 | Field of view number |  |
|  | 2 01 000 | Cancel increase bit width |  |
|  | 2 07 001 | Increase scale, reference value and data width |  |
|  | 0 05 002 | Latitude (coarse accuracy) |  |
|  | 0 06 002 | Longitude (coarse accuracy) |  |
|  | 2 07 000 | Cancel increase scale, reference value and data width |  |
|  | 0 40 027 | Sun glint angle |  |
|  | 0 13 040 | Surface flag |  |
|  | 0 21 120 | Probability of rain |  |
|  | 2 07 003 | Increase scale, reference value and data width |  |
|  | 1 02 003 | Repeat the following 2 descriptors 3 times |  |
|  | 0 02 186 | Capability to detect precipitation phenomena | 1st replication set bit #1 = unknown/unspecified (total precipitation); 2nd replication set bit #6 = solid precipitation; 3rd replication set bit #24 = convective precipitation |
|  | 0 13 155 | Intensity of precipitation (high accuracy) |  |
|  | 2 07 000 | Cancel increase scale, reference value and data width |  |
|  | 0 33 003 | Quality information |  |

in BUFR/CREX Code table 0 13 040 (Surface flag),

10              Standing water

11              Snow

AMEND:

in BUFR/CREX Flag table 0 02 186 (Capability to detect precipitation phenomena),

24	Convective precipitation

ADD:

in Common Code table C-13 under data category 12,

13	Precipitation

- 2017-2.4.2(CM-I)/New BUFR entries for radiosonde descent data [FT2017-2, Validation]

ADD:

in BUFR Table D (Validation),

|  |  | (Sequence for representation of radiosonde descent data) |  |
| --- | --- | --- | --- |
| 3 09 056 | 3 01 150 | WIGOS identifier |  |
|  | 0 02 011 | Radiosonde type |  |
|  | 0 02 013 | Solar and infrared radiation correction |  |
|  | 0 02 014 | Tracking technique/status of system used |  |
|  | 0 02 003 | Type of measuring equipment used |  |
|  | 3 01 128 | Additional information on radiosonde ascent | Valid also for decent |
|  | 3 01 113 | Date/time of launch |  |
|  | 0 08 091 | Coordinates significance | = 2 Start of observation |
|  | 3 01 021 | Latitude/longitude (high accuracy) |  |
|  | 0 07 007 | Height | Begin of descending of radiosonde above mean sea level |
|  | 0 08 091 | Coordinates significance | Set to missing (cancel) |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 002 | Extended delayed descriptor replication factor |  |
|  | 3 03 054 | Temperature, dewpoint and wind data at a pressure level with radiosonde position |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |

in Common Code table C-13 (FT2017-2),

COMMON CODE TABLE C–13:	Data sub-categories of categories defined by entries in BUFR Table A

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group	CREX Edition 2, mmm in Group Annnmmm
Annnmmm of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional alphanumeric
				codes are in brackets)

2	Vertical soundings (other	14	Upper-level temperature/humidity/wind reports from

than satellite)		descent radiosondes originally launched from

fixed-land stations

15	Upper-level temperature/humidity/wind reports from

descent radiosondes originally launched from ships

16	Upper-level temperature/humidity/wind reports from

descent radiosondes originally launched from

mobile land stations

- 2017-2.4.3(CM-I)/New BUFR entries for FY-3 VASS Products [FT2017-2, Validation]

ADD:

in Common Code table C-13 (FT2017-2),

COMMON CODE TABLE C–13:	Data sub-categories of categories defined by entries in BUFR Table A

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group	CREX Edition 2, mmm in Group Annnmmm
Annnmmm of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional alphanumeric
				codes are in brackets)

3	Vertical soundings	8	VASS (Vertical atmospheric sounding system)

(satellite)

in BUFR Table D (Validation),

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- |
| F X Y |  |  |
|  |  | (VASS MWTS report of FY-3) |
| 3 10 070 | 3 10 068 | VASS field of view variables |
|  | 1 01 013 | Replicate 1 descriptor 13 times |
|  | 3 10 069 | VASS channel variables |
|  |  | (VASS MWHS report of FY-3) |
| 3 10 071 | 3 10 068 | VASS field of view variables |
|  | 1 01 015 | Replicate 1 descriptor 15 times |
|  | 3 10 069 | VASS channel variables |
|  |  | (VASS IRAS report of FY-3) |
| 3 10 072 | 3 10 068 | VASS field of view variables |
|  | 1 01 026 | Replicate 1 descriptor 26 times |
|  | 3 10 069 | VASS channel variables |

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- |
| F X Y |  |  |
|  |  | VASS field of view variables |
| 3 10 068 | 0 08 070 | Vertical sounding product qualifier |
|  | 0 01 033 | Identification of originating/generating centre |
|  | 0 01 034 | Identification of originating/generating sub-centre |
|  | 0 01 007 | Satellite identifier |
|  | 0 02 019 | Satellite instruments |
|  | 0 12 064 | Instrument temperature |
|  | 0 05 040 | Orbit number |
|  | 2 01 136 | Increase bit width |
|  | 0 05 041 | Scan line number |
|  | 2 01 000 | Cancel increase bit width |
|  | 0 05 043 | Field of view number |
|  | 3 01 011 | Year, Month, Day |
|  | 3 01 012 | Hour, Minute |
|  | 2 01 138 | Increase bit width |
|  | 2 02 131 | Change scale |
|  | 0 04 006 | Second |
|  | 2 02 000 | Cancel change scale |
|  | 2 01 000 | Cancel increase bit width |
|  | 0 05 001 | Latitude (high accuracy) |
|  | 0 06 001 | Longitude (high accuracy) |
|  | 2 02 126 | Change scale |
|  | 0 07 001 | Height of station |
|  | 2 02 000 | Cancel change scale |
|  | 0 10 007 | Height |
|  | 0 07 024 | Satellite zenith angle |
|  | 0 05 021 | Bearing or azimuth |
|  | 0 07 025 | Solar zenith angle |
|  | 0 05 022 | Solar azimuth |
|  | 0 13 040 | Surface flag |
|  | 0 12 101 | Temperature/air temperature(land or ocean surface temperature) |
|  | 2 01 131 | Increase bit width |
|  | 2 02 129 | Change scale |
|  | 0 11 011 | Wind direction at 10 m (ocean surface wind) |
|  | 2 02 000 | Cancel change scale |
|  | 2 01 000 | Cancel increase bit width |
|  | 2 01 130 | Increase bit width |
|  | 2 02 129 | Change scale |
|  | 0 11 012 | Wind speed at 10 m (ocean surface wind) |
|  | 2 02 000 | Cancel change scale |
|  | 2 01 000 | Cancel increase bit width |
|  | 0 20 029 | Rain flag |
|  | 0 20 010 | Cloud cover (total) |
|  | 0 20 014 | Height of top of cloud |
|  | 0 13 162* | Cloud liquid water |
|  | 0 14 050 | Emissivity |

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- |
| F X Y |  |  |
|  |  | VASS channel variables |
| 3 10 069 | 0 05 042 | Channel number |
|  | 2 01 139 | Increase bit width |
|  | 0 02 155 | Satellite channel wavelength |
|  | 2 01 000 | Cancel increase bit width |
|  | 0 25 077 | Bandwidth correction coefficient 1 |
|  | 0 25 078 | Bandwidth correction coefficient 2 |
|  | 0 33 007 | Per cent confidence |
|  | 2 01 132 | Increase bit width |
|  | 2 02 129 | Change scale |
|  | 0 12 063 | Brightness temperature |
|  | 2 02 000 | Cancel change scale |
|  | 2 01 000 | Cancel increase bit width |

in BUFR/CREX Table B (Validation),

| TABLE REFERENCE F X Y | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 13 162 | Cloud liquid water | kg m–2 | 2 | 0 | 8 | kg m–2 | 2 | 3 |

in Code table 0 13 040 (FT2017-2),

0 13 040

Surface flag

| Code Figure |  |
| --- | --- |
| 7 | Inland water* |
| 8 | Snow cover |
| 9 | Sea ice |

_________________

*Inland water includes river, lake, wetland and swamp.

AMEND:

the element names in BUFR/CREX Table B (FT2017-2),

| TABLE REFERENCE F X Y | ELEMENT NAME |
| --- | --- |
| 0 08 070 | Vertical sounding product qualifier |
| 0 25 077 | Bandwidth correction coefficient 1 |
| 0 25 078 | Bandwidth correction coefficient 2 |

- 2017-2.4.4(CM-I)/BUFR template for surface observations from n-minute period [Validation]

ADD:

TM307092 - BUFR template for surface observations from n-minute period

|  |  | Station identification, time, horizontal and vertical coordinates | Unit, scale |
| --- | --- | --- | --- |
|  |  | WIGOS identifier |  |
| 3 01 150 | 0 01 125 | WIGOS identifier series | Numeric, 0 |
|  | 0 01 126 | WIGOS issuer of identifier | Numeric, 0 |
|  | 0 01 127 | WIGOS issue number | Numeric, 0 |
|  | 0 01 128 | WIGOS local identifier (character) | CCITT IA5 |
| 0 01 015 |  | Station or site name | CCITT IA5, 0 |
| 0 02 001 |  | Type of station | Code table, 0 |
|  |  | Year, month, day |  |
| 3 01 011 | 0 04 001 | Year(2) | Year, 0 |
|  | 0 04 002 | Month(2) | Month, 0 |
|  | 0 04 003 | Day(2) | Day, 0 |
|  |  | Hour, minute |  |
| 3 01 012 | 0 04 004 | Hour(2) | Hour, 0 |
|  | 0 04 005 | Minute(2) | Minute, 0 |
|  |  | Latitude/ longitude (high accuracy) |  |
| 3 01 021 | 0 05 001 | Latitude (high accuracy) | Degree, 5 |
|  | 0 06 001 | Longitude (high accuracy) | Degree, 5 |
| 0 07 030 |  | Height of station ground above mean sea level | m, 1 |
| 0 07 031 |  | Height of barometer above mean sea level | m, 1 |
| 0 01 023 |  | Observation sequence number | Numeric, 0 |
|  |  | Pressure |  |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 10 004 |  | Pressure | Pa, –1 |
| 0 10 051 |  | Pressure reduced to mean sea level | Pa, –1 |
| 0 07 004 |  | Pressure (standard level) | Pa, –1 |
| 0 10 009 |  | Geopotential height of the standard level | gpm, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
|  |  | Wind |  |
| 1 18 000 |  | Delayed replication of 18 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) | m, 2 |
| 0 02 002 |  | Type of instrumentation for wind measurement | Flag table, 0 |
| 0 08 021 |  | Time significance (= 2 Time averaged) | Code table, 0 |
| 0 04 025 |  | Time period or displacement (= –10 minutes, or number of minutes after a significant change of wind) | Minute, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 11 001 |  | Wind direction | Degree true, 0 |
| 0 11 002 |  | Wind speed | m s-1, 1 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 08 021 |  | Time significance (Set to missing) | Code table, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 11 043 |  | Maximum wind gust direction | Degree true, 0 |
| 0 11 041 |  | Maximum wind gust speed | m s-1, 1 |
| 0 11 016 |  | Extreme counterclockwise wind direction of a variable wind | Degree true, 0 |
| 0 11 017 |  | Extreme clockwise wind direction of a variable wind | Degree true, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 04 025 |  | Time period or displacement (= 0 minutes) | Minute, 0 |
|  |  | Temperature and humidity data |  |
| 1 11 000 |  | Delayed replication of 11 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) | m, 2 |
| 0 08 010 |  | Surface qualifier (temperature data) | Code table, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 12 101 |  | Temperature/air temperature (scale 2) | K, 2 |
| 0 12 103 |  | Dewpoint temperature (scale 2) | K, 2 |
| 0 13 009 |  | Relative humidity (original measured value) | %, 1 |
| 0 13 003 |  | Relative humidity | %, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) (Set to missing (Cancel)) | m, 2 |
| 0 08 010 |  | Surface qualifier (temperature data) (Set to missing (Cancel)) | Code table, 0 |
|  |  | Soil temperature and soil moisture |  |
| 1 07 000 |  | Delayed replication of 7 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 0 07 061 |  | Depth below land surface | m, 2 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 12 130 |  | Soil temperature | K, 2 |
| 0 13 111 |  | Soil moisture | g kg-1, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 07 061 |  | Depth below land surface (Set to missing (Cancel)) | m, 2 |
|  |  | Visibility |  |
| 1 07 000 |  | Delayed replication of 7 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
|  |  | Visibility data |  |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) | m, 2 |
| 0 33 041 |  | Attribute of following value | # Code table, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 20 001 |  | Horizontal visibility | m, –1 |
| or |  |  |  |
| 0 15 051 |  | Meteorological optical range | m, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) (Set to missing (Cancel the previous value) | m, 2 |
|  |  | Cloud |  |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 073 |  | Cloud data |  |
|  | 0 20 010 | Cloud cover (total) | %, 0 |
|  | 1 05 004 | Replicate 5 descriptors four times |  |
|  | 0 08 002 | Vertical significance | Code table, 0 |
|  | 0 20 011 | Cloud amount | Code table, 0 |
|  | 0 20 012 | Cloud type | Code table, 0 |
|  | 0 33 041 | Attribute of following value | Code table, 0 |
|  | 0 20 013 | Height of base of cloud | m, –1 |
| 0 08 002 |  | Vertical significance (surface observations) (Set to missing) | Code table, 0 |
|  |  | Ice and State of the ground |  |
| 1 05 000 |  | Delayed replication of 5 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 20 031 |  | Ice deposit (thickness) | m, 2 |
| 0 20 032 |  | Rate of ice accretion (estimated) | Code table, 0 |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 1 04 000 |  | Delayed replication of 4 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 3 02 078 |  | State of ground and snow depth measurement |  |
|  | 0 02 176 | Method of state of ground measurement | Code table, 0 |
|  | 0 20 062 | State of ground (with or without snow) | Code table, 0 |
|  | 0 02 177 | Method of snow depth measurement | Code table, 0 |
|  | 0 13 013 | Total snow depth | m, 2 |
| 2 04 000 |  | Add associated field (Cancel) |  |
|  |  | Present weather |  |
| 1 14 000 |  | Delayed replication of 14 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 04 025 |  | Time period or displacement (= -n minutes) | Minute, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 20 003 |  | Present weather |  |
| 2 04 000 |  | Add associated field (Cancel) |  |
| 0 04 015 |  | Time increment (= -n minutes) |  |
| 0 04 025 |  | Time period or displacement (= -1 minutes) | Minute, 0 |
| 0 04 065 |  | Short time increment (= 1 minutes) |  |
| 1 04 000 |  | Delayed replication of 4 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor (n) | Numeric, 0 |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 20 003 |  | Present weather |  |
| 2 04 000 |  | Add associated field (Cancel) |  |
|  |  | Precipitation |  |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 079 |  | Precipitation measurement |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 02 175 | Method of precipitation measurement | Code table, 0 |
|  | 0 02 178 | Method of liquid water content measurement of Precipitation | Code table, 0 |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 13 011 | Total precipitation / total water equivalent of snow | kg m-2, 1 |
| 0 07 032 |  | Height of sensor above local ground (or deck of marine platform) (Set to missing (Cancel)) | m, 2 |
|  |  | Evaporation |  |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 080 |  | Evaporation measurement |  |
|  | 0 02 185 | # Method of evaporation measurement | Code table, 0 |
|  | 0 04 025 | Time period or displacement ( = - n minutes) | Minute, 0 |
|  | 0 13 033 | Evaporation /evapotranspiration | kg m-2, 1 |
|  |  | Sunshine and radiation |  |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 081 |  | Total sunshine data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 031 | Total sunshine | Minute, 0 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 082 |  | Radiation data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 002 | Long-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 004 | Short-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 016 | Net radiation, integrated over period specified | J m-2, -4 |
|  | 0 14 028 | Global solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  |  | Lightning |  |
| 1 05 000 |  | Delayed replication of 5 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 04 025 |  | Time period or displacement (= -n minutes) | Minute |
| 2 04 006 |  | Add associated field |  |
| 0 31 021 |  | Associated field significance (e.g. = 6) | Code table, 0 |
| 0 13 059 |  | Number of flashes (thunderstorm) | Numeric |
| 2 04 000 |  | Add associated field (Cancel) |  |

- WIGOS Station Identifiers shall be used for n-minute period observations.

- The time identification refers to the end of the n-minute period.

- The height above local ground 0 07 032 referring to ground temperature shall be considered as a variable. After a snowfall, the sensor is placed at the top of the snow layer and the changed value of 0 07 032 shall indicate this procedure (total snow depth is reported in 0 13 013).

- 2017-2.4.5(CM-I)/New BUFR sequence for describing satellites contributing to an observed geophysical quantity [Validation]

ADD:

in BUFR Table D,

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION |
| --- | --- | --- | --- |
| F XY Y |  |  |  |
| 3 04 038 | 1 08 000 | Delayed replication of 8 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | = 28 Start of scan |
|  | 3 01 011 | Year, month, day |  |
|  | 3 01 013 | Hour, minute, second |  |
|  | 0 08 021 | Time significance | = 29 End of scan |
|  | 3 01 011 | Year, month, day |  |
|  | 3 01 013 | Hour, minute, second |  |
|  | 0 01 007 | Satellite identifier |  |
|  | 0 02 019 | Satellite instruments |  |
|  | 0 08 021 | Time significance | Set to missing (cancel) |

- 2017-2.5.1(CM-I)/New Common Code table C-12 entries for CIMSS [FT2017-2]

ADD:

in Common Code table C-12 under the originating centre #176 (=CIMSS),

20	Honolulu (United States)

21	Gilmore Creek (United States)

22	Madison (United States)

23	Miami (United States)

24	Mayaguez (Puerto Rico)

25	Monterey (United States)

26	Guam

27	Corvallis (United States)

28	Hampton (United States)

29	New York City (United States)

- 2017-2.5.2(CM-I)/New Common Code table entries for Spire Global, Inc. [FT2017-2]

Add:

in Common Code tables C-1 and C-11,

178	Spire Global, Inc.

in Common Code table C-5,

269	Spire Lemur 3U CubeSat

in Common Code table C-8,

| Entry # | Agency | Type | Instrument short name | Instrument long name |
| --- | --- | --- | --- | --- |
| 530 | Spire | GNSS occultation sounder | SGNOS-A | Spire global navigation satellite system occultation sounder A |
| 531 | Spire | GNSS occultation sounder | SGNOS-B | Spire global navigation satellite system occultation sounder B |
| 532 | Spire | GNSS occultation sounder | SGNOS-C | Spire global navigation satellite system occultation sounder C |
| 533 | Spire | GNSS occultation sounder | SGNOS-D | Spire global navigation satellite system occultation sounder D |

- 2017-2.5.3(CM-I)/New entries in Common Code tables C-5 and C-8 by India [FT2017-2]

ADD:

in Common Code table C-5,

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6 	(Code table 0 01 007)	 Edition 2

473	473	473	INSAT 3DR

474	474	474	INSAT 3DS

855	855	855	Combination of INSAT 3D and INSAT 3DR

in Common Code table C-8,

Code	Agency	Type	Instrument short name	Instrument long name

289	ISRO	Optical imager	IMG	Imager

- 2017-2.5.4(CM-I)/Common Code table for master table version numbers of GRIB, BUFR and CREX [ABC2018]

AMEND:

Octet 10 of Section 1 in Specifications of Octet Contents of FM 92 GRIB to

10	GRIB master table version number (see Common Code table C–0 and Note 1)

Octet 14 of Section 1 in Specifications of Octet Contents of FM 94 BUFR to

14	BUFR master table version number (see Common Code table C–0 and Note 2)

vv and bb in Group No. 1 of Section 1 in Specifications of Sections of FM 95 CREX to

vv:	CREX master table version number (see Common Code table C–0)

bb:	BUFR master table version number used (see Common Code table C–0)

Notes 2 and 3 to Class 00 of BUFR/CREX Table B to

(2)	BUFR master table version numbers are described in Common Code table C–0 and Note 2 to Section 1 of BUFR regulations.

(3)	CREX master table version numbers are described in Common Code table C–0.

DELETE:

Note 5 to Section 1 of Specifications of Octet Contents of FM 94 BUFR,

Note 3 to Specifications of Sections of FM 95 CREX.

ADD:

a note to GRIB Code table 1.0,

Note: This code table is deprecated. See Common Code table C–0 instead.

Common Code table C–0,

COMMON CODE TABLE C–0: GRIB, BUFR and CREX master table version number

Octet 10 in Section 1 of GRIB Edition 2

Octet 14 in Section 1 of BUFR Edition 4

vv and bb in Group No. 1 in Section 1 of CREX Edition 2

COMMON CODE TABLE C–0: GRIB, BUFR and CREX master table version number

Version number

GRIB	BUFR	CREX	Effective date

0	0	0	Experimental

1		1 November 1988

2		1 November 1993

3		2 November 1994

4		8 November 1995

5		6 November 1996

6		5 November 1997

7		4 November 1998

8	1	3 May 2000

9		8 November 2000

1	10	2	7 November 2001

2	11	3	5 November 2003

3	12	4	2 November 2005

4	13	5	7 November 2007

5	14	6	4 November 2009

6	15	7	15 September 2010

7	16	16	4 May 2011

8	17	17	2 November 2011

9	18	18	2 May 2012

10	19	19	7 November 2012

11	20	20	8 May 2013

12	21	21	14 November 2013

13	22	22	7 May 2014

14	23	23	5 November 2014

15	24	24	6 May 2015

16	25	25	11 November 2015

17	26	26	4 May 2016

18	27	27	2 November 2016

19	28	28	3 May 2017

20	29	29	Pre-operational to be implemented by next

amendment

Notes:

(1)	Introduction of Common Code table C–0 is a legal initiative. WMO Members and other TDCF users could practically deal with the version numbers the same as before until their software becomes capable of referring to the common code table.

(2)	CREX master table version numbers 8–15 are not used.

(3)	In the case of BUFR and CREX, these version numbers apply to the master table 0.

- 2017-2.5.5(CM-I)/New entries to Common Code table C-3/BUFR Table 0 22 067 for Argo floats [FT2017-2]

ADD:

in Common Code table C-3 (Instrument make and type for water temperature profile measurement with fall rate equation coefficients),

Code figure for IxIxIx	Code figure for BUFR (Code table 0 22 067)	Meaning

Instrument make and type	Equation coefficients

a	b

835	835	PROVOR IV		Not applicable

836	836	PROVOR III		Not applicable

870	870	HM2000		Not applicable

871	871	COPEX		Not applicable

872	872	S2X		Not applicable

- 2017-2.5.6(CM-I)/New Common Code table entries in C-5 and C-8 for FY-4A [FT2017-2]

ADD:

in Common Code table C-5,

530		FY-4A

in Common Code table C-8,

Code	Agency	Type	Instrument short name	Instrument long name

961	CMA	Imaging multi-spectral	AGRI	Advanced Geosynchronous Radiation Imager
			radiometer

962	CMA	Atmospheric temperature	GIIRS	Geosynchronous Interferometric Infrared Sounder

and humidity sounder

963	CMA	High-resolution optical	LMI	Lightning Mapping Imager

imager

964	CMA	Space environment	SEP	Space Environment Package

in BUFR/CREX Code table 0 02 020,

383	FY-4

- 2017-2.5.7(CM-I)/New entry in Common Code table C-5 to include satellite identifier for Sentinel 3B [FT2017-2]

ADD:

in Common Code table C-5,

Code figure

65	Sentinel 3B

- 2017-2.6.1(CM-I)/Templates and Templates components for time intervals

# ADD:

# template component, templates and code tables,

# Time Domain Template Component 3.2 – Forecast time interval

| Byte No. | Contents |
| --- | --- |
| 1-2 | Hours of observational data cut-off relative to reference time (see Note 1) |
| 3 | Minutes of observational data cut-off relative to reference time |
| 4 | Type of statistical processing (code table 3.4) |
| 5 | Indicator of unit of time range (see Code table 3.3) |
| 6-9 | Start of interval relative to reference time in units defined by byte 5 |
| 10 | Indicator of unit of time range (see Code table 3.3) |
| 11-14 | Length of time range over which statistical processing is done, in units defined by byte 10 |
| 15-16 | Observable quantity units after statistical processing (code table 3.5) |

# Time Domain Template 3.2 – Forecast time interval

| Component Code | Component Name |
| --- | --- |
| 3.2 | Time interval |

# Time Domain Template 3.3 – Forecast two nested time intervals

| Component Code | Component Name |
| --- | --- |
| 3.2 | Time interval (see note 1) |
| 3.2 | Time interval |

# Time Domain Template 3.3 – Forecast three nested time intervals

| Component Code | Component Name |
| --- | --- |
| 3.2 | Time interval (see note 1) |
| 3.2 | Time interval (see note 1) |
| 3.2 | Time interval (see note 1) |

Note:

(1)	Outer time interval first.

# Code table 3.4 – Type of time statistical processing

Code figure	Meaning

0	Average

1	Accumulation (see Note 1)

2	Maximum

3	Minimum

4	Difference (value at the end of time range minus value at the beginning)

5	Root mean square

6	Standard deviation

7	Covariance (temporal variance) (see Note 2)

8	Difference (value at the start of time range minus value at the end)

9	Ratio

10	Standardized anomaly

11	Summation

12–191	Reserved

192–254	Reserved for local use

255	Missing

# Code table 3.5 – Observable quantity units after statistical processing

Code figure	Meaning

0	Unchanged

1	Multiply by time

65535	Missing

- 2017-2.6.2(CM-I)/Parameters table

Add:

Code table 7.3 – Parameter number by product discipline and parameter category

Notes:

(1)	By convention, the flux sign is positive if downwards.

(2)	When a new parameter is to be added to Code table 7.3 and more than one category applies, the choice of category should be made based on the intended use of the product. The discipline and category are an important part of any product definition, so it is possible to have the same parameter name in more than one category. For example, “water temperature” in discipline 10 (oceanographic products), category 4 (subsurface properties) is used for reporting water temperature in the ocean or open sea, and is not the same as “water temperature” in discipline 1 (hydrological products), category 2 (inland water and sediment properties), which is used for reporting water temperature in freshwater lakes and rivers.

Product discipline 0 – Meteorological products, parameter category 0: temperature

Number	Parameter	Units

0	Temperature	K

1	Virtual temperature	K

2	Potential temperature	K

3	Pseudo-adiabatic potential temperature	K
		or equivalent potential temperature

4	Maximum temperature*	K

5	Minimum temperature*	K

6	Dewpoint temperature	K

7	Dewpoint depression (or deficit)	K

8	Lapse rate	K m–1

9	Temperature anomaly	K

10	Latent heat net flux	W m–2

11	Sensible heat net flux	W m–2

12	Heat index	K

13	Wind chill factor	K

14	Minimum dewpoint depression*	K

15	Virtual potential temperature	K

16	Snow phase change heat flux	W m–2

17	Skin temperature	K

18	Snow temperature (top of snow)	K

19	Turbulent transfer coefficient for heat	Numeric

20	Turbulent diffusion coefficient for heat	m2 s–1

21	Apparent temperature**	K

22	Temperature tendency due to short-wave radiation	K s–1

23	Temperature tendency due to long-wave radiation	K s–1

24	Temperature tendency due to short-wave radiation, 	K s–1
		clear sky

25	Temperature tendency due to long-wave radiation, 	K s–1
		clear sky

26	Temperature tendency due to parameterization 	K s–1

27	Wet-bulb temperature	K

28–191	Reserved

192–254	Reserved for local use

255	Missing

_____________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	Apparent temperature is the perceived outdoor temperature, caused by a combination of phenomena, such as air temperature, relative humidity and wind speed.

Product discipline 0 – Meteorological products, parameter category 1: moisture

Number	Parameter	Units

0	Specific humidity	kg kg–1

1	Relative humidity	%

2	Humidity mixing ratio	kg kg–1

3	Precipitable water	kg m–2

4	Vapour pressure	Pa

5	Saturation deficit	Pa

6	Evaporation	kg m–2

7	Precipitation rate*	kg m–2 s–1

8	Total precipitation***	kg m–2

9	Large-scale precipitation (non-convective)***	kg m–2

10	Convective precipitation***	kg m–2

11	Snow depth	m

12	Snowfall rate water equivalent*	kg m–2 s–1

13	Water equivalent of accumulated snow depth***	kg m–2

14	Convective snow***	kg m–2

15	Large-scale snow***	kg m–2

16	Snow melt	kg m–2

17	Snow age	d

18	Absolute humidity	kg m–3

19	Precipitation type	(Code table 4.201)

20	Integrated liquid water	kg m–2

21	Condensate	kg kg–1

22	Cloud mixing ratio	kg kg–1

23	Ice water mixing ratio	kg kg–1

24	Rain mixing ratio	kg kg–1

25	Snow mixing ratio	kg kg–1

26	Horizontal moisture convergence	kg kg–1 s–1

27	Maximum relative humidity*	%

28	Maximum absolute humidity*	kg m–3

29	Total snowfall***	m

30	Precipitable water category	(Code table 4.202)

31	Hail	m

32	Graupel (snow pellets)	kg kg–1

33	Categorical rain	(Code table 4.222)

34	Categorical freezing rain	(Code table 4.222)

35	Categorical ice pellets	(Code table 4.222)

36	Categorical snow	(Code table 4.222)

37	Convective precipitation rate	kg m–2 s–1

38	Horizontal moisture divergence	kg kg–1 s–1

39	Per cent frozen precipitation	%

40	Potential evaporation	kg m–2

41	Potential evaporation rate	W m–2

42	Snow cover	%

43	Rain fraction of total cloud water	Proportion

44	Rime factor	Numeric

45	Total column integrated rain	kg m–2

46	Total column integrated snow	kg m–2

47	Large scale water precipitation (non-convective)***	kg m–2

48	Convective water precipitation***	kg m–2

49	Total water precipitation***	kg m–2

50	Total snow precipitation***	kg m–2

51	Total column water (Vertically integrated total water 	kg m–2

(vapour + cloud water/ice))

52	Total precipitation rate**	kg m–2 s–1

53	Total snowfall rate water equivalent**	kg m–2 s–1

54	Large scale precipitation rate	kg m–2 s–1

55	Convective snowfall rate water equivalent	kg m–2 s–1

56	Large scale snowfall rate water equivalent	kg m–2 s–1

57	Total snowfall rate	m s–1

58	Convective snowfall rate	m s–1

59	Large scale snowfall rate	m s–1

60	Snow depth water equivalent	kg m–2

61	Snow density	kg m–3

62	Snow evaporation	kg m–2

63	Reserved

64	Total column integrated water vapour	kg m–2

65	Rain precipitation rate	kg m–2 s–1

66	Snow precipitation rate	kg m–2 s–1

67	Freezing rain precipitation rate	kg m–2 s–1

68	Ice pellets precipitation rate	kg m–2 s–1

69	Total column integrated cloud water	kg m–2

70	Total column integrated cloud ice	kg m–2

71	Hail mixing ratio	kg kg–1

72	Total column integrated hail	kg m–2

73	Hail precipitation rate	kg m–2 s–1

74	Total column integrated graupel	kg m–2

75	Graupel (snow pellets) precipitation rate	kg m–2 s–1

76	Convective rain rate	kg m–2 s–1

77	Large scale rain rate	kg m–2 s–1

78	Total column integrated water (all components	kg m–2
		including precipitation)

79	Evaporation rate	kg m–2 s–1

80	Total condensate	kg kg–1

81	Total column-integrated condensate	kg m–2

82	Cloud ice mixing-ratio	kg kg–1

83	Specific cloud liquid water content	kg kg–1

84	Specific cloud ice water content	kg kg–1

85	Specific rainwater content	kg kg–1

86	Specific snow water content	kg kg–1

87–89	Reserved

90	Total kinematic moisture flux	kg kg–1 m s–1

91	u-component (zonal) kinematic moisture flux	kg kg–1 m s–1

92	v-component (meridional) kinematic moisture 	kg kg–1 m s–1
		flux

93	Relative humidity with respect to water	%

94	Relative humidity with respect to ice	%

95	Freezing or frozen precipitation rate	kg m–2 s–1

96	Mass density of rain	kg m–3

97	Mass density of snow	kg m–3

98	Mass density of graupel	kg m–3

99	Mass density of hail	kg m–3

100	Specific number concentration of rain	kg–1

101	Specific number concentration of snow	kg–1

102	Specific number concentration of graupel	kg–1

103	Specific number concentration of hail	kg–1

104	Number density of rain	m–3

105	Number density of snow	m–3

106	Number density of graupel	m–3

107	Number density of hail	m–3

108	Specific humidity tendency due to  	kg kg–1 s–1
		parameterization

109–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	Total precipitation/snowfall rate stands for the sum of convective and large-scale precipitation/snowfall rate.

***	Statistical process 1 (Accumulation) does not change units. It is recommended to use another parameter with “rate” in its name and accumulation in PDT.

Product discipline 0 – Meteorological products, parameter category 2: momentum

Number	Parameter	Units

0	Wind direction (from which blowing)	degree true

1	Wind speed	m s–1

2	u-component of wind	m s–1

3	v-component of wind	m s–1

4	Stream function	m2 s–1

5	Velocity potential	m2 s–1

6	Montgomery stream function	m2 s–2

7	Sigma coordinate vertical velocity	s–1

8	Vertical velocity (pressure)	Pa s–1

9	Vertical velocity (geometric)	m s–1

10	Absolute vorticity	s–1

11	Absolute divergence	s–1

12	Relative vorticity	s–1

13	Relative divergence	s–1

14	Potential vorticity	K m2 kg–1 s–1

15	Vertical u-component shear	s–1

16	Vertical v-component shear	s–1

17	Momentum flux, u-component	N m–2

18	Momentum flux, v-component	N m–2

19	Wind mixing energy	J

20	Boundary layer dissipation	W m–2

21	Maximum wind speed*	m s–1

22	Wind speed (gust)	m s–1

23	u-component of wind (gust)	m s–1

24	v-component of wind (gust)	m s–1

25	Vertical speed shear	s–1

26	Horizontal momentum flux	N m–2

27	u-component storm motion	m s–1

28	v-component storm motion	m s–1

29	Drag coefficient	Numeric

30	Frictional velocity	m s–1

31	Turbulent diffusion coefficient for momentum	m2 s–1

32	Eta coordinate vertical velocity	s–1

33	Wind fetch	m

34	Normal wind component**	m s–1

35	Tangential wind component**	m s–1

36	Amplitude function for Rossby wave envelope 	m s–1
		for meridional wind***

37	Northward turbulent surface stress****	N m–2 s

38	Eastward turbulent surface stress****	N m–2 s

39	Eastward wind tendency due to 	m s–2
		parameterization

40	Northward wind tendency due to 	m s–2
		parameterization

41	u-component of geostrophic wind	m s–1

42	v-component of geostrophic wind 	m s–1

43	Geostrophic wind direction	degree true

44	Geostrophic wind speed	m s–1

45–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	In relation to local coordinate axes at a cell edge.

***	This parameter is described in more detail by (a) Lee, S. and I.M. Held, 1993: Baroclinic wave packets in models and observations. J Atmos. Sci., 50:1413-1428, (b) Chang, E.K.M., 1993: Downstream development of baroclinic waves as inferred from regression analysis. J. Atmos. Sci., 50:2038-2053, (c) Archambault, H.M., D. Keyser and L.F. Bosart, 2010: Relationships between large-scale regime transitions and major cool-season precipitation events in the northeastern United States. Mon Wea. Rev., 138:3454-3473, and (d) Zimin, A.V., I. Szunyogh, B.R. Hung and E. Orr, 2006: Extracting envelopes of nonzonally propagating Rossby wave packets. Mon. Wea. Review, 134:1329–1333.

****	Statistical process 1 (Accumulation) does not change units.

Product discipline 0 – Meteorological products, parameter category 3: mass

Number	Parameter	Units

0	Pressure	Pa

1	Pressure reduced to MSL	Pa

2	Pressure tendency	Pa s–1

3	ICAO Standard Atmosphere Reference Height	m

4	Geopotential	m2 s–2

5	Geopotential height	gpm

6	Geometric height	m

7	Standard deviation of height	m

8	Pressure anomaly	Pa

9	Geopotential height anomaly	gpm

10	Density	kg m–3

11	Altimeter setting	Pa

12	Thickness	m

13	Pressure altitude	m

14	Density altitude	m

15	5-wave geopotential height	gpm

16	Zonal flux of gravity wave stress	N m–2

17	Meridional flux of gravity wave stress	N m–2

18	Planetary boundary layer height	m

19	5-wave geopotential height anomaly	gpm

20	Standard deviation of sub-grid scale orography	m

21	Angle of sub-gridscale orography	rad

22	Slope of sub-gridscale orography	Numeric

23	Gravity wave dissipation	W m–2

24	Anisotropy of sub-gridscale orography	Numeric

25	Natural logarithm of pressure in Pa	Numeric

26	Exner pressure	Numeric

27	Updraught mass flux	kg m–2 s–1

28	Downdraught mass flux 	kg m–2 s–1

29	Updraught detrainment rate 	kg m–3 s–1

30	Downdraught detrainment rate 	kg m–3 s–1

31–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 0 – Meteorological products, parameter category 4: short-wave radiation

Number	Parameter	Units

0	Net short-wave radiation flux (surface)*	W m–2

1	Net short-wave radiation flux (top of atmosphere)*	W m–2

2	Short-wave radiation flux*	W m–2

3	Global radiation flux	W m–2

4	Brightness temperature	K

5	Radiance (with respect to wave number)	W m–1 sr–1

6	Radiance (with respect to wavelength)	W m–3 sr–1

7	Downward short-wave radiation flux	W m–2

8	Upward short-wave radiation flux	W m–2

9	Net short wave radiation flux	W m–2

10	Photosynthetically active radiation	W m–2

11	Net short-wave radiation flux, clear sky	W m–2

12	Downward UV radiation	W m–2

13	Direct short-wave radiation flux	W m–2

14	Diffuse short-wave radiation flux	W m–2

15–49	Reserved

50	UV index (under clear sky)**	Numeric

51	UV index**	Numeric

52–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	The Global Solar UVI is formulated using the International Commission on Illumination (CIE) reference action spectrum for UV-induced erythema on the human skin (ISO 17166:1999/CIE S 007/E-1998).

It is a measure of the UV radiation that is relevant to and defined for a horizontal surface. The UVI is a unitless quantity defined by the formula:

where Eλ is the solar spectral irradiance expressed in W / (m2·nanometre) at wavelength λ and dλ is the wave-length interval used in the summation. Ser λ is the erythema reference action spectrum, and ker is a constant equal to 40 m2 / W.

Product discipline 0 – Meteorological products, parameter category 5: long-wave radiation

Number	Parameter	Units

0	Net long-wave radiation flux (surface)*	W m–2

1	Net long-wave radiation flux (top of atmosphere)*	W m–2

2	Long-wave radiation flux*	W m–2

3	Downward long-wave radiation flux	W m–2

4	Upward long-wave radiation flux	W m–2

5	Net long-wave radiation flux	W m–2

6	Net long-wave radiation flux, clear sky	W m–2

7	Brightness temperature	K

8–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

Product discipline 0 – Meteorological products, parameter category 6: cloud

Number	Parameter	Units

0	Cloud ice	kg m–2

1	Total cloud cover	%

2	Convective cloud cover	%

3	Low cloud cover	%

4	Medium cloud cover	%

5	High cloud cover	%

6	Cloud water	kg m–2

7	Cloud amount	%

8	Cloud type	(Code table 4.203)

9	Thunderstorm maximum tops	m

10	Thunderstorm coverage	(Code table 4.204)

11	Cloud base	m

12	Cloud top	m

13	Ceiling	m

14	Non-convective cloud cover	%

15	Cloud work function	J kg–1

16	Convective cloud efficiency	Proportion

17	Total condensate*	kg kg–1

18	Total column-integrated cloud water*	kg m–2

19	Total column-integrated cloud ice*	kg m–2

20	Total column-integrated condensate*	kg m–2

21	Ice fraction of total condensate	Proportion

22	Cloud cover	%

23	Cloud ice mixing ratio*	kg kg–1

24	Sunshine	Numeric

25	Horizontal extent of cumulonimbus (CB)	%

26	Height of convective cloud base	m

27	Height of convective cloud top	m

28	Number of cloud droplets per unit mass of air	kg–1

29	Number of cloud ice particles per unit mass of air	kg–1

30	Number density of cloud droplets	m–3

31	Number density of cloud ice particles	m–3

32	Fraction of cloud cover	Numeric

33	Sunshine duration	s

34	Surface long-wave effective total cloudiness	Numeric

35	Surface short-wave effective total cloudiness	Numeric

36	Fraction of stratiform precipitation cover	Proportion

37	Fraction of convective precipitation cover	Proportion

38	Mass density of cloud droplets	kg m–3

39	Mass density of cloud ice	kg m–3

40	Mass density of convective cloud water droplets	kg m–3

41–46	Reserved

47	Volume fraction of cloud water droplets**	Numeric

48	Volume fraction of cloud ice particles**	Numeric

49	Volume fraction of cloud (ice and/or water)**	Numeric

50–191	Reserved

192–254	Reserved for local use

255	Missing

_____________________

*	Parameter deprecated. Use another parameter in parameter category 1: moisture instead.

**	The sum of the water and ice fractions may exceed the total due to overlap between the volumes containing ice and those containing liquid water.

Product discipline 0 – Meteorological products, parameter category 7: thermodynamic stability
		indices

Number	Parameter	Units

0	Parcel lifted index (to 500 hPa)	K

1	Best lifted index (to 500 hPa)	K

2	K index	K

3	KO index	K

4	Total totals index	K

5	Sweat index	Numeric

6	Convective available potential energy	J kg–1

7	Convective inhibition	J kg–1

8	Storm relative helicity	J kg–1

9	Energy helicity index	Numeric

10	Surface lifted index	K

11	Best (4-layer) lifted index	K

12	Richardson number	Numeric

13	Showalter index	K

14	Reserved

15	Updraught helicity	m2 s–2

16	Bulk Richardson number	Numeric

17	Gradient Richardson number	Numeric

18	Flux Richardson number	Numeric

19–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 0 – Meteorological products, parameter category 13: aerosols

Number	Parameter	Units

0	Aerosol type	(Code table 4.205)

1–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 0 – Meteorological products, parameter category 14: trace gases

Number	Parameter	Units

0	Total ozone	DU

1	Ozone mixing ratio	kg kg–1

2	Total column integrated ozone	DU

3–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 0 – Meteorological products, parameter category 15: radar

Number	Parameter	Units

0	Base spectrum width	m s–1

1	Base reflectivity	dB

2	Base radial velocity	m s–1

3	Vertically integrated liquid water (VIL)	kg m–2

4	Layer-maximum base reflectivity	dB

5	Precipitation	kg m–2

6	Radar spectra (1)	–

7	Radar spectra (2)	–

8	Radar spectra (3)	–

9	Reflectivity of cloud droplets	dB

10	Reflectivity of cloud ice	dB

11	Reflectivity of snow	dB

12	Reflectivity of rain	dB

13	Reflectivity of graupel	dB

14	Reflectivity of hail	dB

15	Hybrid scan reflectivity	dB

16	Hybrid scan reflectivity height	m

17–191	Reserved

192–254	Reserved for local use

255	Missing

Product Discipline 0 – Meteorological products, parameter category 16: forecast radar imagery

Number	Parameter	Units

0	Equivalent radar reflectivity factor for rain	mm6 m–3

1	Equivalent radar reflectivity factor for snow	mm6 m–3

2	Equivalent radar reflectivity factor for 	mm6 m–3
		parameterized convection

3	Echo top	m

4	Reflectivity	dB

5	Composite reflectivity	dB

6–191	Reserved

192–254	Reserved for local use

255	Missing

Note:	Decibel (dB) is a logarithmic measure of the relative power, or of the relative values of two flux densities, especially of sound intensities and radio and radar power densities.  In radar meteorology, the logarithmic scale (dBZ) is used for measuring radar reflectivity factor (obtained from the American Meteorological Society Glossary of Meteorology).

Product discipline 0 – Meteorological products, parameter category 17: electrodynamics

Number	Parameter	Units

0	Lightning strike density	m–2 s–1

Product discipline 0 – Meteorological products, parameter category 18: nuclear/radiology

Number	Parameter	Units

0	Air concentration of caesium 137	Bq m–3

1	Air concentration of iodine 131	Bq m–3

2	Air concentration of radioactive pollutant	Bq m–3

3	Ground deposition of caesium 137	Bq m–2

4	Ground deposition of iodine 131	Bq m–2

5	Ground deposition of radioactive pollutant	Bq m–2

6	Time-integrated air concentration of caesium	Bq s m–3
		pollutant (see Note 1)

7	Time-integrated air concentration of iodine	Bq s m–3
		pollutant (see Note 1)

8	Time-integrated air concentration of radioactive	Bq s m–3
		pollutant (see Note 1)

9	Reserved

10	Air concentration	Bq m–3

11	Wet deposition	Bq m–2

12	Dry deposition	Bq m–2

13	Total deposition (wet + dry)	Bq m–2

14–191	Reserved

192–254	Reserved for local use

255	Missing

Notes:

(1)	Statistical process 1 (Accumulation) does not change units. It is recommended to use another parameter without the word “time-integrated” in its name and accumulation in PDT.

(2)	Parameters from 10 onward may be used in combination with product definition templates 4.40 – 4.43 and Common Code table C–14 (Code table 4.230) to represent any type of radioisotope.

Product discipline 0 – Meteorological products, parameter category 19: physical atmospheric
		properties

Number	Parameter	Units

0	Visibility	m

1	Albedo	%

2	Thunderstorm probability	%

3	Mixed layer depth	m

4	Volcanic ash	(Code table 4.206)

5	Icing top	m

6	Icing base	m

7	Icing	(Code table 4.207)

8	Turbulence top	m

9	Turbulence base	m

10	Turbulence	(Code table 4.208)

11	Turbulent kinetic energy	J kg–1

12	Planetary boundary-layer regime	(Code table 4.209)

13	Contrail intensity	(Code table 4.210)

14	Contrail engine type	(Code table 4.211)

15	Contrail top	m

16	Contrail base	m

17	Maximum snow albedo (see Note 1)	%

18	Snow free albedo	%

19	Snow albedo	%

20	Icing	%

21	In-cloud turbulence	%

22	Clear air turbulence (CAT)	%

23	Supercooled large droplet probability (see Note 2)	%

24	Convective turbulent kinetic energy	J kg–1

25	Weather	(Code table 4.225)

26	Convective outlook	(Code table 4.224)

27	Icing scenario	(Code table 4.227)

28	Mountain wave turbulence (eddy dissipation rate)	m2/3 s–1

29–191	Reserved

192–254	Reserved for local use

255	Missing

Notes:

(1)	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

(2)	Supercooled large droplets (SLD) are defined as those with a diameter greater than 50 microns.

Product discipline 0 – Meteorological products, parameter category 20: atmospheric chemical
		constituents

Number	Parameter	Units

0	Mass density (concentration)	kg m–3

1	Column-integrated mass density (see Note)	kg m–2

2	Mass mixing ratio (mass fraction in air)	kg kg–1

3	Atmosphere emission mass flux	kg m–2 s–1

4	Atmosphere net production mass flux	kg m–2 s–1

5	Atmosphere net production and emission mass flux	kg m–2 s–1

6	Surface dry deposition mass flux	kg m–2 s–1

7	Surface wet deposition mass flux	kg m–2 s–1

8	Atmosphere re-emission mass flux	kg m–2 s–1

9	Wet deposition by large-scale precipitation mass 	kg m–2 s–1
		flux

10	Wet deposition by convective precipitation mass 	kg m–2 s–1
		flux

11	Sedimentation mass flux	kg m–2 s–1

12	Dry deposition mass flux	kg m–2 s–1

13	Transfer from hydrophobic to hydrophilic	kg kg–1 s–1

14	Transfer from SO2 (sulphur dioxide) to SO4	kg kg–1 s–1
		(sulphate)

15–49	Reserved

50	Amount in atmosphere	mol

51	Concentration in air	mol m–3

52	Volume mixing ratio (fraction in air)	mol mol–1

53	Chemical gross production rate of concentration	mol m–3 s–1

54	Chemical gross destruction rate of concentration	mol m–3 s–1

55	Surface flux	mol m–2 s–1

56	Changes of amount in atmosphere (see Note)	mol s–1

57	Total yearly average burden of the atmosphere	mol

58	Total yearly averaged atmospheric loss (see Note)	mol s–1

59	Aerosol number concentration	m–3

60–99	Reserved

100	Surface area density (aerosol)	m–1

101	Vertical visual range	m

102	Aerosol optical thickness	Numeric

103	Single scattering albedo	Numeric

104	Asymmetry factor	Numeric

105	Aerosol extinction coefficient	m–1

106	Aerosol absorption coefficient	m–1

107	Aerosol lidar backscatter from satellite	m–1 sr–1

108	Aerosol lidar backscatter from the ground	m–1 sr–1

109	Aerosol lidar extinction from satellite	m–1

110	Aerosol lidar extinction from the ground	m–1

111–191	Reserved

192–254	Reserved for local use

255	Missing

Note:	FirstFixedSurface and SecondFixedSurface of Code table 4.5 (Fixed surface types and units) to define the vertical extent, i.e. FirstFixedSurface can be set to 1 (Ground or water surface) and SecondFixedSurface set to 7 (Tropopause) for a restriction to the troposphere.

Product discipline 0 – Meteorological products, parameter category 190: CCITT IA5 string

Number	Parameter	Units

0	Arbitrary text string	CCITT IA5

1–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 0 – Meteorological products, parameter category 191: miscellaneous

Number	Parameter	Units

0	Seconds prior to initial reference time	s
		(defined in Section 1)

1	Geographical latitude	°N

2	Geographical longitude	°E

3	Days since last observation	d

4–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 1 – Hydrological products, parameter category 0: hydrology basic products

Number	Parameter	Units

0	Flash flood guidance 	kg m–2
		(Encoded as an accumulation over a floating
		subinterval of time between the reference time
		and valid time)

1	Flash flood runoff 	kg m–2
		(Encoded as an accumulation over a floating
		subinterval of time)

2	Remotely sensed snow cover	(Code table 4.215)

3	Elevation of snow-covered terrain	(Code table 4.216)

4	Snow water equivalent per cent of normal	%

5	Baseflow-groundwater runoff	kg m–2

6	Storm surface runoff	kg m–2

7	Discharge from rivers or streams	m3 s–1

8	Groundwater upper storage	kg m–2

9	Groundwater lower storage	kg m–2

10	Side flow into river channel	m3 s–1 m–1

11	River storage of water	m3

12	Floodplain storage of water	m3

13	Depth of water on soil surface	kg m–2

14	Upstream accumulated precipitation	kg m–2

15	Upstream accumulated snow melt	kg m–2

16–191	Reserved

192–254	Reserved for local use

255	Missing

Notes:

(1)	Remotely sensed snow cover is expressed as a field of dimensionless, thematic values. The currently accepted values are for no-snow/no-cloud, 50, for clouds, 100, and for snow, 250 (see Code table 4.215).

(2)	A data field representing snow coverage by elevation portrays at which elevations there is a snow pack. The elevation values typically range from 0 to 90 in 100-metre increments. A value of 253 is used to represent a no-snow/no-cloud data point. A value of 254 is used to represent a data point at which snow elevation could not be estimated because of clouds obscuring the remote sensor (when using aircraft or satellite measurements).

(3)	Snow water equivalent per cent of normal is stored in per cent of normal units. For example, a value of 110 indicates 110 per cent of the normal snow water equivalent for a given depth of snow.

Product discipline 1 – Hydrological products, parameter category 1: hydrology probabilities

Number	Parameter	Units

0	Conditional per cent precipitation amount 	kg m–2
		fractile for an overall period
		(Encoded as an accumulation)

1	Per cent precipitation in a sub-period of an	%
		overall period
		(Encoded as per cent accumulation over
		the sub-period)

2	Probability of 0.01 inch of precipitation (POP) 	%

3–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 1 – Hydrological products, parameter category 2: inland water and
		sediment properties

Number	Parameter	Units

0	Water depth	m

1	Water temperature	K

2	Water fraction	Proportion

3	Sediment thickness	m

4	Sediment temperature	K

5	Ice thickness	m

6	Ice temperature	K

7	Ice cover	Proportion

8	Land cover (0 = water, 1 = land)	Proportion

9	Shape factor with respect to salinity profile	–

10	Shape factor with respect to temperature	–
		profile in thermocline

11	Attenuation coefficient of water with respect	m–1
		to solar radiation

12	Salinity	kg kg–1

13	Cross-sectional area of flow in channel	m2

Product discipline 2 – Land surface products, parameter category 0: vegetation/biomass

Number	Parameter	Units

0	Land cover (0 = sea, 1 = land)	Proportion

1	Surface roughness	m

2	Soil temperature***	K

3	Soil moisture content*	kg m–2

4	Vegetation	%

5	Water runoff	kg m–2

6	Evapotranspiration	kg–2 s–1

7	Model terrain height	m

8	Land use	(Code table 4.212)

9	Volumetric soil moisture content**	Proportion

10	Ground heat flux*	W m–2

11	Moisture availability	%

12	Exchange coefficient	kg m–2 s–1

13	Plant canopy surface water	kg m–2

14	Blackadar’s mixing length scale	m

15	Canopy conductance	m s–1

16	Minimal stomatal resistance	s m–1

17	Wilting point*	Proportion

18	Solar parameter in canopy conductance	Proportion

19	Temperature parameter in canopy	Proportion

20	Humidity parameter in canopy conductance	Proportion

21	Soil moisture parameter in canopy conductance	Proportion

22	Soil moisture***	kg m–3

23	Column-integrated soil water***	kg m–2

24	Heat flux	W m–2

25	Volumetric soil moisture	m3 m–3

26	Wilting point	kg m–3

27	Volumetric wilting point	m3 m–3

28	Leaf area index	Numeric

29	Evergreen forest cover	Proportion

30	Deciduous forest cover	Proportion

31	Normalized differential vegetation index (NDVI)	Numeric

32	Root depth of vegetation	m

33	Water runoff and drainage****	kg m–2

34	Surface water runoff****	kg m–2

35	Tile class	Code table 4.243

36	Tile fraction	Proportion

37	Tile percentage	%

38	Soil volumetric ice content (water equivalent) 	m3 m–3
		(see Note)

39–191	Reserved

192–254	Reserved for local use

255	Missing

Note:	For parameter 38 (Parameter category 0), ice volume is expressed as if the ice content were melted to liquid water and then its volume measured in the liquid state. This may be understood in the same manner as water equivalent snow depth.

_____________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	It is recommended not to use this parameter, but another one with a more descriptive unit.

***	Parameter deprecated. Use another parameter in parameter category 3: soil products instead.

****	Statistical process 1 (Accumulation) does not change units.

Product discipline 2 – Land surface products, parameter category 3: soil products

Number	Parameter	Units

0	Soil type	(Code table 4.213)

1	Upper layer soil temperature*	K

2	Upper layer soil moisture*	kg m–3

3	Lower layer soil moisture*	kg m–3

4	Bottom layer soil temperature*	K

5	Liquid volumetric soil moisture (non-frozen)**	Proportion

6	Number of soil layers in root zone	Numeric

7	Transpiration stress-onset (soil moisture)**	Proportion

8	Direct evaporation cease (soil moisture)**	Proportion

9	Soil porosity**	Proportion

10	Liquid volumetric soil moisture (non-frozen)	m3 m–3

11	Volumetric transpiration stress-onset (soil moisture)	m3 m–3

12	Transpiration stress-onset (soil moisture)	kg m–3

13	Volumetric direct evaporation cease (soil moisture)	m3 m–3

14	Direct evaporation cease (soil moisture)	kg m–3

15	Soil porosity	m3 m–3

16	Volumetric saturation of soil moisture	m3 m–3

17	Saturation of soil moisture	kg m–3

18	Soil temperature	K

19	Soil moisture	kg m–3

20	Column-integrated soil moisture	kg m–2

21	Soil ice	kg m–3

22	Column-integrated soil ice	kg m–2

23	Liquid water in snow pack	kg m–2

24	Frost index	K day–1

25	Snow depth at elevation bands	kg m–2

26–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	Parameter deprecated. See Regulation 92.6.2 and use another parameter instead.

**	It is recommended not to use this parameter, but another one with a more descriptive unit.

Product discipline 2 – Land surface products, parameter category 4: fire weather products

Number	Parameter	Units

0	Fire outlook	Code table 4.224

1	Fire outlook due to dry thunderstorm	Code table 4.224

2	Haines index	Numeric

3	Fire burned area	%

4	Fosberg index*	Numeric

5	Forest Fire Weather Index (Canadian Forest	Numeric

Service)

6	Fine Fuel Moisture Code (Canadian Forest	Numeric
		Service)

7	Duff Moisture Code (Canadian Forest Service)	Numeric

8	Drought Code (Canadian Forest Service)	Numeric

9	Initial Fire Spread Index (Canadian Forest Service)	Numeric

10	Fire Buildup Index (Canadian Forest Service)	Numeric

11	Fire Daily Severity Rating (Canadian Forest Service)	Numeric

12–191	Reserved

192–254	Reserved for local use

255	Missing

______________________

*	The Fosberg index denotes the potential influence of weather on a wildland fire. It takes into account the combined effects of temperature, wind speed, relative humidity and precipitation. Higher values indicate a higher potential impact.

Product discipline 2 – Land surface products, parameter category 5: glaciers and inland ice

Number	Parameter	Units

1	Glacier temperature	K

Product discipline 3 – Space products, parameter category 0: image format products

Number	Parameter	Units

0	Scaled radiance	Numeric

1	Scaled albedo	Numeric

2	Scaled brightness temperature	Numeric

3	Scaled precipitable water	Numeric

4	Scaled lifted index	Numeric

5	Scaled cloud top pressure	Numeric

6	Scaled skin temperature	Numeric

7	Cloud mask	Code table 4.217

8	Pixel scene type	Code table 4.218

9	Fire detection indicator	Code table 4.223

10–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 3 – Space products, parameter category 1: quantitative products

Number	Parameter	Units

0	Estimated precipitation	kg m–2

1	Instantaneous rain rate	kg m–2 s–1

2	Cloud top height	m

3	Cloud top height quality indicator	Code table 4.219

4	Estimated u-component of wind	m s–1

5	Estimated v-component of wind	m s–1

6	Number of pixel used	Numeric

7	Solar zenith angle	°

8	Relative azimuth angle	°

9	Reflectance in 0.6 micron channel	%

10	Reflectance in 0.8 micron channel	%

11	Reflectance in 1.6 micron channel	%

12	Reflectance in 3.9 micron channel	%

13	Atmospheric divergence	s–1

14	Cloudy brightness temperature	K

15	Clear-sky brightness temperature	K

16	Cloudy radiance (with respect to wave number)	W m–1 sr–1

17	Clear-sky radiance (with respect to wave	W m–1 sr–1
		number)

18	Reserved

19	Wind speed	m s–1

20	Aerosol optical thickness at 0.635 μm

21	Aerosol optical thickness at 0.810 μm

22	Aerosol optical thickness at 1.640 μm

23	Angstrom coefficient

24–26	Reserved

27	Bidirectional reflectance factor (see Note 1)	numeric

28	Brightness temperature	K

29	Scaled radiance (see Note 2)	numeric

30–191	Reserved

192–254	Reserved for local use

255	Missing

Notes:

(1)	The ratio of the radiant flux reflected by a surface to that reflected into the same reflected-beam geometry and wavelength range by an ideal (lossless) and diffuse (Lambertian) standard surface, irradiated under the same conditions.

(2)	Top of atmosphere radiance observed by a sensor, multiplied by pi and divided by the in-band solar irradiance.

Product discipline 3 – Space products, parameter category 2: cloud properties

Number	Parameter	Units

0	Clear sky probability	%

1	Cloud top temperature	K

2	Cloud top pressure	Pa

3	Cloud type	Code table 4.218

4	Cloud phase	Code table 4.218

5	Cloud optical depth	Numeric

6	Cloud particle effective radius	m

7	Cloud liquid water path	kg m–2

8	Cloud ice water path	kg m–2

9	Cloud albedo	Numeric

10	Cloud emissivity	Numeric

11	Effective absorption optical depth ratio	Numeric

Product discipline 3 – Space products, parameter category 3: flight rule conditions

Number	Parameter	Units

0	Probability of encountering marginal visual flight 	%
		rule conditions

1	Probability of encountering low instrument flight 	%
		rule conditions

2	Probability of encountering instrument flight 	%
		rule conditions

Product discipline 3 – Space products, parameter category 4: volcanic ash

Number	Parameter	Units

0	Volcanic ash probability	%

1	Volcanic ash cloud top temperature	K

2	Volcanic ash cloud top pressure	Pa

3	Volcanic ash cloud top height	m

4	Volcanic ash cloud emissivity	Numeric

5	Volcanic ash effective absorption optical depth 	Numeric
		ratio

6	Volcanic ash cloud optical depth	Numeric

7	Volcanic ash column density	kg m–2

8	Volcanic ash particle effective radius	m

Product discipline 3 – Space products, parameter category 5: sea-surface temperature

Number	Parameter	Units

0	Interface sea-surface temperature (see Note 1)	K

1	Skin sea-surface temperature (see Note 2)	K

2	Sub-skin sea-surface temperature (see Note 3)	K

3	Foundation sea-surface temperature (see Note 4)	K

4	Estimated bias between sea-surface 	K
		temperature and standard

5	Estimated standard deviation between sea- 	K
		surface temperature and standard

Notes:

(1)	Theoretical temperature at the precise air-sea interface.

(2)	Temperature of the water across a very small depth (approximately the upper 20 micrometers).

(3)	Temperature at the base of the thermal skin layer.

(4)	Temperature of the water column free of diurnal temperature variability or equal to the SST sub-skin in the absence of any diurnal signal.

Product discipline 3 – Space products, parameter category 6: solar radiation

Number	Parameter	Units

0	Global solar irradiance (see Note 1)	W m–2

1	Global solar exposure (see Note 2)	J m–2

2	Direct solar irradiance (see Note 3)	W m–2

3	Direct solar exposure (see Note 4)	J m–2

4	Diffuse solar irradiance (see Note 5)	W m–2

5	Diffuse solar exposure (see Note 6)	J m–2

Notes:

(1)	The solar flux per unit area received from a solid angle of 2π sr on a horizontal surface.

(2)	Time integral of global solar irradiance.

(3)	The solar flux per unit area received from the solid angle of the sun’s disc on a surface normal to the sun direction.

(4)	Time integral of direct solar irradiance.

(5)	The solar flux per unit area received from a solid angle of 2π sr, except for the solid angle of the sun's disc, on a horizontal surface.

(6)	Time integral of diffuse solar irradiance.

Product discipline 10 – Oceanographic products, parameter category 0: waves

Number	Parameter	Units

0	Wave spectra (1)	–

1	Wave spectra (2)	–

2	Wave spectra (3)	–

3	Significant height of combined wind waves	m
		and swell

4	Direction of wind waves	degree true

5	Significant height of wind waves	m

6	Mean period of wind waves	s

7	Direction of swell waves	degree true

8	Significant height of swell waves	m

9	Mean period of swell waves	s

10	Primary wave direction	degree true

11	Primary wave mean period	s

12	Secondary wave direction	degree true

13	Secondary wave mean period	s

14	Direction of combined wind waves and swell	degree true

15	Mean period of combined wind waves and swell	s

16	Coefficient of drag with waves	–

17	Friction velocity	m s–1

18	Wave stress	N m–2

19	Normalized wave stress	–

20	Mean square slope of waves	–

21	u-component surface Stokes drift	m s–1

22	v-component surface Stokes drift	m s–1

23	Period of maximum individual wave height	s

24	Maximum individual wave height	m

25	Inverse mean wave frequency	s

26	Inverse mean frequency of wind waves	s

27	Inverse mean frequency of total swell	s

28	Mean zero-crossing wave period	s

29	Mean zero-crossing period of wind waves	s

30	Mean zero-crossing period of total swell	s

31	Wave directional width	–

32	Directional width of wind waves	–

33	Directional width of total swell	–

34	Peak wave period	s

35	Peak period of wind waves	s

36	Peak period of total swell	s

37	Altimeter wave height	m

38	Altimeter corrected wave height	m

39	Altimeter range relative correction	–

40	10-metre neutral wind speed over waves	m s–1

41	10-metre wind direction over waves	°

42	Wave energy spectrum	m2 s rad–1

43	Kurtosis of the sea-surface elevation due to 	–
		waves

44	Benjamin–Feir index	–

45	Spectral peakedness factor	s–1

46–191	Reserved

192–254	Reserved for local use

255	Missing

*	Further information concerning the wave parameters can be found in the Guide to Wave Analysis and Forecasting (WMO-No. 702).

Product discipline 10 – Oceanographic products, parameter category 1: currents

Number	Parameter	Units

0	Current direction	degree true

1	Current speed	m s–1

2	u-component of current	m s–1

3	v-component of current	m s–1

4–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 10 – Oceanographic products, parameter category 2: ice

Number	Parameter	Units

0	Ice cover	Proportion

1	Ice thickness	m

2	Direction of ice drift	degree true

3	Speed of ice drift	m s–1

4	u-component of ice drift	m s–1

5	v-component of ice drift	m s–1

6	Ice growth rate	m s–1

7	Ice divergence	s–1

8	Ice temperature	K

9	Module of ice internal pressure*	Pa m

10	Zonal vector component of vertically	Pa m 
		integrated ice internal pressure

11	Meridional vector component of vertically 	Pa m 
		integrated ice internal pressure

12	Compressive ice strength	N m–1

13–191	Reserved

192–254	Reserved for local use

255	Missing

*	Ice internal pressure or stress (Pa m) is the integrated pressure across the vertical thickness of a layer of ice. It is produced when concentrated ice reacts to external forces such as wind and ocean currents.

Product discipline 10 – Oceanographic products, parameter category 3: surface properties

Number	Parameter	Units

0	Water temperature	K

1	Deviation of sea level from mean	m

2–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 10 – Oceanographic products, parameter category 4: subsurface properties

Number	Parameter	Units

0	Main thermocline depth	m

1	Main thermocline anomaly	m

2	Transient thermocline depth	m

3	Salinity	kg kg–1

4	Ocean vertical heat diffusivity	m2 s–1

5	Ocean vertical salt diffusivity	m2 s–1

6	Ocean vertical momentum diffusivity	m2 s–1

7	Bathymetry	m

8–10	Reserved

11	Shape factor with respect to salinity profile	–

12	Shape factor with respect to temperature	–
		profile in thermocline

13	Attenuation coefficient of water with respect to	m–1
		solar radiation

14	Water depth	m

15	Water temperature	K

16–191	Reserved

192–254	Reserved for local use

255	Missing

Product discipline 10 – Oceanographic products, parameter category 191: miscellaneous

Number	Parameter	Units

0	Seconds prior to initial reference time	s
		(defined in Section 1)

1	Meridional overturning stream function	m3 s–1

2	Reserved

3	Days since last observation	d

4–191	Reserved

192–254	Reserved for local use

255	Missing

- 2017-2.6.3(CM-I)/Amendments to current version of GRIB edition 3

Amend:

in Vertical Coordinate Template Component 5.0 - Vertical Level,

Byte No.	Contents

1		Type of first fixed surface (see Code table 5.1)

2		Scale factor of first fixed surface

3-6		Scaled value of first fixed surface

in Vertical Coordinate Template Component 5.1 - Vertical layer,

Byte No.	Contents

1		Type of first fixed surface (see Code table 5.1)

2		Scale factor of first fixed surface

3-6		Scaled value of first fixed surface

7		Type of second fixed surface (see Code table 5.1)

8		Scale factor of second fixed surface

9-12		Scaled value of second fixed surface

in Data Representation Template Component 8.0 - Simple packing,

Note:

(1)	Management of explicitly missing values is an alternative to bit-map use within Section 9; it is intended to reduce the whole GRIB message size and to provide better performance when decoding data with missing values.

in Data Representation Template Component 8.1 8.4 - IEEE floating point,

Should be component 8.1 not 8.4

- 2017-2.6.4(CM-I)/Template for Gaussian and other grids

# ADD:

Horizontal Domain Template Component 4.5 – Latitudes simple packing

Byte No.	Contents

1-2	Number of parallels (2)

5	Latitudes Generating algorithm (code table 4.x)

6-13	Latitude Reference Value. (IEEE 64-bit floating-point)

14-15	Latitude Binary scale factor

16-17	Latitude Decimal scale factor

18	Number of bits used for each latitude value

19-20	Nblat - Number of bytes of latitude encoded data

21–Nblat		Latitude simple packing encoded data

Notes: The template provides an ordered list of latitudes (parallels) and shall be combined with a definition of the longitudes per parallel.  Unsigned.

Code table 4.x – Latitudes generating algorithm

Code figure	Meaning

Gaussian quadrature points

Regular latitudes

Shifted regular latitudes

Chebyskev nodes

255	Missing

Horizontal Domain Template Component 4.6 –Periodic Longitudes with number of points per parallel

Byte No.	Contents

1-8	Longitude of first point (IEEE 64-bit floating point)

9-10	NP - Number of parallels (1)

11-11+2*NP	Number of points per parallel

Notes: unsigned.

Horizontal Domain Template Component 4.7 – Periodic Longitudes with number of points per parallel and Longitude of first point per parallel

Byte No.	Contents

1-2	NP - Number of parallels (1)

5-5+2*NP	Number of points per parallel

6+NP-6+16*NP	Longitude of first point per parallel (IEEE 64-bit floating point)

Notes: unsigned

Horizontal Domain Template Component 4.8 – Non-periodic Longitudes with number of points per parallel

Byte No.	Contents

1-2	NP - Number of parallels (2)

5-12	Longitude of first point per parallel (IEEE 64-bits floating point)

13-20	Longitude of last point per parallel (IEEE 64-bits floating point)

20-20 +2*NP		Number of points per parallel

# Note: Longitude of first point per parallel shall not be equal to longitude of last point per parallel unsigned.

Horizontal Domain Template Component 4.9 – Regular Latitudes

Byte No.	Contents

1-4	Nj – number of points along a meridian

5-8	Basic angle of the initial production domain (see Note 1)

9-12	Subdivisions of basic angle used to define extreme latitudes, and

direction increments (see Note 1)

13-16	La1 – latitude of first grid point (see Note 1)

17	Resolution and component flags (see Flag table 4.1)

18-21	La2 – latitude of last grid point (see Note 1)

22-25	Dj – j direction increment (see Notes 1 and 2)

Notes:

(1)	Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10–6 degrees is not applicable to describe the extreme longitudes and latitudes, and direction increments. For these last six descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number.  For ordinary cases, zero and missing values should be coded, equivalent to respective values of 1 and 106 (10–6 degrees unit).

(2)	Direction increments are unsigned and direction of increment is represented in the scanning mode.

Horizontal Domain Template Component 4.10 – Regular longitudes

Byte No.	Contents

1-4	Ni – number of points along a parallel

9-12	Basic angle of the initial production domain (see Note 1)

13-16	Subdivisions of basic angle used to define extreme longitudes, and

direction increments (see Note 1)

21-24	Lo1 – longitude of first grid point (see Note 1)

30-33	Lo2 – longitude of last grid point (see Note 1)

34-37	Di – i direction increment (see Notes 1 and 2)

Notes:

(1)	Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10–6 degrees is not applicable to describe the extreme longitudes and latitudes, and direction increments. For these last six descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number.  For ordinary cases, zero and missing values should be coded, equivalent to respective values of 1 and 106 (10–6 degrees unit).

(2)	Direction increments are unsigned and direction of increment is represented in the scanning mode.

Horizontal Domain Template Component 4.9 – Selection of latitudes

Byte No.	Contents

1-2	First latitude rank

3-4	Last latitude rank

# Horizontal Domain Template Component 4.10 – Selection of points per parallel

Byte No.	Contents

1 - 2	NP - Number of parallels (1)

5 - 5+NP 	Parallel first point ranks

5+NP - 5+2*NP 	Parallel last point ranks

Note: unsigned

Horizontal Domain Template 4.x1 – Latitudes simple packing, Periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.6	Periodic longitudes with number of points per parallel

Horizontal Domain Template 4.x2 – Latitudes simple packing, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.7	Periodic longitudes with number of points per parallel and longitude

of first point per parallel

Horizontal Domain Template 4.x3 – Latitudes simple packing, non-periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.8	Non-periodic longitudes with number of points per parallel

Horizontal Domain Template 4.x4 – Latitudes simple packing, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.10	Regular longitudes

Horizontal Domain Template 4.x5 – Regular Latitudes, Periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.6	Periodic longitudes with number of points per parallel

Horizontal Domain Template 4.x6 – Regular Latitudes, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.7	Periodic longitudes with number of points per parallel and longitude

of first point per parallel

Horizontal Domain Template 4.x7 – Regular Latitudes, non-periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.8	Non-periodic longitudes with number of points per parallel

Horizontal Domain Template 4.x8 – Regular Latitudes, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.10	Regular longitudes

Horizontal Domain Template 4.x9 – Latitudes simple packing, Periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.6	Periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x10 – Latitudes simple packing, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.7	Periodic longitudes with number of points per parallel and longitude

of first point per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x11 – Latitudes simple packing, non-periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.8	Non-periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x12 – Latitudes simple packing, regular Longitudes on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.10	Regular longitudes

4.11	Selection of latitudes

4.13	Selection of longitudes

Horizontal Domain Template 4.x13 – Regular Latitudes, Periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.6	Periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x14 – Regular Latitudes, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.7	Periodic longitudes with number of points per parallel and longitude of

first point per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x15 – Regular Latitudes, non-periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.8	Non-periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

Horizontal Domain Template 4.x16 – Regular Latitudes, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.10	Regular longitudes

4.11	Selection of latitudes

4.13	Selection of longitudes

Horizontal Domain Template 4.y1 – Rotated Latitudes simple packing, Periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.6	Periodic longitudes with number of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y2 – Rotated Latitudes simple packing, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.7	Periodic longitudes with number of points per parallel and longitude of

first point per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y3 – Rotated Latitudes simple packing, non-periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.8	Non-periodic longitudes with number of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y4 – Rotated Latitudes simple packing, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.10	Regular longitudes

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y5 – Rotated Regular Latitudes, Periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.6	Periodic longitudes with number of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y6 – Rotated Regular Latitudes, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.7	Periodic longitudes with number of points per parallel and longitude of

first point per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y7 – Rotated Regular Latitudes, non-periodic Longitudes with number of points per parallel on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.8	Non-periodic longitudes with number of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y8 – Rotated Regular Latitudes, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.10	Regular longitudes

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y9 – Rotated Latitudes simple packing, Periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.6	Periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y10 – Rotated Latitudes simple packing, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.7	Periodic longitudes with number of points per parallel and longitude of 		first point per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y11 – Rotated Latitudes simple packing, non-periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.8	Non-periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y12 – Rotated Latitudes simple packing, regular Longitudes on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.5	Latitudes simple packing

4.10	Regular longitudes

4.11	Selection of latitudes

4.13	Selection of longitudes

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y13 – Rotated Regular Latitudes, Periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.6	Periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y14 – Rotated Regular Latitudes, Periodic Longitudes with number of points per parallel and longitude of first point per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.7	Periodic longitudes with number of points per parallel and longitude of 		first point per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y15 – Rotated Regular Latitudes, non-periodic Longitudes with number of points per parallel on ellipsoidal planet, sub-grid

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.8	Non-periodic longitudes with number of points per parallel

4.11	Selection of latitudes

4.12	Selection of points per parallel

4.2	Rotation of latitude/longitude coordinate system

Horizontal Domain Template 4.y16 – Rotated Regular Latitudes, regular Longitudes on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.9	Regular Latitudes

4.10	Regular longitudes

4.11	Selection of latitudes

4.13	Selection of longitudes

4.2	Rotation of latitude/longitude coordinate system

- 2017-2.6.5(CM-I)/Template for unstructured mesh

Add:

# Horizontal Domain Template Component 4.13 – Latitude/longitude unstructured mesh simple packing

Byte No.	Contents

1-4	Number of grid points

5-12	Longitude Reference Value. (IEEE 64-bit floating-point)

13-14	Longitude Binary scale factor

15-16	Longitude Decimal scale factor

17	Number of bits used for each longitude value

18-25	Latitude Reference Value. (IEEE 64-bit floating-point)

26-27	Latitude Binary scale factor

28-29	Latitude Decimal scale factor

30	Number of bits used for each latitude value

31-34	Nblon - Number of bytes of longitude encoded data

35 – 35+Nblon	Longitude simple packing encoded data

36+Nblon - 37+Nblon	Nblat - Number of bytes of latitude encoded data

38+Nblon – 38+Nblon+Nblat   Latitude simple packing encoded data

Horizontal Domain Template 4.z1 – Latitude/longitude unstructured mesh on ellipsoidal planet simple packing

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.13	Latitude/longitude unstructured grid simple packing

Horizontal Domain Template 4.z2 – Rotated Latitude/longitude unstructured mesh on ellipsoidal planet simple packing

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.13	Latitude/longitude unstructured grid simple packing

4.2	Rotation of latitude/longitude coordinate system

- 2017-2.6.6(CM-I)/Horizontal domain URL

Add:

# Horizontal Domain Template Component 4.14 – URL

Byte No.	Contents

1-2	scheme (code table)

3	Nhost - number of bytes used by host

4 - 4+Nhost	host

5+Nhost - 6+Nhost	port (unsigned integer)

7+Nhost - 9+Nhost	Npath -number of bytes used by path

10+Nhost - 10+Nhost+Npath	path

11+Nhost+Npath - 13+Nhost+Npath	Nquery -number of bytes used by query

14+Nhost+Npath - 15+Nhost+Npath+Nquery	query

Note:

(1)	A URL is a Uniform Resource Locator that is identifying a web resource and is used in this context to locate and retrieve a GRIB message providing the template to be used.

The syntax of the URL is:

scheme://[host[:port]]/path[?query]

where port and query are optional and can be missing.

Code table 4.x – URL scheme

Code figure	Meaning

0	http

1	https

2	ftp

3	file

65535	Missing

Horizontal Domain Template 4.9 – URL

Component Code	Component Name

4.14	URL

- 2017-2.6.7(CM-I)/Overlay Template URL

# Overlay Template Component 9.1 – URL

Byte No.	Contents

1-2	scheme (code table 9.0)

3	Nhost - number of bytes used by host

4 - 4+Nhost	host

5+Nhost - 6+Nhost	port (unsigned integer)

7+Nhost - 9+Nhost	Npath - number of bytes used by path

10+Nhost - 10+Nhost+Npath	path

11+Nhost+Npath - 13+Nhost+Npath	Nquery - number of bytes used by query

14+Nhost+Npath - 15+Nhost+Npath+Nquery	query

Note:

(1)	A URL is a Uniform Resource Locator that is identifying a web resource and is used in this context to locate and retrieve a GRIB message providing the overlay field.

The syntax of the URL is:

scheme://host[:port]/path[?query]

where port and query are optional and can be missing.

Code table 9.0 – URL scheme

Code figure	Meaning

0	http

1	https

2	ftp

3	file

65535	Missing

Overlay Template 9.1 – URL

Component Code	Component Name

9.1	URL

- 2017-2.6.8(CM-I)/Vertical Domain template and template component for model levels

# Vertical Domain Template Component 5.2 – Model level parameters list

Byte No.	Contents

1-4	NP - Number of parameters

5	Algorithm to compute height, depth or pressure level (code table 		5.2)

6 - 6+NP*4	List of parameters. (IEEE 32-bit floating-point)

Note:

(1)	The list of parameters is ordered and the algorithm to compute height, depth or pressure has to be clearly explained in code table 5.x.

# Code Table 5.2 – Algorithm to compute height, depth or pressure level

Code figure	Meaning

0	The atmosphere is divided into NLEV layers. These layers are 			defined by the pressures at the interfaces between them (the 			‘half-levels’), and these pressures are given by pk+1/2 = Ak+1/2 + 		Bk+1/2 ps (1) for 0 ≤ k ≤ NLEV. The Ak+1/2 and Bk+1/2 are constants 		whose values effectively define the vertical coordinate and ps is 		the surface pressure field.

The values of the A k+1/2 and Bk+1/2 for all 0 ≤ k ≤ NLEV are the 		list of parameters an ps is the auxiliary field needed to compute 		the pressure pk associated with each model level (middle of 				layer) from pk = 1/2 (pk−1/2 + pk+1/2) with 1 ≤ k ≤ NLEV by using 		(1) and the surface pressure field.

255	Missing

# Vertical Domain Template Component 5.3 – URL of auxiliary fields for model levels

Byte No.	Contents

1-2	scheme (code table 5.3)

3	Nhost - number of bytes used by host

4 - 4+Nhost	host

5+Nhost - 6+Nhost	port (unsigned integer)

7+Nhost - 9+Nhost	Npath -number of bytes used by path

10+Nhost - 10+Nhost+Npath	path

11+Nhost+Npath - 13+Nhost+Npath	Nquery -number of bytes used by query

14+Nhost+Npath - 15+Nhost+Npath+Nquery	query

Note:

(1)	A URL is a Uniform Resource Locator that is identifying a web resource and is used in this context to locate and retrieve a GRIB message providing the fields to be used to compute height, depth or pressure for a given model level.

The syntax of the URL is:

scheme://host[:port]/path[?query]

where port and query are optional and can be missing.

Code table 5.3 – URL scheme

Code figure	Meaning

0	http

1	https

2	ftp

3	file

65535	Missing

# Vertical Domain Template Component 5.4 – Model level

Byte No.	Contents

1-4	Model level

# Vertical Domain Template Component 5.5 – URL of model level parameters list

Byte No.	Contents

1-2	scheme (code table 5.x)

3	Nhost - number of bytes used by host

4 - 4+Nhost	host

5+Nhost - 6+Nhost	port (unsigned integer)

7+Nhost - 9+Nhost	Npath -number of bytes used by path

10+Nhost - 10+Nhost+Npath	path

11+Nhost+Npath - 13+Nhost+Npath	Nquery -number of bytes used by query

14+Nhost+Npath - 15+Nhost+Npath+Nquery	query

Note:

(1)	A URL is a Uniform Resource Locator that is identifying a web resource and is used in this context to locate and retrieve a GRIB message providing the list of model levels and the algorithm to be used to compute height, depth or pressure for a given model level.

The syntax of the URL is:

scheme://host[:port]/path[?query]

where port and query are optional and can be missing.

# Vertical Domain Template 5.2 – Model level with list of parameters and URL of auxiliary fields

Component Code	Component Name

5.4	Model level

5.2	Model level parameters list

5.3	URL of auxiliary fields for model levels

# Vertical Domain Template 5.3 – Model level with URL of list of parameters and URL of auxiliary fields

Component Code	Component Name

5.4	Model level

5.5	URL of model level parameters list

5.3	URL of auxiliary fields for model levels

# Vertical Domain Template 5.5 – Layer of model level with list of parameters and URL of auxiliary fields

Component Code	Component Name

5.4	Model level

5.4	Model level

5.2	Model level parameters list

5.3	URL of auxiliary fields for model levels

# Vertical Domain Template 5.6 – Layer of model level with URL of list of parameters and URL of auxiliary fields

Component Code	Component Name

5.4	Model level

5.4	Model level

5.5	URL of model level parameters list

5.3	URL of auxiliary fields for model levels

- 2017-2.6.9(CM-I)/Template for irregular grid

# Horizontal Domain Template Component 4.14 – Latitude/longitude irregular grid simple packing

Byte No.	Contents

1-2	Ni - Number of columns (1)

3-4	Nj - Number of rows (1)

5-12	Longitude Reference Value. (IEEE 64-bit floating-point)

13-14	Longitude Binary scale factor

15-16	Longitude Decimal scale factor

17	Number of bits used for each longitude value

18-25	Latitude Reference Value. (IEEE 64-bit floating-point)

26-27	Latitude Binary scale factor

28-29	Latitude Decimal scale factor

30	Number of bits used for each latitude value

31-34	Nblon - Number of bytes of longitude encoded data

35 – 35+Nblon	Longitude simple packing encoded data

36+Nblon - 37+Nblon	Nblat - Number of bytes of latitude encoded data

38+Nblon – 38+Nblon+Nblat	Latitude simple packing encoded data

Notes:

(1)	Unsigned

Horizontal Domain Template 4.y1 – Latitude/longitude irregular grid on ellipsoidal planet simple packing

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.14	Latitude/longitude irregular grid simple packing

Horizontal Domain Template 4.y2 – Rotated Latitude/longitude irregular grid on ellipsoidal planet simple packing

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.14	Latitude/longitude irregular grid simple packing

4.2	Rotation of latitude/longitude coordinate system

- 2017-2.6.10(CM-I)/Templates for spherical harmonics

# Horizontal Domain Template Component 4.15 – Spherical harmonic coefficients

Byte No.	Contents

1-2	J – pentagonal resolution parameter (1)

3-4	K – pentagonal resolution parameter (1)

5-6	M – pentagonal resolution parameter (1)

7	Representation type indicating the method used to define the 			norm (see Code table 4.x)

8	Representation mode indicating the order of the coefficients (see 		Code table 4.y)

Notes:

(1) unsigned

(2) The pentagonal representation of resolution is general. Some common truncations are special cases of the pentagonal one:

Triangular:	M = J = K

Rhomboidal:	K = J + M

Trapezoidal:	K = J, K > M

Code table 4.x – Spectral data representation type

Code figure	Meaning

1	The associated Legendre functions of the first kind are defined by:

=

A field F(λ, μ) is represented by:

where    is the longitude,

the sine of latitude,

and  the complex conjugate of

Code table 4.y – Spectral data representation mode

Code figure	Meaning

0	Reserved

1	The complex numbers (see code figure 1 in Code table 3.6) are 		stored for m ≥ 0 as pairs of real numbers Re(), Im() ordered 		with n increasing from m to N(m), first for m = 0 and then for m = 		1, 2, ... M (see Note)

2–254	Reserved

255	Missing

Note:	Values of N(m) for common truncation cases:

Triangular:	M = J = K,	N(m) = J

Rhomboidal:	K = J + M,	N(m) = J + m

Trapezoidal:	K = J, K > M,	N(m) = J

Horizontal Domain Template 4.y1 – Spherical harmonic coefficients on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.15	Spherical harmonic coefficients

Horizontal Domain Template 4.y2 – Rotated Spherical harmonic coefficients on ellipsoidal planet

Component Code	Component Name

4.0	Ellipsoid of revolution defined with axis lengths

4.15	Spherical harmonic coefficients

4.2	Rotation of latitude/longitude coordinate system

# Data Representation Template Component 8.2 – Spherical harmonics coefficients, simple packing

Byte No.	Contents

1-4	Reference value (R) (IEEE 32-bit floating-point value)

5-6	Binary scale factor (E)

7-8	Decimal scale factor (D)

9	Number of bits used for each packed value

10-13	Real part of (0.0) coefficient (IEEE 32-bit floating-point value)

Notes:

(1)	Removal of the real part of (0.0) coefficient from packed data is intended to reduce the variability of the coefficients, in order to improve packing accuracy.

(2)	For some spectral representations, the (0.0) coefficient represents the mean value of the parameter represented.

# Data Representation Template Component 8.3 – Spherical harmonics coefficients, complex packing

Byte No.	Contents

1-4	P – Laplacian scaling factor (expressed in 10–6 units)

5-6	JS – pentagonal resolution parameter of the unpacked subset 			(see Note 1)

7-8	KS	 – pentagonal resolution parameter of the unpacked subset 		(see Note 1)

9-10	MS – pentagonal resolution parameter of the unpacked subset 			(see Note 1)

11-14	TS	 – total number of values in the unpacked subset (see Note 		1)

15	Precision of the unpacked subset (see Code table 8.z)

Notes:

(1)	The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to JS, KS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7.

(2)	The remaining coefficients are multiplied by (n x (n+1))P, scaled and packed. The operator associated with this multiplication is derived from the Laplacian operator on the sphere.

(3)	The retrieval formula for a coefficient of wave number n is then:

Y = (R + X x 2E) x10–D x (n x (n+1))–P where X is the packed scaled value associated with the coefficient.

Data Representation Template 4.z1 – Spherical harmonic coefficients, simple packing

Component Code	Component Name

8.2	Spherical harmonics coefficients, simple packing

Data Representation Template 4.z1 – Spherical harmonic coefficients, complex packing

Component Code	Component Name

8.3	Spherical harmonics coefficients, complex packing

3.		MANUAL ON CODES: REGULATIONS FOR REPORTING TRADITIONAL OBSERVATION DATA IN TABLE-DRIVEN CODE FORMS

- 2017-3.1.1(CM-I)/Regulations for radiosonde descent data [Validation]

ADD:

new notes under B/C 25 Regulations,

(6)	If the radiosonde and sounding system are capable to report data during the descent of the radiosonde, sequence <3 09 056> shall be used for reporting radiosonde descent data as outlined in Annex III.

new Annex III to B/C 25 Regulation,

ANNEX III TO B/C25 – Regulations

TM 309056 – BUFR template for P, T, U and wind vertical profiles suitable for radiosonde descent data

|  |  | (Sequence for representation of radiosonde descent data) |  |
| --- | --- | --- | --- |
| 3 09 056 | 3 01 150 | WIGOS identifier |  |
|  | 0 02 011 | Radiosonde type |  |
|  | 0 02 013 | Solar and infrared radiation correction |  |
|  | 0 02 014 | Tracking technique/status of system used |  |
|  | 0 02 003 | Type of measuring equipment used |  |
|  | 3 01 128 | Additional information on radiosonde ascent | Valid also for radiosonde decent |
|  | 3 01 113 | Date/time of launch | Or drop respectively |
|  | 0 08 091 | Coordinates significance | = 2 Start of observation |
|  | 3 01 021 | Latitude/longitude (high accuracy) |  |
|  | 0 07 007 | Height | Begin of descending of radiosonde above mean sea level |
|  | 0 08 091 | Coordinates significance | Set to missing (cancel) |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 002 | Extended delayed descriptor replication factor |  |
|  | 3 03 054 | Temperature, dewpoint and wind data at a pressure level with radiosonde position |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |

This BUFR template for P, T, U and wind profiles further expands as follows:

|  |  | Identification of drop site and instrumentation |  |
| --- | --- | --- | --- |
|  |  | WIGOS identifier |  |
| 3 01 150 | 0 01 125 | WIGOS identifier series | Numeric, 0 |
|  | 0 01 126 | WIGOS issuer of identifier | Numeric, 0 |
|  | 0 01 127 | WIGOS issue number | Numeric, 0 |
|  | 0 01 128 | WIGOS local identifier (character) | CCITT IA5 |
| 0 02 011 |  | Radiosonde type | Code table |
| 0 02 013 |  | Solar and infrared radiation correction | Code table |
| 0 02 014 |  | Tracking technique/status of system used | Code table |
| 0 02 003 |  | Type of measuring equipment used | Code table |
|  |  | Additional information on radiosonde ascent |  |
| 3 01 128 | 0 01 081 | Radiosonde serial number | CCITT IA5 |
|  | 0 01 082 | Radiosonde ascension number | Numeric, 0 |
|  | 0 01 083 | Radiosonde release number | Numeric, 0 |
|  | 0 01 095 | Observer identification | CCITT IA5 |
|  | 0 02 015 | Radiosonde completeness | Code table |
|  | 0 02 016 | Radiosonde configuration (3 = Parachute) | Flag table |
|  | 0 02 017 | Correction algorithms for humidity measurements | Code table |
|  | 0 02 066 | Radiosonde ground receiving system | Code table |
|  | 0 02 067 | Radiosonde operating frequency | Hz, scale -5 |
|  | 0 02 080 | Balloon manufacturer | Code table |
|  | 0 02 081 | Type of balloon | Code table |
|  | 0 02 082 | Weight of balloon | kg, scale 3 |
|  | 0 02 083 | Type of balloon shelter | Code table |
|  | 0 02 084 | Type of gas used in balloon | Code table |
|  | 0 02 085 | Amount of gas used in balloon | kg, scale 3 |
|  | 0 02 086 | Balloon flight train length | m, scale 1 |
|  | 0 02 095 | Type of pressure sensor | Code table |
|  | 0 02 096 | Type of temperature sensor | Code table |
|  | 0 02 097 | Type of humidity sensor | Code table |
|  | 0 02 103 | Radome | Flag table |
|  | 0 02 191 | Geopotential height calculation | Code table |
|  | 0 25 061 | Software identification and version number | CCITT IA5 |
|  | 0 35 035 | Reason for termination | Code table |
|  |  | Date/time of drop |  |
| 3 01 113 | 0 08 021 | Time significance (= 18 Launch time) | Code table |
|  | 3 01 011 | Year | Year |
|  |  | Month | Month |
|  |  | Day | Day |
|  | 3 01 013 | Hour | Hour |
|  |  | Minute | Minute |
|  |  | Second | Second |
|  |  | Horizontal and vertical coordinates of drop site |  |
| 0 08 091 |  | Coordinates significance (= 2 Start of observation) | Code table |
|  |  | Latitude/longitude (high accuracy) |  |
| 3 01 021 | 0 05 001 | Latitude (high accuracy) | Degree, scale 5 |
|  | 0 06 001 | Longitude (high accuracy) | Degree, scale 5 |
| 0 07 007 |  | Height |  |
| 0 08 091 |  | Coordinates significance (Set to missing (cancel)) |  |
|  |  | Temperature, dewpoint and wind data at pressure levels |  |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 002 |  | Extended delayed descriptor replication factor | Numeric |
|  |  | Temperature, dewpoint and wind data at a pressure level with radiosonde position |  |
| 3 03 054 | 0 04 086 | Long time period or displacement (since launch time) | Second |
|  | 0 08 042 | Extended vertical sounding significance | Flag table |
|  | 0 07 004 | Pressure | Pa, scale –1 |
|  | 0 10 009 | Geopotential height | gpm |
|  | 0 05 015 | Latitude displacement since launch site (high accuracy) | Degree, scale 5 |
|  | 0 06 015 | Longitude displacement since launch site (high accuracy) | Degree, scale 5 |
|  | 0 12 101 | Temperature/air temperature | K, scale 2 |
|  | 0 12 103 | Dewpoint temperature | K, scale 2 |
|  | 0 11 001 | Wind direction | Degree true |
|  | 0 11 002 | Wind speed | m s–1, scale 1 |
|  |  | Wind shear data |  |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric |
|  |  | Wind shear data at a pressure level |  |
| 3 03 051 | 0 04 086 | Long time period or displacement (since launch time) | Second |
|  | 0 08 042 | Extended vertical sounding significance | Flag table |
|  | 0 07 004 | Pressure | Pa, scale –1 |
|  | 0 05 015 | Latitude displacement since launch site (high accuracy) | Degree, scale 5 |
|  | 0 06 015 | Longitude displacement since launch site (high accuracy) | Degree, scale 5 |
|  | 0 11 061 | Absolute wind shear in 1 km layer below | m s–1, scale 1 |
|  | 0 11 062 | Absolute wind shear in 1 km layer above | m s–1, scale 1 |

Regulations:

- Regulations B/C25.1 shall apply with following amendments:

  - Bufr edition 4 is required

  - The international data sub-category shall be included at all observation times as follows:
	= 014 for radiosonde descent data,
	= 015 for radiosonde descent data from ships,
	= 016 for radiosonde descent data from mobile stations.

- Regulations B/C25.3 shall apply

- Regulations B/C25.7 to B/C25.9, inclusive, shall apply.

Notes:

- If a parachute is used, the descriptor <0 02 016> ‘Radiosonde configuration’ in sequence <3 01 128> shall be encoded with bit no. 3 set to 1.

- 2017-3.1.2(CM-I)/Regulations for reporting SHIP data in TDCF (B/C10) [ABC2018]

AMEND:

B/C10.2.2.2,

B/C10.2.2.2	Direction and speed of motion of moving observing platform may be included as missing values in reports from ships that have not been directly recruited and instrumented by an NMHS, except when reporting from an area for which the ship report collecting centre, in order to meet a requirement of a search and rescue centre, has requested inclusion of direction and speed of ship motion as a routine procedure. [12.3.1.2(b)]

- 2017-3.1.3(CM-I)/Implementation of the Decision 15 of EC-69 regarding the International Exchange of Snow Data [ABC2018]

AMEND:

in B/C Regulations,

B/C1.8	State of ground, snow depth, ground minimum temperature 
<3 02 037>

B/C1.8.1	State of ground (with or without snow) – Code table 0 20 062

State of ground without snow or with snow shall be reported using Code table 0 20 062. The synoptic hour at which this datum shall be reported is determined by regional decision.  In addition to the synoptic hour, this datum should be reported at other synoptic hours, i.e. four times a day.

B/C1.8.2	Total snow depth

Total snow depth (0 13 013) shall be reported in metres (with precision in hundredths of a metre). The synoptic hour at which this datum is determined by regional decision.  In addition to the synoptic hour, this datum should be reported at other synoptic hours, i.e. four times a day.

6.	SUMMARY AND CONCLUSION OF PROPOSALS

- 2017-6.1(CM-I)/Summary on amendments since IPET-DRMM-IV

Implementation of amendments during the intersessional period

1.	Pre-operational (22 September 2016)

2.	Fast-track 2016-2 (2 November 2016)

3.	Fast-track 2017-1 (3 May 2017)

4.	Adoption between CBS sessions 2017 (8 November 2017)

Note: English documents are available from the links. Some of them are also available in other languages, replacing "en" with "fr", "ru", "es" or "ar" (Item 4), or with "fr", "ru" and "es" (Items 2 and 3) in the links.

STATUS OF PROPOSALS STATUS OF PROPOSALS 1. The following tables are to highlight the proposals under validation for facilitating validation exercise and to record those implemented or adopted after the previous meeting of IPET-DRMM. 2. Proposals at validation stage have no status of Manual on Codes (WMO-No. 306) and MAY NOT be applied in operational data and products. 3. It should be noted proposals at validation stage may be modified during validation exercise. It is recommended to update the proposals whenever modified. FM 92 GRIB ID; Approval for Validation; Proposals; Status; Remarks. PFC2016-2.2; New GRIB2 entry in code table 4.2; FT2017-1. PFC2016-2.4; New GRIB2 templates related to distribution functions and associated table entries; FT2017-1. 2016-2.2.1; DRMM-IV; New parameter for tropical cyclone forecasts; FT2016-2. 2016-2.2.2; DRMM-IV; New GRIB template for post-processed NWP output; FT2016-2. 2016-2.2.3; DRMM-IV; New GRIB table entries (unbalanced components); FT2016-2. 2016-2.2.4; DRMM-IV; Amendments in Common Table C-14 for Pollen, new parameters in Table 4.2 and new entries in Table 4.5; FT2016-2. 2016-2.2.5; DRMM-IV; New generating process and common codes to represent wildfire-driven pollution in air quality models; FT2016-2. 2016-2.2.6; DRMM-IV; Update of GRIB2 new compression method – CCSDS libaec; FT2016-2. 2016-2.2.7; DRMM-IV; New ensemble templates and one ensemble template correction; FT2016-2. 2016-2.2.8; DRMM-IV; Encoding fields from Limited Area models combining conformal projections and bi-periodic spectral geometry; Val; Ongoing collaborated by ECMWF. 2016-2.2.9; DRMM-IV; New table entries for GRIB2 to represent GRAPES model products; FT2016-2. 2015-2.1.1; DRMM-III; Complete encoding of information required for georeferencing grid points in GRIB; ABC2016. 2015-2.1.2; DRMM-III; New GRIB2 Regulations and notes to make it clear that forecast times may be negative; ABC2017. 2015-2.2.1/2.2.2; DRMM-III; New parameters in GRIB2 Code table 4.2/New GRIB2 parameters and product definition template for observational satellite data; Val; Ongoing No requirement from USA. 2014-2.2.2; DRMM-II; A product definition template for statistics over an ensemble; Val; Ongoing. 2012-2.2.9; DRC-IV; GRIB template for 4-D Trajectory grid definition; Val; Ongoing collaborated by ECMWF, UKMO. FM 94 BUFR/FM 95 CREX ID; Approval for Validation; Proposals; Status; Confirmed at meeting. PFC2016-2.5; Clarification of entry 4 in code table 0 01 150; FT2017-1. 2016-3.2.1; DRMM-IV; New entry in common code table C-2 by Switzerland and Kazakhstan; FT2016-2. 2016-3.2.2; DRMM-IV; New entries for dropsonde data; FT2016-2. 2016-3.2.3 2016-3.2.8; DRMM-IV; New entries in Common Tables C-5 and C-8 New entries in Common Code Table C-5; FT2016-2. 2016-3.2.4; DRMM-IV; New BUFR elements for C-band synthetic aperture radar instrument (C-SAR); FT2016-2. 2016-3.2.6; DRMM-IV; BUFR Sequence for synoptic reports from sea stations suitable for VOS observation data (editorial changes); FT2016-2. 2016-3.2.7; DRMM-IV; Additional bio-geochemical sequences for data from Argo profiling floats; Val; Ongoing C-3: superseded by 2017-2.5.5 (FT2017-2). 2016-3.2.9; DRMM-IV; Revised and additional new descriptors for radar data products; FT2016-2. 2016-3.2.10; DRMM-IV; Revised note for encoding geographic coordinates in BUFR and new element descriptors for encoding coordinate reference systems and fixed reference mean sea level; ABC2017, FT2016-2. 2016-3.2.11; DRMM-IV; Development of a LIDAR & Ceilometer Table D Sequence; FT2017-1. 2016-3.2.12; DRMM-IV; Radiosounding meta-data reporting: clarifications and amendments required; FT2016-2, FT2017-1. 2016-3.2.13; DRMM-IV; Reporting of measurement uncertainties in BUFR; FT2017-1. 2015-5.1; DRMM-III; Clarifications of B/C 30 and B/C 32 Regulations; ABC2016. 2015-5.2; DRMM-III; Strategy for amending Annex II to B/C 25 Regulations; ABC2016. 2015-5.3; DRMM-III; Clarification of upper-air wind reporting nearby the Poles; ABC2016. 2014-3.2.7; DRMM-II; BUFR template for n-minute AWS data (3 07 092); Val; Superseded by 2017-2.2.4. 2014-3.2.9; DRMM-II; Proposed BUFR sequence for reporting observations from offshore platforms; Val; FT2017-2. 2014-11.2; DRMM-II; WIGOS Station Identifier; FT2017-1. 2013-3.2.2; DRMM-I; Proposal for a BUFR template for radar wind profiler; FT2017-1. 2013-3.2.4; DRMM-I; Satellite-derived winds in BUFR; Val; FT2017-2...

7.	LIST OF AMENDMENTS BY FAST-TRACK

- 2017-7.1	(CM-I)/Drafting of list of amendments by fast-track (FT2017-2)

See appendix.

8.	MIGRATION TO TABLE-DRIVEN CODE FORMS

- 2017-8.1(CM-I)/Status of migration by WWW Monitoring Exercise

Table 1 - Percentage of required surface observations received in Table Driven Code format (TDCF) and in Traditional Alphanumeric Code (TAC).

| WMO Region | Average % of TDCF | Average % of TAC |
| --- | --- | --- |
| 1 | 29 | 53 |
| 2 | 70 | 91 |
| 3 | 33 | 60 |
| 4 | 72 | 83 |
| 5 | 62 | 75 |
| 6 | 84 | 90 |
| Antarctica | 33 | 44 |

Table 2 - Percentage of required upper air observations received in Table Driven Code format (TDCF) and in Traditional Alphanumeric Code (TAC).

| WMO Region | Average % of TDCF | Average % of TAC |
| --- | --- | --- |
| 1 | 6 | 26 |
| 2 | 28 | 81 |
| 3 | 21 | 50 |
| 4 | 56 | 94 |
| 5 | 34 | 67 |
| 6 | 42 | 81 |
| Antarctica | 23 | 48 |

Chart 1 - Comparison of percentage of required surface reports received in TAC (left-pointing arrow) and TDCF (right pointing arrow) in the period 1-15 April 2017.

Chart 2 - Comparison of percentage of required upper air reports received in TAC (left-pointing arrow) and TDCF (right pointing arrow) in the period 1-15 April 2017.

- 2017-8.2.1(CM-I)/Status of migration in RA I

Migration in countries

In Kenya and Tunisia:

The migration to TDCF was finished.

In Mauritius:

Migration not yet started at the NMS but the RTH connected to converts TAC SYNOP message and injects it into GTS network.  Any help to migrate to TDCF would be much appreciated.

In Morocco:

The transition to the TDCF format was carried out by the NMS of Morocco in October 2016. TAC messages were replaced by BUFR and transmitted to the GTS through RTH Toulouse.

TEMP messages are issued from the upper-air stations in both formats TAC and BUFR.  Dissemination on the GTS of TEMP data in BUFR will be achieved by the end of 2017, pending the official declaration to the WMO of the two upper-air stations of DAKHLA and ERRACHIDIA.

SYNOP and METAR messages automatically generated by 156 new AWOS stations which are also issued in TAC and BUFR at the national level.  Their transmission on the GTS is dependent on the officialization of the identifiers of these AWOS stations in early 2018.

Migration Status evolution

Recommendations

1.	The revision of the list of focal points mentioned at the May-June 2016 meeting should be re-examined. The list of TDCF focal points should be updated with the aim of having specialized focal points.

Editorial note: If a focal point for codes and data representation matters is inactive, it is recommended to contact the focal point for WIS of the country, instead (escalation approach).

2.	The RTHs should be encouraged to take in charge the responsibility of converting and transmitting in TDCF format all messages received from attached NMS.

3.	The task of monitoring TDCF migration in RA I is not permanent. No NMS nor RTH has this responsibility in charge. It is recommended that WMO or other centre, such as ECMWF, do this and diffuse the results on web site.

- 2017-8.2.2(CM-I)/Status of migration in RA II

RA-II members activities related to TDCF

Republic of Korea

Republic of Korea started dissemination of CLIMAT and surface observation data in BUFR in November 2016 with GTS headings of ISCC01 RKSL (monthly), ISMC01 RKSL (00, 06, 12, 18 UTC) and ISIC20 RKSL (03, 09, 15, 21 UTC).

Japan

In November 2016, Japan started dissemination of drifting buoy data in a new BUFR format using TM315009, with GTS headings of IOB[A-D,I-L]1[1-3] RJTD, to replace IOB[A-D,I-L]0[1-3] RJTD, which do not use TM315009. Dissemination of the old format reports will be discontinued after users’ migration to the new bulletins. Traditional BUOY reports continue to be distributed with headings of SSVB[01-19] RJTD for Japanese drifting buoys.

Uzbekistan

Uzbekistan started dissemination of surface observation data in BUFR format in April 2017 with GTS headings of ISMD01 UTTW (00, 06, 12, 18 UTC) and ISID20 UTTW (03, 09, 15, 21 UTC).

Monitoring and Analysis of Migration Status

Monitoring method

Statistics were collected for the monitoring period (January, April, July and October 1 through 15). Resources were derived from the results of Special MTN Monitoring (SMM) pre-analysis and Integrated WWW Monitoring (IWM) created by WMC Melbourne/RTH Tokyo and from the latest version of the surface and upper-air station list of Regional Basic Synoptic Networks (RBSN) at the time of analysis.

In addition to WWW monitoring, the status of TDCF data communication is also monitored based on a catalogue created by GISC Tokyo (available at http://www.wis-jma.go.jp/csv/catalog.csv).

Migration progress and status

(1)	SYNOP, TEMP and PILOT reports

The figures below show numerical representations of the progress of stations issuing BUFR-format bulletins equivalent to SYNOP and TEMP reports over the past three years. In the latest monitoring period from April 1 to 15, 2017, RTH Tokyo received (i) at least one surface synoptic observation report (excluding NIL reports) in BUFR format from 77% of RA-II observation stations registered as part of RBSN (TAC format from 94%), and (ii) at least one upper-air sounding report in BUFR format from 55% of registered stations (TAC format from 91%). Twelve BUFR reports equivalent to PILOT reports were received by RTH Tokyo in the monitoring period, while TAC bulletins were received from thirteen stations.

Number of RA-II RBSN stations issuing surface synoptic observation (SYNOP) and upper-air sounding (TEMP) reports in TAC and BUFR format from July 2014 to April 2017

(2)	CLIMAT reports

As of June 2017, ten Members out of 34 Members who have Regional Basic Climatological Network (RBCN) stations were reporting CLIMAT data in BUFR format: China; India; Mongolia; Saudi Arabia; Pakistan; Japan; Bangladesh; Hong Kong, China; Macao, China, Republic of Korea and Thailand.

(3)	Marine reports

As of June 2017, India (TESAC), Hong Kong, China (SHIP), Japan (TESAC, TRACKOB, SHIP) and Republic of Korea (TESAC) were routinely disseminating marine observation data in BUFR format.  Adoption of new templates for TESAC and BATHY is limited.

Upper-air reports in native BUFR format

Japan plans to start providing high resolution native BUFR upper-air reports in 2018, replacing BUFR reports converted from TEMP. Japan will continue to provide TEMP in parallel at least one year.

- 2017-8.2.3(CM-I)/Status of migration in RA III

Individual country (migrated to BUFR):

GISC Brasilia has already been generating BUFR bulletins for upper-air sounding, CLIMAT, TEMP, PILOT, SYNOP from automatic and manned weather stations, together with the same information in TAC.  Recently the conversions from TEMP/PILOT into BUFR was stopped in view of getting the BUFR report directly from the Air Force upper air network.  The Air Force agrees with this propose, but a technical solution to implement it was not found yet.

DCPC Argentina generates BUFR bulletins for upper-air sounding (TEMP, PILOT), CLIMAT and SYNOP from automatic and manned weather stations together with the same information in TAC.  Aircraft data are also being provided by Argentina in BUFR.

Chile does not report TAC bulletins any more.  It reports BUFR bulletins from surface weather station with originating centre 45 (Santiago - Chile) and from aircraft data with generate centre 56 (ARINC Centre).

Colombia (Bogota) has already generated BUFR bulletins from surface weather station, using the originating center code 54, and inserting them into the GTS through Brasilia.

Peru, Ecuador and Venezuela have also started to generates BUFR bulletins. Peru is very advanced in this task.

Satellite data (migrated to BUFR):

Satellite data have been generated by Brazil and Argentina. In the case of Brazil, the satellite data are generated by the Brazilian Space Agency (INPE/CPTEC) (originating centre 45) and ingested at the GTS in Brasilia. In Argentina, satellite data are generated by the National Commission for Space Activities (CONAE) (originating centre 147).

Individual country (migrating to BUFR):

The National Centre in Venezuela informed some progress in the creation of BUFR bulletins but they are not producing at the operational level yet.

GISC Brasilia and DCPC Buenos Aires still converting TAC bulletins into TDCF for those centres, which are not able yet to complete their migration work.

Use of BUFR messages:

Other migration aspect is the use of BUFR bulletin in Region III.  At moment the information is that only INPE/CPTEC and very recently the Navy centre besides INMET/Brasilia are capable to use BUFR data internally for generates products or assimilate in modelling.

A training for use of BUFR for two persons from Paraguay will be provided by Mr Jose Mauro de Rezende (INMET/Brasilia).  The same is expected for Uruguay, Bolivia, Suriname and Guyana.

Verification:

Verifications were made by INMET and INPE/CPTEC on the BUFR data provided in Region III. Some issues have been identified, such as the correct use of the originating centre code.  Mr de Rezende (INMET) is in contact with the respective centres to assist in solving these issues.

Monitoring resume of BUFR bulletins available in Region III during 24 hours (16th July 2017 and CLIMAT on 4th July 2017)

|  | T1T2 | BUFR Generate Centre | Category - Description |
| --- | --- | --- | --- |
| SABM – Buenos Aires - Argentina | ISII | 41 – Buenos Aires (RSMC) | 0 - surface |
|  | ISC/ |  | 0- Surface (CLIMAT) |
|  | IUKI, IUJI |  | 2 - Radiosonde |
|  | IUAG |  | 4 - Aircraft |
| SBBR Brasilia – Brazil | ISAI ISII ISME | 43 – Brasilia(RSMC/RAFC) | 0 – Surface Land |
|  | ISC/ |  | 0 – Surface Land (CLIMAT) |
|  | IUKI, IUSI, IUSE,IUSX, IUWA,IUWE,IUWI,IUWX, IUJE |  | 2 – Radiosonde |
| SEQU – Quito - Ecuador | ISME | 43- Brasilia (RSMC/RAFC) | 0 - Surface |
| SKBO – Bogota (Colombia) | ISME | 43 - Brasilia (RSMC/RAFC) | 0 - Surface |
|  | ISII01 | 54 - Montreal (RSMC) | 0 - Surface |
| SCSC - Santiago - Chile | ISAI | 45 - Santiago - Chile | 0 - Surface |
|  | IUAI | 56 - ARINC Centre | 4 - Aircraft |
| SOCA - Cayenne/Rochambeau (Cayenne)- French Guiana | IUKE | 85 - Toulouse (RSMC) | 2 – Radiosonde |
| SPIM – Lima (Peru) | ISMI | 49 – Peru (NMC) | 0 -Surface (*) |

(*) Bulletin previously reported but not reported as of the date of this verification.

Satellite information

|  |  | BUFR Generate Centre | Type |
| --- | --- | --- | --- |
| SABM – Buenos Aires - Argentina | INAI | 147 - National Commission on Space Activities (CONAE) | 3 – Vertical Profile |
| SBBR Brasilia – Brazil | INAX | 46 - Brazilian Space Agency (INPE/CPTEC) | 3 – Vertical Profile |
|  | INCS |  | 21 – Radiance |

Figure 1:BUFR data in region III  on 2017-07-16. Presence of BUFR data  in the grid of 1 x 1 degrees (Except moored buoys and satellite data)

- 2017-8.2.4.1 (CM-I)/Availability of high-resolution radiosonde data from U.S.A.

| WMO block/station ID | GTS headers | Site location |
| --- | --- | --- |
| 70133 | IU[KS]N05 PAOT | Koztebue, AK |
| 70200 | IU[KS]N05 PAOM | Nome, AK |
| 70261 | IU[KS]N05 PAFA | Fairbanks, AK |
| 70308 | IU[KS]N05 PASN | St. Paul Island, AK |
| 70398 | IU[KS]N05 PANT | Annette Island, AK |
| 72206 | IU[KS]N02 KJAX | Jacksonville, FL |
| 72208 | IU[KS]N01 KCHS | Charleston, SC |
| 72215 | IU[KS]N02 KFFC | Peachtree City, GA |
| 72230 | IU[KS]N02 KBMX | Birmingham, AL |
| 72233 | IU[KS]N02 KLIX | Slidell, LA |
| 72235 | IU[KS]N02 KJAN | Jackson, MS |
| 72248 | IU[KS]N02 KSHV | Shreveport, LA |
| 72317 | IU[KS]N01 KGSO | Greensboro, NC |
| 72363 | IU[KS]N02 KAMA | Amarillo, TX |
| 72426 | IU[KS]N01 KILN | Wilmington, OH |
| 72489 | IU[KS]N04 KREV | Reno, NV |
| 72501 | IU[KS]N01 KOKX | Upton, NY |
| 72518 | IU[KS]N01 KALY | Albany, NY |
| 72520 | IU[KS]N01 KPBZ | Pittsburgh, PA |
| 72528 | IU[KS]N01 KBUF | Buffalo, NY |
| 72634 | IU[KS]N03 KAPX | Gaylord, MI |
| 72712 | IU[KS]N01 KCAR | Caribou, ME |
| 74389 | IU[KS]N01 KGYX | Gray, ME |
| 74494 | IU[KS]N01 KCHH | Chatham, MA |
| 78526 | IU[KS]N02 TJSJ | San Juan, PR |

- 2017-8.2.5.1(CM-I)/TDCF Migration for Hi-Res Radiosonde and hourly SYNOP

List of Hi-Res Upper-air Radiosonde BUFR Bulletins

| 94802 | IUKK09 AMMC | 94653 | IUKK72 AMMC | 94229 | IUKK85 AMMC |
| --- | --- | --- | --- | --- | --- |
| 94802 | IUSK09 AMMC | 94653 | IUSK72 AMMC | 94229 | IUSK85 AMMC |
| 94203 | IUKK10 AMMC | 94461 | IUKK73 AMMC | 94711 | IUKK86 AMMC |
| 94203 | IUSK10 AMMC | 94461 | IUSK73 AMMC | 94711 | IUSK86 AMMC |
| 96996 | IUKK19 AMMC | 94821 | IUKK74 AMMC | 94995 | IUKK88 AMMC |
| 96996 | IUSK19 AMMC | 94821 | IUSK74 AMMC | 94995 | IUSK88 AMMC |
| 94638 | IUKK60 AMMC | 94659 | IUKK75 AMMC | 95527 | IUKK89 AMMC |
| 94638 | IUSK60 AMMC | 94659 | IUSK75 AMMC | 95527 | IUSK89 AMMC |
| 94403 | IUKK61 AMMC | 94578 | IUKK76 AMMC | 94996 | IUKK90 AMMC |
| 94403 | IUSK61 AMMC | 94578 | IUSK76 AMMC | 94996 | IUSK90 AMMC |
| 94212 | IUKK62 AMMC | 94510 | IUKK78 AMMC | 94767 | IUKK91 AMMC |
| 94212 | IUSK62 AMMC | 94510 | IUSK78 AMMC | 94767 | IUSK91 AMMC |
| 94637 | IUKK63 AMMC | 94346 | IUKK79 AMMC | 94910 | IUKK92 AMMC |
| 94637 | IUSK63 AMMC | 94346 | IUSK79 AMMC | 94910 | IUSK92 AMMC |
| 94430 | IUKK65 AMMC | 94367 | IUKK80 AMMC | 94776 | IUKK93 AMMC |
| 94430 | IUSK65 AMMC | 94367 | IUSK80 AMMC | 94776 | IUSK93 AMMC |
| 94312 | IUKK67 AMMC | 94332 | IUKK81 AMMC | 94866 | IUKK94 AMMC |
| 94312 | IUSK67 AMMC | 94332 | IUSK81 AMMC | 94866 | IUSK94 AMMC |
| 94326 | IUKK68 AMMC | 94374 | IUKK82 AMMC | 89611 | IUKK95 AMMC |
| 94326 | IUSK68 AMMC | 94374 | IUSK82 AMMC | 89611 | IUSK95 AMMC |
| 94150 | IUKK70 AMMC | 94294 | IUKK83 AMMC | 94998 | IUKK98 AMMC |
| 94150 | IUSK70 AMMC | 94294 | IUSK83 AMMC | 94998 | IUSK98 AMMC |
| 94672 | IUKK71 AMMC | 94170 | IUKK84 AMMC | 89564 | IUKK99 AMMC |
| 94672 | IUSK71 AMMC | 94170 | IUSK84 AMMC | 89564 | IUSK99 AMMC |

- 2017-8.2.7(CM-I)/Status of migration in JCOMM

Number of observations from marine platforms reporting in TAC and BUFR over the periods specified

| Platform | Format | Sampling period | Count | Source |
| --- | --- | --- | --- | --- |
| VOS | BUFR | 2017/06/01 – 2017/06/30 | 145841 | JCOMMOPS |
|  | TAC | 2017/06/01 – 2017/06/30 | 159029 | JCOMMOPS |
|  | BUFR | 2017/06/29 – 2017/06/29 | 6489 | Paul Poli (Météo-France) |
|  | TAC | 2017/06/29 – 2017/06/29 | 7281 | Paul Poli (Météo-France) |
| Argo | BUFR | 2016/03/01 – 2017/02/28 | 12396 / month | Anh Tran (Fisheries and Oceans Canada) |
|  | TAC | 2016/03/01 – 2017/02/28 | 12446 / month | Anh Tran (Fisheries and Oceans Canada) |
| Moored buoy | BUFR | 2017/06/29 – 2017/06/29 | 8867 | Paul Poli (Météo-France) |
|  | TAC | 2017/06/29 – 2017/06/29 | 8562 | Paul Poli (Météo-France) |
| Drifting buoy | BUFR | 2017/06/29 – 2017/06/29 | 17888 | Paul Poli (Météo-France) |
|  | TAC | 2017/06/29 – 2017/06/29 | 14653 | Paul Poli (Météo-France) |

Templates for reporting marine meteorological and oceanographic observations in BUFR

| TAC | Description | Current template(s) | Status | Plans/comments |
| --- | --- | --- | --- | --- |
| FM13-XIV SHIP | VOS data | B/C10 - Regulations for reporting SHIP data in TDCF | Operational (TM308009) |  |
|  | VOS data | Synoptic reports from sea stations suitable for SHIP observation data from VOS stations | Operational (TM308014) | Replaces / supersedes TM308009 |
|  | Offshore platforms | Template for the representation of observations from offshore platforms | Validated (TM308017) | Proposed FT2017-2 implementation |
| FM18-XII BUOY | Drifting buoy data | Template for the representation from drifting buoys | Operational (TM315009) | Simplified template specific to drifting buoys |
|  | Moored buoy data | Template for the representation of data from moored buoys | Operational (TM315008) | Simplified template specific to moored buoys, including directional and non-directional wave data |
|  | Wave buoy data | Template for the representation of data from moored buoys | Operational (TM315008) | Plans to update template to include additional wave parameters. |
|  | Argo data | Sub-surface profiling floats | Operational (TM315003) | Additional sequences being defined as required to extend template. |
| FM36-XI Ext. TEMP SHIP | ASAP data | B/C25 - Regulations for reporting TEMP, TEMP SHIP, TEMP MOBIL data in TDCF | Operational (TM309052) | No specific plan for this template |
|  | ASAP data | UKMO template for representation of radiosonde data with geopotential height as the vertical coordinate | Operational (revisited in July 2010) |  |
| FM62-VIII Ext. TRACKOB | TRACKOB data | TRACKOB data – ThermoSalinoGraph (TSG) data and metadata | Operational (TM308010) | Plans to update template to include additional metadata. |
| FM63-XI Ext. BATHY | XBT data | New BUFR template for XBT Temperature Profile data | Operational (TM315004) |  |
| FM64-XI Ext. TESAC | CTD / TESAC | Template for the representation of data derived from a ship based lowered instrument measuring subsurface seawater temperature, salinity and current profiles. | Operational (TM 315007) |  |
| FM65-XI Ext. WAVEOB | Wave buoy data | Templates for the wave observations from different platforms suitable for WAVEOB data | Operational (TM308015) and (TM308016) |  |
| N/A | Sea-level data | BUFR/CREX templates for tsunameter data and dart buoy system messages | Operational (TM306027) | No specific plan for this template |
|  | Sea-level data | BUFR/CREX templates for reporting time series of tide data | Validation | Template should undergo validation |

10.	IPET-CM AND TASK TEAMS

- 2017-10.1(CM-I)/IPET-CM and establishment of task teams

Task teams agreed by IPET-CM and the Teams Of References (TORs),

Task Team on GRIB (TT-GRIB): Lead - Dr Enrico Fucile

a)	Further develop GRIB edition 3.

b)	Define the GRIB format in a way that is compatible with the WMO Logical Data Model.

Task Team on BUFR (TT-BUFR): Lead - Mr Yves Pelletier

a)	Develop the next edition of BUFR to meet requirements that are not met by previous BUFR editions.

b)	Define the BUFR format in a way that is compatible with the WMO Logical Data Model.

Task Team on Migration to Table-Driven Code Forms (TT-MTDCF): Lead - Mr Abderrazak Lemkhenter

a)	Assist the WIGOS Data Quality Monitoring System (WDQMS) in understanding and dealing with erroneous BUFR messages as and when they are identified by WDQMS.

b)	Monitor progress towards, and coordinate actions to implement, migration to TDCF.

c)	Report annually the status to IPET-CM.

d)	Contribute to the maintenance of WIS competencies related to use of codes and associated training and learning guides and facilitate trainings.

Task Team on Aviation Coding Issues (TT-AvCI): Lead - Mr Yves Pelletier

a)	Develop amendments to the Manual on Codes specific to aviation, including Traditional Alphanumeric Codes (TAC) and Table-Driven Code Forms (TDCF) in collaboration with the International Civil Aviation Organization (ICAO).

b)	Support IPET-DD in formulating amendments to Volume I.3 of the Manual on Codes.

11.	COLLABORATION WITH OTHER ORGANIZATIONS AND TECHNICAL BODIES

- 2017-11.1(CM-I)/Report from CBS-MG Task Team on Upper Air BUFR (TT-UABUFR)

Outline procedure for correcting errors in upper air BUFR reports

Types of error

NWP centres have investigated errors in upper air reports issued in BUFR and have proposed a classification of the type of error as in Table 1.

Table 1. Classification of types of errors in upper air BUFR reports

| Category of issue | Monitoring tasks – aspects to be checked |
| --- | --- |
| Missing | Comparison against bulletins received the previous month, Comparison against TAC reports whilst they are still sent |
| Meta | Locations/elevations compared against local station lists and against OSCAR/Surface in future |
| Obs | Ongoing systematic errors in observations |
| Coding | Ongoing systematic errors in how the data are placed in BUFR; bulletins rejected by well validated BUFR decoders |
| Structure | Stations reporting reformatted BUFR, BUFR by parts, etc. |

Procedure to identify and correct errors

Step 1: Monitor reports and identify errors.

At least once a month, participating NWP centres will use their normal monitoring procedures to prepare a list of those stations whose reports have persistent errors – an issues list.

Each NWP centre will discuss its issues list with at least one other NWP centre and record issues that both centres agree merit further investigation on an “issues list” shared by all participating NWP centres.

Step 2: Identify issues that need to be resolved.

This step is performed by coordinating centres with responsibility for coordinating the response to data quality issues. For the pilot project, GISCs would assume this responsibility for those countries for which they are the Principal GISC.

Some of the issues identified may be the result of transmission or decoding problems at the NWP centres providing the reports. The coordinating centre checks whether it can reproduce the issue. It then provides feedback on the status of the issue to the NWP group. Those issues that are confirmed to be real are passed to the next step.

Step 3. Coordinate with data providers

The coordinating centre notifies data originators of reports associated with each issue and works with them to plan how the issue will be resolved. In some cases, it may be appropriate to work with an intermediate body (such as an RTH that is providing a transformation service between TDCF and TAC on behalf of countries).

The coordinating centre should update the issues list with the current status of the issue.

[END OF ANNEX TO THE REPORT]

[NEW PROPOSALS THROUGH PFC]

The following lists new amendments proposed through the practice between meetings of IPET-DRMM.

1.	PFC2016-2-1	New entry in common code table C-8 [FT2017-1] <para 6.1>

Add an entry:

in common code table C-8,

| Code | Agency | Type | Short name | Long name |
| --- | --- | --- | --- | --- |
| 617 | NOAA | Imaging multi-spectral radiometer | ABI | Advanced baseline imager |
| 618 | NOAA | High-resolution optical imager | GLM | Geostationary lightning mapper |

2.	PFC2016-2-2	New entry in GRIB2 code table 4.2 [FT2017-1] <para 6.1>

Add an entry:

in GRIB2 code table 4.2,

Product discipline 0 – Meteorological products, Parameter category 7: thermodynamical stability

19	Convective available potential energy – shear	m2 s–2

Description:

CAPE-shear parameter is a product of wind shear and sqrt(CAPE):

CAPE-shear parameter = wind shear*sqrt(CAPE)

Wind shear denotes the deep layer shear defined as the absolute value of the wind vector difference between two levels and the second term sqrt(CAPE) is the square root of the standard convective available potential energy.

3.	PFC2016-2-3	New entry in common code table C-12 [FT2017-1] <para 4.1>

Add an entry:

in common code table C-12,

under 254 "EUMETSAT Operation Centre",

125	Ford Island, Hawaii

4.	PFC2016-2-4	New GRIB2 templates related to distribution functions and associated table entries [FT2017-1] <para 6.1>

Add entries:

in code table 4.240,

8	No distribution function.  The encoded variable is derived from variables characterized by type of distribution function of type no. 7 (see above) with fixed variance σ (p1) and fixed particle density ρ (p2)

in code table 4.0,

67	Average, accumulation and/or extreme values or other statistically processed values at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for atmospheric chemical constituents based on a distribution function

68	Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for atmospheric chemical constituents based on a distribution function

Add product definition templates 4.67 and 4.68:

Product definition template 4.67 – Average, accumulation and/or extreme values or other statistically processed values at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for atmospheric chemical constituents based on a distribution function

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12–13	Atmospheric chemical constituent type (see Code table 4.230)

14–15	Number of mode (N) of distribution (see Note 2)

16–17	Mode number (l)

18–19	Type of distribution function (see Code table 4.240, see Note 3)

20	Number of following function parameters (Np), defined by type given in octets 18–19 (Type of distribution function)

Repeat the following 5 octets for the number of function parameters (n = 1, Np), if Np > 0

21+5(n–1)	List of scale factor of fixed distribution function parameter (p1–pNp), defined by type of distribution in octets 18–19

(22+5(n–1))–(25+5(n–1))	List of scaled value of fixed distribution function parameter (p1–pNp), defined by type of distribution in octets 18–19

21+5Np	Type of generating process (see Code table 4.3)

22+5Np	Background generating process identifier (defined by originating centre)

23+5Np	Analysis or forecast generating process identifier (defined by originating centre)

(24+5Np)–(25+5Np)	Hours of observational data cut-off after reference time (see Note 1)

26+5Np	Minutes of observational data cut-off after reference time

27+5Np	Indicator of unit of time range (see Code table 4.4)

(28+5Np)–(31+5Np)	Forecast time in units defined by the previous octet (see Note 4)

32+5Np	Type of first fixed surface (see Code table 4.5)

33+5Np	Scale factor of first fixed surface

(34+5Np)–(37+5Np)	Scaled value of first fixed surface

38+5Np	Type of second fixed surface (see Code table 4.5)

39+5Np	Scale factor of second fixed surface

(40+5Np)–(43+5Np)	Scaled value of second fixed surface

(44+5Np)–(45+5Np)	Year

(46+5Np)	Month

(47+5Np)	Day          Time of end of overall time interval

(48+5Np)	Hour

(49+5Np)	Minute

(50+5Np)	Second

(51+5Np)	n – number of time range specifications describing the time intervals used to calculate the statistically processed field

(52+5Np)–(55+5Np)	Total number of data values missing in statistical process 
(56+5Np)–(67+5Np)   Specification of the outermost (or only) time range over which statistical processing is done

(56+5Np)	Statistical process used to calculate the processed field from the field at each time increment during the time range (see Code table 4.10)

(57+5Np)	Type of time increment between successive fields used in the statistical processing (see Code table 4.11)

(58+5Np)	Indicator of unit of time for time range over which statistical processing is done (see Code table 4.4)

(59+5Np)–(62+5Np)	Length of the time range over which statistical processing is done, in units defined by the previous octet

(63+5Np)	Indicator of unit of time for the increment between the successive fields used (see Code table 4.4)

(64+5Np)–(67+5Np)	Time increment between successive fields, in units defined by the previous octet (see Notes 5 and 6)
(68+5Np)–nn   These octets are included only if n > 1, where nn = (55+5Np) + 12 x n

(68+5Np)–(79+5Np)	As octets (56+5Np) to (67+5Np), next innermost step of processing

(80+5Np)–nn	Additional time range specifications, included in accordance with the value of n. Contents as octets (56+5Np) to (67+5Np), repeated as necessary

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	If Number of mode (N) > 1, then between x N fields with mode number l = 1, …, N define the distribution function.  x is the number of variable parameters in the distribution function.

(3)	For more information, see Attachment III (Distribution functions in GRIB) in Part B of this volume (I.2 – Att.III/GRIB – 1 to 2).

(4)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(5)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples.  Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge.

(6)	The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment.  For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast times.

Product definition template 4. 68 - Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for atmospheric chemical constituents based on a distribution function

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12–13	Atmospheric chemical constituent type (see Code table 4.230)

14–15	Number of mode (N) of distribution (see Note 2)

16–17	Mode number (l)

18–19	Type of distribution function (see Code table 4.240, see Note 3)

20	Number of following function parameters (Np), defined by type given in octets 18–19 (Type of distribution function)

Repeat the following 5 octets for the number of function parameters (n = 1, Np), if Np > 0

21+5(n–1)	List of scale factor of fixed distribution function parameter (p1–pNp), defined by type of distribution in octets 18–19

(22+5(n–1))–(25+5(n–1))	List of scaled value of fixed distribution function parameter (p1–pNp), defined by type of distribution in octets 18–19

21+5Np	Type of generating process (see Code table 4.3)

22+5Np	Background generating process identifier (defined by originating centre)

23+5Np	Analysis or forecast generating process identifier (defined by originating centre)

(24+5Np)–(25+5Np)	Hours of observational data cut-off after reference time (see Note 1)

26+5Np	Minutes of observational data cut-off after reference time

27+5Np	Indicator of unit of time range (see Code table 4.4)

(28+5Np)–(31+5Np)	Forecast time in units defined by the previous octet (see Note 4)

32+5Np	Type of first fixed surface (see Code table 4.5)

33+5Np	Scale factor of first fixed surface

(34+5Np)–(37+5Np)	Scaled value of first fixed surface

38+5Np	Type of second fixed surface (see Code table 4.5)

39+5Np	Scale factor of second fixed surface

(40+5Np)–(43+5Np)	Scaled value of second fixed surface

44+5Np	Type of ensemble forecast (see code table 4.6)

45+5Np	Perturbation number

46+5Np	Number of forecasts in ensemble

(47+5Np)–(48+5Np)	Year

(49+5Np)	Month

(50+5Np)	Day          Time of end of overall time interval

(51+5Np)	Hour

(52+5Np)	Minute

(53+5Np)	Second

(54+5Np)	n – number of time range specifications describing the time intervals used to calculate the statistically processed field

(55+5Np)–(58+5Np)	Total number of data values missing in statistical process
(59+5Np)–(70+5Np)   Specification of the outermost (or only) time range over which statistical processing is done

(59+5Np)	Statistical process used to calculate the processed field from the field at each time increment during the time range (see Code table 4.10)

(60+5Np)	Type of time increment between successive fields used in the statistical processing (see Code table 4.11)

(61+5Np)	Indicator of unit of time for time range over which statistical processing is done (see Code table 4.4)

(62+5Np)–(65+5Np)	Length of the time range over which statistical processing is done, in units defined by the previous octet

(66+5Np)	Indicator of unit of time for the increment between the successive fields used (see Code table 4.4)

(67+5Np)–(70+5Np)	Time increment between successive fields, in units defined by the previous octet (see Notes 5 and 6)

(71+5Np)–nn   These octets are included only if n > 1, where nn = (58+5Np) + 12 x n

(71+5Np)–(82+5Np)	As octets (59+5Np) to (70+5Np), next innermost step of processing

(83+5Np)–nn	Additional time range specifications, included in accordance with the value of n.  Contents as octets (59+5Np) to (70+5Np), repeated as necessary

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	If Number of mode (N) > 1, then between x N fields with mode number l = 1, …, N define the distribution function.  x is the number of variable parameters in the distribution function.

(3)	For more information, see Attachment III (Distribution functions in GRIB) in Part B of this volume (I.2 – Att.III/GRIB – 1 to 2).

(4)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(5)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples.  Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge.

(6)	The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment.  For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast times.

5.	PFC2016-2.5	Clarification of an entry in code table 0 01 150 [FT2017-1] <para 6.1>

Amend the entry 4 and Note 2:

In code table 0 01 150,

| Code figure |  |
| --- | --- |
| 4 | Ellipsoidal datum using the International Reference Meridian and the International Reference Pole as the prime meridian and prime pole, respectively, and the origin of the International Terrestrial Reference System (ITRS) (see Note 2). International Reference Meridian, International Reference Pole and ITRS are maintained by the International Earth Rotation and Reference Systems Service (IERS) |

Notes:

(2)	When Code figure 4 is used to specify a custom coordinate reference system, the ellipsoidal datum shall be an oblate ellipsoid of revolution, where the major axis is uniplanar with the equatorial plane and the minor axis traverses the prime meridian towards the prime pole. North corresponds to the direction from the equator to the prime pole. East corresponds to the counter-clockwise direction from the prime meridian as viewed from above the North Pole. In this case, the semi-major and semi-minor axes must be specified (e.g. by descriptors 0 01 152 and 0 01 153).

6.	PFC2016-2-6	New entry in common code table C-5 [FT2017-1] <para 4.1>

Add an entry:

in Common Code table C-5,

| Code figure for I6I6I6 | Code figure for BUFR | Code figure for GRIB | Short name |
| --- | --- | --- | --- |
| 422 | 422 | 422 | ScatSat-1 |

[REVISIONS AND ADDITIONS IN RELATION TO IPET-DRMM-IV]

The following lists amendments revised or additionally proposed in relation to the IPET-DRMM-IV for approval.

Some amendments editorially corrected may not be shown here, e.g. center to centre, sea surface to sea-surface, dew point to dewpoint.

1.	New entries in Common Code Table C-5 [FT2016-2]

Add entries:

in common code table C-5,

| Code figure for I6I6I6 | Code figure for BUFR | Code figure for GRIB2 |  |
| --- | --- | --- | --- |
| 225 | 225 | 225 | NOAA 20 |
| 226 | 226 | 226 | NOAA 21 |

[PROPOSALS UNDER VALIDATION]

FM 92 GRIB

- 2016-2.2.1(DRMM-IV)/New parameter for tropical cyclone forecasts [FT2016-2] <para 6.1>

Add a new parameter:

in Code Table 4.2, Discipline 10 (Oceanographic products), Category 3 (Surface properties)

Number	Parameter	Units

2		Heat exchange coefficient		  -

- 2016-2.2.2(DRMM-IV)/New GRIB template for post-processed NWP output [FT2016-2] <para 6.1>

Add new templates:

Product definition template 4.70 – Post-processing analysis or forecast at a horizontal level or in a horizontal layer at a point in time

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)
	12-13	Input process identifier (see Note 1)
	14-15	Input originating centre (see common code table C-11 and Note 2)
	16	Type of post-processing (see Note 3)

17	Type of generating process (see code table 4.3)

18	Background generating process identifier (defined by originating centre)

19	Analysis or forecast generating process identifier (defined by originating centre)

20–21	Hours of observational data cut-off after reference time (see Note 4)

22	Minutes of observational data cut-off after reference time

23	Indicator of unit of time range (see code table 4.4)

24–27	Forecast time in units defined by octet 23

28	Type of first fixed surface (see code table 4.5)

29	Scale factor of first fixed surface

30–33	Scaled value of first fixed surface

34	Type of second fixed surface (see code table 4.5)

35	Scale factor of second fixed surface

36-39	Scaled value of second fixed surface

Notes:

(1)	The input process identifier shall have the value of the “analysis or forecast process identifier” of the original GRIB message used as input of the post-processing.

(2)	The input originating centre shall have the value of the “originating centre” of the original GRIB message used as input of the post-processing.

(3)	This identifies which post-processing technique was used. This is defined by the originating centre.

(4)	Hours greater than 65534 will be coded as 65534.

Product definition template 4.71 – Post-processing individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time

Octet No.	Contents

10	Parameter category (see code table 4.1)
	11	Parameter number (see code table 4.2)
	12-13	Input process identifier (see Note 1)
	14-15	Input originating centre (see common code table C-11 and Note 2)
	16	Type of post-processing (see Note 3)

17		Type of generating process (see code table 4.3)

18	Background generating process identifier (defined by originating centre)

19 	Forecast generating process identifier (defined by originating centre)

20–21 	Hours after reference time of data cut-off (see Note 4)

22	Minutes after reference time of data cut-off

23	Indicator of unit of time range (see code table 4.4)

24–27 	Forecast time in units defined by octet 23

28 	Type of first fixed surface (see code table 4.5)

29 	Scale factor of first fixed surface

30–33 	Scaled value of first fixed surface

34	Type of second fixed surface (see code table 4.5)

35	Scale factor of second fixed surface

36–39 	Scaled value of second fixed surface

40	Type of ensemble forecast (see code table 4.6)

41	Perturbation number

42 	Number of forecasts in ensemble

Notes:

(1)	The input process identifier shall have the value of the “analysis or forecast process identifier” of the original GRIB message used as input of the post-processing.

(2)	The input originating centre shall have the value of the “originating centre” of the original GRIB message used as input of the post-processing.

(3)	This identifies which post-processing technique was used. This is defined by the originating centre.

(4)	Hours greater than 65534 will be coded as 65534.

Product definition template 4.72 – Post-processing average, accumulation, extreme values or other statistically processed values at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)

12-13	Input process identifier (see Note 1)

14-15	Input originating centre (see common code table C-11 and Note 2)
	16	Type of post-processing (see Note 3)
	17	Type of generating process (see code table 4.3)

18	Background generating process identifier (defined by originating centre)

19	Analysis or forecast generating process identifier (defined by originating centre)

20–21	Hours after reference time of data cut-off (see Note 4)

22	Minutes after reference time of data cut-off

23	Indicator of unit of time range (see code table 4.4)

24–27	Forecast time in units defined by octet 23 (see Note 5)

28	Type of first fixed surface (see code table 4.5)

29	Scale factor of first fixed surface

30–33	Scaled value of first fixed surface

34	Type of second fixed surface (see code table 4.5)

35	Scale factor of second fixed surface

36–39	Scaled value of second fixed surface

40–41	Year

42	Month

Time of end of overall time interval

43	Day

44	Hour

45	Minute

46	Second

47	n – number of time range specifications describing the time intervals used to calculate the 		statistically processed field

48–51	Total number of data values missing in statistical process

52–63	Specification of the outermost (or only) time range over which statistical processing 		is done

52	Statistical process used to calculate the processed field from the field at each time increment 		during the time range (see code table 4.10)

53	Type of time increment between successive fields used in the statistical processing (see code 		table 4.11)

54	Indicator of unit of time for time range over which statistical processing is done (see code 			table 4.4)

55–58	Length of the time range over which statistical processing is done, in units defined by the 			previous octet

59	Indicator of unit of time for the increment between the successive fields used (see code table 		4.4)

60–63	Time increment between successive fields, in units defined by the previous octet (see Notes 		6 and 7)

64–nn	These octets are included only if n > 1, where nn = 51 + 12 x n

64–75	As octets 52 to 63, next innermost step of processing

76–nn	Additional time range specifications, included in accordance with the value of n. Contents as 		octets 52 to 63, repeated as necessary

Notes:

(1)	The input process identifier shall have the value of the “analysis or forecast process identifier” of the original GRIB message used as input of the post-processing.

(2)	The input originating centre shall have the value of the “originating centre” of the original GRIB message used as input of the post-processing.

(3)	This identifies which post-processing technique was used. This is defined by the originating centre.

(4)	Hours greater than 65534 will be coded as 65534.

(5)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(6)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge.

(7)	The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment (one of octets 63, 65, 77, ...). For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast times.

Product definition template 4.73 – Post-processing individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer, in a continuous or non-continuous time interval

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)
	12-13	Input process identifier (see Note 1)
	14-15	Input originating centre (see common code table C-11 and Note 2)
	16	Type of post-processing (see Note 3)

17	Type of generating process (see code table 4.3)

18	Background generating process identifier (defined by originating centre)

19	Forecast generating process identifier (defined by originating centre)

20–21	Hours after reference time of data cut-off (see Note 4)

22	Minutes after reference time of data cut-off

23	Indicator of unit of time range (see code table 4.4)

24–27	Forecast time in units defined by octet 23 (see Note 5)

28	Type of first fixed surface (see code table 4.5)

29	Scale factor of first fixed surface

30–33	Scaled value of first fixed surface

34	Type of second fixed surface (see code table 4.5)

35	Scale factor of second fixed surface

36–39	Scaled value of second fixed surface

40	Type of ensemble forecast (see code table 4.6)

41	Perturbation number

42	Number of forecasts in ensemble

43–44	Year of end of overall time interval

45	Month of end of overall time interval

46	Day of end of overall time interval

47	Hour of end of overall time interval

48	Minute of end of overall time interval

49	Second of end of overall time interval

50		n – number of time range specifications describing the time intervals used to calculate the 		statistically processed field

51–54	Total number of data values missing in statistical process

55–66	Specification of the outermost (or only) time range over which statistical processing 		is done

55	Statistical process used to calculate the processed field from the field at each time increment 		during the time range (see code table 4.10)

56	Type of time increment between successive fields used in the statistical processing (see code 		table 4.11)

57	Indicator of unit of time for time range over which statistical processing is done (see code 			table 4.4)

58–61	Length of the time range over which statistical processing is done, in units defined by the 			previous octet

62	Indicator of unit of time for the increment between the successive fields used (see code 			table 4.4)

63–66	Time increment between successive fields, in units defined by the previous octet (see Note 		6)

67–nn These octets are included only if n > 1, where nn = 54 + 12 x n

67–78	As octets 55 to 66, next innermost step of processing

79–nn	Additional time range specifications, included in accordance with the value of n. Contents as 		octets 55 to 66, repeated as necessary

Notes:

(1)	The input process identifier shall have the value of the “analysis or forecast process identifier” of the original GRIB message used as input of the post-processing.

(2)	The input originating centre shall have the value of the “originating centre” of the original GRIB message used as input of the post-processing.

(3)	This identifies which post-processing technique was used. This is defined by the originating centre.

(4)	Hours greater than 65534 will be coded as 65534.

(5)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(6)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge. The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment (one of octets 56, 68, 80, ...). For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast times

Add new entries:

in code table 4.0,

Code	Meaning

70	Post-processing analysis or forecast at a horizontal level or in a horizontal layer at a point in 		time
	71	Post-processing individual ensemble forecast, control and perturbed, at a horizontal level or 		in a horizontal layer at a point in time
	72	Post-processing average, accumulation, extreme values or other statistically processed 			values at a horizontal level or in a horizontal layer in a continuous or non-continuous time 			interval
	73	Post-processing individual ensemble forecast, control and perturbed, at a horizontal level or 		in a horizontal layer, in a continuous or non-continuous time interval

- 2016-2.2.3(DRMM-IV)/New GRIB table entries (unbalanced components) [FT2016-2] <para 6.1>

Add entries:

in code table 4.2,

Product Discipline 0 – Meteorological products, Parameter category 0: temperature

| number | parameter | units |
| --- | --- | --- |
| 28 | Unbalanced component of temperature | K |

Description:

28: Residual resulting from subtracting from temperature an approximate "balanced" value derived from relevant variable(s).

Product Discipline 0 – Meteorological products, Parameter category 1: Moisture

| number | parameter | units |
| --- | --- | --- |
| 118 | Unbalanced component of specific humidity | kg kg-1 |
| 119 | Unbalanced component of specific cloud liquid water content | kg kg-1 |
| 120 | Unbalanced component of specific cloud ice water content | kg kg-1 |

Description:

- 118: Residual resulting from subtracting from specific humidity (mass of water vapour/mass of moist air) an approximate "balanced" value derived from relevant variable(s).

- 119: Residual resulting from subtracting from specific cloud liquid water content (mass of condensate/mass of moist air) an approximate "balanced" value derived from relevant variable(s).

- 120: Residual resulting from subtracting from specific cloud ice water content (mass of condensate/mass of moist air) an approximate "balanced" value derived from relevant variable(s).

Product Discipline 0 – Meteorological products, Parameter category 2: momentum

| number | parameter | units |
| --- | --- | --- |
| 45 | Unbalanced component of divergence | s-1 |

Description:

- 45: Residual resulting from subtracting from divergence an approximate "balanced" value derived from relevant variable(s).

Product Discipline 0 – Meteorological products, Parameter category 3: mass

| number | parameter | units |
| --- | --- | --- |
| 31 | Unbalanced component of logarithm of surface pressure | - |

Description:

- 31: Residual resulting from subtracting from logarithm of surface pressure an approximate "balanced" value derived from relevant variable(s). Note that this parameter is dimensionless.

- 2016-2.2.4(DRMM-IV)/Amendments in Common Table C-14 for Pollen, new parameters in Table 4.2 and new entries in Table 4.5 [FT2016-2] <para 6.1>

Add entries:

in COMMON CODE TABLE C–14,

Code figure                    Meaning

62027–62099	Reserved

62100		Alnus (Alder) pollen

62101		Betula (Birch) pollen

62102		Castanea (Chestnut) pollen

62103		Carpinus (Hornbeam) pollen

62104		Corylus (Hazel) pollen

62105		Fagus (Beech) pollen

62106		Fraxinus (Ash) pollen

62107		Pinus (Pine) pollen

62108		Platanus (Plane) pollen

62109		Populus (Cottonwood, Poplar) pollen

62110		Quercus (Oak) pollen

62111		Salix (Willow) pollen

62112		Taxus (Yew) pollen

62113		Tilia (Lime, Linden) pollen

62114		Ulmus (Elm) pollen

62115-62199	Reserved

62200		Ambrosia (Ragweed, Burr-ragweed ) pollen

62201		Artemisia (Sagebrush, Wormwood, Mugwort) pollen

62202		Brassica (Rape, Broccoli, Brussels Sprouts, Cabbage, Cauliflower, Collards, Kale, 				Kohlrabi, Mustard, Rutabaga) pollen

62203		Plantago  (Plantain) pollen

62204		Rumex (Dock, Sorrel) pollen

62205		Urtica (Nettle) pollen

62206-62299	Reserved

62300		Poaceae (Grass family) pollen

62301-65534	Reserved

65535		Missing

Editorial note: Information about pollen species is collected from www.pollen.com, www.pollenflug.de, www.metoffice.gov.uk, www.polleninfo.org and www.meteoswiss.admin.ch.

Add entries:

in GRIB2 Table 4.2, discipline 0 (meteorological products), category 18 (nuclear/radiology),

17	Column-integrated air concentration		Bq m-2

18	Column-averaged air concentration in layer		Bq m-3

in GRIB2 Table 4.2, discipline 0 (meteorological products), category 20 (atmospheric chemical constituents),

63	Column-averaged mass density in layer		kg m-3

in GRIB2 Table 4.5 – Fixed surface types and units,

21	Lowest level where mass density exceeds the specified value

(base for a given threshold of mass density)	kg m-3

22	Highest level where mass density exceeds the specified

value (top for a given threshold of mass density)	kg m-3

23	Lowest level where air concentration exceeds the specified

value (base for a given threshold of air concentration)	Bq m-3

24	Highest level where air concentration exceeds the specified

value (top for a given threshold of air concentration)	Bq m-3

- 2016-2.2.5(DRMM-IV)/New generating process and common codes to represent wildfire-driven pollution in air quality models [FT2016-2] <para 6.1>

Proposed new entries for common code table C-14 and code table 4.3

|  | Proposed new entries for Common Code Table C-14 (Atmospheric chemical or physical constituent type) |  |
| --- | --- | --- |
| Code Figure | Meaning | Chemical formula |
| 62026 | Particulate matter (PM) | none |

|  | Proposed new entries for Code Table 4.3 (Type of generating process) |
| --- | --- |
| Code Figure | Meaning |
| 18 | Difference between two forecasts |

- 2016-2.2.6(DRMM-IV)/Update of GRIB2 new compression method – CCSDS libaec [FT2016-2] <para 6.1>

Add templates:

Preliminary note:	For most templates, details of the packing process are described in regulation 92.9.4.

This template is only valid for Consultative Committee for Space Data Systems: Lossless Data Compression. CCSDS Recommendation for Space Data System Standards, CCSDS 121.0-B-2, Blue Book, May 2012.

| Data Representation Template 5.42 - Grid point and spectral data - CCSDS recommended lossless compression. |  |
| --- | --- |
| Octet No. | Contents |
| 12 – 15 | Reference value (R) (IEEE 32-bit floating-point value) |
| 16 – 17 | Binary scale factor (E) |
| 18 – 19 | Decimal scale factor (D) |
| 20 | Number of bits required to hold the resulting scaled and referenced data values (see Note 1) |
| 21 | Type of original field values (see code table 5.1) |
| 22 | ​CCSDS compression options mask (see Note 3) |
| 23 | Block size |
| 24-25 | Reference sample interval |

Notes:

(1)	The intent of this template is to scale the grid point data to obtain the desired precision, if appropriate, and then subtract the reference value from the scaled field as is done using Data Representation Template 5.0.  After this, the resulting grid point field can be treated as a grayscale image and encoded into the CCSDS recommended standard for lossless data compression code stream format. To unpack the data field, the CCSDS recommended standard for lossless data compression code stream is decoded back into an image, and the original field is obtained from the image data as described in regulation 92.9.4 Note (4).

(2)	The Consultative Committee for Space Data Systems (CCSDS) recommended standard for lossless data compression is the standard used by space agencies for the compression of scientific data transmitted from satellites and other space instruments. CCSDS recommended standard for lossless data compression is a very fast predictive compression algorithm based on the extended-Rice algorithm. It uses Golomb-Rice codes for entropy coding. The sequence of prediction errors is divided into blocks. Each block is compressed using a two-pass algorithm. In the first pass the best coding method for the whole block is determined. In the second pass, the output of the marker of the selected coding method is encoded as ancillary information along with prediction errors.
The coding methods include:

  - Golomb-Rice codes of a chosen rank

  - Unary code for transformed pairs of prediction errors

  - Fixed-length natural binary code if the block is found to be incompressible

  - Signaling to the decoder empty block if all prediction errors are zeroes

(3)	Library flags governing data type, and storage and processing parameters. For further information, see Rosenhauer, Mathis. "Flags." libaec – Adaptive Entropy Coding library. German Climate Computing Centre (Deutsches Klimarechenzentrum, DKRZ), 12 May 2016. Web. 13 June 2016. <http://gitlab.dkrz.de/k202009/libaec/blob/v0.3.3/README.md#flags>.

| Data Template 7.42 - Grid point and spectral data - CCSDS recommended lossless compression. |  |
| --- | --- |
| Octet No. | Contents |
| 6 - nn | CCSDS recommended standard for lossless data compression code stream |

Add an entry:

in code table 5.0 – Data representation template number

| Code figure | Contents |
| --- | --- |
| 42 | Grid point and spectral data - CCSDS recommended lossless compression. |

- 2016-2.2.7(DRMM-IV)/New ensemble templates and one ensemble template correction [FT2016-2] <para 6.1>

Add or amend entries:

in code table 4.0: Product definition template number,

49	Individual ensemble forecast, control and perturbed, at a horizontal level or in a 
horizontal layer at a point in time for optical properties of aerosol

50	Reserved

56	Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters (deprecated)

58	Individual ensemble forecast, control and perturbed, at a horizontal level or in a 
horizontal layer at a point in time for atmospheric chemical constituents based on a distribution function

59	Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters

(corrected version of template 4.56)

Add product definition templates:

Product definition template 4.49 – individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for optical properties of aerosol

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)

12–13	Aerosol type (see common code table C–14)

14	Type of interval for first and second size (see code table 4.91)

15	Scale factor of first size

16–19	Scaled value of first size in metres

20	Scale factor of second size

21–24	Scaled value of second size in metres

25	Type of interval for first and second wavelength (see code table 4.91)

26	Scale factor of first wavelength

27–30	Scaled value of first wavelength in metres

31	Scale factor of second wavelength

32–35	Scaled value of second wavelength in metres

36	Type of generating process (see code table 4.3)

37	Background generating process identifier (defined by originating centre)

38	Analysis or forecast generating process identifier (defined by originating centre)

39–40	Hours of observational data cut-off after reference time (see Note)

41	Minutes of observational data cut-off after reference time

42	Indicator of unit of time range (see code table 4.4)

43–46	Forecast time in units defined by octet 42

47	Type of first fixed surface (see code table 4.5)

48	Scale factor of first fixed surface

49–52	Scaled value of first fixed surface

53	Type of second fixed surface (see code table 4.5)

54	Scale factor of second fixed surface

55–58	Scaled value of second fixed surface

59	Type of ensemble forecast (see code table 4.6)

60	Perturbation number

61	Number of forecasts in ensemble

Note:	Hours greater than 65534 will be coded as 65534.

Product definition template 4.58 – individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for atmospheric chemical constituents based on a distribution function

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)

12–13	Atmospheric chemical constituent type (see code table 4.230)

14–15	Number of mode (N) of distribution (see Note 2)

16–17	Mode number (l)

18–19	Type of distribution function (see code table 4.240)

20	Number of following function parameters (Np), defined by type given in octet 18–19 (Type 		of distribution function)

Repeat the following 5 octets for the number of function parameters (n = 1, Np), if Np > 0

21+5(n–1)	List of scale factor of fixed distribution function parameter (p1–pNp), defined by type of distribution in octet 18–19

(22+5(n–1))–(25+5(n–1))	List of scaled value of fixed distribution function parameter (p1–pNp), defined by type of distribution in octet 18–19

21+5Np	Type of generating process (see code table 4.3)

22+5Np	Background generating process identifier (defined by originating centre)

23+5Np	Analysis or forecast generating process identifier (defined by originating centre)

(24+5Np)–(25+5Np)	Hours of observational data cut-off after reference time (see Note 1)

26+5Np	Minutes of observational data cut-off after reference time

27+5Np	Indicator of unit of time range (see code table 4.4)

(28+5Np)–(31+5Np)	Forecast time in units defined by the previous octet

32+5Np	Type of first fixed surface (see code table 4.5)

33+5Np	Scale factor of first fixed surface

(34+5Np)–(37+5Np)	Scaled value of first fixed surface

38+5Np	Type of second fixed surface (see code table 4.5)

39+5Np	Scale factor of second fixed surface

(40+5Np)–(43+5Np)	Scaled value of second fixed surface

44+5Np	Type of ensemble forecast (see code table 4.6)

45+5Np	Perturbation number

46+5Np	Number of forecasts in ensemble

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	If Number of mode (N) > 1, then between x N fields with mode number l = 1, …, N define the distribution function.  x is the number of variable parameters in the distribution function.

(3)	For more information, see Attachment III (Distribution functions in GRIB) in Part B of this volume (I.2 – Att.III/GRIB – 1 to 2).

Product definition template 4.56 – individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters

Note: This template is deprecated. Template 4.59 should be used instead.

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)

12	Tile classification (see code table 4.242)

13	Total number (NT) of tile/attribute pairs (see Notes 2 and 3)

14	Number of used spatial tiles (NUT) (see Notes 2 and 3)

15	Tile index (ITN = {1,…, NUT}) (see Note 2)

16	Number of used tile attributes (NAT) for tile ITN (see Note 2)

17	Attribute of tile (see Code table 4.241)) (A = {A(1),…, A(NAT(ITN))}) (see Note 2)

18	Type of generating process (see code table 4.3)

19	Background generating process identifier (defined by originating centre)

20	Analysis or forecast generating process identifier (defined by originating centre)

21–22	Hours of observational data cut-off after reference time (see Note 1)

23	Minutes of observational data cut-off after reference time

24	Indicator of unit of time range (see code table 4.4)

25–28	Forecast time in units defined by octet 24

29	Type of first fixed surface (see code table 4.5)

30	Scale factor of first fixed surface

31–34	Scaled value of first fixed surface

35	Type of second fixed surface (see code table 4.5)

36	Scale factor of second fixed surface

37–40	Scaled value of second fixed surface

41	Perturbation number

42	Number of forecasts in ensemble

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	See Note 2 under product definition template 4.55.

(3)	For more information, see Attachment IV (Spatio-temporal changing tiles in GRIB) in Part B of this volume (I.2 – Att.IV/GRIB–1 to 3).

Product definition template 4.59 – individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters

Octet No.	Contents

10	Parameter category (see code table 4.1)

11	Parameter number (see code table 4.2)

12	Tile classification (see code table 4.242)

13	Total number (NT) of tile/attribute pairs (see Notes 2 and 3)

14	Number of used spatial tiles (NUT) (see Notes 2 and 3)

15	Tile index (ITN = {1,…, NUT}) (see Note 2)

16	Number of used tile attributes (NAT) for tile ITN (see Note 2)

17	Attribute of tile (see code table 4.241)) (A = {A(1),…, A(NAT(ITN))}) (see Note 2)

18	Type of generating process (see code table 4.3)

19	Background generating process identifier (defined by originating centre)

20	Analysis or forecast generating process identifier (defined by originating centre)

21–22	Hours of observational data cut-off after reference time (see Note 1)

23	Minutes of observational data cut-off after reference time

24	Indicator of unit of time range (see code table 4.4)

25–28	Forecast time in units defined by octet 24

29	Type of first fixed surface (see code table 4.5)

30	Scale factor of first fixed surface

31–34	Scaled value of first fixed surface

35	Type of second fixed surface (see code table 4.5)

36	Scale factor of second fixed surface

37–40	Scaled value of second fixed surface

41	Type of ensemble forecast (see code table 4.6)

42	Perturbation number

43	Number of forecasts in ensemble

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	See Note 2 under product definition template 4.55.

(3)	For more information, see Attachment IV (Spatio-temporal changing tiles in GRIB) in Part B of this volume (I.2 – Att.IV/GRIB–1 to 3).

- 2016-2.2.8(DRMM-IV)/Encoding fields from Limited Area models combining conformal projections and bi-periodic spectral geometry [Validation] <para 6.1>

Add templates:

Grid definition template 3.13 - Mercator LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15-nn | Same as grid definition template 3.10 |
| [nn+1] - [nn+4] | Nux – Size of model forecast area C in X direction (number of grid points) |
| [nn+5] - [nn+8] | Ncx – Width of coupling area I in X direction (number of grid points) |
| [nn+9] - [nn+12] | Nuy – Size of model forecast area C in Y direction (number of grid points) |
| [nn+13] - [nn+16] | Ncy – Width of coupling area I in Y direction (number of grid points) |

Grid definition template 3.23 - Polar stereographic LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15-65 | Same as grid definition template 3.20 |
| 66-69 | Nux – Size of model forecast area C in X direction (number of grid points) |
| 70-73 | Ncx – Width of coupling area I in X direction (number of grid points) |
| 74-77 | Nuy – Size of model forecast area C in Y direction (number of grid points) |
| 78-81 | Ncy – Width of coupling area I in Y direction (number of grid points) |

Grid definition template 3.33 - Lambert conformal LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15-81 | Same as grid definition template 3.30 |
| 82-85 | Nux – Size of model forecast area C in X direction (number of grid points) |
| 86-89 | Ncx – Width of coupling area I in X direction (number of grid points) |
| 90-93 | Nuy – Size of model forecast area C in Y direction (number of grid points) |
| 94-97 | Ncy – Width of coupling area I in Y direction (number of grid points) |

Grid definition template 3.61 - Spectral Mercator LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15 | Spectral Representation type (see code table 3.6) |
| 16-19 | N – bi-Fourier Resolution Parameter |
| 20-23 | M – Bi-Fourier Resolution Parameter |
| 24 | Bi – Fourier Truncation Type (see code table 3.25) |
| 25-32 | Lx – Size in meters of the domain along X axis |
| 33-40 | Lux – Size in meters of model forecast area C along X axis |
| 41-48 | Lcx – Width in meters of coupling area I along X axis |
| 49-56 | Ly – Size in meters of the domain along Y axis |
| 57-64 | Luy – Size in meters of model forecast area C along Y axis |
| 65-72 | Lcy – Width in meters of coupling area I along Y axis |
| 73 | Shape of the Earth (see code table 3.2) |
| 74 | Scale factor of radius of spherical Earth |
| 75-78 | Scaled value of radius of spherical Earth |
| 79 | Scale factor of major axis of oblate spheroid Earth |
| 80-83 | Scaled value of major axis of oblate spheroid Earth |
| 84 | Scale factor of minor axis of oblate spheroid Earth |
| 85-88 | Scaled value of minor axis of oblate spheroid Earth |
| 89-92 | La1 – latitude of first grid point |
| 93-96 | Lo1 – longitude of first grid point |
| 97-100 | LaD – latitude(s) at which the Mercator projection intersects the Earth (Latitude(s) where Di and Dj are specified) |
| 101-104 | La2 – latitude of last grid point |
| 105-108 | Lo2 – longitude of last grid point |
| 109-112 | Orientation of the grid, angle between i direction on the map and the Equator (see Note 1) |

Notes:

(1) Limited to the range of 0 to 90 degrees.

Grid definition template 3.62 - Spectral polar stereographic LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15 | Spectral Representation type (see code table 3.6) |
| 16-19 | N – bi-Fourier Resolution Parameter |
| 20-23 | M – Bi-Fourier Resolution Parameter |
| 24 | Bi – Fourier Truncation Type (see code table 3.25) |
| 25-32 | Lx – Size in meters of the domain along X axis |
| 33-40 | Lux – Size in meters of model forecast area C along X axis |
| 41-48 | Lcx – Width in meters of coupling area I along X axis |
| 49-56 | Ly – Size in meters of the domain along Y axis |
| 57-64 | Luy – Size in meters of model forecast area C along Y axis |
| 65-72 | Lcy – Width in meters of coupling area I along Y axis |
| 73 | Shape of the Earth (see code table 3.2) |
| 74 | Scale factor of radius of spherical Earth |
| 75-78 | Scaled value of radius of spherical Earth |
| 79 | Scale factor of major axis of oblate spheroid Earth |
| 80-83 | Scaled value of major axis of oblate spheroid Earth |
| 84 | Scale factor of minor axis of oblate spheroid Earth |
| 85-88 | Scaled value of minor axis of oblate spheroid Earth |
| 89-92 | La1 – latitude of first grid point |
| 93-96 | Lo1 – longitude of first grid point |
| 97 | Resolution and component flags (see flag table 3.3) |
| 98-101 | LaD – latitude where Dx and Dy are specified |
| 102-105 | LoV – orientation of the grid |
| 106 | Projection centre flag (see flag table 3.5) |

Grid definition template 3.63 - Spectral Lambert conformal LAM with explicit boundary for bi-periodic field

| Octet No. | Contents |
| --- | --- |
| 15 | Spectral Representation type (see code table 3.6) |
| 16-19 | N – bi-Fourier Resolution Parameter |
| 20-23 | M – Bi-Fourier Resolution Parameter |
| 24 | Bi-Fourier Truncation Type (see code table 3.25) |
| 25-32 | Lx – Size in meters of the domain along X axis |
| 33-40 | Lux – Size in meters of model forecast area C along X axis |
| 41-48 | Lcx – Width in meters of coupling area I along X axis |
| 49-56 | Ly – Size in meters of the domain along Y axis |
| 57-64 | Luy – Size in meters of model forecast area C along Y axis |
| 65-72 | Lcy – Width in meters of coupling area I along Y axis |
| 73 | Shape of the Earth (see code table 3.2) |
| 74 | Scale factor of radius of spherical Earth |
| 75-78 | Scaled value of radius of spherical Earth |
| 79 | Scale factor of major axis of oblate spheroid Earth |
| 80-83 | Scaled value of major axis of oblate spheroid Earth |
| 84 | Scale factor of minor axis of oblate spheroid Earth |
| 85-88 | Scaled value of minor axis of oblate spheroid Earth |
| 89-92 | La1 – latitude of first grid point |
| 93-96 | Lo1 – longitude of first grid point |
| 97-100 | LaD – latitude where Dx and Dy are specified |
| 101-104 | LoV – longitude of meridian parallel to y-axis along which latitude increases as the y-coordinate increases |
| 105 | Projection centre flag (see flag table 3.5) |
| 106-109 | Latin 1 – first latitude from the pole at which the secant cone cuts the sphere |
| 110-113 | Latin 2 – second latitude from the pole at which the secant cone cuts the sphere |
| 114-117 | Latitude of the southern pole of projection |
| 118-121 | Longitude of the southern pole of projection |

Data representation template 5.53 - Spectral LAM data - complex packing

| Octet No. | Contents |
| --- | --- |
| 12-15 | Reference value (R) (IEEE 32-bit floating-point value) |
| 16-17 | Binary scale factor (E) |
| 18-19 | Decimal scale factor (D) |
| 20 | Number of bits used for each packed value (field width) |
| 21 | Bi-Fourier Sub-Truncation Type (see Code table 5.25) |
| 22 | Packing mode for Axes (see Code table 5.26) |
| 23-26 | P - Laplacian scaling factor (expressed in 10-6 units) |
| 27-28 | NS - Bi-Fourier resolution parameter of the unpacked subset (see Note 1) |
| 29-30 | MS - Bi-Fourier resolution parameter of the unpacked subset (see Note 1) |
| 31-34 | TS – total number of values in the unpacked subset (see Note 1) |
| 35 | Precision of the unpacked subset (see Code table 5.7) |

Notes:

(1)	The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to NS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7.

(2)	The remaining coefficients are multiplied by (n2 +m2)P, scaled and packed. The operator associated with this multiplication is derived from the Laplacian operator.

(3)	The retrieval formula for a coefficient of wave number n is then: Y = (R + X x 2E ) x10–D x (m2 +n2)-P where X is the packed scaled value associated with the coefficient.

Data template 7.53 - Spectral LAM data  - complex packing

| Octet No. | Contents |
| --- | --- |
| 6-(5+IxTS) | Data values from the unpacked subset (IEEE floating-point values on I octets) |
| (6+IxTS)–nn | Binary data values – binary string, with each (scaled) data value out of the unpacked subset |

Add entries:

in code table 3.1 – Grid definition template number,

| Code figure | Meaning |
| --- | --- |
| 13 | Mercator LAM with explicit boundary for bi-periodic field |
| 23 | Polar stereographic LAM with explicit boundary for bi-periodic field |
| 33 | Lambert conformal LAM with explicit boundary for bi-periodic field |
| 61 | Spectral Mercator LAM with explicit boundary for bi-periodic field |
| 62 | Spectral polar stereographic LAM with explicit boundary for bi-periodic field |
| 63 | Spectral Lambert conformal LAM with explicit boundary for bi-periodic field |

in code table 3.6 – Spectral data representation type,

| Code figure | Meaning |
| --- | --- |
| 2 | Bi-Fourier representation |

in code table 3.25 - Type of bi-Fourier truncation,

| Code figure | Meaning |
| --- | --- |
| 77 | Rectangular |
| 88 | Elliptic |
| 99 | Diamond |

in code table 5.0 - Data representation template number,

| Code figure | Meaning |
| --- | --- |
| 53 | Spectral LAM data - complex packing |

in code table 5.25 - Type of bi-Fourier subtruncation,

| Code figure | Meaning |
| --- | --- |
| 77 | Rectangular |
| 88 | Elliptic |
| 99 | Diamond |

in code table 5.26 – Packing mode for axes

| Code figure | Meaning |
| --- | --- |
| 0 | Spectral coefficients for axes are packed |
| 1 | Spectral coefficients for axes included in the unpacked subset |

- 2016-2.2.9(DRMM-IV)/New table entries for GRIB2 to represent GRAPES model products [FT2016-2] <para 6.1>

Add entries:

in code table 4.2,

Product discipline 0 – Meteorological products, parameter category 0: temperature

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 29 | Temperature advection | K s-1 | Temperature advection is the advection of temperature by the wind. It refers to the change of temperature caused by movement of air by the wind. Warm advection (positive value) indicates the temperature is increasing, and cold advection (negative value) indicates the temperature is decreasing. |

Product discipline 0 – Meteorological products, parameter category 2: momentum

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 46 | Vorticity advection | s-2 | Vorticity advection is the advection of relative vorticity by the wind. It refers to the change in vorticity caused by the movement of air. A positive value corresponds to rising forcing, while negative value corresponds to sinking forcing. |

in code table 4.5,

| Code figure | Meaning | Unit |
| --- | --- | --- |
| 115 | Sigma height level (see Note 5) | – |

Notes:

(5)	Sigma height level is the vertical model level of the height-based terrain-following coordinate (Gal-Chen and Somerville, 1975). The value of the level = (Height of the level – height of the terrain) / (height of the top level – height of the terrain), which is >=0 and <=1.

- 2015-2.1.1(DRMM-III) Complete encoding of information required for georeferencing grid points in GRIB [ABC2016] <para 6.1>

Add a new note:

(2)	With respect to the Code figures 0, 1, 3, 6, and 7, coordinates can only be unambiguously interpreted, if the coordinate reference system, in which they are embedded, is known. Therefore, defining the shape of the Earth alone without coordinate system axis origins is ambiguous. Generally, the prime meridian defined in the geodetic system WGS84 can be safely assumed to be the longitudinal origin. However, because these code figures do not specify the longitudinal origin explicitly, it is suggested to contact the originating centre, if high precision coordinates are needed in order to obtain the precise details of the coordinate system used.

- 2015-2.1.2(DRMM-III) New GRIB2 Regulations and notes to make it clear that forecast times may be negative [ABC2017] <para 6.1>

Add a regulation:

92.6.3	In product definition templates that refer to a forecast time or offset from reference time, this may be negative to refer to times or intervals that begin before the reference time, if this is applicable.

- 2015-2.2.1/2.2.2(DRMM-III) New parameters in GRIB2 Code table 4.2/New GRIB2 parameters and product definition template for observational satellite data [Validation] <para 6.1>

Add entries:

In Code table 4.1,

Discipline 3 (Space products)

191	Miscellaneous

In Code Table 4.2:

Discipline 3 (Space products), category 1 (Quantitative products)

18	Water temperature	K

Discipline 3 (Space products), Category 191 (Miscellaneous)

0	Seconds prior to initial reference time (defined in Section 1)	s

Add a code table:

Code table 4.16 – Quality value associated with parameter

Code figure	Meaning

0	Confidence index (see Note 2)

1	Quality indicator (see Note 3 and Code table 4.244)

2	Correlation of product with used calibration product (see Note 4)

3	Standard deviation of product from calibration product (see Note 5)

4-191	Reserved

192-254	Reserved for local use

255	Missing

Notes:

(1)	When a non-missing value is used from this code table, the original data value is a quality value associated with the parameter defined by octets 10 and 11 of the product definition template.

(2)	The original data value is a non-dimensional number from 0 to 1, where 0 indicates no confidence and 1 indicates maximal confidence.

(3)	The original data value is defined by Code table 4.244.

(4)	The original data value is a non-dimensional number without units.

(5)	The original data value is in the same units as the parameter defined by octets 10 and 11 of the product definition template.

Code table 4.244 – Quality indicator

Code figure	Meaning

0	No quality information available

1	Failed

2	Passed

3-191	Reserved

192-254	Reserved for local use

255	Missing

Add a template:

Product definition template 4.35 – satellite product with or without associated quality values

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Observation generating process identifier (defined by originating centres)

14	Quality value associated with parameter (see Code Table 4.16)

15	Number of contributing spectral bands (NB)

16–   Repeat the following 11 octets for each contributing band (nb = 1, NB)

(16+11(nb–1))–(17+11(nb–1))	Satellite series of band nb (code table defined by originating/generating centre)

(18+11(nb–1))–(19+11(nb–1))	Satellite numbers of band nb (code table defined by originating/generating centre)

(20+11(nb–1))–(21+11(nb–1))	Instrument types of band nb (code table defined by originating/generating centre)

(22+11(nb–1))	Scale factor of central wave number of band nb

(23+11(nb–1))–(26+11(nb–1))	Scaled value of central wave number of band nb (units: m–1)

Note:	For “satellite series of band nb”, “satellite numbers of band nb” and “instrument types of band nb”, it is recommended to encode the values as per BUFR Code tables 0 02 020, 0 01 007 (Common Code table C–5) and 0 02 019 (Common Code table C–8), respectively.

- 2014-2.2.2(DRMM-II)/A product definition template for statistics over an ensemble

[Validation] <para 6.1>

Add a new template:

Product definition template 4.62 – Statistics over an ensemble reforecast, at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Forecast generating process identifier (defined by originating centre)

15	Indicator of unit of time range (see Code table 4.4)

16-19	Forecast time in units defined by octet 15 (see Note 1)

20	Type of first fixed surface (see Code table 4.5)

21	Scale factor of first fixed surface

22-25	Scaled value of first fixed surface

26	Type of second fixed surface (see Code table 4.5)

27	Scale factor of second fixed surface

28-31	Scaled value of second fixed surface

32	Type of ensemble forecast (see Code table 4.6)

33	Number of forecasts in ensemble

34	Number of years in the ensemble reforecast period (see Note 2)

35	First year of ensemble reforecast period

36	Last year of ensemble reforecast period

37	Total number of data values possible (or expected) in statistical process over the
 		ensemble reforecast

38-39	Total number of data values missing in statistical process over the ensemble
		reforecast

40	Statistical process used to calculate the processed field over the ensemble reforecast
		(see Code table 4.10)

41-42	Year of model version date (see Note 3)

43	Month of model version date

44	Day of model version date

45	Hour of model version date

46	Minute of model version date

47	Second of model version date

48	Month of end of overall time interval (see Note 5)

49	Day of end of overall time interval

50	Hour of end of overall time interval

51	Minute of end of overall time interval

52	Second of end of overall time interval

53	n - number of time range specifications describing the time intervals used

to calculate the statistically processed field

54-57	Total number of data values missing in statistical process

58-69	Specification of the outermost (or only) time range over which statistical

processing is done

58	Statistical process used to calculate the processed field from the field at

each time increment during the time range (see Code table 4.10)

59	Type of time increment between successive fields used in the statistical

processing (see Code table 4.11)

60	Indicator of unit of time for time range over which statistical processing is

done (see Code table 4.4)

61-64	Length of the time range over which statistical processing is done, in units

defined by the previous octet

65	Indicator of unit of time for the increment between the successive fields

used (see Code table 4.4)

66-69	Time increment between successive fields, in units defined by the previous

octet (see Note 3)

70-nn	These octets are included only if n>1, where nn=69 + 12 x n

70-81	As octets 58 to 69, next innermost step of processing

82-nn	Additional time range specifications, included in accordance with the value

of n. Contents as octets 58 to 69, repeated as necessary

Notes:

(1)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(2)	Octets 34-40 define a statistical process over both time and ensemble.

(3)	This is the date to identify the model version that is used to generate the reforecast.

(4)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge. The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment (one of octets 59, 71. 83 ...).  For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast time.

- 2012-2.2.9(DRC-IV)/GRIB template for 4-D Trajectory grid definition [Validation] <para 6.1>

Validate templates:

Grid definition template 3.1010 – 4-D trajectory grid definition

Octet No.	Contents

15	Shape of the Earth (see Code table 3.2)

16	Scale factor of radius of spherical Earth

17–20	Scaled value of radius of spherical Earth

21	Scale factor of major axis of oblate spheroid Earth

22–25	Scaled value of major axis of oblate spheroid Earth

26	Scale factor of minor axis of oblate spheroid Earth

27–30	Scaled value of minor axis of oblate spheroid Earth

31–32	Number of horizontal points in slice (see Note 1)

33–34	Number of vertical points in slice (see Note 1)

35–38	Di – slice horizontal grid length (see Note 2)

39–42	Dj – slice vertical grid length (see Note 2)

43–46	Pi – horizontal location of trajectory point within slice (see Note 3)

47–50	Pj – vertical location of trajectory point within slice (see Note 3)

51	Scanning mode (flags – see Flag table 3.4) (see Note 1)

52–53	NW – Number of way points (see Note 4)

54–(53+NW×24)	Waypoint descriptions to define 4-D coordinates

Waypoint descriptions:

30+(N×24+0–N×24+3)	LaN – latitude of Nth trajectory way point

30+(N×24+4–N×24+7)	LoN – longitude of Nth trajectory way point

30+(N×24+8)	Type of Nth’s trajectory way point surface (see Code table 4.5) (see
		Note 5)

30+(N×24+9)	Scale factor of Nth’s trajectory way point surface

30+(N×24+10–N×24+13)	Scaled value of Nth’s trajectory way point surface

30+(N×24+14)	Indicator of unit of time range (see Code table 4.4)

30+(N×24+15–N×24+18)	Waypoint time in units defined by octet N×24+14 (see Note 6)

30+(N×24+19–N×24+22)	Number of slices per trajectory segment (see Note 7)

30+(N×24+23)	Number of additional leading and trailing slices (see Note 8)

Notes:

(1)		Horizontal and vertical points in slice indicate orthogonal grid slice perpendicular to point along the trajectory segment. Therefore scanning mode describes scanning within single slice.

(2)	Grid lengths are in units of 10–3 m. Trajectory is always positioned in the centre of the slice.

(3)	Location of trajectory point within slice is in units of 10–3 m relative from first grid point of this slice.

(4)	Type of line for way segments is assumed to be Great Circle.

(5)	Each of waypoints can be in different types of surface coordinates. For the purpose of light transition level, point of transition can be repeated in both meters above surface and isobaric level equivalent to height reduction based on QNH valid for FIR at that moment. Waypoint can be also repeated for stopover on the trajectory with same 3-D coordinates but different waypoint time. First point of transition level or stopover description can have MISSING slices in this case.

(6)	Waypoint time is relative to reference time of data defined in Section 1.

(7)	Slices are defined as perpendicular planes which are equidistantly spaced along the trajectory segment. First slice is always located in first point of trajectory segment and last slice in last point of trajectory segment. Therefore minimum number of slices is 2, unless set to MISSING (for last point of trajectory or for transition level repeated point).

Figure 3: Example of trajectory segment with 5 slices (circles represent trajectory waypoints)

(8)	Number of leading slices is same as number of trailing slices, and represents additional slices outside the trajectory segment but within its direction using same equidistant spacing as for corresponding trajectory segment itself.

Figure 4: Example of trajectory segment with 5 slices and 1 leading and trailing slice (circles represent trajectory waypoints)

(9)	A scaled value of radius of spherical Earth, or major or minor axis of oblate spheroid Earth, is derived by applying the appropriate scale factor to the value expressed in metres.

Product definition template 4.1010 – 4-D trajectory

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Analysis or forecast generating process identifier (defined by originating centre)

15–16	Hours of observational data cutoff after reference time (see Note)

17	Minutes of observational data cutoff after reference time

Note: Hours greater than 65534 will be coded as 65534.

Product definition template 4.1011 – 4-D trajectory ensemble forecast, control and perturbed

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating Centre)

14	Forecast generating process identifier (defined by originating Centre)

15–16	Hours after reference time of data cutoff (see Note)

17	Minutes after reference time of data cutoff

18	Type of ensemble forecast (see Code table 4.6)

19	Perturbation number

20	Number of forecasts in ensemble

Note: Hours greater than 65534 will be coded as 65534.

Product definition template 4.1015 – 4-D trajectory probability forecasts

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Forecast generating process identifier (defined by originating centre)

15–16	Hours after reference time of data cutoff (see Note)

17	Minutes after reference time of data cutoff

18	Forecast probability number

19	Total number of forecast probabilities

20	Probability type (see Code table 4.9)

21	Scale factor of lower limit

22–25	Scaled value of lower limit

26	Scale factor of upper limit

27–30	Scaled value of upper limit

Note: Hours greater than 65534 will be coded as 65534.

FM 94 BUFR/FM 95 CREX

- 2016-3.2.1(DRMM-IV)/New entry in common code table C-2 by Switzerland and Kazakhstan [FT2016-2] <para 6.1>

Add entries:

in common code table C-2

| Code figure for rara | Code figure for BUFR |  |
| --- | --- | --- |
| 50 | 150 | Meteolabor SRS-C50/Argus (Switzerland) |
| 73 | 173 | МАRL-A (Russian Federation) – ASPAN-15 (Kazakhstan) |
| N/A | 190 | NCAR research dropsonde NRD94 with GPS and Vaisala RS92-based sensor module (United States) |
| N/A | 191 | NCAR research dropsonde NRD41 with GPS and Vaisala RS41-based sensor module (United States) |
| N/A | 192 | Vaisala/NCAR dropsonde RD94 with GPS and Vaisala RS92-based sensor module (Finland/USA) |
| N/A | 193 | Vaisala/NCAR dropsonde RD41 with GPS and Vaisala RS41-based sensor module (Finland/USA) |

- 2016-3.2.2(DRMM-IV)/New entries for dropsonde data [FT2016-2] <para 6.1>

Add an entry:

in BUFR Table B,

| FXY | Element Name | BUFR Unit | BUFR Scale | BUFR reference value | BUFR width (bits) | CREX Unit | CREX Scale | CREX width (characters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-02-087 | Parachute surface area | m2 | 4 | 0 | 15 | m2 | 4 | 5 |

- 2016-3.2.3(DRMM-IV)/New entries in Common Tables C-5 and C-8 [FT2016-2] <para 6.1>

- 2016-3.2.8(DRMM-IV)/New entries in Common Code Table C-5 [FT2016-2]

Add entries:

in common code table C-5,

| Code figure for I6I6I6 | Code figure for BUFR | Code figure for GRIB2 |  |
| --- | --- | --- | --- |
| 062 | 62 | 62 | Sentinel 1A |
| 063 | 63 | 63 | Sentinel 1B |
| 064 | 64 | 64 | Sentinel 5P |
| 270 | 270 | 270 | GOES 16 |
| 271 | 271 | 271 | GOES 17 |
| 272 | 272 | 272 | GOES 18 |
| 273 | 273 | 273 | GOES 19 |
| 523 | 523 | 523 | FY-3D |
| 812 | 812 | 812 | SCISAT-1 |
| 813 | 813 | 813 | ODIN |

in common code table C-8,

| Code | Agency | Type | Short name | Long name |
| --- | --- | --- | --- | --- |
| 92 | CSA | Limb-scanning sounder | ACE-FTS | Atmospheric Chemistry Experiment - Fourier Transform Spectrometer |
| 151 | ESA | Imaging radar | SAR-C | Synthetic Aperture Radar (C-band) |
| 152 | ESA | Cross-nadir scanning | SW | Sounder TROPOMI Tropospheric Monitoring Instrument |
| 959 | SNSB | Limb-scanning sounder | SMR | Sub-millimetre radiometer |

- 2016-3.2.4(DRMM-IV)/New BUFR elements for C-band synthetic aperture radar  instrument (C-SAR) [FT2016-2] <para 6.1>

Add entries:

in BUFR table B,

Class 03 – BUFR/CREX Instrumentation

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REF. VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char.) | Description |
| 0 03 025 | Cross-track estimation area size | m | 0 | 5000 | 16 | m | 0 | 5 | Ground range size of the estimation area [m]. |
| 0 03 026 | Along-track estimation area size | m | 0 | 5000 | 16 | m | 0 | 5 | Azimuth size of the estimation area [m]. |

Class 05 – BUFR/CREX Location (horizontal – 1)

| TABLE REF. | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REF. VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char.) | Description |
| 0 05 071 | Stripmap identifier | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 | In the case of SM mode the product contained in a single netCDF file will be split in several BUFR messages. We need to have an identifier to rebuild the original product and to mark the single parts to belong to the same product. This is the function of Stripmat identifier |
| 0 05 072 | Number of spectra in range direction | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 | Number of range swell wave spectra cells. For WV mode this dimension is set to 1 as there is 1 swell wave spectra per WV vignette. For SM this dimension is set to the number of cells in the range direction, nominally 4. This parameter does not apply to IW nor EW mode. |
| 0 05 073 | Number of spectra in azimuthal direction | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 | Number of azimuth swell wave spectra cells. For WV mode this dimension is set to 1 as there is 1 swell wave spectra per WV vignette. For SM this dimension is set to the number of cells in the azimuth direction, nominally 4; although, this will vary with the length of the input image strip. This parameter does not apply to IW nor EW mode. |
| 0 05 074 | Index in range direction | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 | Index of range swell wave spectra cell. |
| 0 05 075 | Index in azimuthal direction | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 | Index of azimuth swell wave spectra cell. |

Class 25 – BUFR/CREX Processing information

| TABLE REF. | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REF. VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char.) | Description |
| 0 25 189 | Range cut-off wavelength | m | 0 | 1 | 9 | m | 0 | 3 | The range cut-off wavelength [m] is the shortest wavelength in range direction that can be resolved in the swell wave spectra. The cut-off wavelength is computed from the slant range resolution (or range bandwidth, fsf) and the local incidence angle as: λrange(θ)=c/fsfsinθ |

Class 40 – BUFR/CREX Satellite data

| TABLE REF. | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REF. VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char.) | Description |
| ## 0 40 039 | ## Single Look Complex image intensity | ## Numeric | ## 0 | ## -25 | ## 5 | ## Numeric | ## 0 | ## 3 | The input Single Look Complex (SLC) image intensity estimated within each wave cell |
| ## 0 40 040 | ## Single Look Complex image skewness | ## Numeric | ## 2 | ## 1 | ## 13 | ## Numeric | ## 0 | ## 4 | The skewness of the input SLC image [dimensionless] estimated within each wave cell |
| ## 0 40 041 | ## Single Look Complex image kurtosis | ## Numeric | ## 2 | ## 1 | ## 13 | ## Numeric | ## 0 | ## 4 | The kurtosis of the input SLC image [dimensionless] estimated within each wave cell |
| ## 0 40 042 | ## Single Look Complex image variance | ## Numeric | ## 2 | ## 1 | ## 13 | ## Numeric | ## 0 | ## 4 | The variance of the input SLC image normalized by the square of the mean intensity [dimensionless] estimated within each wave cell. |

Class 42 – BUFR/CREX Oceanographic elements

| TABLE REF. | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REF. VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char.) | Description |
| 0 42 001 | Dominant swell wave direction of spectral partition | degree | 0 | 0 | 9 | degree | 0 | 3 | Dominant wave direction [degree] for each partition of the swell wave spectra. |
| 0 42 002 | Significant swell wave height of spectral partition | m | 1 | 0 | 9 | m | 1 | 3 | Significant wave height [m] for each partition of the swell wave spectra |
| 0 42 003 | Dominant swell wavelength of spectral partition | m | 2 | 100 | 17 | m | 2 | 6 | Dominant wave length [m] for each partition of the swell wave spectra |
| 0 42 004 | Confidence of inversion for each partition of swell wave spectra | Code table | 0 | 0 | 4 | Code table | 0 | 2 | Confidence of inversion for each partition of swell wave spectra |
| 0 42 005 | Ambiguity removal factor for swell wave partition | Numeric | 5 | -100000 | 18 | Numeric | 5 | 6 | The ambiguity removal factor [dimensionless] for each wave partition |
| 0 42 006 | Wave age | Numeric | 2 | 1 | 8 | Numeric | 2 | 3 | The dimensionless parameter derived from the SAR data that describes the state of development of the wind sea component of the wave spectra. The dimensions of the matrix are oswAzSize x oswRaSize. |
| 0 42 007 | Shortest ocean wavelength on spectral resolution | m | 2 | 0 | 16 | m | 2 | 5 | The spectral resolution gives the shortest ocean wavelength [m] that can be detected. |
| 0 42 008 | Nonlinear inverse spectral width | m | 2 | 0 | 16 | m | 2 | 5 | Nonlinear inverse spectral width [m] describing non-linear spectral cut-off computed from the cross-spectra. |
| 0 42 009 | Partition number | Numeric | 0 | 1 | 4 | Numeric | 0 | 2 | Number of wave partitions |

Add a code table:

Code table 0 42 004 – Confidence of inversion for each partition of swell wave spectra

| Code figure |  |
| --- | --- |
| 0 | Wave direction resolved |
| 1 | 180-degree ambiguity not resolve |
| 2-14 | Reserved |
| 15 | Missing |

- 2016-3.2.6(DRMM-IV)/BUFR Sequence for synoptic reports from sea stations suitable for VOS observation data (editorial changes) [FT2016-2/pre-operational] <para 6.1>

Add entries:

in BUFR/CREX Table B,

Class 01 – BUFR/CREX Identification

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 01 | 114 | Encrypted ship or mobile land station identifier (base64 encoding) | CCITT IA5 | 0 | 0 | 352 | Character | 0 | 44 |

Class 03 – BUFR/CREX Instrumentation

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 03 | 001 | Surface station type | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 | 03 | 003 | Thermometer/hygrometer housing | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 | 03 | 004 | Type of screen/shelter/radiation shield | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 | 03 | 005 | Horizontal width of screen or shield (x) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 006 | Horizontal depth of screen or shield (y) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 007 | Vertical height of screen or shield (z) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 008 | Artificially ventilated screen or shield | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 009 | Amount of forced ventilation at time of reading | m s–1 | 1 | 0 | 9 | m s–1 | 1 | 3 |
| 0 | 03 | 020 | Material for thermometer/hygrometer housing | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 021 | Hygrometer heating | Code table | 0 | 0 | 2 | Code table | 0 | 1 |
| 0 | 03 | 022 | Instrument owner | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 023 | Configuration of louvers for thermometer/hygrometer screen | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 024 | Psychrometric coefficient | K–1 | 6 | 0 | 10 | K–1 | 6 | 3 |

Class 10 – BUFR/CREX Non coordinate location (vertical)

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 10 | 038 | Maximum height of deck cargo above summer load line | m | 0 | 0 | 6 | m | 0 | 2 |
| 0 | 10 | 039 | Departure of reference level (summer maximum load line) from actual sea level | m | 0 | –32 | 6 | m | 0 | 3 |

Class 11 – BUFR/CREX Wind and turbulence

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 11 | 007 | Relative wind direction (in degrees off bow) | ° | 0 | 0 | 9 | ° | 0 | 3 |
| 0 | 11 | 008 | Relative wind speed | m s–1 | 1 | 0 | 12 | m s–1 | 1 | 4 |

Class 15 – BUFR/CREX Physical/chemical constituents

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 15 | 028 | Mole fraction of atmospheric constituent/pollutant in dry air | ‰ | 5 | 0 | 16 | ‰ | 5 | 5 |

Class 25 – BUFR/CREX Processing information

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 25 | 185 | Encryption method | Code table | 0 | 0 | 8 | Code table | 0 | 3 |
| 0 | 25 | 186 | Encryption key version | CCITT IA5 | 0 | 0 | 96 | Character | 0 | 12 |
| 0 | 25 | 188 | Method for reducing pressure to sea level | Code table | 0 | 0 | 5 | Code table | 0 | 2 |

Class 41 – BUFR/CREX Oceanographic/bio-geochemical parameters

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 41 | 001 | pCO2 | Pa | 3 | 0 | 18 | Pa | 3 | 6 |
| 0 | 41 | 002 | Fluorescence | kg l–1 | 12 | 0 | 16 | kg l–1 | 12 | 5 |
| 0 | 41 | 003 | Dissolved nitrates | µmol kg–1 | 3 | 0 | 17 | µmol kg–1 | 3 | 5 |
| 0 | 41 | 005 | Turbidity | NTU | 2 | 0 | 12 | NTU | 2 | 4 |

in BUFR table D,

Category 01 – BUFR/CREX Location and identification sequences

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 01 | 018 |  |  |  | (Encrypted ship's call sign and encryption method) | See Notes 1 – 3 |
|  |  |  | 0 | 01 | 114 | Encrypted ship or mobile land station identifier |  |
|  |  |  | 0 | 25 | 185 | Encryption method |  |
|  |  |  | 0 | 25 | 186 | Encryption key version |  |

Notes:

(1)		The ship's call sign or WMO identifier should be reported using descriptor 0 01 011.

(2)		However, if required by shipping companies when VOS ships are recruited or if subsequently requested, for ship reports using template 3 08 014 the Ship call sign or other identifier can be encrypted in BUFR reports using sequence 3 01 018 according to the following method:

- The normal callsign (i.e. descriptor 0 01 011) shall be encoded with missing value;

- The encryption method shall be indicated using the method indicated by 0 25 185;

- The version of the encryption key that is used shall be indicated by 0 25 186;

(3)		The encryption keys will be managed by the JCOMM Focal Point on Ship Masking.

Category 02 – BUFR/CREX Meteorological sequences common to surface data

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship “instantaneous” data) |  |
| 3 | 02 | 062 | 0 | 25 | 188 | Method for reducing pressure report to sea level |  |
|  |  |  | 3 | 02 | 001 | Pressure and 3-hour pressure change |  |
|  |  |  | 3 | 02 | 093 | Extended ship temperature and humidity data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 053 | Ship visibility data |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 004 | General cloud information |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 001 | Delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 005 | Cloud layer |  |
|  |  |  | 0 | 08 | 002 | Vertical significance (surface observations) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 055 | Icing and ice |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 056 | Sea/water temperature |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 021 | Waves |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 024 | Wind and swell waves |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship “period” data) |  |
| 3 | 02 | 063 | 3 | 02 | 038 | Present and past weather |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 040 | Precipitation measurement |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 034 | Precipitation past 24 hours |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 058 | Ship extreme temperature data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 064 | Ship or other marine platform wind data |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (VOSClim data elements) |  |
| 3 | 02 | 092 | 0 | 11 | 104 | True heading of aircraft, ship or other mobile platform | Ship’s true heading |
|  |  |  | 0 | 01 | 012 | Direction of motion of moving observing platform | Ship’s course over ground |
|  |  |  | 0 | 01 | 013 | Speed of motion of moving observing platform | Ship’s speed over ground |
|  |  |  | 0 | 10 | 038 | Maximum height of deck cargo above summer load line |  |
|  |  |  | 0 | 10 | 039 | Departure of reference level (summer maximum load line) from actual sea level |  |
|  |  |  | 0 | 11 | 007 | Relative wind direction (in degrees off bow) |  |
|  |  |  | 0 | 11 | 008 | Relative wind speed |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | X | YYY |  |  |
|  |  |  |  |  |  | (Extended ship temperature and humidity data) |  |
| 3 | 02 | 093 | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature/humidity sensors |  |
|  |  |  | 0 | 12 | 101 | Temperature/air temperature |  |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor | Set to 0 if no change from previous values, 1 if changes |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature/humidity sensors |  |
|  |  |  | 0 | 02 | 039 | Method of wet-bulb temperature measurement |  |
|  |  |  | 0 | 02 | 097 | Type of humidity sensor |  |
|  |  |  | 0 | 03 | 024 | Psychrometric coefficient | Set to missing if type of humidity sensor is not psychrometer |
|  |  |  | 0 | 03 | 021 | Hygrometer heating |  |
|  |  |  | 0 | 12 | 102 | Wet-bulb temperature |  |
|  |  |  | 0 | 12 | 103 | Dewpoint temperature |  |
|  |  |  | 0 | 13 | 003 | Relative humidity |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Cancel (set to missing) |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface | Cancel (set to missing) |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature/humidity sensors | Cancel (set all elements in sequence to missing) |
|  |  |  | 0 | 02 | 039 | Method of wet-bulb temperature measurement | Cancel (set to missing) |
|  |  |  | 0 | 02 | 097 | Type of humidity sensor | Cancel (set to missing) |
|  |  |  | 0 | 03 | 021 | Hygrometer heating | Cancel (set to missing) |
|  |  |  | 0 | 03 | 024 | Psychrometric coefficient | Cancel (set to missing) |

Category 03 – BUFR/CREX Meteorological sequences common to vertical soundings data

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Metadata common to temperature/humidity sensors) |  |
| 3 | 03 | 099 | 0 | 03 | 005 | Horizontal width of screen or shield (x) |  |
|  |  |  | 0 | 03 | 006 | Horizontal depth of screen or shield (y) |  |
|  |  |  | 0 | 03 | 007 | Vertical height of screen or shield (z) |  |
|  |  |  | 0 | 02 | 096 | Type of thermometer |  |
|  |  |  | 0 | 03 | 022 | Instrument owner |  |
|  |  |  | 0 | 03 | 003 | Thermometer/hygrometer housing |  |
|  |  |  | 0 | 03 | 020 | Material for thermometer/hygrometer housing |  |
|  |  |  | 0 | 03 | 004 | Type of screen/shelter/radiation shield |  |
|  |  |  | 0 | 03 | 023 | Configuration of louvers for thermometer/hygrometer screen |  |
|  |  |  | 0 | 03 | 008 | Artificially ventilated screen or shield |  |
|  |  |  | 0 | 03 | 009 | Amount of forced ventilation at time of reading |  |

Category 06 – BUFR/CREX Meteorological or oceanographic sequences common to oceanographic observations

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 06 | 043 |  |  |  | (Marine bio-geochemical and radiation observations) |  |
|  |  |  | 0 | 41 | 001 | pCO2 |  |
|  |  |  | 0 | 08 | 043 | Atmospheric chemical or physical constituent type | set to 3 (carbon dioxide) |
|  |  |  | 0 | 15 | 028 | Mole fraction of atmospheric constituent / pollutant in dry air |  |
|  |  |  | 0 | 08 | 043 | Atmospheric chemical or physical constituent type | Cancel |
|  |  |  | 0 | 13 | 080 | pH |  |
|  |  |  | 0 | 41 | 005 | Turbidity |  |
|  |  |  | 0 | 41 | 003 | Dissolved nitrates |  |
|  |  |  | 0 | 22 | 188 | Dissolved oxygen |  |
|  |  |  | 0 | 41 | 002 | Fluorescence |  |
|  |  |  | 1 | 06 | 000 | Delayed replication of 6 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed replication factor |  |
|  |  |  | 0 | 04 | 024 | Time period or displacement (hours) | Set to -1 (preceding hour) |
|  |  |  | 0 | 14 | 002 | Long-wave radiation, integrated over period specified | Downwelling longwave radiation |
|  |  |  | 0 | 14 | 002 | Long-wave radiation, integrated over period specified | Upwelling longwave radiation |
|  |  |  | 0 | 14 | 012 | Net long-wave radiation, integrated over period specified |  |
|  |  |  | 0 | 14 | 004 | Short-wave radiation, integrated over period specified |  |
|  |  |  | 0 | 04 | 024 | Time period or displacement (hours) | Cancel |

Category 08 – BUFR/CREX Surface report sequences (sea)

| Table reference |  |  | Table References |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Synoptic reports from sea stations suitable for VOS observation data) |  |
| 3 | 08 | 014 | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 01 | 018 | Encrypted ship’s call sign and encryption method |  |
|  |  |  | 0 | 03 | 001 | Surface station type |  |
|  |  |  | 3 | 01 | 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |
|  |  |  | 2 | 08 | 032 | Change width of CCITT IA5 to 32 characters |  |
|  |  |  | 0 | 01 | 079 | Unique identifier for profile | Unique ID for report |
|  |  |  | 2 | 08 | 000 | Change width of CCITT IA5 | Cancel |
|  |  |  | 3 | 02 | 062 | Ship “instantaneous” data |  |
|  |  |  | 3 | 02 | 063 | Ship “period” data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 092 | VOSClim data elements |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 033 | Surface salinity |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 034 | Surface current |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 043 | E-SURFMAR S-AWS Observations |  |

Amend an element name:

in BUFR/CREX Table B,

Class 11 – BUFR/CREX Wind and turbulence

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 11 | 104 | True heading of aircraft, ship or other mobile platform | degree true | 0 | 0 | 9 | degree true | 0 | 3 |

Add and amend entries:

in code tables,

0 02 096

Type of temperature sensor

| Code figure | Meaning |
| --- | --- |
| 0 | Rod thermistor |
| 1 | Bead thermistor |
| 2 | Capacitance bead |
| 3 | Capacitance wire |
| 4 | Resistive sensor |
| 5 | Chip thermistor |
| 6 | Mercury |
| 7 | Alcohol/glycol |
| 8–30 | Reserved (for future use) |
| 31 | Missing value |

0 02 097

Type of humidity sensor

| Code figure | Meaning |
| --- | --- |
| 0 | VIZ Mark II carbon hygristor |
| 1 | VIZ B2 hygristor |
| 2 | Vaisala A-Humicap |
| 3 | Vaisala H-Humicap |
| 4 | Capacitance sensor |
| 5 | Vaisala RS90 |
| 6 | Sippican Mark IIA carbon Hygristor |
| 7 | Twin alternatively heated Humicap capacitance sensor |
| 8 | Humicap capacitance sensor with active de-icing method |
| 9 | Carbon hygristor |
| 10 | Psychrometer |
| 11 | Capacitive (polymer) |
| 12 | Capacitive (ceramic, incl. metal oxide) |
| 13 | Resistive (generic) |
| 14 | Resistive (salt polymer) |
| 15 | Resistive (conductive polymer) |
| 16 | Thermal conductivity |
| 17 | Gravimetric |
| 18 | Paper-metal coil |
| 19 | Ordinary human hair |
| 20 | Rolled hair (torsion) |
| 21 | Goldbeater's skin |
| 22 | Chilled mirror hygrometer |
| 23 | Dew cell |
| 24 | Optical absorption sensor |
| 25–30 | Reserved for future use |
| 31 | Missing value |

Add BUFR/CREX Code tables:

0 03 001

Surface station type

| Code figure | Meaning |
| --- | --- |
| 0 | Land station (synoptic network) |
| 1 | Shallow water station (fixed to sea/lake floor) |
| 2 | Ship |
| 3 | Rig/platform |
| 4 | Moored buoy |
| 5 | Drifting buoy (or drifter) |
| 6 | Ice buoy |
| 7 | Land station (local network) |
| 8 | Land vehicle |
| 9 | Autonomous marine vehicle |
| 10–30 | Reserved (for future use) |
| 31 | Missing value |

0 03 003

Thermometer/hygrometer housing

| Code figure | Meaning |
| --- | --- |
| 0 | Screen |
| 1 | Sling / Whirling |
| 2 | Unscreened |
| 3 | Radiation shield |
| 4 | Aspirated (e.g. Assmann) |
| 5 | Other Shelter |
| 6 | Handheld |
| 7–14 | Reserved for future use |
| 15 | Missing value |

0 03 004

Type of screen/shelter/radiation shield

| Code figure | Meaning |
| --- | --- |
| 0 | Stevenson screen |
| 1 | Marine Stevenson screen |
| 2 | Cylindrical section plate shield |
| 3 | Concentric tube |
| 4 | Rectangular section shield |
| 5 | Square section shield |
| 6 | Triangular section shield |
| 7 | Open covered lean to |
| 8 | Open covered inverted V roof |
| 9 | Integrated (e.g. Chilled Mirror) |
| 10–14 | Reserved for future use |
| 15 | Missing value |

0 03 008

Artificially ventilated screen or shield

| Code figure | Meaning |
| --- | --- |
| 0 | Natural ventilation in use |
| 1 | Artificial aspiration in use: constant flow at time of reading |
| 2 | Artificial aspiration in use: variable flow at time of reading |
| 3–6 | Reserved |
| 7 | Missing value |

0 03 020

Material for thermometer/hygrometer housing

| Code figure | Meaning |
| --- | --- |
| 0 | Wood |
| 1 | Metal alloy |
| 2 | Plastic/GRP |
| 3 | Reed/grass/leaf |
| 4–6 | Reserved for future use |
| 7 | Missing value |

0 03 021

Hygrometer heating

| Code figure | Meaning |
| --- | --- |
| 0 | Unheated |
| 1 | Heated |
| 2 | Not applicable |
| 3 | Missing value |

0 03 022

Instrument owner

| Code figure | Meaning |
| --- | --- |
| 0 | National hydro meteorological/weather service |
| 1 | Other |
| 2 | Standards institute |
| 3–6 | Reserved for future use |
| 7 | Missing value |

0 03 023

Configuration of louvers for thermometer/hygrometer screen

| Code figure | Meaning |
| --- | --- |
| 0 | Single v section louvers |
| 1 | Overlapping louvers |
| 2 | Double v section louvers |
| 3 | Non-overlapping louvers |
| 4 | Vented, non-louvered |
| 5 | Not applicable |
| 6 | Reserved for future use |
| 7 | Missing value |

0 25 185

Encryption method

| Code Figure | Meaning |
| --- | --- |
| 0 | AES 256 |
| 1–254 | Reserved |
| 255 | Missing value |

0 25 188

Method for reducing pressure report to sea level

| Code Figure | Meaning |
| --- | --- |
| 0 | Pressure adjusted to mean sea level following WMO 8 for low level (< 50m) stations |
| 1 | Pressure adjusted to mean sea level following WMO 8 for stations below 750 m |
| 2 | Pressure adjusted to sea level following national practice |
| 3 | Pressure adjusted to local water level following national practice |
| 4 | Pressure not corrected for height |
| 5–14 | Reserved |
| 15 | Missing value |

Add an entry:

in common code table C-6,

| Code figure |  | Conventional abbreviation | Abbreviation in IA5/ASCII | Abbreviation in IA2 | Definition in base units |
| --- | --- | --- | --- | --- | --- |
| 843 | Nephelometric turbidity units | NTU | NTU |  |  |

- 2016-3.2.7(DRMM-IV)/Additional bio-geochemical sequences for data from Argo profiling floats [Validation, FT2017-2] <para 6.1>

Add entries:

in BUFR/CREX Table B,

Class 13 – Hydrographic and hydrological elements

| Table reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (characters) |
| 0 | 13 | 161 | pH scale | Code table | 0 | 0 | 3 | Code table | 0 | 1 |

Class 41 – Marine bio-geochemical data

| Table reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (characters) |
| 0 | 41 | 006 | Backscattering | m-1 | 5 | 0 | 19 | m-1 | 5 | 6 |

in BUFR Table D,

Sequence 3-06-044 for chlorophyll-A profile data

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
| 3 | 06 | 044 |  |  |  | (Chlorophyll-A (fluorescence) profile data) |  |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Gives number of depths |
|  |  |  | 0 | 07 | 062 | Depth below sea / water surface | Code as missing |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class | Code as missing |
|  |  |  | 0 | 33 | 050 | GTSPP quality class | Code as missing |
|  |  |  | 0 | 07 | 065 | Water pressure |  |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 10, indicates pressure at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |
|  |  |  | 0 | 41 | 002 | Chlorophyll-A (fluorescence) | In kg l-1 (= 109 mg m-3) |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 21, chlorophyll-A at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |

Code depth related descriptors as missing as water pressure is used as the vertical axis

Chlorophyll-A specified in range 0 to 65.535 mg m-3 with a resolution of 0.001 mg m-3.

Sequence 3-06-045 for dissolved nitrate profile data

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
| 3 | 06 | 045 |  |  |  | (Dissolved nitrate profile data) |  |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Gives number of depths |
|  |  |  | 0 | 07 | 062 | Depth below sea / water surface | Code as missing |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class | Code as missing |
|  |  |  | 0 | 33 | 050 | GTSPP quality class | Code as missing |
|  |  |  | 0 | 07 | 065 | Water pressure |  |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 10, indicates pressure at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |
|  |  |  | 0 | 41 | 003 | Dissolved nitrate | In μmol kg-1 |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 22, nitrate at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |

Code depth related descriptors as missing as water pressure is used as the vertical axis.

Sequence 3-06-046 for pH profile data

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
| 3 | 06 | 046 |  |  |  | (pH profile data) |  |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Gives number of depths |
|  |  |  | 0 | 07 | 062 | Depth below sea / water surface | Code as missing |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class | Code as missing |
|  |  |  | 0 | 33 | 050 | GTSPP quality class | Code as missing |
|  |  |  | 0 | 07 | 065 | Water pressure |  |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 10, indicates pressure at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |
|  |  |  | 0 | 13 | 161 | pH scale |  |
|  |  |  | 0 | 13 | 080 | pH | dimensionless |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 23, pH at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |

Code depth related descriptors as missing as water pressure is used as the vertical axis.

Sequence 3-06-047 for backscattering profile data

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
| 3 | 06 | 047 |  |  |  | (Backscatter profile data) |  |
|  |  |  | 0 | 02 | 071 | (Spectrographic) wavelength | m |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Gives number of depths |
|  |  |  | 0 | 07 | 062 | Depth below sea / water surface | Code as missing |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class | Code as missing |
|  |  |  | 0 | 33 | 050 | GTSPP quality class | Code as missing |
|  |  |  | 0 | 07 | 065 | Water pressure |  |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 10, indicates pressure at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |
|  |  |  | 0 | 41 | 006 | Backscattering | m-1 |
|  |  |  | 0 | 08 | 080 | Qualifier for quality class (set to 24, backscatter at a level) |  |
|  |  |  | 0 | 33 | 050 | GTSPP quality class |  |

Code depth related descriptors as missing as water pressure is used as the vertical axis.

in BUFR/CREX code tables,

0 02 149

Type of data buoy

| Code figure | Meaning |
| --- | --- |
| 31 | Coastal sub-surface float |
| 32 | Deep sub-surface float |

0 08 080

Qualifier for GTSPP Quality Flag

| Code figure | Meaning |
| --- | --- |
| 21 | Chlorophyll-A at a level |
| 22 | Nitrate at a level |
| 23 | pH at a level |
| 24 | Backscattering at a level |

0 22 067 (FT2017-2)

Instrument type for water temperature/salinity profile measurement

| Code figure | Meaning |
| --- | --- |
| 835 | PROVOR IV |
| 836 | PROVOR III |
| 870 | HM2000 |
| 871 | COPEX |
| 872 | S2X |
| 869 | DOVA (no longer required) |
| 870 | NAMI (no longer required) |
| 871 | HM2000 |

Add a code table:

0 13 161

pH scale

| Code figure | Meaning |
| --- | --- |
| 0 | Seawater scale |
| 1 | Freescale |
| 2 | Total scale |
| 3-6 | Reserved |
| 7 | missing |

- 2016-3.2.9(DRMM-IV)/Revised and additional new descriptors for radar data products [FT2016-2] <para 6.1>

Add entries:

in BUFR/CREX Table B,

| TABLE REFERENCE |  |  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y | ELEMENT NAME | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char) |
| 0 | 01 | 152 | Semi-major axis of rotation ellipsoid | m | 2 | 0 | 31 | m | 2 | 11 |
| 0 | 01 | 153 | Semi-minor axis of rotation ellipsoid | m | 2 | 0 | 31 | m | 2 | 11 |
| 0 | 06 | 32 | X offset (see Note 6) | m | 2 | -1073741824 | 31 | m | 2 | 11 |
| 0 | 05 | 32 | Y offset (see Note 6) | m | 2 | -1073741824 | 31 | m | 2 | 11 |
| 0 | 29 | 14 | Optional list of parameters for an external map projection library | CCITT IA5 | 0 | 0 | 504 | Character | 0 | 63 |

Add Notes:

under Class 05 – BUFR/CREX Location (horizontal–1),

(6)	Y offset is the distance between the projection origin and the upper left corner of the upper left pixel in a map as explained in the following drawing:

Projection origin

Upper left pixel

under Class 06 – BUFR/CREX Location (horizontal–2),

(6)	X offset is the distance between the projection origin and the upper left corner of the upper left pixel in a map as explained in the following drawing:

Projection origin

Upper left pixel

Add entries:

in code table 0 08 091 – Coordinates significance,

| Code figure |  |
| --- | --- |
| 0 | Satellite coordinates |
| 1 | Observations coordinates |
| 2 | Start of observation |
| 3 | End of observation |
| 4 | Horizontal centre of gravity of the observation |
| 5 | Top of the observation |
| 6 | Bottom of the observation |
| 7 | Vertical centre of gravity of the observation |
| 8 | Projection origin |
| 9 | Coordinates of true scale |
| 10 - 254 | Reserved |
| 255 | Missing value |

Add entries:

in BUFR Table B,

Class 8 – BUFR/CREX Significance qualifiers

| TABLE REFERENCE |  |  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y | ELEMENT NAME | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char) |
| 0 | 08 | 088 | Map significance | Code table | 0 | 0 | 6 | Code table | 0 | 2 |
| 0 | 20 | 129 | Lightning density (Stroke, flash or event) | m-2 | 6 | 0 | 10 | m-2 | 6 | 4 |

Add a code table:

Code table 0 08 088 – Map significance

| Code figure |  |
| --- | --- |
| 0 | Top view (geographical longitude on X axis and latitude on Y axis) |
| 1 | North-South view (transect with geographical longitude on X axis and vertical height on Y axis) |
| 2 | East-West view (transect with geographical latitude on X axis and vertical height on Y axis) |
| 3-62 | Reserved |
| 63 | Missing |

http://www.eumetnet.eu/sites/default/files/bufr_sw_desc.pdf

- 2016-3.2.10(DRMM-IV)/Revised note for encoding geographic coordinates in BUFR and new element descriptors for encoding coordinate reference systems and fixed reference mean sea level [ABC2017, FT2016-2] <para 6.1>

Add a note [ABC2017]:

to Representation form of BUFR,

(10)	Position can only be unambiguously interpreted if the coordinate reference system and, if required, fixed reference mean sea level, to which it is attributed, is known. If these are not specified it is assumed that the position shall be interpreted with respect to WGS84 geodetic system and Earth Geodetic Model EGM96.

Add entries [FT2016-2]:

in Class 01 – BUFR/CREX Identification,

| TABLE REFERENCE |  |  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y | ELEMENT NAME | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char) |
| 0 | 01 | 150 | Coordinate reference system | Code table | 0 | 0 | 16 | Code table | 0 | 5 |
| 0 | 01 | 151 | Fixed mean sea level reference datum | Code table | 0 | 0 | 12 | Code table | 0 | 4 |

Add code tables [FT2016-2]:

Code table 0 01 150   Coordinate reference system

| Code figure |  |
| --- | --- |
| 0 | WGS84, as used by ICAO since 1998 |
| 1 | ETRS89, as defined by EPSG::4258 |
| 2 | NAD83, as defined by EPSG::4269 |
| 3 | DHDN, as defined by EPSG::4314 |
| 4 | Ellipsoidal datum using the International Reference Meridian maintained by the International Earth Rotation and Reference Systems Service (IERS) (see Note 2) |
| 5 - 65534 | Reserved |
| 65535 | Missing value |

Notes:

(1)		EPSG is a dataset of coordinate system and coordinate system transformations, originally produced and maintained by the European Petroleum Survey Group. Now it is maintained by the Geodesy Subcommittee of the International Association of Oil and Gas Producers Geomatics Committee.

(2)	If this coordinate reference system is specified, the semi-major and semi-minor axes must be specified (e.g. descriptors 0 01 152 and 0 01 153).

0 01 151   Fixed mean sea level reference datum

| Code figure |  |
| --- | --- |
| 0 | Earth Gravitational Model 1996 |
| 1 | Baltic height system 1977 |
| 2-4094 | Reserved |
| 4095 | Missing value |

- 2016-3.2.11(DRMM-IV)/Development of a LIDAR & Ceilometer Table D Sequence [FT2017-1] <para 6.1>

Add an entry:

in BUFR Table D,

| LIDAR Sequence |  |  |  |  |
| --- | --- | --- | --- | --- |
| TABLE REFERENCE | TABLE REFERENCE | TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME |
| F X Y | F X Y | F X Y | F X Y |  |
| Header Section |  |  |  |  |
| 3 09 023 |  |  |  |  |
|  | 3 01 001 |  |  | WMO block and station numbers |
|  |  | 0 01 001 |  | WMO Block Number |
|  |  | 0 01 002 |  | WMO Station Number |
|  | 0 05 001 |  |  | Latitude (High Accuracy) |
|  | 0 06 001 |  |  | Longitude (High Accuracy) |
|  | 0 07 030 |  |  | Height of station ground above mean sea level |
|  | 3 01 014 |  |  | Time Period |
|  |  | 1 02 002 |  | Replication of 2 descriptors (3 01 011 & 3 01 012) twice |
|  |  | 3 01 011 |  |  |
|  |  |  | 0 04 001 | Year |
|  |  |  | 0 04 002 | Month |
|  |  |  | 0 04 003 | Day |
|  |  | 3 01 012 |  |  |
|  |  |  | 0 04 004 | Hour |
|  |  |  | 0 04 005 | Minute |
|  | 0 02 003 |  |  | Type of measuring equipment used |
|  | Cloud data section |  |  |  |
|  | 3 02 004 |  |  | General Cloud Information |
|  |  | 0 20 010 |  | Cloud cover (total) |
|  |  | 0 08 002 |  | Vertical significance (surface observations) |
|  |  | 0 20 011 |  | Cloud amount |
|  |  | 0 20 013 |  | Height of base of cloud |
|  |  | 0 20 012 |  | Cloud Type |
|  |  | 0 20 012 |  | Cloud Type |
|  |  | 0 20 012 |  | Cloud Type |
|  | 3 02 005 |  |  | Cloud Layer |
|  |  | 0 08 002 |  | Vertical significance (surface observations) |
|  |  | 0 20 011 |  | Cloud amount |
|  |  | 0 20 012 |  | Cloud Type |
|  |  | 0 20 013 |  | Height of base of cloud |
|  | Backscatter data section |  |  |  |
|  | 1 14 000 |  |  | Delayed replication of 14 descriptors |
|  | 0 31 000 |  |  | Delayed replication Factor |
|  | 0 07 007 |  |  | Height |
|  | 3 01 021 | 0 05 001 |  | Latitude High Accuracy |
|  |  | 0 06 001 |  | Longitude High Accuracy |
|  | 1 01 000 |  |  | Delayed replication dependent on number of wavelength measurements present in the data.(note b) |
|  | 0 31 000 |  |  | Delayed replication Factor (see note b) |
|  | 0 02 121 |  |  | Mean Frequency |
|  | 0 15 063 |  |  | Attenuated Backscatter |
|  | 0 15 064 |  |  | Uncertainty in Attenuated Backscatter |
|  | 0 15 065 |  |  | Particle Backscatter Coefficient |
|  | 0 15 066 |  |  | Uncertainty in Particle Backscatter Coefficient |
|  | 0 15 067 |  |  | Particle Extinction Coefficient |
|  | 0 15 068 |  |  | Uncertainty in Particle Extinction Coefficient |
|  | 0 15 069 |  |  | Particle LIDAR Ratio |
|  | 0 15 070 |  |  | Uncertainty in LIDAR Ratio |
|  | 0 15 071 |  |  | Particle Depolarization Ratio |
|  | 0 15 072 |  |  | Uncertainty in Depolarization Ratio |
|  | 0 33 002 |  |  | Quality Information |
|  | 0 10 071 |  |  | Vertical Resolution |
|  | 0 27 079 |  |  | Horizontal Width of sampled volume |

(a)	We have a requirement for coding instruments operating at wavelengths between the following ranges;

100nm for UV Lidar’s  to 30m for VHF radars* (see RWP Template)

This corresponds to a range of between the following when measured in Hz

1e+7 – 1e+16

We intend to make use of the existing descriptor 0 02 121 and use modifiers to obtain the desired range.

*We must also consider the ceilometers/lidar instruments as we plan to use a common header sequence – details in a separate proposal.

(b)	The replication is included to allow for instruments with multiple wavelengths – for most instruments only one wavelength is reported, in which case this should be set to 1 (ie. Only the mean frequency should be included in this replication sequence).

Add entries:

in BUFR/CREX Table B,

|  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE REFERENCE |  |  |  |  | DATA |  |  | DATA |
|  | ELEMENT NAME | UNIT | SCALE | REFERENCE | WIDTH | UNIT | SCALE | WIDTH |
| F* X Y |  |  |  | VALUE | (Bits) |  |  | (Characters) |
| 0 15 063 | Attenuated Backscatter | m-1 Sr-1 | 8 | 0 | 20 | m-1 Sr-1 | 8 | 7 |
| 0 15 064 | Uncertainty in Attenuated Backscatter | m-1 Sr-1 | 8 | 0 | 20 | m-1 Sr-1 | 8 | 7 |
| 0 15 065 | Particle Backscatter Coefficient | m-1 Sr-1 | 8 | 0 | 20 | m-1 Sr-1 | 8 | 7 |
| 0 15 066 | Uncertainty in Particle Backscatter Coefficient | m-1 Sr-1 | 8 | 0 | 20 | m-1 Sr-1 | 8 | 7 |
| 0 15 067 | Particle Extinction Coefficient | m-1 | 8 | 0 | 20 | m-1 | 8 | 7 |
| 0 15 068 | Uncertainty in Particle Extinction Coefficient | m-1 | 8 | 0 | 20 | m-1 | 8 | 7 |
| 0 15 069 | Particle LIDAR Ratio | Sr | 2 | 0 | 14 | Sr | 2 | 5 |
| 0 15 070 | Uncertainty in LIDAR Ratio | Sr | 2 | 0 | 14 | Sr | 2 | 5 |
| 0 15 071 | Particle Depolarization Ratio | % | 2 | 0 | 14 | % | 2 | 5 |
| 0 15 072 | Uncertainty in Depolarization Ratio | % | 2 | 0 | 14 | % | 2 | 5 |
| 0 27 079 | Horizontal Width of sampled volume | m | 0 | 0 | 18 | m | 0 | 6 |
| 0 10 071 | Vertical resolution | m | 0 | 0 | 14 | m | 0 | 5 |

Add entries: (To be confirmed)

in code table  0 02 003,

| 10 | Lidar |
| --- | --- |

- 2016-3.2.12(DRMM-IV)/Radiosounding meta-data reporting: clarifications and amendments required [FT2016-2, FT2017-1] <para 6.1>

Add entries [FT2016-2]:

in code table 0 02 013 “Solar and infrared radiation correction”,

8	Solar and infrared corrected as specified by GRUAN

9	Solar corrected as specified by GRUAN

in code table 0 02 017 “Correction algorithms for humidity measurements”,

7	GRUAN solar radiation and time lag correction

in code table 0 02 081 “Type of balloon”,

7	Totex TA type balloons

8	Totex TX type balloons

in code table 0 02 083 “Type of balloon shelter”,

4	Automated unmanned sounding system

in code table 0 35 035 “Reason for termination”,

16	Increasing pressure

17	Invalid and/or missed data time limits exceeded

Add an element description to 0 35 035 “Reason for termination” [FT2016-2]:

in Table D for 3 01 128,

Reason for ascent termination

Add entries [FT2017-1]:

in BUFR/CREX Table B,

| FXY | Element Name | BUFR Unit | BUFR Scale | BUFR reference value | BUFR width (bits) | CREX Unit | CREX Scale | CREX width (characters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 02 088 | Volume of gas used in balloon | m3 | 3 | 0 | 13 | m3 | 3 | 4 |
| 0 08 037 | Baseline check significance | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 08 038 | Instrument data significance | Code table | 0 | 0 | 8 | Code table | 0 | 3 |
| 0 03 027 | Type of flight rig | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

Add Code tables [FT2017-1]:

##### 0 08 037

Baseline check significance

Code figure

0	Manufacturer’s baseline check unit

1	Weather screen

2	GRUAN Standard humidity chamber

3-30	Reserved

31	Missing value

##### 0 08 038

Instrument data significance

Code figure

0	Verified instrument reading

1	Reference instrument reading

2-254	Reserved

255	Missing value

##### 0 03 027

Type of flight rig

Code figures

0	Solo (single radiosonde)

1	Block

2	Bar

3	Cross

4	T-rig

5	Double T-rig

6	Complex

7-14	Reserved

15	Missing value

- 2016-3.2.13(DRMM-IV)/Reporting of measurement uncertainties in BUFR [FT2017-1] <para 6.1>

Add entries:

in BUFR/CREX Table B,

| FXY | Element Name | BUFR Unit | BUFR Scale | BUFR reference value | BUFR width (bits) | CREX Unit | CREX Scale | CREX width (characters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 08 092 | Measurement uncertainty expression | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 08 093 | Measurement uncertainty significance | Code table | 0 | 0 | 5 | Code table | 0 | 2 |

Add code tables:

0 08 092

Measurement uncertainty expression

Code figure

0	Standard uncertainty
	1-30	Reserved
	31	Missing value

0 08 093

Measurement uncertainty significance

Code figure

0	Total uncertainty
	1	Systematic component of uncertainty
	2	Random component of uncertainty
	3-30	Reserved
	31	Missing value

- 2015-5.1(DRMM-III) Clarifications of B/C30 and B/C32 Regulations [ABC2016] <para 6.1>

# Add a B/C 30.4.4:

B/C 30.4	Regional or national reporting practices

B/C 30.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT data in Manual on Codes, WMO-No. 306, Volume II.

B/C 30.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 30.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 30.4.3	Date/time (of beginning of the period for monthly precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for period different from the period recommended in Note (1) to B/C 30.2.6.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

B/C 30.4.4	Date/time (of beginning of the one-month period for precipitation data on the last day of the previous month)

If the regional or national reporting practices require reporting monthly precipitation data for period which starts on the last day of the previous month in UTC, template TM 307078 should be used. The beginning of the period for monthly precipitation data shall be specified by short time displacement (0 04 074) set to a relevant negative value. The beginning of one-month period for which the normals of precipitation are reported, shall be specified in a similar way.

# Add a B/C 32.4.4:

B/C 32.4	Regional or national reporting practices

B/C 32.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT SHIP data in Manual on Codes, WMO-No. 306, Volume II.

B/C 32.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 32.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 32.4.3	Date/time (of beginning of the one-month period for precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for one-month period different from the period recommended in Note (1) to B/C 32.2.3.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

B/C 32.4.4	Date/time (of beginning of the one-month period for precipitation data on the last day of the previous month)

If the regional or national reporting practices require reporting monthly precipitation data for period which starts on the last day of the previous month in UTC, template TM 308023 should be used. The beginning of the period for monthly precipitation data shall be specified by short time displacement (0 04 074) set to a relevant negative value. The beginning of one-month period for which the normals of precipitation are reported, shall be specified in a similar way.

- 2015-5.2(DRMM-III) Amendment to Annex II to B/C25 Regulations [ABC2016 except for 3 01 128 Additional information on radiosonde ascent (FT2015-2)] <para 6.1>

Editorial note: corrections to the Notes 4 and 5 were agreed as specified here.

Amend the notes to 3 09 052 in B/C25:

Notes:

(1)	Time of launch 3 01 013 shall be reported with the highest possible accuracy available.  If the launch time is not available with second accuracy, the entry for seconds shall be put to zero.

(2)	Long time displacement 0 04 086 represents the time offset from the launch time 3 01 013 (in seconds).

(3)	Latitude displacement 0 05 015 represents the latitude offset from the latitude of the launch site. Longitude displacement 0 06 015 represents the longitude offset from the longitude of the launch site.

(4)	If additional information on sounding is available, the sequence <3 09 052> shall be preceded by sequences suitable for reporting additional information on sounding systems.

(5)	If the sounding data are obtained from upper-air systems where pressure is derived from geopotential height by integration of hydrostatic equation, the geopotential calculation method shall be recorded using 0 02 191 within the preceding sequences.

Delete B/C25.11.

Amend Annex II to B/C25 as follows

Additional information on radiosonde ascent

| 3 01 128 |  | Additional information on radiosonde ascent |
| --- | --- | --- |
|  | 0-01-081 | Radiosonde serial number |
|  | 0-01-082 | Radiosonde ascension number |
|  | 0-01-083 | Radiosonde release number |
|  | 0-01-095 | Observer identification |
|  | 0-02-015 | Radiosonde completeness |
|  | 0-02-016 | Radiosonde configuration |
|  | 0-02-017 | Correction algorithms for humidity measurements |
|  | 0-02-066 | Radiosonde ground receiving system |
|  | 0-02-067 | Radiosonde operating frequency |
|  | 0-02-080 | Balloon manufacturer |
|  | 0-02-081 | Type of balloon |
|  | 0-02-082 | Weight of balloon |
|  | 0-02-083 | Type of balloon shelter |
|  | 0-02-084 | Type of gas used in balloon |
|  | 0-02-085 | Amount of gas used in balloon |
|  | 0-02-086 | Balloon flight train length |
|  | 0-02-095 | Type of pressure sensor |
|  | 0-02-096 | Type of temperature sensor |
|  | 0-02-097 | Type of humidity sensor |
|  | 0-02-103 | Radome |
|  | 0-02-191 | Geopotential height calculation |
|  | 0-25-061 | Software identification and version number |
|  | 0-35-035 | Reason for termination |

Add the entry 3 01 128 in BUFR Table D [FT2015-2]

- 2015-5.3(DRMM-III) Clarification of upper-air wind reporting nearby the Poles [ABC2016] <para 6.1>

- To amend B/C 20.5.2.5 Regulation as follows:

B/C 20.5.2.5 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- To amend B/C 25.7.2.8 Regulation as follows:

B/C 25.7.2.8 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- To amend B/C 26.5.2.8 Regulation as follows:

B/C 26.5.2.8 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured in sounding originated from a launch site within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- 2014-3.2.7(DRMM-II)/BUFR template for n-minute AWS data (3 07 092) <combined with 2013-3.2.1(DRMM-I)> [Validation] <para 6.1>

Add new entries:

BUFR Table D --- Check availability of element descriptors

|  |  | (Temperature and humidity instrumentation) |  |
| --- | --- | --- | --- |
| 3 01 130 | 0 03 002 | Generic type of humidity instrument |  |
|  | 0 03 003 | Configuration of sensors |  |
|  | 0 03 004 | Type of shield or screen |  |
|  | 0 03 005 | Horizontal width of screen or shield (x) |  |
|  | 0 03 006 | Horizontal depth of screen or shield (y) |  |
|  | 0 03 007 | Vertical height of screen or shield (z) |  |
|  | 0 03 008 | Artificially ventilated screen or shield |  |
|  | 0 03 009 | Amount of forced ventilation at time of reading |  |

BUFR/CREX Table B --- Check availability of element descriptors

| F X Y | Element Name | BUFR Units | BUFR Scale | BUFR Ref. | BUFR Width | CREX Units | CREX Scale | CREX Width |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 03 001 | Surface station type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 002 | Generic type of humidity instrument | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 003 | Configuration of sensors | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 03 004 | Type of shield or screen | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 03 005 | Horizontal width of screen or shield (x) | m | 2 |  |  | m | 2 |  |
| 0 03 006 | Horizontal depth of screen or shield (y) | m | 2 |  |  | m | 2 |  |
| 0 03 007 | Vertical height of screen or shield (z) | m | 2 |  |  | m | 2 |  |
| 0 03 008 | Artificially ventilated screen or shield | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 03 009 | Amount of forced ventilation at time of reading | m-3 s-1 | 1 |  |  | m-3 s-1 | 1 |  |

Code tables --- Check availability of code tables

0 03 001  Surface station type

Code figure

0	Land station (synoptic network)

1	Shallow water station (fixed to sea/lake floor)

2	Ship

3	Rig/platform

4	Moored buoy

5	Drifting buoy (or drifter)

6	Ice buoy

7	Land station (local network)

8	Land vehicle

9	Autonomous marine vehicle

10-14	Reserved

15	Missing value

Notes:

(1)	The last three categories are for possible future use.

(2)	"Land station (local network)" distinguishes "non-synoptic" stations these aren't currently distributed on the GTS but might be stored in BUFR in the future.

(3)	There could be separate tables for marine and land stations but there are marginal cases (shallow water fixed stations in the southern North Sea and some rigs reporting in SYNOP code).

0 03 002   Generic type of humidity instrument

Code figure

0	Psychrometer [too generic]

1	Capacitive sensor (unheated)

2	Capacitive sensor (heated)

3	Resistive sensor [too generic]

4	Ordinary human hair

5	Rolled hair

6	Goldbeater’s skin

7	Chilled mirror hygrometer

8	Dew cell

9	Optical absorption sensor

10-14	Reserved

15	Missing value

0 03 003  Configuration of Sensors [already full]

Code figure

0	Solar radiation shield or screen (double v section louvers)

1	No solar radiation shield or screen

2	Solar radiation shield or screen (single v section louvers)

3	Solar radiation shield or screen (overlapping louvers)

4	Solar radiation shield or screen (non-overlapping louvers)

5	Solar radiation shield or screen (not louvered)

6	Integrated e.g. chilled mirror

7	Missing value

0 03 004 Type of Shield or Screen [entries need to be checked]

Code figure

0	Within Stevenson screen (wooden)

1	Within Stevenson screen (plastic)

2	Within marine Stevenson screen (wooden)

3	Within marine Stevenson screen (plastic)

4	Within cylindrical section plate shield (metal)

5	Within cylindrical section plate shield (wooden)

6	Within cylindrical section plate shield (plastic)

7	Within concentric tube (metal)

8	Within concentric tube (wooden)

9	Within concentric tube (plastic)

10	Within rectangular section shield (metal)

11	Within rectangular section shield (wooden)

12	Within rectangular section shield (plastic)

13	Within rectangular section shield (metal)

14	Within square section shield (wooden)

15	Within square section shield (plastic)

16	Within square section shield (metal)

17	Within triangular section shield (wooden)

18	Within triangular section shield (plastic)

19	Within triangular section shield (metal)

20	Within open covered lean-to (reed/grass/leaf)

21	Within open covered inverted v roof (reed/grass/leaf)

22-29	Reserved

30	Not Applicable, e.g. chilled mirror manufacturers enclosure

31	Missing value

0 03 008 Artificially Ventilated Screen or Shield

Code figure

0	Natural ventilation in use

1	Artificial aspiration in use: constant flow at time of reading

2	Artificial aspiration in use: variable flow at time of reading

3 -6	Reserved

7	Missing value

Add a template:

BUFR template for surface observations from n-minute period

# TM 307092

|  |  | National station identification |  |
| --- | --- | --- | --- |
| 3 01 089 | 0 01 101 | State identifier (1) | Code table, 0 |
|  | 0 01 102 | National station number (1) | Numeric, 0 |
|  |  | Fixed surface station identification; time, horizontal and vertical co-ordinates |  |
| 3 01 090 | 3 01 004 | Surface station identification |  |
|  |  | WMO block number | Numeric, 0 |
|  |  | WMO station number | Numeric, 0 |
|  |  | Station or site name | CCITT IA5, 0 |
|  |  | Type of station | Code table, 0 |
|  | 3 01 011 | Year(2) | Year, 0 |
|  |  | Month(2) | Month, 0 |
|  |  | Day(2) | Day, 0 |
|  | 3 01 012 | Hour(2) | Hour, 0 |
|  |  | Minute(2) | Minute, 0 |
|  | 3 01 021 | Latitude (high accuracy) | Degree, 5 |
|  |  | Longitude (high accuracy) | Degree, 5 |
|  | 0 07 030 | Height of station ground above mean sea level | m, 1 |
|  | 0 07 031 | Height of barometer above mean sea level | m, 1 |
| 0 03 001 |  | Surface station type | Code table, 0 |
| 0 08 010 |  | Surface qualifier (for temperature data) | Code table, 0 |
| 3 01 091 |  | Surface station instrumentation |  |
|  | 0 02 180 | Main present weather detecting system | Code table, 0 |
|  | 0 02 181 | Supplementary present weather sensor | Flag table, 0 |
|  | 0 02 182 | Visibility measurement system | Code table, 0 |
|  | 0 02 183 | Cloud detection system | Code table, 0 |
|  | 0 02 184 | Type of lightning detection sensor | Code table, 0 |
|  | 0 02 179 | Type of sky condition algorithm | Code table, 0 |
|  | 0 02 186 | Capability to detect precipitation phenomena | Flag table, 0 |
|  | 0 02 187 | Capability to detect other weather phenomena | Flag table, 0 |
|  | 0 02 188 | Capability to detect obscuration | Flag table, 0 |
|  | 0 02 189 | Capability to discriminate lightning strikes | Flag table, 0 |
| 0 04 015 |  | Time increment (= - n minutes) | Minute, 0 |
| 0 04 065 |  | Short time increment ( = 1 minute) | Minute, 0 |
| 1 33 000 |  | Delayed replication of 33 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor (= n) | Numeric, 0 |
| 0 10 004 |  | Pressure | Pa, –1 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 3 02 070 |  | Wind data |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 11 001 | Wind direction | Degree true, 0 |
|  | 0 11 002 | Wind speed | m s-1, 1 |
|  | 0 11 043 | Maximum wind gust direction | Degree true, 0 |
|  | 0 11 041 | Maximum wind gust speed | m s-1, 1 |
|  | 0 11 016 | Extreme counterclockwise wind direction of a variable wind | Degree true, 0 |
|  | 0 11 017 | Extreme clockwise wind direction of a variable wind | Degree true, 0 |
|  |  | Temperature and humidity instrumentation |  |
| 3 01 130 | 0 03 002 | Generic type of humidity instrument | Code table, 0 |
|  | 0 03 003 | Configuration of sensors | Code table, 0 |
|  | 0 03 004 | Type of Shield or Screen | Code table, 0 |
|  | 0 03 005 | Horizontal Width of Screen or Shield (x) | m, 2 |
|  | 0 03 006 | Horizontal Depth of Screen or Shield (y) | m, 2 |
|  | 0 03 007 | Vertical Height of Screen or Shield (z) | m, 2 |
|  | 0 03 008 | Artificially Ventilated Screen or Shield | Code table, 0 |
|  | 0 03 009 | Degree of Forced Ventilation at time of reading | m-3 s-1, 1 |
|  | 0 33 003 | Quality of humidity measurement | Code table, 0 |
|  |  | Temperature and humidity data |  |
| 3 02 072 | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 12 101 | Temperature/Air-temperature (scale 2) | K, 2 |
|  | 0 12 103 | Dew-point temperature (scale 2) | K, 2 |
|  | 0 13 003 | Relative humidity | %, 0 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 0 07 032 |  | Height of sensor above local ground(6) | m, 2 |
| 0 08 010 |  | Surface qualifier | Code table, 0 |
| 0 12 120 |  | Ground temperature | K, 2 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 0 08 010 |  | Surface qualifier (set to missing to cancel the previous value) | Code table, 0 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 1 01 005 |  | Replicate 1 descriptor five times |  |
| 3 07 063 | 0 07 061 | Depth below land surface | m, 2 |
|  | 0 12 130 | Soil temperature (scale 2) | K, 2 |
| 0 07 061 |  | Depth below land surface (set to missing to cancel the previous value) | m, 2 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 069 |  | Visibility data |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 33 041 | Attribute of following value | # Code table, 0 |
|  | 0 20 001 | Horizontal visibility | m, –1 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 0 07 033 |  | Height of sensor above water surface (set to missing to cancel the previous value) | m, 1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 073 |  | Cloud data |  |
|  | 0 20 010 | Cloud cover (total) | %, 0 |
|  | 1 05 004 | Replicate 5 descriptors four times |  |
|  | 0 08 002 | Vertical significance | Code table, 0 |
|  | 0 20 011 | Cloud amount | Code table, 0 |
|  | 0 20 012 | Cloud type | Code table, 0 |
|  | 0 33 041 | Attribute of following value | Code table, 0 |
|  | 0 20 013 | Height of base of cloud | m, –1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 076 |  | Precipitation, obscuration and other phenomena |  |
|  | 0 20 021 | Type of precipitation | Flag table, 0 |
|  | 0 20 022 | Character of precipitation | # Code table, 0 |
|  | 0 26 020 | Duration of precipitation(3) | Minute, 0 |
|  | 0 20 023 | Other weather phenomena | Flag table, 0 |
|  | 0 20 024 | Intensity of phenomena | Code table, 0 |
|  | 0 20 025 | Obscuration | Flag table, 0 |
|  | 0 20 026 | Character of obscuration | Code table, 0 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 13 155 |  | Intensity of precipitation | kgm-2s-1, 4 |
| 0 13 058 |  | Size of precipitation element | m, 4 |
|  |  | (end of the replicated sequence) |  |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 20 031 |  | Ice deposit (thickness) | m, 2 |
| 0 20 032 |  | Rate of ice accretion | Code table, 0 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 078 |  | State of ground and snow depth measurement |  |
|  | 0 02 176 | Method of state of ground measurement | Code table, 0 |
|  | 0 20 062 | State of ground (with or without snow) | Code table, 0 |
|  | 0 02 177 | Method of snow depth measurement | Code table, 0 |
|  | 0 13 013 | Total snow depth | m, 2 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 079 |  | Precipitation measurement |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 02 175 | # Method of precipitation measurement | Code table, 0 |
|  | 0 02 178 | Method of liquid water content measurement of Precipitation | Code table, 0 |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 13 011 | Total precipitation / total water equivalent of snow | kg m-2, 1 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 080 |  | Evaporation measurement |  |
|  | 0 02 185 | # Method of evaporation measurement | Code table, 0 |
|  | 0 04 025 | Time period or displacement ( = - n minutes) | Minute, 0 |
|  | 0 13 033 | Evaporation /evapotranspiration | kg m-2, 1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 081 |  | Total sunshine data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 031 | Total sunshine | Minute, 0 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 082 |  | Radiation data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 002 | Long-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 004 | Short-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 016 | Net radiation, integrated over period specified | J m-2, -4 |
|  | 0 14 028 | Global solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 04 025 |  | Time period (= - n minutes) | Minute |
| 0 13 059 |  | Number of flashes | Numeric |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 083 |  | First order statistics of P, W, T, U data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 08 023 | First order statistics (= 9; best estimate of standard deviation) (4) | Code table, 0 |
|  | 0 10 004 | Pressure | Pa, –1 |
|  | 0 11 001 | Wind direction | Degree true, 0 |
|  | 0 11 002 | Wind speed | m s-1, 1 |
|  | 0 12 101 | Temperature/air temperature (scale 2) | K, 2 |
|  | 0 13 003 | Relative humidity | %, 0 |
|  | 0 08 023 | First order statistics (= missing value) | Code table, 0 |
| 0 33 005 |  | Quality information (AWS data) | Flag table, 0 |
| 0 33 006 |  | Internal measurement status information (AWS) | Code table, 0 |

Notes:

(1)	0 01 101 (WMO Member State identifier) and 0 01 102 (National AWS number) shall be used to identify a station within the national numbering system that is completely independent of the WMO international numbering system. The WMO international identification 0 01 001 (WMO block number) and 0 01 002 (WMO station number) shall be reported if available for the particular station.

(2)	The time identification refers to the end of the n-minute period.

(3)	Duration of precipitation (in minutes) represents number of minutes in which any precipitation was registered.

(4)	Best estimate of standard deviation is counted out of a set of samples (signal measurements) recorded within the period specified; it should be reported as a missing value, if the measurements of the relevant element are not available from a part of the period specified by 0 04 025.

(5)	If reporting nominal values is required, the template shall be supplemented with 3 07 093.

(6)	The height above local ground 0 07 032 referring to ground temperature shall be considered as a variable. After a snowfall, the sensor is placed at the top of the snow layer and the changed value of 0 07 032 shall indicate this procedure (total snow depth is reported in 0 13 013).

- 2014-3.2.9(DRMM-II)/Proposed BUFR sequence for reporting observations from offshore platforms [FT2017-2] <para 6.1>

Add new entries:

BUFR/CREX Table B

| Table reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (characters) |
| 0 | 02 | 008 | Type of offshore platform | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

BUFR Table D

|  |  | (Sequence for platform identification, type, time and location of the observation report) |  |
| --- | --- | --- | --- |
| 3 01 056 | 0 01 087 | WMO marine observing platform extended identifier | WMO number (extended 7 digit identifier) |
|  | 0 01 011 | Ship or mobile land station identifier | Call sign (where allocated) |
|  | 0 01 015 | Station or site name | Platform name |
|  | 0 02 008 | Type of offshore platform |  |
|  | 0 02 001 | Type of station |  |
|  | 3 01 011 | Year, month, day |  |
|  | 3 01 012 | Hour, minute |  |
|  | 3 01 021 | Latitude/longitude (high accuracy) |  |
|  | 0 07 030 | Height of station ground above mean sea level | Height of station platform above mean sea level |
|  | 0 07 031 | Height of barometer above mean sea level |  |

|  |  | (Sequence for reporting observations from offshore platforms) |  |
| --- | --- | --- | --- |
| 3 08 017 | 3 01 056 | Sequence for platform identification, type, time and location of the observation report |  |
|  | 3 02 001 | Pressure and 3-hour pressure change |  |
|  | 3 02 052 | Ship temperature and humidity data |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 056 | Sea/water temperature | Optional |
|  | 3 02 064 | Ship wind data (see Note) |  |
|  | 3 02 053 | Ship visibility data |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 004 | General cloud information | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 005 | Cloud layer | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 038 | Present and past weather | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 06 039 | Sequence for representation of basic wave measurements | Optional |

Add new code tables:

0 02 008

Type of offshore platform

| Code figure | Meaning |
| --- | --- |
| 0 | Fixed platform |
| 1 | Mobile offshore drill ship |
| 2 | Jack-up rig |
| 3 | Semi-submersible platform |
| 4 | FPSO (floating production storage and offloading unit) |
| 5 | Light vessel |
| 6 – 14 | Reserved |
| 15 | Missing value (or not known) |

- 2014-11.2(DRMM-II)/WIGOS Station Identifier [FT2017-1] <para 6.1>

Add entries in BUFR/CREX Table B.

Class 01 – BUFR/CREX Identification

|  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE |  |  |  |  | DATA |  |  | DATA |
| REFERENCE | ELEMENT NAME | UNIT | SCALE | REFERENCE | WIDTH | UNIT | SCALE | WIDTH |
| F X Y |  |  |  | VALUE | (Bits) |  |  | (Characters) |
| 0 01 125 | WIGOS Identifier Series | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |
| 0 01 126 | WIGOS Issuer of Identifier | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |
| 0 01 127 | WIGOS Issue Number | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |
| 0 01 128 | WIGOS Local Identifier (Character) | CCITT IA5 | 0 | 0 | 128 | Character | 0 | 16 |

Add entries in BUFR/CREX Table D.

Category 01 – Location and identification sequences

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION |
| --- | --- | --- | --- |
| F X Y |  |  |  |
|  |  | (WIGOS Identifier) |  |
| 3 01 150 | 0 01 125 | WIGOS Identifier Series |  |
|  | 0 01 126 | WIGOS Issuer of Identifier |  |
|  | 0 01 127 | WIGOS Issue Number |  |
|  | 0 01 128 | WIGOS Local Identifier (Character) |  |

- 2013-3.2.2(DRMM-I)/Proposal for a BUFR template for radar wind profiler [FT2017-1] <para 6.1>

Add templates:

Radar Wind Profiler Data Sequence:

| TABLE REFERENCE | TABLE REFERENCES | TABLE REFERENCES | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- | --- | --- |
| 3 09 021 |  |  |  | RWP Wind data (product data) NEW |
|  | 3 01 001 |  |  | WMO block and station numbers |
|  |  | 0 01 001 |  | WMO Block Number |
|  |  | 0 01 002 |  | WMO Station Number |
|  | 0 05 001 |  |  | Latitude (high accuracy) |
|  | 0 06 001 |  |  | Longitude (high accuracy) |
|  | 0 07 030 |  |  | Height of station ground above mean sea level |
|  | 3 01 014 |  |  | Time period |
|  |  | 1 02 002 |  | Replicate 2 descriptors 2 times |
|  |  | 3 01 011 |  | Year, month, day |
|  |  |  | 0 04 001 | Year |
|  |  |  | 0 04 002 | Month |
|  |  |  | 0 04 003 | Day |
|  |  | 3 01 012 |  | Hour, minute |
|  |  |  | 0 04 004 | Hour |
|  |  |  | 0 04 005 | Minute |
|  | 0 02 003 |  |  | Type of measuring equipment used |
|  | 0 02 121 |  |  | Mean frequency (see note a) |
|  | 1 12 000 |  |  | Delayed replication of 12 descriptors |
|  | 0 31 001 |  |  | Delayed descriptor replication factor |
|  | 0 07 007 |  |  | Height |
|  | 3 01 021 |  |  | Latitude/longitude (high accuracy) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  | 0 11 003 |  |  | u-component |
|  | 0 11 110 |  |  | Uncertainty in u-component NEW |
|  | 0 11 004 |  |  | v-component |
|  | 0 11 111 |  |  | Uncertainty in v-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 11 006 |  |  | w-component |
|  | 0 11 112 |  |  | Uncertainty in w-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 10 071 |  |  | Vertical resolution NEW |
|  | 0 27 079 |  |  | Horizontal width of sampled volume NEW |

RASS Data Sequence:

| TABLE REFERENCE | TABLE REFERENCES | TABLE REFERENCES | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- | --- | --- |
| 3 09 022 |  |  |  | RASS virtual temperature (product data) NEW |
|  | 3 01 001 |  |  | WMO block and station numbers |
|  |  | 0 01 001 |  | WMO Block Number |
|  |  | 0 01 002 |  | WMO Station Number |
|  | 0 05 001 |  |  | Latitude (high accuracy) |
|  | 0 06 001 |  |  | Longitude (high accuracy) |
|  | 0 07 030 |  |  | Height of station ground above mean sea level |
|  | 3 01 014 |  |  | Time period |
|  |  | 1 02 002 |  | Replicate 2 descriptors 2 times |
|  |  | 3 01 011 |  | Year, month, day |
|  |  |  | 0 04 001 | Year |
|  |  |  | 0 04 002 | Month |
|  |  |  | 0 04 003 | Day |
|  |  | 3 01 012 |  | Hour, minute |
|  |  |  | 0 04 004 | Hour |
|  |  |  | 0 04 005 | Minute |
|  | 0 02 003 |  |  | Type of measuring Equipment used |
|  | 0 02 121 |  |  | Mean frequency (see note a) |
|  | 1 10 000 |  |  | Delayed replication of 10 descriptors |
|  | 0 31 001 |  |  | Delayed descriptor replication factor |
|  | 0 07 007 |  |  | Height |
|  | 3 01 021 |  |  | Latitude/longitude (high accuracy) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  | 0 12 007 |  |  | Virtual temperature |
|  | 0 12 008 |  |  | Uncertainty in virtual temperature NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 11 006 |  |  | w-component |
|  | 0 11 112 |  |  | Uncertainty in w-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 10 071 |  |  | Vertical resolution NEW |
|  | 0 27 079 |  |  | Horizontal width of sampled volume NEW |

Add descriptors:

|  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE |  |  |  |  | DATA |  |  | DATA |
| REFERENCE | ELEMENT NAME | UNIT | SCALE | REFERENCE | WIDTH | UNIT | SCALE | WIDTH |
| F* X Y |  |  |  | VALUE | (Bits) |  |  | (Characters) |
| 0 11 110 | Uncertainty in u-component | m s-1 | 1 | -4096 | 13 | m s-1 | 1 | 4 |
| 0 11 111 | Uncertainty in v-component | m s-1 | 1 | -4096 | 13 | m s-1 | 1 | 4 |
| 0 11 112 | Uncertainty in w-component | m s-1 | 2 | -4096 | 13 | m s-1 | 2 | 4 |
| 0 12 008 | Uncertainty in virtual temperature | K | 1 | 0 | 12 | C | 1 | 4 |

- 2013-3.2.4(DRMM-I)/Satellite-derived winds in BUFR [FT2017-2] <para 6.1>

Add an entry:

| 3 10 067 |  | (Satellite-derived winds) |  |
| --- | --- | --- | --- |
|  |  | Processing information |  |
|  | 0 01 033 | Identification of originating/generating centre |  |
|  | 0 01 034 | Identification of originating/generating sub-centre |  |
|  | 0 25 061 | Software identification and version number |  |
|  | 0 25 062 | Database identification |  |
|  |  | Satellite/Instrument identification |  |
|  | 0 01 007 | Satellite identifier |  |
|  | 0 02 153 | Satellite channel centre frequency |  |
|  | 0 01 012 | Direction of motion of moving observing platform |  |
|  | 2 01 138 | Change data width |  |
|  | 0 02 026 | Cross-track resolution |  |
|  | 0 02 027 | Along-track resolution |  |
|  | 2 01 000 | Cancel change data width |  |
|  |  | Methods |  |
|  | 0 02 028 | Segment size at nadir in x-direction (target box size) |  |
|  | 0 02 029 | Segment size at nadir in y-direction (target box size) |  |
|  | 0 02 161 | Wind processing method |  |
|  | 0 02 164 | Tracer correlation method |  |
|  | 0 02 023 | Satellite derived wind computation method |  |
|  | 0 08 012 | Land/sea qualifier |  |
|  | 0 08 013 | Day/night qualifier |  |
|  |  | Final AMV data |  |
|  | 0 01 124 | Grid point identifier | . |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 04 001 | Year |  |
|  | 0 04 002 | Month |  |
|  | 0 04 003 | Day |  |
|  | 0 04 004 | Hour |  |
|  | 0 04 005 | Minute |  |
|  | 0 04 006 | Second |  |
|  | 0 04 086 | Long time period or displacement (seconds) |  |
|  | 0 11 001 | Wind direction |  |
|  | 0 11 002 | Wind speed |  |
|  | 0 11 003 | Wind u-component |  |
|  | 0 11 004 | Wind v-component |  |
|  | 0 02 162 | Extended height assignment method |  |
|  | 0 07 004 | Pressure |  |
|  | 0 12 001 | Temperature |  |
|  | 0 20 014 | Height of top of cloud |  |
|  | 0 07 024 | Satellite zenith angle |  |
|  | 0 01 023 | Observation sequence number |  |
|  | 1 04 000 | Delayed replication of 4 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 02 162 | Extended height assignment method |  |
|  | 0 07 004 | Pressure |  |
|  | 0 12 001 | Temperature |  |
|  | 0 20 014 | Height of top of cloud |  |
|  |  | Image information (for each image used) |  |
|  | 1 13 000 | Delayed replication of 13 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 086 | Long time period or displacement (seconds) |  |
|  | 0 02 020 | Satellite classification |  |
|  | 0 01 007 | Satellite identifier |  |
|  | 0 02 019 | Satellite instruments |  |
|  | 0 05 042 | Channel number |  |
|  | 0 02 153 | Satellite channel centre frequency |  |
|  | 0 05 040 | Orbit number |  |
|  | 0 07 024 | Satellite zenith angle |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 02 162 | Extended height assignment method |  |
|  | 0 07 004 | Pressure |  |
|  | 0 12 001 | Temperature |  |
|  | 0 20 014 | Height of top of cloud |  |
|  |  | Intermediate vectors (for each component vector) |  |
|  | 1 19 000 | Delayed replication of 19 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 086 | Long time period or displacement (seconds) |  |
|  | 0 04 086 | Long time period or displacement (seconds) |  |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 11 003 | u-component |  |
|  | 0 11 004 | v-component |  |
|  | 0 11 113 | Tracking correlation of vector |  |
|  | 0 25 148 | Coefficient of variation |  |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 023 | First order statistics |  |
|  | 0 11 003 | u-component |  |
|  | 0 11 004 | v-component |  |
|  | 0 08 023 | First order statistics | Set to missing (cancel) |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 111 | x-axis error ellipse major component |  |
|  | 0 20 112 | y-axis error ellipse minor component |  |
|  | 0 20 114 | Angle of x-axis in error ellipse |  |
|  |  | Corresponding forecast data |  |
|  | 0 01 033 | Identification of originating/generating centre |  |
|  | 0 08 021 | Time significance | = 27 First guess |
|  | 0 11 095 | u-component of the model wind vector |  |
|  | 0 11 096 | v-component of the model wind vector |  |
|  | 0 07 004 | Pressure |  |
|  | 0 08 021 | Time significance | = 4 Forecast |
|  | 0 11 095 | u-component of the model wind vector |  |
|  | 0 11 096 | v-component of the model wind vector |  |
|  | 0 07 004 | Pressure |  |
|  | 0 08 021 | Time significance | Set to missing (cancel) |
|  | 0 08 086 | Vertical significance for NWP | = 10 Level of best fit |
|  | 0 11 095 | u-component of the model wind vector |  |
|  | 0 11 096 | v-component of the model wind vector |  |
|  | 0 07 004 | Pressure |  |
|  | 0 08 086 | Vertical significance for NWP | Set to missing (cancel) |
|  |  | Final AMV quality |  |
|  | 1 02 004 | Replicate 2 descriptors 4 times |  |
|  | 0 01 032 | Generating application |  |
|  | 0 33 007 | Per cent confidence |  |
|  | 0 08 092 | Measurement uncertainty expression | = 0 Standard uncertainty |
|  | 0 11 003 | u-component |  |
|  | 0 11 004 | v-component |  |
|  | 0 07 004 | Pressure |  |
|  | 0 08 092 | Measurement uncertainty expression | Set to missing (cancel) |
|  | 0 33 066 | AMV Quality Flag |  |
|  |  | Cloud data and microphysics (refers to the nominal image used for HA) |  |
|  | 0 20 081 | Cloud amount |  |
|  | 0 20 012 | Cloud type |  |
|  | 0 20 056 | Cloud phase |  |
|  | 1 17 000 | Delayed replication of 17 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 023 | First order statistics |  |
|  | 0 20 016 | Pressure at the top of cloud |  |
|  | 0 08 092 | Measurement uncertainty expression | = 0 Standard uncertainty |
|  | 0 08 003 | Vertical significance (satellite observations) | = 2 Cloud top |
|  | 0 12 001 | Temperature |  |
|  | 0 08 003 | Vertical significance (satellite observations) | Set to missing (cancel) |
|  | 0 20 016 | Pressure at the top of cloud |  |
|  | 0 08 092 | Measurement uncertainty expression | Set to missing (cancel) |
|  | 0 25 149 | Optimal estimation cost |  |
|  | 0 20 016 | Pressure at top of cloud |  |
|  | 0 20 014 | Height of top of cloud |  |
|  | 0 13 093 | Cloud optical thickness |  |
|  | 0 13 109 | Ice/liquid water path |  |
|  | 0 40 038 | Cloud particle size |  |
|  | 0 08 011 | Meteorological feature | = 12 Cloud |
|  | 0 14 050 | Emissivity |  |
|  | 0 08 011 | Meteorological feature | Set to missing (cancel) |
|  | 0 08 023 | First order statistics | Set to missing (cancel) |

#### Add new entries to Table B:

|  | Element name | BUFR | CREX |
| --- | --- | --- | --- |

| F X Y |  | Unit | Scale | Ref. value | Data width | Unit | Scale | Data width |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 02 161 | Wind processing method | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |
| 0 02 162 | Extended height assignment method | Code table | 0 | 0 | 6 | Code table | 0 | 2 |
| 0 11 113 | Tracking correlation of vector | Numeric | 3 | -1000 | 12 | Numeric | 3 | 4 |
| 0 13 109 | Ice/liquid water path | kg m-2 | 3 | 0 | 10 | kg m-2 | 3 | 4 |
| 0 25 147 | Size of largest cluster (in pixels) | Numeric | 0 | 0 | 10 | Numeric | 0 | 4 |
| 0 25 148 | Coefficient of variation | Numeric | 2 | -10000 | 15 | Numeric | 2 | 5 |
| 0 25 149 | Optimal estimation cost | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |
| 0 33 066 | AMV quality flag | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |
| 0 40 038 | Cloud particle size | m | 7 | 0 | 28 | m | 7 | 6 |

Add new entries to 0-02-164

3		Stereo matching

Add new entries to 0-08-086

10		Level of best fit

Add new entries to 0-20-056

5		Supercooled Liquid Water

Add new entries to 0-20-012

43		Clear

44		Liquid water

45		Supercooled liquid water

46		Mixed phase

47		Optically thick ice

48		Optically thin ice

49		Multilayered ice

0-02-161 Wind processing method

| Bit | DESCRIPTION |
| --- | --- |
| 1-10 | Reserved |
| 11 | Wind height calculated from median cloud-top pressure of target |
| 12 | Target is cloudy |
| 13 | Low-level inversion |
| 14 | CCC method |
| 15 | Nested tracking |
| All 16 | Missing value |

0-02-162  Extended height assignment method

| Code figure | DESCRIPTION |
| --- | --- |
| 0 | Auto editor |
| 1 | IRW height assignment |
| 2 | WV height assignment |
| 3 | H2O intercept height assignment |
| 4 | CO2 slicing height assignment |
| 5 | Low pixel max gradient |
| 6 | Higher pixel max gradient |
| 7 | Primary height assignment |
| 8 | Layer thickness assignment |
| 9 | Cumulative contribution function - 10 percent height |
| 10 | Cumulative contribution function - 50 percent height |
| 11 | Cumulative contribution function - 90 percent height |
| 12 | Cumulative contribution function - height of maximum gradient |
| 13 | IR/two WV channel rationing method |
| 14 | Composite height assignment |
| 15 | Optimal estimation |
| 16 | Inversion correction |
| 17 | Geometric height assignment |
| 18-62 | Reserved |
| 63 | Missing value |

0-33-066 AMV quality flag

| Bit | DESCRIPTION |
| --- | --- |
| 1-21 | Reserved |
| 22 | Correlation surface constraint fails |
| 23 | Reserved |
| All 24 | Missing value |

http://www.jcommops.org/ftp/SOT/MAPS/2017-06-VOS_TAC_TDC.png