As of 23 December 2015

- ANNEX TO PARAGRAPH 2.1.1 [Validation]

Add a new note:

(2)	With respect to the Code figures 0, 1, 3, 6, and 7, coordinates can only be unambiguously interpreted, if the coordinate reference system, in which they are embedded, is known. Therefore, defining the shape of the Earth alone without coordinate system axis origins is ambiguous. Generally, the prime meridian defined in the geodetic system WGS84 can be safely assumed to be the longitudinal origin. However, because these code figures do not specify the longitudinal origin explicitly, it is suggested to contact the originating center, if high precision coordinates are needed in order to obtain the precise details of the coordinate system used.

- ANNEX TO PARAGRAPH 2.1.2 [Validation]

Add a regulation:

92.6.3	In product definition templates that refer to a forecast time or offset from reference time, this may be negative to refer to times or intervals that begin before the reference time, if this is applicable.

- ANNEX TO PARAGRAPH 2.2.1 [Validation] <para 4.1>

- ANNEX TO PARAGRAPH 2.2.2

Add entries:

In Code table 4.1,

Discipline 3 (Space products)

191	Miscellaneous

In Code Table 4.2:

Discipline 3 (Space products), category 1 (Quantitative products)

18	Water temperature							K

Discipline 3 (Space products), Category 191 (Miscellaneous)

0	Seconds prior to initial reference time (defined in Section 1)			s

Add a code table:

Code table 4.16 – Quality value associated with parameter

Code figure	Meaning

0	Confidence index (see Note 2)

1	Quality indicator (see Note 3 and Code table 4.244)

2	Correlation of product with used calibration product (see Note 4)

3	Standard deviation of product from calibration product (see Note 5)

4-191	Reserved

192-254	Reserved for local use

255	Missing

Notes:

(1)	When a non-missing value is used from this code table, the original data value is a quality value associated with the parameter defined by octets 10 and 11 of the product definition template.

(2)	The original data value is a non-dimensional number from 0 to 1, where 0 indicates no confidence and 1 indicates maximal confidence.

(3)	The original data value is defined by Code table 4.244.

(4)	The original data value is a non-dimensional number without units.

(5)	The original data value is in the same units as the parameter defined by octets 10 and 11 of the product definition template.

Code table 4.244 – Quality indicator

Code figure	Meaning

0	No quality information available

1	Failed

2	Passed

3-191	Reserved

192-254	Reserved for local use

255	Missing

Add a template:

Product definition template 4.35 – satellite product with or without associated quality values

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Observation generating process identifier (defined by originating centres)

14	Quality value associated with parameter (see Code Table 4.16)

15	Number of contributing spectral bands (NB)

16–   Repeat the following 11 octets for each contributing band (nb = 1, NB)

(16+11(nb–1))–(17+11(nb–1))	Satellite series of band nb (code table defined by originating/generating centre)

(18+11(nb–1))–(19+11(nb–1))	Satellite numbers of band nb (code table defined by originating/generating centre)

(20+11(nb–1))–(21+11(nb–1))	Instrument types of band nb (code table defined by originating/generating centre)

(22+11(nb–1))	Scale factor of central wave number of band nb

(23+11(nb–1))–(26+11(nb–1))	Scaled value of central wave number of band nb (units: m–1)

Note:	For “satellite series of band nb”, “satellite numbers of band nb” and “instrument types of band nb”, it is recommended to encode the values as per BUFR Code tables 0 02 020, 0 01 007 (Common Code table C–5) and 0 02 019 (Common Code table C–8), respectively.

- ANNEX TO PARAGRAPH 2.2.3 [FT2015-2]

Add an entry:

In Code table 4.2,

Parameter discipline 0 - meteorological products, parameter category 19: physical atmospheric properties

28	Mountain wave turbulence (eddy dissipation rate)							m2/3 s–1

- ANNEX TO PARAGRAPH 2.2.4 [FT2015-2]

Add an entry:

In Code table 4.2,

Product discipline 2 – Land surface products, parameter category 4: fire weather products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 5 | Fire Weather Index (Canadian Forest Service) | Numeric | The fire weather index (FWI) is a numerical rating of fire intensity that combines the initial spread index (ISI) and the build-up index (BUI), which in turn is calculated from the Duff moisture code (DMC) and the Drought code (DC). (*) |
| 6 | Fine Fuel Moisture Code (Canadian Forest Service) | Numeric | The Fine Fuel Moisture Code is a numerical rating of the moisture content of litter and other cured fine fuels. It indicates the relative ease of ignition and flammability of fine fuel. (*) |
| 7 | Duff Moisture Code (Canadian Forest Service) | Numeric | The Duff Moisture Code is a numerical rating of the average moisture content of loosely compacted organic layers of moderate depth. It gives an indication of fuel consumption in moderate duff layers and medium-size woody material. (*) |
| 8 | Drought Code (Canadian Forest Service) | Numeric | The DC is a numerical rating of the average moisture content of deep, compact, organic layers. It is a useful indicator of seasonal drought effects on forest fuels, and amount of smouldering in deep duff layers and large logs. |
| 9 | Initial Fire Spread Index (Canadian Forest Service) | Numeric | The Initial Fire Spread Index is a numerical rating of the expected rate of fire spread. It combines the effects of wind and the FFMC on rate of spread without the influence of variable quantities of fuel. (*) |
| 10 | Fire Build Up Index (Canadian Forest Service) | Numeric | The Fire Build Up Index is a numerical rating of the total amount of fuel available for combustion that combines the DMC and DC. (*) |
| 11 | Fire Daily Severity Rating (Canadian Forest Service) | Numeric | The Fire Daily Severity Rating is a numerical rating of the difficulty of controlling fires, based on the FWI that is calculated afterward. It reflects more accurately the expected efforts required for fire suppression. (*) |

(*)	Detailed description of this parameter and how it is computed can be found in Van Wagner CE (1987) “Development and structure of the Canadian Forest Fire Weather Index System.  ”Canadian Forestry Service, Technical Report 35", pp 37

- ANNEX TO PARAGRAPH 2.2.5 [FT2015-2]

Add an entry:

In Code table 4.2,

Product discipline 0 – Meteorological products, parameter category 0: temperature

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 22 | Temperature tendency due to short-wave radiation | K s-1 | Temperature tendency due to parameterised short-wave radiation, all sky. |
| 23 | Temperature tendency due to long-wave radiation | K s-1 | Temperature tendency due to parameterised long-wave radiation, all sky. |
| 24 | Temperature tendency due to short-wave radiation, clear sky | K s-1 | Temperature tendency due to parameterised short-wave radiation, clear sky. |
| 25 | Temperature tendency due to long-wave radiation, clear sky | K s-1 | Temperature tendency due to parameterised long-wave radiation, clear sky. |
| 26 | Temperature tendency due to parameterisations | K s-1 | Temperature tendency due to parameterisations. |

Product discipline 0 – Meteorological products, parameter category 1: moisture

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 108 | Specific humidity tendency due to parameterisations | kg kg-1 s-1 | Specific humidity tendency due to parameterisations |

Product discipline 0 – Meteorological products, parameter category 2: momentum

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 39 | Eastward wind tendency due to parameterisations | m s-2 | Eastward wind tendency due to parameterisations. |
| 40 | Northward wind tendency due to parameterisations | m s-2 | Northward wind tendency due to parameterisations. |

Product discipline 0 – Meteorological products, parameter category 3: mass

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 27 | Updraught mass flux | kg m-2 s-1 | Updraught mass flux due to parameterised convection. |
| 28 | Downdraught mass flux | kg m-2 s-1 | Downdraught mass flux due to parameterised convection. |
| 29 | Updraught detrainment rate | kg m-3 s-1 | Updraught detrainment rate due to parameterised convection. |
| 30 | Downdraught detrainment rate | kg m-3 s-1 | Downdraught detrainment rate due to parameterised convection. |

- ANNEX TO PARAGRAPH 2.2.6 [FT2015-2/pre-operational since 27 August]

Add an entry:

In Common Code table C-12 under the originating centre, RSMC Rome (Code figure: 80),

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

80	Rome (RSMC)	102	National Research Council/Institute of

Atmospheric Sciences and Climate

(CNR-ISAC)

- ANNEX TO PARAGRAPH 2.2.7 [FT2015-2]

Add entries:

In Code table 4.1

| Product Discipline | Parameter Category | Description |
| --- | --- | --- |
| 3 | 2 | Cloud Properties |
| 3 | 3 | Flight Rules Conditions |
| 3 | 4 | Volcanic Ash |
| 3 | 5 | Sea Surface Temperature |
| 3 | 6 | Solar Radiation |

In Code table 4.2,

| Product Discipline | Parameter Category | Parameter Number | Parameter | Units |
| --- | --- | --- | --- | --- |
| 3 | 1 | 27 | bidirectional reflectance factor (Note 1) | numeric |
| 3 | 1 | 28 | brightness temperature | K |
| 3 | 1 | 29 | scaled radiance (Note 2) | numeric |
| 3 | 2 | 0 | clear sky probability | % |
| 3 | 2 | 1 | cloud top temperature | K |
| 3 | 2 | 2 | cloud top pressure | Pa |
| 3 | 2 | 3 | cloud type | Code table 4.218 |
| 3 | 2 | 4 | cloud phase | Code table 4.218 |
| 3 | 2 | 5 | cloud optical depth | Numeric |
| 3 | 2 | 6 | cloud particle effective radius | m |
| 3 | 2 | 7 | cloud liquid water path | kg/m2 |
| 3 | 2 | 8 | cloud ice water path | kg/m2 |
| 3 | 2 | 9 | cloud albedo | Numeric |
| 3 | 2 | 10 | cloud emissivity | Numeric |
| 3 | 2 | 11 | effective absorption optical depth ratio | Numeric |
| 3 | 3 | 0 | Probability of encountering Marginal Visual Flight Rules conditions | % |
| 3 | 3 | 1 | Probability of encountering Low Instrument Flight Rules conditions | % |
| 3 | 3 | 2 | Probability of encountering Instrument Flight Rules conditions | % |
| 3 | 4 | 0 | volcanic ash probability | % |
| 3 | 4 | 1 | volcanic ash cloud top temperature | K |
| 3 | 4 | 2 | volcanic ash cloud top pressure | Pa |
| 3 | 4 | 3 | volcanic ash cloud top height | m |
| 3 | 4 | 4 | volcanic ash cloud emissivity | Numeric |
| 3 | 4 | 5 | volcanic ash effective absorption optical depth ratio | Numeric |
| 3 | 4 | 6 | volcanic ash cloud optical depth | Numeric |
| 3 | 4 | 7 | volcanic ash column density | kg/m2 |
| 3 | 4 | 8 | volcanic ash particle effective radius | m |
| 3 | 5 | 0 | interface sea surface temperature (Note 3) | K |
| 3 | 5 | 1 | skin sea surface temperature (Note 4) | K |
| 3 | 5 | 2 | sub-skin sea surface temperature (Note 5) | K |
| 3 | 5 | 3 | foundation sea surface temperature (Note 6) | K |
| 3 | 5 | 4 | estimated bias between sea surface temperature and standard | K |
| 3 | 5 | 5 | estimated standard deviation between sea surface temperature and standard | K |
| 3 | 6 | 0 | global solar irradiance (Note 7) | W/m2 |
| 3 | 6 | 1 | global solar exposure (Note 8) | J/m2 |
| 3 | 6 | 2 | direct solar irradiance (Note 9) | W/m2 |
| 3 | 6 | 3 | direct solar exposure (Note 10) | J/m2 |
| 3 | 6 | 4 | diffuse solar irradiance (Note 11) | W/m2 |
| 3 | 6 | 5 | diffuse solar exposure (Note 12) | J/m2 |

Notes:

1.	The ratio of the radiant flux reflected by a surface to that reflected into the same reflected-beam geometry and wavelength range by an ideal (lossless) and diffuse (Lambertian) standard surface, irradiated under the same conditions.

2.	Top of atmosphere radiance observed by a sensor, multiplied by pi and divided by the in-band solar irradiance.

3.	Theoretical temperature at the precise air-sea interface

4.	Temperature of the water across a very small depth (approximately the upper 20 micrometers)

5.	Temperature at the base of the thermal skin layer

6.	Temperature of the water column free of diurnal temperature variability or equal to the SSTsubskin in the absence of any diurnal signal

7.	The solar flux per unit area received from a solid angle of 2π sr on a horizontal surface

8.	Time integral of global solar irradiance

9.	The solar flux per unit area received from the solid angle of the sun’s disc on a surface normal to the sun direction

10.	Time integral of direct solar irradiance

11.	The solar flux per unit area received from a solid angle of 2π sr, except for the solid angle of the sun's disc, on a horizontal surface

12.	Time integral of diffuse solar irradiance

In Code table 4.218 (Pixel scene type),

| Code Figure | Meaning |
| --- | --- |
| 19 | Warm liquid water cloud |
| 20 | Supercooled liquid water cloud |
| 21 | Mixed phase cloud |
| 22 | Optically thin ice cloud |
| 23 | Optically thick ice cloud |
| 24 | Multi-layered cloud |

In Code table 4.3,

| Code Figure | Meaning |
| --- | --- |
| 16 | Physical retrieval |
| 17 | Regression analysis |

- ANNEX TO PARAGRAPH 2.2.8 [FT2015-2 except for Code table 4.5 (FT2016-1)]

Add entries:

In Code table 4.2,

| Domain | Parameter Category | Parameter Number | Description | Units | Comment or note |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 027 | Wet bulb temperature | K |  |
| 0 | 4 | 013 | Direct short wave radiation flux | W m-2 |  |
| 0 | 4 | 014 | Diffuse short wave radiation flux | W m-2 |  |
| 0 | 6 | 040 | Mass density of convective cloud water droplets | kg m-3 |  |
| 0 | 6 | 047 | Volume fraction of cloud water droplets | Numeric | Note x |
| 0 | 6 | 048 | Volume fraction of cloud ice particles | Numeric | Note x |
| 0 | 6 | 049 | Volume fraction of cloud (ice and/or water) | Numeric | Note x |

Note:

(x)	The sum of the water and ice fractions may exceed the total due to overlap between the volumes containing ice and those containing liquid water.

Add an entry to GRIB2 Code table 4.5 [FT2016-1]

| Code figure | Meaning | Unit |
| --- | --- | --- |
| 013 | Lowest level where vertically integrated cloud cover exceeds the specified percentage (cloud base for a given percentage cloud cover) | % |

- ANNEX TO PARAGRAPH 2.2.9 [FT2015-2]

Add entries:

In Code table 4.2,

Product Discipline 0 – Meteorological products, Parameter category 2: momentum

| Number | #### Parameter | #### Units |
| --- | --- | --- |
| 41 | #### u-component of geostrophic wind | #### m s-1 |
| 42 | #### v-component of geostrophic wind | #### m s-1 |
| 43 | #### Geostrophic wind direction | #### degree true |
| 44 | #### Geostrophic wind speed | #### m s-1 |

Product Discipline 0 – Meteorological products, Parameter category 7: thermodynamic stability

| Number | #### Parameter | #### Units |
| --- | --- | --- |
| 16 | #### Bulk Richardson number | #### Numeric |
| 17 | #### Gradient Richardson number | #### Numeric |
| 18 | #### Flux Richardson number | #### Numeric |

- ANNEX TO PARAGRAPH 2.2.10 [FT2015-2]

Add entries:

In Code table 4.2,

Product discipline 1 – Hydrological products, parameter category 0: hydrology basic products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 8 | Groundwater upper storage | kg m-2 | Ground water storage in the upper saturated zone. Corresponds to fast responding (storm) flow. |
| 9 | Groundwater lower storage | kg m-2 | Ground water storage in the lower saturated zone. Corresponds to Base flow-groundwater runoff. |
| 10 | Side flow into river channel | m3 s-1 m-1 | Rate of runoff that enters the river channel in each grid cell. The runoff consists of the contributions from surface runoff, outflow from ground water, additional runoff from rivers and reservoirs. Calculated as a flow (m3/s) per m river length. |
| 11 | River storage of water | m3 | Total amount of water storage in the river network within a grid cell over the last time step. The term storage refers to the total volume of water |
| 12 | Floodplain storage of water | m3 | Total amount of water storage on the floodplain within a grid cell over the last time step. The term storage refers to the total volume of water. A floodplain is defined as: "A flat or nearly flat land adjacent to a stream or river that stretches from the banks of its channel to the base of the enclosing valley walls and experiences flooding during periods of high discharge" (Goudie, A. S., 2004, Encyclopedia of Geomorphology, vol. 1. Routledge, New York. ISBN 0-415-32737-7) |
| 13 | Depth of water on soil surface | kg m-2 | Total amount on water on soil surface that is not infiltrating the ground or intercepted on vegetation. The parameter can also be defined as water intercepted on soil. |
| 14 | Upstream accumulated precipitation | kg m-2 | Total accumulated precipitation (rain + snowfall) upstream each grid cell, including the value of the grid cell, for each time step |
| 15 | Upstream accumulated snow melt | kg m-2 | Total snow melt from areas upstream each grid cell, including the value of the grid cell, for each time step |

Product discipline 1 – Hydrological products, parameter category 2: inland water and sediment properties

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 13 | Cross sectional area of flow in channel | m2 | Channel cross sectional area is defined as the cross section area of the water flowing in the river channel (wet area). The channel cross section area multiplied by the mean velocity of the flow gives the discharge |

Product discipline 2 – Land surface products, parameter category 3: soil products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 24 | Frost index | K day-1 | When the soil surface is frozen, this affects the hydrological processes occurring near the soil surface. To estimate whether the soil surface is frozen or not, a frost index F is calculated. The equation is based on Molnau & Bissell (1983, cited in Maidment 1993), and adjusted for variable time steps. (*) |
| 25 | Snow depth at elevation bands | kg m-2 | Snow depth in water equivalent at elevation bands. The parameter needs to have several layers to represent elevation bands |

(*)	The rate at which the frost index changes is given by:

dF/dt = (1-Af)F-Tav•e(-0.04•K•ds/wes)

dF/dt is expressed in [K day-1 day-1 ]. Af is a decay coefficient [day-1], K is a a snow depth reduction coefficient [cm-1], ds is the (pixel-average) depth of the snow cover (expressed as mm equivalent water depth), and wes is a parameter called snow water equivalent, which is the equivalent water depth water of a snow cover (Maidment, 1993). The soil is considered frozen when the frost index rises above a critical threshold of 56. For each time step the value of F [K day-1] is updated as:

F(t) = F(t −1) + dF/dt•Δt

F is not allowed to become less than 0.

- ANNEX TO PARAGRAPH 2.2.11 [FT2015-2] <para 4.1>

Add entries:

In Code table 4.0:

| Octet No. | Contents |
| --- | --- |
| 55 | Spatio-temporal changing tiles at a horizontal level or horizontal layer at a point in time. |
| 56 | Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters. |

Add templates:

| Product definition template 4.55 - Spatio-temporal changing tiles at a horizontal level or horizontal layer at a point in time |  |
| --- | --- |
| Octet No. | Contents |
| 10 | Parameter category (see Code table 4.1) |
| 11 | Parameter number (see Code table 4.2) |
| 12 | Tile classification (see Code table 4.242) |
| 13 | Total number (NT) of tile / attribute pairs (see Note 2, 3) |
| 14 | Number of used spatial tiles (NUT) (see Note 2, 3) |
| 15 | Tile index (ITN ={1,…, NUT}) (see Note 2) |
| 16 | Number of used Tile attributes (NAT) for Tile ITN (see Note 2) |
| 17 | Attribute of Tile (see Code Table 4.241) ) (A ={A(1),…, A(NAT(ITN))}) (see Note 2) |
| 18 | Type of generating process (see Code table 4.3) |
| 19 | Background generating process identifier (defined by originating centre) |
| 20 | Analysis or forecast generating process identifier (defined by originating centre) |
| 21 – 22 | Hours of observational data cut-off after reference time (see Note) |
| 23 | Minutes of observational data cut-off after reference time |
| 24 | Indicator of unit of time range (see Code table 4.4) |
| 25 – 28 | Forecast time in units defined by octet 24 |
| 29 | Type of first fixed surface (see Code table 4.5) |
| 30 | Scale factor of first fixed surface |
| 31 – 34 | Scaled value of first fixed surface |
| 35 | Type of second fixed surface (see Code table 4.5) |
| 36 | Scale factor of second fixed surface |
| 37 – 40 | Scaled value of second fixed surface |

Notes:

- Hours greater than 65534 will be coded as 65534.

- The number of used Tiles (NUT) is the number of used different spatial tiles, defining the cover structure of a point. As each of these tiles have one or more different tile attributes A( NAT (ITN)), ( ITN=1,…,NUT), e.g. unmodified, snow-covered,…), there are NT=  fields (that is, the total number of tile/attribute pairs, defined in Octet 13) with indices( ITN, IAN )  with the following meaning (IAN = {1,…, NAT(ITN)}): 
1,1			First tile – first attribute (e.g. unmodified) 
 ….		….
1,NAT(1)		First tile – NAT of first tile (last, e.g. snow-covered)  attribute
2,1			Second tile – first attribute (e.g. unmodified)
….			….
2,NAT(2)		Second tile – NAT of second tile (last, e.g. snow-covered) attribute
.			.
.			. 
NUT,1		NUT tile – first attribute (e.g. unmodified)
….	...
NUT,NAT(NUT)	NUT tile – NAT of last tile (last) attribute

A single tile/attribute index (ITN, IAN) with spatial tile index ITN (1,…,NUT) and attribute A(IAN) with IAN=(1,…,NAT(ITN)) is represented in the template. All NT partitions are linked by the normalisation formula, which states that the sum of all partitions must be equal to a normalisation term (N=1 for fractions and N=100 for percentage) on each point of the grid.
The fields "tile class" and "tile fraction" must be provided in order to obtain the tile structure of each grid point. Please note  that the field “tile fraction” is time-dependent in the case of defined attributes, whereas the field “tile class” is not affected by attributes (NT=NUT).

- For more information, see Attachment IV (Spatio-temporal changing tiles in GRIB) in Part B of this volume (I.2 – Att.IV/GRIB 1 to x).

| # Product definition template 4.56 - Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters. |  |
| --- | --- |
| Octet No. | Contents |
| 10 | Parameter category (see Code table 4.1) |
| 11 | Parameter number (see Code table 4.2) |
| 12 | Tile classification (see Code table 4.242) |
| 13 | Total number (NT) of tile / attribute pairs (see Note 2, 3) |
| 14 | Number of used spatial tiles (NUT) (see Note 2, 3) |
| 15 | Tile index (ITN ={1,…, NUT}) (see Note 2) |
| 16 | Number of used Tile attributes (NAT) for Tile ITN (see Note 2) |
| 17 | Attribute of Tile (see Code Table 4.241) ) (A ={A(1),…, A(NAT(ITN))}) (see Note 2) |
| 18 | Type of generating process (see Code table 4.3) |
| 19 | Background generating process identifier (defined by originating centre) |
| 20 | Analysis or forecast generating process identifier (defined by originating centre) |
| 21 – 22 | Hours of observational data cut-off after reference time (see Note) |
| 23 | Minutes of observational data cut-off after reference time |
| 24 | Indicator of unit of time range (see Code table 4.4) |
| 25 – 28 | Forecast time in units defined by octet 24 |
| 29 | Type of first fixed surface (see Code table 4.5) |
| 30 | Scale factor of first fixed surface |
| 31 – 34 | Scaled value of first fixed surface |
| 35 | Type of second fixed surface (see Code table 4.5) |
| 36 | Scale factor of second fixed surface |
| 37 – 40 | Scaled value of second fixed surface |
| 41 | Perturbation number |
| 42 | Number of forecasts in ensemble |

Notes:

- Hours greater than 65534 will be coded as 65534.

- See note (2) under product definition template 4.55

- For more information, see Attachment IV (Spatio-temporal changing tiles in GRIB) in Part B of this volume (I.2 – Att.IV/GRIB 1 to x).

Add an attachment:

ATTACHMENT IV

Definition of  “tiles” with time-dependent attributes

How to code “tiles” with Templates 4.55 and 4.56

The land surface model is evolving and growing more complex. More complex descriptive capabilities are needed to properly describe the representation of land cover types in state of the art weather and climate models.

This includes the sub-grid scale tiling to represent surface heterogeneity. Each grid box with sub-grid variability is divided into a number of tiles, each representing a single surface type.

The use of Template   4.53 and 4.54 for partitioned parameters implies that for every chosen partition PN(1), PN(2), …, PN(NP) a GRIB message exists. All NP partitions are linked by the normalization formula.

The GRIB code representation of this tile approach takes into account the possibility to encode

- only the dominating tiles, which  could differ from grid box to grid box,

- tile attributes, considering  that tile fractions can be modified according to code table 4.241 (e.g. snow-covered)

Point (1) implies that every grid box has its own subset of tile classes from the land use table

Point (2) allows for the differentiation of tile attributes, the temporal component of this approach.

The fractions  of these N (dominant) classes and their attributes are subject to normalization formula:

In detail, the model grid box is regarded as consisting of a prescribed number of surface types (tiles).

The fractional area of each tile is either given by the geospatial surface data or by one or more prescribed tile attributes (e.g., snow-free and snow-covered). It is important to note that in contrast to the geospatial surface data, the tile attributes according to code table 4.241 could be time dependent.

Fig.1: Generation of the dominating tile structure for NUT=3 of a heterogeneous land surface. The outer circle shows the fractional areas covered by the respective land cover classes for a given grid cell. The inner circle shows the selected dominating tiles. Please note the rescaling of the fractional areas performed in the inner circle.

Given the number of land-use surface types from the geospatial land-use data table in a particular grid box, the approach recognizes the most dominant land-use surface types above a prescribed threshold fraction (e.g. 5%) up to the number of used tiles (NUT). Two model grid boxes always use the same number of tiles but could differ in the most dominant land-use surface types (see Fig. 1, outer circles). The fraction of resulting NUT is always rescaled to the total grid box area (see Fig. 1, inner circles).

For grid boxes with nearly homogeneous land-surface types the approach recognizes only the single dominant type and the fractional area of the other used tiles is considered as zero (see Fig. 2).

Fig. 2:  Generation of the dominating tile structure for NUT=3 of a nearly homogeneous land surface of a coastal region.  In this example, area fractions smaller than 5% are not considered when selecting the dominating tiles.

The tile attributes considered in this approach allow for a modification of the tile fractions, e.g. by a temporal evolution of the snow cover (see code table 4.241– tile attributes). Therefore, a subset of the land-use classes from the geospatial land-use data table can be considered for tile attributes.

Fig. 3:  Generation of the dominating tile structure for NUT=3 with tile attribute “snow-covered” of a heterogeneous land surface, partly covered with snow. Dominating tiles are equipped with two attributes where applicable: “snow-covered” and  ”unmodified”. Shaded areas show the snow-covered tile fractions.

The tiles used in a particular grid box which belong to the attribute are then divided into fractions of the attribute and fractions of the originating dominating tile (see Fig. 3).

- ANNEX TO PARAGRAPH 2.3.1

REQUIREMENTS

Some of the requirements are based on the limitations encountered in encoding the complex set of products generated by the NWP community in GRIB 2.

The most relevant limitations are

1.	The lack of flexibility due to the presence of time, parameter and statistical information in a single template.

2.	The absence of any template mechanism in some sections.

3.	Some level of ambiguity in the definition of some aspects of the encoding standard.

Other requirements are coming from the growing interest around NWP data from a wider community including not only scientific and technical organisations, also decision making bodies and general public. The main requirement in this context is an improved interoperability to reach those communities and may require a complete change in vocabulary, syntax and structure of the message.

The following classification of the requirements is based on the complexity of the changes required for their implementation.

- Requiring minor changes.

  - Ambiguities must be eliminated as much as possible

    - explaining the terms used in regulations and notes,

    - defining the numerical and algorithmic characteristics of the elements,

    - Providing best practices and examples for each regulation or note.

  - Regulations and notes must be clearly understandable, unambiguous and thoroughly explained by examples.

  - The signedness of each element in the message, in the main body of the sections or in the templates must be explicitly expressed in the manual.

  - A machine readable version of the tables must be available in a format suitable to build automatically the decoding rules into the decoding software. Web registries are considered the optimal solution to satisfy this requirement.

  - A machine readable version of the template must be provided to allow an automatic update of the decoding software.

- Requiring changes in structure.

  - A template must be introduced to define the time characteristics of the product represented in the message or a mechanism with equivalent flexibility must be provided.

  - The message structure of the new edition must be composed of single sections or templates or equivalent entities expressing at least the following elementary components of the product description: originator, time, horizontal grid, vertical coordinates, process, property or parameter, data representation.

  - Separation of metadata from data values must be always present also for multiple field messages. It must be possible for decoding software to list the content of the message without decoding the full data set.

  - Multiple, optional bitmaps to represent different types of missing data, e.g. instrumentation error, nothing recorded, not applicable etc. .

  - Multi-dimensional structure has to be developed with a proper template based approach avoiding the simple repetition of sections.

  - A way for a hybrid height field to refer to its orography reference field (and other such referential requirements). A unique way of referencing external fields.

  - Introduction in the message of a checksum element to validate the integrity of data.

- Requiring changes in vocabulary, syntax and structure.

  - Improve interoperability.

  - Harmonisation with the ISO 19156 Observations and Measurement.

  - Redefine the term GRID as more objects which cannot be classified with the classical term of GRID need to be represented in GRIB. The concept of sampling feature used in O&M could be applied in this context.

  - Terminology defining the new format has to be thoroughly reviewed to avoid semantic overloading and ensure clear understanding of the meaning of each term.

  - Ability to represent data for other planets and stars.

  - Harmonisation with BUFR in terms of vocabulary and semantics.

COST BENEFIT ANALYSIS

A cost/benefit analysis of the implementation of these requirements can be performed assuming that the main benefits of implementing a new GRIB edition are: increased flexibility and wider user community while the most important costs are due to the impact of the change on NWP operational systems and on current users’ software. In the following cost/benefit diagrams the categories of requirements presented above are represented in the context of their benefits and costs with the aim to prioritise the implementation of one group or requirements on another.

Flexibility

Impact on NWP and users systems

Low

Low

High

High

1

2

3

Wider user community

Impact on NWP and users systems

Low

Low

High

High

1

2

3

It is clear from the cost/benefit diagrams that the requirements in group 3 have a high associated cost and in terms of flexibility a moderate impact, while they have a very high impact in terms of reaching new users communities. As a consequence the implementation of a completely different vocabulary and syntax in the new GRIB edition to implement a higher level of interoperability is giving little benefits to the current user community with a very high cost on changes in the operational and research systems.

The benefits of implementing group 1 are low in both contexts and their cost is low as well, which makes this group not enough to trigger the development of a new GRIB edition, but is surely the group that we can implement if other requirements are giving enough motivation for a new edition.

Group 2 is providing considerable benefits for both aspects with moderate cost of implementation and can be considered the driver of the new edition implementation.

According to the cost/benefit analysis the priority for the implementation of the consolidated requirements is therefore on the first two groups while the implementation of group 3 is very expensive and provides only partial benefits, therefore some requirements of group 3 can be implemented only if they can be moved to group 2 to decrease the cost of implementation. It is therefore proposed to start with the implementation of group 1 and 2 requirements and keep requirements in group 3 in consideration only if we manage to implement them with a low cost.

METHODOLOGY AND STARTING STRUCTURE

With the aim to start the development process the following structure, already proposed at the IPET-DRC IV, is proposed as a starting point for an iterative development process. We highlight that this is only a starting point for the development of a new structure and a discussion is necessary within the TT-GRIB to consolidate the ideas behind the new sections proposed.

| Section 0 | Indicator Section | - GRIB (constant) - edition - length |
| --- | --- | --- |
| Section 1 | Originator Identification Section | - centre - project (optional) - Local Template |
| Section 2 | Time definition section | - Time Template defining time for the product, including statistics on time. |
| Section 3 | Horizontal grid definition section | - Horizontal Grid Template (including spectral fields) |
| Section 4 | Vertical coordinate definition section | - Vertical Coordinates Template (defining the vertical level or layer) |
| Section 5 | Generating Process definition section | - Process Template defining EPS, deterministic, multi-model, … (model, configuration, version) (originator if different from message originator) - Post-Processing template (clustering, probability, post-processing on the wave frequencies/directions) |
| Section 6 | Physical Quantity or Observable Property definition section | - Observable Property Template contains (discipline, category and parameter). Allows multi-dimensional and complex parameters definitions like for the wave model, chemicals and aerosols. |
| Section 7 | Data Representation Section | - Scanning Template - Pre-processing Template - Data Representation Template - Post-Processing Template |
| Section 8 | Bitmap section | - Binary bitmap |
| Section 9 | Data Section | - Binary data |
| Section 10 | Checksum Section | - Checksum Template |
| Section 11 | End Section |  |

An iterative approach based on the following steps has been already accepted as the methodology to develop the new GRIB edition:

1.	Proposal of a data structure based on the requirements and starting from the structure proposed in this document.

2.	Prototype of encoding/decoding software must be developed to test the proposed structure for performance and general encoding/decoding issues.

3.	Examples must be produced and examined to assess the effectiveness of the new structure.

4.	Iteration of the steps 1 to 3 until all requirements are satisfactory implemented.

SOFTWARE FOR NEW EDITION DEVELOPMENT

ECMWF has offered its new decoding/encoding software ecCodes to support the development of new GRIB and BUFR editions. ecCodes is released with Apache 2.0 license and a collaborative development platform is provided to facilitate the collaborative effort.

Software from other organisations provided for the same purpose with similar licensing scheme and with a collaborative development platform will be accepted for the development of new editions.

- ANNEX TO PARAGRAPH 3.1.1 [Validation]

In the Code form,

Notes:

(10)	Position can only be unambiguously interpreted if the coordinate reference system and, if required, fixed reference mean sea level, to which it is attributed, is known. If opposite is not specified it is assumed that position shall be interpreted with respect to WGS84 geodetic system and Earth Geodetic Model EGM96.

In BUFR Table B,

Class 01 – BUFR/CREX Identification

| TABLE REFERENCE |  |  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y | ELEMENT NAME | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char) |
| 0 | 01 | 150 | Coordinate reference system | Code table | 0 | 0 | 16 | Code table | 0 | 5 |
| 0 | 01 | 151 | Fixed mean sea level reference datum | Code table | 0 | 0 | 11 | Code table | 0 | 4 |

0 01 150   Coordinate reference system

| Code figure |  |
| --- | --- |
| 0 | WGS84, as used by ICAO since 1998 |
| 1 | ETRS89, as defined by EPSG::4258 |
| 2 | NAD83, as defined by EPSG::4269 |
| 3 | DHDN, as defined by EPSG::4314 |
| 4 - 65534 | Reserved |
| 65535 | Missing value |

Note:	EPSG is a dataset of coordinate system and coordinate system transformations, originally produced and maintained by the European Petroleum Survey Group. Now it is maintained by the Geodesy Subcommittee of the International Association of Oil and Gas Producers Geomatics Committee.

0 01 151   Fixed mean sea level reference datum

Not available now

- ANNEX TO PARAGRAPH 3.2.1 [FT2015-2]

Add entries:

In Code table 0 08 040 (Flight level significance)

38	Top of wind sounding

39	Bottom of wind sounding

- ANNEX TO PARAGRAPH 3.2.2 [FT2015-2]

In BUFR/CREX table B,

Class 08 – BUFR/CREX Significance qualifiers

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 08 091 | Coordinates significance | Code table | 0 | 0 | 8 | Code table | 0 | 3 |

Class 05 - BUFR/CREX Location (horizontal)

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 05 063 | Spacecraft Roll | deg | 2 | 0 | 16 | deg | 2 | 5 |
| 0 05 064 | Spacecraft Pitch | deg | 2 | 0 | 16 | deg | 2 | 5 |
| 0 05 066 | Spacecraft Yaw | deg | 2 | 0 | 16 | deg | 2 | 5 |
| 0 05 067 | Number of scan lines | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |

Class 40 – BUFR/CREX Satellite data

| TABLE REFERENCE | ELEMENT NAME |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 40 027 |  | Sun glint angle | deg | 2 | -18000 | 16 | deg | 2 | 5 |
| 0 40 028 |  | GMI quality flag | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

In BUFR table D,

| TABLE REFERENCE | TABLE REFERENCES | Element name | Element description |
| --- | --- | --- | --- |
| 3 40 012 | GPM Microwave Imager (GMI) |  |  |
|  | 0 01 007 | Satellite identifier |  |
|  | 0 02 019 | Satellite instruments |  |
|  | 0 08 091 | Coordinates significance | 0 -> Satellite coordinates |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 07 002 | Height or altitude |  |
|  | 0 05 063 | Spacecraft Roll |  |
|  | 0 05 064 | Spacecraft Pitch |  |
|  | 0 05 066 | Spacecraft Yaw |  |
|  | 0 05 041 | Scan line number |  |
|  | 0 05 067 | Number of scan lines |  |
|  | 3 01 011 | Year, moth, day |  |
|  | 3 01 012 | Hour, minute |  |
|  | 0 04 007 | Seconds within a minute (microsecond accuracy) |  |
|  | 0 08 091 | Coordinates significance | 1 -> Observation coordinates |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 1 07 000 | Delayed replication of 6 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 05 042 | Channel number |  |
|  | 0 02 153 | Satellite channel centre frequency |  |
|  | 0 02 104 | Antenna polarization |  |
|  | 0 40 028 | GMI quality flag |  |
|  | 0 07 024 | Satellite zenith angle |  |
|  | 0 40 027 | Sun glint angle |  |
|  | 0 12 063 | Brightness temperature |  |

Code table 0 08 091 – Coordinates significance

| Code figure |  |
| --- | --- |
| 0 | Satellite coordinates |
| 1 | Observations coordinates |
| 2 | Start of observation |
| 3 | End of observation |
| 4 | Horizontal centre of gravity of the observation |
| 5 | Vertical centre of gravity of the observation |
| 6 | Top of the observation |
| 7 | Bottom of the observation |
| 8-254 | Reserved |
| 255 | Missing value |

Code table 0 40 028 – GMI quality flag

| Code figure |  |
| --- | --- |
| 0 | Good data |
| 1 | Possible sun glint |
| 2 | Possible radio frequency interference |
| 3 | Degraded geolocation data |
| 4 | Data corrected for warm load intrusion |
| 5 | Scan blanking on |
| 6 | Data is missing from file or unreadable |
| 7 | Unphysical brightness temperature |
| 8 | Error in geolocation data |
| 9 | Data missing in one channel |
| 10 | Data missing in multiple channels |
| 11 | Lat/lon values are out of range |
| 12 | Non-normal status modes |
| 13 | Distance to corresponding lf pixel > 7 km |
| 14 | Reserved |
| 15 | Missing value (no quality information available) |

- ANNEX TO PARAGRAPH 3.2.3 [FT2015-2]

Add an entry:

In Common Code table C5, Satellite identifier, 200–299: Numbers allocated to the United States

| Code figure for I6I6I6 | #### Code figure for BUFR (Code table 0 01 007) | #### Code figure for GRIB Edition 2 |  |
| --- | --- | --- | --- |
| 262 | #### 262 | #### 262 | #### JASON 3 |

- ANNEX TO PARAGRAPH 3.2.4 [FT2016-1]

In BUFR/CREX table B,

Class 05 - BUFR/CREX Location (horizontal)

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 05 068 | Profile number | Numeric | 0 | 0 | 32 | Numeric | 0 | 10 |
| 0 05 069 | Observation identifier | Numeric | 0 | 0 | 32 | Numeric | 0 | 10 |
| 0 05 070 | Receiver channel | Code table | 0 | 0 | 2 | Code table | 0 | 1 |

Class 07 - BUFR/CREX Location (vertical)

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 07 071 | Height (high resolution) | m | 3 | -10000000 | 26 | m | 3 | 8 |

Class 08 – BUFR/CREX Significance qualifiers

| TABLE REFERENCE | ELEMENT NAME |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 08 091 |  | Coordinates significance | Code table | 0 | 0 | 8 | Code table | 0 | 3 |

Editorial note: This is operational as of 11 November 2015 (FT2015-2).

Class 25 - BUFR/CREX Processing information

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 25 187 | Confidence flag | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

Class 40 – BUFR/CREX Satellite data

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 40 029 | Horizontal observation integration length | m | 0 | 0 | 26 | m | 0 | 8 |
| 0 40 030 | Horizontal line of sight wind | m s-1 | 2 | -32767 | 16 | m s-1 | 2 | 5 |
| 0 40 031 | Error estimate of horizontal line of sight wind | m s-1 | 2 | 0 | 15 | m s-1 | 2 | 5 |
| 0 40 032 | Derivative wind to pressure | m s-1 Pa-1 | 3 | -100000 | 18 | m s-1 Pa-1 | 3 | 6 |
| 0 40 033 | Derivative wind to temperature | m s-1 K-1 | 3 | -100000 | 18 | m s-1 K-1 | 3 | 6 |
| 0 40 034 | Derivative wind to backscatter ratio | m s-1 | 3 | -200000 | 19 | m s-1 | 3 | 6 |
| 0 40 035 | Satellite range | m | 0 | 380000 | 18 | m | 0 | 6 |
| 0 40 036 | Lidar classification type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 40 037 | Backscatter ratio | Numeric | 3 | 500 | 20 | Numeric | 3 | 7 |

In BUFR Table D,

| TABLE REFERENCE | TABLE REFERENCES | Element name |  |
| --- | --- | --- | --- |
| 3 40 013 | (NAME) |  |  |
|  | 0 01 007 | Satellite identifier |  |
|  | 0 02 019 | Satellite instruments |  |
|  | 0 01 033 | Identification of originating/generating centre |  |
|  | 0 01 034 | Identification of originating/generating sub-centre |  |
|  | 0 04 001 | Year |  |
|  | 0 04 002 | Month |  |
|  | 0 04 003 | Day |  |
|  | 0 04 004 | Hour |  |
|  | 0 04 005 | Minute |  |
|  | 0 04 007 | Seconds within a minute (microsecond accuracy) |  |
|  | 0 05 068 | Profile number |  |
|  | 0 05 069 | Observation identifier |  |
|  | 0 05 070 | Receiver channel |  |
|  | 0 40 036 | Lidar l2b classification type |  |
|  | 0 08 091 | Coordinates significance | 2 -> Start of observation |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 04 016 | Time increment |  |
|  | 0 08 091 | Coordinates significance | 3 -> End of observation |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 04 016 | Time increment |  |
|  | 0 08 091 | Coordinates significance | 4 -> Centre of gravity of observation |
|  | 0 05 001 | Latitude (high accuracy) |  |
|  | 0 06 001 | Longitude (high accuracy) |  |
|  | 0 04 016 | Time increment |  |
|  | 0 08 091 | Coordinates significance | 5 -> Top of observation |
|  | 0 07 070 | Height (high resolution) |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 07 021 | Elevation |  |
|  | 0 40 035 | Satellite range |  |
|  | 0 08 091 | Coordinates significance | 6 -> Bottom of observation |
|  | 0 07 070 | Height (high resolution) |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 07 021 | Elevation |  |
|  | 0 40 035 | Satellite range |  |
|  | 0 08 091 | Coordinates significance | 7 -> Vertical centre of Gravity of the observation |
|  | 0 07 070 | Height (high resolution) |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 07 021 | Elevation |  |
|  | 0 40 035 | Satellite range |  |
|  | 0 40 029 | Horizontal observation integration length |  |
|  | 0 40 030 | Horizontal line of sight wind |  |
|  | 0 40 031 | Error estimate of horizontal line of sight wind |  |
|  | 0 25 187 | Confidence flag |  |
|  | 0 10 004 | Pressure |  |
|  | 0 12 001 | Temperature/air temperature |  |
|  | 0 40 037 | Backscatter ratio |  |
|  | 0 40 032 | Derivative wind to pressure |  |
|  | 0 40 033 | Derivative wind to temperature |  |
|  | 0 40 034 | Derivative wind to backscatter ratio |  |

Code table 0 08 091 – Coordinates significance

| Code figure |  |
| --- | --- |
| 0 | Satellite coordinates |
| 1 | Observations coordinates |
| 2 | Start of observation |
| 3 | End of observation |
| 4 | Horizontal centre of gravity of the observation |
| 5 | Top of the observation |
| 6 | Bottom of the observation |
| 7 | Vertical centre of gravity of the observation |
| 8-254 | Reserved |
| 255 | Missing value |

Editorial note: 0 08 091 and the associated code table are seen in 2015-3.2.2 (DRMM-III).  This is operational as of 11 November 2015 (FT2015-2).

Code table 0 05 070 – Receiver channel

| Code figure |  |
| --- | --- |
| 0 | Mie |
| 1 | Rayleigh |
| 2 | Reserved |
| 3 | Missing |

Code table 0 25 187 – Confidence flag

| Code figure |  |
| --- | --- |
| 0 | Valid |
| 1 | Invalid |
| 2-14 | Reserved |
| 15 | Missing |

Code table 0 40 036 – Lidar classification type

| Code figure |  |
| --- | --- |
| 0 | Clear |
| 1 | Cloud |
| 2-14 | Reserved |
| 15 | Missing |

- ANNEX TO PARAGRAPH 3.2.6 [FT2016-1] <para 4.1>

Add entries:

In BUFR table B,

| Table Reference |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| F X Y | Element name | Unit | Scale | Reference | Data width |
| 0 22 102 | Scaled maximum non-directional spectral wave density by frequency (Note 1) | m2 s | 0 | 0 | 14 |
| 0 22 103 | Scaled maximum non-directional spectral wave density by wavenumber (Note 1) | m3 | 0 | 0 | 14 |
| 0 22 104 | Scaled non-directional spectral wave density by frequency (Note 1) | m2 s | 0 | 0 | 14 |
| 0 22 105 | Scaled non-directional spectral wave density by wavenumber (Note 1) | m3 | 0 | 0 | 14 |
| 0 22 106 | Scaled directional spectral wave density by frequency (Note 1) | m2 s rad-1 | 0 | 0 | 14 |
| 0 22 107 | Scaled directional spectral wave density by wavenumber (Note 1) | m4 | 0 | 0 | 14 |

Editorial note: CREX part should be included.

Notes

- Must be preceded by 0 08 090, possibly with intervening operators. The value is 10x multiplied by the encoded value, where x is the value associated with the preceding 0 08 090 descriptor. The encoded value is the actual value multiplied by 10-x.

With the above values, the representable range of values for the scale is -127 to +127 and the representable values for the spectral energy densities range from 0 with a precision of 10-127 through to (214-2)*10127 (1.6382*10131) with a precision of 10127 (approximately 4 decimal digits), far in excess of requirements. Of course other values could be chosen to reduce BUFR message size, including a narrower width version of 0 08 090. Reducing the decimal scale to 4 bits with a reference of -7 would give 0 with a precision of 10-7 through 1.6382*1011 with a precision of 107 (still approximately 4 decimal digits), which would still be adequate. Reducing the data width for the spectral energy densities would reduce the precision below 4 decimal digits. We believe that each use of 0 08 090 will have to be followed (after the affected descriptors) with another use with the scale set to zero or missing as class 8 descriptors remain in effect until reset.

Add entries:

In Table D,

| Table reference |  |  |  |
| --- | --- | --- | --- |
| F X Y | Table reference | Element name | Note or WAVEOB ref |
|  |  | Identification (WAVEOB Section 0) |  |
| 3 08 015 | 0 01 003 | WMO Region number/geographical area | A1 |
|  | 0 01 020 | WMO Region sub-area | bw |
|  | 0 01 005 | Buoy/platform identifier | nbnbnb |
|  | 0 01 011 | Ship or mobile land station identifier | D…D |
|  | 0 01 007 | Satellite identifier | I6I6I6 |
|  | 0 01 001 | WMO block number | II |
|  | 0 01 002 | WMO station number | Iii |
|  | 0 02 044 | Indicator for method of calculating spectrum | Im |
|  | 0 02 045 | Indicator for type of platform | Ip |
|  | 3 01 011 | Year, month, day | JMMYY |
|  | 3 01 012 | Hour, minute | GGgg |
|  | 3 01 021 | Latitude/longitude (high accuracy) | QcLaLaLaLa LoLoLoLoLo |
|  |  | Basic data (WAVEOB Section 0) |  |
|  | 0 22 063 | Total water depth | 1hhhh |
|  | 0 22 076 | Direction of coming dominant waves | 9dddd |
|  | 0 22 077 | Directional spread of dominant wave | dsds |
|  | 0 22 094 | Total number of wave bands | 111BTBT |
|  | 0 25 044 | Wave sampling interval | SSSS |
|  | 0 22 079 | Length of wave record | D’D’D’D’ |
|  | 1 07 002 | Replicate 5 descriptors 2 times | Over sensor type |
|  | 0 02 046 | Wave measurement instrumentation |  |
|  | 0 22 070 | Significant wave height | 2Hs HsHsHs or 6HseHseHseHse |
|  | 0 22 072 | Spectral peak wave length | 3PpPpPpPp or 7PspPspPspPsp |
|  | 0 22 073 | Maximum wave height | 4HmHmHmHm |
|  | 0 22 075 | Average wave period | 5PaPaPaPa or 8PsaPsaPsaPsa |
|  |  | Spectral data (WAVEOB Sections 1-5) |  |
|  | 1 24 000 | Delayed replication of 24 descriptors | Note 1 |
|  | 0 31 001 | Replication factor | 0, 1 or 2; normally 1 |
|  | 0 02 046 | Wave measurement instrumentation | 2222 or 3333 |
|  | 0 08 090 | Scale to be applied to following element descriptors | x |
|  | 0 22 102 | Maximum non-directional spectral wave density by frequency m2 s | x CmCmCmCm or x CsmCsmCsmCsm |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 084 | Band containing maximum non-directional spectral wave density | nmnm or nsmnsm |
|  | 1 18 000 | Delayed replication of 18 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor | 111BTBT |
|  | 0 22 080 | Waveband central frequency | BB/// 1f1f1f1x 1fdfdfdx… |
|  | 0 22 085 | Spectral wave density ratio | 1c1c1c2c2 or 1cs1cs1cs2cs2 … |
|  | 0 02 086 | Mean direction from which waves are coming | 1da1da1 … |
|  | 0 02 087 | Principal direction from which waves are coming | da2da2… |
|  | 0 22 088 | First normalized polar coordinate from Fourier coefficients | 1r1r1 … |
|  | 0 22 089 | Second normalized polar coordinate from Fourier coefficients | r2r2… |
|  | 1 04 000 | Delayed replication of 4 descriptors | Note 2 |
|  | 0 31 001 | Delayed descriptor replication factor | Ib Note 2 |
|  | 0 08 090 | Scale to be applied to following element descriptors | x |
|  | 0 22 104 | Non-directional spectral estimate by frequency | 1A1A1A1x … |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 186 | Direction from which waves are coming | Note 3 |
|  | 0 22 187 | Directional spread of wave | Note 3 |
|  | 1 04 000 | Delayed replication of 4 descriptors | Note 4 |
|  | 0 31 001 | Delayed descriptor replication factor | Ib Note 4 |
|  | 0 08 090 | Scale to be applied to following element descriptors | x |
|  | 0 22 106 | Directional spectral estimate by frequency | 1A1A1A1x … |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 186 | Direction from which waves are coming | 1d1d1 … |
|  | 0 22 187 | Directional spread of wave | dsds … |

Notes:

- Normally 1, may be 2 if both heave and slope sensors are in use, or 0 if no spectral data.

- Non-directional spectra, (Ib=0 in WAVEOB) or partial directional spectra (Ib=1 in WAVEOB with one direction per wavenumber). Count=0 (full directional spectra) or 1 (non-directional spectra or partial directional spectra). Partial directional spectra have only one direction per wavenumber band.

- Missing for non-directional spectra.

- Full directional spectra (Ib=1 in WAVEOB with more than one direction per wavenumber band). The replication count is the number of directions per wavenumber band which should normally cover the full circle.

| Table reference |  |  |  |
| --- | --- | --- | --- |
| F X Y | Table reference | Element name | Note or WAVEOB ref |
|  |  | Identification (WAVEOB Section 0) |  |
| 3 08 016 | 0 01 003 | WMO Region number/geographical area | A1 |
|  | 0 01 020 | WMO Region sub-area | bw |
|  | 0 01 005 | Buoy/platform identifier | nbnbnb |
|  | 0 01 011 | Ship or mobile land station identifier | D…D |
|  | 0 01 007 | Satellite identifier | I6I6I6 |
|  | 0 01 001 | WMO block number | II |
|  | 0 01 002 | WMO station number | iii |
|  | 0 02 044 | Indicator for method of calculating spectrum | Im |
|  | 0 02 045 | Indicator for type of platform | Ip |
|  | 3 01 011 | Year, month, day | JMMYY |
|  | 3 01 012 | Hour, minute | GGgg |
|  | 3 01 021 | Latitude/longitude (high accuracy) | QcLaLaLaLa LoLoLoLoLo |
|  |  | Basic data (WAVEOB Section 0) |  |
|  | 0 22 063 | Total water depth | 1hhhh |
|  | 0 22 076 | Direction of coming dominant waves | 9dddd |
|  | 0 22 077 | Directional spread of dominant wave | dsds |
|  | 0 22 094 | Total number of wave bands | 111BTBT |
|  | 0 25 044 | Wave sampling interval | SSSS |
|  | 0 22 079 | Length of wave record | D’D’D’D’ |
|  | 1 07 002 | Replicate 5 descriptors 2 times | Over sensor type |
|  | 0 02 046 | Wave measurement instrumentation |  |
|  | 0 22 070 | Significant wave height | 2Hs HsHsHs or 6HseHseHseHse |
|  | 0 22 072 | Spectral peak wave length | 3PpPpPpPp or 7PspPspPspPsp |
|  | 0 22 073 | Maximum wave height | 4HmHmHmHm |
|  | 0 22 075 | Average wave period | 5PaPaPaPa or 8PsaPsaPsaPsa |
|  |  | Spectral data (WAVEOB Sections 1-5) |  |
|  | 1 24 000 | Delayed replication of 24 descriptors | Note 1 |
|  | 0 31 001 | Replication factor | 0, 1 or 2; normally 1 |
|  | 0 02 046 | Wave measurement instrumentation | 2222 or 3333 |
|  | 0 08 090 | Scale to be applied to following element descriptors | x |
|  | 0 22 103 | Maximum non-directional spectral wave density by wavenumber | x CmCmCmCm or x CsmCsmCsmCsm |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 084 | Band containing maximum non-directional spectral wave density | nmnm or nsmnsm |
|  | 1 18 000 | Delayed replication of 18 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor | 111BTBT |
|  | 0 22 081 | Waveband central wave number | BB/// 1f1f1f1x 1fdfdfdx… |
|  | 0 22 085 | Spectral wave density ratio | 1c1c1c2c2 or 1cs1cs1cs2cs2 … |
|  | 0 02 086 | Mean direction from which waves are coming | 1da1da1 … |
|  | 0 02 087 | Principal direction from which waves are coming | da2da2… |
|  | 0 22 088 | First normalized polar coordinate from Fourier coefficients | 1r1r1 … |
|  | 0 22 089 | Second normalized polar coordinate from Fourier coefficients | r2r2… |
|  | 1 03 000 | Delayed replication of 5 descriptors | Note 2 |
|  | 0 31 001 | Delayed descriptor replication factor | Ib Note 2 |
|  | 0 08 090 | Scale to be applied to next element descriptor | x |
|  | 0 22 105 | Non-directional spectral estimate by wave number | 1A1A1A1x … |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 186 | Direction from which waves are coming | Note 3 |
|  | 0 22 187 | Directional spread of wave | Note 3 |
|  | 1 03 000 | Delayed replication of 5 descriptors | Note 4 |
|  | 0 31 001 | Delayed descriptor replication factor | Ib Note 4 |
|  | 0 08 090 | Scale to be applied to next element descriptor | x |
|  | 0 22 107 | Directional spectral estimate by wave number | 1A1A1A1x … |
|  | 0 08 090 | Scale to be applied to following descriptors | Missing |
|  | 0 22 186 | Direction from which waves are coming | 1d1d1 … |
|  | 0 22 187 | Directional spread of wave | dsds … |

Editorial note: Names should be consistent with the table B descriptors.

Notes:

Refer to notes for template 3 08 015.

- ANNEX TO PARAGRAPH 3.2.7 [FT2015-2]

Add entries:

In BUFR/CREX Table B, class 33 (quality information)

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| 0 33 089 | Noise equivalent delta temperature (NEdT) quality indicators for warm target calibration | K | 2 | 0 | 12 | K | 2 | 4 |
| 0 33 090 | Noise equivalent delta temperature (NEdT) quality indicators for cold target calibration | K | 2 | 0 | 12 | K | 2 | 4 |
| 0 33 091 | Noise equivalent delta temperature (NEdT) quality indicators for overall calibration | K | 2 | 0 | 12 | K | 2 | 4 |

- ANNEX TO PARAGRAPH 3.2.8 [Validation]

Class 29 – BUFR/CREX Map data

| TABLE REFERENCE |  |  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y | ELEMENT NAME | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Char) |
| 0 | 29 | 10 | Semi-major axis of rotation ellipsoid | m | 2 | 0 | 33 | m | 0 | 11 |
| 0 | 29 | 11 | Semi-minor axis of rotation ellipsoid | m | 2 | 0 | 33 | m | 0 | 11 |
| 0 | 29 | 12 | Longitude origin | degree | 5 | -18000000 | 26 | degree | 5 | 10 |
| 0 | 29 | 13 | Latitude origin | degree | 5 | -9000000 | 25 | degree | 5 | 9 |
| 0 | 29 | 14 | Latitude of true scale | degree | 5 | -9000000 | 25 | degree | 5 | 9 |
| 0 | 29 | 15 | X-offset (see Note 1) | m | 2 | -3355443200 | 33 | m | 0 | 12 |
| 0 | 29 | 16 | Y-offset (see Note 1) | m | 2 | -3355443200 | 33 | m | 0 | 12 |
| 0 | 29 | 17 | Geographic projection as projection initialisation string | CCITT IA5 | 0 | 0 | 800 | Character | 0 | 100 |

Add Notes under Class 29 – BUFR/CREX Map data:

- X-Offset and Y-Offset are distances between the projection origin and the upper left corner of the upper left pixel in the map as explained in the following drawing:

http://www.eumetnet.eu/sites/default/files/bufr_sw_desc.pdf

- ANNEX TO PARAGRAPH 3.2.9 [FT2015-2]

Add an entry:

In BUFR descriptor 0 08 010 (Surface qualifier (for temperature data))

| Code figure | Meaning |
| --- | --- |
| 12 | Building roof |

- ANNEX TO PARAGRAPH 3.2.10 [FT2015-2]

Add entries:

in Common Code table C-8: Satellite instruments (also for 0-02-019)

Code	Agency	Type	Instrument short name     Instrument long name

958	CMA	GNSS       	GNOS	Global Navigation Satellite System

occultation		Occultation Sounder

sounders

in Code table for BUFR descriptor 0-02-020 (Satellite classification)

Code figure	Meaning

404	              BDS (BeiDou Navigation Satellite System)

- ANNEX TO PARAGRAPH 3.2.11 [FT2015-2]

- New entry in code table for BUFR descriptor 0-02-066 (Radiosonde ground receiving system)

| Code figure | Meaning |
| --- | --- |
| 2 | ShangHai GTC1 |
| 3 | NanJing GTC2 |
| 4 | NanJing GFE(L)1 |
| 5 | MARL-A radar |
| 6 | VEKTOR-M radar |

- New entry in code table for BUFR descriptor 0-02-097 (type of humidity sensor)

| Code figure | Meaning |
| --- | --- |
| 9 | Carbon hygristor |

- New entry in code table for BUFR descriptor 0-35-035(Reason for termination)

| Code figure | Meaning |
| --- | --- |
| 14 | Sudden lost of signal |
| 15 | Tracking lost |

- New entry in flag table for BUFR descriptor 0-08-042(Extended vertical sounding significance)

| Bit | Meaning |
| --- | --- |
| 16 | Freezing level |

Note:

(1)	Freezing level is the level which temperature first decreases to 0°C. The criteria for the selection of freezing level in Upper Air Observations are:

(a)	If the surface temperature is not lower than 0°C when release the radiosonde, then the lowest level which the temperature first decreases to 0°C will be selected as the freezing level.

(b)	If the surface temperature is equal to 0°C, then the surface level will be the freezing level.

(c)	If there is no level which the temperature just equal to 0°C in observation, using closest 2 levels with temperature cross from positive to negative to interpolate the freezing level which temperature equal to 0°C.

(d)	The following elements will be calculated for the freezing level: time, elevation, pressure, humidity, dewpoint temperature, dewpoint depression, deviation of longitude and latitude.

- ANNEX TO PARAGRAPH 3.2.12 [FT2015-2] <para 4.1>

Add an entry:

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION |
| --- | --- | --- | --- |
| F X Y |  |  |  |
|  |  | (Additional synoptical parameters) |  |
| 3 02 067 | 0 01 023 | Observation sequence number |  |
|  |  | Additional “instantaneous” data |  |
|  | 0 04 025 | Time period or displacement | = 0 minutes |
|  | 0 02 177 | Method of snow depth measurement | = 0 Manual observation, = 1 Ultrasonic method, = 2 Video camera method, = 3-13 Reserved, = 14 Others, = 15 Missing value |
|  |  | Additional present weather |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 003 | Present weather | 960ww, 961ww |
|  |  | Visibility in different directions |  |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 05 021 | Bearing or azimuth | 981VV-988VV |
|  | 0 20 001 | Horizontal visibility | VV |
|  | 0 05 021 | Bearing or azimuth | Set to missing (cancel) |
|  |  | Sea data (observations from a coastal station) |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 056 | Sea/water temperature | Sea surface temperature, method of measurement, and depth below sea surface |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 0 33 041 | Attribute of following value |  |
|  | 0 20 058 | Visibility seawards from a coastal station | 980VsVs |
|  | 0 22 061 | State of the sea | 924SVs |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 022 | Wind waves |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 3 02 023 | Swell waves |  |
|  |  | Clouds |  |
|  | 1 04 000 | Delayed replication of 4 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed | Da, Dp |
|  | 0 20 137 | Evolution of clouds | 940Cn3 |
|  | 0 20 012 | Cloud type | 941CDp, 943CLDp |
|  | 0 20 090 | Special clouds | 993CsDa |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 054 | True direction from which a phenomenon or |  |
|  |  | clouds are moving or in which they are observed |  |
|  | 0 20 137 | Evolution of clouds |  |
|  | 0 20 136 | Supplementary cloud type | 948C0Da, 949CaDa, 950Nmn3, 951Nvn4 |
|  |  | Additional "period" data |  |
|  | 0 04 025 | Time period or displacement | Reference period of fresh fallen snow |
|  | 0 13 012 | Depth of fresh snow |  |
|  |  | Additional wind data |  |
|  | 0 04 025 | Time period or displacement | = -60 minutes |
|  | 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff 902tt 912ff .. mandatory ff>=18 |
|  | 1 04 000 | Delayed replication of 4 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | = 30 Time of occurrence, = 17 Start of phenomenon |
|  | 0 04 025 | Time period or displacement | = -xx ¦ 902tt |
|  | 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff |
|  | 0 08 021 | Time significance | Set to missing (cancel) |
|  |  | Significant change in wind speed and/or direction |  |
|  | 1 15 000 | Delayed replication of 15 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | = 30 Time of occurrence, |
|  |  |  | = 17 Start of phenomenon |
|  | 0 04 015 | Time increment | = -xx1 |
|  | 0 08 021 | Time significance | = 2 Time averaged |
|  | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |
|  | 0 11 001 | Wind direction | 915dd |
|  | 0 11 002 | Wind speed | 913ff |
|  | 0 08 021 | Time significance | = 22 Time of occurrence of wind shift |
|  | 0 04 015 | Time increment | = +xx2 |
|  | 0 08 021 | Time significance | = 2 Time averaged |
|  | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |
|  | 0 11 001 | Wind direction | 915dd |
|  | 0 11 002 | Wind speed | 913ff |
|  | 0 08 021 | Time significance | Set to missing (cancel) |
|  | 0 04 025 | Time period or displacement | = 0 minutes |
|  | 0 04 015 | Time increment | = +(xx1-xx2) ¦ Non negative to reset the time to the actual time |
|  |  | Additional weather |  |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 025 | Time period or displacement | = -xx, i.e. from |
|  | 0 04 025 | Time period or displacement | = -xx, i.e. to |
|  | 0 20 003 | Present weather | 962ww, 963w1w1, 964ww, 965w1w1, 966ww, 967w1w1 |
|  |  | Additional 9SpSpspsp groups |  |
|  | 1 10 000 | Delayed replication of 10 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 025 | Time period or displacement | = -xx, i.e. from |
|  | 0 04 025 | Time period or displacement | = -xx, i.e. to |
|  | 0 05 021 | Bearing or azimuth | Da, Dp |
|  | 0 05 021 | Bearing or azimuth | Da, Dp |
|  | 0 20 054 | True direction from which a phenomenon or clouds are moving | Da, Dp |
|  | 0 20 024 | Intensity of phenomena | = 1 Light, = 2 Moderate = 3 Heavy = 4 Violent = 5 Severe |
|  | 0 20 025 | Obscuration |  |
|  | 0 20 026 | Character of obscuration |  |
|  | 0 20 027 | Phenomena occurrence |  |
|  | 0 20 063 | Special phenomena |  |

- ANNEX TO PARAGRAPH 3.2.13 [FT2016-1]

The following new BUFR table B entries and corresponding code tables are proposed:

| TABLE REFERENCE | ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH |
| 0 01 104 | State / federal state identifier | CCITT IA5 | 0 | 0 | 32 | CCITT IA5 | 0 | 4 |
| 0 01 105 | Highway designator | CCITT IA5 | 0 | 0 | 40 | CCITT IA5 | 0 | 5 |
| 0 01 106 | Location along highway as indicated by position markers | m | -2 | 0 | 14 | m | -2 | 5 |
| 0 03 016 | Position of road sensors | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 017 | Extended type of station | Flag table | 0 | 0 | 6 | Flag table | 0 | 2 |
| 0 03 018 | Type of road | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 03 019 | Type of construction | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 12 128 | Road surface temperature | K | 2 | 0 | 16 | C | 2 | 5 |
| 0 12 129 | Road sub-surface temperature | K | 2 | 0 | 16 | C | 2 | 5 |
| 0 13 116 | Water film thickness | m | 3 | 0 | 7 | m | 3 | 2 |
| 0 20 138 | Road surface condition | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

Code and Flag-tables:

0 03 016   Position of road sensors

| Code figure |  |
| --- | --- |
| 0 | Fast lane between the wheel tracks |
| 1 | Fast lane between the wheel tracks in the opposite direction |
| 2 | Fast lane in the wheel tracks |
| 3 | Fast lane in the wheel tracks in the opposite direction |
| 4 | Slow lane between the wheel tracks |
| 5 | Slow lane between the wheel tracks in the opposite direction |
| 6 | Slow lane in the wheel tracks |
| 7 | Slow lane in the wheel tracks in the opposite direction |
| 8 - 14 | Reserved |
| 15 | Missing value |

0 03 017   Extended type of station

| Bit No. |  |
| --- | --- |
| 1 | Automatic |
| 2 | Manned |
| 3 | Event triggered |
| 4 | Longer time period than the standard |
| 5 | Reserved |
| all 6 | Missing value |

0 03 018   Type of road

| Code figure |  |
| --- | --- |
| 0 | Free track without further information |
| 1 | Free track, embankment |
| 2 | Free track, flat relative to surroundings |
| 3 | Free track, water basin(s) in vicinity |
| 4 | Free track, forest |
| 5 | Free track, cleft |
| 6 | Free track, on hilltop |
| 7 | Free track, on hilltop, forest |
| 8 | Free track, in valley |
| 9 | Free track, in valley, forest |
| 10 | Free track, north inclination |
| 11 | Free track, north inclination, forest |
| 12 | Free track, south inclination |
| 13 | Free track, south inclination, forest |
| 14-19 | Reserved |
| 20 | Bridge without further information |
| 21 | Bridge across a valley in a urban area |
| 22 | Bridge across a valley with forest/meadows/fields |
| 23 | Bridge across street/track |
| 24 | Bridge across big river/canal |
| 25 | Bridge across river/canal of medium size |
| 26 | Bridge across a small stream/loading canal |
| 27-30 | Reserved |
| 31 | Missing value |

0 03 019   Type of construction

| Code figure |  |
| --- | --- |
| 0 | Asphalt |
| 1 | Concrete |
| 2 | Concrete construction |
| 3 | Steel-girder construction |
| 4 | Box girder bridge |
| 5 | Orthotrope slab |
| 6 | Drain asphalt |
| 7-14 | Reserved |
| 15 | Missing value |

0 20 138   Road surface condition

| Code figure |  |
| --- | --- |
| 0 | Dry |
| 1 | Moist |
| 2 | Wet |
| 3 | Rime |
| 4 | Snow |
| 5 | Ice |
| 6 | Glaze |
| 7 | Not dry |
| 8-14 | Reserved |
| 15 | Missing value |

Add new BUFR sequence in BUFR table D:

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION |
| --- | --- | --- | --- |
| F X Y |  |  |  |
|  |  | (Road weather information) |  |
|  |  | Station identification |  |
| 3 07 102 | 3 01 089 | National station identification |  |
|  | 0 01 018 | Short station or site name | For identification of the road weather monitoring site |
|  | 0 01 015 | Station or site name |  |
|  | 0 01 104 | State / federal state identifier |  |
|  | 0 01 105 | Highway designator |  |
|  | 0 01 106 | Routes kilometre of highway |  |
|  | 0 03 017 | Extended type of station |  |
|  | 0 03 018 | Type of road |  |
|  | 0 03 019 | Type of construction |  |
|  | 3 01 011 | Year, month, day |  |
|  | 3 01 012 | Hour, minute |  |
|  | 3 01 021 | Latitude/longitude (high accuracy) |  |
|  | 0 07 030 | Height of station ground above mean sea level |  |
|  |  | Temperature, humidity and visibility data |  |
|  | 0 07 032 | Height of sensor above local ground |  |
|  | 0 12 101 | Temperature/dry-bulb temperature |  |
|  | 0 12 103 | Dew-point temperature |  |
|  | 0 13 003 | Relative humidity |  |
|  | 0 07 032 | Height of sensor above local ground | Set to missing (cancel) |
|  | 0 20 001 | Horizontal visibility |  |
|  |  | Road temperature and other data |  |
|  | 1 09 000 | Delayed replication of 9 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 03 016 | Position of road sensors |  |
|  | 0 12 128 | Road surface temperature |  |
|  | 1 02 000 | Delayed replication of 2 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 07 061 | Depth below land surface | = 0.30 m in the first replication, = e.g. 0.15 or 0.07 m in the second replication |
|  | 0 12 129 | Road sub-surface temperature |  |
|  | 0 07 061 | Depth below land surface | Set to missing (cancel) |
|  | 0 13 116 | Water film thickness |  |
|  | 0 20 138 | Road surface condition |  |
|  |  | Precipitation data |  |
|  | 0 04 025 | Time period | = - 15 minutes |
|  | 0 20 024 | Intensity of phenomena | Intensity (light, moderate, heavy) of precipitation |
|  | 0 13 055 | Intensity of precipitation |  |
|  | 0 20 021 | Type of precipitation |  |
|  | 0 13 011 | Total precipitation / total water equivalent of snow |  |
|  |  | Wind data |  |
|  | 0 07 032 | Height of sensor above local ground |  |
|  | 0 08 021 | Time significance | = 2 Time averaged |
|  | 0 04 025 | Time period | = –10 minutes |
|  | 0 11 001 | Wind direction |  |
|  | 0 11 002 | Wind speed |  |
|  | 0 08 021 | Time significance | Set to missing (cancel) |
|  |  | Maximum wind gust |  |
|  | 0 04 025 | Time period in minutes |  |
|  | 0 11 043 | Maximum wind gust direction |  |
|  | 0 11 041 | Maximum wind gust speed |  |
|  |  | State of functionality |  |
|  | 0 33 005 | Quality information (AWS data) |  |

Remarks:

- To represent Intensity of precipitation, type of precipitation and state of functionality, 
0 20 024 (Code table), 0 20 021(Flag table) and 0 33 005 (Flag table) are used, respectively.

- Some more descriptors are required to reduce the workload with respect to the station database, e.g. for identification of the federal state, identification of the highway, etc.

- The majority of stations has only one position on the road and one sub-surface temperature sensor. Delayed replications have been introduced to increase flexibility and volume efficiency.

- Each position of road sensors includes the measurements of

  - road surface temperature

  - road sub-surface temperatures

  - water film thickness

  - road surface condition

Some types of station do not have the ability to identify the surface condition accurately. They can only report conditions such as "not dry" or "glazed". The code table for road surface conditions has been adjusted accordingly.

- ANNEX TO PARAGRAPH 3.2.14 [FT2015-2]

Add entries:

In Common Code table C-13 (Data sub-categories of categories defined by entries in BUFR Table A))

| DATA CATEGORIES BUFR Edition 4, Octet 11 in Section 1 CREX Edition 2, nnn in Group Annnmmm of Section 1 |  | INTERNATIONAL DATA SUB-CATEGORIES BUFR Edition 4, Octet 12 (if = 255, it means other sub-category or undefined) CREX Edition 2, mmm in Group Annnmmm of Section 1 |  |
| --- | --- | --- | --- |
| Code figure | Name | Code figure | Name |
| 0 | Surface data - land | 8 | Radiation observations from one-hour period |
|  |  | 9 | Radiation observations from n-minute period |
| 8 | Physical/chemical constituents | 3 | Acid rain |

- ANNEX TO PARAGRAPH 3.2.15 [FT2016-1 except for 3 02 064 (FT2015-2)] <para 4.1>

Editorial note: Text shaded grey has previously been approved for validation but has not yet been validated. Text in red is either new, awaiting validation status or undergone significant change since last approval for validation status.

BUFR/CREX Table B

Class 01 – BUFR/CREX Identification

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 01 | 114 | Encrypted ship or mobile land station identifier | CCITT IA5 | 0 | 0 | 352 | CCITT IA5 | 0 | 44 |

Class 3 – BUFR/CREX Instrumentation

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 03 | 001 | Surface station type | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 | 03 | 002 | Type of humidity sensor | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 | 03 | 003 | Thermometer / hygrometer housing | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 | 03 | 004 | Type of screen / shelter / radiation shield | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 | 03 | 005 | Horizontal width of screen or shield (x) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 006 | Horizontal depth of screen or shield (y) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 007 | Vertical height of screen or shield (z) | m | 3 | 0 | 16 | m | 3 | 5 |
| 0 | 03 | 008 | Artificially ventilated screen or shield | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 009 | Amount of forced ventilation at time of reading | m s-1 | 1 | 0 | 9 | m s-1 | 1 | 3 |
| 0 | 03 | 013 | Type of thermometer | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 020 | Material for thermometer / hygrometer housing | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 021 | Hygrometer heating | Code table | 0 | 0 | 2 | Code table | 0 | 1 |
| 0 | 03 | 022 | Instrument owner | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 023 | Configuration of louvers for thermometer / hygrometer screen | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 | 03 | 024 | Psychrometric coefficient | K-1 | 6 | 0 | 10 | K-1 | 6 | 3 |

Note: the majority of descriptors above were previously proposed at IPET-DRMM-II but have undergone significant revision.

Class 10 – BUFR/CREX Non coordinate location (vertical)

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 10 | 038 | Maximum height of deck cargo above summer load line | m | 0 | 0 | 6 | m | 0 | 2 |
| 0 | 10 | 039 | Departure of reference level (summer maximum load line) from actual sea level | m | 0 | -32 | 6 | m | 0 | 3 |

Class 11 – BUFR/CREX Wind and turbulence

Amend

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 11 | 104 | True heading of aircraft, ship or other mobile platform | Degree true | 0 | 0 | 9 | Degree true | 0 | 3 |

Note: Amendment is change of name.

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 11 | 007 | Relative wind direction (in degrees off bow) | Degree | 0 | 0 | 9 | Degree | 0 | 3 |
| 0 | 11 | 008 | Relative wind speed | m s-1 | 1 | 0 | 12 | m s-1 | 1 | 4 |

Class 25 – BUFR/CREX Processing information

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 25 | 185 | Encryption method | Code table | 0 | 0 | 8 | Code table | 0 | 3 |
| 0 | 25 | 186 | Encryption key version | CCITT IA5 | 0 | 0 | 96 | CCITT IA5 | 0 | 12 |
| 0 | 25 | 188 | Method for reducing pressure to sea level | Code Table | 0 | 0 | 5 | Code Table | 0 | 2 |

Class 41 – BUFR/CREX Oceanographic / bio-geochemical parameters

Add

| Table Reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 41 | 001 | pCO2 | Pa | 3 | 0 | 18 | Atm | 3 | 6 |
| 0 | 41 | 002 | Fluorescence | kg l-1 | 12 | 0 | 16 | kg l-1 | 12 | 5 |
| 0 | 41 | 003 | Dissolved nitrates | µmol kg-1 | 3 | 0 | 17 | µmol kg-1 | 3 | 5 |
| 0 | 41 | 004 | Atmospheric CO2 | ‰ | 5 | 30000 | 15 | ‰ | 5 | 5 |
| 0 | 41 | 005 | Turbidity | NTU | 2 | 0 | 12 | NTU | 2 | 4 |

BUFR/CREX Table D

Category 01 – BUFR/CREX Location and identification sequences

Add

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 01 | 018 |  |  |  | (Encrypted ship's call sign and encryption method) |  |
|  |  |  | 0 | 01 | 114 | Encrypted ship or mobile land station identifier |  |
|  |  |  | 0 | 25 | 185 | Encryption method |  |
|  |  |  | 0 | 25 | 186 | Encryption key version |  |

Category 02 – BUFR/CREX Meteorological sequences common to surface data

Add

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 02 | 062 |  |  |  | (Ship “instantaneous” data) |  |
|  |  |  | 0 | 25 | 188 | Method for reducing pressure report to sea level |  |
|  |  |  | 3 | 02 | 001 | Pressure and 3-hour pressure change |  |
|  |  |  | 3 | 02 | 093 | Extended ship temperature and humidity data |  |
|  |  |  | 1 | 03 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 053 | Ship visibility data |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface | Set to missing (cancel) |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 004 | General cloud information |  |
|  |  |  | 3 | 02 | 005 | Cloud layer |  |
|  |  |  | 0 | 08 | 002 | Vertical significance (surface observations) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 055 | Icing and ice |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 056 | Sea/water temperature |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 021 | Waves |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 024 | Wind and swell waves |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 02 | 063 |  |  |  | (Ship “period” data) |  |
|  |  |  | 3 | 02 | 038 | Present and past weather |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 040 | Precipitation measurement |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 034 | Precipitation past 24 hours |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 058 | Ship extreme temperature data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 064 | Ship / or other marine platform wind data |  |

Add an entry 3 02 064 [FT2015-2]:

In BUFR Table D,

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 02 | 064 |  |  |  | (Ship / or other marine platform wind data) |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 0 | 02 | 002 | Type of instrumentation for wind measurement |  |
|  |  |  | 0 | 08 | 021 | Time significance | = 2 Time averaged |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 001 | Wind direction |  |
|  |  |  | 0 | 11 | 002 | Wind speed |  |
|  |  |  | 0 | 08 | 021 | Time significance | Set to missing (cancel) |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 001 | Delayed descriptor replication factor |  |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 043 | Maximum wind gust direction |  |
|  |  |  | 0 | 11 | 041 | Maximum wind gust speed |  |

Note: this sequence has been validated and is awaiting approval of its validated status (see document IPET-DRMM-III / Doc 4.2.2 (2).

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 02 | 092 |  |  |  | (VOSClim data elements) |  |
|  |  |  | 0 | 11 | 104 | True heading of aircraft, ship or other mobile platform | Ship’s true heading |
|  |  |  | 0 | 01 | 012 | Direction of motion of moving observing platform | Ship’s course over ground |
|  |  |  | 0 | 01 | 013 | Speed of motion of moving observing platform | Ship’s speed over ground |
|  |  |  | 0 | 10 | 038 | Maximum height of deck cargo above summer load line |  |
|  |  |  | 0 | 10 | 039 | Departure of reference level (summer maximum load line) from actual sea level |  |
|  |  |  | 0 | 11 | 007 | Relative wind direction (in degrees off bow) |  |
|  |  |  | 0 | 11 | 008 | Relative wind speed |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | X | YYY |  |  |
| 3 | 02 | 093 |  |  |  | (Extended ship temperature and humidity data) |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature / humidity sensors |  |
|  |  |  | 0 | 12 | 101 | Temperature / air temperature |  |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed replication factor | set to 0 if no change from previous values, 1 if changes |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature / humidity sensors |  |
|  |  |  | 0 | 02 | 039 | Method of wet-bulb temperature measurement |  |
|  |  |  | 0 | 03 | 002 | Type of humidity sensor |  |
|  |  |  | 0 | 03 | 024 | Psychrometer coefficient | Set to missing if type of humidity sensor is not psychrometer |
|  |  |  | 0 | 03 | 021 | Hygrometer heating |  |
|  |  |  | 0 | 12 | 102 | Wet-bulb temperature |  |
|  |  |  | 0 | 12 | 103 | Dewpoint temperature |  |
|  |  |  | 0 | 13 | 003 | Relative humidity |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Cancel (set to missing) |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface | Cancel (set to missing) |
|  |  |  | 3 | 03 | 099 | Metadata common to temperature / humidity sensors | Cancel (set all elements in sequence to missing) |
|  |  |  | 0 | 02 | 039 | Method of wet-bulb temperature measurement | Cancel (set to missing) |
|  |  |  | 0 | 03 | 002 | Type of humidity sensor | Cancel (set to missing) |
|  |  |  | 0 | 03 | 021 | Hygrometer heating | Cancel (set to missing) |
|  |  |  | 0 | 03 | 024 | Psychrometer coefficient | Cancel (set to missing) |

Note: Significant changes to previously proposed version taking into account need for consistency between land and marine metadata.

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 03 | 099 |  |  |  | Metadata common to temperature / humidity sensors |  |
|  |  |  | 0 | 03 | 005 | Horizontal width of screen or shield (x) |  |
|  |  |  | 0 | 03 | 006 | Horizontal depth of screen or shield (y) |  |
|  |  |  | 0 | 03 | 007 | Vertical height of screen or shield (z) |  |
|  |  |  | 0 | 03 | 013 | Type of thermometer |  |
|  |  |  | 0 | 03 | 022 | Instrument owner |  |
|  |  |  | 0 | 03 | 003 | Thermometer / hygrometer housing |  |
|  |  |  | 0 | 03 | 020 | Material for thermometer / hygrometer housing |  |
|  |  |  | 0 | 03 | 004 | Type of screen / shelter / radiation shield |  |
|  |  |  | 0 | 03 | 023 | Configuration of louvers for thermometer / hygrometer screen |  |
|  |  |  | 0 | 03 | 008 | Artificially ventilated screen or shield |  |
|  |  |  | 0 | 03 | 009 | Amount of forced ventilation at time of reading |  |

Category 06 – BUFR/CREX Meteorological or oceanographic sequences common to oceanographic observations

Add

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 06 | 043 |  |  |  | Ship basic bio-geochemical data |  |
|  |  |  | 0 | 41 | 001 | pCO2 |  |
|  |  |  | 0 | 41 | 004 | Atmospheric CO2 |  |
|  |  |  | 0 | 13 | 080 | pH |  |
|  |  |  | 0 | 41 | 005 | Turbidity |  |
|  |  |  | 0 | 41 | 003 | Dissolved nitrates |  |
|  |  |  | 0 | 22 | 188 | Dissolved oxygen |  |
|  |  |  | 0 | 41 | 002 | Fluorescence |  |

Category 08 – BUFR/CREX Surface report sequences (sea)

Add

| Table reference |  |  | Table References |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
| 3 | 08 | 014 |  |  |  | (Synoptic reports from sea stations suitable for VOS observation data) |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 01 | 018 | Encrypted ship’s call sign and encryption method |  |
|  |  |  | 3 | 01 | 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |
|  |  |  | 2 | 08 | 032 | Change width of CCITT IA5 to 32 characters |  |
|  |  |  | 0 | 01 | 079 | Unique identifier for profile | Unique ID for report. |
|  |  |  | 2 | 08 | 000 | Change width of CCITT IA5 | Cancel |
|  |  |  | 3 | 02 | 062 | Ship “instantaneous” data |  |
|  |  |  | 3 | 02 | 063 | Ship “period” data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 092 | VOSClim data elements |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 033 | Surface salinity |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 034 | Surface current |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 06 | 043 | Ship basic bio-geochemical data |  |

Note: last 3 descriptors (1 01 000 to 3 06 043) to be moved outside 3 08 014 if 3 06 043 not ready for validation.

BUFR/CREX Code tables

Code/flag Tables 03

Add

0 03 001

Surface Station Type

| Code figure | Meaning |
| --- | --- |
| 0 | Land station (synoptic network) |
| 1 | Shallow water station (fixed to sea/lake floor) |
| 2 | Ship |
| 3 | Rig/platform |
| 4 | Moored buoy |
| 5 | Drifting buoy (or drifter) |
| 6 | Ice buoy |
| 7 | Land station (local network) |
| 8 | Land vehicle |
| 9 | Autonomous marine vehicle |
| 10-30 | Reserved (for future use) |
| 31 | Missing value |

0 03 002

Type of humidity sensor

| Code figure | Meaning |
| --- | --- |
| 0 | Psychrometer |
| 1 | Capacitive (Generic) |
| 2 | Capacitive (polymer) |
| 3 | Capacitive (Ceramic inc.Metal-oxide) |
| 4 | Resistive (Generic) |
| 5 | Resistive (Salt polymer) |
| 6 | Resistive (Conductive polymer) |
| 7 | Thermal conductivity |
| 8 | Gravimetric |
| 9 | Paper-metal coil |
| 10 | Ordinary human hair |
| 11 | Rolled hair (Torsion) |
| 12 | Goldbeater's skin |
| 13 | Chilled mirror hygrometer |
| 14 | Dew cell |
| 15 | Optical absorption sensor |
| 16 - 30 | Reserved for future use |
| 31 | Missing value |

0 03 003

Thermometer / hygrometer housing

| Code figure | Meaning |
| --- | --- |
| 0 | Screen |
| 1 | Sling / Whirling |
| 2 | Unscreened |
| 3 | Radiation shield |
| 4 | Aspirated (e.g. Assmann) |
| 5 | Other Shelter |
| 6 | Handheld |
| 7-14 | Reserved for future use |
| 15 | Missing value |

0 03 004

Type of screen / shelter / radiation shield

| Code figure | Meaning |
| --- | --- |
| 0 | Stevenson screen |
| 1 | Marine Stevenson screen |
| 2 | Cylindrical section plate shield |
| 3 | Concentric tube |
| 4 | Rectangular section shield |
| 5 | Square section shield |
| 6 | Triangular section shield |
| 7 | Open covered lean to |
| 8 | Open covered inverted V roof |
| 9 | Integrated (e.g. Chilled Mirror) |
| 10-14 | Reserved for future use |
| 15 | Missing value |

0 03 008

Artificially ventilated screen or shield

| Code figure | Meaning |
| --- | --- |
| 0 | Natural ventilation in use |
| 1 | Artifical aspiration in use: constant flow at time of reading |
| 2 | Artifical aspiration in use: variable flow at time of reading |
| 3-6 | Reserved |
| 7 | Missing value |

0 03 013

Type of thermometer

| Code figure | Meaning |
| --- | --- |
| 0 | Electrical (resistance) |
| 1 | Mercury |
| 2 | Alcohol / glycol |
| 3-6 | Reserved for future use |
| 7 | Missing value |

0 03 020

Material for thermometer / hygrometer housing

| Code figure | Meaning |
| --- | --- |
| 0 | Wood |
| 1 | Metal alloy |
| 2 | Plastic / GRP |
| 3 | Reed / grass / leaf |
| 4-6 | Reserved for future use |
| 7 | Missing value |

0 03 021

Hygrometer heating

| Code figure | Meaning |
| --- | --- |
| 0 | Unheated |
| 1 | Heated |
| 2 | Not applicable |
| 3 | Missing value |

0 03 022

Instrument owner

| Code figure | Meaning |
| --- | --- |
| 0 | National hydro meteorological / weather service |
| 1 | Other |
| 2 | Standards institute |
| 3-6 | Reserved for future use |
| 7 | Missing value |

0 03 023

a for thermometer / hygrometer screen

| Code figure | Meaning |
| --- | --- |
| 0 | Single v section louvers |
| 1 | Overlapping louvers |
| 2 | Double v section louvers |
| 3 | Non-overlapping louvers |
| 4 | Vented, non-louvered |
| 5 | Not applicable |
| 6 | Reserved for future use |
| 7 | Missing value |

Code/flag Tables 25

Add

0 25 185

Encryption method

| Code Figure | Meaning |
| --- | --- |
| 0 | AES 256 |
| 1 – 254 | Reserved |
| 255 | Missing value |

0 25 188

Method for reducing pressure report to sea level

| Code Figure | Meaning |
| --- | --- |
| 0 | Pressure adjusted to mean sea level following WMO 8 |
| 1 | Pressure adjusted to mean sea level following national practice |
| 2 | Pressure adjusted to local water level following WMO 8 |
| 3 | Pressure adjusted to local water level following national practice |
| 4 | Pressure not corrected for height |
| 5 – 14 | Reserved |
| 15 | Missing |

Common Code Table C6

Add

| Code figure |  | Conventional abbreviation | Abbreviation in IA5/ASCII | Abbreviation in IA2 | Definition in base units |
| --- | --- | --- | --- | --- | --- |
| 843 | Nephelometric Turbidity Units | NTU | NTU |  |  |

APPENDIX TO PARAGRAPH 3.2.15 [For reference]

status of BUFR templates for marine DATA

| TAC | Description | Current template(s) | Status | Plans/comments |
| --- | --- | --- | --- | --- |
| FM13-XIV SHIP | VOS data | B/C10 - Regulations for reporting SHIP data in TDCF | Operational (TM308009) |  |
|  | VOS data | Synoptic reports from sea stations suitable for SHIP observation data from VOS stations (see Appendix B) | Validation | Revised template under validation |
| FM18-XII BUOY | Drifting buoy data | Template for the representation from drifting buoys | Operational (TM 315009) | Simplified template specific to drifting buoys |
|  | Moored buoy data | Template for the representation of data from moored buoys | Operational (TM 315008) | Simplified template specific to moored buoys, including directional and non-directional wave data |
|  | Wave buoy data | Template for the representation of data from moored buoys | Operational (TM 315008) | Simplified template specific to moored buoys, including directional and non-directional wave data |
|  | Argo data | Sub-surface profiling floats | Operational (TM 315003) | Note additional sequences for the reporting of high precision / accuracy temperature and salinity data (306017 and 306018) |
| FM36-XI Ext. TEMP SHIP | ASAP data | B/C25 - Regulations for reporting TEMP, TEMP SHIP, TEMP MOBIL data in TDCF | Operational (TM309052) |  |
|  | ASAP data | UKMO template for representation of radiosonde data with geopotential height as the vertical coordinate | Operational (revisited in July 2010) |  |
| FM62-VIII Ext. TRACKOB | TRACKOB data | TRACKOB data – ThermoSalinoGraph (TSG) data and metadata | Operational (TM308010) |  |
| FM63-XI Ext. BATHY | XBT data | New BUFR template for XBT Temperature Profile data | Operational (TM315004) |  |
| FM64-XI Ext. TESAC | CTD / TESAC | Template for the representation of data derived from a ship based lowered instrument measuring subsurface seawater temperature, salinity and current profiles. | Operational (TM 315007) |  |
| FM65-XI Ext. WAVEOB | Wave buoy data | Templates for the wave observations from different platforms suitable for WAVEOB data | Validation | Template should be imminently validated and operational. See also FM18-XII BUOY. |
| N/A | Sea-level data | BUFR/CREX templates for tsunameter data and dart buoy system messages | Operational (TM306027) |  |
|  | Sea-level data | BUFR/CREX templates for reporting time series of tide data | Operational (TM306011, TM306012, TM306013, TM306014, TM306016, TM306029) |  |

- ANNEX TO PARAGRAPH 3.2.16 [Further study]

Add an entry:

In BUFR Code table 0 02 083

0 02 083

Type of balloon shelter

Code figure

4	Automated (unmanned) sounding system

- ANNEX TO PARAGRAPH 3.3.1 [For reference]

A new change management strategy for BUFR editions 4 and 5: The possibility of allowing a BUFR edition 4 to evolve incrementally to a limited degree in two-stream approach.

In the first stream, the Team may consider whether an incremental, “point-release” approach might be applicable. This would allow for light impact changes to BUFR edition 4 specification (thus leading to 4.1, 4.2…).  This is not resource-neutral, but it allows for incremental evolution of the specification with no direct impact on existing templates or operational applications.  It also provides for a suitably long transition period to let the MTDCF take hold with BUFR edition 4.

The second stream would seek to create a BUFR edition 5 platform that would essentially be a reengineering of the BUFR code form, keeping in mind the BUFR design goals below. The most important new design goals would be the implementation of a logical model approach for data representation and conformity with the ISO 19000 series of standards.

The list of design goals presented at the first meeting of the IPET-DRMM in 2013:

a)	Universality in the representation of data and metadata (achieved through the BUFR data representation syntax and the various code tables)	Extensibility (meaning that new data elements or sequences can be added without changes to the encoding/decoding software; achieved by Table-driven approach)

b)	Portability across computer architectures (achieved by use of unsigned octets, scale, reference value and bit-width)

c)	Compactness (achieved by Table-Driven approach and representation of code values in binary form, using as few bits as possible).

d)	Lossless conveyance of information (no lossy compression is used. Also, while current editions do not focus on data integrity in the technical sense of the word,  there is a concern for sufficient precision to preserve the accuracy of the data being conveyed.)

e)	Focus on operational data exchange between data processing centres

f)	Interoperability with ISO standards for data and metadata representation.

- ANNEX TO PARAGRAPH 4.1

Implementation of amendments during the intersessional period

1.	Adoption between CBS sessions 2014 (5 November 2014)

2.	Pre-operational (14 October 2014)

3.	Fast-track 2014-2 (5 November 2014)

4.	Fast-track 2015-1 (6 May 2015)

5.	Pre-Operational (7 May 2015)

6.	Rec. 6 and 7 (CBS Ext. (2014)) (4 November 2015)

Note: The recommendations have been adopted by the World Meteorological Congress Seventeenth session (Cg-17).

STATUS OF PROPOSALS

1.	The following tables are to show the proposals at validation stage to facilitate validation exercise.

2.	Proposals at validation stage have no status of the Manual on Codes (WMO-No. 306) and MAY NOT be used in operational data and products.

3.	It should be noted proposals at validation stage may be modified during validation exercise.  It is recommended to update the proposals whenever modified.

Information as of: 24 July 2015

FM 92 GRIB

| ID | Approval for Validation | Proposals | Last update for validation | Stage |
| --- | --- | --- | --- | --- |
| 2014-2.2.1 | DRMM-II | New parameters and fixed surface types to represent ground surface conditions in analyses or models |  | FT2015-2 Pre-operational 07/05/2015 |
| 2014-2.2.2 | DRMM-II | A product definition template for statistics over an ensemble | ongoing | Val |
| 2014-2.2.5 | DRMM-II | Review of GRIB templates for spatio-temporal changing tiles | Replaced by 2015-2.2.11 (DRMM-III) | FT2015-2 |
| 2014-2.2.7 | DRMM-II | Octets of forecast time in product definition template 4.44 |  | 4 Nov. 2015 CBS-Ext. (2014) |
| 2013-2.1.1 | DRMM-I | Reporting quality information in GRIB2 | Replaced by 2015-2.2.1/2.2.2 (DRMM-III) | Val |
| 2013-2.2.1 | DRMM-I | GRIB2 new compression method – CCSDS libaec | ongoing | Val |
| 2013-2.2.5 | DRMM-I | Proposed amendments to GRIB2 Code tables 4.2, 4.3 and 4.10 (amendments other than Code table 4.10 were implemented by FT2014-1) | Replaced by 2015-2.2.1/2.2.2 (DRMM-III) | Val |
| 2012-2.2.9 | DRC-IV | GRIB template for 4-D Trajectory grid definition | ongoing | Val |
| 2011-2.3.5 | DRC-III | GRIB template for rotated tilted Mercator projection | ongoing | Val |
| 2011-2.3.12 | DRC-III | Space Weather in GRIB2 |  | Withdrawn |
| 2010-2.3.1/2 | DRC-II | Encoding wave spectra in GRIB Edition 2 Waves parameters (amendments other than 46-48 were implemented) |  | Withdrawn |

FM 94 BUFR/FM 95 CREX

| ID | Approval for Validation | Proposals | Last update | Stage |
| --- | --- | --- | --- | --- |
| 2014-3.1.2 | DRMM-II | Clarification of Regulation 94.1.5 |  | 4 Nov. 2015 CBS-Ext. (2014) |
| 2014-3.2.7 | DRMM-II | BUFR template for n-minute AWS data (3 07 092) | ongoing | Val |
| 2014-3.2.8 | DRMM-II | Revised BUFR sequence for synoptic reports from sea stations suitable for VOS observation data | Replaced by 2015-3.2.15 | Val 3 02 064 FT2015-2 |
| 2014-3.2.9 | DRMM-II | Proposed BUFR sequence for reporting observations from offshore platforms | ongoing | Val |
| 2014-3.2.10 | DRMM-II | Proposed modifications to the BUFR template for reporting data from Argo profiling floats |  | FT2015-2 |
| 2014-11.2 | DRMM-II | WIGOS Station Identifier | ongoing | Val |
| 2013-3.2.2 | DRMM-I | Proposal for a BUFR template for radar wind profiler | ongoing | Val |
| 2013-3.2.4 | DRMM-I | Satellite-derived winds in BUFR | ongoing | Val |
| 2012-3.2.7 | DRC-IV | Sequence 3 02 067 | Replaced by 2015-3.2.12 (DRMM-III) | FT2015-2 |
| 2012-3.2.11 | DRC-IV | A BUFR Sequence to append data for national use to Canadian SYNOP reports | ongoing | Val |
| 2012-3.2.12 | DRC-IV | New BUFR/CREX entries for dual polarization radar data |  | FT2015-2 |
| 2009-3.3.2 | DRC-I | Update from JCOMM on developments for BUFR templates: New XBT template and queries about a revised VOS template * Proposals other than 3 06 032 and 3 15 006 have been implemented. 3 06 032 and 3 15 006 were proposed in association with the 2006-5.4.3 and considered under validation. |  | Withdrawn |
| 2008-3.1.15 | JM-MTDCF/DRC | Templates for the wave observations from different platforms suitable for WAVEOB data (revision 4) | Replaced by 2015-3.2.6 (DRMM-III) | FT2016-1 |
| 2006-5.4.3 | JM-MTDCF/DRC | BUFR template for buoy wave data * It was confirmed in 2013 that this template is under validation. |  | Withdrawn |

- ANNEX TO PARAGRAPH 4.2

4.2.1.1	New parameters and fixed surface types to represent ground surface conditions in analyses or models [Pre-operational since May 2015/FT2015-2] <para 4.1>

Add entries:

In Code tables 4.2,

| Parameter | Product Discipline | Parameter Category | Parameter number | Units |
| --- | --- | --- | --- | --- |
| Brightness temperature | 0 | 5 | 7 | K |
| Soil volumetric ice content (water equivalent) (see note) | 2 | 0 | 38 | m3 m-3 |
| Liquid water in snow pack | 2 | 3 | 23 | kg m-2 |
| Glacier temperature | 2 | New parameter category: 5 (Glaciers and inland ice) | 1 | K |

Note: For Parameter 38 (Parameter Category 0), ice volume is expressed as if the ice content were melted to liquid water and then its volume measured in the liquid state. This may be understood in the same manner as water equivalent snow depth.

In Code tables 4.5,

|  | Entries under validation for Code Table 4.5 (Fixed surface types and units) |
| --- | --- |
| Code figure | Meaning |
| 174 | Top surface of ice on sea, lake or river |
| 175 | Top surface of ice, under snow cover, on sea, lake or river |
| 176 | Bottom surface (underside) ice on sea, lake or river |
| 177 | Deep soil (of indefinite depth) |
| 178 | Reserved |
| 179 | Top surface of glacier ice and inland ice |
| 180 | Deep inland or glacier ice (of indefinite depth) |
| 181 | Grid tile land fraction as a model surface |
| 182 | Grid tile water fraction as a model surface |
| 183 | Grid tile ice fraction on sea, lake or river as a model surface |
| 184 | Grid tile glacier ice and inland ice fraction as a model surface |

4.2.1.2	Tile template for GRIB2 [see 2015-2.2.11 (DRMM-III)]

4.2.2.1	BUFR descriptors for dual-polarization radar data [FT2015-2] <para 4.1>

Add entries:

In BUFR/CREX Table B,

|  |  | Element name |  | BUFR |  |  |  |  | CREX |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  |  |  | Unit |  | Scale | Ref. value | Data width | Unit |  | Scale | Data width |
| 0 21 004 |  | Differential reflectivity |  | dB |  | 2 | -800 | 11 | dB |  | 2 | 4 |
| 0 21 028 |  | Differential phase |  | degree |  | 1 | 0 | 12 | degree |  | 1 | 4 |
| 0 21 029 |  | Cross-polarization correlation coefficient |  | Numeric |  | 2 | -100 | 8 | Numeric |  | 2 | 3 |

4.2.2.2	Validation of sequence 3 02 064 (Ship/or other marine platform wind data) [FT2015-2]

<para 4.1>

Add entries:

In BUFRTable D,

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship/or other marine platform wind data) |  |
| 3 | 02 | 064 | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 0 | 02 | 002 | Type of instrumentation for wind measurement |  |
|  |  |  | 0 | 08 | 021 | Time significance | Set to 2, time averaged |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 001 | Wind direction |  |
|  |  |  | 0 | 11 | 002 | Wind speed |  |
|  |  |  | 0 | 08 | 021 | Time significance | Set to missing (cancel) |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 001 | Delayed replication factor |  |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 043 | Maximum wind gust direction |  |
|  |  |  | 0 | 11 | 041 | Maximum wind gust speed |  |

4.2.2.3	Validation of additional Argo BUFR Sequences [FT2015-2] <para 4.1>

# Add entries:

# In BUFR/CREX Table B,

| Reference |  |  | Element Name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Unit | Scale | Data width (char) |
| 0 | 08 | 034 | Temperature/salinity measurement qualifier | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

# In BUFR Table D,

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
|  |  |  |  |  |  | (Sub-surface temperature profile (high accuracy/precision) with quality flags) |  |
| 3 06 017 |  |  | 0 | 02 | 032 | Indicator for digitization | = 0 Fixed sensor depths |
|  |  |  | 0 | 08 | 034 | Temperature/salinity measurement qualifier |  |
|  |  |  | 1 | 06 | 000 | Delayed replication of 6 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Number of depths |
|  |  |  | 0 | 07 | 065 | Water pressure | In Pa |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 045 | Sea/water temperature | In K to 3 decimal places |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 08 | 034 | Temperature/salinity measurement qualifier | Set to missing (cancel) |

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
|  |  |  |  |  |  | (Sub-surface temperature profile (high accuracy/precision) with quality flags) |  |
| 3 06 018 |  |  | 0 | 02 | 032 | Indicator for digitization | = 0 Fixed sensor depths |
|  |  |  | 0 | 08 | 034 | Temperature/salinity measurement qualifier |  |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 002 | Extended delayed descriptor replication factor | Number of depths |
|  |  |  | 0 | 07 | 065 | Water pressure | In Pa |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 045 | Sea/water temperature | In K to 3 decimal places |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 064 | Salinity |  |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 08 | 034 | Temperature/salinity measurement qualifier | Set to missing (cancel) |

# Add a Code table:

0 08 034

Temperature / salinity measurement qualifier

0	Secondary sampling: averaged

1	Secondary sampling: discrete

2	Secondary sampling: mixed

3	Near-surface sampling: averaged, pumped

4	Near-surface sampling: averaged, un-pumped

5	Near-surface sampling: discrete, pumped

6	Near-surface sampling: discrete, un-pumped

7	Near-surface sampling: mixed, pumped

8	Near-surface sampling: mixed, un-pumped

9 – 14	Reserved

15	Missing value

- ANNEX TO PARAGRAPH 5.1 [ABC2016]

# Add a B/C 30.4.4:

B/C 30.4	Regional or national reporting practices

B/C 30.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT data in Manual on Codes, WMO-No. 306, Volume II.

B/C 30.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 30.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 30.4.3	Date/time (of beginning of the period for monthly precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for period different from the period recommended in Note (1) to B/C 30.2.6.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

B/C 30.4.4	Date/time (of beginning of the one-month period for precipitation data on the last day of the previous month)

If the regional or national reporting practices require reporting monthly precipitation data for period which starts on the last day of the previous month in UTC, template TM 307078 should be used. The beginning of the period for monthly precipitation data shall be specified by short time displacement (0 04 074) set to a relevant negative value. The beginning of one-month period for which the normals of precipitation are reported, shall be specified in a similar way.

# Add a B/C 32.4.4:

B/C 32.4	Regional or national reporting practices

B/C 32.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT SHIP data in Manual on Codes, WMO-No. 306, Volume II.

B/C 32.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 32.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 32.4.3	Date/time (of beginning of the one-month period for precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for one-month period different from the period recommended in Note (1) to B/C 32.2.3.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

B/C 32.4.4	Date/time (of beginning of the one-month period for precipitation data on the last day of the previous month)

If the regional or national reporting practices require reporting monthly precipitation data for period which starts on the last day of the previous month in UTC, template TM 308023 should be used. The beginning of the period for monthly precipitation data shall be specified by short time displacement (0 04 074) set to a relevant negative value. The beginning of one-month period for which the normals of precipitation are reported, shall be specified in a similar way.

- ANNEX TO PARAGRAPH 5.2 [ABC2016 except for 3 01 128 (FT2015-2)]

Editorial note: corrections to the Notes 4 and 5 were agreed as specified here.

Amend the notes to 3 09 052 in B/C25:

Notes:

(1)	Time of launch 3 01 013 shall be reported with the highest possible accuracy available.  If the launch time is not available with second accuracy, the entry for seconds shall be put to zero.

(2)	Long time displacement 0 04 086 represents the time offset from the launch time 3 01 013 (in seconds).

(3)	Latitude displacement 0 05 015 represents the latitude offset from the latitude of the launch site.  Longitude displacement 0 06 015 represents the longitude offset from the longitude of the launch site.

(4)	If additional information on sounding is available, the sequence <3 09 052> shall be preceded by sequence <3 01 128> shown in the Annex II to B/C25.

(5)	If the sounding data are obtained from upper-air systems where pressure is derived from geopotential height by integration of hydrostatic equation, the geopotential calculation method shall be recorded using 0 02 191 within the sequence 3 01 128.

Delete B/C25.11.

Amend Annex II to B/C25 as follows

Additional information on radiosonde ascent

| 3 01 128 |  | Additional information on radiosonde ascent |
| --- | --- | --- |
|  | 0-01-081 | Radiosonde serial number |
|  | 0-01-082 | Radiosonde ascension number |
|  | 0-01-083 | Radiosonde release number |
|  | 0-01-095 | Observer identification |
|  | 0-02-015 | Radiosonde completeness |
|  | 0-02-016 | Radiosonde configuration |
|  | 0-02-017 | Correction algorithms for humidity measurements |
|  | 0-02-066 | Radiosonde ground receiving system |
|  | 0-02-067 | Radiosonde operating frequency |
|  | 0-02-080 | Balloon manufacturer |
|  | 0-02-081 | Type of balloon |
|  | 0-02-082 | Weight of balloon |
|  | 0-02-083 | Type of balloon shelter |
|  | 0-02-084 | Type of gas used in balloon |
|  | 0-02-085 | Amount of gas used in balloon |
|  | 0-02-086 | Balloon flight train length |
|  | 0-02-095 | Type of pressure sensor |
|  | 0-02-096 | Type of temperature sensor |
|  | 0-02-097 | Type of humidity sensor |
|  | 0-02-103 | Radome |
|  | 0-02-191 | Geopotential height calculation |
|  | 0-25-061 | Software identification and version number |
|  | 0-35-035 | Reason for termination |

Add the entry 3 01 128 in BUFR Table D [FT2015-2]

- ANNEX TO PARAGRAPH 5.3 [ABC2016]

- To amend B/C 20.5.2.5 Regulation as follows:

B/C 20.5.2.5 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- To amend B/C 25.7.2.8 Regulation as follows:

B/C 25.7.2.8 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- To amend B/C 26.5.2.8 Regulation as follows:

B/C 26.5.2.8 Wind direction and speed

The wind direction (0 11 001) shall be reported in degrees true and the wind speed (0 11 002) shall be reported in meters per second (with precision in tenths of a meter per second).

Note:

- Wind direction measured in sounding originated from a launch site within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

- ANNEX TO PARAGRAPH 6.1 [Withdrawn]

Amend a regulation:

15.13.2.1	Up to three groups of information on recent weather shall be given by the indicator letters RE followed, without a space, by the appropriate abbreviations, in accordance with Regulation 15.8 (but no intensity of the recent weather phenomena shall be indicated) if the following weather phenomena were observed during the period since the last routine report, or last hour, whichever is shorter, but not at the time of observation:

–	Freezing precipitation;

–	Moderate or heavy drizzle, rain or snow;

–	Moderate or heavy: ice pellets, hail, small hail and/or snow pellets;

–	Blowing snow;

–	Sandstorm or duststorm;

–	Thunderstorm;

–	Funnel cloud(s) (tornado or waterspout);

–	Volcanic ash.

Not more than one precipitation type shall be reported in the recent weather group.  For example, abbreviations such as RERASN shall not be used.

When an automatic observing system is used and when the type of the precipitation cannot be identified by this system, the abbreviation REUP shall be used for recent precipitation. It may be combined with the characteristics of the present weather in accordance with Regulation 15.8.6.

Note: The meteorological authority, in consultation with users, may agree not to provide recent weather information where SPECI are issued.

- ANNEX TO PARAGRAPH 7.1

# Figure 1: Surface BUFR reports for RBSN stations captured by the SMM exercise during 1-15 April 2015 shown as a percentage of the expected number of reports.

# Figure 2: Upper-air BUFR reports for RBSN stations captured by the SMM exercise during 1-15 April 2015 shown as a percentage of the expected number of reports.

# Figure 3: Climate BUFR reports for RBSN stations captured by the SMM exercise during 1-15 April 2015 shown as a percentage of the expected number of reports.

# Table 1:  Average by country of the number of surface and upper-air reports from RBSN stations and climate reports from RBCN stations recorded in BUFR messages during the SMM of 1-15 April 2015.  The number of reports expected from each station is 60 (surface), 30 (upper-air) and 1 (climate), respectively.

# Surface RBSN

| Country/Territory | Average of TAC | Average of TDCF |
| --- | --- | --- |
| REGION 1 |  |  |
| Algeria | 92 | 90 |
| Angola | 8 | 0 |
| Benin | 100 | 0 |
| Botswana | 54 | 0 |
| Bouvet Island (Norway) | 0 | 0 |
| Burkina Faso | 93 | 66 |
| Burundi | 62 | 0 |
| Cabo Verde | 99 | 98 |
| Cameroon | 24 | 0 |
| Canary Islands (Spain) | 81 | 85 |
| Central African Republic | 21 | 21 |
| Ceuta And Melilla (Spain) | 70 | 73 |
| Chad | 34 | 15 |
| Comoros | 93 | 49 |
| Congo | 88 | 51 |
| Cote D'ivoire | 80 | 36 |
| Democratic Republic Of The Congo | 5 | 0 |
| Djibouti | 25 | 0 |
| Egypt | 85 | 39 |
| Equatorial Guinea | 0 | 79 |
| Eritrea | 0 | 0 |
| Ethiopia | 43 | 0 |
| Gabon | 55 | 43 |
| Gambia | 91 | 0 |
| Ghana | 54 | 0 |
| Guinea | 16 | 0 |
| Guinea-Bissau | 10 | 8 |
| Kenya | 58 | 2 |
| Lesotho | 13 | 0 |
| Liberia | 28 | 0 |
| Libya | 33 | 0 |
| Madagascar | 60 | 37 |
| Madeira | 99 | 38 |
| Malawi | 43 | 0 |
| Mali | 65 | 40 |
| Mauritania | 53 | 38 |
| Mauritius | 94 | 100 |
| Morocco | 50 | 0 |
| Mozambique | 23 | 0 |
| Namibia | 51 | 0 |
| Niger | 91 | 3 |
| Nigeria | 8 | 0 |
| Ocean Islands | 69 | 36 |
| Rwanda | 48 | 0 |
| Senegal | 90 | 25 |
| Seychelles | 32 | 31 |
| Sierra Leone | 0 | 0 |
| Somalia | 0 | 0 |
| South Africa | 74 | 45 |
| South Sudan | 0 | 0 |
| Sudan | 5 | 0 |
| Swaziland | 45 | 0 |
| Togo | 92 | 0 |
| Tunisia | 77 | 1 |
| Uganda | 23 | 0 |
| United Republic Of Tanzania | 59 | 32 |
| Western Sahara | 38 | 0 |
| Zambia | 11 | 0 |
| Zimbabwe | 57 | 23 |
| REGION 2 |  |  |
| Afghanistan, Islamic State Of | 0 | 0 |
| Bahrain | 19 | 5 |
| Bangladesh | 84 | 15 |
| Cambodia | 5 | 5 |
| China | 100 | 51 |
| Democratic People's Republic Of Korea | 100 | 61 |
| Hong Kong, China | 100 | 57 |
| India | 95 | 87 |
| Iran, Islamic Republic Of | 97 | 0 |
| Iraq | 50 | 0 |
| Japan | 100 | 100 |
| Kazakhstan | 98 | 0 |
| Kuwait | 100 | 13 |
| Kyrgyzstan | 100 | 0 |
| Lao People's Democratic Republic | 73 | 0 |
| Macao, China | 98 | 0 |
| Maldives | 67 | 85 |
| Mongolia | 100 | 39 |
| Myanmar | 76 | 0 |
| Nepal | 71 | 0 |
| Oman | 91 | 11 |
| Pakistan | 72 | 42 |
| Qatar | 63 | 18 |
| Republic Of Korea | 100 | 57 |
| Russian Federation | 96 | 54 |
| Saudi Arabia | 72 | 44 |
| Sri Lanka | 86 | 0 |
| Tajikistan | 94 | 0 |
| Thailand | 100 | 0 |
| Turkmenistan | 100 | 0 |
| United Arab Emirates | 84 | 19 |
| Uzbekistan | 100 | 0 |
| Viet Nam | 96 | 51 |
| Yemen | 0 | 0 |
| REGION 3 |  |  |
| Argentina | 98 | 0 |
| Bolivia | 45 | 0 |
| Brazil | 67 | 50 |
| Chile | 77 | 0 |
| Colombia | 61 | 33 |
| Ecuador | 53 | 6 |
| French Guiana | 98 | 99 |
| Guyana | 27 | 0 |
| Islands (88: 800 - 998) | 0 | 68 |
| Paraguay | 66 | 0 |
| Peru | 55 | 0 |
| South America | 0 | 79 |
| Suriname | 24 | 0 |
| Uruguay | 82 | 0 |
| Venezuela | 7 | 0 |
| REGION 4 |  |  |
| Antigua And Barbuda | 65 | 67 |
| Bahamas | 14 | 0 |
| Barbados | 98 | 0 |
| Belize | 48 | 0 |
| Bermuda | 98 | 98 |
| Canada | 96 | 16 |
| Cayman Islands | 73 | 0 |
| Clipperton | 0 | 0 |
| Colombia (San Andres And Providencia Islands) | 71 | 38 |
| Costa Rica | 59 | 0 |
| Cuba | 76 | 0 |
| Curacao, St Maarten And Aruba (Nl) | 98 | 0 |
| Dominica | 73 | 0 |
| Dominican Republic | 83 | 0 |
| El Salvador | 0 | 0 |
| Grenada | 98 | 0 |
| Guadeloupe, St Martin, St Barthelemy And Other French Islands In The Vicinity | 0 | 0 |
| Guatemala | 34 | 0 |
| Haiti | 0 | 0 |
| Honduras | 82 | 0 |
| Jamaica | 100 | 0 |
| Martinique | 99 | 50 |
| Mexico | 85 | 59 |
| Nicaragua | 0 | 0 |
| Panama | 0 | 0 |
| Puerto Rico And Us Possessions In The Caribbean Area | 66 | 0 |
| Saint Lucia | 82 | 0 |
| St Eustatius And Bonaire (Nl) | 48 | 0 |
| St. Pierre And Miquelon (France) | 100 | 0 |
| Trinidad And Tobago | 93 | 0 |
| United States Of America | 96 | 0 |
| United States Of America (Alaska) | 84 | 0 |
| Venezuela (Isla De Aves) | 0 | 0 |
| REGION 5 |  |  |
| American Samoa | 20 | 0 |
| Australia | 80 | 65 |
| Brunei Darussalam | 92 | 0 |
| Cook Islands | 39 | 37 |
| Detached Islands (91 : 753, 754) | 93 | 0 |
| Detached Islands (91: 960 - 998) | 100 | 100 |
| Fiji | 76 | 22 |
| French Polynesia (Austral Islands) | 100 | 100 |
| French Polynesia (Marquesas Islands) | 100 | 100 |
| French Polynesia (Society Islands) | 100 | 100 |
| French Polynesia (Tuamotu Islands And Gambier Islands) | 98 | 75 |
| Indonesia | 94 | 76 |
| Islands (96: 995, 996) | 100 | 82 |
| Islands In The Pacific Ocean North Of The Equator | 32 | 0 |
| Kiribati | 46 | 0 |
| Malaysia | 2 | 98 |
| Nauru | 0 | 0 |
| New Caledonia | 83 | 50 |
| New Zealand | 100 | 95 |
| Niue | 28 | 27 |
| Papua New Guinea | 24 | 14 |
| Philippines | 89 | 84 |
| Phoenix Islands | 0 | 0 |
| Samoa | 28 | 0 |
| Singapore | 100 | 100 |
| Solomon Islands | 87 | 72 |
| Timor-Leste | 0 | 0 |
| Tokelau (And Swains Is.) | 0 | 0 |
| Tonga | 70 | 69 |
| Tuvalu | 50 | 0 |
| Vanuatu | 78 | 0 |
| REGION 6 |  |  |
| Armenia | 100 | 0 |
| Austria | 100 | 96 |
| Azerbaijan | 100 | 0 |
| Belarus | 100 | 87 |
| Belgium | 100 | 95 |
| Bosnia And Herzegovina | 78 | 78 |
| Bulgaria | 100 | 0 |
| Croatia | 93 | 70 |
| Cyprus | 100 | 7 |
| Czech Republic | 100 | 95 |
| Denmark And Faroe Islands | 99 | 73 |
| Estonia | 100 | 99 |
| Finland | 100 | 100 |
| France | 100 | 100 |
| Georgia | 89 | 0 |
| Germany | 100 | 100 |
| Gibraltar | 98 | 98 |
| Greece | 91 | 87 |
| Greenland | 74 | 28 |
| Hungary | 100 | 100 |
| Iceland | 57 | 55 |
| Ireland | 100 | 100 |
| Israel | 100 | 38 |
| Italy | 92 | 100 |
| Jordan | 96 | 0 |
| Kazakhstan | 100 | 0 |
| Latvia | 100 | 85 |
| Lebanon | 95 | 0 |
| Lithuania | 100 | 100 |
| Luxembourg | 98 | 98 |
| Malta | 95 | 0 |
| Montenegro | 96 | 59 |
| Netherlands | 100 | 100 |
| Norway | 96 | 89 |
| Poland | 97 | 95 |
| Portugal | 96 | 46 |
| Republic Of Moldova | 100 | 0 |
| Romania | 100 | 100 |
| Russian Federation | 99 | 37 |
| Serbia | 93 | 89 |
| Slovakia | 100 | 100 |
| Slovenia | 100 | 78 |
| Spain | 82 | 88 |
| Sweden | 97 | 100 |
| Switzerland And Liechtenstein | 100 | 88 |
| Syrian Arab Republic | 48 | 3 |
| The Former Yugoslav Republic Of Macedonia | 66 | 43 |
| Turkey | 90 | 89 |
| Ukraine | 94 | 2 |
| United Kingdom Of Great Britain And Northern Ireland | 98 | 100 |
| ANTARCTICA |  |  |
| Stations In The Antarctic | 48 | 19 |

# Upper air

| Country/territory | Average of TAC | Average of TDCF |
| --- | --- | --- |
| REGION 1 |  |  |
| Algeria | 79 | 0 |
| Angola | 0 | 0 |
| Botswana | 0 | 0 |
| Burkina Faso | 43 | 47 |
| Cabo Verde | 23 | 0 |
| Cameroon | 53 | 0 |
| Canary Islands (Spain) | 100 | 100 |
| Central African Republic | 0 | 0 |
| Chad | 0 | 0 |
| Congo | 45 | 0 |
| Cote D'ivoire | 100 | 0 |
| Democratic Republic Of The Congo | 0 | 0 |
| Egypt | 36 | 0 |
| Eritrea | 0 | 0 |
| Ethiopia | 8 | 0 |
| Ghana | 0 | 0 |
| Guinea | 0 | 0 |
| Kenya | 47 | 0 |
| Libya | 0 | 0 |
| Madagascar | 3 | 0 |
| Madeira | 50 | 50 |
| Malawi | 0 | 0 |
| Mali | 43 | 0 |
| Mauritania | 0 | 0 |
| Mauritius | 23 | 0 |
| Morocco | 2 | 0 |
| Mozambique | 0 | 0 |
| Namibia | 37 | 0 |
| Niger | 7 | 0 |
| Nigeria | 0 | 0 |
| Ocean Islands | 29 | 29 |
| Senegal | 63 | 48 |
| Somalia | 0 | 0 |
| South Africa | 83 | 83 |
| Sudan | 0 | 0 |
| Tunisia | 32 | 0 |
| United Republic Of Tanzania | 0 | 0 |
| Zambia | 0 | 0 |
| Zimbabwe | 23 | 0 |
| REGION 2 |  |  |
| Afghanistan, Islamic State Of | 20 | 0 |
| Bangladesh | 38 | 0 |
| China | 100 | 93 |
| Democratic People's Republic Of Korea | 20 | 0 |
| Hong Kong, China | 100 | 100 |
| India | 15 | 14 |
| Iran, Islamic Republic Of | 49 | 0 |
| Iraq | 0 | 0 |
| Japan | 100 | 100 |
| Kazakhstan | 100 | 0 |
| Kuwait | 43 | 0 |
| Maldives | 50 | 0 |
| Mongolia | 33 | 32 |
| Myanmar | 0 | 0 |
| Oman | 18 | 0 |
| Pakistan | 0 | 0 |
| Qatar | 0 | 0 |
| Republic Of Korea | 100 | 0 |
| Russian Federation | 98 | 0 |
| Saudi Arabia | 84 | 0 |
| Sri Lanka | 0 | 0 |
| Tajikistan | 0 | 0 |
| Thailand | 49 | 0 |
| Turkmenistan | 97 | 0 |
| United Arab Emirates | 90 | 0 |
| Viet Nam | 100 | 100 |
| REGION 3 |  |  |
| Argentina | 28 | 0 |
| Bolivia | 0 | 0 |
| Brazil | 66 | 59 |
| Chile | 59 | 28 |
| Colombia | 26 | 16 |
| Ecuador | 0 | 0 |
| French Guiana | 100 | 100 |
| Guyana | 0 | 0 |
| Islands (88: 800 - 998) | 47 | 47 |
| Peru | 0 | 0 |
| Venezuela | 0 | 0 |
| REGION 4 |  |  |
| Bahamas | 47 | 50 |
| Barbados | 50 | 43 |
| Belize | 50 | 50 |
| Bermuda | 100 | 100 |
| Canada | 98 | 44 |
| Cayman Islands | 47 | 47 |
| Colombia (San Andres And Providencia Islands) | 0 | 0 |
| Costa Rica | 0 | 0 |
| Curacao, St Maarten And Aruba (Nl) | 45 | 42 |
| Dominican Republic | 50 | 50 |
| Guadeloupe, St Martin, St Barthelemy And Other French Islands In The Vicinity | 0 | 0 |
| Jamaica | 33 | 33 |
| Mexico | 64 | 60 |
| Nicaragua | 0 | 0 |
| Panama | 20 | 20 |
| Puerto Rico And Us Possessions In The Caribbean Area | 100 | 100 |
| Trinidad And Tobago | 40 | 50 |
| United States Of America | 98 | 98 |
| United States Of America (Alaska) | 98 | 97 |
| REGION 5 |  |  |
| American Samoa | 97 | 97 |
| Australia | 49 | 47 |
| Brunei Darussalam | 50 | 0 |
| Cook Islands | 0 | 0 |
| Fiji | 100 | 83 |
| French Polynesia (Austral Islands) | 0 | 0 |
| French Polynesia (Marquesas Islands) | 50 | 50 |
| French Polynesia (Society Islands) | 100 | 100 |
| French Polynesia (Tuamotu Islands And Gambier Islands) | 50 | 50 |
| Indonesia | 96 | 92 |
| Islands (96: 995, 996) | 40 | 40 |
| Islands In The Pacific Ocean North Of The Equator | 89 | 84 |
| Kiribati | 23 | 3 |
| Malaysia | 95 | 0 |
| Nauru | 0 | 0 |
| New Caledonia | 100 | 100 |
| New Zealand | 85 | 88 |
| Papua New Guinea | 0 | 0 |
| Philippines | 98 | 98 |
| Singapore | 100 | 100 |
| Tuvalu | 50 | 10 |
| Vanuatu | 7 | 0 |
| REGION 6 |  |  |
| Armenia | 47 | 0 |
| Austria | 100 | 100 |
| Belgium | 57 | 0 |
| Bulgaria | 50 | 50 |
| Croatia | 100 | 97 |
| Cyprus | 23 | 3 |
| Czech Republic | 100 | 100 |
| Denmark And Faroe Islands | 100 | 100 |
| Estonia | 50 | 50 |
| Finland | 50 | 50 |
| France | 92 | 91 |
| Germany | 97 | 97 |
| Gibraltar | 0 | 0 |
| Greece | 47 | 14 |
| Greenland | 99 | 98 |
| Hungary | 100 | 28 |
| Iceland | 97 | 3 |
| Ireland | 100 | 100 |
| Israel | 100 | 100 |
| Italy | 100 | 100 |
| Jordan | 50 | 50 |
| Latvia | 23 | 0 |
| Lebanon | 0 | 0 |
| Lithuania | 0 | 0 |
| Netherlands | 57 | 57 |
| Norway | 73 | 67 |
| Poland | 97 | 97 |
| Portugal | 47 | 22 |
| Romania | 97 | 93 |
| Russian Federation | 97 | 12 |
| Serbia | 100 | 97 |
| Slovakia | 100 | 97 |
| Slovenia | 0 | 0 |
| Spain | 99 | 83 |
| Sweden | 73 | 73 |
| Switzerland And Liechtenstein | 100 | 100 |
| The Former Yugoslav Republic Of Macedonia | 0 | 0 |
| Turkey | 99 | 95 |
| Ukraine | 30 | 0 |
| United Kingdom Of Great Britain And Northern Ireland | 81 | 79 |
| ANTARCTICA |  |  |
| Stations In The Antarctic | 46 | 36 |

# CLIMAT surface

| Country/territory | Average of TAC | Average of TDCF |
| --- | --- | --- |
| REGION 1 |  |  |
| Algeria | 98 | 0 |
| Angola | 6 | 0 |
| Benin | 50 | 50 |
| Botswana | 0 | 0 |
| Bouvet Island | 0 | 0 |
| Burkina Faso | 100 | 100 |
| Burundi | 0 | 0 |
| Cabo Verde | 100 | 0 |
| Cameroon | 57 | 7 |
| Central African Republic | 92 | 0 |
| Chad | 75 | 8 |
| Comoros | 100 | 100 |
| Congo | 100 | 100 |
| Cote D'ivoire | 77 | 15 |
| Democratic Republic Of The Congo | 86 | 0 |
| Djibouti | 0 | 0 |
| Egypt | 0 | 0 |
| Equatorial Guinea | 100 | 0 |
| Eritrea | 0 | 0 |
| Ethiopia | 13 | 0 |
| Gabon | 83 | 0 |
| Gambia | 50 | 50 |
| Ghana | 53 | 0 |
| Guinea | 100 | 0 |
| Guinea-Bissau | 100 | 0 |
| Kenya | 0 | 0 |
| Lesotho | 0 | 0 |
| Liberia | 100 | 0 |
| Libya | 0 | 0 |
| Madagascar | 31 | 4 |
| Madeira | 100 | 67 |
| Malawi | 0 | 0 |
| Mali | 94 | 56 |
| Mauritania | 100 | 85 |
| Mauritius | 0 | 0 |
| Morocco | 82 | 0 |
| Mozambique | 0 | 0 |
| Namibia | 0 | 0 |
| Niger | 100 | 93 |
| Nigeria | 92 | 8 |
| Ocean Islands | 69 | 54 |
| Rwanda | 0 | 0 |
| Senegal | 100 | 100 |
| Seychelles | 0 | 0 |
| Sierra Leone | 100 | 0 |
| Somalia | 0 | 0 |
| South Africa | 18 | 0 |
| South Sudan | 0 | 0 |
| Spain (Canary Islands) | 100 | 100 |
| Spain (Ceuta And Melilla) | 100 | 100 |
| Sudan | 100 | 0 |
| Swaziland | 0 | 0 |
| Togo | 80 | 0 |
| Tunisia | 18 | 0 |
| Uganda | 0 | 0 |
| United Republic Of Tanzania | 33 | 0 |
| Western Sahara | 100 | 0 |
| Zambia | 0 | 0 |
| Zimbabwe | 11 | 0 |
| REGION 2 |  |  |
| Afghanistan, Islamic State Of | 0 | 0 |
| Bahrain | 0 | 0 |
| Bangladesh | 100 | 0 |
| Cambodia | 0 | 0 |
| China | 97 | 96 |
| Democratic People's Republic Of Korea | 86 | 0 |
| Hong Kong, China | 100 | 100 |
| India | 35 | 0 |
| Iran, Islamic Republic Of | 100 | 0 |
| Iraq | 0 | 0 |
| Japan | 100 | 100 |
| Kazakhstan | 0 | 0 |
| Kuwait | 0 | 0 |
| Kyrgyzstan | 0 | 0 |
| Lao People's Democratic Republic | 0 | 0 |
| Macao, China | 100 | 0 |
| Maldives | 50 | 0 |
| Mongolia | 100 | 0 |
| Myanmar | 0 | 0 |
| Nepal | 0 | 0 |
| Oman | 100 | 0 |
| Pakistan | 100 | 96 |
| Qatar | 0 | 0 |
| Republic Of Korea | 100 | 0 |
| Russian Federation | 12 | 0 |
| Saudi Arabia | 0 | 32 |
| Sri Lanka | 83 | 0 |
| Tajikistan | 0 | 0 |
| Thailand | 100 | 0 |
| Turkmenistan | 0 | 0 |
| United Arab Emirates | 0 | 0 |
| Uzbekistan | 0 | 0 |
| Viet Nam | 88 | 0 |
| Yemen | 0 | 0 |
| REGION 3 |  |  |
| Argentina | 100 | 100 |
| Bolivia | 0 | 0 |
| Brazil | 95 | 92 |
| Chile | 94 | 0 |
| Colombia | 100 | 0 |
| Ecuador | 71 | 14 |
| French Guiana | 100 | 100 |
| Guyana | 0 | 0 |
| Islands (88: 800 - 998) | 80 | 20 |
| Paraguay | 80 | 0 |
| Peru | 41 | 0 |
| Suriname | 0 | 0 |
| Uruguay | 89 | 0 |
| Venezuela | 97 | 0 |
| REGION 4 |  |  |
| Bahamas | 100 | 0 |
| Barbados | 0 | 0 |
| Belize | 100 | 0 |
| Bermuda | 100 | 0 |
| Canada | 95 | 0 |
| Cayman Islands | 0 | 0 |
| Colombia (San Andres And Providencia Islands | 100 | 0 |
| Costa Rica | 0 | 0 |
| Cuba | 100 | 0 |
| Curacao, St Maarten And Aruba (Nl) | 0 | 0 |
| Dominican Republic | 0 | 0 |
| El Salvador | 0 | 0 |
| Guadeloupe, St Martin, St Barthelemy And Other French Islands In The Vicinity | 100 | 100 |
| Guatemala | 0 | 0 |
| Honduras | 25 | 0 |
| Jamaica | 100 | 0 |
| Martinique | 100 | 100 |
| Mexico | 81 | 0 |
| Nicaragua | 100 | 0 |
| Panama | 0 | 0 |
| Puerto Rico And Us Possessions In The Caribbean Area | 100 | 0 |
| Trinidad And Tobago | 100 | 0 |
| United States Of America | 98 | 0 |
| United States Of America (Alaska) | 93 | 0 |
| REGION 5 |  |  |
| American Samoa | 100 | 0 |
| Australia | 54 | 97 |
| Brunei Darussalam | 100 | 0 |
| Cook Islands | 0 | 0 |
| Detached Islands (91 : 753, 754) | 100 | 100 |
| Detached Islands (91: 960 - 998) | 0 | 0 |
| Fiji | 100 | 0 |
| French Polynesia (Austral Islands) | 100 | 50 |
| French Polynesia (Marquesas Islands) | 100 | 100 |
| French Polynesia (Society Islands) | 100 | 100 |
| French Polynesia (Tuamotu Islands And Gambier Islands) | 100 | 100 |
| Indonesia | 77 | 74 |
| Islands (96: 995, 996) | 0 | 100 |
| Islands In The Pacific Ocean North Of The Equator | 85 | 0 |
| Kiribati | 0 | 0 |
| Malaysia | 100 | 0 |
| Nauru | 0 | 0 |
| New Caledonia | 100 | 100 |
| New Zealand | 0 | 0 |
| Niue | 100 | 0 |
| Papua New Guinea | 0 | 0 |
| Philippines | 88 | 88 |
| Phoenix Islands | 0 | 0 |
| Samoa | 0 | 0 |
| Singapore | 100 | 100 |
| Solomon Islands | 0 | 0 |
| Timor-Leste | 0 | 0 |
| Tokelau (And Swains Is.) | 0 | 0 |
| Tonga | 0 | 0 |
| Tuvalu | 0 | 0 |
| Vanuatu | 0 | 0 |
| REGION 6 |  |  |
| Albania | 0 | 0 |
| Armenia | 100 | 0 |
| Austria | 0 | 100 |
| Azerbaijan | 100 | 0 |
| Belarus | 0 | 92 |
| Belgium | 100 | 0 |
| Bosnia And Herzegovina | 100 | 0 |
| Bulgaria | 100 | 0 |
| Croatia | 0 | 100 |
| Cyprus | 50 | 50 |
| Czech Republic | 100 | 100 |
| Denmark And Faroe Islands | 100 | 0 |
| Estonia | 0 | 0 |
| Finland | 100 | 0 |
| France | 100 | 100 |
| Georgia | 100 | 0 |
| Germany | 7 | 100 |
| Gibraltar | 100 | 0 |
| Greece | 100 | 0 |
| Greenland | 100 | 0 |
| Hungary | 71 | 71 |
| Iceland | 83 | 0 |
| Ireland | 100 | 100 |
| Israel | 0 | 100 |
| Italy | 94 | 0 |
| Jordan | 100 | 0 |
| Kazakhstan (In Europe) | 0 | 0 |
| Latvia | 100 | 100 |
| Lebanon | 0 | 0 |
| Lithuania | 0 | 100 |
| Luxembourg | 0 | 0 |
| Malta | 100 | 0 |
| Montenegro | 0 | 0 |
| Netherlands | 67 | 67 |
| Norway | 0 | 100 |
| Poland | 100 | 100 |
| Portugal | 100 | 50 |
| Republic Of Moldova | 0 | 0 |
| Romania | 100 | 100 |
| Russian Federation | 13 | 0 |
| Serbia | 100 | 33 |
| Slovakia | 100 | 60 |
| Slovenia | 100 | 0 |
| Spain | 100 | 100 |
| Sweden | 0 | 100 |
| Switzerland And Liechtenstein | 88 | 13 |
| Syrian Arab Republic | 38 | 0 |
| The Former Yugoslav Republic Of Macedonia | 0 | 0 |
| Turkey | 97 | 97 |
| Ukraine | 7 | 0 |
| United Kingdom Of Great Britain And Northern Ireland | 0 | 95 |
| ANTARCTICA |  |  |
| Stations In The Antarctic | 69 | 44 |

- ANNEX TO PARAGRAPH 7.2.1

Migration Status:

| Country | Migration Status | Encoding Software | Remarks |
| --- | --- | --- | --- |
| Tanzania | Exchanging TDCF reports | Customized ECMWF decoder | BUFR SYNOP being received in Nairobi |
| Kenya | Exchanging TDCF bulletins | CLIMSOFT | TDCF bulletins being forwarded to Toulouse |
| Rwanda | Exchanging TDCF bulletins | CLIMSOFT | BUFR SYNOP being received in Nairobi |
| Uganda | TDCF data exchange not started |  | TDCF Training done |
| Burundi | TDCF data exchange not started | Planning to use CLIMSOFT | TDCF Training done |
| Malawi | TDCF data exchange not started |  | TDCF Training done |
| Botswana | Arrangement with Pretoria to implement TAC to BURF converter |  | TDCF Training done |
| Zambia | TDCF data exchange not started |  | TDCF Training done |
| Madagascar | AWS data being exchanged in BUFR | CLIMSOFT | BUFR AWS being exchanged between Antananarivo and Exeter |
| Zimbabwe | TDCF data exchange not started |  | TDCF Training done |

Migration Challenges

The TDCF migration challenges that have been identified are;

a)	Lack of TDCF encoding and transmission systems

Even after undergoing training and appreciating the importance of TDCF, several countries have been unable to proceed with the migration as they lack capacity to implement TDCF processing systems. The more affected are those that have been using communications means that are only capable of handling TAC data exchanges. Transfer of TDCF messages requires upgrading to more advanced means such as FTP. Some have implemented CLIMSOFT which can encode observations into TDCF but so far they have not been able to transmit the produced BUFR files to the responsible RTHs due to the same reasons. Examples are Uganda, Burundi and Malawi.

b)	Incapability of TDCF Bulletins Compilation in Message Switching Systems

Some message switching systems require upgrading in order to compile BUFR files into GTS bulletins. An example is the Nairobi messages switch which can only forward BUFR files but cannot compile them into bulletins. Application for compiling bulletins has been separately developed.

c)	Parallel Transmission

The requirement for parallel transmission has been seen as an extra work in those NMHS that have started TDC data exchange. In cases where staff capacity is low the newly implemented TDCF operations are given a lower priority thereby reversing the gains made towards full migration. Examples are Kenya and Rwanda.

Recommendations to Region I

a)	The RTCs in RA I should be encouraged to play a guiding role in identifying the suitable connectivity means that can be easily implemented for TDCF data exchange between the centres under their responsibilities.

b)	TAC-TDCF converters be implemented in the existing message switching systems to act as a temporary measure before a full migration is achieved in those NMHS that are far in their TDCF migration path.

c)	System developers be encouraged to develop TDCF processing applications that are simple and integrate well with the region’s data processing practices.

d)	Those countries that have started TDCF data exchange but have only partially migrated be requested to speed up the migration process so that all the data types are exchanged in TDCF.

- ANNEX TO PARAGRAPH 7.2.2

# [RA II]

RA-II Member activities related to TDCF

Several activities related to TDCF were reported by RA II Members since the last meeting in April 2014:

a)	Vietnam

Vietnam started disseminating SYNOP and TEMP in BUFR format in December 2014. GTS headings are: ISMC[01|02|40], ISIC[20|21|40], IUSC01 VNNN. It is in the process of developing encoding software for CLIMAT report.

b)	Bhutan

GTS connection between Bhutan and RTH (GISC) New Delhi, India was established in June 2015. Bhutan was the last RA II Member which had not been linked to GTS network. Bhutan will start disseminating AWS data of one station in BUFR format in near future.  The AWS data in CREX format is collected through satellite data collection system, brought back to the meteorological office and reformatted to BUFR and put into GTS.

c)	Location metadata errors of Bangladesh, Myanmar and Mongolia

Reported errors of location (latitude and longitude) metadata in BUFR surface reports of Myanmar and Mongolia were fixed. Software of Bangladesh meteorology department will be updated shortly to apply the correction.

Monitoring and Analysis of Migration Status

Monitoring method

Statistics were collected for the period of April 1 through 15, 2015. Resources were derived from the results of Special MTN Monitoring (SMM) pre-analysis and Integrated WWW Monitoring (IWM) created by WMC Melbourne/RTH Tokyo and from the latest version of the surface and upper-air (RBSN) station list of Regional Basic Synoptic Networks at the time of analysis.

In addition to WWW monitoring, the status of TDCF data communication is also monitored based on a catalogue created by GISC Tokyo (available at http://www.wis-jma.go.jp/csv/catalog.csv).

Migration progress and status

a)   SYNOP, TEMP and PILOT reports

The figures below show numerical representations of the progress of stations issuing BUFR-format bulletins equivalent to SYNOP and TEMP reports over the past two years. In the latest monitoring period from April 1 to 15, 2015, RTH Tokyo received (i) at least one surface synoptic observation report (excluding NIL reports) in BUFR format from 48% of RA II observation stations registered as part of RBSN (TAC format from 94%), and (ii) at least one upper-air sounding report in BUFR format from 43% of registered stations (TAC format from 87%). Sixteen BUFR reports equivalent to PILOT reports were received by RTH Tokyo in the monitoring period, while TAC bulletins were received from 17 stations.

Number of RA II RBSN stations issuing surface synoptic observation (SYNOP) and upper-air sounding (TEMP) reports in TAC and BUFR format from July 2013 to April 2015

The results were affected by the outage of RTH Bangkok which continued from mid-March to the beginning of May 2015. About 100 surface reports and 10 upper-air reports were not created and/or routed to RTH Tokyo because of the outage.

b) CLIMAT reports

As of July 2015, ten Members were reporting CLIMAT data in BUFR format: China; India; Mongolia; Saudi Arabia; Pakistan; Japan; Bangladesh; Hong Kong, China; Macao, China and Thailand.

c) Marine reports

As of July 2015, India (TESAC), Hong Kong, China (SHIP), Japan (TESAC, TRACKOB, SHIP) and Republic of Korea (TESAC) were routinely disseminating marine observation data in BUFR format.  Adoption of new templates for TESAC and BATHY is limited.

Upper-air reports in BUFR format

In RA II, seven Members are producing upper-air sounding in BUFR format at their own site:   China (converted from TEMP in parts), India (converted from TEMP in parts), Mongolia (converted from TEMP in parts), Hong Kong in China (converted from TEMP in parts), Japan (converted from TEMP in parts), Thailand (converted from TEMP and merged) and Vietnam (converted from TEMP in parts). No Member is reporting native high-resolution BUFR radiosonde data in RA-II.

# [China]

Issues raised with respect to the current TDCF tables

a)	There are still many elements that couldn’t be reported using existing BUFR/CREX Table B descriptors and have to be defined as local descriptors.

b)	Without vocabularies or sufficient definitions and explanations, it’s a big challenge to decide if the descriptor or the code figures in the code table are the exact ones meeting the requirement.

c)	For code tables of BUFR/CREX Table B descriptor, some has entries reserved for local use, but some doesn’t have and the new entry requirements have to be submitted to WMO.

d)	For some code tables like 0 02 081 (type of balloon), the existing code figure couldn’t meet the requirement.  It is therefore needed to request new entries, but without explanation or guide, it’s hard to study and imitate the universal way of coding just from the literal meaning of the existing code figures.

- ANNEX TO PARAGRAPH 7.2.3

[Status of migration in Brazil]

RTH Brasilia developed routine software to generate CLIMAT bulletins in BUFR directly from the data base without conversion, using the libecbufr libraries developed by Environmental Canada.

There is an automatic process to generate BUFR bulletins from the Automatic Weather Station network, which consists of more than 400 stations.  The SYNOP from manned stations are still being converted.  A process to generate BUFR report from the observation point is under implementation. It is expected to have at least 10% of the stations ready by the end of this year.

SYNOP, SHIP and BUOY bulletins received from other organization in Brazil, such as the Air Force and Navy, are converted into BUFR by the Message Switching System at RTH Brasilia.  Both alphanumeric and binary bulletins are inserted on the GTS.

The Air Force has a program to produce AMDAR bulletins directly from the airlines.  At the moment only the cruiser data (FIR Atlantic) are being reported in alphanumeric format.  Soon the ascent and descent data over the land will be reported, too.  It has been also promised to report everything in BUFR.

Regarding the radiosonde reports, most of the ground equipment was updated and they are able to report high resolution observation data, but many of them only report TEMP and PILOT bulletins which are later converted into BUFR at RTH Brasilia.  A small number of stations also send both types of report.  There are some levels of negotiations with the Air Force authorities to make the high resolution BUFR report available, too.

- ANNEX TO PARAGRAPH 7.2.4

Status of BUFR migration in North, Central America and the Caribbean on 9 July 2015

(Prepared by Mr Glendell De Sousa of the Caribbean Meteorological Organization)

| RA IV Members |  |  |
| --- | --- | --- |
| Country | Status | Comments |
| Antigua and Barbuda | No Information available |  |
| Bahamas | No Information available |  |
| Barbados | Migrated SYNOP, TEMP and CLIMAT |  |
| Belize | No Information available |  |
| British Caribbean Territories | Migrated SYNOP |  |
| Canada | SYNOP and partial TEMP/PILOT in parallel distribution |  |
| Colombia | Implementation started for SYNOP and CLIMAT |  |
| Costa Rica | Completed migration |  |
| Cuba | In progress 86% completed |  |
| Curaçao and Sint Maarten | Implementation started in Sint Maarten, No information for Curacao |  |
| Dominica | Migrated SYNOP |  |
| Dominican Republic | Migrated SYNOP |  |
| El Salvador | In progress 91% completed |  |
| Guatemala | In progress 73% completed |  |
| Haiti | No Information available |  |
| Honduras | In progress 73% completed |  |
| Jamaica | Migrated SYNOP |  |
| Mexico | Completed migration |  |
| Nicaragua | In progress 59% completed |  |
| Panama | In progress 86% completed |  |
| Saint Lucia | Migrated SYNOP |  |
| Trinidad and Tobago | Migrated SYNOP |  |
| United States of America | No Information available |  |
| Venezuela | No Information available |  |
| Countries which are not RA IV Members |  |  |
| Country | Status | Comments |
| Grenada | Implementation not started |  |
| Guyana | Migrated SYNOP |  |
| St. Kitts/Nevis | Implementation not started |  |
| St. Vincent and the Grenadines | Implementation not started |  |

Note: TEMP migration in Bahamas, Belize, Cayman Islands (BCT), Curacao and Sint Maarten, Dominican Republic, Jamaica and Trinidad and Tobago will only occur with the changes to hardware and software according to a schedule which is to be determined.

Status of migration in Canada

SYNOP

Canada currently produces BUFR SYNOP data for of its 871 SYNOP sites. The data is being sent in collected reports on the GTS, in parallel with alphanumeric data, under headers ISxx01-39 CWAO. The BUFR SYNOP data is encoded from a real-time central database of observational data (not converted from TAC SYNOP). The main issue is currently with the limited amount of the metadata required in template 3 07 080 that Canada is able to provide. For instance, Canada is able to provide barometer elevation but no other station or instrument elevation metadata at this time. Canada expects to be able to provide more station metadata in the BUFR SYNOP following upgrades to the station metadata systems.

Radiosondes

Currently, there is parallel BUFR data for 15 of 31 upper-air sites in Canada. The BUFR data is produced at the source, using vendor software, and is NOT converted from TAC.  It may, however, be of somewhat lower resolution than similarly produced data from other operators.  Most sites require upgrades to physical network software infrastructure in order to process and transmit BUFR data.  The available bulletins are distributed under the following headers:

IUJB01/IUKB01/IUSB01/IUWB01 CAYT – St. John’s

IUJB01/IUKB01/IUSB01/IUWB01 CWLW - Kelowna

IUJB01/IUKB01/IUSB01/IUWB01 CWMW - Maniwaki

IUJB01/IUKB01/IUSB01/IUWB01 CWEU - Eureka

IUJB01/IUKB01/IUSB01/IUWB01 CWLT - Alert

IUJB01/IUKB01/IUSB01/IUWB01 CWQI – Yarmouth

IUJB01/IUKB01/IUSB01/IUWB01 CWZC - Moosonee

IUJB01/IUKB01/IUSB01/IUWB01 CYJT - Stephenville

IUJB01/IUKB01/IUSB01/IUWB01 CYQD – The Pas

IUJB01/IUKB01/IUSB01/IUWB01 CYPL – Pickle Lake

IUJB01/IUKB01/IUSB01/IUWB01 CYRB - Resolute

IUJB01/IUKB01/IUSB01/IUWB01 CYVP – Kuujjuaq

IUJB01/IUKB01/IUSB01/IUWB01 CYXY – Whitehorse auto-launcher (not on circuit – 14-19 May 2015)

IUJB01/IUKB01/IUSB01/IUWB01 CYZV – Sept-Iles

IUJB01/IUKB01/IUSB01/IUWB01 CZXS – Prince George

Issues USA is experiencing in migration

a)	Lack of advanced notification when countries discontinue their TAC transmission.  This often leads to numerous inquiries and frantic remediation efforts by other countries that were not informed and therefore unprepared for the cessation of the TAC messages.

b)	Migrated BUFR data for certain countries is not always automatically made available or easily accessible.  USA has found some cases where they were receiving TAC TEMP and SYNOP reports from certain countries, but they were not receiving the corresponding migrated BUFR reports from those countries even though previous WMO monitoring periods had noted the availability of such reports.  Upon further investigation, USA learned that migrated BUFR reports from such countries were available, but those bulletins were not being made available to RTH Washington for internal dissemination to US customers.  USA believes all such bulletins should be automatically “pushed” onto the GTS for easy discoverability and access by other countries, rather than each country having to individually notice such discrepancies in their own data receipts and then initiate action to remedy those discrepancies.

c)	Improperly-encoded BUFR reports.  As many others have also noted, USA is finding numerous errors in migrated BUFR reports, including missing or erroneous metadata values, units-conversion errors when translating from TAC to BUFR (e.g. knots vs. m/s), missing significance qualifier values, reporting bit numbers for flag tables rather than the proper corresponding value, etc.  Should there be some minimum mandatory validation through which migrated BUFR reports from a country should be required to pass before that country is permitted to cease their corresponding TAC transmissions or otherwise declare their migration as being “complete”?  USA noted there were at least a few web-based portals available which perform such validations, and though it’s likely that some are more rigorous than others, their usage is entirely voluntary and there is no requirement that new products must pass through any such check before being released onto the GTS.  Should this be re-considered?

d)	One of the many original rationales for the migration to BUFR was that station location metadata would be available in each report alongside the data, rather than having to look up the metadata in a separate station table at each processing center based on an identifier in the TAC report.  However, some of the BUFR-encoding errors USA is seeing are in this metadata, e.g. latitude and longitude reported in degrees, minutes and seconds rather than being converted to fractional values, elevations reported in feet instead of meters, etc.  Furthermore, and despite these conversion issues, some countries apparently believe that the migration to BUFR means that they no longer need to maintain their station information in Volume A.  Until such conversion errors are a thing of the past, USA believes that countries should be directed to continue maintaining their correct information in Volume A and any successor references; otherwise, the ability to diagnose such metadata errors would be significantly degraded.

- ANNEX TO PARAGRAPH 7.2.6

Result of survey during 04/06-19/06.2015 (for CLIMAT data - since 01/06/2015)

a)	Still 7 of 52 RA VI Members demonstrate no evidence of any activity towards the Migration: Azerbaijan, Lebanon, Malta, Monaco, Republic of Moldova, Syrian Arab Republic, The former Yugoslav Republic of Macedonia.

b)	22 Members, from other side, are ready for the Migration: Austria, Belarus, Croatia, Cyprus, Czech Republic, Germany, Hungary, Ireland, Israel, Italy, the Netherlands, Norway, Poland, Portugal, Romania, Serbia, Slovakia, Spain, Sweden, Turkey and United Kingdom of Great Britain and Northern Ireland. Some of them already started ceasing TAC dissemination signifying thus decision point has been passed and the Migration is irreversible.

c)	Lots of wind profiler data are still exchanged using BUFR edition 3, while some are in BUFR edition 2.

- ANNEX TO PARAGRAPH 7.3/7.4

1.	Identified issues in encoding and decoding BUFR reports

### 1.1	Use of BUFR editions and templates

Edition 3 is used by wind profiler and AMDAR data; edition 2 is by wind profiler, but edition is not a matter that is hidden in the system; this has to be informed to producers who use the old editions of the problem.

### 1.2	Drift information in upper-air data

Noted

### 1.3	Metadata

#### 1.3.1	Observation station position, latitude/longitude, height

This should go to the check list; staff training issue; to check list.

#### 1.3.2	Identifier of upper-air data from ships

This is clearly stated in B/C regulations; not an operator training issue; problem of software.

#### 1.3.3	Instruments types and software versions

To check list

### 1.3.4	BUFR content errors due to conversion from TAC, etc.

To check list

### 1.4	Reformatted BUFR upper-air reports separated into parts

Noted (discussed in relevant items)

### 1.5	Use of BUFR reports in NWP systems

Noted

2	Identified issues in managing migration process

### 2.1	Lack of shared understanding of migration goals

It was pointed that conversion is the only intermediate solution for SYNOP; CBS concentrated on application of data rather than migration itself; a briefing pack for migration project to convince senior management could be made available for Members.

### 2.2	Lack of advanced notification of turning off TAC reporting

See summary of the item 7.3. Tools, i.e. METNO, focal points (including NWP centres one) and Operational Newsletter available for notification on migration were reviewed.

### 2.3	Lack of consolidated process for quality checking and reporting issues

It was suggested to undertake validation of upper-air BUFR data produced by manufactures of radiosonde systems.

### 2.4	Training and expertise

Importance of local training was highlighted, noting training organized by WMO.

### 2.5	Lack of capacity and infrastructure

The meeting noticed the role of CREX for Members with no binary communication infrastructures from the IPET-DRMM perspective.

3.	Recommendations

## 3.1	Recommended actions for all WMO Members

## 3.2	Recommendation on BUFR upper-air reports

### 3.3	Scheme for managing issues with representation of data

- ANNEX TO PARAGRAPH 8.1

(see Attachment)

- ANNEX TO PARAGRAPH 11.4

IPET-DRMM decided that:

1)	Someone will act as the team leader.

2)	It would invite ECMWF, UK Met Office, JMA and NOAA/NWS to provide an NWP expert for the team.

USA nominated Dr Patricia M. Pauley,

UK nominated Adam Maycock and Warren Tennant,

Japan nominated Mr Ota,

Canada nominated two persons,

China will nominate .

3)	Weiqing Qu, Kamel Brai and Richard Weedon from IPET-DRMM would participate directly in the work of the team

4)	Mr Alexander Kats contributes on issues associated with the operation of radiosonde systems.

UK nominated David Edwards.

5)	the team would provide by 1 September a plan for resolving the issue of reporting upper air information in BUFR.

[END OF ANNEX TO THE REPORT]

[NEW PROPOSALS THROUGH PFC]

The following lists new amendments proposed through the practice between meetings of IPET-DRMM.

1.	GRIB2 parameters for NCEP forecast products [FT2016-1]

Add entries:

In GRIB Code table 4.2,

Discipline 0 (Meteorological products), Category 19 (Physical atmospheric properties)

| Number | Parameter | Unit |
| --- | --- | --- |

29	Clear air turbulence (CAT)	m2/3 s–1

Discipline 0 (Meteorological products), Category 20 (Atmospheric chemical constituents)

| Number | Parameter | Unit |
| --- | --- | --- |

111	Angstrom exponent	Numeric

2.	GRIB2 parameters and level type for the uncertainties in ensembles of regional reanalyses project [FT2016-1]

Add entries:

In GRIB Code table 4.2,

Product discipline 0 - Meteorological products, parameter category 4: short-wave radiation

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 52 | Downward short-wave radiation flux, clear sky | W m–2 | Downward short-wave radiation flux computed under actual atmospheric conditions but assuming zero cloudiness. |
| 53 | Upward short-wave radiation flux, clear sky | W m–2 | Upward short-wave radiation flux computed under actual atmospheric conditions but assuming zero cloudiness. |

Product discipline 0 - Meteorological products, parameter category 5: long-wave radiation

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 8 | Downward long-wave radiation flux, clear sky | W m–2 | Downward long-wave radiation flux computed under actual atmospheric conditions but assuming zero cloudiness. |

Product discipline 2 - Land surface products, parameter category 3: soil products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 26 | Soil heat flux | W m–2 | The soil heat flux is the energy receive by the soil to heat it per unit of surface and time. The Soil heat flux is positive when the soil receives energy (warms) and negative when the soil loses energy (cools). |

Product discipline 1 - Hydrological products, parameter category 0: hydrology basic products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 16 | Percolation rate | kg m–2 s–1 | The percolation is the downward movement of water under hydrostatic pressure in the saturated zone. This water might still end up in rivers and lakes as discharge but it is a slower process than water runoff or drainage. Such defined percolation is an input for hydrological models together with e.g. water runoff. |

Product discipline 2 - Land surface products, parameter category 3: soil products

| Number | Parameter | Units | Description |
| --- | --- | --- | --- |
| 27 | Soil depth | m | Soil depth, positive downward. It is meant to be used together with the type of level "soil level" to encode the depth of the level at each grid point. |

In GRIB Code table 4.5,

| Code figure | Meaning | Units | Description |
| --- | --- | --- | --- |
| 151 | Soil level (see Note 5) | Numeric | This level represents a soil model level. The aim of this type of the level is to encode a field referred to a soil level that has variable depth across the model domain. The non-constant depth is then encoded as a parameter "soil depth" discipline 2, category 3 and parameter number 27. |

Add a Note:

In GRIB Code table 4.5,

Notes:

(5)	The soil level represents a model level for which the depth is not constant across the model domain. The depth in metres of the level is provided by another GRIB message with the parameter "soil depth" with discipline 2, category 3 and parameter number 27.

3.	GRIB2 parameters for new forecast and post-processing products [FT2016-1]

Add entries:

In GRIB Code table 4.2,

Product Discipline 0 – Meteorological products, Parameter category 1: moisture

| Number | Parameter | Units |
| --- | --- | --- |
| 109 | Mass density of liquid water coating on hail expressed as mass of liquid water per unit volume of air | kg m–3 |
| 110 | Specific mass of liquid water coating on hail expressed as mass of liquid water per unit mass of moist air | kg kg–1 |
| 111 | Mass mixing ratio of liquid water coating on hail expressed as mass of liquid water per unit mass of dry air | kg kg–1 |
| 112 | Mass density of liquid water coating on graupel expressed as mass of liquid water per unit volume of air | kg m–3 |
| 113 | Specific mass of liquid water coating on graupel expressed as mass of liquid water per unit mass of moist air | kg kg–1 |
| 114 | Mass mixing ratio of liquid water coating on graupel expressed as mass of liquid water per unit mass of dry air | kg kg–1 |
| 115 | Mass density of liquid water coating on snow expressed as mass of liquid water per unit volume of air | kg m–3 |
| 116 | Specific mass of liquid water coating on snow expressed as mass of liquid water per unit mass of moist air | kg kg–1 |
| 117 | Mass mixing ratio of liquid water coating on snow expressed as mass of liquid water per unit mass of dry air | kg kg–1 |

Product Discipline 0 – Meteorological products, Parameter category 17: electrodynamics

| Number | Parameter | Units |
| --- | --- | --- |
| 1 | Lightning potential index (LPI) (see Note) | J kg–1 |

Note:	Definition of LPI after Lynn et. al.:

Lynn, B., and Y. Yair, 2010: Prediction of lightning flash density with the WRF model, Adv. Geosci., 23, 11-16

Yair, Y.,  B. Lynn, C. Price, V. Kotroni, K. Lagouvardos, E. Morin, A. Mugnai, and M. Llasat, 2010:

Predicting the potential for lightning activity in Mediterranean storms based on the Weather

Research and Forecasting (WRF) model dynamic and microphysical fields, JGR, 115, D04205,

doi:10.1029/2008JD010868

Product Discipline 0 – Meteorological products, Parameter category 20: atmospheric chemical constituents

| #### Number | #### Parameter | #### Units |
| --- | --- | --- |
| #### 59 | #### Aerosol number concentration (see Note 2) | #### m–3 |
| #### 60 | #### Aerosol specific number concentration (see Note 2) | #### kg–1 |
| #### 61 | #### Maximum of mass density in layer (see Note 1) | #### kg m–3 |
| #### 62 | #### Height of maximum mass density | #### m |

Notes:

(1)	FirstFixedSurface and SecondFixedSurface of Code table 4.5 (Fixed surface types and units) to define the vertical extent, i.e. FirstFixedSurface can be set to 1 (Ground or water surface) and SecondFixedSurface set to 7 (Tropopause) for a restriction to the troposphere.

(2)	The term “number density” is used as well for “number concentration” (code number 59); conversion factor between “number density” (59) and “specific number concentration” (60) is “mass density” [kg m–3].

Amend entries:

In parameter numbers 1, 56 and 58 in Product Discipline 0 – Meteorological products, Parameter category 20: atmospheric chemical constituents of GRIB Code table 4.2,

"see Note" to "see Note 1"

Add a note to parameter number 10 and add entries:

In GRIB Code table 4.2,

Product Discipline 0 – Meteorological products, Parameter category 18: nuclear/radiology

| #### Number | #### Parameter | #### Units |
| --- | --- | --- |
| #### 10 | #### Air concentration (see Note 3) | #### Bq m–3 |
| #### 14 | #### Specific activity concentration (see Note 3) | #### Bq kg–1 |
| #### 15 | #### Maximum of air concentration in layer | #### Bq m–3 |
| #### 16 | #### Height of maximum air concentration | #### m |

Notes:

(3)	Conversion factor between “Specific activity concentration” (14) and “Air concentration” (10) is “mass density” [kg m–3].

Product Discipline 0 – Meteorological products, Parameter category 19: physical atmospheric properties

| #### Number | #### Parameter | #### Units |
| --- | --- | --- |
| #### 30 | #### Eddy dissipation parameter (see Note 3) | #### m2/3 s–1 |
| #### 31 | #### Maximum of Eddy dissipation parameter in layer | #### m2/3 s–1 |

Notes:

(3)	Eddy dissipation parameter is third root of eddy dissipation rate [m2 s–3].

In GRIB Code table 4.5,

| #### Code figure | #### Meaning | #### Unit |
| --- | --- | --- |
| #### 13 | #### Level of free convection (LFC) | #### - |
| #### 14 | #### Convective condensation level (CCL) | #### - |
| #### 15 | #### Level of neutral buoyancy or equilibrium level (LNB) | #### - |

[REVISIONS AND ADDITIONS IN RELATION TO IPET-DRMM-III]

The following lists amendments revised or additionally proposed in relation to the IPET-DRMM-III for approval.

Some amendments editorially corrected may not be shown here, e.g. center to centre, sea surface to sea-surface, dew point to dewpoint.

- ALTERNATIVE TO ANNEX TO PARAGRAPH 5.2 [ABC2016]

Proposed changes in B/C25:

(4)	If additional information on sounding is available, the sequence <3 09 052> shall be preceded by sequences suitable for reporting additional information on sounding systems.

(5)	If the sounding data are obtained from upper-air systems where pressure is derived from geopotential height by integration of hydrostatic equation, the geopotential calculation method shall be recorded using 0 02 191 within the preceding sequences.

[PROPOSALS UNDER VALIDATION]

FM 92 GRIB

- 2014-2.2.1(DRMM-II)/pre-operational from 7 May 2015/FT2015-2/valid from 4 November 2015 <para 4.1>

Add new entries:

Code table 4.2

| Parameter | Product discipline | Parameter category | Parameter number | Units |
| --- | --- | --- | --- | --- |
| Brightness temperature | 0 | 5 | 7 | K |
| Soil volumetric ice content (water equivalent) (see note) | 2 | 0 | 38 | m3 m–3 |
| Liquid water in snow pack | 2 | 3 | 23 | kg m–2 |
| Glacier temperature | 2 | New category: 5 (Glaciers and inland ice) | 1 | K |

Note: For parameter 38 (Parameter category 0), ice volume is expressed as if the ice content were melted to liquid water and then its volume measured in the liquid state. This may be understood in the same manner as water equivalent snow depth.

Code Table 4.5

| Code Figure | Meaning | Unit |
| --- | --- | --- |
| 174 | Top surface of ice on sea, lake or river | none |
| 175 | Top surface of ice, under snow cover, on sea, lake or river | none |
| 176 | Bottom surface (underside) ice on sea, lake or river | none |
| 177 | Deep soil (of indefinite depth) | none |
| 178 | Reserved |  |
| 179 | Top surface of glacier ice and inland ice | none |
| 180 | Deep inland or glacier ice (of indefinite depth) | none |
| 181 | Grid tile land fraction as a model surface | none |
| 182 | Grid tile water fraction as a model surface | none |
| 183 | Grid tile ice fraction on sea, lake or river as a model surface | none |
| 184 | Grid tile glacier ice and inland ice fraction as a model surface | none |

- 2014-2.2.2(DRMM-II)/A product definition template for statistics over an ensemble

<para 4.1>

Add a new template:

Product definition template 4.62 – Statistics over an ensemble reforecast, at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Forecast generating process identifier (defined by originating centre)

15	Indicator of unit of time range (see Code table 4.4)

16-19	Forecast time in units defined by octet 15 (see Note 1)

20	Type of first fixed surface (see Code table 4.5)

21	Scale factor of first fixed surface

22-25	Scaled value of first fixed surface

26	Type of second fixed surface (see Code table 4.5)

27	Scale factor of second fixed surface

28-31	Scaled value of second fixed surface

32	Type of ensemble forecast (see Code table 4.6)

33	Number of forecasts in ensemble

34	Number of years in the ensemble reforecast period (see Note 2)

35	First year of ensemble reforecast period

36	Last year of ensemble reforecast period

37	Total number of data values possible (or expected) in statistical process over the
 		ensemble reforecast

38-39	Total number of data values missing in statistical process over the ensemble
		reforecast

40	Statistical process used to calculate the processed field over the ensemble reforecast
		(see Code table 4.10)

41-42	Year of model version date (see Note 3)

43	Month of model version date

44	Day of model version date

45	Hour of model version date

46	Minute of model version date

47	Second of model version date

48	Month of end of overall time interval (see Note 5)

49	Day of end of overall time interval

50	Hour of end of overall time interval

51	Minute of end of overall time interval

52	Second of end of overall time interval

53	n - number of time range specifications describing the time intervals used

to calculate the statistically processed field

54-57	Total number of data values missing in statistical process

58-69	Specification of the outermost (or only) time range over which statistical

processing is done

58	Statistical process used to calculate the processed field from the field at

each time increment during the time range (see Code table 4.10)

59	Type of time increment between successive fields used in the statistical

processing (see Code table 4.11)

60	Indicator of unit of time for time range over which statistical processing is

done (see Code table 4.4)

61-64	Length of the time range over which statistical processing is done, in units

defined by the previous octet

65	Indicator of unit of time for the increment between the successive fields

used (see Code table 4.4)

66-69	Time increment between successive fields, in units defined by the previous

octet (see Note 3)

70-nn	These octets are included only if n>1, where nn=69 + 12 x n

70-81	As octets 58 to 69, next innermost step of processing

82-nn	Additional time range specifications, included in accordance with the value

of n. Contents as octets 58 to 69, repeated as necessary

Notes:

(1)	The reference time in section 1 and the forecast time together define the beginning of the overall time interval.

(2)	Octets 34-40 define a statistical process over both time and ensemble.

(3)	This is the date to identify the model version that is used to generate the reforecast.

(4)	An increment of zero means that the statistical processing is the result of a continuous (or near continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge. The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment (one of octets 59, 71. 83 ...).  For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast time.

- 2014-2.2.5(DRMM-II)/Review of GRIB templates for spatio-temporal changing tiles <para 4.1>

Add new entries:

Code table 4.0

| Octet No. | Contents |
| --- | --- |
| 55 | Spatio-temporal changing tiles at a horizontal level or horizontal layer at a point in time |
| 56 | Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters |

Add new templates:

Product definition template 4.55 – Spatio-temporal changing tiles at a horizontal level or horizontal layer at a point in time

| Octet No. | Contents |
| --- | --- |
| 10 | Parameter category (see Code table 4.1) |
| 11 | Parameter number (see Code table 4.2) |
| 12 | Tile classification (see Code table 4.242) |
| 13 | Number of used tiles (NUT) (see Notes 2 and 3) |
| 14 | Identification number of tile (ITN = {1,…, NUT}) (see Note 2) |
| 15 | Number of used tile attributes (NAT) for tile ITN (see Note 2) |
| 16 | Identification number of tile attribute (IAN = {1,…, NAT(ITN)}) (see Note 2) |
| 17 | Attribute of tile (see Code table 4.241) |
| 18 | Type of generating process (see Code table 4.3) |
| 19 | Background generating process identifier (defined by originating centre) |
| 20 | Analysis or forecast generating process identifier (defined by originating centre) |
| 21 – 22 | Hours of observational data cut-off after reference time (see Note) |
| 23 | Minutes of observational data cut-off after reference time |
| 24 | Indicator of unit of time range (see Code table 4.4) |
| 25 – 28 | Forecast time in units defined by octet 24 |
| 29 | Type of first fixed surface (see Code table 4.5) |
| 30 | Scale factor of first fixed surface |
| 31 – 34 | Scaled value of first fixed surface |
| 35 | Type of second fixed surface (see Code table 4.5) |
| 36 | Scale factor of second fixed surface |
| 37 – 40 | Scaled value of second fixed surface |

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	The number of used Tiles (NUT) is the number of used different tiles, defining the cover structure of a point. As each of these tiles do have one or more different tile attributes NAT(ITN)  ( ITN=1,…,NUT), e.g. unmodified, snow-covered,…), there are  fields with identification numbers ( ITN, IAN )  with the following meaning:

1,1	First tile – first attribute (e.g. unmodified)

….	….

1,NAT(1)	First tile – NAT of first tile (last, e.g. snow-covered) attribute

2,1	Second tile – first attribute (e.g. unmodified)

….	….

2,NAT(2)	Second tile – NAT of second tile (last, e.g. snow-covered) attribute

.	.

.	.

NUT,1	NUT tile – first attribute (e.g. unmodified)

….	...

NUT,NAT(NUT)	NUT tile – NAT of last tile (last) attribute

A single tile index (ITN, IAN) with code value ITN (1,…,NUT) and attribute identification IAN (1,…,NAT(ITN)) is represented in the template. All partitions are linked by the normalisation formula stating that the sum of all partitions must be equal to a normalisation term (N=1 for fractions and N=100 for percentage) on each point of the grid.

To get the tile structure of each grid point, the fields “tile class” and “tile fraction” have to be provided.

Product definition template 4.56 – Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for spatio-temporal changing tile parameters

| Octet No. | Contents |
| --- | --- |
| 10 | Parameter category (see Code table 4.1) |
| 11 | Parameter number (see Code table 4.2) |
| 12 | Tile classification (see Code table 4.242) |
| 13 | Number of used tiles (NUT) (see Notes 2 and 3) |
| 14 | Identification number of tile (ITN = {1,…, NUT}) (see Note 2) |
| 15 | Number of used tile attributes (NAT) for tile ITN (see Note 2) |
| 16 | Identification number of tile attribute (IAN = {1,…, NAT(ITN)}) (see Note 2) |
| 17 | Attribute of tile (see Code table 4.241) |
| 18 | Type of generating process (see Code table 4.3) |
| 19 | Background generating process identifier (defined by originating centre) |
| 20 | Analysis or forecast generating process identifier (defined by originating centre) |
| 21 – 22 | Hours of observational data cut-off after reference time (see Note) |
| 23 | Minutes of observational data cut-off after reference time |
| 24 | Indicator of unit of time range (see Code table 4.4) |
| 25 – 28 | Forecast time in units defined by octet 24 |
| 29 | Type of first fixed surface (see Code table 4.5) |
| 30 | Scale factor of first fixed surface |
| 31 – 34 | Scaled value of first fixed surface |
| 35 | Type of second fixed surface (see Code table 4.5) |
| 36 | Scale factor of second fixed surface |
| 37 – 40 | Scaled value of second fixed surface |
| 41 | Perturbation number |
| 42 | Number of forecasts in ensemble |

Notes:

(1)	Hours greater than 65534 will be coded as 65534.

(2)	See note (2) under product definition template 4.55.

- 2014-2.2.7(DRMM-II)/Rec. 7 (CBS-Ext.(2014))/valid from 2 November 2015 <para 4.1>

Add a new note:

GRIB Product definition template 4.44

Note: It is recommended not to use this template.  PDT 4.48 should be used instead with optical wave length range set to missing.

- 2013-2.1.1(DRMM-I)/Reporting quality information in GRIB2 <para 4.1>

Add new entries to Code table 4.10:

12	Confidence index (see Note 4)

13	Quality indicator (see Note 5 and Code table 4.244)

Editorial note: the new entry 12 is also used in the 2013-03.

Add new notes to Code table 4.10:

(4)	The original data value is a non-dimensional number from 0 to 1, where 0 indicates no confidence and 1 indicates maximal confidence.

(5)	The original data value is defined by Code table 4.244.

New Code table 4.244:

Quality indicator

Code
figure	        Meaning

0		No quality information available

1	Failed

2 	Passed

- 2013-2.2.1(DRMM-I)/GRIB2 new compression method – CCSDS libaec <para 4.1>

Note: For most templates, details of the packing process are described in regulation 92.9.4

| Data Representation Template 5.42 - Grid point and spectral data - CCSDS recommended lossless compression |  |
| --- | --- |
| Octet No. | Contents |
| 12 – 15 | Reference value (R) (IEEE 32-bit floating-point value) |
| 16 – 17 | Binary scale factor (E) |
| 18 – 19 | Decimal scale factor (D) |
| 20 | Number of bits required to hold the resulting scaled and referenced data values (see Note 1) |
| 21 | Type of original field values (see Code Table 5.1) |
| 22 | compression scheme version number of CCSDS 121.0-B recommended standard blue book (currently 2) (see Note 3) |
| 23 | compression options mask (see Note 3) |
| 24 | compression input/output bits per pixel (see Note 3) |
| 25 – 26 | compression input/output pixels per block (see Note 3) |
| 27 – 28 | compression input/output pixels per scan line (see Note 3) |
| 29 – 36 | length of the uncompressed grib message in octets |
| 37 – 40 | size of uncompressed data in octets |

Notes:

(1)		The intent of this template is to scale the grid point data to obtain the desired precision, if appropriate, and then subtract out reference value from the scaled field as is done using Data Representation Template 5.0.  After this, the resulting grid point field can be treated as a grayscale image and is then encoded into the CCSDS recommended standard for lossless data compression code stream format. To unpack the data field, the CCSDS recommended standard for lossless data compression code stream is decoded back into an image, and the original field is obtained from the image data as described in regulation 92.9.4 Note (4).

(2)		The Consultative Committee for Space Data Systems (CCSDS) recommended standard for lossless data compression is the standard used by space agencies for the compression of scientific data transmitted from satellites and other space instruments. CCSDS recommended standard for lossless data compression is a very fast predictive compression algorithm based on the extended-Rice algorithm. It uses Golomb-Rice codes for entropy coding. The sequence of prediction errors is divided into blocks. Each block is compressed using a two-pass algorithm. In the first pass the best coding method for the whole block is determined. In the second pass, output of the marker of the selected coding method as a side information is done along with prediction errors encoded.

The coding methods include:

  - Golomb-Rice codes of a chosen rank

  - Unary code for transformed pairs of prediction errors

  - Fixed-length natural binary code if the block is found to be incompressible

  - Signaling to the decoder empty block if all prediction errors are zeroes

(3)		 Consultative Committee for Space Data Systems: Lossless Data Compression.

CCSDS Recommendation for Space Data System Standards, 
CCSDS 121.0-B-2, Blue Book, May 2012.

| Data Template 7.42 - Grid point and spectral data - CCSDS recommended lossless compression. |  |
| --- | --- |
| Octet No. | Contents |
| 6 - nn | CCSDS recommended standard for lossless data compression code stream |

Code table 5.0 – Data representation template number

| Octet No. | Contents |
| --- | --- |
| 42 | Grid point and spectral data - CCSDS recommended lossless compression. |

- 2013-2.2.5(DRMM-I)/Proposed amendments to GRIB2 Code tables 4.2, 4.3 and 4.10 
(amendments other than Code table 4.10 were implemented by FT2014-1) <para 4.1>

Add new entries:

GRIB2 Code tables 4.2

| Code Table 4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Discipline | Parameter Category | Parameter number | Parameter | Units |
| 10 | 002 | 010 | Zonal vector component of vertically integrated ice internal pressure | Pa m |
| 10 | 002 | 011 | Meridional vector component of vertically integrated ice internal pressure | Pa m |
| 10 | 002 | 012 | Compressive ice Strength | N/m |
| 10 | 191 | 3 | Days since last observation | Day |
| 0 | 191 | 3 | Days since last observation | Day |

Add new entries:

GRIB2 Code tables 4.3

Code table 4.3 – Type of generating process

| Code figure | Meaning |
| --- | --- |
| 12 | Post-processed analysis |
| 13 | Post-processed forecast |

Note:	Code figures 12 and 13 are intended in cases where code figures 0 and 2 may not be sufficient to indicate that significant post-processing has taken place on an initial analysis or forecast output.

Add new entries: [validation]

GRIB2 Code tables 4.10

Code table 4.10 – Type of statistical processing

| Code figure | Meaning |
| --- | --- |
| 12 | Confidence index |

Note:	It is expected confidence will be a non-dimensional value from 0 to 1, where 0 indicates no confidence and 1 indicates maximal confidence. This may be used, for instance (but not exclusively), in cases where one wishes to show the horizontal distribution of confidence in analyzed values of a given field.

Editorial note: the new entry 12 is also used in the 2013-01.

Delete the Note (4) in Table 4.5 and renumber the Note (5).

Amend number 9 in Product discipline 10, parameter category 2 of Table 4.2 and add a new note to the category:

9	Module of ice internal pressure*	Pa m

*	Ice internal pressure or stress (Pa m) is the integrated pressure across the vertical thickness of a layer of ice. It is produced when concentrated ice reacts to external forces such as wind and ocean currents.

- 2012-2.2.9(DRC-IV)/GRIB template for 4-D Trajectory grid definition <para 4.1>

Validate templates:

Grid definition template 3.1010 – 4-D trajectory grid definition

Octet No.	Contents

15	Shape of the Earth (see Code table 3.2)

16	Scale factor of radius of spherical Earth

17–20	Scaled value of radius of spherical Earth

21	Scale factor of major axis of oblate spheroid Earth

22–25	Scaled value of major axis of oblate spheroid Earth

26	Scale factor of minor axis of oblate spheroid Earth

27–30	Scaled value of minor axis of oblate spheroid Earth

31–32	Number of horizontal points in slice (see Note 1)

33–34	Number of vertical points in slice (see Note 1)

35–38	Di – slice horizontal grid length (see Note 2)

39–42	Dj – slice vertical grid length (see Note 2)

43–46	Pi – horizontal location of trajectory point within slice (see Note 3)

47–50	Pj – vertical location of trajectory point within slice (see Note 3)

51	Scanning mode (flags – see Flag table 3.4) (see Note 1)

52–53	NW – Number of way points (see Note 4)

54–(53+NW×24)	Waypoint descriptions to define 4-D coordinates

Waypoint descriptions:

30+(N×24+0–N×24+3)	LaN – latitude of Nth trajectory way point

30+(N×24+4–N×24+7)	LoN – longitude of Nth trajectory way point

30+(N×24+8)	Type of Nth’s trajectory way point surface (see Code table 4.5) (see
		Note 5)

30+(N×24+9)	Scale factor of Nth’s trajectory way point surface

30+(N×24+10–N×24+13)	Scaled value of Nth’s trajectory way point surface

30+(N×24+14)	Indicator of unit of time range (see Code table 4.4)

30+(N×24+15–N×24+18)	Waypoint time in units defined by octet N×24+14 (see Note 6)

30+(N×24+19–N×24+22)	Number of slices per trajectory segment (see Note 7)

30+(N×24+23)	Number of additional leading and trailing slices (see Note 8)

Notes:

(1)		Horizontal and vertical points in slice indicate orthogonal grid slice perpendicular to point along the trajectory segment. Therefore scanning mode describes scanning within single slice.

(2)	Grid lengths are in units of 10–3 m. Trajectory is always positioned in the centre of the slice.

(3)	Location of trajectory point within slice is in units of 10–3 m relative from first grid point of this slice.

(4)	Type of line for way segments is assumed to be Great Circle.

(5)	Each of waypoints can be in different types of surface coordinates. For the purpose of light transition level, point of transition can be repeated in both meters above surface and isobaric level equivalent to height reduction based on QNH valid for FIR at that moment. Waypoint can be also repeated for stopover on the trajectory with same 3-D coordinates but different waypoint time. First point of transition level or stopover description can have MISSING slices in this case.

(6)	Waypoint time is relative to reference time of data defined in Section 1.

(7)	Slices are defined as perpendicular planes which are equidistantly spaced along the trajectory segment. First slice is always located in first point of trajectory segment and last slice in last point of trajectory segment. Therefore minimum number of slices is 2, unless set to MISSING (for last point of trajectory or for transition level repeated point).

Figure 3: Example of trajectory segment with 5 slices (circles represent trajectory waypoints)

(8)	Number of leading slices is same as number of trailing slices, and represents additional slices outside the trajectory segment but within its direction using same equidistant spacing as for corresponding trajectory segment itself.

Figure 4: Example of trajectory segment with 5 slices and 1 leading and trailing slice (circles represent trajectory waypoints)

(9)	A scaled value of radius of spherical Earth, or major or minor axis of oblate spheroid Earth, is derived by applying the appropriate scale factor to the value expressed in metres.

Product definition template 4.1010 – 4-D trajectory

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Analysis or forecast generating process identifier (defined by originating centre)

15–16	Hours of observational data cutoff after reference time (see Note)

17	Minutes of observational data cutoff after reference time

Note: Hours greater than 65534 will be coded as 65534.

Product definition template 4.1011 – 4-D trajectory ensemble forecast, control and perturbed

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating Centre)

14	Forecast generating process identifier (defined by originating Centre)

15–16	Hours after reference time of data cutoff (see Note)

17	Minutes after reference time of data cutoff

18	Type of ensemble forecast (see Code table 4.6)

19	Perturbation number

20	Number of forecasts in ensemble

Note: Hours greater than 65534 will be coded as 65534.

Product definition template 4.1015 – 4-D trajectory probability forecasts

Octet No.	Contents

10	Parameter category (see Code table 4.1)

11	Parameter number (see Code table 4.2)

12	Type of generating process (see Code table 4.3)

13	Background generating process identifier (defined by originating centre)

14	Forecast generating process identifier (defined by originating centre)

15–16	Hours after reference time of data cutoff (see Note)

17	Minutes after reference time of data cutoff

18	Forecast probability number

19	Total number of forecast probabilities

20	Probability type (see Code table 4.9)

21	Scale factor of lower limit

22–25	Scaled value of lower limit

26	Scale factor of upper limit

27–30 	Scaled value of upper limit

Note: Hours greater than 65534 will be coded as 65534.

- 2011-2.3.5(DRC-III)/GRIB template for rotated tilted Mercator projection <para 4.1>

Add new entry to Code table 3.1

Code table 3.1 - Grid definition template number

Code figure 	Meaning

11	Rotated Mercator projection

Add new Grid definition template

Product Definition Template 3.11:	Rotated Mercator projection

Octet Number(s)		Contents

15	Shape of the earth (see Code Table 3.2)

16	Scale factor of radius of spherical earth

17-20		Scaled value of radius of spherical earth

21		Scale factor of major axis of oblate spheroid earth

22-25		Scaled value of major axis of oblate spheroid earth

26		Scale factor of minor axis of oblate spheroid earth

27-30		Scaled value of minor axis of oblate spheroid earth

31-34		Ni - number of points along a parallel

35-38		Nj - number of points along a meridian

39-42		La1 - latitude of first grid point

43-46		Lo1 - longitude of first grid point

47		Resolution and component flags (see Flag Table 3.3)

48-51		LaD - Latitude(s) at which the Mercator projection intersects the Earth 						(Latitude(s) where Di and Dj are specified)

52-55		La2 - latitude of last grid point

56-59		Lo2 - longitude of last grid point

60		Scanning mode (flags - see Flag Table 3.4)

61-64		Orientation of the grid, angle between i direction on the map and the equator 
		(see Note 1)

65-68		Di - longitudinal direction grid length (see Note 2)

69-72		Dj - latitudinal direction grid length (see Note 2)

73-76		La0 - geographical latitude of the point to be brought to the origin of the 			projection, in the case of a rotation of the sphere prior to the projection

77-80		Lo0 - geographical longitude of the point to be brought to the origin of the 			projection, in the case of a rotation of the sphere prior to the projection

81-84		beta – tilting angle of the sphere around the origin point of the rotated 				sphere

85-nn		List of number of points along each meridian or parallel (These octets are 			only present for quasi-regular grids as described in notes 2 and 3 of GDT 			3.1)

Notes:

(1)	Limited to the range of 0 to 90 degrees; if the angle of orientation of the grid is neither 0 nor 	90 degrees, Di and Dj must be equal to each other.

(2)	Grid lengths are in units of 10-3 m, at the latitude specified by LaD.

(3)	A scaled value of radius of spherical Earth, or major or minor axis of oblate spheroid Earth is 	derived from applying appropriate scale factor to the value expressed in metres.

(4)	Transformation formulas from geographical (lat,lon)=(θ,λ) to projected grid point coordinates 	(x,y) :

Reverse transformation formulas from gridpoint (x,y) to (lat,lon):

Where:

- La0 is θ0

- Lo0 is λ0

- Beta is β

- x and y are metric coordinates in the i and j direction, in standard units (m). (x,y)=(0,0) corresponds to the coordinate of the reference point (La0,Lo0), provided this point is kept as the center of the grid point domain.

- the other variables are intermediate ones. More explanation can be found in the Technical Note by P. Bénard (2011), “rotated/tilted Mercator geometry in Aladin”.

- 2011-2.3.12(DRC-III)/Space Weather in GRIB2 <para 4.1>

Rename Code table 3.2 as “Shape of the reference system” and add the following new entries

Code table 3.2 – Shape of the reference system

Code figure	Meaning

10		Earth model assumed WGS84 with corrected geomagnetic coordinates (latitude and longitude) defined by Gustafsson et al., 1992

11	Sun assumed spherical with radius = 695,990,000 m (Allen, C.W., 1976 Astrophysical Quantities (3rd Ed.; London: Athlone)) and Stonyhurst latitude and longitude system with origin at the intersection of the solar central meridian (as seen from Earth) and the solar equator (Thompson, W, Coordinate systems for solar image data, A&A 449, 791–803 (2006)).

12	Sun assumed spherical with radius = 695,990,000 m (Allen, C.W., 1976 Astrophysical Quantities (3rd Ed.; London: Athlone)) and Carrington latitude and longitude system that rotates with a sidereal period of 25.38 days (Thompson, W, Coordinate systems for solar image data, A&A 449, 791–803 (2006)).

Add the following new entries to Code table 4.5

Code table 4.5 - Fixed surface types and units

Code figure	Meaning

170	Ionospheric D-region level

171	Ionospheric E-region level

172	Ionospheric F1-region level

173	Ionospheric F2-region level

In Code table 0.0, rename existing entry #3 as “Satellite remote sensing products” and add new entry #4

Code table 0.0 - Discipline of processed data in the GRIB message, number of GRIB Master
   	  table

Code figure	Meaning

3	Satellite remote sensing products

4	Space weather products

Add the following new entries to Code table 4.1

Code table 4.1 - Parameter category by product discipline

| Product Discipline 4 – Space Weather Products |  |
| --- | --- |
| Category (octet 10) | Description |
| 0 | Temperature |
| 1 | Momentum |
| 2 | Charged particle mass and number |
| 3 | Electric and magnetic fields |
| 4 | Energetic particles |
| 5 | Waves |
| 6 | Solar electromagnetic emissions |
| 7 | Terrestrial electromagnetic emissions |
| 8 | Imagery |
| 9 | Ion-neutral coupling |
| 10-191 | Reserved |
| 192-254 | Reserved for local use |
| 255 | Missing |

Add the following new entries to Code table 4.2

Code table 4.2 - Parameter number by product discipline and parameter category

Product discipline 4 – Space weather products, parameter category 0: Temperature

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Temperature | K |
| 1 | Electron temperature | K |
| 2 | Proton temperature | K |
| 3 | Ion temperature | K |
| 4 | Parallel temperature | K |
| 5 | Perpendicular temperature | K |
| 6-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 1: Momentum

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Velocity magnitude (Speed) | m s-1 |
| 1 | 1st vector component of velocity (coordinate system dependent) | m s-1 |
| 2 | 2nd vector component of velocity (coordinate system dependent) | m s-1 |
| 3 | 3rd vector component of velocity (coordinate system dependent) | m s-1 |
| 4-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 2: Charged particle mass and number

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Particle number density | m-3 |
| 1 | Electron density | m-3 |
| 2 | Proton density | m-3 |
| 3 | Ion density | m-3 |
| 4 | Vertical electron content | m-2 |
| 5 | HF absorption frequency | Hz |
| 6 | HF absorption | dB |
| 9 | Spread F | m |
| 10 | h’F | m |
| 11 | Critical frequency | Hz |
| 13 | Scintillation | Numeric |
| 14-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 3: Electric and magnetic fields

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Magnetic field magnitude | T |
| 1 | 1st vector component of magnetic field | T |
| 2 | 2nd vector component of magnetic field | T |
| 3 | 3rd vector component of magnetic field | T |
| 4 | Electric field magnitude | V m-1 |
| 5 | 1st vector component of electric field | V m-1 |
| 6 | 2nd vector component of electric field | V m-1 |
| 7 | 3rd vector component of electric field | V m-1 |
| 8-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 4: Energetic particles

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Proton flux (differential) | (m2 s sr eV)-1 |
| 1 | Proton flux (integral) | (m2 s sr )-1 |
| 2 | Electron flux (differential) | (m2 s sr eV)-1 |
| 3 | Electron flux (integral) | (m2 s sr)-1 |
| 4 | Heavy ion flux (differential) | (m2 s sr eV/nuc)-1 |
| 5 | Heavy ion flux (integral) | (m2 s sr)-1 |
| 6 | Cosmic ray neutron flux | h-1 |
| 7-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 5: Waves

| Number | Parameter | Units |
| --- | --- | --- |
| 0-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 6: Solar electromagnetic emissions

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Integrated Solar Irradiance | W m-2 |
| 1 | Solar X-ray Flux (XRS Long) | W m-2 |
| 2 | Solar X-ray Flux (XRS Short) | W m-2 |
| 3 | Solar EUV Irradiance | W m-2 |
| 5 | Solar Spectral Irradiance | W m-2 nm-1 |
| 6 | F10.7 | W m-2 Hz-1 |
| 7 | Solar radio emissions | W m-2 Hz-1 |
| 8-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 7: Terrestrial electromagnetic emissions

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Limb intensity | m-2 s-1 |
| 1 | Disk intensity | m-2 s-1 |
| 2 | Disk intensity day | m-2 s-1 |
| 3 | Disk intensity night | m-2 s-1 |
| 4-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 8: Imagery

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | X-ray radiance | W sr-1 m-2 |
| 1 | EUV radiance | W sr-1 m-2 |
| 2 | H-alpha radiance | W sr-1 m-2 |
| 3 | White light radiance | W sr-1 m-2 |
| 4 | CaII-K radiance | W sr-1 m-2 |
| 5 | White light coronagraph radiance | W sr-1 m-2 |
| 6 | Heliospheric radiance | W sr-1 m-2 |
| 7 | Thematic mask | numeric |
| 8-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

Product discipline 4 – Space weather products, parameter category 9: Ion-neutral coupling

| Number | Parameter | Units |
| --- | --- | --- |
| 0 | Pedersen conductivity | S m-1 |
| 1 | Hall conductivity | S m-1 |
| 2 | Parallel conductivity | S m-1 |
| 3-191 | Reserved |  |
| 192-254 | Reserved for local use |  |
| 255 | Missing |  |

- 2010-2.3.1/2(DRC-II)/Encoding wave spectra in GRIB Edition 2 Waves parameters (amendments other than 46-48 were implemented) <para 4.1>

The prognostic variable of a wave model is the distribution of wave energy with respect to frequency and direction, called the "Wave Spectrum".

The spectrum is a statistical quantity representing the average sea state of a certain region within a certain time period. However, this averaging is implicitly assumed and it is not applied in the post-processing of the model.

In order to reduce the number of degrees of freedom there exists a number of "integrated" parameters that characterize the spectrum. For example, the significant wave height is determined from the spectrum by integrating over all frequencies f (and directions)

Hs = 4 * sqrt(m0), where m0 = integral (f * E(f)) df

Hs approximates the average over the higher third of all waves that might be observed by a ship officer. Similarly, other integrated parameters like the "Mean Wave Period" or the "Maximum Height of an Individual Wave" can be derived (in a more or less complicated way) from the wave spectrum without any averaging in space or time.

Since wave models are not capable of describing the development of single waves it makes no sense to define a parameter "Height of an Individual Wave".

Thus, using expressions like "maximum" or "mean" in the spectral context is not contradictory to the GRIB2-orthonogality requirement.

The "Wave Directional Width" is a relative measure (like the relative humidity) and therefore dimensionless.

#### Add the new entries in Code table 4.2

Product discipline 10:	Oceanographic products, parameter category 0:	Waves

| Number | Parameter | Units |
| --- | --- | --- |
| 16 | Coefficient of drag with waves | - |
| 17 | Friction Velocity | m s-1 |
| 18 | Wave stress | N m-2 |
| 19 | Normalised Wave Stress | - |
| 20 | Mean square slope of waves | - |
| 21 | U-component surface Stokes drift | m s-1 |
| 22 | V-component surface Stokes drift | m s-1 |
| 23 | Period of maximum individual wave height | s |
| 24 | Maximum individual wave height | m |
| 25 | Inverse mean wave frequency | s |
| 26 | Inverse mean frequency of the wind waves | s |
| 27 | Inverse mean frequency of the total swell | s |
| 28 | Mean zero-crossing wave period | s |
| 29 | Mean zero-crossing period of the wind waves | s |
| 30 | Mean zero-crossing period of the total swell | s |
| 31 | Wave directional width | - |
| 32 | Directional width of the wind waves | - |
| 33 | Directional width of the total swell | - |
| 34 | Peak wave period | s |
| 35 | Peak period of the wind waves | s |
| 36 | Peak period of the total swell | s |
| 37 | Altimeter wave height | m |
| 38 | Altimeter corrected wave height | m |
| 39 | Altimeter range relative correction | - |
| 40 | 10 metre neutral wind speed over waves | m s-1 |
| 41 | 10 metre wind direction over waves | degrees |
| 42 | Wave energy spectrum | m2 s radian-1 |
| 43 | Kurtosis of the sea surface elevation due to waves | - |
| 44 | Benjamin-Feir index | - |
| 45 | Spectral peakedness factor | s-1 |
| 46 | 2-dim Spectral Energy Density E(f,θ) | m² s |
| 47 | Frequency Spectral Energy Density E(f) = ∫E(f,θ)dθ | m² s |
| 48 | Directional Spectral Energy Density E(θ)= ∫E(f,θ)df / m0 | - |
| 49-254 | Reserved |  |
| 255 | Missing |  |

Product discipline 10:  Oceanographic products, parameter category 4: Sub-surface properties

Number	Parameter		Units

7	Bathymetry		m

Add the new note to the Product discipline 10, parameter category 0 in Code table 4.2

*	Further information concerning the wave parameters can be found in WMO Publication No 702, “Guide to Wave Analysis and Forecasting (http://www.wmo.int/pages/prog/amp/mmop/documents/WMO%20No%20702/WMO702.pdf)

FM 94 BUFR/FM 95 CREX

- 2014-3.1.2(DRMM-II)/Rec. 7 (CBS-Ext.(2014))/valid from 4 November 2015 <para 4.1>

Amend Regulations:

94.1.5	Missing values shall be expressed by all bits set to 1 within the data width of the element. This shall apply to all Table B elements, including elements defined as CCITT IA5, code tables and flag tables, with the exception of data description operator qualifiers in Class 31.

Note: Flag tables are always augmented to contain an additional bit as the least significant bit of the table.  All bits, including this additional bit, shall be set to 1 to express a missing value, but in all other cases this additional bit shall be set to 0. This note does not apply to Data present indicator 0 31 031.

Amend Regulations:

94.1.6	The convention for representing missing data for compressed data within the binary Data section shall be to set the corresponding increments to fields of all bits set to 1.

94.1.7	When a local reference value for a set of element values for compressed data is represented as all bits set to 1, this shall imply that all values in the set are missing.

94.6.3	...

Notes: ...

(2)	...

(iii)	If all values of an element are missing, R0 shall be coded with all bits set to 1;

(vi)	Missing values will be denoted by setting all bits of the corresponding I to 1;

- 2014-3.2.7(DRMM-II)/BUFR template for n-minute AWS data (3 07 092) <combined with 2013-3.2.1(DRMM-I)> <para 4.1>

Add new entries:

BUFR Table D

|  |  | (Temperature and humidity instrumentation) |  |
| --- | --- | --- | --- |
| 3 01 130 | 0 03 002 | Generic type of humidity instrument |  |
|  | 0 03 003 | Configuration of sensors |  |
|  | 0 03 004 | Type of shield or screen |  |
|  | 0 03 005 | Horizontal width of screen or shield (x) |  |
|  | 0 03 006 | Horizontal depth of screen or shield (y) |  |
|  | 0 03 007 | Vertical height of screen or shield (z) |  |
|  | 0 03 008 | Artificially ventilated screen or shield |  |
|  | 0 03 009 | Amount of forced ventilation at time of reading |  |

BUFR/CREX Table B

| F X Y | Element Name | BUFR Units | BUFR Scale | BUFR Ref. | BUFR Width | CREX Units | CREX Scale | CREX Width |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 03 001 | Surface station type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 002 | Generic type of humidity instrument | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 003 | Configuration of sensors | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 03 004 | Type of shield or screen | Code table | 0 | 0 | 5 | Code table | 0 | 2 |
| 0 03 005 | Horizontal width of screen or shield (x) | m | 2 |  |  | m | 2 |  |
| 0 03 006 | Horizontal depth of screen or shield (y) | m | 2 |  |  | m | 2 |  |
| 0 03 007 | Vertical height of screen or shield (z) | m | 2 |  |  | m | 2 |  |
| 0 03 008 | Artificially ventilated screen or shield | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 03 009 | Amount of forced ventilation at time of reading | m-3 s-1 | 1 |  |  | m-3 s-1 | 1 |  |

Code tables

0 03 001  Surface station type

Code figure

0	Land station (synoptic network)

1	Shallow water station (fixed to sea/lake floor)

2	Ship

3	Rig/platform

4	Moored buoy

5	Drifting buoy (or drifter)

6	Ice buoy

7	Land station (local network)

8	Land vehicle

9	Autonomous marine vehicle

10-14	Reserved

15	Missing value

Notes:

(1)	The last three categories are for possible future use.

(2)	"Land station (local network)" distinguishes "non-synoptic" stations these aren't currently distributed on the GTS but might be stored in BUFR in the future.

(3)	There could be separate tables for marine and land stations but there are marginal cases (shallow water fixed stations in the southern North Sea and some rigs reporting in SYNOP code).

0 03 002   Generic type of humidity instrument

Code figure

0	Psychrometer [too generic]

1	Capacitive sensor (unheated)

2	Capacitive sensor (heated)

3	Resistive sensor [too generic]

4	Ordinary human hair

5	Rolled hair

6	Goldbeater’s skin

7	Chilled mirror hygrometer

8	Dew cell

9	Optical absorption sensor

10-14	Reserved

15	Missing value

0 03 003  Configuration of Sensors [already full]

Code figure

0	Solar radiation shield or screen (double v section louvers)

1	No solar radiation shield or screen

2	Solar radiation shield or screen (single v section louvers)

3	Solar radiation shield or screen (overlapping louvers)

4	Solar radiation shield or screen (non-overlapping louvers)

5	Solar radiation shield or screen (not louvered)

6	Integrated e.g. chilled mirror

7	Missing value

0 03 004 Type of Shield or Screen [entries need to be checked]

Code figure

0	Within Stevenson screen (wooden)

1	Within Stevenson screen (plastic)

2	Within marine Stevenson screen (wooden)

3	Within marine Stevenson screen (plastic)

4	Within cylindrical section plate shield (metal)

5	Within cylindrical section plate shield (wooden)

6	Within cylindrical section plate shield (plastic)

7	Within concentric tube (metal)

8	Within concentric tube (wooden)

9	Within concentric tube (plastic)

10	Within rectangular section shield (metal)

11	Within rectangular section shield (wooden)

12	Within rectangular section shield (plastic)

13	Within rectangular section shield (metal)

14	Within square section shield (wooden)

15	Within square section shield (plastic)

16	Within square section shield (metal)

17	Within triangular section shield (wooden)

18	Within triangular section shield (plastic)

19	Within triangular section shield (metal)

20	Within open covered lean-to (reed/grass/leaf)

21	Within open covered inverted v roof (reed/grass/leaf)

22-29	Reserved

30	Not Applicable, e.g. chilled mirror manufacturers enclosure

31	Missing value

0 03 008 Artificially Ventilated Screen or Shield

Code figure

0	Natural ventilation in use

1	Artificial aspiration in use: constant flow at time of reading

2	Artificial aspiration in use: variable flow at time of reading

3 -6	Reserved

7	Missing value

Add a template:

BUFR template for surface observations from n-minute period

# TM 307092

|  |  | National station identification |  |
| --- | --- | --- | --- |
| 3 01 089 | 0 01 101 | State identifier (1) | Code table, 0 |
|  | 0 01 102 | National station number (1) | Numeric, 0 |
|  |  | Fixed surface station identification; time, horizontal and vertical co-ordinates |  |
| 3 01 090 | 3 01 004 | Surface station identification |  |
|  |  | WMO block number | Numeric, 0 |
|  |  | WMO station number | Numeric, 0 |
|  |  | Station or site name | CCITT IA5, 0 |
|  |  | Type of station | Code table, 0 |
|  | 3 01 011 | Year(2) | Year, 0 |
|  |  | Month(2) | Month, 0 |
|  |  | Day(2) | Day, 0 |
|  | 3 01 012 | Hour(2) | Hour, 0 |
|  |  | Minute(2) | Minute, 0 |
|  | 3 01 021 | Latitude (high accuracy) | Degree, 5 |
|  |  | Longitude (high accuracy) | Degree, 5 |
|  | 0 07 030 | Height of station ground above mean sea level | m, 1 |
|  | 0 07 031 | Height of barometer above mean sea level | m, 1 |
| 0 03 001 |  | Surface station type | Code table, 0 |
| 0 08 010 |  | Surface qualifier (for temperature data) | Code table, 0 |
| 3 01 091 |  | Surface station instrumentation |  |
|  | 0 02 180 | Main present weather detecting system | Code table, 0 |
|  | 0 02 181 | Supplementary present weather sensor | Flag table, 0 |
|  | 0 02 182 | Visibility measurement system | Code table, 0 |
|  | 0 02 183 | Cloud detection system | Code table, 0 |
|  | 0 02 184 | Type of lightning detection sensor | Code table, 0 |
|  | 0 02 179 | Type of sky condition algorithm | Code table, 0 |
|  | 0 02 186 | Capability to detect precipitation phenomena | Flag table, 0 |
|  | 0 02 187 | Capability to detect other weather phenomena | Flag table, 0 |
|  | 0 02 188 | Capability to detect obscuration | Flag table, 0 |
|  | 0 02 189 | Capability to discriminate lightning strikes | Flag table, 0 |
| 0 04 015 |  | Time increment (= - n minutes) | Minute, 0 |
| 0 04 065 |  | Short time increment ( = 1 minute) | Minute, 0 |
| 1 33 000 |  | Delayed replication of 33 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor (= n) | Numeric, 0 |
| 0 10 004 |  | Pressure | Pa, –1 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 3 02 070 |  | Wind data |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 11 001 | Wind direction | Degree true, 0 |
|  | 0 11 002 | Wind speed | m s-1, 1 |
|  | 0 11 043 | Maximum wind gust direction | Degree true, 0 |
|  | 0 11 041 | Maximum wind gust speed | m s-1, 1 |
|  | 0 11 016 | Extreme counterclockwise wind direction of a variable wind | Degree true, 0 |
|  | 0 11 017 | Extreme clockwise wind direction of a variable wind | Degree true, 0 |
|  |  | Temperature and humidity instrumentation |  |
| 3 01 130 | 0 03 002 | Generic type of humidity instrument | Code table, 0 |
|  | 0 03 003 | Configuration of sensors | Code table, 0 |
|  | 0 03 004 | Type of Shield or Screen | Code table, 0 |
|  | 0 03 005 | Horizontal Width of Screen or Shield (x) | m, 2 |
|  | 0 03 006 | Horizontal Depth of Screen or Shield (y) | m, 2 |
|  | 0 03 007 | Vertical Height of Screen or Shield (z) | m, 2 |
|  | 0 03 008 | Artificially Ventilated Screen or Shield | Code table, 0 |
|  | 0 03 009 | Degree of Forced Ventilation at time of reading | m-3 s-1, 1 |
|  | 0 33 003 | Quality of humidity measurement | Code table, 0 |
|  |  | Temperature and humidity data |  |
| 3 02 072 | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 12 101 | Temperature/Air-temperature (scale 2) | K, 2 |
|  | 0 12 103 | Dew-point temperature (scale 2) | K, 2 |
|  | 0 13 003 | Relative humidity | %, 0 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 001 |  | Delayed descriptor replication factor | Numeric, 0 |
| 0 07 032 |  | Height of sensor above local ground(6) | m, 2 |
| 0 08 010 |  | Surface qualifier | Code table, 0 |
| 0 12 120 |  | Ground temperature | K, 2 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 0 08 010 |  | Surface qualifier (set to missing to cancel the previous value) | Code table, 0 |
| 1 03 000 |  | Delayed replication of 3 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 1 01 005 |  | Replicate 1 descriptor five times |  |
| 3 07 063 | 0 07 061 | Depth below land surface | m, 2 |
|  | 0 12 130 | Soil temperature (scale 2) | K, 2 |
| 0 07 061 |  | Depth below land surface (set to missing to cancel the previous value) | m, 2 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 069 |  | Visibility data |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 07 033 | Height of sensor above water surface | m, 1 |
|  | 0 33 041 | Attribute of following value | # Code table, 0 |
|  | 0 20 001 | Horizontal visibility | m, –1 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 0 07 033 |  | Height of sensor above water surface (set to missing to cancel the previous value) | m, 1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 073 |  | Cloud data |  |
|  | 0 20 010 | Cloud cover (total) | %, 0 |
|  | 1 05 004 | Replicate 5 descriptors four times |  |
|  | 0 08 002 | Vertical significance | Code table, 0 |
|  | 0 20 011 | Cloud amount | Code table, 0 |
|  | 0 20 012 | Cloud type | Code table, 0 |
|  | 0 33 041 | Attribute of following value | Code table, 0 |
|  | 0 20 013 | Height of base of cloud | m, –1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 076 |  | Precipitation, obscuration and other phenomena |  |
|  | 0 20 021 | Type of precipitation | Flag table, 0 |
|  | 0 20 022 | Character of precipitation | # Code table, 0 |
|  | 0 26 020 | Duration of precipitation(3) | Minute, 0 |
|  | 0 20 023 | Other weather phenomena | Flag table, 0 |
|  | 0 20 024 | Intensity of phenomena | Code table, 0 |
|  | 0 20 025 | Obscuration | Flag table, 0 |
|  | 0 20 026 | Character of obscuration | Code table, 0 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 13 155 |  | Intensity of precipitation | kgm-2s-1, 4 |
| 0 13 058 |  | Size of precipitation element | m, 4 |
|  |  | (end of the replicated sequence) |  |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 20 031 |  | Ice deposit (thickness) | m, 2 |
| 0 20 032 |  | Rate of ice accretion | Code table, 0 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 078 |  | State of ground and snow depth measurement |  |
|  | 0 02 176 | Method of state of ground measurement | Code table, 0 |
|  | 0 20 062 | State of ground (with or without snow) | Code table, 0 |
|  | 0 02 177 | Method of snow depth measurement | Code table, 0 |
|  | 0 13 013 | Total snow depth | m, 2 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 079 |  | Precipitation measurement |  |
|  | 0 07 032 | Height of sensor above local ground | m, 2 |
|  | 0 02 175 | # Method of precipitation measurement | Code table, 0 |
|  | 0 02 178 | Method of liquid water content measurement of Precipitation | Code table, 0 |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 13 011 | Total precipitation / total water equivalent of snow | kg m-2, 1 |
| 0 07 032 |  | Height of sensor above local ground (set to missing to cancel the previous value) | m, 2 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 080 |  | Evaporation measurement |  |
|  | 0 02 185 | # Method of evaporation measurement | Code table, 0 |
|  | 0 04 025 | Time period or displacement ( = - n minutes) | Minute, 0 |
|  | 0 13 033 | Evaporation /evapotranspiration | kg m-2, 1 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 081 |  | Total sunshine data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 031 | Total sunshine | Minute, 0 |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 082 |  | Radiation data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 14 002 | Long-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 004 | Short-wave radiation, integrated over period specified | J m-2, -3 |
|  | 0 14 016 | Net radiation, integrated over period specified | J m-2, -4 |
|  | 0 14 028 | Global solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
|  | 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified | J m-2, -2 |
| 1 02 000 |  | Delayed replication of 2 descriptors |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 0 04 025 |  | Time period (= - n minutes) | Minute |
| 0 13 059 |  | Number of flashes | Numeric |
| 1 01 000 |  | Delayed replication of 1 descriptor |  |
| 0 31 000 |  | Short delayed descriptor replication factor | Numeric, 0 |
| 3 02 083 |  | First order statistics of P, W, T, U data |  |
|  | 0 04 025 | Time period (= - n minutes) | Minute, 0 |
|  | 0 08 023 | First order statistics (= 9; best estimate of standard deviation) (4) | Code table, 0 |
|  | 0 10 004 | Pressure | Pa, –1 |
|  | 0 11 001 | Wind direction | Degree true, 0 |
|  | 0 11 002 | Wind speed | m s-1, 1 |
|  | 0 12 101 | Temperature/air temperature (scale 2) | K, 2 |
|  | 0 13 003 | Relative humidity | %, 0 |
|  | 0 08 023 | First order statistics (= missing value) | Code table, 0 |
| 0 33 005 |  | Quality information (AWS data) | Flag table, 0 |
| 0 33 006 |  | Internal measurement status information (AWS) | Code table, 0 |

Notes:

(1)	0 01 101 (WMO Member State identifier) and 0 01 102 (National AWS number) shall be used to identify a station within the national numbering system that is completely independent of the WMO international numbering system. The WMO international identification 0 01 001 (WMO block number) and 0 01 002 (WMO station number) shall be reported if available for the particular station.

(2)	The time identification refers to the end of the n-minute period.

(3)	Duration of precipitation (in minutes) represents number of minutes in which any precipitation was registered.

(4)	Best estimate of standard deviation is counted out of a set of samples (signal measurements) recorded within the period specified; it should be reported as a missing value, if the measurements of the relevant element are not available from a part of the period specified by 0 04 025.

(5)	If reporting nominal values is required, the template shall be supplemented with 3 07 093.

(6)	The height above local ground 0 07 032 referring to ground temperature shall be considered as a variable. After a snowfall, the sensor is placed at the top of the snow layer and the changed value of 0 07 032 shall indicate this procedure (total snow depth is reported in 0 13 013).

- 2014-3.2.8(DRMM-II)/Revised BUFR sequence for synoptic reports from sea stations suitable for VOS observation data <para 4.1>

Add new entries:

BUFR/CREX Table B

| Table reference | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F XX YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (char) |
| 0 01 114 | Encrypted ship or mobile land station identifier | CCITT IA5 | 0 | 0 | 352 | CCITT IA5 | 0 | 44 |
| 0 03 014 | Type of marine hygrometer | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 015 | Exposure of marine thermometer / hygrometer | Code table | 0 | 0 | 4 | Code table | 0 | 2 |
| 0 03 013 | Type of marine thermometer | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 10 015 | Maximum height of deck cargo above summer load line | m | 0 | 0 | 6 | m | 0 | 2 |
| 0 10 016 | Departure of reference level (summer maximum load line) from actual sea level | m | 0 | -32 | 6 | m | 0 | 3 |
| 0 11 007 | Relative wind direction (in degrees off bow) | ° | 0 | 0 | 9 | ° | 0 | 3 |
| 0 11 008 | Relative wind speed | m s-1 | 1 | 0 | 12 | m s-1 | 1 | 4 |
| 0 25 185 | Encryption method | Code table | 0 | 0 | 8 | Code table | 0 | 3 |
| 0 25 186 | Encryption key version | CCITT IA5 | 0 | 0 | 96 | CCITT IA5 | 0 | 12 |

BUFR/CREX Table D

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Encrypted ship's call sign and encryption method) |  |
| 3 | 01 | 018 | 0 | 01 | 114 | Encrypted ship or mobile land station identifier |  |
|  |  |  | 0 | 25 | 185 | Encryption method |  |
|  |  |  | 0 | 25 | 186 | Encryption key version |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship “instantaneous” data) |  |
| 3 | 02 | 062 | 3 | 02 | 001 | Pressure and 3-hour pressure change |  |
|  |  |  | 3 | 02 | 093 | Extended ship temperature and humidity data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 053 | Ship visibility data |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface | Set to missing (cancel) |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 004 | General cloud information |  |
|  |  |  | 3 | 02 | 005 | Cloud layer |  |
|  |  |  | 0 | 08 | 002 | Vertical significance (surface observations) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 055 | Icing and ice |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 056 | Sea/water temperature |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 021 | Waves |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 024 | Wind and swell waves |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship “period” data) |  |
| 3 | 02 | 063 | 3 | 02 | 038 | Present and past weather |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 040 | Precipitation measurement |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 034 | Precipitation past 24 hours |  |
|  |  |  | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 058 | Ship extreme temperature data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 064 | Ship wind data |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Ship wind data) |  |
| 3 | 02 | 064 | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 0 | 02 | 002 | Type of instrumentation for wind measurement |  |
|  |  |  | 0 | 08 | 021 | Time significance | = 2 Time averaged |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 001 | Wind direction |  |
|  |  |  | 0 | 11 | 002 | Wind speed |  |
|  |  |  | 0 | 08 | 021 | Time significance | Set to missing (cancel) |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 0 | 04 | 025 | Time period or displacement |  |
|  |  |  | 0 | 11 | 043 | Maximum wind gust direction |  |
|  |  |  | 0 | 11 | 041 | Maximum wind gust speed |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (VOSClim data elements) |  |
| 3 | 02 | 092 | 0 | 01 | 012 | Direction of motion of moving observing platform | Ship’s heading |
|  |  |  | 0 | 01 | 012 | Direction of motion of moving observing platform | Ship’s course over ground |
|  |  |  | 0 | 01 | 013 | Speed of motion of moving observing platform | Ship’s speed over ground |
|  |  |  | 0 | 10 | 038 | Maximum height of deck cargo above summer load line |  |
|  |  |  | 0 | 10 | 039 | Departure of reference level (summer maximum load line) from actual sea level |  |
|  |  |  | 0 | 11 | 007 | Relative wind direction (in degrees off bow) |  |
|  |  |  | 0 | 11 | 008 | Relative wind speed |  |

| Table reference |  |  | Table references |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | X | YYY |  |  |
|  |  |  |  |  |  | (Extended ship temperature and humidity data) |  |
| 3 | 02 | 093 | 0 | 07 | 032 | Height of sensor above local ground (or deck of marine platform) |  |
|  |  |  | 0 | 07 | 033 | Height of sensor above water surface |  |
|  |  |  | 0 | 03 | 013 | Type of marine thermometer |  |
|  |  |  | 0 | 03 | 015 | Exposure of marine thermometer/hygrometer |  |
|  |  |  | 0 | 12 | 101 | Temperature/air temperature |  |
|  |  |  | 0 | 02 | 039 | Method of wet-bulb temperature measurement |  |
|  |  |  | 0 | 03 | 014 | Type of marine hygrometer |  |
|  |  |  | 0 | 03 | 015 | Exposure of marine thermometer/hygrometer |  |
|  |  |  | 0 | 12 | 102 | Wet-bulb temperature |  |
|  |  |  | 0 | 12 | 103 | Dewpoint temperature |  |
|  |  |  | 0 | 13 | 003 | Relative humidity |  |
|  |  |  | 0 | 03 | 015 | Exposure of marine thermometer/hygrometer | Set to missing (cancel) |
|  |  |  | 0 | 03 | 013 | Type of marine thermometer | Set to missing (cancel) |
|  |  |  | 0 | 03 | 014 | Type of marine hygrometer | Set to missing (cancel) |

| Table reference |  |  | Table References |  |  | Element name | Element description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY | F | XX | YYY |  |  |
|  |  |  |  |  |  | (Synoptic reports from sea stations suitable for VOS observation data) |  |
| 3 | 08 | 014 | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 01 | 018 | Encrypted ship’s call sign and encryption method |  |
|  |  |  | 3 | 01 | 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |
|  |  |  | 3 | 02 | 062 | Ship “instantaneous” data |  |
|  |  |  | 3 | 02 | 063 | Ship “period” data |  |
|  |  |  | 1 | 01 | 000 | Delayed replication of 1 descriptor |  |
|  |  |  | 0 | 31 | 000 | Short delayed descriptor replication factor |  |
|  |  |  | 3 | 02 | 092 | VOSClim data elements |  |

Add new code tables:

0 03 013

Type of marine thermometer

| Code Figure | Meaning |
| --- | --- |
| 0 | Alcohol thermometer |
| 1 | Dry bulb mercury thermometer |
| 2 | Electric (resistance) thermometer |
| 3 – 7 | Reserved |

0 03 014

Type of marine hygrometer

| Code Figure | Meaning |
| --- | --- |
| 0 | Capacitance |
| 1 | Chilled mirror |
| 2 | Electric |
| 3 | Hair hygrometer |
| 4 | Hygristor |
| 5 | Psychrometer |
| 6 | Torsion |
| 7 | Other |
| 8 – 15 | Reserved |

0 03 015

Exposure of marine thermometer / hygrometer

| Code Figure | Meaning |
| --- | --- |
| 0 | Aspirated (Assmann type) |
| 1 | Screen (non-ventilated, i.e. natural ventilation) |
| 2 | Screen (ventilated, i.e. assisted ventilation) |
| 3 | Ship's screen (property of the ship) |
| 4 | Ship's sling (property of the ship) |
| 5 | Unscreened |
| 6 | Whirling or sling psychrometer |
| 7 – 15 | Reserved |

0 25 185

Encryption method

| Code Figure | Meaning |
| --- | --- |
| 0 | AES 256 |
| 1 – 254 | Reserved |
| 255 | Missing value |

- 2014-3.2.9(DRMM-II)/Proposed BUFR sequence for reporting observations from offshore platforms <para 4.1>

Add new entries:

BUFR/CREX Table B

| Table reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (characters) |
| 0 | 02 | 008 | Type of offshore platform | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

Editorial note: this descriptor may be replaced by 0 03 001.

BUFR Table D

|  |  | (Sequence for platform identification, type, time and location of the observation report) |  |
| --- | --- | --- | --- |
| 3 01 056 | 0 01 087 | WMO marine observing platform extended identifier | WMO number (extended 7 digit identifier) |
|  | 0 01 011 | Ship or mobile land station identifier | Call sign (where allocated) |
|  | 0 01 015 | Station or site name | Platform name |
|  | 0 02 008 | Type of offshore platform |  |
|  | 0 02 001 | Type of station |  |
|  | 3 01 011 | Year, month, day |  |
|  | 3 01 012 | Hour, minute |  |
|  | 3 01 021 | Latitude/longitude (high accuracy) |  |
|  | 0 07 030 | Height of station ground above mean sea level | Height of station platform above mean sea level |
|  | 0 07 031 | Height of barometer above mean sea level |  |

|  |  | (Sequence for reporting observations from offshore platforms) |  |
| --- | --- | --- | --- |
| 3 08 017 | 3 01 056 | Sequence for platform identification, type, time and location of the observation report |  |
|  | 3 02 001 | Pressure and 3-hour pressure change |  |
|  | 3 02 052 | Ship temperature and humidity data |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 056 | Sea/water temperature | Optional |
|  | 3 02 064 | Ship wind data (see Note) |  |
|  | 3 02 053 | Ship visibility data |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 004 | General cloud information | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 005 | Cloud layer | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 038 | Present and past weather | Optional |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 06 039 | Sequence for representation of basic wave measurements | Optional |

Note: Sequence 3 02 064 has previously been approved for validation and is also used in the sequence for synoptic reports from sea stations suitable for VOS data.

Add new code tables:

0 02 008

Type of offshore platform

| Code figure | Meaning |
| --- | --- |
| 0 | Fixed platform |
| 1 | Mobile offshore drill ship |
| 2 | Jack-up rig |
| 3 | Semi-submersible platform |
| 4 | FPSO (floating production storage and offloading unit) |
| 5 | Light vessel |
| 6 – 14 | Reserved |
| 15 | Missing value (or not known) ["or not known" to be separated from missing value] |

BUFR Template for reporting observations from offshore platforms

Items in red are new descriptors/sequences suggested for review and incorporation in the WMO Manual on Codes.

Template for reporting observations from offshore platforms

| 3 08 017 | 3 01 056 | Sequence for representation of platform identification type, time and location of the observation report |
| --- | --- | --- |
|  | 3 02 001 | Sequence for pressure data |
|  | 3 02 052 | Sequence for temperature and humidity data |
|  | 1 01 000 | Replicate next sequence |
|  | 0 31 000 | Short delayed replication factor |
|  | 3 02 056 | Sequence for sea temperature data (optional) |
|  | 3 02 064 | Sequence for wind data |
|  | 3 02 053 | Sequence for visibility data |
|  | 1 01 000 | Replicate next sequence |
|  | 0 31 000 | Short delayed replication factor |
|  | 3 02 004 | Sequence for cloud information (optional) |
|  | 1 01 000 | Replicate next sequence |
|  | 0 31 000 | Short delayed replication factor |
|  | 3 02 005 | Sequence for cloud layer information (optional) |
|  | 1 01 000 | Replicate next sequence |
|  | 0 31 000 | Short delayed replication factor |
|  | 3 02 038 | Sequence for present and past weather (optional) |
|  | 1 01 000 | Replicate next sequence |
|  | 0 31 000 | Short delayed replication factor |
|  | 3 06 039 | Sequence for representation of basic wave measurements (optional) |

Sequences for platform identification, pressure, temperature, wind and visibility are mandatory. Sequences for sea temperature, cloud data, present/past weather and waves are optional and used only when the relevant data are measured or reported.
Description of sequences

Sequence for platform identification, type, time and location of the observation report

| 3 01 056 |  | 0 01 087 | WMO number (extended 7 digit identifier) |
| --- | --- | --- | --- |
|  |  | 0 01 011 | Call sign (where allocated) |
|  |  | 0 01 015 | Platform name |
|  |  | 0 02 008 | Type of offshore platform* |
|  |  | 0 02 001 | Type of station |
|  | 3 01 011 | 0 04 001 | Year |
|  |  | 0 04 002 | Month |
|  |  | 0 04 003 | Day |
|  | 3 01 012 | 0 04 004 | Hour |
|  |  | 0 04 005 | Minute |
|  | 3 01 021 | 0 05 001 | Latitude (high accuracy) |
|  |  | 0 06 001 | Longitude (high accuracy) |
|  |  | 0 07 030 | Height of station platform above mean sea level |
|  |  | 0 07 031 | Height of barometer above mean sea level |

* suggest new code table to define platform type

Sequence for pressure data

| 3 02 001 |  | 0 10 004 | Pressure (as measured) |
| --- | --- | --- | --- |
|  |  | 0 10 051 | Pressure corrected to mean sea level |
|  |  | 0 10 061 | 3-hour pressure change* |
|  |  | 0 13 003 | Characteristic of pressure tendency* |

* would normally be omitted for hourly (or more frequent) reports

Sequence for temperature and humidity data

| 3 02 052 |  | 0 07 032 | Height of sensor above marine deck platform |
| --- | --- | --- | --- |
|  |  | 0 07 033 | Height of sensor above water surface |
|  |  | 0 12 101 | Temperature (dry bulb) |
|  |  | 0 02 039 | Method of wet-bulb temperature measurement* |
|  |  | 0 12 102 | Wet bulb temperature* |
|  |  | 0 12 103 | Dew point temperature** |
|  |  | 0 13 003 | Relative humidity*** |

* for manual observations only otherwise code as missing

** this will normally be calculated

*** include only when measured otherwise code as missing

Sequence for sea temperature data

| 3 02 056 |  | 0 02 038 | Method of sea temperature measurement |
| --- | --- | --- | --- |
|  |  | 0 07 063 | Depth below sea water surface of sensor |
|  |  | 0 22 043 | Sea/water temperature |
|  |  | 0 07 063 | Depth below sea water surface of sensor (set to missing to cancel) |

Sequence for wind data*

| 3 02 064 |  | 0 07 032 | Height of sensor above marine deck platform |
| --- | --- | --- | --- |
|  |  | 0 07 033 | Height of sensor above water surface |
|  |  | 0 02 002 | Type of instrumentation for wind measurement |
|  |  | 0 08 021 | Time significance (= 2 (time averaged)) |
|  |  | 0 04 025 | Time period (= - 10 minutes, or number of minutes after a significant change of wind) |
|  |  | 0 11 001 | Wind direction |
|  |  | 0 11 002 | Wind speed |
|  |  | 0 08 021 | Time significance (= missing value) |
|  |  | 1 03 000 | Delayed replication of 3 descriptors |
|  |  | 0 31 001 | Delayed descriptor replication factor (0 if no gust data, 1 otherwise) |
|  |  | 0 04 025 | Time period in minutes |
|  |  | 0 11 043 | Maximum wind gust direction |
|  |  | 0 11 041 | Maximum wind gust speed |

* sequence under validation

Sequence for visibility data

| 3 02 053 |  | 0 07 032 | Height of sensor above marine deck platform |
| --- | --- | --- | --- |
|  |  | 0 07 033 | Height of sensor above water surface |
|  |  | 0 20 001 | Horizontal visibility |

Sequences for cloud data

| 3 02 004 |  | 0 20 010 | Cloud cover (total) |
| --- | --- | --- | --- |
|  |  | 0 08 002 | Vertical significance |
|  |  | 0 20 011 | Cloud amount |
|  |  | 0 20 013 | Height of base of cloud |
|  |  | 0 20 012 | Cloud type (low clouds) |
|  |  | 0 20 012 | Cloud type (middle clouds) |
|  |  | 0 20 012 | Cloud type (high clouds) |

| 3 02 005 |  | 0 08 002 | Vertical significance |
| --- | --- | --- | --- |
|  |  | 0 20 011 | Cloud amount |
|  |  | 0 20 012 | Cloud type |
|  |  | 0 20 013 | Height of base of cloud |

Sequence for present and past weather data

| 3 02 038 |  | 0 20 003 | Present weather |
| --- | --- | --- | --- |
|  |  | 0 04 024 | Time period |
|  |  | 0 20 004 | Past weather (1) |
|  |  | 0 20 005 | Past weather (2) |

Sequence for basic wave data

| 3 06 039 |  | 0 22 078 | Duration of wave record |
| --- | --- | --- | --- |
|  |  | 0 22 070 | Significant wave height |
|  |  | 0 22 073 | Maximum wave height |
|  |  | 0 22 074 | Average wave period |
|  |  | 0 22 071 | Spectral peak wave period |
|  |  | 0 22 076 | Direction from which dominant waves are coming |
|  |  | 0 22 077 | Directional spread of dominant waves |

New code table 0 02 008 for platform type

0 – fixed platform

1 – mobile offshore drill ship

2 – jack-up rig

3 – semi-submersible platform

4 – FPSO (floating production storage and offloading unit)

5 – light vessel

6-14 - reserved

15 – missing value (or not known).

- 2014-3.2.10(DRMM-II)/Proposed modifications to the BUFR template for reporting data from Argo profiling floats <para 4.1>

Add new table B class

| F | X | Class | Comments |
| --- | --- | --- | --- |
| 0 | 41 | Marine bio-geochemical data |  |

Add new table B entries

| Table reference |  |  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F | XX | YYY |  | Unit | Scale | Ref. value | Data width (bits) | Units | Scale | Data width (characters) |
| 0 | 08 | 034 | Type of temperature/ salinity measurement | Code table | 0 | 0 | 4 | Code table | 0 | 2 |

Add new table D entries

3 06 017

Near surface depth (pressure) and temperature profile from floats

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
|  |  |  |  |  |  | (Sub-surface temperature profile (high accuracy/precision) with quality flags) |  |
| 3 06 017 |  |  | 0 | 02 | 032 | Indicator for digitization | = 0 Fixed sensor depths |
|  |  |  | 0 | 08 | 034 | Type of temperature/salinity measurement |  |
|  |  |  | 1 | 06 | 000 | Delayed replication of 6 descriptors |  |
|  |  |  | 0 | 31 | 001 | Delayed descriptor replication factor | Number of depths |
|  |  |  | 0 | 07 | 065 | Water pressure | In Pa |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 043 | Sea/water temperature | In K to 2 decimal places |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 08 | 034 | Type of temperature/salinity measurement | Set to missing (cancel) |

3 06 018

Near surface depth (pressure), temperature and salinity profile from floats

| Table Reference |  |  | Table References |  |  | Element Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |  |
|  |  |  |  |  |  | (Sub-surface temperature profile (high accuracy/precision) with quality flags) |  |
| 3 06 018 |  |  | 0 | 02 | 032 | Indicator for digitization | = 0 Fixed sensor depths |
|  |  |  | 0 | 08 | 034 | Type of temperature/salinity measurement |  |
|  |  |  | 1 | 09 | 000 | Delayed replication of 9 descriptors |  |
|  |  |  | 0 | 31 | 001 | Delayed descriptor replication factor | Number of depths |
|  |  |  | 0 | 07 | 065 | Water pressure | In Pa |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 043 | Sea/water temperature | In K to 2 decimal places |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 22 | 064 | Salinity |  |
|  |  |  | 0 | 08 | 080 | Qualifier for GTSPP quality flag |  |
|  |  |  | 0 | 33 | 050 | Global GTSPP quality flag |  |
|  |  |  | 0 | 08 | 034 | Type of temperature/salinity measurement | Set to missing (cancel) |

Add new code tables

0 08 034

Temperature / salinity measurement qualifier

| Code figure | Meaning |
| --- | --- |
| 0 | Un-pumped float temperature and salinity data |
| 1 | Auxiliary STS sensor data |
| 2 – 14 | Reserved |
| 15 | Missing value |

- 2014-5.1(DRMM-II)/Rec. 6 (CBS-Ext.(2014)/valid from 4 November 2015

Amend Regulations B/C 30.4 and 32.4:

B/C 30.4	Data required by Regional or national reporting practices

B/C 30.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT data in Manual on Codes, WMO-No. 306, Volume II.

B/C 30.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 30.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 30.4.3	Date/time (of beginning of the period for monthly precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for period different from the period recommended in Note (1) to B/C 30.2.6.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

B/C 32.4	Data required by Regional or national reporting practices

B/C 32.4.1	Data required by regional or national reporting practices

No additional data are currently required by regional or national reporting practices for CLIMAT SHIP data in Manual on Codes, WMO-No. 306, Volume II.

B/C 32.4.2	Reference period for the data of the month

If the regional or national reporting practices require reporting monthly data (with the exception of precipitation data) for one-month period different from the local time month as recommended in B/C 32.2.2.1, short time displacement (0 04 074) shall be adjusted accordingly.

B/C 32.4.3	Date/time (of beginning of the one-month period for precipitation data)

If the regional or national reporting practices require reporting monthly precipitation data for one-month period different from the period recommended in Note (1) to B/C 32.2.3.1, then hour (0 04 004) shall be adjusted accordingly. This regulation does not apply if the beginning of the period for monthly precipitation data starts on the last day of the previous month in UTC.

Amend entries of BUFR Table D:

In 3 07 071, 3 07 072, 3 08 011 and 3 08 012, modify LST to LT.

Amend templates of B/C Regulations:

In TM3 07 073 and TM3 08 013, modify LST to LT.

In the Notes 1 and 2 to TM3 07 073 and TM3 08 013,

(1)	The time identification refers to the beginning of the one-month period. Except for precipitation measurements, the one-month period is recommended to correspond to the local standard time (LT) month. [7]

(2)	In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. [5]

Amend B/C Regulations:

In B/C 30.2.2.1 and B/C 32.2.2.1,

Monthly data (with the exception of precipitation data) are recommended to be reported for one-month period, corresponding to the local standard time (LT) month [Handbook on CLIMAT and CLIMAT TEMP Reporting (WMO/TD-No.1188)].  In that case, short time displacement (0 04 074) shall specify the difference between UTC and LT (set to non-positive values in the eastern hemisphere, non-negative values in the western hemisphere).

Time period (0 04 023) represents the number of days in the month for which the data are reported, and shall be expressed as a positive value in days.

In Note 1 to B/C 30.2.6.1 and B/C 32.2.3.1,

Day (0 04 003) and hour (0 04 004) of the beginning of the one-month period for monthly precipitation data are reported. Day (0 04 003) shall be set to 1 and hour (0 04 004) shall be set to 6.

Notes:

(1)	In case of precipitation measurements, a month begins at 0600 hours UTC on the first day of the month and ends at 0600 hours UTC on the first day of the following month [Handbook on CLIMAT and CLIMAT TEMP Reporting (WMO/TD-No.1188)].

In B/C 30.3.1.2 and B/C 32.3.1.2,

The one-month period for which the normal values are reported shall be specified by month (0 04 002), day (0 04 003) being set to 1, hour (0 04 004) being set to 0, short time displacement (0 04 074) being set to (UTC – LT) and time period (0 04 022) being set to 1, i.e. 1 month.  Short time displacement (0 04 074) shall be set to non-positive values in the eastern hemisphere, non-negative values in the western hemisphere.

In B/C 30.2.5.3,

B/C 30.2.5.3	Occurrence of the lowest daily mean temperature

………..

Lowest daily mean temperature (0 12 153) shall be reported in degrees Kelvin (with precision in hundredths of a degree Kelvin); if produced in CREX, in degrees Celsius (with precision in hundredths of a degree Celsius).

- 2014-11.1(DRMM-II)/Rec. 6 (CBS-Ext.(2014))/valid from 4 November 2015]

Add new regulations:

B/C 1.4.4.4.4, B/C 5.4.4.4.4 and B/C 10.4.4.4.3

If synoptic data are produced in BUFR or CREX by conversion from a TAC report, the following approach shall be used: Height of base of the lowest cloud 0 20 013 shall be derived from the hshs in the first group 8 in section 3, i.e. from the hshs of the lowest cloud. If and only if groups 8 are not reported in section 3, 0 20 013 may be derived from h. The lower limit of the range defined for hshs and for h shall be used. However, if groups 8 are not reported in section 3 and h = 9 and Nh ≠ 0, then 0 20 013 shall be 4000 m; if groups 8 are not reported in section 3 and h = 9 and Nh = 0, then 0 20 013 shall be 8000 m.

Add items (d), (e) and (f) in the regulations below:

B/C 1.4.4.3.1, B/C 5.4.4.3.1 and B/C 10.4.4.3.1

(d)	If no clouds are observed (clear sky), then the cloud amount shall be reported as 0.

(e)	If sky is obscured by fog and/or other meteorological phenomena, then the cloud amount shall be reported as 9.

(f)	If cloud cover is indiscernible for reasons other than fog or other meteorological phenomena, or observation is not made, the cloud amount shall be reported as missing.

- 2014-11.2(DRMM-II)/WIGOS Station Identifier <para 4.1>

Add entries in BUFR/CREX Table B.

Class 01 – BUFR/CREX Identification

|  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE |  |  |  |  | DATA |  |  | DATA |
| REFERENCE | ELEMENT NAME | UNIT | SCALE | REFERENCE | WIDTH | UNIT | SCALE | WIDTH |
| F X Y |  |  |  | VALUE | (Bits) |  |  | (Characters) |
| 0 01 125 | WIGOS Identifier Series | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |
| 0 01 126 | Issuer of Identifier | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |
| 0 01 127 | Issue Number | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |
| 0 01 128 | Local Identifier (Character) | CCITT IA5 | 0 | 0 | 128 | Character | 0 | 16 |

Add entries in BUFR/CREX Table D.

Category 01 – Location and identification sequences

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION |
| --- | --- | --- | --- |
| F X Y |  |  |  |
|  |  | (WIGOS Identifier) |  |
| 3 01 150 | 0 01 125 | WIGOS Identifier Series |  |
|  | 0 01 126 | Issuer of Identifier |  |
|  | 0 01 127 | Issue Number |  |
|  | 0 01 128 | Local Identifier (Character) |  |

- 2013-3.2.2(DRMM-I)/Proposal for a BUFR template for radar wind profiler <para 4.1>

Add templates:

Radar Wind Profiler Data Sequence:

| TABLE REFERENCE | TABLE REFERENCES | TABLE REFERENCES | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- | --- | --- |
| 3 09 021 |  |  |  | RWP Wind data (product data) NEW |
|  | 3 01 001 |  |  | WMO block and station numbers |
|  |  | 0 01 001 |  | WMO Block Number |
|  |  | 0 01 002 |  | WMO Station Number |
|  | 3 01 022 |  |  | (Latitude/longitude (high accuracy), height of station) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  |  | 0 07 001 |  | Height of station |
|  | 3 01 014 |  |  | Time period |
|  |  | 1 02 002 |  | Replicate 2 descriptors 2 times |
|  |  | 3 01 011 |  | Year, month, day |
|  |  |  | 0 04 001 | Year |
|  |  |  | 0 04 002 | Month |
|  |  |  | 0 04 003 | Day |
|  |  | 3 01 012 |  | Hour, minute |
|  |  |  | 0 04 004 | Hour |
|  |  |  | 0 04 005 | Minute |
|  | 0 02 003 |  |  | Type of measuring equipment used |
|  | 1 12 000 |  |  | Delayed replication of 12 descriptors |
|  | 0 31 001 |  |  | Delayed descriptor replication factor (typically between 20 and 100) |
|  | 0 07 007 |  |  | Height |
|  | 3 01 022 |  |  | (Latitude/longitude (high accuracy)) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  | 0 11 003 |  |  | u-component |
|  | 0 11 099 0 11 110 |  |  | Uncertainty in u-component NEW |
|  | 0 11 004 |  |  | v-component |
|  | 0 11 100 0 11 111 |  |  | Uncertainty in v-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 11 006 |  |  | w-component |
|  | 0 11 101 0 11 112 |  |  | Uncertainty in w-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 10 071 |  |  | Vertical resolution NEW |
|  | 0 05 062 |  |  | Horizontal width of sampled volume NEW |

RASS Data Sequence:

| TABLE REFERENCE | TABLE REFERENCES | TABLE REFERENCES | TABLE REFERENCES | ELEMENT NAME |
| --- | --- | --- | --- | --- |
| 3 09 022 |  |  |  | RASS virtual temperature (product data) NEW |
|  | 3 01 001 |  |  | WMO block and station numbers |
|  |  | 0 01 001 |  | WMO Block Number |
|  |  | 0 01 002 |  | WMO Station Number |
|  | 3 01 022 |  |  | (Latitude/longitude (high accuracy), height of station) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  |  | 0 07 001 |  | Height of station |
|  |  | 0 02 003 |  | Type of measuring equipment |
|  | 3 01 014 |  |  | Time period |
|  |  | 1 02 002 |  | Replicate 2 descriptors 2 times |
|  |  | 3 01 011 |  | Year, month, day |
|  |  |  | 0 04 001 | Year |
|  |  |  | 0 04 002 | Month |
|  |  |  | 0 04 003 | Day |
|  |  | 3 01 012 |  | Hour, minute |
|  |  |  | 0 04 004 | Hour |
|  |  |  | 0 04 005 | Minute |
|  | 0 02 003 |  |  | Type of measuring Equipment used |
|  | 1 10 000 |  |  | Delayed replication of 10 descriptors |
|  | 0 31 001 |  |  | Delayed descriptor replication factor |
|  | 0 07 007 |  |  | Height |
|  | 3 01 022 |  |  | (Latitude/longitude (high accuracy)) |
|  |  | 0 05 001 |  | Latitude (high accuracy) |
|  |  | 0 06 001 |  | Longitude (high accuracy) |
|  | 0 12 007 |  |  | Virtual temperature |
|  | 0 12 008 |  |  | Uncertainty in virtual temperature NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 11 006 |  |  | w-component |
|  | 0 11 101 0 11 112 |  |  | Uncertainty in w-component NEW |
|  | 0 33 002 |  |  | Quality information |
|  | 0 10 071 |  |  | Vertical resolution NEW |
|  | 0 05 062 |  |  | Horizontal width of sampled volume NEW |

Add descriptors:

|  |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE |  |  |  |  | DATA |  |  | DATA |
| REFERENCE | ELEMENT NAME | UNIT | SCALE | REFERENCE | WIDTH | UNIT | SCALE | WIDTH |
| F* X Y |  |  |  | VALUE | (Bits) |  |  | (Characters) |
| 0 11 099 0 11 110 | Uncertainty in u-component | m s-1 | -1 | 0 | 10 | m s-1 | -1 | 4 |
| 0 11 100 0 11 111 | Uncertainty in v-component | m s-1 | -1 | 0 | 10 | m s-1 | -1 | 4 |
| 0 11 101 0 11 112 | Uncertainty in w-component | m s-1 | -2 | 0 | 10 | m s-1 | -2 | 4 |
| 0 10 071 | Vertical resolution | m | 0 | 0 | 14 | m | 0 | 5 |
| 0 05 062 | Horizontal width of sampled volume | m | -1 | 0 | 20 | m | -1-1 | 7 |
| 0 12 008 | Uncertainty in virtual temperature | K | -1 | 0 | 10 | K | 0-1 | 4 |

- 2013-3.2.4(DRMM-I)/Satellite-derived winds in BUFR <para 4.1>

Add an entry:

|  |  | (Satellite-derived winds) |
| --- | --- | --- |
| 3 10 067 | 0 01 007 | Satellite identifier |
|  | 0 01 033 | Identification of originating/generating centre |
|  | 0 01 034 | Identification of originating/generating sub-centre |
|  | 0 02 019 | Satellite instruments |
|  | 0 02 020 | Satellite classification |
|  | 3 01 011 | Year, month, day |
|  | 3 01 012 | Hour, minute |
|  | 2 07 003 | Increase scale, reference value and bit data width |
|  | 0 04 006 | Second |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 3 01 021 | Latitude, L/longitude (high accuracy) |
|  | 0 07 024 | Satellite zenith angle |
|  | 0 02 153 | Satellite channel centre frequency |
|  | 0 02 014 | Tracking technique/status of system used |
|  | 0 02 023 | Satellite derived wind computation method |
|  | 0 08 072 | Pixel(s) type (target type) |
|  | 0 02 028 | Segment size at nadir in X x-direction (target scene width) |
|  | 0 02 029 | Segment size at nadir in Y y-direction (target scene height) |
|  | 0 04 025 | Time period or displacement (in minutes) |
|  | 0 10 004 | Pressure |
|  | 0 12 101 | Temperature/air temperature |
|  | 2 07 002 | Increase scale, reference value and bit data width |
|  | 0 11 001 | Wind direction |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 0 33 007 | Per cent confidence (for wind direction) |
|  | 2 07 001 | Increase scale, reference value and bit data width |
|  | 0 11 002 | Wind speed |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 0 33 007 | Per cent confidence (for wind speed) |
|  | 0 08 041 | Data significance (14 = Expected error) |
|  | 0 11 002 | Wind speed |
|  | 0 08 041 | Data significance (15 = Representative error) |
|  | 0 10 004 | Pressure |
|  | 0 12 101 | Temperature/air temperature |
|  | 0 08 041 | Data significance (Missing = Cancel) |
|  | 0 08 021 | Time significance (4 = Forecast) |
|  | 0 04 004 | Hour |
|  | 0 04 005 | Minute |
|  | 0 04 006 | Second |
|  | 2 07 002 | Increase scale, reference value and bit data width |
|  | 0 11 001 | Wind direction |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 2 07 001 | Increase scale, reference value and bit data width |
|  | 0 11 002 | Wind speed |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 0 08 021 | Time significance (28 = Start of scan) |
|  | 0 04 004 | Hour |
|  | 0 04 005 | Minute |
|  | 0 04 006 | Second |
|  | 3 01 021 | Latitude, L/longitude (high accuracy) |
|  | 2 07 001 | Increase scale, reference value and bit data width |
|  | 0 11 003 | u-component |
|  | 0 11 004 | v-component |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 0 11 110 0 11 113 | Tracking correlation of vector |
|  | 0 08 023 | First order statistics (10 = Standard deviation) |
|  | 0 11 002 | Wind speed |
|  | 0 08 023 | First order statistics (Missing = Cancel) |
|  | 0 25 147 | Size of largest cluster (in pixels) |
|  | 0 08 021 | Time significance (29 = End of scan or time of ending) |
|  | 0 04 004 | Hour |
|  | 0 04 005 | Minute |
|  | 0 04 006 | Second |
|  | 3 01 021 | Latitude, L/longitude (high accuracy) |
|  | 2 07 001 | Increase scale, reference value and bit data width |
|  | 0 11 003 | u-component |
|  | 0 11 004 | v-component |
|  | 2 07 000 | Cancel increase scale, reference value and bit data width |
|  | 0 11 110 0 11 113 | Tracking correlation of vector |
|  | 0 08 023 | First order statistics (10 = Standard deviation) |
|  | 0 11 002 | Wind speed |
|  | 0 08 023 | First order statistics (Missing = Cancel) |
|  | 0 25 147 | Size of largest cluster (in pixels) |
|  | 0 08 021 | Time significance (Missing = Cancel) |
|  | 0 04 004 | Hour |
|  | 0 04 005 | Minute |
|  | 0 04 006 | Second |
|  | 0 08 003 | Vertical significance (satellite observations) (2=Cloud top) |
|  | 0 08 023 | First order statistics (2=Maximum value) |
|  | 0 10 004 | Pressure (Maximum cloud top pressure in target scene) |
|  | 0 12 101 | Temperature/air temperature (Maximum cloud top temperature in target scene) |
|  | 0 08 023 | First order statistics (3=Minimum value) |
|  | 0 10 004 | Pressure (Minimum cloud top pressure in target scene) |
|  | 0 12 101 | Temperature/air temperature (Minimum cloud top temperature in target scene) |
|  | 0 08 023 | First order statistics (10 = Standard deviation) |
|  | 0 10 004 | Pressure (Standard deviation of cloud top pressure in target scene) |
|  | 0 08 023 | First order statistics (Missing = Cancel) |
|  | 0 08 003 | Vertical significance (satellite observations) (Missing = Cancel) |
|  | 0 20 056 | Cloud phase (Dominant cloud phase of target scene) |
|  | 0 12 133 | NWP vertical temperature gradient (+/- 200hpa about pressure assignment of tracer) |
|  | 0 11 111 0 11 114 | NWP vertical wind shear (+/- 200hpa about pressure assignment of tracer) |
|  | 0 12 134 | Low-level inversion flag |

Add entries:

| F X Y | Element name | BUFR Unit | BUFR Scale | BUFR Refval | BUFR Bits | CREX Unit | CREX Scale | CREX Bytes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 11 110 0 11 113 | Tracking correlation of vector | Numeric | 3 | -1000 | 12 | Numeric | 3 | 4 |
| 0 11 111 0 11 114 | NWP vertical wind shear (+/- 200hPa about pressure assignment of tracer) | m/s | 2 | -8192 | 14 | m/s | 2 | 5 |
| 0 12 133 | NWP vertical termperature gradient (+/- 200hPa about pressure assignment of tracer) | K | 2 | 0 | 16 | C | 2 | 4 |
| 0 12 134 | Low-level inversion flag | Code table | 0 | 0 | 2 | Code table | 0 | 1 |
| 0 25 147 | Size of largest cluster (in pixels) | Numeric | 0 | 0 | 10 | Numeric | 0 | 4 |

Add entries:

Code table 0 02 014/Common Code table C-7 (Tracking technique/status of system used)

71 	Nested tracking disabled

Code table 0 20 056 (Cloud phase)

5		Supercooled liquid water

Code table 0 08 041 (Data significance)

14		Expected error

15		Representative error

0 12 134 – Low-level inversion flag

| Code figure | DESCRIPTION |
| --- | --- |
| 0 | No inversion |
| 1 | Inversion |
| 2 | Reserved |
| 3 | Missing value |

- 2012-3.2.7(DRC-IV)/Sequence 3 02 067 <para 4.1>

Validate a new descriptor:

Category 02 – Meteorological sequences common to surface data

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT DESCRIPTION |  |
| --- | --- | --- | --- |
| F X Y |  |  |  |
|  |  | Additional synoptical parameters |  |
| 3 02 067 | 0 01 023 | Observation sequence number |  |
|  | 0 04 025 | Time period or displacement | Time period (= 0 minutes) |
|  | 0 02 177 | Method of snow depth measurement |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 003 | Present weather |  |
|  | 1 02 000 | Delayed replication of 2 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 20 001 | Horizontal visibility |  |
|  | 0 05 021 | Bearing or azimuth | set to missing to cancel previous entry |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 056 | (Sea/water temperature) |  |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 0 33 041 | Attribute of following value |  |
|  | 0 20 058 | Visibility seawards from a coastal station |  |
|  | 0 22 061 | State of the sea |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 3 02 022 | (Wind waves) |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 3 02 023 | (Swell waves) |  |
|  | 1 04 000 | Delayed replication of 4 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed of a phenomenon or clouds |  |
|  | 0 20 137 | Evolution of clouds |  |
|  | 0 20 012 | Cloud type |  |
|  | 0 20 090 | Special clouds |  |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed of a phenomenon or clouds |  |
|  | 0 20 137 | Evolution of clouds |  |
|  | 0 20 136 | Supplementary cloud type |  |
|  | 0 04 025 | Time period or displacement | Reference period of fresh fallen snow |
|  | 0 13 012 | Depth of fresh snow |  |
|  | 0 04 025 | Time period or displacement | Time period in minutes ( =-60 ) |
|  | 0 11 042 | Maximum wind speed (10-minute mean wind) |  |
|  | 1 04 000 | Delayed replication of 4 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | = 30 (time of occurrence) or = 17 (start of phenomenon) |
|  | 0 04 025 | Time period or displacement | Time displacement (=-xx ) |
|  | 0 11 042 | Maximum wind speed (10-minute mean wind) |  |
|  | 0 08 021 | Time significance | set to missing to cancel previous entry |
|  | 1 15 000 | Delayed replication of 15 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | = 30 (time of occurrence) or = 17 (start of phenomenon) |
|  | 0 04 015 | Time increment | Time increment (=-xx1 ) |
|  | 0 08 021 | Time significance | = 2 (time averaged) |
|  | 0 04 025 | Time period or displacement | Time period (= - 10 minutes, or number of minutes after a significant change of wind) |
|  | 0 11 001 | Wind direction |  |
|  | 0 11 002 | Wind speed |  |
|  | 0 08 021 | Time significance | =22 (time of occurrence of wind shift) |
|  | 0 04 015 | Time increment | Time increment (=+xx2 ) |
|  | 0 08 021 | Time significance | = 2 (time averaged) |
|  | 0 04 025 | Time period or displacement | Time period (= - 10 minutes, or number of minutes after a significant change of wind) |
|  | 0 11 001 | Wind direction |  |
|  | 0 11 002 | Wind speed |  |
|  | 0 08 021 | Time significance | set to missing to cancel previous entry |
|  | 0 04 025 | Time period or displacement | Time displacement (= 0 minutes) |
|  | 0 04 015 | Time increment | Time increment (=+(xx1-xx2), non negative to reset the time to the actual time ) |
|  | 1 03 000 | Delayed replication of 3 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 025 | Time period or displacement | Time period in minutes (=-xx i.e. from) |
|  | 0 04 025 | Time period or displacement | Time period in minutes (=-xx i.e. to) |
|  | 0 20 003 | Present weather |  |
|  | 1 10 000 | Delayed replication of 10 descriptors |  |
|  | 0 31 001 | Delayed descriptor replication factor |  |
|  | 0 04 025 | Time period or displacement | Time period in minutes (=-xx i.e. from) |
|  | 0 04 025 | Time period or displacement | Time period in minutes (=-xx i.e. to) |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 05 021 | Bearing or azimuth |  |
|  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed of a phenomenon or clouds |  |
|  | 0 20 024 | Intensity of phenomena |  |
|  | 0 20 025 | Obscuration |  |
|  | 0 20 026 | Character of obscuration |  |
|  | 0 20 027 | Phenomena occurrence |  |
|  | 0 20 063 | Special phenomena |  |

And in Template Form:

| 3 02 067 |  |  | Additional synoptical parameters |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 01 023 |  | Observation Sequence number |  | Numeric, 0 |
|  |  |  | Additional “instantaneous” data |  |  |
|  | 0 04 025 |  | Time period (= 0 minutes) |  | Minute, 0 |
|  | 0 02 177 |  | Method of snow depth measurement 0 Manual observation 1 Ultrasonic method 2 Video camera method 3-13 Reserved 14 Others 15 Missing value |  | Code table, 0 |
|  |  |  | Additional present weather |  |  |
|  | 1 01 000 |  | Delayed replication of 1 descriptor |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 20 003 |  | Present weather | 960ww 961ww | Code table, 0 |
|  |  |  | Visibility in different directions |  |  |
|  | 1 02 000 |  | Delayed replication of 2 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 05 021 |  | Bearing or azimuth | 981VV-988VV | Degreetrue, 0 |
|  | 0 20 001 |  | Horizontal visibility | VV | m, –1 |
|  | 0 05 021 |  | Bearing or azimuth (=set to missing to cancel previous entry) |  | Degreetrue, 0 |
|  |  |  | Sea data (Observations from a coastal station) |  |  |
|  | 1 01 000 |  | Delayed replication of 1 descriptor |  |  |
|  | 0 31 000 |  | Short delayed descriptor replication factor |  | Numeric, 0 |
|  | 3 02 056 |  | Sea surface temperature, method of measurement, and depth below sea surface |  |  |
|  |  | 0 02 038 | Method of sea/water temperature measurement |  | Code table, 0 |
|  |  | 0 07 063 | Depth below sea/water surface (for sea surface temperature measurement) |  | m, 2 |
|  |  | 0 22 043 | Sea/water temperature | 222.. 0sTwTwTw | K, 2 |
|  |  | 0 07 063 | Depth below sea/water surface (set to missing to cancel the previous value) |  | m, 2 |
|  | 1 03 000 |  | Delayed replication of 3 descriptors |  |  |
|  | 0 31 000 |  | Short delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 33 041 |  | Attribute of following value |  | Code tabel, 0 |
|  | 0 20 058 |  | Visibility seawards from a coastal station | 980VsVs | m,-1 |
|  | 0 22 061 |  | State of the sea | 924SVs | Code table, 0 |
|  | 1 01 000 |  | Delayed replication of 1 descriptor |  |  |
|  | 0 31 000 |  | Short delayed descriptor replication factor |  | Numeric, 0 |
|  | 3 02 022 |  | Wind waves |  |  |
|  |  | 0 22 002 | Direction of wind waves |  | Degreetrue, 0 |
|  |  | 0 22 012 | Period of wind waves | 222.. 2PwPw | s, 0 |
|  |  | 0 22 022 | Height of wind waves | HwHw | m, 1 |
|  | 1 01 000 |  | Delayed replication of 1 descriptor |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 3 02 023 |  | Swell waves |  |  |
|  |  | 0 22 003 | Direction of swell waves | 222.. 3dw1dw1 dw2dw2 | Degreetrue, 0 |
|  |  | 0 22 013 | Period of swell waves | 4Pw1Pw1, 5Pw2Pw2 | s, 0 |
|  |  | 0 22 023 | Height of swell waves | Hw1Hw1, Hw2Hw2 | m, 1 |
|  |  |  | Clouds | 941CDp, 943CLDp |  |
|  | 1 04 000 |  | Delayed replication of 4 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 20 054 |  | True direction from which a phenomenon or clouds are moving | Da, Dp | Degreetrue, 0 |
|  | 0 20 137 |  | Evolution of Clouds | 940Cn3 | Code table, 0 |
|  | 0 20 012 |  | Cloud type (C) | 941CDp, 943CLDp | Code table, 0 |
|  | 0 20 090 |  | Special clouds | 993CSDa | Code table, 0 |
|  | 1 03 000 |  | Delayed replication of 3 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 20 054 |  | True direction from which a phenomenon or clouds are moving | Da | Degreetrue, 0 |
|  | 0 20 137 |  | Evolution of Clouds | n3, n4 | Code table, 0 |
|  | 0 20 136 |  | Supplementary Cloud type | 948C0Da, 949CaDa, 950Nmn3, 951Nvn4 | Code table, 0 |
|  |  |  | Additional “period” data |  |  |
|  | 0 04 025 |  | Time period in minutes ( = reference period of fresh fallen snow ) |  | Minute, 0 |
|  | 0 13 012 |  | Depth of fresh snow |  | m, 2 |
|  |  |  | Additional wind data |  |  |
|  | 0 04 025 |  | Time period in minutes ( =-60 ) |  | Minute, 0 |
|  | 0 11 042 |  | Maximum wind gust speed (10 min mean wind) | 912ff | m s-1, 1 |
|  |  |  | 902tt 912ff .. mandatory ff>=18 |  |  |
|  | 1 04 000 |  | Delayed replication of 4 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 08 021 |  | Time significance (=30 time of occurrence or =17 start of phenomenon) |  | Code table, 0 |
|  | 0 04 025 |  | Time displacement (=-xx ) | 902tt | Minute, 0 |
|  | 0 11 042 |  | Maximum wind gust speed (10 min mean wind) | 912ff | m s-1, 1 |
|  | 0 08 021 |  | Time significance (=set to missing to cancel previous entry) |  | Code table, 0 |
|  |  |  | Significant change in wind speed and/or direction | e.g. 913ff 904tt 913ff |  |
|  | 1 15 000 |  | Delayed replication of 15 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 08 021 |  | Time significance (=30 time of occurrence or =17 start of phenomenon) |  | Code table, 0 |
|  | 0 04 015 |  | Time increment (=-xx1 ) |  | Minute, 0 |
|  | 0 08 021 |  | Time significance (= 2 (time averaged)) |  | Code table, 0 |
|  | 0 04 025 |  | Time period (= - 10 minutes, or number of minutes after a significant change of wind) |  | Minute, 0 |
|  | 0 11 001 |  | Wind direction | 915dd | Degreetrue, 0 |
|  | 0 11 002 |  | Wind speed | 913ff | m s-1, 1 |
|  | 0 08 021 |  | Time significance (=22 time of occurrence of wind shift) |  | Code table, 0 |
|  | 0 04 015 |  | Time increment (=+xx2 ) |  | Minute, 0 |
|  | 0 08 021 |  | Time significance (= 2 (time averaged)) |  | Code table, 0 |
|  | 0 04 025 |  | Time period (= - 10 minutes, or number of minutes after a significant change of wind) |  | Minute, 0 |
|  | 0 11 001 |  | Wind direction | 915dd | Degreetrue, 0 |
|  | 0 11 002 |  | Wind speed | 913ff | m s-1, 1 |
|  | 0 08 021 |  | Time significance (=set to missing to cancel previous entry) |  | Code table, 0 |
|  | 0 04 025 |  | Time displacement (= 0 minutes) |  | Minute, 0 |
|  | 0 04 015 |  | Time increment (=+(xx1-xx2), non negative to reset the time to the actual time ) |  | Minute, 0 |
|  |  |  | Additional weather |  |  |
|  | 1 03 000 |  | Delayed replication of 3 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 04 025 |  | Time period in minutes (=-xx i.e. from) |  | Minute, 0 |
|  | 0 04 025 |  | Time period in minutes (=-xx i.e. to) |  | Minute, 0 |
|  | 0 20 003 |  | Present weather | 962ww, 963w1w1, 964ww, 965w1w1, 966ww, 967w1w1 | Code table, 0 |
|  |  |  | Additional 9SpSpspsp groups |  |  |
|  | 1 10 000 |  | Delayed replication of 10 descriptors |  |  |
|  | 0 31 001 |  | Delayed descriptor replication factor |  | Numeric, 0 |
|  | 0 04 025 |  | Time period in minutes (=-xx i.e. from) |  | Minute, 0 |
|  | 0 04 025 |  | Time period in minutes (=-xx i.e. to) |  | Minute, 0 |
|  | 0 05 021 |  | Bearing or azimuth * | Da, Dp | Degreetrue, 0 |
|  | 0 05 021 |  | Bearing or azimuth * | Da, Dp | Degreetrue, 0 |
|  | 0 20 054 |  | True direction from which a phenomenon or clouds are moving | Da, Dp | Degreetrue, 0 |
|  | 0 20 024 |  | Intensity of phenomena (1= Light, 2 = Moderate, 3 = Heavy, 4 = Violent, 5 = Severe) |  | Code table, 0 |
|  | 0 20 025 |  | Obscuration |  | Flag table, 0 |
|  | 0 20 026 |  | Character of obscuration |  | Code table, 0 |
|  | 0 20 027 |  | Phenomenon occurrence |  | Flag table, 0 |
|  | 0 20 063 |  | Special phenomena |  | Code table, 0 |

- 2012-3.2.11(DRC-IV)/A BUFR Sequence to append data for national use to Canadian 
SYNOP reports <para 4.1>

Validate a template, relevant descriptors and a code table:

| TABLE REFERENCE | TABLE REFERENCES | ELEMENT DESCRIPTION |  |
| --- | --- | --- | --- |
| F X Y |  | Synoptic data for national use (Canada) |  |
| 3 07 100 |  |  |  |
|  | 0 04 024 | Time period or displacement | (value = -6): following values apply to the 6 hour period preceding reference time. |
|  | 1 02 000 | Short d Delayed descriptor replication of two 2 descriptors |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 0 13 057 | Time of beginning/end of precipitation | Time of Precipitation beginning/end (National Use SYNOP code 909RtDc) |
|  | 0 26 020 | Duration of precipitation |  |
|  |  | Depth of newly-fallen snow |  |
|  | 1 01 000 | Delayed replication of 1 descriptor |  |
|  | 0 31 000 | Short delayed descriptor replication factor |  |
|  | 0 13 012 | Depth of fresh snow | (National SYNOP code 931ss) |
| 3 01 002 |  |  |  |
|  | 0 04 004 | Hour | set to 6 reporting time is always 0600Z |
|  | 0 04 005 | Minute | set to 0 |
|  | 0 04 024 | Time period or displacement in hours | Set to -24 ; values apply to the 24 hour period ending at the last T=06Z |
|  | 1 03 000 | Delayed replication of 3 descriptors replication | Bright sunshine |
|  | 0 31 000 | Delayed descriptor replication factor |  |
|  | 0 02 006 | Data obtained by | Value=1, obtained by derivation |
|  | 0 14 031 | Bright sunshine | (National Use SYNOP code 5SSSS) |
|  | 0 02 006 | Data obtained by | Value=0, Obtained by measurement |
|  | 1 04 000 | Short Delayed replication of 4 descriptors | Snowfall Amount/Water Equivalent |
|  | 0 31 000 | Short D delayed descriptor replication factor |  |
|  | 0 02 175 | Method of precipitation measurement |  |
|  | 0 13 119 | Snowfall amount | New descriptor (National SYNOP code 1SSSS) |
|  | 0 02 175 | Method of precipitation measurement |  |
|  | 0 13 120 | Amount of water equivalent of snow | New descriptor (National SYNOP code 2SwSwSwSw) |
|  | 1 07 000 | Short Delayed replication of 7 descriptors | Max wind speed/direction |
|  | 0 31 000 | Short D delayed descriptor replication factor |  |
|  | 0 08 021 | Time significance | Value=30, "time of occurrence" |
|  | 0 04 004 | Hour | Time of occurrence in hours (National SYNOP code 4FhFTFTFI) |
|  | 0 04 005 | Minutes | Time of occurrence minutes |
|  | 0 11 001 | Wind direction | (National Use SYNOP code 3dmdm) |
|  | 0 08 023 | First order statistics | max, value=2 |
|  | 0 11 002 | Wind speed | (National Use SYNOP code fmfm) |
|  | 0 08 021 | Time significance | Value=0, cancel |
|  | 0 04 024 | Time period in hours | Value=0, reset to initial time value |
| 3 01 002 |  |  |  |
|  | 0 04 004 | Hour | Reset to original value, the observation time |
|  | 0 04 005 | Minute | Reset to original value, the observation time |

| Table reference |  | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y | Element name | Unit | Scale | Reference value | Data width | Unit | Scale | Data width |
| 0 02 006 | Data obtained by | Code table | 0 | 0 | 3 | Code table | 0 | 1 |
| 0 13 119 | Snowfall amount | m | 3 | -10 | 14 | m | 3 | 5 |
| 0 13 120 | Snow amount, water equivalent | kg/m2 | 1 | -10 | 14 | kg/m2 | 1 | 5 |

0 02 006

Data obtained by

| Code figure |  |
| --- | --- |
| 0 | Measurement |
| 1 | Derivation from other data |
| 2-6 | Reserved |
| 7 | Missing |

- 2012-3.2.12(DRC-IV)/New BUFR/CREX entries for dual polarization radar data <para 4.1>

Validate new entries:

|  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | Unit | Scale | Ref. value | Data width | Unit | Scale | Data width |
| 0 33 029 | Correlation coefficient | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |
| 0 21 028 | Specific differential phase | deg/m | 5 | -200 | 11 | deg/m | 2 | 4 |

Add new entries:

Updated on 19 December 2014

|  | Element name | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F X Y |  | Unit | Scale | Ref. value | Data width | Unit | Scale | Data width |
| 0 21 004 | Differential reflectivity | dB | 2 | -800 | 11 | dB | 2 | 4 |
| 0 21 028 | Differential phase | deg | 1 | 0 | 12 | deg | 1 | 4 |
| 0 33 029 | Correlation coefficient | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |

- 2009-3.3.2(DRC-I)/Update from JCOMM on developments for BUFR templates: New XBT template and queries about a revised VOS template (3 06 032 and 3 15 006 are under validation- being confirmed - others have been implemented with revisions) <para 4.1>

The template makes use of existing BUFR descriptors and proposes additional ones. Existing descriptors are indicated with no colour highlighting in the table. New descriptors are indicated with a coloured background. A notes column has been added to provide additional information.

## The XBT template (3 15 005)

| F |  | X |  | Y |  | Name |  | Unit |  | Scale |  | Ref value |  | Data Width (bits) |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 22 |  | 176 |  | Unique identifier for the profile |  | Numeric |  | 0 |  | 0 |  | 33 |  | (1) |  |
| 0 |  | 01 |  | 011 |  | Ship or mobile land station identifier |  | CCITT IA5 |  | 0 |  | 0 |  | 72 |  | (2) |  |
| 0 |  | 01 |  | 103 |  | IMO Number. Unique Lloyd's registry. |  | Numeric |  | 0 |  | 0 |  | 24 |  | (3) |  |
| 0 |  | 01 |  | 087 |  | WMO Marine observing platform extended identifier |  | Numeric |  | 0 |  | 0 |  | 23 |  | (4) |  |
| 0 |  | 01 |  | 019 |  | Long Station or site name |  | CCITT IA5 |  | 0 |  | 0 |  | 256 |  | (5) |  |
| 0 |  | 01 |  | 080 |  | Ship line number according to SOOP |  | CCITT IA5 |  | 0 |  | 0 |  | 32 |  |  |  |
| 0 |  | 05 |  | 036 |  | Ship transect number according to SOOP |  | Numeric |  | 0 |  | 0 |  | 7 |  | (6) |  |
| 0 |  | 01 |  | 013 |  | Speed of motion of moving observing platform |  | m/s |  | 0 |  | 0 |  | 10 |  |  |  |
| 0 |  | 01 |  | 012 |  | Direction of motion of moving observing platform |  | degree true |  | 0 |  | 0 |  | 9 |  |  |  |
| 3 |  | 01 |  | 011 |  | Date |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | 01 |  | 012 |  | Time |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | 01 |  | 021 |  | Latitude and longitude (high accuracy) |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  | 07 |  | 032 |  | Height of sensor above local ground (or deck of marine platform) |  | m |  | 2 |  | 0 |  | 16 |  | (7) |  |
| 0 |  | 07 |  | 033 |  | Height of sensor above water surface |  | m |  | 1 |  | 0 |  | 12 |  | (7) |  |
| 0 |  | 02 |  | 002 |  | Type of instrumentation for wind measurement |  | Flag table |  | 0 |  | 0 |  | 4 |  | (8) |  |
| 0 |  | 11 |  | 002 |  | Wind speed |  | m/s |  | 1 |  | 0 |  | 12 |  |  |  |
| 0 |  | 11 |  | 001 |  | Wind direction |  | degree true |  | 0 |  | 0 |  | 9 |  |  |  |
| 0 |  | 07 |  | 032 |  | Height of sensor above local ground (or deck of marine platform) |  | m |  | 2 |  | 0 |  | 16 |  | (9) |  |
| 0 |  | 07 |  | 033 |  | Height of sensor above water surface |  | m |  | 1 |  | 0 |  | 12 |  | (9) |  |
| 0 |  | 12 |  | 101 |  | Temperature/Dry-bulb temperature |  | K |  | 2 |  | 0 |  | 16 |  |  |  |
| 0 |  | 12 |  | 103 |  | Dew-point temperature |  | K |  | 2 |  | 0 |  | 16 |  |  |  |
| 0 |  | 07 |  | 032 |  | Height of sensor above local ground (or deck of marine platform) (set to missing to cancel previous value) |  | m |  | 2 |  | 0 |  | 16 |  |  |  |
| 0 |  | 07 |  | 033 |  | Height of sensor above water surface (set to missing to cancel previous value) |  | m |  | 1 |  | 0 |  | 12 |  |  |  |
| 3 |  | 02 |  | 021 |  | Waves |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | 02 |  | 056 |  | Sea Surface Temperature |  |  |  |  |  |  |  |  |  | (10) |  |
| 0 |  | 02 |  | 031 |  | Duration and time of current measurement |  | code table |  | 0 |  | 0 |  | 5 |  |  |  |
| 0 |  | 22 |  | 005 |  | Direction of sea surface current |  | degrees true |  | 0 |  | 0 |  | 9 |  |  |  |
| 0 |  | 22 |  | 032 |  | Speed of sea surface current |  | m/s |  | 0 |  | 0 |  | 13 |  |  |  |
| 0 |  | 02 |  | 032 |  | Indicator for digitization |  | code table |  | 0 |  | 0 |  | 2 |  | (11) |  |
| 3 |  | 15 |  | 004 |  | Water temperature profile (Temperature profile observed by XBT or Buoy) |  |  |  |  |  |  |  |  |  | (12) |  |
| 0 |  | 22 |  | 063 |  | Total depth of water |  | m |  | 0 |  | 0 |  | 14 |  |  |  |
| 0 |  | 08 |  | 080 |  | Qualifier for quality class |  | code table |  | 0 |  | 0 |  | 6 |  | (13) |  |
| 0 |  | 33 |  | 050 |  | Global GTSPP quality class |  | code table |  | 0 |  | 0 |  | 4 |  |  |  |
| 0 |  | 22 |  | 178 |  | XBT/XCTD launcher Type |  | code table |  | 0 |  | 0 |  | 8 |  | (14) |  |
| 0 |  | 22 |  | 177 |  | Height of XBT/XCTD Launcher above sea level |  | Numeric |  | 0 |  | 0 |  | 6 |  | (15) |  |
| 0 |  | 22 |  | 067 |  | Instrument type for water temperature profile measurement |  | code table |  | 0 |  | 0 |  | 10 |  |  |  |
| 0 |  | 02 |  | 191 |  | Instrument serial number for water temperature profile measurement |  | CCITT IA5 |  | 0 |  | 0 |  | 64 |  | (16) |  |
| 0 |  | 08 |  | 041 |  | Data significance |  | Code table |  | 0 |  | 0 |  | 5 |  | (17) |  |
| 0 |  | 26 |  | 021 |  | Year |  | year |  | 0 |  | 0 |  | 12 |  |  |  |
| 0 |  | 26 |  | 022 |  | Month |  | month |  | 0 |  | 0 |  | 4 |  |  |  |
| 0 |  | 26 |  | 023 |  | Day |  | day |  | 0 |  | 0 |  | 6 |  |  |  |
| 0 |  | 22 |  | 068 |  | Water temperature profile recorder type |  | code table |  | 0 |  | 0 |  | 7 |  |  |  |
| 0 |  | 25 |  | 061 |  | Data acquisition software type (or name) and version number |  | CCITT IA5 |  | 0 |  | 0 |  | 96 |  | (18) |  |
| 0 |  | 01 |  | 036 |  | Agency in charge of operating the observing platform |  | code table |  | 0 |  | 0 |  | 20 |  |  |  |

Notes:

(1)	Currently some countries are using a 32 bit CRC calculation to generate a unique identifier for the individual BATHY messages. Since missing values in a template have all bits set to 1 and since this may be a legitimate CRC result, we have set the bit width to be 33.

(2)	Place the ship call sign here.

(3)	Values are restricted to be between 0 and 9999999.

(4)	If field 0-01-011 is used, this field will be left missing and vice versa.

(5)	Place the ship name here.

(6)	Integer, assigned by the operator, incremented for each new transect (i.e. all drops have the same transect number while the ship is moving from one end point of the line to the other end point; as soon as the ship arrived to port and goes back to start a new transect then transect number is incremented). The initial value and subsequent values for transect numbers do not matter provided that each new transect by a ship on a line has a transect number higher than previous transect numbers for the same line and the same ship. In case a single cruise follows more than one SOOP line in a row, then the transect number should be incremented each time the cruise changes line.

(7)	This field records the height of the instrument used to make the wind speed and direction measurements.

(8)	Introduced to ensure that information about the certification, or not, of the instrument is retained as present in BATHY.

(9)	This was added to record the height of the instrument used to make the dry bulb temperature measurement.

(10)	This sequence as it stands allows 2 decimal precision on SST with descriptor 0-22-043. As temperatures are stored in kelvin, to convert we must add 273.15. By allowing only 2 decimals we may incur a rounding error. We propose a new sequence as follows:

3-02-090: Sea/water temperature high precision

0 02 038 Method of sea/water temperature measurement

0 07 063 Depth below sea/water surface (cm).  For sea surface temperature measurement

0 22 045 Sea/water temperature

0 07 063 Depth below sea/water surface (cm). Set to missing value to cancel the previous value.

(11)	This descriptor applies to the method used to select depths for the temperature profile encoded through 3-15-004. If temperatures are reported at signiﬁcant depths, the values shall:

(a) Be sufficient to reproduce basic features of the proﬁle and;

(b) Deﬁne the top and the bottom of isothermal layers.

(12) Proposed new sequence as follows. Note that temperatures are stored in K.

3-15-004: Water Temperature Profile

1-06-000	Delayed replication of 6 descriptors

0-31-001	Delayed descriptor replication factor

0-07-063	Depth below sea surface

0-08-080	Qualifier for quality class. Note: set to qualifier = 13

0-33-050	GTSPP quality class

0-22-043	Subsurface sea temperature

0-08-080	Qualifier for quality class. Note: set to qualifier = 11

0-33-050	GTSPP quality class

With an addition (in yellow) in code table 0-08-080 as follows:

0 08 080  Qualifier for GTSPP quality flag

Code	Meaning

figure

0	Total water pressure profile

1	Total water temperature profile

2	Total water salinity profile

3	Total water conductivity profile

4	Total water depth

5-9	Reserved

10	Water pressure at a level

11	Water temperature at a level

12	Salinity at a level

13	Water depth at a level

14-19	Reserved

20	Position

21-62	Reserved

63	Missing value

And an addition (in yellow) in code table 0-33-050 as follows:

0 33 050  Global GTSPP quality flag

Code	Meaning

figure

0	Unqualified

1	Correct value (all checks passed)

2	Probably good but value inconsistent with statistics (differ from climatology)

3	Probably bad (spike, gradient, … if other tests passed)

4	Bad value, Impossible value (out of scale, vertical instability, constant profile)

5	Value modified during quality control

6-7	Reserved

8	Interpolated value

9	Good for operational use; Caution; check literature for other uses

10-14	Reserved

15	Missing value

(13) We require a new entry in table 0 08 080. This has been inserted as code figure 4 and highlighted in yellow in note 12.

(14)	Propose new code table 0-22-178 as follows:

0 22 178  XBT/XCTD Launcher Type

Code

figure

0	Unknown

1	LM-2A Deck-mounted

2	LM-3A Hand-Held

3	LM-4A Thru-Hull

4-9	Reserved

10	AL-12 TSK Autolauncher (up to 12 Probes)

11-19	Reserved

20	SIO XBT Autolauncher (up to 6 probes)

21-29	Reserved

30	AOML XBT V6 Autolauncher (up to 6 Deep Blue probes)

31	AOML XBT V8.0 Autolauncher (up to 8 Deep Blue probes)

32	AOML XBT V8.1 Autolauncher (up to 8 Deep Blue&Fast Deep probes)

33-89	Reserved

90	CSIRO Devil Autolauncher

91-99	Reserved

100	MFSTEP Autolauncher (Mediterranean)

101-254	Reserved

255	Missing

(15)	Values are restricted to 0 to 50m in units of whole m.

(16)	New descriptor to record XBT serial number. Allows up to 8 characters.

(17)	Set the value for this descriptor to be 8 and we require a new code figure in table 0-08-041:

Code 	Meaning

0 	Parent site

1 	Observation site

2 	Balloon manufacture date

3 	Balloon launch point

4 	Surface observation

5 	Surface observation displacement from launch point

6 	Flight level observation

7 	Flight level termination point

8	Instrument manufacture date

9-30 Reserved

31 Missing value

The subsequent date fields then record year, month and day of the manufacturing date of the instrument.

(18)	If 12 characters is insufficient to recorder both name and version, the field width can be extending with the descriptor 2-08-YYY where YYY is the number of characters of the total field. For example, for a name and version number that requires 16 characters, the descriptor would be 2-08-016 and would preceed the 0-25-061 descriptor in the message format part of the BUFR message.

Add proposed template for buoy data including directional and non-directional wave data for validation.

| 3 | 06 | 032 | 0 | 02 | 032 | Indicator for digitization |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 02 | 033 | Method of salinity/depth measurement |
|  |  |  | 1 | 03 | 000 | Delayed replication of 3 descriptors |
|  |  |  | 0 | 31 | 001 | Delayed descriptor replication factor |
|  |  |  | 0 | 07 | 062 | Depth below sea/water surface |
|  |  |  | 0 | 22 | 043 | Subsurface s Sea/water temperature |
|  |  |  | 0 | 22 | 062 | Salinity |
|  |  |  | 0 | 22 | 066 | Water Conductivity |

Add 3 15 006 as an alternative to 3 15 002 having 3 06 004 replaced by 3 06 032 for validation.

|  |  |  |  |  |  | (Typically reported underwater sounding without optional fields) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 006 | 0 | 01 | 011 | Ship or mobile land station identifier Ship’s call sign |
|  |  |  | 3 | 01 | 011 | Year, month, day Date |
|  |  |  | 3 | 01 | 012 | Hour, minute Time |
|  |  |  | 3 | 01 | 023 | Latitude/ and longitude (coarse accuracy) |
|  |  |  | 3 | 06 | 032 | Buoy data including directional and non-directional wave data Depth, temperature, salinity |

- 2008-3.1.15(JM-MTDCF/DRC)/Templates for the wave observations from different platforms suitable for WAVEOB data (revision 4)

<para 4.1>

TM 308015 – BUFR template for WAVEOB data expressed as frequency (Ia=0 in FM-65 WAVEOB)

| # | Descriptor | Name (unit) | Expanded descriptors | Remark, Corresponding groups in FM-65 WAVEOB |
| --- | --- | --- | --- | --- |
|  |  | Identification |  |  |
| 1 | 001003 | WMO Region number/geographical area | 001003 | A1 - First digit of WMO number (e.g. 62024 => 6) |
| 2 | 001020 | WMO Region sub-area | 001020 | bw - Second digit of WMO number (e.g. 62024 => 2) |
| 3 | 001005 | Buoy/platform identifier | 001005 | nb - Last 3 digits of WMO number (e.g. 62024 => 024) |
| 4 | 001011 | Ship or mobile land station identifier | 001011 | D …… D |
| 5 | 001007 | Satellite Identifier | 001007 | I6 I6 I6 |
| 6 | 001001 | WMO block number | 001001 | II |
| 7 | 001002 | WMO station number | 001002 | iii – IIiii only apply to fixed sea stations. |
| 8 | 002044 | Indicator for method of calculating spectral wave data. | 002044 | Im – TAC code table 1744, BUFR code table 2044 |
| 9 | 002045 | Indicator for type of platform | 002045 | Ip – TAC code table 1747, BUFR code table 2045 |
| 10 | 301011 | Year, month, day Date | 004001 (year) 004002 (month) 004003 (day) | J - Date of observation MM YY |
| 11 | 301012 | Hour, minute Time | 004004 (Hour) 004005 (Minutes) | GG - Time of observation Gg |
| - 12 | - 301021 | - Latitude and /longitude (high accuracy) | - 005001 (Lat; high accuracy) - 006001 (Lon; high accuracy) | - Qc La La La La - Lo Lo Lo Lo Lo |
|  |  | - Basic data (WAVEOB Section 0) |  |  |
| - 13 | - 022063 | - Total water depth (m) | - 022063 | - 1hhhh |
| 14 | 022076 | Direction of coming dominant waves (deg) | 022076 | 9dd dd - section 0 |
| 15 | 022077 | Directional spread of dominant wave (deg) | 022077 | ds ds - section 0 |
| 16 | 022094 | Total number of wave bands | 022094 | 111 BT BT – Section 1 |
| 17 | 025043 | Wave sampling interval (Time, s) | 025043 | SSSS – Section 1 |
| 18 | 022078 | Duration of wave record (s) | 022078 | D’D’D’D’ – Section 1 |
| - 19 | - 105002 | - Replecate ion 5 descriptors 2 times | - 105002 | - Replicate over sensor type |
| - 20 | - 002046 | - Wave measurement instrumentation Type of wave sensor | - 002046 | - Code table 2046, 1=heave sensor. 2=slope sensor. |
| 21 | 022070 | Significant wave height (m) | 022070 | 2HsHsHsHs or 6HseHseHseHse - section 0 |
| 22 | 022071 | Spectral peak wave period (s) | 022071 | 3PpPpPpPp or 7PspPspPspPsp - section 0 |
| 23 | 022073 | Maximum wave height | 022073 | 4Hm Hm Hm Hm |
| 24 | 022074 | Average wave period (s) | 022074 | 5PaPaPaPa or 8PsaPsaPsaPsa - section 0 |
|  |  | Spectral Data (WAVEOB Section 1 – 5) |  |  |
| - 25 | - 121000 | - Delayed Replication of 21 descriptors | - 121000 | - Replication over sensor type |
| - 26 | - 031001 | - Delayed descriptor replication factor | - 031001 | - (0,1,2) normally 1 |
| - 27 | - 002046 | - Wave measurement instrumentation Type of wave sensor | - 002046 | - Code table 2046, 1=heave sensor. 2=slope sensor. |
| 28 | 022082 | Maximum non-directional spectral wave density (m2/Hz) | 022082 | Cm Cm Cm - Section 2 or Csm Csm Csm - Section 3 |
| 29 | 022084 | Band containing maximum non-directional spectral wave density | 022084 | nmnm - Section 2 or nsmnsm - Section 3 |
| 30 | 116000 | Delayed replication 16 descriptors | 116000 | Replication over bands. |
| 31 | 031001 | Replication Factor | 031001 | 111 BT BT – Section 1 (Num of band) |
| 32 | 022080 | Waveband central frequency (Hz) | 022080 | 1f1 f1 f1 f1 x …. – Section 1 |
| 33 | 022085 | Spectral wave density ratio | 022085 | 1 c1 c1 c2 c2 …. – Section 2 or 1 cs1 cs1 cs2 cs2 …. – Section 3 |
| 34 | 022086 | Mean direction from which waves are coming (deg) | 022086 | 1da1da1 – Section 4 |
| 35 | 022087 | Principal direction from which waves are coming (deg) | 022087 | da2da2 - Section 4 |
| 36 | 022088 | First normalized polar coordinate from Fourier coefficients | 022088 | r1r1 - Section 4 |
| 37 | 022089 | Second normalized polar coordinate from Fourier coefficients | 022089 | r2r2 - Section 4 |
| 38 | 103000 | Delayed replication of 3 descriptors | 103000 |  |
| 39 | 031001 | Delayed descriptor replication factor | 031001 | n - Section 5 (Num of directions counted); =0 if Ib = 1 (Directional) |
| 40 | 022090 | Non-directional spectral estimate by wave frequency (m2/Hz) | 022090 | 1 A1 A1 A1 x…… - Section 5 |
| 41 | 022186 | Direction from which waves are coming (deg) | 022186 | 1 d1 d1 - Section 5 |
| 42 | 022187 | Directional spread of wave (deg) | 022187 | ds ds - Section 5 |
| 43 | 103000 | Delayed replication of 3 descriptors | 103000 |  |
| 44 | 031001 | Delayed descriptor replication factor | 031001 | n - Section 5 (Num of directions counted); =0 if Ib = 0 (Non-Drectional) |
| 45 | 022092 | Directional Spectral Estimate By Wave Frequency (M2/Hz/Radian) | 022092 | 1 A1 A1 A1 x…… - Section 5 |
| 46 | 022186 | Direction from which waves are coming (deg) | 022186 | 1 d1 d1 - Section 5 |
| 47 | 022187 | Directional spread of wave (deg) | 022187 | ds ds - Section 5 |

TM 308016 - BUFR template for WAVEOB data expressed as the wave number (Ia=1 in FM-65 WAVEOB).

| # | Descriptor | Name | Expanded descriptors | Remark, Corresponding groups in FM-65 WAVEOB |
| --- | --- | --- | --- | --- |
|  |  | Identification |  |  |
| 1 | 001003 | WMO Region number/geographical area | 001003 | A1 - First digit of WMO number (e.g. 62024 => 6) |
| 2 | 001020 | WMO Region sub-area | 001020 | bw - Second digit of WMO number (e.g. 62024 => 2) |
| 3 | 001005 | Buoy/platform identifier | 001005 | nb - Last 3 digits of WMO number (e.g. 62024 => 024) |
| 4 | 001011 | Ship or mobile land station identifier | 001011 | D …… D |
| 5 | 001007 | Satellite identifier | 001007 | I6 I6 I6 |
| 6 | 001001 | WMO block number | 001001 | II |
| 7 | 001002 | WMO station number | 001002 | iii – IIiii only apply to fixed sea stations. |
| 8 | 002044 | Indicator for method of calculating spectral wave data. | 002044 | Im – TAC code table 1744, BUFR code table 2044 |
| 9 | 002045 | Indicator for type of platform | 002045 | Ip – TAC code table 1747, BUFR code table 2045 |
| 10 | 301011 | Year, month, day Date | 004001 (year) 004002 (month) 004003 (day) | J - Date of observation MM YY |
| 11 | 301012 | Hour, minute Time | 004004 (Hour) 004005 (Minutes) | GG - Time of observation Gg |
| - 12 | - 301021 | - Latitude and /longitude (high accuracy) | - 005001 (Lat; high accuracy) - 006001 (Lon; high accuracy) | - Qc La La La La - Lo Lo Lo Lo Lo |
|  |  | - Basic data (WAVEOB Section 0) |  |  |
| - 13 | - 022063 | - Total water depth (m) | - 022063 | - 1hhhh |
| 14 | 022076 | Direction of coming dominant waves (deg) | 022076 | 9dd dd - section 0 |
| 15 | 022077 | Directional spread of dominant wave (deg) | 022077 | ds ds - section 0 |
| 16 | 022094 | Total number of wave bands | 022094 | 111 BT BT – Section 1 |
| 17 | 025044 | Wave sampling interval (Space, m) | 025044 | SSSS – Section 1 |
| 18 | 022079 | Length of wave record (m) | 022079 | D’D’D’D’ – Section 1 |
| - 19 | - 105002 | - Replecateion 5 descriptors 2 times | - 105002 | - Replication over sensor type |
| - 20 | - 002046 | - Wave measurement instrumentation Type of wave sensor | - 002046 | - Code table 2046, 1=heave sensor. 2=slope sensor. |
| 21 | 022070 | Significant wave height (m) | 022070 | 2HsHsHsHs or 6HseHseHseHse - section 0 |
| 22 | 022072 | Spectral peak wave length (m) | 022072 | 3PpPpPpPp - section 0 |
| 23 | 022073 | Maximum wave height | 022073 | 4Hm Hm Hm Hm |
| 24 | 022075 | Average wave length (m) | 022075 | 5PaPaPaPa - section 0 |
|  |  | Spectral Data (WAVEOB Section 1 – 5) |  |  |
| - 25 | - 121000 | - Delayed replication of 21 descriptor | - 121000 | - Replication over sensor type |
| - 26 | - 031001 | - Delayed descriptor replication factor | - 031001 | - (0,1,2) normally 1 |
| - 27 | - 002046 | - Wave measurement instrumentation Type of wave sensor | - 002046 | - Code table 2046, 1=heave sensor. 2=slope sensor. |
| 28 | 022083 | Maximum non-directional spectral wave number (m3) | 022083 | Cm Cm Cm - Section 2 or Csm Csm Csm - Section 3 |
| 29 | 022084 | Band containing maximum non-directional spectral wave density | 022084 | nmnm - Section 2 or nsmnsm - Section 3 |
| 30 | 116000 | Delayed replication of 16 descriptors | 116000 | Replication over band |
| 31 | 031001 | Delayed descriptor replication factor | 031001 | 111 BT BT – Section 1 (Num of band) |
| 32 | 022081 | Waveband central wave number (1/m) | 022081 | 1f1 f1 f1 f1 x …. – Section 1 |
| 33 | 022085 | Spectral wave density ratio | 022085 | 1 c1 c1 c2 c2 …. – Section 2 or 1 cs1 cs1 cs2 cs2 …. – Section 3 |
| 34 | 022086 | Mean direction from which waves are coming (deg) | 022086 | 1da1da1 - Section 4 |
| 35 | 022087 | Principal direction from which waves are coming (deg) | 022087 | da2da2 - Section 4 |
| 36 | 022088 | First normalized polar coordinate from Fourier coefficients | 022088 | r1r1 - Section 4 |
| 37 | 022089 | Second normalized polar coordinate from Fourier coefficients | 022089 | r2r2 - Section 4 |
| 38 | 103000 | Delayed replication of 3 descriptors | 103000 |  |
| 39 | 031001 | Delayed descriptor replication factor | 031001 | n - Section 5 (Num of directions counted); =0 if Ib = 1 (Directional) |
| 40 | 022091 | Non-directional spectral estimate by wave number (m3) | 022091 | 1 A1 A1 A1 x…… - Section 5 |
| 41 | 022186 | Direction from which waves are coming (deg) | 022186 | 1 d1 d1 - Section 5 |
| 42 | 022187 | Directional spread of wave (deg) | 022187 | ds ds - Section 5 |
| 43 | 103000 | Delayed replication of 3 Descriptors | 103000 |  |
| 44 | 031001 | Delayed descriptor replication factor | 031001 | n - Section 5 (Num of directions counted); =0 if Ib = 0 (Non-Drectional) |
| 45 | 022093 | Directional dpectral estimate by eave number (m4) | 022093 | 1 A1 A1 A1 x…… - Section 5 |
| 46 | 022186 | Direction from which waves are coming (deg) | 022186 | 1 d1 d1 - Section 5 |
| 47 | 022187 | Directional spread of wave (deg) | 022187 | ds ds - Section 5 |

Notes:

1. Because WAVEOB can be reported from different platform, the descriptors for identification of all possible platforms are all included in the template. The irrelevant descriptors for identification shell be set to missing.

2. New descriptors

022186 is introduced to express the direction of “any wave”, as appose to the direction of “dominant wave” (022076), “mean direction” (022086), and “principal direction” (022087).

022187 is introduced to express the directional spread of “any wave”, as appose to the directional spread of “dominant wave” (022077).

| TABLE REFERENCE |  |  | TABLE ELEMENT NAME | BUFR |  |  |  | CREX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UNIT | SCALE | REFERENCE VALUE | DATA WIDTH (Bits) | UNIT | SCALE | DATA WIDTH (Characters) |
| F | X | Y |  |  |  |  |  |  |  |  |
| 0 | 22 | 186 | Direction from which waves are coming | Degree true | 0 | 0 | 9 | Character | 0 | 3 |
| 0 | 22 | 187 | Directional spread of wave | Degree | 0 | 0 | 9 | Character | 0 | 3 |

- 2006-5.4.3(JM-MTDCF/DRC)/BUFR template for buoy wave data <para 4.1>

Proposed new template for buoy data, including directional and non-directional wave data

| # | Descriptor | Name | Expanded descriptors | Comment, encoding |
| --- | --- | --- | --- | --- |
| 1 | 001003 | WMO region | 001003 | First digit of WMO number (e.g. 62024 => 6) |
| 2 | 001020 | WMO region sub-area | 001020 | Second digit of WMO number (e.g. 62024 => 2) |
| 3 | 001005 | Buoy/platform identifier | 001005 | Last 3 digits of WMO number (e.g. 62024 => 024) |
| 4 | 002001 | Type of station | 002001 | 1=Manned station |
| 5 | 002036 | Buoy type | 002036 | 1=Fixed buoy |
| 6 | 002149 | Type of data buoy | 002149 | 16=unspecified moored buoy 24=Omnidirectional waverider 25=Directional waverider |
| 7 | 301011 | Date | 004001 (year) 004002 (month) 004003 (day) | Date of observation |
| 8 | 301012 | Time | 004004 (Hour) 004005 (Minutes) | Time of observation |
| 9 | 008021 | Time significance | 008021 | Value = 26 (time of last known position) |
| 10 | 301011 | Date | 004001 (year) 004002 (month) 004003 (day) | Date of last known position coded here; coded missing for fixed station |
| 11 | 301012 | Time | 004004 (Hour) 004005 (Minutes) | Time of last known position coded here; coded missing for fixed station |
| 12 | 008021 | Time significance | 008021 | Value = “missing” |
| 13 | 301021 | Latitude and longitude (high accuracy) | 005001 (Lat; high accuracy) 006001 (Lon; high accuracy) | Coarse accuracy descriptors (005002 and 006002 respectively) were used with PDE buoys |
| 14 | 027004 | Alternate latitude (high accuracy) | 027004 | Coded if Argos is used for location; otherwise coded missing |
| 15 | 028004 | Alternate longitude (high accuracy) | 028004 | Coded if Argos is used for location; otherwise coded missing |
| 16 | 007030 | Height of station above MSL | 007030 |  |
| 17 | 001051 | Platform Transmitter ID | 001051 | If Argos is used, Argos ID number; |
| 18 | 002148 | Data collection and/or Location system | 002148 | 1=Argos 2=GPS Coded missing if none |
| 19 | 001012 | Platform drift direction | 001012 | Coded missing for moored buoys |
| 20 | 001014 | Platform drift speed | 001014 | Coded missing for moored buoys |
| 21 | 002040 | Method of removing platform direction and speed from current | 002040 | Coded missing for moored buoys |
| 22 | 033022 | Quality of buoy satellite transmission | 033022 | 0=Good 1=Dubious 3=missing |
| 23 | 033023 | Quality of buoy location | 033023 | 0=Reliable 1=Last known position 2=Dubious 3=missinh |
| 24 | 033027 | Location quality class (range of radius of 66% confidence) | 033027 | 0: >= 1500m 1: 500m to 1500m |
| 25 | 022063 | Total water depth | 022063 | Mooring depth; otherwise coded missing |
| 26 | 302021 | Waves | 022001 (direction of waves) 022011 (period of waves) 022021 (height of waves) |  |
| 27 | 302022 | Wind waves | 022002 (direction wind wv) 022012 (period wind wv) 022022 (height wind wv) |  |
| 28 | 302023 | Swell waves | 022003 (direction swell wv) 022013 (period swell wv) 022023 (height swell wv) |  |
| 29 | 008081 | Type of equipment (observing platform) | 008081 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value=3: Equipment = “platform” |
| 30 | 025026 | Battery voltage | 025026 | (New descriptor, Volts, scale=0, ref=0, bits=6) Platform battery voltage |
| 31 | 008081 | Type of equipment (transmitter) | 008081 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value=1: Equipment = “transmitter” |
| 32 | 025026 | Battery voltage | 025026 | (New descriptor, Volts, scale=0, ref=0, bits=6) Transmitter battery voltage |
| 33 | 008081 | Type of equipment (receiver) | 008081 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value=2: Equipment = “receiver” |
| 34 | 025026 | Battery voltage | 025026 | (New descriptor, Volts, scale=0, ref=0, bits=6) Receiver battery voltage |
| 35 | 008081 | Type of equipment – value Missing = cancel | 008081 | 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value = “missing” |
| 36 | 002034 | Drogue type | 002034 | Coded missing for moored buoys |
| 37 | 022060 | Lagrangian drifter drogue status | 022060 | (New descriptor, scale=0, ref=0, bits=3) 0=detached 1=attached 3=missing Coded missing for moored buoys |
| 38 | 007070 | Drogue depth | 007070 | Coded missing for moored buoys |
| 39 | 002190 | Lagrangian drifter submergence | 002190 | Coded missing for moored buoys |
| 40 | 025086 | Depth correction indicator for sub-surface measurements along cable | 025086 | 0=depths are not corrected 1=depths are corrected 3=missing |
| 41 | 002035 | Cable length | 002035 | Depth of hydrostatic pressure sensor at bottom of cable |
| 42 | 002168 | Hydrostatic pressure of lower end of cable | 002168 |  |
| 43 | 020031 | Ice deposit (thickness) | 020031 | Ice thickness |
| 44 | 002038 | Method of temperature and/or velocity measurement | 002038 | e.g. 2=hull contact sensor 8=thermistor chain |
| 45 | 306004 | Digitization, depth/salinity method, depths/salinities/temperatures | 002032 (indicator for digit) 002033 (method sal/depth) 103000 (delayed repl 3 desc) 031001 (replication factor) 007062 (depth) 022043 (sea temperature) 022062 (salinity) | Replication factor indicates number of (depth, temp., salinity) data points that are encoded |
| 46 | 002030 | Method of current measurement | 002030 |  |
| 47 | 306005 | Time/duration of current measurement, depths/directions/speeds | 002031 (method current) 103000 (delayed repl 3 desc) 031001 (replicationfactor) 007062 (depth) 022004 (direction current) 022031 (speed current) | Replication factor indicates number of (pepth, dir, speed) data points that are encoded |
| 48 | 007031 | Height of barometer above MSL | 007031 |  |
| 49 | 008081 | Type of equipment (sensor) | 008081 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value=0: Equipment = “sensor” |
| 50 | 012064 | Instrument temperature | 012064 | Temperature of air pressure sensor |
| 51 | 302001 | Pressure and pressure change | 010004 (pressure at station) 010051 (MSLP) 010061 (3-hour tendency) 010063 (tend. Characteristic) | Mean Seal Level Pressure to be computed based upon pressure at station level and sensor height |
| 52 | 008081 | Type of equipment – value missing = cancel | 008081 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor 1=transmitter 2=receiver 3=observing system Here coded with value = “missing” |
| 53 | 007032 | Height of sensor above marine deck platform (for temp.&hum. measurement) | 007032 | Height of thermometer above marine desck |
| 54 | 007033 | Height of sensor above water surface (for temp.&hum. measurement) | 007033 | Height of thermometer (assumed should be coded with value = 2 metres for PDE buoys) |
| 55 | 012101 | Dry-bulb temperature (scale 2) | 012101 | Dry-bulb temperature at 2m (012004) was used for PDE buoys |
| 56 | 012103 | Dew-point temperature (scale 2) | 012103 |  |
| 57 | 013003 | Relative humidity | 013003 |  |
| 58 | 007032 | Height of sensor above marine deck platform (for wind measurement) | 007032 | Real height of anemometer above marine deck |
| 59 | 007033 | Height of sensor above water surface (for wind measurement) | 007033 | Real height of anemometer above average water surface |
| 60 | 008082 | Artificial correction of sensor height to another value | 008082 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor height is not corrected 1=sensor height is artificially corrected 7=missing Assumed should be coded to value 1 for PDE buoys |
| 61 | 007033 | Height of sensor above water surface (here height of anemometer to which it is artificially corrected) | 007033 | Here height of anemometer to which it is artificially corrected Assumed should be coded with value = 10 metres for PDE buoys |
| 62 | 002169 | Anemometer type | 002169 | e.g. 0=rotor 1=propeller rotor |
| 63 | 002002 | Type of instrumentation for wind measurement | 002002 |  |
| 64 | 008021 | Time significance | 008021 | Value = 2 (time averaged) |
| 65 | 004025 | Time period in minutes | 004025 | Value for averaging period (e.g. 10 minutes) |
| 66 | 011001 | Wind direction | 011001 | Wind direction at 10m (011011) was used with PDE buoys |
| 67 | 011002 | Wind speed | 011002 | Wind speed at 10m (011012) was used with PDE buoys |
| 68 | 008021 | Time significance | 008021 | Value = 23 (monitoring period) |
| 69 | 004025 | Time period in minutes | 004025 | Period during which gust is being monitored prior to observation time |
| 70 | 011043 | Maximum wind gust direction | 011043 |  |
| 71 | 011041 | Maximum wind gust speed | 011041 |  |
| 72 | 008082 | Artificial correction of sensor height to another value (set to missing to reset previous value) | 008082 | (New descriptor, scale=0, ref=0, bits=6) 0=sensor height is not corrected 1=sensor height is artificially corrected 7=missing Here coded with value = “missing” |
| 73 | 007033 | Height of sensor above water surface (set to missing to cancel previous value) | 007033 | Value=”missing”: Redefine height to previous level |
| 74 | 007032 | Height of sensor above marine deck platform (for precipitation measurement) | 007032 | Here height of precipitations |
| 75 | 004024 | Time period in hours | 004024 | Period during which precipitation is being monitored prior to observation time |
| 76 | 013011 | Total precipitation | 013011 | Total precipitation during monitoring period |
| 77 | 007032 | Height of sensor above marine deck platform (set to missing to cancel the previous value) | 007032 | Value = “missing” |
| 78 | 008021 | Time significance | 008021 | Value = 3 (accumulated) |
| 79 | 004024 | Time period in hours | 004024 | Period during which global radiation is being accumulated prior to observation time |
| 80 | 014021 | Global radiation, integrated over period specified | 014021 |  |
| 81 | 008021 | Time significance | 008021 | Value = “missing” |
| 82 | 025028 | Operator or manufacturer defined parameter (#1) | 025028 | (New descriptor, scale=1, ref=-16384, bits=15) Housekeeping parameter number 1 |
| 83 | 025028 | Operator or manufacturer defined parameter (#2) | 025028 | (New descriptor, scale=1, ref=-16384, bits=15) Housekeeping parameter number 2 |
| 84 | 025028 | Operator or manufacturer defined parameter (#3) | 025028 | (New descriptor, scale=1, ref=-16384, bits=15) Housekeeping parameter number 3 |
| 85 | 022073 | Maximum wave height | 022073 |  |
| 86 | 022070 | Significant wave height | 022070 | HsHsHsHs in WAVEOB section 0 |
| 87 | 022074 | Average wave period | 022074 | PaPaPaPa in WAVEOB section 0 |
| 88 | 022076 | Direction from which dominant waves are coming | 022076 | dddd in WAVEOB section 0 |
| 89 | 022077 | Directional spread of dominant waves | 022077 | dsds in WAVEOB section 0 |
| 90 | 022071 | Spectral peak wave period | 022071 | PpPpPpPp in WAVEOB section 0 |
| 91 | 022078 | Duration of wave record | 022078 | D’D’D’D’ in WAVEOB section 1 |
| 92 | 022082 | Maximum non-directional spectral wave density | 022082 | CmCmCm in WAVEOB section 2 |
| 93 | 022084 | Band containing maximum non-directional spectral wave density | 022084 | nmnm in WAVEOB section 2 |
| 94 | 025043 | Wave sampling interval (time) | 025043 | SSSS in WAVEOB (Ia=0) |
| 95 | 025044 | Wave sampling interval (space) | 025044 | SSSS in WAVEOB (Ia=1) |
| 96 | 112000 | Delayed replication of 12 descriptors | 112000 | Replication for frequency bands. PDE buoys did not used delayed replication |
| 97 | 031001 | Replication factor | 031001 | Delayed replication therefore added. Replication factor = Number of frequency bands |
| 98 | 022080 | Waveband central frequency | 022080 | fnfnfn in WAVEOB section 1 |
| 99 | 201134 | Add 6 bits to data width | 201134 |  |
| 100 | 022096 | Spectral band width | 022096 | Here coded with 10 bits as descriptor requires 4 bits and we have 6 bits added due to previous operation descriptor |
| 101 | 201000 | Reset data width to normal | 201000 |  |
| 102 | 022090 | Non-directional spectral estimate by wave frequency | 022090 | AnAnAn in WAVEOB (Ib=0) section 5 |
| 103 | 022086 | Mean direction from which waves are coming | 022086 | da1da1 in WAVEOB section 4 |
| 104 | 022087 | Principal direction from which waves are coming | 022087 | da2da2 in WAVEOB section 4 |
| 105 | 022095 | Directional spread of individual waves | 022095 |  |
| 106 | 022085 | Spectral wave density ratio | 022085 | cncn in WAVEOB section 2 |
| 107 | 022088 | First normalized polar coordinate from Fourier coefficients | 022088 | r1r1 in WAVEOB section 4 |
| 108 | 022089 | Second normalized polar coordinate from Fourier coefficients | 022089 | r2r2 in WAVEOB section 4 |
| 109 | 022092 | Directional spectral estimate by wave frequency | 022092 | AnAnAn in WAVEOB (Ib=1) section 5 |