| WORLD WEATHER WATCH COMMISSION FOR BASIC SYSTEMS |  |  |
| --- | --- | --- |
| Ad hoc inter-programme workshop on representation of WIGOS and climate metadata Melbourne, 22-25 June 2015 |  | IPET-MDRD-2/Final Report |
|  |  | 27 October 2015 |

# Final report of the Ad hoc inter-programme workshop on representation of WIGOS and climate metadata

# DISCLAIMER

Regulation 43

Recommendations of working groups shall have no status within the Organization until they have been approved by the responsible constituent body. In the case of joint working groups the recommendations must be concurred with by the presidents of the constituent bodies concerned before being submitted to the designated constituent body.

Regulation 44

In the case of a recommendation made by a working group between sessions of the responsible constituent body, either in a session of a working group or by correspondence, the president of the body may, as an exceptional measure, approve the recommendation on behalf of the constituent body when the matter is, in his opinion, urgent, and does not appear to imply new obligations for Members. He may then submit this recommendation for adoption by the Executive Council or to the President of the Organization for action in accordance with Regulation 9(5).

© World Meteorological Organization, 2015

The right of publication in print, electronic and any other form and in any language is reserved by WMO. Short extracts from WMO publications may be reproduced without authorization provided that the complete source is clearly indicated. Editorial correspondence and requests to publish, reproduce or translate this publication (articles) in part or in whole should be addressed to:

Chairperson, Publications Board

World Meteorological Organization (WMO)

7 bis, avenue de la Paix				Tel.: +41 (0)22 730 84 03

P.O. Box No. 2300				Fax: +41 (0)22 730 80 40

CH-1211 Geneva 2, Switzerland				E-mail: Publications@wmo.int

NOTE:

The designations employed in WMO publications and the presentation of material in this publication do not imply the expression of any opinion whatsoever on the part of the Secretariat of WMO concerning the legal status of any country, territory, city or area or of its authorities, or concerning the delimitation of its frontiers or boundaries.

Opinions expressed in WMO publications are those of the authors and do not necessarily reflect those of WMO. The mention of specific companies or products does not imply that they are endorsed or recommended by WMO in preference to others of a similar nature which are not mentioned or advertised.

This document (or report) is not an official publication of WMO and has not been subjected to its standard editorial procedures. The views expressed herein do not necessarily have the endorsement of the Organization

# FINAL REPORT OF THE AD HOC INTER-PROGRAMME WORKSHOP ON REPRESENTATION OF WIGOS AND CLIMATE METADATA

## 1.    Organization of the meeting

### 1.1.  Welcome and objectives (Dr Barrell (BoM), WMO, Chair)

Dr Barrell, ‎Deputy Director, Observations and Infrastructure at the Bureau of Meteorology, welcomed participants to the meeting. She stressed the importance of WIGOS in underpinning the work of Members, and the critical role of the WIGOS metadata and its exchange.

Mr Bannerman welcomed participants to the meeting, and outlined the objectives of the meeting:  to develop a common understanding within the team of a range of issues surrounding the development of a logical data model to underpin future exchange of meteorological observations data, together with the context of how that observation was made (metadata); and by using this common understanding, to agree on a work plan to further develop the logical data model and to demonstrate its effective use. He proposed working definitions for different types of metadata for the purposes of the meeting: Observations metadata, Discovery metadata, Provenance metadata and IPR metadata (See Annex 3).

On behalf of the Secretary-General, Dr Foreman, Chief of Data Representation Metadata and Monitoring thanked the Bureau of Meteorology for hosting the meeting, and welcomed participants. He emphasized the importance of the meeting for setting the direction for data representations used by WMO, building on the work started through development of WaterML2 and IWXXM, and especially the need to ensure that future WMO data representations were consistent with data representations used in other communities. Particular challenges would be in describing the information in a way that allowed exchange of only a subset of the available metadata corresponding to the most relevant information for the task in hand, and in standardizing vocabularies and structures so that they were sufficiently tightly controlled for operational activities and long term data analyses, while retaining the flexibility to allow research activities to flourish. He also commented that the meeting would only be able to define the key structure for the representations and that the team would need to continue to work after the meeting to complete a coherent standard.

#### 1.1.1. Introduction to WIGOS.

Dr Barrell outlined WIGOS and its objectives as a framework for planning, operating and delivering observations. Following seventeenth Congress, WIGOS was in the pre-operational phase, where most regulatory materials had been developed, but these needed to be implemented to become standard operational practice.

### 1.2.  Adoption of the agenda

The agenda in Annex 1 was adopted. Participants recognized that it would not be possible to resolve all the issues associated with the tasks, and the key priority was to make sure that the issues were understood well enough that solutions could be agreed by correspondence.  The list of participants is in Annex 2.

### 1.3.  Working arrangements

Participants agreed the working hours for the meeting.

## 2.   Background

### 2.1.  Approach to model-driven development of data exchange standards

Mr Tandy summarized the steps needed to develop a model-driven exchange standard. The formal rules and regulations need to be interpreted in terms of a “requirements model” that can be used to define the “conformance classes” needed for data encoding. The conformance classes are used to create an “application schema” (also known as a “conceptual model”) that is a technology-independent description of the content and structure of the information to be exchanged, and that is expressed in UML (Unified Modelling Language). The logical application schema written in UML is “serialized” to create specific data representations, for example an XML schema together with the “schematron” document containing validation rules associated with it.

The International Organization for Standardization has introduced a generic set of standards for representing geographical information (ISO 191xx). WMO has developed an initial set of standards for representing Weather, Water and Climate information (METCE). The workshop would create a “profile” of METCE to add the detail (and additional functionality) needed to represent WIGOS and climate station metadata.

METCE had taken the approach of using external code lists to define lists of permitted values. These were published through http://codes.wmo.int.

### 2.2. UML basics

Mr Lowe introduced the key concepts of the Unified Modelling Language relevant to the work of the team.

### 2.3. Rules for Application Schema; feature types and data types etc.

Mr Lowe introduced the ISO standards for conceptual modelling that provided the context for the data modelling to be performed by the team.

### 2.4. Model-driven development workflow and tools

Mr Lowe explained the tools used in developing models.

Enterprise Architect was used for UML modelling. It was a Windows application but could run under Linux and Apple Mac operating systems using a Windows interface emulator, although a virtual machine with a full version of Windows was preferable.

Editors should use the Professional edition (or higher).

Mr Bannerman agreed to advise the team on which version of Enterprise Architect should be used (the most recent release at the time of the meeting was version 12).

The ISO models could be obtained from http://www.isotc211.org using the “models” item from the menu on the left.

Subversion was open source software for a version control repository.  A temporary repository has been set up for initial development, but a more permanent repository would be needed.

Dr Foreman would investigate whether WMO could host a configuration management repository.

Mr Lowe would grant all members of the team access to the repository.

In addition to a repository, a tool was needed to interface between the local machine and the repository.

Mr Lowe would provide participants with instructions on how to interface Enterprise Architect to the temporary repository.

### 2.5. Overview of Observations and Measurements

Mr Tandy gave a high level overview of ISO 19156:2011 (also known as OGC Observations and Measurements [O&M]). Mr Lowe provided more detail on the sampling features aspect and their relation to sampled features.

### 2.6. Overview of WMO data model: METCE

Mr Tandy outlined the profile of ISO 19156, called METCE(Modéle pour l’Échange des informations sue le Temps, le Climate et l’Eau), that had been developed by IPET-MDRD to describe information about weather, water and climate. METCE was intended for general application; its first application had been limited to representing information in support of international civil aviation. METCE was based on O&M and restricted the “feature of interest” to the type SF_SpatialSamplingFeature, and the “procedure” to type “Process”. Mr Pan described the process that had been used to convert from the UML version of METCE/IWXXM into XML schema and associated schematron documents.

### 2.7. Timeseries and TimeseriesML - a close up look

Mr Lowe described TimeSeriesML that had been generalized from WaterML2 to describe time series of information. This was being developed through the OGC. It consisted of a conceptual model (the Timeseries Profile of O&M) and an XML encoding (TimeseriesML 1.0). Other encodings were under discussion. Timeseries only considered the time variation (at a point, a value representing an area, or other spatially fixed location), rather than a trajectory of locations that varied in time. Timeseries was expressed as a coverage.

## 3.    Review high level requirements for the use of Observations Metadata

### 3.1. Detailed review of WIGOS Metadata specification

Mr Monnik, co-chair of TT-WMD, outlined the drivers for the development of the WIGOS metadata standard. WIGOS metadata should: ensure maximum usefulness of WIGOS observations as the nature of observations changed (expanding beyond the Global Observing System to observations that did not follow traditional WMO standards, or that were made by organizations outside an NHMS), and in an environment where the integrity of observations was increasingly being challenged.

GCOS climate monitoring principle 3 provided a definition of metadata: “details and history of local conditions, instruments, operating procedures, data processing algorithms and other factors pertinent to interpreting data”, and required that the metadata were preserved together with the observations themselves.

Within the Bureau of Meteorology, the SitesDb was used to store the site specific information relevant to observations. In addition to the information needed to interpret observations, it included information needed for managing the observing network. The database contained both structured information (such as contact details) and unstructured information (such as documents and photographs). Specific applications of SitesDb in respect of climate included: changes in location, instrumentation, exposure or environment, observing practice, or in data processing.

Some information in SitesDb (and other metadata databases) was needed for management purposes, but the details might not be suitable for international exchange (such as which individual took certain actions, though the fact that there was a change in individual may be important information to be distributed).

In developing the WIGOS metadata, it was found that there was very little exchange of metadata between NMHS. Global standards were specific to the application area. Each NMHS had its own method for storing observation metadata, and paper records were still the prevalent method of storage.

Although there had been no detailed discussion with least developed countries in developing the WIGOS metadata standard, during the seventeenth World Meteorological Congress several countries had expressed interest in using OSCAR (the WMO database for WIGOS metadata) to store their own metadata.

There were three locations associated with an observation in the WIGOS metadata standard (the locations could be points, areas or volumes): the location of the station, the location of the sensor, and the location of the entity whose properties were being observed.

Mr Klausen described the WIGOS metadata standard. He emphasized the relevance of the standard for meeting national as well as international needs. Eight underpinning principles had guided the development of the WIGOS metadata standard, and there were ten categories of information in the standard (see Annex 3).

WIGOS metadata had several definitions of time and space coordinates (Annex 3).

Some elements of metadata vary only slowly, whereas others may change with each observation. The efficiency of storage and data exchange would be influenced strongly by the effectiveness of the data structures in differentiating between elements that vary on different timescales.

OSCAR surface implemented the WIGOS metadata standard in a database. It was based on the concept of splitting the time series of metadata into homogeneous segments in which the instruments remained the same.

The OSCAR management console covered: Stations (basic characteristics, photos, and history), observations (location, variable, methods [principles on which measurement is based], instruments, quality and uncertainty, and history), contacts and bibliographic references (with the ability to upload associated documents, or to refer to documents that might not normally be publicly available).

When uploading from Volume A into OSCAR several assumptions had to be made; for example, manual stations were assumed to observe each of the elements that were specified in Technical Regulations for manual stations.

In response to questions, Dr Klausen informed participants that there was a traceability chain in the WIGOS metadata model that allowed calibration evidence to be recorded. It was possible to record several instruments that contribute to observing an observed quantity, and to identify which one was providing the “official record.” Mr Pan noted that in Hong Kong, multiple barometers were used and a “voting mechanism” was used to determine which barometer was used to provide the authoritative measurement; however no record was kept of which barometer was responsible for a particular observation.

### 3.2. Observation data in practice: implications for data encoding

#### 3.2.1. CHy requirements for observations metadata and related experience with the Australian Water Act.

Mr Boston presented the needs of CHy for observational metadata and on the Australian experience with metadata for water observations. Hydrological observations may take the form of samples that are sent off for analysis or continuous observations, an example of a complex processing chain.

The Bureau of Meteorology was responsible for hydrological information in Australia. This involved collecting information from about 200 organizations. Metadata were gathered using an Australian format that was similar to, but pre-dated, WaterML2. Metadata requirements were specified for each type of observation.

Mr Pecora described how CHy was planning to share hydrological information among the CHy community (WHOS – WMO Hydrological Observing System). CHy proposed creating a catalogue of all data providers and the information they provided, contributing to the WIS catalogue. The data providers were responsible for delivering the data.

WHOS, in phase 1, would provide a map of locations of NHS that linked to the web pages of those services. The second phase would create an operational system that used a cloud infrastructure to provide a single data broker to avoid the for users to manage many-many format translations that would be needed if users had to retrieve the data directly from each provider, especially important for hydrology because there was little standardization in the data formats used by the providers.

The WHOS data broker would take the metadata from each provider and transform it to the standard WIGOS metadata representation.

#### 3.2.2 Aggregating crowd-sourced observations: WOW

Mr Tandy described the Met Office Weather Observation Website (WoW) that had been developed by the UK Met Office and the Royal Meteorological Society for collecting observations from a wide variety of observers, notably amateurs. The Bureau Meteorology was also using the system to gather observations, as were the Netherlands and New Zealand. As metadata, the system recorded the name, unique identifier, a subjective quality assessment expressed as a star system, the location and exposure of an observing site. The Met Office did not use the crowd-sourced observations in its NWP models because there were too few metadata available to use for quality control. Forecasters did, however, use the observations to qualitatively check their understanding of the current weather situation.

#### 3.2.3. Managing a historical climate record

##### 3.2.3.1. Experiences of Observations Metadata requirements in Developing Countries

Mr Stuber referred to the Climate Data Management System specifications document (WMO-No. 1131). CLICOM was a climate data management system (CDMS) that had been deployed in more than 100 countries; it had been developed between 1985 and 2000. Those components defining the observation metadata (description of the station geography, observations types and observed elements) were used, but the stored information typically only included the location of the stations; the data discovery metadata were not used, and the data quality component was not widely used. There was not a component to record data provenance. About ten countries still used CLICOM.

Although basic information on location and administrative responsibility was widely available for migrating information from CLICOM into more up-to-date CDMSs, other components of the metadata were not readily available, often only being held locally at the observing station. Not only was the information not shared with other NMHS, in many cases the information was not shared between departments within the organization operating the observing stations.

In many cases the route for information to reach the CDMS from the origin of the observation was not clear, and varied with time. Recording such provenance information was important, particularly because different routes might introduce different types of error. This was related to the issue that it was possible for information to flow through different routes to produce operational products, and the different sources of error could result in different “observations” being used to generate inconsistent products. Without provenance information it was not possible to identify when this is likely to occur.

##### 3.2.3.2. Bureau of Meteorology climate data quality management.

Mr Bannerman illustrated the importance of metadata by referring to recent challenges to the historical record in Australia, and the UK Parliamentary enquiry into “Climategate.”

Climate scientists needed to be able to refer to the analytical data that underpinned their publications, including the quality assured observations data that were used to derive the coverages, the “raw” observations data, and information about the instruments, practices and other aspects of creating the observations end product.

Ms Flannery described the role of quality management in the data lifecycle of climate data and the role of metadata in the processes. She noted that there were no international standards on the quality control procedures to be performed for processing observations for use in climate studies, or in the nomenclature used to describe the outcomes of quality control.

Although fully automated quality control was more effective than manual quality control, a system that included manual review of the automated quality control allowed for data recovery and resulted in a more reliable dataset.

##### 3.2.3.3. Experiences in establishing and using a homogenised high quality climate network (ACORN-SAT).

Mr Trewin described how metadata were used in the development of long term climate datasets, with particular emphasis on Australian surface data. Creation of a climatological series needed access to observations over longer periods than observing stations and their instruments remained unchanged. Examples of changes were moves of site, changes in instruments, changes to the local environment and changes in observing practices. Creating a meaningful time series required the observed values to be corrected for the impacts of these changes, and this in turn needed metadata. Statistical comparison with neighbouring stations could indicate records that needed further investigation using the metadata. The Australian SitesDb contained very little metadata from before 1997. Earlier metadata had to be derived from hard copy files, or from instrument registers that had been completed when instruments were installed (mainly before 1920). Observer instructions or procedures, instrument test reports and organization annual reports were additional sources of information. Explaining discrepancies in the time series required manual investigation using a wide variety of formal and informal sources of information about sites.

##### 3.2.3.4. Defending the Climate Record: Responding to Parliamentary Inquiries and Freedom of Information requests

Mr Trewin and Ms Flannery explained that there had been an increase in the number of Freedom of Information requests for meteorological information within Australia. These sometimes re	quested information that was normally considered to be personal information. Imaging of paper records required the images to be protected against alteration. On occasion the regional practices caused the written records to differ from the “official” recorded value, for example where the maximum temperature had been extracted from a recording thermograph to interpolate between thermometer measurements. Australia placed a time limit on providing the information in response to requests, to meet this requirement additional metadata was required.to indicate where the physical records were stored

Although proving that documents could not be changed was a requirement on data archives, there might also be a need to provide assurance that data have not been modified in transit.

Information held in paper documents would need to be discoverable.

Some of the metadata contained information that in some legal regimes might be considered as personal, and thus transfer standards would need to take account of handling that situation.

The purpose of the exchange standard was to support international exchange. National requirements might require further information, but the exchange of this was outside the scope of the standard to be developed by the team. In particular, the data model would not protect the integrity of the metadata resources (this would be the responsibility of the transport layer and the database management layer), though it might need to describe the legal status of the document.

#### 3.2.4. What is the Authoritative record?

Mr Bannerman described the many sources of observations metadata in the Bureau of Meteorology, some of which describe information that was also in SitesDb.

It was the responsibility of the NMHS to provide the authoritative record for the observational metadata corresponding to their observations. OSCAR would be considered a copy of that. The most important application of OSCAR was to provide global observation metadata in a uniform way.

### 3.3. Compilation of user stories that can be used to validate the formal encoding of WIGOS Metadata (outline level only)

Mr Bannerman explained the concept of user stories and how they support the development of data representations. He gave examples of user stories for exchanging observations metadata, for a CDMS observations data exchange specification for systems development, and climate data quality assurance, climate data homogenization, and for actions following a destroyed climate station.

Ms Flannery introduced a use case for adding digitized data into a CDMS and ensuring that the provenance of the data can be recorded.

TT-WMD had considered the use case of a user of observations receiving the associated metadata needed to allow the data to be interpreted correctly for the intended application. There was also a need for a summary level report of the metadata to allow an overview of the metadata to be exchanged (this would be a requirement for a system managing the metadata rather than exchange).

Mr Tandy highlighted the need to share metadata from, for example, an automatic weather station producing reports regularly, a new instrument being installed and requiring the metadata to be updated, and someone needing a long time series with associated metadata needed for homogenization of the data.

It would take about six months to develop a data exchange model. OSCAR was due to go operational in September, and Members would need to be able to upload Volume A information.

Mr Monnik suggested that the work had at least two phases. The first would be to replicate Volume A (without the remarks/comments field), and the second would be a more complete representations.

At the Bureau of Meteorology since 2009, there had been a programme of encouraging users to provide metadata in an XML format. Although most of the large providers had transferred to the new system, the majority of smaller organizations had not adopted the XML approach.

The group was reminded that the formats proposed would need to remain unchanged for many years.

Mr Tandy introduced user stories.

  - Environmental sensors deployed within a smart-city project, both fixed and on municipal buses, that regularly publishes data (hourly summaries and data-stream of continuous sampling). An NMHS consumes these data for incorporation within their urban air quality forecast model subject to their QMS procedures.

  - A faulty stream flow sensor is replaced at a monitoring point within a hydrological network. The manager of the associated monitoring programme updates the authoritative record within the observing system capability database (e.g. OSCAR) to describe the replacement sensor.

  - A data quality manager investigates a data quality issue flagged buy the automated system. She identifies nearby stations (and other sources, e.g. radar) and acquires the data recorded at those stations (including other data streams from the station being investigated) for the period bracketing the quality issue under investigation. The data from adjacent stations, along with the observing system metadata for those stations, is used to determine if the quality issue is genuine or a false positive.

### 3.4. Extending the WIGOS specification to include CDMS

The definition of climate data was expanding to the GCOS Essential Climate Variables (for the purposes of data modelling these variables needed to be more clearly specified).

Lineage and provenance needed to be expressed clearly

ET-CDMS had developed a specification for CDMS that Members were expected to implement over a five year period.

The specifications of metadata within the CDMS contained more information that the WIGOS metadata standard, and it was likely that in the future even more information would need to be stored and exchanged.

- In the future, more detailed recording of intellectual property information was likely to be required. ET-CDMS would like to use WIS to exchange a consistent global climate record, requiring formal definitions of climate data, application schema to support consistent and semantic data exchange. Such exchange would also require more comprehensive data policies, and place constraints on CDMS and associated databases.

Preliminary planning for an open source CDMS had started, and this would require stable interface specifications. Such a system would also provide a tool for maintaining WIGOS metadata.

Meteo Suisse would be able to contribute the OSCAR data model to an Open CDMS.

## 4.  Data Modelling

### 4.1. Integrating Timeseries with METCE

Mr Tandy proposed that METCE import the TimeSeries model packages. This would allow the use of TimeseriesObservation directly. The participants agreed with this approach.

METCE constrained the type of “procedure” (process class) that could be used. It also limited the type of samplingFeature that was used. The ComplexSamplingMeasurement feature type was too restrictive. Mr Tandy therefore proposed that the METCE model and IWXXM model should avoid using the use of these classes in METCE, and instead that the associated constraints should be expressed in IWXXM to reflect the business rules required for aviation. This would require modification of the schemas, Volume I.3 and the Guidelines on WMO Data Modelling to deprecate these in METCE and include them in IWXXM. Mr Tandy and Mr Lowe would make the required changes.

Removing samplingFeature and ComplexSamplingMeasurement from METCE would enable other application areas to define the specializations they needed.

Mr Tandy commented that the IWXXM XML encoding would not change as a result of these changes. As there would be no change on how the data were formatted, this change should be able to be introduced using the Simple procedure.

TimeSeries was limited to point information. Changes would be needed to include coverages. This would need the assistance of the relevant OGC SWG to implement.

### 4.2.  Create a first-cut Application Schema (UML model) for the formal encoding of WIGOS Metadata (based on requirements captured in Day2)

The WIGOS data model was described in the WIGOS documentation for seventeenth Congress (Resolution 17, Cg-17).

The mapping of the WIGOS metadata standard to the METCE model was discussed, and the following mappings identified (see also Annex 4.).

  - Observed variable (measurand) mapped onto OM_Observation.observedProperty.

  - Measurement unit: for a single point this was an attribute in the result in TimeSeries (normally applied to the whole time series).

    - The unit of measurement was also needed as a feature of the observing system (in addition to the data).

Visually estimate values (for example cloud height, or wind from the Beaufort scale), although reported in physical units, might actually be estimated using a categorized system that results in a physical unit.

The model needed to describe how units related to the procedure used.

  - MeasurementContext.measuringinterval mapped to the “Observable range” in the WIGOS nomenclature.

Uncertainty was missing from MeasurementContext and had to be added in METCE (together with the description of how uncertainty was derived [WIGOS categorized the method of determining uncertainty]).

  - Temporal extend [extent], the duration that the station/instrument had been reporting could be reported  within the observation process description; this would allow description of a dataset (phenomenon time), a segment of the metadata record, or the whole metadata series for the station. In the WIGOS standard it was intended to record the whole period covered by the station.

    - In section 1 temporal extent referred to a period during which an observed variable was derived using a standard procedure. This, for the purpose of the representation, was considered to be a dataset, and thus should use MD_Metadata.

    - The result of the OM_Observation (for OSCAR) would be the pointer to the dataset/serivce containing the actual observations.

  - Spatial extent would be represented using MD_Metadata

  - Representativeness of the observation would use the CIMO code table (a controlled vocabulary) and would be represented using a metadata keyword (MD_Metadata).

Define a methodology for associating keywords (and their thesauri) with the WIGOS metadata element they are describing. Mr Lowe and Mr Tandy.

Review the INSPIRE EnvironmentalMonitoringFacility model to identify what can be re-used for CDMS/WIGOS. Mr Tandy and Mr Lowe.

Ensure that the relationship between the environmental monitoring facility and the volume being observed can be recorded. Mr Tandy and Mr Lowe.

Clarify how the geospatial location in station/platform should be specified (contents and representation). Mr Tandy and Mr Lowe.

Each (Oscar) segment would be an OM_Observation instance. The Procedure contained a method, and the method was associated with one or more sensors (linked by a “deployment”).

Mr Tandy and Mr Lowe would propose how to describe the sampling procedure and associated sampling information in a way that was searchable but that had sufficient flexibility to describe enough categories.

Codes.wmo.int was the probable host for code lists. It was less clear where the catalogue/reference should be published to act as authoritative source for other catalogues that would be needed for a “fully reference-able model.”

Clarify the issue of defining the authoritative source of observations metadata and how to access that authoritative source. Monnik and Klausen to clarify with TT-WMD.

The Chair would nominate an expert to identify where in the data exchange model to  record whether the exchange was of a complete record or of only part records, and to propose how partial records could be exchanged so that their relationship with other entities was unambiguous.

Develop specifications and a plan for a testbed for testing the model that allowed the model to be exercised for all use cases. Mr Klausen.

Mr Lowe would recommend to OGC that the timeseries “accuracy” should be called “uncertainty”. The associated existing definition was appropriate.

The Chair would identify an expert to recommend the treatment of quality flags for stations and/or segments – whether this was required, and if so how it should it be done. The preference was to use the ISO 19157 data quality model.

Use DQ_Quality in the result quality element of OM_Observation should be used for recording uncertainty and quality flags.

## 5. Data Modelling and Developing Work Plan

### 5.1. Encoding of sample data

#### 5.1.1	Test of representing CHy data

The case considered was of a hydrological station that had moved from one management structure to another. It measured streamflow, and the cross-section changed over time, resulting in a need to change the rating curve. These time series ere archived, and the records of the station and rating curves needed to be retained.

There was an existing pattern, in WaterML2 part 2, that described “related observations.” Issues identified in the example were listed under the object that should address them (these objects are listed in the following paragraphs or in Annex 4.).

In order to allow recording of only those metadata that were relevant to the context, the application schema definitions of which items are mandatory need not be the same as those in the WIGOS metadata standard.

The concept of EnvironmentMonitoringFacility was proposed to represent the equivalent of a station or platform.

The concept of a Collection was needed to enable a record to consist of several “segments”.

“Collection” needed to contain a time extent.

OM_Observation

  - The records should only include segments for which a sensor was present and observations were produced.

  - For the observed quantity “Streamflow,” during the period of calibration, when the rating curve was unknown, no observations could be made, but the deployment record should show that the instrument was deployed but not reporting.

  - The duration of a segment would be provided using OM_Observation.

  - The data usage constraints were fixed for the whole of the segment.

  - There should be different collections for observations of different quantities.

Additional attributes of OM_Observation were required, including the observing network.

Compound observed properties would not be used.

  - Related observations would be recorded by the relatedObservation attribute (for example wind speed and wind direction).

### 5.3. Testbed design- how will we validate the formal encoding of WIGOS Metadata?

OSCAR implemented the WIGOS metadata, and had a need to ingest information from other systems; as a consequence it could be used to test the effectiveness of the application schema.

The testbed should include the following:

  - Be designed to prove that the data model works, and not to require significant software engineering or implementation.

  - Definition of test cases.

    - Encoding data to consume in OSCAR (the test is expected to use manual encoding).

  - Test cases

    - Mr Tandy’s three use cases (see paras 63.1 to 63.3)

    - Update a subset of the metadata record (e.g.  a sensor deployment record.)

  - A tool (such as XMLSpy) should be used to validate XML

  - Import of data into OSCAR

  - Testing patterns for “by reference” (linked, rather than explicitly included) metadata, including access to the metadata information associated with data values for observations.

  - Peer review though the OGC MetOcean Domain Working Group and Hydrological Domain Working Group.

The testbed phase should not include the following:

  - Proposing a new OGC Standards Working Group.

  - Further collaboration with WMO Commissions beyond that given by the experts on the team.

Commission representatives in the team should report back to their Commissions and seek feedback on the model.

A decision was needed on whether the following should be included in the testbed (collaboration with other communities, further collaboration with OGC, involvement of vendors, source of sample metadata, and the types of metadata to be included.

Considerations when making the decision on what should be included in the testbed were:

  - Collaboration with other communities: communities should only be considered if they had pre-existing resources or infrastructure that was ready to apply to the problem.

  - Creating sample data from an existing database using Extract-Transform-Load (e.g. SitesDb (BoM), DWD, UK Met Office?, Météo France, Météo Suisse).

  - Types of metadata to consider for inclusion included: climate station, hydrological, monitoring points, radar, Argos buoy, upper air, operational station.

### 5.4. Considerations for future iterations including alternative encodings: BUFR, NetCDF / HDF5, RDF?

The initial objective was to produce an application schema that was serialized to XML. Other representations, while necessary for the future, would need to be developed after the XML serialization had been proven. The work plan for these would need to be made following finalization of the XML version.

### 5.5. Develop the work plan

#### 5.5.1.	Infrastructure to support the work plan

In order to develop logical data models and other data representations, IPET-MDRD required access to suitable configuration and issue management tools to be available to support the team. The Secretariat would to investigate how these could be provided.

#### 5.5.2. Creation of WMO regulation (additions to WMO No 306 Manual on Codes Vol I.3)

Governance of entries in wmo.codes.int needed to be clarified. This would be addressed in a teleconference. Mr Bannerman would ensure this was tackled.

## 6. Any other business

There was no other business,

## 7. Closure of the meeting

The meeting closed at 1721 on 25 June 2015

# Action and Decision Summary

## Actions

A1	Mr Bannerman agreed to advise the team on which version of Enterprise Architect should be used (the most recent release at the time of the meeting was version 12).

A2	Dr Foreman would investigate whether WMO could host a configuration management repository.

A3	Mr Lowe would grant all members of the team access to the repository.

A4	Mr Lowe would provide participants with instructions on how to interface Enterprise Architect to the temporary repository.

A5	METCE constrained the type of “procedure” (process class) that could be used. It also limited the type of samplingFeature that was used. The ComplexSamplingMeasurement feature type was too restrictive. Mr Tandy therefore proposed that the METCE model and IWXXM model should avoid using the use of these classes in METCE, and instead that the associated constraints should be expressed in IWXXM to reflect the business rules required for aviation. This would require modification of the schemas, Volume I.3 and the Guidelines on WMO Data Modelling to deprecate these in METCE and include them in IWXXM. Mr Tandy and Mr Lowe would make the required changes.

A6	Define a methodology for associating keywords (and their thesauri) with the WIGOS metadata element they are describing. Mr Lowe and Mr Tandy.

A7	Review the INSPIRE EnvironmentalMonitoringFacility model to identify what can be re-used for CDMS/WIGOS. Mr Tandy and Mr Lowe.

A8	Ensure that the relationship between the environmental monitoring facility and the volume being observed can be recorded. Mr Tandy and Mr Lowe.

A9	Clarify how the geospatial location in station/platform should be specified (contents and representation). Mr Tandy and Mr Lowe.

A10	Mr Tandy and Mr Lowe would propose how to describe the sampling procedure and associated sampling information in a way that was searchable but that had sufficient flexibility to describe enough categories.

A11	Clarify the issue of defining the authoritative source of observations metadata and how to access that authoritative source. Monnik and Klausen to clarify with TT-WMD.

A12	The Chair would nominate an expert to identify where in the data exchange model to  record whether the exchange was of a complete record or of only part records, and to propose how partial records could be exchanged so that their relationship with other entities was unambiguous.

A13	Develop specifications and a plan for a testbed for testing the model that allowed the model to be exercised for all use cases. Mr Klausen.

A14	Mr Lowe would recommend to OGC that the timeseries “accuracy” should be called “uncertainty”. The associated existing definition was appropriate.

A15	The Chair would identify an expert to recommend the treatment of quality flags for stations and/or segments – whether this was required, and if so how it should it be done. The preference was to use the ISO 19157 data quality model.

A16	“Collection” needed to contain a time extent.

A17	Additional attributes of OM_Observation were required, including the observing network.

A18	Commission representatives in the team should report back to their Commissions and seek feedback on the model.

A19	A decision was needed on whether the following should be included in the testbed (collaboration with other communities, further collaboration with OGC, involvement of vendors, source of sample metadata, and the types of metadata to be included.

A20	In order to develop logical data models and other data representations, IPET-MDRD required access to suitable configuration and issue management tools to be available to support the team. The Secretariat would to investigate how these could be provided.

A21	Governance of entries in wmo.codes.int needed to be clarified. This would be addressed in a teleconference. Mr Bannerman would ensure this was tackled.

## Decisions

D1	The purpose of the exchange standard was to support international exchange. National requirements might require further information, but the exchange of this was outside the scope of the standard to be developed by the team. In particular, the data model would not protect the integrity of the metadata resources (this would be the responsibility of the transport layer and the database management layer), though it might need to describe the legal status of the document.

D2	Mr Tandy proposed that METCE import the TimeSeries model packages. This would allow the use of TimeseriesObservation directly. The participants agreed with this approach.

D3	The model needed to describe how units related to the procedure used.

D4	Uncertainty was missing from MeasurementContext and had to be added in METCE (together with the description of how uncertainty was derived [WIGOS categorized the method of determining uncertainty]).

D5	Use DQ_Quality in the result quality element of OM_Observation should be used for recording uncertainty and quality flags.

D6	In order to allow recording of only those metadata that were relevant to the context, the application schema definitions of which items are mandatory need not be the same as those in the WIGOS metadata standard.

D7	Compound observed properties would not be used.

D8	The initial objective was to produce an application schema that was serialized to XML. Other representations, while necessary for the future, would need to be developed after the XML serialization had been proven. The work plan for these would need to be made following finalization of the XML version.

## Annex 1. Agenda

The agenda accepted at the start of the meeting was as follows. This was adjusted during the meeting to avoid repeating topics.

1.    Organization of the meeting

1.1.  Welcome and objectives (Dr Barrell (BoM), WMO, Chair)

1.1.1. Introduction to WIGOS.

1.2.  Adoption of the agenda

1.3.  Working arrangements

2.	Background

2.1.  Approach to model-driven development of data exchange standards

2.2. UML basics

2.3. Rules for Application Schema; feature types and datatypes etc.

2.4. Model-driven development workflow and tools

2.5. A simple example

2.6. Overview of Observations and Measurements

2.7. Overview of WMO data model: METCE

2.8. Timeseries and TimeseriesML - a close up look

3.    Review high level requirements for the use of Observations Metadata

3.1. Detailed review of WIGOS Metadata specification

3.2. Observation data in practice: implications for data encoding

3.2.1 Managing reference data (e.g. station metadata): OSCAR/Surface, Vol A Station Gazetteer and other systems

3.2.1.1. What NMHS are doing.

3.2.2. Compiling a time-series from regular observations: GAWSIS

3.2.3. CHy requirements for observations metadata and related  experience with the Australian Water Act.

3.2.4. Aggregating crowd-sourced observations: WOW

3.2.5. Managing a historical climate record

3.2.5.1. Experiences of Observations Metadata requirements in Developing Countries

3.2.5.2. Bureau of Meteorology climate data quality management.

3.2.5.3. Experiences in establishing and using an homogenised high quality climate network (ACORN-SAT).

3.2.5.4. Defending the Climate Record: Responding to Parliamentary Inquiries and Freedom of Information requests

3.2.6. What is the Authoritative record?

3.3. Compilation of user stories that can be used to validate the formal encoding of WIGOS Metadata (outline level only)

4.  Data Modelling

4.1. Mapping WIGOS Metadata specification to METCE, Timeseries and WMO Core Metadata Profile

4.2. Integrating Timeseries with METCE

4.3. Extension points: how and where to extend the formal encoding of WIGOS Metadata to meet future requirements

4.3.1. Use of Climate Data Management System Specification to provide extensibility requirements

4.4.  Create a first-cut Application Schema (UML model) for the formal encoding of WIGOS Metadata (based on requirements captured in Day2)

4.5. Identification of new controlled vocabularies for publication for WMO Codes Registry (codes.wmo.int)

4.6. Derive GML Application Schema (XML Schema) from the UML

5. Data Modelling and Developing Work Plan

5.1. Encoding of sample data

5.2. Evaluation of encoded sample data against outline use cases (from Day2)

5.3. Testbed design- how will we validate the formal encoding of WIGOS Metadata?

5.4. Considerations for future iterations including alternative encodings: BUFR, NetCDF / HDF5, RDF?

5.5. Develop the work plan

5.5.1. Identify other organisations (if any) with whom we can collaborate to accelerate the work

5.5.2. Creation of WMO regulation (additions to WMO No 306 Manual on Codes Vol I.3)

6. Any other business

7. Closure of the meeting

## Annex 2. Participants

| Bruce BANNERMAN (Chair) Proposed by: Australia CCl | Bureau of Meteorology G.P.O. Box 1289 3001 MELBOURNE VIC Australia Tel: +61 3 96694093 Email: b.bannerman@bom.gov.au |
| --- | --- |
| Tony BOSTON (Member) Proposed by: Australia CHy | Bureau of Meteorology G.P.O. Box 1289 3001 MELBOURNE VIC Australia Fax: N/A Tel: N/A Email: T.Boston@bom.gov.au |
| Dominic LOWE (Member) Proposed by: Australia IPET-MDRD | Bureau of Meteorology 14 Childers Street 2601 CANBERRA ACT Australia Fax: n/a Tel: +61 2 62323015 Email: D.Lowe@bom.gov.au |
| Karl MONNIK (Member) Proposed by: Australia TT-WMD (WIGOS) | Bureau of Meteorology G.P.O. Box 1289 3001 MELBOURNE VIC Australia Fax: +(61 3) 9669 4168 Tel: +(61 3) 9669 4205 Email: k.monnik@bom.gov.au |
| Denis STUBER (Member) Proposed by: France CCl | Météo-France 42, avenue Gaspard Coriolis 31057 TOULOUSE CEDEX France Tel: +33 5 61 07 83 37 Email: denis.stuber@meteo.fr |
| Chi Kin PAN (Member) Proposed by: Hong Kong TT-AvXML/CAeM | Hong Kong Observatory (HKO) 134A Nathan Road KOWLOON Hong Kong, China Fax: +852 2375 2645 Tel: +852 2968 8357 Email: ckpan@hko.gov.hk |
| Silvano PECORA (Member) Proposed by: Italy CHy | ARPA SIMC - AREA IDROLOGIA Via Garibaldi 75 43121 PARMA Italy Fax: +39 0521 774056 Tel: +39 0521 274378 Email: specora@arpa.emr.it |
| Jörg KLAUSEN (Member) Proposed by: Switzerland CAS / TT-WMD (WIGOS) | MeteoSwiss Krähbühlstrasse 58 Postfach 514 CH-8044 ZURICH Switzerland Fax: +41(0)44 256 9278 Tel: +41(0)44 256 9223 Email: joerg.klausen@meteoswiss.ch |
| Jeremy TANDY (Member) Proposed by: United Kingdom IPET-MDRD | Met Office FitzRoy Road EX1 3PB EXETER Devon United Kingdom of Great Britain and Northern Ireland Fax: +44 870 900 5050 Tel: +44 1392 88 6584 Email: jeremy.tandy@metoffice.gov.uk |
| Meaghan FLANNERY (Interested expert) Proposed by: Australia | Bureau of Meteorology G.P.O. Box 1289 3001 MELBOURNE VIC Australia Email: M.Flannery@bom.gov.au |
| Blair TREWIN (Interested expert) Proposed by: Australia | Bureau of Meteorology G.P.O. Box 1289 3001 MELBOURNE VIC Australia Email: B.Trewin@bom.gov.au |

## Annex 3. Working definitions of types of metadata

### Observations Metadata

Time-series data that describe how, when and where meteorological observations were made and the conditions they were made under

### Discovery Metadata

Information intended to facilitate the discovery and assessment of a dataset to determine if it is fit for reuse for a purpose that may be at odds with the reason for which it was originally created

### Data Provenance Metadata

Information relevant to climate data that allows end-users, including data managers, scientists and the general public, to develop trust in the integrity of the climate data

### Data Intellectual Property Metadata

Information describing the Intellectual Property constraints on the data or information being described, provided so that users know the conditions under which the data or information may be used.

## Annex 4. Mapping the WIGOS metadata standard onto the data representation

The following tables are based on those in the WIGOS metadata standard, and the “Id”, “Name”, “Definition”, “Code Table” and “ItemMCO” columns are taken from that standard. The “Mapping” column records the discussion by participants on how elements should be mapped to a data representation compatible with the OGC/ISO O&M and WMO METCE standards.

### Category 1: Observed variable

Specifies the basic characteristics of the observed variable and the resulting datasets. It includes an element describing the spatial representativeness of the observation as well as the biogeophysical compartment the observation describe.

The contents of this Category were contained within the existing O&M standard.

OM_Observation: Note: Only include segments for which a sensor was present and observations were produced.

Note:  Related observations would be recorded by the relatedObservation attribute (for example wind speed and wind direction).

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 1-01 | Observed variable (measurand) | Variable intended to be measured or observed or derived, including the biogeophysical context |  | 1-01 | M* (Phase 1) |
| 1-02 | Measurement unit | Real scalar quantity, defined and adopted by convention, with which any other quantity of the same kind can be compared to express the ratio of the two quantities as a number [VIM3, 1.9] |  | 1-02 | C* (Phase 1) |
| 1-03 | Temporal extent | Time period covered by a series of observations inclusive of the specified date-time indications (measurement history) |  |  | M* (Phase 1) |
| 1-04 | Spatial extent | Typical spatial georeferenced volume covered by the observations |  |  | M* (Phase 1) |
| 1-05 | Representativeness | Spatial extent of the region around the observation for which it is representative |  | 1-05 | O (Phase 2) |

Condition:

{1-02} variables that are measured, rather than classified

### Category 2: Purpose of Observation

Specifies the main application area(s) of the observation and the observing programme(s) and networks the observation is affiliated to.

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 2-01 | Application area(s) | Context within, or intended application(s) for which the observation is primarily made or which has/have the most stringent requirements | MD_Metadata: keyword from code list from specified thesaurus | 2-01 | M* (Phase 1) |
| 2-02 | Program/Network affiliation | The global, regional or national program/network(s) that the station/platform is associated with | Internationally – controlled vocabulary Need national extensions MD_Identification: refer to CI_Citation that refers to CI_Responsibility that refers to CI_RoleCode – extend the code list to include Programme/Network affiliation (from ISO 19115-1:2014). Gmx:anchor can restrict the identifier to a specific vocabulary | 2-02 | M (Phase 1) |

### Category 3: Station/Platform

Specifies the environmental monitoring facility, including fixed station, moving equipment or remote sensing platform at which the observation is made.

A new class of “EnvironmentalMonitoringFacility” was needed to describe the properties of a station/platform (because SF_SpatialSamplingFeature was tied to the location of the observed entity – whereas a satellite was remote from the entity being observed)

+status

+location

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 3-01 | Region of origin of data | WMO Region | Gazeteer reference | 3-01 | C* (Phase 1) |
| 3-02 | Territory of origin of data | Country or territory name of the location of the observation | Gazeteer reference (WMO list) | 3-02 | C* (Phase 1) |
| 3-03 | Station/platform name | Official name of the station/platform | Human readable |  | M (Phase 1) |
| 3-04 | Station/platform type | A categorization of the type of environmental monitoring facility at which an observed variable is measured | Controlled vocab | 3-04 | M* (Phase 2) |
| 3-05 | Station/platform model | The model of the monitoring equipment used at the station/platform | Park this as an issue to be resolved at a later stage |  | M*# (Phase 3) |
| 3-06 | Station/platform unique identifier | A unique and persistent identifier for an environmental monitoring facility (station/platform), which may be used as an external point of reference | WIGOS identifier (as used for the master record in OSCAR) |  | M* (Phase 1) |
| 3-07 | Geospatial location | Position in space defining the location of the environmental monitoring station/platform at the time of observation | For OSCAR this would a representative location (for example the route of a ship, an orbit for a satellite) Use EX_Extent | 11-01 11-02 | M* (Phase 1) |
| 3-08 | Data communication method | Data communication method between the station/platform and some central facility | Code list | 3-08 | O (Phase 2) |
| 3-09 | Station status | Declared reporting status of the station | Code list | 3-09 | M (Phase 1) |

Conditions:

{3-01, 3-02}: Mandatory for fixed land-based stations, optional for mobile stations

### Category 4: Environment

Describes the geographical environment within which the observation is made. It also provides an unstructured element for additional meta-information that is considered relevant for adequate use of the observations and that is not captured anywhere else in the standard.

These were attributes of the station/platform

EnvironmentalMonitoring Facility

+status: timeboundstatus

| Id | Name | Definition | Mapping | Code table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 4-01 | Surface cover | The observed (bio)physical cover on the Earth’s surface in the vicinity of the observation | Code list | 4-01 | C (Phase 3) |
| 4-02 | Surface cover classification scheme | Name and reference or link to document describing the classification scheme | Code list | 4-02 | C (Phase 3) |
| 4-03 | Topography or bathymetry | The shape or configuration of a geographical feature, represented on a map by contour lines | Code list (4 available) concatenated. Encode as four attributes. | 4-03 | C (Phase 3) |
| 4-04 | Events at station/platform | Description of human action or natural event at the station or in the vicinity that may influence the observation | Description and date/time description applies | 4-04 or free text | O (Phase 2) |
| 4-05 | Site information | Non-formalized information about the location and its surroundings at which an observation is made and that may influence it | A collection that can be text, attached files, with associated descriptive text and citation information. (There are existing models for this sort of information – horizons, photos, letters, log books, etc etc). Some are just plain text, others have attributes associated with them. |  | O (Phase 2) |

Conditions:

Either {4-01 and 4-02 and 4-03} or a nilReason=”not applicable” must be reported. For hydrology and satellite observations, specifying nilReason is appropriate.

### Category 5: Instruments and Methods of Observation

Specifies the method of observation and describes characteristics of the instrument(s) used to make the observation. If multiple instruments are used to generate the observation, then this category should be repeated.

Need to add entities:

Deployment

+timeduration

+operatingstatus

+configuration

+heightabovelocalreferencesurface (must include definition of surface)

+instrumentcontrolschedule

+Maintenance schedule

+Maintenance party

+relativelocation (wrt EMF)

Note: The status of the sensor should record the presence of the sensor, even if no observations are being reported

SensorSpecification

+identifier

+instrument specifications (includes recommended maintenance)

+manufacturer

+model

ControlRecord (a log)

+logentry: timestamped, reason, classification….

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 5-01 | Source of observation | The source of the dataset described by the metadata | Code list (Automatic/manual etc) Part of the method (Source and method must be the same for every observation in a segment) | 5-01 | M (Phase 1) |
| 5-02 | Measurement/observing method | The method of measurement/ observation used | Code list | 5-02 | M# (Phase 1) |
| 5-03 | Instrument specifications | Intrinsic capability of the measurement/observing method to measure the designated element, including range, stability, precision, etc. | Referenced by key manufacturer:model ( and recorded in a catalogue. (SensorSpecification) |  | M*# (Phase 1) |
| 5-04 | Instrument operating status +exposure | The status of an instrument with respect to its operation | Include in Deployment | 5-04 | O (Phase 3) |
| 5-05 | Vertical distance of sensor | Vertical distance of the sensor from a (specified) reference level such as local ground, or deck of a marine platform at the point where the sensor is located; or sea surface | Include in Deployment |  | C* (Phase 1) |
| 5-06 | Configuration of instrumentation | Description of any shielding or configuration/setup of the instrumentation or auxiliary equipment needed to make the observation or to reduce the impact of extraneous influences on the observation | In Deployment |  | C# (Phase 3) |
| 5-07 | Instrument control schedule | Description of schedule for calibrations or verification of instrument | In Deployment |  | C (Phase 3) |
| 5-08 | Instrument control result | The result of an instrument control check, including date, time, location, standard type and period of validity | Log. Attached to sensor +logentry: timestamped, reason, classification…. | 5-08 | C# (Phase 3) |
| 5-09 | Instrument model and serial number | Details of manufacturer, model number, serial number and firmware version if applicable | Include in the sensor specification catalogue |  | C# (Phase 3) |
| 5-10 | Instrument routine maintenance | A description of schedule maintenance that is performed on an instrument | In Deployment |  | C# (Phase 3) |
| 5-11 | Maintenance party | Identifier of the organization or individual who performed the maintenance activity | In Deployment |  | O (Phase 2) |
| 5-12 | Geospatial location | Geospatial location of instrument/sensor | In Deployment | 11-01 11-02 | C* (Phase 2) |
| 5-13 | Maintenance Activity | Description of maintenance performed on instrument | Include in same log as control |  | O (Phase 3) |
| 5-14 | Status of observation | Official status of observation | Binary flag. In the OM_Observations. Segments must be able to overlap (for a variable, one segment per sensor) as a parameter. | 5-14 | O (Phase 3) |
| 5-15 | Exposure of instruments | The degree to which an instrument is affected by external influences and reflects the value of the observed variable | In deployment | 5-15 | C (Phase 2) |

Conditions:

{5-07, 5-08, 5-09, 5-10, 5-15} mandatory for instrumental observations

{5-05} mandatory for instrumental observations and if proximity of reference surface impacts on observation

{5-06} mandatory for instrumental observations and if prescribed by “best practice”.

{5-12} mandatory for instrumental observations and if different from station/platform

### Category 6: Sampling

Specifies how sampling and/or analysis are used to derive the reported observation or how a specimen is collected.

Sampling regime may be better handled as a separate feature to allow it to be re-used between many observing stations.

ObservationProcess

+method

+source

+samplingDescription [samplingProcedure (descriptive), samplingTreatment]

+samplingStrategy

+reportingLatency

+traceability

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 6-01 | Sampling procedures | Procedures involved in obtaining a sample | In ObservationProcess |  | O (Phase 3) |
| 6-02 | Sample treatment | Chemical or physical treatment of sample prior to analysis | In ObservationProcess (as a code list) |  | O (Phase 3) |
| 6-03 | Sampling strategy | The strategy used to generate the observed variable | In ObservationProcess Code list | 6-03 | O* (Phase 1) |
| 6-04 | Sampling time period | The period of time over which a measurement is taken | OM_Observation | 11-03 | M# (Phase 3) |
| 6-05 | Spatial sampling resolution | Spatial resolution refers to the size of the smallest observable object. The intrinsic resolution of an imaging system is determined primarily by the instantaneous field of view of the sensor, which is a measure of the ground area viewed by a single detector element in a given instance in time. | OM_Observation |  | M# (Phase 2) |
| 6-06 | Temporal sampling interval | Time period between the beginning of consecutive sampling periods | OM_Observation |  | M (Phase 3) |
| 6-07 | Diurnal base time | Time to which diurnal statistics are referenced | More work needed |  | M (Phase 1) |
| 6-08 | Schedule of observation | Schedule of observation | More work needed |  | M (Phase 1) |

### Category 7: Data Processing and Reporting

Specifies how raw data are transferred into the observed variables and reported to the users.

This needed to be a separate feature “DataProcessingMethod”. Initially this could be free text, and later versions would include more categorized information.

+dataProcessingDescription 1…* (any type)

+processingSoftware  description, verison, …

+processingParty (processing centre) CI_ResponsibleParty 0…*

+dataProcessingLevel

+referenceTimeSource

+numericalResolution (more flexible that METCE resolutionScale because does not have to be a power of 10)

Further investigation was needed to determine the need for a feature TimeSeriesMetadata. that would be based on TimeseriesML to record the history in an appropriate way.

| Id | Name | Definition | Mapping | Code table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 7-01 | Data processing methods and algorithms | A description of the processing used to generate the observation and list of algorithms utilized to derive the resultant value |  |  | O (Phase 3) |
| 7-02 | Processing/analysis center | Center at which the observation is processed. |  |  | O (Phase 2) |
| 7-03 | Temporal reporting period | Time period over which the observed variable is reported |  | 11-03 | M* (Phase 1) |
| 7-04 | Spatial reporting interval | Spatial interval at which the observed variable is reported |  |  | C* (Phase 1) |
| 7-05 | Software/processor and version | Name and version of the software or processor utilized to derive the element value |  |  | O (Phase 3) |
| 7-06 | Level of data | Level of data processing | +dataProcessingLevel | 7-06 | O (Phase 2) |
| 7-07 | Data format | Description of the format in which the observed variable is being provided. | In the collection section of the report |  | M (Phase 3) |
| 7-08 | Version of data format | Version of the data format in which the observed variable is being provided. | In the data collection section of the report |  | M (Phase 3) |
| 7-09 | Aggregation period | Time period over which individual samples/observations are aggregated | TimeseriesMetadata | 11-03 | M (Phase 2) |
| 7-10 | Reference time | Time base to which date and time stamps refer | In dataprocessingmethod +referenceTimeSource | 7-10 | M (Phase 2) |
| 7-11 | Reference datum | Reference datum used to convert observed quantity to reported quantity | +referenceDatum (name, type, value, unit) |  | C (Phase 1) |
| 7-12 | Numerical resolution | Measure of the detail in which a numerical quantity is expressed | METCE – resolutionScale (but not in the right place) +numericalResolution (use same definition as METCE) |  | O (Phase 3) |
| 7-13 | Latency (of reporting) | The typical time between completion of the observation or collection of the datum and when the datum is reported | In ObservationProces +reportingLatency (duration) |  | M (Phase 3) |

Conditions:

{7-04}: mandatory for remote sensing observations and mobile platforms in general

{7-11}: mandatory for stations/platforms that report a derived observation value that depends on a local datum

### Category 8: Data Quality

Specifies the data quality and traceability of the observation

dataQualityCharacteristics

+indicativeUncertainty

+uncertaintyEstimationProcedure

Use DQ_Quality in the result quality element of OM_Observation

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 8-01 | Uncertainty of measurement | Non-negative parameter, associated with the result of a measurement, that characterizes the dispersion of the values that could reasonably be attributed to the observation/ measurand | This is indicative of the typical uncertainty of observations from the source – if known for every value, it would be reported as part of the data set. +indicativeUncertainty For individual data items, use TimeSeries “accuracy” |  | C*# (Phase 2) |
| 8-02 | Procedure used to estimate uncertainty | A reference or link pointing to a document describing the procedures/algorithms used to derive the uncertainty statement | Document or pointer +uncertaintyEstimationProcedure |  |  |
| 8-03 | Quality flag | An ordered list of qualifiers indicating the result of a quality control process applied to the observation | An assessment of the collection can be made using DQ_Element in the Collection For individual reports use TimeSeries | 8-03 | M# (Phase 2) |
| 8-04 | Quality flagging system | Reference to the system used to flag the quality of the observation | An assessment of the collection can be made using DQ_Element in the Collection For individual reports use TimeSeries | 8-04 |  |
| 8-05 | Traceability | Statement defining traceability to a standard, including sequence of measurement standards and calibrations that is used to relate a measurement result to a reference [VIM 3 2.4.2] | Code list In ObservationProcess | 8-05 | C*# (Phase 2) |

Conditions:

{8-01, 8-02 and 8-05} variables that are measured, rather than classified

### Category 9: Ownership & Data Policy

Specifies who is responsible for the observation and owns it.

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 9-01 | Supervising organization | Name of organization who owns the observation | In CI_ResponsibleParty as a parameter of OM_Observation |  | M (Phase 2) |
| 9-02 | Data policy/use constraints | Details relating to the use and limitations surrounding data imposed by the supervising organization | Include parameter dataUsageConstraint, of type MD_LegalConstraints (probably) If different data policies applied to observations at different times, (eg main synoptic hours were Essential, and other hours were Additional), then the data policy could be recorded in otherConstraints usage constraint, record each of WMOEssential, WMOAdditional and a third entry to explain which applies to which data. | 9-02 | M* (Phase 1) |

### Category 10: Contact

Specifies where information about an observation or dataset can be obtained.

| Id | Name | Definition | Mapping | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 10-01 | Contact (Nominated Focal Point) | Principal contact (Nominated Focal Point, FP) for resource | +principalContact: CI_ResponsibleParty in Collection |  | M (Phase 1) |

## Annex 5. Initial Draft Structure of Logical Data Model

## Annex 6. Outline plan

### Key constraints

- CBS-16: formal recommendation of data exchange format. Needs: UML, documentation, serialization to have been written tested and reviewed by June 2016 (to allow time for translation)

- OSCAR needs to import Volume A information

- OSCAR needs to import JCOMM Ops information

### Work plan

| Id | Deliverable | Responsible | Due date |
| --- | --- | --- | --- |
|  | Modified IWXXM and METCE, to provide facilities needed for WIGOS metadata | Tandy | Early September 2015 (before TT-AvXML) |
|  | Presentation to TT-AvXML on proposed changes to IWXXM and METCE | Tandy | TT-AvXML, September 2015 |
|  | Technical regulation changes specified for IWXXM and METCE | Tandy, Lowe, Bannerman | February 2016 |
|  | Complete Fast Track approvals process for IWXXM and METCE | Foreman | August 2016 |
|  | Definition of data quality representation in data model (more detailed mapping required) | Klausen, Monnik, Lowe, Bannerman | August 2015 |
|  | Definition of representation of sampling regime in data model (requirements to be determined, and then model to be updated) | Klausen, Monnik, Lowe, Tandy, Flannery | August 2015 |
|  | Publish initial communications plan | Bannerman | August 2015 |
|  | All developers have access to EA12 and version control software | Lowe, Tandy, Klausen, Monnik, Stuber, Pan, Choy, Pecora | July 2015 |
|  | Team trained in use of EA 12 | Lowe | October 2015 |
|  | Logical data model for WIGOS suitable for review as to its viability for deriving an exchange format. Zeroeth draft. | Lowe | End August 2015 |
|  | Logical data model for WIGOS suitable for review as to its viability for deriving an exchange format. Zeroeth draft. | Lowe | End September 2015 |
|  | Logical data model for WIGOS suitable for review as to its viability for deriving an exchange format. First draft. | Lowe | End October 2015 |
|  | Logical data model for WIGOS suitable for review as to its viability for deriving an exchange format. Second draft. | Lowe | End November 2015 |
|  | Results of Logical Data Model testing in test bed provided to team for technical response. | Klausen | March 2016 |
|  | Logical data model for WIGOS suitable for review as to its viability for deriving an exchange format. Release candidate 1. | Lowe, | End January 2016 |
|  | OGC TC informed of forthcoming public review of logical data model. Other potential reviewers informed. MetOcean DWG, Hydro DWG and others as determined by team. | Bannerman | November 2015 |
|  | Confirmation by representatives from Commissions on team that the November draft of the Logical Data Model is consistent with the needs of their domain requirements | Stuber, Bannerman, Monnik, Pecora, Klausen | December 2015 |
|  | Request for public comments issued (WIS focal points, WIGOS focal points, DWGs, Commission focal points, Programme focal points). Objective to ensure that it is consistent with domain requirements | Bannerman, Foreman | End January 2016 |
|  | End of public review of Logical Data Model | Bannerman, Lowe, Tandy | End March 2016 |
|  | Review public comments, requests for change and results of testbed and recommend modifications to Logical Data Model to accommodate priority requirements. | Monnik | End April 2016 |
|  | Monthly telephone conferences | Foreman | Monthly from July 2015 to June 2016 |
|  | Logical data model documentation (EAP project) available for deriving GML schema and for semi-automatically generating Technical Regulations. Includes response to requests for change from testbed and reviews. Ready for submission to CBS-16 for their recommendation. | Lowe | End of June 2016 |
|  | Recommendation of GML and Technical Regulations by CBS-16 | Foreman | Nov 2016 |
|  | Pre-operational use based on CBS-16 approved versions | Foreman | January 2017 |
|  | Communication list for team set up (cbs-adhoc-01@wmo.int) | Foreman | 26 September 2015 |
|  | Approval by Executive Council | Foreman | June 2016 |
|  | Operational use |  | November 2016 |
|  | Review TT-WMD Terms of Reference to make sure that they cover work to develop the data reference (so TT-WMD members can participate in the work). IPET-MDRD members already covered by IPET-MDRD ToRs, as are ET-CDMS by theirs. [WIGOS Logical Data Model – WLDM] | Monnik | July 2015 |
|  | Wiki repository for documents available | Foreman | End June 2015 |

### Communications Plan

| Id | Stakeholder | Objective | Responsible | Due |
| --- | --- | --- | --- | --- |
| C1 | Techical Commissions | Inform about developments. Seek review comments before submission to CBS | Bannerman | Complete review March 2016 |
| C2 | BALTRAD (Daniel Michelson) | Build confidence that this user community will be able to use the Logical Data Model | Monnik | Complete review March 2016 |
| C3 | JCOMM | Build confidence that this user community will be able to use the Logical Data Model | Monnik | Complete review March 2016 |
| C4 | TT-AvXML | Ensure developments of the Logical Data Model are consistent with the evolution of IWXXM | Tandy | Initial September 2015 |
| C5 | CBS Codes Focal Points | Prepare for consultation on the Logical Data Model so that meaningful review comments are received | Secretariat | Ongoing |
| C6 | OGC MetOcean DWG, HydroDWG (plus mailing lists) | Request public comment and participation in testbed | Bannerman | December 2016 |

## Annex 7. Testbed

The purpose of the testbed was to involve as many impacted communities as possible in the review of the draft of the WIGOS Logical Data Model and to give confidence in the version of the Logical Data Model that would be prepared for formal approval and inclusion in the Manual on Codes.

### Aspects to be included in the testbed

- Proving that the logical data model works (rather than an operational software environment)

- Test cases

  - Environmental sensors deployed within a smart-city project, both fixed and on municipal buses, that regularly publishes data (hourly summaries and data-stream of continuous sampling). An NMHS consumes these data for incorporation within their urban air quality forecast model subject to their QMS procedures.

  - A faulty stream flow sensor is replaced at a monitoring point within a hydrological network. The manager of the associated monitoring programme updates the authoritative record within the observing system capability database (e.g. OSCAR) to describe the replacement sensor.

  - A data quality manager investigates a data quality issue flagged buy the automated system. She identifies nearby stations (and other sources, e.g. radar) and acquires the data recorded at those stations (including other data streams from the station being investigated) for the period bracketing the quality issue under investigation. The data from adjacent stations, along with the observing system metadata for those stations, is used to determine if the quality issue is genuine or a false positive.

  - Updating a subset of a record

- Simple (e.g. XMLSpy) validation of XML

- Manual encoding of data for ingestion to OSCAR

- Importing data into OSCAR

- Testing by reference patterns

- Peer review through OGC MetOcean DWG and HydroDWG

### Aspects to be excluded from the testbed

- Formation of an OGC SWG

- Additional mechanisms beyond those of IPET-MDRD to consult with Technical Commissions

- Creation of software management processes

- Close involvement of vendors

- Validation of approaches for recording observations, the metadata catatolgue

### Aspects that needed more consideration as to whether they should be included in the testbed

- Collaboration with other communities that

  - Unless they are able to provide their own resources and infrastructure and make no demands on the team

- Future collaboration with OGC

- Sources of example metadata (such as BoM Sites Db, DWD, UK Met Office,  MeteoSuisse, climate station, hydrological monitoring point, radar, ocean buioy, upper air, operational land station).

ISO 19757-3:2006. Schematron is defined as “a language for making assertions about the presence or absence of patterns in XML documents.” (This definition is not part of the ISO standard, but was taken (on 14 August 2015) from: http://www.schematron.com/ that is used within the ISO standard as the reference for the definition of Schematron).

OGC: Open Geospatial  Consortium.

An asterisk (*) denotes the element is required for the WIGOS Rolling Review of Requirements (RRR) process. A hash sign (#) denotes that it is acceptable to record a "mandatory" element with a value of nilReason (that indicates that the metadata is either “unknown”, or “not applicable”, or “not available”).

Provided as part of the WIS metadata records

Provided as part of the WIS metadata records