# Proposed modifications to Manual on Codes Vol 3 to accommodate WIGOS metadata

# Prepared by Jeremy Tandy, 21 aUGUST 2015
The attached Vol I.3 has changes tracked ... although it's easier to see what's happening with the changes not shown (e.g. 'final' without markup). I have also created a new branch in the WMO data modelling SVN repository: (https://subversion.assembla.com/svn/xmirepo/branches/metce-amendment-sep2015) within which all the necessary model changes have been made. Note that I have not yet incorporated TimeseriesML- I am expecting this to be completed by another group.

Please can you review the changes if you are able. I will be communicating these changes to the participants of TT-AvXML4 which meets in mid-Sept.

Many thanks, Jeremy

-- Outstanding tasks --

a) implement Schematron contraint for IWXXM classes ‘MeteorologicalAerodromeObservation’, ‘MeteorologicalAerodromeTrendForecast’, ‘MeteorologicalAerodromeForecast’, ‘SIGMETEvolvingConditionAnalysis' and ‘SIGMETPositionAnalysis’ such that the sub-element of om:OM_Observation/om:procedure MUST be metce:Process

b) Updates to 'Guidelines on data modelling for WMO codes’ to reflect the amended modelling practices … (published within WMO No 1140 Abridged final report from CBS Extraordinary Session 2014, Annex 2 to Recommendation 10 (CBS-Ext.(2014)), pp. 265-387)

c) add TimeseriesML as a Code Form within WMO No. 306 Vol I.3; FM-206? … this should be done as part of another task.

d) updating http://codes.wmo.int/common/observation-type/METCE/2013 register with additional observation types from Code Table D-3

e) updating Table numbers in draft Vol I.3, FM-202 and FM-205 … the table numbers are set per section; my version of MS Word wants continuous numbers throughout. Please can I leave this editorial to the Secretariat?

f) update edition information (and publication history table) for Vol I.3. Please can I leave this editorial to the Secretariat?

-- Briefing note --

Based on discussions at the 'Ad hoc inter-programme workshop on representation of WIGOS and climate metadata' (Melbourne, 22-25 June 2015) a number of changes are proposed to the METCE data model relating to the manner by which ISO 19156 Observations and Measurements is extended. The goals of these changes are to enable:

i) TimeseriesML to be integrated within the METCE ‘stack’; and

ii) make METCE applicable to a broader range of meteorological observations by allowing specimens and domain features to be used as subjects for observations (OM_Observation/featureOfInterest).

Analysis of TimeseriesML indicates a very similar approach to METCE; it is a profile of ISO 19156. However, the approach taken regarding the ‘procedure’ and ‘featureOfInterest’ conflict with the rigid constraints applied in METCE.

Firstly, TimeseriesML Requirement http://www.opengis.net/spec/timeseries/1.0/req/uml-timeseries-observation/procedure states

A TimeseriesObservation shall have a procedure property of subtype of either OM_Process (as defined in ISO 19156) or SWE AbstractProcess (as defined in the SWE Common Data Model 2.0, OGC 08-094), or a reference to one of these types. This includes ObservationProcess defined in this standard as a subtype of OM_Process.

In addition to permitting the ‘procedure' property to refer to an instance of any class derived from OM_Process or SWE AbstractProcess, TimeseriesML defines a new sub-class of OM_Process named ‘ObservationProcess’ that provides properties pertinent to a time series observation. In contrast, METCE is constrained such that the ‘procedure’ property shall refer to an instance of class ‘Process' defined within METCE’s ‘Procedure’ Application Schema. Such a tight constraint prevents the use of ‘ObservationProcess’ (from TimeseriesML), sub-class of AbstractProcess (from SWE) or any other sub-class of OM_Process (from ISO 19156) that does not include METCE’s ‘Process’ class in the inheritance chain.

Secondly, TimeseriesML Requirement http://www.opengis.net/spec/timeseries/1.0/req/uml-timeseries-observation/featureOfInterest states:

If the feature of interest of the observation is not a domain feature (e.g. river section, dam, lake) then the featureOfInterest shall be of a subtype of SF_SpatialSamplingFeature, as defined by ISO 19156, which includes MonitoringFeature defined in this standard.

Here we see that ‘featureOfInterest’ property may be a domain feature or spatial sampling feature, whereas METCE constrains this property to only allow use of spatial sampling features.

Finally, when considering meteorological observations in their broadest scope, the workshop participants recognised that it is not uncommon to use specimens, say using ice cores to measure historic atmospheric contaminants. ISO 19156 provides a data model for describing specimens: SF_Specimen, a sub-class of SF_SamplingFeature. As stated above, METCE is currently constrained such that only spatial sampling features (SF_SpatialSamplingFeature) are permitted as the value of the feature of interest.

Given these factors, the participants at the ad-hoc workshop considered the constraints relating to OM_Observation/procedure and OM_Observation/featureOfInterest that are currently specified within METCE’s ‘Observations and Measurements Types’ to be overly restrictive.

Therefore, in order the goals outlined above (integration of TimeseriesML and applicability to a broader scope of meteorological observations) the constraints on properties OM_Observation/procedure and OM_Observation/featureOfInterest specified in METCE’s ‘Observations and Measurement Types’ Application Schema shall be relaxed.

METCE’s ‘Observations and Measurement Types’ Application Schema contains three classes:

i) ComplexSamplingMeasurement - a sub-class of OM_ComplexObservation (as specified in ISO 19156);

ii) SamplingCoverageMeasurement - a sub-class of OM_DiscreteCoverageObservation (as specified in ISO 19156); and

iii) SamplingObservation - a sub-class of OM_Observation.

In each case, the only thing that differentiates the sub-classes from their parent class are the constraints on 'featureOfInterest' and 'procedure' properties. By relaxing the constraints on these properties, there remains no semantic difference between the sub-classes and their parent class; thus removing the need for these specialised sub-classes and, by implication, the ‘Observation and Measurement Types’ Application Schema.

Therefore, we propose to deprecate (or remove) the ‘Observation and Measurement Types’ Application Schema from METCE. In its place, Application Schema, such as IWXXM, would be expected to use the appropriate observation and measurement types specified within ISO 19156. METCE then becomes a ‘toolkit’ of classes that can be incorporated into those Application Schema- such as the ‘Process’ class (from METCE’s ‘Procedure’ Application Schema) that provides a concrete ‘ready to use’ generic class for describing simple observation procedures.

As a result of this proposal, the following changes are required within IWXXM:

- 'METAR/SPECI’ Application Schema

  - Class ‘MeteorologicalAerodromeObservation’ shall be specified as a direct child of ‘OM_ComplexObservation’ (as specified in ISO 19156)

  - Class ‘MeteorologicalAerodromeObservation’ shall specify property ‘procedure’ (referring to type ‘Process’ from METCE’s ‘Procedure’ Application Schema)

  - Class ‘MeteorologicalAerodromeTrendForecast’ shall be specified as a direct child of ‘OM_ComplexObservation’ (as specified in ISO 19156)

  - Class ‘MeteorologicalAerodromeTrendForecast’ shall specify property ‘procedure’ (referring to type ‘Process’ from METCE’s ‘Procedure’ Application Schema)

- 'TAF’ Application Schema

  - Class ‘MeteorologicalAerodromeForecast’ shall be specified as a direct child of ‘OM_ComplexObservation’ (as specified in ISO 19156)

  - Class ‘MeteorologicalAerodromeForecast’ shall specify property ‘procedure’ (referring to type ‘Process’ from METCE’s ‘Procedure’ Application Schema)

- ‘SIGMET’ Application Schema

  - Class ’SIGMETEvolvingConditionAnalysis’ shall be specified as a direct child of ‘OM_Observation’ (as specified in ISO 19156)

  - Class ’SIGMETEvolvingConditionAnalysis’ shall specify property ‘procedure’ (referring to type ‘Process’ from METCE’s ‘Procedure’ Application Schema)

  - Class ’SIGMETPositionAnalysis’ shall be specified as a direct child of ‘OM_Observation’ (as specified in ISO 19156)

  - Class ’SIGMETPositionAnalysis’ shall specify property ‘procedure’ (referring to type ‘Process’ from METCE’s ‘Procedure’ Application Schema)

Note that these changes have no material impact on the XML encoding of IWXXM as this uses the approach defined in OMXML (e.g. a ’soft-typed’ OM_Observation XML element with a ’type’ sub-element that is used to specify the particular sub-class of OM_Observation that is used and thus indicate the semantics that apply). The constraint on the ‘procedure’ property is implemented using Schematron.

These modifications (along with removal of the ‘Observations and Measurement Types’ Application Schema) have been completed in a new branch of the WMO data modelling SVN repository: https://subversion.assembla.com/svn/xmirepo/branches/metce-amendment-sep2015.

WMO No. 306 Manual on Codes Vol I.3 'Part D – Representations derived from data models’ also needs modification in the light of these changes.

- FM-202: METCE

  - Requirements Classes http://def.wmo.int/metce/2013/req/xsd-complex-sampling-measurement (Part D, 202.4), http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement (Part D, 202.5) and http://def.wmo.int/metce/2013/req/xsd-sampling-observation (Part D, 202.6) are removed.

  - All of observation and measurement types defined in ISO 19156 may be used within METCE, therefore new Requirements Classes are added for each of these. These requirements classes assert the dependencies on requirements from OMXML rather than duplicating the information in WMO No. 306 Vol I.3.

    - 202.12 Requirements class: Generic observation data (http://def.wmo.int/metce/2013/req/xsd-observation)

    - 202.13 Requirements class: Measurement data (http://def.wmo.int/metce/2013/req/xsd-measurement)

    - 202.14 Requirements class: Category observation data (http://def.wmo.int/metce/2013/req/xsd-category-observation)

    - 202.15 Requirements class: Count observation data (http://def.wmo.int/metce/2013/req/xsd-count-observation)

    - 202.16 Requirements class: Truth observation data (http://def.wmo.int/metce/2013/req/xsd-truth-observation)

    - 202.17 Requirements class: Geometry observation data (http://def.wmo.int/metce/2013/req/xsd-geometry-observation)

    - 202.18 Requirements class: Temporal observation data (http://def.wmo.int/metce/2013/req/xsd-temporal-observation)

    - 202.19 Requirements class: Complex observation data (http://def.wmo.int/metce/2013/req/xsd-complex-observation)

    - 202.20 Requirements class: Discrete coverage observation data (http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation)

  - Schema location is amended to http://schemas.wmo.int/metce/1.2/metce.xsd

  - The existing members (see below) of Code Table D-3 ‘METCE Observation Types’ (pp.239-240) shall deprecated; there are no observation types now defined in METCE as these will be provided by other Application Schema such as IWXXM. The register http://codes.wmo.int/common/observation-type/METCE/2013 are updated to reflect this change.

    - ComplexSamplingMeasurement

    - SamplingCoverageMeasurement

    - SamplingObservation

  - New members (see below) shall be added to Code Table D-3 for each of the observation and measurement types defined in ISO 19156. The register http://codes.wmo.int/common/observation-type/METCE/2013 shall be updated to reflect this change.

    - OM_CategoryObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CategoryObservation)

    - OM_ComplexObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_ComplexObservation)

    - OM_CountObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CountObservation)

    - OM_DiscreteCoverageObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_DiscreteCoverageObservation)

    - XX [OM_DiscretePointCoverageObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_DiscretePointCoverageObservation)]

    - XX [OM_DiscreteTimeSeriesObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_DiscreteTimeSeriesObservation)]

    - OM_GeometryObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_GeometryObservation)

    - OM_Measurement (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Measurement)

    - OM_Observation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Observation)

    - OM_TemporalObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TemporalObservation)

    - OM_TruthObservation (http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TruthObservation)

  - Editorial changes to sub-clause 202.3 Virtual typing

- FM-205: IWXXM

  - 205.14 Requirements class: Meteorological aerodrome observation

    - Editorial changes: remove references to ComplexSamplingMeasurement

    - Update sub-clause 205.14.4 to refer to requirements and dependencies from '202.19 Requirements class: Complex observation data' (http://def.wmo.int/metce/2013/req/xsd-complex-observation)

    - Update table 51

    - Remove dependency:

      - http://def.wmo.int/metce/2013/req/xsd-complex-sampling-measurement (Part D, 202.4)

    - Add dependency:

      - http://def.wmo.int/metce/2013/req/xsd-process (Part D, 202.10)

    - Add requirement:

      - http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/procedure-metce-process "The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process.”

  - 205.17 Requirements class: Meteorological aerodrome trend forecast

    - ditto as per 205.14 (above) - noting changes to specific references (sub-clause 205.17.4, table 55, requirement […]/xsd-meteorological-aerodrome-trend-forecast/procedure-metce-process)

  - 205.24 Requirements class: Meteorological aerodrome forecast

  - ditto as per 205.14 (above) - noting changes to specific references (sub-clause 205.24.4, table 63, requirement […]/xsd-meteorological-aerodrome-forecast/procedure-metce-process)

- 205.27 Requirements class: SIGMET evolving condition analysis

  - Editorial changes: remove references to SamplingObservation

  - Remove dependency:

    - http://def.wmo.int/metce/2013/req/xsd-sampling-observation (Part D, 202.6)

  - Add dependencies:

    - http://def.wmo.int/metce/2013/req/xsd-process (Part D, 202.10)

  - Add requirement:

    - http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/procedure-metce-process "The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process.”

- 205.30 Requirements class: SIGMET position analysis

  - ditto as per 205.27 (above) - noting changes to specific references (requirement […]/xsd-sigmet-position-analysis/procedure-metce-process)

# Manual on Codes

International Codes

Volume I.3

(Annex II to WMO Technical Regulations)

Part D – Representations derived from data models

WMO-No. 306

2016 edition

Contents

Introduction	11

a. FM system of numbering XML Markup Language Application Schemas	15

FM System of extensible mark up language representations	15

1	Representation of information in extensible markup language	17

2	Unique identifiers to identify Code Table Items and definitions	18

3	Tables and code lists supporting the WMO Logical Data Model	19

3.1	Application of Code Tables and Code Lists	19

3.2	Nil Reasons	19

3.3	Physical Quantities	19

FM-201: COLLECTion of features	22

FM 201-15 Ext	COLLECT-XML Collection of Features	22

201.1	Scope	22

201.2	XML Schema for COLLECT-XML	22

201.3	Requirements Class: Meteorological bulletin	23

FM 202: METCE	25

FM 202-15 Ext	METCE-XML   Foundation building blocks	25

202.1	Scope	25

202.2	XML Schema for METCE-XML	26

202.3	Virtual typing	27

202.7 	Requirements Class: Volcano	28

202.8 Requirements Class: Erupting volcano	29

202.9 Requirements Class: Tropical cyclone	30

202.10 Requirements Class: Process	31

202.11 Requirements Class: Measurement context	33

202.12	Requirements Class: Generic observation data	34

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).	35

202.13	Requirements Class: Measurement data	35

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/measurement has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/measurement (OMXML clause A.2).	36

202.14	Requirements Class: Category observation data	36

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/categoryObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/categoryObservation (OMXML clause A.3).	37

202.15	Requirements Class: Count observation data	37

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/countObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/countObservation (OMXML clause A.4).	38

202.16	Requirements Class: Truth observation data	38

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/truthObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/truthObservation (OMXML clause A.5).	39

202.17	Requirements Class: Geometry observation data	39

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/geometryObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/geometryObservation (OMXML clause A.6).	40

202.18	Requirements Class: Temporal observation data	40

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/temporalObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/temporalObservation (OMXML clause A.7).	41

202.19	Requirements Class: Complex observation data	41

202.20	Requirements Class: Discrete coverage observation data	42

FM 203-15 Ext	OPM-XML  Observable property Model	46

203.1	Scope	46

203.2	XML Schema for OPM-XML	47

203.3	Requirements Class: Observable property	48

203.4	Requirements Class: Composite observable property	49

203.5	Requirements Class: Qualified observable property	50

203.6	Requirements Class: Statistical qualifier	52

203.7	Requirements Class: Constraint	53

203.8	Requirements Class: Category constraint	55

203.9	Requirements Class: Scalar constraint	56

203.10	Requirements Class: Range constraint	57

FM-204: SAF	60

FM 204-15 Ext	SAF-XML  Simple Aeronautical Features	60

204.1	Scope	60

204.2	XML Schema for SAF-XML	61

204.3	Requirements Class: Unique identification	62

204.4	Requirements Class: Aerodrome	63

204.5	Requirements Class: Runway	65

204.6	Requirements Class: Runway direction	66

204.7	Requirements Class: Aeronautical service provision units	68

204.8	Requirements Class: Airspace volume	70

204.9	Requirements Class: Airspace	71

FM-205: IWXXM	74

FM 205-15 Ext	IWXXM-XML  ICAO Meteorological informtion Exchange Model	74

205.1	Scope	74

205.2	XML Schema for IWXXM-XML	76

205.3	Virtual typing	78

205.4	Requirements Class: Cloud layer	78

205.5	 Requirements Class: Aerodrome cloud forecast	80

205.6	 Requirements Class: Aerodrome runway state	82

205.7	 Requirements Class: Aerodrome wind shear	85

205.8	 Requirements Class: Aerodrome observed clouds	86

205.9	 Requirements Class: Aerodrome runway visual range	88

205.10	Requirements Class: Aerodrome sea state	90

205.11	Requirements Class: Aerodrome horizontal visibility	92

205.12	Requirements Class: Aerodrome surface wind	94

205.13	Requirements Class: Meteorological aerodrome observation record	96

205.14	Requirements Class: Meteorological aerodrome observation	101

205.15	Requirements Class: Aerodrome surface wind trend forecast	105

205.16	Requirements Class: Meteorological aerodrome trend forecast record	106

205.17	Requirements Class: Meteorological aerodrome trend forecast	110

205.18	Requirements Class: Meteorological aerodrome observation report	113

205.19	Requirements Class: METAR	116

205.20	Requirements Class: SPECI	117

205.21	Requirements Class: Aerodrome surface wind forecast	117

205.22	Requirements Class: Aerodrome air temperature forecast	118

205.23	Requirements Class: Meteorological aerodrome forecast record	120

205.24	Requirements Class: Meteorological aerodrome forecast	125

205.25	Requirements Class: TAF	128

205.26	Requirements Class: Evolving meteorological condition	133

205.27	Requirements Class: SIGMET evolving condition analysis	135

205.28	Requirements Class: Meteorological position	138

205.29	Requirements Class: Meteorological position collection	139

205.30	Requirements Class: SIGMET position analysis	140

205.31	Requirements Class: SIGMET	143

205.32	Requirements Class: Volcanic ash SIGMET	147

205.33	Requirements Class: Tropical cyclone SIGMET	148

Code Table D-1: Nil Reasons	151

CODE TABLE D-2: Physical quantity kInds	152

Meteorological quantities	153

Oceanographic quantities	154

Aeronautical quantities	154

Code table: FM201 – Collect	157

Code table: FM202 – Metce	158

Code Table D-3: METCE observation types	159

Code table: FM203 – OPM	164

Code table: FM204 – SAF	165

Code table: FM205 – IWXXM	166

Code Table D-4: IWXXM observation types	166

Code Table D-4: IWXXM observable properties	169

Code Table D-5: Aerodrome recent weather	170

Code Table D-6: Aerodrome PRESENT or forecast weather	172

Code Table D-7: Cloud amount reported at aerodrome	197

Code Table D-8: Significant convective cloud type	199

Code Table D-9: Significant weather phenomena	200

Appendix B. Definition of Schemas	203

1	COLLECT-XML	203

2	METCE-XML (Modéle pour l'échange des informations sur le Temps, le Climat et l'Eau)	203

3	OPM-XML (Modéle pour l'échange des informations sur le Temps, le Climat et l'Eau)	203

4	SAF-XML (Simple Aeronautical Features)	203

5	IWXXM-XML (ICAO Meteorological Information Exchange Model)	204

-

-

-

Publication Revision Track Record

| Date | Part/chapter/section | Purpose of amendment | Proposed by | Approval resolution of of Cg or EC |
| --- | --- | --- | --- | --- |
| July 2015 | Part D | Introduce IWXXM. Create Vol I.3. | EC-67 | Resolution ?? Cg-17 |
| Sept 2015 | Part D, FM-202 and FM-205 | Amendments to make METCE more widely applicable |  |  |

EDITORIAL NOTE

Typefaces employed in this volume do not signify standard or recommended practices, and are used solely for legibility.

For the purposes of drafting only:

Text copied from Volume I.1 or Volume I.2 of the Manual on Codes is denoted thus, using Word style "_Moved_destination"

Text deleted from Volume I.1 or Volume I.2 of the Manual on Codes is denoted thus, using Word style "_Deleted text".

Text added to from Volume I.1 or Volume I.2 of the Manual on Codes is denoted thus, using Word style "_Added text".

New text that is not contained within a section of text copied from Volume I.1 or Volume I.2 of the Manual on Codes is in the font and style intended for publication.

[Note to editors: even though they appear to be URLs or standard text, the items highlighted in blue are not intended to link to pages when viewed in a browser. They must not be translated because they are character strings used as identifiers.]

Preface

[Instruction to editors: Copy the Preface from the existing volumes of the Manual on Codes. The Third paragraph of the Preface should be modified in the existing Vol I.1 and Vol I.2 as well as in the new Volume I.3 to match the changes below.]

Rules concerning the selection of code forms to be exchanged for international purposes, and the selection of their symbolic words, figure groups and letters, are laid down in the WMO Technical Regulations, Volume I, Chapter A.2.3 (1988 edition). These code forms are contained in Volume I of the Manual on Codes, issued as Volume I.1 – Part A, and Volume I.2 – Part B and Part C and Volume I.3 – Part D.

[Instruction to Editors: The final paragraph of the Preface (that is specific to the Volume) in Vol I.3 is]

This is the first Edition of Volume I.3 of the Manual on Codes and introduces the use of Extensible Markup Language (XML).

_________

Introduction

[Instruction to Editors: copy the Introduction from the existing volumes I.1 and I.2 into Volume I.3. The changes marked must be made in all three volumes.]

Volume I of the Manual on Codes contains WMO international codes for meteorological data and other geophysical data relating to meteorology; it constitutes Annex II of the WMO Technical Regulations and has therefore the status of a Technical Regulation. It is issued in two three volumes: Volume I.1, containing PART A, and Volume I.2, containing PART B and PART C, Volume I.3 containing PART D.

VOLUME I.1:

Part A – Alphanumeric Codes consists of five sections. The standard coding procedures are distinguished by the use of the term “shall” in the English text, and by suitable equivalent terms in the French, Russian and Spanish texts. Where national practices do not conform with these regulations, Members concerned shall formally notify the Secretary-General of WMO for the benefit of other Members.

VOLUME I.2:

Part B – Binary Codes consists of the list of binary codes with their specifications and associated code tables. Explanatory notes are sometimes added to regulations.

Part C – Common Features to Binary and Alphanumeric Codes consists of the list of table-driven alphanumeric codes with their specifications and associated code tables, and of common code tables to binary and alphanumeric codes and other WMO data representations.

The attachments (yellow background) to Volume I.2 do not have the status of WMO Technical Regulations and are given for information only.

Volume I.3:

Part D – Representations derived from data models consists of the specification of the list of standard representations derived from data models, including those using Extensible Markup Language (XML) with their specifications and associated code tables.

[Note to Editors: the following paragraph is only needed if the draft text contains non-regulatory attachments]

The attachments (yellow background) to Volume I.3 do not have the status of WMO Technical Regulations and are given for information only.

[Instruction to editors: copy the definitions section from Volume I.1 into Volume I.3 Add the following into the definitions section at the appropriate place in all three Volumes].

All-components schema document

An XML schema document that includes, either directly, or indirectly, all the components defined and declared in a namespace.

Application Schema

An Application Schema is a conceptual schema for data required by one or more applications. (Source: ISO 19101:2002, definition 4.2)

Extensible Markup Language (XML)

Extensible Markup Language (XML) is a markup language that defines a set of rules for encoding documents in a format that is both human-readable and machine-readable. It is defined in the W3C XML 1.0 Specification.

GML application schema

A GML application schema is an application schema written in XML Schema in accordance with the rules specified in ISO 19136:2007.

GML document

An XML document with a root element that is one of the XML elements Abstract Feature, Dictionary or TopoComplex specified in the GML schema  or any element of a substitution group of any of these XML elements.

GML schema

The XML schema components in the XML namespace http://www.opengis.net/gml/3.2 as specified in ISO 19136:2007.

Namespace

A namespace is a collection of names, identified by a URI reference, which are used in XML documents as element names and attribute names.

Root element

Each XML document has exactly one root element. This element, also known as the document element, encloses all the other elements and is therefore the sole parent element to all the other elements. The root element provides the starting point for processing the document.

Schematron

Schematron is a definition language for making assertions about patterns found in XML documents, differing in basic concept from other schema languages in that it not based on grammars but on finding patterns in the parsed document.

Uniform Resource Identifier (URI)

A Uniform Resource Identifier (URI) is a compact sequence of characters that identifies an abstract or physical resource. URI syntax is defined in IETF RFC3986.

XML Attribute

A start tag delimiting a XML Element may contain one or more attributes. Attributes are Name-Value pairs, with the Name in each pair referred to as the attribute name and the Value (the text between the ' or " delimiters) as the attribute value. The order of attribute specifications in a start-tag or empty-element tag is not significant.

XML document

A structured document conforming to the rules specified in Extensible Markup Language (XML) 1.0 (Second Edition).

XML element

Each XML document contains one or more elements, the boundaries of which are either delimited by start-tags and end-tags, or, for empty elements, by an empty-element tag. Each element has a type, identified by name, sometimes called its "generic identifier" (GI), and may have a set of attribute specifications. An XML element may contain other XML elements, XML attributes or character data.

XML schema

XML Schema is a definition language offering facilities for describing the structure and constraining the contents of XML documents. The set of definitions for describing a particular XML document structure and associated constraints is referred to an XML Schema Document (XSD).

XML schema document

An XML document containing XML schema component definitions and declarations.

___________

a. FM system of numbering XML Markup Language Application Schemas

Each extensible markup language (XML) application schema bears a number, preceded by the letters FM. This number is followed by a numeral to identify the session of the Commission for Basic Systems that either approved the XML application schema as a new one or made the latest amendment to its previous version. An extensible markup language application schema approved or amended by correspondence after a session of the Commission for Basic Systems receives the number of that session.

Furthermore, an indicator term is used to designate the XML representation colloquially and is therefore called a "code name".

Notes on nomenclature:

(a)	Changes and augmentations to the structure of the XML data representation shall be identified as different “editions”. Each edition of the XML code is allocated a unique namespace. To distinguish between editions, namespaces include EITHER a year field, denoting the year in which those changes and augmentations were begun, OR a version number. For example, FM 202-15 Ext. METCE-XML has namespace http://def.wmo.int/metce/2013 (initial year of work 2013) whilst FM 205-15 Ext. IWXXM- XML has namespace http://icao.int/iwxxm/1.1 (version number 1.1).

(b)	Changes to the content of any of the supporting tables are backward compatible. Terms used within the supporting tables may be deprecated; they will not be deleted. Once changes to the supporting tables are approved, a snapshot containing all the supporting tables required for XML codes will be published. Each snapshot is referred to as a “table version”. The current table version for XML codes is Version 1.

(c)	Backward compatible changes, including addition of new elements or attributes to supporting tables, do not require a new edition of the XML code.

(d)	Further XML code editions and table versions may be generated independently of one another in the future as requirements dictate.

The following table lists XML application schemas included within the FM numbering system, together with the corresponding code names and their reference list of Congress, Executive Council or CBS approval decisions.

FM System of extensible mark up language representations

| FM 201-15 Ext. COLLECT-XML | Collection of reports that use the same XML application schemas. Res. X (Cg-17) |
| --- | --- |
| FM 202-15 Ext. METCE-XML | Meteorological Information. “Modéle pour l’Échange des informations sur le Temps, le Climat et l’Eau.” A set of foundation building blocks to support application schema in the domains of interest to WMO, notably the weather, climate, hydrology, oceanography and space weather disciplines. Res. X (Cg-17) |
| FM 203-15 Ext. OPM-XML | Observable Property Model. Based on work by the Open Geospatial Consortium Sensor Web Enablement Working Group, this allows observable properties (also known as “quantity kinds”) to be aggregated into groups, and for any qualification or constraint relating to those observable properties to be described explicitly. Res. X (Cg-17) |
| FM 204-15 Ext. SAF-XML | Simple Aeronautical Features. Allows items such as airports, or runways to be described to the level of detail required for reporting weather information for international civil aviation purposes. Res. X (Cg-17) |
| FM 205-15 Ext. IWXXM-XML | International Civil Aviation Organization Meteorological Information Exchange Model - defines the reports required by ICAO (with information content equivalent to that in the alphanumeric METAR/SPECI, TAF and SIGMET code forms) that are built from the components of the packages managed by WMO. Res. X (Cg-17) |

1	Representation of information in extensible markup language

1.1	Extensible markup language (XML) documents shall be well-formed with respect to XML 1.0 [Extensible Markup Language (XML) 1.0 (Second Edition)].

Note: the XML implementation specified in this Manual (WMO-No.306 the Manual on Codes) is described using the XML Schema language (XSD) [XML Schema Part 1: Structures (Second Edition), XML Schema Part 2: Datatypes (Second Edition)] and Schematron [ISO/IEC 19757-3:2006, Information technology – Document Schema Definition Languages (DSDL) – Part 3: Rule-based validation – Schematron].

Note: within this Manual, XPath [XML Path Language (XPath) 2.0 (Second Edition)] is used to refer to particular elements and attributes within an XML document.

1.2	XML documents conforming to XML schema that have been allocated an FM identifier within this Manual shall, in addition to conforming to the Regulation for the specified code form, conform to the requirements specified in Clause 2.4 of ISO 19136:2007 [ISO 19136:2007, Geographic information – Geography markup language (GML)].

Note: XML Schema defined in this Manual are conformant to the encoding rules specified in ISO 19136:2007 and are categorized as “GML application schema”. Similarly, XML documents conforming to requirements from ISO 19136:2007 are termed “GML documents”.

Note: Conformance tests for GML documents are provided in Annex A.3 of ISO 19136:2007 - Abstract test suite for GML documents.

Note: the Content-Type [IETF RFC 2387 MIME Multipart/Related Content Type] for GML documents is “application/gml+xml”.

1.3	Information exchanged in XML using the WMO Information System (WIS) shall conform to publicly available GML application schemas.

1.4	Information that is exchanged as XML using the WIS and that capable of being represented according to the GML application schema defined in this Manual should conform to the GML application schema defined within this Manual.

1.5	Creators of GML documents conforming to GML application schema defined in this Manual shall ensure that their GML documents are valid with respect to the associated XML schema documents (XSD).

1.6	Creators of GML documents conforming to GML application schema defined in this Manual shall ensure that their GML documents validate against the associated Schematron schema(s) that test conformance with the specified GML application schema.

Note: it is not necessary for recipients to validate each document.

1.7	All date-time elements shall be encoded using ISO 8601 extended time format [ISO 8601:2004, Data elements and interchange formats – Information interchange – Representation of dates and times].

1.8	The value of each time element shall include a time zone definition according to the ISO 8601 standard. The time zone provided should be UTC.

Note: a time zone is specified using a signed 4 digit character or a ‘Z’ to represent Zulu or Coordinated Universal Time (UTC)  according to the following regular expression: (Z|[+-]HH:MM)

1.9	All units of measure shall use the appropriate code from the Unified Code for Units of Measure (UCUM) code system. The unit of measure shall be identified by encoding the UCUM code in the “uom” attribute of the gml:MeasureType. Where no UCUM code is provided for the unit of measure, the unit of measure should be identified using a URI that resolves to an online definition that is recognised by some level of authority,

Note: the UCUM base codes are available in XML form at: http://unitsofmeasure.org/ucum-essence.xml

Note: a list of unit of measures appropriate to the weather, water and climate domains are provided at http://codes.wmo.int/common/c-6. Each unit of measurement listed therein has a URI identifier.

1.10	Where an xlink:href attribute is used to reference a resource from within an XML document, the xlink:title attribute should not be used to provide a textual description of that resource.

2	Unique identifiers to identify Code Table Items and definitions

2.1	The GML application schemas defined in this Manual make extensive use of externally managed codes and vocabulary items, with majority drawn from Code Tables or Code Lists in Part A, Part B or Part C of WMO-No. 306 Manual on Codes.

2.2	Code or vocabulary items are referenced from within XML documents using the xlink:href attribute [XML Linking Language (XLink) Version 1.1].

2.3	The target code table or vocabulary from which codes or vocabulary items shall, should or may be drawn is defined within the GML application schema using the //annotation/appinfo/vocabulary element within the XML Type definition.

2.4	The level of validation applied when assessing membership of codes or vocabulary items within the target code table or vocabulary is defined within the GML application schema using the //annotation/appinfo/extensibility element within the XML Type definition.

2.4.1	<extensibility> “none” indicates that codes or vocabulary items shall be drawn from the target code table or vocabulary.

2.4.2	<extensibility> “narrower” indicates that codes or vocabulary items shall be drawn from the target code table or vocabulary, or that the code or vocabulary item used shall be derived from another term within the target code table or vocabulary using a more refined, or narrower, definition.

2.4.3	<extensibility> “any” indicates that codes or vocabulary items may be drawn from the target code table, code list or vocabulary or any other code table or vocabulary deemed appropriate by the author.

2.5	Code or vocabulary items referenced from within GML documents should have an available online definition and have been recognised by some level of authority.

2.6	Each code list managed by WMO in support of XML application schemas shall have a unique identifier of the form: http://codes.wmo.int/<identifier>.

Note: The recommended practice for selecting <identifier> is to base it on the WMO Number of the document defining the appropriate regulation, and the table within that document. An example of a unique identifier is http://codes.wmo.int/306/4678/BLSN.

Note: WMO provides a web service that makes the unique references are "resolvable". That means that if unique identifier, such as http://codes.wmo.int/306/4678/BLSN, is entered as a URL into a browser the definition of the item corresponding to the unique reference is displayed.

3	Tables and code lists supporting the WMO Logical Data Model

3.1	Application of Code Tables and Code Lists

Regulations specified in Code Tables or Code Lists in Part A, Part B or Part C of WMO-No. 306 Manual on Codes shall apply to the corresponding entries in code tables used within the GML application schemas defined in this Manual.

3.2	Nil Reasons

3.2.1	Nil reason terms from Part D code table D-1 shall, where permitted within the GML application schemas defined in this manual, be used to provide an explanation for recording a missing (or void) value within a GML document.

Note: Part D code table D-1 is described in Appendix A.

Note: Part D code table D-1 is published online at http://codes.wmo.int/common/nil.

3.2.2	Each nil reason term is identified with a URI [IETF RFC 3986 Uniform Resource Identifier (URI): Generic Syntax]. The URI shall comprise the “Code-space” column concatenated with the “Notation” column of Part D code table D-1.

3.3	Physical Quantities

3.3.1	Terms from Part D code Table D-2 shall be used within the GML application schemas defined in this Manual to describe physical quantity kinds.

3.3.2	Each physical quantity kind is identified with a URI [IETF RFC 3986 Uniform Resource Identifier (URI): Generic Syntax]. The URI shall comprise the path http://codes.wmo.int/common/c-15/ concatenated with the value listed in the “Notation” column of Part D code Table D-2: Physical quantity kinds.

4	References

4.1	Normative References

Extensible Markup Language (XML) 1.0 (Second Edition), W3C Recommendation (6 October 2000)

XML Schema Part 1: Structures (Second Edition), W3C Recommendation (28 October 2004)

XML Schema Part 2: Datatypes (Second Edition), W3C Recommendation (28 October 2004)

Namespaces in XML 1.0 (Third Edition), W3C Recommendation  (8 December 2009)

XML Linking Language (XLink) Version 1.1, W3C Recommendation  (6 May 2010)

Note for translators: these documents are only available in English, French or Russian. The title of the document for these languages should be copied from and the exact title is needed - so for other languages, use the English version

. http://www.iso.org/iso/home/store/catalogue_ics/catalogue_ics_browse.htm?ics1=35&ics2=240&ics3=30&=

The choice of language is at the top of the web page next to the shopping cart.

ISO/IEC 19757-3:2006, Information technology – Document Schema Definition Languages (DSDL) – Part 3: Rule-based validation – Schematron

ISO 8601:2004, Data elements and interchange formats – Information interchange – Representation of dates and times

ISO 19136:2007, Geographic information – Geography markup language (GML)

ISO/TS 19139:2007, Geographic information – Metadata – XML schema implementation

OGC/IS 08-094r1 SWE Common Data Model Encoding Standard 2.0

OGC/SAP 09-146r2 GML Application Schema – Coverages 1.0.1

OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation

4.2	Informative References

XML Path Language (XPath) 2.0 (Second Edition), W3C Recommendation (14 December 2010; correction 3 January 2011)

Note for translators: the following (unhighlighted) se documents are only available in English, French or Russian. The title of the document for these languages should be copied from and the exact title is needed - so for other languages, use the English version

. http://www.iso.org/iso/home/store/catalogue_ics/catalogue_ics_browse.htm?ics1=35&ics2=240&ics3=30&=

The choice of language is at the top of the web page next to the shopping cart.

ISO 19103:2005 Geographic information – Conceptual schema language

ISO 19109:2005 Geographic information – Rules for application schema

ISO 19123:2005 Geographic information – Schema for coverage and geometry functions

ISO 19156:2011 Geographic information – Observations and measurements

IETF RFC 2387 MIME Multipart/Related Content Type (August 1998)

IETF RFC 3986 Uniform Resource Identifier (URI): Generic Syntax (January 2005)

FM-201: COLLECTion of features

FM 201-15 Ext	COLLECT-XML	Collection of Features

201.1	Scope

COLLECT-XML should be used to represent a collection of GML feature instances of the same type of meteorological information. The intent is to allow XML encoded meteorological information to be packaged in a way that emulates the existing data distribution practices used within the Global Telecommunication System (GTS) and Aeronautical Fixed System (AFS).

Note: the collection of meteorological information is often referred to as a bulletin.

Note: XML encodings of meteorological information are defined in Part D; for example FM 205-15 EXT IWXXM-XML.

Note: aggregation of meteorological information in form of meteorological bulletins usually takes place at a station or centre originating or compiling the bulletin, as agreed internationally.  A meteorological bulletin may have one or more instances of meteorological information.  If meteorological reports of routine messages are not available during compilation, a NIL report of that station should be included in the published contents of the bulletin.

The Requirements Classes defined in COLLECT-XML are listed in Table 1.

Table 1. Requirements Classes defined in COLLECT-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin, Part D, 201.3 |

201.2	XML Schema for COLLECT-XML

201.2.1	Representations of information in COLLECT-XML shall declare the XML namespaces listed in Table 2 and Table 3.

Note: additional namespace declarations may be required depending on the XML elements used within COLLECT-XML. In particular, the meteorological information included within the bulletin is likely to imply specific requirements regarding namespace declaration.

Note: Schematron schemas providing additional constraints are provided as an external file to the XSD defining COLLECT-XML. The canonical location of this file is: http://schemas.wmo.int/rule/1.1/collect.sch.

Table 2. XML namespaces defined for COLLECT-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://def.wmo.int/collect/2014 | collect | http://schemas.wmo.int/collect/1.1/collect.xsd |

Table 3. External XML namespaces used in COLLECT-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XSLT v2 | http://www.w3.org/1999/XSL/Transform | xsl |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink |  |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |

201.3	Requirements Class: Meteorological bulletin

201.3.1	This requirements class is used to describe the collection of GML feature instances of meteorological information.

201.3.2	XML elements describing a meteorological bulletin shall conform to all Requirements specified in Table 4.

Table 4. Requirements class xsd-meteorological-bulletin

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin |  |
| Target Type | Data instance |
| Name | Meteorological bulletin |
| Requirement | http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin/valid The content model of this element shall have a value that matches the content model of collect:MeteorologicalBulletin. |
| Requirement | http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin/bulletin-identifier The value of XML element collect:MeteorologicalBulletin/bulletinIdentifier shall conform to the general file naming convention described in the Manual on the Global Telecommunication System (WMO No. 386), Attachment II-15. |
| Requirement | http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin/meteorological-information The XML element collect:MeteorologicalBulletin shall contain one or more child elements collect:MeteorologicalBulletin/collect:meteorologicalInformation, each of which shall contain one and only one child element expressing a report of meteorological information. |
| Requirement | http://def.wmo.int/collect/2014/req/xsd-meteorological-bulletin/consistent-meteorological-information-type An instance of collect:MeteorologicalBulletin shall contain only one type of meteorological information reports. All child elements of XML element collect:MeteorologicalBulletin/collect:meteorologicalInformation shall be of the same type, and hence have the same qualified name. |

Note: in the context of the file naming convention, abbreviated headings are described in Manual on the Global Telecommunication System (WMO No. 386), Attachment II-15, 2.3.2.

Note: meteorological information reports include METAR, SPECI, TAF and SIGMET – represented using XML elements iwxxm:METAR, iwxxm:SPECI, iwxxm:TAF and iwxxm:SIGMET.

Note: the qualified name of a METAR is iwxxm:METAR which is of type iwxxm:METARType.

FM 202: METCE

FM 202-15 Ext	METCE-XML	 	Foundation building blocks

202.1	Scope

METCE-XML shall be used for the exchange in extensible markup language (XML) of meteorological information conforming to the “Modéle pour l’Échange des informations sur le Temps, le Climat et l’Eau.” (METCE) application schema. METCE-XML may be used directly to encode meteorological information or incorporated as components within other XML encodings.

Note: the METCE application schema is described in the informal document Guidelines on Data Modelling for WMO Codes (available in English only from http://wis.wmo.int/metce-uml).

The Requirements Classes defined in METCE-XML are listed in Table 1.

Table 1. Requirements Classes defined in METCE-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-volcano, Part D, 202.7 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-erupting-volcano, Part D, 202.8 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone, Part D, 202.9 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-measurement-context, Part D, 202.11 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-observation, Part D, 202.12 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-measurement, Part D, 202.13 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-category-observation, Part D, 202.14 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-count-observation, Part D, 202.15 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-truth-observation, Part D, 202.16 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-geometry-observation, Part D, 202.17 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-temporal-observation, Part D, 202.18 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-complex-observation, Part D, 202.19 |
| Requirements Class | http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation, Part D, 202.20 |

202.2	XML Schema for METCE-XML

202.2.1	Representations of information in METCE-XML shall declare the XML namespaces listed in Table 2 and Table 3.

Note: additional namespace declarations may be required depending on the XML elements used within the METCE-XML.

Note: the XML Schema is packaged in three XML schema documents (XSD) describing one XML namespace: http://def.wmo.int/metce/2013.

Note: Schematron schemas providing additional constraints are embedded within the XSD defining METCE-XML.

Table 2. XML namespaces defined for METCE-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://def.wmo.int/metce/2013 | metce | http://schemas.wmo.int/metce/1.2/metce.xsd |

Table 3. External XML namespaces used in METCE-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XSLT v2 | http://www.w3.org/1999/XSL/Transform | xsl |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink |  |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |
| ISO/TS 19139:2007 metadata XML implementation | http://www.isotc211.org/2005/gmd | gmd | http://standards.iso.org/ittf/PubliclyAvailableStandards/ ISO_19139_Schemas/gmd/gmd.xsd |
| OGC OMXML | http://www.opengis.net/om/2.0 | om | http://schemas.opengis.net/om/2.0/observation.xsd |
| FM 203-15 Ext. OPM-XML | http://def.wmo.int/opm/2013 | opm | http://schemas.wmo.int/opm/1.1/opm.xsd |

202.3	Virtual typing

202.3.1	In accordance with OMXML (clause 7.2), the specialization of OM_Observation is provided through schematron restriction. The om:type element shall be used to specify the type of OM_Observation that is being encoded using the URI for the corresponding observation type listed in Part D code table D-3.

Note: Part D code Table D-3 is described in Part D Appendix A.

Note: Part D code table D-3 is published online at http://codes.wmo.int/common/observation-type/METCE/2013.

Note: the URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of OM_ComplexObservation is http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_ComplexObservation.

Note: Each URI will resolve to provide further information about the associated observation type.

Note: The terms “observation” and “measurement” evoke a particular concept to meteorologists (e.g. the measurement of physical phenomena using an instrument or sensor). As defined in ISO 19156:2011, Geographic information – Observations and Measurements, an instance of OM_Observation is defined as an “estimate of the value of some property of some feature of interest using a specified procedure”. OM_Measurement is clearly applicable to the measurement of some physical property values using an instrument or sensor, but is equally applicable to the numerical simulation of physical property values using a computational model (e.g. a forecast or reanalysis).

202.4	Requirements Class: Complex Sampling Measurement

202.4.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a set of values relating to a specified location and time instant or duration, the ‘feature of interest’ is a representative subset of the atmosphere or body of  wateretc. based on a predetermined sampling regime and the ‘procedure’ provides the set of information as specified by WMO.

Note: ComplexSamplingMeasurement (a subclass of OM_ComplexObservation) is intended for use where the observation event is concerned with the evaluation of multiple measurands at a specified location and time instant or duration. OM_ComplexObservation is used because the 'result' of this class of observations is a group of measures, provided as a Record (as defined in ISO 19103:2005 Geographic information – Conceptual schema language).

202.4.2	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/common/observation-type/METCE/2013/ComplexSamplingMeasurement shall conform to all Requirements specified in Table 4.

202.4.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/common/observation-type/METCE/2013/ComplexSamplingMeasurement shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of metce:ComplexSamplingMeasurement is dependent on:

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

SWE Common 2.0 [OGC/IS 08-094r1 SWE Common Data Model Encoding Standard 2.0]

Table 4. Requirements class xsd-complex-sampling-measurement

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-complex-sampling-measurement |  |
| Target Type | Data instance |
| Name | Complex sampling measurement |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/complexObservation, OMXML clause 7.10 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components, SWE Common 2.0 clause 8.1 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components, SWE Common 2.0 clause 8.2 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings, SWE Common 2.0 clause 8.5 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/general-encoding-rules, SWE Common 2.0 clause 9.1 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/text-encoding-rules, SWE Common 2.0 clause 9.2 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xml-encoding-rules, SWE Common 2.0 clause 9.3 |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-complex-sampling-measurement/xmlns-declaration-swe The OGC SWE Common 2.0 namespace http://www.opengis.net/swe/2.0 shall be declared within the XML document. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-complex-sampling-measurement/procedure-metce-process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Recommendation | The default namespace prefix used for http://www.opengis.net/swe/2.0 should be “swe”. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/complexObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/complexObservation (OMXML clause A.8).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-components (SWE Common 2.0 clause A.8).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-record-components (SWE Common 2.0 clause A.9).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-encodings (SWE Common 2.0 clause A.12).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/general-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/general-encoding-rules (SWE Common 2.0 clause A.14).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/text-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/text-encoding-rules (SWE Common 2.0 clause A.15).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xml-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xml-encoding-rules (SWE Common 2.0 clause A.16).

Note: the canonical schema location for OGC SWE Common 2.0 (http://www.opengis.net/swe/2.0) is http://schemas.opengis.net/sweCommon/2.0/swe.xsd.

202.5	Requirements Class: Sampling Coverage Measurement

202.5.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a set of values describing the variation of properties with space and/or time, the ‘feature of interest’ is a representative subset of the atmosphere or body of water etc. based on a predetermined sampling regime and the ‘procedure’ provides the set of information as specified by WMO.

Note: SamplingCoverageMeasurement (a subclass of OM_DiscreteCoverageObservation) is intended for use where the observation event is concerned with the evaluation of measurands that vary with respect to space and/or time. OM_DiscreteCoverageObservation is used because the 'result' of this class of observations is a discrete coverage (as defined in ISO 19123:2005 Geographic information – Schema for coverage and geometry functions).

Note: SamplingCoverageMeasurement is based on the informative SamplingCoverageObservation specialization of OM_Observation outlined in ISO 19156:2011 clause D.3.4. Within METCE, additional restrictions are applied to the ‘procedure’. Furthermore, the name is changed from “SamplingCoverageObservation” to “SamplingCoverageMeasurement” in an attempt to disambiguate the two classes and to mitigate confusion arising from use of the term “observation”.

202.5.2	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/common/observation-type/METCE/2013/SamplingCoverageMeasurement shall conform to all Requirements specified in Table 5.

202.5.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/common/observation-type/METCE/2013/SamplingCoverageMeasurement shall conform to all Requirements of all relevant dependencies specified in Table 5.

Note: XML implementation of metce:ComplexSamplingMeasurement is dependent on:

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

SWE Common 2.0 [OGC/IS 08-094r1 SWE Common Data Model Encoding Standard 2.0]

GMLCOV 1.0 [OGC/SAP 09-146r2 GML Application Schema – Coverages 1.0.1]

Table 5. Requirements class xsd-sampling-coverage-measurement

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement |  |
| Target Type | Data instance |
| Name | Sampling coverage measurement |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components, SWE Common 2.0 clause 8.1 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components, SWE Common 2.0 clause 8.2 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-block-components, SWE Common 2.0 clause 8.4 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings, SWE Common 2.0 clause 8.5 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/general-encoding-rules, SWE Common 2.0 clause 9.1 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/text-encoding-rules, SWE Common 2.0 clause 9.2 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xml-encoding-rules, SWE Common 2.0 clause 9.3 |
| Dependency | http://www.opengis.net/spec/gmlcov/1.0/req/gml-coverage, GMLCOV 1.0 clause 6 |
| Dependency | http://www.opengis.net/spec/gmlcov/1.0/req/gml, GMLCOV 1.0 clause 7 |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/xmlns-declaration-swe The OGC SWE Common 2.0 namespace http://www.opengis.net/swe/2.0 shall be declared within the XML document. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/xmlns-declaration-gmlcov The OGC GMLCOV 1.0 namespace http://www.opengis.net/gmlcov/1.0 shall be declared within the XML document. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/result-discrete-or-grid-coverage The XML element om:result shall contain a child element gml:DiscreteCoverage (or any element of a substitution group of gml:DiscreteCoverage), gml:GridCoverage, gml:RectifiedGridCoverage or gml:ReferenceableGridCoverage. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/result-coverage-gml-encoding The child element of om:result shall be represented in GML as defined in GMLCOV 1.0 clause 7. Multipart representation and special format representation shall not be used. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/procedure-metce-process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Recommendation | The default namespace prefix used for http://www.opengis.net/swe/2.0 should be “swe”. |
| Recommendation | The default namespace prefix used for http://www.opengis.net/gmlcov/1.0 should be “gmlcov”. |

202.4	This clause has been removed.

202.5	This clause has been removed. Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-components (SWE Common 2.0 clause A.8).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-record-components (SWE Common 2.0 clause A.9).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-block-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-block-components (SWE Common 2.0 clause A.11).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-encodings (SWE Common 2.0 clause A.12).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/general-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/general-encoding-rules (SWE Common 2.0 clause A.14).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/text-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/text-encoding-rules (SWE Common 2.0 clause A.15).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xml-encoding-rules has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xml-encoding-rules (SWE Common 2.0 clause A.16).

Note: the canonical schema location for OGC SWE Common 2.0 (http://www.opengis.net/swe/2.0) is http://schemas.opengis.net/sweCommon/2.0/swe.xsd.

Note: dependency http://www.opengis.net/spec/gmlcov/1.0/req/gml-coverage has associated conformance class (GMLCOV 1.0 clause A.1)

Note: dependency http://www.opengis.net/spec/gmlcov/1.0/req/gml has associated conformance class (GMLCOV 1.0 clause A.2)

Note: the canonical schema location for OGC GMLCOV 1.0 (http://www.opengis.net/gmlcov/1.0) is http://schemas.opengis.net/gmlcov/1.0/gmlcovAll.xsd.

202.6	This clause

has been removed.

202.7 	Requirements Class: Volcano

202.7.1	This requirements class is used to describe the representation of a volcano. The class is targeted at providing a basic description of the volcano as a meteorological phenomenon.

Note: representations providing more detailed information may be used if required.

202.7.2	XML elements describing volcanoes shall conform to all Requirements specified in Table 7.

Table 7. Requirements class xsd-volcano

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-volcano |  |
| Target Type | Data instance |
| Name | Volcano |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-volcano/valid The content model of this element shall have a value that matches the content model of metce:Volcano. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-volcano/name The XML element metce:name shall provide an authoritative name for the given volcano as a literal character string. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-volcano/position The XML element metce:position shall contain a valid child element gml:Point that provides the reference location of the volcano in question. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-volcano/name-as-block-caps The authoritative name for the given volcano should be expressed in block capitals. |

Note: the Global Volcanism Program provides an online, searchable catalogue of volcanoes which may assist in identifying the authoritative name for a given volcano. The catalogue is accessed at the URL http://www.volcano.si.edu/world/. No guarantee is made regarding the availability of this catalogue service.

202.8 Requirements Class: Erupting volcano

202.8.1	This requirements class is used to describe the representation of a currently erupting, or recently erupted, volcano, that is the source of volcanic ash or other significant meteorological phenomena described in weather reports.

Note: representations providing more detailed information may be used if required.

202.8.2	XML elements describing volcanoes where the date of a particular eruption is deemed important shall conform to all Requirements specified in Table 8.

202.8.2	XML elements describing volcanoes where the date of a particular eruption is deemed important shall conform to all Requirements of all relevant dependencies specified in Table 8.

Table 8. Requirements class xsd-erupting-volcano

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-erupting-volcano |  |
| Target Type | Data instance |
| Name | Erupting volcano |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-volcano |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-erupting-volcano/valid The content model of this element shall have a value that matches the content model of metce:EruptingVolcano. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-erupting-volcano/eruption-date The XML element metce:eruptionDate shall provide the date at which the current or recent eruption began expressed in ISO 8601 date-time format. |

202.9 Requirements Class: Tropical cyclone

202.9.1	This requirements class is used to describe the representation of a tropical cyclone.

Note: in this release of METCE-XML, the information expressed about a tropical cyclone is limited to the cyclone’s name. Representations providing more detailed information may be used if required.

202.9.2	XML elements describing tropical cyclones shall conform to all Requirements specified in Table 9.

Table 9. Requirements class xsd-tropical-cyclone

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone |  |
| Target Type | Data instance |
| Name | Tropical cyclone |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone/valid The content model of this element shall have a value that matches the content model of metce:TropicalCyclone. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone/name The XML element metce:name shall provide an authoritative name for the given tropical cyclone as a literal character string. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone/name-as-block-caps The authoritative name for the given tropical cyclone should be expressed in block capitals. |

202.10 Requirements Class: Process

202.10.1	This requirements class is used to describe the procedures involved in generating an observation or measurement.

Note: Process provides a concrete implementation of the abstract OM_Process class.

Note: an instance of Process is often an instrument or sensor (perhaps even a sensor in a given calibrated state), but it may be a human observer executing a set of repeatable instructions, a simulator or a process algorithm.

Note: Process is intended to allow the provision of reference(s) to supporting documentation (e.g. online documentation describing the procedure in detail) plus the resolution (e.g. the smallest quantity being measured that causes a perceptible change in the corresponding indication) and measuring interval (e.g. the range of values for a given quantity kind that an instrument or sensor can detect under the defined conditions) for each physical quantity kind observed.

Note: Process is targeted at providing a basic process description; representations providing more detailed information may be used.

202.10.2	An instance of Process should provide sufficient information for one to interpret the result of an observation.

Note: the recalibration of a sensor such as an anemometer, or modifying its height above local ground, is likely to affect the values that that sensor records; a new instance of Process may be required to express such changes enabling accurate interpretation of the observation result.

202.10.3	XML elements describing procedures relating to observations or measurements shall conform to all Requirements specified in Table 10.

Table 10. Requirements class xsd-process

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-process |  |
| Target Type | Data instance |
| Name | Process |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-process/valid The content model of this element shall have a value that matches the content model of metce:Process. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-process/description A description of the procedure, or citation to some well known description of the procedure, should be provided using the gml:description element. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-process/documentation-reference Where more information about the procedure is accessible online, a reference to that information should be provided using the xlink:href attribute of the metce:documentationRef element to indicate the URL of the online documentation. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-process/configuration Where more information about the procedure configuration is available, such as details of a sensor’s calibration or deployment environment, that information should be included in the procedure description. For each configuration item, an XML element metce:parameter shall be provided, each with a child element om:NamedValue. XML element //metce:parameter/om:NamedValue/om:name shall indicate the meaning of the parameter. The parameter name should be taken from a well-governed source. Furthermore, to avoid ambiguity, there should be no more than one parameter with the same name within a given procedure description. XML element //metce:parameter/om:NamedValue/om:value provides the value of the parameter using any suitable concrete type. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-process/measurement-context Where additional information about the quantity kind(s) observed or measured by the procedure, such as qualification or constraint of the quantity kind, or details of the resolution and/or range with which the procedure is able to measure a given quantity kind is available, that information should be included in the procedure description. For each quantity kind that additional information is to be provided for, an XML element metce:context should be provided, each with a child element metce:MeasurementContext (or any element of a substitution group of metce:MeasurementContext). |

Note: where a procedure is common for many observations, the metce:Process describing that procedure may be published online at an accessible location and referenced from each observation using xlink:href to indicate the URL.

202.11 Requirements Class: Measurement context

202.11.1	This requirements class is used to describe the additional context that may be provided for a quantity kind measured by a given procedure.

Note: the measurement context allows the resolution scale (e.g. the smallest change in quantity being measure that causes a perceptible change in the corresponding indication) and/or the measuring interval (e.g. the range of values that can be measured) to be defined for a given quantity kind for the associated procedure. For example, it is possible to state that a given procedure, say a thermometer, is able to measure air temperature to a resolution of 0.5 degree Celsius in the range -30 degree Celsius to +50 degree Celsius.

202.11.2	XML elements describing procedures relating to observations or measurements shall conform to all Requirements specified in Table 11.

Table 11. Requirements class xsd-measurement-context

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-measurement-context |  |
| Target Type | Data instance |
| Name | Measurement context |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-observable-property, Part D, 203.3 |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-measurement-context/valid The content model of this element shall have a value that matches the content model of metce:MeasurementContext. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-measurement-context/measurand The quantity kind to which this element applies shall be specified via the XML element metce:measurand. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-measurement-context/unit-of-measure-consistent The unit of measurement specified in XML element metce:unitOfMeasure shall be consistent with the unit of measurement used to express resolution scale and/or measuring interval. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-measurement-context/unit-of-measure-provision Where either XML elements metce:resolutionScale or metce:measuringInterval or both are present, XML element metce:unitOfMeasure shall be provided. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-measurement-context/measuring-interval-range-bounds-order Where the measuring interval is specified, the lower limit of the interval, expressed via XML element //metce:measuringInterval/metce:RangeBounds/metce:rangeStart, shall be less than the upper limit of the interval, expressed via XML element //metce:measuringInterval/metce:RangeBounds/metce:rangeEnd. |

Note: the XML element metce:measurand may reference a quantity kind provided by some authority using xlink:href to indicate the URI of the quantity kind, or provide a child element opm:ObservableProperty (or element within the substitution group of opm:ObservableProperty). The latter case may be useful where additional qualification or constraint relating to the quantity kind needs to be embedded within the GML document as XML element //om:OM_Observation/om:observedProperty only permits the observed quantity kind to be expressed by reference using xlink:href.

Note: units of measurement are specified in accordance with Part D, 1.9.

202.12	Requirements Class: Generic observation data

202.12.1	This requirements class mandates use of the XML element om:OM_Observation for encoding observation data.

Note: The term ‘observation’ evokes a particular concept to meteorologists: the measurement of physical phenomena using a human observer, instrument or sensor. However, OM_Observation, as defined in ISO 19156:2011, provides a generalised pattern for describing an event that estimates the value(s) of a property for a subject using a specified process. These semantics are equally applicable to describe forecasts and other numerical simulations.

202.12.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Observation or where element om:type is not specified shall conform to all Requirements specified in Table 4.

202.12.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Observation or where element om:type is not specified shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_Observation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-observation |  |
| Target Type | Data instance |
| Name | Observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

202.13	Requirements Class: Measurement data

202.13.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a scaled number. This ‘result’ is represented in XML as a measure from the Geography Markup Language (GML).

Note: OM_Measurement, as defined in ISO 19156:2011, is intended for use where the observation event is concerned with measurement of a single property at a specified location and time instant or duration. The 'result' of this class of observations is a single scaled value with associated unit of measurement.

Note: a description of the XML implementation of measures (gml:MeasureType) is provided in ISO 19136:2007 Clause 16.3.

202.13.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Measurement shall conform to all Requirements specified in Table 4.

202.13.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_Measurement shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_Measurement is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-measurement

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-measurement |  |
| Target Type | Data instance |
| Name | Measurement |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/measurement, OMXML clause 7.4 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/measurement has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/measurement (OMXML clause A.2).

202.14	Requirements Class: Category observation data

202.14.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a term from a controlled vocabulary or ontology.

Note: a description of the XML implementation of terms from a controlled vocabulary (gml:ReferenceType) is provided in ISO 19136:2007 Clause 7.2.3.

202.14.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CategoryObservation shall conform to all Requirements specified in Table 4.

202.14.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CategoryObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_CategoryObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-category-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-category-observation |  |
| Target Type | Data instance |
| Name | Category observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/categoryObservation, OMXML clause 7.5 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/categoryObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/categoryObservation (OMXML clause A.3).

202.15	Requirements Class: Count observation data

202.15.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is an integer.

Note: the XML implementation of integer values is provided in XML Schema Part 2: Datatypes, Clause 3.3.13.

202.15.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CountObservation shall conform to all Requirements specified in Table 4.

202.15.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_CountObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_CountObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-count-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-count-observation |  |
| Target Type | Data instance |
| Name | Count observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/countObservation, OMXML clause 7.6 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/countObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/countObservation (OMXML clause A.4).

202.16	Requirements Class: Truth observation data

202.16.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a Boolean that can be used to express ‘true’ or ‘false’.

Note: the XML implementation of Boolean values is provided in XML Schema Part 2: Datatypes, Clause 3.2.2.

202.16.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TruthObservation shall conform to all Requirements specified in Table 4.

202.16.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TruthObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_TruthObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-truth-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-truth-observation |  |
| Target Type | Data instance |
| Name | Truth observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/truthObservation, OMXML clause 7.7 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/truthObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/truthObservation (OMXML clause A.5).

202.17	Requirements Class: Geometry observation data

202.17.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a geometry object.

Note: the XML implementation of geometry objects (geometric primitives) is provided in ISO 19136:2007 Clause 10.

202.17.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_GeometryObservation shall conform to all Requirements specified in Table 4.

202.17.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_GeometryObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_GeometryObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-geometry-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-geometry-observation |  |
| Target Type | Data instance |
| Name | Geometry observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/geometryObservation, OMXML clause 7.8 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/geometryObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/geometryObservation (OMXML clause A.6).

202.18	Requirements Class: Temporal observation data

202.18.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a temporal object.

Note: the XML implementation of temporal objects is provided in ISO 19136:2007 Clause 14.

202.18.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TemporalObservation shall conform to all Requirements specified in Table 4.

202.18.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_TemporalObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_TemporalObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 4. Requirements class xsd-temporal-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-temporal-observation |  |
| Target Type | Data instance |
| Name | Temporal observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/temporalObservation, OMXML clause 7.9 |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/temporalObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/temporalObservation (OMXML clause A.7).

202.19	Requirements Class: Complex observation data

202.19.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a set of values relating to a specified location and time instant or duration. This ‘result’ is represented in XML as a simple data record or vector from the SWE Common Data Model.

Note: OM_ComplexObservation, as defined in ISO 19156:2011, is intended for use where the observation event is concerned with the evaluation of multiple properties at a specified location and time instant or duration. The 'result' of this class of observations is a group of values, provided as a Record (as defined in ISO 19103:2005 Geographic information – Conceptual schema language).

Note: a description of data records, vectors and their XML implementation is provided in SWE Common 2.0 Clauses 7.3 and 8.2.

202.19.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_ComplexObservation shall conform to all Requirements specified in Table 4.

202.19.3	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_ComplexObservation shall conform to all Requirements of all relevant dependencies specified in Table 4.

Note: XML implementation of OM_ComplexObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

SWE Common 2.0 [OGC/IS 08-094r1 SWE Common Data Model Encoding Standard 2.0]

Table 4. Requirements class xsd-complex-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-complex-observation |  |
| Target Type | Data instance |
| Name | Complex observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/complexObservation, OMXML clause 7.10 |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-complex-observation/xmlns-declaration-swe The OGC SWE Common 2.0 namespace http://www.opengis.net/swe/2.0 shall be declared within the XML document. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-complex-observation/xmlns-prefix-swe The default namespace prefix used for http://www.opengis.net/swe/2.0 should be “swe”. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/complexObservation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/complexObservation (OMXML clause A.8).

Note: the canonical schema location for OGC SWE Common 2.0 (http://www.opengis.net/swe/2.0) is http://schemas.opengis.net/sweCommon/2.0/swe.xsd.

202.20	Requirements Class: Discrete coverage observation data

202.20.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation is a set of values describing the variation of properties with space and/or time.

Note: OM_DiscreteCoverageObservation, as defined in ISO 19156:2011, is intended for use where the observation event is concerned with the evaluation of properties that vary with respect to space and/or time. The 'result' of this class of observations is a discrete coverage (as defined in ISO 19123:2005 Geographic information – Schema for coverage and geometry functions).

202.20.2	Instances of om:OM_Observation with element om:type specifying http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_DiscreteCoverageObservation shall conform to all Requirements specified in Table 5.

202.20.3	Instances of om:OM_Observation with element om:type specifying  http://www.opengis.net/def/observationType/OGC-OM/2.0/OM_DiscreteCoverageObservation shall conform to all Requirements of all relevant dependencies specified in Table 5.

Note: XML implementation of OM_DiscreteCoverageObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

SWE Common 2.0 [OGC/IS 08-094r1 SWE Common Data Model Encoding Standard 2.0]

GMLCOV 1.0 [OGC/SAP 09-146r2 GML Application Schema – Coverages 1.0.1]

Table 5. Requirements class xsd-discrete-coverage-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation |  |
| Target Type | Data instance |
| Name | Discrete coverage observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components, SWE Common 2.0 clause 8.1 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components, SWE Common 2.0 clause 8.2 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-block-components, SWE Common 2.0 clause 8.4 |
| Dependency | http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings, SWE Common 2.0 clause 8.5 |
| Dependency | http://www.opengis.net/spec/gmlcov/1.0/req/gml-coverage, GMLCOV 1.0 clause 6 |
| Dependency | http://www.opengis.net/spec/gmlcov/1.0/req/gml, GMLCOV 1.0 clause 7 |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation/xmlns-declaration-swe The OGC SWE Common 2.0 namespace http://www.opengis.net/swe/2.0 shall be declared within the XML document. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation/xmlns-declaration-gmlcov The OGC GMLCOV 1.0 namespace http://www.opengis.net/gmlcov/1.0 shall be declared within the XML document. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/result-discrete-or-grid-coverage The XML element om:result shall contain a child element gml:DiscreteCoverage (or any element of a substitution group of gml:DiscreteCoverage), gml:GridCoverage, gml:RectifiedGridCoverage or gml:ReferenceableGridCoverage. |
| Requirement | http://def.wmo.int/metce/2013/req/xsd-sampling-coverage-measurement/result-coverage-gml-encoding The child element of om:result shall be represented in GML as defined in GMLCOV 1.0 clause 7. Multipart representation and special format representation shall not be used. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation/xmlns-prefix-swe The default namespace prefix used for http://www.opengis.net/swe/2.0 should be “swe”. |
| Recommendation | http://def.wmo.int/metce/2013/req/xsd-discrete-coverage-observation/xmlns-prefix-gmlcov The default namespace prefix used for http://www.opengis.net/gmlcov/1.0 should be “gmlcov”. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-components (SWE Common 2.0 clause A.8).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-record-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-record-components (SWE Common 2.0 clause A.9).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-block-components has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-block-components (SWE Common 2.0 clause A.11).

Note: dependency http://www.opengis.net/spec/SWE/2.0/req/xsd-simple-encodings has associated conformance class http://www.opengis.net/spec/SWE/2.0/conf/xsd-simple-encodings (SWE Common 2.0 clause A.12).

Note: the canonical schema location for OGC SWE Common 2.0 (http://www.opengis.net/swe/2.0) is http://schemas.opengis.net/sweCommon/2.0/swe.xsd.

Note: dependency http://www.opengis.net/spec/gmlcov/1.0/req/gml-coverage has associated conformance class (GMLCOV 1.0 clause A.1)

Note: dependency http://www.opengis.net/spec/gmlcov/1.0/req/gml has associated conformance class (GMLCOV 1.0 clause A.2)

Note: the canonical schema location for OGC GMLCOV 1.0 (http://www.opengis.net/gmlcov/1.0) is http://schemas.opengis.net/gmlcov/1.0/gmlcovAll.xsd.

FM-203: OPM

FM 203-15 Ext	OPM-XML		Observable property Model

203.1	Scope

OPM-XML should be used to represent complex observable properties (also known as “quantity kinds”) where individual observable properties are aggregated into groups or where the qualification and/or constraint applied to an observable property needs to be explicitly described.

Note: an “observable property” is a physical property that can be observed; typically this will be a quantitative property, such as dew point temperature.

Note: OPM, the Observable Property Model, was developed to a draft level by the Open Geospatial Consortium Sensor Web Enablement Working Group and was re-used in the INSPIRE Generic Conceptual Model. It is published by WMO to ensure that there is a stable definition of its XML schemas.

Note: the Observable Property Model application schema is described in the Guidelines for data modelling for WMO codes (available in English only  from http://wis.wmo.int/metce-uml).

The Requirements Classes defined in OPM-XML are listed in Table 1.

Table 1. Requirements Classes defined in OPM-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-observable-property, Part D, 203.3 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-composite-observable-property, Part D, 203.4 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property, Part D, 203.5 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier, Part D, 203.6 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-constraint, Part D, 203.7 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-category-constraint, Part D, 203.8 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-scalar-constraint, Part D, 203.9 |
| Requirements Class | http://def.wmo.int/opm/2013/req/xsd-range-constraint, Part D, 203.10 |

203.2	XML Schema for OPM-XML

203.2.1	Representations of information in OPM-XML shall declare the XML namespaces listed in Table 2 and Table 3.

Note: additional namespace declarations may be required depending on the XML elements used within OPM-XML.

Note: the XML Schema is packaged in two XML schema documents (XSD) describing one XML namespace: http://def.wmo.int/opm/2013.

Note: Schematron schemas providing additional constraints are embedded within the XSD defining OPM-XML.

Table 2. XML namespaces defined for OPM-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://def.wmo.int/opm/2013 | opm | http://schemas.wmo.int/opm/1.1/opm.xsd |

Table 3. External XML namespaces used in OPM-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XSLT v2 | http://www.w3.org/1999/XSL/Transform | xsl |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink |  |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |

203.3	Requirements Class: Observable property

203.3.1	This requirements class is used to describe the representation of an observable property.

Note: representations providing more detailed information – such as composite observable property and qualified observable property (see 203.4 and 203.5 below) – may be used if required.

203.3.2	XML elements describing observable properties shall conform to all Requirements specified in Table 4.

Table 4. Requirements class xsd-observable-property

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-observable-property |  |
| Target Type | Data instance |
| Name | Observable property |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-observable-property/valid The content model of this element shall have a value that matches the content model of opm:AbstractObservableProperty. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-observable-property/label The primary human-readable label for the observable property should be specified using the opm:label XML element. |

Note: alternative human-readable labels may be specified using one or more instances of the XML element opm:altLabel.

Note: the XML element opm:notation may be used to specify a notation or code-value that is used to identify the observable property within a given context (e.g. providing a local identifier).

203.4	Requirements Class: Composite observable property

203.4.1	This requirements class is used to describe the representation of an aggregate set of observable properties.

203.4.2	XML elements describing composite observable properties shall conform to all Requirements specified in Table 5.

203.4.3	XML elements describing composite observable properties shall conform to all Requirements of all relevant dependencies specified in Table 5.

Table 5. Requirements class xsd-composite-observable-property

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-composite-observable-property |  |
| Target Type | Data instance |
| Name | Composite observable property |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-observable-property, Part D, 203.3 |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-composite-observable-property/valid The content model of this element shall have a value that matches the content model of opm:CompositeObservableProperty. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-composite-observable-property/set A composite observable property shall contain a minimum of 2 child observable properties. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-composite-observable-property/child-property For each child observable property within the composite observable property, the XML element //opm:CompositeObservableProperty/opm:property shall either contain a valid child element in the substitution group of opm:AbstractObservableProperty or provide a reference to the definition of the child observable property using the xlink:href attribute to indicate the URL where a description is located. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-composite-observable-property/count The XML attribute //opm:CompositeObservableProperty/@count shall specify the number of child observable properties from which the composite observable property is comprised. |

Note: a child observable property specified within a composite observable property instance may itself be a composite observable property, thus allowing arbitrarily complex nesting of sets of observable properties.

203.5	Requirements Class: Qualified observable property

203.5.1	This requirements class is used to describe the representation of an observable property subject to additional qualification or constraint.

Note: the observable property to which the additional qualification or constraint is applied is known as the base property.

203.5.2	XML elements describing qualified observable properties shall conform to all Requirements specified in Table 6.

203.5.3	XML elements describing qualified observable properties shall conform to all Requirements of all relevant dependencies specified in Table 6.

Table 6. Requirements class xsd-qualified-observable-property

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property |  |
| Target Type | Data instance |
| Name | Qualified observable property |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-observable-property, Part D, 203.3 |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/valid The content model of this element shall have a value that matches the content model of opm:QualifiedObservableProperty. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/base-property The XML element //opm:QualifiedObservableProperty/opm:baseProperty shall either contain a valid child element opm:ObservableProperty (or element within the substitution group of opm:ObservableProperty) that describes the base property or provide a reference to the definition of the base property using the xlink:href attribute to indicate the URL where a description is located. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/specified-unit-of-measure If the base property is qualified such that values of the qualified observable property are always provided using a given unit of measurement, the XML attribute //opm:QualifiedObservableProperty/opm:unitOfMeasure/@uom shall be used to specify that unit of measurement. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/valid-unit-of-measure If specified, the unit of measurement referenced via XML attribute //opm:QualifiedObservableProperty/opm:unitOfMeasure/@uom shall be appropriate for the base property. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/qualifier If specified, the XML element //opm:QualifiedObservableProperty/opm:qualifier shall contain a valid child element opm:StatisticalQualifier that provides details of any statistical qualification applied to the base property. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/constraint If specified, the XML element //opm:QualifiedObservableProperty/opm:constraint shall contain a valid child element opm:Constraint, or element in the substitution group of opm:Constraint, that provides details of any constraint applied to the base property. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-qualified-observable-property/minimal-qualification At least one of the XML elements //opm:QualifiedObservableProperty/opm:unitOfMeasure, //opm:QualifiedObservableProperty/opm:qualifier or //opm:QualifiedObservableProperty/opm:constraint should be included within a qualified observable property. |

Note: units of measurement are specified in accordance with Part D, 1.9.

203.6	Requirements Class: Statistical qualifier

203.6.1	This requirements class is used to describe the representation of statistical qualifiers applied to an observable property.

Note: typically statistical qualification is based on some geometric or temporal aggregation using a given statistical function; for example, the maximum temperature in a 24-hour duration.

203.6.2	XML elements describing statistical qualification of observable properties shall conform to all Requirements specified in Table 7.

Table 7. Requirements class xsd-statistical-qualifier

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier |  |
| Target Type | Data instance |
| Name | Statistical qualifier |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/valid The content model of this element shall have a value that matches the content model of opm:StatisticalQualifier. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/statistical-function-code The XML element //opm:StatisticalQualifier/opm:statisticalFunction shall reference the function used in the statistical qualification using the xlink:href attribute to specify the URI used to identify the target statistical function. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/single-qualification-domain One, and only one, of XML elements //opmStatisticalQualifier/opm:aggregationArea, //opmStatisticalQualifier/opm:aggregationLength, //opmStatisticalQualifier/opm:aggregationTimePeriod, //opmStatisticalQualifier/opm:aggregationVolume and //opmStatisticalQualifier/opm:otherAggregation shall be included within a statistical qualification. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/description A textual description of the statistical qualification applied to the observable property should be provided using the //opmStatisticalQualifier/opm:description XML element. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/statistical-function-code-online-definition The URI used to identify the statistical function should have an available online definition and have been recognised by some level of authority. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-statistical-qualifier/qualification-domain-type Where the statistical qualification domain relates to geometric area, geometric length, time period or geometric volume then XML elements //opmStatisticalQualifier/opm:aggregationArea, //opmStatisticalQualifier/opm:aggregationLength, //opmStatisticalQualifier/opm:aggregationTimePeriod or //opmStatisticalQualifier/opm:aggregationVolume should be used in preference to XML element //opmStatisticalQualifier/opm:otherAggregation to describe the statistical qualification domain. |

Note: groups of statistical qualifiers may be applied to a given base property; in such a case the order of the statistical qualification is important. For example, mean daily maximum temperature over a month period comprises two statistical operations with respect to the base property “air temperature” – a maximum over a 24-hour duration followed by a mean over a 1-month duration. A collection of statistical qualifiers can be linked using the XML element //opmStatisticalQualifier/opm:derivedFrom to establish an ordered set.

Note: terms from Part B, FM 92 GRIB Code table 4.10, Type of statistical processing, may be use used to describe the statistical function. An alternative source of statistical function codes are provided in Part B, FM 94 BUFR Code table 0 08 023, First-order statistics. For convenience, these code tables have been published online at http://codes.wmo.int/grib2/codeflag/4.10 and http://codes.wmo.int/bufr4/codeflag/0-08-023 respectively.

203.7	Requirements Class: Constraint

203.7.1	This requirements class is used to describe the representation of constraints applied to an observable property.

Note: the observable property that is used to constrain the base property is known as the constraining property. For example, the observed property “radiance” may be constrained such that one is concerned only with the radiance between wavelengths 50nm to 100nm – in which case the constraining property is “wavelength”.

Note: representations providing more detail – such as scalar constraint, range constraint or category constraint – may be used if required.

203.7.2	XML elements describing the constraint of observable properties shall conform to all Requirements specified in Table 8.

Table 8. Requirements class xsd-constraint

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-constraint |  |
| Target Type | Data instance |
| Name | Constraint |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-constraint/valid The content model of this element shall have a value that matches the content model of opm:Constraint. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-constraint/constraint-property The XML element opm:constraintProperty shall either contain a valid child element opm:ObservableProperty (or element within the substitution group of opm:ObservableProperty) that describes the constraining property or provide a reference to the definition of the constraining property using the xlink:href attribute to indicate the URL where a description is located. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-constraint/description A textual description of the constraint applied to the observable property should be provided using the opm:description XML element. |

203.8	Requirements Class: Category constraint

203.8.1	This requirements class is used to describe the representation of category-based constraints applied to an observable property.

Note: for example, where one is interested only in the cloud-base height of convective clouds, the base property is “cloud-base height”, the constraining property is “cloud type” and the values of the category constraint element list the particular cloud types of interest (e.g. cumulonimbus, towering cumulus etc.).

203.8.2	XML elements describing a category-based constraint of observable properties shall conform to all Requirements specified in Table 9.

203.8.3	XML elements describing a category-based constraint of observable properties shall conform to all Requirements of all relevant dependencies specified in Table 9.

Table 9. Requirements class xsd-category-constraint

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-category-constraint |  |
| Target Type | Data instance |
| Name | Category constraint |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-constraint, Part D, 203.7 |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-category-constraint/valid The content model of this element shall have a value that matches the content model of opm:CategoryConstraint. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-category-constraint/category-member One or more instances of the XML element //opm:CategoryConstraint/opm:value shall be used to specify the category members relevant to this constraint. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-category-constraint/category-member-appropriate-to-constraining-property Each of the category members defined using XML element //opm:CategoryConstraint/opm:value shall appropriate to the constraining property. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-category-constraint/category-value-code-space The XML attribute //opm:CategoryConstraint/opm:value/@gml:codeSpace should be provided when specifying each category member. |
| Recommendation | http://def.wmo.int/opm/2013/req/xsd-category-constraint/category-value-online-definition Appending the content of the XML element //opm:CategoryConstraint/opm:value to the content of XML attribute //opm:CategoryConstraint/opm:value/@gml:codeSpace should create a URI that resolves to an online definition that is recognised by some authority. |

203.9	Requirements Class: Scalar constraint

203.9.1	This requirements class is used to describe the representation of scalar constraints applied to an observable property.

Note: for example, the base property “air temperature” may be constrained such that one is concerned only with air temperature at 1.2 metres above the local ground level (e.g. a screen temperature); height above local ground level is the constraining property.

203.9.2	XML elements describing the constraint of observable properties shall conform to all Requirements specified in Table 10.

203.9.3	XML elements describing the constraint of observable properties shall conform to all Requirements of all relevant dependencies specified in Table 10.

Table 10. Requirements class xsd-scalar-constraint

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-scalar-constraint |  |
| Target Type | Data instance |
| Name | Scalar constraint |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-constraint, Part D, 203.7 |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-scalar-constraint/valid The content model of this element shall have a value that matches the content model of opm:ScalarConstraint. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-scalar-constraint/comparison-operator The XML attribute //opm:ScalarConstraint/@comparisonOperator shall specify the mathematical operator relating the scalar constraint to the supplied numeric value. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-scalar-constraint/comparison-operator-enumeration The value of XML attribute //opm:ScalarConstraint/@comparisonOperator shall be one of the enumeration: “ne” (not equal to), “lt” (less than), “le” (less than or equal to), “eq” (equal to), “ge” (greater than or equal to) or “gt” (greater than). |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-scalar-constraint/unit-of-measure Unless the constraining property is dimensionless, a unit of measurement shall be indicated that is appropriate for the constraining property via XML attribute //opm:ScalarConstraint/opm:unitOfMeasure/@uom. |

Note: units of measurement are specified in accordance with Part D, 1.9.

203.10	Requirements Class: Range constraint

203.10.1	This requirements class is used to describe the representation of constraints applied to an observable property according to a range of values.

Note: for example, the base property “radiance” may be constrained such that we are only concerned with radiance between wavelengths of 50nm and 100nm – “wavelength” is the constraining property and is limited to the range 50nm to 100nm.

203.10.2	XML elements describing the constraint of observable properties shall conform to all Requirements specified in Table 11.

203.10.3	XML elements describing the constraint of observable properties shall conform to all Requirements of all relevant dependencies specified in Table 11.

Table 11. Requirements class xsd-range-constraint

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/opm/2013/req/xsd-range-constraint |  |
| Target Type | Data instance |
| Name | Range constraint |
| Dependency | http://def.wmo.int/opm/2013/req/xsd-constraint, Part D, 203.7 |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/valid The content model of this element shall have a value that matches the content model of opm:RangeConstraint. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/unit-of-measure Unless the constraining property is dimensionless, a unit of measurement shall be indicated that is appropriate for the constraining property via XML attribute //opm:ScalarConstraint/opm:unitOfMeasure/@uom. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/value The XML element //opm:RangeConstraint/opm:value shall contain a valid child element opm:RangeBounds wherein the start and end values of the constraining property range are defined. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/valid-range The numeric value of XML element //opm:RangeConstraint/opm:value/opm:RangeBounds/rangeStart shall be less than the numeric value of XML element //opm:RangeConstraint/opm:value/opm:RangeBounds/rangeEnd. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/start-comparison The XML attribute //opm:RangeConstraint/opm:value/opm:RangeBounds/@startComparison shall specify the mathematical operator relating the range constraint to the supplied numeric value at the lower limit of the range. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/end-comparison The XML attribute //opm:RangeConstraint/opm:value/opm:RangeBounds/@endComparison shall specify the mathematical operator relating the range constraint to the supplied numeric value at the upper limit of the range. |
| Requirement | http://def.wmo.int/opm/2013/req/xsd-range-constraint/comparison-operator-enumeration The value of XML attributes //opm:RangeConstraint/opm:value/opm:RangeBounds/@startComparison and //opm:RangeConstraint/opm:value/opm:RangeBounds/@endComparison shall be one of the enumeration: “ne” (not equal to), “lt” (less than), “le” (less than or equal to), “eq” (equal to), “ge” (greater than or equal to) or “gt” (greater than). |

Note: units of measurement are specified in accordance with Part D, 1.9.

### FM-204: SAF

FM 204-15 Ext	SAF-XML		Simple Aeronautical Features

204.1	Scope

SAF-XML shall be used to represent features relating to the provision of meteorological services for aviation, such as aerodromes, runways, air traffic management units and flight information regions.

Note: the entities provided in SAF-XML are intended to describe only the level of detail required for reporting meteorological information for international civil aviation purposes. Representations providing more detailed information may be used if required.

The Requirements Classes defined in SAF-XML are listed in Table 1.

Table 1. Requirements Classes defined in SAF-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-aerodrome, Part D, 204.4 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-runway, Part D, 204.5 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-runway-direction, Part D, 204.6 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units, Part D, 204.7 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-airspace-volume, Part D, 204.8 |
| Requirements Class | http://icao.int/saf/1.1/req/xsd-airspace, Part D, 204.9 |

204.2	XML Schema for SAF-XML

204.2.1	Representations of information in SAF-XML shall declare XML namespaces listed in Table 2 and Table 3.

Note: additional namespace declarations may be required depending on the XML elements used within SAF-XML.

Note: the XML Schema is packaged into four XML schema documents (XSD) describing one XML namespace: http://icao.int/saf/1.0.

Note: Schematron schemas providing additional constraints are embedded within the XSD defining SAF-XML.

Table 2. XML namespaces defined for SAF-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://icao.int/saf/1.0 | saf | http://schemas.wmo.int/saf/1.1/saf.xsd |

Table 3. External XML namespaces used in SAF-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XSLT v2 | http://www.w3.org/1999/XSL/Transform | xsl |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink |  |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |

204.3	Requirements Class: Unique identification

204.3.1	This requirements class is used to describe how the information representations of aeronautical features are identified.

Note: examples of aeronautical features include aerodromes, air traffic management units and flight information regions.

Note: to achieve consistency with the Aeronautical Information Exchange Model (AIXM 5), the method of identification defined therein is adopted here. More details may be found in the document “AIXM5 Feature Identification and Reference”.

Note: the identifier does not identify the real-world aeronautical feature itself – rather it identifies the information representation about a given aeronautical feature. The originator of information about a given aeronautical feature uniquely identifies the information record they maintain within their data management systems about a given real-world aeronautical feature. The same identifier is then re-used in downstream information systems when referring to that information record. This ensures that all parties can be confident that they are working with the same information about a given aeronautical feature. Thus if multiple systems use the same identifier for an aeronautical feature, this indicates (i) the data is from the same source, or (ii) there are processes in place to ensure the consistency of data between those systems.

204.3.2	XML elements describing aeronautical features shall conform to all Requirements specified in Table 4.

Table 4. Requirements class xsd-unique-identification

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-unique-identification |  |
| Target Type | Data instance |
| Name | Unique identification |
| Requirement | http://icao.int/saf/1.1/req/xsd-unique-identification/uniqueness An identifier scheme shall be used for assigning identity to information records that describe real-world aeronautical features which ensures that there is a reasonable confidence that a given identifier will never be unintentionally used by anyone for anything else. Different versions of the information record describing a given aeronautical feature shall be assigned different identifiers. |
| Requirement | http://icao.int/saf/1.1/req/xsd-unique-identification/gml-identifier The identifier for the information record describing a real-world aeronautical feature shall be specified using the XML element //gml:identifier. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-unique-identification/uuid The identifier scheme that should be used for assigning identity to information records that describe real-world aeronautical features is UUID version 4 based on random number generation. The corresponding value of XML attribute //gml:identifier/@codeSpace associated with the use of UUID is "urn:uuid:". |

Note: UUID generators are widely available; for example, please refer to the International Telecommunication Union’s UUID generator, accessed at the URL http://www.itu.int/ITU-T/asn1/cgi-bin/uuid_generate. No guarantee is made regarding the availability of this UUID generation service.

204.4	Requirements Class: Aerodrome

204.4.1	This requirements class is used to describe the representation of an aerodrome. The class is targeted at providing a basic description of the aerodrome required for reporting meteorological information for international civil aviation purposes.

Note: representations providing more detailed information may be used if required.

Note: an aerodrome is a defined area on land or water (including any buildings, installations and equipment) intended to be used either wholly or in part for the arrival, departure and surface movement of aircraft/helicopters.

204.4.2	XML elements describing aerodromes shall conform to all Requirements specified in Table 5.

204.4.3	XML elements describing aerodromes shall conform to all Requirements of all relevant dependencies specified in Table 5.

Table 5. Requirements class xsd-aerodrome

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-aerodrome |  |
| Target Type | Data instance |
| Name | Aerodrome |
| Dependency | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Requirement | http://icao.int/saf/1.1/req/xsd-aerodrome/valid The content model of this element shall have a value that matches the content model of saf:Aerodrome. |
| Requirement | http://icao.int/saf/1.1/req/xsd-aerodrome/icao-location-indicator If the aerodrome has a 4-letter ICAO location indicator, this shall be specified using the XML element //saf:Aerodrome/saf:locationIndicatorICAO. |
| Requirement | http://icao.int/saf/1.1/req/xsd-aerodrome/iata-deisgnator If the aerodrome has a 3-letter IATA designator, this shall be specified using the XML element //saf:Aerodrome/saf:designatorIATA. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aerodrome/designator The XML element //saf:Aerodrome/saf:designator should be used to specify the designator code for the aerodrome. If the aerodrome has a 4-letter ICAO location indicator, this should be used as the designator code. If the aerodrome does not have a 4-letter ICAO location indicator but does have a 3-letter IATA code, this should be used as the designator code. Alternatively, an artificially generated code shall be used. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aerodrome/name The XML element //saf:Aerodrome/saf:name should be used to specify the primary official name of the aerodrome as designated by the appropriate authority. The name should be provided in block capitals. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aerodrome/field-elevation The XML element //saf:Aerodrome/saf:fieldElevation should be used to specify the vertical distance above mean sea level of the highest point of the landing area. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aerodrome/field-elevation-unit-of-measure If specified, the vertical distance above mean sea level of the highest point of the landing area (field elevation) should be expressed in metres using the XML attribute //saf:Aerodrome/saf:fieldElevation/@uom with the value “m”. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aerodrome/aerodrome-reference-point The XML element //saf:Aerodrome/saf:ARP should be used to specify location of the aerodrome reference point. Coordinate reference system EPSG 4326 should be used to report the location in latitude and longitude. Coordinate reference system EPSG 4979 should be used to report the location in latitude, longitude and altitude. |

Note: ICAO designators are listed in ICAO document 7910.

Note: units of measurement are specified in accordance with Part D, 1.9.

204.5	Requirements Class: Runway

204.5.1	This requirements class is used to describe the representation of a runway. The class is targeted at providing a basic description of the runway required for reporting meteorological information for international civil aviation purposes.

Note: representations providing more detailed information may be used if required.

Note: a runway is a defined rectangular area on a land aerodrome prepared for the landing and take-off of aircraft. This includes the concept of Final Approach and Take-Off Area (FATO) for helicopters.

204.5.2	XML elements describing runways shall conform to all Requirements specified in Table 6.

204.5.32	XML elements describing runway shall conform to all Requirements of all relevant dependencies specified in Table 6.

Table 6. Requirements class xsd-runway

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-runway |  |
| Target Type | Data instance |
| Name | Runway |
| Dependency | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Dependency | http://icao.int/saf/1.1/req/xsd-aerodrome, Part D, 204.4 |
| Requirement | http://icao.int/saf/1.1/req/xsd-runway/valid The content model of this element shall have a value that matches the content model of saf:Runway. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway/associated-aerodrome The XML element //saf:Runway/saf:associatedAirportHeliport should be used to indicate the aerodrome that the runway is situated at using a value that matches the content model of saf:Aerodrome. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway/designator Where an aerodrome has more than one runway, the XML element //saf:Runway/saf:designator should be used to specify the unique identifier for the runway within the aerodrome. |

204.6	Requirements Class: Runway direction

204.6.1	This requirements class is used to describe the representation of one of the two landing and take-off directions of a runway.

Note: representations providing more detailed information may be used if required.

204.6.2	XML elements describing aerodromes shall conform to all Requirements specified in Table 7.

204.6.3	XML elements describing aerodromes shall conform to all Requirements of all relevant dependencies specified in Table 7.

Table 7. Requirements class xsd-runway-direction

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-runway-direction |  |
| Target Type | Data instance |
| Name | Runway direction |
| Dependency | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Dependency | http://icao.int/saf/1.1/req/xsd-runway, Part D, 204.5 |
| Requirement | http://icao.int/saf/1.1/req/xsd-runway-direction/valid The content model of this element shall have a value that matches the content model of saf:RunwayDirection. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/used-runway The XML element //saf:RunwayDirection/saf:usedRunway should be used to indicate the associated runway using a value that matches the content model of saf:Runway. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/designator The textual designator for the landing and take-off direction of the associated runway should be specified using the XML element //saf:RunwayDirection/saf:designator. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/true-bearing The measured angle between true north and the landing and take-off direction should be specified using the XML element //saf:RunwayDirection/saf:trueBearing. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/true-bearing-unit-of-measure The measured angle between true north and the landing and take-off direction should be expressed in degrees using the XML attribute //saf:RunwayDirection/saf:trueBearing/@uom with the value “deg”. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/elevation The vertical distance above mean sea level of the highest point of the runway touchdown zone should be specified using the XML element //saf:RunwayDirection/saf:elevationTDZ. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-runway-direction/elevation-unit-of-measure If specified, the vertical distance above mean sea level of the highest point of the runway touchdown zone should be expressed in metres using the XML attribute //saf:RunwayDirection/saf:elevationTDZ/@uom with the value “m”. |

Note: examples of runway direction designators include “27”, “35L” and “01R”.

Note: the True North is the north point at which the meridian lines meet.

Note: units of measurement are specified in accordance with Part D, 1.9.

204.7	Requirements Class: Aeronautical service provision units

204.7.1	This requirements class is used to describe the representation of aeronautical service provision units.

Note: aeronautical service provision units include Aerodrome Reporting Office (ARO), Air Traffic Control Centre (ATCC), Air Traffic Services Unit (ATSU), Flight Information Centre (FIC) and Meteorological Watch Office (MWO).

Note: representations providing more detailed information may be used if required.

204.7.2	XML elements describing aeronautical service provision units shall conform to all Requirements specified in Table 8.

204.7.3	XML elements describing aeronautical service provision units shall conform to all Requirements of all relevant dependencies specified in Table 8.

Table 8. Requirements class xsd-aeronautical-service-provision-units

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units |  |
| Target Type | Data instance |
| Name | Aeronautical service provision units |
| Dependency | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Requirement | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units/valid The content model of this element shall have a value that matches the content model of saf:Unit. |
| Requirement | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units/unit-type-enumeration If specified, the value of XML element //saf:Unit/saf:type shall be one of the enumeration: “ARO” (Aerodrome Reporting Office), “ATCC” (Air Traffic Control Centre), “ATSU” (Air Traffic Services Unit), “FIC” (Flight Information Centre) or “MWO” (Meteorological Watch Office). |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units/name The XML element //saf:Unit/saf:name should be used to specify the primary official name of the aeronautical service provision unit as designated by the appropriate authority. The name should be provided in block capitals. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units/type The type of the aeronautical service provision unit should be indicated using the XML element //saf:Unit/saf:type. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units/designator The coded designator used to identify the aeronautical service provision unit should be indicated using the XML element //saf:Unit/saf:designator. |

Note: coded designators for aeronautical service provision units are specified in ICAO document 7910.

Note: the location of the aeronautical service provision unit, expressed as a reference point, may be specified using XML element //saf:Unit/saf:position.

204.8	Requirements Class: Airspace volume

204.8.1	This requirements class is used to describe the geometric representation of a three dimensional airspace volume.

Note: representations providing more detailed information may be used if required.

Note: the three dimensional region of space is specified as a two dimensional horizontal region with bounded vertical extent.

204.8.2	XML elements describing airspaces shall conform to all Requirements specified in Table 9.

Table 9. Requirements class xsd-airspace-volume

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-airspace-volume |  |
| Target Type | Data instance |
| Name | Airspace volume |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace-volume/valid The content model of this element shall have a value that matches the content model of saf:AirspaceVolume. |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace-volume/upper-limit If the upper limit of the vertical extent of the airspace is specified (using XML element //saf:AirspaceVolume/saf:upperLimit) then the XML element //saf:AirspaceVolume/saf:upperLimitReference shall be used to specify the associated vertical reference system. |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace-volume/lower-limit If the lower limit of the vertical extent of the airspace is specified (using XML element //saf:AirspaceVolume/saf:lowerLimit) then the XML element //saf:AirspaceVolume/saf:lowerLimitReference shall be used to specify the associated vertical reference system. |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace-volume/limit-type The values of XML elements //saf:AirspaceVolume/saf:upperLimitReference and //saf:AirspaceVolume/saf:lowerLimitReference specifying a vertical reference system shall be one of the enumeration: “SFC” (distance measured from the surface of the earth), “MSL” (distance measured from mean sea level), “W84” (distance measured from the WGS84 ellipsoid) or “STD” (distance measured with an altimeter set to the standard atmosphere). |
| Recommendation | http://icao.int/saf/1.1/req/xsd-airspace-volume/horizontal-projection The XML element //saf:Airspace/saf:horizontalProjection should be used to describe the geometry of the horizontal extent of the airspace volume. |

Note: omission of the upper limit of the vertical extent (the airspace ceiling) indicates that the airspace extends upward to, or beyond, the limit of aeronautical operations, whilst omission of the lower limit of the vertical extent (the airspace floor) indicates that the airspace extends to the land / sea surface.

Note: distance measured from mean sea level is equivalent to “altitude”.

204.9	Requirements Class: Airspace

204.9.1	This requirements class is used to describe the representation of airspaces.

Note: an airspace is a defined three dimensional region of space relevant to air traffic. Airspace types include flight information region (FIR), upper flight information region (UIR) and controlled airspace (CTA).

Note: representations providing more detailed information may be used if required.

204.9.2	XML elements describing airspaces shall conform to all Requirements specified in Table 10.

204.9.3	XML elements describing airspaces shall conform to all Requirements of all relevant Dependencies specified in Table 10.

Table 10. Requirements class xsd-airspace

| Requirements Class |  |
| --- | --- |
| http://icao.int/saf/1.1/req/xsd-airspace |  |
| Target Type | Data instance |
| Name | Airspace |
| Dependency | http://icao.int/saf/1.1/req/xsd-unique-identification, Part D, 204.3 |
| Dependency | http://icao.int/saf/1.1/req/xsd-airspace-volume, Part D, 204.8 |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace/valid The content model of this element shall have a value that matches the content model of saf:Airspace. |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace/icao-designator-indication If the coded designator used to identify the airspace is an ICAO recognised designator then the XML element //saf:Airspace/saf:designatorICAO should have value “true”. |
| Requirement | http://icao.int/saf/1.1/req/xsd-airspace/airspace-type-enumeration If specified, the value of XML element //saf:Airspace/saf:type shall be one of the enumeration: “FIR” (Flight Information Region), “UIR” (Upper Flight Information Region), “FIR_UIR” (Flight Information Region or Upper Flight Information Region) or “CTA” (Controlled Airspace). |
| Recommendation | http://icao.int/saf/1.1/req/xsd-airspace/type The type of the airspace should be indicated using the XML element //saf:Airspace/saf:type. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-airspace/designator The coded designator used to identify the airspace should be indicated using the XML element //saf:Airspace/saf:designator. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-airspace/name The XML element //saf:Airspace/saf:name should be used to specify the official name of the airspace as designated by the appropriate authority. The name should be provided in block capitals. |
| Recommendation | http://icao.int/saf/1.1/req/xsd-airspace/geometry-component The XML element //saf:Airspace/saf:geometryComponent should be used to describe the geometric volume of the airspace using a value that matches the content model of saf:AirspaceVolume. |

Note: ICAO designators are listed in ICAO document 7910.

Note: an airspace may comprise of multiple geometry elements.

## FM-205: IWXXM

FM 205-15 Ext	IWXXM-XML		ICAO Meteorological informtion Exchange Model

205.1	Scope

IWXXM-XML shall be used to represent observations and forecasts, and reports thereof, for international civil aviation as specified by Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation.

IWXXM-XML includes provision for: routine aerodrome meteorological reports (METAR), special aerodrome meteorological reports (SPECI), aerodrome forecasts (TAF) and SIGMET information.

Note: SIGMET information is information issued by a meteorological watch office concerning the occurrence or expected occurrence of specified en-route weather phenomena which may affect the safety of aircraft operations.

The Requirements Classes defined in OPM-XML are listed in Table 1.

Table 1. Requirements Classes defined in IWXXM-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer, Part D, 205.4 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast, Part D, 205.5 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state, Part D, 205.6 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear, Part D, 205.7 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds, Part D, 205.8 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range, Part D, 205.9 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state, Part D, 205.10 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility, Part D, 205.11 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind, Part D, 205.12 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record, Part D, 205.13 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation, Part D, 205.14 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast, Part D, 205.15 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record, Part D, 205.16 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast, Part D, 205.17 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report, Part D, 205.18 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-metar, Part D, 205.19 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-speci, Part D, 205.20 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-forecast, Part D, 205.21 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast, Part D, 205.22 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record, Part D, 205.23 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast, Part D, 205.24 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-taf, Part D, 205.25 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition, Part D, 205.26 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis, Part D, 205.27 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position, Part D, 205.28 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position-collection, Part D, 205.29 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis, Part D, 205.30 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-sigmet, Part D, 205.31 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet, Part D, 205.32 |
| Requirements Class | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet, Part D, 205.33 |

205.2	XML Schema for IWXXM-XML

205.2.1	Representations of information in IWXXM-XML shall declare the XML namespaces listed in Table 2 and Table 3.

Note: additional namespace declarations may be required depending on the XML elements used within IWXXM-XML.

Note: the XML Schema is packaged in five XML schema documents (XSD) describing one XML namespace: http://icao.int/iwxxm/1.0.

Note: Schematron schemas providing additional constraints are embedded within the XSD defining IWXXM-XML.

Table 2. XML namespaces defined for IWXXM-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://icao.int/iwxxm/1.0 | iwxxm | http://schemas.wmo.int/iwxxm/1.1/iwxxm.xsd |

Table 3. External XML namespaces used in IWXXM-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XSLT v2 | http://www.w3.org/1999/XSL/Transform | xsl |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink |  |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |
| ISO/TS 19139:2007 metadata XML implementation | http://www.isotc211.org/2005/gmd | gmd | http://standards.iso.org/ittf/PubliclyAvailableStandards/ISO_19139_Schemas/gmd/gmd.xsd |
| OGC OMXML | http://www.opengis.net/om/2.0 | om | http://schemas.opengis.net/om/2.0/observation.xsd |
| OGC OMXML | http://www.opengis.net/sampling/2.0 | sam | http://schemas.opengis.net/sampling/2.0/samplingFeature.xsd |
| OGC OMXML | http://www.opengis,net/samplingSpatial/2.0 | sams | http://schemas.opengis.net/samplingSpatial/2.0/spatialSamplingFeature.xsd |
| FM 202-15 Ext METCE-XML | http://def.wmo.int/metce/2013 | metce | http://schemas.wmo.int/metce/1.1/metce.xsd |
| FM 203-15 Ext. OPM-XML | http://def.wmo.int/opm/2013 | opm | http://schemas.wmo.int/opm/1.1/opm.xsd |
| FM 204-15 Ext SAF-XML | http://icao.int/saf/1.0 | saf | http://schemas.wmo.int/saf/1.1/saf.xsd |

205.3	Virtual typing

202.3.1	In accordance with OMXML (clause 7.2), the specialization of OM_Observation is provided through schematron restriction. The om:type element shall be used to specify the type of OM_Observation that is being encoded using the URI for the corresponding observation type listed in Part D Code Table D-4.

Note: Part D Code Table D-4 is described in Part D Appendix A.

Note: Part D Code table D-4 is published online at http://codes.wmo.int/49-2/observation-type/IWXXM/1.0.

Note: the URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of MeteorologicalAerodromeForecast is http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast.

Note: Each URI will resolve to provide further information about the associated observation type.

205.4	Requirements Class: Cloud layer

205.4.1	This requirements class is used to describe the representation of a cloud layer. The class is targeted at providing a basic description of the cloud layer as required for civil aviation purposes.

Note: representations providing more detailed information may be used if required.

Note: the requirements for reporting cloud information are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 [C.3.1.] 4.5, Appendix 5 [C.3.1.] 2.2.5 and Appendix 5 [C.3.1.] 1.2.4.

205.4.2	XML elements describing cloud layers shall conform to all Requirements specified in Table 4.

Table 4. Requirements class: xsd-cloud-layer

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-cloud-layer |  |
| Target Type | Data instance |
| Name | Cloud layer |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/valid The content model of this element shall have a value that matches the content model of iwxxm:CloudLayer. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-amount The XML element //iwxxm:CloudLayer/iwxxm:amount shall be used to report an operationally significant amount of cloud. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-amount-code If cloud amount is reported, the value of XML attribute //iwxxm:CloudLayer/iwxxm:amount/@xlink:href shall be the URI of valid term from Part D Code table D-7: Cloud amount reported at aerodrome. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-base The XML element //iwxxm:CloudLayer/iwxxm:base shall either indicate the height of the lowest level in the atmosphere that contains a perceptible quantity of cloud particles or the reason for not reporting the cloud base shall be expressed using the XML attribute //iwxxm:CloudLayer/iwxxm:base/@nilReason to indicate the appropriate nil-reason code. If a nil-reason code is provided, the XML attributes //iwxxm:CloudLayer/iwxxm:base/@xsi:nil and //iwxxm:CloudLayer/iwxxm:base/@uom shall have the values “true” and “N/A” respectively. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-base-unit-of-measure If the cloud base is reported then the vertical distance shall be expressed in metres or feet. The unit of measure shall be indicated using the XML attribute //iwxxm:CloudLayer/iwxxm:base/@uom with values “m” (metres) or “[ft_i]” (feet). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-type-code If cloud type is reported, the value of XML attribute //iwxxm:CloudLayer/iwxxm:cloudType/@xlink:href shall be the URI of valid cloud type from Part D Code table D-8: Significant convective cloud type. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/cloud-type If reporting observed cloud, then the XML element //iwxxm:CloudLayer/iwxxm:cloudType should be used to report the most operationally significant type of cloud in the layer of cloud. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer/nil-significant-cloud If no operationally significant cloud is reported, then the value of XML attribute //iwxxm:CloudLayer/iwxxm:amount/@nilReason should be set to http://codes.wmo.int/common/nil/nothingOfOperationalSignificance. If reporting observed cloud, then the value of XML attribute iwxxm:CloudLayer/iwxxm:cloudType/@nilReason should also be set to http://codes.wmo.int/common/nil/nothingOfOperationalSignificance. |

Note: operationally significant cloud includes cloud below 1500 metres or the highest minimum sector altitude, whichever is greatest, and Cumulonimbus whenever present.

Note: Part D Code table D-1 provides a set of nil reason codes and is published at http://codes.wmo.int/common/nil.

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: Part D Code table D-7 is published online at http://codes.wmo.int/49-2/CloudAmountReportedAtAerodrome.

Note: Part D Code table D-8 is published online at http://codes.wmo.int/49-2/SigConvectiveCloudType.

205.5		Requirements Class: Aerodrome cloud forecast

205.5.1	This requirements class is used to describe forecast cloud conditions at an aerodrome. The class is targeted at providing a basic description of the forecast cloud conditions as required for civil aviation purposes.

Note: representations providing more detailed information may be used if required.

Note: the requirements for reporting forecast cloud conditions are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5 [C.3.1.] 2.2.5 and Appendix 5 [C.3.1.] 1.2.4.

205.5.2	XML elements describing forecast cloud conditions shall conform to all Requirements specified in Table 5.

205.5.3	XML elements describing forecast cloud conditions shall conform to all Requirements of all relevant dependencies specified in Table 5.

Table 5. Requirements class xsd-aerodrome-cloud-forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast |  |
| Target Type | Data instance |
| Name | Aerodrome cloud forecast |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer, Part D, 205.4 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeCloudForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/vertical-visibility When cloud of operational significance is forecast, then the XML element //iwxxm:AerodromeCloudForecast/iwxxm:verticalVisibility shall be used to report the vertical visual range. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/vertical-visibility-unit-of-measure If the vertical visibility is reported then the vertical distance shall be expressed in metres or feet. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeCloudForecast/iwxxm:verticalVisibility/@uom with values “m” (metres) or “[ft_i]” (feet). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/cloud-layers When cloud of operational significance is forecast, then the XML element //iwxxm:AerodromeCloudForecast/iwxxm:layer, containing a valid child element //iwxxm:AerodromeCloudForecast/iwxxm:layer/iwxxm:CloudLayer, shall be used to describe the each cloud layer. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/number-of-cloud-layers No more than four cloud layers shall be reported. If more than four significant cloud layers are forecast, then the four most significant cloud layers with respect to aviation operations shall be prioritized. |

Note: operationally significant cloud includes cloud below 1500 metres or the highest minimum sector altitude, whichever is greatest, and Cumulonimbus whenever present.

Note: vertical visibility is defined as the vertical visual range into an obscuring medium.

Note: units of measurement are specified in accordance with Part D, 1.9.

205.6		Requirements Class: Aerodrome runway state

205.6.1	This requirements class is used to describe the observed runway state.

Note: representations providing more detailed information may be used if required.

Note: the requirements for reporting runway state are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 [C.3.1.] 4.8.1.4.

205.6.2	XML elements describing forecast runway state shall conform to all Requirements specified in Table 6.

205.6.23	XML elements describing forecast runway state shall conform to all Requirements of all relevant dependencies specified in Table 6.

Table 6. Requirements class xsd-aerodrome-runway-state

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state |  |
| Target Type | Data instance |
| Name | Aerodrome runway state |
| Dependency | http://icao.int/saf/1.1/req/xsd-runway-direction, Part D, 204.6 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeRunwayState. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/applicable-runway If XML attribute //iwxxm:AerodromeRunwayState/@allRunways is absent or has value “false”, then XML element //iwxxm:AerodromeRunwayState/iwxxm:runway, with valid child element //iwxxm:AerodromeRunwayState/iwxxm:runway/saf:RunwayDirection, shall be used to indicate the runway direction to which these conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/all-runways If XML attribute //iwxxm:AerodromeRunwayState/@allRunways has value “true”, then XML element //iwxxm:AerodromeRunwayState/iwxxm:runway shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/snow-closure If the aerodrome is closed due to an extreme deposit of snow, XML attribute //iwxxm:AerodromeRunwayState/@snowClosure shall have value “true”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/cleared If the runway has been cleared of meteorological deposits, then XML attribute //iwxxm:AerodromeRunwayState/@cleared shall have value “true” and XML elements //iwxxm:AerodromeRunwayState/iwxxm:depositType, //iwxxm:AerodromeRunwayState/iwxxm:contamination, //iwxxm:AerodromeRunwayState/iwxxm:depthOfDeposit and //iwxxm:AerodromeRunwayState/iwxxm:estimatedSurfaceFriction shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/surface-friction-estimate If reported, the estimated surface friction shall be stated using the XML element //iwxxm:AerodromeRunwayState/iwxxm:estimatedSurfaceFriction and shall have numeric value greater than 0.0 and less than or equal to 0.9. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/surface-friction-estimate-unit-of-measure If reported, the estimated surface friction shall be expressed as a unitless ratio with the value of XML attribute //iwxxm:AerodromeRunwayState/iwxxm:estimatedSurfaceFriction/@uom specified as “http://www.opengis.net/def/uom/OGC/1.0/unity”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/unreliable-surface-friction-estimate If the surface friction estimate for the runway is considered to be unreliable, then XML attribute //iwxxm:AerodromeRunwayState/@estimatedSurfaceFrictionUnreliable shall have value “true”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/unreliable-surface-friction-estimate-true If XML attribute //iwxxm:AerodromeRunwayState/@estimatedSurfaceFrictionUnreliable has value “true”, then XML element //iwxxm:AerodromeRunwayState/iwxxm:estimatedSurfaceFriction shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/deposit-type-code If deposit type is reported, then the value of XML attribute //iwxxm:AerodromeRunwayState/iwxxm:depositType/@xlink:href shall be the URI of valid term from Part B FM 94 BUFR Code table 0 20 086: Runway deposits. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/contamination-code If runway contamination is reported, then the value of XML attribute //iwxxm:AerodromeRunwayState/iwxxm:contamination/@xlink:href shall be the URI of valid term from Part B FM 94 BUFR Code table 0 20 087: Runway contamination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/snow-closure-affects-all-runways If XML attribute //iwxxm:AerodromeRunwayState/@snowClosure has value “true”, then XML //iwxxm:AerodromeRunwayState/@allRunways should also have value “true”; snow closure affects all runways at an aerodrome. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state/deposit-depth-unit-of-measure If reported, the depth of deposit should be expressed in millimetres with the value of XML attribute //iwxxm:AerodromeRunwayState/iwxxm:depthOfDeposit/@uom specified as “mm”. |

Note: For convenience, Part B FM 94 BUFR Code table 0 20 086 is published online at http://codes.wmo.int/bufr4/codeflag/0-20-086.

Note: runway contamination is expressed as a percentage the total runway area that is contaminated according to a predefined set of categories: less than 10%, between 11% and 25%, between 25% and 50% and more than 50%. These categories are listed in Part B FM 94 BUFR Code table 0 20 087: Runway contamination. For convenience, this code table is published on line at http://codes.wmo.int/bufr4/codeflag/0-20-087.

Note: units of measurement are specified in accordance with Part D, 1.9.

205.7		Requirements Class: Aerodrome wind shear

205.7.1	This requirements class is used to describe the aerodrome wind shear. The class is targeted at providing a basic description of the wind shear as required for civil aviation purposes – currently limited to indicating whether a wind shear threshold has been exceeded.

Note: the information on wind shear includes, but is not necessarily limited to, wind shear of a non-transitory nature such as might be associated with low-level temperature inversions or local topography.

Note: representations providing more detailed information may be used if required.

Note: the requirements for reporting aerodrome wind shear are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 [C.3.1.] 4.8.1.3.

205.7.2	XML elements describing forecast runway state shall conform to all Requirements specified in Table 7.

205.7.3	XML elements describing forecast runway state shall conform to all Requirements of all relevant dependencies specified in Table 7.

Table 7. Requirements class xsd-aerodrome-wind-shear

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear |  |
| Target Type | Data instance |
| Name | Aerodrome wind shear |
| Dependency | http://icao.int/saf/1.1/req/xsd-runway-direction, Part D, 204.6 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeWindShear. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear/applicable-runways If XML attribute //iwxxm:AerodromeWindShear/@allRunways is absent or has value “false”, then one or more XML elements //iwxxm:AerodromeWindShear/iwxxm:runway, each with valid child element //iwxxm:AerodromeWindShear/iwxxm:runway/saf:RunwayDirection, shall be used to indicate the set of runway directions to which wind shear conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear/all-runways If XML attribute //iwxxm:AerodromeWindShear/@allRunways has value “true”, then XML element //iwxxm:AerodromeWindShear/iwxxm:runway shall be absent. |

205.8		Requirements Class: Aerodrome observed clouds

205.8.1	This requirements class is used to describe observed cloud conditions at an aerodrome. The class is targeted at providing a basic description of the observed cloud conditions as required for civil aviation purposes.

Note: representations providing more detailed information may be used if required.

Note: the requirements for reporting observed cloud conditions are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.5.

205.8.2	XML elements describing observed cloud conditions shall conform to all Requirements specified in Table 8.

205.8.2	XML elements describing observed cloud conditions shall conform to all Requirements of all relevant dependencies specified in Table 8.

Table 8. Requirements class xsd-aerodrome-observed-clouds

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds |  |
| Target Type | Data instance |
| Name | Aerodrome observed clouds |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-cloud-layer, Part D, 205.4 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeObservedClouds. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/amount-and-height-not-detectable-by-auto-system When an automatic observing system observes cumulonimbus clouds or towering cumulus clouds but the amount and height cannot be observed, the XML attribute //iwxxm:AerodromeObservedClouds/@amountAndHeightUnobservableByAutoSystem shall have value set to “true” and XML element //iwxxm:AerodromeObservedClouds/iwxxm:layer shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/either-vertical-visibility-or-cloud-layers When vertical visibility is reported, cloud layers shall not be reported. When cloud layers are reported, vertical visibility shall not be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/vertical-visibility When cloud of operational significance is observed but the amount and height cannot be observed, then the XML element //iwxxm:AerodromeObservedClouds/iwxxm:verticalVisibility shall be used to report the vertical visibility. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/vertical-visibility-unit-of-measure If the vertical visibility is reported then the vertical distance shall be expressed in metres or feet. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeObservedClouds/iwxxm:verticalVisibility/@uom with values “m” (metres) or “[ft_i]” (feet). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds/cloud-layers When the amount and height of operationally significant cloud is observed, then the XML element //iwxxm:AerodromeObservedClouds/iwxxm:layer, containing a valid child element //iwxxm:AerodromeObservedClouds/iwxxm:layer/iwxxm:CloudLayer, shall be used to describe the each cloud layer. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast/number-of-cloud-layers No more than four cloud layers shall be reported. If more than four significant cloud layers are observed, then the four most significant cloud layers with respect to aviation operations shall be prioritized. |

Note: operationally significant cloud includes cloud below 1500 metres or the highest minimum sector altitude, whichever is greatest, and Cumulonimbus whenever present.

Note: vertical visibility is defined as the vertical visual range into an obscuring medium.

Note: units of measurement are specified in accordance with Part D, 1.9.

205.9		Requirements Class: Aerodrome runway visual range

205.9.1	This requirements class is used to describe runway visual range for a specific runway direction at an aerodrome.

Note: the requirements for reporting runway visual range are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.3.

205.9.2	XML elements describing runway visual range shall conform to all Requirements specified in Table 9.

205.9.3	XML elements describing runway visual range shall conform to all Requirements of all relevant dependencies specified in Table 9.

Table 9. Requirements class xsd-aerodrome-runway-visual-range

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range |  |
| Target Type | Data instance |
| Name | Aerodrome runway visual range |
| Dependency | http://icao.int/saf/1.1/req/xsd-runway-direction, Part D, 204.6 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/valid The content model of this element shall have a value that matches the content model of iwxxm:RunwayVisualRange. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/applicable-runway The XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:runway, with valid child element //iwxxm:AerodromeRunwayState/iwxxm:runway/saf:RunwayDirection, shall be used to indicate the runway direction to which these visual range conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/mean-rvr The XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVR shall be used to express the 10-minute average for observed runway visual range or, if a marked discontinuity in visual range occurs during the 10-minute period, the average runway visual range following that marked discontinuity. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/mean-rvr-unit-of-measure The mean runway visual range shall be reported in metres. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVR/@uom with value “m”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/mean-rvr-exceeds-2000m If the mean runway visual range exceeds 2000 metres, then the numeric value of XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVR shall be set to 2000 and the XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVROperator shall have value “ABOVE”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/mean-rvr-comparison-operator If present, the value XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVROperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/upward-or-downward-visual-range-tendency If the runway visual range values observed in the 10-minute period have shown a distinct tendency, such that the mean during the first 5-minutes varies by 100 metres or more when compared to the second 5-minutes, this should be indicated using the XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:pastTendency with the value “UPWARD” (visual range is increasing) or “DOWNWARD” (visual range is decreasing) as appropriate. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range/no-change-in-visual-range-tendency If the runway visual range values observed in the 10-minute period have not shown a distinct tendency, this should be indicated using the XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:pastTendency with the value “NO_CHANGE”. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: the absence of XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:meanRVROperator indicates that the mean runway visual range has the numeric value reported.

Note: the absence of XML element //iwxxm:AerodromeRunwayVisualRange/iwxxm:pastTendency indicates that no distinct tendency in visual range has been observed.

205.10	Requirements Class: Aerodrome sea state

205.10.1	This requirements class is used to describe aggregated set of sea state conditions reported at an aerodrome.

Note: the requirements for reporting the sea state are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.8.1.4.

205.10.2	XML elements describing sea state shall conform to all Requirements specified in Table 10.

Table 10. Requirements class xsd-aerodrome-sea-state

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state |  |
| Target Type | Data instance |
| Name | Aerodrome sea state |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeSeaState. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/sea-surface-temperature The sea surface temperature shall be reported in Celsius (°C) using the XML element //iwxxm:AerdromeSeaState/iwxxm:seaSurfaceTemperature. The value of the associated XML attribute //iwxxm:AerdromeSeaState/iwxxm:seaSurfaceTemperature/@uom shall be “Cel”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/either-significant-wave-height-or-sea-state When significant wave height is reported, sea state shall not be reported. When sea state is reported, significant wave height shall not be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/significant-wave-height If reported, the observed significant wave height shall be expressed using the XML element //iwxxm:AerodromeSeaState/iwxxm:significantWaveHeight. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/sea-state-code If sea state is reported, then the value of XML attribute //iwxxm:AerodromeSeaState/iwxxm:seaState/@xlink:href shall be the URI of valid term from Part B FM 94 BUFR Code table 0 22 061: State of the sea. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state/significant-wave-height-unit-of-measure The significant wave height should be reported in metres. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeSeaState/iwxxm:significantWaveHeight/@uom with value “m”. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: the term sea surface temperature is generally meant to be representative of the upper few metres of the ocean as opposed to the skin temperature.

Note: For convenience, Part B FM 94 BUFR Code table 0 22 061 is published online at http://codes.wmo.int/bufr4/codeflag/0-22-061.

205.11	Requirements Class: Aerodrome horizontal visibility

205.11.1	This requirements class is used to describe the horizontal visibility conditions observed at an aerodrome.

Note: the requirements for reporting the horizontal visibility are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.2.

205.11.2	XML elements describing horizontal visibility shall conform to all Requirements specified in Table 11.

Table 11. Requirements class xsd-aerodrome-horizontal-visibility

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility |  |
| Target Type | Data instance |
| Name | Aerodrome horizontal visibility |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeHorizontalVisibility. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/prevailing-visibility The prevailing visibility shall be stated using the XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibility with the unit of measure metres, indicated using the XML attribute //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibility/@uom with value “m”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/prevailing-visibility-exceeds-10000m If the prevailing visibility exceeds 10000 metres, then the numeric value of XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibility shall be set to 10000 and the XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/prevailing-visibility-comparison-operator If present, the value XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/minimum-visibility If reported, the minimum visibility shall be expressed using XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:minimumVisibility with the unit of measure metres, indicated using the XML attribute //iwxxm:AerodromeHorizontalVisibility/iwxxm:minimumVisibility/@uom with value “m”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility/minimum-visibility-direction If reported, the observed angle between true north and the direction of minimum visibility shall be expressed in degrees using XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:minimumVisibilityDirection, with the unit of measure indicated using the XML attribute //iwxxm:AerodromeHorizontalVisibility/iwxxm:minimumVisibilityDirection/@uom having value “deg”. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1000 candelas can be seen and identified against an unlit background.

Note: prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

Note: the absence of XML element //iwxxm:AerodromeHorizontalVisibility/iwxxm:prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

Note: the conditions for reporting minimum visibility are that the visibility is not the same in different directions and (i) when the lowest visibility is different from the prevailing visibility and less than 1500 metres or less than 50% of the prevailing visibility and less than 5000 metres, or (ii) when the visibility is fluctuating rapidly and the prevailing visibility cannot be determined.

Note: when reporting minimum visibility, the general direction of the minimum visibility in relation to the aerodrome should be reported unless the visibility is fluctuating rapidly.

Note: the True North is the north point at which the meridian lines meet.

205.12	Requirements Class: Aerodrome surface wind

205.12.1	This requirements class is used to describe the surface wind conditions observed at an aerodrome.

Note: the requirements for reporting the surface wind conditions are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.1.

205.12.2	XML elements describing surface wind conditions shall conform to all Requirements specified in Table 12.

Table 12. Requirements class xsd-aerodrome-surface-wind

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind |  |
| Target Type | Data instance |
| Name | Aerodrome surface wind |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeSurfaceWind. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/mean-wind-speed The mean wind speed shall be stated using the XML element //iwxxm:AerodromeSurfaceWind/iwxxm:meanWindSpeed with the unit of measure metres per second, knots or kilometres per hour. The unit of measure is indicated using the XML attribute //iwxxm:AerodromeSurfaceWind/iwxxm:meanWindSpeed/@uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/variable-wind-direction If the wind direction is variable, then the XML attribute //iwxxm:AerodromeSurfaceWind/@variableDirection shall have the value “true” and XML element //iwxxm:AerodromeSurfaceWind/iwxxm:meanWindDirection shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/steady-wind-direction If the wind direction is not variable, then - the observed angle between true north and the mean direction from which the wind is blowing shall be expressed using XML element //iwxxm:AerodromeSurfaceWind/iwxxm:meanWindDirection, with the unit of measure indicated using the XML attribute //iwxxm:AerodromeSurfaceWind/iwxxm:meanWindDirection/@uom having value “deg”; - the XML attribute //iwxxm:AerodromeSurfaceWind/@variableDirection shall be absent or have the value “false”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/extreme-wind-direction If the extremes of wind direction variability are reported, then - the observed angle between true north and extreme clockwise direction from which the wind is blowing shall be expressed using XML element //iwxxm:AerodromeSurfaceWind/iwxxm:extremeClockWiseWindDirection; - the observed angle between true north and extreme counter-clockwise direction from which the wind is blowing shall be expressed using XML element //iwxxm:AerodromeSurfaceWind/iwxxm:extremeCounterClockWiseWindDirection; - the unit of measure for each extreme wind direction shall indicated using the XML attribute @uom with value “deg”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind/gust-speed If reported, the observed gust speed should be stated using the XML element //iwxxm:AerodromeSurfaceWind/iwxxm:windGustSpeed and expressed in metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeSurfaceWind/iwxxm:windGustSpeed/@uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |

Note: the mean wind speed is the average wind speed observed over the previous ten minutes.

Note: the gust speed is the maximum wind speed observed over the previous ten minutes.

Note: wind direction is reported as variable (VRB) if, during the ten minute observation of mean wind speed, the variation of wind direction is (i) 180 degrees or more, or (ii) 60 degrees or more when the wind speed is less than 1.5 metres per second (3 knots).

Note: extreme directional variations of wind are reported if, during the ten minute observation of mean wind speed, the variation of wind direction is 60 degrees or more and less than 180 degrees and the wind speed is 1.5 metres per second (3 knots) or more.

Note: the absence of XML attribute //iwxxm:AerodromeSurfaceWind/@variableDirection implies a “false” value; e.g. the wind direction is not variable.

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: the True North is the north point at which the meridian lines meet.

205.13	Requirements Class: Meteorological aerodrome observation record

205.13.1	This requirements class is used to describe the aggregated set of meteorological conditions observed at an aerodrome.

205.13.2	XML elements describing the set of meteorological conditions observed at an aerodrome shall conform to all Requirements specified in Table 13.

205.13.3	XML elements describing the set of meteorological conditions observed at an aerodrome shall conform to all Requirements of all relevant dependencies specified in Table 13.

Table 13. Requirements class xsd-meteorological-aerodrome-observation-record

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome observation record |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-state, Part D, 205.6 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-wind-shear, Part D, 205.7 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-observed-clouds, Part D, 205.8 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-runway-visual-range, Part D, 205.9 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-sea-state, Part D, 205.10 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-horizontal-visibility, Part D, 205.11 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind, Part D, 205.12 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalAerodromeObservationRecord. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/cavok If the conditions associated with CAVOK are observed, then - the XML attribute //iwxxm:MeteorologicalAerodromeObservationRecord/@cloudAndVisibilityOK shall have value “true”; and - the following XML elements shall be absent: //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:visibility, //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:rvr, //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:presentWeather and //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:cloud. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/air-temperature The air temperature observed at the aerodrome shall be reported in Celsius (°C) using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:airTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/dew-point-temperature The dew-point temperature observed at the aerodrome shall be reported in Celsius (°C) using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:dewpointTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/qnh The atmospheric pressure, QNH, observed at the aerodrome shall be reported in hectopascals (hPa) using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:qnh. The value of the associated XML attribute @uom shall be “hPa”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/present-weather If present weather is reported, the value of XML attribute //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:presentWeather/@xlink:href shall be the URI of a valid weather phenomenon code from Part D Code table D-6: Aerodrome present or forecast weather. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/number-of-present-weather-codes No more than three present weather codes shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/recent-weather If recent weather is reported, the value of XML attribute //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:recentWeather/@xlink:href shall be the URI of a valid weather phenomenon code from Part D Code table D-5: Aerodrome recent weather. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/number-of-recent-weather-codes No more than three recent weather codes shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/surface-wind Surface wind conditions observed at the aerodrome shall be reported using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:surfaceWind containing a valid child element iwxxm:AerodromeSurfaceWind. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/runway-state If reported, the surface conditions for a given runway direction shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:runwayState containing a valid child element iwxxm:AerodromeRunwayState. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/wind-shear If reported, the wind shear conditions for the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:windShear containing a valid child element iwxxm:AerodromeWindShear. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/cloud If reported, the cloud conditions observed at the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:cloud containing a valid child element iwxxm:AerodromeObservedClouds. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/runway-visual-range If reported, the visual range conditions for a given runway direction shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:rvr containing a valid child element iwxxm:AerodromeRunwayVisualRange. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/number-of-rvr-groups Visual range conditions shall be reported for no more than four runway directions. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/sea-state If reported, the sea state conditions observed at the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:seaState containing a valid child element iwxxm:AerodromeSeaState. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/visibility If reported, the horizontal visibility conditions observed at the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:visibility containing a valid child element iwxxm:AerodromeHorizontalVisibility. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record/present-weather-not-observable If present weather is not observable due to sensor failure or obstruction the value of XML attribute //iwxxm:MeteorologicalAerodromeObservationRecord/iwxxm:presentWeather/@nilReason should indicate the URI “http://codes.wmo.int/common/nil/notObservable” |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1500 metres or the minimum sector altitude, whichever is greatest, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as “CAVOK”. Use of CAVOK is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 2.2.

Note: the requirements for reporting air temperature and dew-point temperature are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.6.

Note: the requirements for reporting atmospheric pressure (QNH) are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.7.

Note: the requirements for reporting present weather are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.4.

Note: Part D Code table D-6 is published online at http://codes.wmo.int/49-2/AerodromePresentOrForecastWeather.

Note: the requirements for reporting recent weather are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.8.1.1.

Note: Part D Code table D-5 is published online at http://codes.wmo.int/49-2/AerodromeRecentWeather.

Note: the requirements for reporting the surface wind conditions are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.1.

Note: the requirements for reporting runway state are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 [C.3.1.] 4.8.1.4.

Note: the requirements for reporting aerodrome wind shear are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 [C.3.1.] 4.8.1.3.

Note: the requirements for reporting observed cloud conditions are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.5.

Note: information on runway visual range shall be omitted if the prevailing visibility exceeds 1500 metres. Details of the requirements for reporting runway visual range are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.3.

Note: the requirements for reporting the sea state are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.8.1.4.

Note: the requirements for reporting the horizontal visibility are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.2.

205.14	Requirements Class: Meteorological aerodrome observation

205.14.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation describes the aggregated set of meteorological conditions observed at an aerodrome, the ‘feature of interest’ is a representative point location within the aerodrome at which the meteorological conditions were observed and the ‘procedure’ provides the set of information as specified by WMO.

Note: MeteorologicalAerodromeObservation is a subclass of OM_ComplexObservation defined within ISO 19156:2011.

205.14.2	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeObservation shall conform to all Requirements in Table 14.

205.14.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeObservation shall conform to all Requirements of all relevant dependencies in Table 14.

205.14.4 This clause has been removed.

Note: XML implementation of iwxxm:MeteorologicalAerodromeObservation is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 14. Requirements class xsd-meteorological-aerodrome-observation

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome observation |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Dependency | http://icao.int/saf/1.1/req/xsd-aerodrome, Part D, 204.4 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-record, Part D, 205.13 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/feature-of-interest The XML element //om:OM_Observation/om:featureOfInterest shall contain a valid child element sams:SF_SpatialSamplingFeature that describes the reference point to which the observed meteorological conditions apply. The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:type shall have value “http://www.opengis.net/def/samplingFeatureType/OGC-OM/2.0/SF_SamplingPoint”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/sampled-feature The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_ SpatialSamplingFeature/sam:sampledFeature shall contain a valid child element saf:Aerodrome that describes the aerodrome to which the observed meteorological conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/result If reported, the XML element //om:OM_Observation/om:result shall contain a valid child element iwxxm:MeteorologicalAerodromeObservationRecord that describes the aggregated set of meteorological conditions observed at the target aerodrome. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/phenomenon-time The XML element //om:OM_Observation/om:phenomenonTime shall contain a valid child element gml:TimeInstant that describes the time at which the observation occurred. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/result-time The XML element //om:OM_Observation/om:resultTime shall contain a valid child element gml:TimeInstant that describes the time at which the observation was made available for dissemination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/observed-property The XML attribute //om:OM_Observation/om:observedProperty/@xlink:href should have value “http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeObservation”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation/procedure The value of XML element //om:OM_Observation/om:procedure/metce:Process/gml:description should be used to cite the Technical Regulations relating to meteorological aerodrome observations. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: URI http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeObservation refers to an XML document that defines the aggregate set of observable properties relevant to a meteorological aerodrome observation.

Note: the Technical Regulations relating to meteorological observations may be cited as: “Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3: Technical specifications related to meteorological observations and reports”.

Note: the time at which the observation is made available for dissemination may be a few minutes after the observation occurred.

Note: in the case of NIL report (e.g. to indicate that an anticipated meteorological aerodrome observation report is considered to be “MISSING”), no meteorological conditions are provided. In these cases, the XML element //om:OM_Observation/om:result has no child elements and the XML attribute //om:OM_Observation/om:result/@nilReason is used to indicate why the “result” is absent.

205.15	Requirements Class: Aerodrome surface wind trend forecast

205.15.1	This requirements class is used to describe the surface wind conditions forecast at an aerodrome as appropriate for inclusion in a trend forecast of a routine or special meteorological aerodrome report.

Note: the requirements for reporting the surface wind conditions within a trend forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.2.2.

205.15.2	XML elements describing surface wind conditions shall conform to all Requirements specified in Table 16.

Table 16. Requirements class xsd-aerodrome-surface-wind-trend-forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast |  |
| Target Type | Data instance |
| Name | Aerodrome surface wind trend forecast |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeSurfaceWindTrendForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast/mean-wind-speed The forecast mean wind speed shall be stated using the XML element //iwxxm:AerodromeSurfaceWindTrendForecast/iwxxm:meanWindSpeed with the unit of measure metres per second, knots or kilometres per hour. The unit of measure is indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour) |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast/wind-direction If the forecast mean wind direction is reported, then angle between true north and the mean direction from which the wind is forecast to be blowing shall be expressed using XML element //iwxxm:AerodromeSurfaceWindTrendForecast/iwxxm:meanWindDirection, with the unit of measure indicated using the XML attribute @uom having value “deg”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast/gust-speed If reported, the forecast gust speed should be stated using the XML element //iwxxm:AerodromeSurfaceWindTrendForecast/iwxxm:windGustSpeed and expressed in metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute //iwxxm:AerodromeSurfaceWind/iwxxm:windGustSpeed/@uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: the True North is the north point at which the meridian lines meet.

205.16	Requirements Class: Meteorological aerodrome trend forecast record

205.16.1	This requirements class is used to describe the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in a trend forecast of a routine or special meteorological aerodrome report.

205.16.2	XML elements describing the set of meteorological conditions for inclusion in a trend forecast shall conform to all Requirements specified in Table 17.

205.16.3	XML elements describing the set of meteorological conditions for inclusion in a trend forecast shall conform to all Requirements of all relevant dependencies specified in Table 17.

Table 17. Requirements class xsd-meteorological-aerodrome-trend-forecast-record

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome trend forecast record |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast, Part D, 205.5 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast, Part D, 205.15 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalAerodromeTrendForecastRecord. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/change-indicator-nosig If no operationally significant changes to the meteorological conditions are forecast for the aerodrome, then the XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/@changeIndicator shall have the value “NO_SIGNIFICANT_CHANGES”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/change-indicator-becmg If the meteorological conditions forecast for the aerodrome are expected to reach or pass through specified values at a regular or irregular rate, then the XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/@changeIndicator shall have the value “BECOMING”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/change-indicator-tempo If temporary fluctuations in the meteorological conditions forecast for the aerodrome are expected occur, then the XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/@changeIndicator shall have the value “TEMPORARY_FLUCTUATIONS”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/cavok If the conditions associated with CAVOK are forecast, then - the XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/@cloudAndVisibilityOK shall have value “true”; and - the following XML elements shall be absent: //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibility, //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibilityOperator, //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:forecastWeather and //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:cloud. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/prevailing-visiblity If reported, the prevailing visibility shall be stated using the XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibility with the unit of measure metres, indicated using the XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibility/@uom with value “m”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/prevailing-visibility-exceeds-10000m If the prevailing visibility exceeds 10000 metres, then the numeric value of XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibility shall be set to 10000 and the XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/prevailing-visibility-comparison-operator If present, the value XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/forecast-weather If forecast weather is reported, the value of XML attribute //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:forecastWeather/@xlink:href shall be the URI of a valid weather phenomenon code from Part D Code table D-6: Aerodrome present or forecast weather. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/number-of-forecast-weather-codes No more than three forecast weather codes shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/surface-wind Surface wind conditions forecast for the aerodrome shall be reported using the XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:surfaceWind containing a valid child element iwxxm:AerodromeSurfaceWindTrendForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record/cloud If reported, the cloud conditions forecast for the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:cloud containing a valid child element iwxxm:AerodromeCloudForecast. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: temporary fluctuations in the meteorological conditions occur when those condition reach or pass specified values and last for a period of time less than one hour in each instance and, in the aggregate, cover less than one-half the period during which the fluctuations are forecast to occur (Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.3.3).

Note: the use of change indicators is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.3 and Appendix 3, [C.3.1.] Table A3-3

Note: cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1500 metres or the minimum sector altitude, whichever is greatest, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as “CAVOK”. Use of CAVOK is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 2.2.

Note: visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1000 candelas can be seen and identified against an unlit background.

Note: prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

Note: the requirements for reporting prevailing visibility conditions within a trend forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.2.3.

Note: the absence of XML element //iwxxm:MeteorologicalAerodromeTrendForecastRecord/iwxxm:prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

Note: the requirements for reporting forecast weather within a trend forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.2.4.

Note: Part D Code table D-6 is published online at http://codes.wmo.int/49-2/AerodromePresentOrForecastWeather.

Note: the requirements for reporting the surface wind conditions within a trend forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.2.2.

Note: the requirements for reporting cloud conditions within a trend forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.2.5.

205.17	Requirements Class: Meteorological aerodrome trend forecast

205.17.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation describes the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in a trend forecast, the ‘feature of interest’ is a representative point location within the aerodrome for which the meteorological conditions were forecast and the ‘procedure’ provides the set of information as specified by WMO.

Note: MeteorologicalAerodromeTrendForecast is a subclass of OM_ComplexObservation defined within ISO 19156:2011.

205.17.2	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeTrendForecast shall conform to all Requirements in Table 18.

205.17.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeTrendForecast shall conform to all Requirements of all relevant dependencies in Table 18.

205.17.4 This clause has been removed.

Note: XML implementation of iwxxm:MeteorologicalAerodromeTrendForecast is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 18. Requirements class xsd-meteorological-aerodrome-trend-forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome trend forecast |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Dependency | http://icao.int/saf/1.1/req/xsd-aerodrome, Part D, 204.4 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast-record, Part D, 205.16 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-trend-forecast/feature-of-interest The XML element //om:OM_Observation/om:featureOfInterest shall contain a valid child element sams:SF_SpatialSamplingFeature that describes the reference point to which the forecast meteorological conditions apply. The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:type shall have value “http://www.opengis.net/def/samplingFeatureType/OGC-OM/2.0/SF_SamplingPoint”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/sampled-feature The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_ SpatialSamplingFeature/sam:sampledFeature shall contain a valid child element saf:Aerodrome that describes the aerodrome to which the forecast meteorological conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/result The XML element //om:OM_Observation/om:result shall contain a valid child element iwxxm:MeteorologicalAerodromeTrendForecastRecord that describes the aggregated set of meteorological conditions forecast for the target aerodrome. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/phenomenon-time The XML element //om:OM_Observation/om:phenomenonTime shall contain a valid child element gml:TimeInstant or gml:TimePeriod that describes the time for which the forecast is valid. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/result-time The XML element //om:OM_Observation/om:resultTime shall contain a valid child element gml:TimeInstant that describes the time at which the trend forecast was made available for dissemination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/observed-property The XML attribute //om:OM_Observation/om:observedProperty/@xlink:href should have value “http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeTrendForecast”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast/procedure The value of XML element //om:OM_Observation/om:procedure/metce:Process/gml:description should be used to cite the Technical Regulations relating to trend forecasts for aerodromes. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: URI http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeTrendForecast refers to an XML document that defines the aggregate set of observable properties relevant to a trend forecast.

Note: the Technical Regulations relating to trend forecasts may be cited as: “Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2: Criteria related to trend forecasts”.

Note: a forecast may be provided for a specific time instant or a time period. Requirements regarding the specification of time for which the forecast is valid are specified at Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.3.

205.18	Requirements Class: Meteorological aerodrome observation report

205.18.1	This requirements class is used to describe the report within which meteorological aerodrome observations, and optionally one or more trend forecasts, are provided.

Note: the reporting requirements for routine and special meteorological aerodrome reports are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 and Appendix 5, [C.3.1] 2.

205.18.2	XML elements describing routine or special meteorological aerodrome reports shall conform to all Requirements specified in Table 20.

205.18.3	XML elements describing routine or special meteorological aerodrome reports shall conform to all Requirements of all relevant dependencies specified in Table 20.

Table 20. Requirements class xsd-meteorological-aerodrome-observation-report

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome observation report |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation, Part D, 205.14 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-trend-forecast, Part D, 205.17 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalAerodromeObservationReport. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/status The status of the report shall be indicated using the XML attribute @status with the value being one of the enumeration: “NORMAL”, “MISSING” or “CORRECTION”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/automated-station If the meteorological aerodrome observation included within the report has been generated by an automated system, the value of XML attribute @automatedStation shall be set to “true”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/observation The XML element iwxxm:observation shall contain a valid child element om:OM_Observation of type MeteorologicalAerodromeObservation. The value of the XML attribute iwxxm:observation/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeObservation”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/trend-forecast If trend forecasts are reported, the value of XML element iwxxm:trendForecast shall be a valid child element om:OM_Observation of type MeteorologicalAerodromeTrendForecast. For each trend forecast, the value of the XML attribute iwxxm:trendForecast/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeTrendForecast”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/number-of-trend-forecasts No more than three trend forecasts shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/unique-subject-aerodrome The observation and, if reported, trend forecasts shall refer to the same aerodrome. All values of XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:sampledFeature/saf:Aerodrome/gml:identifier within the meteorological aerodrome observation report shall be identical. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/nil-report If XML attribute has value @status has value “MISSING”, then a NIL report shall be provided: - XML element iwxxm:observation/om:OM_Observation/om:result shall have no child elements and XML attribute iwxxm:observation/om:OM_Observation/om:result/@nilReason shall provide an appropriate nil-reason. - XML attribute @automatedStation shall be absent; and - XML element iwxxm:trendForecast shall be absent. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/nosig If no change of operational significance is forecast, then a single XML element iwxxm:trendForecast should be included with no child elements therein and the value XML attribute iwxxm:trendForecast/@nilReason should indicate “inapplicable”. |

Note: a report with status “CORRECTED” indicates that content has been amended to correct an error identified in an earlier report. The XML element //om:OM_Observation/om:resultTime/gml:TimeInstant is used to reflect the dissemination time of the corrected report.

Note: a report with status “MISSING” indicates that a routine report has not been provided on the anticipated timescales. Such a report does not contain details of any observed or forecast meteorological conditions and is often referred to as a “NIL” report.

Note: the requirements relating to reporting the use of an automated system are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.9.

Note: if XML attribute @automatedStation is absent then the value “false” is inferred; e.g. the meteorological aerodrome observation included within the report has not been generated by an automated system.

Note: within an XML encoded meteorological aerodrome report it is likely that only one instance of saf:Aerodrome will physically be present; subsequent assertions about the aerodrome may use xlinks to refer to the previously defined saf:Aerodrome element in order to keep the XML document size small. As such, validation of requirement http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report/unique-subject-aerodrome is applied once any xlinks, if used, have been resolved.

Note: Part D Code table D-1 provides a set of nil reason codes and is published at http://codes.wmo.int/common/nil.

205.19	Requirements Class: METAR

205.19.1	This requirements class is used to describe the routine meteorological aerodrome reports (METAR).

205.19.2	XML elements describing METARs shall conform to all Requirements specified in Table 21.

205.19.2	XML elements describing METARs shall conform to all Requirements of all relevant dependencies specified in Table 21.

Table 21. Requirements class xsd-metar

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-metar |  |
| Target Type | Data instance |
| Name | METAR |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report, Part D, 205.18 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-metar/valid The content model of this element shall have a value that matches the content model of iwxxm:METAR. |

205.20	Requirements Class: SPECI

205.20.1	This requirements class is used to describe the special meteorological aerodrome reports (SPECI).

205.20.2	XML elements describing SPECIs shall conform to all Requirements specified in Table 22.

205.20.3	XML elements describing SPECIs shall conform to all Requirements of all relevant dependencies specified in Table 22.

Table 22. Requirements class xsd-speci

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-speci |  |
| Target Type | Data instance |
| Name | SPECI |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-observation-report, Part D, 205.18 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-speci/valid The content model of this element shall have a value that matches the content model of iwxxm:SPECI. |

205.21	Requirements Class: Aerodrome surface wind forecast

205.21.1	This requirements class is used to describe the surface wind conditions forecast at an aerodrome as appropriate for inclusion in an aerodrome forecast report (TAF).

Note: the requirements for reporting the surface wind conditions within a TAF specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.1.

205.21.2	XML elements describing surface wind conditions shall conform to all Requirements specified in Table 23.

205.21.2	XML elements describing surface wind conditions shall conform to all Requirements of all dependencies specified in Table 23.

Table 23. Requirements class xsd-aerodrome-surface-wind-forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-forecast |  |
| Target Type | Data instance |
| Name | Aerodrome surface wind forecast |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-trend-forecast, Part D, 205.12 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-forecast/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeSurfaceWindForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-forecast/variable-wind-direction If the wind direction is variable, then the XML attribute //iwxxm:AerodromeSurfaceWindForecast/@variableWindDirection shall have the value “true” and XML element //iwxxm:AerodromeSurfaceWindForecast/iwxxm:meanWindDirection shall be absent. |

Note: wind direction is reported as variable (VRB) if is not possible to forecast a prevailing surface wind direction due to expected variability, for example, during light wind conditions (less than 3 knots) or thunderstorms.

205.22	Requirements Class: Aerodrome air temperature forecast

205.22.1	This requirements class is used to describe the temperature conditions forecast at an aerodrome as appropriate for inclusion in an aerodrome forecast report (TAF), including the maximum and minimum temperature values and their time of occurrence.

Note: the requirements for reporting the surface wind conditions within a TAF specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.5.

205.22.2	XML elements describing temperature conditions shall conform to all Requirements specified in Table 24.

Table 24. Requirements class xsd-aerodrome-air-temperature-forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast |  |
| Target Type | Data instance |
| Name | Aerodrome air temperature forecast |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast/valid The content model of this element shall have a value that matches the content model of iwxxm:AerodromeAirTemperatureForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast/maximum-temperature The maximum air temperature anticipated during the forecast period shall be reported in Celsius (°C) using the XML element //iwxxm:AerodromeAirTemperatureForecast/iwxxm:maximumAirTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast/maximum-temperature-time The XML element //iwxxm:AerodromeAirTemperatureForecast/iwxxm:maximumAirTemperatureTime shall have a valid child element gml:TimeInstant that describes the time at which the maximum air temperature is anticipated to occur. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast/minimum-temperature The minimum air temperature anticipated during the forecast period shall be reported in Celsius (°C) using the XML element //iwxxm:AerodromeAirTemperatureForecast/iwxxm:minimumAirTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast/minimum-temperature-time The XML element //iwxxm:AerodromeAirTemperatureForecast/iwxxm:minimumAirTemperatureTime shall have a valid child element gml:TimeInstant that describes the time at which the minimum air temperature is anticipated to occur. |

Note: units of measurement are specified in accordance with Part D, 1.9.

205.23	Requirements Class: Meteorological aerodrome forecast record

205.23.1	This requirements class is used to describe the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in a aerodrome forecast report (TAF).

205.23.2	XML elements describing the set of meteorological conditions for inclusion in an aerodrome forecast shall conform to all Requirements specified in Table 25.

205.23.3	XML elements describing the set of meteorological conditions for inclusion in an aerodrome forecast shall conform to all Requirements of all relevant dependencies specified in Table 25.

Table 25. Requirements class xsd-meterological-aerodrome-forecast-record

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome forecast record |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-cloud-forecast, Part D, 205.5 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-surface-wind-forecast, Part D, 205.21 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-aerodrome-air-temperature-forecast, Part D, 205.22 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalAerodromeForecastRecord. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/prevailing-forecast-conditions The XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall be absent if the forecast describes the prevailing meteorological conditions. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-fm If the meteorological conditions forecast for the aerodrome are expected to change significantly and more or less completely to a different set of conditions, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “FROM”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-becmg If the meteorological conditions forecast for the aerodrome are expected to reach or pass through specified values at a regular or irregular rate, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “BECOMING”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-tempo If temporary fluctuations in the meteorological conditions forecast for the aerodrome are expected occur, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “TEMPORARY_FLUCTUATIONS”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-prob30 If meteorological conditions forecast for the aerodrome have a 30% probability of occurring, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “PROBABILITY_30”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-prob30-tempo If the forecast of temporary fluctuations in meteorological conditions has a 30% probability of occurring, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “PROBABILITY_30_TEMPORARY_FLUCTUATIONS”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-prob40 If meteorological conditions forecast for the aerodrome have a 40% probability of occurring, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “PROBABILITY_40”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/change-indicator-prob40-tempo If the forecast of temporary fluctuations in meteorological conditions has a 40% probability of occurring, then the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall have the value “PROBABILITY_40_TEMPORARY_FLUCTUATIONS”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/cavok If the conditions associated with CAVOK are forecast, then - the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/@cloudAndVisibilityOK shall have value “true”; and - the following XML elements shall be absent: //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibility, //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibilityOperator, //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:weather and //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:cloud. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/prevailing-visiblity If reported, the prevailing visibility shall be stated using the XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibility with the unit of measure metres, indicated using the XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibility/@uom with value “m”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/prevailing-visibility-exceeds-10000m If the prevailing visibility exceeds 10000 metres, then the numeric value of XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibility shall be set to 10000 and the XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/prevailing-visibility-comparison-operator If present, the value XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/temperature If reported, the temperature conditions forecast for the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:temperature containing a valid child element iwxxm:AerodromeAirTemperatureForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/number-of-temperature-groups No more than two sets of temperature conditions shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/cloud If reported, the cloud conditions forecast for the aerodrome shall be expressed using the XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:cloud containing a valid child element iwxxm:AerodromeCloudForecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/forecast-weather If forecast weather is reported, the value of XML attribute //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:forecastWeather/@xlink:href shall be the URI of a valid weather phenomenon code from Part D Code table D-6: Aerodrome present or forecast weather. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/number-of-forecast-weather-codes No more than three forecast weather codes shall be reported. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record/surface-wind Surface wind conditions forecast for the aerodrome shall be reported using the XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:surfaceWind containing a valid child element iwxxm:AerodromeSurfaceWindForecast. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: temporary fluctuations in the meteorological conditions occur when those condition reach or pass specified values and last for a period of time less than one hour in each instance and, in the aggregate, cover less than one-half the period during which the fluctuations are forecast to occur (Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 2.3.3).

Note: the use of change groups and time indicators within a TAF is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.3 and Table A5-2.

Note: the use of probability groups and time indicators within a TAF is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.4 and Table A5-2.

Note: cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1500 metres or the minimum sector altitude, whichever is greatest, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as “CAVOK”. Use of CAVOK is specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 2.2.

Note: visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1000 candelas can be seen and identified against an unlit background.

Note: prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

Note: the requirements for reporting prevailing visibility conditions within an aerodrome forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.2.

Note: the absence of XML element //iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

Note: the requirements for reporting the temperature conditions within an aerodrome forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.5.

Note: the requirements for reporting cloud conditions within an aerodrome forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.4.

Note: the requirements for reporting forecast weather within an aerodrome forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.3.

Note: Part D Code table D-6 is published online at http://codes.wmo.int/49-2/AerodromePresentOrForecastWeather.

Note: the requirements for reporting the surface wind conditions within an aerodrome forecast are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.2.1.

205.24	Requirements Class: Meteorological aerodrome forecast

205.24.1	This requirements class restricts the content model for the XML element om:OM_Observation such that the ‘result’ of the observation describes the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in an aerodrome forecast report (TAF), the ‘feature of interest’ is a representative point location within the aerodrome for which the meteorological conditions were forecast and the ‘procedure’ provides the set of information as specified by WMO.

Note: MeteorologicalAerodromeForecast is a subclass of of OM_ComplexObservation defined within ISO 19156:2011.

205.24.2	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast shall conform to all Requirements in Table 26.

205.24.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast shall conform to all Requirements of all relevant dependencies in Table 26.

205.24.4 This clause has been removed.

Note: XML implementation of iwxxm:MeteorologicalAerodromeForecast is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 26. Requirements class xsd-meteorological forecast

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast |  |
| Target Type | Data instance |
| Name | Meteorological aerodrome forecast |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Dependency | http://icao.int/saf/1.1/req/xsd-aerodrome, Part D, 204.4 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast-record, Part D, 205.23 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-forecast/feature-of-interest The XML element //om:OM_Observation/om:featureOfInterest shall contain a valid child element sams:SF_SpatialSamplingFeature that describes the reference point to which the forecast meteorological conditions apply. The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:type shall have value “http://www.opengis.net/def/samplingFeatureType/OGC-OM/2.0/SF_SamplingPoint”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/sampled-feature The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_ SpatialSamplingFeature/sam:sampledFeature shall contain a valid child element saf:Aerodrome that describes the aerodrome to which the forecast meteorological conditions apply. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/result If reported, the XML element //om:OM_Observation/om:result shall contain a valid child element iwxxm:MeteorologicalAerodromeForecastRecord that describes the aggregated set of meteorological conditions forecast for the target aerodrome. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/phenomenon-time The XML element //om:OM_Observation/om:phenomenonTime shall contain a valid child element gml:TimeInstant or gml:TimePeriod that describes the time for which the forecast is valid. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/result-time The XML element //om:OM_Observation/om:resultTime shall contain a valid child element gml:TimeInstant that describes the time at which the forecast was made available for dissemination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/observed-property The XML attribute //om:OM_Observation/om:observedProperty/@xlink:href should have value “http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeForecast”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast/procedure The value of XML element //om:OM_Observation/om:procedure/metce:Process/gml:description should be used to cite the Technical Regulations relating to meteorological aerodrome forecasts. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: URI http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeForecast refers to an XML document that defines the aggregate set of observable properties relevant to an aerodrome forecast.

Note: the Technical Regulations relating to forecasts may be cited as: “Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1: Criteria related to TAF”.

Note: a forecast may be provided for a specific time instant or a time period. Requirements regarding the specification of time for which the forecast is valid are specified at Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.3.

Note: in the case of NIL report (e.g. to indicate that an anticipated TAF is considered to be “MISSING”), no meteorological conditions are provided. In these cases, the XML element //om:OM_Observation/om:result has no child elements and the XML attribute //om:OM_Observation/om:result/@nilReason is used to indicate why the “result” is absent.

205.25	Requirements Class: TAF

205.25.1	This requirements class is used to describe the aerodrome forecast report within which a base forecast, and optionally one or more change forecasts, is provided.

Note: the reporting requirements for aerodrome forecasts are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3 and Appendix 5, [C.3.1] 1.

205.25.2	XML elements describing aerodrome forecast reports shall conform to all Requirements specified in Table 28.

205.25.3	XML elements describing aerodrome forecast reports shall conform to all Requirements of all relevant dependencies specified in Table 28.

Table 28. Requirements class xsd-taf

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-taf |  |
| Target Type | Data instance |
| Name | TAF |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-aerodrome-forecast, Part D, 205.24 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/valid The content model of this element shall have a value that matches the content model of iwxxm:TAF. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/status The status of the TAF shall be indicated using the XML attribute //iwxxm:TAF/@status with the value being one of the enumeration: “NORMAL”, “AMENDMENT”, “CANCELLATION”, “CORRECTION” or “MISSING”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/issue-time The XML element //iwxxm:TAF/iwxxm:issueTime shall contain a valid child element gml:TimeInstant that describes the time at which the TAF was issued. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/base-forecast If the prevailing forecast conditions for the valid period of the TAF are reported, then - the XML element //iwxxm:TAF/iwxxm:baseForecast shall contain a valid child element om:OM_Observation of type MeteorologicalAerodromeForecast; - the value of the XML attribute //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast”; and - the XML attribute //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:result/iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/change-forecast If change forecasts or alternative forecasts with probability of occurrence are reported, then - the XML element //iwxxm:TAF/iwxxm:changeForecast shall contain a valid child element om:OM_Observation of type MeteorologicalAerodromeForecast; - the value of the XML attribute //iwxxm:TAF/iwxxm:changeForecast/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast”; - XML element //iwxxm:TAF/iwxxm:changeForecast/om:OM_Observation/om:result/iwxxm:MeteorologicalAerodromeForecastRecord/iwxxm:temperature shall be absent; and - the XML attribute //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:result/iwxxm:MeteorologicalAerodromeForecastRecord/@changeIndicator shall be one of the enumeration: “BECOMING”, “TEMPORARY_FLUCTUATIONS”, “FROM”, “PROBABILITY_30”, “PROBABILITY_30_TEMPORARY_FLUCTUATIONS”, “PROBABILITY_40”, or “PROBABILITY_40_TEMPORARY_FLUCTUATIONS”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/unique-subject-aerodrome The base forecast and, if reported, change forecasts shall refer to the same aerodrome. All values of XML element //iwxxm:TAF/*/om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:sampledFeature/saf:Aerodrome/gml:identifier within the TAF shall be identical. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/status-normal If the status of the TAF is “NORMAL” (as specified by XML attribute //iwxxm:TAF/@status), then: - the prevailing meteorological conditions anticipated during the valid period of the TAF shall be reported using the XML element //iwxxm:TAF/iwxxm:baseForecast; - the valid time period of the TAF shall be given using the XML element //iwxxm:TAF/iwxxm:validTime/gml:TimePeriod - the XML element //iwxxm:TAF/iwxxm:previousReportAerodrome shall be absent; and - the XML element //iwxxm:TAF/iwxxm:previousReportValidPeriod shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/status-amendment-or-correction If the status of the TAF is “AMENDMENT” or “CORRECTION” (as specified by XML attribute //iwxxm:TAF/@status), then: - the prevailing meteorological conditions anticipated during the valid period of the TAF shall be reported using the XML element //iwxxm:TAF/iwxxm:baseForecast; - the valid time period of the TAF shall be given using the XML element //iwxxm:TAF/iwxxm:validTime/gml:TimePeriod; and - the valid time period for the TAF that has been amended or corrected shall be reported using the XML element //iwxxm:TAF/iwxxm:previousReportValidPeriod/gml:TimePeriod. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/status-cancellation If the status of the TAF is “CANCELLATION” (as specified by XML attribute //iwxxm:TAF/@status), then: - the XML element //iwxxm:TAF/iwxxm:baseForecast shall be absent; - the XML element //iwxxm:TAF/iwxxm:changeForecast shall be absent; - the time period for which TAF reports at the subject aerodrome are cancelled shall be given using the XML element //iwxxm:TAF/iwxxm:validTime/gml:TimePeriod - the aerodrome for which TAF reports are cancelled shall be reported using the XML element //iwxxm:TAF/iwxxm:previousReportAerodrome/saf:Aerodrome; and - the valid time period for the TAF that has been cancelled shall be reported using the XML element //iwxxm:TAF/iwxxm:previousReportValidPeriod/gml:TimePeriod. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-taf/nil-report-status-missing If the status of the TAF is “MISSING” (as specified by XML attribute //iwxxm:TAF/@status), then: - the XML element //iwxxm:TAF/iwxxm:baseForecast shall be have valid child element om:OM_Observation of type MeteorologicalAerodromeForecast - the value of XML element //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:sampledFeature/saf:Aerodrome shall indicate the aerodrome for which the TAF is missing; - the XML element //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:result shall have no child elements and XML attribute //iwxxm:TAF/iwxxm:baseForecast/om:OM_Observation/om:result/@nilReason shall provide an appropriate nil-reason; - the XML element //iwxxm:TAF/iwxxm:changeForecast shall be absent; - the XML element //iwxxm:TAF/iwxxm:validTime shall be absent; - the XML element //iwxxm:TAF/iwxxm:previousReportAerodrome shall be absent; and - the XML element //iwxxm:TAF/iwxxm:previousReportValidPeriod shall be absent. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-taf/number-of-change-forecasts The number of change forecasts should be kept to a minimum, and no more than five change forecasts should be reported in normal circumstances. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-taf/issue-time-matches-result-time The TAF issue time (specified by XML element //iwxxm:TAF/iwxxm:issueTime/gml:TimeInstant) should match the result time for each of the forecasts provided within the TAF (specified by XML element //iwxxm:TAF/*/om:OM_Observation/om:resultTime/gml:TimeInstant). |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-taf/valid-time-includes-all-phenomenon-times The valid times of all forecasts included in the TAF (specified by XML element //iwxxm:TAF/*/om:OM_Observation/om:phenomenonTime/*) should occur within the valid time period of the TAF (specified by XML element //iwxxm:TAF/iwxxm:validTime/gml:TimePeriod). |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-taf/status-amendment-or-correction-previous-aerodrome If the status of the TAF is “AMENDMENT” or “CORRECTION” (as specified by XML attribute //iwxxm:TAF/@status), then the aerodrome that was the subject of the TAF that has been amended or corrected should be reported using the XML element //iwxxm:TAF/iwxxm:previousReportAerodrome/saf:Aerodrome. |

Note: the requirements relating to the reporting of change forecasts are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.3.

Note: the requirements relating to the reporting of alternative forecasts with probability of occurrence are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.4.

Note: guidance regarding the number of change forecasts and alternative forecasts is given in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 5, [C.3.1.] 1.5.

Note: a report with status “MISSING” indicates that a routine report has not been provided on the anticipated timescales. Such a report does not contain details of any forecast meteorological conditions and is often referred to as a “NIL” report.

Note: within an XML encoded TAF it is likely that only one instance of saf:Aerodrome will physically be present; subsequent assertions about the aerodrome may use xlinks to refer to the previously defined saf:Aerodrome element in order to keep the XML document size small. As such, validation of requirement http://icao.int/iwxxm/1.1/req/xsd-taf/unique-subject-aerodrome is applied once any xlinks, if used, have been resolved.

Note: Part D Code table D-1 provides a set of nil reason codes and is published at http://codes.wmo.int/common/nil.

205.26	Requirements Class: Evolving meteorological condition

205.26.1	This requirements class is used to describe the presence of a specific SIGMET phenomenon such as volcanic ash or thunderstorm, along with expected changes to the intensity of the phenomenon, its speed and direction of motion. The geometric extent of the SIGMET phenomenon is specified as a two dimensional horizontal region with bounded vertical extent.

205.26.2	XML elements describing the characteristics of a SIGMET phenomenon shall conform to all Requirements specified in Table 29.

205.26.3	XML elements describing the characteristics of a SIGMET phenomenon shall conform to all Requirements of all relevant dependencies specified in Table 29.

Table 29. Requirements class xsd-evolving-meteorological-condition

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition |  |
| Target Type | Data instance |
| Name | Evolving meteorological condition |
| Dependency | http://icao.int/saf/1.1/req/xsd-airspace-volume, Part D, 204.8 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/valid The content model of this element shall have a value that matches the content model of iwxxm:EvolvingMeteorologicalCondition. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/intensity-change The anticipated change in intensity of the SIGMET phenomenon shall be indicated using the XML attribute //iwxxm:EvolvingMeteorologicalCondition/@intensityChange with the value being one of the enumeration: “NO_CHANGE”, “WEAKEN” or “INTENSIFY”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/geometry The geometric extent of the SIGMET phenomenon shall be reported using the XML element //iwxxm:EvolvingMeteorologicalCondition/iwxxm:geometry with valid child element saf:AirspaceVolume. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/speed-of-motion The speed of motion of the SIGMET phenomenon shall be reported using the XML element //iwxxm:EvolvingMeteorologicalCondition/iwxxm:speedOfMotion with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute //iwxxm:EvolvingMeteorologicalCondition/iwxxm:speedOfMotion/@uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/direction-of-motion If reported, the angle between true north and the direction of motion of the SIGMET phenomenon shall be given in degrees using the XML element //iwxxm:EvolvingMeteorologicalCondition/iwxxm:directionOfMotion. The unit of measure shall be indicated using the XML attribute //iwxxm:EvolvingMeteorologicalCondition/iwxxm:directionOfMotion/@uom with value “deg”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition/stationary-phenomenon If the SIGMET phenomenon is not moving, (indicated by the XML element //iwxxm:EvolvingMeteorologicalCondition/iwxxm:speedOfMotion having numeric value zero) XML element //iwxxm:EvolvingMeteorologicalCondition/iwxxm:directionOfMotion should be absent. |

Note: units of measurement are specified in accordance with Part D, 1.9.

Note: the True North is the north point at which the meridian lines meet.

205.27	Requirements Class: SIGMET evolving condition analysis

205.27.1	This requirements class is used to describe the details of how the characteristics of a SIGMET phenomenon were evaluated and is based on the observation pattern from ISO 19156:2011, Geographic information – Observations and measurements. This requirements class is applicable to both the observation and forecasting of SIGMET phenomenon characteristics.

205.27.2	This requirements class restricts the content model of om:OM_Observation such that the ‘result’ of the observation describes the characteristics of a SIGMET phenomenon (including geometric extent, expected intensity change, speed and direction of motion), the ‘feature of interest’ is the bounded extent of the airspace for which the SIGMET report is issued and the ‘procedure’ provides the set of information as specified by WMO.

Note: SIGMETEvolvingConditionAnalysis is a subclass of OM_Observation defined within ISO 19156:2011.

205.27.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETEvolvingConditionAnalysis shall conform to all Requirements in Table 30.

205.27.4	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETEvolvingConditionAnalysis shall conform to all Requirements of all relevant dependencies in Table 30.

Note: XML implementation of iwxxm:SIGMETEvolvingConditionAnalysis is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 30. Requirements class xsd-sigmet-evolving-condition-analysis

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis |  |
| Target Type | Data instance |
| Name | SIGMET evolving condition analysis |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Dependency | http://icao.int/saf/1.1/req/xsd-airspace, Part D, 204.9 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-evolving-meteorological-condition, Part D, 205.26 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/feature-of-interest The XML element //om:OM_Observation/om:featureOfInterest shall contain a valid child element sams:SF_SpatialSamplingFeature that describes the horizontal extent of the airspace for which the SIGMET report is issued – a sampling surface. The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:type shall have value “http://www.opengis.net/def/samplingFeatureType/OGC-OM/2.0/SF_SamplingSurface”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/sampled-feature The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_ SpatialSamplingFeature/sam:sampledFeature shall contain a valid child element saf:Airspace that describes the airspace for which the SIGMET report is issued. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/result If reported, the XML element //om:OM_Observation/om:result shall contain a valid child element iwxxm:EvolvingMeteorologicalCondition that describes the characteristics of the SIGMET phenomenon (geometric extent, expected intensity change, speed and direction of motion). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/phenomenon-time The XML element //om:OM_Observation/om:phenomenonTime shall contain a valid child element gml:TimeInstant that describes the time at which the SIGMET phenomenon was observed or the time for which the characteristics of the SIGMET phenomenon have been forecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/result-time The XML element //om:OM_Observation/om:resultTime shall contain a valid child element gml:TimeInstant that describes the time at which the details of the SIGMET phenomenon were made available for dissemination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/observed-property The XML attribute //om:OM_Observation/om:observedProperty/@xlink:href should have a value that is the URI of a valid term from Part D Code table D-9: Significant weather phenomena. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis/procedure The value of XML element //om:OM_Observation/om:procedure/metce:Process/gml:description should be used to cite the Technical Regulations relating to provision of SIGMET reports. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: Part D Code table D-9 is published online at http://codes.wmo.int/49-2/SigWxPhenomena.

Note: the Technical Regulations relating to provision of SIGMET reports may be cited as: “Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1.] 1: Specifications related to SIGMET information”.

Note: in the case of SIGMET cancellation, no characteristics of a SIGMET phenomenon are provided. In these cases, the XML element //om:OM_Observation/om:result has no child elements and the XML attribute //om:OM_Observation/om:result/@nilReason is used to indicate why the “result” is absent.

205.28	Requirements Class: Meteorological position

205.28.1	This requirements class is used to describe the forecast position and extent of a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the end of the valid period of the SIGMET report. The geometric extent of the SIGMET phenomenon is specified as a two dimensional horizontal region with bounded vertical extent.

205.28.2	XML elements describing only the geometry of a SIGMET phenomenon shall conform to all Requirements specified in Table 31.

205.28.3	XML elements describing only the geometry of a SIGMET phenomenon shall conform to all Requirements of all relevant dependencies specified in Table 31.

Table 31. Requirements class xsd-meteorological-position

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-position |  |
| Target Type | Data instance |
| Name | Meteorological position |
| Dependency | http://icao.int/saf/1.1/req/xsd-airspace-volume, Part D, 204.8 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalPosition. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position/geometry The geometric extent of the SIGMET phenomenon shall be reported using the XML element //iwxxm:MeteorologicalPosition/iwxxm:geometry with valid child element saf:AirspaceVolume. |

205.29	Requirements Class: Meteorological position collection

205.28.1	This requirements class is used to describe a collection of geometries for a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the end of the valid period of the SIGMET report.

205.28.2	XML elements describing a collection of geometries for a SIGMET phenomenon shall conform to all Requirements specified in Table 32.

205.28.3	XML elements describing a collection of geometries for a SIGMET phenomenon shall conform to all Requirements of all relevant dependencies specified in Table 32.

Table 32. Requirements class xsd-meteorological-position-collection

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-meteorological-position-collection |  |
| Target Type | Data instance |
| Name | Meteorological position collection |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position, Part D, 205.28 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position/valid The content model of this element shall have a value that matches the content model of iwxxm:MeteorologicalPositionCollection. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position-collection/members If reported, the geometries for a specific SIGMET phenomenon shall be provided using the XML element //iwxxm:MeteorologicalPositionCollection/iwxxm:member with valid child element iwxxm:MeteorologicalPosition. |

205.30	Requirements Class: SIGMET position analysis

205.30.1	This requirements class is used to describe the details of how the forecast position of a SIGMET phenomenon at the end of the valid period of a SIGMET report was evaluated.

205.30.2	This requirements class restricts the content model of om:OM_Observation such that the ‘result’ of the observation describes the collection of forecast positions of a specific SIGMET phenomenon, the ‘feature of interest’ is the bounded extent of the airspace for which the SIGMET report is issued and the ‘procedure’ provides the set of information as specified by WMO.

Note: SIGMETPositionAnalysis is a subclass of OM_Observation defined within ISO 19156:2011.

205.30.3	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETPositionAnalysis shall conform to all Requirements in Table 33.

205.30.4	Instances of om:OM_Observation with element om:type specifying http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETPositionAnalysis shall conform to all Requirements of all relevant dependencies in Table 33.

Note: XML implementation of iwxxm:SIGMETPositionAnalysis is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML [OGC/IS 10-025r1 Observations and Measurements 2.0 – XML Implementation]

Table 33. Requirements class xsd-sigmet-position-analysis

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis |  |
| Target Type | Data instance |
| Name | SIGMET position analysis |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/observation, OMXML clause 7.3 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/sampling, OMXML clause 7.14 |
| Dependency | http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling, OMXML clause 7.15 |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-process, Part D, 202.10 |
| Dependency | http://icao.int/saf/1.1/req/xsd-airspace, Part D, 204.9 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-meteorological-position-collection, Part D, 205.29 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/feature-of-interest The XML element //om:OM_Observation/om:featureOfInterest shall contain a valid child element sams:SF_SpatialSamplingFeature that describes the horizontal extent of the airspace for which the SIGMET report is issued – a sampling surface. The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:type shall have value “http://www.opengis.net/def/samplingFeatureType/OGC-OM/2.0/SF_SamplingSurface”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/sampled-feature The XML element //om:OM_Observation/om:featureOfInterest/sams:SF_ SpatialSamplingFeature/sam:sampledFeature shall contain a valid child element saf:Airspace that describes the airspace for which the SIGMET report is issued. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/process The XML element om:procedure shall contain a child element metce:Process or any element of a substitution group of metce:Process. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/result The XML element //om:OM_Observation/om:result shall contain a valid child element iwxxm:MeteorologicalPositionCollection that describes the collection of forecast positions of a specific SIGMET phenomenon. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/phenomenon-time The XML element //om:OM_Observation/om:phenomenonTime shall contain a valid child element gml:TimeInstant that describes the time for which the collection of positions of the SIGMET phenomenon have been forecast. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/result-time The XML element //om:OM_Observation/om:resultTime shall contain a valid child element gml:TimeInstant that describes the time at which the details of the SIGMET phenomenon positions were made available for dissemination. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/observed-property The XML attribute //om:OM_Observation/om:observedProperty/@xlink:href should have a value that is the URI of a valid term from Part D Code table D-9: Significant weather phenomena. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis/procedure The value of XML element //om:OM_Observation/om:procedure/metce:Process/gml:description should be used to cite the Technical Regulations relating to provision of SIGMET reports. |

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/observation has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/observation (OMXML clause A.1).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/sampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/sampling (OMXML clause A.12).

Note: dependency http://www.opengis.net/spec/OMXML/2.0/req/spatialSampling has associated conformance class http://www.opengis.net/spec/OMXML/2.0/conf/spatialSampling (OMXML clause A.13).

Note: Part D Code table D-9 is published online at http://codes.wmo.int/49-2/SigWxPhenomena.

Note: the Technical Regulations relating to provision of SIGMET reports may be cited as: “Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1.] 1: Specifications related to SIGMET information”.

205.31	Requirements Class: SIGMET

205.31.1	This requirements class is used to describe the SIGMET report within which the characteristics of a specific SIGMET phenomenon are described.

Note: the reporting requirements for SIGMETs are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1] 1.

205.31.2	XML elements describing SIGMET reports shall conform to all Requirements specified in Table 34.

205.31.3	XML elements describing SIGMET reports shall conform to all Requirements of all relevant dependencies specified in Table 34.

Table 34. Requirements class xsd-sigmet

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-sigmet |  |
| Target Type | Data instance |
| Name | SIGMET |
| Dependency | http://icao.int/saf/1.1/req/xsd-aeronautical-service-provision-units, Part D, 204.7 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-sigmet-evolving-condition-analysis, Part D, 205.27 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/valid The content model of this element shall have a value that matches the content model of iwxxm:SIGMET. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/status The status of the SIGMET shall be indicated using the XML attribute @status with the value being one of the enumeration: “NORMAL”, or “CANCELLATION”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/issuing-air-traffic-service-unit The air traffic services unit responsible for the subject airspace shall be indicated using the XML element iwxxm:issuingAirTrafficServicesUnit with a valid child element saf:Unit. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/originating-meteorological-watch-office The meteorological watch office that originated the SIGMET report shall be indicated using the XML element iwxxm:originatingMeteorologicalWatchOffice with a valid child element saf:Unit. The value of XML element iwxxm:issuingAirTrafficServicesUnit/saf:Unit/saf:type shall be “MWO” (Meteorological Watch Office). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/sequence-number The sequence number of this SIGMET report shall be indicated using XML element iwxxm:sequenceNumber. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/valid-period The period of validity of this SIGMET report shall be indicated using XML element iwxxm:validPeriod with valid child element gml:TimePeriod. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/phenomenon The XML attribute iwxxm:phenomenon/@xlink:href shall have a value that is the URI of a valid term from Part D Code table D-9: Significant weather phenomena. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/unique-subject-airspace All SIGMET analyses included in the report shall refer to the same airspace. All values of XML element //om:OM_Observation/om:featureOfInterest/sams:SF_SpatialSamplingFeature/sam:sampledFeature/saf:Airspace/gml:identifier within the SIGMET shall be identical. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/analysis If reported, XML element iwxxm:analysis shall contain a valid child element om:OM_Observation of type SIGMETEvolvingConditionAnalysis. The value of XML attribute iwxxm:analysis/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETEvolvingConditionAnalysis”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/status-normal If the status of the SIGMET is “NORMAL” (as specified by XML attribute @status), then: - the characteristics of the SIGMET phenomenon shall be reported using one or more of the XML element iwxxm:analysis; - each XML element iwxxm:analysis shall contain a valid element iwxxm:analysis/om:OM_Observation/om:result/iwxxm:EvolvingMeteorologicalCondition within which the characteristics of the SIGMET phenomenon are described; - the XML element iwxxm:cancelledSequenceNumber shall be absent; and - the XML element iwxxm:cancelledValidPeriod shall be absent. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-sigmet/status-cancellation If the status of the SIGMET is “CANCELLATION” (as specified by XML attribute @status), then: - the details of the airspace for which the SIGMET has been cancelled shall be provided by a single instance of XML element iwxxm:analysis; - the XML element iwxxm:analysis/om:OM_Observation/om:result shall have no child elements and XML attribute iwxxm:analysis/om:OM_Observation/om:result/@nilReason shall provide an appropriate nil-reason; - the value XML element iwxxm:cancelledSequenceNumber shall indicated the sequence number of the SIGMET that has been cancelled; and - the XML element iwxxm:cancelledValidPeriod shall have a valid child element gml:TimePeriod that indicates the validity period of the SIGMET that has been cancelled. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet/issuing-air-traffic-service-unit-type The value of XML element //iwxxm:SIGMET/iwxxm:issuingAirTrafficServicesUnit/saf:Unit/saf:type should be one of the enumeration: “ATSU” (Air Traffic Services Unit) or “FIC” (Flight Information Centre). |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet/valid-period-start-matches-result-time The start time of the validity period of the SIGMET report (expressed using XML element //iwxxm:validPeriod/gml:TimePeriod/gml:beginPosition) should match the result time of each SIGMET analysis included within the report (expressed using XML element //om:OM_Observation/om:resultTime/gml:TimeInstant/gml:timePosition). |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet/valid-time-includes-all-phenomenon-times The observation and/or forecast times of all SIGMET analyses included in the report (specified by XML element //om:OM_Observation/om:phenomenonTime/*) should occur within the valid time period of the SIGMET (specified by XML element //iwxxm:validPeriod/gml:TimePeriod). |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-sigmet/7-point-definition-of-airspace-volume The horizontal extent of any airspace volumes enclosing a SIGMET phenomenon (reported using XML element //om:OM_Observation/om:result/*/iwxxm:geometry/saf:AirspaceVolume/saf:horizontalProjection) should use no more than seven points to define the bounding polygon. |

Note: requirements relating to sequence numbers within SIGMET reports are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1.] 1.1.3.

Note: requirements relating to reporting of the SIGMET phenomenon are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1.] 1.1.4.

Note: within an XML encoded SIGMET it is likely that only one instance of saf:Airspace will physically be present; subsequent assertions about the airspace may use xlinks to refer to the previously defined saf:Airspace element in order to keep the XML document size small. As such, validation of requirement http://icao.int/iwxxm/1.1/req/xsd-sigmet/unique-subject-airspace is applied once any xlinks, if used, have been resolved.

Note: Part D Code table D-1 provides a set of nil reason codes and is published at http://codes.wmo.int/common/nil.

Note: Part D Code table D-9 is published online at http://codes.wmo.int/49-2/SigWxPhenomena.

205.32	Requirements Class: Volcanic ash SIGMET

205.32.1	This requirements class is used to describe the Volcanic Ash (VA) SIGMET report, which includes additional information about the source volcano and the forecast position of the volcanic ash at the end of the validity period of the SIGMET.

205.32.2	XML elements describing VA-SIGMET reports shall conform to all Requirements specified in Table 35.

205.32.3	XML elements describing VA-SIGMET reports shall conform to all Requirements of all relevant dependencies specified in Table 35.

Table 35. Requirements class xsd-volcanic-ash-sigmet

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet |  |
| Target Type | Data instance |
| Name | Volcanic ash SIGMET |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-erupting-volcano, Part D, 202.8 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis, Part D, 205.30 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-sigmet, Part D, 205.31 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet/valid The content model of this element shall have a value that matches the content model of iwxxm:VolcanicAshSIGMET. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet/source-volcano Details of the volcano that is the source of the volcanic ash shall be reported using the XML element iwxxm:eruptingvolcano with valid child element metce:Volcano (or element in the substitution group of metce:Volcano). |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet/phenomenon The XML attribute iwxxm:phenomenon/@xlink:href shall have a value that is the URI “http://codes.wmo.int/49-2/SigWxPhenomena/VA”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet/forecast-position-analysis The forecast position of the volcanic ash shall be reported using the XML element iwxxm:forecastPositionAnalysis with valid child element om:OM_Observation of type SIGMETPositionAnalysis. The value of XML attribute iwxxm:forecastPositionAnalysis/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETPositionAnalysis”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-volcanic-ash-sigmet/forecast-position-phenomenon-time The valid time of the forecast position of the volcanic ash (indicated with XML element iwxxm:forecastPositionAnalysis/om:OM_Observation/om:phenomenonTime/gml:TimeInstant/gml:timePosition) should match the end of the validity period of the SIGMET (indicated with XML element iwxxm:validPeriod/gml:TimePeriod/gml:endPosition). |

205.33	Requirements Class: Tropical cyclone SIGMET

205.33.1	This requirements class is used to describe the Tropical Cyclone (TC) SIGMET report, which includes additional information about the tropical cyclone itself and the forecast position of the tropical cyclone at the end of the validity period of the SIGMET.

205.33.2	XML elements describing TC-SIGMET reports shall conform to all Requirements specified in Table 36.

205.33.2	XML elements describing TC-SIGMET reports shall conform to all Requirements of all relevant dependencies specified in Table 36.

Table 36. Requirements class xsd-tropical-cyclone-sigmet

| Requirements Class |  |
| --- | --- |
| http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet |  |
| Target Type | Data instance |
| Name | Tropical cyclone SIGMET |
| Dependency | http://def.wmo.int/metce/2013/req/xsd-tropical-cyclone, Part D, 202.9 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-sigmet-position-analysis, Part D, 205.30 |
| Dependency | http://icao.int/iwxxm/1.1/req/xsd-sigmet, Part D, 205.31 |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet/valid The content model of this element shall have a value that matches the content model of iwxxm:TropicalCycloneSIGMET. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet/cyclone Details of the tropical cyclone shall be reported using the XML element iwxxm:tropicalCyclone with valid child element metce:TropicalCyclone. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet/phenomenon The XML attribute iwxxm:phenomenon/@xlink:href shall have a value that is the URI “http://codes.wmo.int/49-2/SigWxPhenomena/TC”. |
| Requirement | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet/forecast-position-analysis The forecast position of the tropical cyclone shall be reported using the XML element iwxxm:forecastPositionAnalysis with valid child element om:OM_Observation of type SIGMETPositionAnalysis. The value of XML attribute iwxxm:forecastPositionAnalysis/om:OM_Observation/om:type/@xlink:href shall be the URI “http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/SIGMETPositionAnalysis”. |
| Recommendation | http://icao.int/iwxxm/1.1/req/xsd-tropical-cyclone-sigmet/forecast-position-phenomenon-time The valid time of the forecast position of the tropical cyclone (indicated with XML element iwxxm:forecastPositionAnalysis/om:OM_Observation/om:phenomenonTime/gml:TimeInstant/gml:timePosition) should match the end of the validity period of the SIGMET (indicated with XML element iwxxm:validPeriod/gml:TimePeriod/gml:endPosition). |

Appendix A: Code Tables

Code Table D-1: Nil Reasons

Nil-reason terms are used to provide an explanation for recording a missing (or void) value within a data product. Terms are drawn from authorities in addition to WMO including ISO/TC 211 (from ISO 19136:2007 Geography markup language clause 8.2.3.1; published on their behalf by the Open Geospatial Consortium). The code-space indicates the authority under which the nil-reason terms are published. A URI for each nil-reason term is composed by appending the notation to the code-space. As an example, the URI of notDetectedByAutoSystem is http://codes.wmo.int/common/nil/notDetectedByAutoSystem. The URI is also a URL providing additional information about the associated nil-reason term. This code table is published at http://codes.wmo.int/common/nil.

| Label | Notation | Code-space | Description |
| --- | --- | --- | --- |
| Above detection range | AboveDetectionRange | http://www.opengis.net/def/nil/OGC/0/ | The value was above the detection range of the instrument used to estimate it. |
| Below detection range | BelowDetectionRange | http://www.opengis.net/def/nil/OGC/0/ | The value was above the detection range of the instrument used to estimate it. |
| Inapplicable | inapplicable | http://www.opengis.net/def/nil/OGC/0/ | There is no value. |
| Missing | missing | http://www.opengis.net/def/nil/OGC/0/ | The correct value is not readily available to the sender of this data. Furthermore, a correct value may not exist. |
| No significant change (NOSIG) | noSignificantChange | http://codes.wmo.int/common/nil/ | No significant change is expected to occur. |
| Nothing detected by automated system | notDetectedByAutoSystem | http://codes.wmo.int/common/nil/ | The automated observing system did not detect a value (e.g. no cloud detected ‘NCD’). |
| Not observable | notObservable | http://codes.wmo.int/common/nil/ | A system failure, sensor failure, or sensor obstruction prevented the intended observation of the value. |
| Nothing of operational significance | nothingOfOperationalSignificance | http://codes.wmo.int/common/nil/ | Nothing was observed or forecast of operational significance (e.g. nil significant cloud ‘NSC’, nil significant weather ‘NSW’). |
| Template | template | http://www.opengis.net/def/nil/OGC/0/ | The value will be available later. |
| Unknown | unknown | http://www.opengis.net/def/nil/OGC/0/ | The correct value is not known to, and not computable by, the sender of the data. However, a correct value probably exists. |
| Withheld | withheld | http://www.opengis.net/def/nil/OGC/0/ | The value is not divulged. |

CODE TABLE D-2: Physical quantity kInds

The Unique Resource Identifier (URI) of each physical quantity kind is composed by the prefix http://codes.wmo.int/common/quantity-kind/ and the notation. As an example the URI of airTemperature is http://codes.wmo.int/common/quantity-kind/airTemperature . The URI can be used in the XML code format and is also a URL providing comprehensive information regarding the physical quantity kind.

### Meteorological quantities

| Label | Notation | Description | Dimensions |
| --- | --- | --- | --- |
| Air temperature | airTemperature | The temperature indicated by a thermometer exposed to the air in a place sheltered from direct solar radiation. | Θ |
| Atmospheric pressure | atmosphericPressure | The atmospheric pressure on a given surface is the force per unit area exerted by virtue of the weight of the atmosphere above. The pressure is thus equal to the weight of a vertical column of air above a horizontal projection of the surface, extending to the outer limit of the atmosphere. | ML-1T-2 |
| Dew-point temperature | dewPointTemperature | The temperature to which a given air parcel must be cooled at constant pressure and constant water vapour content in order for saturation to occur. | Θ |
| Height of base of cloud | heightOfBaseOfCloud | For a given cloud or cloud layer, vertical distance (measured from local ground surface) of the lowest level in the atmosphere at which the air contains a perceptible quantity of cloud particles. | L |
| Horizontal visibility | horizontalVisibility | The greatest distance determined in the horizontal plane at the ground surface that prominent objects can be seen and identified by unaided, normal eyes. | L |
| Maximum wind gust speed | maximumWindGustSpeed | Nominal maximum speed of wind during a given period; usually determined as a mean wind speed over a short duration (e.g. 1-minute) within a longer period (e.g. 10-minutes). | LT-1 |
| Sea surface temperature | seaSurfaceTemperature | Temperature of the sea water at surface. | Θ |
| Vertical visibility | verticalVisibility | Maximum distance at which an observer can see and identify an object on the same vertical as himself, above or below. | L |

### Oceanographic quantities

| Label | Notation | Description | Dimensions |
| --- | --- | --- | --- |
| Sea surface temperature | seaSurfaceTemperature | Temperature of the sea water at surface. | Θ |

### Aeronautical quantities

| Label | Notation | Description | Dimensions |
| --- | --- | --- | --- |
| Aerodrome maximum wind gust speed | aerodromeMaximumWindGustSpeed | Maximum wind speed in the 10 minute period of observation. It is reported only if exceeds the mean speed by 5 m s–1 (10 knots). | LT-1 |
| Aerodrome mean wind direction | aerodromeMeanWindDirection | The mean true direction in degrees from which the wind is blowing over the 10-minute period immediately preceding the observation. When the 10-minute period includes a marked discontinuity in the wind characteristics (1), only data after the discontinuity shall be used for mean wind direction and variations of the wind direction, hence the time interval in these circumstances shall be correspondingly reduced. | dimensionless |
| Aerodrome mean wind speed | aerodromeMeanWindSpeed | The mean speed of the wind over the 10-minute period immediately preceding the observation. When the 10-minute period includes a marked discontinuity in the wind characteristics (1), only data after the discontinuity shall be used for obtaining mean wind speed, hence the time interval in these circumstances shall be correspondingly reduced. | LT-1 |
| Aerodrome minimum horizontal visibility | aerodromeMinimumHorizontalVisibility | The minimum horizontal visibility that is reported when the horizontal visibility is not the same in different directions and when the minimum visibility is different from the prevailing visibility, and less than 1500 metres or less than 50% of the prevailing visibility, and less than 5000 metres. | L |
| Aerodrome minimum visibility direction | aerodromeMinimumVisibilityDirection | When the minimum horizontal visibility is reported, its general direction in relation to the aerodrome reference point has to be reported and indicated by reference to one of the eight points of the compass. If the minimum visibility is observed in more than one direction, the Dv shall represent the most operationally significant direction. | dimensionless |
| Aeronautical prevailing horizontal visibility | aeronauticalPrevailingHorizontalVisibility | The greatest visibility value, observed in accordance with the definition of “visibility”, which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors. | L |
| Aeronautical visibility | aeronauticalVisibility | The greater of: (a) The greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; (b) The greatest distance at which lights in the vicinity of 1000 candelas can be seen and identified against an unlit background. | L |
| Altimeter setting (QNH) | altimeterSettingQnh | Altimeter setting (also known as QNH) is defined as barometric pressure adjusted to sea level. It is a pressure setting used by pilots, air traffic control (ATC), and low frequency weather beacons to refer to the barometric setting which, when set on an aircraft's altimeter, will cause the altimeter to read altitude above mean sea level within a certain defined region. | ML-1T-2 |
| Depth of runway deposit | depthOfRunwayDeposit | Depth of deposit on surface of runway | L |
| Runway contamination coverage | runwayContaminationCoverage | Proportion of runway that is contaminated. A runway is considered to be contaminated when more than 25% of the runway surface area (whether in isolated areas or not) within the required length and width being used is covered by the following: (a) Surface water more than 3 mm deep, or by slush or loose snow equivalent to more than 3 mm of water; (b) Snow which has been compressed into a solid mass which resists further compression and will hold together or break into lumps if picked up (compacted snow); or (c) Ice, including wet ice. | dimensionless |
| Runway friction coefficient | runwayFrictionCoefficient | Quantitative assessment of friction coefficient of runway surface. | dimensionless |
| Runway visual range (RVR) | runwayVisualRangeRvr | The range over which the pilot of an aircraft on the centre line of a runway can see the runway surface markings or the lights delineating the runway or identifying its centre line. | L |

NOTES:

- A marked discontinuity occurs when there is an abrupt and sustained change in wind direction of 30°  or  more,  with  a  wind  speed  of  5  m s–1 (10  KT)  or  more  before  or  after  the change, or a change in wind speed of 5 m s–1 (10 KT) or more, lasting at least two minutes.

Code table: FM201 – Collect

There are no code tables  that are specific to FM201.

Code table: FM202 – Metce

Code Table D-3: METCE observation types

The items within this code table are specialized observation or measurement types defined within 'Modèle pour l'Échange des informations sur le Temps, le Climat et l'Eau' (METCE). Each observation or measurement type listed herein is specified as a class in METCE that derives from the OM_Observation class (defined in ISO 19156, Geographic information - Observations and measurements, clause 6.2) or sub-class thereof. A URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of ComplexSamplingMeasurement is http://codes.wmo.int/common/observation-type/METCE/2013/ComplexSamplingMeasurement. The URI is also a URL providing additional information about the associated observation type. This code table is published at http://codes.wmo.int/common/observation-type/METCE/2013.

| Label | Notation | Code-space | Description |
| --- | --- | --- | --- |
| Complex sampling measurement | ComplexSamplingMeasurement | http://codes.wmo.int/common/observation-type/METCE/2013/ | ComplexSamplingMeasurement (a subclass of OM_ComplexObservation) is intended for use where the observation event is concerned with the evaluation of multiple measurands at a specified location and time instant or duration. The result of this observation type shall refer to an entity of metatype Record (from ISO 19103). ComplexSamplingMeasurement enforces the following additional constraints: 'featureOfInterest' shall refer to an entity of type SF_SpatialSamplingFeature (from ISO 19156), or subclass thereof; and 'procedure' shall refer to an entity of type Process (from METCE), or subclass thereof. The OM_ComplexObservation is used because the 'result' of this class of observations is a group of measures, provided as a record. Again, this matches the WMO application domain wherein multiple phenomena are measured within a single 'observation event'. The term ‘measurement’ is used in the name in an attempt to reduce confusion arising from the overloading of the term ‘observation’. This term is deprecated. |
| Sampling coverage measurement | SamplingCoverageMeasurement | http://codes.wmo.int/common/observation-type/METCE/2013/ | SamplingCoverageMeasurement (a subclass of OM_DiscreteCoverageObservation) is intended for use where the observation is concerned with the evaluation of measurands that vary with respect to space and/or time - the result of this observation type shall refer to an entity of type CV_DiscreteCoverage (from ISO 19123). ComplexSamplingMeasurement enforces the following additional constraints: 'featureOfInterest' shall refer to an entity of type SF_SpatialSamplingFeature (from ISO 19156), or subclass thereof; and 'procedure' shall refer to an entity of type Process (from METCE), or subclass thereof. The Class ‘SamplingCoverageMeasurement’ is based on the SamplingCoverageObservation which is defined in an informative annex of ISO 19156. The term ‘measurement’ is used in the name in an attempt to reduce confusion arising from the overloading of the term ‘observation’. This term is deprecated. |
| Sampling observation | SamplingObservation | http://codes.wmo.int/common/observation-type/METCE/2013/ | SamplingObservation (a subclass of OM_Observation) provides a general purpose observation type. It enforces the following additional constraints: 'featureOfInterest' shall refer to an entity of type SF_SpatialSamplingFeature (from ISO 19156), or subclass thereof; and 'procedure' shall refer to an entity of type Process (from METCE), or subclass thereof. SamplingObservation is intended for use where measurement of physical phenomena is not the goal of the procedure. For example, the procedure executed to define SIGMET reports results in the identification of areas of turbulence, icing or other meteorological phenomena. This term is deprecated. |
| Category observation | OM_CategoryObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a Scoped Name (as defined in ISO 19156:2011 Clause 7.2.2). For example: A category observation of the "taxon" (property-type) of "specimen 123" (feature-of-interest) by "Amy Bachrach" (procedure) had the result "Eucalyptus caesia" (from the Flora of Australia). |
| Complex observation | OM_ComplexObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a Record (as defined in ISO 19156:2011 Clause 7.2.2). For example: A complex observation of "major element composition" (property-type) for "specimen h8j" (feature-of-interest) using the "ICPMS" (procedure) had the result "(… array of element proportions …)". OM_ComplexObservation is intended for use where the observation event is concerned with the evaluation of multiple properties at a specified location and time instant or duration. The 'result' of this class of observations is a group of values, provided as a Record (as defined in ISO 19103:2005 Geographic information – Conceptual schema language). |
| Count observation | OM_CountObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is an Integer (as defined in ISO 19156:2011 Clause 7.2.2). For example: A count observation of "the number of votes cast" (property-type) at "the municipal election" (feature-of-interest) using the "electronic voting machine tally" (procedure) had the result "3542". |
| Discrete coverage observation | OM_DiscreteCoverageObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a Discrete Coverage (as defined in ISO 19156:2011 Clause 7.3.2). Examples include: (i) The colour of a scene varies with position. The result of an observation of the property ”colour” of the scene is a coverage. Each domain element is a pixel whose index allows the spatial location within the scene to be obtained. (ii) Temperature may be sampled using an array of weather stations. The temperature field of the region covered by the array may be represented as a discrete point coverage, whose domain-elements correspond to the station locations. OM_DiscreteCoverageObservation is intended for use where the observation event is concerned with the evaluation of properties that vary with respect to space and/or time. The 'result' of this class of observations is a discrete coverage (as defined in ISO 19123:2005 Geographic information – Schema for coverage and geometry functions). |
| Geometry observation | OM_GeometryObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a geometry (as defined in ISO 19156:2011 Clause 7.2.2). For example: A geometry observation of "perimeter" (property-type) for "plot 987" (feature-of-interest) using the "field survey GHJ" (procedure) had the result "(… description of polygon …)". |
| Measurement | OM_Measurement | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a scaled number or Measure (as defined in ISO 19156:2011 Clause 7.2.2). For example: A measurement of "mass" (property-type) of "the seventh banana" (feature-of-interest) using the "kitchen scales" (procedure) had the result "150g". |
| Observation | OM_Observation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result type is unconstrained (as defined in ISO 19156:2011 Clause 7). An observation is an act that results in the estimation of the value of a feature property, and involves application of a specified procedure, such as a sensor, instrument, algorithm or process chain. The procedure may be applied in-situ, remotely, or ex-situ with respect to the sampling location. Use of a common model allows observation data using different procedures to be combined unambiguously. Observation details are also important for data discovery and for data quality estimation. Observation feature types are defined by the properties that support these applications. |
| Temporal observation | OM_TemporalObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a TM_Object (as defined in ISO 19156:2011 Clause 7.2.2). For example: A temporal observation of "duration" (property-type) for "Usain Bolt 100m dash" (feature-of-interest) using the "stop watch" (procedure) had the result "9.6s". |
| Truth observation | OM_TruthObservation | http://www.opengis.net/def/observationType/OGC-OM/2.0/ | Observation whose result is a Boolean (as defined in ISO 19156:2011 Clause 7.2.2). For example: A truth observation of "presence" (property-type) of "intruder" (feature-of-interest) using the "CCTV" (procedure) had the result "false". |

Code table: FM203 – OPM

There are no code tables that are specific to FM203.

Code table: FM204 – SAF

There are no code tables that are specific to FM204.

Code table: FM205 – IWXXM

Code Table D-4: IWXXM observation types

The items within this code table are specialized observation or measurement types defined within the ICAO Meteorological Information Exchange Model (IWXXM). Each observation or measurement type listed herein is specified as a class in IWXXM that derives from the OM_Observation class (defined in ISO 19156, Geographic information - Observations and measurements, clause 6.2) or sub-class thereof. A URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of MeteorologicalAerodromeForecast is http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/MeteorologicalAerodromeForecast. The URI is also a URL providing additional information about the associated observation type. This code table is published at http://codes.wmo.int/49-2/observation-type/IWXXM/1.0.

| Label | Notation | Code-space | Description |
| --- | --- | --- | --- |
| Meteorological aerodrome forecast | MeteorologicalAerodromeForecast | http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/ | MeteorologicalAerodromeForecast (a subclass of ComplexSamplingMeasurement from METCE) is intended for use when reporting an aggregate set of forecast meteorological conditions at an aerodrome. The result of this observation type shall refer to an entity of type MeteorologicalAerodromeForecastRecord. MeteorologicalAerodromeForecast enforces the following additional constraints: ‘featureOfInterest’ shall refer to an entity of type SF_SamplingPoint and the associated ‘sampledFeature’ must be an Aerodrome. This class is also related but not identical to MeteorologicalAerodromeTrendForecast which is reported on a METAR/SPECI - conditions reported in trend forecasts in METAR/SPECI differ from forecast groups in a TAF. The TAF forecast group from/to variants (FM, TL, AT, etc.) are represented on the OM_Observation validTime, which is always an instance of TM_Period. When there is only an instant at which a condition occurs, the start and end time are the same. |
| Meteorological aerodrome observation | MeteorologicalAerodromeObservation | http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/ | MeteorologicalAerodromeObservation (a subclass of ComplexSamplingMeasurement from METCE) is intended for use when reporting an aggregate set of observed meteorological conditions at an aerodrome. The result of this observation type shall refer to an entity of type MeteorologicalAerodromeObservationRecord. MeteorologicalAerodromeObservation enforces the following additional constraints: ‘featureOfInterest’ shall refer to an entity of type SF_SamplingPoint and the associated ‘sampledFeature’ must be an Aerodrome. MeteorologicalAerodromeObservation has a peer class for forecast information at an aerodrome: MeteorologicalAerodromeTrendForecast. |
| Meteorological aerodrome trend forecast | MeteorologicalAerodromeTrendForecast | http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/ | MeteorologicalAerodromeTrendForecast (a subclass of ComplexSamplingMeasurement from METCE) is intended for use when reporting an aggregate set of forecast meteorological conditions at an aerodrome. The result of this observation type shall refer to an entity of type MeteorologicalAerodromeTrendForecastRecord. MeteorologicalAerodromeTrendForecast enforces the following additional constraints: ‘featureOfInterest’ shall refer to an entity of type SF_SamplingPoint and the associated ‘sampledFeature’ must be an aerodrome. MeteorologicalAerodromeTrendForecasts are reported in surface observation reports such as SPECI and METAR. MeteorologicalAerodromeTrendForecast has a peer class for observation information at an aerodrome (MeteorologicalAerodromeObservation) which is also reported on a METAR and SPECI for observed phenomena. This class is also related but not identical to MeteorologicalAerodromeForecast which is reported on a TAF - conditions reported in trend forecasts in METAR/SPECI differ from forecast groups in a TAF. |
| SIGMET evolving condition analysis | SIGMETEvolvingConditionAnalysis | http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/ | SIGMETEvolvingConditionAnalysis (a subclass of SamplingObservation from METCE) is intended for use when reporting an observed or forecast aggregate set of meteorological conditions hazardous to flight over a large airspace, including anticipated intensity change plus speed and direction of motion. The result of this observation type shall refer to a single EvolvingMeteorologicalCondition which represents a SIGMET observation or forecast of meteorological conditions. SIGMETEvolvingConditionAnalysis enforces the following additional constraints: ‘featureOfInterest’ shall refer to an entity of type SF_SamplingSurface and the associated ‘sampledFeature’ must be an airspace. |
| SIMGET position analysis | SIGMETPositionAnalysis | http://codes.wmo.int/49-2/observation-type/iwxxm/1.1/ | SIGMETPositionAnalysis (a subclass of SamplingObservation from METCE) is intended for use when reporting the forecast position of meteorological conditions hazardous to flight. The result of this observation type shall refer to one or more MeteorologicalPositions which represents the forecast positions of SIGMET phenomena. SIGMETPositionAnalysis enforces the following additional constraints: ‘featureOfInterest’ shall refer to an entity of type SF_SamplingSurface and the associated ‘sampledFeature’ must be an airspace. |

Code Table D-4: IWXXM observable properties

The items within this code table are composite observable properties that define the set of physical properties evaluated as a result of regulated procedures specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. These include aerodrome observation and forecast reports (e.g. METAR, SPECI and TAF). A URI for each observable property is composed by appending the notation to the code-space. As an example, the URI of MeteorologicalAerodromeForecast is http://codes.wmo.int/49-2/observable-property/MeteorologicalAerodromeForecast. The URI is also a URL providing additional information about the associated observable property. This code table is published at http://codes.wmo.int/49-2/observable-property.

| Label | Notation | Code-space | Description |
| --- | --- | --- | --- |
| Meteorological aerodrome forecast | MeteorologicalAerodromeForecast | http://codes.wmo.int/49-2/observable-property/ | The set of physical properties evaluated as a result of an aerodrome forecast (TAF), as specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. |
| Meteorological aerodrome observation | MeteorologicalAerodromeObservation | http://codes.wmo.int/49-2/observable-property/ | The set of physical properties evaluated as a result of the observation procedure of a routine or special aerodrome meteorological report (METAR or SPECI), as specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. |
| Meteorological aerodrome trend forecast | MeteorologicalAerodromeTrendForecast | http://codes.wmo.int/49-2/observable-property/ | The set of physical properties evaluated as a result of the trend forecast procedure of a routine or special aerodrome meteorological report (METAR or SPECI), as specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. |

Code Table D-5: Aerodrome recent weather

The items within this code table are the weather types that may be reported within a meteorological aerodrome observation report that have occurred during the period since the last issued routine report or last hour, whichever is shorter, but are not observed at the time of the observation. Requirements for reporting recent weather at an aerodrome are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.8.1.1.

This code table contains the set of weather types that are permitted for reporting recent weather. These are a subset of the enumerated set of meteorologically valid combinations specified in Part A, Code table 4678 comprising elements ‘intensity or proximity’, ‘descriptor’, ‘precipitation’, ‘obscuration’ and/or ‘other’.

Each weather type is uniquely identified using a URI. The URI is also a URL providing additional information about the associated weather type. This code table is published at http://codes.wmo.int/49-2/AerodromeRecentWeather.

| Label | Notation | URI |
| --- | --- | --- |
| Blowing snow | REBLSN | http://codes.wmo.int/306/4678/BLSN |
| Duststorm | REDS | http://codes.wmo.int/306/4678/DS |
| Precipitation of drizzle | REDZ | http://codes.wmo.int/306/4678/DZ |
| Funnel cloud(s) (tornado or water spout) | REFC | http://codes.wmo.int/306/4678/FC |
| Precipitation of freezing drizzle | REFZDZ | http://codes.wmo.int/306/4678/FZDZ |
| Precipitation of freezing rain | REFZRA | http://codes.wmo.int/306/4678/FZRA |
| Unidentified freezing precipitation | REFZUP | http://codes.wmo.int/306/4678/FZUP |
| Precipitation of ice pellets | REPL | http://codes.wmo.int/306/4678/PL |
| Precipitation of rain | RERA | http://codes.wmo.int/306/4678/RA |
| Precipitation of snow grains | RESG | http://codes.wmo.int/306/4678/SG |
| Showery precipitation of hail | RESHGR | http://codes.wmo.int/306/4678/SHGR |
| Showery precipitation of snow pellets/small hail | RESHGS | http://codes.wmo.int/306/4678/SHGS |
| Showery precipitation of rain | RESHRA | http://codes.wmo.int/306/4678/SHRA |
| Showery precipitation of snow | RESHSN | http://codes.wmo.int/306/4678/SHSN |
| Unidentified showery precipitation | RESHUP | http://codes.wmo.int/306/4678/SHUP |
| Precipitation of snow | RESN | http://codes.wmo.int/306/4678/SN |
| Sandstorm | RESS | http://codes.wmo.int/306/4678/SS |
| Thunderstorm | RETS | http://codes.wmo.int/306/4678/TS |
| Thunderstorm with precipitation of hail | RETSGR | http://codes.wmo.int/306/4678/TSGR |
| Thunderstorm with precipitation of snow pellets/small hail | RETSGS | http://codes.wmo.int/306/4678/TSGS |
| Thunderstorm with precipitation of rain | RETSRA | http://codes.wmo.int/306/4678/TSRA |
| Thunderstorm with precipitation of snow | RETSSN | http://codes.wmo.int/306/4678/TSSN |
| Thunderstorm with unidentified precipitation | RETSUP | http://codes.wmo.int/306/4678/TSUP |
| Unidentified precipitation | REUP | http://codes.wmo.int/306/4678/UP |
| Volcanic ash | REVA | http://codes.wmo.int/306/4678/VA |

Code Table D-6: Aerodrome PRESENT or forecast weather

The items within this code table are the weather phenomena that may be reported as forecast to occur or have been observed at an aerodrome. Requirements for reporting present or forecast weather at an aerodrome are specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 3, [C.3.1.] 4.4 (observation), Appendix 5, [C.3.1] 2.2.4 (trend forecast) and [C.3.1.] 1.2.3 (for TAF).

The weather phenomena listed here are a subset of the enumerated set of meteorologically valid combinations specified in Part A, Code table 4678 comprising elements ‘intensity or proximity’, ‘descriptor’, ‘precipitation’, ‘obscuration’ and/or ‘other’.

Each weather type is uniquely identified using a URI. The URI is also a URL providing additional information about the associated weather type. This code table is published at http://codes.wmo.int/49-2/AerodromePresentOrForecastWeather.

| Label | Notation | URI |
| --- | --- | --- |
| Light precipitation of drizzle | -DZ | http://codes.wmo.int/306/4678/-DZ |
| Light precipitation of rain | -RA | http://codes.wmo.int/306/4678/-RA |
| Light precipitation of snow | -SN | http://codes.wmo.int/306/4678/-SN |
| Light precipitation of snow grains | -SG | http://codes.wmo.int/306/4678/-SG |
| Light precipitation of ice pellets | -PL | http://codes.wmo.int/306/4678/-PL |
| Light unidentified precipitation | -UP | http://codes.wmo.int/306/4678/-UP |
| Light precipitation of drizzle and rain | -DZRA | http://codes.wmo.int/306/4678/-DZRA |
| Light precipitation of rain and drizzle | -RADZ | http://codes.wmo.int/306/4678/-RADZ |
| Light precipitation of snow and drizzle | -SNDZ | http://codes.wmo.int/306/4678/-SNDZ |
| Light precipitation of snow grains and drizzle | -SGDZ | http://codes.wmo.int/306/4678/-SGDZ |
| Light precipitation of ice pellets and drizzle | -PLDZ | http://codes.wmo.int/306/4678/-PLDZ |
| Light precipitation of drizzle and snow | -DZSN | http://codes.wmo.int/306/4678/-DZSN |
| Light precipitation of rain and snow | -RASN | http://codes.wmo.int/306/4678/-RASN |
| Light precipitation of snow and rain | -SNRA | http://codes.wmo.int/306/4678/-SNRA |
| Light precipitation of snow grains and rain | -SGRA | http://codes.wmo.int/306/4678/-SGRA |
| Light precipitation of ice pellets and rain | -PLRA | http://codes.wmo.int/306/4678/-PLRA |
| Light precipitation of drizzle and snow grains | -DZSG | http://codes.wmo.int/306/4678/-DZSG |
| Light precipitation of rain and snow grains | -RASG | http://codes.wmo.int/306/4678/-RASG |
| Light precipitation of snow and snow grains | -SNSG | http://codes.wmo.int/306/4678/-SNSG |
| Light precipitation of snow grains and snow | -SGSN | http://codes.wmo.int/306/4678/-SGSN |
| Light precipitation of ice pellets and snow | -PLSN | http://codes.wmo.int/306/4678/-PLSN |
| Light precipitation of drizzle and ice pellets | -DZPL | http://codes.wmo.int/306/4678/-DZPL |
| Light precipitation of rain and ice pellets | -RAPL | http://codes.wmo.int/306/4678/-RAPL |
| Light precipitation of snow and ice pellets | -SNPL | http://codes.wmo.int/306/4678/-SNPL |
| Light precipitation of snow grains and ice pellets | -SGPL | http://codes.wmo.int/306/4678/-SGPL |
| Light precipitation of ice pellets and snow grains | -PLSG | http://codes.wmo.int/306/4678/-PLSG |
| Light precipitation of drizzle, rain and snow | -DZRASN | http://codes.wmo.int/306/4678/-DZRASN |
| Light precipitation of drizzle, snow and rain | -DZSNRA | http://codes.wmo.int/306/4678/-DZSNRA |
| Light precipitation of rain, drizzle and snow | -RADZSN | http://codes.wmo.int/306/4678/-RADZSN |
| Light precipitation of rain, snow and drizzle | -RASNDZ | http://codes.wmo.int/306/4678/-RASNDZ |
| Light precipitation of snow, drizzle and rain | -SNDZRA | http://codes.wmo.int/306/4678/-SNDZRA |
| Light precipitation of snow, rain and drizzle | -SNRADZ | http://codes.wmo.int/306/4678/-SNRADZ |
| Light precipitation of drizzle, rain and snow grains | -DZRASG | http://codes.wmo.int/306/4678/-DZRASG |
| Light precipitation of drizzle, snow grains and rain | -DZSGRA | http://codes.wmo.int/306/4678/-DZSGRA |
| Light precipitation of rain, drizzle and snow grains | -RADZSG | http://codes.wmo.int/306/4678/-RADZSG |
| Light precipitation of rain, snow grains and drizzle | -RASGDZ | http://codes.wmo.int/306/4678/-RASGDZ |
| Light precipitation of snow grains, drizzle and rain | -SGDZRA | http://codes.wmo.int/306/4678/-SGDZRA |
| Light precipitation of snow grains, rain and drizzle | -SGRADZ | http://codes.wmo.int/306/4678/-SGRADZ |
| Light precipitation of drizzle, rain and ice pellets | -DZRAPL | http://codes.wmo.int/306/4678/-DZRAPL |
| Light precipitation of drizzle, ice pellets and rain | -DZPLRA | http://codes.wmo.int/306/4678/-DZPLRA |
| Light precipitation of rain, drizzle and ice pellets | -RADZPL | http://codes.wmo.int/306/4678/-RADZPL |
| Light precipitation of rain, ice pellets and drizzle | -RAPLDZ | http://codes.wmo.int/306/4678/-RAPLDZ |
| Light precipitation of ice pellets, drizzle and rain | -PLDZRA | http://codes.wmo.int/306/4678/-PLDZRA |
| Light precipitation of ice pellets, rain and drizzle | -PLRADZ | http://codes.wmo.int/306/4678/-PLRADZ |
| Light precipitation of rain, snow and snow grains | -RASNSG | http://codes.wmo.int/306/4678/-RASNSG |
| Light precipitation of rain, snow grains and snow | -RASGSN | http://codes.wmo.int/306/4678/-RASGSN |
| Light precipitation of snow, rain and snow grains | -SNRASG | http://codes.wmo.int/306/4678/-SNRASG |
| Light precipitation of snow, snow grains and rain | -SNSGRA | http://codes.wmo.int/306/4678/-SNSGRA |
| Light precipitation of snow grains, rain and snow | -SGRASN | http://codes.wmo.int/306/4678/-SGRASN |
| Light precipitation of snow grains, snow and rain | -SGSNRA | http://codes.wmo.int/306/4678/-SGSNRA |
| Light precipitation of rain, snow and ice pellets | -RASNPL | http://codes.wmo.int/306/4678/-RASNPL |
| Light precipitation of rain, ice pellets and snow | -RAPLSN | http://codes.wmo.int/306/4678/-RAPLSN |
| Light precipitation of snow, rain and ice pellets | -SNRAPL | http://codes.wmo.int/306/4678/-SNRAPL |
| Light precipitation of snow, ice pellets and rain | -SNPLRA | http://codes.wmo.int/306/4678/-SNPLRA |
| Light precipitation of ice pellets, rain and snow | -PLRASN | http://codes.wmo.int/306/4678/-PLRASN |
| Light precipitation of ice pellets, snow and rain | -PLSNRA | http://codes.wmo.int/306/4678/-PLSNRA |
| Light precipitation of ice pellets, snow and snow grains | -PLSNSG | http://codes.wmo.int/306/4678/-PLSNSG |
| Light precipitation of ice pellets, snow grains and snow | -PLSGSN | http://codes.wmo.int/306/4678/-PLSGSN |
| Light precipitation of snow, ice pellets and snow grains | -SNPLSG | http://codes.wmo.int/306/4678/-SNPLSG |
| Light precipitation of snow, snow grains and ice pellets | -SNSGPL | http://codes.wmo.int/306/4678/-SNSGPL |
| Light precipitation of snow grains, ice pellets and snow | -SGPLSN | http://codes.wmo.int/306/4678/-SGPLSN |
| Light precipitation of snow grains, snow and ice pellets | -SGSNPL | http://codes.wmo.int/306/4678/-SGSNPL |
| Precipitation of drizzle | DZ | http://codes.wmo.int/306/4678/DZ |
| Precipitation of rain | RA | http://codes.wmo.int/306/4678/RA |
| Precipitation of snow | SN | http://codes.wmo.int/306/4678/SN |
| Precipitation of snow grains | SG | http://codes.wmo.int/306/4678/SG |
| Precipitation of ice pellets | PL | http://codes.wmo.int/306/4678/PL |
| Unidentified precipitation | UP | http://codes.wmo.int/306/4678/UP |
| Precipitation of drizzle and rain | DZRA | http://codes.wmo.int/306/4678/DZRA |
| Precipitation of rain and drizzle | RADZ | http://codes.wmo.int/306/4678/RADZ |
| Precipitation of snow and drizzle | SNDZ | http://codes.wmo.int/306/4678/SNDZ |
| Precipitation of snow grains and drizzle | SGDZ | http://codes.wmo.int/306/4678/SGDZ |
| Precipitation of ice pellets and drizzle | PLDZ | http://codes.wmo.int/306/4678/PLDZ |
| Precipitation of drizzle and snow | DZSN | http://codes.wmo.int/306/4678/DZSN |
| Precipitation of rain and snow | RASN | http://codes.wmo.int/306/4678/RASN |
| Precipitation of snow and rain | SNRA | http://codes.wmo.int/306/4678/SNRA |
| Precipitation of snow grains and rain | SGRA | http://codes.wmo.int/306/4678/SGRA |
| Precipitation of ice pellets and rain | PLRA | http://codes.wmo.int/306/4678/PLRA |
| Precipitation of drizzle and snow grains | DZSG | http://codes.wmo.int/306/4678/DZSG |
| Precipitation of rain and snow grains | RASG | http://codes.wmo.int/306/4678/RASG |
| Precipitation of snow and snow grains | SNSG | http://codes.wmo.int/306/4678/SNSG |
| Precipitation of snow grains and snow | SGSN | http://codes.wmo.int/306/4678/SGSN |
| Precipitation of ice pellets and snow | PLSN | http://codes.wmo.int/306/4678/PLSN |
| Precipitation of drizzle and ice pellets | DZPL | http://codes.wmo.int/306/4678/DZPL |
| Precipitation of rain and ice pellets | RAPL | http://codes.wmo.int/306/4678/RAPL |
| Precipitation of snow and ice pellets | SNPL | http://codes.wmo.int/306/4678/SNPL |
| Precipitation of snow grains and ice pellets | SGPL | http://codes.wmo.int/306/4678/SGPL |
| Precipitation of ice pellets and snow grains | PLSG | http://codes.wmo.int/306/4678/PLSG |
| Precipitation of drizzle, rain and snow | DZRASN | http://codes.wmo.int/306/4678/DZRASN |
| Precipitation of drizzle, snow and rain | DZSNRA | http://codes.wmo.int/306/4678/DZSNRA |
| Precipitation of rain, drizzle and snow | RADZSN | http://codes.wmo.int/306/4678/RADZSN |
| Precipitation of rain, snow and drizzle | RASNDZ | http://codes.wmo.int/306/4678/RASNDZ |
| Precipitation of snow, drizzle and rain | SNDZRA | http://codes.wmo.int/306/4678/SNDZRA |
| Precipitation of snow, rain and drizzle | SNRADZ | http://codes.wmo.int/306/4678/SNRADZ |
| Precipitation of drizzle, rain and snow grains | DZRASG | http://codes.wmo.int/306/4678/DZRASG |
| Precipitation of drizzle, snow grains and rain | DZSGRA | http://codes.wmo.int/306/4678/DZSGRA |
| Precipitation of rain, drizzle and snow grains | RADZSG | http://codes.wmo.int/306/4678/RADZSG |
| Precipitation of rain, snow grains and drizzle | RASGDZ | http://codes.wmo.int/306/4678/RASGDZ |
| Precipitation of snow grains, drizzle and rain | SGDZRA | http://codes.wmo.int/306/4678/SGDZRA |
| Precipitation of snow grains, rain and drizzle | SGRADZ | http://codes.wmo.int/306/4678/SGRADZ |
| Precipitation of drizzle, rain and ice pellets | DZRAPL | http://codes.wmo.int/306/4678/DZRAPL |
| Precipitation of drizzle, ice pellets and rain | DZPLRA | http://codes.wmo.int/306/4678/DZPLRA |
| Precipitation of rain, drizzle and ice pellets | RADZPL | http://codes.wmo.int/306/4678/RADZPL |
| Precipitation of rain, ice pellets and drizzle | RAPLDZ | http://codes.wmo.int/306/4678/RAPLDZ |
| Precipitation of ice pellets, drizzle and rain | PLDZRA | http://codes.wmo.int/306/4678/PLDZRA |
| Precipitation of ice pellets, rain and drizzle | PLRADZ | http://codes.wmo.int/306/4678/PLRADZ |
| Precipitation of rain, snow and snow grains | RASNSG | http://codes.wmo.int/306/4678/RASNSG |
| Precipitation of rain, snow grains and snow | RASGSN | http://codes.wmo.int/306/4678/RASGSN |
| Precipitation of snow, rain and snow grains | SNRASG | http://codes.wmo.int/306/4678/SNRASG |
| Precipitation of snow, snow grains and rain | SNSGRA | http://codes.wmo.int/306/4678/SNSGRA |
| Precipitation of snow grains, rain and snow | SGRASN | http://codes.wmo.int/306/4678/SGRASN |
| Precipitation of snow grains, snow and rain | SGSNRA | http://codes.wmo.int/306/4678/SGSNRA |
| Precipitation of rain, snow and ice pellets | RASNPL | http://codes.wmo.int/306/4678/RASNPL |
| Precipitation of rain, ice pellets and snow | RAPLSN | http://codes.wmo.int/306/4678/RAPLSN |
| Precipitation of snow, rain and ice pellets | SNRAPL | http://codes.wmo.int/306/4678/SNRAPL |
| Precipitation of snow, ice pellets and rain | SNPLRA | http://codes.wmo.int/306/4678/SNPLRA |
| Precipitation of ice pellets, rain and snow | PLRASN | http://codes.wmo.int/306/4678/PLRASN |
| Precipitation of ice pellets, snow and rain | PLSNRA | http://codes.wmo.int/306/4678/PLSNRA |
| Precipitation of ice pellets, snow and snow grains | PLSNSG | http://codes.wmo.int/306/4678/PLSNSG |
| Precipitation of ice pellets, snow grains and snow | PLSGSN | http://codes.wmo.int/306/4678/PLSGSN |
| Precipitation of snow, ice pellets and snow grains | SNPLSG | http://codes.wmo.int/306/4678/SNPLSG |
| Precipitation of snow, snow grains and ice pellets | SNSGPL | http://codes.wmo.int/306/4678/SNSGPL |
| Precipitation of snow grains, ice pellets and snow | SGPLSN | http://codes.wmo.int/306/4678/SGPLSN |
| Precipitation of snow grains, snow and ice pellets | SGSNPL | http://codes.wmo.int/306/4678/SGSNPL |
| Heavy precipitation of drizzle | +DZ | http://codes.wmo.int/306/4678/+DZ |
| Heavy precipitation of rain | +RA | http://codes.wmo.int/306/4678/+RA |
| Heavy precipitation of snow | +SN | http://codes.wmo.int/306/4678/+SN |
| Heavy precipitation of snow grains | +SG | http://codes.wmo.int/306/4678/+SG |
| Heavy precipitation of ice pellets | +PL | http://codes.wmo.int/306/4678/+PL |
| Heavy unidentified precipitation | +UP | http://codes.wmo.int/306/4678/+UP |
| Heavy precipitation of drizzle and rain | +DZRA | http://codes.wmo.int/306/4678/+DZRA |
| Heavy precipitation of rain and drizzle | +RADZ | http://codes.wmo.int/306/4678/+RADZ |
| Heavy precipitation of snow and drizzle | +SNDZ | http://codes.wmo.int/306/4678/+SNDZ |
| Heavy precipitation of snow grains and drizzle | +SGDZ | http://codes.wmo.int/306/4678/+SGDZ |
| Heavy precipitation of ice pellets and drizzle | +PLDZ | http://codes.wmo.int/306/4678/+PLDZ |
| Heavy precipitation of drizzle and snow | +DZSN | http://codes.wmo.int/306/4678/+DZSN |
| Heavy precipitation of rain and snow | +RASN | http://codes.wmo.int/306/4678/+RASN |
| Heavy precipitation of snow and rain | +SNRA | http://codes.wmo.int/306/4678/+SNRA |
| Heavy precipitation of snow grains and rain | +SGRA | http://codes.wmo.int/306/4678/+SGRA |
| Heavy precipitation of ice pellets and rain | +PLRA | http://codes.wmo.int/306/4678/+PLRA |
| Heavy precipitation of drizzle and snow grains | +DZSG | http://codes.wmo.int/306/4678/+DZSG |
| Heavy precipitation of rain and snow grains | +RASG | http://codes.wmo.int/306/4678/+RASG |
| Heavy precipitation of snow and snow grains | +SNSG | http://codes.wmo.int/306/4678/+SNSG |
| Heavy precipitation of snow grains and snow | +SGSN | http://codes.wmo.int/306/4678/+SGSN |
| Heavy precipitation of ice pellets and snow | +PLSN | http://codes.wmo.int/306/4678/+PLSN |
| Heavy precipitation of drizzle and ice pellets | +DZPL | http://codes.wmo.int/306/4678/+DZPL |
| Heavy precipitation of rain and ice pellets | +RAPL | http://codes.wmo.int/306/4678/+RAPL |
| Heavy precipitation of snow and ice pellets | +SNPL | http://codes.wmo.int/306/4678/+SNPL |
| Heavy precipitation of snow grains and ice pellets | +SGPL | http://codes.wmo.int/306/4678/+SGPL |
| Heavy precipitation of ice pellets and snow grains | +PLSG | http://codes.wmo.int/306/4678/+PLSG |
| Heavy precipitation of drizzle, rain and snow | +DZRASN | http://codes.wmo.int/306/4678/+DZRASN |
| Heavy precipitation of drizzle, snow and rain | +DZSNRA | http://codes.wmo.int/306/4678/+DZSNRA |
| Heavy precipitation of rain, drizzle and snow | +RADZSN | http://codes.wmo.int/306/4678/+RADZSN |
| Heavy precipitation of rain, snow and drizzle | +RASNDZ | http://codes.wmo.int/306/4678/+RASNDZ |
| Heavy precipitation of snow, drizzle and rain | +SNDZRA | http://codes.wmo.int/306/4678/+SNDZRA |
| Heavy precipitation of snow, rain and drizzle | +SNRADZ | http://codes.wmo.int/306/4678/+SNRADZ |
| Heavy precipitation of drizzle, rain and snow grains | +DZRASG | http://codes.wmo.int/306/4678/+DZRASG |
| Heavy precipitation of drizzle, snow grains and rain | +DZSGRA | http://codes.wmo.int/306/4678/+DZSGRA |
| Heavy precipitation of rain, drizzle and snow grains | +RADZSG | http://codes.wmo.int/306/4678/+RADZSG |
| Heavy precipitation of rain, snow grains and drizzle | +RASGDZ | http://codes.wmo.int/306/4678/+RASGDZ |
| Heavy precipitation of snow grains, drizzle and rain | +SGDZRA | http://codes.wmo.int/306/4678/+SGDZRA |
| Heavy precipitation of snow grains, rain and drizzle | +SGRADZ | http://codes.wmo.int/306/4678/+SGRADZ |
| Heavy precipitation of drizzle, rain and ice pellets | +DZRAPL | http://codes.wmo.int/306/4678/+DZRAPL |
| Heavy precipitation of drizzle, ice pellets and rain | +DZPLRA | http://codes.wmo.int/306/4678/+DZPLRA |
| Heavy precipitation of rain, drizzle and ice pellets | +RADZPL | http://codes.wmo.int/306/4678/+RADZPL |
| Heavy precipitation of rain, ice pellets and drizzle | +RAPLDZ | http://codes.wmo.int/306/4678/+RAPLDZ |
| Heavy precipitation of ice pellets, drizzle and rain | +PLDZRA | http://codes.wmo.int/306/4678/+PLDZRA |
| Heavy precipitation of ice pellets, rain and drizzle | +PLRADZ | http://codes.wmo.int/306/4678/+PLRADZ |
| Heavy precipitation of rain, snow and snow grains | +RASNSG | http://codes.wmo.int/306/4678/+RASNSG |
| Heavy precipitation of rain, snow grains and snow | +RASGSN | http://codes.wmo.int/306/4678/+RASGSN |
| Heavy precipitation of snow, rain and snow grains | +SNRASG | http://codes.wmo.int/306/4678/+SNRASG |
| Heavy precipitation of snow, snow grains and rain | +SNSGRA | http://codes.wmo.int/306/4678/+SNSGRA |
| Heavy precipitation of snow grains, rain and snow | +SGRASN | http://codes.wmo.int/306/4678/+SGRASN |
| Heavy precipitation of snow grains, snow and rain | +SGSNRA | http://codes.wmo.int/306/4678/+SGSNRA |
| Heavy precipitation of rain, snow and ice pellets | +RASNPL | http://codes.wmo.int/306/4678/+RASNPL |
| Heavy precipitation of rain, ice pellets and snow | +RAPLSN | http://codes.wmo.int/306/4678/+RAPLSN |
| Heavy precipitation of snow, rain and ice pellets | +SNRAPL | http://codes.wmo.int/306/4678/+SNRAPL |
| Heavy precipitation of snow, ice pellets and rain | +SNPLRA | http://codes.wmo.int/306/4678/+SNPLRA |
| Heavy precipitation of ice pellets, rain and snow | +PLRASN | http://codes.wmo.int/306/4678/+PLRASN |
| Heavy precipitation of ice pellets, snow and rain | +PLSNRA | http://codes.wmo.int/306/4678/+PLSNRA |
| Heavy precipitation of ice pellets, snow and snow grains | +PLSNSG | http://codes.wmo.int/306/4678/+PLSNSG |
| Heavy precipitation of ice pellets, snow grains and snow | +PLSGSN | http://codes.wmo.int/306/4678/+PLSGSN |
| Heavy precipitation of snow, ice pellets and snow grains | +SNPLSG | http://codes.wmo.int/306/4678/+SNPLSG |
| Heavy precipitation of snow, snow grains and ice pellets | +SNSGPL | http://codes.wmo.int/306/4678/+SNSGPL |
| Heavy precipitation of snow grains, ice pellets and snow | +SGPLSN | http://codes.wmo.int/306/4678/+SGPLSN |
| Heavy precipitation of snow grains, snow and ice pellets | +SGSNPL | http://codes.wmo.int/306/4678/+SGSNPL |
| Light showery precipitation of rain | -SHRA | http://codes.wmo.int/306/4678/-SHRA |
| Light showery precipitation of snow | -SHSN | http://codes.wmo.int/306/4678/-SHSN |
| Light showery precipitation of hail | -SHGR | http://codes.wmo.int/306/4678/-SHGR |
| Light showery precipitation of snow pellets/small hail | -SHGS | http://codes.wmo.int/306/4678/-SHGS |
| Light unidentified showery precipitation | -SHUP | http://codes.wmo.int/306/4678/-SHUP |
| Light showery precipitation of rain and snow | -SHRASN | http://codes.wmo.int/306/4678/-SHRASN |
| Light showery precipitation of snow and rain | -SHSNRA | http://codes.wmo.int/306/4678/-SHSNRA |
| Light showery precipitation of hail and rain | -SHGRRA | http://codes.wmo.int/306/4678/-SHGRRA |
| Light showery precipitation of snow pellets/small hail and rain | -SHGSRA | http://codes.wmo.int/306/4678/-SHGSRA |
| Light showery precipitation of rain and hail | -SHRAGR | http://codes.wmo.int/306/4678/-SHRAGR |
| Light showery precipitation of snow and hail | -SHSNGR | http://codes.wmo.int/306/4678/-SHSNGR |
| Light showery precipitation of hail and snow | -SHGRSN | http://codes.wmo.int/306/4678/-SHGRSN |
| Light showery precipitation of snow pellets/small hail and snow | -SHGSSN | http://codes.wmo.int/306/4678/-SHGSSN |
| Light showery precipitation of rain and snow pellets/small hail | -SHRAGS | http://codes.wmo.int/306/4678/-SHRAGS |
| Light showery precipitation of snow and snow pellets/small hail | -SHSNGS | http://codes.wmo.int/306/4678/-SHSNGS |
| Light showery precipitation of rain, snow and hail | -SHRASNGR | http://codes.wmo.int/306/4678/-SHRASNGR |
| Light showery precipitation of rain, hail and snow | -SHRAGRSN | http://codes.wmo.int/306/4678/-SHRAGRSN |
| Light showery precipitation of snow, rain and hail | -SHSNRAGR | http://codes.wmo.int/306/4678/-SHSNRAGR |
| Light showery precipitation of snow, hail and rain | -SHSNGRRA | http://codes.wmo.int/306/4678/-SHSNGRRA |
| Light showery precipitation of hail, rain and snow | -SHGRRASN | http://codes.wmo.int/306/4678/-SHGRRASN |
| Light showery precipitation of hail, snow and rain | -SHGRSNRA | http://codes.wmo.int/306/4678/-SHGRSNRA |
| Light showery precipitation of rain, snow and snow pellets/small hail | -SHRASNGS | http://codes.wmo.int/306/4678/-SHRASNGS |
| Light showery precipitation of rain, snow pellets/small hail and snow | -SHRAGSSN | http://codes.wmo.int/306/4678/-SHRAGSSN |
| Light showery precipitation of snow, rain and snow pellets/small hail | -SHSNRAGS | http://codes.wmo.int/306/4678/-SHSNRAGS |
| Light showery precipitation of snow, snow pellets/small hail and rain | -SHSNGSRA | http://codes.wmo.int/306/4678/-SHSNGSRA |
| Light showery precipitation of snow pellets/small hail, rain and snow | -SHGSRASN | http://codes.wmo.int/306/4678/-SHGSRASN |
| Light showery precipitation of snow pellets/small hail, snow and rain | -SHGSSNRA | http://codes.wmo.int/306/4678/-SHGSSNRA |
| Showery precipitation of rain | SHRA | http://codes.wmo.int/306/4678/SHRA |
| Showery precipitation of snow | SHSN | http://codes.wmo.int/306/4678/SHSN |
| Showery precipitation of hail | SHGR | http://codes.wmo.int/306/4678/SHGR |
| Showery precipitation of snow pellets/small hail | SHGS | http://codes.wmo.int/306/4678/SHGS |
| Unidentified showery precipitation | SHUP | http://codes.wmo.int/306/4678/SHUP |
| Showery precipitation of rain and snow | SHRASN | http://codes.wmo.int/306/4678/SHRASN |
| Showery precipitation of snow and rain | SHSNRA | http://codes.wmo.int/306/4678/SHSNRA |
| Showery precipitation of hail and rain | SHGRRA | http://codes.wmo.int/306/4678/SHGRRA |
| Showery precipitation of snow pellets/small hail and rain | SHGSRA | http://codes.wmo.int/306/4678/SHGSRA |
| Showery precipitation of rain and hail | SHRAGR | http://codes.wmo.int/306/4678/SHRAGR |
| Showery precipitation of snow and hail | SHSNGR | http://codes.wmo.int/306/4678/SHSNGR |
| Showery precipitation of hail and snow | SHGRSN | http://codes.wmo.int/306/4678/SHGRSN |
| Showery precipitation of snow pellets/small hail and snow | SHGSSN | http://codes.wmo.int/306/4678/SHGSSN |
| Showery precipitation of rain and snow pellets/small hail | SHRAGS | http://codes.wmo.int/306/4678/SHRAGS |
| Showery precipitation of snow and snow pellets/small hail | SHSNGS | http://codes.wmo.int/306/4678/SHSNGS |
| Showery precipitation of rain, snow and hail | SHRASNGR | http://codes.wmo.int/306/4678/SHRASNGR |
| Showery precipitation of rain, hail and snow | SHRAGRSN | http://codes.wmo.int/306/4678/SHRAGRSN |
| Showery precipitation of snow, rain and hail | SHSNRAGR | http://codes.wmo.int/306/4678/SHSNRAGR |
| Showery precipitation of snow, hail and rain | SHSNGRRA | http://codes.wmo.int/306/4678/SHSNGRRA |
| Showery precipitation of hail, rain and snow | SHGRRASN | http://codes.wmo.int/306/4678/SHGRRASN |
| Showery precipitation of hail, snow and rain | SHGRSNRA | http://codes.wmo.int/306/4678/SHGRSNRA |
| Showery precipitation of rain, snow and snow pellets/small hail | SHRASNGS | http://codes.wmo.int/306/4678/SHRASNGS |
| Showery precipitation of rain, snow pellets/small hail and snow | SHRAGSSN | http://codes.wmo.int/306/4678/SHRAGSSN |
| Showery precipitation of snow, rain and snow pellets/small hail | SHSNRAGS | http://codes.wmo.int/306/4678/SHSNRAGS |
| Showery precipitation of snow, snow pellets/small hail and rain | SHSNGSRA | http://codes.wmo.int/306/4678/SHSNGSRA |
| Showery precipitation of snow pellets/small hail, rain and snow | SHGSRASN | http://codes.wmo.int/306/4678/SHGSRASN |
| Showery precipitation of snow pellets/small hail, snow and rain | SHGSSNRA | http://codes.wmo.int/306/4678/SHGSSNRA |
| Heavy showery precipitation of rain | +SHRA | http://codes.wmo.int/306/4678/+SHRA |
| Heavy showery precipitation of snow | +SHSN | http://codes.wmo.int/306/4678/+SHSN |
| Heavy showery precipitation of hail | +SHGR | http://codes.wmo.int/306/4678/+SHGR |
| Heavy showery precipitation of snow pellets/small hail | +SHGS | http://codes.wmo.int/306/4678/+SHGS |
| Heavy unidentified showery precipitation | +SHUP | http://codes.wmo.int/306/4678/+SHUP |
| Heavy showery precipitation of rain and snow | +SHRASN | http://codes.wmo.int/306/4678/+SHRASN |
| Heavy showery precipitation of snow and rain | +SHSNRA | http://codes.wmo.int/306/4678/+SHSNRA |
| Heavy showery precipitation of hail and rain | +SHGRRA | http://codes.wmo.int/306/4678/+SHGRRA |
| Heavy showery precipitation of snow pellets/small hail and rain | +SHGSRA | http://codes.wmo.int/306/4678/+SHGSRA |
| Heavy showery precipitation of rain and hail | +SHRAGR | http://codes.wmo.int/306/4678/+SHRAGR |
| Heavy showery precipitation of snow and hail | +SHSNGR | http://codes.wmo.int/306/4678/+SHSNGR |
| Heavy showery precipitation of hail and snow | +SHGRSN | http://codes.wmo.int/306/4678/+SHGRSN |
| Heavy showery precipitation of snow pellets/small hail and snow | +SHGSSN | http://codes.wmo.int/306/4678/+SHGSSN |
| Heavy showery precipitation of rain and snow pellets/small hail | +SHRAGS | http://codes.wmo.int/306/4678/+SHRAGS |
| Heavy showery precipitation of snow and snow pellets/small hail | +SHSNGS | http://codes.wmo.int/306/4678/+SHSNGS |
| Heavy showery precipitation of rain, snow and hail | +SHRASNGR | http://codes.wmo.int/306/4678/+SHRASNGR |
| Heavy showery precipitation of rain, hail and snow | +SHRAGRSN | http://codes.wmo.int/306/4678/+SHRAGRSN |
| Heavy showery precipitation of snow, rain and hail | +SHSNRAGR | http://codes.wmo.int/306/4678/+SHSNRAGR |
| Heavy showery precipitation of snow, hail and rain | +SHSNGRRA | http://codes.wmo.int/306/4678/+SHSNGRRA |
| Heavy showery precipitation of hail, rain and snow | +SHGRRASN | http://codes.wmo.int/306/4678/+SHGRRASN |
| Heavy showery precipitation of hail, snow and rain | +SHGRSNRA | http://codes.wmo.int/306/4678/+SHGRSNRA |
| Heavy showery precipitation of rain, snow and snow pellets/small hail | +SHRASNGS | http://codes.wmo.int/306/4678/+SHRASNGS |
| Heavy showery precipitation of rain, snow pellets/small hail and snow | +SHRAGSSN | http://codes.wmo.int/306/4678/+SHRAGSSN |
| Heavy showery precipitation of snow, rain and snow pellets/small hail | +SHSNRAGS | http://codes.wmo.int/306/4678/+SHSNRAGS |
| Heavy showery precipitation of snow, snow pellets/small hail and rain | +SHSNGSRA | http://codes.wmo.int/306/4678/+SHSNGSRA |
| Heavy showery precipitation of snow pellets/small hail, rain and snow | +SHGSRASN | http://codes.wmo.int/306/4678/+SHGSRASN |
| Heavy showery precipitation of snow pellets/small hail, snow and rain | +SHGSSNRA | http://codes.wmo.int/306/4678/+SHGSSNRA |
| Thunderstorm with light precipitation of rain | -TSRA | http://codes.wmo.int/306/4678/-TSRA |
| Thunderstorm with light precipitation of snow | -TSSN | http://codes.wmo.int/306/4678/-TSSN |
| Thunderstorm with light precipitation of hail | -TSGR | http://codes.wmo.int/306/4678/-TSGR |
| Thunderstorm with light precipitation of snow pellets/small hail | -TSGS | http://codes.wmo.int/306/4678/-TSGS |
| Thunderstorm with light unidentified precipitation | -TSUP | http://codes.wmo.int/306/4678/-TSUP |
| Thunderstorm with light precipitation of rain and snow | -TSRASN | http://codes.wmo.int/306/4678/-TSRASN |
| Thunderstorm with light precipitation of snow and rain | -TSSNRA | http://codes.wmo.int/306/4678/-TSSNRA |
| Thunderstorm with light precipitation of hail and rain | -TSGRRA | http://codes.wmo.int/306/4678/-TSGRRA |
| Thunderstorm with light precipitation of snow pellets/small hail and rain | -TSGSRA | http://codes.wmo.int/306/4678/-TSGSRA |
| Thunderstorm with light precipitation of rain and hail | -TSRAGR | http://codes.wmo.int/306/4678/-TSRAGR |
| Thunderstorm with light precipitation of snow and hail | -TSSNGR | http://codes.wmo.int/306/4678/-TSSNGR |
| Thunderstorm with light precipitation of hail and snow | -TSGRSN | http://codes.wmo.int/306/4678/-TSGRSN |
| Thunderstorm with light precipitation of snow pellets/small hail and snow | -TSGSSN | http://codes.wmo.int/306/4678/-TSGSSN |
| Thunderstorm with light precipitation of rain and snow pellets/small hail | -TSRAGS | http://codes.wmo.int/306/4678/-TSRAGS |
| Thunderstorm with light precipitation of snow and snow pellets/small hail | -TSSNGS | http://codes.wmo.int/306/4678/-TSSNGS |
| Thunderstorm with light precipitation of rain, snow and hail | -TSRASNGR | http://codes.wmo.int/306/4678/-TSRASNGR |
| Thunderstorm with light precipitation of rain, hail and snow | -TSRAGRSN | http://codes.wmo.int/306/4678/-TSRAGRSN |
| Thunderstorm with light precipitation of snow, rain and hail | -TSSNRAGR | http://codes.wmo.int/306/4678/-TSSNRAGR |
| Thunderstorm with light precipitation of snow, hail and rain | -TSSNGRRA | http://codes.wmo.int/306/4678/-TSSNGRRA |
| Thunderstorm with light precipitation of hail, rain and snow | -TSGRRASN | http://codes.wmo.int/306/4678/-TSGRRASN |
| Thunderstorm with light precipitation of hail, snow and rain | -TSGRSNRA | http://codes.wmo.int/306/4678/-TSGRSNRA |
| Thunderstorm with light precipitation of rain, snow and snow pellets/small hail | -TSRASNGS | http://codes.wmo.int/306/4678/-TSRASNGS |
| Thunderstorm with light precipitation of rain, snow pellets/small hail and snow | -TSRAGSSN | http://codes.wmo.int/306/4678/-TSRAGSSN |
| Thunderstorm with light precipitation of snow, rain and snow pellets/small hail | -TSSNRAGS | http://codes.wmo.int/306/4678/-TSSNRAGS |
| Thunderstorm with light precipitation of snow, snow pellets/small hail and rain | -TSSNGSRA | http://codes.wmo.int/306/4678/-TSSNGSRA |
| Thunderstorm with light precipitation of snow pellets/small hail, rain and snow | -TSGSRASN | http://codes.wmo.int/306/4678/-TSGSRASN |
| Thunderstorm with light precipitation of snow pellets/small hail, snow and rain | -TSGSSNRA | http://codes.wmo.int/306/4678/-TSGSSNRA |
| Thunderstorm with precipitation of rain | TSRA | http://codes.wmo.int/306/4678/TSRA |
| Thunderstorm with precipitation of snow | TSSN | http://codes.wmo.int/306/4678/TSSN |
| Thunderstorm with precipitation of hail | TSGR | http://codes.wmo.int/306/4678/TSGR |
| Thunderstorm with precipitation of snow pellets/small hail | TSGS | http://codes.wmo.int/306/4678/TSGS |
| Thunderstorm with unidentified precipitation | TSUP | http://codes.wmo.int/306/4678/TSUP |
| Thunderstorm with precipitation of rain and snow | TSRASN | http://codes.wmo.int/306/4678/TSRASN |
| Thunderstorm with precipitation of snow and rain | TSSNRA | http://codes.wmo.int/306/4678/TSSNRA |
| Thunderstorm with precipitation of hail and rain | TSGRRA | http://codes.wmo.int/306/4678/TSGRRA |
| Thunderstorm with precipitation of snow pellets/small hail and rain | TSGSRA | http://codes.wmo.int/306/4678/TSGSRA |
| Thunderstorm with precipitation of rain and hail | TSRAGR | http://codes.wmo.int/306/4678/TSRAGR |
| Thunderstorm with precipitation of snow and hail | TSSNGR | http://codes.wmo.int/306/4678/TSSNGR |
| Thunderstorm with precipitation of hail and snow | TSGRSN | http://codes.wmo.int/306/4678/TSGRSN |
| Thunderstorm with precipitation of snow pellets/small hail and snow | TSGSSN | http://codes.wmo.int/306/4678/TSGSSN |
| Thunderstorm with precipitation of rain and snow pellets/small hail | TSRAGS | http://codes.wmo.int/306/4678/TSRAGS |
| Thunderstorm with precipitation of snow and snow pellets/small hail | TSSNGS | http://codes.wmo.int/306/4678/TSSNGS |
| Thunderstorm with precipitation of rain, snow and hail | TSRASNGR | http://codes.wmo.int/306/4678/TSRASNGR |
| Thunderstorm with precipitation of rain, hail and snow | TSRAGRSN | http://codes.wmo.int/306/4678/TSRAGRSN |
| Thunderstorm with precipitation of snow, rain and hail | TSSNRAGR | http://codes.wmo.int/306/4678/TSSNRAGR |
| Thunderstorm with precipitation of snow, hail and rain | TSSNGRRA | http://codes.wmo.int/306/4678/TSSNGRRA |
| Thunderstorm with precipitation of hail, rain and snow | TSGRRASN | http://codes.wmo.int/306/4678/TSGRRASN |
| Thunderstorm with precipitation of hail, snow and rain | TSGRSNRA | http://codes.wmo.int/306/4678/TSGRSNRA |
| Thunderstorm with precipitation of rain, snow and snow pellets/small hail | TSRASNGS | http://codes.wmo.int/306/4678/TSRASNGS |
| Thunderstorm with precipitation of rain, snow pellets/small hail and snow | TSRAGSSN | http://codes.wmo.int/306/4678/TSRAGSSN |
| Thunderstorm with precipitation of snow, rain and snow pellets/small hail | TSSNRAGS | http://codes.wmo.int/306/4678/TSSNRAGS |
| Thunderstorm with precipitation of snow, snow pellets/small hail and rain | TSSNGSRA | http://codes.wmo.int/306/4678/TSSNGSRA |
| Thunderstorm with precipitation of snow pellets/small hail, rain and snow | TSGSRASN | http://codes.wmo.int/306/4678/TSGSRASN |
| Thunderstorm with precipitation of snow pellets/small hail, snow and rain | TSGSSNRA | http://codes.wmo.int/306/4678/TSGSSNRA |
| Thunderstorm with heavy precipitation of rain | +TSRA | http://codes.wmo.int/306/4678/+TSRA |
| Thunderstorm with heavy precipitation of snow | +TSSN | http://codes.wmo.int/306/4678/+TSSN |
| Thunderstorm with heavy precipitation of hail | +TSGR | http://codes.wmo.int/306/4678/+TSGR |
| Thunderstorm with heavy precipitation of snow pellets/small hail | +TSGS | http://codes.wmo.int/306/4678/+TSGS |
| Thunderstorm with heavy precipitation of unidentified precipitation | +TSUP | http://codes.wmo.int/306/4678/+TSUP |
| Thunderstorm with heavy precipitation of rain and snow | +TSRASN | http://codes.wmo.int/306/4678/+TSRASN |
| Thunderstorm with heavy precipitation of snow and rain | +TSSNRA | http://codes.wmo.int/306/4678/+TSSNRA |
| Thunderstorm with heavy precipitation of hail and rain | +TSGRRA | http://codes.wmo.int/306/4678/+TSGRRA |
| Thunderstorm with heavy precipitation of snow pellets/small hail and rain | +TSGSRA | http://codes.wmo.int/306/4678/+TSGSRA |
| Thunderstorm with heavy precipitation of rain and hail | +TSRAGR | http://codes.wmo.int/306/4678/+TSRAGR |
| Thunderstorm with heavy precipitation of snow and hail | +TSSNGR | http://codes.wmo.int/306/4678/+TSSNGR |
| Thunderstorm with heavy precipitation of hail and snow | +TSGRSN | http://codes.wmo.int/306/4678/+TSGRSN |
| Thunderstorm with heavy precipitation of snow pellets/small hail and snow | +TSGSSN | http://codes.wmo.int/306/4678/+TSGSSN |
| Thunderstorm with heavy precipitation of rain and snow pellets/small hail | +TSRAGS | http://codes.wmo.int/306/4678/+TSRAGS |
| Thunderstorm with heavy precipitation of snow and snow pellets/small hail | +TSSNGS | http://codes.wmo.int/306/4678/+TSSNGS |
| Thunderstorm with heavy precipitation of rain, snow and hail | +TSRASNGR | http://codes.wmo.int/306/4678/+TSRASNGR |
| Thunderstorm with heavy precipitation of rain, hail and snow | +TSRAGRSN | http://codes.wmo.int/306/4678/+TSRAGRSN |
| Thunderstorm with heavy precipitation of snow, rain and hail | +TSSNRAGR | http://codes.wmo.int/306/4678/+TSSNRAGR |
| Thunderstorm with heavy precipitation of snow, hail and rain | +TSSNGRRA | http://codes.wmo.int/306/4678/+TSSNGRRA |
| Thunderstorm with heavy precipitation of hail, rain and snow | +TSGRRASN | http://codes.wmo.int/306/4678/+TSGRRASN |
| Thunderstorm with heavy precipitation of hail, snow and rain | +TSGRSNRA | http://codes.wmo.int/306/4678/+TSGRSNRA |
| Thunderstorm with heavy precipitation of rain, snow and snow pellets/small hail | +TSRASNGS | http://codes.wmo.int/306/4678/+TSRASNGS |
| Thunderstorm with heavy precipitation of rain, snow pellets/small hail and snow | +TSRAGSSN | http://codes.wmo.int/306/4678/+TSRAGSSN |
| Thunderstorm with heavy precipitation of snow, rain and snow pellets/small hail | +TSSNRAGS | http://codes.wmo.int/306/4678/+TSSNRAGS |
| Thunderstorm with heavy precipitation of snow, snow pellets/small hail and rain | +TSSNGSRA | http://codes.wmo.int/306/4678/+TSSNGSRA |
| Thunderstorm with heavy precipitation of snow pellets/small hail, rain and snow | +TSGSRASN | http://codes.wmo.int/306/4678/+TSGSRASN |
| Thunderstorm with heavy precipitation of snow pellets/small hail, snow and rain | +TSGSSNRA | http://codes.wmo.int/306/4678/+TSGSSNRA |
| Light precipitation of freezing drizzle | -FZDZ | http://codes.wmo.int/306/4678/-FZDZ |
| Light precipitation of freezing rain | -FZRA | http://codes.wmo.int/306/4678/-FZRA |
| Light unidentified freezing precipitation | -FZUP | http://codes.wmo.int/306/4678/-FZUP |
| Light precipitation of freezing drizzle and rain | -FZDZRA | http://codes.wmo.int/306/4678/-FZDZRA |
| Light precipitation of freezing rain and drizzle | -FZRADZ | http://codes.wmo.int/306/4678/-FZRADZ |
| Precipitation of freezing drizzle | FZDZ | http://codes.wmo.int/306/4678/FZDZ |
| Precipitation of freezing rain | FZRA | http://codes.wmo.int/306/4678/FZRA |
| Unidentified freezing precipitation | FZUP | http://codes.wmo.int/306/4678/FZUP |
| Precipitation of freezing drizzle and rain | FZDZRA | http://codes.wmo.int/306/4678/FZDZRA |
| Precipitation of freezing rain and drizzle | FZRADZ | http://codes.wmo.int/306/4678/FZRADZ |
| Heavy precipitation of freezing drizzle | +FZDZ | http://codes.wmo.int/306/4678/+FZDZ |
| Heavy precipitation of freezing rain | +FZRA | http://codes.wmo.int/306/4678/+FZRA |
| Heavy unidentified freezing precipitation | +FZUP | http://codes.wmo.int/306/4678/+FZUP |
| Heavy precipitation of freezing drizzle and rain | +FZDZRA | http://codes.wmo.int/306/4678/+FZDZRA |
| Heavy precipitation of freezing rain and drizzle | +FZRADZ | http://codes.wmo.int/306/4678/+FZRADZ |
| Duststorm | DS | http://codes.wmo.int/306/4678/DS |
| Heavy duststorm | +DS | http://codes.wmo.int/306/4678/+DS |
| Duststorm in the vicinity | VCDS | http://codes.wmo.int/306/4678/VCDS |
| Sandstorm | SS | http://codes.wmo.int/306/4678/SS |
| Heavy sandstorm | +SS | http://codes.wmo.int/306/4678/+SS |
| Sandstorm in the vicinity | VCSS | http://codes.wmo.int/306/4678/VCSS |
| Fog | FG | http://codes.wmo.int/306/4678/FG |
| Funnel cloud(s) (tornado or water-spout) | FC | http://codes.wmo.int/306/4678/FC |
| Funnel cloud(s) (tornado or water-spout) | +FC | http://codes.wmo.int/306/4678/+FC |
| Dust/sand whirls (dust devils) | PO | http://codes.wmo.int/306/4678/PO |
| Volcanic ash | VA | http://codes.wmo.int/306/4678/VA |
| Fog in the vicinity | VCFG | http://codes.wmo.int/306/4678/VCFG |
| Funnel cloud(s) (tornado or water-spout) in the vicinity | VCFC | http://codes.wmo.int/306/4678/VCFC |
| Dust/sand whirls (dust devils) in the vicinity | VCPO | http://codes.wmo.int/306/4678/VCPO |
| Volcanic ash in the vicinity | VCVA | http://codes.wmo.int/306/4678/VCVA |
| Thunderstorm | TS | http://codes.wmo.int/306/4678/TS |
| Thunderstorm in the vicinity | VCTS | http://codes.wmo.int/306/4678/VCTS |
| Shower(s) in the vicinity | VCSH | http://codes.wmo.int/306/4678/VCSH |
| Blowing sand in the vicinity | VCBLSA | http://codes.wmo.int/306/4678/VCBLSA |
| Blowing dust in the vicinity | VCBLDU | http://codes.wmo.int/306/4678/VCBLDU |
| Blowing snow in the vicinity | VCBLSN | http://codes.wmo.int/306/4678/VCBLSN |
| Blowing sand | BLSA | http://codes.wmo.int/306/4678/BLSA |
| Blowing dust | BLDU | http://codes.wmo.int/306/4678/BLDU |
| Blowing snow | BLSN | http://codes.wmo.int/306/4678/BLSN |
| Low drifting sand | DRSA | http://codes.wmo.int/306/4678/DRSA |
| Low drifting dust | DRDU | http://codes.wmo.int/306/4678/DRDU |
| Low drifting snow | DRSN | http://codes.wmo.int/306/4678/DRSN |
| Sand | SA | http://codes.wmo.int/306/4678/SA |
| Dust | DU | http://codes.wmo.int/306/4678/DU |
| Shallow fog | MIFG | http://codes.wmo.int/306/4678/MIFG |
| Partial fog (covering part of the aerodrome) | PRFG | http://codes.wmo.int/306/4678/PRFG |
| Patches of fog | BCFG | http://codes.wmo.int/306/4678/BCFG |
| Freezing fog | FZFG | http://codes.wmo.int/306/4678/FZFG |
| Mist | BR | http://codes.wmo.int/306/4678/BR |
| Haze | HZ | http://codes.wmo.int/306/4678/HZ |
| Smoke | FU | http://codes.wmo.int/306/4678/FU |
| Squalls | SQ | http://codes.wmo.int/306/4678/SQ |

Code Table D-7: Cloud amount reported at aerodrome

The items within this code table are the cloud amount categories of operational significance for aviation as specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. This code table contains a subset of the cloud amount categories defined in Part B FM 94 BUFR Code table 0 20 008. Each code item is uniquely identified using a URI. The URI is also a URL providing additional information about the associated cloud amount category. This code table is published at http://codes.wmo.int/49-2/CloudAmountReportedAtAerodrome.

| Label | Notation | URI | Description |
| --- | --- | --- | --- |
| Broken | BKN | http://codes.wmo.int/bufr4/codeflag/0-20-008/3 | Broken (5-7 oktas). |
| Embedded | EMBD | http://codes.wmo.int/bufr4/codeflag/0-20-008/16 | Embedded. Applicable only to cumulonimbus (CB). |
| Few | FEW | http://codes.wmo.int/bufr4/codeflag/0-20-008/1 | Few (1-2 oktas). |
| Frequent | FRQ | http://codes.wmo.int/bufr4/codeflag/0-20-008/12 | Frequent. Applicable only to cumulonimbus (CB). |
| Isolated | ISOL | http://codes.wmo.int/bufr4/codeflag/0-20-008/8 | Isolated. Applicable only to cumulonimbus (CB). |
| Layers | LYR | http://codes.wmo.int/bufr4/codeflag/0-20-008/14 | Layers. Applicable only to cumulonimbus (CB). |
| Occasional | OCNL | http://codes.wmo.int/bufr4/codeflag/0-20-008/10 | Occasional. Applicable only to cumulonimbus (CB). |
| Overcast | OVC | http://codes.wmo.int/bufr4/codeflag/0-20-008/4 | Overcast (8 oktas). |
| Scattered | SCT | http://codes.wmo.int/bufr4/codeflag/0-20-008/2 | Scattered (3-4 oktas). |
| Sky clear | SKC | http://codes.wmo.int/bufr4/codeflag/0-20-008/0 | Sky clear (0 oktas). |

Code Table D-8: Significant convective cloud type

The items within this code table are the cloud types of operational significance for aviation as specified in Technical Regulations, Volume II (WMO-No. 49): Meteorological Services for International Air Navigation. This code table contains a subset of the cloud types defined in Part B FM 94 BUFR Code table 0 20 012. Each cloud type is uniquely identified using a URI. The URI is also a URL providing additional information about the associated cloud type. This code table is published at http://codes.wmo.int/49-2/SigConvectiveCloudType.

| Label | Notation | URI | Description |
| --- | --- | --- | --- |
| Cumulonimbus | CB | http://codes.wmo.int/bufr4/codeflag/0-20-012/9 | A principal cloud type, exceptionally dense and vertically developed, occurring either as isolated clouds or as a line or wall of clouds with separated upper portions. |
| Towering cumulous | TCU | http://codes.wmo.int/bufr4/codeflag/0-20-012/32 | Cumulus mediocris or congestus, towering cumulus (TCU), with or without Cumulus of species fractus or humilis or Stratocumulus, all having their bases at the same level. |

Code Table D-9: Significant weather phenomena

The items within this code table are the types of weather phenomena of significance to aeronautical operations – as used in SIGMET and AIRMET reports and specified in Technical Regulations, Volume II (WMO-No. 49), Part II, Appendix 6, [C.3.1.] 1.1.4. Each weather phenomenon type is uniquely identified using a URI. The URI is also a URL providing additional information about the associated weather phenomena type. This code table is published at http://codes.wmo.int/49-2/SigWxPhenomena.

| Label | Notation | URI | Description |
| --- | --- | --- | --- |
| Embedded thunderstorm | EMBD_TS | http://codes.wmo.int/49-2/SigWxPhenomena/EMBD_TS |  |
| Embedded thunderstorm with hail | EMBD_TSGR | http://codes.wmo.int/49-2/SigWxPhenomena/EMBD_TSGR |  |
| Frequent thunderstorm | FRQ_TS | http://codes.wmo.int/49-2/SigWxPhenomena/FRQ_TS |  |
| Frequent thunderstorm with hail | FRQ_TSGR | http://codes.wmo.int/49-2/SigWxPhenomena/FRQ_TSGR |  |
| Heavy dust storm | HVY_DS | http://codes.wmo.int/49-2/SigWxPhenomena/HVY_DS |  |
| Heavy sand storm | HVY_SS | http://codes.wmo.int/49-2/SigWxPhenomena/HVY_SS |  |
| Obscured thunderstorm | OBSC_TS | http://codes.wmo.int/49-2/SigWxPhenomena/OBSC_TS |  |
| Obscured thunderstorm with hail | OBSC_TSGR | http://codes.wmo.int/49-2/SigWxPhenomena/OBSC_TSGR |  |
| Radioactive cloud | RDOACT_CLD | http://codes.wmo.int/49-2/SigWxPhenomena/RDOACT_CLD |  |
| Severe airframe icing | SEV_ICE | http://codes.wmo.int/49-2/SigWxPhenomena/SEV_ICE |  |
| Severe airframe icing from freezing rain | SEV_ICE_FZRA | http://codes.wmo.int/49-2/SigWxPhenomena/SEV_ICE_FZRA |  |
| Severe mountain wave | SEV_MTW | http://codes.wmo.int/49-2/SigWxPhenomena/SEV_MTW |  |
| Severe turbulence | SEV_TURB | http://codes.wmo.int/49-2/SigWxPhenomena/SEV_TURB |  |
| Squall line | SQL_TS | http://codes.wmo.int/49-2/SigWxPhenomena/SQL_TS |  |
| Squall line with hail | SQL_TSGR | http://codes.wmo.int/49-2/SigWxPhenomena/SQL_TSGR |  |
| Tropical cyclone | TC | http://codes.wmo.int/49-2/SigWxPhenomena/TC |  |
| Volcanic ash | VA | http://codes.wmo.int/49-2/SigWxPhenomena/VA |  |

Appendix B. Definition of Schemas

[Note to editors: the contents of the sub-sections are URLs. They must not be translated]

1	COLLECT-XML

1.1	http://schemas.wmo.int/collect/1.1/collect.xsd

2	METCE-XML (Modéle pour l'échange des informations sur le Temps, le Climat et l'Eau)

2.1	http://schemas.wmo.int/metce/1.1/metce.xsd

2.2	http://schemas.wmo.int/metce/1.1/procedure.xsd

2.3	http://schemas.wmo.int/metce/1.1/phenomena.xsd

3	OPM-XML (Modéle pour l'échange des informations sur le Temps, le Climat et l'Eau)

3.1	http://schemas.wmo.int/opm/1.1/opm.xsd

3.2	http://schemas.wmo.int/opm/1.1/observable-property.xsd

4	SAF-XML (Simple Aeronautical Features)

4.1	http://schemas.wmo.int/saf/1.1/saf.xsd

4.2	http://schemas.wmo.int/saf/1.1/dataTypes.xsd

4.3	http://schemas.wmo.int/saf/1.1/features.xsd

4.4	http://schemas.wmo.int/saf/1.1/measures.xsd

5	IWXXM-XML (ICAO Meteorological Information Exchange Model)

5.1	http://schemas.wmo.int/iwxxm/1.1/iwxxm.xsd

5.2	http://schemas.wmo.int/iwxxm/1.1/common.xsd

5.3	http://schemas.wmo.int/iwxxm/1.1/metarSpeci.xsd

5.4	http://schemas.wmo.int/iwxxm/1.1/taf.xsd

5.5	http://schemas.wmo.int/iwxxm/1.1/sigmet.xsd