| World Meteorological Organization COMMISSION FOR BASIC SYSTEMS Inter-Programme Expert Team on Metadata and Data Representation Development Fourth Meeting Geneva, 9 to 12 May 2016 | IPET-MDRD-4/Doc. 3.4(1) |
| --- | --- |
|  | Submitted by: Lowe 28.V.2016 DRAFT 2 |

AGENDA ITEM 3:	Development of Model Driven Code Forms

AGENDA ITEM 3.3:	WaterML2

# Recommended Text

### 3.3	TimeSeriesML

IPET-MDRD considered the draft text defining the XML representation of hydrological time series  information using WaterML2 (input document 3.3(1).   It noted that the standard had been prepared jointly by OGC and CHy. IPET-MDRD agreed with the recommendation to use FM222 as the formal code identification. It advised that the requirements classes in the definitions of model driven code forms are to be treated as [descriptive names suitable for translation | identifiers that must remain the same in each language].

Accepting that WaterML2ML had undergone considerable peer review during the Open Geospatial Consortium’s (OGC) review procedures, IPET-MDRD recommended that ICT-ISS include the proposed amendment in the documentation for CBS-16 and for CHy-15.

In the WaterML2 specification, the requirement xsd-timeseries-core is missing. IPET-MDRD decided to include the requirement, taking into account that the corresponding conformance class A2.6 exists. This should be added to FM 231 after section 231.9.

Any suggestions for modifications to the text of the proposal should be sent to wis-help@wmo.int before 31 July.

URIs for those code list items that were incomplete in input document 3.4(1) would be provided by ?????? before ?????.

IPET-MDRD recommended that CHy should produce interim versions of code tables D-22, D-23, D-24 and D-26. These could be updated by the simple (fast track) procedure if CHy identified additional needs not met by the initial versions.

## Background information

Table numbers and cross-referencing need to be checked and to run continuously from the existing sections. These will also need to be merged with the proposals for WaterML2 and IWXXM.

IPET-MDRD should advise on whether requirements classes are formal identifiers that have to be the same in every language, or are they descriptive text that can be translated?

Table D-16 (data quality indicators) differs from that needed to support the WIGOS metadata standard.

There are no code tables defined for tables D-22, D-23, D-25 and D-26 of WaterML 2.0 Part 2. This means that users are free to create their own code tables. In this case it might have been more appropriate to provide unstructured keywords. IPET-MDRD might like to suggest that CHy develop initial versions of these tables that could be expanded as the needs for entries became clearer.

Draft text for the amendment is in Annex 1.

## Annex 1: Draft amendment to WMO-No. 306 Manual on Codes Volume I.3

Notes to Editors and translators:

1) New text to be added to the Manual is identified in the instructions to editors (in italics).

2) Where text currently in the Manual is modified, text added to the Manual is marked as added text

3) Where text currently in the Manual is modified, text deleted from the Manual is marked as deleted text

4) Additional headings and sub-headings should be included in the index.

5) Not all text that looks like a URL will actually link to a web page.

6) Some text, including all text where works are run together in camelCase and text that looks like a URL, is to be retained in English in all versions of the Manual. These items are denoted with a double underline that must not be transferred to the published manual.

## Amend WMO-No. 306 Manual on Codes Volume I.3 as follows.

In the section “FM system of extensible markup language representations” add the following entry.

| FM 331 Ext. WMLTS-XML | Hydrological Timeseries. Allows a monotonic series of observations over time to be described to the level of detail required for accurate representation as timeseries, with specific consideration for Hydrological data. Res. X (EC-69) |
| --- | --- |
| FM 332 Ext. WMLRGS-XML | Ratings, Gaugings and Sections. Allows the description of the process and conversions used to determine hydrological observations such as river discharge. Res. X (EC-69) |

Add new section FM231: WMLTS-XML and FM232 WMLRGS-XML  following section FM 2221: TMSL-XML.

## FM 231: WaterML2 – Time series

FM 231-16	WMLTS -XML	WATERML2 TIME SERIES OBSERVATIONS

231.1	Scope

WMLTS-XML shall be used for the exchange in extensible markup language (XML) of time series of hydrological  information conforming to the “WaterML2.0 Part 1 Timeseries” conceptual model. WMLTS-XML may be used directly to represent time series information or incorporated as components within other XML encodings.

Note: the WMLTS-XML application schema and XML encoding are both described in the document OGC/IS 10-126r4 WaterML 2.0: Part 1 - Timeseries.

Note: further information on handling application schema and data modelling can be found in the informal document Guidelines on Data Modelling for WMO Codes (available in English only from http://wis.wmo.int/metce-uml).

Note: representation of non-hydrological information in time series should use FM-221 TMSL.

The Requirements Classes defined in WMLTS-XML are listed in Table 231-1.

Table 231-1. Requirements Classes defined in WMLTS-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-xml-rules, Part D, 231.4 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-timeseries-observation, Part D, 231.5 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-timeseries-tvp-observation, Part D, 231.6 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-measurement-timeseries-tvp-observation, Part D, 231.7 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-categorical-timeseries-tvp-observation, Part D, 231.8 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-timeseries-tvp, Part D, 231.9 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-measurement-timeseries-tvp, Part D, 231.10 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-categorical-timeseries-tvp, Part D, 231.11 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-feature-of-interest-monitoring-point, Part D, 231.12 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-observation-process, Part D, 231.13 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-monitoring-point, Part D, 231.14 |
| Requirements Class | http://def.wmo/int/wmlts/2016/req/xsd-collection, Part D, 231.15 |

231.2 XML Schema for WMLTS-XML

231.2.1	Representations of information in WMLTS-XML shall declare the XML namespaces listed in Tables 231-2 and 231-3.

Note: additional namespace declarations may be required depending on the XML elements used within the WMLTS -XML.

Note: the XML Schema is packaged in six XML schema documents (XSD) describing one XML namespace: http://schemas.opengis.net/waterml/2.0

Note: Schematron schemas providing additional constraints are provided alongside the XSD defining WMLTS-XML.

Table 231-2. XML namespaces defined for WMLTS-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://www.opengis.net/waterml/2.0 | wml2 | http://schemas.opengis.net/waterml/2.0/waterml2.xsd |

Table 231-3. External XML namespaces used in WMLTS-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink | https://www.w3.org/1999/xlink.xsd |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |
| OGC OMXML | http://www.opengis.net/om/2.0 | om | http://schemas.opengis.net/om/2.0/observation.xsd |

231.3	Virtual typing

231.3.1	In accordance with OMXML (clause 7.2), the specialization of OM_Observation result type is provided through schematron restriction. The om:type element shall be used to specify the type of OM_Observation that is being encoded using the URI for the corresponding observation type listed in Part D code table D-15.

Note: Part D code Table D-15 is described in Part D Appendix A.

Note: Part D code table D-15 is published online at http://codes.wmo.int/<url-tbc>/

Note: the URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of MeasurementTimeseriesTVPObservation is http://www.opengis.net/def/observationType/waterml/2.0 /MeasurementTimeseriesTVPObservation.

Note: Each URI will resolve to provide further information about the associated observation type.

Note: The terms “observation” and “measurement” evoke a particular concept to meteorologists (e.g. the measurement of physical phenomena using an instrument or sensor). As defined in ISO 19156:2011, Geographic information – Observations and Measurements, an instance of OM_Observation is defined as an “estimate of the value of some property of some feature of interest using a specified procedure”. OM_Measurement is clearly applicable to the measurement of some physical property values using an instrument or sensor, but is equally applicable to the numerical simulation of physical property values using a computational model (e.g. a forecast or reanalysis).

231.4 	Requirements Class: XML Rules

231.4.1	This requirements class contains a set of general rules applicable to the XML encoding.

231.4.2	XML elements shall conform to all Requirements specified in Table 231-4.

Note: XML implementation of WMLTS-XML is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML[OGC/IS 10-025r1 Observations and Measurements 2.0 - XML Implementation]

Table 231-4. Requirements class xsd-xml-rules

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-xml-rules |  |
| Target Type | Data instance |
| Name | XML Rules |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-xml-rules WaterML 2.0 Part 1 clause 10.3 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-xml-rules has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-xml-rules, WaterML 2.0 Part 1 clause A.2.1.

231.5 	Requirements Class: Time series Observation

231.5.1	This requirements class restricts the content model for the XML element OM_Observation relating specifically to time series observations. The requirements classes that depend on this class describe specific result types of time series.

231.5.2	Instances of om:OM_Observation with element om:type specifying an observation type listed in Part D code table D-15 shall conform to all Requirements of all relevant dependencies specified in Table 231-5.

Table 231-5. Requirements class xsd-timeseries-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-timeseries-observation |  |
| Target Type | Data instance |
| Name | Time series Observation |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-timeseries-observation WaterML 2.0 Part 1 clause 10.4 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd- timeseries-observation  has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-timeseries-observation, WaterML 2.0 Part 1 clause A.2.2.

231.6 	Requirements Class: Time series Time-Value Pair (Interleaved) Observation

231.6.1	This requirements class describes the core type of timeseries observation – one with a result of an interleaved time-value pair timeseries.

231.6.2	XML elements describing a Time series Time-Value Pair (Interleaved) Observation shall conform to all Requirements specified in Table 231-6.

Table 231-6. Requirements class xsd-timeseries-tvp-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-timeseries-tvp-observation |  |
| Target Type | Data instance |
| Name | Time series Time-Value Pair (Interleaved) Observation |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-timeseries-tvp-observation WaterML 2.0 Part 1 clause 10.5 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-timeseries-tvp-observation  has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-timeseries-tvp-observation, WaterML 2.0 Part 1 clause A.2.3.

231.7 	Requirements Class: Measurement Time Series Time-Value Pair (Interleaved) Observation

231.7.1	This requirements class describes an interleaved time series time-value pair Observation where the result is a set of Measurement values

231.7.2	XML elements describing a Measurement Time Series Time-Value Pair (Interleaved) Observation shall conform to all Requirements specified in Table 231.7.

Table 231-7. Requirements class xsd-measurement-timeseries-tvp-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-measurement-timeseries-tvp-observation |  |
| Target Type | Data instance |
| Name | Measurement Time Series Time Value Pair (Interleaved) Observation |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-measurement-timeseries-tvp-observation WaterML 2.0 Part 1 clause 10.6 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-measurement-timeseries-tvp-observation  has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-measurement-timeseries-tvp-observation, WaterML 2.0 Part 1 clause A.2.4.

231.8 	Requirements Class: Categorical Time Series Time-Value Pair (Interleaved) Observation

231.8.1	This requirements class describes an interleaved time series time-value pair Observation where the result is a set of Categorical values.

231.8.2	XML elements describing a Categorical Time Series Time-Value Pair (Interleaved) Observation shall conform to all Requirements specified in Table 231-8.

Table 231-8. Requirements class xsd-categorical-timeseries-tvp-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-categorical-timeseries-tvp-observation |  |
| Target Type | Data instance |
| Name | Categorical Time Series Time Value Pair (Interleaved) Observation |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-categorical-tvp-timeseries-observation WaterML 2.0 Part 1 clause 10.7 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-categorical-timeseries-tvp-observation has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-categorical-timeseries-tvp-observation, WaterML 2.0 Part 1 clause A.2.5.

231.9 	Requirements Class: Time Series Time-Value Pair

231.9.1	This requirements class describes the structure of the interleaved time series. The structure supports time-value pair representation with the ability to associate metadata on a point-by-point basis.

231.9.2	XML elements describing a Time Series Time-Value Pair shall conform to all Requirements specified in Table 231-9.

Table 231-9. Requirements class xsd-timeseries-tvp

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-timeseries-tvp |  |
| Target Type | Data instance |
| Name | Time Sseries Time-Value Pair |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-timeseries-tvp WaterML 2.0 Part 1 clause 10.8 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-timeseries-tvp has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-timeseries-tvp, WaterML 2.0 Part 1 clause A.2.7.

231.10 	Requirements Class: Measurement Time Series Time-Value Pair

231.10.1	This requirements class describes a time-value pair where the values are Measurements.

231.10.2	XML elements describing a Measurement Time Series Time-Value Pair shall conform to all Requirements specified in Table 231-10.

Table 231-10. Requirements class xsd-measurement-timeseries-tvp

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-measurement-timeseries-tvp |  |
| Target Type | Data instance |
| Name | Measurement Time Series Time-Value Pair |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-measurement-timeseries-tvp WaterML 2.0 Part 1 clause 10.9 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-measurement-timeseries-tvp   has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-measurement-timeseries-tvp, WaterML 2.0 Part 1 clause A.2.8.

231.11 	Requirements Class: Categorical Time Series Time-Value Pair

231.11.1	This requirements class describes a time-value pair where the values are Categorical values.

231.11.2	XML elements describing a Categorical Time Series Time-Value Pair shall conform to all Requirements specified in Table 231-11.

Table 231-11. Requirements class xsd-categorical-timeseries-tvp

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-categorical-timeseries-tvp |  |
| Target Type | Data instance |
| Name | Categorical Time Series Time-Value Pair |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-categorical-timeseries-tvp WaterML 2.0 Part 1 clause 10.10 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-categorical-timeseries-tvp has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-categorical-timeseries-tvp, WaterML 2.0 Part 1 clause A.2.9.

231.12 	Requirements Class: Monitoring Point as Feature of Interest

231.12.1	This requirements class captures the restriction of OM_Observation where the feature of interest is a MonitoringPoint.

231.12.2	XML elements describing a Monitoring Point as a Feature Of Interest shall conform to all Requirements specified in Table 231.12.

Table 231-12. Requirements class xsd-feature-of-interest-monitoring-point

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-timeseries-observation |  |
| Target Type | Data instance |
| Name | Monitoring Point Feature of Interest |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-feature-of-interest-monitoring-point WaterML 2.0 Part 1 clause 10.11 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-feature-of-interest-monitoring-point has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-feature-of-interest-monitoring-point, WaterML 2.0 Part 1 clause A.2.10.

231.13 	Requirements Class: Observation Process

231.13.1	This requirements class is used to describe the process involved in making time series observation.

231.13.2	XML elements describing a process shall conform to all Requirements specified in Table 231.13.

Table 231.13. Requirements class xsd-observation-process

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-observation-process |  |
| Target Type | Data instance |
| Name | Observation Process |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-observation-process WaterML 2.0 Part 1 clause 10.12 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-observation-process has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-observation-process, WaterML 2.0 Part 1 clause A.2.11.

231.14 	Requirements Class: Monitoring Point

231.14.1	This requirements class is to be used to describe monitoring points (sampling points) that are described using a point-based geometry. WaterML2.0 extends the OMXML Sampling Point type to add metadata specific to hydrological monitoring.

231.14.2	XML elements describing a Monitoring Point shall conform to all Requirements specified in Table 231.14.

Table 231-14. Requirements class xsd-monitoring-point

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-monitoring-point |  |
| Target Type | Data instance |
| Name | Monitoring Point |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-monitoring-point WaterML 2.0 Part 1 clause 10.13 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-monitoring-point has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-monitoring-point, WaterML 2.0 Part 1 clause A.2.12.

231.15 	Requirements Class: Collection

231.15.1	This requirements class is to be used to describe collections of objects from WaterML2.0. This includes observations, sampling features, sampling feature groups and inline dictionaries.

231.15.2	XML elements describing a Collection shall conform to all Requirements specified in Table 231-15.

Table 231-15. Requirements class xsd-collection

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlts/2016/req/xsd-collection |  |
| Target Type | Data instance |
| Name | Collection |
| Dependency | http://www.opengis.net/spec/waterml/2.0/req/xsd-collection WaterML 2.0 Part 1 clause 10.14 |

Note: Dependency http://www.opengis.net/spec/waterml/2.0/req/xsd-collection has associated conformance class http://www.opengis.net/spec/waterml/2.0/conf/xsd-collection, WaterML 2.0 Part 1 clause A.2.13.

## FM 232: WaterML2 – Ratings, Gaugings and Sections

FM 232-16	WMLRGS -XML	 WATERML Ratings,  gaugings and sections

232.1	Scope

WMLRGS-XML shall be used for the exchange in extensible markup language (XML) of hydrological information conforming to the “WaterML2.0 Part 2 Ratings, Gaugings and Sections” conceptual model. WMLRGS-XML may be used directly to encode Ratings, Guagings and Sections information or incorporated as components within other XML encodings.

Note: the WMLRGS-XML application schema and XML encoding are both described in the document OGC/IS 15-018rs WaterML 2.0: Part 2 – Ratings, Gaugings and Sections

Note: further information on handling application schema and data modelling can be found in the informal document Guidelines on Data Modelling for WMO Codes (available in English only from http://wis.wmo.int/metce-uml).

The Requirements Classes defined in WMLRGS-XML are listed in Table 1.

Table 1. Requirements Classes defined in WMLRGS-XML

| Requirements Classes |  |
| --- | --- |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xsd-xml-rules, Part D, 232.4 |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xml-collection, Part D, 232.5 |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xml-conversions, Part D 232.6 |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xml-range-values, Part D 232.7 |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xml-gaugings, Part D, 232.8 |
| Requirements Class | http://def.wmo/int/wmlrgs/2016/req/xml-section-observation, Part D, 232.9 |

232.2 XML Schema for WMLRGS-XML

232.2.1	Representations of information in WMLRGS-XML shall declare the XML namespaces listed in Tables 232-2 and 232-3.

Note: additional namespace declarations may be required depending on the XML elements used within the WMLRGS -XML.

Note: the XML Schema is packaged in six XML schema documents (XSD) describing one XML namespace: http://schemas.opengis.net/waterml/part2/1.0

Note: Schematron schemas providing additional constraints are provided alongside the XSD defining WMLRGS-XML.

Table 232-2. XML namespaces defined for WMLRGS-XML

| XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http://www.opengis.net/waterml/part2/1.0 | wmlrgs | http://schemas.opengis.net/waterml/part2/1.0/waterml2-part2.xsd. |

Table 232-3. External XML namespaces used in WMLRGS-XML

| Standard | XML Namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML Schema | http://www.w3.org/2001/XMLSchema | xs |  |
| Schematron | http://purl.oclc.org/dsdl/schematron | sch |  |
| XML Linking Language | http://www.w3.org/1999/xlink | xlink | https://www.w3.org/1999/xlink.xsd |
| ISO 19136:2006 GML | http://www.opengis.net/gml/3.2 | gml | http://schemas.opengis.net/gml/3.2.1/gml.xsd |
| OGC OMXML | http://www.opengis.net/om/2.0 | om | http://schemas.opengis.net/om/2.0/observation.xsd |
| WaterML2.0 Part 1 – Timeseries | http://www.opengis.net/waterml/2.0 | wml2 | http://schemas.opengis.net/waterml/2.0/waterml2.xsd |

232.3 	Requirements Class: XML Rules

232.3.1	This requirements class contains a set of general rules applicable to the XML encoding.

232.3.2	XML elements shall conform to all Requirements specified in Table 232-4.

Note: XML implementation of WMLRGS-XML is dependent on:

GML [ISO 19136:2007 Geographic Information – Geography Markup Language]

OMXML[OGC/IS 10-025r1 Observations and Measurements 2.0 - XML Implementation]

Table 232-4. Requirements class xml-rules

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-rules |  |
| Target Type | Data instance |
| Name | XML Rules |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-rules WaterML 2.0 Part 2 clause 9.1 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-rules has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-rules, WaterML 2.0 Part 2 clause B.1.

232.4 	Requirements Class: Collections

232.4.1	This requirements class defines a Collection class which extends the WMLTS-XML Collection class to provide support for conversions, gauging and cross-sections.

232.4.2	XML Elements describing a WMLRGS-XML Collection shall conform to all Requirements specified in Table 232-5.

Table 232-5. Requirements class xml-collection

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-collection |  |
| Target Type | Data instance |
| Name | XML Collection |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-collection WaterML 2.0 Part 2 clause 9.2 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-collection has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-collection, WaterML 2.0 Part 2 clause B.2.

232.5 	Requirements Class: Conversions

232.5.1	This requirements class contains a set of general rules applicable to the XML encoding.

232.5.2	XML Elements describing a Conversion shall conform to all Requirements specified in Table 232-6.

Table 232-6. Requirements class xml-conversions

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-conversions |  |
| Target Type | Data instance |
| Name | XML Conversions |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-conversions WaterML 2.0 Part 2 clause 9.3 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-conversions has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-conversions, WaterML 2.0 Part 2 clause B.3.

232.6 	Requirements Class: Range Values

232.6.1	This requirements class contains a set of general rules applicable to the XML encoding.

232.6.2	XML Elements describing Range Values shall conform to all Requirements specified in Table 232-7.

Table 232-7. Requirements class xml-range-values

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-range-values |  |
| Target Type | Data instance |
| Name | XML Rules |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-range-values WaterML 2.0 Part 2 clause 9.4 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-range-values has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-range-values, WaterML 2.0 Part 2 clause B.4.

232.7 	Requirements Class: Gaugings

232.7.1	This requirements class contains a set of general rules applicable to the XML encoding.

232.7.2	XML Elements describing Gaugings shall conform to all Requirements specified in Table 232-8.

Table 232-8. Requirements class xml-gaugings

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-gaugings |  |
| Target Type | Data instance |
| Name | XML Gaugings |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-gaugings WaterML 2.0 Part 2 clause 9.5 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-gaugings has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-gaugings, WaterML 2.0 Part 2 clause B.5.

232.8 	Requirements Class: Section Observation

232.8.1	This requirements class contains a set of general rules applicable to the XML encoding.

232.8.2	XML Elements describing a SectionObservation shall conform to all Requirements specified in Table 232-9.

Table 232-9. Requirements class xml-section-observation

| Requirements Class |  |
| --- | --- |
| http://def.wmo.int/wmlrgs/2016/req/xml-section-observation |  |
| Target Type | Data instance |
| Name | XML Section Observation |
| Dependency | http://www.opengis.net/spec/waterml/part2/1.0/req/xml-section-observation WaterML 2.0 Part 2 clause 9.6 |

Note: Dependency http://www.opengis.net/spec/waterml/part2/1.0/req/xml-section-observation has associated conformance class http://www.opengis.net/spec/req/waterml/part2/1.0/conf/xml-section-observation, WaterML 2.0 Part 2 clause B.6.

Add the following to Appendix A

### Code Table D-15: WMLTS-XML  (WATERML2) – TIME SERIES OBSERVATION TYPES

The items within this code table are specialized observation or measurement types defined within WaterML2.0 Part 1 Time Series. Each observation or measurement type listed herein is implemented in WMLTS-XML using the OM_Observation class (defined in ISO 19156, Geographic information - Observations and measurements, clause 6.2). A URI for each observation type is composed by appending the notation to the code-space. As an example, the URI of MeasurementTimeseriesTVPObservation is http://www.opengis.net/def/observationType/waterml/2.0 /MeasurementTimeseriesTVPObservation.

| Label | Notation | Code-space | Description |
| --- | --- | --- | --- |
| Measurement TVP Timeseries Observation | MeasurementTimeseriesTVPObservation | http://www.opengis.net/def/observationType/waterml/2.0/ | An OM_Observation where the result is a measurement time series |
| Categorical TVP Timeseries Observation | CategoricalTVPTimeseriesObservation | http://www.opengis.net/def/observationType/waterml/2.0/ | An OM_Observation where the result is a categorical time series |

Are other observation types needed?

### Code Table D-16: WMLTS-XML  (WATERML2) – TIME SERIES DATA QUALITY CODES

The items within this code table are the data quality codes that may be used to indicate the quality of individual data points within a WMLTS-XML time series.

Each data quality code is uniquely identified using a URI. The URI is also a URL providing additional information about the quality assessment. This code table is published at http://opengis.net/def/waterml/2.0/quality/.

Note: This table is a subset of the values permitted for FM 221 TSML-XML.

| Label | URI | Description |
| --- | --- | --- |
| Good | http://opengis.net/def/waterml/2.0/quality/good | The data has been examined and represents a reliable measurement. |
| Suspect | http://opengis.net/def/waterml/2.0/quality/suspect | The data should be treated as suspect. |
| Estimate | http://opengis.net/def/waterml/2.0/quality/estimate | The data is an estimate only, not a direct measurement. |
| Poor | http://opengis.net/def/waterml/2.0/quality/poor | The data should be considered as low quality and may have been rejected. |
| Unchecked | http://opengis.net/def/waterml/2.0/quality/unchecked | The data has not been checked by any qualitative method. |
| Missing | http://opengis.net/def/waterml/2.0/quality/missing | The data is missing. |

### Code Table D-17: WMLTS-XML  (WATERML2) – TIME SERIES INTERPOLATION CODES

The items within this code table are used to indicate how data should be interpolated between neighbouring points in a WMLTS-XML time series.

Each quality code is uniquely identified using a URI. The URI is also a URL providing additional information about the interpolation method. This code table is published at http://opengis.net/def/waterml/2.0/interpolationType.

| Label | URI | Description |
| --- | --- | --- |
| Continuous | http://www.opengis.net/def/waterml/2.0/interpolationType/Continuous | A continuous time series indicates the observation result is the value of a property at the indicated instant in time. The points are essentially connected and interpolation may occur between points in order to estimate the value of the property between points. The appropriate time spacing between successive points to minimise interpolation errors is related to rate of change (wrt time) of the property. |
| Discontinuous | http://www.opengis.net/def/waterml/2.0/interpolationType/Discontinuous | The sampling of the property occurs such that it is not possible to regard the series as continuous. The time between samples is too large to classify the measurements as continuous. Example: An infrequent water sample measuring pH. |
| Instant Total | http://www.opengis.net/def/waterml/2.0/interpolationType/InstantTotal | Value represents a total attributed to a specific time instant. This is normally generated from an event based measuring device. Example: An individual tip of a tipping bucket rain gauge. |
| Average Preceding | http://www.opengis.net/def/waterml/2.0/interpolationType/AveragePrec | Value represents the average value over the preceding interval. |
| Maximum Preceding | http://www.opengis.net/def/waterml/2.0/interpolationType/MaxPrec | Value represents the maximum value that was measured during the preceding time interval. |
| Minimum Preceding | http://www.opengis.net/def/waterml/2.0/interpolationType/MinPrec | Value represents the minimum value that was measured during the preceding time interval. |
| Preceding Total | http://www.opengis.net/def/waterml/2.0/interpolationType/TotalPrec | Value represents the total of measurements taken within the previous time interval. |
| Average Succeeding | http://www.opengis.net/def/waterml/2.0/interpolationType/AverageSucc | Value represents the average value over the following interval. |
| Total Succeeding | http://www.opengis.net/def/waterml/2.0/interpolationType/TotalSucc | Value represents the average value over the following interval. |
| Minimum Succeeding | http://www.opengis.net/def/waterml/2.0/interpolationType/MinSucc | Value represents the minimum value for the following interval. |
| Maximum Succeeding | http://www.opengis.net/def/waterml/2.0/interpolationType/MaxSucc | Value represents the maximum value for the following interval. |
| Constant Preceding | http://www.opengis.net/def/waterml/2.0/interpolationType/ConstPrec | Value is constant in the preceding interval. |
| Constant Succeeding | http://www.opengis.net/def/waterml/2.0/interpolationType/ConstSucc | Value is constant in the succeeding interval. |

### Code Table D-18: WMLTS-XML  (WATERML2) – TIME SERIES PROCESS TYPE CODES

The items within this code table are the process type codes that may be used to indicate the type of process that was used in a WMLTS-XML time series observation.

Each process type code is uniquely identified using a URI. The URI is also a URL providing additional information about the process. This code table is published at http://www.opengis.net/def/waterml/2.0/processType/.

| Label | URI | Description |
| --- | --- | --- |
| Algorithm | http://www.opengis.net/def/waterml/2.0/processType/Algorithm | Timeseries data is generated by applying an algorithm to input data |
| Manual Method | http://www.opengis.net/def/waterml/2.0/processType/ManualMethod | Timeseries data is collected manually |
| Sensor | http://www.opengis.net/def/waterml/2.0/processType/Sensor | Timeseries data is collected from an automated sensor |
| Simulation | http://www.opengis.net/def/waterml/2.0/processType/Simulation | Timeseries is generated from a simulation |
| Unknown | http://www.opengis.net/def/waterml/2.0/processType/Unknown | Timeseries is collected or generated by an unknown process |

### Code Table D-19: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections – Timeseriesmedium  Codes

The items within this code table are the medium type codes that may be used to indicate in what medium a WMLTS-XML time series observation occurred.

Each medium type (Timeseriesmedium) code is uniquely identified using a URI. The URI is also a URL providing additional information about the medium. This code table is published at http://www.opengis.net/def/waterml/2.0/mediumType/.

| Label | URI | Description |
| --- | --- | --- |
| Atmosphere | http://www.opengis.net/def/waterml/2.0/medium/Atmosphere | The layer of gases surrounding the planet Earth |
| Ground Snow | http://www.opengis.net/def/waterml/2.0/medium/GroundSnow | Frozen water in the form of snow collecting on the ground |
| Ground Water | http://www.opengis.net/def/waterml/2.0/medium/GroundWater | Water located beneath the ground surface in soil pore spaces and in the fractures of rock formations |
| Interstitial Water / Pore Water. | http://www.opengis.net/def/waterml/2.0/medium/PoreWater | Subsurface water in an interstice or pore |
| Pore Air | http://www.opengis.net/def/waterml/2.0/medium/PoreAir | Subsurface air in an interstice or pore |
| Sediment Water | http://www.opengis.net/def/waterml/2.0/medium/SedimentWater | Water at the boundary between bed sediment and an overlying water column |
| Soil | http://www.opengis.net/def/waterml/2.0/medium/Soil | Particles of broken rock that have been altered by chemical and mechanical processes that include weathering and erosion; the depth of regolith that influence and have been influenced by plant roots |
| Soil Air | http://www.opengis.net/def/waterml/2.0/medium/SoilAir | All forms of air contained within soil |
| Soil Water | http://www.opengis.net/def/waterml/2.0/medium/SoilWater | All forms of water contained within soil |
| Surface Water | http://www.opengis.net/def/waterml/2.0/medium/SurfaceWater | Water collecting on the ground or in a stream, river, lake, wetland, or ocean |
| Tissue | http://www.opengis.net/def/waterml/2.0/medium/Tissue | An ensemble of biological cells |
| Unknown | http://www.opengis.net/def/waterml/2.0/medium/Unknown | An unknown medium |
| Water | http://www.opengis.net/def/waterml/2.0/medium/Water | All forms of water |

### Code Table D-21: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections DevelopmentMethodCode

The items within this code table are the development method codes that may be used to indicate the way the conversion was developed.

Each development method code (DevelopmentMethodCode) is uniquely identified using a URI. The URI is also a URL providing additional information about the development method. This code table is published at http://www.opengis.net/def/waterml/part2/1.0/DevelopmentMethodCode/.

| Label | URI | Description |
| --- | --- | --- |
| equation | http://www.opengis.net/def/waterml/part2/1.0/DevelopmentMethodCode/equation | The conversion was developed using a standard equation (e.g. from a known control structure). |
| estimated | http://www.opengis.net/def/waterml/part2/1.0/DevelopmentMethodCode/estimated | The relationship was estimated using modeling, mass balance or other quantifiable techniques. |
| fromGauging | http://www.opengis.net/def/waterml/part2/1.0/DevelopmentMethodCode/fromGauging | The conversion was developed using regular gaugings. |
| unknown | http://www.opengis.net/def/waterml/part2/1.0/DevelopmentMethodCode/unknown | Unknown development method. |

### Code Table D-22: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections DomainFeatureTypeCode

The items within this code table are the domain feature codes that may be used to indicate the feature type that the conversion relates to. E.g. ‘River’, ‘Reservoir’.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-23: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections PropertyCode

The items within this code table are the property codes that may be used to indicate the observable property E.g. ‘river level’, ‘discharge’.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-24: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections – StatusCode

The items within this code table are codes that may be used to indicate where the conversion is in its development lifecycle.

Each status code (StutusCode) is uniquely identified using a URI. The URI is also a URL providing additional information about the status. This code table is published at http://www.opengis.net/def/waterml/part2/1.0/StatusCode/.

| Label | URI | Description |
| --- | --- | --- |
| working | http://www.opengis.net/def/waterml/part2/1.0/StatusCode/working | The conversion relationship is the working version. This is the currently active conversion. |
| inReview | http://www.opengis.net/def/waterml/part2/1.0/StatusCode/inReview | The conversion is under review. |
| approved | http://www.opengis.net/def/waterml/part2/1.0/StatusCode/approved | The conversion has been approved for use. |
| submitted | http://www.opengis.net/def/waterml/part2/1.0/StatusCode/submitted | The conversion has been reviewed and submitted for approval but has not yet been approved. |
| provisional | http://www.opengis.net/def/waterml/part2/1.0/StatusCode/provisional | The conversion has been modified to include the latest gauging information. It is released for real time purposes only. |

### Code Table D-25: WMLRGS-XML (WaterML2)  – Ratings, Gaugings Guagings and Sections – RangeDefinitionCode

The items within this code table are the range definition codes that may be used to indicate the nature of the range value, e.g. ‘a textual description of different flood levels’.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-26: WMLRGS-XML (WaterML2)  – Ratings, Gaugings Guagings and Sections –  OutputMethodCode

The items within this code table are the output method codes that may be used to indicate the method used when measuring the output value. E.g. code item that describes an acoustic Doppler current profiler.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-27: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections –  InputMethodCode

The items within this code table are the input method codes that may be used to indicate the method used when measuring the input value. E.g. manual staff gauge reading.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-28: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections –  ControlConditionCodes

The items within this code table are the control condition codes that may be used to indicate the condition of the control. E.g. a code indicating ‘control is affected by weed growth’.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-29: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections – RelativeDirectionCode

The items within this code table may be used to indicate the location of the gauging measurement relative to the monitoring point.

Each relative direction code (RelativeDirectionCode) is uniquely identified using a URI. The URI is also a URL providing additional information about the relative direction code. This code table is published at http://www.opengis.net/def/waterml/part2/1.0/RelativeDirectionCode/.

| Label | URI | Description |
| --- | --- | --- |
| Upstream | http://www.opengis.net/def/waterml/part2/1.0/RelativeDirectionCode/upstream | The measurement was performed upstream of the monitoring point. |
| Downstream | http://www.opengis.net/def/waterml/part2/1.0/RelativeDirectionCode/downstream | The measurement was performed downstream of the monitoring point. |

### Code Table D-30: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections –StreamStateCode

The items within this code table may be used to indicate the stream state during the period of observation

Each stream state code (StreamStateCode) is uniquely identified using a URI. The URI is also a URL providing additional information about the stream state code. This code table is published at http://www.opengis.net/def/waterml/part2/1.0/StreamStateCode/.

| Label | URI | Description |
| --- | --- | --- |
| Rising | http://www.opengis.net/def/waterml/part2/1.0/StreamStateCode/Rising | The level of the stream was rising during observation. |
| Falling | http://www.opengis.net/def/waterml/part2/1.0/StreamStateCode/Falling | The level of the stream was falling during observation. |
| Steady | http://www.opengis.net/def/waterml/part2/1.0/StreamStateCode/Steady | The level of the stream was steady during observation. |

### Code Table D-31: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections –TerminationTypeCode

The items within this code table may be used to indicate the way the section observation has been terminated.

| Label | URI | Description |
| --- | --- | --- |

There are no pre-defined entries for this code table.

### Code Table D-32: WMLRGS-XML (WaterML2)  – Ratings, Gaugings and Sections –SectionPropertyCode

The items within this code table may be used to indicate observed properties of section observations.

Each section property code (SectionPropertyCode) is uniquely identified using a URI. The URI is also a URL providing additional information about the section property. This code table is published at http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/.

| Label | URI | Description |
| --- | --- | --- |
| longSection | http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/longSection | An observation of a longitudinal section. |
| gaugingCrossSection | http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/gaugingCrossSection | An observation of a gauging cross section. |
| crossSection | http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/crossSection | A cross section observation. |
| controlCrossSection | http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/controlCrossSection | An observation of a control cross section. |
| bedSlope | http://www.opengis.net/def/waterml/part2/1.0/SectionPropertyCode/bedSlope | An observation of a section of bed slope. |