| World Meteorological Organization COMMISSION FOR BASIC SYSTEMS Inter-Programme Expert Team on Metadata and Data Representation Development Fourth Meeting Geneva, 9 to 12 May 2016 | IPET-MDRD-4/Inf. 4.1(1) |
| --- | --- |
|  | Submitted by: TT-WMD 04.MAY.2016 DRAFT 1 |

AGENDA ITEM 4:	Management of code lists

AGENDA ITEM 4.1:	Consistent management of code lists

The WIGOS metadata standard is being revised to take account of lessons learned since Cg-17 and to add missing sections. The current draft is provided in the body of this document.

The term “observing facility” is being used to replace “station/platform”, but at this version there may still be some instances of the previous term.

Most of the code lists have been updated since the earlier version. These are in the Annex to the standard.

- Code table 01-01 is of particular interest to IPET-MDRD as it is intended to provide a complete list of physical parameters observed by WMO Programmes.

- Although the Fast Track procedures apply to the WIS Discovery Metadata code lists, ICG-WIGOS and CBS-MG did not decide to seek permission for the same flexibility for code lists in the WIGOS metadata standard.

DRAFT INTERIM VERSION OF THE WIGOS METADATA STANDARD 3 MAY 2016

(Attachment to Appendix 2.4 of the Manual on WIGOS)

WIGOS Metadata Standard

Inter-Commission Coordination Group on the WMO Integrated Global Observing System (ICG-WIGOS) Task Team on WIGOS Metadata (TT-WMD)

Membership:

CBS: Karl Monnik, Bureau of Meteorology, Australia (co-chair 2014-)

CAS: Jörg Klausen, Federal Office of Meteorology and Climatology MeteoSwiss, Switzerland (co- chair 2014-)

CIMO: Brian Howe, Environment Canada, Canada (Chair, 2013-2014), Ercan Büyükbas, Turkish State Meteorological Service, Turkey (2014-)

JCOMM: Joe Swaykos, NOAA National Data Buoy Center, United States

CCl: Manuel Bañón Garcia, Antonio Mestre, State Meteorological Agency (AEMET), Spain

CAeM: Stewart Taylor, Met Office, United Kingdom

CHy: Tony Boston, Bureau of Meteorology, Australia

Member: ZHAO Licheng, China Meteorological Administration, China

Associate Member: Tim Oakley (GCOS)

WMO Secretariat

Roger Atkinson, Steve Foreman, Luis Nunes

Interim version

3 May 2016

Version Control

| Version | Date | Who | What |
| --- | --- | --- | --- |
| 0.0.0 | 06-06-2013 | J. Klausen | Consolidate input received from Brian Howe after TT-WMD telecom-2 |
| 0.0 | 06-06-2013 | J. Klausen | Same as v0.0.0 w/o track changes; new definition of 1-04, code list 1-05 |
| 0.0.1 | 10-06-2013 | J. Klausen | Included content for category 4 (environment) |
| 0.0.2 | 30-06-2013 | S Taylor | Included content for category 10 (contact) |
| 0.0.3 | 01-07-2013 | T Boston | Edits to category 7 (station/platform) |
| 0.0.4 | 02-07-2013 | K Monnik |  |
| 0.0.5 | 16-07-2013 | J. Klausen, B. Howe | Version after Telecon-3 |
| 0.0.6 | 18-07-2013 | T. Boston | Edits to category 4 (environment), category 7 (station/platform); code tables 4-02; 7-03 |
| 0.0.7 | 06-08-2013 | J. Klausen | After Telecon-4 |
| 0.0.8 | 02-09-2013 | T. Boston, B. Howe | Edits to topography category 5 and platform/station model corresponding code table. |
| 0.0.9 | 03-09-2013 | J. Klausen | After Telecon-5 |
| 0.0.10 | ?? | ?? | Intermediate version of uncertain origin |
| 0.0.11 | 03-10-2013 | J,.Klausen | After Telecon-6, with expansions not discussed during telecom |
| 0.0.12 | 03-10-2013 | B. Howe | After Telecon-6 with changes accepted. |
| 0.0.13 | 24-10-2013 | B. Howe | After Telecon-7 |
| 0.0.13.ra | 31-10-2013 | R. Atkinson | Responses to a number of comments in 0.0.13 |
| 0.0.13.ra+km | 04-11-2013 | K. Monnik | General edits, additions to Cat 8, added examples to Cat 1, 5, 7. |
| 0.0.14 | 04-11-2013 | J. Klausen | After Telecon-8 |
| 0.0.14 km | 06-11-2013 | K. Monnik | Minor changes to 6.06, 8.03, 8.10, plus selected comments from Blair Trewin (AU) |
| 0.0.15 | 11-11-2013 | J. Klausen | After Telecon-9, and including feed-back from P. Pilon/R. Atkinson |
| 0.0.16 |  |  | After Telecon-10 |
| 0.0.17 | 19-12-2013 | J. Klausen | After Telecon-11 |
| 0.0.18 | 06-02-2014 | J. Klausen, K Monnik | Response to Wiel Wauben, Bruce Forgan; version after Telecon-12, with further additions and edits, formatting |
| 0.0.19 |  |  |  |
| 0.0.20 | 12-03-2014 18-03-2014 | B. Howe J. Klausen | After Telecon-15, accepted ICG-WIGOS MCO classifications and added two requested fields. Numerous other updates accepted. Comments by ET-SUP carried over. |
| 0.0.21 | 27-03-2014 | J. Klausen | Element 5-04 (Reporting interval (space)) explicitly listed; code table 5-05 included; element 5-11 (reference time) defined and explained; numbering in list of category 5 corrected; Figures 1 and 2 updated |
| 0.0.22 | 03-04-2014 | J. Klausen | After Telecon-16 |
| 0.0.23 | 28-04-2014 | J. Klausen | After Telecon-17, several changes accepted, minor editing, fixed a few cross-references |
| 0.1 | 15-05-2014 | J. Klausen | Version after TT-WMD-2; dropped notion of “Core” in favor of a phased implementation; added element 8-00; dropped 4-04; moved element 8-05 to become 4-04; editorial improvements |
| 0.1.01 | 19-05-2014 | WIGOS PO | Editorial |
| 0.1.02 | 03-07-2014 | WIGOS PO | Review with comments and proposed changes |
| 0.1.03 | 10-07-2014 | TT-WMD | WebEx Sessions (03rd and 10th July 2014) |
| 0.1.04 | 25-11-2014 | J. Klausen | Includes several corrections and changes as they accumulated. |
| 0.1.05 | 04-12-2014 | TT-WMD-3 | As of the end of TT-WMD 3rd session |
| 0.1.06a | 17-12-2014 | L. Nunes | Intermediate editing version |
| 0.1.06b | 20-01-2015 | L. Nunes | Second intermediate editing version |
| 0.1.06 | 26-01-2015 | Co-chairs and Secretariat | WebEx session on 20150123, plus Secretariat edits on 20150126 |
| 0.2 | 27-01-2015 | TT-WMD | WebEx, plus co-chairs and Secretariat edits on 20150128 |
| 1.0 | May 2015 | Secretariat | Minor edits for approval by Cg-17 |
| 1.0a | 16-10-2015 | Secretariat | Proposed changes with comments, including those from the workshop on WIGOS metadata for surface-based observations |
| 1.01 | 03-05-2016 | Secretariat | Interim version to accommodate proposals from various sources, including outcomes of the Workshop on MD for satellites (2015) |

Table of Contents

I - Purpose and Scope of WIGOS Metadata	5

II - WIGOS Metadata Categories	6

III - A Note on Space and Time	12

IV - Reporting Obligations for WIGOS Metadata	14

V - Technical Implementation and Use of Standard	15

VI - Adoption through a Phased Approach	17

VII – Detailed specification of WIGOS metadata elements	19

Category 1: Observed variable	20

Category 2: Purpose of Observation	24

Category 3: Observing facility	25

Category 4: Environment	28

Category 5: Instruments and Methods of Observation	30

Category 6: Sampling	35

Category 7: Data Processing and Reporting	38

Category 8: Data Quality	43

Category 9: Ownership & Data Policy	45

Category 10: Contact	46

VIII - References	47

ANNEX – Code Tables	49

List of Acronyms (to be completed):

WIGOS - WMO Integrated Global Observing System

WIS - WMO Information System

ICG-WIGOS - Inter-Commission Coordination Group on WIGOS

CAS – WMO Technical Commission for Atmospheric Sciences

CBS – WMO Technical Commission for Basic Systems

CIMO – WMO Technical Commission for Instruments and Methods of Observations

CCl – WMO Technical Commission for Climatology

CHy – WMO Technical Commission for Hydrology

CAeM – WMO Technical Commission for Aeronautical Meteorology

JCOMM – Joint WMO-IOC Commission for Oceanography and Marine Meteorology

GCOS – Global Climate Observing System

UML – Unified Modelling Language

RRR - Rolling Review of Requirements

VIM - International Vocabulary of Metrology

AWS - Automatic Weather Station

AMDAR - Aircraft Meteorological Data Relay

XML - Extensible Markup Language

OSCAR - Observing Systems Capability Analysis and Review tool

WIR - WIGOS Information Resource

JCGM - Joint Committee for Guides in Metrology

ISO - International Organization for Standardization

# I - Purpose and Scope of WIGOS Metadata

An important aspect of WIGOS (WMO Integrated Global Observing System) implementation is ensuring maximum usefulness of WIGOS observations. Observations without metadata are of very limited use: it is only when accompanied by adequate metadata (data describing the data) that the full potential of the observations can be utilized. Metadata of two complementary types are required. The first of these is discovery metadata – information that facilitates data discovery, access and retrieval. These metadata are WIS (WMO Information System) metadata and are specified and handled as part of WIS. The second type is interpretation/description or observational metadata – information that enables data values to be interpreted in context. These latter metadata are WIGOS metadata and are the subject of this standard, which provides a WIGOS standard for the interpretation metadata required for the effective utilization of observations from all WIGOS component observing systems by all users.

WIGOS metadata should describe the observed variable, the conditions under which it was observed, how it was measured, and how the data has been processed, in order to provide data users with confidence that the use of the data is appropriate for their application. GCOS (Global Climate Observing System) Climate Monitoring Principle #3 describes the relevance of metadata as:

“The details and history of local conditions, instruments, operating procedures, data processing algorithms and other factors pertinent to interpreting data (i.e., metadata) should be documented and treated with the same care as the data themselves.”

WIGOS observations consist of an exceedingly wide range of data from the manual observations to complex combinations of satellite hyper-spectral frequency bands, measured in situ or remotely, from single dimension to multiple dimensions, and those involving processing. A comprehensive metadata standard to cover all types of observations is by nature complex to define. A user should be able to use the WIGOS metadata to identify the conditions under which the observation (or measurement) was made, and any aspects which may affect its use or understanding, i.e. to determine whether the observations are fit for the purpose.

# II - WIGOS Metadata Categories

Ten categories of WIGOS metadata have been identified. These are listed in Table 1 below. They define the WIGOS metadata standard, each category consisting of one or more metadata elements. All of the categories listed are considered to be important for the documentation and interpretation of observations made, and even for their use in the distant future. Hence, the standard currently declares many elements that are clearly not needed for applications focusing on more immediate use of observations. For these applications, such as numerical weather prediction, aeronautical or other transport sector applications, advisories, etc., profiles of the standard may be developed. The categories are in no particular order but reflect the need to specify the observed variable; to answer why, where and how the observation was made; how the raw data were processed; and what the quality of the observation is.

A schematic composition of all categories, containing the individual elements is shown in Figure 1. Note that some of these elements will most likely be implemented using several individual entities (e.g., geospatial location will consist of the a combination of elements, such as latitude, longitude, elevation or a set of polar coordinates, as well as a reference to the geo-positioning methods used). Chapter VII contains a set of tables detailing all the elements, including definition, notes/examples, and obligations/implementation phase. Code tables enabling users to select from pre-defined vocabularies to facilitate the application of the WIGOS metadata standard and the exchange of metadata are presented in Annex I.

Table 1. WIGOS Metadata Categories

| # | Category | Description |
| --- | --- | --- |
| 1 | Observed variable | Specifies the basic characteristics of the observed variable and the resulting datasets. |
| 2 | Purpose of observation | Specifies the main application area(s) of the observation and the observing programme(s) and networks the observation is affiliated to. |
| 3 | Observing facility | Specifies the environmental monitoring facility, including fixed station, moving equipment or remote sensing platform, at which the observation is made. |
| 4 | Environment | Describes the geographical environment within which the observation is made. It also provides an unstructured element for additional meta-information that is considered relevant for adequate use of the data and that is not captured anywhere else in this standard. |
| 5 | Instruments and methods of observation | Specifies the method of observation and describes characteristics of the instrument(s) used to make the observation. If multiple instruments are used to generate the observation, then this category should be repeated. |
| 6 | Sampling | Specifies how sampling and/or analysis are used to derive the reported observation or how a specimen is collected. |
| 7 | Data processing and reporting | Specifies how raw data are transferred into the observed variable and reported to the users. |
| 8 | Data quality | Specifies the data quality and traceability of the observation. |
| 9 | Ownership and data policy | Specifies who is responsible for the observation and owns it. |
| 10 | Contact | Specifies where information about the observation or dataset can be obtained. |

For example, an observation/dataset may have the following metadata categories associated with it:

•	One or several purpose(s) of observation:

•	Data processing procedures associated with the instruments;

•	Instruments which have been used to make the observation;

•	A observing facility to which the instrument(s) belong(s);

•	Ownership and data policy restriction;

•	Contact.

An instrument output may contribute to observations of one or more variables. For example:

•	A four wire humidity probe can produce temperature and humidity, as well as dew point;

•	A sonic anemometer does report wind speed, wind direction and can report air temperature;

•	A spectrometer can report absorption due to many different chemical species.

An instrument typically will be associated with the categories:

•	Instruments and methods of observation;

•	Sampling (e.g. 10 Hz samples of air temperature);

•	Data processing and reporting (e.g. ceilometer reporting of 10 min statistics of cloud height following processing through sky condition algorithm).

An observed variable may be influenced or characterized by the environment, for example:

•	Wind speed (observed variable) on top of a hill (environment);

•	River yield (observed variable) characterized by the upstream catchment and land use.

Figure 1. UML diagram specifying the WIGOS Metadata Standard (**: code tables expected; [0..1*]: optional or conditional elements. Conditional elements become mandatory if a given condition is met. Conditions are referenced in parentheses. Optional elements may be declared mandatory as part of profiling the standard for specific application areas; [1]: mandatory elements. These elements must be reported, and if no value is available, a nilReason must be reported, which indicates that the metadata is “unknown”, or “not available”)

Table 2. Names and Definition of Elements

An asterisk (*) denotes the element is required for the WMO Rolling Review of Requirements (RRR) process. A hash sign (#) denotes that it is acceptable to record a "mandatory" element with a value of nilReason (that indicates that the metadata is either “unknown”, or “not applicable”, or “not available”). M = Mandatory, C = Conditional, O = Optional.

| Category | Id | Name | Definition | MCO |  |  | Phase |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Observed variable | 1-01 | Observed variable – measurand | Variable intended to be measured or observed or derived, including the biogeophysical context | M* |  |  | 1 |  |
|  | 1-02 | Measurement unit | Real scalar quantity, defined and adopted by convention, with which any other quantity of the same kind can be compared to express the ratio of the two quantities as a number [VIM3, 1.9] | C* |  |  | 1 |  |
|  | 1-03 | Temporal extent | Time period covered by a series of observations inclusive of the specified date-time indications (measurement history) | M* |  |  | 1 |  |
|  | 1-04 | Spatial extent | Typical georeferenced volume covered by the observations | M* |  |  | 1 |  |
|  | 1-05 | Representativeness | Spatial extent of the region around the observation for which it is representative | O |  |  | 2 |  |
| Purpose of observ. | 2-01 | Application area(s) | Context within, or intended application(s) for which the observation is primarily made or which has/have the most stringent requirements | M* |  |  | 1 |  |
|  | 2-02 | Programme/Network affiliation | The global, regional or national Programmes/network(s) that the observing facility is associated with | M |  |  | 1 |  |
| Station/platformObserving facility | 3-01 | Region of origin of data | WMO Region |  | C* | 1 |  |
|  | 3-02 | Territory of origin of data | Country or territory name of the location of the observation |  | C* | 1 |  |
|  | 3-03 | Station/platformObserving facility name | Official name of the observing facility |  | M | 1 |  |
|  | 3-04 | Station/platformObserving facility type | A categorization of the type of environmental monitoring facility at which an observation is made |  | M* | 2 |  |
|  | 3-05 | Station/platformObserving facility model | The model of the monitoring equipment used at the observing facility |  | M*# | 3 |  |
|  | 3-06 | Station/platformObserving facility unique identifier | A unique and persistent identifier for an observing facility, which may be used as an external point of reference |  | M* | 1 |  |
|  | 3-07 | Geospatial location | Position in space defining the location of the environmental monitoring observing facility at the time of observation |  | M* | 1 |  |
|  | 3-08 | Data communication method | Data communication method between the observing facility and some central facility |  | O | 2 |  |
|  | 3-09 | Station Operating Status | Declared reporting status of the station |  | M | 1 |  |
| Environment | 4-01 | Surface cover | The observed (bio)physical cover on the earth’s surface in the vicinity of the observation |  | C | 3 |  |
|  | 4-02 | Surface cover classification scheme | Name and reference or link to document describing the classification scheme |  | C | 3 |  |
|  | 4-03 | Topography or bathymetry | The shape or configuration of a geographical feature, represented on a map by contour lines |  | C | 3 |  |
|  |  | Surface roughness |  |  | O | 3 |  |
|  | 4-04 | Events at observing facility | Description of human action or natural event at the station or at the vicinity that may influence the observation |  | O | 2 |  |
|  |  | Climate zone |  |  | O | 3 |  |
|  | 4-05 | Site information | Non-formalized information about the location and its surroundings at which an observation is made and that may influence it |  | O | 2 |  |

| Instruments and methods of observation | 5-01 | Source of observation | The source of the dataset described by the metadata | M | 1 |
| --- | --- | --- | --- | --- | --- |
|  | 5-02 | Measurement/observing method | The method of measurement/observation used | M# | 1 |
|  | 5-03 | Instrument specifications | Intrinsic capability of the measurement/observing method to measure the designated element, including range, stability, precision, etc | MC*# | 1 |
|  | 5-04 | Instrument operating status | The status of an instrument with respect to its operation | O | 3 |
|  | 5-05 | Vertical distance of sensor | Vertical distance of the sensor from a (specified) reference level such as local ground, or deck of a marine platform at the point where the sensor is located; or sea surface | C* | 1 |
|  | 5-06 | Configuration of instrumentation | Description of any shielding or configuration/setup of the instrumentation or auxiliary equipment needed to make the observation or to reduce the impact of extraneous influences on the observation | C# | 3 |
|  | 5-07 | Instrument control schedule | Description of schedule for calibrations or verification of instrument | C | 3 |
|  | 5-08 | Instrument control result | The result of an instrument control check, including date, time, location, standard type and period of validity | C# | 3 |
|  | 5-09 | Instrument model and serial number | Details of manufacturer, model number, serial number and firmware version if applicable | C# | 3 |
|  | 5-10 | Instrument routine maintenance | A description of maintenance that is routinely performed on an instrument | C# | 3 |
|  | 5-11 | Maintenance party | Identifier of the organization or individual who performed the maintenance activity | O | 2 |
|  | 5-12 | Geospatial location | Geospatial location of instrument/sensor | C* | 2 |
|  | 5-13 | Maintenance Activity | Description of maintenance performed on instrument | O | 3 |
|  | 5-14 | Status of observation | Official status of observation | O | 3 |
|  | 5-15 | Exposure of instrument | The degree to which an instrument is affected by external influences and reflects the value of the observed variable | C | 2 |
| Sampling | 6-01 | Sampling procedures | Procedures involved in obtaining a sample | O | 3 |
|  | 6-02 | Sample treatment | Chemical or physical treatment of sample prior to analysis | O | 3 |
|  | 6-03 | Sampling strategy | The strategy used to generate the observed variable | O* | 1 |
|  | 6-04 | Sampling time period | The period of time over which a measurement is taken | M# | 3 |
|  | 6-05 | Spatial sampling resolution | Spatial resolution refers to the size of the smallest observable object. The intrinsic resolution of an imaging system is determined primarily by the instantaneous field of view of the sensor, which is a measure of the ground area viewed by a single detector element in a given instance in time | M# | 2 |
|  | 6-06 | Temporal sampling interval | Time period between the beginning of consecutive sampling periods | M | 3 |
|  | 6-07 | Diurnal base time | Time to which diurnal statistics are referenced | MC | 1 |
|  | 6-08 | Schedule of observation | Schedule of observation | M | 1 |

| Data processing and Reporting | 7-01 | Data processing methods and algorithms | A description of the processing used to generate the observation and list of algorithms utilized to derive the resultant value | O | 3 |
| --- | --- | --- | --- | --- | --- |
|  | 7-02 | Processing/analysis center | Center at which the observation is processed | O | 2 |
|  | 7-03 | Temporal reporting period | Time period over which the observable variable is reported | M* | 1 |
|  | 7-04 | Spatial reporting interval | Spatial interval at which the observed variable is reported | C* | 1 |
|  | 7-05 | Software/processor and version | Name and version of the software or processor utilized to derive the element value | O | 3 |
|  | 7-06 | Level of data | Level of data processing | O | 2 |
|  | 7-07 | Data format | Description of the format in which the observed variable is being provided | M | 3 |
|  | 7-08 | Version of data format | Version of the data format in which the observed variable is being provided | M | 3 |
|  | 7-09 | Aggregation period | Time period over which individual samples/observations are aggregated | M | 2 |
|  | 7-10 | Reference time | Time base to which date and time stamps refer | M | 2 |
|  | 7-11 | Reference datum | Reference datum used to convert observed quantity to reported quantity | C | 1 |
|  | 7-12 | Numerical resolution | Measure of the detail in which a numerical quantity is expressed | O | 3 |
|  | 7-13 | Latency (of reporting) | The typical time between completion of the observation or collection of the datum and when the datum is reported | M | 3 |
| Data quality | 8-01 | Uncertainty of measurement | Non-negative parameter, associated with the result of a measurement, that characterizes the dispersion of the values that could reasonably be attributed to the observation/measurand | C*# | 2 |
|  | 8-02 | Procedure used to estimate uncertainty | A reference or link pointing to a document describing the procedures / algorithms used to derive the uncertainty statement | C*# | 2 |
|  | 8-03 | Quality flag | An ordered list of qualifiers indicating the result of a quality control process applied to the observation | M# | 2 |
|  | 8-04 | Quality flagging system | Reference to the system used to flag the quality of the observation | M# | 2 |
|  | 8-05 | Traceability | Statement defining traceability to a standard, including sequence of measurement standards and calibrations that is used to relate a measurement result to a reference [VIM 3 2.4.2] | C*# | 2 |
| Ownership and data policy | 9-01 | Supervising organization | Name of organization who owns the observation | M | 2 |
|  | 9-02 | Data policy/use constraints | Details relating to the use and limitations surrounding data imposed by the supervising organization | M* | 1 |
| Contact | 10-01 | Contact (Nominated Focal Point) | Principal contact (Nominated Focal Point, FP) for resource | M | 1 |

# III - A Note on Space and Time

It is important to understand that WIGOS metadata are intended to describe an individual observation or a dataset, i.e. one or several observations, including the where, when, how, and even why the observations were made. As a consequence, references to space and time are made in several places throughout the standard.

Figure 2 illustrates the concepts and terms used to describe the temporal aspects of an observation or dataset, including sampling strategy, analysis, data processing and reporting.

The concepts and terms used to describe spatial aspects (i.e., geospatial location) of observations are even more complex (cf. Fig. 3). For example, for ground-based in-situ observations, the spatial extent of the observation coincides with the geospatial location of the sensor, which in most cases will be time-invariant and is normally close to the geospatial location of the observing facility (station/platform) where the observation was made. For a satellite-based lidar system, the situation is quite different. Depending on the granularity of metadata desired, the spatial extent of the individual observation may be an individual pixel in space, the straight line probed during an individual laser pulse, or perhaps an entire swath. In any case, the spatial extent of the observation will not coincide with the location of the sensor. The WIGOS metadata standard therefore needs to take into account such elements as:

1.	The spatial extent of the observed variable (e.g. atmospheric column above a Dobson Spectrophotometer) (cf. 1-04)

2.	The geospatial location of the observing facility (e.g. radar transmitter/receiver or aircraft position/route) (cf. 3-07)

3.	The geospatial location of the instrument (e.g. the anemometer is adjacent to a runway) (cf. 5-05 Vertical Distance and 5-12 geospatial location)

4.	The spatial representativeness of the observation (cf. 1-05)

All these are expressed in terms of geospatial location, specifying either a zero-dimensional geographic extent (a point), a one-dimensional geographic extent (a line, either straight or curved), a two-dimensional geographic extent (a plane or other surface), or a three-dimensional geographic extent (a volume).

A station/platform can be:

1.	Collocated with the observed quantity as for in situ surface observing station (e.g. an Automatic Weather Station - AWS)

2.	Collocated with the instrument but remote to the observed quantity (e.g. radar)

3.	Remote from where the instrument may transmit data to the station (e.g. airport surface station where instruments are located across the airport, or a balloon atmosphere profiling station)

4.	In motion and travelling through the observed medium (e.g. AMDAR - Aircraft Meteorological Data Relay - equipped aircraft)

5.	In motion and remote to the observed medium (e.g. satellite platform)

An instrument can be:

1.	Collocated with the observed variable (e.g. surface temperature sensor)

2.	Remote to the observed variable (e.g. radar transmitter/receiver)

3.	In motion but located in the observed medium (e.g. radiosonde)

4.	In motion and remote from the observed quantity (e.g. satellite based radiometer)

5.	Located within a standardized enclosure (e.g. a temperature sensor within a Stevenson screen)

Figure 2. Graphical representation of temporal elements referenced in WIGOS Metadata categories

Figure 3. Graphical representation of spatial elements referenced in WIGOS Metadata categories

# IV - Reporting Obligations for WIGOS Metadata

According with the International Organization for Standardization (ISO), the metadata elements are classified as either mandatory (M), conditional (C), or optional (O).

Mandatory metadata elements shall always be made available. The content of the corresponding fields shall never be empty, either the metadata “value” or the reason for no-value, shall be made available.

Most of the elements in this standard are considered mandatory in view of enabling adequate future use of observations by all WMO Application Areas. Metadata providers are expected to report mandatory metadata elements, and a formal validation of a metadata record will fail if mandatory elements are not reported. If Members cannot provide all the Mandatory elements the reason for that shall be reported as “not applicable” or “unknown” or “not available”. The motivation for this is that knowledge of the reason why a mandatory metadata element is not available provides more information than not reporting a mandatory element at all. In the tables of chapter VII, these cases are indicated with M#.

Conditional metadata elements shall be made available when the specified condition or conditions are met, in which case the content of the corresponding fields shall never be empty, either the metadata “value” or the reason for no-value, shall be made available. For example, the element “Spatial reporting interval” is classified as conditional, because it only applies to remote sensing observations and mobile platforms. Therefore, the elements in this category should be considered mandatory for remote sensing and mobile observing systems but not so for e.g., surface land stations.

Optional metadata elements should also be made available. They provide useful information that can help to better understand an observation. In this version of the standard, very few elements are considered optional. Optional elements are likely to be important for a particular community, but less so for others.

# V - Technical Implementation and Use of Standard

This document is a semantic standard that specifies the elements that exist and that can be recorded and reported. It does not specify how the information shall be encoded or exchanged. However, the following are likely scenarios and important aspects that may help the reader appreciate what lies ahead.

1.	The most likely implementation will be in XML (Extensible Markup Language), in line with the specifications for WIS metadata and common interoperability standards. Regardless of the final implementation, the full metadata record describing a dataset can be envisioned as a tree with the categories as branches off the stem, and the individual elements as leaves on these branches. Some branches may occur more than once, e.g., a dataset may have been generated using more than one instrument at once, in which case two branches for ‘instrument’ may be required.

2.	Not all of the elements specified in this document need to be updated at the same frequency. Some elements, such as position of a land-based station are more or less time-invariant, while others, such as a specific sensor, may change regularly every year. Still other elements, such as environment, may change gradually or rarely, but perhaps abruptly. Finally, elements restricting the application of an observation, e.g., to road condition forecasting, may have to be transmitted with every observation. The implementation of the WIGOS metadata needs to be able to deal with this.

3.	Not all applications of observations require the full suite of metadata as specified in this standard at any given time. The amount of metadata that needs to be provided to be able to make adequate use of an observation, for example for the purpose of issuing a heavy precipitation warning, is much less than for the adequate use of even the same observation for a climatological analysis. On the other hand, the metadata needed for near-real-time applications may also need to be provided in near-real-time. This is important to realize, as it makes the task of providing WIGOS metadata much more tractable. The implementation of WIGOS metadata needs to be able to cope with vastly different update intervals, and incremental submission of additional metadata to allow the creation of ‘complete’ metadata records.

4.	Users will want to obtain and filter datasets according to certain criteria/properties as described within each WIGOS metadata record. This functionality requires either a central repository for WIGOS metadata or full interoperability of the archives collecting WIGOS metadata.

How, then can these requirements be met? In the case where observations are clearly only used for some near-real-time application and there is clearly no long-term use or re-analysis application to be expected, a profile of the WIGOS metadata standard may be specified that declares a specific subset of metadata elements as mandatory. This is depicted schematically in Figure 4.

Figure 4. Schematic of the relationship of WIS and WIGOS metadata and the scope of the ISO19115 standard. The WMO Core is a profile of ISO19115. WIGOS metadata exceed the scope of ISO19115 standard. A possible profile (subset) of WIGOS metadata elements for some specific near-real-time application is also shown.

Importantly, all WIGOS metadata elements (or group of elements) will have to be time-stamped with the time of validity and associated to a unique identifier for a dataset during transmission and for archiving. The specification of time stamps should also include a statement on the use of daylight savings time. Using this approach, increments of a ‘full’ WIGOS metadata record can be transmitted anytime changes occur and updates are deemed necessary. At the archive, the increments can be added to the existing metadata record for that dataset, establishing the full history of a particular observation with time.

# VI - Adoption through a Phased Approach

Making available WIGOS metadata will generate substantial benefits for Members, but developing the capacity to make available these metadata also requires a substantial effort on the part of (meta)data providers. To help Members comply with obligations, guidance material will be developed and provided.

Moreover, obligations will be enforced in phases in order to allow Members sufficient time to develop the capacity to comply. Balancing the effort required to generate and make available the metadata elements, and the need to have this information to make adequate use of observations, implementation will proceed through three phases as shown in Table 3. Importantly, elements required by the end of Phase I are either listed as mandatory elements in WMO-No. 9, Vol. A or are of critical importance for the Observing Systems Capability Analysis and Review (OSCAR) tool of the WIR (WIGOS Information Resource), and are considered of benefit for all application areas. Phase II adds elements recognized to be more challenging for Members, but the knowledge of which is still of rather immediate need for the adequate use of observations, in particular for assessing quality of observations. Phase III adds the remaining elements specified in this version of the standard.

Elements emerging as being important for specific application areas or observing programmes will be added to the standard as it evolves.

| Category | Phase I | Phase II | Phase III |
| --- | --- | --- | --- |
|  | 2016 | 2017-2018 | 2019-2020 |
| 1. Observed variable | 1-01 Observed variable – measurand (M) | 1-05 Representativeness (O) |  |
|  | 1-02 Measurement unit (C) |  |  |
|  | 1-03 Temporal extent (M) |  |  |
|  | 1-04 Spatial extent (M) |  |  |
| 2. Purpose of observation | 2-01 Application area(s) (M) |  |  |
|  | 2-02 Programmes/Network affiliation (M) |  |  |
| 3. Observing facility | 3-01 Region of origin of data (C) | 3-04 Observing facility type (M) | 3-05 Observing facility model (M) |
|  | 3-02 Territory of origin of data (C) | 3-08 Data communication method (O) |  |
|  | 3-03 Observing facility name (M) |  |  |
|  | 3-06 Observing facility unique identifier (M) |  |  |
|  | 3-07 Geospatial location (M) |  |  |
|  | 3-09 Station operating status (M) |  |  |
| 4. Environment |  | 4-04 Events at Observing facility (O) | 4-01 Surface cover (C) |
|  |  | 4-05 Site information (O) | 4-02 Surface Cover classification scheme (C) |
|  |  |  | 4-03 Topography or Bathymetry (C) |
|  |  |  | Surface roughness (O) |
|  |  |  | Climate zone (O) |

| 5. Instruments and Methods of Observation | 5-01 Source of observation (M) | 5-11 Maintenance party (O) | 5-04 Instrument operating status (O) |
| --- | --- | --- | --- |
|  | 5-02 Measurement/observing method (M) | 5-12 Geospatial location (C) | 5-06 Configuration of instrumentation (C) |
|  | 5-03 Instrument specifications (C) | 5-15 Exposure of instrument (C) | 5-07 Instrument control schedule (C) |
|  | 5-05 Vertical distance of sensor (C) |  | 5-08 Instrument control result (C) |
|  |  |  | 5-09 Instrument model and serial number (C) |
|  |  |  | 5-10 Instrument routine maintenance (C) |
|  |  |  | 5-13 Maintenance Activity (O) |
|  |  |  | 5-14 Status of observation (O) |
| 6. Sampling | 6-03 Sampling strategy (O) | 6-05 Spatial sampling resolution (M) | 6-01 Sampling procedures (O) |
|  | 6-07 Diurnal base time (C) |  | 6-02 Sample treatment (O) |
|  | 6-08 Schedule of observation (M) |  | 6-04 Sampling time period (M) |
|  |  |  | 6-06 Temporal sampling interval (M) |
| 7. Data Processing and Reporting | 7-03 Temporal reporting period (M) | 7-02 Processing/analysis center (O) | 7-01 Data processing methods and algorithms (O) |
|  | 7-04 Spatial reporting interval (C) | 7-06 Level of data (O) | 7-05 Software/processor and version (O) |
|  | 7-11 Reference datum (C) | 7-09 Aggregation period (M) | 7-07 Data format (M) |
|  |  | 7-10 Reference time (M) | 7-08 Version of data format (M) |
|  |  |  | 7-12 Numerical resolution (O) |
|  |  |  | 7-13 Latency (of reporting) (M) |
| 8. Data Quality |  | 8-01 Uncertainty of measurement (C) |  |
|  |  | 8-02 Procedure used to estimate uncertainty (C) |  |
|  |  | 8-03 Quality flag (M) |  |
|  |  | 8-04 Quality flagging system (M) |  |
|  |  | 8-05 Traceability (C) |  |
| 9. Ownership and Data Policy | 9-02 Data policy/use constraints (M) | 9-01 Supervising organization (M) |  |
| 10. Contact | 10-01 Contact (Nominated Focal Point) (M) |  |  |

Table 3. List of elements specified in the WIGOS Metadata Standard and the phases for implementation by Members.

# VII – Detailed specification of WIGOS metadata elements

# Category 1: Observed variable

Specifies the basic characteristics of the observed variable and the resulting datasets. It includes an element describing the spatial representativeness of the observation as well as the biogeophysical compartment the observation describe.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 1-01 | Observed variable (measurand) | Variable intended to be measured or observed or derived, including the biogeophysical context | [ISO19156] NOTE 1: In conventional measurement theory the term “measurement” is used. However, a distinction between measurement and category-observation has been adopted in more recent work so the term “observation” is used for the general concept. “Measurement” may be reserved for cases where the result is a numeric quantity. NOTE 2: The biogeophysical context is expressed in terms of Domain, Subdomain/Matrix, and Layer, and variables are organized hierarchically using these dimensions. Relevant domains, matrices and layers include atmosphere, aerosol, lake, river, ocean, soil, cloud water, aerosol particulate phase, land surface, troposphere, upper troposphere/lower stratosphere, space, etc. EXAMPLES: In hydrology, this would typically be stage or discharge. Present weather; Air temperature near the surface; CO2 mixing ratio in the atmosphere | 1-01 | M* (Phase 1) |
| 1-02 | Measurement unit | Real scalar quantity, defined and adopted by convention, with which any other quantity of the same kind can be compared to express the ratio of the two quantities as a number [VIM3, 1.9] | [JCGM 200:2012, 1.9] NOTE 1. Measurement units are designated by conventionally assigned names and symbols. [JCGM 200:2012, 1.9] NOTE 2 Measurement units of quantities of the same quantity dimension may be designated by the same name and symbol even when the quantities are not of the same kind. For example, joule per kelvin and J/K are respectively the name and symbol of both a measurement unit of heat capacity and a measurement unit of entropy, which are generally not considered to be quantities of the same kind. However, in some cases special measurement unit names are restricted to be used with quantities of a specific kind only. For example, the measurement unit ‘second to the power minus one’ (1/s) is called hertz (Hz) when used for frequencies and becquerel (Bq) when used for activities of radionuclides. [JCGM 200:2012, 1.9] NOTE 3 Measurement units of quantities of dimension one are numbers. In some cases these measurement units are given special names, e.g. radian, steradian, and decibel, or are expressed by quotients such as millimole per mole equal to 10-3 and microgram per kilogram equal to 10-9. [JCGM 200:2012, 1.9] NOTE 4 For a given quantity, the short term “unit” is often combined with the quantity name, such as “mass unit” or “unit of mass”. EXAMPLE In hydrology, this would typically be m for stage or m3/s for discharge. | 1-02 | C* (Phase 1) |
| 1-03 | Temporal extent | Time period covered by a series of observations inclusive of the specified date-time indications (measurement history) | NOTE 1: The Temporal Extent is defined through the begin and end dates of observations. NOTE 2: If the data are still being added to, omit the End date (but specify a Begin date). NOTE 3: If there are gaps in the data collection (e.g. 1950-1955 then collection resumes 1960-present) then the first date recorded should be the earliest date and the last the most recent, ignoring the gap. EXAMPLES: Surface temperature at the station Säntis has been observed since 1 September 1882. The CO2 record at Mauna Loa extends from 1958 to today. Continuous, 1-hourly aggregates are available from the World Data Centre for Greenhouse Gases for the period 1974-01-01 to 2011-12-31 |  | M* (Phase 1) |
| 1-04 | Spatial extent | Typical spatial georeferenced volume covered by the observations | NOTE 1: The spatial extent of an observed quantity can be a zero-, one-, two-, or three-dimensional feature and will be expressed in terms of a series of geospatial locations describing a geometric shape; in the case of space-based observations the spatial coverage should be stated in terms of global (e.g. for LEO, characteristics like swathe width and repeat cycle), disk (GEO), vertical (soundings), etc. NOTE 2: A zero-dimensional geospatial location of an observation implies either an in-situ (point) observation or, by convention, a column-averaged quantity above the specified geospatial location in nadir. One-dimensional geospatial location of an observation implies a distribution / profile of a quantity along a trajectory (e.g., a straight line from the ground up with a given zenith angle). A two-dimensional geospatial location of an observation implies an area or hyper-surface (e.g., a radar image, or a satellite pixel of a property near the surface). A three-dimensional geospatial location of an observation implies a volume-averaged quantity (e.g., a radar pixel in 3D-space). EXAMPLES: i) Air temperature at a surface observing site: Sydney Airport NSW: Lat. -33.9465 N; Lon. 151.1731 E, Alt: 6.0 m above msl. ii) The projected area or volume of the cone around a particular weather radar with a maximum range of 370 km (radar reflectivity) and 150 km (Doppler); to be expressed as a geometric shape. iii) 3-dimensional grid of radar pixels iv) For Infrared and visible imagery by meteorological satellite (sunsynchronous): Global coverage v) For nadir sounding: atmospheric column above ocean vi) NASA Aura satellite (705 km altitude) has a 16-day (233 orbit) repeat cycle vii) River discharge by gauge: size and geometric shape of a river Catchment. | Free text, that could be complemented by one or more URLs | M* (Phase 1) |
| 1-05 | Representativeness | Spatial extent of the region around the observation for which it is representative | NOTE: The representativeness of an observation is the degree to which it describes the value of the variable needed for a specific purpose. Therefore, it is not a fixed quality of any observation, but results from joint appraisal of instrumentation, measurement interval and exposure against the requirements of some particular application (WMO-No. 8, 2008). Representativeness of an observed value describes the concept that the result of an observation made at a given geospatial location would be compatible with the result of other observations of the same quantity at other geospatial locations. In statistics, the term describes the notion that a sample of a population allows an adequate description of the whole population. Assessing representativeness can only be accomplished in the context of the question the data [or observations] are supposed to address. In the simplest terms, if the data [or observations] can answer the question, it is representative (Ramsey and Hewitt, 2005). The representativeness of an environmental observation depends on the spatio-temporal dynamics of the observed quantity (Henne et al., 2010). Representativeness of an observation can sometimes be specified quantitatively, in most cases qualitatively, based on experience or heuristic arguments. | 1-05 | O (Phase 2) |

Condition:

{1-02} variables that are measured, rather than classified

# Category 2: Purpose of Observation

Specifies the main application area(s) of the observation and the observing programme(s) and networks the observation is affiliated to.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 2-01 | Application area(s) | Context within, or intended application(s) for which the observation is primarily made or which has/have the most stringent requirements | NOTE: Many observations serve more than one purpose, meeting the requirements of various applications areas. In such cases, the application area for which the station or platform was originally established should be listed first. | 2-01 | M* (Phase 1) |
| 2-02 | Program/Network affiliation | The global, regional or national program/network(s) that the observing facility is associated with | EXAMPLES: GUAN, AMDAR, GAW, RBSN, WHOS, etc. (full names to be referenced in code table) | 2-02 | M (Phase 1) |

# Category 3: Observing facility

Specifies the environmental monitoring facility, including fixed station, moving equipment or remote sensing platform at which the observation is made.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 3-01 | Region of origin of data | WMO Region | NOTE: WMO divides Member countries into six Regional Associations responsible for coordination of meteorological, hydrological and related activities within their respective Regions. | 3-01 | C* (Phase 1) |
| 3-02 | Territory of origin of data | Country or territory name of the location of the observation | NOTE: Mandatory for fixed stations, optional for mobile stations EXAMPLE: Australia. | 3-02 | C* (Phase 1) |
| 3-03 | Station/platformObserving facility name | Official name of the observing facility | EXAMPLES: Mauna Loa, South Pole |  | M (Phase 1) |
| 3-04 | Station/platformObserving facility type | A categorization of the type of environmental monitoring facility at which an observation is made | NOTE: Code table according to See [INSPIRE D2.8.III.7, 2013] | 3-04 | M* (Phase 2) |
| 3-05 | Station/platformObserving facility model | The model of the monitoring equipment used at the observing facility | EXAMPLES: ‘Landsat 8’ is a platform/station model of ‘satellite’; ‘Almos Automatic Weather Station (AWS)’ is a model of a ‘land station’; ’Airbus A340-600’ is a model of an ‘aircraft’. |  | M*# (Phase 3) |
| 3-06 | Station/platformObserving facility unique identifier | A unique and persistent identifier for an environmental monitoring facility (observing facility), which may be used as an external point of reference | NOTE: A globally unique identifier assigned by WMO for a station. Where a station has multiple identifiers, there must be a way of recording that they are synonyms. To be defined according to WMO guidelines. EXAMPLE: Ship: Call sign. |  | M* (Phase 1) |
| 3-07 | Geospatial location | Position in space defining the location of the environmental monitoring observing facility at the time of observation | NOTE 1: Required for fixed stations; for stations following pre-determined trajectory (e.g. satellites); NOTE 2: The elevation of a fixed terrestrial station is defined as the height above sea level of the ground on which the station stands (“Hha” in WMO Pub. 9 Vol A). NOTE 3: The geospatial location can be a zero-, one-, two-, or three-dimensional feature. NOTE 4: Geographical coordinates can be specified in decimal degrees. Latitudes are specified with reference to the equator, with positive sign for latitudes north of the equator, and negative sign for latitudes south of the equator. Longitudes are specified with reference to the Greenwich meridian, with positive sign for longitudes east of Greenwich, and negative sign for meridians west of Greenwich. Elevation is a signed number specified in some distance measure (e.g., meters) relative to a reference elevation, with positive sign in the direction away from the Earth centre. NOTE 5: The latitudinal and longitudinal positions of a station referred to in the World Geodetic System 1984 (WGS-84) Earth Geodetic Model 1996 (EGM96) must be recorded to a resolution of at least 0.001 decimal degrees (WMO-No. 8, 2008, Part I, Chapter 1, 1.3.3.2). NOTE 6: This element comprises 3 entities, the coordinates (Lat/Long/Alt), the "geopositioning method" (code table 11-01) which produced the coordinates, as well as the "geospatial reference system" (code table 11-02) used. EXAMPLES: (i) The station Jungfraujoch is located at 46.54749°N 7.98509°E (3580.00 m a.m.s.l.). The reference system is WGS-84. (ii) Voluntary Observing Ship Route: WMO Regional Association 5, Sub Area 6 (R56) (iii) [geostationary satellite] Meteosat-8 (MSG-1) 3.6°E (iv) [sun-synchronous satellite] NOAA-19 Height 870 km; Local Solar Time (LST) 13:39 (v) Weather Watch Radar: Warruwi NT -11.6485° N, 133.3800 E, Height 19.1 m amsl. (vi) River discharge gauge: Warrego River at Cunnamulla Weir 28.1000 S, 145.6833 E, Height: 180 m amsl. | 11-01 11-02 | M* (Phase 1) |
| 3-08 | Data communication method | Data communication method between the observing facility and some central facility | EXAMPLES: Inmarsat-C, ARGOS, Cellular, Globalstar, GMS(DCP), Iridium, Orbcomm, VSat, landline telephone, mail | 3-08 | O (Phase 2) |
| 3-09 | Station operating status | Declared reporting status of the station | NOTE: Refer to the code table | 3-09 | M (Phase 1) |

Conditions:

{3-01, 3-02}: Mandatory for fixed land-based stations, optional for mobile stations

# Category 4: Environment

Describes the geographical environment within which the observation is made. It also provides an unstructured element for additional meta-information that is considered relevant for adequate use of the observations and that is not captured anywhere else in the standard.

| Id | Name | Definition | Note or Example | Code table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 4-01 | Surface cover | The observed (bio)physical cover on the Earth’s surface in the vicinity of the observation | NOTE 1: To be applied to 3 different geographic scales of the vicinity of the observation, namely horizontal radii of <100 m, of 100 m to 3 km, and of 3 km to 100 km. NOTE 2: Surface cover or land cover is distinct from land use despite the two terms often being used interchangeably. Land use is a description of how people utilize the land and socio-economic activity – urban and agricultural land uses are two of the most commonly known land use classes. At any one point or place, there may be multiple and alternate land uses, the specification of which may have a political dimension. NOTE 3: There are various classification methods for ‘land cover’. The MODIS product MCD12Q1 provides 5 different classifications on 500 m resolution grid (https://lpdaac.usgs.gov/products/modis_products_table/mcd12q1). These include the IGBP, UMD, LAI/fPAR, NPP and PFT classifications. NOTE 4: An alternative approach is the ‘Land Cover Classification System’ (LCCS) (Di Gregorio, 2005) adopted by the Food and Agriculture Organization of the United Nations. Translation of other systems to LCCS has been explored by Herold et al. (2009). Eight major land cover types are identified during the first, dichotomous classification phase. These are refined in a subsequent so-called Modular-Hierarchical Phase, in which land cover classes are created by the combination of sets of pre-defined classifiers. These classifiers are tailored to each of the eight major land cover types. This process can be supported by software (http://www.glcn.org/sof_7_en.jsp) or manually using a field log sheet (http://commons.wikimedia.org/wiki/File:LCCS_field_protokoll.png) | 4-01 | C (Phase 3) |
| 4-02 | Surface cover classification scheme | Name and reference or link to document describing the classification scheme | IGBP, UMD, LAI/fPAR, NPP and PFT, LCCS (recommended implementation as a URI pointing to the code table) | 4-02 | C (Phase 3) |
| 4-03 | Topography or bathymetry | The shape or configuration of a geographical feature, represented on a map by contour lines | NOTE 1:Topography shall be formally expressed with the four elements ‘local topography’, ‘relative elevation’, ‘topographic context’, and ‘altitude/depth’. NOTE 2: The term ‘altitude’ is used for elevations above mean sea level. The term ‘depth’ is used for elevations below mean sea level. EXAMPLES (can be converted into entries of the code table): “a ridge at low relative elevation within valleys of middle altitude” “a depression within plains of very low depth” | 4-03 | C (Phase 3) |
|  | Surface roughness | The Terrain classification in terms of aerodynamic roughness length z0 | NOTE: The Terrain classification (Davenport and Wieringa) as mentioned in WMO-No. 8, 2008, Updated 2010, Annex to Chapter 5 |  | O (Phase 3) |
| 4-04 | Events at observing facility | Description of human action or natural event at the station or in the vicinity that may influence the observation | NOTE 1: This information may be frequently changing (for example ocean debris impacting buoys). NOTE 2: The start and end time/date of the event should be included | 4-04 or free text or an URL | O (Phase 2) |
|  | Climate zone | The Koppen climate classification of the region where the observing facility is located |  |  | O (Phase 3) |
| 4-05 | Site information | Non-formalized information about the location and its surroundings at which an observation is made and that may influence it | NOTE 2: In hydrology, description and dating of activities occurring in the basin that can affect the observed discharge, e.g., construction of a regulation structure upstream of the gauging location that significantly affects the hydrological regime, inter-basin diversion of water into or from the basin upstream of the gauging location, significant change in consumptive use, land cover or land use. EXAMPLES: maps, plans, photographs, descriptions and other unique site information that is difficult to express in words or that cannot easily be quantified. | Free text and/or URL(s) | O (Phase 2) |

Conditions:

Either {4-01 and 4-02 and 4-03} or a nilReason=”not applicable” must be reported. For hydrology and satellite observations, specifying nilReason is appropriate.

# Category 5: Instruments and Methods of Observation

Specifies the method of observation and describes characteristics of the instrument(s) used to make the observation. If multiple instruments are used to generate the observation, then this category should be repeated.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 5-01 | Source of observation | The source of the dataset described by the metadata | NOTE: Refer to the Code table | 5-01 | M (Phase 1) |
| 5-02 | Measurement/observing method | The method of measurement/ observation used | EXAMPLES: Temperature can be determined using different principles: liquid in glass; mechanical; electrical resistance; thermistor; thermocouple. Likewise, humidity is determined in AMDAR as amass mixing ratio. Several chemical variables can be determined using infrared absorption spectroscopy. In hydrology, stage would be observed using a staff gauge, electric tape, pressure transducer, gas bubbler, or acoustics. Examples of satellite observation principles: Cross-nadir scanning IR sounder; MW imaging/sounding radiometer, conical scanning, etc. Visual observation of weather, cloud type, etc. | 5-02 | M# (Phase 1) |
| 5-03 | Instrument specifications | Intrinsic capability of the measurement/observing method to measure the designated element, including range, stability, precision, etc. | NOTE 1: The metadata record can be "not available". NOTE 2: Includes the Upper limit of operational range and the Lower limit of operational range; For space-based observations, the channels and their frequencies should be included EXAMPLES: 1) Barometer measurement range 800-1100 hPa (i.e. unsuitable for some mountain ranges, Mt Everest ~300hPa) 2) Maximum distance a human observer can observe given the topography. | An URL is acceptable in case of space-based observations | MC*# (Phase 1) |
| 5-04 | Instrument operating status | The status of an instrument with respect to its operation | NOTE: To be recorded by data providers for each individual observation | 5-04 | O (Phase 3) |
| 5-05 | Vertical distance of sensor | Vertical distance of the sensor from a (specified) reference level such as local ground, or deck of a marine platform at the point where the sensor is located; or sea surface | NOTE 1: The reference surface (generally a surface which will strongly influence the observation) must be specified. NOTE 2: Away from center of earth, positive. Negative values indicate position below reference surface. EXAMPLES: i) Air temperature: height of the temperature sensor is 1.50 m above ground surface (station level). ii) Surface wind: 10.0 m above ground surface (station level) iii) Soil temperature: 0.50 m below soil surface; iv) Ship: Visual Obs Height: 22.0 m a.s.l. v) Weather Watch Radar: Warruwi AU 24.3 m above ground surface (see 7-07) vi) Transmissometer 2.55 above runway surface vii) depth of buoy relative to lowest astronomical tide viii) Pressure sensor: vertical distance above mean sea level ix) |  | C* (Phase 1) |
| 5-06 | Configuration of instrumentation | Description of any shielding or configuration/setup of the instrumentation or auxiliary equipment needed to make the observation or to reduce the impact of extraneous influences on the observation | NOTE: An URL could be provided in case of space-based observations EXAMPLES (for surface-based observations): shelter, temperature control, etc. Internal volume: [m3] Aspirated: [Natural/forced/na] Aspiration rate: m3s-1 Shielding from: [radiation/precipitation/ wind/etc.] |  | C# (Phase 3) |
| 5-07 | Instrument control schedule | Description of schedule for calibrations or verification of instrument | NOTE: For space-based observations, it applies only to major changes; For very frequent changes on parameters, a specific link to external source should be provided EXAMPLE: Every year on first week of February |  | C (Phase 3) |
| 5-08 | Instrument control result | The result of an instrument control check, including date, time, location, standard type and period of validity | NOTE 1: For the result of control check code table 5-08 is to be used NOTE 2: record even if "not available" NOTE 3: Information should contain at least the following elements: Standard type: [International, Primary, Secondary, Reference, Working, Transfer, Travelling, collective] Standard name: [free text] Standard reference: [serial number or equivalent] Within verification limit [Y/N] NOTE 4: Can be implemented with a URI pointing to a document containing this information NOTE 5: For space-based observations, it applies only to major changes; For very frequent changes on parameters, a specific link to external source should be provided EXAMPLE: 20140207 15:30 UTC, travelling standard, <name>, <S/N>, field calibration, result: in calibration, validity: 4 years | 5-08 | C# (Phase 3) |
| 5-09 | Instrument model and serial number | Details of manufacturer, model number, serial number and firmware version if applicable | NOTE 1: Record "not available" NOTE 2: Use the following formats: Instrument manufacturer: [free text] Instrument model: [free text] Instrument serial number: [free text] Firmware version: [free text] EXAMPLE: Vaisala PTB330B G2120006 |  | C# (Phase 3) |
| 5-10 | Instrument routine maintenance | A description of schedule maintenance that is performed on an instrument | EXAMPLE: Daily cleaning of a radiation sensor. |  | C# (Phase 3) |
| 5-11 | Maintenance party | Identifier of the organization or individual who performed the maintenance activity |  |  | O (Phase 2) |
| 5-12 | Geospatial location | Geospatial location of instrument/sensor | NOTE 1: Geographic location of instrument such as airfield anemometer or transmissometer. NOTE 2: This element comprises 3 entities, the coordinates (Lat/Long/Alt), the "geopositioning method" (code table 11-01) which produced the coordinates, as well as the "geospatial reference system" (code table 11-02) used. EXAMPLES: 1) Melbourne Airport AU (East anemometer) -37.6602 N, 144.8443 E, 122.00 m amsl. 2) relative position of wind sensor aboard ship 3) 30 km upstream of river mouth | 11-01 11-02 | C* (Phase 2) |
| 5-13 | Maintenance Activity | Description of maintenance performed on instrument | NOTE: A log of actual maintenance activity, both planned and corrective |  | O (Phase 3) |
| 5-14 | Status of observation | Official status of observation | NOTE: A binary flag | 5-14 | O (Phase 3) |
| 5-15 | Exposure of instruments | The degree to which an instrument is affected by external influences and reflects the value of the observed variable | NOTE: The exposure of an instrument results from joint appraisal of the environment, measurement interval and exposure against the requirements of some particular application. Expressed in terms of code table. | 5-15 | C (Phase 2) |

Conditions:

{5-07, 5-08, 5-09, 5-10, 5-15} mandatory for instrumental observations

{5-05} mandatory for instrumental observations and if proximity of reference surface impacts on observation

{5-06} mandatory for instrumental observations and if prescribed by “best practice”.

{5-12} mandatory for instrumental observations and if different from observing facility

{5-05, 5-06, 5-10, 5-12, 5-15} A nilReason=”not applicable” is acceptable for space-based observations;

# Category 6: Sampling

Specifies how sampling and/or analysis are used to derive the reported observation or how a specimen is collected.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 6-01 | Sampling procedures | Procedures involved in obtaining a sample | EXAMPLES: Temperature measurements are made using a XYZ thermometer and reported results are an average of 10 measurements made in a given hour. Aerosols may be sampled with an inlet with size-cutoff at 2.5 µm and be deposited on a teflon filter. Manual reading of a liquid-in-glass thermometer every three hours. As an exception, an observer may observe the state of the sky from home rather than at the station during night. Rain fall is accumulated during the whole week-end and distributed evenly over these 2 days. | An URL in case of space-based observations | O (Phase 3) |
| 6-02 | Sample treatment | Chemical or physical treatment of sample prior to analysis | EXAMPLES: Homogenization, milling, mixing, drying, sieving, heating, melting, freezing, evaporation… |  | O (Phase 3) |
| 6-03 | Sampling strategy | The strategy used to generate the observed variable | EXAMPLES: Continuous: global radiation, atmospheric pressure, or continuous ozone monitoring with a UV monitor; Discrete: gas chromatographic analysis of carbon monoxide, radar rainfall; Event: grab water samples, flask sampling of air, etc. | 6-03 or an URL in case of space-based observations | O* (Phase 1) |
| 6-04 | Sampling time period | The period of time over which a measurement is taken | NOTE: Includes the sampling time period, plus the meaning of time stamp (11-03). For each example, Time stamp indicates “end of period” | 11-03 | M# (Phase 3) |
| 6-05 | Spatial sampling resolution | Spatial resolution refers to the size of the smallest observable object. The intrinsic resolution of an imaging system is determined primarily by the instantaneous field of view of the sensor, which is a measure of the ground area viewed by a single detector element in a given instance in time. | NOTE: a representative value (L × L × L), where “L” is a length, is expected, according to the dimension (1-D, 2-D or 3-D), but free text is allowed for the characteristics to be explained. EXAMPLES: The sample is a point in space or a very small volume resembling a point, e.g., a temperature sampled by a thermocouple element: No size to be reported; The sample is a line, either straight (e.g., a line of sight of a DOAS instrument) or curved (e.g., the humidity sampled by an aircraft in flight). The ‘length’ of the line is to be reported; The sample is an area, either rectangular or of any other shape, e.g., the reach of a radar image. The ‘length x length’ of the area is to be reported; The sample is a volume, e.g. a water sample or a well-mixed volume of air sampled by flask. The ‘length x length x length’ of the volume is to be reported. |  | M# (Phase 2) |
| 6-06 | Temporal sampling interval | Time period between the beginning of consecutive sampling periods | EXAMPLES: surface winds sampled every 0.25 s (frequency 4 Hz) (WMO, 2008); surface winds measured once per hour; Barometric pressure measured once every 6 minutes; water column height measured every 15 seconds; water temperature measured once per hour (NOAA, 2009); For each example, Time stamp indicates “end of period”. |  | M (Phase 3) |
| 6-07 | Diurnal base time | Time to which diurnal statistics are referenced | Examples Rain fall observation is accumulated for 24 hours up until 0700z, the diurnal base time here is 0700z. Daily temperature maxima refer to the period 0600 local time, the diurnal base time here is 0600z. |  | MC (Phase 1) |
| 6-08 | Schedule of observation | Schedule of observation | EXAMPLES: AMDAR profiling observations are available from Zurich airport between 0600 and 1200 local time; Radio-sondes are collected at a particular station from January to August on weekdays at 0000z and 1200z | Free text | M (Phase 1) |

{6-05, 6-06, 6-07, 6-08} A nilReason=”not applicable” is acceptable for space-based observations;

# Category 7: Data Processing and Reporting

Specifies how raw data are transferred into the observed variables and reported to the users.

| Id | Name | Definition | Note or Example | Code table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 7-01 | Data processing methods and algorithms | A description of the processing used to generate the observation and list of algorithms utilized to derive the resultant value | NOTE: In hydrology, this would be the equation(s) defining the rating curve and any shifts or corrections applied to the data or the curve. EXAMPLE: Radiation correction and calculation of geo-potential height for upper-air soundings | Free text or an URL | O (Phase 3) |
| 7-02 | Processing/analysis center | Center at which the observation is processed. | EXAMPLES: Chemical analysis, AMDAR processing center, National Hydrological Service office. |  | O (Phase 2) |
| 7-03 | Temporal reporting period | Time period over which the observed variable is reported | NOTE: Includes the temporal reporting interval, plus the meaning of time stamp. EXAMPLES: Hourly, daily, monthly, seasonal, event-based, 80 seconds interval during the day, etc. In each case, the meaning, “beginning, “middle”, or “end” of period is indicated. | 11-03 | M* (Phase 1) |
| 7-04 | Spatial reporting interval | Spatial interval at which the observed variable is reported | NOTE: This is applicable only to remote sensing observations and mobile platforms in general. For most remote-sensing observations, this will be redundant with element 6-05. EXAMPLES: - An observation from a satellite may be reported with a spatial resolution of 10 km x 20 km; Other examples in the Appendix. - An aircraft may sample every 1 km along its trajectory ( cf. 6-05), but may report at a spatial interval of 10 km. |  | C* (Phase 1) |
| 7-05 | Software/processor and version | Name and version of the software or processor utilized to derive the element value | EXAMPLES: Avionics version, retrieval algorithm version; MCH Database Management System version 25/10/2013. |  | O (Phase 3) |
| 7-06 | Level of data | Level of data processing | NOTE: Pre or Post processing. | 7-06 | O (Phase 2) |
| 7-07 | Data format | Description of the format in which the observed variable is being provided. | EXAMPLES: ASCII, BUFR, NASA AMES, HDF, XML, AMDAR, comma-separated (CSV), tab-separated (.txt), MCH (for interchange) | 7-07 | M (Phase 3) |
| 7-08 | Version of data format | Version of the data format in which the observed variable is being provided. | EXAMPLES: FM 12–XIV Ext. SYNOP; FM 42-XI Ext. AMDAR, FM 94-XIV BUFR Version 20.0.0, Radar: ODIM_H5 |  | M (Phase 3) |
| 7-09 | Aggregation period | Time period over which individual samples/observations are aggregated | NOTE: Includes the aggregation interval, plus the meaning of time stamp. EXAMPLES: 5 minute mean, meaning of time stamp is “middle of period”; daily maximum, meaning of time stamp is “end of period”; event based, meaning of time stamp is “beginning of period”. | 11-03 | M (Phase 2) |
| 7-10 | Reference time | Time base to which date and time stamps refer | NOTE: The reference time must not be confused with the time zone (which is part of the representation of the time stamp), but indicates what the source of the time stamp is, i.e., to which reference time the time stamps of the observation are aligned. EXAMPLES: NIST time server NTP pool project | 7-10 | M (Phase 2) |
| 7-11 | Reference datum | Reference datum used to convert observed quantity to reported quantity | NOTE 1: Atmospheric pressure can be reported as (i) Field elevation Pressure (QFE), where the reference datum is the elevation corresponding to the official elevation of the aerodrome; (ii) Atmospheric pressure at nautical height (QNH), where the reference datum is mean sea level and the pressure altitude relationship of the ICAO standard atmosphere is used. Where observed atmospheric pressure cannot be reduced to mean sea level, a station should, by regional agreement, report either the geopotential of an agreed ‘constant pressure level’ or the pressure reduced to an agreed datum for the station. The level chosen for the station should be reported in this field. (Ref: WMO-No. 8 3.11.1). NOTE 2: Hydrology may report a gauge zero which is the gauge height of zero flow. |  | C (Phase 1) |
| 7-12 | Numerical resolution | Measure of the detail in which a numerical quantity is expressed | NOTE 1: The resolution of a numerical quantity is a measure of the detail in which the quantity is expressed. It can be expressed as the smallest possible difference between two numbers. It can also be expressed as the number of significant figures of a number, which are those digits that carry meaning contributing to its resolution. EXAMPLE: if a measurement resolution to four decimal places (0.0001) is given as 12.23 then it might be understood that only two decimal places of resolution are available. Stating the result as 12.2300 makes clear that it is precise to four decimal places (in this case, six significant figures). NOTE 2: The notion of measurement resolution is related but must not be confounded with the uncertainty of an observation EXAMPLES: - An anemometer may measure wind speed with a measurement resolution of 0.1 ms-1 with a 1 Hz scan rate. Observations may be aggregated to 1-minute values and may be rounded and reported with a (reduced) measurement resolution of 1 ms-1. - A barometer may be capable of measuring atmospheric pressure with a readout resolution of 1 hPa and an uncertainty of 5 hPa (k=2). The data can be reported to the nearest hPa, however, the measurement resolution should be stated as “5 hPa” or “3 significant digits”. - An ocean thermometer measures temperature to 0.0001 °C. - Seawater salinity measured to 0.001 salinity units (derived from conductivity measurements with a resolution of 0.01 Sm-1) |  | O (Phase 3) |
| 7-13 | Latency (of reporting) | The typical time between completion of the observation or collection of the datum and when the datum is reported | - For satellite data, the “observation” (e.g. a complete image) can take 20 minutes to generate. Hence the latency would be the time between the completion of the image collection, and when it is available. Typically this can be 2-3 minutes. Some satellite products such as SST can take about 10 minutes of processing until it is available. (ii) A radar volumetric scan can take 6 – 10 minutes (in Australia), so the latency would be the time between the completion of the scan and when the data is locally available. In Australia, this varies between a few seconds to several minutes depending on delays in data communications. (iii) AWS data may have a latency of 1- 20 seconds (or considerably more in some places) between the completion of the observation and arrival of the data at a central archive. |  | M (Phase 3) |

Conditions:

{7-04}: mandatory for remote sensing observations and mobile platforms in general

{7-11}: mandatory for stations/platforms that report a derived observation value that depends on a local datum

# Category 8: Data Quality

Specifies the data quality and traceability of the observation

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 8-01 | Uncertainty of measurement | Non-negative parameter, associated with the result of a measurement, that characterizes the dispersion of the values that could reasonably be attributed to the observation/ measurand | NOTE 1: In principle, an uncertainty statement needs to be reported for each observation, as it can change from observation to observation. If the uncertainty of observations remain virtually constant over time, it is sufficient to report the uncertainty at the beginning of the period and then again when substantial changes of the uncertainty occur. The actual uncertainty statements should be reported with the observations. NOTE 2: Complex observations such as gridded satellite imagery may contain large error covariance matrices that are not useful for the purpose of this standard. Such information must be kept with the data, and it is sufficient to report an aggregate (e.g., mean or median) uncertainty in the metadata. NOTE 3: Uncertainty may be expressed, for example, as a standard deviation (or a given multiple of it), or the half-width of an interval having a stated level of confidence. NOTE 4: Uncertainty of measurement comprises, in general, many components. Some of these components may be evaluated from the statistical distribution of the results of series of measurements and can be characterized by experimental standard deviations. The other components, which also can be characterized by standard deviations, are evaluated from assumed probability distributions based on experience or other information. NOTE 5: It is understood that the result of the measurement is the best estimate of the value of the measurand, and that all components of uncertainty, including those arising from systematic effects, such as components associated with corrections and reference standards, contribute to the dispersion. NOTE 6: For space-based observations, the implementation should be taken in a simple way. Examples are provided in the Appendix EXAMPLE: A thermometer reading may yield a value of 13.7 °C. A quality assessment of that observation may indicate that it has an expanded uncertainty of +/- 0.3 °C (k=2), where k=2 is a coverage factor corresponding approximately to a confidence interval of 95% |  | C*# (Phase 2) |
| 8-02 | Procedure used to estimate uncertainty | A reference or link pointing to a document describing the procedures/algorithms used to derive the uncertainty statement | NOTE: Uncertainty is a well-defined term, and guidance material exists to assist in the assessment of the uncertainty of observations and a formulation of adequate uncertainty statements. The authoritative source is the “Guide for the Expression of Uncertainty in Measurement” (JCGM 100:2008). |  |  |
| 8-03 | Quality flag | An ordered list of qualifiers indicating the result of a quality control process applied to the observation | NOTE 1: BUFR code table series 0-33 contains data quality flags/definitions. NOTE 2: To be recorded by data providers for each individual observation | 8-03 | M# (Phase 2) |
| 8-04 | Quality flagging system | Reference to the system used to flag the quality of the observation | NOTE 1: At present, there is no single, globally accepted flagging system. The purpose of this element is to make reference to the flagging system used. This reference should either be a URL to a document explaining the meaning of the quality flag, or a link to a code table where this information can be found. NOTE 2: The use of the BUFR quality codes listed above is recommended (WMO, 2013a) | 8-04 |  |
| 8-05 | Traceability | Statement defining traceability to a standard, including sequence of measurement standards and calibrations that is used to relate a measurement result to a reference [VIM 3 2.4.2] | NOTE 1: A metrological traceability chain is defined through a calibration hierarchy; VIM 3, 2.4.2] NOTE 2: A metrological traceability chain is used to establish metrological traceability of a measurement result. [VIM 3, 2.4.2] NOTE 3: A comparison between two measurement standards may be viewed as a calibration if the comparison is used to check and, if necessary, correct the quantity value and measurement uncertainty attributed to one of the measurement standards; [VIM 3, 2.4.2] NOTE 4: For the statement on traceability, code table 8-05 is to be used NOTE 5: For space-based observations, the implementation should be taken in a simple way. Examples are provided in the Appendix | 8-05 | C*# (Phase 2) |

Conditions:

{8-01, 8-02 and 8-05} variables that are measured, rather than classified

# Category 9: Ownership & Data Policy

Specifies who is responsible for the observation and owns it.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 9-01 | Supervising organization | Name of organization who owns the observation | EXAMPLES: for satellite operators EUMETSAT, ESA, NOAA, NASA, CMA, RapidEye, ISRO |  | M (Phase 2) |
| 9-02 | Data policy/use constraints | Details relating to the use and limitations surrounding data imposed by the supervising organization | NOTE: Only one single use constraint with a value taken from WMO_DataLicenseCode is allowed to ensure unambiguity. (WMO, 2013b, p51) | 9-02 | M* (Phase 1) |

# Category 10: Contact

Specifies where information about an observation or dataset can be obtained.

| Id | Name | Definition | Note or Example | Code Table | ItemMCO |
| --- | --- | --- | --- | --- | --- |
| 10-01 | Contact (Nominated Focal Point) | Principal contact (Nominated Focal Point, FP) for resource | NOTE: The FP would be able to provide data users with information regarding individual observing platforms and their observations. EXAMPLES: Programme or Network Manager, e.g. E-AMDAR Technical Coordinator (TC) has responsibility for data quality of several airlines’ fleets, has information on aircraft type/software/known errors etc. |  | M (Phase 1) |

# VIII – References

Antonio Di Gregorio (2005), Land Cover Classification System. Classification concepts and user manual. Software version 2, Food and Agricultural Organization of the United Nations, ISBN 92-5-105327-8, 212pp, http://www.glcn.org/downs/pub/docs/manuals/lccs/LCCS2-manual_en.pdf

Hammond, E.H. (1954) Small-scale continental landform maps, Annals of the Association of American Geographers, 44(1):33-42. DOI:10.1080/00045605409352120

INSPIRE – D2.8.III.7, 2013 Data Specification on Environmental Monitoring Facilities – Draft Technical Guidelines http://inspire.jrc.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_EF_v3.0rc3.pdf. SpecialisedEMFTypeValue, p 33

ISO/TC 211. (2003). ISO 19115:2003, Geographic information – Metadata. Oslo, Norway: International Standards Organization.

ISO/TC 211. (2011). ISO19156:2011, Geographic Information – Observations and Measurements. Geneva, Switzerland: International Standards Organization.

JCGM 100:2008. Evaluation of measurement data — Guide to the Expression of Uncertainty in Measurement, available at: http://www.bipm.org/utils/common/documents/jcgm/JCGM_100_2008_E.pdf

JCGM 200:2012. International vocabulary of metrology — Basic and general concepts and associated terms (VIM), 3rd Edition, available at: http://www.bipm.org/utils/common/documents/jcgm/JCGM_200_2012.pdf

M. Herold, R. Hubald, and G. Di Gregorio (2009), Translating and evaluating land cover legends using the UN Land Cover Classification system (LCCS), GOFC-GOLD Report Nr. 43, Jena, Germany, http://nofc.cfs.nrcan.gc.ca/gofc-gold/Report%20Series/GOLD_43.pdf

MODIS Land Cover Type Product: https://lpdaac.usgs.gov/products/modis_products_table/mcd12q1

Ramsey and Hewitt, 2005.Charles A. Ramsey and Alan D. Hewitt, A Methodology for Assessing Sample Representativeness, Environmental Forensics, 6:71–75, 2005, doi: 10.1080/15275920590913877

S. Henne, D. Brunner, D. Folini, S. Solberg, J. Klausen, and B. Buchmann, Assessment of parameters describing representativeness of air quality in-situ measurement sites, Atmos. Chem. Phys., 10, 3561–3581, 2010.

Speight, J.G. (2009) Landform, in Australian Soil and Land Survey Field Handbook (3rdedn), National Committee on Soil and Terrain, CSIRO Publishing, Melbourne.http://www.publish.csiro.au/nid/22/pid/5230.htm.http://books.google.com.au/books?id=zywc39z4LgAC

WMO (2008) Guide to Meteorological Instruments and Methods of Observation, (WMO-No. 8 – Updated 2010). Geneva, Switzerland: World Meteorological Organization.

–

W2013a: Manual on Codes (WMO-No. 306) – International Codes VOLUME I.2 PART B – Binary Codes http ://www.wmo.int/pages/prog/www/WMOCodes/WMO306_vI2/Publications/2011editionUP2013/WMO306_vI2_2011UP2013.pdf. Geneva, Switzerland: World Meteorological Organization.

___2013b: Manual on the WMO Information System (WMO-No. 1060), http://wis.wmo.int/WIS-Manual

WMO___2013c: Weather Reporting (WMO-No. 9), Volume A, Observing Stations. http://www.wmo.int/pages/prog/www/ois/ois-home.html

# ANNEX – Code Tables

### Code table: 1-01

Code table title: Observed variable – measurand [Code table under development]

| # | Path | Name | Definition | WMO306_CD |
| --- | --- | --- | --- | --- |
| 6 | >Earth>Crustal motion (horizontal and vertical) | Crustal motion (horizontal and vertical) | Changes in time of the position and height of the Earth plates. Indicative of the lithosphere dynamics, thus useful for earthquake prediction - Physical unit: [ mm/y ] - Accuracy unit: [ mm/y ]. | NA |
| 7 | >Earth>Crustal plates positioning | Crustal plates positioning | Basis for monitoring the evolution of the lithosphere dynamics - Physical unit: [ cm ] - Accuracy unit: [ cm ]. | NA |
| 8 | >Earth>Earth Rotation | Earth Rotation | Rotation velocity of the Earth, determining the length of the day - Physical unit: [Rad/s] - Accuracy unit: [Arcsec/century]. | NA |
| 9 | >Earth>Geoid | Geoid | Equipotential surface which would coincide exactly with the mean ocean surface of the Earth, if the oceans were in equilibrium, at rest, and extended through the continents (such as with very narrow channels) - Physical unit: [ cm ] - Accuracy unit: [ cm ]. | NA |
| 10 | >Earth>Geomagnetic field | Geomagnetic field | Magnitude and direction of the 3D magnetic field on the surface of Earth and within the magnetosphere (i.e., in low-Earth orbit and in geosynchronous orbit). | NA |
| 11 | >Earth>Gravity field | Gravity field | Indicative of the statics and dynamics of the lithosphere and the mantle - Physical unit: [ mGal ] where : 1 Gal = 0.01 m.s<sup>-2</sup> | NA |
| 12 | >Earth>Gravity gradients | Gravity gradients | Gradient of the Earthҳ gravity field measured at the satellite orbital height - Physical unit: [ E ] , E?tv?s (1 E = 1 mGal / 10 km) - Accuracy unit: [ E ]. | NA |
| 13 | >Ocean>Topography>Bathymetry | Bathymetry | NA | oceanBathymetry |
| 18 | >Ocean>Basic Physical Properties>Pressure | Pressure | Pressure(depth) measured by CTD | oceanPressure |
| 37 | >Atmosphere>Aerological soundings | Aerological soundings | NA | NA |
| 39 | >Atmosphere>Atmospheric density | Atmospheric density | 3D field of density of the atmosphere | NA |
| 44 | >Atmosphere>Integrated air samples | Integrated air samples | NA | NA |
| 56 | >Atmosphere>Special phenomena | Special phenomena | NA | NA |
| 57 | >Atmosphere>State of ground | State of ground | NA | NA |
| 62 | >Ocean>Currents>Ocean Current - Direction | Ocean Current - Direction | NA | oceanCurrentDirection |
| 63 | >Ocean>Currents>Ocean Current - Speed | Ocean Current - Speed | Ocean motion measured at various depth levels. | oceanCurrentSpeed |
| 64 | >Ocean>Topography>Ocean dynamic topography (ODT) | Ocean dynamic topography (ODT) | Deviation of sea level from the geoid caused by ocean currents (after corrections for tides and atmospheric pressure effects) - Physical unit: [ cm ] - Accuracy unit: [ cm ]. | oceanDynamicTopography |
| 65 | >Ocean>Currents>Ocean surface currents (vector) | Ocean surface currents (vector) | Water flow on ocean surface - Physical unit: [ cm/s ] - Accuracy unit: [ cm/s ] intended as vector error, i.e. the module of the vector difference between the observed vector and the true vector. | oceanSurfaceCurrent |
| 67 | >Ocean>Miscellaneous>Carbon species>Dissolved inorganic carbon (DIC) | Dissolved inorganic carbon (DIC) | The cumulated concentration of inorganic carbon species (dissolved carbon dioxide, carbonic acid, bicarbonate and carbonate) in solution. | oceanDissolvedInorganicCarbon |
| 68 | >Ocean>Miscellaneous>Carbon species>Dissolved organic carbon | Dissolved organic carbon | NA | oceanDissolvedOrganicCarbon |
| 69 | >Ocean>Miscellaneous>Nitrogen species>Dissolved organic nitrogen | Dissolved organic nitrogen | NA | oceanDissolvedOrganicNitrogen |
| 70 | >Ocean>Miscellaneous>Diffuse attenuation coefficient (DAC), turbidity | Diffuse attenuation coefficient (DAC), turbidity | Indicator of water turbidity and vertical processes in the ocean, extracted from ocean colour observation. | oceanTurbidity |
| 71 | >Ocean>Biology>Chlorophyll concentration | Chlorophyll concentration | Indicator of living phytoplankton biomass, extracted from ocean colour observation. Uncertainty is expressed in mg/m<sup>3</sup> for a given concentration of 1 mg/m<sup>3</sup> . | oceanChlorophyllConcentration |
| 72 | >Ocean>Miscellaneous>Ocean suspended sediments concentration | Ocean suspended sediments concentration | Variable extracted from ocean colour observation. Indicative of river outflow, re-suspension or pollution of other-than-biological origin. Uncertainty expressed in g/m<sup>3</sup>&nbsp;at a specific concentration (e.g., 2 g/m<sup>3</sup>&nbsp;). | oceanSuspendedSedimentConcentration |
| 73 | >Ocean>Miscellaneous>Oil spill cover | Oil spill cover | Fraction of an ocean area polluted by hydrocarbons released from ships or off-shore platforms, accidentally or deliberately. | oceanOilSpillCover |
| 74 | >Ocean>Biology>Primary production | Primary production | NA | oceanPrimaryProduction |
| 75 | >Ocean>Miscellaneous>Sea surface Mass Flux | Sea surface Mass Flux | Sea Surface Mass Flux | oceanSeaSurfaceMassFlux |
| 76 | >Ocean>Miscellaneous>Sea surface heat flux | Sea surface heat flux | Sea Surface Heat Flux | oceanSeaSurfaceHeatFlux |
| 77 | >Ocean>Topography>Sea surface height anomaly (SSHA) | Sea surface height anomaly (SSHA) | Sea surface height anomaly | oceanSeaSurfaceHeightAnomaly |
| 78 | >Ocean>Miscellaneous>Total alkalinity (AT) | Total alkalinity (AT) | The amount of acid necessary to titrate all weak bases in seawater. | oceanTotalAlkalinity |
| 79 | >Ocean>Miscellaneous>pH | pH | The hydrogen ion concentration in seawater, which is a measure of acidity and alkalinity | oceanPH |
| 82 | >Ocean>Biology>SiO3= (metasilicate) | SiO3= (metasilicate) | NA | oceanMetaSilicate |
| 84 | >Ocean>Other Gas>N2, Ar, O2 | N2, Ar, O2 | NA | oceanN2ArO2 |
| 85 | >Ocean>Other Gas>O2/Ar ratio | O2/Ar ratio | NA | oceanO2ArRatio |
| 86 | >Ocean>Other Gas>Ocean subsurface dissolved oxygen concentration | Ocean subsurface dissolved oxygen concentration | Concentration of dissolved oxygen | oceanSubsurfaceDissolvedOxygenConcentration |
| 87 | >Ocean>Other Gas>Oxygen, dissolved | Oxygen, dissolved | NA | oceanDissolvedOxygen |
| 89 | >Ocean>Radiation>Water-leaving spectral radiance | Water-leaving spectral radiance | Upward spectral radiance from the ocean surface, i.e. amount of light leaving the ocean per unit area, per wavelength and per solid angle. | oceanOutgoingspectralRadiance |
| 90 | >Ocean>Reactive Gas>C2H6S (dimethylsulfide, DMS) | C2H6S (dimethylsulfide, DMS) | NA | oceanDMS |
| 91 | >Ocean>Basic Physical Properties>Ocean salinity | Ocean salinity | 3D field of salinity of the ocean Requested in upper and deep ocean - Physical unit: [ psu ], Practical Salinity Unit, close to 1, determined by CTD | oceanSalinity |
| 92 | >Ocean>Basic Physical Properties>Sea surface salinity | Sea surface salinity | Salinity of sea water in the surface layer (upper ~ 1 m if observed in MW). In the open ocean the correct term should be “halinity” with reference of the diversity of salts involved. | oceanSurfaceSalinity |
| 94 | >Ocean>Basic Physical Properties>Ocean temperature | Ocean temperature | 3D field of temperature of the ocean Requested in upper and deep ocean. | oceanTemperature |
| 95 | >Ocean>Basic Physical Properties>Sea surface temperature | Sea surface temperature | Temperature of the sea water at surface. The “bulk” temperature refers to the depth of typically 2 m, the “skin” temperature refers to within the upper 1 mm. | oceanSurfaceTemperature |
| 96 | >Ocean>Topography>Coastal sea level (tide) | Coastal sea level (tide) | Deviation of sea level from local references in coastal zones, caused by local currents and tides (astronomical and wind-induced). | oceanCoastalSeaLevel |
| 98 | >Ocean>Waves>Dominant wave direction | Dominant wave direction | One feature of the ocean wave spectrum. It is the direction of the most energetic wave in the spectrum - Physical unit: [ degrees ] - Accuracy unit: [ degrees ]. | oceanDominantWaveDirection |
| 99 | >Ocean>Waves>Dominant wave period | Dominant wave period | The period of the most energetic wave in the ocean wave spectrum. | oceanDominantWavePeriod |
| 100 | >Ocean>Waves>Significant wave height | Significant wave height | Average amplitude of the highest 30 of 100 waves. | oceanDominantWaveHeight |
| 101 | >Ocean>Waves>Wave 1D energy frequency spectrum | Wave 1D energy frequency spectrum | 1D variable colloquially referred to as &ldquo wave spectrum&rdquo. Describes the wave energy in each frequency band (e.g. 25 frequency bands) regardless of the direction of propagation | oceanWave1DEnergySpectrum |
| 102 | >Ocean>Waves>Wave directional energy frequency spectrum | Wave directional energy frequency spectrum | 2D variable colloquially referred to as wave spectrum. Describes the wave energy travelling in each direction and frequency band (e.g., 24 distinct azimuth sectors each 15&deg; wide, and 25 frequency bands) | oceanWaveDirectionalEnergySpectrum |
| 103 | >Ocean>Wind>Wind stress | Wind stress | The shear force per unit area exerted by wind blowing over the sea surface - | oceanWaveStress |
| 104 | >Outer Space>Energetic particles / solar wind >Electron flux energy spectrum | Electron flux energy spectrum | Flux density energy spectrum of low-, medium-, and high-energy electrons from the magnetosphere, the radiation belts or the interplanetary medium. | NA |
| 105 | >Outer Space>Energetic particles / solar wind >Electrostatic charge | Electrostatic charge | Measurement of the electrostatic charge of a satellite accumulated because of the environmental situation. | NA |
| 106 | >Outer Space>Energetic particles / solar wind >Heavy ion angular flux energy and mass spectrum | Heavy ion angular flux energy and mass spectrum | Flux density energy and mass spectrum of heavy ions ranging from Helium to Iron, per unit solid angle | NA |
| 107 | >Outer Space>Energetic particles / solar wind >Heavy ion flux energy and mass spectrum | Heavy ion flux energy and mass spectrum | Flux density energy and mass spectrum of heavy ions ranging from Helium to Iron. | NA |
| 108 | >Outer Space>Energetic particles / solar wind >Interplanetary magnetic field | Interplanetary magnetic field | Vector magnetic field (3D) in the solar wind.The reference frame is Cartesian or Cylindrical with many different axis orientations. Important in Space Weather to monitor magnetic disturbances of the near earth environment. | NA |
| 111 | >Outer Space>Energetic particles / solar wind >Radiation Dose Rate | Radiation Dose Rate | 3D field of the dose rate of energetic particles | NA |
| 112 | >Outer Space>Energetic particles / solar wind >Solar wind temperature | Solar wind temperature | Temperature of solar wind protons | NA |
| 113 | >Outer Space>Energetic particles / solar wind >Solar wind velocity | Solar wind velocity | Vector velocity of solar wind plasma | NA |
| 114 | >Outer Space>Ionospheric disturbances>Aurora | Aurora | Occurence of fluorescence of the upper atmosphere through ionization by energetic charged particles accelerated by the Earth's magnetic field. | NA |
| 115 | >Outer Space>Ionospheric disturbances>Electric Field | Electric Field | Magnitude and direction of the Earth's electriv field | NA |
| 116 | >Outer Space>Ionospheric disturbances>Electron Density | Electron Density | 3-D field of the electron density in the ionosphere | NA |
| 117 | >Outer Space>Ionospheric disturbances>Ionospheric Radio Absorption | Ionospheric Radio Absorption | Attenuation of a radio wave passing through the lower ionosphere. | NA |
| 118 | >Outer Space>Ionospheric disturbances>Ionospheric Scintillation | Ionospheric Scintillation | Random fluctuations of radio waves resulting of small scale variations of the ionospheric electron density in space and time. | NA |
| 119 | >Outer Space>Ionospheric disturbances>Ionospheric Vertical Total Electron Content (VTEC) | Ionospheric Vertical Total Electron Content (VTEC) | Number of electrons between two points. | NA |
| 120 | >Outer Space>Ionospheric disturbances>Ionospheric plasma density | Ionospheric plasma density | Total number of ionized particles in a volume unit of ionospheric plasma | NA |
| 121 | >Outer Space>Ionospheric disturbances>Ionospheric plasma velocity | Ionospheric plasma velocity | Velocity of bulk plasma or electrons (depending on measurement technique) as a function of altitude in the ionosphere. | NA |
| 122 | >Outer Space>Ionospheric disturbances>Spread F | Spread F | Vertical thickness of highly structured ion density in the F-region of the ionosphere. | NA |
| 123 | >Outer Space>Ionospheric disturbances>foEs | foEs | The highest ordinary-wave frequency reflected back from a sporadic E layer and observed by an ionosonde. | NA |
| 124 | >Outer Space>Ionospheric disturbances>foF2 | foF2 | Critical frequency of the F2 layer of the ionosphere. This critical frequency (f, in MHz) is associated with the electron density (Ne, in cm<sup>-3</sup>): f=9*10<sup>-3</sup>*sqrt(Ne). | NA |
| 125 | >Outer Space>Ionospheric disturbances>h'F | h'F | Virtual height of the bottom of the ionospheric F-layer. | NA |
| 126 | >Outer Space>Ionospheric disturbances>hmF2 | hmF2 | Altitude of the peak density in the ionospheric F2 layer. | NA |
| 127 | >Outer Space>Solar monitoring>EUV flux | EUV flux | Radiative flux integrated over the EUV band. | NA |
| 128 | >Outer Space>Solar monitoring>EUV flux spectrum | EUV flux spectrum | Energy spectrum of the radiative flux measured in the EUV band. | NA |
| 129 | >Outer Space>Solar monitoring>EUV sky image | EUV sky image | EUV image of the sky | NA |
| 130 | >Outer Space>Solar monitoring>Gamma-ray flux | Gamma-ray flux | Radiative flux integrated over the gamma-ray domain. | NA |
| 131 | >Outer Space>Solar monitoring>Gamma-ray flux spectrum | Gamma-ray flux spectrum | Energy spectrum of the radiative flux measured in the gamma-ray domain. | NA |
| 132 | >Outer Space>Solar monitoring>Heliospheric image | Heliospheric image | Image of the interplanetary space between the Sun and Earth | NA |
| 133 | >Outer Space>Solar monitoring>Radio-waves | Radio-waves | Travelling radio-waves of solar, galactic and extragalactic origin, entering the Earth's environment. | NA |
| 134 | >Outer Space>Solar monitoring>Solar Ca II-K image | Solar Ca II-K image | Image of the Sun in the K-line of Ca-II(393.4 nm) | NA |
| 135 | >Outer Space>Solar monitoring>Solar EUV flux | Solar EUV flux | Integrated EUV flux over the solar disk | NA |
| 136 | >Outer Space>Solar monitoring>Solar EUV flux spectrum | Solar EUV flux spectrum | Energy spectrum of the EUV flux integrated over the solar disk. | NA |
| 137 | >Outer Space>Solar monitoring>Solar EUV image | Solar EUV image | Images of the Sun in the Extreme Ultra-Violet (EUV) wavelengths in order to identify features such as filaments, active regions and coronal holes. | NA |
| 138 | >Outer Space>Solar monitoring>Solar H-alpha image | Solar H-alpha image | Image of the Sun in the Hydrogen-alpha transition wavelength (656.3 nm). | NA |
| 139 | >Outer Space>Solar monitoring>Solar Lyman-alpha flux | Solar Lyman-alpha flux | Solar flux in the Hydrogen Lyman-alpha transition wavelength (121.6 nm) | NA |
| 140 | >Outer Space>Solar monitoring>Solar Lyman-alpha image | Solar Lyman-alpha image | Image of the Sun in the Hydrogen Lyman-alpha transition wavelength (121.6 nm) | NA |
| 141 | >Outer Space>Solar monitoring>Solar UV flux | Solar UV flux | Integrated UV flux over the solar disk. | NA |
| 142 | >Outer Space>Solar monitoring>Solar UV flux spectrum | Solar UV flux spectrum | Energy spectrum of the UV flux integrated over the solar disk. | NA |
| 143 | >Outer Space>Solar monitoring>Solar UV image | Solar UV image | Images of the Sun in the UV wavelengths. | NA |
| 144 | >Outer Space>Solar monitoring>Solar VIS flux | Solar VIS flux | Integrated VIS flux over the solar disk | NA |
| 145 | >Outer Space>Solar monitoring>Solar VIS flux spectrum | Solar VIS flux spectrum | Energy spectrum of the integrated VIS flux over the solar disk. | NA |
| 146 | >Outer Space>Solar monitoring>Solar VIS image | Solar VIS image | Images of the Sun in the VIS wavelengths. | NA |
| 147 | >Outer Space>Solar monitoring>Solar X-ray flux | Solar X-ray flux | Integrated X-ray flux over the solar disk | NA |
| 148 | >Outer Space>Solar monitoring>Solar X-ray flux spectrum | Solar X-ray flux spectrum | Energy spectrum of the X-ray flux integrated over the solar disk. | NA |
| 149 | >Outer Space>Solar monitoring>Solar X-ray image | Solar X-ray image | Image of the Sun in X-ray wavelengths | NA |
| 150 | >Outer Space>Solar monitoring>Solar coronagraphic image | Solar coronagraphic image | Image of the solar corona surrounding the Sun. | NA |
| 151 | >Outer Space>Solar monitoring>Solar electric field | Solar electric field | Map of magnitude and direction of the electric field at the solar surface (photosphere). | NA |
| 152 | >Outer Space>Solar monitoring>Solar gamma-ray flux | Solar gamma-ray flux | Integrated gamma-ray flux over the solar disk | NA |
| 153 | >Outer Space>Solar monitoring>Solar gamma-ray flux spectrum | Solar gamma-ray flux spectrum | Energy spectrum of the gamma-ray flux integrated over the solar disk | NA |
| 154 | >Outer Space>Solar monitoring>Solar magnetic field | Solar magnetic field | Vector magnetic field (1D or 3D) at the solar surface (photosphere/chromosphere) | NA |
| 155 | >Outer Space>Solar monitoring>Solar radio flux | Solar radio flux | Integrated radio flux over the solar disk. | NA |
| 156 | >Outer Space>Solar monitoring>Solar radio flux spectrum | Solar radio flux spectrum | Solar radio flux energy spectrum integrated over the solar disk. | NA |
| 157 | >Outer Space>Solar monitoring>Solar radio image | Solar radio image | 2D image of the solar radio flux | NA |
| 158 | >Outer Space>Solar monitoring>Solar velocity fields | Solar velocity fields | Map of 3D velocity of particles of the Solar atmosphere, significant of the Sun interior. | NA |
| 159 | >Outer Space>Solar monitoring>Solar white light image | Solar white light image | Image of the Sun in white light | NA |
| 160 | >Outer Space>Solar monitoring>UV flux | UV flux | Integrated UV flux. | NA |
| 161 | >Outer Space>Solar monitoring>UV sky image | UV sky image | UV image of the sky | NA |
| 162 | >Outer Space>Solar monitoring>X-ray flux | X-ray flux | Integrated X-ray flux. | NA |
| 163 | >Outer Space>Solar monitoring>X-ray flux spectrum | X-ray flux spectrum | Energy spectrum of the integrated X-ray flux. | NA |
| 164 | >Outer Space>Solar monitoring>X-ray sky image | X-ray sky image | X-ray image of the sky | NA |
| 165 | >Terrestrial>Ground water>Ground water (amount per unit area) | Ground water (amount per unit area) | Amount of water present beneath the ground surface per unit area | NA |
| 166 | >Terrestrial>Ground water>Ground water (level) | Ground water (level) | NA | NA |
| 167 | >Terrestrial>Lake>Lake area | Lake area | Aera extent of the surface of a lake | NA |
| 168 | >Terrestrial>Lake>Lake level | Lake level | Map of the height of the lake surface. | NA |
| 171 | >Terrestrial>River>River discharge | River discharge | Volume of water flowing through a river per unit of time | NA |
| 172 | >Terrestrial>River>River stage (level above reference) | River stage (level above reference) | NA | NA |
| 174 | >Terrestrial>Well>Well discharge | Well discharge | NA | NA |
| 179 | >Atmosphere>Clouds>Cloud amount | Cloud amount | NA | NA |
| 180 | >Atmosphere>Clouds>Cloud cover | Cloud cover | NA | NA |
| 181 | >Atmosphere>Clouds>Cloud drop effective radius | Cloud drop effective radius | Size distribution of liquid water drops, assimilated to spheres of the same volume. Considered as both a 3D field throughout the troposphere and a 2D field at the top of cloud surface. | NA |
| 182 | >Atmosphere>Clouds>Cloud hydrometeor concentration | Cloud hydrometeor concentration | NA | NA |
| 183 | >Atmosphere>Clouds>Effective radius of cloud hydrometeors | Effective radius of cloud hydrometeors | NA | NA |
| 186 | >Atmosphere>Clouds>Melting layer depth in clouds | Melting layer depth in clouds | Depth of the atmospheric layer in cloud where liquid-solid states transform into each other | NA |
| 188 | >Atmosphere>Clouds>PSC occurrence | PSC occurrence | 3D field of Polar Stratospheric Clouds occurrence. - Accuracy expressed as Hit Rate [ HR ] and False Alarm Rate [ FAR ].Simplified: [ FAR/HR] | NA |
| 192 | >Atmosphere>Greenhouse Gas>CH4 | CH4 | 1D variable colloquially referred to as &ldquo wave spectrum&rdquo. Describes the wave energy in each frequency band (e.g. 25 frequency bands) regardless of the direction of propagation | NA |
| 193 | >Atmosphere>Greenhouse Gas>CH4 [C-13] | CH4 [C-13] | NA | NA |
| 194 | >Atmosphere>Greenhouse Gas>CO2 | CO2 | NA | NA |
| 195 | >Atmosphere>Greenhouse Gas>CO2 [C-13] | CO2 [C-13] | NA | NA |
| 196 | >Atmosphere>Greenhouse Gas>CO2 [O-18] | CO2 [O-18] | NA | NA |
| 201 | >Atmosphere>Greenhouse Gas>N2O | N2O | NA | NA |
| 203 | >Atmosphere>Greenhouse Gas>SF6 | SF6 | NA | NA |
| 204 | >Atmosphere>Greenhouse Gas>SO2F2 (sulphuryl fluoride) | SO2F2 (sulphuryl fluoride) | NA | NA |
| 205 | >Atmosphere>Other Gas>H2 | H2 | NA | NA |
| 206 | >Atmosphere>Other Gas>H2O (as a chemical species) | H2O (as a chemical species) | 3D field of mole fraction of H<sub>2</sub>O = Water vapour (intended as a chemical species relevant for atmospheric chemistry). | NA |
| 207 | >Atmosphere>Other Gas>HDO (as a chemical species) | HDO (as a chemical species) | 3D field of mole fraction of HDO = Water vapour (with one hydrogen nucleus replaced by its deuterium isotope) | NA |
| 210 | >Atmosphere>Precipitation>Amount of precipitation | Amount of precipitation | NA | NA |
| 211 | >Atmosphere>Precipitation>Duration | Duration | NA | NA |
| 212 | >Atmosphere>Precipitation>Intensity of precipitation | Intensity of precipitation | NA | NA |
| 213 | >Atmosphere>Precipitation>Occurrence during last period (yes/no) | Occurrence during last period (yes/no) | NA | NA |
| 214 | >Atmosphere>Precipitation>Rate of ice accretion | Rate of ice accretion | NA | NA |
| 215 | >Atmosphere>Precipitation>Type of precipitation | Type of precipitation | NA | NA |
| 216 | >Atmosphere>Pressure>Atmospheric pressure | Atmospheric pressure | NA | NA |
| 217 | >Atmosphere>Pressure>Characteristic of pressure tendency | Characteristic of pressure tendency | NA | NA |
| 218 | >Atmosphere>Pressure>Pressure tendency | Pressure tendency | NA | NA |
| 219 | >Atmosphere>Radionuclide>Beryllium [Be-7] | Beryllium [Be-7] | NA | NA |
| 220 | >Atmosphere>Radionuclide>CO2 [C-14] | CO2 [C-14] | NA | NA |
| 221 | >Atmosphere>Radionuclide>Krypton [Kr-85] | Krypton [Kr-85] | NA | NA |
| 222 | >Atmosphere>Radionuclide>Lead [Pb-210] | Lead [Pb-210] | NA | NA |
| 223 | >Atmosphere>Radionuclide>Radon [Rn-222] | Radon [Rn-222] | NA | NA |
| 224 | >Atmosphere>Temperature>Air temperature (at specified distance from reference surface) | Air temperature (at specified distance from reference surface) | NA | NA |
| 225 | >Atmosphere>Temperature>Dew-point temperature (at specified distance from reference surface) | Dew-point temperature (at specified distance from reference surface) | NA | NA |
| 226 | >Atmosphere>Temperature>Extreme temperature (min, max) (at specified distance from reference surface) | Extreme temperature (min, max) (at specified distance from reference surface) | NA | NA |
| 227 | >Atmosphere>Temperature>Temperature profile | Temperature profile | NA | NA |
| 229 | >Atmosphere>Visibility>Runway visual range | Runway visual range | NA | NA |
| 230 | >Atmosphere>Visibility>Visibility | Visibility | NA | NA |
| 231 | >Terrestrial>Land surface>Braking action/friction coefficient | Braking action/friction coefficient | NA | NA |
| 232 | >Terrestrial>Land surface>Coastlines | Coastlines | Location of coastlines (Lat/Long) | NA |
| 236 | >Terrestrial>Land surface>Land cover | Land cover | Processed from land surface imagery by assigning identified cluster(s) within a given area to specific classes of objects - Accuracy expressed as number of classes. Actually [ classes-1 ] is used, so that smaller figure corresponds to better performance, as usual. | NA |
| 237 | >Terrestrial>Land surface>Land surface topography | Land surface topography | Map of land surface heights - Physical unit: [ m ] - Accuracy unit: [ m ]. | NA |
| 239 | >Terrestrial>Land surface>Runway conditions | Runway conditions | NA | NA |
| 243 | >Terrestrial>Land surface>Wetland extent | Wetland extent | Fraction of an area flood by water | NA |
| 245 | >Terrestrial>Soil>Soil heat Flux | Soil heat Flux | NA | NA |
| 246 | >Terrestrial>Soil>Soil type | Soil type | Result of the classification of different types of soil within an area - Accuracy expressed as inverse of the number of classes, so that smaller figures correspond to better performance. | NA |
| 248 | >Atmosphere>Dynamics>Transport calculations (e.g., trajectories) | Transport calculations (e.g., trajectories) | NA | NA |
| 249 | >Atmosphere>Dynamics>Turbulence | Turbulence | 3D field of kinetic energy density of turbulent motion of the air | NA |
| 250 | >Atmosphere>Dynamics>Turbulence type (low levels and wake vortex) | Turbulence type (low levels and wake vortex) | NA | NA |
| 251 | >Atmosphere>Humidity>Humidity (at specified distance from reference surface) | Humidity (at specified distance from reference surface) | NA | NA |
| 252 | >Atmosphere>Humidity>Integrated water vapour | Integrated water vapour | NA | NA |
| 253 | >Atmosphere>Humidity>Mass mixing ratio | Mass mixing ratio | NA | NA |
| 254 | >Atmosphere>Humidity>Object wetness duration | Object wetness duration | NA | NA |
| 255 | >Atmosphere>Humidity>Water vapour pressure | Water vapour pressure | NA | NA |
| 256 | >Atmosphere>Humidity>Watervapor profile | Watervapor profile | NA | NA |
| 257 | >Atmosphere>Lightning>Lightning discharge energy | Lightning discharge energy | NA | NA |
| 258 | >Atmosphere>Lightning>Lightning discharge polarity | Lightning discharge polarity | NA | NA |
| 259 | >Atmosphere>Lightning>Lightning discharge rates | Lightning discharge rates | NA | NA |
| 260 | >Atmosphere>Lightning>Lightning discharge type (cloud to cloud, cloud to surface) | Lightning discharge type (cloud to cloud, cloud to surface) | NA | NA |
| 262 | >Atmosphere>Ozone>Surface ozone | Surface ozone | NA | NA |
| 263 | >Atmosphere>Ozone>Total column ozone | Total column ozone | NA | NA |
| 264 | >Atmosphere>Ozone>Vertical ozone profile | Vertical ozone profile | NA | NA |
| 265 | >Atmosphere>Past weather>Past weather | Past weather | NA | NA |
| 266 | >Atmosphere>Present weather>Present weather | Present weather | NA | NA |
| 267 | >Atmosphere>Radiation>Background luminance | Background luminance | NA | NA |
| 270 | >Atmosphere>Radiation>Sunshine duration | Sunshine duration | NA | NA |
| 271 | >Atmosphere>Radiation>Surface albedo | Surface albedo | NA | NA |
| 273 | >Atmosphere>Radiation>UV-B radiation | UV-B radiation | NA | NA |
| 274 | >Atmosphere>Reactive Gas>BrO (bromine monoxide) | BrO (bromine monoxide) | NA | NA |
| 275 | >Atmosphere>Reactive Gas>C2Cl4 (tetrachloroethene) | C2Cl4 (tetrachloroethene) | NA | NA |
| 276 | >Atmosphere>Reactive Gas>C2HCl3 (trichloroethene) | C2HCl3 (trichloroethene) | NA | NA |
| 277 | >Atmosphere>Reactive Gas>CH2Br2 (dibrimomethane) | CH2Br2 (dibrimomethane) | NA | NA |
| 278 | >Atmosphere>Reactive Gas>CH2Cl2 (dichloromethane) | CH2Cl2 (dichloromethane) | NA | NA |
| 279 | >Atmosphere>Reactive Gas>CH3Br (methylbromide) | CH3Br (methylbromide) | NA | NA |
| 280 | >Atmosphere>Reactive Gas>CH3Cl (methylchloride) | CH3Cl (methylchloride) | NA | NA |
| 281 | >Atmosphere>Reactive Gas>CH3I (methyliodide) | CH3I (methyliodide) | NA | NA |
| 282 | >Atmosphere>Reactive Gas>CHBr3 (bromoform) | CHBr3 (bromoform) | NA | NA |
| 283 | >Atmosphere>Reactive Gas>CHCl3 (chloroform) | CHCl3 (chloroform) | NA | NA |
| 284 | >Atmosphere>Reactive Gas>CO | CO | NA | NA |
| 285 | >Atmosphere>Reactive Gas>COF2 (difluoromethanal, carbonyl fluoride) | COF2 (difluoromethanal, carbonyl fluoride) | NA | NA |
| 286 | >Atmosphere>Reactive Gas>ClO (chlorine monoxide) | ClO (chlorine monoxide) | NA | NA |
| 287 | >Atmosphere>Reactive Gas>H2O2 (hydrogen peroxide) | H2O2 (hydrogen peroxide) | NA | NA |
| 288 | >Atmosphere>Reactive Gas>HCl (hydrochloric acid) | HCl (hydrochloric acid) | NA | NA |
| 289 | >Atmosphere>Reactive Gas>HF (hydrofluoric acid) | HF (hydrofluoric acid) | NA | NA |
| 291 | >Atmosphere>Reactive Gas>ROOH | ROOH | NA | NA |
| 294 | >Atmosphere>Total Atmospheric Deposition>Acidity/Alkalinity | Acidity/Alkalinity | NA | NA |
| 295 | >Atmosphere>Total Atmospheric Deposition>Electric conductivity | Electric conductivity | NA | NA |
| 296 | >Atmosphere>Total Atmospheric Deposition>Hydrogen ion (H+) or pH | Hydrogen ion (H+) or pH | NA | NA |
| 302 | >Atmosphere>Total Atmospheric Deposition>Sulphur (S) | Sulphur (S) | NA | NA |
| 304 | >Atmosphere>Wind>Direction of cloud movement | Direction of cloud movement | NA | NA |
| 305 | >Atmosphere>Wind>Gust Speed | Gust Speed | NA | NA |
| 307 | >Atmosphere>Wind>Wind (Z component, vertical) | Wind (Z component, vertical) | Z component of wind vector (horizontal and vertical profile) | NA |
| 309 | >Atmosphere>Wind>Wind (surface wind direction and speed, horizontal) | Wind (surface wind direction and speed, horizontal) | NA | NA |
| 310 | >Atmosphere>Wind>Wind (X, Y components, horizontal) | Wind (X, Y components, horizontal) | NA | NA |
| 311 | >Ocean>Biology>Nitrogen species>NO2- (nitrite) | NO2- (nitrite) | nitrite concentration in sea water | oceanNitrite |
| 312 | >Ocean>Biology>Nitrogen species>NO3- (nitrate) | NO3- (nitrate) | nitrate concentration in sea water | oceanNitrate |
| 314 | >Atmosphere>Aerosol>Optical properties>Aerosol Absorption Optical Depth | Aerosol Absorption Optical Depth | Vertical column integral of spectral aerosol absorption coefficient: AAOD = exp(-K. &Delta z) where K is the absorption coefficient [km<sup>-1</sup>] and &Delta z the vertical path [km] | NA |
| 315 | >Atmosphere>Aerosol>Optical properties>Aerosol Extinction Coefficient | Aerosol Extinction Coefficient | 3D field of spectral volumetric extinction cross-section of aerosol particles. | NA |
| 316 | >Atmosphere>Aerosol>Optical properties>Light absorption coefficient, PM1 | Light absorption coefficient, PM1 | NA | NA |
| 317 | >Atmosphere>Aerosol>Optical properties>Light absorption coefficient, PM10 | Light absorption coefficient, PM10 | NA | NA |
| 318 | >Atmosphere>Aerosol>Optical properties>Light absorption coefficient, total aerosol | Light absorption coefficient, total aerosol | NA | NA |
| 319 | >Atmosphere>Aerosol>Optical properties>Light backscattering coefficient, PM1 | Light backscattering coefficient, PM1 | NA | NA |
| 320 | >Atmosphere>Aerosol>Optical properties>Light backscattering coefficient, PM10 | Light backscattering coefficient, PM10 | NA | NA |
| 321 | >Atmosphere>Aerosol>Optical properties>Light backscattering coefficient, total aerosol | Light backscattering coefficient, total aerosol | NA | NA |
| 322 | >Atmosphere>Aerosol>Optical properties>Light scattering coefficient, PM1 | Light scattering coefficient, PM1 | NA | NA |
| 323 | >Atmosphere>Aerosol>Optical properties>Light scattering coefficient, PM10 | Light scattering coefficient, PM10 | NA | NA |
| 324 | >Atmosphere>Aerosol>Optical properties>Light scattering coefficient, total aerosol | Light scattering coefficient, total aerosol | NA | NA |
| 325 | >Atmosphere>Aerosol>Optical properties>Multiwavelength optical depth, total aerosol | Multiwavelength optical depth, total aerosol | NA | NA |
| 326 | >Atmosphere>Aerosol>Optical properties>Vertical distribution of properties | Vertical distribution of properties | NA | NA |
| 327 | >Atmosphere>Clouds>Ice>Cloud ice | Cloud ice | 3D field of atmospheric water in the solid phase (precipitating or not). | NA |
| 328 | >Atmosphere>Clouds>Ice>Cloud ice (total column) | Cloud ice (total column) | Field of atmospheric water in the solid phase (precipitating or not) as total column. | NA |
| 329 | >Atmosphere>Clouds>Ice>Cloud ice effective radius | Cloud ice effective radius | Size distribution of ice particles, assimilated to spheres of the same volume. Considered as both a 3D field throughout the troposphere and a 2D field at the top of cloud surface | NA |
| 330 | >Atmosphere>Clouds>Ice>Freezing level height in clouds | Freezing level height in clouds | Height of the atmospheric layer in cloud where liquid-solid states transform into each other | NA |
| 331 | >Atmosphere>Clouds>Ice>Icing potential | Icing potential | 3D field of super-cooled liquid water, enabling the formation of ice coating on an object. | NA |
| 332 | >Atmosphere>Greenhouse Gas>Halon>CBrClF2 (Halon 1211) | CBrClF2 (Halon 1211) | NA | NA |
| 333 | >Atmosphere>Greenhouse Gas>Halon>CBrF3 (bromotrifluoromethane, Halon 1301) | CBrF3 (bromotrifluoromethane, Halon 1301) | NA | NA |
| 334 | >Atmosphere>POPs>PAH>C13H10 (9H-fluorene, fluorene), in air | C13H10 (9H-fluorene, fluorene), in air | NA | NA |
| 335 | >Atmosphere>POPs>PAH>C14H10 (anthracene), PM10 | C14H10 (anthracene), PM10 | NA | NA |
| 336 | >Atmosphere>POPs>PAH>C14H10 (anthracene), in aerosol | C14H10 (anthracene), in aerosol | NA | NA |
| 337 | >Atmosphere>POPs>PAH>C14H10 (anthracene), in aerosol and air | C14H10 (anthracene), in aerosol and air | NA | NA |
| 338 | >Atmosphere>POPs>PAH>C14H10 (anthracene), in air | C14H10 (anthracene), in air | NA | NA |
| 339 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), PM10 | C14H10 (phenanthrene), PM10 | NA | NA |
| 340 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), in aerosol | C14H10 (phenanthrene), in aerosol | NA | NA |
| 341 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), in air | C14H10 (phenanthrene), in air | NA | NA |
| 342 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), in air and aerosol | C14H10 (phenanthrene), in air and aerosol | NA | NA |
| 343 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), in precipitation | C14H10 (phenanthrene), in precipitation | NA | NA |
| 344 | >Atmosphere>POPs>PAH>C14H10 (phenanthrene), in total precipitation | C14H10 (phenanthrene), in total precipitation | NA | NA |
| 345 | >Atmosphere>POPs>PAH>C16H10 (pyrene), in air and aerosol | C16H10 (pyrene), in air and aerosol | NA | NA |
| 346 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), PM10 | C18H12 (Benz(a)anthracene), PM10 | NA | NA |
| 347 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), in aerosol | C18H12 (Benz(a)anthracene), in aerosol | NA | NA |
| 348 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), in air | C18H12 (Benz(a)anthracene), in air | NA | NA |
| 349 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), in air and aerosol | C18H12 (Benz(a)anthracene), in air and aerosol | NA | NA |
| 350 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), in precipitation | C18H12 (Benz(a)anthracene), in precipitation | NA | NA |
| 351 | >Atmosphere>POPs>PAH>C18H12 (Benz(a)anthracene), in total precipitation | C18H12 (Benz(a)anthracene), in total precipitation | NA | NA |
| 354 | >Terrestrial>Land surface>Snow / Ice / Glacier>Permafrost | Permafrost | NA | NA |
| 356 | >Atmosphere>Radiation>UV>UV Broadband | UV Broadband | NA | NA |
| 357 | >Atmosphere>Radiation>UV>UV Erythemally weighted | UV Erythemally weighted | NA | NA |
| 358 | >Atmosphere>Radiation>UV>UV Multiband | UV Multiband | NA | NA |
| 359 | >Atmosphere>Radiation>UV>UV Spectral | UV Spectral | NA | NA |
| 361 | >Atmosphere>Aerosol>Physical properties - primary>Aerosol column burden (mass density) | Aerosol column burden (mass density) | 2D field of the column burden of condensed particles in the atmosphere | NA |
| 362 | >Atmosphere>Aerosol>Physical properties - primary>Aerosol effective radius | Aerosol effective radius | 3D field of mean aerosol particle size, defined as the ratio of the third and second moments of the number size distribution of aerosol particles. Requested in the troposphere (assumed height: 12 km) and as columnar average. | NA |
| 364 | >Atmosphere>Aerosol>Physical properties - primary>Mass concentration (coarse) or Mass PM10 | Mass concentration (coarse) or Mass PM10 | NA | NA |
| 365 | >Atmosphere>Aerosol>Physical properties - primary>Mass concentration (fine) or Mass PM1 | Mass concentration (fine) or Mass PM1 | NA | NA |
| 366 | >Atmosphere>Aerosol>Physical properties - primary>Mass concentration (medium), PM2.5 | Mass concentration (medium), PM2.5 | NA | NA |
| 367 | >Atmosphere>Aerosol>Physical properties - primary>Mass concentration (size fractionated) | Mass concentration (size fractionated) | NA | NA |
| 368 | >Atmosphere>Aerosol>Physical properties - primary>Mass concentration (total aerosol) or Mass TSP | Mass concentration (total aerosol) or Mass TSP | NA | NA |
| 369 | >Atmosphere>Aerosol>Physical properties - primary>Number concentration | Number concentration | NA | NA |
| 370 | >Atmosphere>Aerosol>Physical properties - primary>Number size distribution, PM10 | Number size distribution, PM10 | NA | NA |
| 371 | >Atmosphere>Aerosol>Physical properties - primary>Number size distribution, total aerosol | Number size distribution, total aerosol | NA | NA |
| 373 | >Atmosphere>Clouds>Liquid water>Cloud liquid water (CLW) | Cloud liquid water (CLW) | 3D field of atmospheric water in the liquid phase (precipitating or not). | NA |
| 374 | >Atmosphere>Clouds>Liquid water>Cloud liquid water (CLW, total column) | Cloud liquid water (CLW, total column) | Field of atmospheric water in the liquid phase (precipitating or not). | NA |
| 375 | >Atmosphere>POPs>POP>C12H8Cl6 (aldrin), in aerosol | C12H8Cl6 (aldrin), in aerosol | NA | NA |
| 376 | >Atmosphere>POPs>POP>C12H8Cl6 (aldrin), in air | C12H8Cl6 (aldrin), in air | NA | NA |
| 377 | >Atmosphere>POPs>POP>C12H8Cl6 (aldrin), in air and aerosol | C12H8Cl6 (aldrin), in air and aerosol | NA | NA |
| 378 | >Atmosphere>POPs>POP>C12H8Cl6 (aldrin), in total precipitation | C12H8Cl6 (aldrin), in total precipitation | NA | NA |
| 379 | >Atmosphere>POPs>POP>C12H8Cl6 (aldrin), in wet precipitation | C12H8Cl6 (aldrin), in wet precipitation | NA | NA |
| 380 | >Atmosphere>POPs>POP>C6Cl6 (hexachlorobenzene, HCB), in air | C6Cl6 (hexachlorobenzene, HCB), in air | NA | NA |
| 381 | >Atmosphere>POPs>POP>C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in aerosol | C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in aerosol | NA | NA |
| 382 | >Atmosphere>POPs>POP>C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in air | C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in air | NA | NA |
| 383 | >Atmosphere>POPs>POP>C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in wet precipitation | C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, alpha-lindane, alpha-HCH), in wet precipitation | NA | NA |
| 384 | >Atmosphere>POPs>POP>C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, gamma-lindane, gamma-HCH), in air | C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, gamma-lindane, gamma-HCH), in air | NA | NA |
| 385 | >Atmosphere>POPs>POP>C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, gamma-lindane, gamma-HCH), in wet precipitation | C6H6Cl6 (1,2,3,4,5,6-hexachlorocyclohexane, gamma-lindane, gamma-HCH), in wet precipitation | NA | NA |
| 386 | >Terrestrial>Land surface>Fire>Fire fractional cover | Fire fractional cover | Fraction of a land area where fire is occurring | NA |
| 387 | >Terrestrial>Land surface>Fire>Fire radiative power | Fire radiative power | Power radiated by the fire occurring within an area | NA |
| 388 | >Terrestrial>Land surface>Fire>Fire temperature | Fire temperature | Temperature of the fire occurring within an area | NA |
| 389 | >Terrestrial>Land surface>Temperature>Land surface temperature | Land surface temperature | Temperature of the apparent surface of land (bare soil or vegetation) - Physical unit: [ K ] - Accuracy unit: [ K ]. | NA |
| 390 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>Ammonia (NH3) | Ammonia (NH3) | NA | NA |
| 391 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>C2H3O5N (peroxyacetylnitrate, PAN) | C2H3O5N (peroxyacetylnitrate, PAN) | NA | NA |
| 392 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>ClONO2 (chlorine nitrate) | ClONO2 (chlorine nitrate) | NA | NA |
| 393 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>HCN (formonitrile, hydrogen cyanide) | HCN (formonitrile, hydrogen cyanide) | NA | NA |
| 394 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>HNO3 (nitric acid) | HNO3 (nitric acid) | NA | NA |
| 395 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>NO | NO | NA | NA |
| 396 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>NO2 | NO2 | NA | NA |
| 397 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>NOx | NOx | NA | NA |
| 398 | >Atmosphere>Reactive Gas>Nitrogen containing compounds>NOy | NOy | NA | NA |
| 399 | >Ocean>Biology>Phosporus species>PO4= (phosphate) | PO4= (phosphate) | phosphate conetration in sea water | oceanPhosphate |
| 400 | >Ocean>Biology>Phosporus species>Total organic phosphorus | Total organic phosphorus | NA | oceanTotalOrganicPhosphorous |
| 401 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice cover | Sea-ice cover | Fraction of an ocean area where ice is present | seaIceCover |
| 402 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice elevation | Sea-ice elevation | Elevation of the surface of the sea-ice sheet above sea level | seaIceElevation |
| 403 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice motion | Sea-ice motion | Sea-ice motion | seaIceMotion |
| 404 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice surface characteristics | Sea-ice surface characteristics | Sea-ice surface characteristics (albedo, meltpond, dust, snow properties, temperature) | seaIceSurfaceCharacteristics |
| 405 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice surface temperature | Sea-ice surface temperature | Temperature of the surface of sea-ice. | seaIceSurfaceTemperature |
| 406 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice thickness | Sea-ice thickness | Thickness of the ice sheet. It is related to sea-ice elevation and ice density | seaIceThickness |
| 407 | >Ocean>Snow / Ice / Glacier>Ice>Sea-ice type | Sea-ice type | Variable convolving several factors (age, roughness, density, etc.) - Accuracy expressed as number of classes. Actually [ classes^-1 ] is used, so that smaller figure corresponds to better performance as usual. | seaIceType |
| 408 | >Ocean>Transmissivity>CTD>Transmissivity | Transmissivity | NA | oceanTransmissivity |
| 409 | >Outer Space>Energetic particles / solar wind >Particle density>Solar wind density | Solar wind density | Density of solar wind plasma | NA |
| 410 | >Terrestrial>Lake>Temperature>Lake Surface Temperature | Lake Surface Temperature | Temperature of the lake surface - Physical unit: [ K ] - Accuracy unit: [ K ]. | NA |
| 411 | >Atmosphere>Aerosol>Physical properties - secondary>CCN concentration at single supersaturation | CCN concentration at single supersaturation | NA | NA |
| 412 | >Atmosphere>Aerosol>Physical properties - secondary>CCN concentration spectra | CCN concentration spectra | NA | NA |
| 413 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, 110 μm equivalent | Hygroscopic growth factor, 110 μm equivalent | NA | NA |
| 414 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, 165 μm equivalent | Hygroscopic growth factor, 165 μm equivalent | NA | NA |
| 415 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, 35 μm equivalent | Hygroscopic growth factor, 35 μm equivalent | NA | NA |
| 416 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, 50 μm equivalent | Hygroscopic growth factor, 50 μm equivalent | NA | NA |
| 417 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, 75 μm equivalent | Hygroscopic growth factor, 75 μm equivalent | NA | NA |
| 418 | >Atmosphere>Aerosol>Physical properties - secondary>Hygroscopic growth factor, total aerosol | Hygroscopic growth factor, total aerosol | NA | NA |
| 419 | >Atmosphere>Greenhouse Gas>PFCs>C2F6 (hexafluoroethane, PFC-116) | C2F6 (hexafluoroethane, PFC-116) | NA | NA |
| 420 | >Atmosphere>Greenhouse Gas>PFCs>C3F8 (octafluoropropane, PFC-218) | C3F8 (octafluoropropane, PFC-218) | NA | NA |
| 421 | >Atmosphere>Greenhouse Gas>PFCs>CF4 (tetrafluoromethane, carbon tetrafluoride, perfluoromethane, PFC-14) | CF4 (tetrafluoromethane, carbon tetrafluoride, perfluoromethane, PFC-14) | NA | NA |
| 422 | >Terrestrial>Land surface>Greenhouse Gas>CO2 flux | CO2 flux | Flux of carbon dioxide from the surface to the atmosphere | NA |
| 423 | >Terrestrial>Land surface>Vegetation>Biomass | Biomass | Total amount of vegetation in a reference area - Physical unit: [ t/ha (tons/hectare) ] - Accuracy unit: [ t/ha ] | NA |
| 424 | >Terrestrial>Land surface>Vegetation>Fraction of vegetated land | Fraction of vegetated land | Fraction of a land area where vegetation is present | NA |
| 425 | >Terrestrial>Land surface>Vegetation>Leaf Area Index (LAI) | Leaf Area Index (LAI) | LAI is the total one-sided area of photosynthetic tissue per unit ground surface area | NA |
| 426 | >Terrestrial>Land surface>Vegetation>Normalised Difference Vegetation Index (NDVI) | Normalised Difference Vegetation Index (NDVI) | Difference between maximum (in NIR) and minimum (around the Red) vegetation reflectance, normalised to the summation. Representative of total biomass, supportive for computing LAI if not directly measured | NA |
| 427 | >Terrestrial>Land surface>Vegetation>Vegetation type | Vegetation type | Result of the classification of different types of vegetation within a vegetated area - Accuracy expressed as inverse of the number of classes, so that smaller figures correspond to better performance. | NA |
| 428 | >Atmosphere>Reactive Gas>Sulfur containing compounds>COS (OCS, carbon oxide sulfide, carbonyl sulfide) | COS (OCS, carbon oxide sulfide, carbonyl sulfide) | NA | NA |
| 429 | >Atmosphere>Reactive Gas>Sulfur containing compounds>CS2 (Carbon disulfide) | CS2 (Carbon disulfide) | NA | NA |
| 430 | >Atmosphere>Reactive Gas>Sulfur containing compounds>SO2 | SO2 | NA | NA |
| 431 | >Atmosphere>Lightning>Position>Lightning detection (time and location) | Lightning detection (time and location) | Detection of the time and location (latitude, longitude) of lightning events. Accuracy expressed in terms of Hit Rate and False Alarm Rate, which requires predetermination of a specific distance and time tolerance . | NA |
| 432 | >Atmosphere>Lightning>Position>Lightning direction from station | Lightning direction from station | NA | NA |
| 433 | >Atmosphere>Lightning>Position>Lightning distance from station | Lightning distance from station | NA | NA |
| 434 | >Atmosphere>Reactive Gas>VOC>C2H2 (ethyne, acetylene) | C2H2 (ethyne, acetylene) | NA | NA |
| 435 | >Atmosphere>Reactive Gas>VOC>C2H2O2 (oxaldehyde, ethanedial) | C2H2O2 (oxaldehyde, ethanedial) | NA | NA |
| 436 | >Atmosphere>Reactive Gas>VOC>C2H4 (ethene) | C2H4 (ethene) | NA | NA |
| 437 | >Atmosphere>Reactive Gas>VOC>C2H6 (ethane) | C2H6 (ethane) | NA | NA |
| 438 | >Atmosphere>Reactive Gas>VOC>C2H6S (dimethylsulfide, DMS) | C2H6S (dimethylsulfide, DMS) | NA | NA |
| 439 | >Atmosphere>Reactive Gas>VOC>C3H4 (propyne) | C3H4 (propyne) | NA | NA |
| 440 | >Atmosphere>Reactive Gas>VOC>C3H4O (prop-2-enal, acrolein) | C3H4O (prop-2-enal, acrolein) | NA | NA |
| 441 | >Atmosphere>Reactive Gas>VOC>C3H4O2 (2-oxopropanal, methylglyoxal) | C3H4O2 (2-oxopropanal, methylglyoxal) | NA | NA |
| 442 | >Atmosphere>Reactive Gas>VOC>C3H6 (propene) | C3H6 (propene) | NA | NA |
| 443 | >Atmosphere>Reactive Gas>VOC>C3H6O (acetone, propanone) | C3H6O (acetone, propanone) | NA | NA |
| 444 | >Atmosphere>Reactive Gas>VOC>C3H6O (propanal) | C3H6O (propanal) | NA | NA |
| 445 | >Atmosphere>Reactive Gas>VOC>C3H8 (propane) | C3H8 (propane) | NA | NA |
| 446 | >Atmosphere>Reactive Gas>VOC>C4H6 (1,3-butadiene, butadiene) | C4H6 (1,3-butadiene, butadiene) | NA | NA |
| 447 | >Atmosphere>Reactive Gas>VOC>C4H6O (2-methylprop-2-enal, methacrolein) | C4H6O (2-methylprop-2-enal, methacrolein) | NA | NA |
| 448 | >Atmosphere>Reactive Gas>VOC>C4H6O (but-3-en-2-one, methylvinylketone) | C4H6O (but-3-en-2-one, methylvinylketone) | NA | NA |
| 449 | >Atmosphere>Reactive Gas>VOC>C4H8 (1-butene) | C4H8 (1-butene) | NA | NA |
| 450 | >Atmosphere>Reactive Gas>VOC>C4H8 (sum of butenes) | C4H8 (sum of butenes) | NA | NA |
| 451 | >Atmosphere>Reactive Gas>VOC>C4H8O (butan-2-one, methylethylketone) | C4H8O (butan-2-one, methylethylketone) | NA | NA |
| 452 | >Atmosphere>Reactive Gas>VOC>C4H8O (butanal isomers) | C4H8O (butanal isomers) | NA | NA |
| 453 | >Atmosphere>Reactive Gas>VOC>C5H10 (1-pentene) | C5H10 (1-pentene) | NA | NA |
| 454 | >Atmosphere>Reactive Gas>VOC>C5H10 (2-methyl-1-butene) | C5H10 (2-methyl-1-butene) | NA | NA |
| 455 | >Atmosphere>Reactive Gas>VOC>C5H10 (2-methyl-2-butene) | C5H10 (2-methyl-2-butene) | NA | NA |
| 456 | >Atmosphere>Reactive Gas>VOC>C5H10 (3-methyl-1-butene) | C5H10 (3-methyl-1-butene) | NA | NA |
| 457 | >Atmosphere>Reactive Gas>VOC>C5H10 (cyclopentane) | C5H10 (cyclopentane) | NA | NA |
| 458 | >Atmosphere>Reactive Gas>VOC>C5H10 (sum of 1-pentene & 2-methyl-2-butene) | C5H10 (sum of 1-pentene & 2-methyl-2-butene) | NA | NA |
| 459 | >Atmosphere>Reactive Gas>VOC>C5H10 (sum of pentenes) | C5H10 (sum of pentenes) | NA | NA |
| 460 | >Atmosphere>Reactive Gas>VOC>C5H10O (pentanal, valeraldehyde) | C5H10O (pentanal, valeraldehyde) | NA | NA |
| 461 | >Atmosphere>Reactive Gas>VOC>C5H12 & C4H8 (sum of 2,2-dimethylpropane & c-2-butene) | C5H12 & C4H8 (sum of 2,2-dimethylpropane & c-2-butene) | NA | NA |
| 462 | >Atmosphere>Reactive Gas>VOC>C5H12 (2,2-dimethylpropane) | C5H12 (2,2-dimethylpropane) | NA | NA |
| 463 | >Atmosphere>Reactive Gas>VOC>C5H14 (2,3-dimethylbutane) | C5H14 (2,3-dimethylbutane) | NA | NA |
| 464 | >Atmosphere>Reactive Gas>VOC>C5H8 (2-methyl-1,3-butadiene, isoprene) | C5H8 (2-methyl-1,3-butadiene, isoprene) | NA | NA |
| 465 | >Atmosphere>Reactive Gas>VOC>C5H8 (cyclopentene) | C5H8 (cyclopentene) | NA | NA |
| 466 | >Atmosphere>Reactive Gas>VOC>C5Hn (n={10, 8}; sum of 3-methyl-1-butene & cyclopentene) | C5Hn (n={10, 8}; sum of 3-methyl-1-butene & cyclopentene) | NA | NA |
| 467 | >Atmosphere>Reactive Gas>VOC>C5Hn (n={12, 10}; sum of n-pentane & cyclopentane) | C5Hn (n={12, 10}; sum of n-pentane & cyclopentane) | NA | NA |
| 468 | >Atmosphere>Reactive Gas>VOC>C6H12 (cyclohexane) | C6H12 (cyclohexane) | NA | NA |
| 469 | >Atmosphere>Reactive Gas>VOC>C6H12 (methylcyclopentane) | C6H12 (methylcyclopentane) | NA | NA |
| 470 | >Atmosphere>Reactive Gas>VOC>C6H12O (hexanal isomers) | C6H12O (hexanal isomers) | NA | NA |
| 471 | >Atmosphere>Reactive Gas>VOC>C6H14 (2,2-dimethylbutane) | C6H14 (2,2-dimethylbutane) | NA | NA |
| 472 | >Atmosphere>Reactive Gas>VOC>C6H14 (2-methylpentane) | C6H14 (2-methylpentane) | NA | NA |
| 473 | >Atmosphere>Reactive Gas>VOC>C6H14 (3-methylpentane) | C6H14 (3-methylpentane) | NA | NA |
| 474 | >Atmosphere>Reactive Gas>VOC>C6H14 (n-hexane) | C6H14 (n-hexane) | NA | NA |
| 475 | >Atmosphere>Reactive Gas>VOC>C6H14 (sum of methylpentanes) | C6H14 (sum of methylpentanes) | NA | NA |
| 476 | >Atmosphere>Reactive Gas>VOC>C6H6 (benzene) | C6H6 (benzene) | NA | NA |
| 477 | >Atmosphere>Reactive Gas>VOC>C6Hn (n={14, 12}; sum of 3-methylpentane & cyclohexane) | C6Hn (n={14, 12}; sum of 3-methylpentane & cyclohexane) | NA | NA |
| 478 | >Atmosphere>Reactive Gas>VOC>C6Hn (n={14, 12}; sum of 2,2-dimethylbutane & methylcyclopentane | C6Hn (n={14, 12}; sum of 2,2-dimethylbutane & methylcyclopentane | NA | NA |
| 479 | >Atmosphere>Reactive Gas>VOC>C6Hn (n={14, 12}; sum of 2,3-dimethylbutane & methylcyclopentane | C6Hn (n={14, 12}; sum of 2,3-dimethylbutane & methylcyclopentane | NA | NA |
| 480 | >Atmosphere>Reactive Gas>VOC>C7H16 (sum of methylhexanes) | C7H16 (sum of methylhexanes) | NA | NA |
| 481 | >Atmosphere>Reactive Gas>VOC>C7H6O (benzene carbaldehyde) | C7H6O (benzene carbaldehyde) | NA | NA |
| 482 | >Atmosphere>Reactive Gas>VOC>C7H8 (toluene) | C7H8 (toluene) | NA | NA |
| 483 | >Atmosphere>Reactive Gas>VOC>C8H10 (ethylbenzene) | C8H10 (ethylbenzene) | NA | NA |
| 484 | >Atmosphere>Reactive Gas>VOC>C8H10 (m-xylene) | C8H10 (m-xylene) | NA | NA |
| 485 | >Atmosphere>Reactive Gas>VOC>C8H10 (o-xylene) | C8H10 (o-xylene) | NA | NA |
| 486 | >Atmosphere>Reactive Gas>VOC>C8H10 (p,m-xylene) | C8H10 (p,m-xylene) | NA | NA |
| 487 | >Atmosphere>Reactive Gas>VOC>C9H12 (1,2,4-trimethylbenzene) | C9H12 (1,2,4-trimethylbenzene) | NA | NA |
| 488 | >Atmosphere>Reactive Gas>VOC>C9H12 (1,3,5-trimethylbenzene) | C9H12 (1,3,5-trimethylbenzene) | NA | NA |
| 489 | >Atmosphere>Reactive Gas>VOC>CH2O (formaldehyde) | CH2O (formaldehyde) | NA | NA |
| 490 | >Atmosphere>Reactive Gas>VOC>CH3CH2OH (ethanol) | CH3CH2OH (ethanol) | NA | NA |
| 491 | >Atmosphere>Reactive Gas>VOC>CH3CHO (acetaldehyde, ethanal) | CH3CHO (acetaldehyde, ethanal) | NA | NA |
| 492 | >Atmosphere>Reactive Gas>VOC>CH3CN (acetonitrile) | CH3CN (acetonitrile) | NA | NA |
| 493 | >Atmosphere>Reactive Gas>VOC>CH3OH (methanol) | CH3OH (methanol) | NA | NA |
| 494 | >Atmosphere>Reactive Gas>VOC>NMHC | NMHC | NA | NA |
| 495 | >Atmosphere>Reactive Gas>VOC>c-C4H8 (cis-2-butene) | c-C4H8 (cis-2-butene) | NA | NA |
| 496 | >Atmosphere>Reactive Gas>VOC>c-C5H10 (cis-2-pentene) | c-C5H10 (cis-2-pentene) | NA | NA |
| 497 | >Atmosphere>Reactive Gas>VOC>i-C4H10 (2-methylpropane, iso-butane) | i-C4H10 (2-methylpropane, iso-butane) | NA | NA |
| 498 | >Atmosphere>Reactive Gas>VOC>i-C4H8 (2-methylpropene, iso-butene) | i-C4H8 (2-methylpropene, iso-butene) | NA | NA |
| 499 | >Atmosphere>Reactive Gas>VOC>i-C5H12 (2-methylbutane, iso-pentane) | i-C5H12 (2-methylbutane, iso-pentane) | NA | NA |
| 500 | >Atmosphere>Reactive Gas>VOC>i-C7H16 (2-methylhexane, isoheptane) | i-C7H16 (2-methylhexane, isoheptane) | NA | NA |
| 501 | >Atmosphere>Reactive Gas>VOC>n-C4H10 (n-butane) | n-C4H10 (n-butane) | NA | NA |
| 502 | >Atmosphere>Reactive Gas>VOC>n-C5H12 (n-pentane) | n-C5H12 (n-pentane) | NA | NA |
| 503 | >Atmosphere>Reactive Gas>VOC>n-C7H16 (n-heptane) | n-C7H16 (n-heptane) | NA | NA |
| 504 | >Atmosphere>Reactive Gas>VOC>t-C4H8 (trans-2-butene) | t-C4H8 (trans-2-butene) | NA | NA |
| 505 | >Atmosphere>Reactive Gas>VOC>t-C5H10 (trans-2-pentene) | t-C5H10 (trans-2-pentene) | NA | NA |
| 506 | >Atmosphere>Clouds>Optical properties>Cloud optical depth | Cloud optical depth | Effective depth of a cloud from the viewpoint of radiation propagation. OD = exp(-K.&Deltaz) where K is the extinction coefficient [km<sup>-1</sup> ] and &Delta z the vertical path [km] between the base and the top of the cloud | NA |
| 507 | >Atmosphere>Clouds>Optical properties>Optical depth of fog | Optical depth of fog | NA | NA |
| 508 | >Atmosphere>Clouds>Optical properties>Optical depth within each layer | Optical depth within each layer | NA | NA |
| 509 | >Atmosphere>Clouds>Optical properties>Short-wave cloud reflectance | Short-wave cloud reflectance | Reflectance of the solar radiation from clouds | NA |
| 510 | >Terrestrial>Land surface>Humidity>Evaporation | Evaporation | Quantity of water evaporated from the soil and plants when the ground is at its natural moisture content. | NA |
| 511 | >Outer Space>Energetic particles / solar wind >Particle flux>Alpha particles differential directional flux | Alpha particles differential directional flux | NA | NA |
| 512 | >Outer Space>Energetic particles / solar wind >Particle flux>Alpha particles integral directional flux | Alpha particles integral directional flux | An alpha particle has positive charge and consists of two protons and two neutrons (the nucleus of a helium atom). Flux is the rate of flow through a reference surface, measured in particles per unit area.The directional flux is the flux limited to a certain solid angle as a function of the direction (pitch angle). | NA |
| 513 | >Outer Space>Energetic particles / solar wind >Particle flux>Cosmic ray neutron flux spectrum | Cosmic ray neutron flux spectrum | Flux density energy spectrum of neutrons from the Sun or as component of the cosmic radiation. | NA |
| 514 | >Outer Space>Energetic particles / solar wind >Particle flux>Electron differential directional flux | Electron differential directional flux | Flux density energy spectrum of low-, medium-, and high-energy electrons from the magnetosphere, the radiation belts or the interplanetary medium, per unit solid angle | NA |
| 515 | >Outer Space>Energetic particles / solar wind >Particle flux>Electron flux density | Electron flux density | Flux density of low-, medium-, and high-energy electrons from the magnetosphere, the radiation belts or the interplanetary medium. | NA |
| 516 | >Outer Space>Energetic particles / solar wind >Particle flux>Electron integral directional flux | Electron integral directional flux | Flux density of low-, medium-, and high-energy electrons from the magnetosphere, the radiation belts or the interplanetary medium, per unit solid angle. | NA |
| 517 | >Outer Space>Energetic particles / solar wind >Particle flux>Heavy ion differential directional flux | Heavy ion differential directional flux | Flux density of heavy ions (eavier than Helium) per unit solid angle | NA |
| 518 | >Outer Space>Energetic particles / solar wind >Particle flux>Heavy ion integral directional flux | Heavy ion integral directional flux | Flux density of heavy ions ranging from Helium to Iron. | NA |
| 519 | >Outer Space>Energetic particles / solar wind >Particle flux>Proton differential directional flux | Proton differential directional flux | Flux density energy spectrum of low-, medium-, and high-energy protons from the magnetosphere, the radiation belts or the interplanetary medium, per unit solid angle. | NA |
| 520 | >Outer Space>Energetic particles / solar wind >Particle flux>Proton integral directional flux | Proton integral directional flux | Flux density of low-, medium-, and high-energy protons from the magnetosphere, the radiation belts or the interplanetary medium. | NA |
| 521 | >Atmosphere>Visibility>Obscurations>Extinction coefficient | Extinction coefficient | NA | NA |
| 522 | >Atmosphere>Visibility>Obscurations>Hydrometeor radius | Hydrometeor radius | NA | NA |
| 523 | >Atmosphere>Visibility>Obscurations>Hydrometeor type | Hydrometeor type | NA | NA |
| 524 | >Atmosphere>Visibility>Obscurations>Lithometeor type | Lithometeor type | NA | NA |
| 525 | >Atmosphere>Visibility>Obscurations>Meteorological Optical Range | Meteorological Optical Range | NA | NA |
| 526 | >Atmosphere>Visibility>Obscurations>Obscuration type | Obscuration type | NA | NA |
| 527 | >Terrestrial>Soil>Humidity>Soil moisture (in the roots region) | Soil moisture (in the roots region) | Sub-soil 3D field of the fractional content of water in a volume of wet soil. Requested from surface down to ~ 3 m | NA |
| 528 | >Terrestrial>Soil>Humidity>Soil moisture at surface | Soil moisture at surface | Fractional content of water in a volume of wet soil. Surface layer (upper few centimetres) | NA |
| 529 | >Atmosphere>Total Atmospheric Deposition>Inorganic acid>Strong acids | Strong acids | NA | NA |
| 531 | >Atmosphere>Clouds>Position>Height of cloud base | Height of cloud base | NA | NA |
| 532 | >Atmosphere>Clouds>Position>Height of cloud top | Height of cloud top | NA | NA |
| 533 | >Atmosphere>Clouds>Position>Height of inversion | Height of inversion | NA | NA |
| 534 | >Atmosphere>Greenhouse Gas>HCFCs>C2H3Cl2F (1,1-dichloro-1-fluoroethane, HCFC-141b) | C2H3Cl2F (1,1-dichloro-1-fluoroethane, HCFC-141b) | NA | NA |
| 535 | >Atmosphere>Greenhouse Gas>HCFCs>C2H3ClF2 (1-chloro-1,1-difluoroethane, HCFC-142b) | C2H3ClF2 (1-chloro-1,1-difluoroethane, HCFC-142b) | NA | NA |
| 536 | >Atmosphere>Greenhouse Gas>HCFCs>C2H3F3 (1,1,1-trifluoroethane, HCFC-143a) | C2H3F3 (1,1,1-trifluoroethane, HCFC-143a) | NA | NA |
| 537 | >Atmosphere>Greenhouse Gas>HCFCs>CHClF2 (chlorodifluoromethane, HCFC-22) | CHClF2 (chlorodifluoromethane, HCFC-22) | NA | NA |
| 538 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Bromide (Br-) | Bromide (Br-) | NA | NA |
| 539 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Chloride (Cl-) | Chloride (Cl-) | NA | NA |
| 540 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Fluoride (F-) | Fluoride (F-) | NA | NA |
| 541 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Iodide (I-) | Iodide (I-) | NA | NA |
| 542 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Phosphate (PO4---) | Phosphate (PO4---) | NA | NA |
| 543 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Sulphate (SO4=), corrected | Sulphate (SO4=), corrected | NA | NA |
| 544 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions>Sulphate (SO4=), total | Sulphate (SO4=), total | NA | NA |
| 545 | >Atmosphere>Total Atmospheric Deposition>Inorganic cations>Calcium (Ca++) | Calcium (Ca++) | NA | NA |
| 546 | >Atmosphere>Total Atmospheric Deposition>Inorganic cations>Magnesium (Mg++) | Magnesium (Mg++) | NA | NA |
| 547 | >Atmosphere>Total Atmospheric Deposition>Inorganic cations>Potassium (K+) | Potassium (K+) | NA | NA |
| 548 | >Atmosphere>Total Atmospheric Deposition>Inorganic cations>Sodium (Na+) | Sodium (Na+) | NA | NA |
| 549 | >Ocean>Other Gas>CTD>pO2 | pO2 | NA | oceanPO2 |
| 550 | >Atmosphere>Clouds>Type>Type of cloud | Type of cloud | Result of cloud type classification - Accuracy expressed as inverse of number of classes, so that smaller figures correspond to better performance. | NA |
| 551 | >Atmosphere>Clouds>Type>Type of high clouds | Type of high clouds | NA | NA |
| 552 | >Atmosphere>Clouds>Type>Type of low clouds | Type of low clouds | NA | NA |
| 553 | >Atmosphere>Clouds>Type>Type of middle clouds | Type of middle clouds | NA | NA |
| 554 | >Atmosphere>Greenhouse Gas>HFCs>C2H2F4 (1,1,1,2-tetrafluoroethane, HFC-134a) | C2H2F4 (1,1,1,2-tetrafluoroethane, HFC-134a) | NA | NA |
| 555 | >Atmosphere>Greenhouse Gas>HFCs>C2H4F2 (1,1-difluoroethane, HFC-152a) | C2H4F2 (1,1-difluoroethane, HFC-152a) | NA | NA |
| 556 | >Atmosphere>Greenhouse Gas>HFCs>C2HF5 (pentafluoroethane, HFC-125) | C2HF5 (pentafluoroethane, HFC-125) | NA | NA |
| 557 | >Atmosphere>Greenhouse Gas>HFCs>C3H2F6 (1,1,1,2,3,3-hexafluoropropane, HFC-236fa) | C3H2F6 (1,1,1,2,3,3-hexafluoropropane, HFC-236fa) | NA | NA |
| 558 | >Atmosphere>Greenhouse Gas>HFCs>C3H3F5 (1,1,1,3,3-Pentafluoropropane, HFC-245fa) | C3H3F5 (1,1,1,3,3-Pentafluoropropane, HFC-245fa) | NA | NA |
| 559 | >Atmosphere>Greenhouse Gas>HFCs>C3HF7 (1,1,1,2,3,3,3-Heptafluoropropane, HFC-227ea) | C3HF7 (1,1,1,2,3,3,3-Heptafluoropropane, HFC-227ea) | NA | NA |
| 560 | >Atmosphere>Greenhouse Gas>HFCs>C4H5F5 (1,1,1,3,3-pentafluorobutane, HFC-365mfc) | C4H5F5 (1,1,1,3,3-pentafluorobutane, HFC-365mfc) | NA | NA |
| 561 | >Atmosphere>Greenhouse Gas>HFCs>CH2F2 (difluoromethane, HFC-32) | CH2F2 (difluoromethane, HFC-32) | NA | NA |
| 562 | >Atmosphere>Greenhouse Gas>HFCs>CHF3 (trifluoromethane, HFC-23) | CHF3 (trifluoromethane, HFC-23) | NA | NA |
| 563 | >Terrestrial>Land surface>Radiation>Fraction of Absorbed PAR (FAPAR) | Fraction of Absorbed PAR (FAPAR) | Fraction of PAR absorbed by vegetation (land or marine) for photosynthesis processes (generally around the 'red' ) | NA |
| 564 | >Terrestrial>Land surface>Radiation>Photosynthetically Active Radiation (PAR) | Photosynthetically Active Radiation (PAR) | Flux of downwelling photons of wavelength 0.4-0.7 | NA |
| 565 | >Atmosphere>Radiation>IR>Long-wave radiation (direction unspecified) | Long-wave radiation (direction unspecified) | NA | NA |
| 566 | >Atmosphere>Radiation>IR>Long-wave radiation (downwelling) | Long-wave radiation (downwelling) | NA | NA |
| 567 | >Atmosphere>Radiation>IR>Long-wave radiation (upwelling) | Long-wave radiation (upwelling) | NA | NA |
| 568 | >Atmosphere>Total Atmospheric Deposition>Inorganic nitrogen species>Ammonium (NH4+) | Ammonium (NH4+) | NA | NA |
| 569 | >Atmosphere>Total Atmospheric Deposition>Inorganic nitrogen species>Nitrate (NO3-) | Nitrate (NO3-) | NA | NA |
| 570 | >Atmosphere>Total Atmospheric Deposition>Inorganic nitrogen species>Nitrite (NO2-) | Nitrite (NO2-) | NA | NA |
| 571 | >Atmosphere>Radiation>Solar>Diffuse solar radiation | Diffuse solar radiation | NA | NA |
| 572 | >Atmosphere>Radiation>Solar>Direct solar radiation | Direct solar radiation | NA | NA |
| 573 | >Atmosphere>Radiation>Solar>Global solar radiation (downwelling) | Global solar radiation (downwelling) | NA | NA |
| 574 | >Atmosphere>Radiation>Solar>Global solar radiation (upwelling) | Global solar radiation (upwelling) | NA | NA |
| 575 | >Atmosphere>Radiation>Solar>Reflected solar radiation | Reflected solar radiation | NA | NA |
| 576 | >Atmosphere>Total Atmospheric Deposition>Organic acid>Acetate (CH3COO-) | Acetate (CH3COO-) | NA | NA |
| 577 | >Atmosphere>Total Atmospheric Deposition>Organic acid>Formate (HCOO-) | Formate (HCOO-) | NA | NA |
| 578 | >Atmosphere>Total Atmospheric Deposition>Organic acid>Propionate (C2H5COO-) | Propionate (C2H5COO-) | NA | NA |
| 579 | >Atmosphere>Aerosol>Composition>Acidity/Alkalinity, total aerosol | Acidity/Alkalinity, total aerosol | NA | NA |
| 589 | >Atmosphere>Greenhouse Gas>CFCs>C2Cl2F4 (1,2-dichlorotetrafluoroethane, CFC-114) | C2Cl2F4 (1,2-dichlorotetrafluoroethane, CFC-114) | NA | NA |
| 590 | >Atmosphere>Greenhouse Gas>CFCs>C2Cl3F3 (1,1,2-trichloro-1,2,2-trifluoroethane, CFC-113) | C2Cl3F3 (1,1,2-trichloro-1,2,2-trifluoroethane, CFC-113) | NA | NA |
| 591 | >Atmosphere>Greenhouse Gas>CFCs>C2ClF5 (1-chloro-1,1,2,2,2-pentafluoroethane, CFC-115) | C2ClF5 (1-chloro-1,1,2,2,2-pentafluoroethane, CFC-115) | NA | NA |
| 592 | >Atmosphere>Greenhouse Gas>CFCs>CCl2F2 (dichlorodifluoromethane, CFC-12) | CCl2F2 (dichlorodifluoromethane, CFC-12) | NA | NA |
| 593 | >Atmosphere>Greenhouse Gas>CFCs>CCl3F (trichlorofluoromethane, CFC-11) | CCl3F (trichlorofluoromethane, CFC-11) | NA | NA |
| 594 | >Atmosphere>Greenhouse Gas>Halocarbons>CCl4 (carbon tetrachloride) | CCl4 (carbon tetrachloride) | NA | NA |
| 595 | >Atmosphere>Greenhouse Gas>Halocarbons>CH3CCl3 (1,1,1-trichloroethane) | CH3CCl3 (1,1,1-trichloroethane) | NA | NA |
| 596 | >Terrestrial>Soil>Temperature>Soil temperature | Soil temperature | The standard depths for soil temperature measurements are 5, 10, 20, 50 and 100 cm below the surface; additional depths may be included. The site for such measurements should be a level plot of bare ground (about 75 cm^2 ) and typical of the surrounding soil for which information is required. | NA |
| 597 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Aluminum (Al) | Aluminum (Al) | NA | NA |
| 598 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Arsenic (As) | Arsenic (As) | NA | NA |
| 599 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Cadmium (Cd) | Cadmium (Cd) | NA | NA |
| 600 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Chromium (Cr) | Chromium (Cr) | NA | NA |
| 601 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Cobalt (Co) | Cobalt (Co) | NA | NA |
| 602 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Copper (Cu) | Copper (Cu) | NA | NA |
| 603 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Iron (Fe) | Iron (Fe) | NA | NA |
| 604 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Lead (Pb) | Lead (Pb) | NA | NA |
| 605 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Manganese (Mn) | Manganese (Mn) | NA | NA |
| 606 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Mercury (Hg) | Mercury (Hg) | NA | NA |
| 607 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Nickel (Ni) | Nickel (Ni) | NA | NA |
| 608 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Vanadium (V) | Vanadium (V) | NA | NA |
| 609 | >Atmosphere>Total Atmospheric Deposition>Trace elements>Zinc (Zn) | Zinc (Zn) | NA | NA |
| 610 | >Terrestrial>Land surface>Snow / Ice / Glacier>Glacier>Glacier cover | Glacier cover | Fraction of a land area covered by permanent ice | NA |
| 611 | >Terrestrial>Land surface>Snow / Ice / Glacier>Glacier>Glacier motion | Glacier motion | Velocity of the ice measured at the surface of a glacier | NA |
| 612 | >Terrestrial>Land surface>Snow / Ice / Glacier>Glacier>Glacier topography | Glacier topography | Map of the height of the glacier surface. | NA |
| 613 | >Terrestrial>Land surface>Snow / Ice / Glacier>Ice>Ice sheet topography | Ice sheet topography | Map of ice sheet height over land . | NA |
| 614 | >Atmosphere>Aerosol>Composition>Inorganic anions>Chloride (Cl-), PM1 | Chloride (Cl-), PM1 | NA | NA |
| 615 | >Atmosphere>Aerosol>Composition>Inorganic anions>Chloride (Cl-), PM10 | Chloride (Cl-), PM10 | NA | NA |
| 616 | >Atmosphere>Aerosol>Composition>Inorganic anions>Chloride (Cl-), PM2.5 | Chloride (Cl-), PM2.5 | NA | NA |
| 617 | >Atmosphere>Aerosol>Composition>Inorganic anions>Chloride (Cl-), total aerosol | Chloride (Cl-), total aerosol | NA | NA |
| 618 | >Atmosphere>Aerosol>Composition>Inorganic anions>Fluoride (F-), total aerosol | Fluoride (F-), total aerosol | NA | NA |
| 619 | >Atmosphere>Aerosol>Composition>Inorganic anions>Sulphate (SO4=), corrected | Sulphate (SO4=), corrected | NA | NA |
| 620 | >Atmosphere>Aerosol>Composition>Inorganic anions>Sulphate (SO4=), total | Sulphate (SO4=), total | NA | NA |
| 621 | >Atmosphere>Aerosol>Composition>Inorganic anions>Sulphate (SO4=), total, PM10 | Sulphate (SO4=), total, PM10 | NA | NA |
| 622 | >Atmosphere>Aerosol>Composition>Inorganic anions>Sulphate (SO4=), total, PM2.5 | Sulphate (SO4=), total, PM2.5 | NA | NA |
| 623 | >Atmosphere>Aerosol>Composition>Inorganic carbonaceous>Elemental carbon (coarse), PM10 | Elemental carbon (coarse), PM10 | NA | NA |
| 624 | >Atmosphere>Aerosol>Composition>Inorganic carbonaceous>Elemental carbon, PM1 | Elemental carbon, PM1 | NA | NA |
| 625 | >Atmosphere>Aerosol>Composition>Inorganic carbonaceous>Elemental carbon, PM2.5 | Elemental carbon, PM2.5 | NA | NA |
| 626 | >Atmosphere>Aerosol>Composition>Inorganic carbonaceous>Total carbon (coarse), PM10 | Total carbon (coarse), PM10 | NA | NA |
| 627 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow>Depth of fresh snowfall | Depth of fresh snowfall | NA | NA |
| 628 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow>Snow cover (fraction of area) | Snow cover (fraction of area) | Fraction of a given area which is covered by snow | NA |
| 629 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow>Snow depth | Snow depth | Vertical distance from the snow surface to the underlying surface (ground, glacier ice or sea ice). | NA |
| 630 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow>Snow status (wet/dry) | Snow status (wet/dry) | Binary product (dry or melting/thawing) expressing the presence of liquid water in a snow layer- Accuracy expressed as Hit Rate [ HR ] and False Alarm Rate [ FAR ]. | NA |
| 631 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow>Snow water equivalent | Snow water equivalent | Vertical depth of the water that would be obtained by melting a snow layer. Linked to snow depth through the density of the snow layer. | NA |
| 632 | >Atmosphere>Aerosol>Composition>Inorganic cations>Calcium (Ca++), PM10 | Calcium (Ca++), PM10 | NA | NA |
| 633 | >Atmosphere>Aerosol>Composition>Inorganic cations>Calcium (Ca++), PM2.5 | Calcium (Ca++), PM2.5 | NA | NA |
| 634 | >Atmosphere>Aerosol>Composition>Inorganic cations>Calcium (Ca++), total aerosol | Calcium (Ca++), total aerosol | NA | NA |
| 635 | >Atmosphere>Aerosol>Composition>Inorganic cations>Magnesium (Mg++), PM10 | Magnesium (Mg++), PM10 | NA | NA |
| 636 | >Atmosphere>Aerosol>Composition>Inorganic cations>Magnesium (Mg++), PM2.5 | Magnesium (Mg++), PM2.5 | NA | NA |
| 637 | >Atmosphere>Aerosol>Composition>Inorganic cations>Magnesium (Mg++), total aerosol | Magnesium (Mg++), total aerosol | NA | NA |
| 638 | >Atmosphere>Aerosol>Composition>Inorganic cations>Potassium (K+), PM10 | Potassium (K+), PM10 | NA | NA |
| 639 | >Atmosphere>Aerosol>Composition>Inorganic cations>Potassium (K+), PM2.5 | Potassium (K+), PM2.5 | NA | NA |
| 640 | >Atmosphere>Aerosol>Composition>Inorganic cations>Potassium (K+), total aerosol | Potassium (K+), total aerosol | NA | NA |
| 641 | >Atmosphere>Aerosol>Composition>Inorganic cations>Sodium (Na+), PM10 | Sodium (Na+), PM10 | NA | NA |
| 642 | >Atmosphere>Aerosol>Composition>Inorganic cations>Sodium (Na+), PM2.5 | Sodium (Na+), PM2.5 | NA | NA |
| 643 | >Atmosphere>Aerosol>Composition>Inorganic cations>Sodium (Na+), total aerosol | Sodium (Na+), total aerosol | NA | NA |
| 644 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonia (NH3), PM2.5 | Ammonia (NH3), PM2.5 | NA | NA |
| 645 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonium (NH4+), PM10 | Ammonium (NH4+), PM10 | NA | NA |
| 646 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonium (NH4+), PM2.5 | Ammonium (NH4+), PM2.5 | NA | NA |
| 647 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonium (NH4+), total aerosol | Ammonium (NH4+), total aerosol | NA | NA |
| 648 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonium nitrate (NH4NO3), total aerosol | Ammonium nitrate (NH4NO3), total aerosol | NA | NA |
| 649 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Ammonium nitrate (NH4NO3, PM1 | Ammonium nitrate (NH4NO3, PM1 | NA | NA |
| 650 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Nitrate (NO3-), PM10 | Nitrate (NO3-), PM10 | NA | NA |
| 651 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Nitrate (NO3-), PM2.5 | Nitrate (NO3-), PM2.5 | NA | NA |
| 652 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Nitrate (NO3-), total aerosol | Nitrate (NO3-), total aerosol | NA | NA |
| 653 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Nitrite (NO2-), total aerosol | Nitrite (NO2-), total aerosol | NA | NA |
| 654 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Sum of ammonia (NH3) and ammonium (NH4+), in air and aerosol | Sum of ammonia (NH3) and ammonium (NH4+), in air and aerosol | NA | NA |
| 655 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species>Sum of nitric acid (HNO3) and nitrate (NO3-), in air and aerosol | Sum of nitric acid (HNO3) and nitrate (NO3-), in air and aerosol | NA | NA |
| 656 | >Atmosphere>Aerosol>Composition>Major inorganic components>Major chemical components (size fractionated) | Major chemical components (size fractionated) | NA | NA |
| 657 | >Atmosphere>Aerosol>Composition>Major inorganic components>Major inorganic components (TSP) | Major inorganic components (TSP) | NA | NA |
| 658 | >Atmosphere>Aerosol>Composition>Major inorganic components>Major inorganic components (coarse) | Major inorganic components (coarse) | NA | NA |
| 659 | >Atmosphere>Aerosol>Composition>Major inorganic components>Major inorganic components (fine) | Major inorganic components (fine) | NA | NA |
| 660 | >Atmosphere>Aerosol>Composition>Major inorganic components>Other chemical components (coarse) | Other chemical components (coarse) | NA | NA |
| 661 | >Atmosphere>Aerosol>Composition>Major inorganic components>Other chemical components (fine) | Other chemical components (fine) | NA | NA |
| 662 | >Atmosphere>Aerosol>Composition>Organic anions>C2H3O2- (CH3COO-, acetate), PM1 | C2H3O2- (CH3COO-, acetate), PM1 | NA | NA |
| 663 | >Atmosphere>Aerosol>Composition>Organic anions>C2H3O2- (CH3COO-, acetate), in aerosol | C2H3O2- (CH3COO-, acetate), in aerosol | NA | NA |
| 664 | >Atmosphere>Aerosol>Composition>Organic anions>C2O4= (oxalate, ethanedioate), PM1.0 | C2O4= (oxalate, ethanedioate), PM1.0 | NA | NA |
| 665 | >Atmosphere>Aerosol>Composition>Organic anions>C2O4= (oxalate, ethanedioate), PM10 | C2O4= (oxalate, ethanedioate), PM10 | NA | NA |
| 666 | >Atmosphere>Aerosol>Composition>Organic anions>C2O4= (oxalate, ethanedioate), total aerosol | C2O4= (oxalate, ethanedioate), total aerosol | NA | NA |
| 667 | >Atmosphere>Aerosol>Composition>Organic anions>CH3O3S- (methanesulphonate), PM1.0 | CH3O3S- (methanesulphonate), PM1.0 | NA | NA |
| 668 | >Atmosphere>Aerosol>Composition>Organic anions>CH3O3S- (methanesulphonate), total aerosol | CH3O3S- (methanesulphonate), total aerosol | NA | NA |
| 669 | >Atmosphere>Aerosol>Composition>Organic anions>CHO2- (HCOO-, formate), in aerosol | CHO2- (HCOO-, formate), in aerosol | NA | NA |
| 670 | >Atmosphere>Aerosol>Composition>Organic carbonaceous>Carbonaceous/organic material (coarse), PM10 | Carbonaceous/organic material (coarse), PM10 | NA | NA |
| 671 | >Atmosphere>Aerosol>Composition>Organic carbonaceous>Carbonaceous/organic material (fine) | Carbonaceous/organic material (fine) | NA | NA |
| 672 | >Atmosphere>Aerosol>Composition>Trace elements>Titanium (Ti), total aerosol | Titanium (Ti), total aerosol | NA | NA |
| 673 | >Atmosphere>Aerosol>Composition>Trace elements>Aluminium (Al), PM2.5 | Aluminium (Al), PM2.5 | NA | NA |
| 674 | >Atmosphere>Aerosol>Composition>Trace elements>Aluminium (Al), total aerosol | Aluminium (Al), total aerosol | NA | NA |
| 675 | >Atmosphere>Aerosol>Composition>Trace elements>Antimony (Sb), PM2.5 | Antimony (Sb), PM2.5 | NA | NA |
| 676 | >Atmosphere>Aerosol>Composition>Trace elements>Antimony (Sb), total aerosol | Antimony (Sb), total aerosol | NA | NA |
| 677 | >Atmosphere>Aerosol>Composition>Trace elements>Arsenic (As), PM10 | Arsenic (As), PM10 | NA | NA |
| 678 | >Atmosphere>Aerosol>Composition>Trace elements>Arsenic (As), total aerosol | Arsenic (As), total aerosol | NA | NA |
| 679 | >Atmosphere>Aerosol>Composition>Trace elements>Barium (Ba), PM2.5 | Barium (Ba), PM2.5 | NA | NA |
| 680 | >Atmosphere>Aerosol>Composition>Trace elements>Barium (Ba), total aerosol | Barium (Ba), total aerosol | NA | NA |
| 681 | >Atmosphere>Aerosol>Composition>Trace elements>Bismuth (Bi), PM2.5 | Bismuth (Bi), PM2.5 | NA | NA |
| 682 | >Atmosphere>Aerosol>Composition>Trace elements>Bismuth (Bi), total aerosol | Bismuth (Bi), total aerosol | NA | NA |
| 683 | >Atmosphere>Aerosol>Composition>Trace elements>Cadmium (Cd), PM1 | Cadmium (Cd), PM1 | NA | NA |
| 684 | >Atmosphere>Aerosol>Composition>Trace elements>Cadmium (Cd), PM10 | Cadmium (Cd), PM10 | NA | NA |
| 685 | >Atmosphere>Aerosol>Composition>Trace elements>Cadmium (Cd), PM2.5 | Cadmium (Cd), PM2.5 | NA | NA |
| 686 | >Atmosphere>Aerosol>Composition>Trace elements>Cadmium (Cd), PM2.5 thru PM10 | Cadmium (Cd), PM2.5 thru PM10 | NA | NA |
| 687 | >Atmosphere>Aerosol>Composition>Trace elements>Cadmium (Cd), total aerosol | Cadmium (Cd), total aerosol | NA | NA |
| 688 | >Atmosphere>Aerosol>Composition>Trace elements>Cerium (Ce), PM2.5 | Cerium (Ce), PM2.5 | NA | NA |
| 689 | >Atmosphere>Aerosol>Composition>Trace elements>Cerium (Ce), total aerosol | Cerium (Ce), total aerosol | NA | NA |
| 690 | >Atmosphere>Aerosol>Composition>Trace elements>Chromium (Cr), total aerosol | Chromium (Cr), total aerosol | NA | NA |
| 691 | >Atmosphere>Aerosol>Composition>Trace elements>Cobalt (Co), total aerosol | Cobalt (Co), total aerosol | NA | NA |
| 692 | >Atmosphere>Aerosol>Composition>Trace elements>Copper (Cu), PM10 | Copper (Cu), PM10 | NA | NA |
| 693 | >Atmosphere>Aerosol>Composition>Trace elements>Copper (Cu), total aerosol | Copper (Cu), total aerosol | NA | NA |
| 694 | >Atmosphere>Aerosol>Composition>Trace elements>Iron (Fe), total aerosol | Iron (Fe), total aerosol | NA | NA |
| 695 | >Atmosphere>Aerosol>Composition>Trace elements>Lanthanum (La), PM2.5 | Lanthanum (La), PM2.5 | NA | NA |
| 696 | >Atmosphere>Aerosol>Composition>Trace elements>Lanthanum (La), total aerosol | Lanthanum (La), total aerosol | NA | NA |
| 697 | >Atmosphere>Aerosol>Composition>Trace elements>Lead (Pb), PM10 | Lead (Pb), PM10 | NA | NA |
| 698 | >Atmosphere>Aerosol>Composition>Trace elements>Lead (Pb), total aerosol | Lead (Pb), total aerosol | NA | NA |
| 699 | >Atmosphere>Aerosol>Composition>Trace elements>Lithium (Li), PM2.5 | Lithium (Li), PM2.5 | NA | NA |
| 700 | >Atmosphere>Aerosol>Composition>Trace elements>Lithium (Li), total aerosol | Lithium (Li), total aerosol | NA | NA |
| 701 | >Atmosphere>Aerosol>Composition>Trace elements>Manganese (Mn), PM10 | Manganese (Mn), PM10 | NA | NA |
| 702 | >Atmosphere>Aerosol>Composition>Trace elements>Manganese (Mn), total aerosol | Manganese (Mn), total aerosol | NA | NA |
| 703 | >Atmosphere>Aerosol>Composition>Trace elements>Mercury (Hg), total aerosol | Mercury (Hg), total aerosol | NA | NA |
| 704 | >Atmosphere>Aerosol>Composition>Trace elements>Nickel (Ni), PM10 | Nickel (Ni), PM10 | NA | NA |
| 705 | >Atmosphere>Aerosol>Composition>Trace elements>Nickel (Ni), total aerosol | Nickel (Ni), total aerosol | NA | NA |
| 706 | >Atmosphere>Aerosol>Composition>Trace elements>Phosphorous (P), PM2.5 | Phosphorous (P), PM2.5 | NA | NA |
| 707 | >Atmosphere>Aerosol>Composition>Trace elements>Phosphorous (P), total aerosol | Phosphorous (P), total aerosol | NA | NA |
| 708 | >Atmosphere>Aerosol>Composition>Trace elements>Rubidium (Rb), PM2.5 | Rubidium (Rb), PM2.5 | NA | NA |
| 709 | >Atmosphere>Aerosol>Composition>Trace elements>Rubidium (Rb), total aerosol | Rubidium (Rb), total aerosol | NA | NA |
| 710 | >Atmosphere>Aerosol>Composition>Trace elements>Selenium (Se), total aerosol | Selenium (Se), total aerosol | NA | NA |
| 711 | >Atmosphere>Aerosol>Composition>Trace elements>Strontium (Sr), PM2.5 | Strontium (Sr), PM2.5 | NA | NA |
| 712 | >Atmosphere>Aerosol>Composition>Trace elements>Strontium (Sr), total aerosol | Strontium (Sr), total aerosol | NA | NA |
| 713 | >Atmosphere>Aerosol>Composition>Trace elements>Thallium (Tl), PM2.5 | Thallium (Tl), PM2.5 | NA | NA |
| 714 | >Atmosphere>Aerosol>Composition>Trace elements>Thallium (Tl), total aerosol | Thallium (Tl), total aerosol | NA | NA |
| 715 | >Atmosphere>Aerosol>Composition>Trace elements>Thorium (Th), PM2.5 | Thorium (Th), PM2.5 | NA | NA |
| 716 | >Atmosphere>Aerosol>Composition>Trace elements>Thorium (Th), total aerosol | Thorium (Th), total aerosol | NA | NA |
| 717 | >Atmosphere>Aerosol>Composition>Trace elements>Tin (Sn), PM2.5 | Tin (Sn), PM2.5 | NA | NA |
| 718 | >Atmosphere>Aerosol>Composition>Trace elements>Tin (Sn), total aerosol | Tin (Sn), total aerosol | NA | NA |
| 719 | >Atmosphere>Aerosol>Composition>Trace elements>Uranium (U), PM2.5 | Uranium (U), PM2.5 | NA | NA |
| 720 | >Atmosphere>Aerosol>Composition>Trace elements>Uranium (U), total aerosol | Uranium (U), total aerosol | NA | NA |
| 721 | >Atmosphere>Aerosol>Composition>Trace elements>Vanadium (V), PM2.5 | Vanadium (V), PM2.5 | NA | NA |
| 722 | >Atmosphere>Aerosol>Composition>Trace elements>Vanadium (V), total aerosol | Vanadium (V), total aerosol | NA | NA |
| 723 | >Atmosphere>Aerosol>Composition>Trace elements>Zinc (Zn), PM2.5 | Zinc (Zn), PM2.5 | NA | NA |
| 724 | >Atmosphere>Aerosol>Composition>Trace elements>Zinc (Zn), total aerosol | Zinc (Zn), total aerosol | NA | NA |
| 725 | >Terrestrial>Lake>Snow / Ice / Glacier>Ice>Ice thickness | Ice thickness | NA | NA |
| 726 | >Terrestrial>River>Snow / Ice / Glacier>Ice>Ice thickness | Ice thickness | NA | NA |
| 727 | >Atmosphere>Aerosol>Physical properties - primary>Dust>Aerosol dust concentration (mass) | Aerosol dust concentration (mass) | 3-D field of concentration of dust or sand in the atmosphere | NA |
| 728 | >Atmosphere>Aerosol>Physical properties - primary>Volcanic ash>Aerosol volcanic ash (mass concentration) | Aerosol volcanic ash (mass concentration) | 3D field of mass mixing ratio of volcanic ash | NA |
| 729 | >Atmosphere>Aerosol>Physical properties - primary>Volcanic ash>Aerosol volcanic ash (Total column) | Aerosol volcanic ash (Total column) | Field of total column mass of volcanic ash | NA |
| 731 | >Ocean>Basic Physical Properties>Conductivity | Conductivity | Conductivity measured by CTD or MicroCAT | oceanConductivity |
| 732 | >Ocean>Basic Physical Properties>Density | Density | Density measured by CTD | oceanDensity |
| 733 | >Ocean>Biology>Zooplankton (size and/or development stage) | Zooplankton (size and/or development stage) | the aggregate of animal or animal-like organisms in plankton, as protozoans | oceanZooplanktonSizeDev |
| 735 | >Ocean>Greenhouse Gas>pCO2 | pCO2 | Partial pressure of CO2 | oceanPCO2 |
| 736 | >Ocean>Greenhouse Gas>pCO2 air-sea flux | pCO2 air-sea flux | Flux of CO2 from the ocean into the atmosphere | oceanPCO2AirSeaFlux |
| 739 | >Ocean>Miscellaneous>Fluorescence (DOM) | Fluorescence (DOM) | Flurometric colored dissolved Organic matter concentration | oceanFluorescence |
| 740 | >Ocean>Topography>Mean dynamic topography (MDT) | Mean dynamic topography (MDT) | NA | oceanMeanDynamicTopography |
| 741 | >Ocean>Topography>Mean sea surface (MSS) | Mean sea surface (MSS) | NA | oceanMeanSeaSurface |
| 742 | >Ocean>Topography>Sea surface height (SSH, OST) | Sea surface height (SSH, OST) | NA | oceanSeaSurfaceHeight |
| 743 | >Ocean>Waves>Wave height | Wave height | average of height (trough to crest) of the waves collected through a specified time | oceanWaveHeight |
| 744 | >Ocean>Waves>Wave period | Wave period | The time required for two successive wave crests to pass a fixed point, or the time for a single wave crest to travel a distance equal to the length of the wave. This is averaged over a specified period of time. | oceanWavePeriod |
| 745 | >Ocean>Waves>Directional waves | Directional waves | Waves measured in X,Y,Z directions | oceanDirectionalWaves |
| 10000 | >Atmosphere | Atmosphere | NA | NA |
| 10001 | >Atmosphere>Aerosol | Aerosol | NA | NA |
| 10002 | >Atmosphere>Aerosol>Composition | Composition | NA | NA |
| 10003 | >Atmosphere>Aerosol>Composition>Inorganic anions | Inorganic anions | NA | NA |
| 10004 | >Atmosphere>Aerosol>Composition>Inorganic carbonaceous | Inorganic carbonaceous | NA | NA |
| 10005 | >Atmosphere>Aerosol>Composition>Inorganic cations | Inorganic cations | NA | NA |
| 10006 | >Atmosphere>Aerosol>Composition>Inorganic nitrogen species | Inorganic nitrogen species | NA | NA |
| 10007 | >Atmosphere>Aerosol>Composition>Major inorganic components | Major inorganic components | NA | NA |
| 10008 | >Atmosphere>Aerosol>Composition>Organic anions | Organic anions | NA | NA |
| 10009 | >Atmosphere>Aerosol>Composition>Organic carbonaceous | Organic carbonaceous | NA | NA |
| 10010 | >Atmosphere>Aerosol>Composition>Trace elements | Trace elements | NA | NA |
| 10011 | >Atmosphere>Aerosol>Optical properties | Optical properties | NA | NA |
| 10012 | >Atmosphere>Aerosol>Physical properties - primary | Physical properties - primary | NA | NA |
| 10013 | >Atmosphere>Aerosol>Physical properties - primary>Dust | Dust | NA | NA |
| 10014 | >Atmosphere>Aerosol>Physical properties - primary>Volcanic ash | Volcanic ash | NA | NA |
| 10015 | >Atmosphere>Aerosol>Physical properties - secondary | Physical properties - secondary | NA | NA |
| 10016 | >Atmosphere>Clouds | Clouds | NA | NA |
| 10017 | >Atmosphere>Clouds>Ice | Ice | NA | NA |
| 10018 | >Atmosphere>Clouds>Liquid water | Liquid water | NA | NA |
| 10019 | >Atmosphere>Clouds>Optical properties | Optical properties | NA | NA |
| 10020 | >Atmosphere>Clouds>Position | Position | NA | NA |
| 10021 | >Atmosphere>Clouds>Type | Type | NA | NA |
| 10022 | >Atmosphere>Dynamics | Dynamics | NA | NA |
| 10023 | >Atmosphere>Greenhouse Gas | Greenhouse Gas | NA | NA |
| 10024 | >Atmosphere>Greenhouse Gas>CFCs | CFCs | NA | NA |
| 10025 | >Atmosphere>Greenhouse Gas>Halocarbons | Halocarbons | NA | NA |
| 10026 | >Atmosphere>Greenhouse Gas>Halon | Halon | NA | NA |
| 10027 | >Atmosphere>Greenhouse Gas>HCFCs | HCFCs | NA | NA |
| 10028 | >Atmosphere>Greenhouse Gas>HFCs | HFCs | NA | NA |
| 10029 | >Atmosphere>Greenhouse Gas>PFCs | PFCs | NA | NA |
| 10030 | >Atmosphere>Humidity | Humidity | NA | NA |
| 10031 | >Atmosphere>Lightning | Lightning | NA | NA |
| 10032 | >Atmosphere>Lightning>Position | Position | NA | NA |
| 10033 | >Atmosphere>Other Gas | Other Gas | NA | NA |
| 10034 | >Atmosphere>Ozone | Ozone | NA | NA |
| 10035 | >Atmosphere>Past weather | Past weather | NA | NA |
| 10036 | >Atmosphere>POPs | POPs | NA | NA |
| 10037 | >Atmosphere>POPs>PAH | PAH | NA | NA |
| 10038 | >Atmosphere>POPs>POP | POP | NA | NA |
| 10039 | >Atmosphere>Precipitation | Precipitation | NA | NA |
| 10040 | >Atmosphere>Present weather | Present weather | NA | NA |
| 10041 | >Atmosphere>Pressure | Pressure | NA | NA |
| 10042 | >Atmosphere>Radiation | Radiation | NA | NA |
| 10043 | >Atmosphere>Radiation>IR | IR | NA | NA |
| 10044 | >Atmosphere>Radiation>Solar | Solar | NA | NA |
| 10045 | >Atmosphere>Radiation>UV | UV | NA | NA |
| 10046 | >Atmosphere>Radionuclide | Radionuclide | NA | NA |
| 10047 | >Atmosphere>Reactive Gas | Reactive Gas | NA | NA |
| 10048 | >Atmosphere>Reactive Gas>Nitrogen containing compounds | Nitrogen containing compounds | NA | NA |
| 10049 | >Atmosphere>Reactive Gas>Sulfur containing compounds | Sulfur containing compounds | NA | NA |
| 10050 | >Atmosphere>Reactive Gas>VOC | VOC | NA | NA |
| 10051 | >Atmosphere>Temperature | Temperature | NA | NA |
| 10052 | >Atmosphere>Total Atmospheric Deposition | Total Atmospheric Deposition | NA | NA |
| 10053 | >Atmosphere>Total Atmospheric Deposition>Inorganic acid | Inorganic acid | NA | NA |
| 10054 | >Atmosphere>Total Atmospheric Deposition>Inorganic anions | Inorganic anions | NA | NA |
| 10055 | >Atmosphere>Total Atmospheric Deposition>Inorganic cations | Inorganic cations | NA | NA |
| 10056 | >Atmosphere>Total Atmospheric Deposition>Inorganic nitrogen species | Inorganic nitrogen species | NA | NA |
| 10057 | >Atmosphere>Total Atmospheric Deposition>Organic acid | Organic acid | NA | NA |
| 10058 | >Atmosphere>Total Atmospheric Deposition>Trace elements | Trace elements | NA | NA |
| 10059 | >Atmosphere>Visibility | Visibility | NA | NA |
| 10060 | >Atmosphere>Visibility>Obscurations | Obscurations | NA | NA |
| 10061 | >Atmosphere>Wind | Wind | NA | NA |
| 10062 | >Earth | Earth | NA | NA |
| 10063 | >Ocean | Ocean | NA | NA |
| 10064 | >Ocean>Topography | Topography | NA | NA |
| 10065 | >Ocean>Currents | Currents | NA | NA |
| 10066 | >Ocean>Miscellaneous | Miscellaneous | NA | NA |
| 10067 | >Ocean>Biology | Biology | NA | NA |
| 10068 | >Ocean>Biology>Nitrogen species | Nitrogen species | NA | NA |
| 10069 | >Ocean>Biology>Phosporus species | Phosporus species | NA | NA |
| 10070 | >Ocean>Other Gas | Other Gas | NA | NA |
| 10071 | >Ocean>Other Gas>CTD | CTD | NA | NA |
| 10072 | >Ocean>Pressure | Pressure | NA | NA |
| 10073 | >Ocean>Pressure>CTD | CTD | NA | oceanCTD |
| 10074 | >Ocean>Radiation | Radiation | NA | NA |
| 10075 | >Ocean>Reactive Gas | Reactive Gas | NA | NA |
| 10076 | >Ocean>Basic Physical Properties | Basic Physical Properties | NA | NA |
| 10077 | >Ocean>Snow / Ice / Glacier | Snow / Ice / Glacier | NA | NA |
| 10078 | >Ocean>Snow / Ice / Glacier>Ice | Ice | NA | NA |
| 10079 | >Ocean>Temperature | Temperature | NA | oceanTemperature |
| 10080 | >Ocean>Tides | Tides | NA | oceanTides |
| 10081 | >Ocean>Transmissivity | Transmissivity | NA | NA |
| 10082 | >Ocean>Transmissivity>CTD | CTD | NA | NA |
| 10083 | >Ocean>Waves | Waves | NA | NA |
| 10084 | >Ocean>Wind | Wind | NA | NA |
| 10085 | >Outer Space | Outer Space | NA | NA |
| 10086 | >Outer Space>Energetic particles / solar wind | Energetic particles / solar wind | NA | NA |
| 10087 | >Outer Space>Energetic particles / solar wind >Particle density | Particle density | NA | NA |
| 10088 | >Outer Space>Energetic particles / solar wind >Particle flux | Particle flux | NA | NA |
| 10089 | >Outer Space>Ionospheric disturbances | Ionospheric disturbances | NA | NA |
| 10090 | >Outer Space>Solar monitoring | Solar monitoring | NA | NA |
| 10091 | >Terrestrial | Terrestrial | NA | NA |
| 10092 | >Terrestrial>Ground water | Ground water | NA | NA |
| 10093 | >Terrestrial>Lake | Lake | NA | NA |
| 10094 | >Terrestrial>Lake>Snow / Ice / Glacier | Snow / Ice / Glacier | NA | NA |
| 10095 | >Terrestrial>Lake>Snow / Ice / Glacier>Ice | Ice | NA | NA |
| 10096 | >Terrestrial>Lake>Temperature | Temperature | NA | NA |
| 10097 | >Terrestrial>Land surface | Land surface | NA | NA |
| 10098 | >Terrestrial>Land surface>Fire | Fire | NA | NA |
| 10099 | >Terrestrial>Land surface>Greenhouse Gas | Greenhouse Gas | NA | NA |
| 10100 | >Terrestrial>Land surface>Humidity | Humidity | NA | NA |
| 10101 | >Terrestrial>Land surface>Radiation | Radiation | NA | NA |
| 10102 | >Terrestrial>Land surface>Snow / Ice / Glacier | Snow / Ice / Glacier | NA | NA |
| 10103 | >Terrestrial>Land surface>Snow / Ice / Glacier>Glacier | Glacier | NA | NA |
| 10104 | >Terrestrial>Land surface>Snow / Ice / Glacier>Ice | Ice | NA | NA |
| 10105 | >Terrestrial>Land surface>Snow / Ice / Glacier>Snow | Snow | NA | NA |
| 10106 | >Terrestrial>Land surface>Temperature | Temperature | NA | NA |
| 10107 | >Terrestrial>Land surface>Vegetation | Vegetation | NA | NA |
| 10108 | >Terrestrial>River | River | NA | NA |
| 10109 | >Terrestrial>River>Snow / Ice / Glacier | Snow / Ice / Glacier | NA | NA |
| 10110 | >Terrestrial>River>Snow / Ice / Glacier>Ice | Ice | NA | NA |
| 10111 | >Terrestrial>Soil | Soil | NA | NA |
| 10112 | >Terrestrial>Soil>Humidity | Humidity | NA | NA |
| 10113 | >Terrestrial>Soil>Temperature | Temperature | NA | NA |
| 10114 | >Terrestrial>Well | Well | NA | NA |
| 10115 | >Ocean>Greenhouse Gas | Greenhouse Gas | NA | NA |
| 10116 | >Ocean>Miscellaneous>Carbon species | Carbon species | NA | NA |
| 10117 | >Ocean>Miscellaneous>Nitrogen species | Nitrogen species | NA | NA |

### Code table: 1-02

Code table title: Measurement unit [according to common code table C–6 (WMO, 2013a)]

| # | Name_Short | Name_Long | Source | WMO306_CD |
| --- | --- | --- | --- | --- |
| 1 | m | metre | Base SI | 1 |
| 2 | kg | kilogram | Base SI | 2 |
| 3 | s | second | Base SI | 3 |
| 4 | A | ampere | Base SI | 4 |
| 5 | K | kelvin | Base SI | 5 |
| 6 | mol | mole | Base SI | 6 |
| 7 | cd | candela | Base SI | 7 |
| 8 | rad | radian | Supplementary SI | 21 |
| 9 | sr | steradian | Supplementary SI | 22 |
| 10 | Hz | hertz | Derived SI | 30 |
| 11 | N | newton | Derived SI | 31 |
| 12 | Pa | pascal | Derived SI | 32 |
| 13 | J | joule | Derived SI | 33 |
| 14 | W | watt | Derived SI | 34 |
| 15 | C | coulomb | Derived SI | 35 |
| 16 | V | volt | Derived SI | 36 |
| 17 | F | farad | Derived SI | 37 |
| 18 | Ω | ohm | Derived SI | 38 |
| 19 | S | siemens | Derived SI | 39 |
| 20 | Wb | weber | Derived SI | 40 |
| 21 | T | tesla | Derived SI | 41 |
| 22 | H | henry | Derived SI | 42 |
| 23 | °C | degree Celsius | Derived SI | 60 |
| 24 | lm | lumen | Derived SI | 70 |
| 25 | lx | lux | Derived SI | 71 |
| 26 | Bq | becquerel | Derived SI | 80 |
| 27 | Gy | gray | Derived SI | 81 |
| 28 | Sv | sievert | Derived SI | 82 |
| 29 | ° | degree (angle) | Angle | 110 |
| 30 |  | minute (angle) | Angle | 111 |
| 31 | " | second (angle) | Angle | 112 |
| 32 | l or L | litre | CGPM accepted | 120 |
| 33 | min | minute (time) | CGPM accepted | 130 |
| 34 | h | hour | CGPM accepted | 131 |
| 35 | d | day | CGPM accepted | 132 |
| 36 | t | tonne | CGPM accepted | 150 |
| 37 | eV | electron volt | CGPM accepted | 160 |
| 38 | u | atomic mass unit | CGPM accepted | 161 |
| 39 | AU | astronomic unit | CGPM accepted | 170 |
| 40 | pc | parsec | CGPM accepted | 171 |
| 41 | NA | nautical mile | non-SI conventional | 200 |
| 42 | kt | knot | non-SI conventional | 201 |
| 43 | dB | decibel | non-SI conventional | 210 |
| 44 | ha | hectare | non-SI conventional | 220 |
| 45 | NA | week | non-SI conventional | 230 |
| 46 | a | year | non-SI conventional | 231 |
| 47 | % | per cent | WMO conventional | 300 |
| 48 | ‰ | parts per thousand | WMO conventional | 301 |
| 49 | okta | eighths of cloud | WMO conventional | 310 |
| 50 | ° | degrees true | WMO conventional | 320 |
| 51 | degree/s | degrees per second | WMO conventional | 321 |
| 52 | °C | degrees Celsius | WMO conventional | 350 |
| 53 | °C/m | degrees Celsius per metre | WMO conventional | 351 |
| 54 | °C/100 m | degrees Celsius per 100 metres | WMO conventional | 352 |
| 55 | DU | Dobson Unit | WMO conventional | 360 |
| 56 | mon | month | WMO conventional | 430 |
| 57 | s⁻¹ | per second (same as hertz) | WMO conventional | 441 |
| 58 | s⁻² | per second squared | WMO conventional | 442 |
| 59 | kt/1000 m | knots per 1000 metres | WMO conventional | 501 |
| 60 | ft | foot | WMO conventional | 510 |
| 61 | in | inch | WMO conventional | 511 |
| 62 | dPa s⁻¹ | decipascals per second (microbar per second) | WMO conventional | 520 |
| 63 | cb s⁻¹ | centibars per second | WMO conventional | 521 |
| 64 | cb/12 h | centibars per 12 hours | WMO conventional | 522 |
| 65 | daPa | dekapascal | WMO conventional | 523 |
| 66 | hPa | hectopascal | WMO conventional | 530 |
| 67 | hPa s⁻¹ | hectopascals per second | WMO conventional | 531 |
| 68 | hPa h⁻¹ | hectopascals per hour | WMO conventional | 532 |
| 69 | hPa/3 h | hectopascals per 3 hours | WMO conventional | 533 |
| 70 | nbar | nanobar = hPa 10⁻⁶ | WMO conventional | 535 |
| 71 | g kg⁻¹ | grams per kilogram | WMO conventional | 620 |
| 72 | g kg⁻¹ s⁻¹ | grams per kilogram per second | WMO conventional | 621 |
| 73 | kg kg⁻¹ | kilograms per kilogram | WMO conventional | 622 |
| 74 | kg kg⁻¹ s⁻¹ | kilograms per kilogram per second | WMO conventional | 623 |
| 75 | kg m⁻² | kilograms per square metre | WMO conventional | 624 |
| 76 | g | acceleration due to gravity | WMO conventional | 630 |
| 77 | gpm | geopotential metre | WMO conventional | 631 |
| 78 | mm | millimetre | WMO conventional | 710 |
| 79 | mm s⁻¹ | millimetres per second | WMO conventional | 711 |
| 80 | mm h⁻¹ | millimetres per hour | WMO conventional | 712 |
| 81 | mm⁶ m⁻³ | millimetres to the sixth power per cubic metre | WMO conventional | 713 |
| 82 | cm | centimetre | WMO conventional | 715 |
| 83 | cm s⁻¹ | centimetres per second | WMO conventional | 716 |
| 84 | cm h⁻¹ | centimetres per hour | WMO conventional | 717 |
| 85 | dm | decimetre | WMO conventional | 720 |
| 86 | m s⁻¹ | metres per second | WMO conventional | 731 |
| 87 | m s⁻¹/m | metres per second per metre | WMO conventional | 732 |
| 88 | m s⁻¹/1000 m | metres per second per 1000 metres | WMO conventional | 733 |
| 89 | m² | square metres | WMO conventional | 734 |
| 90 | m² s⁻¹ | square metres per second | WMO conventional | 735 |
| 91 | km | kilometre | WMO conventional | 740 |
| 92 | km h⁻¹ | kilometres per hour | WMO conventional | 741 |
| 93 | km/d | kilometres per day | WMO conventional | 742 |
| 94 | m⁻¹ | per metre | WMO conventional | 743 |
| 95 | Bq l⁻¹ | becquerels per litre | WMO conventional | 750 |
| 96 | Bq m⁻² | becquerels per square metre | WMO conventional | 751 |
| 97 | Bq m⁻³ | becquerels per cubic metre | WMO conventional | 752 |
| 98 | mSv | millisievert | WMO conventional | 753 |
| 99 | m s⁻² | metres per second squared | WMO conventional | 760 |
| 100 | m² s | square metres second | WMO conventional | 761 |
| 101 | m² s⁻² | square metres per second squared | WMO conventional | 762 |
| 102 | m² rad⁻¹ s | square metres per radian second | WMO conventional | 763 |
| 103 | m² Hz⁻¹ | square metres per hertz | WMO conventional | 764 |
| 104 | m³ | cubic metres | WMO conventional | 765 |
| 105 | m³ s⁻¹ | cubic metres per second | WMO conventional | 766 |
| 106 | m³ m⁻³ | cubic metres per cubic metre | WMO conventional | 767 |
| 107 | m⁴ | metres to the fourth power | WMO conventional | 768 |
| 108 | m⅔ s⁻¹ | metres to the two thirds power per second | WMO conventional | 769 |
| 109 | log (m⁻¹) | logarithm per metre | WMO conventional | 772 |
| 110 | log (m⁻²) | logarithm per square metre | WMO conventional | 773 |
| 111 | kg m⁻¹ | kilograms per metre | WMO conventional | 775 |
| 112 | kg m⁻² s⁻¹ | kilograms per square metre per second | WMO conventional | 776 |
| 113 | kg m⁻³ | kilograms per cubic metre | WMO conventional | 777 |
| 114 | kg⁻² s⁻¹ | per square kilogram per second | WMO conventional | 778 |
| 115 | s m⁻¹ | seconds per metre | WMO conventional | 779 |
| 116 | K m s⁻¹ | kelvin metres per second | WMO conventional | 785 |
| 117 | K m⁻¹ | kelvins per metre | WMO conventional | 786 |
| 118 | K m² kg⁻¹ s⁻¹ | kelvin square metres per kilogram per second | WMO conventional | 787 |
| 119 | mol mol⁻¹ | moles per mole | WMO conventional | 788 |
| 120 | rad m⁻¹ | radians per metre | WMO conventional | 790 |
| 121 | N m⁻² | newtons per square metre | WMO conventional | 795 |
| 122 | Pa s⁻¹ | pascals per second | WMO conventional | 800 |
| 123 | kPa | kilopascal | WMO conventional | 801 |
| 124 | J m⁻² | joules per square metre | WMO conventional | 805 |
| 125 | J kg⁻¹ | joules per kilogram | WMO conventional | 806 |
| 126 | W m⁻¹ sr⁻¹ | watts per metre per steradian | WMO conventional | 810 |
| 127 | W m⁻² | watts per square metre | WMO conventional | 811 |
| 128 | W m⁻² sr⁻¹ | watts per square metre per steradian | WMO conventional | 812 |
| 129 | W m⁻² sr⁻¹ cm | watts per square metre per steradian centimetre | WMO conventional | 813 |
| 130 | W m⁻² sr⁻¹ m | watts per square metre per steradian metre | WMO conventional | 814 |
| 131 | W m⁻³ sr⁻¹ | watts per cubic metre per steradian | WMO conventional | 815 |
| 132 | S m⁻¹ | siemens per metre | WMO conventional | 820 |
| 133 | degree² | square degrees | WMO conventional | 825 |
| 134 | Bq s m⁻³ | becquerel seconds per cubic metre | WMO conventional | 830 |
| 135 | dB m⁻¹ | decibels per metre | WMO conventional | 835 |
| 136 | dB degree⁻¹ | decibels per degree | WMO conventional | 836 |
| 137 | pH unit | pH unit | WMO conventional | 841 |
| 138 | N units | N units | WMO conventional | 842 |
| 139 | NA | (unknown) | NA | NA |

### Code table: 1-05

Code table title: Representativeness [(WMO, 2008)], plus extension

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1-05-01 | (unspecified) | Not applicable in the context of the observed quantity; or: unknown; or: not available | unspecified |
| 1-05-02 | microscale | An area or volume less than 100 m horizontal extent (for example, evaporation) | microscale |
| 1-05-03 | toposcale, local scale | An area or volume of 100 m to 3 km horizontal extent (for example, air pollution, tornadoes) | toposcale |
| 1-05-04 | mesoscale | An area or volume of 3 km to 100 km horizontal extent (for example, thunderstorms, sea and mountain breezes) | mesoscale |
| 1-05-05 | large scale | An area or volume of 100 km to 3000 km horizontal extent (for example, fronts, various cyclones, cloud clusters) | largeScale |
| 1-05-06 | planetary scale | An area or volume of more than 3000 km horizontal extent (for example, long upper tropospheric waves) | planetaryScale |
| 1-05-07 | drainage area | An area (also known as 'catchment') having a common outlet for its surface runoff, in km2 | drainageArea |

### Code table: 2-01

Code table title: Application area(s) [Code table under development]

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Global numerical weather prediction (GNWP) | Global numerical weather prediction (GNWP) covers numerical weather prediction on a global scale, required for weather forecasting for more than a few days ahead. NWP uses mathematical models of the atmosphere and oceans to predict the weather based on current conditions of the atmosphere and oceans. http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-Global-NWP.pdf | globalNumericalWeatherPrediction |
| 2 | High-resolution numerical weather prediction (HRNWP) | High-resolution numerical weather prediction (HRNWP) covers forecasting of meteorological events over a limited area with a 1-5 km horizontal resolution, for 1 or 2 days ahead. Its high spatial resolutions allows (i) to get more realistic descriptions of the surface and of atmospheric phenomena such as clouds and precipitation or boundary layer processes, and (ii) to use denser and more frequent observations than global NWP over the area of interest. http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-HighRes-NWP.pdf | highResolutionNumericalWeatherPrediction |
| 3 | Nowcasting and very short range forecasting (NVSRF) | Nowcasting and very short range forecasting (NVSRF) covers the 0-12 hours forecasting range with high time and space resolution, focusing on rapid evolving phenomena like convection and using dedicated algorithms with combined relevant observed and model data, to produce forecasts usually near the surface, in a seamless way for the forecast range.http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-Nowcasting-VSRF.pdf | nowcasting |
| 4 | Sub-seasonal to longer predictions | Sub-seasonal to longer predictions covers predictions in sub-seasonal to decadal time-ranges (roughly two weeks to 10 years), using numerical prediction models based on current and past observations and analyses. http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-SSLP.pdf | seasonalForecasting |
| 5 | Aeronautical meteorology | Aeronautical meteorology covers worldwide, reliable provision of high quality, timely and cost-effective meteorological services to aviation users. | aeronauticalMeteorology |
| 7 | Ocean applications | http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-Ocean.pdf | oceanApplications |
| 8 | Agricultural meteorology | Agricultural Meteorology relates to specific weather and climate services addressed to the agricultural community to help develop sustainable agricultural production systems. http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-Agriculture.pdf | agriculturalMeteorology |
| 9 | Hydrology | Hydrology covers the assessment of the quantity and quality of water resources, both surface and groundwater (in support of the National Authorities responsible for water management in a wide range of activities), in order to meet the needs of society, to permit mitigation of water-related hazards, and to maintain or enhance the condition of the global environment. http://www.wmo.int/pages/prog/www/OSY/SOG/SOG-Hydrology.pdf | hydrology |
| 10 | Climate monitoring (as supported through the Global Climate Observing System, GCOS) | GCOS, an internationally coordinated network of global observing systems for climate, is designed to meet the requirements for climate observations, which are essential to climate monitoring. Climate observations are fundamental to detect, model and assess climate change, support adaptation to climate change, monitor the effectiveness of policies for mitigating climate change, develop climate information services, promote sustainable national economic development and meet other requirements of the UNFCCC and other convention and agreements. | climateMonitoring |
| 11 | Climate applications | fosters the effective application of climate knowledge and information for the benefit of society and the provision of climate services, including the prediction of significant climate variations both natural and as a result of human activity http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-Climate- CCl.pdf | climateApplications |
| 12 | Space weather | Space weather is a discipline which aims at observing, understanding and predicting the state of the Sun, of the interplanetary and planetary environments, their disturbances, and the potential impacts of these disturbances on biological and technological systems. http://www.wmo.int/pages/prog/www/OSY/SOG/SoG-SW.pdf | spaceWeather |
| 13 | Cryosphere | EGOS-IP | cryosphere |
| 14 | Energy sector | Economic activities that produce or distribute energy, such as renewable energies, e.g. solar, thermal, wind | energy |
| 15 | Transportation sector | Economic activities related to transportation of people and goods, on land, air and water | transport |
| 16 | Health sector | Health-related services, often provided to the public, particularly those related to prevention, e.g. pollen allergies, UV radiation, heat wave alerts | health |
| 17 | Terrestrial ecology | Activities and services related to the terrestrial environment, e.g., relating to bio-diversity | terrestrialEcology |
| 21 | Atmospheric composition forecasting | Applications from global to regional scales (with horizontal resolutions similar to global NWP (~10 km and coarser) with stringent timeliness requirements (NRT) to support operations such as sand and dust storm and chemical weather forecasts [source: http://www.wmo.int/pages/prog/arep/gaw/documents/FINAL_GAW_221.pdf] | atmosphericCompositionForecasting |
| 22 | Atmospheric composition monitoring and analysis | Applications related to evaluating and analysing changes (temporally and spatially) in atmospheric composition regionally and globally to support treaty monitoring, climatologies and re-analyses, assessing trends in composition and emissions/fluxes, and to better understand processes, using data of controlled quality (and with less stringent time requirements (not needed in NRT)), and used in products such as Ozone and Greenhouse Gas Bulletins, and State/Health of the Atmosphere reports [source: http://www.wmo.int/pages/prog/arep/gaw/documents/FINAL_GAW_221.pdf] | atmosphericCompositionMonitoring |
| 23 | Integrated urban services | Applications that target limited areas (with horizontal resolution of a few km or smaller and stringent timeliness requirements to support services related to weather/climate/pollution, such as air quality forecasting [source: http://www.wmo.int/pages/prog/arep/gaw/documents/FINAL_GAW_221.pdf] | integratedUrbanServices |

### Code table: 2-02

Code table title: Programme/Network affiliation [Code table under development]

| # | Path | Name | P_ID | AA_ID | Definition | URL | WMO306_CD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | >Co-sponsored | Co-sponsored | NA | 2 | WMO contributions to co-sponsored observing systems | NA | NA |
| 2 | >Co-sponsored>GCOS | GCOS | 17 | 2 | Global Climate Observing System | NA | gcos |
| 9 | >Co-sponsored>GCOS>GRUAN | GRUAN | 2 | 2 | GCOS Reference Upper-Air Network | NA | gruan |
| 10 | >Co-sponsored>GCOS>GSN | GSN | 2 | 2 | GCOS Surface Network | http://www.wmo.int/pages/prog/gcos/index.php?name=ObservingSystemsandData#surface | gsn |
| 11 | >Co-sponsored>GCOS>GUAN | GUAN | 2 | 2 | GCOS Upper-Air Network | http://www.wmo.int/pages/prog/gcos/index.php?name=ObservingSystemsandData#upper-air | guan |
| 62 | >Co-sponsored>GOOS | GOOS | 17 | 2 | Global Ocean Observing System | www.ioc-goos.org/ | goos |
| 13 | >Co-sponsored>GOOS>Argo | Argo | 62 | 2 | IOC/WMO Argo program of drifting sub-surface floats | NA | argo |
| 73 | >Co-sponsored>GOOS>Argo>ARGO ARGENTINA | ARGO ARGENTINA | 13 | 2 | NA | NA | argoArgentina |
| 74 | >Co-sponsored>GOOS>Argo>ARGO AUSTRALIA | ARGO AUSTRALIA | 13 | 2 | NA | NA | argoAustralia |
| 75 | >Co-sponsored>GOOS>Argo>ARGO AUSTRALIA EQ. | ARGO AUSTRALIA EQ. | 13 | 2 | NA | NA | aroEqAustralia |
| 76 | >Co-sponsored>GOOS>Argo>ARGO AWI | ARGO AWI | 13 | 2 | NA | NA | argoAWI |
| 77 | >Co-sponsored>GOOS>Argo>ARGO BRAZIL | ARGO BRAZIL | 13 | 2 | NA | NA | argoBrazil |
| 318 | >Co-sponsored>GOOS>Argo>ARGO BRAZIL NAVY | ARGO BRAZIL NAVY | 13 | 2 | NA | NA | NA |
| 78 | >Co-sponsored>GOOS>Argo>ARGO BSH | ARGO BSH | 13 | 2 | NA | NA | argoBSH |
| 79 | >Co-sponsored>GOOS>Argo>ARGO CANADA | ARGO CANADA | 13 | 2 | NA | NA | argoCanada |
| 80 | >Co-sponsored>GOOS>Argo>ARGO CHILE | ARGO CHILE | 13 | 2 | NA | NA | argoChile |
| 81 | >Co-sponsored>GOOS>Argo>ARGO CHINA | ARGO CHINA | 13 | 2 | NA | NA | argoChina |
| 82 | >Co-sponsored>GOOS>Argo>ARGO CHINA SOA | ARGO CHINA SOA | 13 | 2 | NA | NA | argoChinaSOA |
| 83 | >Co-sponsored>GOOS>Argo>ARGO COSTA RICA | ARGO COSTA RICA | 13 | 2 | NA | NA | argoCostaRica |
| 84 | >Co-sponsored>GOOS>Argo>ARGO DENMARK | ARGO DENMARK | 13 | 2 | NA | NA | argoDenmark |
| 85 | >Co-sponsored>GOOS>Argo>ARGO ECUADOR | ARGO ECUADOR | 13 | 2 | NA | NA | argoEcuador |
| 86 | >Co-sponsored>GOOS>Argo>ARGO EQ. AOML | ARGO EQ. AOML | 13 | 2 | NA | NA | argoEqAOML |
| 87 | >Co-sponsored>GOOS>Argo>ARGO EQ. AWI | ARGO EQ. AWI | 13 | 2 | NA | NA | argoEqAWI |
| 88 | >Co-sponsored>GOOS>Argo>ARGO EQ. BSH | ARGO EQ. BSH | 13 | 2 | NA | NA | argoEqBSH |
| 89 | >Co-sponsored>GOOS>Argo>ARGO EQ. CHINA | ARGO EQ. CHINA | 13 | 2 | NA | NA | argoEqChina |
| 90 | >Co-sponsored>GOOS>Argo>ARGO EQ. ESP-OMZ | ARGO EQ. ESP-OMZ | 13 | 2 | NA | NA | argoEqESPOMZ |
| 91 | >Co-sponsored>GOOS>Argo>ARGO EQ. FERHRI | ARGO EQ. FERHRI | 13 | 2 | NA | NA | argoEqFERHRI |
| 92 | >Co-sponsored>GOOS>Argo>ARGO EQ. FSU | ARGO EQ. FSU | 13 | 2 | NA | NA | argoEqFSU |
| 93 | >Co-sponsored>GOOS>Argo>ARGO EQ. HNFRI | ARGO EQ. HNFRI | 13 | 2 | NA | NA | argoEqHNFRI |
| 94 | >Co-sponsored>GOOS>Argo>ARGO EQ. IFM | ARGO EQ. IFM | 13 | 2 | NA | NA | argoEqIFM |
| 95 | >Co-sponsored>GOOS>Argo>ARGO EQ. IFM2 | ARGO EQ. IFM2 | 13 | 2 | NA | NA | argoEqIFM2 |
| 96 | >Co-sponsored>GOOS>Argo>ARGO EQ. IRELAND | ARGO EQ. IRELAND | 13 | 2 | NA | NA | argoEqIreland |
| 97 | >Co-sponsored>GOOS>Argo>ARGO EQ. JAMSTEC | ARGO EQ. JAMSTEC | 13 | 2 | NA | NA | argoEqJamstec |
| 98 | >Co-sponsored>GOOS>Argo>ARGO EQ. JMA | ARGO EQ. JMA | 13 | 2 | NA | NA | argoEqJMA |
| 99 | >Co-sponsored>GOOS>Argo>ARGO EQ. NAVOCEANO | ARGO EQ. NAVOCEANO | 13 | 2 | NA | NA | argoEqNavoceano |
| 100 | >Co-sponsored>GOOS>Argo>ARGO EQ. NDBC | ARGO EQ. NDBC | 13 | 2 | NA | NA | argoEqNDBC |
| 101 | >Co-sponsored>GOOS>Argo>ARGO EQ. NIPR | ARGO EQ. NIPR | 13 | 2 | NA | NA | argoEqNIPR |
| 102 | >Co-sponsored>GOOS>Argo>ARGO EQ. NRIFS | ARGO EQ. NRIFS | 13 | 2 | NA | NA | argoEqNRIFS |
| 103 | >Co-sponsored>GOOS>Argo>ARGO EQ. OIST | ARGO EQ. OIST | 13 | 2 | NA | NA | argoEqOIST |
| 104 | >Co-sponsored>GOOS>Argo>ARGO EQ. ORI | ARGO EQ. ORI | 13 | 2 | NA | NA | argoEqORI |
| 105 | >Co-sponsored>GOOS>Argo>ARGO EQ. PMEL | ARGO EQ. PMEL | 13 | 2 | NA | NA | argoEqPMEL |
| 106 | >Co-sponsored>GOOS>Argo>ARGO EQ. POMME | ARGO EQ. POMME | 13 | 2 | NA | NA | argoEqPomme |
| 107 | >Co-sponsored>GOOS>Argo>ARGO EQ. SAGE | ARGO EQ. SAGE | 13 | 2 | NA | NA | argoEqSage |
| 108 | >Co-sponsored>GOOS>Argo>ARGO EQ. TNFRI | ARGO EQ. TNFRI | 13 | 2 | NA | NA | argoEqTNFRI |
| 109 | >Co-sponsored>GOOS>Argo>ARGO EQ. TSK | ARGO EQ. TSK | 13 | 2 | NA | NA | argoEqTSK |
| 110 | >Co-sponsored>GOOS>Argo>ARGO EQ. TU | ARGO EQ. TU | 13 | 2 | NA | NA | argoEqTU |
| 111 | >Co-sponsored>GOOS>Argo>ARGO EQ. UH | ARGO EQ. UH | 13 | 2 | NA | NA | argoEqUH |
| 112 | >Co-sponsored>GOOS>Argo>ARGO EQ. UHH | ARGO EQ. UHH | 13 | 2 | NA | NA | argoEqUHH |
| 113 | >Co-sponsored>GOOS>Argo>ARGO EQ. UM-OSU | ARGO EQ. UM-OSU | 13 | 2 | NA | NA | argoEqUMOSU |
| 114 | >Co-sponsored>GOOS>Argo>ARGO EQ. VOCALS | ARGO EQ. VOCALS | 13 | 2 | NA | NA | argoEqVocals |
| 115 | >Co-sponsored>GOOS>Argo>ARGO EQ. WHOI | ARGO EQ. WHOI | 13 | 2 | NA | NA | argoEqWHOI |
| 116 | >Co-sponsored>GOOS>Argo>ARGO FINLAND | ARGO FINLAND | 13 | 2 | NA | NA | argoFinland |
| 117 | >Co-sponsored>GOOS>Argo>ARGO GABON | ARGO GABON | 13 | 2 | NA | NA | argoGabon |
| 118 | >Co-sponsored>GOOS>Argo>ARGO GERMANY | ARGO GERMANY | 13 | 2 | NA | NA | argoGermany |
| 119 | >Co-sponsored>GOOS>Argo>ARGO GREECE | ARGO GREECE | 13 | 2 | NA | NA | argoGreece |
| 120 | >Co-sponsored>GOOS>Argo>ARGO IFM-GEOMAR | ARGO IFM-GEOMAR | 13 | 2 | NA | NA | argoIFMGeomar |
| 121 | >Co-sponsored>GOOS>Argo>ARGO INDIA | ARGO INDIA | 13 | 2 | NA | NA | argoIndia |
| 122 | >Co-sponsored>GOOS>Argo>ARGO IRELAND | ARGO IRELAND | 13 | 2 | NA | NA | argoIreland |
| 123 | >Co-sponsored>GOOS>Argo>ARGO ITALY | ARGO ITALY | 13 | 2 | NA | NA | argoItaly |
| 124 | >Co-sponsored>GOOS>Argo>ARGO JAMSTEC | ARGO JAMSTEC | 13 | 2 | NA | NA | argoJamstec |
| 125 | >Co-sponsored>GOOS>Argo>ARGO JMA | ARGO JMA | 13 | 2 | NA | NA | argoJMA |
| 126 | >Co-sponsored>GOOS>Argo>ARGO KENYA | ARGO KENYA | 13 | 2 | NA | NA | argoKenya |
| 127 | >Co-sponsored>GOOS>Argo>ARGO KIOST | ARGO KIOST | 13 | 2 | NA | NA | argoKiost |
| 128 | >Co-sponsored>GOOS>Argo>ARGO LEBANON | ARGO LEBANON | 13 | 2 | NA | NA | argoLebanon |
| 129 | >Co-sponsored>GOOS>Argo>ARGO MAURITIUS | ARGO MAURITIUS | 13 | 2 | NA | NA | argoMauritius |
| 130 | >Co-sponsored>GOOS>Argo>ARGO MEX-CO-US | ARGO MEX-CO-US | 13 | 2 | NA | NA | argoMexCoUS |
| 131 | >Co-sponsored>GOOS>Argo>ARGO MEXICO | ARGO MEXICO | 13 | 2 | NA | NA | argoMexico |
| 132 | >Co-sponsored>GOOS>Argo>ARGO NETHERLANDS | ARGO NETHERLANDS | 13 | 2 | NA | NA | argoNetherlands |
| 133 | >Co-sponsored>GOOS>Argo>ARGO NEW ZEALAND | ARGO NEW ZEALAND | 13 | 2 | NA | NA | argoNewZealand |
| 134 | >Co-sponsored>GOOS>Argo>ARGO NIMR/KMA | ARGO NIMR/KMA | 13 | 2 | NA | NA | argoMIMRKMA |
| 135 | >Co-sponsored>GOOS>Argo>ARGO NORWAY | ARGO NORWAY | 13 | 2 | NA | NA | argoNorway |
| 136 | >Co-sponsored>GOOS>Argo>ARGO PMEL | ARGO PMEL | 13 | 2 | NA | NA | argoPMEL |
| 137 | >Co-sponsored>GOOS>Argo>ARGO POLAND | ARGO POLAND | 13 | 2 | NA | NA | argoPoland |
| 138 | >Co-sponsored>GOOS>Argo>ARGO RUSSIA | ARGO RUSSIA | 13 | 2 | NA | NA | argoRussia |
| 139 | >Co-sponsored>GOOS>Argo>ARGO SAUDI ARABIA | ARGO SAUDI ARABIA | 13 | 2 | NA | NA | argoSaudiArabia |
| 140 | >Co-sponsored>GOOS>Argo>ARGO SIO | ARGO SIO | 13 | 2 | NA | NA | argoSIO |
| 141 | >Co-sponsored>GOOS>Argo>ARGO SIO EQ (ASIRI) | ARGO SIO EQ (ASIRI) | 13 | 2 | NA | NA | argoSIOEqASIRI |
| 142 | >Co-sponsored>GOOS>Argo>ARGO SIO EQ. (OKMC) | ARGO SIO EQ. (OKMC) | 13 | 2 | NA | NA | argoSIOEqOKMC |
| 143 | >Co-sponsored>GOOS>Argo>ARGO SOUTH AFRICA | ARGO SOUTH AFRICA | 13 | 2 | NA | NA | argoSouthAfrica |
| 144 | >Co-sponsored>GOOS>Argo>ARGO SPAIN | ARGO SPAIN | 13 | 2 | NA | NA | argoSpain |
| 145 | >Co-sponsored>GOOS>Argo>ARGO SRI LANKA | ARGO SRI LANKA | 13 | 2 | NA | NA | argoSriLanka |
| 146 | >Co-sponsored>GOOS>Argo>ARGO TWR ENGINEERING | ARGO TWR ENGINEERING | 13 | 2 | NA | NA | argoTWREng |
| 147 | >Co-sponsored>GOOS>Argo>ARGO UK | ARGO UK | 13 | 2 | NA | NA | argoUK |
| 148 | >Co-sponsored>GOOS>Argo>ARGO UK BIO | ARGO UK BIO | 13 | 2 | NA | NA | argoUKBIO |
| 149 | >Co-sponsored>GOOS>Argo>ARGO UK EQ. | ARGO UK EQ. | 13 | 2 | NA | NA | argoUKEq |
| 150 | >Co-sponsored>GOOS>Argo>ARGO UW | ARGO UW | 13 | 2 | NA | NA | argoUW |
| 151 | >Co-sponsored>GOOS>Argo>ARGO UW EQ. | ARGO UW EQ. | 13 | 2 | NA | NA | argoUWEq |
| 152 | >Co-sponsored>GOOS>Argo>ARGO UW-APL EQ. | ARGO UW-APL EQ. | 13 | 2 | NA | NA | argoUWAplEq |
| 153 | >Co-sponsored>GOOS>Argo>ARGO UW-MBARI EQ. | ARGO UW-MBARI EQ. | 13 | 2 | NA | NA | argoUWMbariEq |
| 154 | >Co-sponsored>GOOS>Argo>ARGO UW-SOCCOM EQ. | ARGO UW-SOCCOM EQ. | 13 | 2 | NA | NA | argoUWSoccomEq |
| 155 | >Co-sponsored>GOOS>Argo>ARGO UW-SPURS EQ. | ARGO UW-SPURS EQ. | 13 | 2 | NA | NA | argoUWSpursEq |
| 156 | >Co-sponsored>GOOS>Argo>ARGO UW-UA EQ. | ARGO UW-UA EQ. | 13 | 2 | NA | NA | argoUWUAEq |
| 157 | >Co-sponsored>GOOS>Argo>ARGO WHOI | ARGO WHOI | 13 | 2 | NA | NA | argoWHOI |
| 158 | >Co-sponsored>GOOS>Argo>ARGO WHOI EQ. IR | ARGO WHOI EQ. IR | 13 | 2 | NA | NA | argoWHOIEqIR |
| 159 | >Co-sponsored>GOOS>Argo>ARGO WHOI-MRV EQ. | ARGO WHOI-MRV EQ. | 13 | 2 | NA | NA | argoWHOIMRVEq |
| 169 | >Co-sponsored>GOOS>Argo>BULARGO | BULARGO | 13 | 2 | NA | NA | argoBulargo |
| 175 | >Co-sponsored>GOOS>Argo>CORIOLIS | CORIOLIS | 13 | 2 | NA | NA | argoCoriolis |
| 176 | >Co-sponsored>GOOS>Argo>CORIOLIS-BIOARGO | CORIOLIS-BIOARGO | 13 | 2 | NA | NA | argoCoriolisBioargo |
| 177 | >Co-sponsored>GOOS>Argo>CORIOLIS-CANOA | CORIOLIS-CANOA | 13 | 2 | NA | NA | argoCoriolisCanoa |
| 178 | >Co-sponsored>GOOS>Argo>CORIOLIS-CONGAS | CORIOLIS-CONGAS | 13 | 2 | NA | NA | argoCoriolisCongas |
| 179 | >Co-sponsored>GOOS>Argo>CORIOLIS-DRAKE | CORIOLIS-DRAKE | 13 | 2 | NA | NA | argoCoriolisDrake |
| 180 | >Co-sponsored>GOOS>Argo>CORIOLIS-EGEE | CORIOLIS-EGEE | 13 | 2 | NA | NA | argoCoriolisEGEE |
| 181 | >Co-sponsored>GOOS>Argo>CORIOLIS-EGYPT | CORIOLIS-EGYPT | 13 | 2 | NA | NA | argoCoriolisEqypt |
| 182 | >Co-sponsored>GOOS>Argo>CORIOLIS-FLOPS | CORIOLIS-FLOPS | 13 | 2 | NA | NA | argoCoriolisFlops |
| 183 | >Co-sponsored>GOOS>Argo>CORIOLIS-FLOSTRAL | CORIOLIS-FLOSTRAL | 13 | 2 | NA | NA | argoCoriolisFlostral |
| 184 | >Co-sponsored>GOOS>Argo>CORIOLIS-FNOB-JCOMMOPS | CORIOLIS-FNOB-JCOMMOPS | 13 | 2 | NA | NA | argoCoriolisFNOBJCOMMOPS |
| 185 | >Co-sponsored>GOOS>Argo>CORIOLIS-FRONTALIS | CORIOLIS-FRONTALIS | 13 | 2 | NA | NA | argoCoriolisFrontalis |
| 186 | >Co-sponsored>GOOS>Argo>CORIOLIS-GOOD HOPE | CORIOLIS-GOOD HOPE | 13 | 2 | NA | NA | argoCoriolisGoodHope |
| 187 | >Co-sponsored>GOOS>Argo>CORIOLIS-OVIDE | CORIOLIS-OVIDE | 13 | 2 | NA | NA | argoCoriolisOvide |
| 188 | >Co-sponsored>GOOS>Argo>CORIOLIS-PIRATA | CORIOLIS-PIRATA | 13 | 2 | NA | NA | argoCoriolisPirata |
| 189 | >Co-sponsored>GOOS>Argo>CORIOLIS-PREVIMER | CORIOLIS-PREVIMER | 13 | 2 | NA | NA | argoCoriolisPrevimer |
| 190 | >Co-sponsored>GOOS>Argo>CORIOLIS-PROSAT | CORIOLIS-PROSAT | 13 | 2 | NA | NA | argoCoriolisProsat |
| 191 | >Co-sponsored>GOOS>Argo>CORIOLIS-REMOCEAN | CORIOLIS-REMOCEAN | 13 | 2 | NA | NA | argoCoriolisRemocean |
| 320 | >Co-sponsored>GOOS>Argo>CORIOLIS-REMOCEAN EQ. | CORIOLIS-REMOCEAN EQ. | 13 | 2 | NA | NA | NA |
| 192 | >Co-sponsored>GOOS>Argo>CORIOLIS-SPICE | CORIOLIS-SPICE | 13 | 2 | NA | NA | argoCoriolisSpice |
| 193 | >Co-sponsored>GOOS>Argo>CORIOLIS-TRACK | CORIOLIS-TRACK | 13 | 2 | NA | NA | argoCoriolisTrack |
| 196 | >Co-sponsored>GOOS>Argo>DEKOSIM | DEKOSIM | 13 | 2 | NA | NA | argoDekosim |
| 198 | >Co-sponsored>GOOS>Argo>E-AIMS | E-AIMS | 13 | 2 | NA | NA | argoEAims |
| 207 | >Co-sponsored>GOOS>Argo>EUROARGO | EUROARGO | 13 | 2 | NA | NA | argoEurargo |
| 213 | >Co-sponsored>GOOS>Argo>GYROSCOPE | GYROSCOPE | 13 | 2 | NA | NA | argoGyroscope |
| 228 | >Co-sponsored>GOOS>Argo>ITP-DAMOCLES | ITP-DAMOCLES | 13 | 2 | NA | NA | argoDamocles |
| 229 | >Co-sponsored>GOOS>Argo>ITP-WHOI | ITP-WHOI | 13 | 2 | NA | NA | argoWHOI |
| 234 | >Co-sponsored>GOOS>Argo>MEDARGO | MEDARGO | 13 | 2 | NA | NA | argoMedargo |
| 235 | >Co-sponsored>GOOS>Argo>MERIDIAN GOODHOPE | MERIDIAN GOODHOPE | 13 | 2 | NA | NA | argoMeridianGoodHope |
| 236 | >Co-sponsored>GOOS>Argo>MERSEA | MERSEA | 13 | 2 | NA | NA | argoMersea |
| 240 | >Co-sponsored>GOOS>Argo>NAOS-FRANCE | NAOS-FRANCE | 13 | 2 | NA | NA | argoNaosFrance |
| 68 | >Co-sponsored>GOOS>DBCP | DBCP | 62 | 2 | NA | NA | NA |
| 70 | >Co-sponsored>GOOS>DBCP>AOML | AOML | 68 | 2 | NA | NA | dbcpAOML |
| 71 | >Co-sponsored>GOOS>DBCP>AOML-E-SURFMAR | AOML-E-SURFMAR | 68 | 2 | NA | NA | dbcpAOMLeSurfmar |
| 330 | >Co-sponsored>GOOS>DBCP>AOML-MF DB | AOML-MF DB | 68 | 2 | NA | NA | NA |
| 72 | >Co-sponsored>GOOS>DBCP>APL-UW | APL-UW | 68 | 2 | NA | NA | dbcpAPLUW |
| 161 | >Co-sponsored>GOOS>DBCP>AWI-DB | AWI-DB | 68 | 2 | NA | NA | dbcpAWIDB |
| 162 | >Co-sponsored>GOOS>DBCP>BOM DB | BOM DB | 68 | 2 | NA | NA | dbcpBomDB |
| 163 | >Co-sponsored>GOOS>DBCP>BOM-TSU | BOM-TSU | 68 | 2 | NA | NA | dbcpBomTSU |
| 164 | >Co-sponsored>GOOS>DBCP>BP INC | BP INC | 68 | 2 | NA | NA | dbcpBPInc |
| 165 | >Co-sponsored>GOOS>DBCP>BRAZIL PNBOIA-INPE | BRAZIL PNBOIA-INPE | 68 | 2 | NA | NA | dbcpBrazilPNBOIAINPE |
| 166 | >Co-sponsored>GOOS>DBCP>BSH-DWD-FIXED | BSH-DWD-FIXED | 68 | 2 | NA | NA | dbcpBshDWDFixed |
| 167 | >Co-sponsored>GOOS>DBCP>BSH-DWD-MB | BSH-DWD-MB | 68 | 2 | NA | NA | dbcpBshDWDMD |
| 168 | >Co-sponsored>GOOS>DBCP>BSH-MB | BSH-MB | 68 | 2 | NA | NA | dbcpBshMB |
| 170 | >Co-sponsored>GOOS>DBCP>CANDHIS | CANDHIS | 68 | 2 | NA | NA | dbcpCandhis |
| 171 | >Co-sponsored>GOOS>DBCP>CARICOOS | CARICOOS | 68 | 2 | NA | NA | dbcpCaricoos |
| 172 | >Co-sponsored>GOOS>DBCP>CARIOCA | CARIOCA | 68 | 2 | NA | NA | dbcpCarioca |
| 173 | >Co-sponsored>GOOS>DBCP>CDIP | CDIP | 68 | 2 | NA | NA | dbcpCDIP |
| 174 | >Co-sponsored>GOOS>DBCP>COLOMBIA-TSU | COLOMBIA-TSU | 68 | 2 | NA | NA | dbcpColombiaTSU |
| 194 | >Co-sponsored>GOOS>DBCP>CORMP-NC | CORMP-NC | 68 | 2 | NA | NA | dbcpCormpNC |
| 195 | >Co-sponsored>GOOS>DBCP>DBCP | DBCP | 68 | 2 | NA | NA | dbcp |
| 197 | >Co-sponsored>GOOS>DBCP>DFO DB | DFO DB | 68 | 2 | NA | NA | cbcpDFODB |
| 205 | >Co-sponsored>GOOS>DBCP>EC DB | EC DB | 68 | 2 | NA | NA | dbcpECDB |
| 206 | >Co-sponsored>GOOS>DBCP>EC MB | EC MB | 68 | 2 | NA | NA | dbcpECMB |
| 321 | >Co-sponsored>GOOS>DBCP>EC-IABP | EC-IABP | 68 | 2 | NA | NA | NA |
| 200 | >Co-sponsored>GOOS>DBCP>E-SURFMAR | E-SURFMAR | 68 | 2 | NA | NA | eSurfmar |
| 201 | >Co-sponsored>GOOS>DBCP>E-SURFMAR-IRISH IMI | E-SURFMAR-IRISH IMI | 68 | 2 | NA | NA | eSurfmarIrishIMI |
| 202 | >Co-sponsored>GOOS>DBCP>E-SURFMAR-NOAA | E-SURFMAR-NOAA | 68 | 2 | NA | NA | eSurfmarNOAA |
| 203 | >Co-sponsored>GOOS>DBCP>E-SURFMAR-UK MB | E-SURFMAR-UK MB | 68 | 2 | NA | NA | eSurfmarUKMB |
| 204 | >Co-sponsored>GOOS>DBCP>E-SURFMAR-UK/FR MB | E-SURFMAR-UK/FR MB | 68 | 2 | NA | NA | eSurfmarUKFRMB |
| 208 | >Co-sponsored>GOOS>DBCP>EUROSITES PLANIER | EUROSITES PLANIER | 68 | 2 | NA | NA | dbcpEurositesPlanier |
| 209 | >Co-sponsored>GOOS>DBCP>FERHRI-TSU | FERHRI-TSU | 68 | 2 | NA | NA | dbcpFerhriTSU |
| 210 | >Co-sponsored>GOOS>DBCP>GERMANY MB | GERMANY MB | 68 | 2 | NA | NA | dbcpGermanyMB |
| 211 | >Co-sponsored>GOOS>DBCP>GLOSCAL | GLOSCAL | 68 | 2 | NA | NA | dbcpGloscal |
| 212 | >Co-sponsored>GOOS>DBCP>GREECE MB | GREECE MB | 68 | 2 | NA | NA | dbcpGreeceMB |
| 322 | >Co-sponsored>GOOS>DBCP>HK-OBS-DB | HK-OBS-DB | 68 | 2 | NA | NA | NA |
| 214 | >Co-sponsored>GOOS>DBCP>IABP | IABP | 68 | 2 | NA | NA | dbcpIABP |
| 215 | >Co-sponsored>GOOS>DBCP>IABP CANADA | IABP CANADA | 68 | 2 | NA | NA | dbcpIABPCanada |
| 216 | >Co-sponsored>GOOS>DBCP>IABP NIC | IABP NIC | 68 | 2 | NA | NA | dbcpIABPNIC |
| 217 | >Co-sponsored>GOOS>DBCP>IABP US | IABP US | 68 | 2 | NA | NA | dbcpIABPUS |
| 218 | >Co-sponsored>GOOS>DBCP>IABP-EC | IABP-EC | 68 | 2 | NA | NA | dbcpIABPEC |
| 219 | >Co-sponsored>GOOS>DBCP>IABP-UW | IABP-UW | 68 | 2 | NA | NA | dbcpIABPUW |
| 220 | >Co-sponsored>GOOS>DBCP>IBPIO | IBPIO | 68 | 2 | NA | NA | dbcpIBPIO |
| 221 | >Co-sponsored>GOOS>DBCP>INCOIS-TSU | INCOIS-TSU | 68 | 2 | NA | NA | dbcpINCOISTSU |
| 222 | >Co-sponsored>GOOS>DBCP>INDIA NIO | INDIA NIO | 68 | 2 | NA | NA | dbcpIndiaNIO |
| 223 | >Co-sponsored>GOOS>DBCP>INDIA NIO-TSU | INDIA NIO-TSU | 68 | 2 | NA | NA | dbcpIndiaNIOTSU |
| 224 | >Co-sponsored>GOOS>DBCP>INDIA-NIOT | INDIA-NIOT | 68 | 2 | NA | NA | dbcpIndiaNIOT |
| 225 | >Co-sponsored>GOOS>DBCP>INOCAR-TSU | INOCAR-TSU | 68 | 2 | NA | NA | dbcpINOCARTSU |
| 226 | >Co-sponsored>GOOS>DBCP>ITALY DB | ITALY DB | 68 | 2 | NA | NA | dbcpItalyDB |
| 227 | >Co-sponsored>GOOS>DBCP>ITALY-OGS | ITALY-OGS | 68 | 2 | NA | NA | dbcpItalyOGS |
| 230 | >Co-sponsored>GOOS>DBCP>JMA DB | JMA DB | 68 | 2 | NA | NA | dbcpJMADB |
| 231 | >Co-sponsored>GOOS>DBCP>JMA-TSU | JMA-TSU | 68 | 2 | NA | NA | dbcpJMATSU |
| 232 | >Co-sponsored>GOOS>DBCP>KMA MB | KMA MB | 68 | 2 | NA | NA | dbcpKMAMB |
| 233 | >Co-sponsored>GOOS>DBCP>LOCEAN | LOCEAN | 68 | 2 | NA | NA | dbcpLocean |
| 323 | >Co-sponsored>GOOS>DBCP>MARLIN-DB | MARLIN-DB | 68 | 2 | NA | NA | NA |
| 237 | >Co-sponsored>GOOS>DBCP>METEO FRANCE DB | METEO FRANCE DB | 68 | 2 | NA | NA | dbcpMeteoFranceDB |
| 238 | >Co-sponsored>GOOS>DBCP>METEO FRANCE MB | METEO FRANCE MB | 68 | 2 | NA | NA | dbcpMeteoFranceMB |
| 239 | >Co-sponsored>GOOS>DBCP>METEO FRANCE WB | METEO FRANCE WB | 68 | 2 | NA | NA | dbcpMeteoFranceWB |
| 241 | >Co-sponsored>GOOS>DBCP>NCCOOS | NCCOOS | 68 | 2 | NA | NA | dbcpNCCOOS |
| 242 | >Co-sponsored>GOOS>DBCP>NDBC-BP | NDBC-BP | 68 | 2 | NA | NA | dbcpNDBCBP |
| 243 | >Co-sponsored>GOOS>DBCP>NDBC-COE | NDBC-COE | 68 | 2 | NA | NA | dbcpNDBCCOE |
| 244 | >Co-sponsored>GOOS>DBCP>NDBC-CSI | NDBC-CSI | 68 | 2 | NA | NA | dbcpNDBCCSI |
| 245 | >Co-sponsored>GOOS>DBCP>NDBC-MB | NDBC-MB | 68 | 2 | NA | NA | dbcpNDBCMB |
| 246 | >Co-sponsored>GOOS>DBCP>NDBC-MBARI | NDBC-MBARI | 68 | 2 | NA | NA | dbcpNDBCMBARI |
| 247 | >Co-sponsored>GOOS>DBCP>NDBC-NASA | NDBC-NASA | 68 | 2 | NA | NA | dbcpNDBCNASA |
| 248 | >Co-sponsored>GOOS>DBCP>NDBC-NOAA-NWS | NDBC-NOAA-NWS | 68 | 2 | NA | NA | dbcpcNOAANWS |
| 249 | >Co-sponsored>GOOS>DBCP>NDBC-TABS | NDBC-TABS | 68 | 2 | NA | NA | dbcpNDBCTABS |
| 250 | >Co-sponsored>GOOS>DBCP>NDBC-TSU | NDBC-TSU | 68 | 2 | NA | NA | dbcpNDBCTSU |
| 251 | >Co-sponsored>GOOS>DBCP>NDBC-USCG | NDBC-USCG | 68 | 2 | NA | NA | dbcpNDBCUSCG |
| 252 | >Co-sponsored>GOOS>DBCP>NDWC-TSU | NDWC-TSU | 68 | 2 | NA | NA | dbcpNDWCTSU |
| 253 | >Co-sponsored>GOOS>DBCP>NEW ZEALAND DB | NEW ZEALAND DB | 68 | 2 | NA | NA | dbcpNewZealandDB |
| 324 | >Co-sponsored>GOOS>DBCP>NIC-IABP | NIC-IABP | 68 | 2 | NA | NA | NA |
| 254 | >Co-sponsored>GOOS>DBCP>NOAA-CBIBS | NOAA-CBIBS | 68 | 2 | NA | NA | dbcpNOAACBIBS |
| 255 | >Co-sponsored>GOOS>DBCP>NORWAY-IMR | NORWAY-IMR | 68 | 2 | NA | NA | dbcpNorwayIMR |
| 256 | >Co-sponsored>GOOS>DBCP>NWS-NDBC-MB | NWS-NDBC-MB | 68 | 2 | NA | NA | dbcpNWSNDBCMB |
| 257 | >Co-sponsored>GOOS>DBCP>NWS-NDBC-WB | NWS-NDBC-WB | 68 | 2 | NA | NA | dbcpNDBCWB |
| 258 | >Co-sponsored>GOOS>DBCP>PACIOOS | PACIOOS | 68 | 2 | NA | NA | dbcpPacioos |
| 259 | >Co-sponsored>GOOS>DBCP>PETROBRAS | PETROBRAS | 68 | 2 | NA | NA | dbcpPetrobras |
| 325 | >Co-sponsored>GOOS>DBCP>PMEL MB | PMEL MB | 68 | 2 | NA | NA | NA |
| 260 | >Co-sponsored>GOOS>DBCP>SCRIPS | SCRIPS | 68 | 2 | NA | NA | dbcpScrips |
| 261 | >Co-sponsored>GOOS>DBCP>SHELL | SHELL | 68 | 2 | NA | NA | dbcpShell |
| 262 | >Co-sponsored>GOOS>DBCP>SHOA-TSU | SHOA-TSU | 68 | 2 | NA | NA | dbcpShoaTSU |
| 263 | >Co-sponsored>GOOS>DBCP>SIO-DB | SIO-DB | 68 | 2 | NA | NA | dbcpSIODB |
| 264 | >Co-sponsored>GOOS>DBCP>SPAIN-PDE MB | SPAIN-PDE MB | 68 | 2 | NA | NA | dbcpSpainPDEMB |
| 265 | >Co-sponsored>GOOS>DBCP>STRATUS | STRATUS | 68 | 2 | NA | NA | dbcpStratus |
| 266 | >Co-sponsored>GOOS>DBCP>TAO ATLAS | TAO ATLAS | 68 | 2 | NA | NA | dbcpTaoAtlas |
| 267 | >Co-sponsored>GOOS>DBCP>TAO PIRATA | TAO PIRATA | 68 | 2 | NA | NA | dbcpTaoPirata |
| 268 | >Co-sponsored>GOOS>DBCP>TAO PIRATA US | TAO PIRATA US | 68 | 2 | NA | NA | dbcpTaoPirataUS |
| 269 | >Co-sponsored>GOOS>DBCP>TAO RAMA US-INDIA | TAO RAMA US-INDIA | 68 | 2 | NA | NA | dbcpTaoRamaUSIndia |
| 270 | >Co-sponsored>GOOS>DBCP>TAO TRITON | TAO TRITON | 68 | 2 | NA | NA | dbcpTaoTriton |
| 271 | >Co-sponsored>GOOS>DBCP>UCSD-SCRIPPS | UCSD-SCRIPPS | 68 | 2 | NA | NA | dbcpUCSDScripps |
| 272 | >Co-sponsored>GOOS>DBCP>UK-JERSEY MB | UK-JERSEY MB | 68 | 2 | NA | NA | dbcpUKJerseyMB |
| 273 | >Co-sponsored>GOOS>DBCP>UK-MO DB | UK-MO DB | 68 | 2 | NA | NA | dbcpUKMODB |
| 274 | >Co-sponsored>GOOS>DBCP>UK-MO FIXED | UK-MO FIXED | 68 | 2 | NA | NA | dbcpUKMOFixed |
| 327 | >Co-sponsored>GOOS>DBCP>UK-MO LV | UK-MO LV | 68 | 2 | NA | NA | NA |
| 275 | >Co-sponsored>GOOS>DBCP>UK-MO MB | UK-MO MB | 68 | 2 | NA | NA | dbcpUKMOMB |
| 276 | >Co-sponsored>GOOS>DBCP>US-CG DB | US-CG DB | 68 | 2 | NA | NA | dbcpUSCGDB |
| 69 | >Co-sponsored>GOOS>SOT | SOT | 62 | 2 | NA | NA | sot |
| 14 | >Co-sponsored>GOOS>SOT>ASAP | ASAP | 69 | 2 | GOS Automated Shipboard Upper-Air Sounding Program | http://www.jcommops.org/sot/asap/ | asap |
| 160 | >Co-sponsored>GOOS>SOT>ASAP>ASAP-AWI | ASAP-AWI | 14 | 2 | NA | NA | asapAWI |
| 199 | >Co-sponsored>GOOS>SOT>ASAP>E-ASAP | E-ASAP | 14 | 2 | NA | NA | eAsap |
| 319 | >Co-sponsored>GOOS>SOT>ASAP-ZA | ASAP-ZA | 69 | 2 | NA | NA | NA |
| 304 | >Co-sponsored>GOOS>SOT>GO-SHIP | GO-SHIP | 69 | 2 | Global Ocean Ship-Based Hydrographic Investigations Programme | NA | goShip |
| 326 | >Co-sponsored>GOOS>SOT>SOOP-BOM | SOOP-BOM | 69 | 2 | NA | NA | NA |
| 305 | >Co-sponsored>GOOS>SOT>SOOT | SOOT | 69 | 2 | Ship Of Opportunity Programme | NA | soot |
| 19 | >Co-sponsored>GOOS>SOT>VOS | VOS | 69 | 2 | GOS Voluntary Observing Ship Program | https://www.wmo.int/pages/prog/amp/mmop/JCOMM/OPA/SOT/vos.html | vos |
| 277 | >Co-sponsored>GOOS>SOT>VOS>VOS-3P | VOS-3P | 19 | 2 | NA | NA | vos3P |
| 278 | >Co-sponsored>GOOS>SOT>VOS>VOS-AU | VOS-AU | 19 | 2 | NA | NA | vosAU |
| 279 | >Co-sponsored>GOOS>SOT>VOS>VOS-CA | VOS-CA | 19 | 2 | NA | NA | vosCA |
| 280 | >Co-sponsored>GOOS>SOT>VOS>VOS-CL | VOS-CL | 19 | 2 | NA | NA | vosCL |
| 281 | >Co-sponsored>GOOS>SOT>VOS>VOS-DE | VOS-DE | 19 | 2 | NA | NA | vosDE |
| 282 | >Co-sponsored>GOOS>SOT>VOS>VOS-ES | VOS-ES | 19 | 2 | NA | NA | vosES |
| 283 | >Co-sponsored>GOOS>SOT>VOS>VOS-EU | VOS-EU | 19 | 2 | NA | NA | vosEU |
| 284 | >Co-sponsored>GOOS>SOT>VOS>VOS-FR | VOS-FR | 19 | 2 | NA | NA | vosFR |
| 285 | >Co-sponsored>GOOS>SOT>VOS>VOS-GB | VOS-GB | 19 | 2 | NA | NA | vosGB |
| 286 | >Co-sponsored>GOOS>SOT>VOS>VOS-GR | VOS-GR | 19 | 2 | NA | NA | vosGR |
| 287 | >Co-sponsored>GOOS>SOT>VOS>VOS-HK | VOS-HK | 19 | 2 | NA | NA | vosHK |
| 288 | >Co-sponsored>GOOS>SOT>VOS>VOS-IE | VOS-IE | 19 | 2 | NA | NA | vosIE |
| 289 | >Co-sponsored>GOOS>SOT>VOS>VOS-IL | VOS-IL | 19 | 2 | NA | NA | vosIL |
| 290 | >Co-sponsored>GOOS>SOT>VOS>VOS-IN | VOS-IN | 19 | 2 | NA | NA | vosIN |
| 291 | >Co-sponsored>GOOS>SOT>VOS>VOS-IS | VOS-IS | 19 | 2 | NA | NA | vosIS |
| 292 | >Co-sponsored>GOOS>SOT>VOS>VOS-JP | VOS-JP | 19 | 2 | NA | NA | vosJP |
| 293 | >Co-sponsored>GOOS>SOT>VOS>VOS-KR | VOS-KR | 19 | 2 | NA | NA | vosKR |
| 294 | >Co-sponsored>GOOS>SOT>VOS>VOS-MY | VOS-MY | 19 | 2 | NA | NA | vosMY |
| 295 | >Co-sponsored>GOOS>SOT>VOS>VOS-NL | VOS-NL | 19 | 2 | NA | NA | vosNL |
| 296 | >Co-sponsored>GOOS>SOT>VOS>VOS-NO | VOS-NO | 19 | 2 | NA | NA | vosNO |
| 297 | >Co-sponsored>GOOS>SOT>VOS>VOS-NZ | VOS-NZ | 19 | 2 | NA | NA | vosNZ |
| 298 | >Co-sponsored>GOOS>SOT>VOS>VOS-RU | VOS-RU | 19 | 2 | NA | NA | vosRU |
| 299 | >Co-sponsored>GOOS>SOT>VOS>VOS-SE | VOS-SE | 19 | 2 | NA | NA | vosSE |
| 300 | >Co-sponsored>GOOS>SOT>VOS>VOS-SHIP-MASK | VOS-SHIP-MASK | 19 | 2 | NA | NA | vosShipMask |
| 301 | >Co-sponsored>GOOS>SOT>VOS>VOS-US | VOS-US | 19 | 2 | NA | NA | vosUS |
| 302 | >Co-sponsored>GOOS>SOT>VOS>VOS-ZA | VOS-ZA | 19 | 2 | NA | NA | vosZA |
| 328 | >Co-sponsored>GOOS>SOT>VOS-PL | VOS-PL | 69 | 2 | NA | NA | NA |
| 63 | >Co-sponsored>GTOS | GTOS | 17 | 2 | Global Terrestrial Observing System | www.fao.org/gtos/ | gtos |
| 303 | >Non-affiliated | Non-affiliated | NA | 2 | Observing elements that are not affiliated with WIGOS or any of the co-sponsored programs | NA | nonAffiliated |
| 15 | >WIGOS | WIGOS | NA | 2 | WMO Integrated Observing System components | www.wmo.int/wigos | NA |
| 1 | >WIGOS>GAW | GAW | 15 | 1 | Global Atmosphere Watch | www.wmo.int/gaw | gaw |
| 5 | >WIGOS>GAW>GAW Contributing networks | GAW Contributing networks | 1 | 1 | Networks contributing to GAW that have signed an agreement with GAW | NA | gawContributing |
| 38 | >WIGOS>GAW>GAW Contributing networks>ALINE | ALINE | 5 | 1 | ALINE Latin America Lidar Network | http://lalinet.org | aline |
| 34 | >WIGOS>GAW>GAW Contributing networks>CASTNET | CASTNET | 5 | 1 | Clean Air Status and Trends Network | http://epa.gov/castnet/javaweb/index.html | castnet |
| 36 | >WIGOS>GAW>GAW Contributing networks>GALION | GALION | 5 | 1 | GAW Aerosol Lidar Observation Network | http://alg.umbc.edu/galion/ | NA |
| 37 | >WIGOS>GAW>GAW Contributing networks>GALION>ADNet | ADNet | 36 | 1 | AD-Net Asian Dust and Lidar Observation Network | http://www-lidar.nies.go.jp/AD-Net/ | galionAdnet |
| 35 | >WIGOS>GAW>GAW Contributing networks>GALION>EARLINET | EARLINET | 36 | 1 | EARLINET European Lidar | http://www.earlinet.org/ | galionEarlinet |
| 41 | >WIGOS>GAW>GAW Contributing networks>GALION>MPLNET | MPLNET | 36 | 1 | Micro-Pulse Lidar Network | http://mplnet.gsfc.nasa.gov/ | galionMplnet |
| 40 | >WIGOS>GAW>GAW Contributing networks>GALION>NDACC | NDACC | 36 | 1 | Network for the Detection of Atmospheric Composition Change | http://www.ndsc.ncep.noaa.gov/ | galionNdacc |
| 8 | >WIGOS>GAW>GAW Contributing networks>GAW-PFR | GAW-PFR | 5 | 1 | GAW Precision Filter Radiometer Network | http://www.pmodwrc.ch/worcc/ | gawPFR |
| 316 | >WIGOS>GAW>GAW Contributing networks>IDAF | IDAF | 5 | 2 | IGAC/DEBITS Africa | NA | idaf |
| 28 | >WIGOS>GAW>GAW Contributing networks>IMPROVE | IMPROVE | 5 | 1 | IMPROVE Optical Aerosol | http://vista.cira.colostate.edu/improve/ | improve |
| 22 | >WIGOS>GAW>GAW Contributing networks>NADP | NADP | 5 | 1 | National Atmospheric Deposition Program | http://nadp.sws.uiuc.edu/ | nadp |
| 31 | >WIGOS>GAW>GAW Contributing networks>TCCON | TCCON | 5 | 1 | Total Carbon Column Observing Network | http://tccon.ipac.caltech.edu/ | tccon |
| 306 | >WIGOS>GAW>GAW Global | GAW Global | 1 | 1 | Global GAW station | NA | gawGlobal |
| 308 | >WIGOS>GAW>GAW Local | GAW Local | 1 | 1 | Local GAW station | NA | gawLocal |
| 315 | >WIGOS>GAW>GAW Other elements | GAW Other elements | 1 | 1 | Other international/national networks that provide atmospheric composition observations but are not (yet) affiliated with GAW | NA | gawOther |
| 23 | >WIGOS>GAW>GAW Other elements>AGAGE | AGAGE | 315 | 1 | AGAGE/SOGE/NIES | http://agage.eas.gatech.edu/ | agage |
| 7 | >WIGOS>GAW>GAW Other elements>AOD | AOD | 315 | 1 | Aerosol Optical Depth | NA | globalAOD |
| 43 | >WIGOS>GAW>GAW Other elements>AOD>AEROCAN | AEROCAN | 7 | 1 | AEROCAN, federated to AERONET | http://www.aerocanonline.com/ | aerocan |
| 44 | >WIGOS>GAW>GAW Other elements>AOD>AERONET | AERONET | 7 | 1 | AERONET | http://aeronet.gsfc.nasa.gov/new_web/index.html | aeronet |
| 47 | >WIGOS>GAW>GAW Other elements>AOD>German AOD network | German AOD network | 7 | 1 | German AOD network | NA | germanAOD |
| 45 | >WIGOS>GAW>GAW Other elements>AOD>PHOTONS | PHOTONS | 7 | 1 | PHOTONS, federated to AERONET | http://loaphotons.univ-lille1.fr/ | photons |
| 46 | >WIGOS>GAW>GAW Other elements>AOD>PolarAOD | PolarAOD | 7 | 1 | PolarAOD | http://polaris.nipr.ac.jp/~ipy/usr/sympo/proc-files/76-Vitale.pdf | polarAOD |
| 48 | >WIGOS>GAW>GAW Other elements>AOD>SibRad | SibRad | 7 | 1 | SibRad | NA | sibrad |
| 49 | >WIGOS>GAW>GAW Other elements>AOD>SKYNET | SKYNET | 7 | 1 | SKYNET | http://atmos.cr.chiba-u.ac.jp/ | skynet |
| 24 | >WIGOS>GAW>GAW Other elements>CAPMoN | CAPMoN | 315 | 1 | Canadian Air and Precipitation Monitoring Network | http://www.msc.ec.gc.ca/capmon/index_e.cfm | capmon |
| 39 | >WIGOS>GAW>GAW Contributing networks>GALION>CIS-LiNet | CIS-LiNet | 36 | 1 | Atmosphere aerosol and ozone monitoring in CIS regions through lidar stations network | http://www.cis-linet.basnet.by/ | cislinet |
| 42 | >WIGOS>GAW>GAW Other elements>CLN | CLN | 315 | 1 | The NOAA Cooperative Remote Sencing Science and Technology (CREST) Lidar network | http://crest.ccny.cuny.edu/ | cln |
| 25 | >WIGOS>GAW>GAW Other elements>EANET | EANET | 315 | 1 | EANET | http://www.eanet.cc/ | eanet |
| 21 | >WIGOS>GAW>GAW Other elements>EMEP | EMEP | 315 | 1 | European Monitoring and Evaluation Programme | http://www.emep.int/ | emep |
| 26 | >WIGOS>GAW>GAW Other elements>ESRLCCG | ESRLCCG | 315 | 1 | NOAA-ESRL (CCG) | http://www.esrl.noaa.gov/gmd/ccgg/iadv/ | esrlccg |
| 27 | >WIGOS>GAW>GAW Other elements>IAGOS | IAGOS | 315 | 1 | MOZAIC-IAGOS | http://www.iagos.fr/web/ | iagos |
| 33 | >WIGOS>GAW>GAW Other elements>NDACC | NDACC | 315 | 1 | Network for the Detection of Atmospheric Composition Change | http://www.ndsc.ncep.noaa.gov/ | ndacc |
| 317 | >WIGOS>GAW>GAW Other elements>Non-affiliated | Non-affiliated | 315 | 1 | Observing elements that are not affiliated with GAW or any of the co-sponsored programs | NA | NA |
| 30 | >WIGOS>GAW>GAW Other elements>SHADOZ | SHADOZ | 315 | 1 | Southern Hemisphere ADditional Ozonesondes | http://croc.gsfc.nasa.gov/shadoz/ | shadoz |
| 307 | >WIGOS>GAW>GAW Regional | GAW Regional | 1 | 1 | Regional GAW station | NA | gawRegional |
| 3 | >WIGOS>GCW | GCW | 15 | 2 | Global Cryosphere Watch | http://globalcryospherewatch.org/ | gcw |
| 66 | >WIGOS>GCW>CryoNet | CryoNet | 3 | 2 | NA | NA | cryonet |
| 4 | >WIGOS>GOS | GOS | 15 | 2 | Global Observing System | https://www.wmo.int/pages/prog/www/OSY/GOS.html | gos |
| 310 | >WIGOS>GOS>GOS Contributing networks | GOS Contributing networks | 4 | 1 | Networks contributing to the Global Observing System (GOS) | NA | gosContributing |
| 55 | >WIGOS>GOS>GOS Contributing networks>CTBTO | CTBTO | 310 | 2 | Comprehensive Nuclear-Test Ban Treaty Organization International Monitoring System Network monitoring station | NA | ctbto |
| 311 | >WIGOS>GOS>GOS Other elements | GOS Other elements | 4 | 1 | Other elements of the Global Observing System (GOS) | NA | gosOther |
| 392 | >WIGOS>GOS>GOS Other elements>ADD | ADD | 311 | 2 | Aircraft Derived Data (Mode-S/ADS-B) | NA | add |
| 391 | >WIGOS>GOS>GOS Other elements>ADS-C | ADS-C | 311 | 2 | Automatic Data Surveillance (ICAO). These are transmitted at selected points of the flightpath. | NA | adsc |
| 394 | >WIGOS>GOS>GOS Other elements>AFIRS | AFIRS | 311 | 2 | Automated Flight Information & Reporting System (Commercial/FLYHT). Not currently open to GTS, contract with Panasonic and airline and/or NMHS | NA | afirs |
| 390 | >WIGOS>GOS>GOS Other elements>AIREP | AIREP | 311 | 2 | Aircraft Report (Manual). These are transmitted by the aircraft when a weather event encountered e.g. severe turbulence or icing. | NA | airep |
| 12 | >WIGOS>GOS>GOS Other elements>AMDAR | AMDAR | 311 | 2 | Aircraft Meteorological Data Relay | https://www.wmo.int/pages/prog/www/GOS/ABO/AMDAR/ | amdar |
| 64 | >WIGOS>GOS>GOS Other elements>Ceilometers | Ceilometers | 311 | 2 | Elastic lidar (ceilometer) networks | www.wmo.int/hwrp/ | ceilometer |
| 314 | >WIGOS>GOS>GOS Other elements>GNSS radio-occultation system | GNSS radio-occultation system | 311 | 2 | NA | NA | gnssRadioOccultation |
| 313 | >WIGOS>GOS>GOS Other elements>Lightning detection system | Lightning detection system | 311 | 2 | Surface-based lightning detection system | NA | lightningDetection |
| 393 | >WIGOS>GOS>GOS Other elements>TAMDAR | TAMDAR | 311 | 2 | Tropospheric Aircraft Meteorological Data Relay (Commercial/Panasonic). Not currently open to GTS, contract with Panasonic and airline and/or NMHS | NA | tamdar |
| 312 | >WIGOS>GOS>GOS Other elements>Wind Profilers | Wind Profilers | 311 | 2 | Wind profilers for upper-air wind observations | NA | windProfiler |
| 20 | >WIGOS>GOS>GOS Other elements>WRO | WRO | 311 | 2 | GOS Weather Radar | https://www.wmo.int/pages/prog/www/WRO/index_en.html | wro |
| 309 | >WIGOS>GOS>GOS Surface networks | GOS Surface networks | 4 | 2 | Surface-based observing networks of the Global Observing System (GOS) | NA | gosSurface |
| 50 | >WIGOS>GOS>GOS Surface networks>ANTON | ANTON | 309 | 2 | Antarctic Observing Network surface synoptic station (SYNOP/CLIMAT) | NA | anton |
| 51 | >WIGOS>GOS>GOS Surface networks>ANTON>ANTON(T) | ANTON(T) | 50 | 2 | Antarctic Observing Network upper-air station (TEMP) | NA | antonT |
| 32 | >WIGOS>GOS>GOS Surface networks>BSRN | BSRN | 309 | 1 | Baseline Surface Radiation Network | http://www.bsrn.awi.de/ | bsrn |
| 56 | >WIGOS>GOS>GOS Surface networks>RBCN | RBCN | 309 | 2 | Regional Basic Climatological Networks | NA | rbcn |
| 52 | >WIGOS>GOS>GOS Surface networks>RBCN>CLIMAT(C) | CLIMAT(C) | 56 | 2 | Station for which monthly climatological means of surface elements are transmitted | NA | rbcnClimatC |
| 54 | >WIGOS>GOS>GOS Surface networks>RBCN>CLIMAT(CT) | CLIMAT(CT) | 56 | 2 | Station for which monthly climatological means of both surface and upper- air elements are transmitted | NA | rbcnClimatCT |
| 53 | >WIGOS>GOS>GOS Surface networks>RBCN>CLIMAT(T) | CLIMAT(T) | 56 | 2 | Station for which monthly climatological means of upper-air elements are transmitted | NA | rbcnClimatT |
| 16 | >WIGOS>GOS>GOS Surface networks>RBSN | RBSN | 309 | 2 | GOS Regional Basic Synoptic Networks | NA | rbsn |
| 59 | >WIGOS>GOS>GOS Surface networks>RBSN>RBSN(P) | RBSN(P) | 16 | 2 | Regional Basic Synoptic Network upper-air station (PILOT) | NA | rbsnP |
| 57 | >WIGOS>GOS>GOS Surface networks>RBSN>RBSN(S) | RBSN(S) | 16 | 2 | Regional Basic Synoptic Network surface station (SYNOP) | NA | rbsnS |
| 61 | >WIGOS>GOS>GOS Surface networks>RBSN>RBSN(SP) | RBSN(SP) | 16 | 2 | NA | NA | rbsnSP |
| 60 | >WIGOS>GOS>GOS Surface networks>RBSN>RBSN(ST) | RBSN(ST) | 16 | 2 | Regional Basic Synoptic Network surface and upper-air station (SYNOP/TEMP) | NA | rbsnST |
| 58 | >WIGOS>GOS>GOS Surface networks>RBSN>RBSN(T) | RBSN(T) | 16 | 2 | Regional Basic Synoptic Network upper-air station (TEMP) | NA | rbsnT |
| 329 | >WIGOS>GOS>OceanSITES | OceanSITES | 4 | 2 | NA | NA | NA |
| 331 | >WIGOS>GOS>OceanSITES>OS-AWI | OS-AWI | 329 | 2 | NA | NA | NA |
| 332 | >WIGOS>GOS>OceanSITES>OS-BAS | OS-BAS | 329 | 2 | NA | NA | NA |
| 333 | >WIGOS>GOS>OceanSITES>OS-BCCR | OS-BCCR | 329 | 2 | NA | NA | NA |
| 334 | >WIGOS>GOS>OceanSITES>OS-BIOS | OS-BIOS | 329 | 2 | NA | NA | NA |
| 335 | >WIGOS>GOS>OceanSITES>OS-CENTRE D'OCEANOLOGIE DE MARSEILLE | OS-CENTRE D'OCEANOLOGIE DE MARSEILLE | 329 | 2 | NA | NA | NA |
| 336 | >WIGOS>GOS>OceanSITES>OS-CNR | OS-CNR | 329 | 2 | NA | NA | NA |
| 337 | >WIGOS>GOS>OceanSITES>OS-CNRS | OS-CNRS | 329 | 2 | NA | NA | NA |
| 338 | >WIGOS>GOS>OceanSITES>OS-CSIRO | OS-CSIRO | 329 | 2 | NA | NA | NA |
| 339 | >WIGOS>GOS>OceanSITES>OS-DFO | OS-DFO | 329 | 2 | NA | NA | NA |
| 340 | >WIGOS>GOS>OceanSITES>OS-DML | OS-DML | 329 | 2 | NA | NA | NA |
| 341 | >WIGOS>GOS>OceanSITES>OS-EU-THOR | OS-EU-THOR | 329 | 2 | NA | NA | NA |
| 342 | >WIGOS>GOS>OceanSITES>OS-GEOMAR | OS-GEOMAR | 329 | 2 | NA | NA | NA |
| 343 | >WIGOS>GOS>OceanSITES>OS-HAMBURG UNIVERSITY | OS-HAMBURG UNIVERSITY | 329 | 2 | NA | NA | NA |
| 344 | >WIGOS>GOS>OceanSITES>OS-HAV | OS-HAV | 329 | 2 | NA | NA | NA |
| 345 | >WIGOS>GOS>OceanSITES>OS-HELLENIC CENTRE FOR MARINE RESEARCH | OS-HELLENIC CENTRE FOR MARINE RESEARCH | 329 | 2 | NA | NA | NA |
| 346 | >WIGOS>GOS>OceanSITES>OS-ICCM - ACIISI - GOBIERNO DE CANARIAS | OS-ICCM - ACIISI - GOBIERNO DE CANARIAS | 329 | 2 | NA | NA | NA |
| 347 | >WIGOS>GOS>OceanSITES>OS-IFREMER | OS-IFREMER | 329 | 2 | NA | NA | NA |
| 348 | >WIGOS>GOS>OceanSITES>OS-IMARPE | OS-IMARPE | 329 | 2 | NA | NA | NA |
| 349 | >WIGOS>GOS>OceanSITES>OS-INSTITUTE OF MARINE RESEARCH | OS-INSTITUTE OF MARINE RESEARCH | 329 | 2 | NA | NA | NA |
| 350 | >WIGOS>GOS>OceanSITES>OS-IO-NTU | OS-IO-NTU | 329 | 2 | NA | NA | NA |
| 351 | >WIGOS>GOS>OceanSITES>OS-IO-WARNEMUENDE | OS-IO-WARNEMUENDE | 329 | 2 | NA | NA | NA |
| 352 | >WIGOS>GOS>OceanSITES>OS-IRD | OS-IRD | 329 | 2 | NA | NA | NA |
| 353 | >WIGOS>GOS>OceanSITES>OS-JAMSTEC | OS-JAMSTEC | 329 | 2 | NA | NA | NA |
| 354 | >WIGOS>GOS>OceanSITES>OS-KIOST | OS-KIOST | 329 | 2 | NA | NA | NA |
| 355 | >WIGOS>GOS>OceanSITES>OS-LDEO | OS-LDEO | 329 | 2 | NA | NA | NA |
| 356 | >WIGOS>GOS>OceanSITES>OS-MARUM | OS-MARUM | 329 | 2 | NA | NA | NA |
| 357 | >WIGOS>GOS>OceanSITES>OS-MBARI | OS-MBARI | 329 | 2 | NA | NA | NA |
| 358 | >WIGOS>GOS>OceanSITES>OS-NIO | OS-NIO | 329 | 2 | NA | NA | NA |
| 359 | >WIGOS>GOS>OceanSITES>OS-NIOT | OS-NIOT | 329 | 2 | NA | NA | NA |
| 360 | >WIGOS>GOS>OceanSITES>OS-NIWA | OS-NIWA | 329 | 2 | NA | NA | NA |
| 361 | >WIGOS>GOS>OceanSITES>OS-NOAA/AOML | OS-NOAA/AOML | 329 | 2 | NA | NA | NA |
| 362 | >WIGOS>GOS>OceanSITES>OS-NOAA/PMEL | OS-NOAA/PMEL | 329 | 2 | NA | NA | NA |
| 363 | >WIGOS>GOS>OceanSITES>OS-NOC | OS-NOC | 329 | 2 | NA | NA | NA |
| 364 | >WIGOS>GOS>OceanSITES>OS-OGS | OS-OGS | 329 | 2 | NA | NA | NA |
| 365 | >WIGOS>GOS>OceanSITES>OS-ROYAL NETHERLANDS INSTITUTE FOR SEA RESEARCH (NIOZ) | OS-ROYAL NETHERLANDS INSTITUTE FOR SEA RESEARCH (NIOZ) | 329 | 2 | NA | NA | NA |
| 366 | >WIGOS>GOS>OceanSITES>OS-SIO | OS-SIO | 329 | 2 | NA | NA | NA |
| 367 | >WIGOS>GOS>OceanSITES>OS-SNU | OS-SNU | 329 | 2 | NA | NA | NA |
| 368 | >WIGOS>GOS>OceanSITES>OS-SOA | OS-SOA | 329 | 2 | NA | NA | NA |
| 369 | >WIGOS>GOS>OceanSITES>OS-UN_HAWAII_MANOA | OS-UN_HAWAII_MANOA | 329 | 2 | NA | NA | NA |
| 370 | >WIGOS>GOS>OceanSITES>OS-UN_MIAMI | OS-UN_MIAMI | 329 | 2 | NA | NA | NA |
| 371 | >WIGOS>GOS>OceanSITES>OS-USP | OS-USP | 329 | 2 | NA | NA | NA |
| 372 | >WIGOS>GOS>OceanSITES>OS-UW | OS-UW | 329 | 2 | NA | NA | NA |
| 373 | >WIGOS>GOS>OceanSITES>OS-WHOI | OS-WHOI | 329 | 2 | NA | NA | NA |
| 374 | >WIGOS>GOS>OceanSITES>OS-WHOI/SIO/OSU | OS-WHOI/SIO/OSU | 329 | 2 | NA | NA | NA |
| 65 | >WIGOS>WHOS | WHOS | 15 | 2 | NA | NA | whos |
| 67 | >WIGOS>WHOS>WHYCOS | WHYCOS | 65 | 2 | NA | www.whycos.org/ | whycos |

### Code table: 3-01

Code table title: Region of origin of data

| # | WMO_RA_CD | Name | Name_FR | WMO306_CD |
| --- | --- | --- | --- | --- |
| 1 | I | Africa | Afrique | africa |
| 2 | II | Asia | Asie | asia |
| 3 | III | South America | Amérique du Sud | southAmerica |
| 4 | IV | North America, Central America and the Caribbean | Amérique du Nord, Amérique Centrale et Caraïbes | northCentralAmericaCaribbean |
| 5 | V | South-West Pacific | Pacifique Sud-Ouest | southWestPacific |
| 6 | VI | Europe | Europe | europe |
| 7 | NA | Antarctica | Antarctique | antarctica |
| 8 | NA | (unknown) | (unknown) | unknown |

### Code table: 3-02

Code table title: Territory of origin of data

| # | Name | ISO2_COUNTRY_CD | ISO3_COUNTRY_CD | ISO_COUNTRY_NU | WMO306_CD |
| --- | --- | --- | --- | --- | --- |
| 1 | Afghanistan | AF | AFG | 4 | NA |
| 2 | Albania | AL | ALB | 8 | NA |
| 3 | Algeria | DZ | DZA | 12 | NA |
| 4 | Angola | AO | AGO | 24 | NA |
| 5 | Anguilla | AI | AIA | 660 | NA |
| 6 | Antarctica | AQ | ATA | 10 | NA |
| 7 | Antigua and Barbuda | AG | ATG | 28 | NA |
| 8 | Argentina | AR | ARG | 32 | NA |
| 9 | Armenia | AM | ARM | 51 | NA |
| 10 | Australia | AU | AUS | 36 | NA |
| 11 | Austria | AT | AUT | 40 | NA |
| 12 | Azerbaijan | AZ | AZE | 31 | NA |
| 13 | Bahamas | BS | BHS | 44 | NA |
| 14 | Bahrain | BH | BHR | 48 | NA |
| 15 | Bangladesh | BD | BGD | 50 | NA |
| 16 | Barbados | BB | BRB | 52 | NA |
| 17 | Belarus | BY | BLR | 112 | NA |
| 18 | Belgium | BE | BEL | 56 | NA |
| 19 | Belize | BZ | BLZ | 84 | NA |
| 20 | Benin | BJ | BEN | 204 | NA |
| 21 | Bermuda | BM | BMU | 60 | NA |
| 22 | Bhutan | BT | BTN | 64 | NA |
| 23 | Bolivia, Plurinational State of | BO | BOL | 68 | NA |
| 24 | Bosnia and Herzegovina | BA | BIH | 70 | NA |
| 25 | Botswana | BW | BWA | 72 | NA |
| 26 | Brazil | BR | BRA | 76 | NA |
| 27 | British Caribbean Territories | BC | BCT | NA | NA |
| 28 | British Virgin Islands | VG | VGB | 92 | NA |
| 29 | Brunei Darussalam | BN | BRN | 96 | NA |
| 30 | Bulgaria | BG | BGR | 100 | NA |
| 31 | Burkina Faso | BF | BFA | 854 | NA |
| 32 | Burundi | BI | BDI | 108 | NA |
| 33 | Cabo Verde | CV | CPV | 132 | NA |
| 34 | Cambodia | KH | KHM | 116 | NA |
| 35 | Cameroon | CM | CMR | 120 | NA |
| 36 | Canada | CA | CAN | 124 | NA |
| 37 | Cayman Islands | KY | CYM | 136 | NA |
| 38 | Central African Republic | CF | CAF | 140 | NA |
| 39 | Chad | TD | TCD | 148 | NA |
| 40 | Chile | CL | CHL | 152 | NA |
| 41 | China | CN | CHN | 156 | NA |
| 42 | Christmas Island | CX | CXR | 162 | NA |
| 43 | Cocos (Keeling) Islands | CC | CCK | 166 | NA |
| 44 | Colombia | CO | COL | 170 | NA |
| 45 | Comoros | KM | COM | 174 | NA |
| 46 | Congo | CG | COG | 178 | NA |
| 47 | Congo (Democratic Republic of the) | CD | COD | 180 | NA |
| 48 | Cook Islands | CK | COK | 184 | NA |
| 49 | Costa Rica | CR | CRI | 188 | NA |
| 50 | CÃ´te d'Ivoire | CI | CIV | 384 | NA |
| 51 | Croatia | HR | HRV | 191 | NA |
| 52 | Cuba | CU | CUB | 192 | NA |
| 53 | Curacao and Sint Maarten | CW | CUW | 531 | NA |
| 54 | Cyprus | CY | CYP | 196 | NA |
| 55 | Czech Republic | CZ | CZE | 203 | NA |
| 56 | Denmark | DK | DNK | 208 | NA |
| 57 | Djibouti | DJ | DJI | 262 | NA |
| 58 | Dominica | DM | DMA | 212 | NA |
| 59 | Dominican Republic | DO | DOM | 214 | NA |
| 60 | Ecuador | EC | ECU | 218 | NA |
| 61 | Egypt | EG | EGY | 818 | NA |
| 62 | El Salvador | SV | SLV | 222 | NA |
| 63 | Equatorial Guinea | GQ | GNQ | 226 | NA |
| 64 | Eritrea | ER | ERI | 232 | NA |
| 65 | Estonia | EE | EST | 233 | NA |
| 66 | Ethiopia | ET | ETH | 231 | NA |
| 67 | Falkland Islands (Malvinas) | FK | FLK | 238 | NA |
| 68 | Fiji | FJ | FJI | 242 | NA |
| 69 | Finland | FI | FIN | 246 | NA |
| 70 | France | FR | FRA | 250 | NA |
| 71 | French Polynesia | PF | PYF | 258 | NA |
| 72 | Gabon | GA | GAB | 266 | NA |
| 73 | Gambia | GM | GMB | 270 | NA |
| 74 | Georgia | GE | GEO | 268 | NA |
| 75 | Germany | DE | DEU | 276 | NA |
| 76 | Ghana | GH | GHA | 288 | NA |
| 77 | Gibraltar | GI | GIB | 292 | NA |
| 78 | Greece | GR | GRC | 300 | NA |
| 79 | Greenland | GL | GRL | 304 | NA |
| 80 | Grenada | GD | GRD | 308 | NA |
| 81 | Guatemala | GT | GTM | 320 | NA |
| 82 | Guinea | GN | GIN | 324 | NA |
| 83 | Guinea-Bissau | GW | GNB | 624 | NA |
| 84 | Guyana | GY | GUY | 328 | NA |
| 85 | Haiti | HT | HTI | 332 | NA |
| 86 | Honduras | HN | HND | 340 | NA |
| 87 | Hong Kong, China | HK | HKG | 344 | NA |
| 88 | Hungary | HU | HUN | 348 | NA |
| 89 | Iceland | IS | ISL | 352 | NA |
| 90 | India | IN | IND | 356 | NA |
| 91 | Indonesia | ID | IDN | 360 | NA |
| 92 | Iran (Islamic Republic of) | IR | IRN | 364 | NA |
| 93 | Iraq | IQ | IRQ | 368 | NA |
| 94 | Ireland | IE | IRL | 372 | NA |
| 95 | Israel | IL | ISR | 376 | NA |
| 96 | Italy | IT | ITA | 380 | NA |
| 97 | Jamaica | JM | JAM | 388 | NA |
| 98 | Japan | JP | JPN | 392 | NA |
| 99 | Jordan | JO | JOR | 400 | NA |
| 100 | Kazakhstan | KZ | KAZ | 398 | NA |
| 101 | Kenya | KE | KEN | 404 | NA |
| 102 | Kiribati | KI | KIR | 296 | NA |
| 103 | Korea (Democratic People's Republic of) | KP | PRK | 408 | NA |
| 104 | Korea (the Republic of) | KR | KOR | 410 | NA |
| 105 | Kuwait | KW | KWT | 414 | NA |
| 106 | Kyrgyzstan | KG | KGZ | 417 | NA |
| 107 | Lao People's Democratic Republic | LA | LAO | 418 | NA |
| 108 | Latvia | LV | LVA | 428 | NA |
| 109 | Lebanon | LB | LBN | 422 | NA |
| 110 | Lesotho | LS | LSO | 426 | NA |
| 111 | Liberia | LR | LBR | 430 | NA |
| 112 | Libya | LY | LBY | 434 | NA |
| 113 | Liechtenstein | LI | LIE | 438 | NA |
| 114 | Lithuania | LT | LTU | 440 | NA |
| 115 | Luxembourg | LU | LUX | 442 | NA |
| 116 | Macao, China | MO | MAC | 446 | NA |
| 117 | Macedonia (the former Yugoslav Republic of) | MK | MKD | 807 | NA |
| 118 | Madagascar | MG | MDG | 450 | NA |
| 119 | Malawi | MW | MWI | 454 | NA |
| 120 | Malaysia | MY | MYS | 458 | NA |
| 121 | Maldives | MV | MDV | 462 | NA |
| 122 | Mali | ML | MLI | 466 | NA |
| 123 | Malta | MT | MLT | 470 | NA |
| 124 | Marshall Islands | MH | MHL | 584 | NA |
| 125 | Mauritania | MR | MRT | 478 | NA |
| 126 | Mauritius | MU | MUS | 480 | NA |
| 127 | Mexico | MX | MEX | 484 | NA |
| 128 | Micronesia, Federated States of | FM | FSM | 583 | NA |
| 129 | Moldova (the Republic of) | MD | MDA | 498 | NA |
| 130 | Monaco | MC | MCO | 492 | NA |
| 131 | Mongolia | MN | MNG | 496 | NA |
| 132 | Montenegro | ME | MNE | 499 | NA |
| 133 | Montserrat | MS | MSR | 500 | NA |
| 134 | Morocco | MA | MAR | 504 | NA |
| 135 | Mozambique | MZ | MOZ | 508 | NA |
| 136 | Myanmar | MM | MMR | 104 | NA |
| 137 | Namibia | NA | NAM | 516 | NA |
| 138 | Nauru | NR | NRU | 520 | NA |
| 139 | Nepal | NP | NPL | 524 | NA |
| 140 | Netherlands | NL | NLD | 528 | NA |
| 141 | New Caledonia | NC | NCL | 540 | NA |
| 142 | New Zealand | NZ | NZL | 554 | NA |
| 143 | Nicaragua | NI | NIC | 558 | NA |
| 144 | Niger | NE | NER | 562 | NA |
| 145 | Nigeria | NG | NGA | 566 | NA |
| 146 | Niue | NU | NIU | 570 | NA |
| 147 | Norway | NO | NOR | 578 | NA |
| 148 | Oman | OM | OMN | 512 | NA |
| 149 | Pakistan | PK | PAK | 586 | NA |
| 150 | Palau | PW | PLW | 585 | NA |
| 151 | Palestine, State of | PS | PSE | 275 | NA |
| 152 | Panama | PA | PAN | 591 | NA |
| 153 | Papua New Guinea | PG | PNG | 598 | NA |
| 154 | Paraguay | PY | PRY | 600 | NA |
| 155 | Peru | PE | PER | 604 | NA |
| 156 | Philippines | PH | PHL | 608 | NA |
| 157 | Pitcairn | PN | PCN | 612 | NA |
| 158 | Poland | PL | POL | 616 | NA |
| 159 | Portugal | PT | PRT | 620 | NA |
| 160 | Puerto Rico | PR | PRI | 630 | NA |
| 161 | Qatar | QA | QAT | 634 | NA |
| 162 | Romania | RO | ROM | 642 | NA |
| 163 | Russian Federation | RU | RUS | 643 | NA |
| 164 | Rwanda | RW | RWA | 646 | NA |
| 165 | Saint Helena | SH | SHN | 654 | NA |
| 166 | Saint Kitts and Nevis | KN | KNA | 659 | NA |
| 167 | Saint Lucia | LC | LCA | 662 | NA |
| 168 | Saint Pierre and Miquelon | PM | SPM | 666 | NA |
| 169 | Saint Vincent and Grenadines | VC | VCT | 670 | NA |
| 170 | Samoa | WS | WSM | 882 | NA |
| 171 | Sao Tome and Principe | ST | STP | 678 | NA |
| 172 | Saudi Arabia | SA | SAU | 682 | NA |
| 173 | Senegal | SN | SEN | 686 | NA |
| 174 | Serbia | RS | SRB | 688 | NA |
| 175 | Seychelles | SC | SYC | 690 | NA |
| 176 | Sierra Leone | SL | SLE | 694 | NA |
| 177 | Singapore | SG | SGP | 702 | NA |
| 178 | Slovakia | SK | SVK | 703 | NA |
| 179 | Slovenia | SI | SVN | 705 | NA |
| 180 | Solomon Islands | SB | SLB | 90 | NA |
| 181 | Somalia | SO | SOM | 706 | NA |
| 182 | South Africa | ZA | ZAF | 710 | NA |
| 183 | South Georgia and the South Sandwich Islands | GS | SGS | 239 | NA |
| 184 | South Sudan | SS | SSD | 728 | NA |
| 185 | Spain | ES | ESP | 724 | NA |
| 186 | Sri Lanka | LK | LKA | 144 | NA |
| 187 | Sudan | SD | SDN | 729 | NA |
| 188 | Suriname | SR | SUR | 740 | NA |
| 189 | Swaziland | SZ | SWZ | 748 | NA |
| 190 | Sweden | SE | SWE | 752 | NA |
| 191 | Switzerland | CH | CHE | 756 | NA |
| 192 | Syrian Arab Republic | SY | SYR | 760 | NA |
| 193 | (unknown) | NA | NA | NA | NA |
| 194 | Taiwan, Province of China | TW | TWN | NA | NA |
| 195 | Tajikistan | TJ | TJK | 762 | NA |
| 196 | Tanzania, United Republic of | TZ | TZA | 834 | NA |
| 197 | Thailand | TH | THA | 764 | NA |
| 198 | Timor-Leste | TL | TLS | 626 | NA |
| 199 | Togo | TG | TGO | 768 | NA |
| 200 | Tokelau | TK | TKL | 772 | NA |
| 201 | Tonga | TO | TON | 776 | NA |
| 202 | Trinidad and Tobago | TT | TTO | 780 | NA |
| 203 | Tunisia | TN | TUN | 788 | NA |
| 204 | Turkey | TR | TUR | 792 | NA |
| 205 | Turkmenistan | TM | TKM | 795 | NA |
| 206 | Turks and Caicos Islands | TC | TCA | 796 | NA |
| 207 | Tuvalu | TV | TUV | 798 | NA |
| 208 | Uganda | UG | UGA | 800 | NA |
| 209 | Ukraine | UA | UKR | 804 | NA |
| 210 | United Arab Emirates (the) | AE | ARE | 784 | NA |
| 211 | United Kingdom (the) | GB | GBR | 826 | NA |
| 212 | United States (the) | US | USA | 840 | NA |
| 213 | Uruguay | UY | URY | 858 | NA |
| 214 | Uzbekistan | UZ | UZB | 860 | NA |
| 215 | Vanuatu | VU | VUT | 548 | NA |
| 216 | Venezuela, Bolivarian Republic of | VE | VEN | 862 | NA |
| 217 | Viet Nam | VN | VNM | 704 | NA |
| 218 | Western Sahara | EH | ESH | 732 | NA |
| 219 | Yemen | YE | YEM | 887 | NA |
| 220 | Zambia | ZM | ZMB | 894 | NA |
| 221 | Zimbabwe | ZW | ZWE | 716 | NA |

### Code table: 3-04

Code table title: Station/platform type (simplified) [WMO, 2013a]

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Land (fixed) | Station/platform on solid terrain, at fixed position | landFixed |
| 2 | Land (mobile) | Station/platform on solid terrain, moving around | landMobile |
| 3 | Sea (fixed) | Station/platform at sea surface, at fixed position | seaFixed |
| 4 | Sea (mobile) | Station/platform at sea surface, moving around | seaMobile |
| 5 | Air (fixed) | Airborne station/platform, at fixed position | airFixed |
| 6 | Air (mobile) | Airborne station/platform, moving around | airMobile |
| 7 | Underwater (fixed) | Station/platform under water, at fixed horizontal position | underwaterFixed |
| 8 | Underwater (mobile) | Station/platform under water, moving around | underwaterMobile |
| 9 | Land (on ice) | Station/platform on ice-covered ground, moving with the ice | landOnIce |
| 10 | Sea (on ice) | Station/platform on floating ice, moving with the ice | seaOnIce |
| 11 | Lake/River (fixed) | Station/platform at lake/river surface, at fixed position | lakeRiverFixed |
| 12 | Lake/River (mobile) | station/platform at lake/river surface, moving around | lakeRiverMobile |
| 13 | Space-based | Satellite platform in orbit | spaceBased |

### Code table: 3-08

Code table title: Data communication method

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Voice/landline | Voice communications using a fixed terrestrial telecommunications network | voiceLandline |
| 2 | Voice/cellular | Voice communications using a cellular or similar terrestrial telecommunications network | voiceCellular |
| 3 | Voice/radio | Voice communications using a direct radio system (such as VHF, HF SSB) | voiceRadio |
| 4 | Voice/satellite | Voice communications using a satellite telecommunications network | voiceSatellite |
| 5 | Data/landline | Data communications (digital or modem) using a fixed terrestrial telecommunications network (copper or fibre) | dataLandline |
| 6 | Data/cellular | Data communications (digital or modem) using a cellular or similar terrestrial telecommunications network | dataCellular |
| 7 | Data/radio | Data communications (digital or modem) using a direct radio system (such as VHF, HF SSB, microwave) | dataRadio |
| 8 | Data/satellite/geostationary | Data communications (digital or modem) using a geostationary satellite service (such as METEOSAT) | dataSatGeostationary |
| 9 | Data/satellite/constellation | Data communications (digital or modem) using a satellite constellation service (such as IRIDIUM) | dataSatConstellation |
| 10 | Data/satellite/intermittent | Data communications (digital or modem) using a satellite service with intermittent cover (such as ARGOS) | dataSatIntermittent |
| 11 | Fax/landline | Facsimile using fixed terrestrial telecommunications network | faxLandline |
| 12 | Fax/cellular | Facsimile using a cellular or similar terrestrial telecommunications network | faxCellular |
| 13 | Fax/radio | Facsimile using a direct radio system (such as VHF, HF SSB) | faxRadio |
| 14 | Post | Physical transfer of information by postal, delivery service, courier service or similar | post |

### Code table: 3-09

Code table title: Station operating status

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Operational | The station fully complies with the reporting obligations of the observation programme/network concerned | operational |
| 2 | Partly operational | The station partially complies with the reporting obligations of the observation programme/network concerned | partlyOperational |
| 3 | Non-reporting | The station is considered non-operational/non-reporting temporarily, but is expected to resume its operations/reporting after the temporary suspension | nonReporting |
| 4 | Closed | The station has been declared as closed by the responsible supervising organization | closed |
| 5 | Planned | The station is planned to be deployed sometime in the future, and all information | planned |
| 6 | Pre-operational | The station is deployed and producing data but still not fully ready to start reporting operationally | preOperational |
| 7 | Stand-by | NA | standBy |

### Code table: 4-01-01

Code table title: Land cover types (IGBP) (Cf. https://lpdaac.usgs.gov/products/modis_products_table/mcd12q1)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-01-01 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) | notApplicable |
| 4-01-01-02 | Water | NA | water |
| 4-01-01-03 | Evergreen Needleleaf forest | NA | evergreenNeedleleafForest |
| 4-01-01-04 | Evergreen Broadleaf forest | NA | evergreenBroadleafForest |
| 4-01-01-05 | Deciduous Needleleaf forest | NA | deciduousNeedleleafForest |
| 4-01-01-06 | Deciduous Broadleaf forest | NA | deciduousBroadleafForest |
| 4-01-01-07 | Mixed forest | NA | mixedForest |
| 4-01-01-08 | Closed shrublands | NA | closedShrublands |
| 4-01-01-09 | Open shrublands | NA | openShrublands |
| 4-01-01-10 | Woody savannas | NA | woodySavannas |
| 4-01-01-11 | Savannas | NA | svannas |
| 4-01-01-12 | Grasslands | NA | grasslands |
| 4-01-01-13 | Permanent wetlands | NA | permanentWetlands |
| 4-01-01-14 | Croplands | NA | croplands |
| 4-01-01-15 | Urban and built-up | NA | urbanBuiltup |
| 4-01-01-16 | Cropland/Natural vegetation mosaic | NA | mosaic |
| 4-01-01-17 | Snow and ice | NA | snowIce |
| 4-01-01-18 | Barren or sparsely vegetated | NA | barren |
| 4-01-01-19 | Unclassified | NA | unclassified |

### Code table: 4-01-02

Code table title: Land cover types (UMD)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-02-01 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) | notApplicable |
| 4-01-02-02 | Water | NA | water |
| 4-01-02-03 | Evergreen Needleleaf forest | NA | evergreenNeedleleafForest |
| 4-01-02-04 | Evergreen Broadleaf forest | NA | evergreenBroadleafForest |
| 4-01-02-05 | Deciduous Needleleaf forest | NA | deciduousNeedleleafForest |
| 4-01-02-06 | Deciduous Broadleaf forest | NA | deciduousBroadleafForest |
| 4-01-02-07 | Mixed forest | NA | mixedForest |
| 4-01-02-08 | Closed shrublands | NA | closedShrublands |
| 4-01-02-09 | Open shrublands | NA | openShrublands |
| 4-01-02-10 | Woody savannas | NA | woodySavannas |
| 4-01-02-11 | Savannas | NA | savannas |
| 4-01-02-12 | Grasslands | NA | grasslands |
| 4-01-02-13 | Croplands | NA | croplands |
| 4-01-02-14 | Urban and built-up | NA | urbanBuiltup |
| 4-01-02-15 | Barren or sparsely vegetated | NA | barren |
| 4-01-02-16 | Unclassified | NA | unclassified |

### Code table: 4-01-03

Code table title: Land cover types (LAI/fPAR)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-03-01 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) | notApplicable |
| 4-01-03-02 | Water | NA | water |
| 4-01-03-03 | Grasses/Cereal crops | NA | grassesCerealCrops |
| 4-01-03-04 | Shrubs | NA | shrubs |
| 4-01-03-05 | Broadleaf crops | NA | broadleafCrops |
| 4-01-03-06 | Savanna | NA | savannas |
| 4-01-03-07 | Evergreen broadleaf forest | NA | evergreenBroadleafForest |
| 4-01-03-08 | Deciduous broadleaf forest | NA | deciduousBroadleafForest |
| 4-01-03-09 | Evergreen needleleaf forest | NA | evergreenNeedleleafForest |
| 4-01-03-10 | Deciduous needleleaf forest | NA | deciduousNeedleleafForest |
| 4-01-03-11 | Non vegetated | NA | nonVegetated |
| 4-01-03-12 | Urban | NA | urban |
| 4-01-03-13 | Unclassified | NA | unclassified |

### Code table: 4-01-04

Code table title: Land cover types (NPP)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-04-01 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) | notApplicable |
| 4-01-04-02 | Water | NA | water |
| 4-01-04-03 | Evergreen needleleaf vegetation | NA | evergreenNeedleleafVegetation |
| 4-01-04-04 | Evergreen broadleaf vegetation | NA | evergreenBroadleafVegetation |
| 4-01-04-05 | Deciduous needleleaf vegetation | NA | deciduousNeedleleafVegetation |
| 4-01-04-06 | Deciduous broadleaf vegetation | NA | deciduousBroadleafVegetation |
| 4-01-04-07 | Annual broadleaf vegetation | NA | annualBroadleafVegetation |
| 4-01-04-07 | Non-vegetated land | NA | nonVegetated |
| 4-01-04-09 | Urban | NA | urban |
| 4-01-04-10 | Unclassified | NA | unclassified |

### Code table: 4-01-05

Code table title: Land cover types (PFT)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-05-01 | Water | None of the codes in the table are applicable in the context of this particular observation (nilReason) | water |
| 4-01-05-02 | Evergreen Needleleaf trees | NA | evergreenNeedleTrees |
| 4-01-05-03 | Evergreen Broadleaf trees | NA | evergreenBroadleafTrees |
| 4-01-05-04 | Deciduous Needleleaf trees | NA | deciduousNeedleafTrees |
| 4-01-05-05 | Deciduous Broadleaf trees | NA | deciduousBroadleafTrees |
| 4-01-05-06 | Shrub | NA | shrub |
| 4-01-05-07 | Grass | NA | grass |
| 4-01-05-08 | Cereal crops | NA | cerealCrops |
| 4-01-05-09 | Broad-leaf crops | NA | broadLeafCrops |
| 4-01-05-10 | Urban and built-up | NA | urbanBuiltUp |
| 4-01-05-11 | Snow and ice | NA | snowIce |
| 4-01-05-12 | Barren or sparse vegetation | NA | barren |
| 4-01-05-13 | Unclassified | NA | unclassified |
| 4-01-05-14 | Fill Value | NA | NA |

### Code table: 4-01-06

Code table title: Land cover types (LCCS) (cf. Antonio Di Gregorio (2005))

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-01-06-01 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) | notApplicable |
| 4-01-06-02 | Cultivated and Managed Terrestrial Areas |  | cultivatedManagedTerrestrial |
| 4-01-06-03 | Natural and Semi-Natural Terrestrial Vegetation |  | naturalSemiNaturalTerrestrial |
| 4-01-06-04 | Cultivated Aquatic or Regularly Flooded Areas |  | cultivatedAquatic |
| 4-01-06-05 | Natural and Semi-Natural Aquatic or Regularly Flooded Vegetation |  | naturalSemiNaturalAquatic |
| 4-01-06-06 | Artificial Surfaces and Associated Areas |  | artificialSurfaces |
| 4-01-06-07 | Bare Areas |  | bareAreas |
| 4-01-06-08 | Artificial Waterbodies, Snow and Ice |  | artificialWaterbodiesSnowIce |
| 4-01-06-09 | Natural Waterbodies, Snow and Ice |  | naturalWaterbodiesSnowIce |
| 4-01-06-10 | Unclassified |  | unclassified |

### Code table: 4-02

Code table title: Surface cover classification scheme

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 4-02-01 | Land cover types (IGBP) | International Geosphere-Biosphere Programme https://lpdaac.usgs.gov/products/modis_products_table/mcd12q1 | igbp |
| 4-02-02 | Land cover types (UMD) | The University of Maryland Department of Geography generated global land cover classification collection from 1998. http://glcf.umd.edu/data/landcover/ | umd |
| 4-02-03 | Land cover types (LAI/fPAR) | Leaf Area Index (LAI) and Fractional Photosynthetically Active Radiation (FPAR). FPAR/LAI is the Fraction of Absorbed Photosynthetically Active radiation that a plant canopy absorbs for photosynthesis and growth in the 0.4 – 0.7nm spectral range. | laifpar |
| 4-02-04 | Land cover types (NPP) | Net Primary Production (NPP) land cover scheme | npp |
| 4-02-05 | Land cover types (PFT) | Plant Functional Types (PFT) land cover scheme | pft |
| 4-02-06 | Land cover types (LCCS) | Land cover classification scheme (LCCS) | lccs |

### Code table: 4-03-01

Code table title: Local topography (based on Speight 2009)

| # | Name | Definition |
| --- | --- | --- |
| 4-03-01-1 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) |
| 4-03-01-2 | Hilltop | Higher than all or nearly all of the surrounding land or subsurface. |
| 4-03-01-3 | Ridge | Higher than all or nearly all of the surrounding land or subsurface, but elongated and extending beyond a 50 m radius. |
| 4-03-01-4 | Slope | Neither crest nor depression or valley bottom, and with a slope more than 3%. |
| 4-03-01-5 | Flat | Slope less than 3% and not a top, ridge, valley bottom or depression. Use for plains. |
| 4-03-01-6 | Valley bottom | Lower than nearly all of surrounding land or subsurface, but water can flow out. |
| 4-03-01-7 | Depression | Lower than surrounding land or subsurface, with no above-ground outlet for water. |

### Code table: 4-03-02

Code table title: Relative elevation

| # | Name | Definition |
| --- | --- | --- |
| 4-03-02-1 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) |
| 4-03-02-2 | Lowest | In the bottom 5% of the elevation range |
| 4-03-02-3 | Low | Between 5% and 25% of the elevation range |
| 4-03-02-4 | Middle | Between 25% and 75% of the elevation range |
| 4-03-02-5 | High | Between 75% and 95% of the elevation range |
| 4-03-02-6 | Highest | In the highest 5% of the elevation range |

### Code table: 4-03-03

Code table title: Topographic context (based on Hammond 1954)

| # | Name | Definition |
| --- | --- | --- |
| 4-03-03-1 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) |
| 4-03-03-2 | Plains | Very low relief |
| 4-03-03-3 | Hollows | Low relief, tending to convergent form |
| 4-03-03-4 | Rises | Low relief, tending to divergent form |
| 4-03-03-5 | Valleys | Medium relief, tending to convergent form |
| 4-03-03-6 | Hills | Medium relief, tending to divergent form |
| 4-03-03-7 | Mountains | High relief |

### Code table: 4-03-04

Code table title: Altitude/Depth

| # | Name | Definition |
| --- | --- | --- |
| 4-03-04-1 | Not applicable | None of the codes in the table are applicable in the context of this particular observation (nilReason) |
| 4-03-04-2 | Very small | between -100 m and 100 m |
| 4-03-04-3 | Small | Between -300 and -100 m or between 100 and 300 m |
| 4-03-04-4 | Middle | Between -1000 and -300 m or between 300 and 1000 m |
| 4-03-04-5 | Large | Between -3000 and -1000 m Between 1000 and 3000 m |
| 4-03-04-6 | Very large | Deeper than -3000 m or above 3000 m |

### Code table: 4-04

Code table title: Events at station/platform

| # | Path | Parent_ID | Name | Definition | WMO306_CD |
| --- | --- | --- | --- | --- | --- |
| 1 | >(not applicable) | NA | (not applicable) | None of the codes in the table is applicable in the context of the observing facility; or: Unknown; or: Information not available | notApplicable |
| 2 | >Incident | NA | Incident | An unexpected event that has (or may have) impacted the observations | incident |
| 3 | >Intervention | NA | Intervention | Human activity that has (or may have) impacted the observations | intervention |
| 4 | >Incident>Environmental | 2 | Environmental | NA | environmental |
| 5 | >Incident>Fire | 2 | Fire | Fire inside building | fire |
| 6 | >Incident>Severe weather | 2 | Severe weather | A natural severe weather event that has (or may have) impacted the observations | severeWeather |
| 7 | >Incident>Space weather | 2 | Space weather | An event in outer space that has (or may have) impacted the observations | spaceWeather |
| 8 | >Incident>Technical | 2 | Technical | NA | technical |
| 9 | >Intervention>Agricultural activity | 3 | Agricultural activity | NA | agriculturalActivity |
| 10 | >Intervention>Biomass burning (planned) | 3 | Biomass burning (planned) | NA | biomassBurningPlanned |
| 11 | >Intervention>Construction | 3 | Construction | NA | construction |
| 12 | >Intervention>Equipment cleaning/ painting | 3 | Equipment cleaning/ painting | NA | equipmentCleaning |
| 13 | >Intervention>Equipment maintenance | 3 | Equipment maintenance | how is this different from "preventive maintenance"? | equipmentMaintenance |
| 14 | >Intervention>Equipment repair/ replacement | 3 | Equipment repair/ replacement | NA | repairReplacement |
| 15 | >Intervention>Equipment verification/ calibration | 3 | Equipment verification/ calibration | NA | equipmentVerificationCalibration |
| 16 | >Intervention>Personnel absent | 3 | Personnel absent | NA | personnelAbsence |
| 17 | >Intervention>Infrastructure works | 3 | Infrastructure works | NA | infrastructureWorks |
| 18 | >Intervention>Inspection/ auditing | 3 | Inspection/ auditing | Corrosion of components, seizing | inspectionAuditing |
| 19 | >Intervention>Land use change | 3 | Land use change | NA | landUseChange |
| 20 | >Intervention>Planned service outage | 3 | Planned service outage | NA | plannedServiceOutage |
| 21 | >Intervention>Preventative maintenance | 3 | Preventative maintenance | NA | preventiveMaintenance |
| 22 | >Intervention>Road work | 3 | Road work | NA | roadWork |
| 23 | >Intervention>Theft of equipment | 3 | Theft of equipment | NA | theft |
| 24 | >Intervention>Unrest/strike | 3 | Unrest/strike | NA | unrestStrike |
| 25 | >Intervention>Urban development | 3 | Urban development | NA | urbanDevelopment |
| 26 | >Intervention>Vandalism | 3 | Vandalism | NA | vandalism |
| 27 | >Intervention>Vegetation trimming/ cutting | 3 | Vegetation trimming/ cutting | NA | vegetationTrimming |
| 28 | >Incident>Environmental>Cyclone/tornado | 4 | Cyclone/tornado | NA | cycloneTornado |
| 29 | >Incident>Environmental>Dust/sand storm | 4 | Dust/sand storm | NA | dustSandStorm |
| 30 | >Incident>Environmental>Extreme cold | 4 | Extreme cold | NA | extremeCold |
| 31 | >Incident>Environmental>Extreme heat | 4 | Extreme heat | NA | extremeHeat |
| 32 | >Incident>Environmental>Hail | 4 | Hail | NA | hail |
| 33 | >Incident>Environmental>Heavy precipitation/flooding | 4 | Heavy precipitation/flooding | NA | flooding |
| 34 | >Incident>Environmental>High wind/ blowing debris | 4 | High wind/ blowing debris | NA | windDebris |
| 35 | >Incident>Environmental>Snow/blizzard/icing | 4 | Snow/blizzard/icing | NA | snowBlizzardIcing |
| 36 | >Incident>Environmental>Thunderstorm/ lightning strike | 4 | Thunderstorm/ lightning strike | NA | thunderstormLightning |
| 37 | >Incident>Fire>Magnetic storm | 5 | Magnetic storm | NA | magneticStorm |
| 38 | >Incident>Fire>Solar flares | 5 | Solar flares | NA | solarFlares |
| 39 | >Incident>Severe weather>Avalanche | 6 | Avalanche | NA | avalanche |
| 40 | >Incident>Severe weather>Biological fouling | 6 | Biological fouling | NA | biologicalFouling |
| 41 | >Incident>Severe weather>Biomass burning (unplanned) | 6 | Biomass burning (unplanned) | NA | biomassBurningUnplanned |
| 42 | >Incident>Severe weather>Corrosion | 6 | Corrosion | NA | corrosion |
| 43 | >Incident>Severe weather>Earthquake | 6 | Earthquake | NA | earthQuake |
| 44 | >Incident>Severe weather>Erosion | 6 | Erosion | NA | erosion |
| 45 | >Incident>Severe weather>Guano fouling | 6 | Guano fouling | NA | guanoFouling |
| 46 | >Incident>Severe weather>Insect/animal infestation | 6 | Insect/animal infestation | NA | infestation |
| 47 | >Incident>Severe weather>Land slide/mudslide | 6 | Land slide/mudslide | Hazard event | landslide |
| 48 | >Incident>Severe weather>Pollution | 6 | Pollution | NA | pollution |
| 49 | >Incident>Severe weather>Storm surge | 6 | Storm surge | NA | stormSurge |
| 50 | >Incident>Severe weather>Tsunami | 6 | Tsunami | NA | tsunami |
| 51 | >Incident>Severe weather>Volcanc erruption/ Ash | 6 | Volcanc erruption/ Ash | NA | volcanicEruption |
| 52 | >Incident>Space weather>Cyber security penetration/ virus/ worm | 7 | Cyber security penetration/ virus/ worm | NA | cyberAttack |
| 53 | >Incident>Space weather>Data communications outage | 7 | Data communications outage | NA | dataCommOutage |
| 54 | >Incident>Space weather>Equipment malfunction | 7 | Equipment malfunction | NA | malfunction |
| 55 | >Incident>Space weather>Ground clutter/ signal blockage/ degradation | 7 | Ground clutter/ signal blockage/ degradation | NA | groundClutter |
| 56 | >Incident>Space weather>Hardware failure | 7 | Hardware failure | NA | hardwareFailure |
| 57 | >Incident>Space weather>Power outage | 7 | Power outage | NA | powerOutage |
| 58 | >Incident>Space weather>Radio frequency interference | 7 | Radio frequency interference | NA | radioFrequencyInterference |
| 59 | >Incident>Space weather>Software failure | 7 | Software failure | NA | softwareFailure |

### Code table: 5-01

Code table title: Source of observation

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Instrumental automatic reading | Automatically produced measurement result | automaticReading |
| 2 | Instrumental manual reading | Manual reading of instrument, both analog and digital outputs | manualReading |
| 3 | Human observation | Human, non-instrumented observation | humanObservation |
| 4 | (unspecified) | Procedure used to obtain result unspecified | unspecified |

### Code table: 5-02

Code table title: Measurement/observing method [Code table under development]

### Code table: 5-04

Code table title: Instrument operating status

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | Operational | The instrument is declared operational and subject to routine maintenance | operational |
| 2 | Testing / commissioning | The instrument is deployed for testing purposes and the information provided may not be reliable | testing |
| 3 | Not in service / inactive | The instrument is deployed but presently not in service | inactive |

### Code table: 5-08-01

Code table title: control standard type

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 5-08-01-01 | International | NA | internationalStandard |
| 5-08-01-02 | Primary | NA | primaryStandard |
| 5-08-01-03 | Secondary | NA | secondaryStandard |
| 5-08-01-04 | Reference | NA | referenceStandard |
| 5-08-01-05 | Working | NA | workingStandard |
| 5-08-01-06 | Transfer | NA | transferStandard |
| 5-08-01-07 | Travelling | NA | travellingStandard |
| 5-08-01-08 | Collective | NA | collectiveStandard |

### Code table: 5-08-02

Code table title: control location

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 5-08-02-01 | Field calibration | Instrument control activity performed on site | onSite |
| 5-08-02-02 | Laboratory calibration | Instrument control activity performed in the laboratory | laboratory |
| 5-08-02-03 | Remote (non in-situ) | Instrument control activity performed remotely | remote |

### Code table: 5-08-03

Code table title: Instrument control result

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 5-08-03-01 | no changes - in calibration | Instrument verified and found to be in calibration | inCalibration |
| 5-08-03-02 | no changes - out of calibration | Instrument checked and found to be out of calibration; no changes to calibration function | outOfCalibration |
| 5-08-03-03 | no changes – calibration unknown | Instrument visited but calibration could not be carried out | unknown |
| 5-08-03-04 | recalibrated - in calibration | Instrument checked and found to be out of calibration; instrument recalibrated (calibration function changed) | recalibrated |

### Code table: 5-14

Code table title: Status of observation

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 5-14-01 | Primary | The primary or official observation of the observed variable | primary |
| 5-14-02 | Additional | Additional or supplemented observation of the observed variable | additional |

### Code table: 5-15

Code table title: Exposure of instrument

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | (unspecified) | Classification not specified | unspecified |
| 2 | Class 1 | Exposure of instrument allows reference level measurements | class1 |
| 3 | Class 2 | Exposure of instrument has small or infrequence influence on measurement | class2 |
| 4 | Class 3 | Exposure of instrument leads to increased uncertainty or occasional invalid measurements | class3 |
| 5 | Class 4 | Exposure of instrument leads to high uncertainty or regular invalid measurements | class4 |
| 6 | Class 5 | Exposure of instrument leads to invalid measurements | class5 |

### Code table: 6-03

Code table title: Sampling strategy

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 6-03-1 | Continuous | Sampling is done continuously, but not necessarily at regular time intervals. Sampling is integrating, i.e., none of the medium escapes observations. | continuous |
| 6-03-2 | Discrete | Sampling is done at regular time intervals for certain sampling periods that are smaller than the time interval. Sampling is not integrating, i.e., parts of the medium escape observation. | discrete |
| 6-03-3 | Event | Sampling is done at irregular time intervals. | event |

### Code table: 7-06

Code table title: Level of data

| # | Name | WMO306_CD | Definition |  |
| --- | --- | --- | --- | --- |
|  |  |  | CIMO (WMO-No. 8, 2008, Updated 2010) | CEOS (http://www.ceos.org/images/WGISS/Documents/Handbook.pdf) |
| 7-06-1 | Unknown | unknown |  |  |
| 7-06-2 | Raw | raw |  | Physical information: Data in their original packets, as received from a satellite |
| 7-06-3 | Level 0 | level0 | Analogue/digital electric signals | Physical information: Reconstructed unprocessed instrument data at full space time resolution with all available supplemental information to be used in subsequent processing (e.g., ephemeris, health and safety) appended. |
| 7-06-4 | Level I | level1 | Level I data (Primary Data): in general, are instrument readings expressed in appropriate physical units, and referred to Earth geographical coordinates. They require conversion to the normal meteorological variables (identified in Part I, Chapter 1). Level I data themselves are in many cases obtained from the processing of electrical signals such as voltages, referred to as raw data. Examples of these data are satellite radiances and water-vapour pressure, positions of constant-level balloons, etc. but not raw telemetry signals. Level I data still require conversion to the meteorological parameters specified in the data requirements. | Physical information: Unpacked, reformatted level 0 data, with all supplemental information to be used in subsequent processing appended. Optional radiometric and geometric correction applied to produce parameters in physical units. Data generally presented as full time/space resolution. A wide variety of sub level products are possible. |
| 7-06-5 | Level II | level2 | Level II Data (Meteorological parameters). They may be obtained directly from many kinds of simple instruments, or derived from Level I data. For example, a sensor cannot measure visibility, which is a Level II quantity; instead, sensors measure the extinction coefficient, which is a Level I quantity. | Geophysical information. Retrieved environmental variables (e.g., ocean wave height, soil moisture, ice concentration) at the same resolution and location as the level 1 source data. |
| 7-06-6 | Level III | level3 | Level III (Initial state parameters) are internally consistent data sets, generally in grid‑point form obtained from level II data by applying established initialization procedures. NOTE: Data exchanged internationally are level II or level III data. | Geophysical information. Data or retrieved environmental variables which have been spatially and/or temporally re-sampled (i.e., derived from level 1 or 2 products). Such re-sampling may include averaging and compositing. |
| 7-06-7 | Level IV | level4 |  | Thematic information. Model output or results from analyses of lower level data (i.e., variables that are not directly measured by the instruments, but are derived from these measurements). |

### Code table: 7-07

Code table title: Reference time

| # | Name | Definiton | WMO306_CD |
| --- | --- | --- | --- |
| 1 | ASCII | ASCII (American Standard Code for Information Interchange) is a character-encoding scheme used for text files. Plain text files are often called "ASCII" files. | ascii |
| 3 | NASA AMES | The NASA AMES format is a set of nine ASCII-based file formats that comply with an overall common structure but make provision of different features adapted to various cases. It consists of a header and a data part. | nasaAmes |
| 4 | HDF4 | Hierarchical Data Format (HDF) is a set of file formats (HDF4, HDF5) designed to store and organize large amounts of data. | hdf4 |
| 5 | HDF5 | Hierarchical Data Format (HDF) is a set of file formats (HDF4, HDF5) designed to store and organize large amounts of data. | hdf5 |
| 6 | XML | Extensible Markup Language (XML) is a markup language that defines a set of rules for encoding documents in a format which is both human-readable and machine-readable. | xml |
| 8 | CSV | A comma-separated values (CSV) file stores tabular data (numbers and text) in plain text. Each line of the file is a data record. Each record consists of one or more fields, separated by commas, hence the name for this file format. | csv |
| 10 | NetCDF | A set of software libraries and self-describing, machine-independent data formats that support the creation, access, and sharing of array-oriented scientific data. http://www.unidata.ucar.edu/software/netcdf/ | netcdf |
| 12 | FM 13 - SHIP | Report of surface observation from a fixed sea station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | ship |
| 13 | FM14 - SYNOP MOBIL | Report of a surface observation from a mobile land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | synopMobil |
| 14 | FM 15 - METAR | Aerodrome routine meteorological report (with or without trend forecast). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | metar |
| 15 | FM 16 - SPECI | Aerodrome special meteorological report (with or withoug trend forecast). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | speci |
| 16 | FM 18 - BUOY | Report of a buoy observation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | buoy |
| 17 | FM 20 - RADOB | Report of ground radar weather observation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | radob |
| 18 | FM 22 - RADREP | Radiological data report (monnitoried on a routine basis and/or in case of accident). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | radrep |
| 19 | FM 32 - PILOT | Upper-wind report froma fixed land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | pilot |
| 20 | FM33 - PILOT SHIP | Upper-wind report from a sea station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | pilotShip |
| 21 | FM 34 - PILOT MOBIL | Upper wind report from a mobile land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | pilotMobil |
| 22 | FM 35 - TEMP | Upper-level pressure, temperature, humidity and wind report from a fixed land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | temp |
| 23 | FM 36 - TEMP SHIP | Upper-level pressure, temperature, humidity and wind report from a sea station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | tempShip |
| 24 | FM 37 - TEMP DROP | Upper-level pressure, temperature, humidity and wind report from a sonde released by carrier balloons or aircraft. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | tempDrop |
| 25 | FM 38 - TEMP MOBIL | Upper-level pressure, temperature, humidity and wind report from a mobile land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | tempMobil |
| 26 | FM 39 - ROCOB | Upper-level temperature, wind and air density report from a land rocketsonde station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | rocob |
| 27 | FM 40 - ROCOB SHIP | Upper-level temperature, wind and air density report from a rocketsonde station on a ship. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | rocobShip |
| 28 | FM 41 - CODAR | Upper-air report from an aircraft (other than weather reconnaissance aircraft). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | codar |
| 29 | FM 42 - AMDAR | Aircraft report (aircraft meteorological data relay). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | amdar |
| 30 | FM 44 - ICEAN | Ice analysis. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | icean |
| 31 | FM 45 - IAC | Analysis in full form. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | IAC |
| 32 | FM 46 - IAC FLEET | Analysis in abreviated form. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | iacFleet |
| 33 | FM 47 - GRID | Processed data in the form of grid-point values. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | grid |
| 34 | FM 49 - GRAF | Processed data in the form of grid-point vlues (abbreviated code form). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | graf |
| 35 | FM 50 - WINTEM | Forecast upper wind and temperature for aviation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | wintem |
| 36 | FM 51 - TAF | Aerodrome forecast. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | taf |
| 37 | FM 53 - ARFOR | Area forecast for aviation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | arfor |
| 38 | FM 54 - ROFOR | Route forecast for aviation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | rofor |
| 39 | FM 57 - RADOF | Radiological trajectory dose forecast (defined time of arrival and location). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | radof |
| 40 | FM 61 - MAFOR | Forecast for shipping. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | mafor |
| 41 | FM 62 - TRACKOB | Report of marine surface observation along a ship's track. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | trackob |
| 42 | FM 63 - BATHY | Rport of bathythermal observation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | bathy |
| 43 | FM 64 - TESAC | Termperature, salinity and current report from a sea station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | tesac |
| 44 | FM 65 - WAVEOB | Report of spectral wave information from a sea station or from a remote platform (aircraft or satellite). Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | waveob |
| 45 | FM 67 - HYDRA | Report of hydrological observation from a hydrological station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | hydra |
| 46 | FM 68 - HYFOR | Hydrological forecast. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | hyfor |
| 47 | FM 71 - CLIMAT | Report of monthly values from a land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | climat |
| 48 | FM 72 - CLIMAT SHIP | Report of monthly means and totals from an ocean weather station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | climatShip |
| 49 | FM 73 - NACLI, CLINP, SPCLI, CLISA, INCLI | Report of monthly means for an oceanic area. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | nacli |
| 50 | FM 75 - CLIMAT TEMP | Report of monthly aerological means form a land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | climatTemp |
| 51 | FM 76 - CLIMAT TEMP SHIP | Report of monthly aerological means form an ocean weather station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | climatTempShip |
| 52 | FM 81 - SFAZI | Synoptic report of bearings of sources of atmospherics. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | sfazi |
| 53 | FM 82 - SFLOC | Synoptic repoort of the geographical location of sources of atospherics. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | sfloc |
| 54 | FM 83 - SFAZU | Detailed report of the distribution of sources of atmospherics by bearings for any period up to and including 24 hours. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | sfazu |
| 55 | FM 85 - SAREP | Report of synoptic interpretatoin of cloud data obtained by a meteorological satellite. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | sarep |
| 56 | FM 86 - SATEM | Report of satellite remote upper-air soundings of pressure, temperature and humidity. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | satem |
| 57 | FM 87 - SARAD | Report of satellite clear rdiance observations.Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | sarad |
| 58 | FM 88 - SATOB | Report of satellite observations of wind, surface termperature, cloud, humidity and radiation. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | satob |
| 59 | FM 92 - GRIB | Processed data in the form of grid-point values expressed in binary form. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes2 | grib |
| 60 | FM 94 - BUFR | Binary Universal Form for the Representation of meteorological data. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes2 | bufr |
| 61 | FM 95 - CREX | Character form for the representation and exchange of data. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | crex |
| 62 | FM 201 - COLLECT | Collection of reports that use the same XML application schemas. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes3 | collect |
| 63 | FM 202 - METCE | Meteorological information. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes3 | metce |
| 64 | FM 203 - OPM-XML | Observable property model. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes3 | opmXML |
| 65 | FM 204 - SAF-XML | Simple aeronautical features. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes3 | safXML |
| 66 | FM 205 - IWXXM-XML | International Civil Aviation Organization meteorological information exchange model. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes3 | iwxxmXML |
| 11 | FM 12 - SYNOP | Report of surface observation from a fixed land station. Manual on Codes Volume I.1. http://wis.wmo.int/ManCodes1 | synop |

### Code table: 7-10

Code table title: Reference time

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | (unknown) | Source of reference time unknown | unknown |
| 2 | Global navigation satellite system (GNSS) | A Global Navigation Satellite System (GNSS) signal controlled clock, which receives a signal that comes from the atomic clock of a GNSS constellation of satellites. | gnss |
| 3 | Manual comparison | A regular procedure of manual comparison with a reference clock | manualComparison |
| 4 | Radio clock | A radio controlled clock, which receives a radio signal that comes from a reference/atomic clock. | radioClock |
| 5 | Time server (internet based) | A server computer that reads the actual time from a reference/atomic clock and distributes this information to its clients using a computer network. The time server may be a local network time server or an internet time server. | timeServer |

### Code table: 8-03-01

Code table title: Quality flag [From BUFR code table 0 33 020 (WMO, 2013a)]

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 8-03-01-1 | Good | The data has been examined and represents a reliable measurement. | good |
| 8-03-01-2 | Inconsistent | The data has been examined and one or more parameters are inconsistent; the relationship between different elements does not meet defined criteria. | inconsistent |
| 8-03-01-3 | Doubtful | The data should be treated as suspect. | suspect |
| 8-03-01-4 | Wrong | The data has been examined and determined to contain errors | erroneous |
| 8-03-01-5 | Not checked | The data has not been checked by any qualitative or quantitative method. | unchecked |
| 8-03-01-6 | Has been changed | The original value has been replaced with another value. | changed |
| 8-03-01-7 | Estimated | The value has been estimated rather than measured. | estimate |
| 8-03-01-8 | Missing value | The data is missing. | missing |

### Code table: 8-03-02

Code table title: Quality flag [From OGC WaterML 2.0]

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 8-03-02-1 | Good | The data has been examined and represents a reliable measurement. | good |
| 8-03-02-2 | Suspect | The data should be consisdred as low quality and may have been rejected. | poor |
| 8-03-02-3 | Estimate | The data should be treated as suspect. | suspect |
| 8-03-02-4 | Poor | The data has not been checked by any qualitative or quantitative method. | unchecked |
| 8-03-02-5 | Unchecked | The value has been estimated rather than measured. | estimate |
| 8-03-02-6 | Missing | The data is missing. | missing |

### Code table: 8-04

Code table title: Quality Flag System

| # | Name | Quality_Flag_Sys_URL | Definition | WMO306_CD |
| --- | --- | --- | --- | --- |
| 1 | (unknown) | NA | Quality flagging system not known | unknown |
| 2 | WMO BUFR table 0 33 020 | http://codes.wmo.int/bufr4/codeflag/0-33-020 | Quality flags are specified according to BUFR table 0 33 020 (http://codes.wmo.int/bufr4/codeflag/0-33-020) | bufr033020 |
| 3 | Other quality flagging system | NA | Quality flags are specified according to another system | other |
| 4 | WaterML2 | http://www.opengis.net/def/waterml/2.0/quality/ | Quality flags are specified according to WaterML2, http://www.opengis.net/def/waterml/2.0/quality/ | waterML2.0 |

### Code table: 8-05

Code table title: Traceability

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 8-05-1 | (unknown) | The traceability of the observations is unknown. | unknown |
| 8-05-2 | Traceable to international standard | The observations made are traceable to an internationally agreed standard. | traceableToInternationalStandard |
| 8-05-3 | Traceable to non-international standard | The observations made are traceable to some other standard. | traceableToOtherStandard |

### Code table: 9-02

Code table title: Data policy/use constraints (WMO_DataLicenseCode, WMO 2013b, Table 14)

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 9-02-1 | WMOEssential | WMO Essential Data: free and unrestricted international exchange of basic data and products. | wmoEssential |
| 9-02-2 | WMOAdditional | WMO Additional Data: free and unrestricted access to data and products exchanged under the auspices of WMO to the research and education communities for non-commercial activities. A more precise definition of the data policy may be additionally supplied within the metadata. In all cases it shall be the responsibility of the data consumer to ensure that they understand the data policy specified by the data provider – which may necessitate dialogue with the data publisher for confirmation of terms and conditions. | wmoAdditional |
| 9-02-3 | WMOOther | Data identified for global distribution via WMO infrastructure (GTS / WIS) that is not covered by WMO Resolution 25 neither WMO Resolution 40; e.g. aviation OPMET data. Data marked with “WMOOther” data policy shall be treated like “WMOAdditional” where a more precise definition of the data policy may be additionally supplied within the metadata. In all cases it shall be the responsibility of the data consumer to ensure that they understand the data policy specified by the data provider – which may necessitate dialogue with the data publisher for confirmation of terms and conditions. | wmoOther |
| 9-02-4 | (unspecified) | No limitation on distribution or use | noLimitation |

ADDITIONAL CODE TABLES, NOT SPECIFIC TO A PARTICULAR METADATA CATEGORY OR ELEMENT

### Code table: 11-01

Code table title: “Coordinates Source/Service”

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | GPS | Location determined using the Global Positioning System. http://www.gps.gov/ | gps |
| 2 | ARGOS DOPPLER | Location determined using the frequency shift of tramsmissions to the ARGOS communications satellite service. http://www.argos-system.org/web/en/333-why-choose-argos.php | argosDoppler |
| 3 | IRIDIUM DOPPLER | Location determined using the frequency shift of transmissions to the Iridium communications satellite constellation. https://www.iridium.com/ | iridiumDoppler |
| 4 | ARGOS Kalman | Location determined using the kalman filtering technique and the ARGOS communications satellite service. http://www.argos-system.org/web/en/393-faq-kalman.php. | argosKalman |
| 5 | GALILEO | Location determined using the European GALILEO global navigation satellite system. http://galileognss.eu/ | galileo |
| 6 | LORAN | Location determined using the LORAN (LOng RAnge Navigation) hyperbolic radio system. | loran |
| 7 | Survey | Location determined using a geographic survey. | survey |
| 8 | From map | Location extracted by locating the object on a map. | map |

### Code table: 11-02

Code table title: “Coordinates reference” [Code table under development]

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | WGS84 | An Earth-centered, Earth-fixed terrestrial reference system and geodetic datum. Currently, WGS84 uses the EGM96 (Earth Gravitational Model 1996) geoid, revised in 2004. | wgs84 |

### Code table: 11-03

Code table title: Meaning of time stamp

| # | Name | Definition | WMO306_CD |
| --- | --- | --- | --- |
| 1 | (unknown) | The beginning and end of the period described by the timestamp are unknown. | unknown |
| 2 | Beginning | Time stamps indicate the end of a period covering the range up to but excluding the preceding time stamp | beginning |
| 3 | End | Time stamps indicate the middle of a period beginning at the middle of the range described by this and the preceding time stamp and ending right before the middle of the range described by this and the following time stamp. | end |
| 4 | Middle | Time stamps indicate the middle of a period beginning at the middle of the range described by this and the preceding time stamp and ending right before the middle of the range described by this and the following time stamp. | middle |

Appendix

EXAMPLES OF WMDS APPLIED TO SPACE-BASED OBSERVATIONS

| WMDS |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |
| --- | --- | --- | --- | --- | --- | --- |
| Category | Id | Name | Definition |  |  |  |
| Observed variable | 1-01 | Observed variable – measurand | Variable intended to be measured or observed or derived, including the biogeophysical context | units that we measure that are not present | Brightness Temperature |  |
|  | 1-02 | Measurement unit | Real scalar quantity, defined and adopted by convention, with which any other quantity of the same kind can be compared to express the ratio of the two quantities as a number [VIM3, 1.9] | unitless quantities? enumerated types (cloud type) | kelvin | 1-02-5 |
|  | 1-03 | Temporal extent | Time period covered by a series of observations inclusive of the specified date-time indications (measurement history) | end date could be the end of a processing system | 7/7/2015 |  |
|  | 1-04 | Spatial extent | Typical georeferenced volume covered by the observations |  | GEOS @ 140.7 (80N-80S, 60E-220E) |  |
|  | 1-05 | Representativeness | Spatial extent of the region around the observation for which it is representative | I expect we could ignore - or specify the spatial resolution | NA |  |

| Purpose of observ. |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Id | Name | Definition |  |  |  |  |
|  | 2-01 | Application area(s) | Context within, or intended application(s) for which the observation is primarily made or which has/have the most stringent requirements |  | Nowcasting, General forecasting; ocean applications |  | 2-01-3; 2-01-5; 2-01-7 |
|  | 3-01 | Region of origin of data | WMO Region | NilReason |  |  |  |
| Station/platformObserving facility | 3-02 | Territory of origin of data | Country or territory name of the location of the observation | NilReason |  |  |  |
|  | 3-03 | Station/platformObserving facility name | Official name of the observing facility | is this the full name? | Himawari-8 |  | 173 |
|  | 3-04 | Station/platformObserving facility type | A categorization of the type of environmental monitoring facility at which an observed variable is measured |  |  |  | satellite |
|  | 3-05 | Station/platformObserving facility model | The model of the monitoring equipment used at the observing facility | perhaps cosmic is better ex. most hw platforms are diff. (except 8 9) | Himawari-8 |  |  |
|  | 3-06 | Station/platformObserving facility unique identifier | A unique and persistent identifier for an environmental monitoring facility (station/ platform), which may be used as a external point of reference | WMO ID | 173 |  |  |
| Station/platformObserving facility | 3-07 | Geospatial location | Position in space defining the location of the environmental monitoring observing facility at the time of observation | space/orbit; what about drift in orbit NOAA-15 into terminator? |  |  |  |
|  |  |  |  |  | moving |  | space based |
|  |  |  |  |  | Orbit type |  | GEO |
|  |  |  |  |  | Central longitude |  | 140.7 |
|  |  |  |  |  | height |  | 38500 |
|  |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |  |
|  | Id | Name | Definition |  |  |  |  |
| Station/platformObserving facility | 3-09 | Station Status | Declared reporting status of the station | Do not include gaps of a day or two = covered by data set level metadata. Do we need a standby status | Operational |  | 3-09-3 |
| Environment | 4-01 | Surface cover | The observed (bio)physical cover on the earth’s surface in the vicinity of the observation |  | NA |  | 4-01-01-00 |
|  | 4-02 | Surface cover classification scheme | Name and reference or link to document describing the classification scheme |  |  |  |  |
|  | 4-03 | Topography or bathymetry | The shape or configuration of a geographical feature, represented on a map by contour lines |  |  |  |  |
|  | 4-04 | Events at observing facility | Description of human action or natural event at the station or at the vicinity that may influence the observation | space weather (specify start/stop) | can be realized as a specific URL pointing to this information |  |  |
|  | 4-05 | Site information | Non-formalized information about the location and its surroundings at which an observation is made and that may influence it |  |  |  |  |
| Instruments and methods of observation | 5-01 | Source of observation | The source of the dataset described by the metadata |  | automatic |  | 5-01-1 |
|  | 5-02 | Measurement/observing method | The method of measurement/observation used | add to table form - hierarchy of methods for review (surface bias). Verify that these match | Moderate-resolution optical imager |  |  |
| Instruments and methods of observation |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |  |
|  | Id | Name | Definition |  |  |  |  |
|  | 5-04 | Instrument operating status | The status of an instrument with respect to its operation | perhaps add satellite termino-logy such as "commissioning" | operational |  | 5-04-1 |
|  | 5-05 | Vertical distance of sensor | Vertical distance of the sensor from a (specified) reference level such as local ground, or deck of a marine platform at the point where the sensor is located; or sea surface | NA for satellite. |  |  |  |
|  | 5-06 | Configuration of instrumentation | Description of any shielding or configuration/setup of the instrumentation or auxiliary equipment needed to make the observation or to reduce the impact of extraneous influences on the observation | NA | NA or a URL |  |  |
|  | 5-07 | Instrument control schedule | Description of schedule for calibrations or verification of instrument | In case of AVHRR this could be each scan (PRT-bb) - no, this is data. MODIS lunar calibration is more appropriate | generic short statements and/or URL |  |  |
|  | 5-08 | Instrument control result | The result of an instrument control check, including date, time, location, standard type and period of validity | Perhaps the table could be expanded - | URL to where this info can be found |  |  |
|  | 5-09 | Instrument model and serial number | Details of manufacturer, model number, serial number and firmware version if applicable |  | Advanced Himawari Imager - 01 |  |  |
|  | 5-10 | Instrument routine maintenance | A description of maintenance that is routinely performed on an instrument |  | NilReason=not applicable |  |  |
|  |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |  |
|  | Id | Name | Definition |  |  |  |  |
| Instruments and methods of observation | 5-12 | Geospatial location | Geospatial location of instrument/sensor | NA | same as geolocation of platform |  |  |
|  | 5-13 | Maintenance Activity | Description of maintenance performed on instrument | Station keeping, calibration | NilReason=not applicable |  |  |
|  | 5-14 | Status of observation | Official status of observation |  | n/a |  |  |
|  | 5-15 | Exposure of instruments | The degree to which an instru-ment is affected by external influences and reflects the value of the observed variable |  | n/A |  |  |
| Sampling | 6-01 | Sampling procedures | Procedures involved in obtaining a sample |  | optional, can be a URL |  |  |
|  | 6-02 | Sample treatment | Chemical or physical treatment of sample prior to analysis |  | n/a |  |  |
|  | 6-03 | Sampling strategy | The strategy used to generate the observed variable |  | optional, can be a URL |  |  |
|  | 6-04 | Sampling time period | The period of time over which a measurement is taken |  | instantateous, repeat every 10 |  |  |
|  | 6-05 | Spatial sampling resolution | Spatial resolution refers to the size of the smallest observable object. The intrinsic resolution of an imaging system is determined primarily by the instantaneous field of view of the sensor, which is a measure of the ground area viewed by a single detector element in a given instance in time |  | n/a |  |  |
|  | 6-06 | Temporal sampling interval | Time period between the beginning of consecutive sampling periods |  | N/a |  |  |
|  | 6-07 | Diurnal base time | Time to which diurnal statistics are referenced |  | n/a |  |  |
|  | 6-08 | Schedule of observation | Schedule of observation |  | n/a |  |  |

|  |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data processing and Reporting | Id | Name | Definition |  |  |  |
|  | 7-02 | Processing/analysis center | Center at which the observation is processed |  | ABOM |  |
|  | 7-03 | Temporal reporting period | Time period over which the observable variable is reported |  | 10-minute |  |
|  | 7-04 | Spatial reporting interval | Spatial interval at which the observed variable is reported |  | 2x2 |  |
|  | 7-05 | Software/processor and version | Name and version of the software or processor utilized to derive the element value | Provider would know |  | AGLS r5700 (svn revision ID or git) |
|  | 7-06 | Level of data | Level of data processing |  | L1 | 7-06-3 |
|  | 7-07 | Data format | Description of the format in which the observed variable is being provided |  | NetCDF |  |
|  | 7-08 | Version of data format | Version of the data format in which the observed variable is being provided | Provider would know | AGLS_data-product-specification_rev01.05 |  |
|  | 7-09 | Aggregation period | Time period over which individual samples/observations are aggregated | ? |  |  |
|  | 7-10 | Reference time | Time base to which date and time stamps refer |  | 1/1/1970 |  |
|  | 7-11 | Reference datum | Reference datum used to convert observed quantity to reported quantity | what about geoid? |  |  |
|  | 7-12 | Numerical resolution | Measure of the detail in which a numerical quantity is expressed |  |  | 1dp |
|  | 7-13 | Latency (of reporting) | The typical time between completion of the observation or collection of the datum and when the datum is reported |  |  | 8 mins |

|  |  |  |  | questions | HW-8 Brightness temperature at 10.3um |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data quality | Id | Name | Definition |  |  |  |
|  | 8-02 | Procedure used to estimate uncertainty | A reference or link pointing to a document describing the procedures / algorithms used to derive the uncertainty statement |  |  |  |
|  | 8-03 | Quality flag | An ordered list of qualifiers indicating the result of a quality control process applied to the observation |  |  |  |
|  | 8-04 | Quality flagging system | Reference to the system used to flag the quality of the observation | ? |  |  |
|  | 8-05 | Traceability | Statement defining traceability to a standard, including sequence of measurement standards and calibrations that is used to relate a measurement result to a reference [VIM 3 2.4.2] | difficult to specify | GSICS |  |
| Ownership and data policy | 9-01 | Supervising organization | Name of organization who owns the observation | code table? | ABOM+JMA |  |
|  | 9-02 | Data policy/use constraints | Details relating to the use and limitations surrounding data imposed by the supervising organization |  | None | 9-02-4 |
| Contact | 10-01 | Contact (Nominated Focal Point) | Principal contact (Nominated Focal Point, FP) for resource |  | ABOM |  |

| WMDS |  | SARAL SLA |  | MTSAT-2 CSR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Category | Id |  |  |  |  |  |
| Observed variable | 1-01 | Sea Level Anomaly |  | Clear sky radiance (Brightness Temperatures at multiple wavelengthts), % of area that is clear, Standard Deviation of brightness temperatures within box |  |  |
|  | 1-02 | m | 1-02-1 | kelvin, %, kelvin | 1-02-5 |  |
|  | 1-03 | 2012- |  | 2009- |  |  |
|  | 1-04 | Global |  | GEOS @ 145 (80N-80S, 65E-225E) |  |  |
|  | 1-05 | NA |  | NA |  |  |
| Purpose of observ. | 2-01 | Ocean | 2-01-7 | GNWP | 2-01-1 |  |
|  | 2-02 | ? |  | ? |  |  |
| Station/platformObserving facility | 3-01 |  |  |  |  |  |
|  | 3-02 |  |  |  |  |  |
|  | 3-03 | Satellite with ARGOS and AltiKa | 441 | Himawari-7 | 172 |  |
|  | 3-04 |  | satellite |  | satellite |  |
|  | 3-05 | Satellite with ARGOS and AltiKa |  | Himawari-7 |  |  |
|  | 3-06 | 441 |  | 172 |  |  |
|  | 3-07 |  |  |  |  |  |
|  |  | moving | space based | moving | space based |  |
|  |  | Orbit type | LEO | Orbit type | GEO |  |
|  |  | Sun synchronous | yes | Central longitude | 145 |  |
|  |  | Equator crossing time (ascending) | 6:00 | height | 35800 |  |
|  |  | height | 800 |  |  |  |
|  | 3-08 | Global downlink | 3-08-XX | rebroadcast via MTSAT-1R | 3-08-XX |  |
|  | 3-09 | Operational | 3-09-3 | Operational | 3-09-3 |  |
| WMDS |  | SARAL SLA |  | MTSAT-2 CSR |  |  |
| Category | Id |  |  |  |  |  |
| Environment | 4-01 | NA | 4-01-01-00 | NA | 4-01-01-00 |  |
|  | 4-02 |  |  |  |  |  |
|  | 4-03 |  |  |  |  |  |
|  | 4-04 |  |  |  |  |  |
|  | 4-05 |  |  |  |  |  |
| Instruments and methods of observation | 5-01 | automatic | 5-01-1 | automatic | 5-01-1 |  |
|  | 5-02 | Radar Altimiter |  | Moderate-resolution optical imager |  |  |
|  | 5-03 |  |  |  |  |  |
|  | 5-04 | operational | 5-04-1 | operational | 5-04-1 |  |
|  | 5-05 |  |  |  |  |  |
|  | 5-06 |  |  | ? |  |  |
|  | 5-07 |  |  | ? |  |  |
|  | 5-08 |  |  |  |  |  |
|  | 5-09 | ALTIKA (but also MWR) |  | IMAGER |  |  |
|  | 5-10 |  |  |  |  |  |
|  | 5-11 |  |  |  |  |  |
|  | 5-12 |  |  |  |  |  |
|  | 5-13 |  |  |  |  |  |
|  | 5-14 |  |  |  |  |  |
|  | 5-15 |  |  |  |  |  |
| Sampling | 6-01 |  |  |  |  |  |
|  | 6-02 |  |  |  |  |  |
|  | 6-03 | Pulse |  | Continuous |  |  |
|  | 6-04 | 20 Hz |  | instantateous, repeat every 60 |  |  |
|  | 6-05 | point, 7km apart |  | 60x60 |  |  |
|  | 6-06 |  |  |  |  |  |
| WMDS |  | SARAL SLA |  | MTSAT-2 CSR |  |  |
| Category | Id |  |  |  |  |  |
| Sampling | 6-07 |  |  |  |  |  |
|  | 6-08 | every second |  | hourly full disk |  |  |
| Data processing and Reporting | 7-01 |  |  |  |  |  |
|  | 7-02 | AVISO/RADS |  | JMA |  |  |
|  | 7-03 | 1-second |  | Hourly |  |  |
|  | 7-04 | 7km along track |  | 60x60 |  |  |
|  | 7-05 |  | RADS v4 |  |  |  |
|  | 7-06 | L2 | 7-06-4 | L2 | 7-06-4 |  |
|  | 7-07 | NetCDF |  | BUFR |  |  |
|  | 7-08 | RADS v4 |  |  |  |  |
|  | 7-09 | 1s |  |  |  |  |
|  | 7-10 | 1/1/1980 |  | 1/1/1970 |  |  |
|  | 7-11 |  |  |  |  |  |
|  | 7-12 |  | 0.001m |  | 1dp |  |
|  | 7-13 |  | 2 days |  | 30 mins |  |
| Data quality | 8-01 | ?mm |  | ? |  |  |
|  | 8-02 |  |  |  |  |  |
|  | 8-03 |  |  |  |  |  |
|  | 8-04 |  |  |  |  |  |
|  | 8-05 | multi-sensor and GPS buoys |  | depends on cloud mask product, GSICS |  |  |
| Ownership data policy | 9-01 | ISRO |  | JMA |  |  |
|  | 9-02 | Restricted | 9-02-4 | WMO | 9-02-1 |  |
| Contact | 10-01 | RADS |  | JMA |  |  |

An asterisk (*) denotes the element is required for the WMO Rolling Review of Requirements (RRR) process. A hash sign (#) denotes that it is acceptable to record a "mandatory" element with a value of nilReason (that indicates that the metadata is either “unknown”, or “not applicable”, or “not available”).

Provided as part of the WIS metadata records

Provided as part of the WIS metadata records