## ATU preliminary position on WRC 19 Agenda Items of WMO Prime interest

- Introduction

The First ATU Preparatory Meeting for 2019 World Radio communications Conference (APM19-1) was held at the Intercontinental Hotel, Nairobi, Kenya from 19th to September 2016.

One hundred ninety (190) participants attended the meeting.

Mr. Daniel Obam from Kenya chaired the meeting while Mr. Augustine NWAULUNE was vice chairperson.  Mr. Khalilou NIANE from Senegal and Mr. Jonathan PINIFOLO/Mr. Micious MATSITS from Malawi were the Rapporteurs

- Objectives of the meeting

The objectives of the meeting were to:

- Consider the key lessons from WRC-15;

- Consider ATU and ITU preparatory work plans for WRC-19;

- Appoint the Preparatory Group teams for WRC-19;

- Consider proposals towards an ATU recommendation for mutual licence and type approval recognition;

- Consider some key post WRC-15 Issues (channelling plans and PPDR frequency

- harmonisation), and

- Consider preliminary views on WRC-19 agenda issues.

  - Preparatory work plans and key issues

The meeting noted the preparatory work plans and key issues for the WRC 19 from the three sub regions EACO, ECOWAS and SADC.

  - ATU Preliminary position on agenda items of WRC 19 of prime interest to WMO

The meeting considered all the WRC-19 agenda items based on the compilations of input contributions from sub-regions, administrations, associate members and relevant interested organizations. At this preparatory stage, no preliminary positions existed for the various agenda items.

- Action by SG-RFC proposed

Note the information in the attached table

- Recommended Text

At this preparatory stage, ATU did not come up with preliminary positions for all the agenda items of the WRC19. The working groups and administrations were urged to actively participate and/or follow the ITU-R studies in order to start developing preliminary positions.

--------------------

| 1.2 | To consider In-band power limits for earth stations operating in the mobile-satellite service, meteorological-satellite service and Earth exploration-satellite service in the frequency bands 401-403 MHz and 399.9-400.05 MHz | No Preliminary position |
| --- | --- | --- |
| 1.3 | Meteorological-satellite service (space-to-Earth) and Earth exploration-satellite service (space-to-Earth) in the frequency band 460 470 MHz | No preliminary position |
| 1.6 | to consider the development of a regulatory framework for non-GSO FSS satellite systems that may operate in the frequency bands 37.5-39.5 GHz (space-to-Earth), 39.5-42.5 GHz (space-to-Earth), 47.2-50.2 GHz (Earth-to-space) and 50.4-51.4 GHz (Earth-to-space), in accordance with Resolution 159 (WRC-15) | ATU Administrations proposes to consider support for the ongoing studies in ITU-R Working Party 4A. No preliminary Draft Proposals at this point. |
| 1.7 | Telemetry, tracking and command in the space operation service for non-GSO satellites with short duration missions | No Preliminary Position |
| 1.13 | to consider identification of frequency bands for the future development of International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution 238 (WRC-15) | A number of administrations gave their preferred Frequency bands but no Prioritization of the bands at this early stage. Possible prioritization to after sufficient studies and assessment has been carried out. |
| 1.14 | to consider, on the basis of ITU-R studies in accordance with Resolution 160 (WRC-15), appropriate regulatory actions for high-altitude platform stations (HAPS), within existing fixed-service allocations | studies should consider protection of incumbent services and systems of fixed and mobile services, this protection has been implemented in previous bands identified for HAPS |
| 1.15 | to consider identification of frequency bands for use by administrations for the land-mobile and fixed services applications operating in the frequency range 275-450 GHz, in accordance with Resolution 767 (WRC-15) | No preliminary Position |
| 1.16 | to consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15) | No preliminary Position |
| 9.1.5 | Consideration of the technical and regulatory impacts of referencing Recommendations ITU-R M.1638-1 and ITU-R M.1849-1 in Nos. 5.447F and 5.450A of the Radio Regulations (Resolution 764(WRC-2015)) | No preliminary Position |
| 9.1.6 | Wireless Power Transmission (Resolution 958 (WRC-2015)) | No preliminary position |
| 9.1.9 | Spectrum needs and possible allocation of the frequency band 51.4-52.4 GHz to the fixed-satellite service (Earth-to-space) | At this early stage, the question of which administrations supported the allocation. 52.4% supported, 14.3% did not support and 33.3% were not sure. |