| World Meteorological Organization Commission for Basic Systems Management Group, seventeenth Session Geneva, 27 February - 1 March 2017 | CBS-MG-17/Doc 8.2.1 |
| --- | --- |
|  | Submitted by: Secretariat 24.Jan.2017 DRAFT 2 |

ICAO REQUIREMENTS FOR XML DATA REPRESENTATION

### SUMMARY

ICAO establishes the requirements for meteorological information to be provided to the international civil aviation community. WMO assists ICAO in this connection through the establishment of meteorological technical methods and practices to fulfil the aeronautical requirements. This is achieved through ICAO Annex 3 (which is reproduced as WMO-No 49 Vol 2). ICAO is moving to “digital exchange” of information that requires the use of XML/GML for the representation of information. CBS has already developed two versions of this (called IWXXM) , but the next amendment (Amendment 78) to Annex 3 is expected to introduce requirements for the exchange of information supporting space weather activities that will require additions to IWXXM to be developed by IPET-DD.

### DECISIONS/ACTIONS REQUIRED:

1. Request OPAG ISS to use the “between sessions” approval procedure for the amendments to IWXXM that had been requested by ICAO.

ANNEX:

- ICAO Template Defining Space Weather Information to be Exchanged

# __________

DECISIONS/ACTIONS REQUIRED

CBS-MG is invited to note the additional requirements from the International Civil Aviation Organization (ICAO) for the representation of information in XML/GML that will require modifications of the IWXXM standard.

Background notes:
a) The ICAO Meteorology Panel (METP) has recommended to the Air Navigation Commission (its parent technical body) that, for changes that require modification of the IWXXM, a standing working practice be introduced requiring that the applicability date should not be sooner than eighteen months following publication of the amendment to Annex 3 (METP/2 Recommendation 5/4 refers);
b) The recommendation at a) will not be confirmed until endorsed by the Air Navigation Commission in March 2017;
c) This information is taken from ICAO AN-WP/9098;
d) Resolution 21 (Cg-17) authorized the use of the “between sessions” for amendments to the Manual on Codes that were consequent on amendments to WMO No. 49 Volume 2. Recommendation 25 (CBS-16) asked for permission to apply the Fast Track procedure to amendments to the Manual on Codes.

Requirements

- Space Weather Advisories

  - ICAO has described the information to be exchanged using the table in the Annex.

  - “Plain language” (Traditional Alphanumeric Code) versions may be used until 5 November 2010, but will need to be introduced.

  - Use of the XML/GML form is optional from the date of implementation of the amendment until 5 November 2020, on which date it becomes mandatory.

- SIGMETs in relation to release of radioactive material

  - Introduce a vertical cylinder as a permitted volume of release, with a default of radius of 30km if no further information is available.

- Introduce a status indicator into SIGMET and AIRMET information

  - Introduce “TEST” and “EXERCISE” indicators for SIGMET and AIRMET messages to allow such messages to be distinguished from operational messages;

  - Note that the indicators would be applied to text versions of the messages as well as to IWXXM versions.

Decision 8.2(1)/1.	CBS-MG noted that Resolution 21 (Cg-18) had authorized the use of the between sessions procedure for amendments to the Manual on Codes that resulted from amendments to WMO-No. 49 Volume 2. It further noted that decided that the amendments to IWXXM resulting from the request from ICAO to represent information on space weather and to amend the information content of SIGMET reports were likely to be too complex to permit a “fast track” procedure. It therefore requested OPAG ISS to use the “between sessions” procedure to implement the amendments to IWXXM that had been requested by ICAO.

__________

Annex 1

# Annex – ICAO template defining space weather information to be exchanged

(Note- the text from this point onwards has been extracted from a PDF and so the typesetting is erratic).

Insert new Table A2-3 as follows:

### Table A2-3.  Template for advisory message for space weather information

Key

:	M = inclusion mandatory, part of every message O = inclusion optional

Note  1.—  The  explanations  for  the  abbreviations  can  be  found  in  the  Procedures  for  Air Navigation Services — ICAO Abbreviations and Codes (PANS-ABC, Doc 8400).

2

TEST or

EXERCISE

indicator (C)*

(C)*

Note 2.— The spatial resolutions are shown in Attachment E.

| Element |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- |
| 1 | Identification of the type of message (M) | Type of message | SWX ADVISORY | SWX ADVISORY |
|  |  | TEST or EXERCISE indicator (C)* | Indicator of TEST or EXERCISE | TEST or EXERCISE |
| 3 | Time of origin (M) | Year, month, day, time in UTC |  |  |
| 4 | Name of centre (M) | Name of SWXC |  |  |
| 5 | Advisory number (M) | Number with year in full and unique message number |  |  |
| 6 | Space weather effect and intensity | Kind of effect and intensity from the space weather event (HF communication, GNSS navigation and surveillance, radiation level exposure environment) | SWX EFFECT: HF COM MOD or SEV, or GNSS MOD or SEV, or HF COM MOD or SEV AND GNSS MOD or SEV, or RADIATION1 MOD or SEV | SWX EFFECT: HF COM MOD GNSS SEV HF COM MOD AND GNSS MOD RADIATION MOD |

| Element |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- |
| 7 | Observed or expected extent of space weather event (M) | Specify time: year, month, day, time in UTC (time T) Observed (or forecast if event has yet to occur) space weather horizontal extent (latitude bands and longitude in degrees) and/or altitude at time T. | OBS or FCST SWX: nnnnnnnn/nnnnZ DAYLIGHT SIDE and/or HNH and/or MNH and/or EQN and/or EQS and/or MSH and/or HSH Wnnn(nn) or Ennn(nn) – Wnnn(nn) or Ennn(nn) and/or ABV FLnnn or FLnnn–nnn or Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or NO SWX EXP | OBS SWX: FCST SWX: 20161108/0100Z DAYLIGHT SIDE HNH HSH HNH MNH MSH HSH EQN EQS W18000 – W09000 ABV FL350 S3000 E09000 – S3000 E18000 – S4000 E18000 – S4000 E09000 NO SWX EXP |
| 8 | Forecast for the | Day and time (in | FCST SWX +6 HR: | FCST SWX +6 HR: |
|  | next 6 hours (M) | UTC) (6 hours from |  |  |
|  |  | time given in item 6, | nn/nnnnZ | 20161108/0700Z |
|  |  | rounded to the next |  |  |
|  |  | full hour) | DAYLIGHT SIDE | DAYLIGHT SIDE |
|  |  | Forecast extent | and/or | HNH |
|  |  | and/or altitude for | HNH | HSH |
|  |  | the fixed valid time. | and/or |  |
|  |  |  | MNH | HNH |
|  |  |  | and/or | MNH |
|  |  |  | EQN | MSH |
|  |  |  | and/or | HSH |
|  |  |  | EQS |  |
|  |  |  | and/or | EQN |
|  |  |  | MSH | EQS |
|  |  |  | and/or |  |
|  |  |  | HSH | W09000 – W00000 |
|  |  |  | and/or | ABV FL350 |
|  |  |  | Wnnn(nn) or Ennn(nn) – | S3000 E09000 – S3000 E18000 – S4000 |
|  |  |  | Wnnn(nn) or Ennn(nn) | E18000 – S4000 E09000 |
|  |  |  | and/or | NO SWX EXP |
|  |  |  | ABV FLnnn | NOT AVBL |
|  |  |  | or FLnnn–nnn |  |
|  |  |  | or |  |

| Element |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or NO SWX EXP or NOT AVBL |  |
| 9 | Forecast for the | Day and time (in | FCST SWX +12 HR: | FCST SWX +12 HR: |
|  | next 12 hours | UTC) (12 hours from |  |  |
|  | (M) | time of onset given | nn/nnnnZ | 20161108/1300Z |
|  |  | in item 6, rounded to |  |  |
|  |  | the next full hour) | DAYLIGHT SIDE | DAYLIGHT SIDE |
|  |  | Forecast extent | or | HNH |
|  |  | and/or altitude for |  | HSH |
|  |  | the fixed valid time. | HNH |  |
|  |  |  | and/or | HNH |
|  |  |  | MNH | MNH |
|  |  |  | and/or | MSH |
|  |  |  | EQN | HSH |
|  |  |  | and/or |  |
|  |  |  | EQS | EQN |
|  |  |  | and/or | EQS |
|  |  |  | MSH |  |
|  |  |  | and/or | E00000 – E09000 |
|  |  |  | HSH |  |
|  |  |  |  | ABV FL350 |
|  |  |  | and/or |  |
|  |  |  |  | S3000 E09000 – S3000 E18000 – S4000 |
|  |  |  | Wnnn(nn) or Ennn(nn) – | E18000 – S4000 E09000 |
|  |  |  | Wnnn(nn) or Ennn(nn) |  |
|  |  |  |  | NO SWX EXP |
|  |  |  | and/or |  |
|  |  |  |  | NOT AVBL |
|  |  |  | ABV FLnnn |  |
|  |  |  | or FLnnn–nnn |  |
|  |  |  | or |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |
|  |  |  | [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] |  |
|  |  |  | or |  |
|  |  |  | NO SWX EXP |  |
|  |  |  | or |  |
|  |  |  | NOT AVBL |  |
| 10 | Forecast for the next 18 hours (M) | Day and time (in UTC) (18 hours from time of onset given in item 6, rounded to the next full hour) Forecast extent and/or altitude for the fixed valid time. | FCST SWX +18 HR: nn/nnnnZ DAYLIGHT SIDE or HNH and/or | FCST SWX +18 HR: 20161108/1900Z DAYLIGHT SIDE HNH HSH HNH |

| Element |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- |
|  |  |  | MNH and/or EQN and/or EQS and/or MSH and/or HSH and/or Wnnn(nn) or Ennn(nn) – Wnnn(nn) or Ennn(nn) and/or ABV FLnnn or FLnnn–nnn or Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or NO SWX EXP or NOT AVBL | MNH MSH HSH EQN EQS E09000 – E18000 ABV FL350 S3000 E09000 – S3000 E18000 – S4000 E18000 – S4000 E09000 NO SWX EXP NOT AVBL |
| 11 | Forecast for the | Day and time (in | FCST SWX +24 HR: | FCST SWX +24 HR: |
|  | next 24 hours | UTC) (24 hours from |  |  |
|  | (M) | time of onset given | nn/nnnnZ | 20161109/0100Z |
|  |  | in item 6, rounded to |  |  |
|  |  | the next full hour) | DAYLIGHT SIDE | DAYLIGHT SIDE |
|  |  | Forecast extent | or | HNH |
|  |  | and/or altitude for |  | HSH |
|  |  | the fixed valid time. | HNH |  |
|  |  |  | and/or | HNH |
|  |  |  | MNH | MNH |
|  |  |  | and/or | MSH |
|  |  |  | EQN | HSH |
|  |  |  | and/or |  |
|  |  |  | EQS | EQN |
|  |  |  | and/or | EQS |
|  |  |  | MSH |  |
|  |  |  | and/or | W18000 – W09000 |
|  |  |  | HSH |  |
|  |  |  |  | ABV FL350 |
|  |  |  | and/or |  |
|  |  |  |  | S3000 E09000 – S3000 E18000 – S4000 |
|  |  |  | Wnnn(nn) or Ennn(nn) – | E18000 – S4000 E09000 |
|  |  |  | Wnnn(nn) or Ennn(nn) |  |
|  |  |  |  | NO SWX EXP |
|  |  |  | and/or |  |
|  |  |  |  | NOT AVBL |
|  |  |  | ABV FLnnn |  |
|  |  |  | or FLnnn–nnn |  |
|  |  |  | or |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |
|  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |

M)

| Element |  |  |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or NO SWX EXP or NOT AVBL |  |
| 12 | Remarks ( | M) |  | Remarks, as necessary. | RMK: Free text up to 256 characters. | RMK: SWX EVENT HAS CEASED RMK: WWW.SPACEWEATHERPROVIDER.GOV RMK: NIL |
| 13 | Next advisory (M) |  |  | Year, month, day, time in UTC. | NXT ADVISORY: nnnnnnnn/nnnnZ or Free text up to XX (TBD) characters or NO FURTHER ADVISORIES | NXT ADVISORY: 20161108/0700Z. NO FURTHER ADVISORIES |

*	Use only when the message issued is a TEST or EXERCISE and is not to be used for operational decision-making. When TEST or EXERCISE is indicated, the message may contain information (not to be used operationally) or will otherwise end immediately after the word "TEST".

Example A2-3: Space weather advisory message (GNSS and HF COM effects)

| (communication header) |  |
| --- | --- |
| SWX ADVISORY |  |
| DTG: | 20161108/0100Z |
| SWXC: | (to be determined) |
| SWX EFFECT: | GNSS MOD AND HF COM MOD |
| ADVISORY NR: | 2016/1 |
| OBS SWX: | 20161108/0100Z HNH HSH E18000 – W18000 |
| FCST SWX +6 HR: | 20121108/0700Z HNH HSH E18000 – W18000 |
| FCST SWX +12 HR: | 20161108/1300Z HNH HSH E18000 – W18000 |
| FCST SWX +18 HR: | 20161108/1900Z HNH HSH E18000 – W18000 |
| FCST SWX +24 HR: | 20161109/0100Z NO SWX EXP |
| RMK: | LOW-LEVEL GEOMAGNETIC STORMING IS CAUSING INCREASED AURORAL ACTIVITY AND SUBSEQUENT MOD DEGRADATION OF GNSS ACCURACY AND HF COM AVAILABILITY IN THE AURORAL ZONE. THIS STORMING IS EXPECTED TO SUBSIDE IN THE FORECAST PERIOD. SEE WWW.SPACEWEATHERPROVIDER.WEB |
| NXT ADVISORY: | NO FURTHER ADVISORIES |

Example A2-4: Space weather advisory message (RADIATION effects)

| (communication header) |  |
| --- | --- |
| SWX ADVISORY |  |
| DTG: | 20161108/0000Z |
| SWXC: | (to be determined) |
| SWX EFFECT: | RADIATION MOD |
| ADVISORY NR: | 2016/2 |

| FCST SWX: | 20161108/0100Z HNH HSH E18000 – W18000 ABV FL350 |
| --- | --- |
| FCST SWX +6 HR: | 20121108/0700Z HNH HSH E18000 – W18000 ABV FL350 |
| FCST SWX +12 HR: | 20161108/1300Z HNH HSH E18000 – W18000 ABV FL350 |
| FCST SWX +18 HR: | 20161108/1900Z HNH HSH E18000 – W18000 ABV FL350 |
| FCST SWX +24 HR: | 20161109/0100Z NO SWX EXP |
| RMK: | RADIATION LEVELS HAVE EXCEEDED 100 PERCENT OF BACKGROUND LEVELS AT FL350 AND ABOVE. THE CURRENT EVENT HAS PEAKED AND LEVELS ARE SLOWLY RETURNING TO BACKGROUND LEVELS. SEE WWW.SPACEWEATHERPROVIDER.WEB |
| NXT ADVISORY: | NO FURTHER ADVISORIES |

Example A2-5: Space weather advisory message (HF COM effects)

| (communication header) |  |
| --- | --- |
| SWX ADVISORY |  |
| DTG: | 20161108/0100Z |
| SWXC: | (to be determined) |
| SWX EFFECT: | HF COM SEV |
| ADVISORY NR: | 2016/1 |
| OBS SWX: | 20161108/0100Z DAYLIGHT SIDE |
| FCST SWX +6 HR: | 20121108/0700Z DAYLIGHT SIDE |
| FCST SWX +12 HR: | 20161108/1300Z DAYLIGHT SIDE |
| FCST SWX +18 HR: | 20161108/1900Z DAYLIGHT SIDE |
| FCST SWX +24 HR: | 20161109/0100Z DAYLIGHT SIDE |
| RMK: | PERIODIC HF COM ABSORPTION HAS BEEN OBSERVED AND IS LIKELY TO CONTINUE IN THE NEAR TERM. COMPLETE AND PERIODIC LOSS OF HF ON THE SUNLIT SIDE OF THE EARTH EXPECTED. CONTINUED HF COM DEGRADATION LIKELY OVER THE NEXT 7 DAYS. SEE WWW.SPACEWEATHERPROVIDER.WEB |
| NXT ADVISORY: | 20161108/0700Z |

End of new Table A2-3.

### Table A2-2.	Template for advisory message for tropical cyclones

Key:	=	=	a double line indicates that the text following it should be placed on the subsequent line.

Note 1.— The ranges and resolutions for the numerical elements included in advisory messages for tropical cyclones are shown in Appendix 6, Table A6-4.

Note 2.— The explanations for the abbreviations can be found in the PANS-ABC (Doc 8400).

Note 3.— All the elements are mandatory.

| or |  |  |  |
| --- | --- | --- | --- |
| WI4 Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |
| Ennn[nn] – |  |  |  |
| Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |
| Ennn[nn] – |  |  |  |
| Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |
| Ennn[nn] – |  |  |  |
| [Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |
| Ennn[nn] – |  |  |  |
| Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |
| Ennn[nn]] |  |  |  |

Note 43.— Inclusion of a “colon” after each element heading is mandatory.

| TEST or |  |  |
| --- | --- | --- |
| EXERCISE |  |  |
| indicator (C)1 |  |  |

Note 54.— The numbers 1 to 1921 are included only for clarity and they are not part of the advisory message, as shown in the example.

| Element |  | Detailed content | Template(s) | Examples |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Identification of the type of message | Type of message | TC ADVISORY | TC ADVISORY |  |  |
| 2 |  | Indicator of TEST or EXERCISE |  |  |  |  |
|  |  |  | TEST TEST or EXERCISE EXERCISE | TEST or EXERCISE |  | TEST or EXERCISE |
|  |  |  |  | indicator (C)1 | indicator (C)1 |  |
| 23 | Time of origin | Year, month, day and time in UTC of issue | DTG: nnnnnnnn/nnnnZ | DTG: 20040925/19600Z |  |  |
| 34 | Name of TCAC | Name of TCAC (location indicator or full name) | TCAC: nnnn or nnnnnnnnnn | TCAC: YUFO12 TCAC: MIAMI |  |  |
| 45 | Name of tropical cyclone | Name of tropical cyclone or “NN” for unnamed tropical cyclone | TC: nnnnnnnnnnnn or NN | TC: GLORIA |  |  |
| 56 | Advisory number | Advisory number: Year in full and message number (separate sequence starting with “0 1” for each cyclone) | ADVISORY NR: nnnn/[n][n]nn | ADVISORY NR: 2004/1301 |  |  |
| 67 | Observed pPosition of the centre | Day and time (in UTC) and pPosition of the centre of the tropical cyclone (in degrees and minutes) | OBS PSN: nn/nnnnZ Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | OBS PSN: 25/1800Z N2706 W07306 |  |  |
| 8 | Observed CB cloud3 | Location of CB cloud (referring to latitude and longitude (in degrees and minutes)) and vertical extent (flight level) | CB: WI nnnKM (or nnnNM) OF TC CENTRE TOP [ABV or BLW] FLnnn | CB: WI 250NM OF TC CENTRE TOP FL500 |  |  |
| 79 | Direction and speed of movement | Direction and speed of movement given in sixteen compass points and km/h (or kt), respectively, or moving slowly (< 6 km/h (3 kt)) or stationary (< 2 km/h (1 kt)) | MOV: N nnKMH (or KT) or NNE nnKMH (or KT) or NE nnKMH (or KT) or ENE nnKMH (or KT) or E nnKMH (or KT) or ESE nnKMH (or KT) or SE nnKMH (or KT) or SSE nnKMH (or KT) or S nnKMH (or KT) or SSW nnKMH (or KT) or SW nnKMH (or KT) or WSW nnKMH (or KT) or W nnKMH (or KT) or | MOV: NW 20KMH |  |  |

| Element |  |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | WNW nnKMH (or KT) or NW nnKMH (or KT) or NNW nnKMH (or KT) or SLW or STNR |  |
| 8 | 10 | Central pressure | Central pressure (in hPa) | C: nnnHPA | C: 965HPA |
| 9 | 11 | Maximum surface wind | Maximum surface wind near the centre (mean over 10 minutes, in m/s (or kt)) | MAX WIND: nn[n]MPS (or nn[n]KT) | MAX WIND: 22MPS |
| 1012 |  | Forecast of centre position (+6 HR) | Day and time (in UTC) (6 hours from the “DTG” given in Item 6); Forecast position (in degrees and minutes) of the centre of the tropical cyclone | FCST PSN +6 HR: nn/nnnnZ Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | FCST PSN +6 HR: 26/0000Z N2748 W07350 |
| 1113 |  | Forecast of maximum surface wind (+6 HR) | Forecast of maximum surface wind (6 hours after the “DTG” given in Item 6) | FCST MAX nn[n]MPS WIND +6 HR: (or nn[n]KT) | FCST MAX 22MPS WIND +6 HR: |
| 1214 |  | Forecast of centre position (+12 HR) | Day and time (in UTC) (12 hours from the “DTG” given in Item 6); Forecast position (in degrees and minutes) of the centre of the tropical cyclone | FCST PSN +12 HR: nn/nnnnZ Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | FCST PSN +12 HR: 26/0400Z N2830 W07430 |
| 1315 |  | Forecast of maximum surface wind (+12 HR) | Forecast of maximum surface wind (12 hours after the “DTG” given in Item 6) | FCST MAX WIND nn[n]MPS +12 HR: (or nn[n]KT) | FCST MAX WIND 22MPS +12 HR: |
|  |  | Forecast of centre position (+18 HR) | Day and time (in UTC) (18 hours from the “DTG” given in Item 6); Forecast position (in degrees and minutes) of the centre of the tropical cyclone | FCST PSN +18 HR: nn/nnnnZ Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | FCST PSN +18 HR: 26/1000Z N2852 W07500 |
| 1416 |  |  |  |  |  |
|  |  | Forecast of maximum surface wind (+18 HR) | Forecast of maximum surface wind (18 hours after the “DTG” given in Item 6) | FCST MAX WIND nn[n]MPS +18 HR: (or nn[n]KT) | FCST MAX WIND 21MPS +18 HR: |
| 1517 |  |  |  |  |  |
|  |  | Forecast of centre position (+24 HR) | Day and time (in UTC) (24 hours from the “DTG” given in Item 6); Forecast position (in degrees and minutes) of the centre of the tropical cyclone | FCST PSN +24 HR: nn/nnnnZ Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | FCST PSN +24 HR: 26/1600Z N2912 W07530 |
| 1618 |  |  |  |  |  |
|  |  | Forecast of maximum surface wind (+24 HR) | Forecast of maximum surface wind (24 hours after the “DTG” given in Item 6) | FCST MAX WIND nn[n]MPS +24 HR: (or nn[n]KT) | FCST MAX WIND 20MPS +24 HR: |
| 1719 |  |  |  |  |  |
|  |  | Remarks | Remarks, as necessary | RMK: Free text up to 256 characters or NIL | RMK: NIL |
| 1820 |  |  |  |  |  |

| Element |  |  | Detailed content | Template(s) | Examples |
| --- | --- | --- | --- | --- | --- |
| 192 | 1 | Expected time of issuance of next advisory | Expected year, month, day and time (in UTC) of issuance of next advisory | NXT MSG: [BFR] nnnnnnnn/nnnnZ or NO MSG EXP | NXT MSG: 20040925/2000Z |

1.	Use only when the message issued is a TEST or EXERCISE and is not to be used for operational decision-making. When TEST is indicated, the message may contain information (not to be used operationally) or will otherwise end immediately after the word "TEST".

Notes.—

| 1.2 | . Fictitious location. |  |
| --- | --- | --- |
| 3. Optional field |  | . |
| 4. The number of coordinates should be kept to a minimum and should not normally exceed seven. |  |  |

Table A6-1A.	Template for SIGMET and AIRMET messages

Key:	M	=	inclusion mandatory, part of every message;

C	=	inclusion conditional, included whenever applicable;

=	=	a double line indicates that the text following it should be placed on the

subsequent line.

Note 1.— The ranges and resolutions for the numerical elements included in SIGMET/AIRMET messages are shown in Table A6-4 of this appendix.

Note 2.— In accordance with 1.1.5 and 2.1.5, severe or moderate icing and severe or moderate turbulence (SEV ICE, MOD ICE, SEV TURB, MOD TURB) associated with  thunderstorms, cumulonimbus clouds or tropical cyclones should not be included.

| Element | Detailed content | SIGMET template | AIRMET template | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- |
| Location indicator of FIR/CTA (M)1 | ICAO location indicator of the ATS unit serving the FIR or CTA to which the SIGMET/AIRMET refers | nnnn |  | YUCC2 YUDD2 |  |
| Identification (M) | Message identification and sequence number3 | SIGMET [n][n]n | AIRMET [n][n]n | SIGMET 1 SIGMET 01 SIGMET A01 | AIRMET 9 AIRMET 19 AIRMET B19 |
| Validity period (M) | Day-time groups indicating the period of validity in UTC | VALID nnnnnn/nnnnnn |  | VALID 010000/010400 VALID 221215/221600 VALID 101520/101800 VALID 251600/252200 VALID 152000/160000 VALID 192300/200300 |  |
| Location indicator of MWO (M) | Location indicator of MWO originating the message with a separating hyphen | nnnn– |  | YUDO–2 YUSO–2 |  |
| Name of the FIR/CTA (M) | Location indicator and name of the FIR/CTA4 for which the SIGMET/AIRMET is issued | nnnn nnnnnnnnnn FIR[/UIR] or UIR or FIR/UIR or nnnn nnnnnnnnnn CTA | nnnn nnnnnnnnnn FIR[/n] | YUCC AMSWELL FIR2 YUDD SHANLON2 FIR/UIR2 UIR FIR/UIR YUDD SHANLON CTA2 | YUCC AMSWELL FIR/22 YUDD SHANLON FIR2 |
| IF THE SIGMET OR AIRMET MESSAGE IS TO BE CANCELLED, SEE DETAILS AT THE END OF THE TEMPLATE. |  |  |  |  |  |
| TEST or EXERCISE indicator (C)5 | Indicator of TEST or EXERCISE | TEST or EXERCISE | TEST or EXERCISE | TEST EXERCISE | TEST EXERCISE |
| Phenomenon (M)56 | Description of phenomenon causing the issuance of SIGMET/AIRMET | OBSC67 TS[GR78] EMBD89 TS[GR78] FRQ910 TS[GR78] SQL1011 TS[GR78] TC nnnnnnnnnn PSN Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] CB or TC NN1112 PSN Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] CB SEV TURB1213 SEV ICE1314 SEV ICE (FZRA)1314 SEV MTW1415 HVY DS HVY SS [VA ERUPTION] [MT nnnnnnnnnn] [PSN Nnn[nn] or Snn[nn] Ennn[nn] or Wnnn[nn]] | SFC WIND nnn/nn[n]MPS (or SFC WIND nnn/nn[n]KT) SFC VIS nnnnM (nn)1516 ISOL1617 TS[GR78] OCNL1718 TS[GR78] MT OBSC BKN CLD nnn/[ABV] nnnnM (or BKN CLD [n]nnn/[ABV] [n]nnnnFT) or BKN CLD SFC/[ABV] nnnnM (or BKN CLD SFC/[ABV][n]nnnnFT) OVC CLD nnn/[ABV]nnnnM (or OVC CLD [n]nnn/[ABV] [n]nnnnFT) | OBSC TS OBSC TSGR EMBD TS EMBD TSGR FRQ TS FRQ TSGR SQL TS SQL TSGR TC GLORIA PSN N10 W060 CB TC NN PSN S2030 E06030 CB SEV TURB SEV ICE SEV ICE (FZRA) SEV MTW HVY DS HV Y SS VA ERUPTION MT ASHVAL2 PSN S15 E073 | SFC WIND 040/40MPS SFC WIND 310/20KT SFC VIS 1500M (BR) ISOL TS ISOL TSGR OCNL TS OCNL TSGR MT OBSC BKN CLD 120/900M BKN CLD 400/3000FT BKN CLD 1000/5000FT BKN CLD SFC/3000M BKN CLD SFC/ABV 10000FT OVC CLD 270/ABV3000M OVC CLD 900/ABV10000FT OVC CLD 1000/5000FT OVC CLD SFC/3000M |

5

| Element | Detailed content | SIGMET template | AIRMET template |  | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VA CLD RDOACT CLD | or OVC CLD SFC/[ABV]nnnnM (or OVC CLD SFC/[ABV][n]nnnnFT) ISOL1617 CB1819 OCNL1718 CB1819 FRQ910 CB1819 ISOL1617 TCU1819 OCNL1718 TCU1819 FRQ910 TCU1819 MOD TURB1213 MOD ICE1314 |  | VA CLD RDOACT CLD | OVC CLD SFC/ABV 10000FT ISOL CB OCNL CB FRQ CB ISOL TCU OCNL TCU FRQ TCU MOD TURB MOD ICE MOD MTW |
|  |  |  | MOD MTW141 |  |  |  |
| Observed or forecast phenomenon (M) | Indication whether the information is observed and expected to continue, or forecast | OBS [AT nnnnZ] or FCST [AT nnnnZ] |  |  | OBS OB S AT 1210Z FCST FCST AT 1815Z |  |
| Location (C)1920 | Location (referring to | Nnn[nn] Wnnn[nn] or Nnn[nn] Ennn[nn] or |  |  | N2020 W07005 |  |
|  | latitude and longitude (in | Snn[nn] Wnnn[nn] or Snn[nn] Ennn[nn] |  |  | N48 E010 |  |
|  | degrees and minutes)) |  |  |  | S60 W160 |  |
|  |  | or |  |  | S0530 E16530 |  |
|  |  | N OF Nnn[nn] or S OF Nnn[nn] or N OF Snn[nn] or |  |  |  |  |
|  |  | S OF Snn[nn] [AND] |  |  | N OF N50 |  |
|  |  | W OF Wnnn[nn] or E OF Wnnn[nn] or W OF Ennn[nn] or |  |  | S OF N5430 |  |
|  |  | E OF Ennn[nn] |  |  | N OF S10 |  |
|  |  |  |  |  | S OF S4530 |  |
|  |  | or |  |  | W OF W155 |  |
|  |  | N OF Nnn[nn] or N OF Snn[nn] AND S OF Nnn[nn] or |  |  | E OF W45 |  |
|  |  | S OF Snn[nn] |  |  | W OF E15540 |  |
|  |  |  |  |  | E OF E09015 |  |
|  |  |  |  |  | N OF N1515 AND W OF E13530 |  |
|  |  | or |  |  | S OF N45 AND N OF N40 |  |
|  |  | W OF Wnnn[nn] or W OF Ennn[nn] AND |  |  |  |  |
|  |  | E OF Wnnn[nn] or E OF Ennn[nn] |  |  | N OF LINE S2520 W11510 – S2520 W12010 |  |
|  |  |  |  |  | SW OF LINE N50 W005 – N60 W020 |  |
|  |  | or |  |  | SW OF LINE N50 W020 – N45 E010 AND NE OF LINE |  |
|  |  | N OF LINE2021 or NE OF LINE2021 or E OF LINE2021 or SE |  |  | N45 W020 – N40 E010 |  |
|  |  | OF LINE2021 or S OF LINE2021 or SW OF LINE2021 or W OF |  |  |  |  |
|  |  | LINE2021 or NW OF LINE2021 Nnn[nn] or Snn[nn] Wnnn[nn] |  |  | WI N6030 E02550 – N6055 E02500 – |  |
|  |  | or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] |  |  | N6050 E02630 – N6030 E02550 |  |
|  |  | [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] [– Nnn[nn] |  |  |  |  |
|  |  | or Snn[nn] Wnnn[nn] or Ennn[nn]] |  |  | APRX 50KM WID LINE BTN N64 W017 – N60 W010 – |  |
|  |  | [AND N OF LINE2021 or NE OF LINE2021 or E OF LINE2021 |  |  | N57 E010 |  |
|  |  | or SE OF LINE2021 or S OF LINE2021 or SW OF LINE2021 or |  |  |  |  |
|  |  | W OF LINE2021 or NW OF LINE2021 Nnn[nn] or Snn[nn] |  |  |  |  |
|  |  | Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |  |
|  |  | Ennn[nn] |  |  |  |  |
|  |  | [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] [– Nnn[nn] |  |  |  |  |
|  |  | or Snn[nn] Wnnn[nn] or Ennn[nn]]] |  |  |  |  |
|  |  | or WI2021, 2122 Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – [Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – |  |  | ENTIRE FIR ENTIRE UIR ENTIRE FIR/UIR ENTIRE CTA |  |

| ABV 7000FT |  |  |
| --- | --- | --- |
| TOP ABV 9000FT |  |  |

| Element | Detailed content | SIGMET template |  |  | AIRMET template |  | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] or APRX nnKM WID LINE2021 BTN (or nnNM WID LINE2021 BTN) Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] or ENTIRE FIR[/UIR] or ENTIRE FIR or ENTIRE FIR/UIR or ENTIRE CTA or2223 WI nnnKM (or nnnNM) OF TC CENTRE |  |  |  |  | WI 400KM OF TC CENTRE WI 250NM OF TC CENTRE WI 30KM OF N6030 E02550 |  |
|  |  | or29 |  |  |  |  |  |  |
|  |  | WI nnnNM or nnnKM OF Nnn[nn] or Snn[nn] Wnnn[nn] or |  |  |  |  |  |  |
|  |  | Ennn[nn] |  |  |  |  |  |  |
| Level (C) 1920 | Flight level or altitude23 | [SFC/]FLnnn or [SFC/]nnnnM (or [SFC/][n]nnnnFT) or FLnnn/nnn or TOP FLnnn or [TOP] ABV FLnnn or (or [TOP] ABV [n]nnnnFT) [nnnn/]nnnnM (or [[n]nnnn/][n]nnnnFT) or [nnnnM/]FLnnn (or [[n]nnnnFT/]FLnnn) or 2223 TOP [ABV or BLW] FLnnn |  |  |  |  | FL180 SFC/FL070 SFC/3000M SFC/10000FT FL050/080 TOP FL390 ABV FL250 TO P ABV FL100 TOP ABV 10000FT 3000M 2000/3000M 8000FT 6000/12000FT 2000M/FL150 10000FT/FL250 TOP FL500 TOP ABV FL500 TOP BLW FL450 |  |
| Movement or expected movement (C) 1920, 24 | Movement or expected movement (direction and speed) with reference to one of the sixteen points of compass, or stationary | MOV N [nnKMH] or MOV NNE [nnKMH] or MOV NE [nnKMH] or MOV ENE [nnKMH] or MOV E [nnKMH] or MOV ESE [nnKMH] or MOV SE [nnKMH] or MOV SSE [nnKMH] or MOV S [nnKMH] or MOV SSW [nnKMH] or MOV SW [nnKMH] or MOV WSW [nnKMH] or MOV W [nnKMH] or MOV WNW [nnKMH] or MOV NW [nnKMH] or MOV NNW [nnKMH] (or MOV N [nnKT] or MOV NNE [nnKT] or MOV NE [nnKT] or MOV ENE [nnKT] or MOV E [nnKT] or MOV ESE [nnKT] or MOV SE [nnKT] or MOV SSE [nnKT] or MOV S [nnKT] or MOV SSW [nnKT] or |  |  |  |  | MOV SE MOV NNW MOV E 40KMH MOV E 20KT MOV WSW 20KT STNR |  |

| Element | Detailed content |  |  |  | SIGMET template | AIRMET template | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MOV SW [nnKT] or MOV WSW [nnKT] or MOV W [nnKT] or MOV WNW [nnKT] or MOV NW [nnKT] or MOV NNW [nnKT]) or STNR |  |  |  |
| Changes in intensity (C) 1920 | Expected changes in intensity |  |  |  | INTSF or WKN or NC |  | INTSF WKN NC |  |
| Forecast time (C)24 | Indication of the forecast time of phenomenon |  |  |  | FCST AT nnnnZ | — | FCST AT 2200Z | — |
| TC forecast position (C)22 |  |  |  |  | TC CENTRE PSN Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] | — | TC CENTRE PSN N1030 or E1600015 | TC forecast position (C)22 |
|  | Forecast position of TC centre at the end of the |  |  |  |  |  |  |  |
|  | validity period of the |  |  |  |  |  |  |  |
|  | SIGMET message |  |  |  |  |  |  |  |
| Forecast position (C) 1920, 24, 25 | Forecast position of phenomenon at the end of the validity period of the SIGMET message |  |  |  | Nnn[nn] Wnnn[nn] or Nnn[nn] Ennn[nn] or Snn[nn] Wnnn[nn] or Snn[nn] Ennn[nn] or N OF Nnn[nn] or S OF Nnn[nn] or N OF Snn[nn] or S OF Snn[nn] [AND] W OF Wnnn[nn] or E OF Wnnn[nn] or W OF Ennn[nn] or E OF Ennn[nn] or N OF Nnn[nn] or N OF Snn[nn] AND S OF Nnn[nn] or S OF Snn[nn] or W OF Wnnn[nn] or W OF Ennn[nn] AND E OF Wnnn[nn] or E OF Ennn[nn] or N OF LINE2021 or NE OF LINE2021 or E OF LINE2021 or SE OF LINE2021 or S OF LINE2021 or SW OF LINE2021 or W OF LINE2021 or NW OF LINE2021 Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] | — | N30 W170 N OF N30 S OF S50 AND W OF E170 S OF N46 AND N OF N39 NE OF LINE N35 W020 – N45 W040 SW OF LINE N48 W020 – N43 E010 AND NE OF LINE N43 W020 – N38 E010 WI N20 W090 – N05 W090 – N10 W100 – N20 W100 – N20 W090 APRX 50KM WID LINE BTN N64 W017 – N57 W005 – N55 E010 – N55 E030 ENTIRE FIR ENTIRE UIR ENTIRE FIR/UIR ENTIRE CTA TC CENTRE PSN N2740 W07345 NO VA LONGER EXP | — |

| Element | Detailed content | SIGMET template | AIRMET template | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- |
|  |  | [AND N OF LINE2021 or NE OF LINE2021 or E OF LINE2021 or SE OF LINE2021 or S OF LINE2021 or SW OF LINE2021 or W OF LINE2021 or NW OF LINE2021 Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]]] or WI2021, 2122 Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or APRX nnKM WID LINE2021 BTN (nnNM WID LINE2021 BTN) Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] [– Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] [ – Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn]] or ENTIRE FIR[/UIR] or ENTIRE UIR or ENTIRE FIR/UIR or ENTIRE CTA or22 TC CENTRE PSN Nnn[nn] or Snn[nn] Wnnn[nn] or Ennn[nn] or26 NO VA LONGER EXP |  |  |  |

| Element | Detailed content | SIGMET template | AIRMET template | SIGMET message examples | AIRMET message examples |
| --- | --- | --- | --- | --- | --- |
| Repetition of elements (C)27 | Repetition of elements included in a SIGMET message for volcanic ash cloud or tropical cyclone | [AND]27 | — | AND | — |

OR

| Cancellation of SIGMET/ AIRMET (C)28 | Cancellation of SIGMET/AIRMET referring to its identification | CNL SIGMET [n][n]n nnnnnn/nnnnnn or26 CNL SIGMET [n][n]n nnnnnn/nnnnnn VA MOV TO nnnn FIR | CNL AIRMET [n][n]n nnnnnn/nnnnnn | CNL SIGMET 2 101200/101600 CNL SIGMET A13 251030/251430 VA MOV TO YUDO FIR2 | CNL AIRMET 05 151520/151800 |
| --- | --- | --- | --- | --- | --- |

Notes.—

1.	See 4.1.

- Fictitious location.

- In accordance with 1.1.3 and 2.1.2. 4.	See 2.1.3.

5.   Used only when the message is issued as a TEST or EXERCISE.  Under such circumstances the information contained within the message is not to be used for operational decision-making. When this field is omitted, information contained within the message is intended to be used operationally. When TEST is indicated, the message may contain information (not to be used operationally) or will otherwise end immediately after the word “TEST”.

56.    In accordance with 1.1.4 and 2.1.4.

67.    In accordance with 4.2.1 a).

78.    In accordance with 4.2.4.

89.    In accordance with 4.2.1 b). 910.  In accordance with 4.2.2. 1011. In accordance with 4.2.3.

1112. Used for unnamed tropical cyclones. 1213. In accordance with 4.2.5 and 4.2.6. 1314. In accordance with 4.2.7.

1415. In accordance with 4.2.8. 1516. In accordance with 2.1.4. 1617. In accordance with 4.2.1 c). 1718. In accordance with 4.2.1 d).

1819. The use of cumulonimbus (CB) and towering cumulus (TCU) is restricted to AIRMETs in accordance with 2.1.4.

1920. In the case of volcanic ash cloud or cumulonimbus clouds associated with a tropical cyclone covering more than one area within the FIR, these elements can be repeated, as necessary.

2021. A straight line is to be used between two points drawn on a map in the Mercator projection or between two points which crosses lines of longitude at a constant angle.

2122. The number of coordinates should be kept to a minimum and should not normally exceed seven. 2223. Only for SIGMET messages for tropical cyclones.

- Only for SIGMET messages for volcanic ash cloud and tropical cyclones.

- The elements “forecast time” and “forecast position” are not to be used in conjunction with the element “movement or expected movement”.

- The levels of the phenomena remain fixed throughout the forecast period.

- Only for SIGMET messages for volcanic ash.

- To be used for two volcanic ash clouds or two centres of tropical cyclones simultaneously affecting the FIR concerned..

- End of the message (as the SIGMET/AIRMET message is being cancelled).

When using SIGMET for RDOACT CLD, when detailed information on the release is not available, a radius of up to 30 km may be applied based on the International Atomic Energy Agency (IAEA) recommendation for surface contamination contained in IAEA Safety Guide GS-G-2.1 - Arrangements for Preparedness for a Nuclear or Radiological Emergency (2007); and a vertical extent from surface (SFC) to the upper limit of the flight information region/upper flight information regio

__________