GISC Moscow Status Report - 2018

- GISC Moscow area of responsibility (AoR)

GISC Moscow operates as a principal GISC for 11 NMHSs - Armenia, Azerbaijan, Belarus, Georgia, Kazakhstan, Kyrgyzstan, Moldova, Tajikistan, Turkmenistan, Uzbekistan and Ukraine.

- AMDCN connectivity

All the above mentioned NMHSs are connected to GISC Moscow directly via logical channels (digital channels, Internet network) to transmit data for their area of responsibility directly to GISC Moscow. The connectivity details are given below:

  - NC Yerevan Armenia – Internet, FTP

  - NC Baku Azerbaijan – Internet, Socket-special+http

  - NC Minsk Belarus – Dedicated connection CIR:128kbps (both directions) + Internet /Backup via Internet, Socket-special (512kbps)+FTP

  - NC Tbilisi Georgia – Internet, FTP + http

  - NC Astana Kazakhstan – Internet, FTP (Kazakhstan made efforts for GTS connectivity through the RMDCN in 2018). Kazakhstan nominated a new NC Astana, which became the main NC, and the NC Almaty is a back-up NC.

  - NC Bishkek Kyrgystan  – Internet, Socket-special (512kbps)

  - NC Kishinev Republic of Moldova - Dedicated connection CIR:256 kbps (both directions), Socket-special+FTP_http/Backup via Internet, email

  - NC Dushanbe Tajikistan - Internet, Socket-special(512kbps)

  - NC Ashgabat Turkmenistan - Internet, FTP

  - NC Tashkent Uzbekistan - Dedicated connection CIR:64 kbps (both directions), Socket-special+FTP/Backup via Internet, email

  - NC Ukraine – Internet, FTP

- Compliance between DCs and WIS

GISC Moscow DC (Main Computing Centre of Roshydromet + Hydrometeorological Centre of Russia) is compliant to WIS in terms of data exchange. The DC metadata compliance with WIS will be provided in future.

- Training – capacity building activity

Roshydromet developed the subjects of webinars for the area of responsibility of an NC.

  - The webinar, conducted in Q2/2018, examined some theoretical issues (metadata management in the Moscow Central Control Center, Virtual National Centers in GISC Moscow), including:

    - general information about the remote workplace for accessing and editing NC metadata in GISCs by the operators of NCs within the GISC AMDCN. Organization of the work space of a “Virtual NC” (GISC profile);

    - how to obtain the profile of the work space of an NC specialist;

    - loading of data and creation of metadata.

  - At the next webinar, which is planned to be conducted near the end of 2018, the focus will be placed on the issues of data uploading and metadata generation based on GTS data of the NC area of responsibility, etc.

  - The webinar, scheduled for 2019, will examine the issues of organization and interaction of GISC Moscow and NC/DCPCs in its area of responsibility for implementing the WIS monitoring.

  - As part of the webinars, GISC Moscow compiled a number of training documents on how to search and get information at GISC Moscow as well as on how to establish virtual NCs based on GISC Moscow and manage metadata for GISC virtual centre operators. The training documents in Russian are placed on the web-page of GISC Moscow.

- Operational issues in the past period

GISC Moscow is harvesting the following metadata sets:

WIS-GISC-BEIJING

WIS-GISC-BRASILIA

WIS-GISC-EXETER

WIS-GISC-JEDDAH

WIS-GISC-MELBOURNE

WIS-GISC-OFFENBACH

WIS-GISC-SEOUL

WIS-GISC-TEHRAN

WIS-GISC-TOKYO

WIS-GISC-TOULOUSE

WIS-UNASSOCIATED

WIS-GISC-NEWDELHI

- GISC back-up and activities

6.1.   GISC Moscow has concluded backup agreements with GISC Offenbach and GISC Toulouse. The following centers from GISC Moscow AoR are connected to the back-up GISCs:

- NC Minsk Belarus, NC Bishkek Kyrgystan, NC Astana Kazakhstan, DCPC/RTH of Russian Federation (Khabarovsk, Novosibirsk) are backuped at GISC Offenbach.

- NC Kiev Ukraine, NCs Tbilisi Georgia and Erevan Armenia are backuped at GISC Toulouse.

- There are 5 countries in the GISC Moscow AoR that are not backuped yet:  Azerbaijan, Moldova, Tajikistan, Turkmenistan, Uzbekistan.

6.2.   In 2018, measures were taken to connect the main NC of Kazakhstan (Astana) to the GISC Moscow, including the modification of the abbreviated headings of the Astana meteorological bulletins (CCCC=UAST), the generation and provision of metadata of Kazakhstan to the WIS.

6.3.	NC Tashkent is connected to GISC Moscow, the provision of Uzbekistan's metadata in WIS is transferred from GISC Tokyo to GISC Moscow.

- Monitoring activity & volume

GISC Moscow monitors the functioning of GISC hardware/software portal, communications status (RMDCN IPVPN health, status of metadata catalogue synchronization with other GISCs, 24h data cache volume). The monitoring results are provided in the form of JSON files to WMO WIS Monitoring Site (http://www.wis-jma.go.jp/wcd/v1/top.html.

- Traffic size and volumes, uptime etc, user registration

8.1.    Metadata Catalogue status:

•	Number of Metadata in WIS-CATALOGUE: ~ 99380 records

•	Number of Metadata in WIS-GISC-MOSCOW: 3432 records

8.2. 	24h Cache  status:

•	Number of product instances: 564 434

•	Size of cache: 12,6 Gb

•	Number of product instances missing metadata: 394 309

•	Size of product instances missing metadata: 10,4 Gb

•	Number of unique products missing metadata: 1 535

•	Number of unique products missing metadata AoR: 0

- Progress on action items

Training materials have been published on GISC Moscow (http://portal.gisc-msk.wis.mecom.ru:8080/portal/portal/gisc-user/wmo)

- Outlook (planned changes/improvements)

GISC Moscow is planning to establish direct connection with GISC Tokyo, GISC Melbourne via RMDCN connection and with GISC Washington via Internet.

Also, GISC Moscow continues work with NCs Azerbaijan, Republic of Moldova, Tajikistan, Turkmenistan and Uzbekistan to connect them to backup GISC Offenbach.

The certification of NCs in the area of responsibility of GISC Moscow is scheduled to be completed in Q1/2019.

- Any other issues of interest

GISC Moscow is making preparations for holding the re-audit in Q1/2019.

- Summary (text for report)

GISC Moscow made a GISC status report in accordance with the structure and content accepted by ET-WISC/TT-GISC 2016, aiming to continue WIS implementation at the centre and in its area of responsibility.