Guide to Virtual Private Networks (VPN) via the Internet between GTS centres

WMO-No. 1116

WMO-No. 1116

© World Meteorological Organization, 2013

The right of publication in print, electronic and any other form and in any language is reserved by WMO. Short extracts from WMO publications may be reproduced without authorization, provided that the complete source is clearly indicated. Editorial correspondence and requests to publish, reproduce or translate this publication in part or in whole should be addressed to:

Chair, Publications Board

World Meteorological Organization (WMO)

7 bis, avenue de la Paix	Tel.: +41 (0) 22 730 84 03

P.O. Box 2300	Fax: +41 (0) 22 730 80 40

CH-1211 Geneva 2, Switzerland	E-mail: publications@wmo.int

ISBN 978-92-63-11116-6

NOTE

The designations employed in WMO publications and the presentation of material in this publication do not imply the expression of any opinion whatsoever on the part of the Secretariat of WMO concerning the legal status of any country, territory, city or area, or of its authorities, or concerning the delimitation of its frontiers or boundaries.

Opinions expressed in WMO publications are those of the authors and do not necessarily reflect those of WMO. The mention of specific companies or products does not imply that they are endorsed or recommended by WMO in preference to others of a similar nature which are not mentioned or advertised.

WORLD METEOROLOGICAL ORGANIZATION

COMMISSION FOR BASIC SYSTEMS

OPAG ON INFORMATION SYSTEMS & SERVICES

Guide for Virtual Private Networks (VPN) via the Internet between Global Telecommunication System centres

TABLE OF CONTENTS

1	INTRODUCTION	6

2	WHAT IS A VIRTUAL PRIVATE NETWORK?	7

2.1	Definition	7

2.2	Types of virtual private networks	8

2.2.1	Link Layer solutions	9

2.2.1.1	Asynchronous Transfer Mode and Frame Relay	9

2.2.1.2	Multiprotocol Label Switching	9

2.2.1.3	Layer 2 Tunnelling Protocol and Point-to-Point Tunnelling Protocol	9

2.2.2	Transport and application layers	11

2.2.2.1	Secure Socket Layer and Transport Layer Security	11

2.2.2.2	Secure Shell	11

2.2.3	So, why Internet Protocol Security?	12

3	WHAT IS INTERNET PROTOCOL SECURITY?	13

3.1	Internet Protocol Security architecture	13

3.2	Internet Protocol Security services and modes	14

3.3	Security associations	16

3.4	Authentication Headers	16

3.5	Encapsulating Security Payload	17

3.6	Internet Key Exchange	17

3.7	Conclusion	18

4	INTERNET PROTOCOL SECURITY AND THE GTS	19

4.1	Cryptography and the law	19

4.2	A quick view of the Global Telecommunication System (GTS)	20

4.2.1	Physical layers	20

4.2.2	Upper layers	20

4.3	Internet	20

4.4	Suggested approach	20

4.5	Solution suppliers	21

4.6	Network architecture	21

4.7	Implementation scenarios	22

4.8	Multicast over Internet Protocol Security Virtual Private Networks	23

4.9	Dynamic Multipoint Virtual Private Network solutions	23

4.10	OpenVPN solutions	24

5	OPERATOR SOLUTIONS	27

5.1	Star, partially meshed and fully meshed networks	27

5.2	Brief introduction to Multiprotocol Label Switching	28

5.3	Internet Protocol Security Virtual Private Networks over the Internet	28

5.4	Conclusion	28

APPENDIX A	30

1.	Example: Cisco-based configuration	30

1.1	Test platform	30

1.2	Asymmetric Digital Subsriber Line (ADSL) connection with Network Address Translation	30

1.3	Virtual Private Networks between routers	31

1.3.1	Internet Key Exchange	34

1.3.2	Authentication Header and Encapsulated Security Payload	34

1.3.3	Security Association	35

1.4	Final configuration	35

1.5	Performance	38

2.	Example: Cisco DMVPN configuration	38

2.1	Test environment	38

2.2	HUB1 configuration	39

2.3	HUB2 configuration	40

2.4	SPOKE configuration	41

3.	Configuration example for an OpenVPN-based solution	43

3.1	Server configuration	43

3.2	Generate the Certificate Authority	43

3.3	Generate the certificate and key for the server	44

3.4	Generate certificates and keys for the clients (Met Centres)	45

3.5	Generate Diffie–Hellman parameters	46

3.6	Copying files here and there	46

3.7	Client configuration	47

GLOSSARY	49

BIBLIOGRAPHY	60

Blank

# INTRODUCTION

After describing various concepts related to Virtual Private Networks (VPNs) and Internet Protocol Security (IPSec), this document presents a potential methodology to introduce a technical solution to the Global Telecommunication System (GTS) and shows why these tools can enhance communication capabilities among World Meteorological Organization (WMO) Members for operational traffic exchanges.

Chapter 2 introduces the concept of a VPN and presents various technical alternatives to IPSec. As IPSec is the most appropriate protocol for secure network-to-network communication, the document will then focus on this solution.

Chapter 3 briefly describes what IPSec is, as it is not a simple solution to understand. It covers many technical aspects, allows various types of algorithms, and is better described as a framework rather than a protocol.

Chapter 4 focuses on the application of IPSec to the GTS. A selection of protocols is proposed. It shows what benefits WMO Members could expect from the use of IPSec.

Chapter 5 shows the technical evolution of Internet Protocol (IP) services from the operators. Evolving from Frame Relay, now deprecated, to IP solutions, operators offer VPN solutions that are either Multiprotocol Label Switching (MPLS) based or IPSec based.

Appendix A gives complete configuration examples of Cisco routers with the protocols selected in chapter 3. This is followed by a glossary of terms related to VPNs and some references on IPSec.

# WHAT IS A VIRTUAL PRIVATE NETWORK?

## Definition

The definition below comes from What is a VPN? (Ferguson and Huston, June 1998).

Perhaps the simplest method of attempting to arrive at a simple definition for a VPN is to look at each word in the acronym individually, and then subsequently tie each of them together in a simple, common sense, and meaningful fashion.

Let’s start by examining the word “network” This is perhaps the least difficult term for us to define and understand, since the commonly accepted definition is fairly uncontroversial and generally accepted throughout the industry. A network consists of any number of devices which can communicate through some arbitrary method. Devices of this nature include computers, printers, routers, and so forth, and may reside in geographically diverse locations. The methods in which they may communicate are numerous, since there are countless electronic signalling specifications, and data-link, transport, and application layer protocols. For the purposes of simplicity, let’s just agree that a “network” is a collection of devices that can communicate in some fashion, and can successfully transmit and receive data amongst themselves.

The term “private” is fairly straightforward, and is intricately related to the concept of “virtualization” insofar as a VPN is concerned, as we’ll discuss in a moment. In the simplest of definitions, “private” means that communications between two (or more) devices is, in some fashion, secret – that the devices which are not participating in the “private” nature of communications are not privy to the communicated content, and that they are indeed completely unaware of the private relationship altogether. Accordingly, data privacy and security (data integrity) are also important aspects of a VPN which need to be taken into consideration when considering any particular VPN implementation.

Another means of expressing this definition of “private” is through its antonym, “public.” A “public” facility is one which is openly accessible, and is managed within the terms and constraints of a common public resource, often via a public administrative entity. By contrast, a “private” facility is one where access is restricted to a defined set of entities, and third parties cannot gain access. Typically, the private resource is managed by the entities that have exclusive right of access. Examples of this type of private network can be found in any organizational network which is not connected to the Internet, or to any other external organizational network, for that matter.

With this definition the current GTS and the WIS core network are is a private networks.

"These networks are private due to the fact that there is no external connectivity, and thus no external network communications.

"Another important aspect of privacy in a VPN is through its technical definition. For example, privacy in an addressing and routing system means that the addressing used within a VPN community of interest is separate and discrete from that of the underlying shared network, and from that of other VPN communities. The same holds true for the routing system used within the VPN and that of the underlying shared network. The routing and addressing scheme within a VPN should, in general, be self-contained, but this scenario degenerates into a philosophical discussion of the context of the term "VPN." Also, it is worthwhile to examine the differences between the "peer" and "overlay" models of constructing VPNs—both of which are discussed in more detail later under the heading "Network-Layer VPNs."

"Virtual" is a concept that is slightly more complicated. The New Hacker's Dictionary (formerly known as the Jargon File) [2] defines virtual as:

virtual /adj./ [via the technical term "virtual memory," prob. from the term "virtual image" in optics] 1. Common alternative to{logical}; often used to refer to the artificial objects (like addressable virtual memory larger than physical memory) simulated by a computer system as a convenient way to manage access to shared resources. 2. Simulated; performing the functions of something that isn't really there. An imaginative child's doll may be a virtual playmate. Oppose {real}."

For a VPN, the definition given in 2. is perhaps the most appropriate comparison for virtual networks.

"The “virtualization” aspect is one that is similar to what we briefly described above as “private,” however, the scenario is slightly modified – the private communication is now conducted across a network infrastructure that is shared by more than a single organization. Thus, the private resource is actually constructed by using the foundation of a logical partitioning of some underlying common shared resource, rather than by using a foundation of discrete and dedicated physical circuits and communications services. Accordingly, the “private” network has no corresponding “private” physical communications system. Instead, the “private” network is a virtual creation which has no physical counterpart. The virtual communications between two (or more) devices are due to the fact that the devices which are not participating in the virtual communications are not privy to the content of the data, and that they are also altogether unaware of the private relationship between the virtual peers. The shared network infrastructure could, for example, be the global Internet and the number of organizations or other users not participating in the virtual network may literally number into the thousands, hundreds of thousands, or even millions.

A VPN can also said to be a discrete network – discrete \dis*crete"\, a. [L. discretus, p. p. of discernere. See Discreet.] 1. Separate; distinct; disjunct.

The discrete nature of VPN’s allow both privacy and virtualization. While VPN’s are not completely separate, per se, the distinction is that they operate in a discrete fashion across a shared infrastructure, providing exclusive communications environments which do not share any points of interconnection. The combination of these terms produces VPN – a private network, where the privacy is introduced by some method of virtualization. A VPN could be built between two end-systems or between two organizations, between several end-systems within a single organization or between multiple organizations across the global Internet, between individual applications, or any combination of the above.

The common and somewhat formal characterization of the VPN, and perhaps the most straightforward and strict definition, is:

A VPN is a communications environment in which access is controlled to permit peer connections only within a defined community of interest, and is constructed through some form of partitioning of a common underlying communications medium, where this underlying communications medium provides services to the network on a non-exclusive basis.

A VPN is a communications environment in which access is controlled to permit peer connections only within a defined community of interest, and is constructed through some form of partitioning of a common underlying communications medium, where this underlying communications medium provides services to the network on a non-exclusive basis.

This definition introduces a concept, the VPN, not related to any technical implementation.

There are quite a lot of technical implementations of VPN."

## Types of virtual private networks

A simplified version of the Transmission Control Protocol (TCP)/IP layer model is shown in Figure 1.

The technical implementation of the VPN is related to this model:

- On the link layer, one can find:

  - Asynchronous Transfer Mode (ATM) and Frame Relay connection;

  - Multiprotocol Label Switching;

  - Virtual Private LAN Service;

  - Link-Layer Encryption (Layer 2 Tunnelling Protocol (L2TP) or Point-to-Point Tunnelling Protocol (PPTP));

  - Virtual eXtensible Local Area Network (VXLAN) IETF RFC 7348

- On the network layer:

  - IPSec;

- On the transport and application layer:

  - Transport Layer Security (TLS) is a proposed standard by the Internet Engineering Task Force (IETF);

  - Secure Shell (SSH).

Figure 1. A simplified version of the Transmission Control Protocol (TCP)/IP layer model

### Link Layer solutions

#### Asynchronous Transfer Mode and Frame Relay

Following the definition of VPN given in chapter 1, early data network, such as ATM and Frame Relay solutions, must be considered as VPNs. In this case, the VPNs rely on the operator.

#### Multiprotocol Label Switching

Nowadays, as IP is the base protocol, most of the provider solutions are based on MPLS, which is a protocol originated by Cisco (the Tag Switching initiative), and which is now widely adopted.

In the traditional IP world, every router must route every packet on the network. Routing is rather complex and slow. Multiprotocol Label Switching introduces (or uses) the concepts of tags. Packets are tagged at the entrance of the Wide Area Network (WAN). Inside the WAN, packets are switched (not routed) based on the tag. Tags are removed at the network exit.

This solution is now widely offered by operators.

Virtual Private LAN Service

VPLS is a way to provide Ethernet-based multipoint to multipoint communication over IP or MPLS networks. It allows geographically dispersed sites to share an Ethernet broadcast domain by connecting sites through pseudo-wires, and VPLS allows any-to-any (multipoint) connectivity.

#### Layer 2 Tunnelling Protocol and Point-to-Point Tunnelling Protocol

Two solutions that are mainly dedicated to remote access are L2TP and PPTP. In a normal situation, a remote user who wants to connect to the Intranet uses a Point-to-Point Protocol (PPP) connection to a remote-access server. In this case, the username and password (and also data) are transferred in plain text and therefore might be sniffed by potential intruders. The encryption of traffic between peers is permitted by L2TP and PPTP, leading to better security.

##### Point-to-Point Tunnelling Protocol

Point-to-Point Tunnelling Protocol is a layer 2 protocol that encapsulates PPP frames in IP datagrams for transmission over an IP internetwork, such as the Internet. Point-to-Point Tunnelling Protocol can be used for remote-access and router-to-router VPN connections. Point-to-Point Tunnelling Protocol is documented in Request for Comments (RFC) 2637.

Point-to-Point Tunnelling Protocol uses a TCP connection for tunnel maintenance and a modified version of Generic Routing Encapsulation (GRE) to encapsulate PPP frames for tunnelled data. The payloads of the encapsulated PPP frames can be encrypted and/or compressed. Figure 2 shows the structure of a PPTP packet containing user data.

Figure 2. The structure of a PPTP packet

##### Layer 2 Tunnelling Protocol

Layer 2 Tunnelling Protocol is a combination of PPTP and Layer 2 Forwarding (L2F), a technology proposed by Cisco Systems, Inc. Layer 2 Tunnelling Protocol represents the best features of PPTP and L2F, and encapsulates PPP frames to be sent over IP, X.25, Frame Relay or ATM networks. When configured to use IP as its datagram transport, L2TP can be used as a tunnelling protocol over the Internet. Layer 2 Tunnelling Protocol is documented in RFC 2661.

Over IP networks, L2TP uses User Datagram Protocol (UDP) and a series of L2TP messages for tunnel maintenance. It also uses UDP to send L2TP-encapsulated PPP frames as tunnelled data (see Figure 3). The payloads of encapsulated PPP frames can be encrypted and/or compressed.

Figure 3.Structure of IP Packet containing L2TP encapsulated PPP frame using UDP

In Windows operating system versions, starting from Windows 2000, IPSec Encapsulated Security Payload (ESP) is used to encrypt the L2TP packet. This is known as L2TP/IPSec. The result after applying ESP is shown in Figure 4.

Figure 4. IP Packet contating information encrypted using L2TP/IPSec

#### Virtual eXtensible Local Area Network (VXLAN)

Virtual eXtensible LAN (VXLAN) is a network virtualization technology that attempts to address the need for overlay networks within virtualized data centers (enterprise datacenters and cloud service providers).

It is described in IETF RFC 7348.

### Transport and application layers

These layers mainly cover host-based solutions.

#### Secure Socket Layer and Transport Layer Security

Netscape created the protocol SSL. Transport Layer Security is a cryptographic protocol. In the TCP/IP layering model it is on top of the TCP layer. Therefore, it could be used for adding security (strong authentication and encryption) for all TCP-based application (telnet, File Transfer Protocol (FTP), etc.). SSL is no longer recommended for use, all SSL versions are deprecated by now. The noe recommended protocol  is TLS. Companies use this technology to provide secure remote access to their internal infrastructure by using SSL/TLS VPNs with additional strong authentication, often using either one-time password generators or X.509 certificates. The advantage is that no VPN client software has to be installed on the client computers in advance. Instead, the VPN client software is downloaded as ActiveX controls or Java applets just before the VPN connection is established.

In addition to commercial SSL TLS VPN solutions, there are also powerful public domain VPN solutions such as OpenVPN (Open Source VPM) and stunnel, which can be used free of charge.

Other implementations exist for these protocols, but the success story of SSL TLS is Hypertext Transfer Protocol Secure (HTTPS). It is used in e-commerce applications to allow secure information exchanges between client and servers. Transport layer security is the IETF proposed standard and the successor of SSL and is also an IETF standard. TLS is a proposed Internet Engineering Task Force (IETF) standard, first defined in 1999 and updated in RFC 5246 (August 2008) and RFC 6176 (March 2011)  (https://en.wikipedia.org/wiki/Transport_Layer_Security).

The use of TLS 1.2 and later versions is recommended, as older versions are deprecated.

#### Secure Shell

Secure Shell is another application layer authentication and encryption protocol. Secure Shell Frequently Asked Questions give the following definition of SSH:

"Secure Shell is a program to log into another computer over a network, to execute commands in a remote machine, and to move files from one machine to another. It provides strong authentication and secure communications over insecure channels. It is intended as a replacement for telnet, rlogin, rsh and rdist"

For SSH2, there also is a replacement for FTP: sftp.

Therefore, the main use of SSH is within organizations. In theory, to manage security-based devices (firewalls, etc.), or to gain root access on hosts, the network/system administrator should avoid connecting remotely using telnet to the box. If telnet is used, it is very easy, with a sniffer, to capture and analyse the packets to gain administrative access on the firewall/system. With direct access, no clear user/password will be exchanged on the Local Area Network (LAN). However, network administrators are often lazy; SSH is the answer in this case!

Among other things, SSH includes an encrypted replacement tool for telnet.

Secure Shell is becoming very popular the strongly recommended tool for secure remote management and also for secure remote data transfer.

### So, why Internet Protocol Security?

In the Manual on the GTS, (WMO No 386) presents two solutions to exchange traffic between Message Switching Systems (MSS) using the IP protocol. One is based on FTP and the other on sockets.

This Manual does not cover the WAN infrastructure. The current GTS is a mix of leased lines, peer-to-peer Frame Relay links, global Frame Relay services and MPLS networks (such as RMDCN in WMO Region VI). For economic reasons, and with regards to the overall high quality of the Internet service, it might be a good opportunity to study the potential use of the Internet to complement the GTS.

However, if reliable (but no real Service Level Agreement (SLA)), the Internet is, by nature, an insecure network. Various documents within WMO show how National Meteorological Centres (NMCs) should connect to the Internet (firewalls, etc.).

In order to allow a smooth introduction of the Internet to complement the GTS, the following rules should apply:

- Permit the use of the current protocols (FTP and socket) on the Internet

- Avoid any impact on MSS

- Guarantee an acceptable level of trust for members

The first two points mean that the proposed solution should be transparent to the application and the hosts. Among the protocols described above, IPSec is the only one that is completely application independent.

To offer a minimum level of trust, authentication (who wants to talk to me with whom) and encryption (no one except me  the 2 authenticated partners/sites can understand the data) are both needed. Internet Protocol Security offers these two services.

# WHAT IS INTERNET PROTOCOL SECURITY?

## Internet Protocol Security architecture

From RFC 2401 (replaced by RFC 4301; Kent and Seo, 2005；RFC 4301 partly replaced by RFC 6040, Briscoe, 2010 and RFC 7619, Smyslov and Wouters, 2015):

IPSec is designed to provide interoperable high quality, cryptographically-based security for IPv4 and IPv6. The set of security services offered includes access control, connectionless integrity, data origin authentication, protection against replays, confidentiality, and limited traffic flow confidentiality. These services are provided at the IP layer offering protection for the IP and upper layer protocols.

IPSec is designed to provide interoperable high quality, cryptographically-based security for IPv4 and IPv6. The set of security services offered includes access control, connectionless integrity, data origin authentication, protection against replays, confidentiality, and limited traffic flow confidentiality. These services are provided at the IP layer offering protection for the IP and upper layer protocols.

Specification of IPSec is rather complex. The overall architecture of the specification can be seen as a suite of interacting protocols. The organization of the specifications is given in RFC 4301 (Figure 5).

Figure 5. IPSec document overview

- Architecture covering the general concepts, security requirements, definitions and mechanisms defining IPSec technology:

- Defines the capabilities that hosts and routers should provide;

- For example, it is required that the hosts provide confidentiality using ESP, however, this document does not specify the header format;

- Describes the interaction between IPSec and the rest of TCP/IP;

(b)	Encapsulated security payload (ESP) and authentication header (AH):

- Define the protocol, the payload header format and the services they provide;

- Define the packet processing rules;

- Do not specify the cryptographic transforms that are used to provide these capabilities; this allows the transforms to be changed if they become cryptographically insecure, without any change in the base protocol;

(c)	Encryption algorithm and authentication algorithm:

- A set of documents that describes how various encryption algorithms are used in ESP or how various authentication algorithms are used in AH and the authentication part of ESP;

- Define the algorithm, the key sizes, the derivation of keys, transformation processes and any algorithm-specific information;

- The definitions have to be very specific in order to obtain interoperability;

- Key management describing the key management schemes:

- Keys are generated with Internet Key Exchange (IKE) in IPSec protocols;

- The payload format of IKE is very generic, it can be used to negotiate keys in any protocol, and IKE is also used for negotiating keys for other protocols outside IPSec;

- Genericity is achieved by separating the parameters IKE negotiates from the protocol itself;

(e)	Domain of interpretation (DOI) contains values required for the other documents to relate to each other, that is, identifiers for approved encryption and authentication algorithms, and operational parameters such as key lifetime:

- The parameters negotiated by IKE are defined in DOI;

(f)	Policy is an important component:

- It determines if two entities will be able to communicate with each other, and if so, which transforms to use;

- Policy representation deals with definition, storage and retrieval of policy;

- Policy implementation addresses the application of policy for actual communication involving, for example, the application of negotiated keys in the communication.

All these documents are RFCs, and, as often occurs with RFCs, they can be difficult to understand!!

The following paragraphs introduce IPSec and some key points in it.

## Internet Protocol Security services and modes

Internet Protocol Security framework has been built and defined in such a way to guarantee maximum independence between different parts of the system (encryption algorithm, authentication algorithm, etc. are not linked to ESP and AH protocols, for example). The goal of IPSec is to offer security through encryption, and it has been decided to split the solution into several parts. This led to a very powerful solution, and depending on the goal one wants to reach, the good protocols may be chosen among a large choice. However, it may lead to some incompatibility.

It must also be noted that in the first design (the first set of RFCs was published in 1995), the ESP protocol was not usable for authentication. In the second set (published in late 1998), the ESP protocol also offered authentication solutions. The third set of RFCs related to IPSEC, in 2005, confirms this architecture.

When two systems want to exchange data using IPSec, they must first determine the services they want to use from IPSec. Table 1 summarizes the services offered by ESP and AH.

Table 1. Internet Protocol Security services

|  | Authentication Header | Encapsulating Security Payload (encryption only) | Encapsulating Security Payload (encryption and authentication) |
| --- | --- | --- | --- |
| Access control |  |  |  |
| Connectionless integrity |  |  |  |
| Data origin authentication |  |  |  |
| Rejection of replayed packets |  |  |  |
| Confidentiality |  |  |  |
| Limited traffic flow confidentiality |  |  |  |

This is the first step, but not the last!

The next step is to decide if tunnel mode or transport mode should be used. Transport mode is mainly designed for host-to-host communication where IPSec is embedded in the host operating system. In tunnel mode, the hosts are not in charge of IPSec, some boxes between the hosts carry out this job.

Figures 6 and 7 show the structures of the packets in the two modes.

Figure 6. IPSec operations within transport mode

In this mode, the real IP header of the packet is used along the way, and the IP addresses of the hosts are used to route the packet. It means that if the packet is going on the Internet, IP addresses of both hosts must be routable.

Figure 7. IPSec operations within tunnel mode

The real IP addresses are embedded in the new IPSec packet. If the packet is routed over the Internet, the new IP addresses must be routable.

## Security associations

Security Associations (SAs) in IPSec terminology form the basis for IPSec operations. Security Associations are contracts between two entities determining the IPSec protocols used for securing the packets, the transforms, the keys and their duration, plus many other things.

Before two entities are able to exchange packets using IPSec, they must first create SAs, which are always one way (simplex). Therefore, if hosts A and B are communicating with IPSec, each host will have two SAs: SAin and SAout. Parameters SAin for host A and SAout for host B will share the same cryptographic parameters. Likewise, SAs are protocol specific. There is an SA for each protocol: ESP and AH. Each IPSec host therefore has to keep a database (the Security Association Database (SADB)) to store all SAs; the Security Parameter Index (SPI) is a 32 bit element used to identify the SA in the SADB. Security Associations are managed (created and deleted) either manually or with a protocol, and in this case, the protocol used is IKE. It is out of the scope of this article to describe the exact and complete use of SAs, but it must be clear that SAs are the basis of the IPSec framework.

In the framework presented above, SA is linked to policy and DOI. Security associations are the links between concepts (IPSec framework) and reality (how to really use all these things in data communication).

## Authentication Headers

Authentication Headers provide data security and authentication of IP packets:

- Data integrity ensures that undetected modification to a packet content in transit is not possible

- Authentication features enable end systems or network devices to authenticate the user or the application and filter traffic accordingly

- Prevent address spoofing attacks

- Protect against replay attacks

The first three functions are guaranteed by an authentication hash function. A Message Authentication Code (MAC) function is an authentication process combined with a symmetrical key. To make it short, these functions calculate a digest of the message, and then the digest is encrypted with a shared secret between the two hosts.

A digest is the result of a fixed-length mathematical calculation. It has been proven that it is very unlikely to recreate the digest if the original information was altered, as it would be too time consuming.

Such techniques are used, for example, on public FTP servers. When publishing a file on a server, it is useful to also put the digest of the file on the same server. The digest guarantees that the original file has not been modified by anyone.

The most well-known digest calculation method is MD5. However, since it is also an insecure method, the use of Secure Hash Algorithm (SHA)-256 is now recommended.

The required protocol for AH is Hashed Message Authentication Code (HMAC)-SHA-1, but HMAC-SHA-256 is recommended.

The last function in AH (anti-replay attack) is built with a mechanism of a sliding window. Each packet receives a sequence number and various techniques guarantee that the packet cannot be replayed without notice.

## Encapsulating Security Payload

Encapsulating Security Payload offers two services:

- Encryption: The data is encrypted with a predefined protocol between the two hosts

- Authentication: See AH

In the first release of RFCs regarding IPSEC, AH was used for authentication and ESP for encryption only. It has been shown by further study that encryption without authentication was not secure. As the old RFCs allowed the use of ESP without AH, it was decided to add the capability to authenticate within ESP to the new RFC.

There is now some redundancy. It is possible to use AH to authenticate, and to use ESP to also authenticate and encrypt. Bruce Schneier, a well-known cryptographic specialist, suggests using ESP for both, and to forget about AH.

The authentication algorithms usable in ESP are:

- Triple Data Encryption Standard (3DES) in Cipher Block Chaining mode (this is a mandatory protocol)

- Advanced Encryption Standard (AES) in Cipher Block Chaining mode

- Advanced Encryption Standard counter mode

3DES has been deprecated in the latest version of the protocol, and therefore must not be used.

AES is now the de facto standard and should be used for encryption.

## Internet Key Exchange

In order to talk to each other, peers must use SAs, which define the security parameters and authentication keys to use. Internet Key Exchange is the protocol used to create and exchange SAs.

However, nothing is simple in IPSec theory. In fact, IKE covers three different protocols:

- Internet Security Association and Key Management protocol (ISAKMP)

- OAKLEY Key Determination Protocol

- SKEME Versatile Secure Key Exchange Mechanism

Internet Security Association and Key Management Protocol defines the language for negotiation. It defines the payload format, the mechanics of implementing a key exchange and the negotiation of an SA. It does not define the key exchange algorithm, but rather the message types in order to exchange keys.

OAKLEY and SKEME are two key exchange protocols usable under the ISAKMP umbrella in the IKE world.

In fact, ISAKMP, OAKLEY and SKEME were already developed protocols and IKE is the binding element between all of these.

## Conclusion

In this brief introduction to IPSec, we have seen that:

- Two modes exist: tunnel and transport

- Two protocols that are partially redundant are defined: ESP and AH

- Security Associations are created and maintained by three different protocols

- Authentication can be achieve through two possible algorithms

- Encryption relies on 3DES or AES

We have neither covered nor introduced other aspects that may also be relevant in IPSec:

- Diffie–Helmann protocol to exchange keys

- X.509v3 certificates

- Public Key Infrastructure (PKI)

In some aspects, VPN is quite a vague concept; IPSec, one of the VPN solutions, is not much clearer!

Therefore, despite the rather ugly face of the protocol, the various options and the risk of incompatibility, IPSec is now widely implemented and available on many platforms, including, routers, dedicated boxes, firewalls, hosts, etc.

In fact, in most cases, the solutions implemented only cover part of the possible options. It is therefore very important to guarantee interoperability to share the same subset among peers.

In the following section, we suggest a possible scenario taking into account the need of security, some legal aspects and interoperability issues.

# INTERNET PROTOCOL SECURITY AND THE GTS

## Cryptography and the law

In VPN and IPSec, cryptography is one of the key aspects to ensure privacy of communication. Governments in all countries still consider cryptography as some sort of weapon. It has often been explained by different governments in the world that lawful activities perhaps need privacy, but the main use of crypto tools was for unlawful activities. Therefore, every country has defined rules about the use of cryptography within the country.

The website http://www.ieee-security.org/Cipher/ConfReports/CryptoLawSurvey.html  resents the situation regarding cryptography in many countries (from the website):

In the last years, a group of countries worked on the Wassenaar Arrangement. It controls the export of weapons and of dual-use goods, that is, goods that can be used both for a military and for a civil purpose; cryptography is such a dual-use good.

In 1995, 28 countries decided to establish a follow-up to COCOM, the Wassenaar Arrangement on Export Controls for Conventional Arms and Dual-Use Goods and Technologies. The negotiations on the Arrangement were finished in July 1996, and the agreement was signed by 31 countries (Argentina, Australia, Austria, Belgium, Canada, the Czech Republic, Denmark, Finland, France, Germany, Greece, Hungary, Ireland, Italy, Japan, Luxembourg, the Netherlands, New Zealand, Norway, Poland, Portugal, the Republic of Korea, Romania, the Russian Federation, the Slovak Republic, Spain, Sweden, Switzerland, Turkey, the United Kingdom and the United States). Later, Bulgaria and Ukraine also became a participating state to the Arrangement.

The initial provisions were largely the same as old COCOM regulations. The General Software Note (applicable until the December 1998 revision) excepted mass-market and public-domain crypto software from the controls. Australia, France, New Zealand, Russia, and the US deviated from the GSN and controlled the export of mass-market and public-domain crypto software. Export via the Internet did not seem to be covered by the regulations.

There is a personal-use exemption, allowing export of products “accompanying their user for the user’s personal use” (e.g., on a laptop).

The Wassenaar Arrangement was revised in December 1998. Negotiations were held on 2 and 3 December 1998 in Vienna, which resulted in restrictions on the General Software Note and in some relaxations:

- free for export are: all symmetric crypto products of up to 56 bits, all asymmetric crypto products of up to 512 bits, and all subgroup-based crypto products (including elliptic curve) of up to 112 bits;

- mass-market symmetric crypto software and hardware of up to 64 bits are free for export (the 64-bit limit was deleted on 1 December 2000, see below);

- the export of products that use encryption to protect intellectual property (such as DVDs) is relaxed;

- export of all other crypto still requires a license.

Import or use of cryptographic solution is controlled in a large number of countries.

Before any use of cryptographic solution, everyone and especially government agencies such as National Meteorological Centres MUST verify if the solution they want to use is legal in their country.

Before any use of cryptographic solution, everyone and especially government agencies such as National Meteorological Centres MUST verify if the solution they want to use is legal in their country.

## A quick view of the Global Telecommunication System (GTS)

### Physical layers

The current GTS is a mix of various technical solutions:

- Leased line

- Frame Relay lines: The leased line is replaced by a connection to a Frame Relay network, but only two peers share the same Frame Relay network

- An MPLS network such as the RMDCN （as WIS core network ) is such a network

- Connections using the Internet

—    Satellite communication channels

The reproaches for the first three solutions are quite the same:

- Costs: For (relatively) slow-speed lines, the price is high

- Both the leased line and the Private Virtual Channel (PVCs) in Frame Relay are point-to-point links and therefore work well for point-to-point communication, but not for many-to-many nor any-to-any communication

Multiprotocol Label Switching with the any-to-any feature improves the situation, but the cost may still be an issue in some cases.

However, the current requirements in telecommunications become increasingly higher in terms of:

- Traffic

- Global exchange

With the growth of products and data, and with the WMO Information System (WIS) network, National Centres (NCs), Data Collection or Production Centres (DCPCs) and Global Information System Centres (GISCs) will want to exchange much more information. The architecture based on the concentration through a hub site and costly slow-speed links is not suitable for these new challenges.

### Upper layers

At the application layer, for GTS data exchange, two protocols are usable on top of the IP layer: FTP and sockets. Every new solution must be compatible with these standards.

## Internet

Most GTS sites are now connected to the Internet. The Internet connection is in most cases:

- Usually reliable: The Internet Service Providers (ISPs) deliver the connections to the Internet. They are operated just like other connecting lines and are therefore as reliable as the lines. However, it must be noted that no end-to-end SLAs could be defined on the Internet.

- Powerful: The Internet connection is often a high-speed connection.

- Secure: The sites must be protected by security systems such as security gateways (firewalls), malware filters, etc.

Therefore, the Internet is now a possible media to complement the current GTS private infrastructure, and it is already used as the connection to GTS in a few countries (see also section 4.2.1).

## Suggested approach

As seen previously, the only real application and host-independent VPN solution is IPSec.

Therefore, for WMO use, we recommend IPSec as the VPN solution.

However, in order to guarantee interoperability between NMCs without redefining the protocols to use each time, we suggest the following implementation solution:

- Tunnel mode: As IPSec will mostly be configured on routers, firewalls or dedicated boxes, and taking into account that neither encryption nor authentication are mandatory on LAN, tunnel mode is the most appropriate solution

- Authentication header should not be used, ESP used for authentication, and authentication should be carried out with HMAC-SHA-256

- Pre-shared secrets: Using the X.509v3 certificate is probably a more elegant solution, but in practice, it is more difficult to implement in WMO situations

If necessary, encryption using AES-256 should be used.

These recommendations mostly come from A Cryptographic Evaluation of IPSec (Ferguson and Schneier, December 2003.

## Solution suppliers

With the subset of protocols defined above, each GTS site can choose any supplier for VPN equipment.

In addition to commercial products, there is also a wide range of open-source software offering IPSec-based VPN functionality.

All IPSec implementations should be compatible. However, as seen previously, IPSec offers a large choice of configuration parameters. The solution proposed in section 4.4 is included in the mandatory subset of protocols described in the RFCs. Therefore, any RFC-compliant solution should interoperate with any other. However, in some cases, different IPSec end points do not interoperate. Before choosing a VPN box, sites should check with their potential VPN neighbours if their respective solutions are compatible.

Over the last seven years, the European Centre for Medium-Range Weather Forecasts (ECMWF) and its partners in Europe made many tests on IPSec. The latest one in 2007 showed that interoperability is still an issue. The tests involving Cisco, Nortel (with Checkpoint software) and Nokia (also with Checkpoint) were successful. However, the hassle to configure multiple different devices convinced ECMWF and its partners to agree on a common platform before a large-scale implementation.

## Network architecture

The placement of the VPN gateway in a security solution is of paramount importance. Improper placement of the VPN gateway will affect the effectiveness of the solution. In this context, all the VPN gateways operate in tunnel mode only. The following few scenarios of VPN gateway placement highlight this point. Emphasis is placed on the Intranet infrastructure and the demilitarization zone (DMZ), which is typically meant for limited authorized access. Servers that are put into the DMZ include web servers and external mail servers.

Several different solutions are available, and two solutions are highlighted in the following:

- The simplest one is a firewall and a VPN gateway within the same box

- The second one is a VPN gateway adjacent to the firewall

It should be noted that this architecture must be coherent with the security policy of the site.

Figure 8. VPN gateway is implemented on the firewall

In this case, a VPN gateway function is implemented on a firewall and the firewall connects to the Internet. The flow of traffic is from the Internet to the firewall/VPN gateway directly. This will be a more cost-effective solution since there is no need for a separate VPN gateway. Placing both the functions firewall and VPN onto the same box results in easier management. Note that this set-up is typically used for smaller VPN solutions.

The disadvantage of this solution is that the Central Processing Unit (CPU) utilization of the firewall/VPN gateway will be very high, and adequate measures have to be taken to address this issue. In other words, this set-up introduces additional software components in the firewall and hence results in further performance degradation. Another disadvantage of this solution is that it requires more services/ports to be opened. As a result, the security implementation may suffer from additional security holes. In addition, the more services that are running on a single (security) system, the higher the risk of being hacked. Finally, the firewall poses a single point of failure for the VPN solution.

Figure 9. VPN gateway in dedicated DMZ

In this case, the VPN gateway is connected to a (dedicated) interface of the Internet firewall. The flow of traffic into the Intranet will be through the firewall. However, the Internet traffic will arrive either:

- From the Internet via the firewall or

- From the Internet directly to the external interface of the VPN gateway and then to the firewall

The advantage of this set-up is that both types of clients (VPN and non-VPN) can be supported. All the clients will be subject to the firewall policy.

The most advanced scenario introduces another firewall dedicated to control VPN traffic. From one box for all the dedicated boxes, there are many different options, from the cheapest to the most expensive. However, as a general statement, the best solution for one site is the solution that conforms with the security policy and where the pros and the cons of the solutions are well understood.

## Implementation scenarios

In order for two GTS sites to establish a VPN link, they must:

- Agree upon the protocols to be used (mode, authentication and/or encryption, message digest algorithm, site authentication);

- Define the pre-shared secret – this password must be defined and must be the same on both sides;

- Confirm the VPN platforms to be used;

- Agree on IP addresses for the exchange of data over the link;

- Set up appropriate filter rules on the firewall, the following rules:

  - Allow UDP port 500 to be used for ISAKMP;

  - Allow UDP port 4500 (Network Address Translation (NAT) traversal);

  - Allow IP protocol number 50 (ESP protocol);

are the basic rules for an IPSec connection;

- Implement the defined configuration;

- Test.

Once everything is running, the main risk is the potential failure of the virtual link created.

## Multicast over Internet Protocol Security Virtual Private Networks

For communication in future WIS systems, it could be an advantage to exchange certain sets of data using multicast instead of unicast communication.

Since the IPSec protocol is a point-to-point protocol, and does not have point-to-multipoint capabilities by definition, sending multicast traffic over an IPsec VPN with more than two sites will require the use of additional tunnels (GRE or IP-in-IP) today.

An example of such a solution is the Dynamic Multipoint VPN (DMVPN) solution from Cisco, which is briefly described in section 4.9.

## Dynamic Multipoint Virtual Private Network solutions

A DMVPN is a proprietary Cisco Internetworking Operating System (IOS) solution for building IPSec and GRE VPNs in a scalable way. It relies on two Cisco technologies: Next Hop Resolution Protocol (NHRP) and multipoint GRE tunnel interfaces. A DMVPN does not alter the standards-based IPSec VPN tunnels, but it does change their configuration.

A spoke router registers its real public IP interface address when it boots and a hub maintains an NHRP database of all the spoke’s real public interface addresses. Subsequently, spoke routers query the NHRP database for real addresses of destination spoke routers to dynamically build direct tunnels. The multipoint GRE tunnel interface allows single GRE interfaces to support multiple IPSec tunnels.

In a typical hub-and-spoke IPSec design, all VPN traffic travels via the hub site, which acts as a gateway. Therefore, the hub bandwidth and the CPU utilization limit the VPN size. In the DMVPN dynamic-mesh approach, spoke-to-spoke tunnels can be dynamically built when required. This means that spoke routers only need to support tunnels currently in use, and hub routers only support spoke–hub traffic and overflow from spoke-to-spoke traffic.

In the context of the GTS, a DMVPN-based infrastructure was deployed as a backup for the RMDCN in a large-scale operational pilot at the beginning of February 2010 for the following reasons:

- The RMDCN Customer Edge (CE) router is a Cisco router running IOS

- The traffic re-routing between the RMDCN and the DMVPN infrastructures can be carried out in a fast, automatic and reliable manner

The following constraints apply to the DMVPN-based infrastructure:

- The solution maintains the RMDCN any-to-any connectivity

- The use of dedicated Cisco routers

- The use of a subset of Cisco routers running specific IOS versions in order to limit troubleshooting and maintenance issues

- The DMVPN equipment is locally managed

- The use of DMVPN is an additional option to the portfolio of the available RMDCN backup mechanisms (mission critical and Integrated Services Digital Network (ISDN))

The DMVPN topology selected is “single DMVPN/dual hub”, and the spoke site deployed by the DMVPN router is in accordance with the following technical requirements:

- The Cisco DMVPN router has to be installed on the same physical LAN/DMZ segment as the RMDCN CE router

- The DMVPN router needs to have a public IP address reachable through the Internet – no NAT should be deployed

- The DMVPN router must be able to establish IPSec tunnels to other DMVPN RMDCN sites and send encrypted RMDCN traffic through them: allow the IPSec protocols to and from the DMVPN router’s public IP address, that is, ESP, GRE, IKE (UDP 500) and NAT-T (UDP 4500) protocols

In addition to providing more scalability and flexibility, the DMVPN solution offers fewer configurations overhead.

## OpenVPN solutions

To provide a connectivity environment to allow all 13 National Meteorological Centres in Region III to be connected to each other using a VPN over the Internet, a VPN solution based on freely available software (OpenVPN) and the operating system Linux was set up.
Although the described VPN is not in operational use at the moment, the description below remains in this document for reference.

OpenVPN is a full-featured open-source SSL VPN solution that accommodates a wide range of configurations, including remote access, site-to-site VPNs, Wi-Fi security, and enterprise-scale remote-access solutions with load balancing, failover and fine-grained access controls. Starting with the fundamental premise that complexity is the enemy of security, OpenVPN offers a cost-effective, lightweight alternative to other VPN technologies.

The lightweight design of OpenVPN sheds many of the complexities that characterize other VPN implementations. The OpenVPN security model is based on SSL, the industry standard for secure communications via the Internet. OpenVPN implements Open Systems Interconnection (OSI) layer 2 or 3 secure network extensions using the SSL/TLS protocol, supports flexible client authentication methods based on certificates, smart cards and/or two-factor authentication, and allows user or group-specific access control policies using firewall rules applied to the VPN virtual interface. OpenVPN is not a web application proxy and does not operate through a web browser.

It was decided to create three networks using the OpenVPN software to facilitate the configuration and let the operation be more flexible:

- One core network connection between the three Regional Telecommunication Hubs (RTHs)

- One network covering the north part of the region

- One network covering the south part of the region

This set-up is shown in Figure 10.

Figure 10. VPN map for WMO Region III

RTH Buenos Aires will provide a server to collect the data from their neighbouring centres. RTH Maracay will provide the same approach to get data from the centres concerned to areas of responsibility. The three RTHs will have a special configuration in order to provide bidirectional communication between the centres involved.

The network connection of the three RTHs in the region will be called the core network. It will be the access point for the GTS though the message switching system. Each RTH will host the servers for the VPN connections.

The south part of Region III, comprising the NMCs for Colombia, Ecuador, Suriname, Guyana and French Guyana, will preferentially be a client for the server hosted by Maracay. In case of emergency or backup, they can connect to RTH Brasilia or RTH Buenos Aires.

The north part of Region III, comprising the NMCs for Chile, Peru, Bolivia, Paraguay and Uruguay will preferentially be a client for the server hosted by Buenos Aires. In case of emergency or backup, they can connect to RTH Brasilia or RTH Maracay.

In all cases, the servers will provide a fixed and private IP address for the logical tunnel between the NMC and the RTH in order to facilitate and guarantee the communication.

Figure 11 illustrates the designed set-up.

Figure 11. VPN setup in RA III

# OPERATOR SOLUTIONS

In past years, the operator market solutions for managed networks have consisted mainly of the following technologies:

- Frame relay

- Multiprotocol Label Switching

- Internet Protocol Security VPNs

Frame relay is increasingly being replaced by the other two technologies.

The first two solutions are layer 2 based, and are therefore completely independent of layer 3 protocols.

The main disadvantage of Frame Relay is its static nature. The client must define exactly what peers are allowed to exchange traffic and the throughput they want. This solution is not flexible. It is costly and often creates administrative complexity to change the parameters and to allow a more dynamic approach.

Frame relay is now deprecated and MPLS is the preferred solution from the providers.

## Star, partially meshed and fully meshed networks

Figure 12. Possible VPN paths on a Frame Relay network

Figure 13. Possible VPN paths on MPLS network

In a Frame Relay environment, as establishment of PVC has its costs, only a star or a partially meshed network is available. This gives less flexibility to the network.

An MPLS network, in general, is a fully meshed network. Therefore, it easily provides any-to-any connectivity.

## Brief introduction to Multiprotocol Label Switching

When data enter the carrier network, a label is attached to each packet. This label uniquely identifies the VPN using the so-called vpnv4/v6 address in a shared infrastructure and keeps it private. While travelling through the network, another label (for MPLS switching) is added. Upon reaching its destination, the labels are removed, returning the data packet to its original state. The process is seamless and unnoticeable to end-users.

Figure 14. Example topology of an MPLS network

The labels thus replace traditional Internet packet forwarding, where complicated address matching is performed at each hop in the network. The MPLS label describes how the packet should be handled within the network and thus assigns the packet to a Class of Service (CoS). Therefore, all packets that belong to the same CoS are treated in the same way and are quickly sped along their way.

In most cases, the providers manage the complete service (including local terminating routers).

## Internet Protocol Security Virtual Private Networks over the Internet

The Internet is a collection of interconnected ISP networks. Each ISP owns its infrastructure and interconnects with others to provide Internet access to its clients.

Within its network, an ISP can guarantee some level of service and therefore, with add-ons can propose to build a (service provider based) VPN based on IPSec. In this case, the VPN is a value-added service using a specific Internet connection. The SLA, in such a situation, will be very limited compared to an MPLS network.

## Software-Defined WAN (SD-WAN)

Software-Defined WAN (SD-WAN) is a way to simplify the management of a WAN by separating the networking hardware from its control mechanism.

It allows the use of various network links (e.g.: Internet ADSL, Internet LTE, MPLS) and route different kind of traffic through the appropriate one (e.g.: business through MPLS, Internet browsing through Internet ADSL) and dynamic backups.

Additional functionalities, such as security, QoS, etc. can be added.

It should allow companies to build higher performance WANs and lower costs.

Traditional VPN technologies, such as IPSec, can be used on SD-WAN.

## Conclusion

For the Main Telecommunication Network (MTN) or a regional network, a managed service from a provider (either MPLS VPN or IPSec VPN) offers an SLA. Depending on the technology used, this SLA will cover guaranteed bandwidth, performance, CoS, round-trip time, recovery time in case of problems or failures, etc.

The technical benefits of a managed network relying on MPLS or other VPNs compared to point-to-point solutions are better reliability, more flexibility in the traffic flows, and guaranteed and common SLAs.

However, whatever the technical solution used, the issues are similar:

- The cost of the network, in particular, for an MPLS solution

- The same operator must be available on each site

- The network will need a management entity

- A common agreement must be signed between all the countries involved and with the selected provider

The underlying technology for operator services is important, but this is not the only issue to consider before launching such a project.

# APPENDIX A

## Example: Cisco-based configuration

### Test platform

The test platform aims to demonstrate the connection of two Cisco routers using IPSec over the Internet.

Both sites are connected to the Internet. The left site (see Figure A1) is connected through a permanent ADSL connection. The right site connects through ISDN.

Both sites must keep normal Internet access. Internet accesses are not dedicated to the IPSEC connection, and depending on destination, the connection must be direct or use the ad hoc VPN.

The ADSL site must have a permanent IP address on the ADSL side of the router. The ISDN site receives a dynamic IP address for every new connection. Network address translation is used to masquerade internal IP addresses.

Figure A1. IPSec VPN over the Internet

In section 1.4 of this appendix, the full configurations are shown with an analysis of the main configuration topics.

For a full Cisco to-do list, see http://www.cisco.com/go/IPSec.

### Asymmetric Digital Subsriber Line (ADSL) connection with Network Address Translation

In order to understand the step-by-step connection mechanism, we first indicate the ADSL connection including NAT. In this case, there is no IPSec.

!

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo

!

memory-size iomem 15

ip subnet-zero

no ip finger

!

!

interface Ethernet0/0

ip address 130.1.8.1 255.255.0.0

ip nat inside

!

interface Ethernet0/1

no ip address

shutdown

!

interface ATM1/0

no ip address

atm vc-per-vp 4096

no atm ilmi-keepalive

pvc 2/32

encapsulation aal5mux ppp Virtual-Template1

!

interface Virtual-Template1

ip address negotiated

no ip redirects

no ip proxy-arp

ip nat outside

pulse-time 0

ppp chap hostname UserNetissimo

ppp chap password 7 00574404035D1207

no ppp chap wait

ppp pap sent-username UserNetissimo password 7 04085C040827554F

!

ip nat pool NET 193.253.191.14 193.253.191.14 netmask 255.255.255.0

ip nat inside source list 1 pool NET overload

ip classless

ip route 0.0.0.0 0.0.0.0 193.253.191.1

ip http server

!

access-list 1 permit any

!

line con 0

transport input none

line aux 0

line vty 0 4

password PWD

login

!

no scheduler allocate

end

### Virtual Private Networks between routers

ADSL router

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo

!

memory-size iomem 15

ip subnet-zero

no ip finger

!

crypto isakmp policy 1

authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0

!

crypto IPSec transform-set CMT esp-null esp-sha-hmac

!

crypto dynamic-map MYMAP 10

set security-association lifetime seconds 600

set transform-set CMT

match address 100

!

crypto map CMMYMAP 30 IPSec-isakmp dynamic MYMAP discover

!

interface Ethernet0/0

ip address 130.1.8.1 255.255.0.0

!

interface Ethernet0/1

no ip address

shutdown

!

interface ATM1/0

no ip address

atm vc-per-vp 4096

no atm ilmi-keepalive

pvc 2/32

encapsulation aal5mux ppp Virtual-Template1

!

interface Virtual-Template1

ip address negotiated

no ip redirects

no ip proxy-arp

pulse-time 0

ppp chap hostname UserNetissimo

ppp chap password 7 00574404035D1207

no ppp chap wait

ppp pap sent-username UserNetissimo password 7 04085C040827554F

crypto map CMMYMAP

!

ip classless

ip route 0.0.0.0 0.0.0.0 193.253.191.1

ip http server

!

access-list 1 permit any

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

!

line con 0

transport input none

line aux 0

line vty 0 4

password PWD

login

!

end

ISDN router

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ISDN

!

enable secret 5 $1$n6eZ$UXI24gGl7lZu/wB3OZVnC1

enable password PWD

!

memory-size iomem 25

ip subnet-zero

no ip finger

no ip domain-lookup

!

isdn switch-type vn3

!

crypto isakmp policy 1

authentication pre-share

crypto isakmp key IPSECKEY address 193.253.191.14

!

crypto IPSec transform-set CMT esp-null esp-sha-hmac

!

crypto map CMYMAP 1 IPSec-isakmp

set peer 193.253.191.14

set security-association lifetime seconds 600

set transform-set CMT

match address 100

!

cns event-service server

!

interface BRI0

no ip address

encapsulation ppp

no ip mroute-cache

dialer pool-member 1

isdn switch-type vn3

isdn send-alerting

no cdp enable

crypto map CMYMAP

!

interface FastEthernet0

ip address 131.1.8.1 255.255.0.0

no ip mroute-cache

speed auto

full-duplex

!

interface Dialer0

ip address negotiated

encapsulation ppp

dialer remote-name LibertySurf

dialer pool 1

dialer string 0860155555

dialer-group 1

pulse-time 0

no cdp enable

ppp authentication chap pap callin

ppp chap hostname bane0000@lsurf.fr

ppp chap password 7 1307160B04020A2F

ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

crypto map CMYMAP

!

ip classless

ip route 0.0.0.0 0.0.0.0 Dialer0

no ip http server

!

access-list 1 permit any

access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

dialer-list 1 protocol ip permit

!

line con 0

transport input none

line aux 0

line vty 0 4

password PWD

login

!

no scheduler allocate

end

In the two configurations, the red and underlined lines show the IPSEC set-up. Let us focus on this part of the configuration.

Table A1. IPSec config for ADSL and ISDN lines

| ADSL | ISDN |
| --- | --- |
| crypto isakmp policy 1 authentication pre-share crypto isakmp key IPSECKEY address 0.0.0.0 ! crypto IPSec transform-set CMT esp-null esp-sha-hmac ! crypto dynamic-map MYMAP 10 set security-association lifetime seconds 600 set transform-set CMT match address 100 ! crypto map CMMYMAP 30 IPSec-isakmp dynamic MYMAP discover ! access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255 ! interface ATM1/0 crypto map CMMYMAP ! | crypto isakmp policy 1 authentication pre-share crypto isakmp key IPSECKEY address 193.253.191.14 ! crypto IPSec transform-set CMT esp-null esp-sha-hmac ! crypto map CMYMAP 1 IPSec-isakmp set peer 193.253.191.14 set security-association lifetime seconds 600 set transform-set CMT match address 100 ! access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255 ! interface Dialer0 crypto map CMYMAP |

#### Internet Key Exchange

In the configuration, this is related to the lines:

crypto isakmp policy 1

authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0

The protocol used for IKE is isakmp, and it is an established pre-shared secret. This secret will be used by both routers to exchange real encryption keys.

IPSECKEY is the secret shared between the two routers.

On the ADSL side, the original IP address of the ISDN router is not known (in dialup connections such as ISDN, this address will change for every new connection); therefore, at this stage, the 0.0.0.0 means accept this IPSECKEY for every address. This may lead to potential security risks. We will see below how Cisco permits to minimize this.

#### Authentication Header and Encapsulated Security Payload

Authentication header protocol and ESP are then selected. Depending on the hashing protocol available, the encryption solution, the choice to rely on ESP for both authentication and encryption or to use AH, the protocols to be used for this particular VPN are described by the following command:

crypto IPSec transform-set CMT esp-null esp-sha-hmac

In our case, ESP is the preferred method for both authentication and encryption.

Here, ESP-NULL means use NULL for ESP (this is no encryption), and ESP-SHA-HMAC means use SHA-HMAC for authentication with ESP. Of course, the choice must be the same for both sides of the tunnel.

#### Security Association

The first two parts have shown the protocol part, let us now work on SA. The ADSL must accept an incoming IPSEC connection from an unknown peer:

crypto dynamic-map MYMAP 10

set security-association lifetime seconds 600

set transform-set CMT

match address 100

!

crypto map CMMYMAP 30 IPSec-isakmp dynamic MYMAP discover

!

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

Therefore, Cisco offers the dynamic-map solution. In this case, the ADSL router will accept every new connection using the right protocols (esp-null and esp-sha-hmac) from any IP address using the right password IPSECKEY. In order to restrict the use of this IPSEC tunnel, Cisco uses the mechanism of access-list. The access-list 100 must be the same on both sides of the tunnel in order to allow the IPSEC connection. This is a Cisco-proprietary feature.

### Final configuration

A mixture of the above configurations, NAT and IPSEC, are gathered in one configuration.

Comments in red help the reader to identify the parts in the configuration that are site dependent. This includes:

- IP addresses on both sites

- Usernames and passwords used to connect to the ISPs

- Access-lists

- IPSec parameters

This configuration can be used as an example or a to-do list when configuring IPSec between two sites.

ADSL router

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo	Username and password used to connect

!	to the Internet through ADSL. This

memory-size iomem 15	depends on agreements with local service

ip subnet-zero	providers (ISPs).

no ip finger

!

crypto isakmp policy 1

authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0

!

crypto IPSec transform-set CMT esp-null esp-sha-hmac

!

crypto dynamic-map MYMAP 10

set security-association lifetime seconds 600

set transform-set CMT

match address 100

!

crypto map CMMYMAP 30 IPSec-isakmp dynamic MYMAP discover

!

interface Ethernet0/0

ip address 130.1.8.1 255.255.0.0	Internet protocol address of local site.

ip nat inside

!

interface Ethernet0/1

no ip address

shutdown

!

interface ATM1/0	Asynchronous transfer mode interface for ADSL. link.

no ip address	These parameters are ISP dependent.

atm vc-per-vp 4096

no atm ilmi-keepalive

pvc 2/32

encapsulation aal5mux ppp Virtual-Template1

!

interface Virtual-Template1	Interface for virtual link on top

ip address negotiated	of ADSL connection. All the PPP

no ip redirects	settings are site dependent.

no ip proxy-arp

ip nat outside

pulse-time 0

ppp chap hostname UserNetissimo

ppp chap password 7 00574404035D1207

no ppp chap wait

ppp pap sent-username UserNetissimo password 7 04085C040827554F

crypto map CMMYMAP

!

ip nat pool NET 193.253.191.14 193.253.191.14 netmask 255.255.255.0

ip nat inside source list 101 pool NET overload	The IP address 193.253.191.14 is

ip classless	the IP address given by the ISP.

ip route 0.0.0.0 0.0.0.0 193.253.191.1	It is used to translate internal

ip http server	private addresses to IP addresses

!	in conformance with Internet rules.

access-list 1 permit any

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

access-list 101 deny   ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

access-list 101 permit ip any any	See comments on ACL above.

!

line con 0

transport input none

line aux 0

line vty 0 4

password PWD

login

!

end

ISDN router

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ISDN

!

enable secret 5 $1$n6eZ$UXI24gGl7lZu/wB3OZVnC1

enable password PWD

!

memory-size iomem 25

ip subnet-zero

no ip finger

no ip domain-lookup

!

isdn switch-type vn3

!

crypto isakmp policy 1

authentication pre-share

crypto isakmp key CléIPSEC address 193.253.191.14

!

crypto IPSec transform-set CMT esp-null esp-sha-hmac

!

crypto map CMYMAP 1 IPSec-isakmp

set peer 193.253.191.14	This IP address is the other end

set security-association lifetime seconds 600	of the VPN tunnel. It depends on the

set transform-set CMT	local situation.

match address 100

!

cns event-service server

!

interface BRI0

no ip address

ip nat outside

encapsulation ppp

no ip mroute-cache

dialer pool-member 1

isdn switch-type vn3

isdn send-alerting

no cdp enable

crypto map CMYMAP

!

interface FastEthernet0

ip address 131.1.8.1 255.255.0.0	Internet protocol address of local site.

ip nat inside

no ip mroute-cache

speed auto

full-duplex

!

interface Dialer0	The two diallers below are ISP

ip address negotiated	dependent: phone number, username

encapsulation ppp	and password.

dialer remote-name LibertySurf

dialer pool 1

dialer string 0860155555

dialer-group 1

pulse-time 0

no cdp enable

ppp authentication chap pap callin

ppp chap hostname bane0000@lsurf.fr

ppp chap password 7 1307160B04020A2F

ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

crypto map CMYMAP

!

interface Dialer1

ip address negotiated

ip nat outside

encapsulation ppp

dialer pool 1

dialer string 0860155555

dialer-group 1

pulse-time 0

no cdp enable

ppp authentication chap pap callin

ppp chap hostname bane0000@lsurf.fr

ppp chap password 7 1307160B04020A2F

ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

!

ip nat inside source list 101 interface Dialer1 overload

ip classless

ip route 0.0.0.0 0.0.0.0 Dialer1

ip route 130.1.0.0 255.255.0.0 Dialer0

ip route 193.253.191.14 255.255.255.255 Dialer0

no ip http server

!

access-list 1 permit any

access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

access-list 101 deny   ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

access-list 101 permit ip any any	See comments on ACL above.

dialer-list 1 protocol ip permit

!

line con 0

transport input none

line aux 0

line vty 0 4

password PWD

login

!

end

### Performance

The categorization of Cisco routers is a good start for many situations. However, it can be quite tricky when it involves building a complex and/or large VPN solution. The deployment of the right router at the right place will contribute significantly to the overall VPN design in terms of performance as well as cost.

It is important to note that additional Cisco IOS processing will be required for all packets that require security services. The IPSec technology has shown to be slower when compared with Cisco encryption technology. One of the reasons is because of the deployment of authentication algorithms that are considered to be slow.

Nonetheless, security literature has highlighted lack of authentication as a security vulnerability.

Without compromising security, IPSec must be the technology to deploy. In addition, IPSec introduces packet expansion, which is more likely to require fragmentation and reassembly of IPSec-protected IP datagrams. The encrypted packets will also be authenticated, which means that most packets will have two cryptographic operations performed on them.

Performance figures for the different solutions can be found at http://www.cisco.com/go/IPSec.

Performance impact can also occur when using VPN connections via satellite Internet links. Experience from testing carried out by Meteo France showed the following. In some configurations, especially when crossing satellite links, the Maximum Transfer Unit (MTU) is significantly reduced, and large IPSEC packets need to be fragmented. If the Don't Fragment (DF) bit is set, such packets are dropped. The result is that small packets (authentication sequences) can pass, whereas large packets (data transfers) are dropped. To avoid this, a recommendation is to clear the DF bit (on a Cisco router: “crypto IPSec df-bit-clear”).

## Example: Cisco DMVPN configuration

### Test environment

The configurations presented in this section are valid for a “Single DMVPN/Dual Hub” DMVPN topology in which HUB1 is the primary DMVPN hub router and HUB2 is the secondary one.

Figure A2. DMVPN setup with primary and secondary hub router

Each spoke site has permanent IPSec tunnels towards both hubs in order to exchange information about the DMVPN topology. All the routers have direct Internet access, and no NAT is deployed. X.509 certificates are used for device authentication. Enhanced Interior Gateway Routing Protocol (EIGRP) is used to exchange routing information within the DMVPN cloud.

### HUB1 configuration

HUB1 router

This configuration file was truncated to allow for easier reading.

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname HUB1

!

crypto pki trustpoint w3cert

enrollment terminal

fqdn HUB1.rmdcn.com

revocation-check none

rsakeypair DMVPN.test.com

!

crypto pki certificate chain w3cert

certificate 6306820854F75290

…

F0363478 D576

quit

certificate ca 7399989A358E3CE2

30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

...

4E0D71EE 6E55FA0F 7AF0

quit

!

crypto isakmp policy 1

encr aes 256

group 2

!

crypto isakmp policy 2

encr 3des

group 2

crypto isakmp nat keepalive 5

!

crypto IPSec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

mode transport

crypto IPSec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

mode transport

!

crypto IPSec profile DMVPN_PROFILE

set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

description HUB1 DMVPN Interface

bandwidth 1000

ip address 192.168.200.1 255.255.255.0

no ip redirects

ip mtu 1400

no ip next-hop-self eigrp 1

ip nhrp authentication DMVPNVPN

ip nhrp map multicast dynamic

ip nhrp network-id 1

ip nhrp holdtime 360

ip nhrp cache non-authoritative

ip tcp adjust-mss 1360

no ip split-horizon eigrp 1

no ip mroute-cache

delay 900

cdp enable

tunnel source FastEthernet0

tunnel mode gre multipoint

tunnel key 100000

tunnel protection IPSec profile DMVPN_PROFILE

!

interface FastEthernet0

description To the INTERNET

ip address 172.16.1.1 255.255.255.0

!

interface FastEthernet1

description To local LAN

ip address 10.157.254.1 255.255.255.0

!

router eigrp 1

redistribute static

network 192.168.200.0

distribute-list route-map ADD-TAG out Tunnel0

no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.1.254

!

ip prefix-list EIGRP-Local-Networks-To-Cloud description Routes locally advertised to Cloud

ip prefix-list EIGRP-Local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

description Tag any local route with local number

match ip address prefix-list EIGRP-Local-Networks-To-Cloud

set tag 1

!

route-map ADD-TAG permit 20

description Allow HUB sites to advertise spoke sites networks

match tag 1

!

route-map ADD-TAG deny 30

description Deny anything else

!

end

### HUB2 configuration

HUB2 router

This configuration file was truncated to allow for easier reading.

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname HUB2

!

crypto pki trustpoint w3cert

enrollment terminal

revocation-check none

rsakeypair DMVPN.test.com

!

crypto pki certificate chain TP-self-signed-3064311813

crypto pki certificate chain w3cert

certificate 0D7B18996CB51149

30820303 3082026C A0030201 0202080D 7B18996C B5114930 0D06092A 864886F7

...

BB12CFFA FE67A6

quit

certificate ca 7399989A358E3CE2

30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

...

4E0D71EE 6E55FA0F 7AF0

quit

!

crypto isakmp policy 1

encr aes 256

group 2

!

crypto isakmp policy 2

encr 3des

group 2

crypto isakmp nat keepalive 5

!

crypto IPSec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

mode transport

crypto IPSec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

mode transport

!

crypto IPSec profile DMVPN_PROFILE

set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

description HUB2 DMVPN Interface

bandwidth 1000

ip address 192.168.200.8 255.255.255.0

no ip redirects

ip mtu 1400

no ip next-hop-self eigrp 1

ip nhrp authentication DMVPNVPN

ip nhrp map multicast dynamic

ip nhrp map 192.168.200.1 172.16.1.1

ip nhrp map multicast 172.16.1.1

ip nhrp network-id 1

ip nhrp holdtime 360

ip nhrp nhs 192.168.200.1

ip nhrp cache non-authoritative

ip tcp adjust-mss 1360

no ip split-horizon eigrp 1

no ip mroute-cache

delay 1000

cdp enable

tunnel source FastEthernet0/0

tunnel mode gre multipoint

tunnel key 100000

tunnel protection IPSec profile DMVPN_PROFILE

!

interface FastEthernet0/0

description To the INTERNET

ip address 172.16.8.1 255.255.255.0

!

interface FastEthernet0/1

description To local LAN

ip address 10.157.254.8 255.255.255.0

!

router eigrp 1

network 192.168.200.0

distribute-list route-map ADD-TAG out Tunnel0

no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.8.254

!

ip prefix-list EIGRP-Local-Networks-To-Cloud description Routes locally advertised to Cloud

ip prefix-list EIGRP-Local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

description Tag any local route with local number

match ip address prefix-list EIGRP-Local-Networks-To-Cloud

set tag 1

!

route-map ADD-TAG permit 20

description Allow HUB sites to distribute spoke sites networks

match tag 1

!

route-map ADD-TAG deny 30

description Deny anything else

!

end

### SPOKE configuration

SPOKE router

This configuration file was truncated to allow for easier reading.

!

version 12.4

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname SPOKE

!

crypto pki trustpoint w3cert

enrollment terminal

revocation-check none

rsakeypair DMVPN.rmdcn.com

!

crypto pki certificate chain TP-self-signed-266594605

crypto pki certificate chain w3cert

certificate 00D481DE90432A3E

30820304 3082026D A0030201 02020800 D481DE90 432A3E30 0D06092A 864886F7

...

7ED1CD42 68E8FDD1

quit

certificate ca 7399989A358E3CE2

30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

...

4E0D71EE 6E55FA0F 7AF0

quit

!

crypto isakmp policy 1

encr aes 256

group 2

!

crypto isakmp policy 2

encr 3des

group 2

crypto isakmp nat keepalive 5

!

crypto IPSec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

mode transport

crypto IPSec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

mode transport

!

crypto IPSec profile DMVPN_PROFILE

set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

description SPOKE DMVPN Interface

bandwidth 1000

ip address 192.168.200.4 255.255.255.0

no ip redirects

ip mtu 1400

ip nhrp authentication DMVPNVPN

ip nhrp map 192.168.200.1 172.16.1.1

ip nhrp map multicast 172.16.1.1

ip nhrp map multicast 172.16.8.1

ip nhrp map 192.168.200.8 172.16.8.1

ip nhrp network-id 1

ip nhrp holdtime 360

ip nhrp nhs 192.168.200.1

ip nhrp nhs 192.168.200.8

ip nhrp cache non-authoritative

ip tcp adjust-mss 1360

no ip mroute-cache

delay 1000

cdp enable

tunnel source FastEthernet4

tunnel mode gre multipoint

tunnel key 100000

tunnel protection IPSec profile DMVPN_PROFILE

!

interface FastEthernet4

description To the INTERNET

ip address 172.16.4.1 255.255.255.0

!

interface Vlan1

description To local LAN

ip address 10.157.254.4 255.255.255.0

!

router eigrp 1

network 192.168.200.0

distribute-list route-map ADD-TAG out Tunnel0

no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.4.254

!

ip prefix-list EIGRP-local-Networks-To-Cloud description locally advertised to Cloud

ip prefix-list EIGRP-local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

description Tag any local route with local number

match ip address prefix-list EIGRP-local-Networks-To-Cloud

set tag 4

!

route-map ADD-TAG deny 30

description Deny anything else

!

end

## Configuration example for an OpenVPN-based solution

### Server configuration

By starting up with the server configuration, the first thing we have to do is to create a series of certificates and keys. For all these, we take advantage of the OpenVPN features by having a number of pre-set scripts to carry out the job, so that to follow an order, the recommended action is to copy the directory where those scripts are stored into the “/etc/openvpn” files configuration directory.

Copy the “easy-rsa” directory to /etc/openvpn; this can usually be found at /usr/share/openvpn/2.0/easy-rsa or /usr/share/doc/openvpn-2*/easy-rsa:

#cp -a /usr/share/doc/openvpn-2*/easy-rsa /etc/openvpn

#cd /etc/openvpn/easy-rsa

Note: Former configuration files that remain in the /etc/openvpn directory should have to be removed in order to avoid making futures mistakes.

### Generate the Certificate Authority

This certificate has to be generated just once in the server and then will be one of the files to be copied and submitted to the clients (Met Centres).

#source vars

By executing vars, we are initializing the environment variables and something like this is displayed:

NOTE: when you run ./clean-all, we are doing a “rm –rf” on /etc/openvpn/easy-rsa/keys

Then, let us initialize the keys directory (/etc/openvpn/easy-rsa/keys): just in case old files remain, the script below cleans them up:

#sh clean-all   or   #./clean-all

Then, we execute the following script:

#sh build-ca

The Certificate Authority (CA) is generated and some information is asked which is recommendable that we shall provide as precisely as possible since in future configurations, some data will be required, as an example:

Generating a 1024 bit RSA private key

...............++++++

...............................++++++

writing a new private key to ‘ca.key’

-----

You are about to be asked to enter information that will be incorporated

in your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter ‘.’, the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR

State or Province Name (full name) [NA]:Buenos Aires

Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires

Organization Name (eg, company) [OpenVPN-TEST]: press “enter” (default)

Organizational Unit Name (eg, section) []:CRTBA

Common Name (eg, your name or your server’s hostname) []:SMN

Email Address [me@myhost.mydomain]:xxx1@yyy.ar (changed to avoid use of real e-mail address)

Note: don’t let the Common Name blank!

### Generate the certificate and key for the server

#sh build-key-server servidor

Again, some information is queried: country name, state or province name, locality name and organization name have to be the same entries as in the CA certificate; the rest could be different, especially common name, in this example, “servidor”:

# sh build-key-server servidor

Generating a 1024 bit RSA private key

.........++++++

.........++++++

Writing a new private key to ‘servidor.key’

-----

You are about to be asked to enter information that will be incorporated

into your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter ‘.’, the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR

State or Province Name (full name) [NA]:Buenos Aires

Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires

Organization Name (eg, company) [OpenVPN-TEST]: press “enter” (default)

Organizational Unit Name (eg, section) []:CRTBA

Common Name (eg, your name or your server’s hostname) []:servidor

Email Address [me@myhost.mydomain]:xxx2@yyy.ar (changed to avoid use of real e-mail address)

Please enter the following ‘extra’ attributes

to be sent with your certificate request

A challenge password []:press “enter” (default)

An optional company name []:press “enter” (default)

Using configuration from /etc/openvpn/easy-rsa/openssl.cnf

Check that the request matches the signature

Signature ok

The Subject’s Distinguished Name is as follows

country Name           :PRINTABLE:‘AR’

stateOrProvince Name   :PRINTABLE:‘Buenos Aires’

localityName          :PRINTABLE:‘Ciudad de Buenos Aires’

organizationName      :PRINTABLE:‘OpenVPN-TEST’

organizationalUnitName:PRINTABLE:‘CRTBA’

commonName            :PRINTABLE:‘servidor’

emailAddress          :IA5STRING:‘ xxx2@yyy.ar’ (changed to avoid use of real e-mail address)

Certificate is to be certified until Nov 21 18:52:04 2018 GMT (3650 days)

Sign the certificate? [y/n]:y

1 out of 1 certificate requests certified, commit? [y/n]y

Write out database with 1 new entries

Data Base Updated

Two files have been created inside /etc/openvpn/easy-rsa/keys and have to be copied to /etc/openvpn/

* servidor.crt

* servidor.key

### 3.4	Generate certificates and keys for the clients (Met Centres)

As carried out previously for the server, we shall do the same for each of the clients, that is, certificates and encrypt keys are created uniquely for all of them.

# sh build-key clienteBR

Similar to the server, queried information such as country name, state or province name, locality name and organization name have to be the same as the input in the CA certificate; the rest could be different, especially common name, in this example, “clienteBR”:

Generating a 1024 bit RSA private key

............++++++

..............................++++++

writing a new private key to ‘clienteBR.key’

-----

You are about to be asked to enter information that will be incorporated

in your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter ‘.’, the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR

State or Province Name (full name) [NA]:Buenos Aires

Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires

Organization Name (eg, company) [OpenVPN-TEST]:

Organizational Unit Name (eg, section) []:CRTBA

Common Name (eg, your name or your server’s hostname) []:clienteBR

Email Address [me@myhost.mydomain]: xxx3@yyy.br (changed to avoid use of real e-mail address)

Please enter the following ‘extra’ attributes

to be sent with your certificate request

A challenge password []:

An optional company name []:

Using configuration from /etc/openvpn/easy-rsa/openssl.cnf

Check that the request matches the signature

Signature ok

The Subject’s Distinguished Name is as follows

countryName           :PRINTABLE:‘AR’

stateOrProvinceName   :PRINTABLE:‘Buenos Aires’

localityName          :PRINTABLE:‘Ciudad de Buenos Aires’

organizationName      :PRINTABLE:‘OpenVPN-TEST’

organizationalUnitName:PRINTABLE:‘CRTBA’

commonName            :PRINTABLE:‘clienteBR’

emailAddress          :IA5STRING:‘ xxx3@yyy.br’ (changed to avoid use of real e-mail address)

Certificate is to be certified until Nov 21 19:00:49 2018 GMT (3650 days)

Sign the certificate? [y/n]:y

1 out of 1 certificate requests certified, commit? [y/n]y

Write out database with 1 new entries

Data Base Updated

If more certificates or keys have to be generated for other clients, the previous step should be repeated, and just the passed parameter should be changed, for example:

#sh build-key clienteVE

#sh build-key clienteUY

#sh build-key clientePY

In this way, we are generating certificates and encrypted keys with the names clienteVE, clienteUY, etc.

### Generate Diffie–Hellman parameters

The Diffie–Hellman parameters are generated by executing this command, which will provide us with a safety key exchange:

#sh build-dh

Generating DH parameters, 1024 bit long safe prime, generator 2

This is going to take a long time

................................................+......................................+.....+.........................................................+.....

.+...........+.............................................................+...........................................................+.....................

....................................................+...............................+............+..........+..........................................+.....

............................................+................................+...............................+.....................+.........................

.+.................+..............................................+.........................................................+...................++*++*++*

The above command shall create the file dh1024.pem inside the directory /etc/openvpn/easy-rsa/, which should be copied to the /etc/openvpn directory.

### Copying files here and there

Once the previous steps have been carried out, we have several files stored in /etc/openvpn/easy-rsa/keys; any of them have to be copied into the client and others into the server:

Server: The following files have to be copied into /etc/openvpn:

ca.crt

ca.key

servidor.key

servidor.crt

dh1024.pem

Client: The following files have to be copied into the /etc/openvpn/ directory:

ca.crt

clienteBR.crt

clienteBR.key

Server configuration

We need to rely on the existence of a configuration file either in the client or in the server. This will contain all configuration parameters, it should have the .conf extension, and of course will be stored in the /etc/openvpn directory:

servidor.conf

port 1194

proto udp

dev tun

ca /etc/openvpn/ca.crt

cert /etc/openvpn/servidor.crt

key /etc/openvpn/servidor.key

dh /etc/openvpn/dh1024.pem

server 172.16.0.0 255.255.255.0

ifconfig-pool-persist ipp.txt

ifconfig-pool-linear

push “route 10.1.0.0 255.255.0.0”

keepalive 10 120

#comp-lz0

user nobody

group nobody

persist-key

persist-tun

status openvpn-status.log

verb 4

Some parameters include:

ca: Name of the certificate authority

cert: Name of the server certificate

key: Name of the encrypt key

dh: Name of the Diffie–Hellman parameter

Server: IP range assigned to the client’s “Tun” interface. The server will have .1, for our example, 172.16.0.1

push: Indicates to the server which IP subnet could be reached by the connected clients

### Client configuration

client

dev tun

proto udp

remote 200.16.116.114

resolv-retry infinite

nobind

user nobody

group nobody

persist-key

persist-tun

ca /etc/openvpn/ca.crt

cert /etc/openvpn/clienteBr.crt

key /etc/openvpn/clienteBr.key

#comp-lzo

verb 4

Client: Act as a client and some configurations will be taken from the server:

nobind: Do not act as a server just a client

ca: Name of the certificate authority

cert: Name of the client certificate

key: Name of the encrypt key

Note: In addition to the present configuration, it is possible, with a small amount of configuration expanding the scope of the VPN, to include additional machines on either the client or server subnet.

More information can be found at: http://en.wikipedia.org/wiki/Virtual_private_network and http://openvpn.net/.

## GLOSSARY

The following glossary is based on http://www.freeswan.org/freeswan_trees/freeswan-1.95/doc/glossary.html.

3DES (triple DES)

Using three Data Encryption Standard (DES) encryptions on a single data block, with at least two different keys, to obtain higher security than available from a single DES pass. Internet Protocol Security (IPSec) always carries out 3DES with three different keys, as required by Request for Comments (RFC) 2451. For an explanation of the two-key variant, see two-key triple DES. Both use an encrypt–decrypt–encrypt (EDE) sequence of operations. Single DES is insecure. Double DES is ineffective. Using two 56 bit keys, one might expect an attacker to have to do 2112 operations to break it. In fact, only 257 operations are required with a meet-in-the-middle attack, though a large amount of memory is also required. Triple DES is vulnerable to a similar attack, but that just reduces the operations factor from the 2168 that one might expect to 2112. This provides adequate protection against brute-force attacks, and no better attack is known. Triple DES can be somewhat slow compared to other ciphers. It requires three DES encryptions per block. Data Encryption Standard was designed for hardware implementation and includes some operations that are difficult in software. However, the speed we obtain is quite acceptable for many uses.

AES

The Advanced Encryption Standard, a new block cipher standard to replace DES being developed by the United States National Institute of Standards and Technology (NIST). Data Encryption Standard used 64 bit blocks and a 56 bit key. Advanced Encryption Standard ciphers use a 128 bit block and are required to support 128, 192 and 256 bit keys. Some of them also support other sizes. The larger block size helps resist birthday attacks, while the large key size prevents brute-force attacks. Fifteen proposals meeting the basic criteria of NIST were submitted in 1998 and subjected to intense discussion and analysis, round one evaluation. In August 1999, NIST narrowed the field to five round two candidates:

    - Mars from IBM

    - RC6 from RSA

    - Rijndael from two Belgian researchers

    - Serpent, a British–Norwegian–Israeli research collaboration

    - Twofish from the consulting firm Counterpane

Three of the five finalists – Rijndael, Serpent and Twofish – have completely open licences. In October 2000, NIST announced the winner: Rijndael.

For more information, see:

    - National Institute of Standards and Technology AES homepage

    - The Block Cipher Lounge AES page

    - Brian Gladman’s code and benchmarks

    - Helger Lipmaa’s survey of implementations

Adding one or more AES ciphers to Linux FreeS/WAN would be a useful undertaking. It is likely that one would add all three of the round two candidates with good licences. A complication is that our code is built for a 64 bit block cipher and AES uses a 128 bit block. Volunteers via the mailing lists would be welcome.

AH

Internet Protocol Security Authentication Header (AH), added after the Internet Protocol (IP) header. For details, see IPSec documents and/or RFC 2402.

Authentication

Ensures that a message originated from the expected sender and has not been altered on the route. Internet Protocol Security uses authentication in two places:

    - Peer authentication, authenticating the players in Internet Key Exchange (IKE) Diffie–Hellman key exchanges to prevent man-in-the-middle attacks. This can be carried out in a number of ways. The methods supported by FreeS/WAN can be found at http://www.freeswan.org.

    - Packet authentication, authenticating packets on an established Security Association (SA), either with a separate AH or with the optional authentication in the Encapsulating Security Payload (ESP) protocol. In either case, packet authentication uses a hashed message authentication code (HMAC) technique.

Outside IPSEC, passwords are perhaps the most common authentication mechanism. Their function is essentially to authenticate the person’s identity to the system. Passwords are generally only as secure as the network they travel over. If you send a cleartext password over a tapped phone line or over a network with a packet sniffer on it, the security provided by that password becomes zero. Sending an encrypted password is no better; the attacker merely records it and reuses it at his convenience. This is called a replay attack. A common solution to this problem is a challenge-response system. This defeats simple eavesdropping and replay attacks. Of course, an attacker might still try to break the cryptographic algorithm used, or the random number generator.

Birthday paradox

Not really a paradox, just a rather counter-intuitive mathematical fact. In a group of 23 people, the chance of at least one pair having the same birthday is over 50 per cent. The second person has 1 chance in 365 (ignoring leap years) of matching the first. If they do not match, the third person’s chances of matching one of them are 2/365, the fourth, 3/365, and so on. The total of these chances grows more quickly than one might guess.

Block cipher

A symmetric cipher that operates on fixed-sized blocks of plaintext giving a block of ciphertext for each. Contrast with stream cipher. Block ciphers can be used in various modes when multiple blocks are to be encrypted. Data Encryption Standard is among the best-known and widely used block ciphers, but it is now obsolete. Its 56 bit key size makes it highly insecure today. The current generation of block ciphers – such as Blowfish, the symmetric block cipher CAST-128 and the International Data Encryption Algorithm (IDEA) – all use 64 bit blocks and 128 bit keys. The next generation, AES, uses 128 bit blocks and supports key sizes up to 256 bits. More information can be found at http://www.cryptolounge.org/wiki/Block_Cipher.

Blowfish

A block cipher using 64 bit blocks and keys of up to 448 bits. Designed by Bruce Schneier, and used in several products. This is not required by IPSEC RFCs.

Brute-force attack (exhaustive search)

This means breaking a cipher by trying all possible keys. This is always possible in theory (except against a one-time pad), but it becomes practical only if the key size is inadequate. Longer keys protect against brute-force attacks. Each extra bit in the key doubles the number of possible keys and therefore doubles the work a brute-force attack must do. A large enough key defeats any brute-force attack. For example, the DES Cracker from Electronic Frontier Foundation (EFF) searches a 56 bit key space in an average of a few days. Let us assume an attacker that can find a 64 bit key (256 times harder) by brute-force search in 1 second (a few hundred thousand times faster). For a 96 bit key, that attacker needs 232 seconds, about 135 years. Against a 128 bit key, he needs 232 times that, over 500 billion years. Your data are then obviously secure against brute-force attacks. Even if our estimate of the attacker’s speed is off by a factor of a million, it still takes him over 500 thousand years to crack a message. This is why:

    - Single DES is now considered dangerously insecure

    - All current generations of block ciphers use 128 bit or longer keys

    - Advanced Encryption Standard ciphers support keysizes of 128, 192 and 256 bits

    - Any cipher we add to Linux FreeS/WAN will have at least a 128 bit key

Cautions

Inadequate keylength always indicates a weak cipher, but it is important to note that adequate keylength does not necessarily indicate a strong cipher. There are many attacks other than brute force, and adequate keylength only guarantees resistance to brute force. Any cipher, whatever its key size, will be weak if design or implementation flaws allow other attacks. In addition, once you have an adequate keylength (somewhere about 90 or 100 bits), adding more key bits makes no practical difference, even against brute force. Consider our 128 bit example above that takes 500 billion years to break by brute force. We really do not care how many zeros there are on the end of that, as long as the number remains ridiculously large. That is, we do not care exactly how large the key is as long as it is large enough. There may be reasons of convenience in the design of the cipher to support larger keys. For example, Blowfish allows up to 448 bits and RC4 up to 2 048 bits, but beyond 100 odd bits, it makes no difference to practical security.

CBC mode

Cipher block chaining (CBC) mode, a method of using a block cipher in which, for each block except the first, the result of the previous encryption is  Exclusive Or’d (XORed) into the new block before it is encrypted. Cipher block chaining is the mode used in IPSEC. An initialization vector (IV) must be provided. It is XORed into the first block before encryption. The IV need not be secret, but should be different for each message and unpredictable.

Cipher modes

Different ways of using a block cipher when encrypting multiple blocks. Four standard modes were defined for DES in Federal Information Processing Standard (FIPS) 81. They can actually be applied with any block cipher:

- Electronic Code Book (ECB): Encrypt each block independently

- Cipher Block Chaining: XOR previous block cipher text into new block plaintext before encrypting new block

- Cipher feedback

- Output feedback

IPSec uses CBC mode since this is only marginally slower than ECB mode, and is more secure. In ECB mode, the same plaintext always encrypts to the same ciphertext, unless the key is changed. In CBC mode, this does not occur. Various other modes are also possible, but none of them are used in IPSec.

Ciphertext

The encrypted output of a cipher, as opposed to the unencrypted plaintext input.

Client

This term has at least two distinct uses in discussing IPSec:

    - The clients of an IPSEC gateway are the machines it protects, typically on one or more subnets behind the gateway. In this usage, all the machines on an office network are clients of that office’s IPSEC gateway. Laptops or home machines connecting to the office, however, are not clients of that gateway. They are remote gateways, running the other end of an IPSEC connection. Each of them is also its own client.

    - IPSec client software is used to describe software that runs on various stand-alone machines to let them connect to IPSEC networks. In this usage, a laptop or home machine connecting to the office is a client machine.

We generally use the term in the first sense. Vendors of Windows IPSEC solutions often use it in the second.

Denial of service (DoS) attack

This is an attack that aims at denying some service to legitimate users of a system, rather than providing a service to the attacker:

    - One variant is a flooding attack, overwhelming the system with too many packets, too much e-mail, or whatever.

    - A closely related variant is a resource exhaustion attack. For example, consider a TCP SYN flood attack. Setting up a TCP connection involves a three-packet exchange:

- Initiator: connection please (SYN);

- Responder: OK (ACK);

- Initiator: OK here too.

If the attacker puts bogus source information in the first packet, such that the second is never delivered, the responder may wait a long time for the third to come back. If the responder has already allocated memory for the connection data structures, and if many of these bogus packets arrive, the responder may run out of memory.

- Another variant is to feed the system indigestible data, hoping to make it sick. For example, IP packets are limited in size to 64 Kbytes, and a fragment carries information on where it starts within that 64 K and how long it is. The ping of death delivers fragments that say, for example, start at 60 K and are 20 K long. Attempting to reassemble these without checking for overflow can be fatal.

The two example attacks discussed were both quite effective when first discovered, capable of crashing or disabling many operating systems. They were also well publicized, and today, far fewer systems are vulnerable to them.

DES

The Data Encryption Standard, a block cipher with 64 bit blocks and a 56 bit key. Probably the most widely used symmetric cipher ever devised. The DES has been a United States government standard for their own use (only for unclassified data), and for some regulated industries such as banking, since the late 1970s. DES is seriously insecure against current attacks.

DH

See Diffie–Hellman.

Diffie–Hellman (DH) key exchange protocol

This is a protocol that allows two parties without any initial shared secret to create one in a manner immune to eavesdropping. Once they have done this, they can communicate privately by using that shared secret as a key for a block cipher or as the basis for key exchange. The protocol is secure against all passive attacks, but it is not at all resistant to active man-in-the-middle attacks. If a third party can impersonate Bob to Alice and vice versa, then no useful secret can be created. Authentication of the participants is a prerequisite for safe Diffie–Hellman key exchange. IPSec can use any of several authentication mechanisms. Those supported by FreeS/WAN are discussed in our configuration section. The Diffie–Hellman key exchange is based on the discrete logarithm problem and is secure unless someone finds an efficient solution to that problem. Given a prime p and generator g (explained under discrete log below), Alice:

    - Generates a random number a

    - Calculates A = g^a modulo p

    - Sends A to Bob

Meanwhile, Bob:

    - Generates a random number b

    - Calculates B = g^b modulo p

    - Sends B to Alice

Now, Alice and Bob can both calculate the shared secret s = g^(ab). Alice knows a and B, so she calculates s = B^a. Bob knows A and b, so he calculates s = A^b. An eavesdropper will know p and g since these are made public, and can intercept A and B, but, short of solving the discrete log problem, these do not let him or her discover the secret s.

Digital signature

Sender:

    - Calculates a message digest of a document

    - Encrypts the digest with his or her private key, using some public key cryptosystem

    - Attaches the encrypted digest to the document as a signature

Receiver:

    - Calculates a digest of the document (not including the signature)

    - Decrypts the signature with the signer’s public key

    - Verifies that the two results are identical

If the public key system is secure and the verification succeeds, then the receiver knows:

    - That the document was not altered between signing and verification

    - That the signer had access to the private key

Such an encrypted message digest can be treated as a signature since it cannot be created without both the document and the private key, which only the sender should possess. The legal issues are complex, but several countries are moving in the direction of legal recognition for digital signatures.

DMVPN

Dynamic Multipoint Virtual Private Network: a proprietary solution that allows for easy configuration and set up of meshed IPSEC VPNs.

EAR (Export Administration Regulations)

The United States government’s export administration regulations, administered by the Bureau of Export Administration. These have replaced the earlier International Traffic in Arms Regulations (ITAR) regulations as the controls on export of cryptography.

EDE

The sequence of operations normally used in either the three-key variant of triple DES used in IPSec or the two-key variant used in some other systems. The sequence is:

    - Encrypt with key1

    - Decrypt with key2

    - Encrypt with key3

For the two-key version, key1 = key3. The advantage of this EDE order of operations is that it makes it simple to interoperate with older devices offering only single DES. Set key1 = key2 = key3, and you have the worst of both worlds, the overhead of triple DES with the security of single DES. Since single DES is insecure, this is an extremely dubious advantage. The EDE two-key variant can also interoperate with the EDE three-key variant used in IPSEC; just set key1 = key3.

Encryption

Techniques for converting a readable message (plaintext) into apparently random material (ciphertext) that cannot be read if intercepted. A key is required to read the message. Major variants include symmetric encryption in which sender and receiver use the same secret key and public key methods in which the sender uses one of a matched pair of keys and the receiver uses the other. Many current systems, including IPSEC, are hybrids combining the two techniques.

ESP

Encapsulated security payload, the IPSEC protocol that provides encryption. It can also provide an authentication service and may be used with null encryption (which we do not recommend). For details, see RFC 2406.

FIPS

Federal Information Processing Standards, the United States government’s standards for products it buys. These are issued by NIST. Among other things, DES and SHA are defined in FIPS documents. There is an FIPS homepage at NIST.

Hash

See message digest algorithm.

Hashed Message Authentication Code (HMAC)

Uses keyed message digest functions to authenticate a message. This differs from other uses of these functions:

    - In normal usage, the hash function’s internal variables are initialized in some standard way. Anyone can reproduce the hash to check that the message has not been altered.

    - For HMAC usage, you initialize the internal variables from the key. Only someone with the key can reproduce the hash. A successful check of the hash indicates not only that the message is unchanged, but also that the creator knew the key.

The exact techniques used in IPSEC are defined in RFC 2104. They are referred to as HMAC-MD5-96 and HMAC-SHA-96 because they output only 96 bits of the hash. This makes some attacks on the hash functions harder.

HMAC

See Hashed Message Authentication Code.

HMAC-MD5-96

See Hashed Message Authentication Code.

HMAC-SHA-256

See Hashed Message Authentication Code.

HTTP

Hypertext Transfer Protocol.

IDEA

International Data Encryption Algorithm, developed in Europe as an alternative to exportable American ciphers such as DES, which were too weak for serious use. IDEA is a block cipher using 64 bit blocks and 128 bit keys, and is used in products such as Pretty Good Privacy (PGP). IDEA is not required by the IPSEC RFCs and not currently used in Linux FreeS/WAN. IDEA is patented and, with strictly limited exceptions for personal use, using it requires a licence from Ascom.

IKE

Internet Key Xxchange, based on the Diffie–Hellman key exchange protocol.

Initialization Vector (IV)

Some cipher modes, including the CBC mode that IPSec uses, require some extra data at the beginning. These data are called the initialization vector. It need not be secret, but should be different for each message. Its function is to prevent messages that begin with the same text from encrypting to the same ciphertext. That might give an analyst an opening, so it is best prevented.

IP

Internet Protocol.

IPSec

Internet Protocol Security, security functions (authentication and encryption) implemented at the IP level of the protocol stack. It is optional for IPv4 and mandatory for IPv6.

ISAKMP

Internet Security Association and Key Management Protocol, defined in RFC 2408.

LT2P

Layer 2 Tunnelling Protocol.

Man-in-the-middle attack

This is an active attack in which the attacker impersonates each of the legitimate players in a protocol to the other. For example, if Alice and Bob are negotiating a key via the Diffie–Hellman key agreement, and are not using authentication to be certain they are talking to each other, then an attacker able to insert himself in the communication path can deceive both players. Call the attacker Mallory. For Bob, he pretends to be Alice. For Alice, he pretends to be Bob. Two keys are then negotiated, Alice-to-Mallory and Bob-to-Mallory. Alice and Bob each think the key they have is Alice-to-Bob. A message from Alice to Bob then goes to Mallory who decrypts it, reads it and/or saves a copy, re-encrypts using the Bob-to-Mallory key and sends it along to Bob. Bob decrypts successfully and sends a reply that Mallory decrypts, reads, re-encrypts and forwards to Alice. To make this attack effective, Mallory must:

    - Subvert some part of the network in some way that lets him carry out the deception of possible targets: Domain Name System (DNS) servers, router, Alice or Bob’s machine, mail server, etc.

    - Beat any authentication mechanism Alice and Bob use

Strong authentication defeats the attack entirely; this is why IKE requires authentication:

    - Work in real time, delivering messages without introducing a delay large enough to alert the victims

It is not hard if Alice and Bob are using email, but it can be quite difficult in some situations. If he manages it, however, it is devastating. He not only gets to read all the messages, he can alter messages, inject his own, forge anything he likes, etc. In fact, he controls the communication completely.

Manual keying

This is an IPSec mode in which the keys are provided by the administrator. The alternative, automatic keying, is preferred in most cases.

MD4

Message Digest Algorithm four from Ron Rivest of RSA. It was widely used a few years ago, but is now considered obsolete. It has been replaced by its descendants MD5 and SHA.

MD5

Message Digest Algorithm five from Ron Rivest of RSA, an improved variant of MD4. Like MD4, it produces a 128 bit hash. For details, see RFC 1321. It is one of two message digest algorithms available in IPSEC. The other is SHA, which produces a longer hash and is therefore more resistant to birthday attacks, but this is not a concern for IPSEC. The HMAC method used in IPSEC is secure, even if the underlying hash is not particularly strong against this attack.

Meet-in-the-middle attack

A divide-and-conquer attack that breaks a cipher into two parts that work against each other separately, and compares results. Probably the best-known example is an attack on double DES. This applies in principle to any pair of block ciphers, for example, to an encryption system using, say, CAST-128 and Blowfish, but we will describe it for double DES. Double DES encryption and decryption can be written as:

C = E(k2,E(k1,P))

P = D(k1,D(k2,C))

where C is ciphertext, P is plaintext, E is encryption, D is decryption, k1 is one key and k2 is the other key. If we know a P, C pair, we can try to find the keys with a brute-force attack, trying all possible k1, k2 pairs. Since each key is 56 bits, there are 2 112 such pairs, and this attack is painfully inefficient.

The meet-in-the middle attack rewrites the equations to calculate a middle value M:

M = E(k1,P)

M = D(k2,C)

Now, we can try some large number of D(k2,C) decryptions with various values of k2 and store the results in a table. Then, start doing E(k1,P) encryptions, checking each result to see if it is in the table.

With enough table space, this breaks double DES with 256 + 256 = 257 work. Against triple DES, you need 256 + 2 112 ~= 2 112. The memory requirements for such attacks can be prohibitive, but there is a whole body of research literature on methods of reducing them.

Message digest algorithm

An algorithm that takes a message as input and produces a hash or digest of it, a fixed-length set of bits that depend on the message contents in some highly complex manner. Design criteria include making it extremely difficult for anyone to counterfeit a digest or to change a message without altering its digest. One essential property is collision resistance. The main applications are in message authentication and digital signature schemes. Widely used algorithms include MD5 and SHA. In IPSEC, message digests are used for HMAC authentication of packets.

MPLS

Multiprotocol Label Switching.

NIST

United States National Institute of Standards and Technology, responsible for FIPS standards including DES and its replacement, AES.

Non-routable IP address

This is an IP address not normally allowed in the “to” or “from” IP address field header of IP packets. Almost invariably, the phrase “non-routable address” means one of the addresses reserved by RFC 1918 for private networks:

    - 10.anything

    - 172.x.anything with 16 <= x <= 31

    - 192.168.anything

These addresses are commonly used on private networks, for example, behind a Linux machine doing an IP masquerade. Machines within the private network can address each other with these addresses. All packets going outside that network, however, have these addresses replaced before they reach the Internet. If any packets using these addresses do leak out, they do not go far. Most routers automatically discard all such packets. Various other addresses – the 127.0.0.0/8 block reserved for local use, 0.0.0.0, various broadcast and network addresses – cannot be routed over the Internet, but are not normally included in the meaning when the phrase “non-routable address” is used.

Oakley

A key determination protocol, defined in RFC 2412.

Oakley groups

The groups used as the basis of Diffie–Hellman key exchange in the Oakley protocol, and in IKE. Four were defined in the original RFC, and a fifth has been added since.

One-time pad

A cipher in which the key is:

    - As long as the total set of messages to be enciphered

    - Absolutely random

    - Never reused

Given these three conditions, it can easily be proved that the cipher is perfectly secure, in the sense that an attacker with an intercepted message in hand has no better chance of guessing the message than an attacker who has not intercepted the message and only knows the message length. No such proof exists for any other cipher. There are, however, several problems with this perfect cipher. First, it is wildly impractical for most applications. Key management is at best difficult, often completely impossible. Second, it is extremely fragile. Small changes that violate the conditions listed above do not just weaken the cipher a bit; quite often, they destroy its security completely:

    - Reusing the pad weakens it to the point where it can be broken with pencil and paper. With a computer, the attack is trivially easy.

    - Using computer-generated pseudo-random numbers instead of a really random pad completely invalidates the security proof. Depending on the random number generator used, this may also give an extremely weak cipher.

Marketing claims about the unbreakable security of various products that somewhat resemble one-time pads are common. Such claims are one of the surest signs of cryptographic snake oil. Systems marketed with such claims are usually completely worthless. Finally, even if the system is implemented and used correctly, it is highly vulnerable to certain types of attack. If an attacker knows the plaintext and has an intercepted message, he can discover the pad. This does not matter if the attacker is just a passive eavesdropper. It gives him no plaintext that he did not already know and we do not care that he learns a pad that we will never reuse. However, knowing the pad lets an active attacker perform a man-in-the-middle attack, replacing your message with whatever he chooses.

OpenVPN

An open-source solution for an IPSEC VPN.

PKI

Public Key Infrastructure, the things an organization or community needs to set up in order to make public key cryptographic technology a standard part of their operating procedures. There are several PKI products on the market. Typically, they use a hierarchy of certificate authorities. Often, they use Lightweight Directory Access Protocol (LDAP) access to X.509 directories to implement this.

Plaintext

The unencrypted input to a cipher, as opposed to the encrypted ciphertext output.

PPTP

Point-to-point Tunnelling Protocol. Papers discussing weaknesses in it are on http://www.counterpane.com.

Public Key Cryptography

In public key cryptography, keys are created in matched pairs. Encrypt with one half of a pair and only the matching other half can decrypt it. This contrasts with symmetric or secret key cryptography in which a single key known to both parties is used for encryption and decryption. One half of each pair, called the public key, is made public. The other half, called the private key, is kept secret. Messages can then be sent by anyone who knows the public key to the holder of the private key. Encrypt with the public key and you know only someone with the matching private key can decrypt. Public key techniques can be used to create digital signatures and to deal with key management issues, perhaps the hardest part of effective deployment of symmetric ciphers. The resulting hybrid cryptosystems use public key methods to manage keys for symmetric ciphers. Many organizations are currently creating PKIs, to make these benefits widely available.

Public Key Infrastructure

See PKI

RC4

Rivest Cipher four, designed by Ron Rivest of RSA and widely used. Believed to be highly secure with adequate keylength, but often implemented with inadequate keylength to comply with export restrictions.

RC6

Rivest Cipher six, RSA’s AES candidate cipher.

Replay attack

An attack in which the attacker records data and later replays it in an attempt to deceive the recipient.

RFC

Request for Comments, an Internet document. Some RFCs are just informative.

Routable IP address

Most IP addresses can be used as “to” and “from” addresses in packet headers. These are the routable addresses; we expect routing to be possible for them. If we send a packet to one of them, we expect (in most cases, there are various complications) that it will be delivered if the address is in use and will cause an ICMP error packet to come back to us if not. There are also several classes of non-routable IP addresses.

RSA algorithm

Rivest Shamir Adleman public key encryption method, named after its three inventors. The algorithm is widely used and likely to become more so since it became free of patent encumbrances in September 2000. For a full explanation of the algorithm, consult Applied Cryptography (Schneier, 1998). A simple explanation is the following. The great seventeenth century French mathematician Fermat proved that, for any prime p and number x, 0 x^p == x modulo p x^(p − 1) == 1 modulo p, non-zero x. From this, it follows that if we have a pair of primes p, q and two numbers e, d such that:

ed == 1			modulo lcm( p − 1, q − 1)

where lcm() is the least common multiple, then for all x, 0 x^(ed − 1) == 1 modulo pq, non-zero x x^ed == x modulo pq. So, we construct such a set of numbers p, q, e, d and publish the product N = pq and e as the public key. Encryption is then:

c = x^e			modulo N

An attacker cannot deduce x from the ciphertext c, short of either factoring N or solving the discrete logarithm problem for this field. If p, q are large primes (hundreds or thousands of bits), no efficient solution to either problem is known. The receiver, knowing the private key (N and d), can readily find x since:

c^d == (x^e)^d		modulo N

== x^ed		modulo N

== x		modulo N

This gives an effective public key technique, with only a couple of problems. It uses a good deal of computer time, since calculations with large integers are not cheap, and there is no proof it is necessarily secure since no one has proven either factoring or discrete log cannot be carried out efficiently.

SA

Security association, the channel negotiated by the higher levels of an IPSec implementation and used by the lower levels. Security associations are unidirectional; you need a pair of them for two-way communication. An SA is defined by three things – the destination, the protocol (AH or ESP) and the security parameter index (SPI). It is used to index other things such as session keys and initialization vectors. For more details, see RFC 2401.

Security Association

See SA.

SHA

Secure Hash Algorithm, a message digest algorithm developed by the National Security Agency (NSA) for use in the digital signature standard, FIPS number 186 from NIST. Secure hash algorithm is an improved variant of MD4 producing a 160 bit hash, and is one of two message digest algorithms available in IPSec. The other is MD5. Some people do not trust SHA because it was developed by NSA. There is, as far as we know, no cryptographic evidence that SHA is untrustworthy, but this does not prevent that view from being strongly held.

SPI

Security Parameter Index, an index used within IPSEC to keep connections distinct. An SA is defined by destination, protocol and SPI. Without the SPI, two connections to the same gateway using the same protocol could not be distinguished. For more details, see RFC 2401.

SSH

Secure Shell, an encrypting replacement for the insecure Berkeley commands whose names begin with “r” for “remote”: rsh, rlogin, etc.

SSL

Secure Sockets Layer, a set of encryption and authentication services for web browsers, developed by Netscape. Widely used in Internet commerce. It's now replaced by  Transport Layer Security (TLS).

Symmetric cryptography

Symmetric cryptography, also referred to as conventional or secret key cryptography, relies on a shared secret key, identical for sender and receiver. The sender encrypts with that key, the receiver decrypts with it. The idea is that an eavesdropper without the key is unable to read the messages. There are two main types of symmetric cipher: block ciphers and stream ciphers. Symmetric cryptography contrasts with public key or asymmetric systems where the two players use different keys. The great difficulty in symmetric cryptography is, of course, key management. The sender and receiver must have identical keys, and those keys must be kept secret from everyone else. This is not too much of a problem if only two people are involved and they can conveniently meet privately or employ a trusted courier. It is quite a problem, though, in other circumstances. It becomes much worse if there are many people. An application might be written to use only one key for communication among 100 people, for example, but there would be serious problems. Do you actually trust all of them that much? Do they trust each other that much? Should they? What is at risk if that key is compromised? How are you going to distribute that key to everyone without risking its secrecy? What do you do when one of them leaves the company? Will you even know? On the other hand, if you need unique keys for every possible connection between a group of 100, then each user must have 99 keys. You need either 99 × 100/2 = 4 950 secure key exchanges between users or a central authority that securely distributes 100 key packets, each with a different set of 99 keys. Either of these is possible, though tricky, for 100 users. Either becomes an administrative nightmare for larger numbers. Moreover, keys must be changed regularly, so the problem of key distribution comes up again and again. If you use the same key for many messages, then an attacker has more text to work with in an attempt to crack that key. Moreover, one successful crack will give them the text of all those messages. In short, the hardest part of conventional cryptography is key management. Today, the standard solution is to build a hybrid system using public key techniques to manage keys.

TLS

Transport Layer Security, the successor of SSL.

Transport mode

An IPSEC application in which the IPSec gateway is the destination of the protected packets, a machine acts as its own gateway. It contrasts with tunnel mode.

Triple DES

See 3DES.

Tunnel mode

An IPSec application in which an IPSec gateway provides protection for packets to and from another system. It contrasts with transport mode.

Two-key Triple DES

A variant of triple DES or 3DES in which only two keys are used. As in the three-key version, the order of operations is EDE or encrypt–decrypt–encrypt, but in the two-key variant, the first and third keys are the same. Triple DES with three keys has 3 × 56 = 168 bits of key, but has only 112 bit strength against a meet-in-the-middle attack, so it is possible that the two-key versions are just as strong. Triple DES for IPSEC is defined as the three-key variant by RFC 2451. The two-key variant should not be used and is not implemented directly in Linux FreeS/WAN. It cannot be used in automatically keyed mode without major fiddles in the source code.

Virtual Private Network

See VPN.

VPN

Virtual Private Network, a network that can safely be used as if it were private, even though some of its communication uses insecure connections. All traffic on these connections is encrypted. IPSec is not the only technique available for building VPNs, but it is the only method defined by RFCs and supported by many vendors. VPNs are by no means the only thing you can do with IPSec, but they may be the most important application for many users.

Wassenaar Arrangement

International agreement restricting the export of munitions and other tools of war. Unfortunately, cryptographic software is also restricted under the current version of the agreement.

X.509

Standard from the International Telecommunication Union for hierarchical directories with authentication services, used in many PKI implementations.

XOR

“Exclusive Or” is a logical operation that outputs true whenever both inputs differ (one is true, the other is false).

## BIBLIOGRAPHY

Black, D. and D. McGrew, 2008: RFC 5282: Using Authenticated Encryption Algorithms with the Encrypted Payload of the Internet Key Exchange version 2 (IKEv2) Protocol.

Eastlake, D., 2007: RFC 4835, 4305: Cryptographic Algorithm Implementation Requirements for Encapsulating Security Payload (ESP) and Authentication Header (AH).

Ferguson, P. and G. Huston, 1998: What is a VPN?, http://www.potaroo.net/papers/vpn.pdf.

Ferguson, P. and B. Schneier, 2003: A Cryptographic Evaluation of IPSec, http://www.schneier.com/paper-IPSec.html.

Hoffman, P., 2005: RFC 4308: Cryptographic Suites for IPSec.

Housley, R., 2005: RFC 4309: Using Advanced Encryption Standard (AES) CCM Mode with IPSec Encapsulating Security Payload (ESP).

Kent S., 2005a: RFC 4302: IP Authentication Header.

———, 2005b: RFC 4303: IP Encapsulating Security Payload (ESP).

———, 2005c: RFC 4304: Extended Sequence Number (ESN) Addendum to IPSec Domain of Interpretation (DOI) for Internet Security Association and Key Management Protocol (ISAKMP).

Kent, S. and K. Seo, 2005: RFC 4301: Security Architecture for the Internet Protocol.

Schiller, J., 2005: RFC 4307: Cryptographic Algorithms for Use in the Internet Key Exchange Version 2 (IKEv2).

Tiller, J.S., 2000: A Technical Guide to IPSec Virtual Private Networks. ISBN 0-8493-0876-3.