#### FM 205-?? 	IWXXM-XML 	ICAO METEOROLOGICAL INFORMATION

EXCHANGE MODEL (IWXXM 3.0)

205-??.1 		Scope

205-??.1.1 	IWXXM-XML shall be used to represent observations and forecasts, and reports thereof, for international civil aviation, as specified by the Technical Regulations (WMO-No. 49), Volume II – Meteorological Service for International Air Navigation.

205-??.1.2 	IWXXM-XML includes provision for aerodrome routine meteorological reports (METAR), aerodrome special meteorological reports (SPECI), aerodrome forecast (TAF) reports, SIGMET information, AIRMET information, Tropical Cyclone Advisory, Volcanic Ash Advisory and Space Weather Advisory.

Notes:

- SIGMET information is information issued by a meteorological watch office concerning the occurrence or expected occurrence of specified en-route weather phenomena that may affect the safety of aircraft operations.

- AIRMET Information is information issued by a meteorological watch office concerning the occurrence or expected occurrence of specified en-route weather phenomena that may affect the safety of low-level aircraft operations and which was not already included in the forecast issued for low-level flights in the flight information region concerned or sub-area thereof.

- Tropical Cyclone Advisory Information is information issued by a Tropical Cyclone Advisory Centre (TCAC) regarding the position, forecast direction and speed of movement, central pressure and maximum surface wind of tropical cyclones.

- Volcanic Ash Advisory Information is information issued by a Volcanic Ash Advisory Centre (VAAC) regarding the lateral and vertical extent and forecast movement of volcanic ash in the atmosphere following volcanic eruptions.

- Space Weather Advisory Information is information issued by a Space Weather Centre (SWXC) regarding the space weather phenomena expected to affect high-frequency radio communications, communications via satellite, GNSS-based navigation and surveillance systems and/or pose a radiation risk to aircraft occupants.

205-??.1.3 	The requirements classes defined in IWXXM-XML are listed in Table 205-??.1.

#### Table 205-?? .1 . Requirements classes defined in IWXXM-XML

| Requirements classes |  |
| --- | --- |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report, 205-??.3 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- speci, 205-??.4 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- metar, 205-??.5 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast, 205-??.6 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation, 205-??.7 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state, 205-??.8 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range, 205-??.9 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- condition, 205-??.10 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear, 205-??.11 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud, 205-??.12 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind, 205-??.13 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility, 205-??.14 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf, 205-??.15 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast, 205-??.16 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast, 205-??.17 |
| Requirements classes |  |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection, 205-??.18 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- position- collection, 205-??.19 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- evolving- condition- collection, 205-??.20 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection, 205-??.21 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection, 205-??.22 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection, 205-??.23 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition, 205-??.24 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position, 205-??.25 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet, 205-??.26 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet, 205-??.27 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet, 205-??.28 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection, 205-??.29 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition, 205-??.30 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet, 205-??.31 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory, 205-??.32 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions, 205-??.33 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- forecast- conditions, 205-??.34 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory, 205-??.35 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- observed- or- estimated- conditions, 205-??.36 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions, 205-??.37 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated, 205-??.38 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated, 205-??.39 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- forecast, 205-??.40 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory, 205-??.41 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis, 205-??.42 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region, 205-??.43 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- report, 205-??.44 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast, 205-??.45 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- forecast, 205-??.46 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- string- with- nil- reason, 205-??.47 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast, 205-??.48 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer, 205-??.49 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- angle- with- nil- reason, 205-??.50 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- distance- with- nil- reason, 205-??.51 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- length- with- nil- reason, 205-??.52 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- measure- with- nil- reason, 205-??.53 |
| Requirements class | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- velocity- with- nil- reason, 205-??.54 |

#### 205-??.2 	XML schema for IWXXM-XML

Representations of information in IWXXM-XML shall declare the XML namespaces listed in Table 205-??.2 and Table 205-??.3.

Notes:

- Additional namespace declarations may be required depending on the XML elements used within IWXXM-XML.

- The XML schema is packaged in ten XML schema documents (XSD) describing one XML namespace: http:// icao. int/ iwxxm/ 3. 0.

- Schematron schemas providing additional constraints are provided as external files to the XSD defining IWXXM-XML. The canonical location of this file is http:// schemas. wmo. int/ iwxxm/ 3. 0/ rule/ iwxxm. sch.

#### Table 205-?? .2 . XML namespaces defined for IWXXM-XML

| XML namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- |
| http:// icao. int/ iwxxm/ 3. 0 | iwxxm | http:// schemas. wmo. int/ iwxxm/ 3. 0/ iwxxm. xsd |

#### Table 205-?? .3 . External XML namespaces used in IWXXM-XML

| Standard | XML namespace | Default namespace prefix | Canonical location of all-components schema document |
| --- | --- | --- | --- |
| XML schema | http:// www. w3. org/ 2001/ XMLSchema | xs |  |
| XML Schema Instance | http:// www. w3. org/ 2001/ XMLSchema- instance | xsi |  |
| Schematron | http:// purl. oclc. org/ dsdl/ schematron | sch |  |
| XSLT v2 | http:// www. w3. org/ 1999/ XSL/ Transform | xsl |  |
| XML Linking Language | http:// www. w3. org/ 1999/ xlink | xlink | http:// www. w3. org/ 1999/ xlink. xsd |
| ISO 19136:2007 GML | http:// www. opengis. net/ gml/ 3. 2 | gml | http:// schemas. opengis. net/ gml/ 3. 2. 1/ gml. xsd |
| FM 202-16 METCE-XML | http:// def. wmo. int/ metce/ 2013 | metce | http:// schemas. wmo. int/ metce/ 1. 2/ metce. xsd |
| AIXM 5.1.1 | http:// www. aixm. aero/ schema/ 5. 1. 1/ | aixm | http:// www. aixm. aero/ schema/ 5. 1. 1/ AIXM_ Features. xsd |

#### 205-??.3 	Requirements class: Meteorological aerodrome observation report

205-??.3.1 	This requirements class is used to describe the report within which meteorological aerodrome observations, and optionally one or more trend forecasts, are provided.

Note: 	The reporting requirements for routine and special meteorological aerodrome reports are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3 and Appendix 5, section 2.

205-??.3.2 	XML elements describing routine or special meteorological aerodrome reports shall conform to all requirements specified in Table 205-??.4.

205-??.3.3 	XML elements describing routine or special meteorological aerodrome reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.4.

#### Table 205-?? .4 . Requirements class xsd-meteorological-aerodrome-observation-report

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report |  |
| Target type | Data instance |
| Name | Meteorological aerodrome observation report |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation, 205-??.7 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast, 205-??.6 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- report, 205-??.44 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ valid The content model of this element shall have a value that matches the content model of iwxxm: MeteorologicalAerodromeObservationReport which extends iwxxm: Report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ issue- time The XML element iwxxm: issueTime shall contain a valid child XML element gml: TimeInstant to indicate the time of issuance of the report. This should be the same as iwxxm: observationTime except for corrections. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ aerodrome The XML element iwxxm: aerodrome shall contain a valid child element aixm: AirportHeliport to indicate the aerodrome at which the observation took place. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ observation- time The XML element iwxxm: observationTime shall contain a valid child element gml: TimeInstant showing the time at which the phenomena were observed. This may differ from the time reported for forecast conditions |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ observation The XML element iwxxm: observation shall contin a valid child XML element iwxxm: MeteorologicalAerodromeObservation to indicate the meteorological conditions observed at the aerodrome. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ trend- forecast If trend forecasts are reported, the value of XML element iwxxm: trendForecast shall have a valid child element iwxxm: MeteorologicalAerodromeTrendForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ automated- station If the meteorological aerodrome observation included within the report has been generated by an automated system, the value of XML attribute @automatedStation shall be set to “true”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report/ nil- report A NIL report shall have appropriately filled elements including iwxxm: issueTime, iwxxm: aerodrome and iwxxm: observationTime. XML elements iwxxm: observation shall be empty with @nilReason = “http: // codes. wmo. int/ common/ nil/ missing” and iwxxm: trendForecast shall be absent. |

Notes:

- The requirements for reporting the use of an automated system are specified in the Technical Regulations (WMONo. 49), Volume II, Part II, Appendix 3, 4.8.

- If XML attribute @automatedStation is absent, then the value “false” is inferred; for example, the meteorological aerodrome observation included within the report has not been generated by an automated system.

- A NIL report indicates that a routine report has not been provided on the anticipated timescales. Such a report does not contain details of any observed and forecast meteorological conditions

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

#### 205-??.4 	Requirements class: SPECI

205-??.4.1 	This requirements class is used to describe the special meteorological aerodrome reports (SPECI).

205-??.4.2 	XML elements describing SPECIs shall conform to all requirements specified in Table 205-??.5.

205-??.4.3 	XML elements describing SPECIs shall conform to all requirements of all relevant dependencies specified in Table 205-??.5.

#### Table 205-?? .5 . Requirements class xsd-speci

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- speci |  |
| Target type | Data instance |
| Name | SPECI |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report, 205-??.3 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- speci/ valid The content model of this element shall have a value that matches the content model of iwxxm: SPECI which extends iwxxm: MeteorologicalAerodromeObservationReport. |

#### 205-??.5 	Requirements class: METAR

205-??.5.1 	This requirements class is used to describe the routine meteorological aerodrome reports (METAR).

205-??.5.2 	XML elements describing METARs shall conform to all requirements specified in Table 205-??.6.

205-??.5.3 	XML elements describing METARs shall conform to all requirements of all relevant dependencies specified in Table 205-??.6.

#### Table 205-?? .6 . Requirements class xsd-metar

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- metar |  |
| Target type | Data instance |
| Name | METAR |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- report, 205-??.3 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- metar/ valid The content model of this element shall have a value that matches the content model of iwxxm: METAR which extends iwxxm: MeteorologicalAerodromeObservationReport. |

#### 205-??.6 	Requirements class: Meteorological aerodrome trend forecast

205-??.6.1 	This requirements class is used to describe the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in a trend forecast of a routine or special meteorological aerodrome report.

205-??.6.2 	XML elements describing the set of meteorological conditions for inclusion in a trend forecast shall conform to all requirements specified in Table 205-??.7.

205-??.6.3 	XML elements describing the set of meteorological conditions for inclusion in a trend forecast shall conform to all requirements of all relevant dependencies specified in Table 205-??.7.

#### Table 205-?? .7 . Requirements class xsd-meteorological-aerodrome-trend-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast |  |
| Target type | Data instance |
| Name | Meteorological aerodrome trend forecast |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast, 205-??.48 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast, 205-??.45 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: MeteorologicalAerodromeTrendForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml: TimePeriod or gml:TimeInstant to indicate the time at which meteorological phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ phenomenon- time- becmg- without- a- specified- time When a trend forecast is reported with a BECMG block without a specified time and the period commences at the beginning of the trend forecast period and ceases by the end of the trend forecast period this should be represented with an empty element iwxxm: phenomenonTime with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ missing". Otherwise if the BECMG time is uncertain within the trend forecast period this should be represented with an empty element iwxxm: phenomenonTime with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ unknown" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ phenomenon- time- tempo- without- a- specified- time When a trend forecast is reported with a TEMPO block without a specified time and the period commences at the beginning of the trend forecast period and ceases by the end of the trend forecast period this should be represented with an empty element iwxxm: phenomenonTime with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ missing" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ time- indicator The XML element iwxxm: timeIndicator is a human-readable time indicator including “FROM_UNTIL”, “FROM”, “UNTIL”, and “AT” for the trend forecast. This shall match the time period or time instant reported in the phenomenon time. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ prevailing- visiblity If reported, the prevailing visibility shall be stated using the XML element iwxxm: prevailingVisibility with the unit of measure metres, indicated using the XML attribute iwxxm: prevailingVisibility/ @uom with value “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ prevailing- visibility- comparison- operator If present, the value of XML element iwxxm: prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ prevailing- visibility- exceeds- 10000m If the prevailing visibility exceeds 10 000 metres, then the numeric value of XML element iwxxm: prevailingVisibility shall be set to 10000 and the XML element iwxxm: prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ surface- wind Surface wind conditions forecast for the aerodrome shall be reported using the XML element iwxxm: surfaceWind containing a valid child element iwxxm: AerodromeSurfaceWindTrendForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ weather If forecast weather is reported, the value of XML attribute iwxxm: forecastWeather/ @xlink: href shall be the URI of a valid weather phenomenon code from Code table D-7: Aerodrome present or forecast weather. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ weather- nsw The expected end of occurrence of weather phenomena shall be indicated by a single XML element iwxxm: weather with @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ number- of- forecast- weather- codes No more than three forecast weather codes shall be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ cloud If reported, the cloud conditions forecast for the aerodrome shall be expressed using the XML element iwxxm: cloud containing a valid child element iwxxm: AerodromeCloudForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ cloud- nsc When no cloud of operational significance is forecast, and no restriction on vertical visibility and 'CAVOK' is not appropriate, XML element iwxxm: cloud shall be empty with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ change- indicator- becmg If the meteorological conditions forecast for the aerodrome are expected to reach or pass through specified values at a regular or irregular rate, then the XML attribute @changeIndicator shall have the value “BECOMING”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ change- indicator- tempo If temporary fluctuations in the meteorological conditions forecast for the aerodrome are expected to occur, then the XML attribute @changeIndicator shall have the value “TEMPORARY_FLUCTUATIONS”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ cavok If the conditions associated with CAVOK are forecast, then: - The XML attribute @cloudAndVisibilityOK shall the have value “true”; and - The following XML elements shall be absent: iwxxm: prevailingVisibility, iwxxm: prevailingVisibilityOperator, iwxxm: weather and iwxxm: cloud. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- trend- forecast/ nosig If no change of operational significance is forecast, then a single XML element iwxxm: trendForecast shall be empty with attribute @nilReason = “http: // codes. wmo. int/ common/ nil/ noSignificantChange”. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- Temporary fluctuations in the meteorological conditions occur when those conditions reach or pass specified values and last for a period of time less than one hour in each instance and, in the aggregate, cover less than one half the period during which the fluctuations are forecast to occur (Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 2.3.3).

- The use of change groups is specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 2.3 and Appendix 3, Table A3-3.

- Cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1 500 metres or the minimum sector altitude, whichever is greater, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as CAVOK. Use of CAVOK is specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 2.2.

- Visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1 000 candelas can be seen and identified against an unlit background.

- Prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

- The requirements for reporting the following within a trend forecast are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5:

(a) 	Prevailing visibility conditions 	paragraph 2.2.3 (b) Forecast weather phenomena 	section 2.2.4

  - Surface wind conditions 	paragraph 2.2.2

  - Cloud conditions 	paragraph 2.2.5

- The absence of XML element iwxxm: prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

- Code table D-7 is published online at http:// codes. wmo. int/ 49- 2/ AerodromePresentOrForecastWeather.

#### 205-??.7 	Requirements class: Meteorological aerodrome observation

205-??.7.1 	This requirements class is used to describe the aggregated set of meteorological conditions observed at an aerodrome.

205-??.7.2 	XML elements describing the set of meteorological conditions observed at an aerodrome shall conform to all requirements specified in Table 205-??.8.

205-??.7.3 	XML elements describing the set of meteorological conditions observed at an aerodrome shall conform to all requirements of all relevant dependencies specified in Table 205-??.8.

#### Table 205-?? .8 . Requirements class xsd-meteorological-aerodrome-observation

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation |  |
| Target type | Data instance |
| Name | Meteorological aerodrome observation |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind, 205-??.13 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility, 205-??.14 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range, 205-??.9 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud, 205-??.12 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear, 205-??.11 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- condition, 205-??.10 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state, 205-??.8 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ valid The content model of this element shall have a value that matches the content model of iwxxm: MeteorologicalAerodromeObservation. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ air- temperature The air temperature observed at the aerodrome shall be reported in Celsius (°C) using the XML element iwxxm: airTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ air- temperature- not- observable If air temperature cannot be reported due to sensor failure, the XML element iwxxm: airTemperature shall have attributes @xsi:nil = "true", @uom = "N/A", and @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ dew- point- temperature The dewpoint temperature observed at the aerodrome shall be reported in Celsius (°C) using the XML element iwxxm: dewpointTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ dew-point- temperature- not- observable If dew point temperature cannot be reported due to sensor failure, the XML element iwxxm: dewpointTemperature should have attributes @xsi:nil = "true", @uom = "N/A", and @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ qnh The atmospheric pressure, known as QNH, observed at the aerodrome shall be reported in hectopascals (hPa) using the XML element iwxxm: qnh. The value of the associated XML attribute @uom shall be “hPa”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ qnh- not- observable If QNH cannot be reported due to sensor failure, the XML element iwxxm: qnh should have attributes @xsi:nil = "true", @uom = "N/A", and @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ surface- wind Surface wind conditions observed at the aerodrome shall be reported using the XML element iwxxm: surfaceWind containing a valid child element iwxxm: AerodromeSurfaceWind. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ visibility If reported, the horizontal visibility conditions observed at the aerodrome shall be expressed using the XML element iwxxm: visibility containing a valid child element iwxxm: AerodromeHorizontalVisibility. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ visibility- not- observable If visibility cannot be reported due to sensor failure, the XML element iwxxm: visibility should have attributes @xsi:nil = "true", @uom = "N/A", and @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ runway- visual- range If reported, the visual range conditions for a given runway direction shall be expressed using the XML element iwxxm: rvr containing a valid child element iwxxm: AerodromeRunwayVisualRange. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation- record/ number- of- rvr- groups Visual range conditions shall be reported for no more than four runway directions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ runway- visual- range- not- observable If runway visual range cannot be reported due to sensor failure, the XML element iwxxm: rvr should have attributes @xsi:nil = "true", @uom = "N/A", and @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ present- weather If present weather is reported, the value of XML attribute iwxxm: presentWeather/ @xlink: href shall be the URI of a valid weather phenomenon code from Code table D-7: Aerodrome present or forecast weather. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ number- of- present- weather- codes No more than three present weather codes shall be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ present- weather- not- observable If present weather is not observable due to sensor failure or obstruction, the XML element iwxxm: presentWeather should be empty with attribute @nilReason = “http:// codes. wmo. int/ common/ nil/ notObservable”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud If reported, the cloud conditions observed at the aerodrome shall be expressed using the XML element iwxxm: cloud containing a valid child element iwxxm: AerodromeObservedClouds. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud- amount- and- base- not- observed When cumulonimbus clouds or towering cumulus clouds are detected by the automatic observing system and the cloud amount and the height of cloud base cannot be observed, this shall be indicated by an empty XML element iwxxm: cloud/ layer with an attribute @nilReason of either: - "http: // codes. wmo. int/ common/ nil/ notObservable" if the value could not be observed due to a sensor or system failure, or - "http: // codes. wmo. int/ common/ nil/ notDetectedByAutoSystem" if the automated system is working properly but did not observe any values |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud- amount- not- observed When cumulonimbus clouds or towering cumulus clouds are detected by the automatic observing system and the cloud amount cannot be observed, this shall be indicated by an empty XML element iwxxm: cloud/ layer/ amount with an atribute @nilReason of either: - "http: // codes. wmo. int/ common/ nil/ notObservable" if the value could not be observed due to a sensor or system failure, or - "http: // codes. wmo. int/ common/ nil/ notDetectedByAutoSystem" if the automated system is working properly but did not observe any values |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud- base- not- observed When cumulonimbus clouds or towering cumulus clouds are detected by the automatic observing system and the height of cloud base cannot be observed, this shall be indicated by an empty XML element iwxxm: cloud/ layer/ base and an attribute @nilReason of either: - "http: // codes. wmo. int/ common/ nil/ notObservable" if the value could not be observed due to a sensor or system failure, or - "http: // codes. wmo. int/ common/ nil/ notDetectedByAutoSystem" if the automated system is working properly but did not observe any values |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud- type- not- observed When cloud type cannot be observed by an auto system due to sensor limitations, the XML element iwxxm: cloudType shall be empty with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ notObservable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cloud- clr- or- skc When CAVOK is not present but clear sky is reported by automated system or observer, the XML element iwxxm: cloud/ base shall be empty with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ recent- weather If recent weather is reported, the value of XML attribute iwxxm: recentWeather/ @xlink: href shall be the URI of a valid weather phenomenon code from Code table D-6: Aerodrome recent weather. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ number- of- recent- weather- codes No more than three recent weather codes shall be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ wind- shear If reported, the wind shear conditions for the aerodrome shall be expressed using the XML element iwxxm: windShear containing a valid child element iwxxm: AerodromeWindShear. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ sea- condition If reported, the sea-state conditions observed at the aerodrome shall be expressed using the XML element iwxxm: seaCondition containing a valid child element iwxxm: AerodromeSeaCondition. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ runway- state If reported, the surface conditions for a given runway direction shall be expressed using the XML element iwxxm: runwayState containing a valid child element iwxxm: AerodromeRunwayState. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- observation/ cavok If the conditions associated with CAVOK are observed, then: - The XML attribute @cloudAndVisibilityOK shall have the value “true”; and - The following XML elements shall be absent: iwxxm: visibility, iwxxm: rvr, iwxxm: presentWeather andiwxxm: cloud. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- Cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1 500 metres or the minimum sector altitude, whichever is greater, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as CAVOK. Use of CAVOK is specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 2.2.

- The requirements for reporting the following are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3:

  - Air temperature and dewpoint temperature  	section 4.6

  - Atmospheric pressure (QNH)  			section 4.7

  - Present weather  				section 4.4

  - Recent weather  				paragraph 4.8.1.1

  - Surface wind conditions  			section 4.1

  - Runway state  				paragraph 4.8.1.5

(g) 	Aerodrome wind shear  			paragraph 4.8.1.4

(h) 	Observed cloud conditions  			section 4.5

  - Sea state  					paragraph 4.8.1.5

  - Horizontal visibility  				section 4.2

- Code table D-7 is published online at http:// codes. wmo. int/ 49- 2/ AerodromePresentOrForecastWeather.

- Code table D-6 is published online at http:// codes. wmo. int/ 49- 2/ AerodromeRecentWeather.

- Information on runway visual range shall be omitted if the prevailing visibility exceeds 1 500 metres. Details of the requirements for reporting runway visual range are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.3.

#### 205-??.8 	Requirements class: Aerodrome runway state

205-??.8.1 	This requirements class is used to describe the observed runway state.

Notes:

- Representations providing more detailed information may be used if required.

- The requirements for reporting runway state are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.8.1.5.

205-??.8.2 	XML elements describing observed runway state shall conform to all requirements specified in Table 205-??.9.

205-??.8.3 	XML elements describing observed runway state shall conform to all requirements of all relevant dependencies specified in Table 205-??.9.

#### Table 205-?? .9 . Requirements class xsd-aerodrome-runway-state

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state |  |
| Target type | Data instance |
| Name | Aerodrome runway state |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeRunwayState. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ runway If XML attribute @allRunways is absent or has value “false”, then XML element iwxxm: runway, with valid child element iwxxm: runway/ aixm: RunwayDirection, shall be used to indicate the runway direction to which these conditions apply. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ runway- all If XML attribute @allRunways has value “true”, then XML element iwxxm: runway shall be absent. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ runway- snow- closure If the aerodrome is closed due to an extreme deposit of snow, the XML element iwxxm: runway shall be empty with an attribute @nilReason = "http: // codes. wmo. int/ bufr4/ codeflag/ 0-20-085/ 1". |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ deposit- type If deposit type is reported, then the value of XML attribute iwxxm: depositType/ @xlink: href shall be the URI of the valid term from Volume I.2, FM 94 BUFR, Code table 0 20 086: Runway deposits. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ contamination If runway contamination is reported, then the value of XML attribute iwxxm: contamination/ @xlink: href shall be the URI of the valid term from Volume I.2, FM 94 BUFR, Code table 0 20 087: Runway contamination. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ depth- of- deposit If reported, the depth of deposit shall be expressed in millimetres, with the value of XML attribute iwxxm: depthOfDeposit/@uom specified as “mm”. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ depth- of- deposit- unknown When the runway depth of deposits is operationally insignificant or is not measurable this should be indicated by an empty XML element iwxxm: depthOfDeposits with attribute @nilReason of either "http: // codes. wmo. int/ common/ nil/ notObservable" or "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ depth- of- deposit- not- reported When the runway depth of deposits is not reported due to runway or runways non-operational due to snow, slush, ice, large drifts or runway clearance, this should be indicated by an empty XML element iwxxm: depthOfDeposits with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ unknown" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ estimated- surface- friction- or- braking- action If reported, then the value of XML attribute iwxxm: estimatedSurfaceFrictionOrBrakingAction/ @xlink: href shall be the URI of the valid term from Volume I.2, FM 94 BUFR, Code table 0 20 089: Runway friction coefficient. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ cleared If the runway has been cleared of meteorological deposits, then XML attribute @cleared shall have the value “true” and XML elements iwxxm: depositType, iwxxm: contamination and iwxxm: depthOfDeposit shall be absent. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- state/ from- previous- report When a new runway state report is not available in time for dissemination of the METAR message, attribute "fromPreviousReport" shall be set to “true”, XML element attribute iwxxm: runway shall be empty with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable" |

Notes:

- For convenience, FM 94 BUFR, Code table 0 20 086 from Volume I.2, is published online at http:// codes. wmo. int/ bufr4/ codeflag/ 0- 20- 086.

- Runway contamination is expressed as a percentage of the total runway area that is contaminated according to a predefined set of categories: less than 10%, between 11% and 25%, between 25% and 50% and more than 50%. These categories are listed in Volume I.2, FM 94 BUFR, Code table 0 20 087: Runway contamination. For convenience, this code table is published online at http:// codes. wmo. int/ bufr4/ codeflag/ 0- 20- 087.

- FM 94 BUFR, Code table 0 20 089 from Volume I.2, is published online at http:// codes. wmo. int/ bufr4/ codeflag/ 0- 20- 089.

- Units of measurement are specified in accordance with 1.9 above.

#### 205-??.9 	Requirements class: Aerodrome runway visual range

205-??.9.1 	This requirements class is used to describe runway visual range for a specific runway direction at an aerodrome.

Note: 	The requirements for reporting runway visual range are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.3.

205-??.9.2 	XML elements describing runway visual range shall conform to all requirements specified in Table 205-??.10.

205-??.9.3 	XML elements describing runway visual range shall conform to all requirements of all relevant dependencies specified in Table 205-??.10.

#### Table 205-?? .10 . Requirements class xsd-aerodrome-runway-visual-range

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range |  |
| Target type | Data instance |
| Name | Aerodrome runway visual range |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ valid The content model of this element shall have a value that matches the content model of iwxxm: RunwayVisualRange. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ runway The XML element iwxxm: runway, with valid child element aixm: RunwayDirection, shall be used to indicate the runway direction to which these visual range conditions apply. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ mean- rvr The XML element iwxxm: meanRVR shall be used to express the 10-minute average for observed runway visual range measured in metres or, if a marked discontinuity in visual range occurs during the 10-minute period, the average runway visual range following that marked discontinuity, with the value of XML attribute iwxxm: meanRVR/ @uom specified as “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ mean- rvr- operator If present, the value of XML element iwxxm: meanRVROperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ mean- rvr- exceeds- 2000m If the mean runway visual range exceeds 2 000 metres, then the numeric value of XML element iwxxm: meanRVR shall be set to 2000 and the XML element iwxxm: meanRVROperator shall have the value “ABOVE”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ past- tendency If the runway visual range values observed in the 10-minute period have shown a distinct tendency, such that the mean during the first 5 minutes varies by 100 metres or more when compared with the second 5 minutes, this shall be indicated using the XML element iwxxm: pastTendency with value “UPWARD” (visual range is increasing) or “DOWNWARD” (visual range is decreasing) as appropriate. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ past- tendency- no- change If the runway visual range values observed in the 10-minute period have not shown a distinct tendency, this should be indicated using the XML element iwxxm: pastTendency with value “NO_CHANGE”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- runway- visual- range/ past- tendency- missing If it is not possible to determine the tendency, this should be indicated using the XML element iwxxm: pastTendency with value “MISSING”. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- The absence of XML element //iwxxm: AerodromeRunwayVisualRange/ iwxxm: meanRVROperator indicates that the mean runway visual range has the numeric value reported.

- The absence of XML element //iwxxm: AerodromeRunwayVisualRange/ iwxxm: pastTendency indicates that no distinct tendency in visual range has been observed.

#### 205-??.10 	Requirements class: Aerodrome sea condition

205-??.10.1 	This requirements class is used to describe an aggregated set of sea-state conditions reported at an aerodrome.

Note: 	The requirements for reporting sea state are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.8.1.5.

205-??.10.2 	XML elements describing sea condition shall conform to all requirements specified in Table 205-??.11.

205-??.10.3 	XML elements describing sea condition shall conform to all requirements of all relevant dependencies specified in Table 205-??.11.

#### Table 205-?? .11 . Requirements class xsd-aerodrome-sea-condition

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- condition |  |
| Target type | Data instance |
| Name | Aerodrome sea condition |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- state/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeSeaCondition. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- state/ sea- surface- temperature The sea-surface temperature shall be reported in Celsius (°C) using the XML element iwxxm: seaSurfaceTemperature. The value of the associated XML attribute iwxxm: seaSurfaceTemperature/ @uom shall be “Cel”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- state/ significant- wave- height If reported, the observed significant wave height reported in metres shall be expressed using the XML element iwxxm: significantWaveHeight and the unit of measure shall be indicated using the XML attribute iwxxm: significantWaveHeight/ @uom with value “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- state/ sea- state If sea state is reported, then the value of XML attribute iwxxm: seaState/ @xlink: href shall be the URI of the valid term from Volume I.2, FM 94 BUFR, Code table 0 22 061: State of the sea. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- sea- state/ either- significant-w ave- height- or- sea- state When significant wave height is reported, sea state shall not be reported. When sea state is reported, significant wave height shall not be reported. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- The term sea-surface temperature is generally meant to be representative of the upper few metres of the ocean as opposed to the skin temperature.

- For convenience, FM 94 BUFR, Code table 0 22 061 from Volume I.2 is published online at http:// codes. wmo. int/ bufr4/ codeflag/ 0- 22- 061.

#### 205-??.11 	Requirements class: Aerodrome wind shear

205-??.11.1 	This requirements class is used to describe the aerodrome wind shear. The class is targeted at providing a basic description of the wind shear as required for civil aviation purposes – currently limited to indicating whether a wind shear threshold has been exceeded.

Notes:

- The information on wind shear includes, but is not necessarily limited to, wind shear of a non-transitory nature such as might be associated with low-level temperature inversions or local topography.

- Representations providing more detailed information may be used if required.

- The requirements for reporting aerodrome wind shear are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.8.1.4.

205-??.11.2 	XML elements describing aerodrome wind shear shall conform to all requirements specified in Table 205-??.12.

205-??.11.3 	XML elements describing aerodrome wind shear shall conform to all requirements of all relevant dependencies specified in Table 205-??.12.

#### Table 205-?? .12 . Requirements class xsd-aerodrome-wind-shear

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear |  |
| Target type | Data instance |
| Name | Aerodrome wind shear |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeWindShear. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear/ runways If XML attribute @allRunways is absent or has value “false”, then one or more XML elements iwxxm: runway, each with valid child element aixm: RunwayDirection, shall be used to indicate the set of runway directions to which wind shear conditions apply. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- wind- shear/ all- runways If XML attribute @allRunways has value “true”, then XML element iwxxm: runway shall be absent. |

#### 205-??.12 	Requirements class: Aerodrome cloud

205-??.12.1 	This requirements class is used to describe observed cloud conditions at an aerodrome. The class is targeted at providing a basic description of the observed cloud conditions as required for civil aviation purposes.

Notes:

- Representations providing more detailed information may be used if required.

- The requirements for reporting observed cloud conditions are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.5.

205-??.12.2 	XML elements describing observed cloud conditions shall conform to all requirements specified in Table 205-??.13.

205-??.12.3 	XML elements describing observed cloud conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.13.

#### Table 205-?? .13 . Requirements class xsd-aerodrome-observed-clouds

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud |  |
| Target type | Data instance |
| Name | Aerodrome cloud |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer, 205-??.49 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeCloud. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ vertical- visibility When cloud of operational significance is observed but the amount and height cannot be observed, then the XML element iwxxm: verticalVisibility shall be used to report the vertical visibility. The vertical distance shall be expressed in metres or feet which shall be indicated using the XML attribute iwxxm: verticalVisibility/ @uom with value “m” (metres) or “[ft_i]” (feet). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ cloud- layer When the amount and height of cloud of operational significance are observed, then the XML element iwxxm: layer, containing a valid child element iwxxm: CloudLayer, shall be used to describe each cloud layer. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ number- of- cloud- layers No more than four cloud layers shall be reported. If more than four significant cloud layers are observed, then the four most significant cloud layers with respect to aviation operations shall be prioritized. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ either- vertical- visibility- or- cloud- layers When vertical visibility is reported, cloud layers shall not be reported. When cloud layers are reported, vertical visibility shall not be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ nsc When no cloud of operational significance is observed, and no restriction on vertical visibility and 'CAVOK' is not appropriate, XML element iwxxm: cloud shall be empty with a XML attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud/ ncd When no clouds are detected by an automatic observing system, XML element iwxxm: cloud shall be empty with a XML attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ notDetectedByAutoSystem" |

Notes:

- Cloud of operational significance includes cloud below 1 500 metres or the highest minimum sector altitude, whichever is greater, and cumulonimbus whenever present.

- Vertical visibility is defined as the vertical visual range into an obscuring medium.

- Units of measurement are specified in accordance with 1.9 above.

#### 205-??.13 	Requirements class: Aerodrome surface wind

205-??.13.1 	This requirements class is used to describe the surface wind conditions observed at an aerodrome.

Note: 	The requirements for reporting surface wind conditions are specified in the Technical Regulations (WMO No. 49), Volume II, Part II, Appendix 3, 4.1.

205-??.13.2 	XML elements describing surface wind conditions shall conform to all requirements specified in Table 205-??.14.

205-??.13.3 	XML elements describing surface wind conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.14.

#### Table 205-?? .14 . Requirements class xsd-aerodrome-surface-wind

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind |  |
| Target type | Data instance |
| Name | Aerodrome surface wind |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeSurfaceWind. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ mean- wind- speed The mean wind speed shall be stated using the XML element iwxxm: meanWindSpeed, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute iwxxm: meanWindSpeed/ @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ mean- wind- speed- operator If present, the value of XML element iwxxm: meanWindSpeedOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ wind- gust- speed If reported, the observed gust speed shall be stated using the XML element iwxxm: windGustSpeed and expressed in metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute iwxxm: windGustSpeed/ @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ wind- gust- speed- operator If present, the value of XML element iwxxm: meanWindGustSpeedOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ extreme- clockwise- wind- direction The extreme clockwise direction from true north from which the wind has varied shall be expressed using XML element iwxxm: extremeClockWiseWindDirection. The unit of measure for the extreme wind direction shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ extreme- counter- clockwise- wind- direction The extreme counterclockwise direction from true north from which the wind has varied shall be expressed using XML element iwxxm: extremeCounterClockWiseWindDirection. The unit of measure for the extreme wind direction shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind/ variable- wind- direction If the wind direction is variable, then the XML attribute @variableDirection shall have the value “true” and XML element iwxxm: meanWindDirection shall be absent. |

Notes:

- The mean wind speed is the average wind speed observed over the previous 10 minutes.

- The gust speed is the maximum wind speed observed over the previous 10 minutes.

- Wind direction is reported as variable (VRB) if, during the 10-minute observation of mean wind speed, the variation of wind direction is (i) 180 degrees or more, or (ii) 60 degrees or more when the wind speed is less than 1.5 metres per second (3 knots).

- Extreme directional variations of wind are reported if, during the 10-minute observation of mean wind speed, the variation of wind direction is 60 degrees or more and less than 180 degrees and the wind speed is 1.5 metres per second (3 knots) or more.

- The absence of XML attribute iwxxm: AerodromeSurfaceWind/ @variableDirection implies a “false” value; for example, the wind direction is not variable.

- Units of measurement are specified in accordance with 1.9 above.

- The true north is the north point at which the meridian lines meet.

#### 205-??.14 	Requirements class: Aerodrome horizontal visibility

205-??.14.1 	This requirements class is used to describe the horizontal visibility conditions observed at an aerodrome.

Note: 	The requirements for reporting horizontal visibility are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.2.

205-??.14.2 	XML elements describing horizontal visibility shall conform to all requirements specified in Table 205-??.15.

205-??.14.3 	XML elements describing horizontal visibility shall conform to all requirements of all relevant dependencies specified in Table 205-??.15.

#### Table 205-?? .15 . Requirements class xsd-aerodrome-horizontal-visibility

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility |  |
| Target type | Data instance |
| Name | Aerodrome horizontal visibility |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeHorizontalVisibility. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ prevailing- visibility The prevailing visibility shall be stated using the XML element iwxxm: prevailingVisibility with the unit of measure metres, indicated using the XML attribute iwxxm: prevailingVisibility/ @uom with value “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ prevailing- visibility- operator If present, the value of XML element i wxxm: prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ prevailing- visibility- exceeds- 10000m If the prevailing visibility exceeds 10 000 metres, then the numeric value of XML element iwxxm: prevailingVisibility shall be set to 10000 and the XML element iwxxm: prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ minimum- visibility If reported, the minimum visibility shall be expressed using XML element iwxxm: minimumVisibility with the unit of measure metres, indicated using the XML attribute iwxxm: minimumVisibility/ @uom with value “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- horizontal- visibility/ minimum- visibility- direction If reported, the observed angle between true north and the direction of minimum visibility shall be expressed in degrees using XML element iwxxm: mininumVisibilityDirection, with the unit of measure indicated using the XML attribute iwxxm: mininumVisibilityDirection/ @uom with value “deg”. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- Visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1 000 candelas can be seen and identified against an unlit background.

- Prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

- The absence of XML element  iwxxm: prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

- The conditions for reporting minimum visibility are that the visibility is not the same in different directions and (i) when the lowest visibility is different from the prevailing visibility and less than 1 500 metres or less than 50% of the prevailing visibility and less than 5 000 metres, or (ii) when the visibility is fluctuating rapidly and the prevailing visibility cannot be determined.

- When reporting minimum visibility, the general direction of the minimum visibility in relation to the aerodrome should be reported unless the visibility is fluctuating rapidly.

- The true north is the north point at which the meridian lines meet.

#### 205-??.15 	Requirements class: TAF

205-??.15.1 	This requirements class is used to describe the aerodrome forecast (TAF) report within which a base forecast, and optionally one or more change forecasts, is provided.

Note: 	The reporting requirements for aerodrome forecasts are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3 and Appendix 5, section 1.

205-??.15.2 	XML elements describing TAFs shall conform to all requirements specified in Table 205-??.15.

205-??.15.3 	XML elements describing TAFs shall conform to all requirements of all relevant dependencies specified in Table 205-??.16.

#### Table 205-?? .16 . Requirements class xsd-taf

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf |  |
| Target type | Data instance |
| Name | TAF |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- report, 205-??.44 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast, 205-??.16 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ valid The content model of this element shall have a value that matches the content model of iwxxm: TAF which extends iwxxm: Report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ issue- time The XML element iwxxm: issueTime shall contain a valid child element gml: TimeInstant that describes the time at which the TAF was issued. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ aerodrome The XML element iwxxm: aerodrome shall contain a valid child element aixm: AirportHeliport to indicate the aerodrome at which the observation took place. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ valid- period The XML element iwxxm: validPeriod shall contain a valid child element gml:TimePeriod to indicate the time frame at which this report is valid. All forecast elements should be valid within this period. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ cancelled- report- valid- period The XML element iwxxm: cancelledReportValidPeriod shall contain a valid child element gml:TimePeriod to indicate the valid time period of the previously cancelled report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ base- forecast Prevailing forecast conditions are indicated in XML element iwxxm: baseForecast which shall contain a valid child element iwxxm: MeteorologicalAerodromeForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ change- forecast Forecast that modifies the base forecast are indicated in XML element iwxxm: changeForecast which shall contain a valid child element iwxxm: MeteorologicalAerodromeForecast. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ number- of- change- forecasts The number of change forecasts should be kept to a minimum, and no more than five change forecasts should be reported in normal circumstances. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ is- cancel- report If the report is intended to cancel a previously issued TAF the XML attribute @isCancelReport shall be set to “true”, iwxxm: cancelledReportValidPeriod shall be appropriately filled and iwxxm: validPeriod, iwxxm: baseForecast and iwxxm: changeForecast shall be empty. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- taf/ nil- report A NIL report shall be represented by making XML element iwxxm:baseForecast empty with attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ missing" |

Notes:

- The requirements for reporting the following are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5:

  - Change forecasts  	section 1.3

  - Probability forecasts with probability of occurrence 	paragraph 1.4

- Guidance regarding the number of change forecasts or forecasts with probability of occurrence is given in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 1.5.

- A NIL report indicates that a routine report has not been provided on the anticipated timescales. Such a report does not contain details of any forecast meteorological conditions

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

#### 205-??.16 	Requirements class: Meteorological aerodrome forecast

205-??.16.1 	This requirements class is used to describe the aggregated set of meteorological conditions forecast at an aerodrome as appropriate for inclusion in a aerodrome forecast (TAF) report.

205-??.16.2 	XML elements describing the set of meteorological conditions for inclusion in an aerodrome forecast shall conform to all requirements specified in Table 205-??.17.

205-??.16.3 	XML elements describing the set of meteorological conditions for inclusion in an aerodrome forecast shall conform to all requirements of all relevant dependencies specified in Table 205-??.17.

#### Table 205-?? .17 . Requirements class xsd-meterological-aerodrome-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast |  |
| Target type | Data instance |
| Name | Meteorological aerodrome forecast |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast, 205-??.45 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- forecast, 205-??.46 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast, 205-??.17 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: MeteorologicalAerodromeForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ phenomenon- time The XML element iwxxm: phenomenonTime shall have a valid child element gml:TimePeriod that describes the time at which the reported phenomena or change of phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ prevailing- visiblity If reported, the prevailing visibility shall be stated using the XML element iwxxm: prevailingVisibility with the unit of measure metres, indicated using the XML attribute @uom with value “m”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ prevailing- visibility- operator If present, the value of XML element iwxxm: prevailingVisibilityOperator shall be one of the enumeration: “ABOVE” or “BELOW”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ prevailing- visibility- exceeds- 10000m If the prevailing visibility exceeds 10 000 metres, then the numeric value of XML element iwxxm: prevailingVisibility shall be set to 10000 and the XML element iwxxm: prevailingVisibilityOperator shall have the value “ABOVE”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ surface- wind Surface wind conditions forecast for the aerodrome shall be reported using the XML element iwxxm: surfaceWind containing a valid child element iwxxm: AerodromeSurfaceWindForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ weather If forecast weather is reported, the value of XML attribute iwxxm: weather/ @xlink: href shall be the URI of a valid weather phenomenon code from Code table D-7: Aerodrome present or forecast weather. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ weather- nsw The expected end of occurrence of weather phenomena shall be indicated by an empty XML element iwxxm: weather with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ number- of- forecast- weather- codes No more than three forecast weather codes shall be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/cloud If reported, the cloud conditions forecast for the aerodrome shall be expressed using the XML element iwxxm: cloud containing a valid child element iwxxm: AerodromeCloudForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/cloud- nsc When no cloud of operational significance is observed/forecast, and 'CAVOK' is not appropriate, the XML element iwxxm: cloud shall be empty with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance” |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/cloud- vertical- visibility- not- available When the vertical visibility is not available for any reason, XML element iwxxm: cloud/ iwxxm: AerodromeCloudForecast/ iwxxm:verticalVisibility" shall be missing with no nilReason. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ temperature If reported, the temperature conditions forecast for the aerodrome shall be expressed using the XML element iwxxm: temperature containing a valid child element iwxxm: AerodromeAirTemperatureForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ number- of- temperature- groups No more than two sets of temperature conditions shall be reported. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ change- indicator The XML attribute @changeIndicator shall be absent if the forecast describes the prevailing meteorological conditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ change- indicator- becmg If the meteorological conditions forecast for the aerodrome are expected to reach or pass through specified values at a regular or irregular rate, then the XML attribute @changeIndicator shall have the value “BECOMING”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ change- indicator- tempo If temporary fluctuations in the meteorological conditions forecast for the aerodrome are expected to occur, then the XML attribute @changeIndicator shall have the value “TEMPORARY_FLUCTUATIONS”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast/ change- indicator- fm If the meteorological conditions forecast for the aerodrome are expected to change significantly and more or less completely to a different set of conditions, then the XML attribute @changeIndicator shall have the value “FROM”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast- record/ change- indicator- prob30 If meteorological conditions forecast for the aerodrome have a 30% probability of occurring, then the XML attribute @changeIndicator shall have the value “PROBABILITY_30”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast- record/ change- indicator- prob30- tempo If the temporary fluctuations in meteorological conditions forecast have a 30% probability of occurring, then the XML attribute @changeIndicator shall have the value “PROBABILITY_30_TEMPORARY_FLUCTUATIONS”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast- record/ change- indicator- prob40 If meteorological conditions forecast for the aerodrome have a 40% probability of occurring, then the XML attribute @changeIndicator shall have the value “PROBABILITY_40”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast- record/ change- indicator- prob40- tempo If the temporary fluctuations in meteorological conditions forecast have a 40% probability of occurring, then the XML attribute @changeIndicator shall have the value “PROBABILITY_40_TEMPORARY_FLUCTUATIONS”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- meteorological- aerodrome- forecast- record/ cavok If the conditions associated with CAVOK are forecast, then: (i) The XML attribute @cloudAndVisibilityOK shall have the value “true”; and (ii) The following XML elements shall be absent: iwxxm: prevailingVisibility, iwxxm: prevailingVisibilityOperator, iwxxm: weather and iwxxm: cloud. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- Temporary fluctuations in the meteorological conditions occur when those conditions reach or pass specified values and last for a period of time less than one hour in each instance and, in the aggregate, cover less than one half the period during which the fluctuations are forecast to occur (Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 2.3.3).

- The use of change groups and time indicators within a TAF is specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 1.3 and Table A5-2.

- The use of probability groups and time indicators within a TAF is specified in the Technical Regulations (WMO No. 49), Volume II, Part II, Appendix 5, 1.4 and Table A5-2.

- Cloud and visibility information is omitted when considered to be insignificant to aeronautical operations at an aerodrome. This occurs when: (i) visibility exceeds 10 kilometres, (ii) no cloud is present below 1 500 metres or the minimum sector altitude, whichever is greater, and there is no cumulonimbus at any height, and (iii) there is no weather of operational significance. These conditions are referred to as CAVOK. Use of CAVOK is specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 2.2.

- Visibility for aeronautical purposes is defined as the greater of: (i) the greatest distance at which a black object of suitable dimensions, situated near the ground, can be seen and recognized when observed against a bright background; or (ii) the greatest distance at which lights in the vicinity of 1 000 candelas can be seen and identified against an unlit background.

- Prevailing visibility is defined as the greatest visibility value observed which is reached within at least half the horizon circle or within at least half of the surface of the aerodrome. These areas could comprise contiguous or non-contiguous sectors.

- The requirements for reporting the following within an aerodrome forecast are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5:

(a) Prevailing visibility conditions paragraph 1.2.2 (b) Temperature conditions paragraph 1.2.5

  - Cloud conditions 	paragraph 1.2.4

  - Forecast weather phenomena 	paragraph 1.2.3

  - Surface wind conditions 	paragraph 1.3.0

- The absence of XML element //iwxxm: MeteorologicalAerodromeForecastRecord/ iwxxm:

prevailingVisibilityOperator indicates that the prevailing visibility has the numeric value reported.

- Code table D-7 is published online at http:// codes. wmo. int/ 49- 2/ AerodromePresentOrForecastWeather.

#### 205-??.17 	Requirements class: Aerodrome air temperature forecast

205-??.17.1 	This requirements class is used to describe the temperature conditions forecast at an aerodrome as appropriate for inclusion in an aerodrome forecast (TAF) report, including the maximum and minimum temperature values and their time of occurrence.

Note: 	The requirements for reporting the temperature conditions at an aerodrome within a TAF are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 1.2.5.

205-??.17.2 	XML elements describing temperature conditions at an aerodrome shall conform to all requirements specified in Table 205-??.18.

205-??.17.3 	XML elements describing temperature conditions at an aerodrome shall conform to all requirements of all relevant dependencies specified in Table 205-??.18.

#### Table 205-?? .18 . Requirements class xsd-aerodrome-air-temperature-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast |  |
| Target type | Data instance |
| Name | Aerodrome air temperature forecast |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeAirTemperatureForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast/ maximum- temperature The maximum air temperature anticipated during the forecast period shall be reported in Celsius (°C) using the XML element iwxxm: maximumAirTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast/ maximum- temperature- time The XML element iwxxm: maximumAirTemperatureTime shall contain a valid child element gml: TimeInstant that describes the time at which the maximum air temperature is anticipated to occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast/ minimum- temperature The minimum air temperature anticipated during the forecast period shall be reported in Celsius (°C) using the XML element iwxxm: minimumAirTemperature. The value of the associated XML attribute @uom shall be “Cel”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- air- temperature- forecast/ minimum- temperature- time The XML element iwxxm: minimumAirTemperatureTime shall contain a valid child element gml: TimeInstant that describes the time at which the minimum air temperature is anticipated to occur. |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.18 	Requirements class: Tropical cyclone SIGMET evolving condition collection

205-??.18.1 	This requirements class is used to describe a collection of tropical cyclone observed or forecast conditions.

205-??.18.2 	XML elements describing a collection of tropical cyclone observed or forecast conditions shall conform to all requirements specified in Table 205-??.19.

205-??.18.3 	XML elements describing a collection of tropical cyclone observed or forecast conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.19.

#### Table 205-?? .19 . Requirements class xsd-tropical-cyclone-sigmet-evolving-condition-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection |  |
| Target type | Data instance |
| Name | Tropical cyclone SIGMET evolving condition collection |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- tropical- cyclone, 202-16.24 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection, 205-??.22 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneSIGMETEvolvingConditionCollection which extends iwxxm: SIGMETEvolvingConditionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection / tropical- cyclone- position The observed or forecast position of a tropical cyclone shall be reported using the XML element iwxxm: tropicalCyclonePosition with valid child element gml: Point. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection / tropical- cyclone- id The identity of a tropical cyclone shall be reported using the XML element iwxxm: tropicalCycloneId. It shall be the gml:id of the XML element metce: TropicalCyclone defined for the respective tropical cyclone. |

#### 205-??.19 	Requirements class: Tropical cyclone SIGMET position collection

205-??.19.1 	This requirements class is used to describe a collection of tropical cyclone forecast conditions.

205-??.19.2 	XML elements describing only a collection of tropical cyclone forecast conditions shall conform to all requirements specified in Table 205-??.20.

205-??.19.3 	XML elements describing only a collection of tropical cyclone forecast conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.20.

#### Table 205-?? .20 . Requirements class xsd-tropical-cyclone-sigmet-evolving-condition

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- position- collection |  |
| Target type | Data instance |
| Name | Tropical cyclone SIGMET position collection |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- tropical- cyclone, 202-16.24 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection, 205-??.28 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- position- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneSIGMETPositionCollection which extends iwxxm: SIGMETPositionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- position- collection / tropical- cyclone- position The forecast position of a tropical cyclone shall be reported using the XML element iwxxm: tropicalCyclonePosition with valid child element gml: Point. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection / tropical- cyclone- id The identity of a tropical cyclone shall be reported using the XML element iwxxm: tropicalCycloneId. It shall be the gml:id of the XML element metce: TropicalCyclone defined for the respective tropical cyclone. |

#### 205-??.20 	Requirements class: Volcanic ash SIGMET evolving condition collection

205-??.20.1 	This requirements class is used to describe a collection of volcanic ash observed or forecast conditions.

205-??.20.2 	XML elements describing only a collection of volcanic ash observed or forecast conditions shall conform to all requirements specified in Table 205-??.21.

205-??.20.3 	XML elements describing only a collection of volcanic ash observed or forecast conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.21.

#### Table 205-?? .21 . Requirements class xsd-volcanic-ash-sigmet-evolving-condition-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- evolving- condition- collection |  |
| Target type | Data instance |
| Name | Volcanic ash SIGMET evolving condition collection |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- volcano, 202-16.7 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection, 205-??.22 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- evolving- condition- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshSIGMETEvolvingConditionCollection which extends iwxxm: SIGMETEvolvingConditionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- evolving- condition- collection / volcano- id The identity of a volcano shall be reported using the XML element iwxxm: volcanoId. It shall be the gml:id of the XML element metce: Volcano defined for the respective volcano. |

#### 205-??.21 	Requirements class: Volcanic ash SIGMET position collection

205-??.21.1 	This requirements class is used to describe a collection of volcanic ash forecast conditions.

205-??.21.2 	XML elements describing a collection of volcanic ash forecast conditions shall conform to all requirements specified in Table 205-??.22.

205-??.21.3 	XML elements describing a collection of volcanic ash forecast conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.22.

#### Table 205-?? .22 . Requirements class xsd-volcanic-ash-sigmet-position-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection |  |
| Target type | Data instance |
| Name | Volcanic ash SIGMET position collection |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- volcano, 202-16.7 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection, 205-??.28 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshSIGMETPositionCollection which extends iwxxm: SIGMETPositionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection / volcano- id The identity of a volcano shall be reported using the XML element iwxxm: volcanoId. It shall be the gml:id of the XML element metce: Volcano defined for the respective volcano. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection/ expected- end- of- occurrence- of- volcanic- ash The expected end of occurrence of volcanic ash shall be indicated with an empty XML element iwxxm: member with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance" |

#### 205-??.22 	Requirements class: SIGMET evolving condition collection

205-??.22.1 	This requirements class is used to describe a collection of observed and forecast conditions of a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the start of the valid period of the SIGMET report.

205-??.22.2 	XML elements describing a collection of observed and forecast SIGMET conditions shall conform to all requirements specified in Table 205-??.23.

205-??.22.3 	XML elements describing a collection of observed and forecast SIGMET conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.23.

#### Table 205-?? .23 . Requirements class xsd-sigmet-evolving-condition-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection |  |
| Target type | Data instance |
| Name | SIGMET evolving condition collection |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition, 205-??.25 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: SIGMETEvolvingConditionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection/ phenomenon- time The XML element iwxxm: phenomenonTime shall have a valid child element gml:TimeInstant that describes the time at which the reported phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection/ time- indicator The XML attribute @timeIndicator shall indicate whether the information is observed and expected to continue, or forecast with one of the enumeration: “OBSERVATION” or “FORECAST” |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection/ member If reported, the conditions of a specific SIGMET phenomenon shall be provided using the XML element iwxxm: member with valid child element iwxxm: SIGMETEvolvingCondition. |

#### 205-??.23 	Requirements class: SIGMET position collection

205-??.23.1 	This requirements class is used to describe a collection of forecast conditions of a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the end of the valid period of the SIGMET report.

205-??.23.2 	XML elements describing a collection of forecast conditions of a SIGMET phenomenon shall conform to all requirements specified in Table 205-??.24.

205-??.23.3 	XML elements describing a collection of forecast conditions of a SIGMET phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.24.

#### Table 205-?? .24 . Requirements class xsd-sigmet-position-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection |  |
| Target type | Data instance |
| Name | SIGMET position collection |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position, 205-??.25 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: SIGMETPositionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position- collection/ phenomenon- time The XML element iwxxm: phenomenonTime shall have a valid child element gml:TimeInstant that describes the time at which the reported phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection/ member If reported, the conditions of a specific SIGMET phenomenon shall be provided using the XML element iwxxm: member with valid child element iwxxm: SIGMETPosition. |

#### 205-??.24 	Requirements class: SIGMET evolving condition

205-??.24.1 	This requirements class is used to describe the observed or forecast conditions of a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the start of the valid period of the SIGMET report. The geometric extent of the SIGMET phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.24.2 	XML elements describing the observed or forecast conditions of a SIGMET phenomenon shall conform to all requirements specified in Table 205-??.25.

205-??.24.3 	XML elements describing the observed or forecast conditions of a SIGMET phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.25.

#### Table 205-?? .25 . Requirements class xsd-sigmet-evolving-condition

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition |  |
| Target type | Data instance |
| Name | SIGMET evolving condition |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ valid The content model of this element shall have a value that matches the content model of iwxxm: SIGMETEvolvingCondition. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ geometry The geometric extent of the SIGMET phenomenon shall be reported using the XML element iwxxm: geometry with valid child element aixm: AirspaceVolume. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ 7- point- definition- of- airspace-volume The horizontal extent of any airspace volumes enclosing a SIGMET phenomenon should use no more than seven points to define the bounding polygon. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ geometry- is- a- single- point When a SIGMET position is reported at a single point the location should be indicated as a gml: CircleByCenterPoint with a gml: radius of 0 |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ direction- of- motion The expected direction of movement which the phenomenon is moving towards shall be reported using the XML element iwxxm: directionOfMotion. The unit of measure shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ speed- of- motion The expected speed of movement of a meteorological condition shall be reported using the XML element iwxxm: speedOfMotion. The unit of measure shall be indicated using the XML attribute @uom with value of either “km/h” or “[kn_i]” (knots). |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ no- movement When no movement is expected, iwxxm: speedOfMotion shall have a value of 0, iwxxm: directionOfMotion shall be empty with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ intensity- change The expected change in intensity for the reported meteorological condition determined at the time of SIGMET analysis based on the current state of the meteorological condition of the SIGMET phenomenon shall be reported using the XML element iwxxm: intensityChange with one of the enumeration: “NO_CHANGE”, “WEAKEN” or “INTENSIFY”. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ approximate- location The Indicator of whether the location of the phenomena is approximate shall be reported using the XML attribute @approximateLocation. If the attribute is “false” or not reported locations are not approximate. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- The true north is the north point at which the meridian lines meet.

#### 205-??.25 	Requirements class: SIGMET position

205-??.25.1 	This requirements class is used to describe the forecast conditions of a specific SIGMET phenomenon, such as volcanic ash or thunderstorm, at the end of the valid period of the SIGMET report. The geometric extent of the SIGMET phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.25.2 	XML elements describing the forecast conditions of a SIGMET phenomenon shall conform to all requirements specified in Table 205-??.26.

205-??.25.3 	XML elements describing the forecast conditions of a SIGMET phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.26.

#### Table 205-?? .26 . Requirements class xsd-sigmet-position

| Requirements class |  |
| --- | --- |
| http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position |  |
| Target type | Data instance |
| Name | SIGMET position |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position/ valid The content model of this element shall have a value that matches the content model of iwxxm: SIGMETPosition. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position/ geometry The geometric extent of the SIGMET phenomenon shall be reported using the XML element iwxxm: geometry with valid child element aixm: AirspaceVolume. |
| Recommendation | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position/ 7- point- definition- of- airspace-volume The horizontal extent of any airspace volumes enclosing a SIGMET phenomenon should use no more than seven points to define the bounding polygon. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position/ geometry- is- a- single- point When a SIGMET position is reported at a single point the location should be indicated as a gml: CircleByCenterPoint with a gml: radius of 0 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition/ approximate- location The Indicator of whether the location of the phenomena is approximate shall be reported using the XML attribute @approximateLocation. If the attribute is “false” or not reported locations are not approximate |

#### 205-??.26 	Requirements class: SIGMET

205-??.26.1 	This requirements class is used to describe the SIGMET report within which the characteristics of a specific SIGMET phenomenon are described.

Note: 	The reporting requirements for SIGMETs are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, section 1.

205-??.26.2 	XML elements describing SIGMET reports shall conform to all requirements specified in Table 205-??.27.

205-??.26.3 	XML elements describing SIGMET reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.27.

#### Table 205-?? .27 . Requirements class xsd-sigmet

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet |  |
| Target type | Data instance |
| Name | SIGMET |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- evolving- condition- collection, 205-??.22 or (for tropical cyclone SIGMET) http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- evolving- condition- collection, 205-??.18 or (for volcanic ash SIGMET) http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- evolving- condition- collection, 205-??.20 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet- position-collection, 205-??.23 or (for tropical cyclone SIGMET) http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet- position- collection, 205-??.19 or (for volcanic ash SIGMET) http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet- position- collection, 205-??.21 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ valid The content model of this element shall have a value that matches the content model of iwxxm: SIGMET. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ issue- time The XML element iwxxm: issueTime shall contain a valid child element gml: TimeInstant that describes the time at which the SIGMET was issued. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ issuing- air- traffic- services- unit The air traffic services unit responsible for the subject airspace shall be indicated using the XML element iwxxm: issuingAirTrafficServicesUnit with a valid child element aixm: Unit. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ issuing- air- traffic- services- unit- type The value of XML element iwxxm: issuingAirTrafficServicesUnit/ aixm: Unit/ aixm: timeSlice/ aixm: UnitTimeSlice/ aixm: type should be one of the enumeration: “ATSU” (Air Traffic Services Unit) or “FIC” (Flight Information Centre). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ originating- meteorological- watch- office The meteorological watch office that originated the SIGMET report shall be indicated using the XML element iwxxm: originatingMeteorologicalWatchOffice with a valid child element aixm: Unit. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ originating- meteorological- watch- office- type The value of XML element iwxxm: originatingMeteorologicalWatchOffice/ aixm: Unit/ aixm: timeSlice/ aixm: UnitTimeSlice/ aixm: type should be “MWO” (Meteorological Watch Office). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ issuing- air- traffic- services- region The air traffic services region (FIR, UIR, CTA, or FIR/UIR) shall be indicated using the XML element iwxxm: issuingAirTrafficServicesRegion with a valid child element aixm: Airspace. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ sequence- number The sequence number of this SIGMET report shall be indicated using XML element iwxxm: sequenceNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ valid- period The period of validity of this SIGMET report shall be indicated using XML element iwxxm: validPeriod with valid child element gml: TimePeriod. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ cancelled- report- sequence- number The sequence number of a previous SIGMET that is cancelled by this SIGMET shall be indicated using XML element iwxxm: cancelledReportSequenceNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ cancelled- report- valid- period The valid period of a previous SIGMET that is cancelled by this SIGMET shall be indicated using XML element iwxxm: cancelledReportValidPeriod with valid child element gml: TimePeriod. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ phenomenon The XML attribute iwxxm: phenomenon/ @xlink: href shall have a value that is the URI of a valid term from Code table D-10: Significant weather phenomena. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ analysis If reported, XML element iwxxm: analysis shall contain a valid child element of either: - For SIGMET: iwxxm: SIGMETEvolvingConditionCollection - For tropical cyclone SIGMET: iwxxm: TropicalCycloneSIGMETEvolvingConditionCollection - For volcanic ash SIGMET: iwxxm: VolvanicAshSIGMETEvolvingConditionCollection |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ forecast- position- analysis If reported, XML element iwxxm: forecastPositionAnalysis shall contain a valid child element of either: - For SIGMET: iwxxm: SIGMETPositionCollection - For tropical cyclone SIGMET: iwxxm: TropicalCycloneSIGMETPositionCollection - For volcanic ash SIGMET: iwxxm: VolvanicAshSIGMETPositionCollection |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ valid- time- includes- all- phenomenon- times The observation and/or forecast times of all SIGMET analyses and, if reported, forecast position analyses included in the report should occur within the valid time period in XML element iwxxm: validPeriod of the SIGMET. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ valid- period- start- matches- phenomenon- time The start time of the validity period of the SIGMET report should match the phenomenon time of each SIGMET analysis included within the report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet/ is- cancel- report If the SIGMET report is a 'CANCELLATION' report which cancels a previously issued report, the XML attribute @isCancelReport shall be “true”. Iwxxm: calcelledReportSequenceNumber and iwxxm: cancelledReportValidPeriod shall be appropriately filled and iwxxm: phenomenon and iwxxm: analysis empty. |

Notes:

- Requirements relating to sequence numbers within SIGMET reports are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, 1.1.3.

- Requirements for reporting the SIGMET phenomenon are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, 1.1.4.

- A forecast position may be provided for a volcanic ash cloud, the centre of a tropical cyclone or other hazardous phenomena at the end of the validity period of the SIGMET message.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

- Code table D-10 is published online at http:// codes. wmo. int/ 49- 2/ SigWxPhenomena.

#### 205-??.27 	Requirements class: Tropical cyclone SIGMET

205-??.27.1 	This requirements class is used to describe the tropical cyclone (TC) SIGMET report, which includes additional information about the tropical cyclone itself and the forecast position of the tropical cyclone at the end of the validity period of the SIGMET.

205-??.27.2 	XML elements describing TC SIGMET reports shall conform to all requirements specified in Table 205-??.28.

205-??.27.3 	XML elements describing TC SIGMET reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.28.

#### Table 205-?? .28 . Requirements class xsd-tropical-cyclone-sigmet

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet |  |
| Target type | Data instance |
| Name | Tropical cyclone SIGMET |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- tropical- cyclone, 202-16.24 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet, 205-??.26 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneSIGMET which extends iwxxm: SIGMET. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- sigmet/ tropical-cyclone Details of the tropical cyclone shall be reported using the XML element iwxxm: tropicalCyclone with valid child element metce: TropicalCyclone. |

#### 205-??.28 	Requirements class: Volcanic ash SIGMET

205-??.28.1 	This requirements class is used to describe the volcanic ash (VA) SIGMET report, which includes additional information about the source volcano and the forecast position of the volcanic ash at the end of the validity period of the SIGMET.

205-??.28.2 	XML elements describing VA SIGMET reports shall conform to all requirements specified in Table 205-??.29.

205-??.28.3 	XML elements describing VA SIGMET reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.29.

#### Table 205-?? .29 . Requirements class xsd-volcanic-ash-sigmet

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet |  |
| Target type | Data instance |
| Name | Volcanic ash SIGMET |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- volcano, 202-16.7 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- sigmet, 205-??.26 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshSIGMET. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet/ erupting- volcano Details of the volcano that is the source of the volcanic ash shall be reported using the XML element iwxxm: eruptingvolcano with valid child element metce: Volcano. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- sigmet/ volcanic- ash- moved- to- fir If reported, the XML element iwxxm: volcanicAshMovedToFIR shall indicate the FIR to which volcanic ash has moved. Reported only on SIGMET cancellation reports. |

#### 205-??.29 	Requirements class: AIRMET evolving condition collection

205-??.29.1 	This requirements class is used to define a collection of AIRMET phenomena described by the requirements class AIRMET evolving condition, each representing a location where AIRMET observed or forecast conditions exist.

205-??.29.2 	XML elements describing the characteristics of an AIRMET phenomenon shall conform to all requirements specified in Table 205-??.30.

205-??.29.3 	XML elements describing the characteristics of an AIRMET phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.30.

#### Table 205-?? .30 . Requirements class xsd-airmet-evolving-condition-collection

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection |  |
| Target type | Data instance |
| Name | AIRMET evolving condition collection |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition, 205-??.30 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection/ valid The content model of this element shall have a value that matches the content model of iwxxm: AIRMETEvolvingConditionCollection. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection/ phenomenon- time The XML element iwxxm: phenomenonTime shall have a valid child element gml:TimeInstant that describes the time at which the reported phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection/ time- indicator The XML attribute @timeIndicator shall indicate whether the information is observed and expected to continue, or forecast with one of the enumeration: “OBSERVATION” or “FORECAST” |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition- collection/ member If reported, the conditions of a specific AIRMET phenomenon shall be provided using the XML element iwxxm: member with valid child element iwxxm: AIRMETEvolvingCondition. |

#### 205-??.30 	Requirements class: AIRMET evolving condition

205-??.30.1 	This requirements class is used to describe the presence of a specific AIRMET phenomenon such as thunderstorms or mountain obscuration, along with expected changes to the intensity of the phenomenon, its speed and direction of motion. The geometric extent of the AIRMET phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.30.2 	XML elements describing the characteristics of an AIRMET phenomenon shall conform to all requirements specified in Table 205-??.31.

205-??.30.3 	XML elements describing the characteristics of an AIRMET phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.31.

Table 205-?? . 31 . Requirements class xsd-airmet-evolving-condition

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition |  |
| Target type | Data instance |
| Name | AIRMET evolving condition |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ valid The content model of this element shall have a value that matches the content model of iwxxm: AIRMETEvolvingCondition. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ geometry The geometric extent of the AIRMET phenomenon shall be reported using the XML element iwxxm: geometry with valid child element aixm: AirspaceVolume. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ direction- of- motion If reported, the angle between true north and the direction of motion of the AIRMET phenomenon shall be given in degrees using the XML element iwxxm: directionOfMotion. The unit of measure shall be indicated using the XML attribute @uom with value “deg”. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ 7- point- definition- of- airspace-volume The horizontal extent of any airspace volumes enclosing a AIRMET phenomenon should use no more than seven points to define the bounding polygon. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ geometry- is- a- single- point When an AIRMET position is reported at a single point the location should be indicated as a gml: CircleByCenterPoint with a gml: radius of 0 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ speed- of- motion The speed of motion of the AIRMET phenomenon shall be reported using the XML element iwxxm: speedOfMotion, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ intensity- change The anticipated change in intensity of the AIRMET observed phenomenon shall be indicated using the XML attribute @intensityChange with the value being one of the enumeration: “NO_CHANGE”, “WEAKEN” or “INTENSIFY”. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ stationary- phenomenon If the AIRMET phenomenon is not moving (indicated by the XML element //iwxxm: AIRMETEvolvingCondition/ iwxxm: speedOfMotion having numeric value zero), XML element //iwxxm: AIRMET EvolvingCondition/iwxxm: directionOfMotion should be absent. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ cloud- base If reported, the cloud base of reported clouds shall be given using XML element iwxxm: cloudBase. The unit of measure shall be indicated using the XML attribute @uom with value "m" (metres) or "[ft_i]" (feet). Only used with an AIRMET reporting cloud phenomenon. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ cloud- base- reference If reported, the reference surface used for the value of the cloud base shall be given using XML element iwxxm: cloudBaseReference with one of the enumeration: “SFC”, “MSL” or “STD”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ cloud- top If reported, the cloud top of reported clouds shall be given using XML element iwxxm: cloudTop. The unit of measure shall be indicated using the XML attribute @uom with value "m" (metres) or "[ft_i]" (feet). Only used with an AIRMET reporting cloud phenomenon. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ cloud- top- reference If reported, the reference surface used for the value of the cloud base shall be given using XML element iwxxm: cloudBaseReference with one of the enumeration: “SFC”, “MSL” or “STD”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ surface- visibility If reported, the surface visibility shall be given using XML element iwxxm: surfaceVisibility. The unit of measure shall be indicated using XML attribute @uom with value "m" (metre). Only used with AIRMET reporting surface visibility. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ surface- visibility- cause If reported, the weather condition(s) causing reduced visibility shall be given by XML attribute iwxxm: surfaceVisibility/ @xlink: href and the value shall be the URI of a valid weather phenomenon code from ICAO Annex 3 / WMO No. 49-2: Appendix 6 Section 2.1. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ surface- wind- direction If reported, the widespread surface wind direction shall be given using XML element iwxxm: surfaceWindDirection. The unit of measure shall be indicated using XML attribute @uom with value "deg" representing the angle in degrees from true north. Only used with AIRMET reporting surface wind. A true north wind is indicated with a direction of 360. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ surface- wind- speed If reported, the widespread surface wind speed shall be given using XML element iwxxm: surfaceWindSpeed. The unit of measure shall be indicated using XML attribute @uom with value "m/s" or "[kn_i]" (knot). Only used with AIRMET reporting surface wind. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition/ approximate- location The Indicator of whether the location of the phenomena is approximate shall be reported using the XML attribute @approximateLocation. If the attribute is “false” or not reported locations are not approximate |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- The true north is the north point at which the meridian lines meet.

#### 205-??.31 	Requirements class: AIRMET

205-??.31.1 	This requirements class is used to describe the AIRMET report within which the characteristics of a specific AIRMET phenomenon are described.

Note: 	The reporting requirements for AIRMETs are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, section 2.

205-??.31.2 	XML elements describing AIRMET reports shall conform to all requirements specified in Table 205-??.32.

205-??.31.3 	XML elements describing AIRMET reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.32.

#### Table 205-?? .32 . Requirements class xsd-airmet

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet |  |
| Target type | Data instance |
| Name | AIRMET |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet- evolving- condition, 205-??.29 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ valid The content model of this element shall have a value that matches the content model of iwxxm: AIRMET. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ issue- time The XML element iwxxm: issueTime shall contain a valid child element gml: TimeInstant that describes the time at which the AIRMET was issued. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ issuing- air- traffic- services- unit The air traffic services unit responsible for the subject airspace shall be indicated using the XML element iwxxm: issuingAirTrafficServicesUnit with a valid child element aixm: Unit. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ issuing- air- traffic- services- unit- type The value of XML element iwxxm: issuingAirTrafficServicesUnit/ aixm: Unit/ aixm: timeSlice/ aixm: UnitTimeSlice/ aixm: type should be one of the enumeration: “ATSU” (Air Traffic Services Unit) or “FIC” (Flight Information Centre). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ originating- meteorological- watch- office The meteorological watch office that originated the AIRMET report shall be indicated using the XML element iwxxm: originatingMeteorologicalWatchOffice with a valid child element aixm: Unit. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ originating- meteorological- watch- office- type The value of XML element iwxxm: originatingMeteorologicalWatchOffice/ aixm: Unit/ aixm: timeSlice/ aixm: UnitTimeSlice/ aixm: type should be “MWO” (Meteorological Watch Office). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ issuing- air- traffic- services- region The air traffic services region (FIR, UIR, CTA, or FIR/UIR) shall be indicated using the XML element iwxxm: issuingAirTrafficServicesRegion with a valid child element aixm: Airspace. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ sequence- number The sequence number of this AIRMET report shall be indicated using XML element iwxxm: sequenceNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ valid- period The period of validity of this AIRMET report shall be indicated using XML element iwxxm: validPeriod with valid child element gml: TimePeriod. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ cancelled- report- sequence- number The sequence number of a previous AIRMET that is cancelled by this AIRMET shall be indicated using XML element iwxxm: cancelledReportSequenceNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ cancelled- report- valid- period The valid period of a previous AIRMET that is cancelled by this AIRMET shall be indicated using XML element iwxxm: cancelledReportValidPeriod with valid child element gml: TimePeriod. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ phenomenon The XML attribute iwxxm: phenomenon/ @xlink: href shall have a value that is the URI of a valid term from Code table D-10: Significant weather phenomena. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ analysis If reported, XML element iwxxm: analysis shall contain a valid child element iwxxm: AIRMETEvolvingConditionAnalysis. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ valid- time- includes- all- phenomenon- times The observation and/or forecast times of all AIRMET analyses included in the report should occur within the valid time period in XML element iwxxm: validPeriod of the AIRMET. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ valid- period- start- matches- phenomenon- time The start time of the validity period of the AIRMET report should match the phenomenon time of each AIRMET analysis included within the report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- airmet/ is- cancel- report If the AIRMET report is a 'CANCELLATION' report which cancels a previously issued report, the XML attribute @isCancelReport shall be “true”. Iwxxm: calcelledReportSequenceNumber and iwxxm: cancelledReportValidPeriod shall be appropriately filled and iwxxm: phenomenon and iwxxm: analysis empty. |

Notes:

- Requirements relating to sequence numbers within AIRMET reports are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, 3.0.2.

- Requirements for reporting the AIRMET phenomenon are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 6, 3.0.4.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

- Code table D-10 is published online at http:// codes. wmo. int/ 49- 2/ AirWxPhenomena.

#### 205-??.32 	Requirements class: Tropical Cyclone Advisory

205-??.32.1 	This requirements class is used to describe the Tropical Cyclone Advisory report within which the characteristics of a specific Tropical Cyclone Advisory phenomenon are described.

Note: 	The reporting requirements for Tropical Cyclone Advisories are specified in the Technical Regulations (WMONo. 49), Volume II, Part II, Appendix 2, section 5.

205-??.32.2 	XML elements describing Tropical Cyclone Advisory reports shall conform to all requirements specified in Table 205-??.33.

205-??.32.3 	XML elements describing Tropical Cyclone Advisory reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.33.

#### Table 205-?? .33 . Requirements class xsd-tropical-cyclone-advisory

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory |  |
| Target type | Data instance |
| Name | Tropical Cyclone Advisory |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- tropical- cyclone, 202-16.24 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions, 205-??.33 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- forecast- conditions, 205-??.34 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneAdvisory. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ issue- time The issuance time of this Tropical Cyclone Advisory report shall be indicated using XML element iwxxm: issueTime with valid child element gml: TimeInstant. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ issuing- tropical- cyclone- advisory- centre The air traffic services unit responsible for the subject airspace shall be indicated using the XML element iwxxm: issuingTropicalCycloneAdvisoryCentre with a valid child element aixm: Unit. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ tropical- cyclone- name Details of the tropical cyclone shall be reported using the XML element iwxxm: tropicalCycloneName with valid child element metce: TropicalCyclone. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ tropical- cyclone- is- unnamed An unnamed tropical cyclone should be indicated with an XML element iwxxm: tropicalCycloneName having a value of “UNNAMED”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ advisory- number The advisory number of this Tropical Cyclone Advisory report shall be indicated using XML element iwxxm: advisoryNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ observation If reported, XML element iwxxm: observation shall contain a valid child element iwxxm: TropicalCycloneObservationConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ forecast If reported, XML element iwxxm: forecast shall contain a valid child element iwxxm:TropicalCycloneForecastConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ remarks The XML element iwxxm: remarks shall contain the remarks of the advisory. When there are no remarks, it shall be indicated with an empty XML element iwxxm: remarks with an attribute @nilReason = “http: // codes. wmo. int/ common/ nil/ inapplicable”. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ next- advisory- time The time at which the next advisory is expected to be issued shall be indicated with XML element iwxxm: nextAdvisoryTime. Use attribute gml: timePosition/ @indeterminatePosition with a value from enumeration “before” or “after” to indicate if the actual temporal position is before or after the specified value. When no subsequent advisory is expected to be issued it shall be indicated by an empty iwxxm: nextAdvisoryTime with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable”. |

Notes:

- Requirements for reporting the Tropical Cyclone Advisory phenomenon are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 2, 5.1.1.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

#### 205-??.33 	Requirements class: Tropical cyclone observed conditions

205-??.33.1 	This requirements class is used to describe the presence of a specific condition observed phenomenon, along with expected changes. The geometric extent of the phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.33.2 	XML elements describing the characteristics of a tropical cyclone observed phenomenon shall conform to all requirements specified in Table 205-??.34.

205-??.33.3 	XML elements describing the characteristics of a tropical cyclone observed phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.34.

#### Table 205-?? .34 . Requirements class xsd-tropical-cyclone-observed-conditions

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions |  |
| Target type | Data instance |
| Name | Tropical cyclone observed conditions |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneObservedConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions / phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml:TimeInstant to indicate the time at which meteorological phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ tropical- cyclone- position The tropical cyclone position shall be reported using the XML element iwxxm: tropicalCyclonePosition with valid child element gml: Point. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ cumulonimbus- cloud- location The location of the cumulonimbus cloud shall be reported using the XML element iwxxm: cumulonimbusCloudLocation with valid child element aixm: AirspaceVolume |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ movement The movement observed for the tropical cyclone shall be reported using the XML element iwxxm: movement in which the value shall be one of the enumeration: “MOVING” or “STATIONARY” |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ movement- direction The movement direction observed. Only reported when the tropical cyclone is moving at least 6 km/hour or 3 knots. If reported, the angle between true north and the direction of motion of the tropical cyclone observed phenomenon shall be given in degrees using the XML element iwxxm: movementDirection. The unit of measure shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ movement- speed The movement speed observed. Only reported when the tropical cyclone is moving at least 6 km/hour or 3 knots. If reported, the speed of motion of the tropical cyclone shall be reported using the XML element iwxxm: movementSpeed, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ central- pressure The central pressure of the tropical cyclone shall be reported using the XML element iwxxm: centralPressure, with the unit of measure hectoPascals (hPa). The unit of measure shall be indicated using the XML attribute @uom with value “hPa”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ maximum- surface- wind- speed The maximum 10-minute mean surface wind speed near the centre of the tropical cyclone which shall be provided in XML element iwxxm: maximumSurfaceWindSpeed with attribute @uom indicating either of the two units of measures: "m/s" or "[kn_i]" (knot). |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.34 	Requirements class: Tropical cyclone forecast conditions

205-??.34.1 	This requirements class is used to describe the presence of a specific tropical cyclone forecast phenomenon, along with expected changes. The geometric extent of the phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.34.2 	XML elements describing the characteristics of a tropical cyclone forecast phenomenon shall conform to all requirements specified in Table 205-??.35.

205-??.34.3  	XML elements describing the characteristics of a tropical cyclone forecast phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.35.

#### Table 205-?? .35 . Requirements class xsd-tropical-cyclone-forecast-conditions

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- forecast- conditions |  |
| Target type | Data instance |
| Name | Tropical cyclone forecast conditions |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- forecast- conditions/ valid The content model of this element shall have a value that matches the content model of iwxxm: TropicalCycloneForecastConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions / phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml:TimeInstant to indicate the time at which meteorological phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ tropical- cyclone- position The tropical cyclone position shall be reported using the XML element iwxxm: tropicalCyclonePosition with valid child element gml: Point. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- observed- conditions/ maximum- surface- wind- speed The maximum 10-minute mean surface wind speed near the centre of the tropical cyclone which shall be provided in XML element iwxxm: maximumSurfaceWindSpeed with attribute @uom indicating either of the two units of measures: "m/s" or "[kn_i]" (knot). |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.35 	Requirements class: Volcanic Ash Advisory

205-??.35.1 	This requirements class is used to describe the volcanic ash advisory report within which the characteristics of a specific Volcanic Ash Advisory phenomenon are described.

Note: 	The reporting requirements for Volcanic Ash Advisories are specified in the Technical Regulations (WMONo. 49), Volume II, Part II, Appendix 2, section 3.

205-??.35.2 	XML elements describing Volcanic Ash Advisory reports shall conform to all requirements specified in Table 205-??.36.

205-??.35.3 	XML elements describing Volcanic Ash Advisory reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.36.

#### Table 205-?? .36 . Requirements class xsd-volcanic-ash-advisory

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory |  |
| Target type | Data instance |
| Name | Volcanic Ash Advisory |
| Dependency | http:// def. wmo. int/ metce/ 2013/ req/ xsd- erupting- volcano, 202-16.8 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- observed- or- estimated- conditions, 205-??.36 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions, 205-??.37 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshAdvisory. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ issue- time The issuance time of this Volcanic Ash Advisory report shall be indicated using XML element iwxxm: issueTime with valid child element gml: TimeInstant. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ issuing- volcanic- ash- advisory- centre The air traffic services unit responsible for the subject airspace shall be indicated using the XML element iwxxm: issuingVolcanicAshAdvisoryCentre with a valid child element aixm: Unit. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ volcano Details of the volcano that is the source of the volcanic ash shall be reported using the XML element iwxxm: volcano with valid child element metce: EruptingVolcano. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ volcano- unnamed An unnamed volcano should be indicated with an XML element iwxxm: volcano/ metce: EruptingVolcano/ metce: name having a value "UNNAMED" |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ volcano- unknown An unknown volcano name should be indicated with an XML element iwxxm: volcano/ metce: EruptingVolcano/ metce: name having a value "UNKNOWN" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ volcano- location- unknown An unknown volcano location should be indicated with an empty XML element iwxxm: volcano/ metce: EruptingVolcano/ metce: position with an attribute @nilReason = “http: // codes. wmo. int/ common/ nil/ unknown ” |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ advisory- number The advisory number of this Volcanic Ash Advisory report shall be indicated using XML element iwxxm: advisoryNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ information- source The information source of the Volcanic Ash Advisory report shall be indicated using XML element iwxxm: informationSource. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ colourCode The aviation colour code of the Volcanic Ash Advisory report shall be indicated with XML attribute iwxxm: colourCode/ @xlink: href and the value shall be the URI of a valid weather phenomenon code from ICAO Annex 15: Volcanic Aviation Colour Code. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ colourCode- unkown An unknown aviation colour code shall be indicated with an empty XML element iwxxm: colourCode with an attribute @nilReason = "http: // codes. wmo .int/ common/ nil/ unknown" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ colourCode- not- given An aviation colour code that is not given shall be indicated with an empty XML element iwxxm: colourCode with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ withheld" |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ colourCode- nil A NIL aviation colour code shall be indicated with an empty XML element iwxxm: colourCode with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ missing". |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ observation If reported, XML element iwxxm: observation shall contain a valid child element iwxxm: VolcanicAshObservedOrEstimatedConditions |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ forecast If reported, XML element iwxxm: forecast shall contain a valid child element iwxxm: VolcanicAshForecastConditions |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ remarks The XML element iwxxm: remarks shall contain the remarks of the advisory. When there are no remarks, it shall be indicated with an empty XML element iwxxm: remarks with an attribute @nilReason = “http: // codes. wmo. int/ common/ nil/ inapplicable”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- advisory/ next- advisory- time The time at which the next advisory is expected to be issued shall be indicated with XML element iwxxm: nextAdvisoryTime. Use attribute gml: timePosition/ @indeterminatePosition with a value from enumeration “before” or “after” to indicate if the actual temporal position is before or after the specified value. When no subsequent advisory is expected to be issued it shall be indicated by an empty iwxxm: nextAdvisoryTime with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable”. |

Notes:

- Requirements for reporting the Volcanic Ash Advisory phenomenon are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 2, 3.1.1.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

#### 205-??.36 	Requirements class: Volcanic ash observed or estimated conditions

205-??.36.1 	This requirements class is used to describe the observed or estimated conditions of volcanic ash phenomenon.

205-??.36.2 	XML elements describing the observed or estimated conditions of volcanic ash phenomenon shall conform to all requirements specified in Table 205-??.37.

205-??.36.3 	XML elements describing the observed or estimated conditions of volcanic ash phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.37.

Table 205-?? .37 . Requirements class xsd-volcanic-ash-observed-or-estimated-conditions

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- observed- or- estimated- conditions |  |
| Target type | Data instance |
| Name | Volcanic ash observed or estimated conditions |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated, 205-??.39 |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated, 205-??.38 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml:TimeInstant to indicate the time at which volcanic ash is observed or estimated to occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ wind The XML element iwxxm: wind shall contain a valid child element iwxxm: WindObservedOrEstimated. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ ash- cloud The XML element iwxxm: ashCloud shall contain a valid child element iwxxm: VolcanicAshCloudObservedOrEstimated |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ is- estimated The XML attribute @isEstimated indicates whether the characteristics of the phenomenon is observed (empty or false) or estimated (true). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- conditions/ status The XML attribute @status indicates volcanic ash observation (or estimation) status which has value in enumeration: “IDENTIFIABLE” or “NOT_IDENTIFIABLE”. |

#### 205-??.37 	Requirements class: Volcanic ash forecast conditions

205-??.37.1 	This requirements class is used to describe the forecast conditions of volcanic ash phenomenon.

205-??.37.2 	XML elements describing the forecast conditions of volcanic ash phenomenon shall conform to all requirements specified in Table 205-??.38.

205-??.37.3 	XML elements describing the forecast conditions of volcanic ash phenomenon shall conform to all requirements of all relevant dependencies specified in Table 205-??.38.

Table 205-?? .38 . Requirements class xsd-volcanic-ash-forecast-conditions

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions |  |
| Target type | Data instance |
| Name | Volcanic ash forecast conditions |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- forecast, 205-??.40 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forcast- conditions/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshForecastConditions. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions/ phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml:TimeInstant to indicate the time at which volcanic ash is forecast to occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions/ ash- cloud The XML element iwxxm: ashCloud shall contain a valid child element iwxxm: VolcanicAshCloudForecast |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- forecast- conditions/ status The XML attribute @status indicates volcanic ash forecast status which has value in enumeration: “IDENTIFIABLE” or “NOT_IDENTIFIABLE”. |

#### 205-??.38 	Requirements class: Volcanic ash cloud observed or estimated

205-??.38.1 	This requirements class is used to describe the observed or estimated volcanic ash clouds.

205-??.38.2 	XML elements describing the observed or estimated volcanic ash cloud shall conform to all requirements specified in Table 205-??.39.

205-??.38.3 	XML elements describing the observed or estimated volcanic ash cloud shall conform to all requirements of all relevant dependencies specified in Table 205-??.39.

#### Table 205-?? .39 . Requirements class xsd-volcanic-ash-cloud-observed-or-estimated

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated |  |
| Target type | Data instance |
| Name | Volcanic ash cloud observed or estimated |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshCloudObservedOrEstimated. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated/ ashCloudExtend The geometric extent of the volcanic ash cloud shall be reported using the XML element iwxxm: ashCloudExtent with valid child element aixm: AirspaceVolume. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud/ direction- of- motion If reported, the angle between true north and the direction of motion of the volcanic ash cloud shall be given in degrees using the XML element iwxxm: directionOfMotion. The unit of measure shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- observed- or- estimated/ speed- of- motion The speed of motion of the volcanic ash cloud observed phenomenon shall be reported using the XML element iwxxm: speedOfMotion, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.39 	Requirements class: Wind observed or estimated

205-??.39.1 	This requirements class is used to describe the observed or estimated wind.

205-??.39.2 	XML elements describing the observed or estimated wind shall conform to all requirements specified in Table 205-??.40.

205-??.39.3 	XML elements describing the observed or estimated wind shall conform to all requirements of all relevant dependencies specified in Table 205-??.40.

#### Table 205-?? .40 . Requirements class xsd-wind-observed-or-estimated

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated |  |
| Target type | Data instance |
| Name | Wind observed or estimated |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated/ valid The content model of this element shall have a value that matches the content model of iwxxm: WindObservedOrEstimated. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated/ verticalLayer The vertical layer in terms of flight levels in which wind is provided shall be described using the XML element iwxxm: verticalLayer with valid child element aixm: AirspaceLayer. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated/ wind- direction If reported, the angle between true north and the wind direction within the layer shall be given in degrees using the XML element iwxxm: windDirection. The unit of measure shall be indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated/ wind- speed The wind speed within the layer shall be reported using the XML element iwxxm: windSpeed, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- wind- observed- or- estimated/ variable- wind- direction The attribute @variableWindDirection, when true, indicates that the wind direction is variable. In this case, iwxxm: windDirection shall be empty. |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.40 	Requirements class: Volcanic ash cloud forecast

205-??.40.1 	This requirements class is used to describe the forecast volcanic ash clouds.

205-??.40.2 	XML elements describing the forecast volcanic ash cloud shall conform to all requirements specified in Table 205-??.41.

205-??.40.3 	XML elements describing the forecast volcanic ash cloud shall conform to all requirements of all relevant dependencies specified in Table 205-??.41.

#### Table 205-?? .41 . Requirements class xsd-volcanic-ash-cloud-forcast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- forecast |  |
| Target type | Data instance |
| Name | Volcanic ash cloud forecast |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: VolcanicAshCloudForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- volcanic- ash- cloud- forecast/ ashCloudExtend The geometric extent of the volcanic ash cloud shall be reported using the XML element iwxxm: ashCloudExtent with valid child element aixm: AirspaceVolume. |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.41 	Requirements class: Space Weather Advisory

205-??.41.1 	This requirements class is used to describe the Space Weather Advisory report within which the characteristics of a specific Space Weather Advisory phenomenon are described.

Note: 	The reporting requirements for Space Weather Advisories are specified in the Technical Regulations (WMO No. 49), Volume II, Part II, Appendix 2, section 6.

205-??.41.2 	XML elements describing Space Weather Advisory reports shall conform to all requirements specified in Table 205-??.42.

205-??.41.3	XML elements describing Space Weather Advisory reports shall conform to all requirements of all relevant dependencies specified in Table 205-??.42.

#### Table 205-?? .42 . Requirements class xsd-space-weather-advisory

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory |  |
| Target type | Data instance |
| Name | Space Weather Advisory |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis, 205-??.42 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ valid The content model of this element shall have a value that matches the content model of iwxxm: SpaceWeatherAdvisory. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- tropical- cyclone- advisory/ issue- time The issuance time of this Space Weather Advisory report shall be indicated using XML element iwxxm: issueTime with valid child element gml: TimeInstant. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ issuing- space- weather- centre The space weather centre responsible for the issuance of the report shall be indicated using the XML element iwxxm: issuingSpaceWeatherCentre with a valid child element aixm: Unit. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ advisory- number The advisory number of this Space Weather Advisory report shall be indicated using XML element iwxxm: advisoryNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ replaced- advisory- number The number of the advisory being replaced shall be indicated using XML element iwxxm: replacedAdvisoryNumber. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ phenomenon If reported, the space weather phenomenon shall be given by XML attribute iwxxm: phenomenon/ @xlink: href and the value shall be the URI of a valid space weather phenomenon code from ICAO Annex 3 / WMO No. 49-2: Appendix 6 Section 6.1. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ analysis Observed and forecast space weather information. Analyses should be reported in the order in which they occur, starting with the initial observed/ forecast conditions and proceeding through each subsequent forecast to the end of the period. If reported, XML element iwxxm: analysis shall contain a valid child element iwxxm: SpaceWeatherAnalysis. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ remarks The XML element iwxxm: remarks shall contain the remarks of the advisory. When there are no remarks, it shall be indicated with an empty XML element iwxxm: remarks with an attribute @nilReason = “http: // codes. wmo. int/ common/ nil/ inapplicable”. |
| Requirement | http:// icao. Int/ iwxxm/ 3. 0/ req/ xsd- space- weather- advisory/ next- advisory- time The time at which the next advisory is expected to be issued shall be indicated with XML element iwxxm: nextAdvisoryTime. Use attribute gml: timePosition/ @indeterminatePosition with a value from enumeration “before” or “after” to indicate if the actual temporal position is before or after the specified value. When no subsequent advisory is expected to be issued it shall be indicated by an empty iwxxm: nextAdvisoryTime with an attribute @nilReason = "http: // codes. wmo. int/ common/ nil/ inapplicable”. |

Notes:

- Requirements for reporting the Tropical Cyclone Advisory phenomenon are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 2, 6.1.1.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

#### 205-??.42 	Requirements class: Space weather analysis

205-??.42.1 	This requirements class is used to describe observed or forecast space weather phenomena.

205-??.42.2 	XML elements describing the observed or forecast space weather phenomena shall conform to all requirements specified in Table 205-??.43.

205-??.42.3 	XML elements describing the characteristics of observed or forecast space weather phenomena shall conform to all requirements of all relevant dependencies specified in Table 205-??.43.

#### Table 205-?? .43 . Requirements class xsd-space-weather-analysis

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis |  |
| Target type | Data instance |
| Name | Space weather analysis |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region, 205-??.43 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis/ valid The content model of this element shall have a value that matches the content model of iwxxm: SpaceWeatherAnalysis. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis / phenomenon- time The XML element iwxxm: phenomenonTime shall contain a valid child element gml:TimeInstant to indicate the time at which meteorological phenomena occur. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis/ region The location(s) affected by the space weather phenomena shall be described using the XML element iwxxm: region with valid child element SpaceWeatherRegion. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- analysis/ time- indicator The XML attribute @timeIndicator shall indicate whether the information is observed and expected to continue, or forecast with one of the enumeration: “OBSERVATION” or “FORECAST” |

Note: 	Units of measurement are specified in accordance with 1.9 above.

#### 205-??.43 	Requirements class: Space weather region

205-??.43.1 	This requirements class is used to describe the location affected by space weather phenomena. The geometric extent of the phenomenon is specified as a two-dimensional horizontal region with bounded vertical extent.

205-??.43.2 	XML elements describing the location affected by space weather phenomena shall conform to all requirements specified in Table 205-??.44.

205-??.43.3  	XML elements describing the location affected by space weather phenomena shall conform to all requirements of all relevant dependencies specified in Table 205-??.44.

#### Table 205-?? .44 . Requirements class xsd-space-weather-region

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region |  |
| Target type | Data instance |
| Name | Space weather region |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region/ valid The content model of this element shall have a value that matches the content model of iwxxm: SpaceWeatherRegion. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region / location The XML element iwxxm: location shall contain a valid child element aixm: AirspaceVolume to indicate the geographic location at which space weather phenomena occur |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- space- weather- region/ location- indicator The XML attribute iwxxm: locationIndicator/ xlink: href shall have a value that is the URI of a valid latitude bands for advisories from ICAO Annex 3 / WMO No. 49-2 Attachment E: Spatial Ranges and Resolutions for Space Weather Advisory Information. |

#### 205-??.44 	Requirements class: Report

205-??.44.1 	This requirements class is used to describe the report within which the characteristics common to all IWXXM reports (such as METAR and SIGMET) are described. The report type cannot be instantiated directly, but report members are included on all sub-types.

205-??.44.2 	XML elements describing report shall conform to all requirements specified in Table 205-??.45.

205-??.44.3 	XML elements describing report shall conform to all requirements of all relevant dependencies specified in Table 205-??.45.

#### Table 205-?? .45 . Requirements class xsd-report

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 2. 1/req / xsd- report |  |
| Target type | Abstract data instance |
| Name | Report |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- report/ valid The content model of this element shall have a value that matches the content model of iwxxm: Report. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- report/ report- status The status of the report shall be indicated using the XML attribute @reportStatus with the value being one of the enumeration: “NORMAL”, “AMENDMENT” or “CORRECTION”. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ permissible- usage The permissible usage (operational or non-operational) for the report shall be indicated using the XML attribute @permissibleUsage. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ permissible- usage- reason When the permissible usage is non-operational, the reason for this usage (test or exercise) shall be indicated using XML attribute @permissibleUsageReason. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ permissible- usage- supplementary When the permissible usage is non-operational, the human-readable descriptive supplementary information shall be indicated using XML attribute @permissibleUsageSupplementary. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translatedBulletinId If reported on a report that was translated from the Traditional Alphanumeric Code (TAC) report, XML attribute @translatedBulletinId shall contain the bulletin ID of the form ‘TTAAiiCCCYYGGgg’. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translatedBulletinReceptionTime If reported on a report that was translated from TAC, XML attribute @translatedBulletinReceptionTime shall contain the time at which the bulletin was received by the translation centre. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translatedCentreDesignator If reported on a report that was translated from TAC, XML attribute @translatedCentreDesignator shall contain the ICAO designator of the translation centre. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translatedCentreName If reported on a report that was translated from TAC, XML attribute @translatedCentreName shall contain the name of the translation centre. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translationTime If reported on a report that was translated from TAC, XML attribute @translationTime shall contain the time at which the bulletin was translated. |
| Requirement | http:// icao. int/ iwxxm/ 2. 1/ req/ xsd- report/ translatedFailedTAC If reported on a report that was translated from TAC which could not be completely translated, XML attribute @translationFailedTAC shall contain the original TAC report that was not translated. When translation fails, only the report type (i.e. SIGMET or METAR), translation information and other basic report metadata shall be provided. In this case no translated content shall be included other than the original TAC. Permissible usage may be set as normal and TAC that failed translation may still be used for operational purposes, but under no circumstances should partially translated content be distributed or marked as operational. |

#### 205-??.45	Requirements class: Aerodrome cloud forecast

205-??.45.1 	This requirements class is used to describe forecast cloud conditions at an aerodrome. The class is targeted at providing a basic description of the forecast cloud conditions as required for civil aviation purposes.

Notes:

- A report with status “CORRECTED” indicates that content has been amended to correct an error identified in an earlier report. The XML element //om: OM_ Observation/ om: resultTime/ gml: TimeInstant is used to reflect the dissemination time of the corrected report.

- A report with status “MISSING” indicates that a routine report has not been provided on the anticipated timescales. Such a report does not contain details of any observed or forecast meteorological conditions and is often referred to as a “NIL” report.

- Representations providing more detailed information may be used if required.

- The requirements for reporting forecast cloud conditions are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 2.2.5 and 1.2.4.

205-??.45.2 	XML elements describing forecast cloud conditions shall conform to all requirements specified in Table 205-??.46.

205-??.45.3 	XML elements describing forecast cloud conditions shall conform to all requirements of all relevant dependencies specified in Table 205-??.46.

#### Table 205-?? .46 . Requirements class xsd-aerodrome-cloud-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast |  |
| Target type | Data instance |
| Name | Aerodrome cloud forecast |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer, 205-??.49 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeCloudForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast/ vertical- visibility When cloud of operational significance is forecast, then the XML element iwxxm: AerodromeCloudForecast/ iwxxm: verticalVisibility shall be used to report the vertical visual range. The vertical distance shall be expressed in metres or feet and the unit of measure shall be indicated using the XML attribute @uom with value “m” (metres) or “[ft_i]” (feet). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast/ cloud- layers When cloud of operational significance is forecast, then the XML element iwxxm: layer, containing a valid child element iwxxm: CloudLayer, shall be used to describe each cloud layer. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- cloud- forecast/ number- of- cloud- layers No more than four cloud layers shall be reported. If more than four significant cloud layers are forecast, then the four most significant cloud layers with respect to aviation operations shall be prioritized. |

Notes:

- Cloud of operational significance includes cloud below 1 500 metres or the highest minimum sector altitude, whichever is greater, and cumulonimbus whenever present.

- Vertical visibility is defined as the vertical visual range into an obscuring medium.

- Units of measurement are specified in accordance with 1.9 above.

#### 205-??.46 	Requirements class: Aerodrome surface wind forecast

205-??.46.1 	This requirements class is used to describe the surface wind conditions forecast at an aerodrome as appropriate for inclusion in an aerodrome forecast (TAF) report.

Note: 	The requirements for reporting the surface wind conditions within a TAF are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 1.3.0.

205-??.46.2 	XML elements describing surface wind conditions forecast shall conform to all requirements specified in Table 205-??.47.

205-??.46.3 	XML elements describing surface wind conditions forecast shall conform to all requirements of all relevant dependencies specified in Table 205-??.47.

#### Table 205-?? .47 . Requirements class xsd-aerodrome-surface-wind-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- forecast |  |
| Target type | Data instance |
| Name | Aerodrome surface wind forecast |
| Dependency | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast, 205-??.48 |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeSurfaceWindForecast. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- forecast/ variable- wind- direction If the wind direction is variable, then the XML attribute @variableWindDirection shall have the value “true” and XML element meanWindDirection shall be absent. |

Note: 	Wind direction is reported as variable (VRB) if is not possible to forecast a prevailing surface wind direction due to expected variability, for example, during light wind conditions (less than 3 knots) or thunderstorms.

#### 205-??.47 	Requirements class: String with nil reason

205-??.47.1 	This requirements class is a nillable string. Unlike the base string, references to this type may be nil and may include a nilReason.

205-??.47.2 	XML elements describing a string with nil reason shall conform to all requirements specified in Table 205-??.48.

205-??.47.3 	XML elements describing a string with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.48.

#### Table 205-?? .48 . Requirements class: xsd-string-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- string- with- nil- reason |  |
| Target type | Data instance |
| Name | String with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- string- with- nil- reason/ value The XML element iwxxm: StringWithNilReason shall contain valid numerical content or shall have the “nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |

#### 205-??.48 	Requirements class: Aerodrome surface wind trend forecast

205-??.48.1 	This requirements class is used to describe the surface wind conditions forecast at an aerodrome as appropriate for inclusion in a trend forecast of a routine or special meteorological aerodrome report.

Note: 	The requirements for reporting the surface wind conditions within a trend forecast are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 5, 2.2.2.

205-??.48.2 	XML elements describing surface wind conditions within a trend forecast shall conform to all requirements specified in Table 205-??.49.

205-??.48.3 	XML elements describing surface wind conditions within a trend forecast shall conform to all requirements of all relevant dependencies specified in Table 205-??.49.

#### Table 205-?? .49 . Requirements class xsd-aerodrome-surface-wind-trend-forecast

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast |  |
| Target type | Data instance |
| Name | Aerodrome surface wind trend forecast |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ valid The content model of this element shall have a value that matches the content model of iwxxm: AerodromeSurfaceWindTrendForecast. |
|  | Requirements class |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ wind- direction If the forecast mean wind direction is reported, then the angle between true north and the mean direction from which the wind is forecast to be blowing shall be expressed using XML element iwxxm: meanWindDirection, with the unit of measure indicated using the XML attribute @uom with value “deg”. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ mean- wind- speed The forecast mean wind speed shall be stated using the XML element iwxxm: meanWindSpeed, with the unit of measure metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ mean- wind- speed- operator Indication that the mean wind speed is above or below the reported value. If reported, the XML element iwxxm: meanWindSpeedOperator shall be one of the values in enumeration: “ABOVE” or “BELOW”. When no operator is reported, wind speed is an exact value with identical semantics to other measured quantities. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ wind- gust- speed If reported, the forecast gust speed shall be stated using the XML element iwxxm:windGustSpeed and expressed in metres per second, knots or kilometres per hour. The unit of measure shall be indicated using the XML attribute @uom with value “m/s” (metres per second), “[kn_i]” (knots) or “km/h” (kilometres per hour). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- aerodrome- surface- wind- trend- forecast/ wind- gust- speed- operator Indication that the wind gust speed is above or below the reported value. If reported, the XML element iwxxm: windGustSpeedOperator shall be one of the values in enumeration: “ABOVE” or “BELOW”. When no operator is reported, wind gust is an exact value with identical semantics to other measured quantities. |

Notes:

- Units of measurement are specified in accordance with 1.9 above.

- The true north is the north point at which the meridian lines meet.

#### 205-??.49 	Requirements class: Cloud layer

205-??.39.1 	This requirements class is used to describe the representation of a cloud layer. The class is targeted at providing a basic description of the cloud layer as required for international civil aviation purposes.

Notes:

- Representations providing more detailed information may be used if required.

- The requirements for reporting cloud information are specified in the Technical Regulations (WMO-No. 49), Volume II, Part II, Appendix 3, 4.5 and Appendix 5, 2.2.5 and 1.2.4.

205-??.49.2 	XML elements describing cloud layers shall conform to all requirements specified in Table 205-??.50.

205-??.49.3 	XML elements describing cloud layers shall conform to all requirements of all relevant dependencies specified in Table 205-??.50.

#### Table 205-?? .50 . Requirements class: xsd-cloud-layer

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer |  |
| Target type | Data instance |
| Name | Cloud layer |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ valid The content model of this element shall have a value that matches the content model of iwxxm: CloudLayer. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ amount The XML element iwxxm: amount shall be used to report an amount of cloud of operational significance. If cloud amount is reported, the value of XML attribute iwxxm: amount/ @xlink: href shall be the URI of the valid term from Code table D-8: Cloud amount reported at aerodrome. |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ base The XML element iwxxm: base shall indicate the height of the lowest level in the atmosphere that contains a perceptible quantity of cloud particles. If the cloud base is reported, then the vertical distance shall be expressed in metres or feet. The unit of measure shall be indicated using the XML attribute @uom with value “m” (metres) or “[ft_i]” (feet). |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ cloud- type If cloud type is reported, the value of XML attribute iwxxm: cloudType/ @xlink: href shall be the URI of the valid cloud type from Code table D-9: Significant convective cloud type. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ cloud- type- of- operational- significance If reporting observed cloud, then the XML element //iwxxm: CloudLayer/ iwxxm: cloudType should be used to report the most cloud of operational significance type in the layer of cloud. |
| Recommendation | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- cloud- layer/ cloud- type- none- of- operational- significance If no cloud of operational significance is reported, then the value of XML attribute iwxxm: amount/ @nilReason should be set to http:// codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance. If reporting observed cloud, then the value of XML attribute iwxxm: cloudType/ @nilReason should also be set to http:// codes. wmo. int/ common/ nil/ nothingOfOperationalSignificance. |

Notes:

- Cloud of operational significance includes cloud below 1 500 metres or the highest minimum sector altitude, whichever is greater, and cumulonimbus whenever present.

- Code table D-1 provides a set of nil-reason codes and is published at http:// codes. wmo. int/ common/ nil.

- Units of measurement are specified in accordance with 1.9 above.

- Code table D-8 is published online at http:// codes. wmo. int/ 49- 2/ CloudAmountReportedAtAerodrome.

- Code table D-9 is published online at http:// codes. wmo. int/ 49- 2/ SigConvectiveCloudType.

#### 205-??.50 	Requirements class: Angle with nil reason

205-??.50.1 	This requirements class is a nillable angle quantity. Unlike the base angle measure, references to this type may be nil and may include a nilReason.

205-??.50.2 	XML elements describing an angle with nil reason shall conform to all requirements specified in Table 205-??.51.

205-??.50.3 	XML elements describing an angle with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.51.

#### Table 205-?? .51 . Requirements class: xsd-angle-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- angle- with- nil- reason |  |
| Target type | Data instance |
| Name | Angle with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- angle- with- nil- reason/ value The XML element iwxxm: AngleWithNilReason shall contain valid numerical content or shall have the “xsi:nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |

#### 205-??.51 	Requirements class: Distance with nil reason

205-??.51.1 	This requirements class is a nillable distance quantity. Unlike the base distance measure, references to this type may be nil and may include a nilReason.

205-??.51.2 	XML elements describing a distance with nil reason shall conform to all requirements specified in Table 205-??.52.

205-??.51.3 	XML elements describing a distance with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.52.

#### Table 205-?? .52 . Requirements class: xsd-distance-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- distance- with- nil- reason |  |
| Target type | Data instance |
| Name | Distance with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- distance- with- nil- reason/ value The XML element iwxxm: DistanceWithNilReason shall contain valid numerical content or shall have the “nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |

#### 205-??.52 	Requirements class: Length with nil reason

205-??.52.1 	This requirements class is a nillable length quantity. Unlike the base length measure, references to this type may be nil and may include a nilReason.

205-??.52.2 	XML elements describing a length with nil reason shall conform to all requirements specified in Table 205-??.53.

205-??.52.3 	XML elements describing a length with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.53.

#### Table 205-?? .53 . Requirements class: xsd-length-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- length- with- nil- reason |  |
| Target type | Data instance |
| Name | Length with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- length- with- nil- reason/ value The XML element iwxxm: LengthWithNilReason shall contain valid numerical content or shall have the “nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |

#### 205-??.53 	Requirements class: Measure with nil reason

205-??.53.1 	This requirements class is a nillable measure. Unlike the base measure, references to this type may be nil and may include a nilReason.

205-??.53.2 	XML elements describing a measure with nil reason shall conform to all requirements specified in Table 205-??.54.

205-??.53.3 	XML elements describing a measure with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.54.

#### Table 205-?? .54 . Requirements class: xsd-measure-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- measure- with- nil- reason |  |
| Target type | Data instance |
| Name | Measure with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- measure- with- nil- reason/ value The XML element iwxxm: MeasureWithNilReason shall contain valid numerical content or shall have the “nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |

#### 205-??.54 	Requirements class: Velocity with nil reason

205-??.54.1 	This requirements class is a nillable velocity quantity. Unlike the base velocity measure, references to this type may be nil and may include a nilReason.

205-??.54.2 	XML elements describing a velocity with nil reason shall conform to all requirements specified in Table 205-??.55.

205-??.54.3 	XML elements describing a velocity with nil reason shall conform to all requirements of all relevant dependencies specified in Table 205-??.55.

#### Table 205-?? .55 . Requirements class: xsd-velocity-with-nil-reason

| Requirements class |  |
| --- | --- |
| http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- velocity- with- nil- reason |  |
| Target type | Data instance |
| Name | Velocity with nil reason |
| Requirement | http:// icao. int/ iwxxm/ 3. 0/ req/ xsd- velocity- with- nil- reason/ value The XML element iwxxm: VelocityWithNilReason shall contain valid numerical content or shall have the “nil” attribute set to “true” and the “nilReason” attribute set with the reason for nil content. |