| World Meteorological Organization COMMISSION FOR INFRASTRUCTURE Steering Group on Radio-Frequency Coordination SG-RFC 2020 meeting Darmstadt, Germany, 11 to 13 February 2020 | SG-RFC-2020 Agenda item: 3 |
| --- | --- |
|  | Submitted by: David Franc 3.FEB.2020 Restricted access: N |

# Information Relating to WRc-23 Agenda Item 1.4

## DISCUSSION

WRC-23 Agenda item 1.4 addresses the potential use of high-altitude platforms for deploying IMT base stations (high altitude IMT base stations, or HIBS).  The frequency band 2500-2690 is included in the list of bands under consideration for use by HIBS.  Operational experience in the United States has shown that transmissions from broadband wireless base stations operating below 2690 MHz can interfere with meteorological radars operating in the lower part of the 2700-2900 MHz frequency band.

The interference from broadband wireless base stations into the meteorological radars in the United States was found to be due to out-of-band emissions from wireless broadband base stations falling into the radar band (above 2700 MHz), and not due to the radar receiver selectivity extending into the broadband wireless band (below 2700 MHz).  The interference cases were limited to relatively small separation distances, however, the interference was occurring into the radar via radar antenna side and backlobes.  The scenario under WRC-23 Agenda Item 1.4 will place broadband base stations in locations where they can fall within the metrological radar antenna main beam, resulting in an antenna gain increase by as much as 35 dB.

Attached is a Report prepared by the National Telecommunications and Information Administration (NTIA), Institute for Telecommunication Sciences (ITS).  This report may be of interest to others who operate meteorological radars in 2700-2900 MHz and are concerned about WRC-23 Agenda Item 1.4.

The Report may also be found at the following URL:

https://www.its.bldrdoc.gov/publications/details.aspx?pub=2684

## References

[1] Provisional Final Acts of WRC-19

[2] Result of CPM23-1

## Recommended Text

Text for meeting report to be developed based on discussion during the SG-RFC meeting

____________