TT-ApMD-1/Doc 4(1) rev 2.
Uses for GTS metadata catalogue (Vol A, Vol C1, Routeing)
	Use
	Information need
	How metadata must support 

	I’m a User.How can I obtain observations from stations in some area?  I can receive only alphanumeric code form.
	Station identification iiiii by identifier, 
Should be in the metadata
Exchange schedule
Not done consistently in metadata or even within the same metadata element
Code form for routine exchange
Dataset identification (Abbreviated heading = TTAAii CCCC)

	Info on left must be listed in catalogue of exchanged data

	Does the station iiiii more frequent observation than reports I receive?
	Station identification iiiii

Observation schedule


	Info on left must be listed in catalogue of observing stations

	I’m RTH supporting the user.  Which RTH should I ask about delivery of requested dataset?
	Dataset identification

Source RTH

Destination RTH


	Info on left must be listed in catalogue of dissemination

	(future) I’m GISC supporting the user.  Am I in position to serve the data of interest, or should I advice the user to contact originator?
	Dataset identification

Extent of exchange (global, regional, bilateral)


	Info on left must be listed in catalogue of dissemination

	I’m monitoring observation data.  Reports from a station iiiii are unavailable for 1200 UTC.  Can I suppose that should be disseminated?
	Dataset identification

Station identification iiiii

Reports scheduled to be transmitted 

	Info on left must be available in catalogues of exchanged data and observing stations.  It may  take any of forms: single record in complex structure, linked two records, or two relational tables (as it has been).

	Are products yyyy available?
	In future will need: Product descriptions and identifiers in searchable form
(at least, production time, forecast time, vertical level, physical quantity, and horizontal resolution are necessary to identify a traditional GRIB Edition 1 message) – at present this is probably encoded in the bulletin header
To reproduce what we can do with Vol C1:

Bulletin name for product that implicitly defines the parameters within the product file
Area covered by product
Exchange schedules

Code forms for routine exchange
	Info available as tagged information



	What is the policy applied to the data?
	Essential/Additional/Other

Reference to data owner and how to find information about specific policy (desirable that the information is in the metadata, codified if possible)
	May need more information about usage constraints than in current guidance.
Needs a new process for authorising additional data.

	Who is responsible for data?
	Vol C1 gives centre within country that is responsible for collecting the data.

Vol C1 also lists the RTH responsible for placing the data on the GTS.
In future perhaps use:
Data owner – may not be possible (eg SHIP bulletins)
Centre responsible for injecting information (NC or DCPC)

Centre responsible for transmission on WIS circuits (RTH/GISC)
	May need to add information to allow communications responsibilities to be identified

	
	
	


