| WORLD METEOROLOGICAL ORGANIZATION _________________________ COMMISSION FOR BASIC SYSTEMS OPAG ON INFORMATION SYSTEMS & SERVICES Expert Team on Communication Techniques and Structure Geneva, 30 Apr – 03 May 2012 |  | ET-CTS/Doc. 5.3.1 (19 IV 2012) ____________ ITEM 5 ENGLISH only |
| --- | --- | --- |

Consideration for Allocation of IPv6 address space

(Submitted by RTH Offenbach)

Summary and purpose of document This paper presents a recommendation for the allocation of IPv6 address space.

ACTION PROPOSED: The meeting is invited to review the document.

- request global /48 PI or larger if justified (multihoming possible, no NAT required)

- subnet into /64:

  - recommended by RFC3177

  - consistency makes management easy

  - MUST for SLAAC (Stateless Address AutoConfiguration)

  - Could be seen as loss of address space

- Subnets > 64 (address space conservation)

  - recommended for special cases

    - /126 – valid for p2p

    - /127 – not valid for p2p (RFC3627)

    - /128 – loopback

  - can complicate management

  - must avoid overlap with specific addresses:

    - Router Anycast (RFC3513)

    - Embedded RP (RFC3956)

    - ISATAP addresses (Intra-Site Automatic Tunnel Adressing Protocol)

- Use ULA addresses only where no NAT will ever be required (eg. Internal VoIP)

(ULA = unique local address  is an IPv6 address in the block fc00::/7)

Additional information:

IPv6 Address Type Identification

Address type         Binary prefix        IPv6 notation

------------         -------------        -------------

Unspecified          00...0  (128 bits)   ::/128

Loopback             00...1  (128 bits)   ::1/128

Multicast            11111111             FF00::/8

Link-local unicast   1111111010           FE80::/10

Site-local unicast   1111111011           FEC0::/10

Global unicast       (everything else)