CBS/ISS/VPN via the Internet, p. 4

WORLD METEOROLOGICAL ORGANIZATION

COMMISSION FOR BASIC SYSTEMS

OPAG ON INFORMATION SYSTEMS & SERVICES 

Guide for Virtual Private Networks (VPN) via the Internet between GTS centres

TABLE OF CONTENT

41
Introduction


52
What is a VPN ?


52.1
Definition


62.2
Types of VPN


62.2.1
The Link Layer solutions


62.2.1.1
ATM and Frame Relay


72.2.1.2
MPLS


72.2.1.3
PPTP and L2TP


82.2.2
Transport and application Layer


82.2.2.1
SSL and TLS


82.2.2.2
SSH


92.2.3
So, why IPSEC?


103
What is IPSEC ?


103.1
IPSec architecture


113.2
IPSec services and modes


133.3
Security associations


133.4
Authentication Header - AH


143.5
Encapsulating Security Payload – ESP


143.6
Internet Key Exchange – IKE


153.7
Conclusion


164
IPSEC and the GTS


164.1
Cryptography and the law


174.2
A quick view on the GTS


174.2.1
Physical layer


174.2.2
Upper layers


174.3
The Internet


174.4
The suggested approach


184.5
Solution suppliers


184.6
Network architecture


194.7
Implementation scenario


204.8
Multicast over an IPSEC VPN


204.9
DMVPN solution


214.10
OpenVPN solution


245
Operators solutions


245.1
Star, partially meshed, fully meshed networks


255.2
A brief introduction to MPLS


255.3
IPSec VPN over the Internet


255.4
Conclusion


266
Appendices


266.1
Example: A Cisco based configuration


266.1.1
The test platform


266.1.2
ADSL connection with NAT


276.1.3
VPN between the routers


296.1.3.1
IKE


306.1.3.2
AH and ESP


306.1.3.3
Security Association


306.1.4
Final configuration


336.1.5
Performance


336.2
Example: A Cisco DMVPN configuration


336.2.1
The test environment


336.2.2
HUB1 configuration


356.2.3
HUB2 configuration


366.2.4
SPOKE configuration


376.3
Configuration Example for an OpenVPN-based solution


376.3.1
Server Configuration


386.3.2
Generate the Certificate Authority (CA).


386.3.3
Generate certificate and key for the Server


406.3.4
Generate certificates and keys for the clients (Met Centers).


416.3.5
Generate Diffie-Hellman parameter


416.3.6
Coping files here and there


426.3.7
Client configuration


436.4
Glossary


536.5
Reference



































































Blank
1 Introduction

After describing various concepts related to VPN and IPSEC, this document presents a potential methodology to introduce the technical solution on the GTS and presents why these tools can enhance communication capabilities among WMO Members for operational traffic exchanges.

Chapter 2 introduces the concept of VPN and presents various technical alternatives to IPSEC. As IPSEC is the most appropriate protocol for secure network-to-network communication the document will then focus on this solution.

Chapter 3 briefly describes what  IPSEC is. IPSEC is not a simple solution to understand. It covers a lot of technical aspects, allows various flavours of algorithms, and is better described as a framework instead of a protocol.

Chapter 4 focuses on application of IPSEC on the GTS. A selection of protocols is proposed. It shows what benefits WMO Members could expect from the use of IPSEC. 

Chapter 5 shows the technical evolution of IP services from the operators. Evolving from Frame Relay, now deprecated, to IP solutions operators offer VPN solutions either MPLS based or IPSEC based.  

Chapter 6 gives:
· complete configuration examples on Cisco routers with the protocols selected in chapter 3.

· a glossary of terms related to VPN

· some references on IPSEC

2 What is a VPN ?

2.1 Definition

The definition below comes from “What is a VPN” – P.Ferguson and G.Huston

Perhaps the simplest method of attempting to arrive at a simple definition for a VPN is to look at each word in the acronym individually, and then subsequently tie each of them together in a simple, common sense, and meaningful fashion.

Let’s start by examining the word “network” This is perhaps the least difficult term for us to define and understand, since the commonly accepted definition is fairly uncontroversial and generally accepted throughout the industry. A network consists of any number of devices which can communicate through some arbitrary method. Devices of this nature include computers, printers, routers, and so forth, and may reside in geographically diverse locations. The methods in which they may communicate are numerous, since there are countless electronic signalling specifications, and data-link, transport, and application layer protocols. For the purposes of simplicity, let’s just agree that a “network” is a collection of devices that can communicate in some fashion, and can successfully transmit and receive data amongst themselves.

The term “private” is fairly straightforward, and is intricately related to the concept of “virtualization” insofar as a VPN is concerned, as we’ll discuss in a moment. In the simplest of definitions, “private” means that communications between two (or more) devices is, in some fashion, secret – that the devices which are not participating in the “private” nature of communications are not privy to the communicated content, and that they are indeed completely unaware of the private relationship altogether. Accordingly, data privacy and security (data integrity) are also important aspects of a VPN which need to be taken into consideration when considering any particular VPN implementation.

Another means of expressing this definition of "private" is through its antonym, "public." A “public” facility is one which is openly accessible, and is managed within the terms and constraints of a common public resource, often via a public administrative entity. By contrast, a “private” facility is one where access is restricted to a defined set of entities, and third parties cannot gain access. Typically, the private resource is managed by the entities that have exclusive right of access. Examples of this type of private network can be found in any organizational network which is not connected to the Internet, or to any other external organizational network, for that matter. With this definition the current GTS is a private network

These networks are private due to the fact that there is no external connectivity, and thus no external network communications. Another important aspect of “privacy” in a VPN is through its technical definition, as describing the privacy of addressing and routing system, meaning that the addressing used within a VPN community of interest is separate and discrete from that of the underlying shared network, and from that of other VPN communities. The same holds true for the routing system used within the VPN and that of the underlying shared network. The routing and addressing scheme within a VPN should, for all intents and purposes, be self-contained, but this degenerates into a philosophical discussion on the context of the term “VPN.”  

“Virtual” is a concept that is slightly more complicated. The New Hacker’s Dictionary [2] defines virtual as –

virtual /adj./ [via the technical term “virtual memory”, prob. from the term “virtual image” in optics] 1. Common alternative to {logical}; often used to refer to the artificial objects (like addressable virtual memory larger than physical memory) simulated by a computer system as a convenient way to manage access to shared resources. 2. Simulated; performing the functions of something that isn’t really there. An imaginative child’s doll may be a virtual playmate. Oppose {real}.

For a VPN, the definition given in 2. is perhaps the most appropriate comparison for virtual networks. The “virtualization” aspect is one that is similar to what we briefly described above as “private,” however, the scenario is slightly modified – the private communication is now conducted across a network infrastructure that is shared by more than a single organization. Thus, the private resource is actually constructed by using the foundation of a logical partitioning of some underlying common shared resource, rather than by using a foundation of discrete and dedicated physical circuits and communications services. Accordingly, the “private” network has no corresponding “private” physical communications system. Instead, the “private” network is a virtual creation which has no physical counterpart. The virtual communications between two (or more) devices are due to the fact that the devices which are not participating in the virtual communications are not privy to the content of the data, and that they are also altogether unaware of the private relationship between the virtual peers. The shared network infrastructure could, for example, be the global Internet and the number of organizations or other users not participating in the virtual network may literally number into the thousands, hundreds of thousands, or millions.

A VPN can also said to be a discrete network –

discrete \dis*crete"\, a. [L. discretus, p. p. of discernere. See Discreet.] 1. Separate; distinct; disjunct.

The discrete nature of VPN’s allow both privacy and virtualization. While VPN’s are not completely separate, per se, the distinction is that they operate in a discrete fashion across a shared infrastructure, providing exclusive communications environments which do not share any points of interconnection.

The combination of these terms produces VPN – a private network, where the privacy is introduced by some method of virtualization. A VPN could be built between two end-systems or between two organizations, between several end-systems within a single organization or between multiple organizations across the global Internet, between individual applications, or any combination of the above.

The common and somewhat formal characterization of the VPN, and perhaps the most straightforward and strict definition, is:

[image: image15.wmf]HeadQuarters

Site 4

Site 1

Site 3

Site 2

Fully meshed

This definition introduces a concept, the VPN, not related to any technical implementation.

There are quite a lot of technical implementations of VPN.

2.2 Types of VPN

[image: image16.png]Application
Layer

Transport
Layer

Network Layer

Link Layer





A simplified version of the TCP/IP layer model is shown on left. 

The technical implementation of the VPN is related to this model:

· On the link layer one can find :

· ATM and Frame Relay connection

· MPLS (Multi Protocol Label Switching)

· Link-Layer Encryption (L2TP or PPTP)

· On the network layer :

· IPSEC

· On the transport and application layer

· SSL (Secure Socket Layer) is a protocol proposed by Netscape mainly for http traffic encryption ; the successor of SSL is TLS
· TLS (Transport Layer Security) is a proposed standard by IETF (Internet Engineering Task Force) based on SSL

· SSH

2.2.1 The Link Layer solutions

2.2.1.1 ATM and Frame Relay

Following the definition of VPN gave on 1, ATM and Frame Relay solution must be considered as VPN. By construction a Frame Relay (and ATM) network, like the RMDCN is a VPN. The provider Equant has a network, which is securely divided among all the customers. Therefore, on a global telecommunication system, multiple isolated “sub-“networks coexist. In this case, the VPN rely on the operator.

2.2.1.2 MPLS

Nowadays, as IP is the base protocol, most of the provider solutions are based on MPLS. Multi Protocol Layer Switching is a protocol originated by Cisco (the Tag Switching initiative), but now widely adopted. 

In the traditional IP world, every router must route every packet on the network. Routing is rather complex and by the way slow. MPLS introduce (or use) the concepts of tags. Packets are “tagged” at the “entrance” of the WAN. Inside the WAN packets are switched (not routed) based on the tag. Tags are removed at the network exit.

This solution is now widely offered by operators. 

2.2.1.3 PPTP and L2TP 

L2TP (Layer 2 Tunnelling Protocol) and PPTP (Point to Point Tunnelling Protocol) are two solutions mainly dedicated to remote access. In the “normal” situation a remote user who wants to connect to the intranet use a PPP connection to a Remote Access Server. In this case, username and password (and also data) are transferred in plain text and therefore might be “sniffed” by potential intruders. L2TP and PPTP permit to encrypt traffic between peers leading to better security.
2.2.1.3.1 Point-to-Point Tunnelling Protocol (PPTP)

PPTP is a Layer 2 protocol that encapsulates PPP frames in IP datagrams for transmission over an IP internetwork, such as the Internet. PPTP can be used for remote access and router-to-router VPN connections. PPTP is documented in RFC 2637.

The Point-to-Point Tunnelling Protocol (PPTP) uses a TCP connection for tunnel maintenance and a modified version of Generic Routing Encapsulation (GRE) to encapsulate PPP frames for tunnelled data. The payloads of the encapsulated PPP frames can be encrypted and/or compressed. Figure 6 shows the structure of a PPTP packet containing user data.

[image: image1.png]Encrypted

1P
header

GRE
header

PPP
header

PPP payload
(IP datagram, IPX datagram, NetBEUI frame)

PPP frame





2.2.1.3.2 Layer Two Tunnelling Protocol (L2TP)

L2TP is a combination of PPTP and Layer 2 Forwarding (L2F), a technology proposed by Cisco Systems, Inc. L2TP represents the best features of PPTP and L2F. L2TP encapsulates PPP frames to be sent over IP, X.25, Frame Relay, or Asynchronous Transfer Mode (ATM) networks. When configured to use IP as its datagram transport, L2TP can be used as a tunnelling protocol over the Internet. L2TP is documented in RFC 2661.

L2TP over IP networks uses UDP and a series of L2TP messages for tunnel maintenance. L2TP also uses UDP to send L2TP-encapsulated PPP frames as the tunnelled data. The payloads of encapsulated PPP frames can be encrypted and/or compressed. 

[image: image2.png]1P ubP L2TP PPP PPP payload
header header header header (IP datagram, IPX datagram, NetBEUI frame)

- PPP frame >

- [ TP frame —————»|

- UDP message >




In Windows operating system versions, starting from Windows 2000, IPSec Encapsulating Security Payload (ESP) is used to encrypt the L2TP packet. This is known as L2TP/IPSec. The result after applying ESP is shown below

[image: image3.png]P we | e | eep PPP payload
header | header | header | header | (P datagram, IPX datagrem, NetBEU! frame)
PSec | IPS
Pl B3| we | e | eee PPP payload =
header | =S| header | header | header | (P detagram, IPX datagram, NetBEUI frame) | yanor | geor

I
Encrypted by IPSec





2.2.2 Transport and application Layer

These layers covers mainly host based solution. 

2.2.2.1 SSL and TLS
Netscape created the protocol SSL (Secure Socket Layer). In the TCP/IP layering model it is on top of the TCP layer.

Therefore, it could be used for adding security (that is strong authentication and encryption) for all TCP-based application (Telnet, FTP…).
Companies use this technology to provide secure remote access to their internal infrastructure by using SSL-VPN’s with additional strong authentication, oftentimes using either via one time password generators or X.509 certificates. The advantage is, that no VPN client software has to be installed on the client computers in advance. Instead, the VPN client software is downloaded as ActiveX controls or a Java applets just before the VPN connection is established.

Besides commercial SSL VPN solutions there are as well powerful Public Domain VPN solutions like OpenVPN (sec Fehler! Verweisquelle konnte nicht gefunden werden.) and stunnel which can be used free of charge..
Some other implementations exist for these protocols but the success story of SSL is HTTPS. HTTPS is used in e-commerce application to allow secure information exchanges between client and servers. TSL is the IETF proposed standard and the successor of SSL and an IETF standard as well.

2.2.2.2 SSH

SSH (Secure Shell) is another application layer authentication and encryption protocol. The SSH FAQ (Frequently Asked Question) give the following definition of SSH:

Secure Shell is a program to log into another computer over a network, to execute commands in a remote machine, and to move files from one machine to another. It provides strong authentication and secure communications over insecure channels. It is intended as a replacement for telnet, rlogin, rsh, and rcp. For SSH2, there is a replacement for FTP: sftp. 

Therefore, the main use of SSH is within organizations. In theory to manage security based devices (firewalls…), or to gain root access on hosts the network/system administrator should avoid connecting remotely using telnet to the box. If telnet is used, it is very easy, with a sniffer, to capture and to analyze the packets to gain administrative access on the firewall/system. With a direct access no clear user/password will be exchange on the LAN. But, network administrators are often lazy… SSH is the answer in this case!
SSH among other things includes an encrypted replacement tool for telnet.

SSH is becoming very popular for secure remote management and also for secure remote data transfer
2.2.3 So, why IPSEC?

In the “Guide on the use of TCP/IP on the GTS”, WMO presents two solutions to exchange traffic between MSS using the IP protocol. One is based on FTP and the other on sockets.

This guide does not cover the WAN infrastructure. The current GTS is a mixed of leased lines, peer-to-peer Frame Relay links, global Frame Relay services and MPLS network  (like RMDCN in RA VI). For economical reasons, and in regards the overall good quality of service of the Internet, it might be a good opportunity to study the potential use of the Internet to complement the GTS.

However, if reliable (but no real SLA –Service Level Agreement-) the Internet is by nature an insecure network. Various documents within WMO show how NMCs should connect to the Internet (firewalls…).

In order to allow a smooth introduction of the Internet to complement the GTS the following rules should apply:
· Permit the use of the current protocols (FTP and socket) on the Internet

· Avoid any impact on the MSS 

· Guarantee an acceptable level of trust for members

The two first points mean that the proposed solution should be transparent to the application and the hosts. Among the protocols describe above, IPSec is the only one completely application independent.

To offer a minimum level of trust, authentication (who wants to talk to me) and encryption (no one except me can understand the data) are both needed. IPSec offers these two services.

3 What is IPSEC ?

3.1 IPSec architecture

From RFC 2401 (now replaced by RFC 4301) :

[image: image17.wmf]HeadQuarters

Site 4

Site 1

Site 3

Site 2

Fully meshed


The IPSec specification is rather complex. The overall architecture of the specification can be seen as a suite of interacting protocols. The RFC4301 gives the organization of the specifications. It can be seen as:

[image: image4.wmf]Architecture

RFC 

4301

ESP Protocol

RFC 

4303

AH Protocol

RFC 

4302

Authentication

Algorithm

DOI

RFC 

5282

Key

Management

Policy

Encryption

Algorithm

IPSec document overview 


• Architecture covering the general concepts, security requirements, definitions and mechanisms defining IPSec technology.

· defines the capabilities hosts and routers should provide

· for example, it is required that the hosts provide confidentiality using ESP. However this document does not specify the header format.

· describes the interaction between IPSec and rest of TCP/IP

• Encapsulation security payload ESP and Authentication header (AH)

· Defines the protocol, the payload header format and the services they provide.

· define the packet processing rules

· Does not specify the cryptographic transforms that are used to provide these capabilities. This allows the transforms to be changed if they become cryptographically insecure without any change in the base protocol.

• Encryption algorithm and Authentication algorithm

· a set of documents that describe how various encryption algorithms are used in ESP or how various authentication algorithms are used in AH and authentication part of ESP.

· Defines the algorithm, the key sizes, the derivation of keys, transformation process, any algorithm-specific information.

· The definitions have to be very specific in order to obtain interoperability.

• Key management describing the key management schemes.

· keys are generated with Internet Key Exchange (IKE) in IPSec protocols

· The payload format of IKE is very generic. It can be used to negotiate keys in any protocol. IKE is also used for negotiating keys for other protocols outside IPSec.

· The genericity is achieved by separating the parameters IKE negotiates from the protocol itself.

• Domain of Interpretation (DOI) contains values needed for the other documents to relate to each other, i.e. identifiers for approved encryption and authentication algorithms, operational parameters like key lifetime.

· the parameters negotiated by IKE are defined in DOI

• Policy is an important component

· Determines if two entities will be able to communicate with each other, and if so, which transforms to use.

· Policy representation deals with definition, storage and retrieval of policy.

· Policy implementation addresses the application of policy for actual communication involving e.g. the application of negotiated keys in the communication.

All these documents are RFCs, and as often with RFC, not very readable !!

The following paragraphs introduce IPSec and some key point in it.

3.2 IPSec services and modes

The IPSec framework has been build and defined that way, to guarantee the maximum independence between the different part of the system (encryption algorithm, authentication algorithm… are not linked to ESP and AH protocols for example)

The goal of IPSec is to offer security through encryption and it has been decided to split the solution in several parts. This lead to a very powerful solution, depending on the goal one wants to reach the “good” protocols may be chosen among a large choice. However, it may lead to some incompatibility…

It must also been already noted that in the first design (the first sets of RFCs was published in 1995), the ESP protocol was not usable for authentication. In the second set (published late 1998), the ESP protocol also offers authentication solution. The third set of RFC related to IPSEC, in 2005, confirms this architecture.
When two systems want to exchange data using IPSec they must first determine the services they want to use from IPSec. The table below summarize the services offer by ESP and AH.

IPSec Services

	
	AH
	ESP (Encryption Only)
	ESP (Encryption and 

authentication)

	Access Control
	(
	(
	(

	Connectionless Integrity
	(
	
	(

	Data origin 

authentication
	(
	
	(

	Rejection of replayed packets
	(
	(
	(

	Confidentiality
	
	(
	(

	Limited traffic flow 

confidentiality
	
	(
	(


This is the first step… but not the last!

The next one is to decide if Tunnel mode or Transport mode should be used. The transport mode is mainly designed for host-to-host communication where IPSec is embedded in the host operating system. In Tunnel mode, the hosts are not in charge of IPSec, some boxes between the hosts are doing the job. 

See below the structure of the packets in the two modes.


[image: image5.wmf]Application

Transport

Network

Data

TCP

Header

Data

TCP

Header

Data

IP

Header

IP

Header

TCP

Header

Data

IP

Header

TCP

Header

Data

IP

Header

TCP

Header

Data

ESP/

AH

ESP

TRL

IP

Header

ESP/

AH

ESP

TRL

Data created from upper layer

Transport information is applied

for end-to end communication

Network information is applied

(Normal Network Layer Information)

IPSec isolates IP Header

from upper layer data

IPSec encrypts upper layer data

IPSec applies the security protocol

The packet is reassembled

IPSec operations within transport mode

Data

TCP

Header


In this mode, the real IP header of the packet is used along the way. The IP addresses of the hosts are used to route the packet. It means that if the packet is going on the Internet, the IP addresses of both hosts must be routable…


[image: image6.wmf]Application

Transport

Network

Data

TCP

Header

Data

TCP

Header

Data

IP

Header

IP

Header

TCP

Header

Data

IP

Header

IP

Header

Data created from upper layer

Transport information is applied

for end-to end communication

Network information is applied

(Normal Network Layer Information)

IPSec encrypts datagram

IPSec build new

IP header

The packet is reassembled

IPSec operations within tunnel mode

ESP/

AH

IP

Header

TCP

Header

Data

IPSec applies the security

protocol

ESP

TRL

ESP/

AH

IP

Header

TCP

Header

ESP

TRL

Data

ESP/

AH

IP

Header

TCP

Header

ESP

TRL

Data


The real IP addresses are embedded in the new IPSec packet. If the packet is routed over the Internet, the new IP addresses must be routable.

3.3  Security associations

The security associations or SA in IPSec terminology form the basis for IPSec operations. The SAs are the contracts between two entities determining the IPSec protocols used for securing the packets, the transforms, the keys and their duration plus many other things.

Before two entities are able to exchange packets using IPSec they must first create SAs. SA are always one-way (simplex). So if hosts A and B are communicating with IPSec, each host will have two SA: SAin and SAout.  SAin for host A and SAout for host B will share the same cryptographic parameters. Likewise SAs are protocol specific. There is an SA for each protocol: ESP and AH. Each IPSec host therefore has to keep a database (the SADB) to store all SA, the SPI (Security Parameter Index) is a 32-bit element used to identify the SA in the SADB. 

SAs are managed (created and deleted) either manually or with a protocol, and in this case the protocol used is IKE (Internet Key Exchange).

It is out of the scope of this paper to describe the exact and complete use of SAs but, it must be clear that SAs are the basis of the IPSec framework.

In the framework presented above, SA is linked to “Policy” and “DOI” (Domain of Interpretation). Security Association is the link between the concepts (IPSec framework) and the reality (how to really use all these things in data communication).

3.4 Authentication Header - AH

AH (Authentication Header) provides data security and authentication of IP packets:

· data integrity ensures that undetected modification to a packet content in transit is not possible

· Authentication feature enables end system or network devices to authenticate the user or the application and filter traffic accordingly.

· prevents the address spoofing attacks

· protects against the replay attack

The three first functions are guaranteed by an authentication hash function. A MAC function (Message Authentication Code) is an authentication process combined with a symmetrical key. To make it short, these functions calculate a digest of the message, afterwards the digest is encrypted with a shared secret between the two hosts.

A digest is the result of a fixed length mathematical calculation. It has been proven that is was very unlikely  to recreate the digest if the original information was altered because it would be too time-consuming.
Such techniques are used, for example, on public FTP server. When publishing a file on a server, it is useful to also put the digest of the file on the same server. The digest guarantees that the original file has not been modified by anyone.

The most well-known digest calculation method is called MD5. But since it is also an insecure method, the use of SHA-256 is now recommended.
The required protocols for AH is HMAC-SHA-1, but recommended is HMAC-SHA-256
The last function in AH (anti-replay attack) is built with a mechanism of sliding window. Each packet receives a sequence number and various techniques guarantee that packet can not be replayed without notice.

3.5 Encapsulating Security Payload – ESP
ESP offers two services:
· encryption : the data is encrypted with a predefined protocol between the two hosts

· authentication : see AH

In the first release of RFC regarding IPSEC, AH was used for authentication and ESP for encryption only. It has been shown by further study, that encryption without authentication was not secure. As the old RFCs allow using ESP without AH it was decided to add in the new RFC the capability to authenticate within ESP.

There is now some redundancy! It is possible to use AH to authenticate, and to use ESP also to authenticate and encrypt. Bruce Schneier, a well-known cryptographic specialist suggests to use ESP for both and to forget about AH…

The authentication algorithms usable in ESP are:
· 3DES in cypher Block Chaining mode (this is a mandatory protocol)
· AES in cypher Block Chaining mode
· AES in counter mode
DES (Data Encryption Standard) has been deprecated in the latest version of the protocol, and therefore must not be used.

AES is now the de-facto standard and should be used for encryption..

3.6 Internet Key Exchange – IKE

In order to talk to each other, peers must use SAs. An SA defines the security parameters and authentication keys to use. IKE is the protocol used to create and exchange SAs.

However, nothing simple in IPSec theory… In fact IKE covers three different protocols:

· ISAKMP – Internet Security Association and Key Management Protocol

· OAKLEY

· SKEME

ISAKMP defines the language for negotiation. It defines the payload format, the mechanics of implementing a key exchange and the negotiation of a security association. ISAKMP does not define the key exchange algorithm but rather the message types in order to exchange keys.

OAKLY and SKEME are two key exchange protocols usable under ISAKMP umbrella in the IKE world.

In fact, ISAKMP, OAKLEY and SKEME were already developed protocols and IKE is the binding element between all these.

3.7 Conclusion

In this brief introduction to IPSec, we have seen that:

· Two modes exist: Tunnel and Transport

· Two protocols partially redundant are defined : ESP and AH

· Security Association are created and maintained by three different protocols

· Authentication can be achieve through two possible algorithms

· Encryption relies on 3DES or AES
We have neither covered nor introduced other aspects which may also be relevant in IPSec:

· Diffie-Helmann protocol to exchange keys

· X.509v3 certificates

· PKI –Public Key Infrastructure

· …

VPN, in some aspect, is quite a vague concept, IPSec, one of the VPN solutions is not much clear!

So, despite the rather ugly face of the protocol, the various options, the risk of incompatibility, IPSec is now widely implemented and available on a lot of platforms, including, routers, dedicated boxes, firewalls, hosts…

In fact, in most cases, the solutions implemented only cover a part of the possible options. It is therefore, very important to guarantee interoperability to share the same subset among the peers.

In the following part, we suggest a possible scenario taking into account the need of security, some legal aspects and interoperability issues.

4 IPSEC and the GTS

4.1 Cryptography and the law

In VPNs and also in IPSec, cryptography is one of the key aspects to ensure privacy of the communication. The government in all the countries have and still do consider cryptography as some sort of weapon.

It has often be explained by different government in the world, that lawful activities perhaps needs privacy but, the main use of crypto tools was for unlawful activities. Therefore, every country has defined rules about the use of cryptography within the country.

The web site http://rechten.uvt.nl/koops/cryptolaw/ presents the situation regarding cryptography in a lot of countries (from the web site) :

In the last years a group of countries worked on the Wassenaar Arrangement. It controls the export of weapons and of dual-use goods, that is, goods that can be used both for a military and for a civil purpose; cryptography is such a dual-use good. 

In 1995, 28 countries decided to establish a follow-up to COCOM, the Wassenaar Arrangement on Export Controls for Conventional Arms and Dual-Use Goods and Technologies. The negotiations on the Arrangement were finished in July 1996, and the agreement was signed by 31 countries (Argentina, Australia, Austria, Belgium, Canada, the Czech Republic, Denmark, Finland, France, Germany, Greece, Hungary, Ireland, Italy, Japan, Luxembourg, the Netherlands, New Zealand, Norway, Poland, Portugal, the Republic of Korea, Romania, the Russian Federation, the Slovak Republic, Spain, Sweden, Switzerland, Turkey, the United Kingdom and the United States). Later, Bulgaria and Ukraine also became a participating state to the Arrangement. 

The initial provisions were largely the same as old COCOM regulations. The General Software Note (applicable until the December 1998 revision) excepted mass-market and public-domain crypto software from the controls. Australia, France, New Zealand, Russia, and the US deviated from the GSN and controlled the export of mass-market and public-domain crypto software. Export via the Internet did not seem to be covered by the regulations.

There is a personal-use exemption, allowing export of products "accompanying their user for the user's personal use" (e.g., on a laptop).

The Wassenaar Arrangement was revised in December 1998. Negotiations were held on 2 and 3 December 1998 in Vienna, which resulted in restrictions on the General Software Note and in some relaxations:

· free for export are: all symmetric crypto products of up to 56 bits, all asymmetric crypto products of up to 512 bits, and all subgroup-based crypto products (including elliptic curve) of up to 112 bits; 

· mass-market symmetric crypto software and hardware of up to 64 bits are free for export (the 64-bit limit was deleted on 1 December 2000, see below); 

· the export of products that use encryption to protect intellectual property (such as DVDs) is relaxed; 

· export of all other crypto still requires a license

Import or use of cryptographic solution is controlled in a large number of countries.

.

4.2 A quick view on the GTS

4.2.1 Physical layer

The current GTS is a mixed of various technical solutions:

· leased line

· Frame-Relay lines. The leased line is replaced by a connection to a Frame Relay network but only two peers share the same Frame Relay network.  

· MPLS network like the RMDCN in region VI is such a network
· Connections using the Internet.
The reproaches for the first three solutions are quite the same:
· costs : for (relatively) slow speed lines the price are high

· both the leased line and the PVCs in Frame Relay are point-to-point links and therefore work well for point-to-point communication, but neither for many-to-many nor any-to-any.
MPLS with the any-to-any feature improve the situation, but, nevertheless the cost may still be an issue in some cases.
But, the current needs on telecommunication become higher and higher in terms of:
· traffic

· global exchange

With the growth of products and data, and with the upcoming WIS network, NCs, DCPCs and GISCs will want to exchange much more information. The architecture based on the concentration through a Hub site and costly slow speed links is not suitable for those new challenges. 

4.2.2 Upper layers

The “Guide on the use of TCP/IP on the GTS” describes how to use IP as a replacement of the legacy X25 protocol. The IP protocol is now widely used on the GTS.
At the application layer, for the GTS data exchange two protocols are usable on top of the IP layer: FTP and sockets. Every new solution must be compatible to these standards. 

4.3 The Internet

Most GTS sites are now connected to the Internet. The Internet connection is in most cases:

· Usually reliable. The ISP (Internet Service Providers) delivers the connection to the Internet. They are operated just like other connecting lines and are therefore as reliable as the lines. However it must be noted that no end-to-end SLA (Service Level Agreement) could be defined on the Internet.

· Powerful. The Internet connection is often a high speed connection.

· Secured. The sites must be protected by security systems like security gateways (firewalls), malware filter, etc.
Therefore, the Internet is now a possible media to complement the current GTS private infrastructure, and it is used as the connection to the GTS in a few countries already (see also 4.2.1).

4.4 The suggested approach

As seen before, the only real application and host independent VPN solution is IPSec.

Therefore, for WMO use we recommend IPSec as the VPN solution.

But in order to guarantee interoperability between NMCs without redefining the protocols to use each time we suggest the following implementation solution:

· Tunnel mode : as IPSec will  mostly be configured on routers, firewall or dedicated boxes, and taking into account that neither encryption nor authentication are mandatory on LAN, tunnel mode is the most appropriate solution

· AH ( Authentication Header) should not be used

· ESP (Encapsulation Security Payload) is used for authentication 

· Authentication should be done with HMAC-SHA-256
· Pre-Shared secrets: using X.509v3 certificated is probably a more elegant solution, but, in practical, more difficult to implement in WMO situation.

If necessary, encryption using AES-256 should be used.
These recommendations mostly come from “A cryptographic evaluation of IPSec” form B.Schneier and N.Ferguson. (http://www.schneier.com/paper-ipsec.html).

4.5 Solution suppliers

With the subset of protocols defined above, each GTS site can choose any supplier for VPN equipment.

Besides commercial products there is also a wide range of Open Source software offering IPSec based VPN functionality.

All IPSec implementations should be compatible. But, as seen before, IPSec offers a large choice of configuration parameters. The solution proposed in 4.4 is included in the mandatory subset of protocols described in the RFCs. So any RFC compliant solution should interoperate with any other. However, in some cases different IPSec endpoints do not interoperate. So before choosing a VPN box, sites should check with their potential VPN neighbours if their respective solutions are compatible.

Over the last 7 years, ECMWF and its partners in Europe made a lot of tests on IPSEC. The latest one in 2007, showed that interoperability is still an issue. The tests involving Cisco, Nortel (with Checkpoint software), Nokia (also with Checkpoint) were successful. However, the hassle to configure multiple different devices convinced ECMWF and its partners to agree on a common platform before a large scale implementation. 
4.6 Network architecture

The placement of the VPN gateway in a security solution is of paramount importance. Improper placement of the VPN gateway will impact effectiveness of the solution. In this context, all the VPN gateways are operating in tunnel mode only. The following few scenarios of VPN gateway placement highlight this point. Emphasis is placed on the Intranet infrastructure and the demilitarization zone (DMZ). The DMZ is typically meant for limited authorized access. Servers that are put into the DMZ include web servers and external Mail servers.

Quite a lot of different solutions are available. 

Two solutions are here highlighted in the following:

· the simplest one is a firewall and a VPN gateway within the same box

· the second one is a VPN gateway adjacent to the firewall

It must be noted that this architecture must be coherent with the security policy of the site.

[image: image7.png]



In this case a VPN gateway function is implemented on a firewall and the firewall connects to the Internet. The flow of traffic is from the Internet to the firewall/VPN gateway directly. This will be a more cost effective solution since there is no need for a separate VPN gateway. Placing both the functions firewall and VPN onto the same box results in an easier management. Note that this setup is typically used for smaller VPN solutions.

The disadvantage of this solution is that the CPU  utilization of the firewall/VPN gateway will be very high and adequate measures have to be taken to address this issue. In other words, this setup introduces additional software components in the firewall and hence results in further performance degradation. Another disadvantage of this solution is that it requires more services/ports to be opened. As a result, the security implementation may suffer from additional security holes. Also, the more services are running on a single (security) system the higher is the risk of getting hacked. Finally, the firewall poses a single point of failure for the VPN solution.

[image: image8.png]



In this case the VPN gateway is connected to a (dedicated) interface of the Internet firewall.  The flow of traffic into the intranet will be through the firewall. However, the Internet traffic will arrive either
· from the Internet via the firewall or
· from the Internet directly to the external interface of the VPN gateway and then to the firewall

The advantage of this setup is that both type of clients (VPN and non-VPN) can be supported. All the clients will be subject to the firewall policy

The most advanced scenario introduces another firewall dedicated to control VPN traffic. From one box for all the dedicated boxes there are quite a lot of different options, from the cheapest to the most expensive. But as a general statement, the best solution for one site is the solution which is conform with the security policy and where the pros and the cons of the solutions are well understood.

4.7 Implementation scenario

In order for two GTS sites to establish a VPN link they must:

· Agree upon the protocols to be used (mode, authentication and/or encryption, message digest algorithm, site authentication)
· Define the pre-shared secret. This  “password” must be defined and must be the same on both sides

· Confirm the VPN platforms to be used

· Agree on IP addresses for  the exchange of data over the link

· Setup appropriate filter rules on the firewall. The following rules 

· allow UDP port 500 is used for ISAKMP

· allow UDP port 4500 (NAT traversal)
· allow IP protocol number 50 (ESP protocol)

are the basic rules for an IPSec connection.
· implement the defined configuration

· test

Once everything is running, the main risk is the potential failure of the virtual link created. 
4.8 Multicast over an IPSEC VPN

For the communication in the future WIS system, it could be an advantage to exchange certain sets of data using multicast instead of unicast.

Since the IPSEC protocol is a point-to-point protocol, and does not have point-to-multipoint capabilities by definition, sending multicast traffic over an IPSEC VPN with more than 2 sites will require the use of additional tunnels (GRE or IP-in-IP) today.

An example for such a solution is the DMVPN solution from Cisco, which is briefly described in chapter 4.9.

4.9 DMVPN solution

Dynamic Multipoint VPN (DMVPN) is a proprietary Cisco Internetworking Operating System (IOS) solution for building IPSec and GRE (Generic Routing Encapsulation) VPN in a scalable way. It relies on two Cisco technologies: Next Hop Resolution Protocol (NHRP) and Multipoint GRE Tunnel Interfaces. DMVPN does not alter the standards-based IPSec VPN tunnels, but it changes their configuration.

A “spoke” router registers its real public IP interface address when it boots and a “hub” maintains a NHRP database of all the spoke’s real public interface addresses. Subsequently, spoke routers query the NHRP database for real addresses of destination spoke routers to dynamically build direct tunnels. The Multipoint GRE Tunnel Interface allows single GRE interface to support multiple IPSec tunnels.
In a typical “hub-and-spoke” IPSec design, all VPN traffic goes via the hub site, which acts as a gateway. Therefore, the hub bandwidth and the CPU utilisation limit the VPN size. In the DMVPN “dynamic-mesh” approach, spoke-to-spoke tunnels can be dynamically built when required. This means that spoke routers only need to support tunnels currently in use and hub routers only support spoke-hub traffic and overflow from spoke-to-spoke traffic.

In the context of the GTS, a DMVPN-based infrastructure was deployed as a backup for the RMDCN in a large-scale operational pilot at the beginning of February 2010 for the following reasons:

· The RMDCN CE (Customer Edge) router is a Cisco router running IOS.

· The traffic re-routing between the RMDCN and the DMVPN infrastructures can be done in a fast, automatic and reliable manner.

The following constraints apply to the DMVPN-based infrastructure:

· The solution maintains the RMDCN any-to-any connectivity.

· The use of dedicated Cisco routers.

· The use of a subset of Cisco routers running specific IOS versions in order to limit troubleshooting and maintenance issues.

· The DMVPN equipment is locally managed. 

· The use of DMVPN is an additional option to the portfolio of the available RMDCN backup mechanisms (Mission Critical and ISDN). 

The DMVPN topology selected is “single DMVPN/dual hub” and the spoke site deployed the DMVPN router in accordance with the following technical requirements:

· The Cisco DMVPN router has to be installed on the same physical LAN/DMZ segment as the RMDCN CE router.

· The DMVPN router needs to have a public IP address reachable through the Internet. No network address translation (NAT) should be deployed. 

· The DMVPN router must be able to establish IPSec tunnels to other DMVPN RMDCN sites and sent encrypted RMDCN traffic through them: allow the IPSec protocols to and from the DMVPN router’s public IP address, i.e.: ESP, GRE, IKE (UDP 500) and NAT-T (UDP 4500) protocols.
In addition to providing more scalability and flexibility, the DMVPN solution offers fewer configurations overhead. 
4.10 OpenVPN solution

To provide a connectivity environment to allow all the 13 National Meteorological Centers in Region III be connected to each other using a Virtual Private Network (VPN) over the Internet, a VPN solution based on freely available Software (OpenVPN) and the operating system Linux was set up.

OpenVPN is a full-featured open source SSL VPN solution that accommodates a wide range of configurations, including remote access, site-to-site VPNs, Wi-Fi security, and enterprise-scale remote access solutions with load balancing, failover, and fine-grained access-controls. Starting with the fundamental premise that complexity is the enemy of security, OpenVPN offers a cost-effective, lightweight alternative to other VPN technologies.

OpenVPN's lightweight design sheds many of the complexities that characterize other VPN implementations. The OpenVPN security model is based on SSL, the industry standard for secure communications via the internet. OpenVPN implements OSI layer 2 or 3 secure network extension using the SSL/TLS protocol, supports flexible client authentication methods based on certificates, smart cards, and/or 2-factor authentication, and allows user or group-specific access control policies using firewall rules applied to the VPN virtual interface. OpenVPN is not a web application proxy and does not operate through a web browser.
It was decided to create 3 networks using the OpenVPN software to facilitate the configuration and let the operation be more flexible:
1) One core network connection the three RTH´s.

2) One network covering the north part of the Region.

3) One network covering the south part of the Region.

This setup is shown in the following picture:
[image: image9.jpg]



The RTH Buenos Aires will provide a server to collect the data from their neighboring centers. The RTH Maracay will provide the same approach do get data from the centers concerned to area of responsibility. The tree RTHs will have a special configuration in order to provide bi-directional communication between the centers involved.

The network connection the three RTHs in the Region will be called Core Network. It will be the access point for the GTS though the Message Switching System. Each RTH will host the servers for the VPN connections.

The south part of Region III, comprising the NMCs for Colombia, Ecuador, Suriname, Guyana and French Guyana will preferentially be client for the server hosted by Maracay. In case of emergency or back-up they can connect to RTH Brasilia or RTH Buenos Aires.

The north part of Region III, comprising the NMCs for Chile, Peru, Bolivia, Paraguay and Uruguay will preferentially be client for the server hosted by Buenos Aires. In case of emergency or back-up they can connect to RTH Brasilia or RTH Maracay.

In all cases, the servers will provide a fixed and private IP address for the logical tunnel between the NMC and the RTH in order to facilitate and guarantee the communication.
The following diagram illustrates the designed setup:

[image: image10.jpg]Message Switching Systern

Message Switching Systern

Brasilia

SVMR
w1702

i L Il
e
4 3 8 8 3
& o

svim
101732
2732

soca
101742
121742

P

w152
2s2

Wiz a2
12 a2

Granain17 1001
Ve 25550

Washington

REGIONAL ASSOCIATION Il

Message Switching Systern
Buenos Aires

Buenos Aires |

&

SaEm
101802
121802

Sartiago]

0

scsc
0112
Wiz

sie

winaz
222

soas

0152
wma2 w2

Gravayin 161001
Vs 26000




5 Operators solutions

In the past years the operator market solutions for managed networks consist mainly of the following technologies:

· Frame Relay 

· MPLS 
· IPSec VPN,

Frame Relay is being replaced more and more by the other 2 technologies.

The first two solutions are layer 2 based, and therefore are completely independent of layer 3 protocols. 

The main disadvantage of Frame Relay is its static nature.  The client must define exactly what peers are allowed to exchange traffic and the throughput they want. This solution is not flexible. It is costly and often creates administrative complexity to change the parameters and to allow a more dynamic approach.

Frame Relay is now deprecated and MPLS is the preferred solution from the providers.
5.1 Star, partially meshed, fully meshed networks


[image: image11.wmf]HeadQuarters

Site 4

Site 1

Site 3

Site 2

HeadQuarters

Site 4

Site 1

Site 3

Site 2

Star network

Partially meshed



In a Frame Relay environment, as establishment of PVC has its costs, only a start or a partially meshed network is available. This gives less flexibility to the network.
An MPLS network, in general, is a fully meshed network. Therefore, it easily provides any-to-any connectivity.

5.2 A brief introduction to MPLS
When data enters the carrier network, a label is attached to each packet. This label uniquely identifies the Virtual Private Network (VPN) using the so called vpnv4/v6 address in a shared infrastructure and keeps it private. While travelling through the network, another label (for MPLS switching) is added. Upon reaching its destination, the labels are removed, returning the data packet to its original state. The process is seamless and unnoticeable to end-users. [image: image12.png]MPLS Network
\





The “labels” thus replace traditional Internet packet forwarding, where complicated address matching is performed at each hop in the network. The MPLS label describes how the packet should be handled within the network and thus assigns the packet to a Class of Service (CoS). So all packets which belong to the same CoS get treated in the same way and are quickly sped along their way. 
In most cases, the providers manage the complete service (including local terminating routers).
5.3 IPSec VPN over the Internet
The Internet is a collection of interconnected Internet Service Providers (ISP) networks. Each ISP owns its infrastructure and interconnects with others to provide Internet access to its clients.

Within its network, an ISP can guarantee some level of service and therefore, with add-on can propose to build a (service provider based) VPN based on IPSec. In this case, the VPN is a value added service using a specific Internet connection. The Service Level Agreement, in such a situation, will be much limited compared to an MPLS network. 
5.4 Conclusion

For the MTN or a regional network, a managed service from a provider (either MPLS VPN ode IPSec VPN) offers a service level agreement (SLA). Depending on the technology used, this SLA will cover guaranteed bandwidth, performance, class of service, Round Trip Time, recovery time in case of problems or failures, etc.
The technical benefits of a managed network relying on MPLS or other Virtual Private Network compared to point-to-point solutions are better reliability, more flexibility in the traffic flows, guaranteed and common service level agreement.
However, whatever the technical solution is used, the issues are similar:
· the cost of the network, in particular for an MPLS solution
· the same operator must be available on each   site

· the network will need a management entity
· a common agreement must be signed between all the countries involved and with the selected provider

The underlying technology for operator’s services is important but, this is not the only issue to consider before launching such a project.
6 Appendices

6.1 Example: A Cisco based configuration

6.1.1 The test platform

This test platform aims to demonstrate the connection of two Cisco routers using IPSEC over the Internet.


[image: image13.wmf]Cisco 1720

Ethernet/IP

Cisco 2611

Internet

ATM25

Modem

ADSL

An IPSEC VPN over the Internet

VPN

Serveur

IBM Compatible

130.1.x.x

131.1.x.x

ISDN


Both sites are connected to the Internet. The left one (see above) is connected through a permanent ADSL connection. The right one connects through ISDN.

Both site musts keep normal Internet access. Internet accesses are not dedicated to IPSEC connection, and depending on destination the connection must be direct or use the ad-hoc VPN.

ADSL site must have a permanent IP address on the ADSL side of the router. ISDN site receive a dynamic IP address for every new connection. NAT (Network Address Translation) is used to masquerade internal IP addresses.

In paragraph 6.1.4 the full configuration are shown with an analysis of the main configuration topics.

For a full Cisco to-do list: 

http://cco.cisco.com/go/ipsec
6.1.2 ADSL connection with NAT

In order to understand the step-by-step connection mechanism we first indicate the ADSL connexion including NAT. In this case, there is no IPSEC.

!

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo

!

memory-size iomem 15

ip subnet-zero

no ip finger

!

!

interface Ethernet0/0

 ip address 130.1.8.1 255.255.0.0

 ip nat inside

!

interface Ethernet0/1

 no ip address

 shutdown

!

interface ATM1/0

 no ip address

 atm vc-per-vp 4096

 no atm ilmi-keepalive

 pvc 2/32 

  encapsulation aal5mux ppp Virtual-Template1

 !

interface Virtual-Template1

 ip address negotiated

 no ip redirects

 no ip proxy-arp

 ip nat outside

 pulse-time 0

 ppp chap hostname UserNetissimo

 ppp chap password 7 00574404035D1207

 no ppp chap wait

 ppp pap sent-username UserNetissimo password 7 04085C040827554F

!

ip nat pool NET 193.253.191.14 193.253.191.14 netmask 255.255.255.0

ip nat inside source list 1 pool NET overload

ip classless

ip route 0.0.0.0 0.0.0.0 193.253.191.1

ip http server

!

access-list 1 permit any

!

line con 0

 transport input none

line aux 0

line vty 0 4

 password PWD

 login

!

no scheduler allocate

end

6.1.3 VPN between the routers

ADSL Router :
!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo

!

memory-size iomem 15

ip subnet-zero

no ip finger

!

crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0        

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac 

!

crypto dynamic-map MYMAP 10

 set security-association lifetime seconds 600

 set transform-set CMT 

 match address 100

!

crypto map CMMYMAP 30 ipsec-isakmp dynamic MYMAP discover

!

interface Ethernet0/0

 ip address 130.1.8.1 255.255.0.0

!

interface Ethernet0/1

 no ip address

 shutdown

!

interface ATM1/0

 no ip address

 atm vc-per-vp 4096

 no atm ilmi-keepalive

 pvc 2/32 

  encapsulation aal5mux ppp Virtual-Template1

!

interface Virtual-Template1

 ip address negotiated

 no ip redirects

 no ip proxy-arp

 pulse-time 0

 ppp chap hostname UserNetissimo

 ppp chap password 7 00574404035D1207

 no ppp chap wait

 ppp pap sent-username UserNetissimo password 7 04085C040827554F

 crypto map CMMYMAP

!

ip classless

ip route 0.0.0.0 0.0.0.0 193.253.191.1

ip http server

!

access-list 1 permit any

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

!

line con 0

 transport input none

line aux 0

line vty 0 4

 password PWD

 login

!

end

ISDN Router :

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ISDN

!

enable secret 5 $1$n6eZ$UXI24gGl7lZu/wB3OZVnC1

enable password PWD

!

memory-size iomem 25

ip subnet-zero

no ip finger

no ip domain-lookup

! 

isdn switch-type vn3

!

crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 193.253.191.14

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac

!

crypto map CMYMAP 1 ipsec-isakmp

 set peer 193.253.191.14

 set security-association lifetime seconds 600

 set transform-set CMT

 match address 100

!

cns event-service server

!

interface BRI0

 no ip address 

 encapsulation ppp

 no ip mroute-cache

 dialer pool-member 1

 isdn switch-type vn3

 isdn send-alerting

 no cdp enable

 crypto map CMYMAP

!

interface FastEthernet0

 ip address 131.1.8.1 255.255.0.0

 no ip mroute-cache

 speed auto

 full-duplex

!

interface Dialer0

 ip address negotiated

 encapsulation ppp

 dialer remote-name LibertySurf

 dialer pool 1

 dialer string 0860155555

 dialer-group 1

 pulse-time 0

 no cdp enable  

 ppp authentication chap pap callin

 ppp chap hostname bane0000@lsurf.fr

 ppp chap password 7 1307160B04020A2F

 ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

 crypto map CMYMAP

!

ip classless

ip route 0.0.0.0 0.0.0.0 Dialer0

no ip http server

!

access-list 1 permit any

access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

dialer-list 1 protocol ip permit

!

line con 0

 transport input none

line aux 0

line vty 0 4

 password PWD

 login

!

no scheduler allocate

end
In the two configurations the red and underlined lines show the IPSEC set-up. Let’s focuses on this part of the configuration.

	ADSL
	ISDN

	crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0        

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac 

!

crypto dynamic-map MYMAP 10

 set security-association lifetime seconds 600

 set transform-set CMT 

 match address 100

!

crypto map CMMYMAP 30 ipsec-isakmp dynamic MYMAP discover

!

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

!
interface ATM1/0

 crypto map CMMYMAP
!


	crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 193.253.191.14

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac

!

crypto map CMYMAP 1 ipsec-isakmp

 set peer 193.253.191.14

 set security-association lifetime seconds 600

 set transform-set CMT

 match address 100

!

access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

!

 interface Dialer0

 crypto map CMYMAP


6.1.3.1 IKE

In the configuration, this is related to the lines:

crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0        
The protocol used for IKA is isakmp and it has been established a pre-shared secret. This secret will be used by both routers to exchanges real encryption key.

IPSECKEY is the secret shared between the two routers.

On ADSL side, the originate IP address of the ISDN router is not known (in dialup connection like ISDN this address will change for every new connection) therefore at this stage the 0.0.0.0 mean accept this IPSECKEY for every address. This may lead to potential security risk. We will see below how Cisco permits to minimize it.

6.1.3.2 AH and ESP

The Authentication header protocol and the encapsulation security payload are then selected. Depending on the hashing protocol available, the encryption solution, the choice to rely on ESP for both authentication and encryption or to AH, the protocols to be used for this particular VPN are described with the following command
crypto ipsec transform-set CMT esp-null esp-sha-hmac
In our case, ESP is the preferred method for both authentication and encryption.

ESP-NULL means use NULL for ESP (this is no encryption). ESP-SHA-HMAC means use SHA-HMAC for authentication with ESP. Of course the choice must be the same for both sides of the tunnel.

6.1.3.3  Security Association

The first two parts show the protocol part, let’s now work on Security Association. The ADSL must accept incoming IPSEC connection from unknown peer:

crypto dynamic-map MYMAP 10

 set security-association lifetime seconds 600

 set transform-set CMT 

 match address 100

!

crypto map CMMYMAP 30 ipsec-isakmp dynamic MYMAP discover

!

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255
Therefore, Cisco offers the “dynamic-map” solution. In this case, the ADSL router will accept every new connection using the right protocols (esp-null and esp-sha-hmac) from any IP address using the right password IPSECKEY. In order to restrict the use of this IPSEC tunnel Cisco use the mecanism of access-list. The access-list 100 must be the same on both side of the tunnel in order to allow the IPSEC connection. This is a Cisco dependant feature.

6.1.4 Final configuration

A mixed of above configuration NAT and IPSEC are gathered in one configuration.

Comments in red help the reader to identify the parts in the configuration, which are site dependant. This includes:

· IP addresses on both sites

· Username and password used to connect to the ISPs (Internet Service Provider)

· Access-lists

· IPSEC parameters

This configuration can be used as an example or a “to-do list” when configuring IPSEC between two sites.

ADSL Router:

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ADSL

!

enable secret 5 $1$zuPR$fsH7hiZKiW3ePfCXlBAAg.

enable password PWD

!

username UserNetissimo password 0 PwdNetissimo
username and password used to connect
!
to the internet through ADSL. This 

memory-size iomem 15
depend on agreement with local service
ip subnet-zero
provider (ISP)
no ip finger



!

crypto isakmp policy 1

 authentication pre-share

crypto isakmp key IPSECKEY address 0.0.0.0        

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac 

!

crypto dynamic-map MYMAP 10

 set security-association lifetime seconds 600

 set transform-set CMT 

 match address 100

!

crypto map CMMYMAP 30 ipsec-isakmp dynamic MYMAP discover

!

interface Ethernet0/0

 ip address 130.1.8.1 255.255.0.0
IP address of local site
 ip nat inside

!

interface Ethernet0/1

 no ip address

 shutdown

!

interface ATM1/0
ATM interface for ADSL link
 no ip address
These parameters are ISP dependant
 atm vc-per-vp 4096

 no atm ilmi-keepalive

 pvc 2/32 

  encapsulation aal5mux ppp Virtual-Template1

!

interface Virtual-Template1
Interface for virtual link on top
 ip address negotiated
of ADSL connection. All the PPP
 no ip redirects
settings are site dependant
 no ip proxy-arp

 ip nat outside

 pulse-time 0

 ppp chap hostname UserNetissimo

 ppp chap password 7 00574404035D1207

 no ppp chap wait

 ppp pap sent-username UserNetissimo password 7 04085C040827554F

 crypto map CMMYMAP

!

ip nat pool NET 193.253.191.14 193.253.191.14 netmask 255.255.255.0

ip nat inside source list 101 pool NET overload
The IP address 193.253.191.14 is
ip classless
the IP address given by the ISP
ip route 0.0.0.0 0.0.0.0 193.253.191.1
It is used to translate internal
ip http server
private addresses to IP addresses
!
in conformance to the Internet rules
access-list 1 permit any

access-list 100 permit ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

access-list 101 deny   ip 130.1.0.0 0.0.255.255 131.1.0.0 0.0.255.255

access-list 101 permit ip any any
See comments on ACL above
!

line con 0

 transport input none

line aux 0

line vty 0 4

 password PWD

 login

!

end

ISDN Router:

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

!

hostname ISDN

!

enable secret 5 $1$n6eZ$UXI24gGl7lZu/wB3OZVnC1

enable password PWD

!

memory-size iomem 25

ip subnet-zero

no ip finger

no ip domain-lookup

!                      

isdn switch-type vn3

!

crypto isakmp policy 1

 authentication pre-share

crypto isakmp key CléIPSEC address 193.253.191.14

!

crypto ipsec transform-set CMT esp-null esp-sha-hmac

!

crypto map CMYMAP 1 ipsec-isakmp

 set peer 193.253.191.14
This IP address is the “other end”


 set security-association lifetime seconds 600
of the VPN tunnel. It depends on the
 set transform-set CMT
local situation
 match address 100

!

cns event-service server

!

interface BRI0

 no ip address          

 ip nat outside

 encapsulation ppp

 no ip mroute-cache

 dialer pool-member 1

 isdn switch-type vn3

 isdn send-alerting

 no cdp enable

 crypto map CMYMAP

!

interface FastEthernet0


 ip address 131.1.8.1 255.255.0.0
IP address of local site
 ip nat inside

 no ip mroute-cache

 speed auto

 full-duplex

!

interface Dialer0
The two dialers below are ISP 

 ip address negotiated
dependant : phone number, username
 encapsulation ppp
and password
 dialer remote-name LibertySurf

 dialer pool 1

 dialer string 0860155555

 dialer-group 1 

 pulse-time 0

 no cdp enable

 ppp authentication chap pap callin

 ppp chap hostname bane0000@lsurf.fr

 ppp chap password 7 1307160B04020A2F

 ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

 crypto map CMYMAP

!

interface Dialer1

 ip address negotiated

 ip nat outside

 encapsulation ppp

 dialer pool 1

 dialer string 0860155555

 dialer-group 1

 pulse-time 0

 no cdp enable

 ppp authentication chap pap callin

 ppp chap hostname bane0000@lsurf.fr

 ppp chap password 7 1307160B04020A2F

 ppp pap sent-username bane0000@lsurf.fr password 7 14151312030A242E

!

ip nat inside source list 101 interface Dialer1 overload 

ip classless

ip route 0.0.0.0 0.0.0.0 Dialer1

ip route  130.1.0.0 255.255.0.0 Dialer0

ip route 193.253.191.14 255.255.255.255 Dialer0

no ip http server

!

access-list 1 permit any

access-list 100 permit ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

access-list 101 deny   ip 131.1.0.0 0.0.255.255 130.1.0.0 0.0.255.255

access-list 101 permit ip any any
See comments on ACL above
dialer-list 1 protocol ip permit

!

line con 0

 transport input none

line aux 0

line vty 0 4

 password PWD

 login

!

end

6.1.5 Performance

The categorization of Cisco routers is a good start for many situations. However, it can be quite tricky when it involves building a complex and/or large VPN solution. The deployment of the right router at the right place will contribute significantly to the overall VPN design in term of performance as well as cost. 

It is important to note that additional Cisco IOS processing will be required for all packets that require security services. The IPSec technology has shown to be slower when compared with Cisco Encryption Technology (CET). One of the reasons is because of the deployment of authentication algorithms which are considered to be slow. 

Nonetheless, security literature has highlighted that lack of authentication as a security vulnerability.

Without compromising security, IPSec must be the technology to deploy. In addition, IPSec introduces packet expansion, which is more likely to require fragmentation and reassembly of IPSec-protected IP datagrams. The encrypted packets will also be authenticated which means that most packets will have two cryptographic operations performed on them.

Performance figures for the different solutions can also be found on http://cco.cisco.com/go/ipsec.
Performance impact can also occur when using VPN connection via satellite Internet links. Experience from testing done by Meteo France showed the following:

In some configurations (i.e.: when crossing satellite links, MTU is significantly reduced), large IPSEC packets need to be fragmented; if the DF bit is set, such packets are dropped. The result is that small packets (i.e.: authentication sequence) can pass, whereas large packets (i.e.: data transfers) are dropped. To avoid that, a recommendation is to clear the DF bit (on a Cisco router: “crypto ipsec df-bit-clear”).

6.2 Example: A Cisco DMVPN configuration

6.2.1 The test environment
The configurations presented in this section are valid for a “Single DMVPN/Dual Hub” DMVPN topology in which “HUB1” is the primary DMVPN hub router and “HUB2” is the secondary one.
[image: image14.png]) Primary DMVPN
"~ Secondary DMVPN





Each “SPOKE” site has permanent IPSec tunnels towards both hubs in order to exchange information about the DMVPN topology. All the routers have direct Internet access and no NAT is deployed. X509 certificates are used for device authentication. The EIGRP routing protocol is used to exchange routing information within the DMVPN cloud.
6.2.2 HUB1 configuration
HUB1 Router

This configuration file was truncated to allow for an easier reading.
!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname HUB1

!

crypto pki trustpoint w3cert

 enrollment terminal

 fqdn HUB1.rmdcn.com

 revocation-check none

 rsakeypair DMVPN.test.com

!

crypto pki certificate chain w3cert

 certificate 6306820854F75290

 …

  F0363478 D576

        quit

 certificate ca 7399989A358E3CE2

  30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

 ...

  4E0D71EE 6E55FA0F 7AF0

        quit

!

crypto isakmp policy 1

 encr aes 256

 group 2

!

crypto isakmp policy 2

 encr 3des

 group 2

crypto isakmp nat keepalive 5

!

crypto ipsec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

 mode transport

crypto ipsec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

 mode transport

!

crypto ipsec profile DMVPN_PROFILE

 set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

 description HUB1 DMVPN Interface

 bandwidth 1000

 ip address 192.168.200.1 255.255.255.0

 no ip redirects

 ip mtu 1400

 no ip next-hop-self eigrp 1

 ip nhrp authentication DMVPNVPN

 ip nhrp map multicast dynamic

 ip nhrp network-id 1

 ip nhrp holdtime 360

 ip nhrp cache non-authoritative

 ip tcp adjust-mss 1360

 no ip split-horizon eigrp 1

 no ip mroute-cache

 delay 900

 cdp enable

 tunnel source FastEthernet0

 tunnel mode gre multipoint

 tunnel key 100000

 tunnel protection ipsec profile DMVPN_PROFILE

!

interface FastEthernet0

 description To the INTERNET

 ip address 172.16.1.1 255.255.255.0

!

interface FastEthernet1

 description To local LAN

 ip address 10.157.254.1 255.255.255.0

!

router eigrp 1

 redistribute static

network 192.168.200.0

distribute-list route-map ADD-TAG out Tunnel0

 no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.1.254

!

ip prefix-list EIGRP-Local-Networks-To-Cloud description Routes locally advertised to Cloud

ip prefix-list EIGRP-Local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

 description Tag any local route with local number

 match ip address prefix-list EIGRP-Local-Networks-To-Cloud

 set tag 1

!

route-map ADD-TAG permit 20

 description Allow HUB sites to advertise spoke sites networks

 match tag 1
!

route-map ADD-TAG deny 30

 description Deny anything else

!

end

6.2.3 HUB2 configuration
HUB2 Router
This configuration file was truncated to allow for an easier reading.

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname HUB2

!

crypto pki trustpoint w3cert

 enrollment terminal

 revocation-check none

 rsakeypair DMVPN.test.com

!

crypto pki certificate chain TP-self-signed-3064311813

crypto pki certificate chain w3cert

 certificate 0D7B18996CB51149

  30820303 3082026C A0030201 0202080D 7B18996C B5114930 0D06092A 864886F7

 ...

  BB12CFFA FE67A6

        quit

 certificate ca 7399989A358E3CE2

  30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

 ...

  4E0D71EE 6E55FA0F 7AF0

        quit

!

crypto isakmp policy 1

 encr aes 256

 group 2

!

crypto isakmp policy 2

 encr 3des

 group 2

crypto isakmp nat keepalive 5

!

crypto ipsec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

 mode transport

crypto ipsec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

 mode transport

!

crypto ipsec profile DMVPN_PROFILE

 set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

 description HUB2 DMVPN Interface

 bandwidth 1000

 ip address 192.168.200.8 255.255.255.0

 no ip redirects

 ip mtu 1400

 no ip next-hop-self eigrp 1

 ip nhrp authentication DMVPNVPN

 ip nhrp map multicast dynamic

 ip nhrp map 192.168.200.1 172.16.1.1

 ip nhrp map multicast 172.16.1.1

 ip nhrp network-id 1

 ip nhrp holdtime 360

 ip nhrp nhs 192.168.200.1

 ip nhrp cache non-authoritative

 ip tcp adjust-mss 1360

 no ip split-horizon eigrp 1

 no ip mroute-cache

 delay 1000

 cdp enable

 tunnel source FastEthernet0/0

 tunnel mode gre multipoint

 tunnel key 100000

 tunnel protection ipsec profile DMVPN_PROFILE

!

interface FastEthernet0/0

 description To the INTERNET

 ip address 172.16.8.1 255.255.255.0

!

interface FastEthernet0/1

 description To local LAN

 ip address 10.157.254.8 255.255.255.0

!

router eigrp 1

 network 192.168.200.0

 distribute-list route-map ADD-TAG out Tunnel0

 no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.8.254

!

ip prefix-list EIGRP-Local-Networks-To-Cloud description Routes locally advertised to Cloud

ip prefix-list EIGRP-Local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

 description Tag any local route with local number

 match ip address prefix-list EIGRP-Local-Networks-To-Cloud

 set tag 1

!

route-map ADD-TAG permit 20

 description Allow HUB sites to distribute spoke sites networks

 match tag 1

!

route-map ADD-TAG deny 30

 description Deny anything else

!

end

6.2.4 SPOKE configuration
SPOKE Router
This configuration file was truncated to allow for an easier reading.
!

version 12.4

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname SPOKE

!

crypto pki trustpoint w3cert

 enrollment terminal

 revocation-check none

 rsakeypair DMVPN.rmdcn.com

!

crypto pki certificate chain TP-self-signed-266594605

crypto pki certificate chain w3cert

 certificate 00D481DE90432A3E

  30820304 3082026D A0030201 02020800 D481DE90 432A3E30 0D06092A 864886F7

 ...

  7ED1CD42 68E8FDD1

  quit

 certificate ca 7399989A358E3CE2

  30820286 308201EF A0030201 02020873 99989A35 8E3CE230 0D06092A 864886F7

 ...

  4E0D71EE 6E55FA0F 7AF0

  quit

!

crypto isakmp policy 1

 encr aes 256

 group 2

!

crypto isakmp policy 2

 encr 3des

 group 2

crypto isakmp nat keepalive 5

!

crypto ipsec transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

 mode transport

crypto ipsec transform-set ESP-AES-SHA esp-aes esp-sha-hmac

 mode transport

!

crypto ipsec profile DMVPN_PROFILE

 set transform-set ESP-AES-SHA ESP-3DES-SHA

!

interface Tunnel0

 description SPOKE DMVPN Interface

 bandwidth 1000

 ip address 192.168.200.4 255.255.255.0

 no ip redirects

 ip mtu 1400

 ip nhrp authentication DMVPNVPN

 ip nhrp map 192.168.200.1 172.16.1.1

 ip nhrp map multicast 172.16.1.1

 ip nhrp map multicast 172.16.8.1

 ip nhrp map 192.168.200.8 172.16.8.1

 ip nhrp network-id 1

 ip nhrp holdtime 360

 ip nhrp nhs 192.168.200.1

 ip nhrp nhs 192.168.200.8

 ip nhrp cache non-authoritative

 ip tcp adjust-mss 1360

 no ip mroute-cache

 delay 1000

 cdp enable

 tunnel source FastEthernet4

 tunnel mode gre multipoint

 tunnel key 100000

 tunnel protection ipsec profile DMVPN_PROFILE

!

interface FastEthernet4

 description To the INTERNET

 ip address 172.16.4.1 255.255.255.0

!

interface Vlan1

 description To local LAN

 ip address 10.157.254.4 255.255.255.0

!

router eigrp 1

network 192.168.200.0

 distribute-list route-map ADD-TAG out Tunnel0

 no auto-summary

!

ip route 0.0.0.0 0.0.0.0 172.16.4.254

!

ip prefix-list EIGRP-local-Networks-To-Cloud description locally advertised to Cloud

ip prefix-list EIGRP-local-Networks-To-Cloud seq 5 permit <LOCAL LAN IP SUBNETS>/24

!

route-map ADD-TAG permit 10

 description Tag any local route with local number

 match ip address prefix-list EIGRP-local-Networks-To-Cloud

 set tag 4

!

route-map ADD-TAG deny 30

 description Deny anything else

!

end
6.3 Configuration Example for an OpenVPN-based solution

6.3.1 Server Configuration

By starting up with the Server configuration first thing we have to do is to create a series of certificates and keys. For all these we take advantage of the OpenVpn features by having a number of pre-set scripts to carry out the job, so that to follow an order, first thing to recommend is to copy the directory where those scripts are stored into the "/etc/openvpn" files configuration directory.

-Copy the "easy-rsa" directory to /etc/openvpn, this one can usually be found on
/usr/share/openvpn/2.0/easy-rsa or /usr/share/doc/openvpn-2*/easy-rsa

#cp -a /usr/share/doc/openvpn-2*/easy-rsa /etc/openvpn

#cd /etc/openvpn/easy-rsa

Note: former configuration files that remain into the /etc/openvpn directory should have to be removed in order to avoid making futures mistakes.

6.3.2 Generate the Certificate Authority (CA).

This Certificate has to be generated just once in the Server and then will be one of the files to be copy and submitted to the clients (Met Centers).

#source vars

By executing vars we are initializing the environment variables and something like this is displayed:  

NOTE: when you run  ./clean-all,  we are doing a  “rm –rf” on /etc/openvpn/easy-rsa/keys

Later on let's initialize the "keys" directory( /etc/openvpn/easy-rsa/keys): just in case that old files remain the script below is going to cleaning them up:

#sh clean-all   or   #./clean-all

And then on we execute the following script

#sh build-ca

CA is generated and  some information is asked which it is recommendable we shall provide the more precisely as possible since in coming configurations some data will be required, as an example:

Generating a 1024 bit RSA private key

...............++++++

...............................++++++

writing a new private key to 'ca.key'

-----

You are about to be asked to enter information that will be incorporated

in your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter '.', the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR
State or Province Name (full name) [NA]:Buenos Aires
Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires
Organization Name (eg, company) [OpenVPN-TEST]: press “enter” (default)

Organizational Unit Name (eg, section) []:CRTBA
Common Name (eg, your name or your server's hostname) []:SMN
Email Address [me@myhost.mydomain]:xxx1@yyy.ar (changed to avoid use of real e-mail address)
Note: don't let the Common Name blank!

6.3.3 Generate certificate and key for the Server

#sh build-key-server servidor

Again some information are queried which Country name, State or Province Name, Locality Name and Organization name have to be the same entries as in CA 

certificate; the rest could be different specially for Common Name, at this example "servidor":

# sh build-key-server servidor
Generating a 1024 bit RSA private key

.........++++++

.........++++++

Writing a new private key to 'servidor.key'

-----

You are about to be asked to enter information that will be incorporated

into your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter '.', the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR
State or Province Name (full name) [NA]:Buenos Aires
Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires
Organization Name (eg, company) [OpenVPN-TEST]: press  “enter” (default)

Organizational Unit Name (eg, section) []:CRTBA
Common Name (eg, your name or your server's hostname) []:servidor
Email Address [me@myhost.mydomain]:xxx2@yyy.ar (changed to avoid use of real e-mail address)
Please enter the following 'extra' attributes

to be sent with your certificate request

A challenge password []:press “enter” (default)

An optional company name []:press “enter” (default)

Using configuration from /etc/openvpn/easy-rsa/openssl.cnf

Check that the request matches the signature

Signature ok

The Subject's Distinguished Name is as follows

country Name           :PRINTABLE:'AR'

stateOrProvince Name   :PRINTABLE:'Buenos Aires'

localityName          :PRINTABLE:'Ciudad de Buenos Aires'

organizationName      :PRINTABLE:'OpenVPN-TEST'

organizationalUnitName:PRINTABLE:'CRTBA'

commonName            :PRINTABLE:'servidor'

emailAddress          :IA5STRING:' xxx2@yyy.ar' (changed to avoid use of real e-mail address)
Certificate is to be certified until Nov 21 18:52:04 2018 GMT (3650 days)

Sign the certificate? [y/n]:y

1 out of 1 certificate requests certified, commit? [y/n]y

Write out database with 1 new entries

Data Base Updated

Two files have been created inside /etc/openvpn/easy-rsa/keys and have to be copied to /etc/openvpn/ 

* servidor.crt

* servidor.key

6.3.4 Generate certificates and keys for the clients (Met Centers).

Likewise as we did before for the server we shall do for each of the clients, i.e. certificates and encrypt keys are created and unique for any of them.

# sh build-key clienteBR
Similarly to the Server, queried information such as Country name, State or Province Name, Locality Name and Organization name have to be the same as input in CA certificate; the rest could be different especially for Common Name, at this example "clienteBR":

Generating a 1024 bit RSA private key

............++++++

..............................++++++

writing a new private key to 'clienteBR.key'

-----

You are about to be asked to enter information that will be incorporated

in your certificate request.

What you are about to enter is what is called a Distinguished Name or a DN.

There are quite a few fields but you can leave some blank

For some fields there will be a default value,

If you enter '.', the field will be left blank.

-----

Country Name (2 letter code) [KG]:AR
State or Province Name (full name) [NA]:Buenos Aires
Locality Name (eg, city) [BISHKEK]:Ciudad de Buenos Aires
Organization Name (eg, company) [OpenVPN-TEST]:

Organizational Unit Name (eg, section) []:CRTBA
Common Name (eg, your name or your server's hostname) []:clienteBR
Email Address [me@myhost.mydomain]: xxx3@yyy.br (changed to avoid use of real e-mail address)
Please enter the following 'extra' attributes

to be sent with your certificate request

A challenge password []:

An optional company name []:

Using configuration from /etc/openvpn/easy-rsa/openssl.cnf

Check that the request matches the signature

Signature ok

The Subject's Distinguished Name is as follows

countryName           :PRINTABLE:'AR'

stateOrProvinceName   :PRINTABLE:'Buenos Aires'

localityName          :PRINTABLE:'Ciudad de Buenos Aires'

organizationName      :PRINTABLE:'OpenVPN-TEST'

organizationalUnitName:PRINTABLE:'CRTBA'

commonName            :PRINTABLE:'clienteBR'

emailAddress          :IA5STRING:' xxx3@yyy.br' (changed to avoid use of real e-mail address)
Certificate is to be certified until Nov 21 19:00:49 2018 GMT (3650 days)

Sign the certificate? [y/n]:y

1 out of 1 certificate requests certified, commit? [y/n]y

Write out database with 1 new entries

Data Base Updated

Whether more certificates or keys have to be generated to other clients last step should be repeated just changing the passed parameter, for example: 

#sh build-key clienteVE
#sh build-key clienteUY
#sh build-key clientePY
In this way we are generating certificates and encrypted keys with the names clienteVE, clienteUY,etc.

6.3.5 Generate Diffie-Hellman parameter
Diffie-Hellman parameter is generated executing this command which will provide us of a safety key exchange.

#sh build-dh

Generating DH parameters, 1024 bit long safe prime, generator 2

This is going to take a long time

................................................+......................................+.....+.........................................................+.....

.+...........+.............................................................+...........................................................+.....................

....................................................+...............................+............+..........+..........................................+.....

............................................+................................+...............................+.....................+.........................

.+.................+..............................................+.........................................................+...................++*++*++*

The above command shall create the file dh1024.pem inside the directory /etc/openvpn/easy-rsa/ which should be copied at /etc/openvpn directory.

6.3.6 Coping files here and there

Once carried out previously steps we've got several files stored in /etc/openvpn/easy-rsa/keys; any of them have to be copied into the client and other ones into the Server:

Server: the following files have to be copied into /etc/openvpn

ca.crt

ca.key

servidor.key 

servidor.crt

dh1024.pem

Client: the following files have to be copied into /etc/openvpn/ directory

 ca.crt

 clienteBR.crt

 clienteBR.key

Server configuration

We need to count on a configuration file either in the client as in the Server.

This will contain all configuration parameters and should be have the .conf extension

and of course shall be stored in the /etc/openvpn directory   

servidor.conf

port 1194

proto udp

dev tun

ca /etc/openvpn/ca.crt

cert /etc/openvpn/servidor.crt

key /etc/openvpn/servidor.key

dh /etc/openvpn/dh1024.pem

server 172.16.0.0 255.255.255.0

ifconfig-pool-persist ipp.txt

ifconfig-pool-linear

push "route 10.1.0.0 255.255.0.0"

keepalive 10 120

#comp-lz0

user nobody

group nobody

persist-key

persist-tun

status openvpn-status.log

verb 4

Some parameters are:

ca: name of the certificate authority

cert: name of the server certificate

key: name of the encrypt key

dh: name of the Diffie-Hellman parameter

Server:Ip range assigned to the clients "Tun" interface. Server

will have the .1 ie for our example 172.16.0.1 

push: indicate the Server which Ip subnet could be reached by the connected clients

6.3.7 Client configuration
client

dev tun

proto udp

remote 200.16.116.114

resolv-retry infinite

nobind

user nobody

group nobody

persist-key

persist-tun

ca /etc/openvpn/ca.crt

cert /etc/openvpn/clienteBr.crt

key /etc/openvpn/clienteBr.key

#comp-lzo

verb 4

Client: act as a client and some configurations will be taken from 

the server

nobind: don't act as a server just a client

ca: name of the certificate authority

cert: name of the client certificate

key: name of the encrypt key

Note: besides the present configuration it is possible with a little bit of configuration expanding the scope of the VPN to include additional machines on either the client or Server subnet.

More information can be found under:

http://en.wikipedia.org/wiki/Virtual_private_network
http://openvpn.net/.

6.4 Glossary
The following glossary is based on http://www.freeswan.org/freeswan_trees/freeswan-1.95/doc/glossary.html
3DES (Triple DES) 

Using three DES encryptions on a single data block, with at least two different keys, to get higher security than is available from a single DES pass. 

IPSEC always does 3DES with three different keys, as required by RFC 2451. For an explanation of the two-key variant, see two key triple DES. Both use an EDE encrypt-decrypt-encrpyt sequence of operations.

Single DES is insecure.

Double DES is ineffective. Using two 56-bit keys, one might expect an attacker to have to do 2112 work to break it. In fact, only 257 work are required with a meet-in-the-middle attack, though a large amount of memory is also required. Triple DES is vulnerable to a similar attack, but that just reduces the work factor from the 2168 one might expect to 2 112. That provides adequate protection against brute force attacks, and no better attack is known.

3DES can be somewhat slow compared to other ciphers. It requires three DES encryptions per block. DES was designed for hardware implementation and includes some operations which are difficult in software. However, the speed we get is quite acceptable for many uses. See our performance document for details.

AES 

The Advanced Encryption Standard, a new block cipher standard to replace DES being developed by NIST, the US National Institute of Standards and Technology. DES used 64-bit blocks and a 56-bit key. AES ciphers use a 128-bit block and are required to support 128, 192 and 256-bit keys. Some of them support other sizes as well. The larger block size helps resist birthday attacks while the large key size prevents brute force attacks. 

Fifteen proposals meeting NIST's basic criteria were submitted in 1998 and subjected to intense discussion and analysis, "round one" evaluation. In August 1999, NIST narrowed the field to five "round two" candidates:

·
Mars from IBM 

·
RC6 from RSA 

·
Rijndael from two Belgian researchers 

·
Serpent, a British-Norwegian-Israeli research collaboration 

·
Twofish from the consulting firm Counterpane 

Three of the five finalists -- Rijndael, Serpent and Twofish -- have completely open licenses. 

In October 2000, NIST announced the winner -- Rijndael.

For more information, see: 

·
NIST's AES home page 

·
The Block Cipher Lounge AES page 

·
Brian Gladman's code and benchmarks 

·
Helger Lipmaa's survey of implementations 

Adding one or more AES ciphers to Linux FreeS/WAN would be a useful undertaking. Likely one would add all three of the Round Two candidates with good licenses. A complication is that our code is built for a 64-bit block cipher and AES uses a 128-bit block. Volunteers via the mailing lists would be welcome.

AH 

The IPsec Authentication Header, added after the IP header. For details, see IPsec document and/or RFC 2402. 

Asymmetric cryptography 

See public key cryptography. 

Authentication 

Ensures that a message originated from the expected sender and has not been altered on the route. IPsec uses authentication in two places: 

·
peer authentication, authenticating the players in IKE's Diffie-Hellman key exchanges to prevent man-in-the-middle attacks. This can be done in a number of ways. The methods supported by FreeS/WAN are discussed in our configuration document. 

·
Packet authentication, authenticating packets on an established SA, either with a separate authentication header or with the optional authentication in the ESP protocol. In either case, packet authentication uses a hashed message authentication code technique. 

Outside IPSEC, passwords are perhaps the most common authentication mechanism. Their function is essentially to authenticate the person's identity to the system. Passwords are generally only as secure as the network they travel over. If you send a cleartext password over a tapped phone line or over a network with a packet sniffer on it, the security provided by that password becomes zero. Sending an encrypted password is no better; the attacker merely records it and reuses it at his convenience. This is called a replay attack.

A common solution to this problem is a challenge-response system. This defeats simple eavesdropping and replay attacks. Of course an attacker might still try to break the cryptographic algorithm used, or the random number generator.

Birthday attack 

A cryptographic attack based on the mathematics exemplified by the birthday paradox. This math turns up whenever the question of two cryptographic operations producing the same result becomes an issue: 

·
Collisions in message digest functions. 

·
Identical output blocks from a block cipher 

·
Repetition of a challenge in a challenge-response system 

Resisting such attacks is part of the motivation for:

·
Hash algorithms such as SHA and RIPEMD-160 giving a 160-bit result rather than the 128 bits of MD4, MD5 and RIPEMD-128. 

·
AES block ciphers using a 128-bit block instead of the 64-bit block of most current ciphers 

·
IPSEC using a 32-bit counter for packets sent on an automatically keyed SA and requiring that the connection always be rekeyed before the counter overflows. 

Birthday paradox 

Not really a paradox, just a rather counter-intuitive mathematical fact. In a group of 23 people, the chance of a least one pair having the same birthday is over 50%. 

The second person has 1 chance in 365 (ignoring leap years) of matching the first. If they don't match, the third person's chances of matching one of them are 2/365, the 4th, 3/365, and so on. The total of these chances grows more quickly than one might guess.

Block cipher 

It is a symmetric cipher which operates on fixed-sized blocks of plaintext giving a block of ciphertext for each. Contrast with stream cipher. Block ciphers can be used in various modes when multiple blocks are to be encrypted. 

DES is among the best known and widely used block ciphers, but is now obsolete. Its 56-bit key size makes it highly insecure today. 

The current generation of block ciphers -- such as Blowfish, CAST-128 and IDEA -- all use 64-bit blocks and 128-bit keys. The next generation, AES, uses 128-bit blocks and supports key sizes up to 256 bits.

The Block Cipher Lounge web site has more information.

Blowfish 

It is a block cipher using 64-bit blocks and keys of up to 448 bits. Designed by Bruce Schneier, and used in several products. 

This is not required by the IPSEC RFCs 

Brute force attack (exhaustive search) 

This means breaking a cipher by trying all possible keys. This is always possible in theory (except against a one-time pad), but it becomes practical only if the key size is inadequate. For an important example, see our document on the insecurities of DES with its 56-bit key. For an analysis of key sizes required to resist plausible brute force attacks, see this paper. 

Longer keys protect against brute force attacks. Each extra bit in the key doubles the number of possible keys and therefore doubles the work a brute force attack must do. A large enough key defeats any brute force attack.

For example, the EFF's DES Cracker searches a 56-bit key space in an average of a few days. Let us assume an attacker that can find a 64-bit key (256 times harder) by brute force search in a second (a few hundred thousand times faster). For a 96-bit key, that attacker needs 232 seconds, about 135 years. Against a 128-bit key, he needs 232 times that, over 500,000,000,000 years. Your data is then obviously secure against brute force attacks. Even if our estimate of the attacker's speed is off by a factor of a million, it still takes him over 500,000 years to crack a message.

This is why

·
Single DES is now considered dangerously insecure 

·
All of the current generation of block ciphers use a 128-bit or longer key 

·
AES ciphers support keysizes 128, 192 and 256 bits 

·
Any cipher we add to Linux FreeS/WAN will have at least a 128-bit key 

Cautions: 

Inadequate keylength always indicates a weak cipher but it is important to note that adequate keylength does not necessarily indicate a strong cipher. There are many attacks other than brute force, and adequate keylength only guarantees resistance to brute force. Any cipher, whatever their key sizes, will be weak if design or implementation flaws allow other attacks.

Also, once you have an adequate key length (somewhere around 90 or 100 bits), adding more key bits make no practical difference, even against brute force. Consider our 128-bit example above that takes 500,000,000,000 years to break by brute force. We really don't care how many zeroes there are on the end of that, as long as the number remains ridiculously large. That is, we don't care exactly how large the key is as long as it is large enough.

There may be reasons of convenience in the design of the cipher to support larger keys. For example Blowfish allows up to 448 bits and RC4 up to 2048, but beyond 100-odd bits it makes no difference to practical security.

CBC mode 

Cipher Block Chaining mode, a method of using a block cipher in which for each block except the first, the result of the previous encryption is XORed into the new block before it is encrypted. CBC is the mode used in IPSEC. 

An initialisation vector (IV) must be provided. It is XORed into the first block before encryption. The IV need not be secret but should be different for each message and unpredictable. 

Cipher Modes 

Different ways of using a block cipher when encrypting multiple blocks. 

Four standard modes were defined for DES in FIPS 81. They can actually be applied with any block cipher.


ECB
Electronic Code Book
encrypt each block independently


CBC
Cipher Block Chaining 
XOR previous block cipher text into new block plaintext before encrypting new block


CFB
Cipher Feed Back



OFB
Output Feed Back


IPSEC uses CBC mode since this is only marginally slower than ECB and is more secure. In ECB mode the same plaintext always encrypts to the same ciphertext, unless the key is changed. In CBC mode, this does not occur.

Various other modes are also possible, but none of them are used in IPsec.

Ciphertext 

The encrypted output of a cipher, as opposed to the unencrypted plaintext input.  

Client 

This term has at least two distinct uses in discussing IPsec: 

·
The clients of an IPSEC gateway are the machines it protects, typically on one or more subnets behind the gateway. In this usage, all the machines on an office network are clients of that office's IPSEC gateway. Laptop or home machines connecting to the office, however, are not clients of that gateway. They are remote gateways, running the other end of an IPSEC connection. Each of them is also its own client. 

·
IPSEC client software is used to describe software which runs on various standalone machines to let them connect to IPSEC networks. In this usage, a laptop or home machine connecting to the office is a client machine. 

We generally use the term in the first sense. Vendors of Windows IPSEC solutions often use it in the second.

Denial of service (DoS) attack 

This is an attack that aims at denying some service to legitimate users of a system, rather than providing a service to the attacker. 

·
One variant is a flooding attack, overwhelming the system with too many packets, to much email, or whatever. 

·
A closely related variant is a resource exhaustion attack. For example, consider a "TCP SYN flood" attack. Setting up a TCP connection involves a three-packet exchange: 

· 
Initiator: Connection please (SYN) 

· 
Responder: OK (ACK) 

· 
Initiator: OK here too 

If the attacker puts bogus source information in the first packet, such that the second is never delivered, the responder may wait a long time for the third to come back. If responder has already allocated memory for the connection data structures, and if many of these bogus packets arrive, the responder may run out of memory.

·
Another variant is to feed the system undigestible data, hoping to make it sick. For example, IP packets are limited in size to 64K bytes and a fragment carries information on where it starts within that 64K and how long it is. The "ping of death" delivers fragments that say, for example, that they start at 60K and are 20K long. Attempting to re-assemble these without checking for overflow can be fatal. 

The two example attacks discussed were both quite effective when first discovered, capable of crashing or disabling many operating systems. They were also well-publicised, and today far fewer systems are vulnerable to them.

DES 

The Data Encryption Standard, a block cipher with 64-bit blocks and a 56-bit key. Probably the most widely used symmetric cipher ever devised. DES has been a US government standard for their own use (only for unclassified data), and for some regulated industries such as banking, since the late 70's. 

DES is seriously insecure against current attacks.

DH 

See Diffie-Hellman 

Diffie-Hellman (DH) key exchange protocol 

This is a protocol that allows two parties without any initial shared secret to create one in a manner immune to eavesdropping. Once they have done this, they can communicate privately by using that shared secret as a key for a block cipher or as the basis for key exchange. 

The protocol is secure against all passive attacks, but it is not at all resistant to active man-in-the-middle attacks. If a third party can impersonate Bob to Alice and vice versa, then no useful secret can be created. Authentication of the participants is a prerequisite for safe Diffie-Hellman key exchange. IPSEC can use any of several authentication mechanisms. Those supported by FreeS/WAN are discussed in our configuration section.

The Diffie-Hellman key exchange is based on the discrete logarithm problem and is secure unless someone finds an efficient solution to that problem.

Given a prime p and generator g (explained under discrete log below), Alice:

·
generates a random number a 

·
calculates A = g^a modulo p 

·
sends A to Bob 

Meanwhile Bob:

·
generates a random number b 

·
calculates B = g^b modulo p 

·
sends B to Alice 

Now Alice and Bob can both calculate the shared secret s = g^(ab). Alice knows a and B, so she calculates s = B^a. Bob knows A and b so he calculates s = A^b.

An eavesdropper will know p and g since these are made public, and can intercept A and B but, short of solving the discrete log problem, these do not let him or her discover the secret s.

Digital signature 

Sender: 

·
calculates a message digest of a document 

·
encrypts the digest with his or her private key, using some public key cryptosystem. 

·
attaches the encrypted digest to the document as a signature 

Receiver:

·
calculates a digest of the document (not including the signature) 

·
decrypts the signature with the signer's public key 

·
verifies that the two results are identical 

If the public-key system is secure and the verification succeeds, then the receiver knows

·
That the document was not altered between signing and verification 

·
That the signer had access to the private key 

Such an encrypted message digest can be treated as a signature since it cannot be created without both the document and the private key which only the sender should possess. The legal issues are complex, but several countries are moving in the direction of legal recognition for digital signatures. 

DMVPN

Dynamic Multipoint VPN; a proprietary solution which allows for an easy configuration and setup of meshed IPSEC VPNs

EAR 

The US government's Export Administration Regulations, administered by the Bureau of Export Administration. These have replaced the earlier ITAR regulations as the controls on export of cryptography. 

EDE 

The sequence of operations normally used in either the three-key variant of triple DES used in IPSEC or the two-key variant used in some other systems. 

The sequence is:

·
Encrypt with key1 

·
Decrypt with key2 

·
Encrypt with key3 

For the two-key version, key1=key3.

The "advantage" of this EDE order of operations is that it makes it simple to interoperate with older devices offering only single DES. Set key1=key2=key3 and you have the worst of both worlds, the overhead of triple DES with the security of single DES. Since single DES is insecure, this is an extremely dubious "advantage".

The EDE two-key variant can also interoperate with the EDE three-key variant used in IPSEC; just set k1=k3.

Encryption 

Techniques for converting a readable message ( plaintext) into apparently random material (ciphertext) which cannot be read if intercepted. A key is required to read the message. 

Major variants include symmetric encryption in which sender and receiver use the same secret key and public key methods in which the sender uses one of a matched pair of keys and the receiver uses the other. Many current systems, including IPSEC, are hybrids combining the two techniques.

ESP 

ESP means Encapsulated Security Payload, the IPSEC protocol which provides encryption. It can also provide authentication service and may be used with null encryption (which we do not recommend). For details see our IPSEC document and/or RFC 2406.

FIPS 

Federal Information Processing Standard, the US government's standards for products it buys. These are issued by NIST. Among other things, DES and SHA are defined in FIPS documents. NIST have a FIPS home page. 

Hash 

See message digest 

Hashed Message Authentication Code (HMAC) 

Using keyed message digest functions to authenticate a message. This differs from other uses of these functions: 

·
In normal usage, the hash function's internal variables are initialised in some standard way. Anyone can reproduce the hash to check that the message has not been altered. 

·
For HMAC usage, you initialise the internal variables from the key. Only someone with the key can reproduce the hash. A successful check of the hash indicates not only that the message is unchanged but also that the creator knew the key. 

The exact techniques used in IPSEC are defined in RFC 2104. They are referred to as HMAC-MD5-96 and HMAC-SHA-96 because they output only 96 bits of the hash. This makes some attacks on the hash functions harder.

HMAC 

See Hashed Message Authentication Code 

HMAC-MD5-96 

See Hashed Message Authentication Code 

HMAC-SHA-256 

See Hashed Message Authentication Code 

ICMP 

This means Internet Control Message Protocol. This is used for various IP-connected devices to manage the network. 

IDEA 

The International Data Encryption Algorithm, developed in Europe as an alternative to exportable American ciphers such as DES which were too weak for serious use. IDEA is a block cipher using 64-bit blocks and 128-bit keys, and is used in products such as PGP. 

IDEA is not required by the IPSEC RFCs and not currently used in Linux FreeS/WAN.

IDEA is patented and, with strictly limited exceptions for personal use, using it requires a license from Ascom.

IKE 

Internet Key Exchange, based on the Diffie-Hellman key exchange protocol. 

Initialisation Vector (IV) 

Some cipher modes, including the CBC mode which IPsec uses, require some extra data at the beginning. This data is called the initialisation vector. It need not be secret, but should be different for each message. Its function is to prevent messages which begin with the same text from encrypting to the same ciphertext. That might give an analyst an opening, so it is best prevented. 

IP 

The Internet Protocol 

IPSEC 

Internet Protocol Security, security functions (authentication and encryption) implemented at the IP level of the protocol stack. It is optional for IPv4 and mandatory for IPv6. 

ISAKMP 

Internet Security Association and Key Management Protocol, defined in RFC 2408. 

Man-in-the-middle attack 

This is an active attack in which the attacker impersonates each of the legitimate players in a protocol to the other. 

For example, if Alice and Bob are negotiating a key via the Diffie-Hellman key agreement, and are not using authentication to be certain they are talking to each other, then an attacker able to insert himself in the communication path can deceive both players.

Call the attacker Mallory. For Bob, he pretends to be Alice. For Alice, he pretends to be Bob. Two keys are then negotiated, Alice-to-Mallory and Bob-to-Mallory. Alice and Bob each think the key they have is Alice-to-Bob.

A message from Alice to Bob then goes to Mallory who decrypts it, reads it and/or saves a copy, re-encrypts using the Bob-to-Mallory key and sends it along to Bob. Bob decrypts successfully and sends a reply which Mallory decrypts, reads, re-encrypts and forwards to Alice.

To make this attack effective, Mallory must

·
subvert some part of the network in some way that lets him carry out the deception of possible targets: DNS, router, Alice or Bob's machine, mail server, ... 

·
beat any authentication mechanism Alice and Bob use 

Strong authentication defeats the attack entirely; this is why IKE requires authentication 

·
Work in real time, delivering messages without introducing a delay large enough to alert the victims 

It’s not hard if Alice and Bob are using email; quite difficult in some situations. 

If he manages it, however, it is devastating. He not only gets to read all the messages; he can alter messages, inject his own, forge anything he likes, etc. In fact, he controls the communication completely.

Manual keying 

Is an IPSEC mode in which the keys is provided by the administrator.  The alternative, automatic keying, is preferred in most cases. 

MD4 

Message Digest Algorithm Four from Ron Rivest of RSA. MD4 was widely used a few years ago, but is now considered obsolete. It has been replaced by its descendants MD5 and SHA. 

MD5 

Message Digest Algorithm Five from Ron Rivest of RSA, an improved variant of his MD4. Like MD4, it produces a 128-bit hash. For details see RFC 1321. 

MD5 is one of two message digest algorithms available in IPSEC. The other is SHA. SHA produces a longer hash and is therefore more resistant to birthday attacks, but this is not a concern for IPSEC. The HMAC method used in IPSEC is secure even if the underlying hash is not particularly strong against this attack.

Meet-in-the-middle attack 

A divide-and-conquer attack which breaks a cipher into two parts works against each separately, and compares results. Probably the best known example is an attack on double DES. This applies in principle to any pair of block ciphers, e.g. to an encryption system using, say, CAST-128 and Blowfish, but we will describe it for double DES. 

Double DES encryption and decryption can be written:

        C = E(k2,E(k1,P))

        P = D(k1,D(k2,C))

Where C is ciphertext, P is plaintext, E is encryption, D is decryption, k1 is one key, and k2 is the other key. If we know a P, C pair, we can try and find the keys with a brute force attack, trying all possible k1, k2 pairs. Since each key is 56 bits, there are 2 112 such pairs and this attack is painfully inefficient.

The meet-in-the middle attack re-writes the equations to calculate a middle value M:

        M = E(k1,P)

        M = D(k2,C)

Now we can try some large number of D(k2,C) decryptions with various values of k2 and store the results in a table. Then start doing E(k1,P) encryptions, checking each result to see if it is in the table.

With enough table space this breaks double DES with 256 + 256 = 257 work. Against triple DES, you need 256 + 2112 ~= 2112. 

The memory requirements for such attacks can be prohibitive, but there is a whole body of research literature on methods of reducing them.

Message Digest Algorithm 

An algorithm which takes a message as input and produces a hash or digest of it, a fixed-length set of bits which depend on the message contents in some highly complex manner. Design criteria include making it extremely difficult for anyone to counterfeit a digest or to change a message without altering its digest. One essential property is collision resistance. The main applications are in message authentication and digital signature schemes. Widely used algorithms include MD5 and SHA. In IPSEC, message digests are used for HMAC authentication of packets. 

MPLS


Multi Protocol Label Switching

NIST 

Is the US National Institute of Standards and Technology, responsible for FIPS standards including DES and it’s  replacement, AES. 

Non-routable IP address 

Is an IP address not normally allowed in the "to" or "from" IP address field header of IP packets. 

Almost invariably, the phrase "non-routable address" means one of the addresses reserved by RFC 1918 for private networks:

·
10.anything 

·
172.x.anything with 16 <= x <= 31 

·
192.168.anything 

These addresses are commonly used on private networks, e.g. behind a Linux machines doing IP masquerade. Machines within the private network can address each other with these addresses. All packets going outside that network, however, have these addresses replaced before they reach the Internet.

If any packets using these addresses do leak out, they do not go far. Most routers automatically discard all such packets.

Various other addresses -- the 127.0.0.0/8 block reserved for local use, 0.0.0.0, various broadcast and network addresses -- cannot be routed over the Internet, but are not normally included in the meaning when the phrase "non-routable address" is used. 

Oakley 

A key determination protocol, defined in RFC 2412. 

Oakley groups 

The groups used as the basis of Diffie-Hellman key exchange in the Oakley protocol, and in IKE. Four were defined in the original RFC, and a fifth has been added since. 

One time pad 

A cipher in which the key is 

·
As long as the total set of messages to be enciphered,
·
Absolutely random, 

·
Never re-used.
Given those three conditions, it can easily be proved that the cipher is perfectly secure, in the sense that an attacker with intercepted message in hand has no better chance of guessing the message than an attacker who has not intercepted the message and only knows the message length. No such proof exists for any other cipher.

There are, however, several problems with this "perfect" cipher. 

First, it is wildly impractical for most applications. Key management is at best difficult, often completely impossible.

Second, it is extremely fragile. Small changes which violate the conditions listed above do not just weaken the cipher a bit; quite often they destroy its security completely. 

·
Re-using the pad weakens it to the point where it can be broken with pencil and paper. With a computer, the attack is trivially easy. 

·
Using computer-generated pseudo-random numbers instead of a really random pad completely invalidates the security proof. Depending on random number generator used, this may also give an extremely weak cipher. 

Marketing claims about the "unbreakable" security of various products which somewhat resemble one-time pads are common. Such claims are one of the surest signs of cryptographic snake oil. Systems marketed with such claims are usually completely worthless.

Finally, even if the system is implemented and used correctly, it is highly vulnerable to certain types of attack. If an attacker knows the plaintext and has an intercepted message, he can discover the pad. This does not matter if the attacker is just a passive eavesdropper. It gives him no plaintext he didn't already know and we don't care that he learns a pad which we'll never re-use. However, knowing the pad lets an active attacker perform a man-in-the-middle attack, replacing your message with whatever he chooses.

OpenVPN


Is an open source solution for an IPSEC VPN.

Photuris 

Another key negotiation protocol, an alternative to IKE, described in RFCs 2522 and 2523. 

PPTP 

Is the Point-to-Point Tunnelling Protocol. Papers discussing weaknesses in it are on counterpane.com. 

PKI 

Public Key Infrastructure, the things an organisation or community needs to set up in order to make public key cryptographic technology a standard part of their operating procedures. 

There are several PKI products on the market. Typically they use a hierarchy of Certification Authorities (CAs). Often they use LDAP access to X.509 directories to implement this.

Plaintext 

Is the unencrypted input to a cipher, as opposed to the encrypted ciphertext output. 

Public Key Cryptography 

In public key cryptography, keys are created in matched pairs. Encrypt with one half of a pair and only the matching other half can decrypt it. This contrasts with symmetric or secret key cryptography in which a single key known to both parties is used for both encryption and decryption. 

One half of each pair, called the public key, is made public. The other half, called the private key, is kept secret. Messages can then be sent by anyone who knows the public key to the holder of the private key. Encrypt with the public key and you know only someone with the matching private key can decrypt.

Public key techniques can be used to create digital signatures and to deal with key management issues, perhaps the hardest part of effective deployment of symmetric ciphers. The resulting hybrid cryptosystems use public key methods to manage keys for symmetric ciphers.

Many organisations are currently creating PKIs, public key infrastructures to make these benefits widely available.

Public Key Infrastructure 

See PKI 

RC4 

Rivest Cipher four, designed by Ron Rivest of RSA and widely used. Believed highly secure with adequate key length, but often implemented with inadequate key length to comply with export restrictions. 

RC6 

Rivest Cipher six, RSA's AES candidate cipher. 

Replay attack 

Is an attack in which the attacker records data and later replays it in an attempt to deceive the recipient. 

RFC 

Means Request For Comments, an Internet document. Some RFCs are just informative.
Routable IP address 

Most IP addresses can be used as "to" and "from" addresses in packet headers. These are the routable addresses; we expect routing to be possible for them. If we send a packet to one of them, we expect (in most cases; there are various complications) that it will be delivered if the address is in use and will cause an ICMP error packet to come back to us if not. 

There are also several classes of non-routable IP addresses. 

RSA algorithm 

Means Rivest Shamir Adleman public key encryption method, named for its three inventors. The algorithm is widely used and likely to become moreso since it became free of patent encumbrances in September 2000. 

For a full explanation of the algorithm, consult Applied Cryptography from B.Schneier. A simple explanation is: 

The great 17th century French mathematician Fermat proved that, for any prime p and number x, 0 x^p == x modulo p x^(p-1) == 1 modulo p, non-zero x From this it follows that if we have a pair of primes p, q and two numbers e, d such that: 


ed == 1


modulo lcm( p-1, q-1)

where lcm() is least common multiple, then for all x, 0 x^(ed-1) == 1 modulo pq, non-zero x x^ed == x modulo pq So we construct such as set of numbers p, q, e, d and publish the product N=pq and e as the public key. Encryption is then: 


c = x^e


modulo N

An attacker cannot deduce x from the cyphertext c, short of either factoring N or solving the discrete logarithm problem for this field. If p, q are large primes (hundreds or thousands of bits) no efficient solution to either problem is known. 

The receiver, knowing the private key (N and d), can readily find x sixce: 


c^d == (x^e)^d

modulo N


    == x^ed

modulo N


    == x

modulo N

This gives an effective public key technique, with only a couple of problems. It uses a good deal of computer time, since calculations with large integers are not cheap, and there is no proof it is necessarily secure since no-one has proven either factoring or discrete log cannot be done efficiently. 

RSA Data Security 

A company founded by the inventors of the RSA public key algorithm. 

SA 

Security Association, the channel negotiated by the higher levels of an IPsec implementation and used by the lower. SAs are unidirectional; you need a pair of them for two-way communication. 

An SA is defined by three things -- the destination, the protocol (AH orESP) and the SPI , security parameters index. It is used to index other things such as session keys and initialization vectors.

For more detail, see our section on IPsec and/or RFC 2401.

Security Association 

See SA 

SHA 

Secure Hash Algorithm, a message digest algorithm developed by the NSA for use in the Digital Signature standard, FIPS number 186 from NIST. SHA is an improved variant of MD4 producing a 160-bit hash. 

SHA is one of two message digest algorithms available in IPsec. The other is MD5. Some people do not trust SHA because it was developed by the NSA. There is, as far as we know, no cryptographic evidence that SHA is untrustworthy, but this does not prevent that view from being strongly held.

SKIP 

Is a Simple Key management for Internet Protocols, an alternative to IKE developed by Sun and being marketed by their Internet Commerce Group. 

SPI 

Security Parameter Index, an index used within IPSEC to keep connections distinct. A Security Association (SA) is defined by destination, protocol and SPI. Without the SPI, two connections to the same gateway using the same protocol could not be distinguished. 

For more detail, see our IPSEC section and/or RFC 2401.

SSH 

Secure Shell, an encrypting replacement for the insecure Berkeley commands whose names begin with "r" for "remote": rsh, rlogin, etc. 

SSL 

Secure Sockets Layer, a set of encryption and authentication services for web browsers, developed by Netscape. Widely used in Internet commerce. It is also known as TLS. 

Symmetric cryptography 

Symmetric cryptography, also referred to as conventional or secret key cryptography, relies on a shared secret key, identical for sender and receiver. Sender encrypts with that key, receiver decrypts with it. The idea is that an eavesdropper without the key be unable to read the messages. There are two main types of symmetric cipher, block ciphers and stream ciphers. 

Symmetric cryptography contrasts with public key or asymmetric systems where the two players use different keys.

The great difficulty in symmetric cryptography is, of course, key management. Sender and receiver must have identical keys and those keys must be kept secret from everyone else. Not too much of a problem if only two people are involved and they can conveniently meet privately or employ a trusted courier. This is quite a problem, though, in other circumstances.

It gets much worse if there are many people. An application might be written to use only one key for communication among 100 people, for example, but there would be serious problems. Do you actually trust all of them that much? Do they trust each other that much? Should they? What is at risk if that key is compromised? How are you going to distribute that key to everyone without risking its secrecy? What do you do when one of them leaves the company? Will you even know?

On the other hand, if you need unique keys for every possible connection between a group of 100, then each user must have 99 keys. You need either 99*100/2 = 4950 secure key exchanges between users or a central authority that securely distributes 100 key packets, each with a different set of 99 keys.

Either of these is possible, though tricky, for 100 users. Either becomes an administrative nightmare for larger numbers. Moreover, keys must be changed regularly, so the problem of key distribution comes up again and again. If you use the same key for many messages then an attacker has more text to work with in an attempt to crack that key. Moreover, one successful crack will give them the text of all those messages.

In short, the hardest part of conventional cryptography is key management. Today the standard solution is to build a hybrid system using public key techniques to manage keys.

TLS 

Transport Layer Security, the successor of SSL. 

Transport mode 

An IPSEC application in which the IPsec gateway is the destination of the protected packets, a machine acts as its own gateway. Contrast with tunnel mode. 

Triple DES 

See 3DES 

Tunnel mode 

Is an IPSEC application in which an IPSEC gateway provides protection for packets to and from another system. It contrasts with transport mode. 

Two-key Triple DES 

Is a variant of triple DES or 3DES in which only two keys are used. As in the three-key version, the order of operations is EDE or encrypt-decrypt-encrypt, but in the two-key variant the first and third keys are the same. 

3DES with three keys has 3*56 = 168 bits of key but has only 112-bit strength against a meet-in-the-middle attack, so it is possible that the two key versions are just as strong. 

RFC 2451 defines triple DES for IPSEC as the three-key variant. The two-key variant should not be used and is not implemented directly in Linux FreeS/WAN. It cannot be used in automatically keyed mode without major fiddles in the source code.  

Virtual Private Network 

See VPN 

VPN 

Virtual Private Network, a network which can safely be used as if it were private, even though some of its communication uses insecure connections. All traffic on those connections is encrypted. 

IPSEC is not the only technique available for building VPNs, but it is the only method defined by RFCs and supported by many vendors. VPNs are by no means the only thing you can do with IPSEC, but they may be the most important application for many users.

Wassenaar Arrangement 

Is an international agreement restricting the export of munitions and other tools of war. Unfortunately, cryptographic software is also restricted under the current version of the agreement. 

X.509 

A standard from the ITU (International Telecommunication Union), for hierarchical directories with authentication services, used in many PKI implementations. 

6.5 Reference

RFC 4301: Security Architecture for the Internet Protocol. S. Kent, K. Seo.December 2005. 
RFC 4302: IP Authentication Header. S. Kent. December 2005     

RFC 4303: IP Encapsulating Security Payload (ESP). S. Kent. December 2005.

RFC 4304: Extended Sequence Number (ESN) Addendum to IPsec Domain of Interpretation (DOI) for Internet Security Association and Key Management Protocol (ISAKMP). S. Kent. December 2005.
RFC 4835, 4305: Cryptographic Algorithm Implementation Requirements for Encapsulating Security Payload (ESP) and Authentication Header (AH).D. Eastlake 3rd. April 2007
RFC 5282: Using Authenticated Encryption Algorithms with the Encrypted Payload of the Internet Key Exchange version 2 (IKEv2) Protocol. D.Black, D. McGrew. August 2008. 
RFC 4307: Cryptographic Algorithms for Use in the Internet Key Exchange Version 2 (IKEv2). J. Schiller. December 2005. 
RFC 4308: Cryptographic Suites for IPsec. P. Hoffman. December 2005.

RFC 4309: Using Advanced Encryption Standard (AES) CCM Mode with IPsec Encapsulating Security Payload (ESP). R. Housley. December 2005.

What is a VPN – P.Ferguson and G.Huston - http://www.potaroo.net/papers.html

A Technical Guide to IPSEC Virtual Private Networks – James S.Tiller - ISBN 0-8493-0876-3

A VPN is a communications environment in which access is controlled to permit peer connections only within a defined community of interest, and is constructed through some form of partitioning of a common underlying communications medium, where this underlying communications medium provides services to the network on a non-exclusive basis.








IPSec is designed to provide interoperable high quality, cryptographically-based security for IPv4 and IPv6. The set of security services offered includes access control, connectionless integrity, data origin authentication, protection against replays, confidentiality, and limited traffic flow confidentiality. These services are provided at the IP layer offering protection for the IP and upper layer protocols.





Before any use of cryptographic solution, everyone and especially government agencies such as National Meteorological Centres MUST verify if the solution they want to use is legal in their country





� EMBED Visio.Drawing.6  ���











_1082704324.vsd

_1083332009.vsd

_1283607063.vsd
Architecture
RFC 4301


ESP Protocol
RFC 4303


AH Protocol
RFC 4302


Encryption
Algorithm


Authentication
Algorithm


DOI
RFC 5282


Policy


Key
Management


IPSec document overview 



_1083331859.vsd

_1082704258.vsd

_1082275995.vsd

