ISS/ET-OI2010/Doc. 2.x(1), p. 1
ISS/ET-OI 2010/Doc. 2.1(8), p. 15

	WORLD METEOROLOGICAL ORGANIZATION

_________________________

COMMISSION FOR BASIC SYSTEMS

OPAG ON INFORMATION SYSTEMS & SERVICES
Meeting of the CBS Expert Team on GTS-WIS Operations and Implementation (ET-OI)

Geneva, 22-24 September 2010
	
	ISS/ET-OI 2010/Doc. 2.1(8)

(03.IX.2010)

Item 2
ENGLISH only


Status of RTH Tokyo
(Submitted by Kenji Tsunoda (JMA))

Summary and Purpose of the Document
This document includes the report on the status of WIS implementation Part A (GTS) at RTH Tokyo.
1.  GTS new Connections: Sep.2008 – Sep.2010


After the ET-OI meeting 2008, two Frame Relay (FR) services were terminated and then seven GTS FR circuits have been migrated to alternate networks.
· Established a new MPLS Network; 2009

· Tokyo: 2.0Mbps

· Bangkok: 128Kbps

· Hong Kong: 1.0Mbps

· Manila: 64Kbps
· Upgrade the Seoul-Tokyo link to 128Kbps dedicated circuit; 2009
· Established a new link to Exeter; 2009

· IMTN Cloud-I move to the RMDCN; 2009

· Exeter, Melbourne, Washington and Tokyo

· Details are shown here;
· http://www.jma.go.jp/jma/jma-eng/jma-center/rth/GTS_upgrades.html 

2.  Current status of implementation of the GTS circuits


Current status of GTS circuits connected with the RTH Tokyo is shown in Table 1 and configuration on connectivity is shown in Figure 2.

[image: image1]
Table 1   Implementation status of GTS circuits connected to RTH Tokyo 
3.   Implementation status of TCP/IP protocol and applications

3.1 TCP/IP


TCP/IP protocol is used on all the GTS circuits connected to the RTH Tokyo.  Simple FTP is used to receive satellite data from Washington, Melbourne, Beijing, Seoul and Hong Kong and to relay subset of them to Washington, Melbourne, Beijing, Seoul and Hong Kong.  Products of JMA's one week ensemble forecast model are also distributed to Hong Kong by simple FTP.  Currently, data and products are exchanged in the following manner.

From 

   Washington:
GET by Tokyo

   Melbourne:
PUT by Melbourne with file renaming

   Beijing:
PUT by Beijing with file renaming

   Seoul:
PUT by Seoul with file renaming

   Hong Kong:
PUT by Hong Kong with file renaming

To

   Washington:
PUT by Tokyo with file renaming

   Melbourne:
PUT by Tokyo with file renaming

   Beijing:
GET by Beijing

   Hong Kong:
PUT by Tokyo with file renaming

   Seoul:
PUT by Tokyo with file renaming

Note) GTS file naming convention is used in these file exchange.
[image: image2.png]Beijing
el NG b eter - Bangkok
elbourne L
_ ~New Delhi Hong Kong
Washington ~—Manila

Khabarovsk
3Mbps + 3Mbps

(load balancing )

Seoul




Figure 2: Configuration of the GTS circuit connected to the RTH Tokyo
4.  Use of the Internet

4.1 Internet accessing systems and servers


There are two permanent connections to Internet Service Providers (ISPs), which provide more stable and reliable access to the Internet than the case of one ISP.
All the Internet servers such as JMA's DDB (Distributed Data Base), NEAR-GOOS (North East Asia Regional - Global Ocean Observing System) Regional Real Time Data Base and WDCGG (World Data Centre for Greenhouse Gases) are placed in DMZ (Demilitarized Zone) for security reasons.  Firewalls are monitored and maintained appropriately with contractual vendor support.
4.2 RSMC Data Serving System


The RSMC Data Serving System (DSS) has been providing NWP products and global observation data to NMHSs mainly in the Asian region through the Internet since 1995.  To meet user requirements in expanding services, JMA replaced the DSS by the new one in October 2005.  The current DSS is a cluster system consists of three servers (Figure 3) in DMZ.  It has been confirmed that the server system based on clustering architecture has enough reliability and maintainability to keep around-the-clock operation, and also cost-effectiveness.


Users can access to the DSS by standard FTP commands with authentication procedures.  The DSS identifies accessing from an authenticated user by a user ID, a user password and an IP address of a client computer.  E-mails are used for communication between the administrator and users of DSS on operational information, such as password change, data to be newly available and so on.

[image: image3]
Figure 3    RSMC Data Serving System
5. Traffic analyze 
5.1 Traffic volume is increasing twice in two years.

[image: image4.emf]
Figure 4   Statistics of data volume

5.2 Data volume in each channel

[image: image5.emf]2.8GB/day

3.5GB/day

August 1st2010


5.3 Traffic status of the Tokyo-Washington link

[image: image6]

[image: image7]
5.4 Traffic status of the Tokyo-Melbourne link

[image: image8]

[image: image9]
5.5 Traffic status of the Tokyo-Beijing link

[image: image10]

[image: image11]
5.6 Traffic status of the Tokyo-Exeter link

[image: image12]

[image: image13]
5.7 Traffic status of the Tokyo-New Delhi link

[image: image14]

[image: image15]
5.8 Traffic status of the Tokyo-Bangkok link

[image: image16]

[image: image17]
5.9 Traffic status of the Tokyo-Khabarovsk link

[image: image18]

[image: image19]
5.10 Traffic status of the Tokyo-Hong Kong link

[image: image20]

[image: image21]
5.11 Traffic status of the Tokyo-Seoul link

[image: image22]

[image: image23]
5.12 Traffic status of the Tokyo-Manila link

[image: image24]

[image: image25]
Storage system





Management Server 





Firewall













































































Storage system





Public Server 





Super computer


system





GTS





MSS





� EMBED Unknown  ���





Storage system





Fiber-sw





Fiber-sw





L2SW





L2SW











Network





Internet





User Ｂ





User Ａ





DMZ





L2SW





L2SW





L2SW





L2SW





Load balancer








[image: image26.wmf][image: image27.png]OBS MPLS
RMDCN
(3Mbps+3Mbps)

NTT MPLS
(2Mbps)

Dedicated 64Kbps
Dedicated 128Kbps

Beijing
Exeter
Jelbourne
ew Delhi
Washington
Bangkok
Hong Kong

Janila

habarovsk

Seoul

TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP
TCP/IP

AN, BL, FX
AN, BL, FX
AN, BL, FX
AN, BI
AN, BL, FX
AN, BI
AN, BI
AN, BI
AN, BL, FX

AN, BL, FX

Sat.

Sat.

Sat.

Sat, Radar

771.4/531.0MB
325.7/780.0MB
560.5/174.9MB
95.2/2.0MB
183.2/12233MB
74.4/21MB
679.3/22.0MB
93.8/0.1MB.
102.6/2AMB

623.3/25.4MB



[image: image28.png]MByte/20min
9.0

Traffic from Tokyo to Washington
(August 1st 2010)

HFILE

8.0
7.0

uBIN
AN

6.0

5.0

4.0

3.0

2.0
1.0
0.0
0000 0300

0600 0900 1200 1500

1800

2100

2400




[image: image29.wmf][image: image30.png]


[image: image31.png]


[image: image32.png]MByte/20min Traffic fromWashington to Tokyo

(August 1st 2010) EFILE
60 HFAX
50 uBIN
AN
40
30
20 B
10 B
0

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image33.png]MByte/20min Traffic from Tokyo to Melbourne WFEILE

(August 1st 2010)

25 FAX
uBIN
20 AN
15
10
5 |
0

0000

0300 0600 0900 1200 1500 1800 2100

2400




[image: image34.png]MByte/20min

Traffic fromMelbourne to Tokyo

(August 1st 2010) uFILE
8.0 HFAX
7.0 “BIN
6.0 BAN
5.0
4.0
3.0 |
2.0 |
1.0 |
0.0 L

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image35.png]MByte/20min

Traffic from Tokyo to Beijing

(August 1st 2010)

MFILE

0000 0300

0600 0900 1200

1500 1800 2100 2400




[image: image36.png]MByte/20min Traffic from Beijing to Tokyo

16 (August 1st 2010) HFILE
14 uBIN
HAN

12

10 l -
8
6 -
4 -
2 -
0 |

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image37.png]MByte/20min Traffic from Tokyo to Exeter WFILE

16 (August 1st 2010) HFAX

14 BIN

12 AN

10

8

6

4 |
2 -
0

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image38.png]MByte/20min
40

Traffic from Exeter to tokyo
(August 1st 2010)

MFAX

35

HAN

EBIN

30

25

20
15

10

5
0
0000 0300

0600

0900

1200

1500

1800

2100

2400




[image: image39.png]MByte/20min

Traffic from Tokyo to New Delhi
(August 1st 2010)

EBIN

9.0
8.0

AN

7.0

6.0

5.0

4.0
3.0

2.0

1.0
0.0

0000 0300

0600

0900

1200

1500

1800

2100 2400




[image: image40.png]KByte/20min
160

Traffic from New Delhi to Tokyo
(August 1st 2010)

=BIN

140

AN

120

100

80
60

40
20
0
0000

0300

0600 0900 1200 1500

1800

2100 2400




[image: image41.png]. Traffic from Tokyo to Bangkok
MByte/20min (August 1st 2010) uBIN

8.0 HAN

1.0
0.0 I]Lll..n L

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image42.png]KByte/20min Traffic from Bangkok to Tokyo
(August 1st 2010)
120 AN -
100
80
60
40
20 -

0

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image43.png]Traffic from Tokyo to Khabarovsk

MByte/20min uFAX
August 1st 2010
(Aug ) =BIN

10.0 HAN

8.0

6.0

4.0

2.0

0.0 —

0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image44.png]KByte/20min

Traffic from Khabarovsk to Tokyo
(August 1st 2010)

uFAX
AN

0000

0300

0600

0900

1200

1500

1800

2100

2400




[image: image45.png]MByte/20min

Traffic from Tokyo to Hong Kong
(August 1st 2010)

HFILE

90
80

u BIN

HAN

70
60

50

40

30

20

10

0
0000

0300

0600 0900 1200 1500

1800

2100

2400




[image: image46.png]MByte/20min Traffic fromHong Kong to Tokyo
August 1st 2010
12 (August1st 2019 wFILE
uBIN
1.0 AN
0.8
0.6
0.4
0.2
0.0 L
0000 0300 0600 0900 1200 1500 1800 2100 2400




[image: image47.png]i Traffic from Tokyo to Seoul
MBytes/20min (August 1st)£010) SFILE
20 uFAX
18 EBIN
16 AN
14
12
10 i
8
6
4
2
0
0000 0300 0800 0900 1200 1500 1800 2100 2400




[image: image48.png]MByte/20min

Traffic from Seoul to Tokyo

(August 1st 2010)

WFILE

uBIN

1.6

AN

1.4

1.2

1.0

0.8

0.6

0.4
0.2
0.0

0000

0300

0600

0900 1200

1500

1800

2100

2400




[image: image49.png]MByte/20min

Traffic from Tokyo to Manila
(August 1st 2010)

10.0

uBIN
HAN

9.0
8.0

7.0

6.0

5.0
4.0

3.0

2.0

1.0

0000

0300

0600 0900 1200 1500

1800

2100

2400




[image: image50.png]KByte/20min Traffic fromManila to Tokyo
(August 1st 2010)

45 HAN

2.5

2.0

1.5

1.0

0.5 |

0.0 .

0000 0300 0600 0900 1200 1500 1800 2100 2400




_1249807789

