ET-CTS 2010/Doc.2(4) p.1

	WORLD METEOROLOGICAL ORGANIZATION

_________________________

CBS Expert Team on WIS-GTS Communication Techniques and Structure (ET-CTS)
Geneva 21-24 September 2010 
	
	ET-CTS 2010/Doc 5
(21.09.2010)
____________

Agenda item 5
ENGLISH only


UPDATE OF THE MANUAL ON THE GTS, WMO MAN 386, ATTACHMENT II-15
(Submitted by Jean-François Gagnon (Canada))

	Summary and purpose of document

This document proposes a new version of the “Manual on the GTS” (WMO Manual No 386, Attachment II-15), which has been reviewed to be current and more readable.
Decision required:

The Expert Team is invited to discuss and approve the new version.


The new version of the Manual on the GTS (Attachment II-15 to the WMO Manual No. 386) is attached in Annex and should replace the current version.
ATTACHMENT II-15

RECOMMENDED PRACTICES AND PROCEDURES FOR THE

IMPLEMENTATION, USE AND APPLICATION OF TCP/IP ON THE GTS

FOREWORD 

Over the years, the GTS has evolved tremendously.  Various protocols were used including X.25 in the recent years.  Most GTS links have now been converted to the industry standard TCP/IP, either using direct point-to-point links or more sophisticated networks.

The use of TCP/IP protocols and associated procedures continues to deliver direct savings in financial and human resource costs to Members by:

(a) Reducing costs for communications equipment purchase and maintenance; and

(b) Reducing software development work through use of industry standard software systems.


Considerable efforts have been applied in defining the framework for applying TCP/IP to the GTS and for the orderly transition from the OSI/X.25 based origin of the GTS.  Furthermore, it is understood that TCP/IP will be the basis for all new telecommunication functions implemented in support of the WMO Information Systems (WIS).   

Procedures are defined to ensure that the primary function of the GTS in carrying real time operational traffic with minimum delay is preserved.  Also, the issue of securing the GTS from interference from the Internet and other networks is also addressed in general terms. Reliance must however be placed on all Members with a TCP/IP based connection to the GTS, who are also connected to the Internet and other networks, to implement and maintain thorough security practices. 

This Attachment and the information related to this topic which is available on the WMO World Wide Web pages provide details of the technical implementation of many of TCP/IP procedures for the GTS. The information is available on the WMO web pages at:
http://www.wmo.int/pages/prog/www/manuals.html

http://www.wmo.int/pages/prog/www/documents.html

Members are strongly advised to take account of the adoption of the TCP/IP based strategy for the future development of GTS, in planning the future development of systems within their national Centres.

41
Introduction


41.1
Historical perspective


41.2
Purpose of this Attachment


41.3
Relationship of the Internet and GTS


51.4
Evolution of the GTS


61.5
Other related issues


72
Principles governing the use of TCP/IP on the GTS


72.1
Basic Concepts


82.2
Overall topology of interconnection


102.3
TCP/IP addressing


122.4
Routing and traffic management


122.4.1
Routing algorithms


122.4.2
Static routing


122.4.3
Dynamic routing


122.4.4
Recommended routing method


122.5
Network Segregation and Zoning


132.6
Traffic management


153
Implementation Guidelines


153.1
Addressing details


153.1.1
Addressing for X.25 packet switching between Centres


153.1.2
Addressing for IP over X.25


163.1.3
Addressing for Direct IP


173.1.4
Addressing for X.25 over IP


183.2
TCP/IP Routing


183.2.1
Autonomous System Numbers


193.2.2
Implementation details


303.3
Management and allocation of addresses and AS numbers


314
Adapting Message Switching Systems to TCP/IP


314.1
Introduction


314.2
TCP Sockets based MSS


324.3
FTP Procedures and File Naming Convention


324.3.1
Introduction


324.3.2
Accumulating messages into files


334.3.3
File naming conventions for existing message types (existing AHL)


344.3.4
General file naming conventions


374.3.5
File renaming


374.3.6
Use of directories


384.3.7
Account names and passwords


384.3.8
FTP Sessions


384.3.9
Local FTP requirements


384.3.10
Use of file compression


384.3.11
Backup with an IP based GTS


385
Trouble shooting and problem resolution


385.1
IP Layer Tools


395.1.1
PING


395.1.2
TRACEROUTE


405.1.3
NETSTAT


415.2
Other monitoring tools


415.2.1
SNMP


425.2.2
MRTG


435.2.3
SYSLOG


435.2.4
Bandwidth Management


446
Appendix 1 - High Level TCP/IP Topology and TCP/IP Data Flows


477
APPENDIX 2 - Cisco Router Configurations


528
APPENDIX 3 - Sample Socket Send and Receive Routines


619
APPENDIX 4 - Some security arrangements for small GTS Centres


619.1
Security policy


619.2
Coexistence of Internet and dedicated GTS links


629.3
Protecting the GTS links from the Internet


639.4
GTS using the Internet


6410
APPENDIX 5 -  Reference material


6410.1
General references on TCP/IP


6510.2
References on Security


6511
APPENDIX 6 - Suggested password management practices


6612
Appendix 7 - IP addresses for use on the GTS


6612.1
INTRODUCTION


6612.2
WHO CAN PROVIDE OFFICIAL IP ADDRESSES?


6612.3
CONNECTING NETWORKS WITH OFFICIAL IP ADDRESSES


6712.3.1
Using official IP addresses assigned directly to an organization (e.g. the NMS)


6712.3.2
Using official IP addresses provided by a telecommunications supplier


6712.3.3
Using IP version 6 (IPv6) addresses


6712.4
CONNECTING NETWORKS WITHOUT OFFICIAL IP ADDRESSES


6712.4.1
Using the “ip unnumbered” feature


6712.4.2
Using RFC1918 – Addresses for private internets


6812.5
RECOMMENDATION


6912.6
CONFIGURATION EXAMPLES





Introduction

Historical perspective

The GTS at present is predominantly used to support the message switching application using message exchange in WMO format.  This exchange is done using:

· TCP/IP protocols,

· limited OSI transport service based on point to point X.25;

and is supplemented by broadcasts.

This implementation is adequate for the legacy application of message switching but requires continuous improvements to fully support the various WMO programs and the WIS. For example, the GTS should support:

Distributed Data Bases (DDB);

Data exchange between non adjacent centres;

Exchange of information that cannot readily be handled by message switching systems (MSSs).
It should be noted that there are no plans to replace the GTS network with a new WIS network, but rather to favour an evolution of the GTS network to provide telecommunication mostly for the critical real-time data exchanged by the MSSs, compatible with the WIS architecture.
Purpose of this Attachment

This Attachment is intended to assist Centres to implement Transmission Control Protocol/Internet Protocol (TCP/IP) based services on the GTS. Throughout this document, it is understood that the implementation of TCP/IP protocols include all essential protocols that are normally part of the TCP/IP protocol suite, as described in the Internet Engineering Task Force (IETF) reference documents RFC1122 and RFC1123. These documents are available from the IETF website at: http://www.ietf.org/ .

The aim of this Attachment is to describe those aspects of the application of TCP/IP that apply specifically to the GTS to meet new requirements and also the long established routine data exchange undertaken by Message Switching Systems (MSSs). The Attachment takes account of the technical evolution of the GTS from an X.25 based network, and maintains the philosophy that Centres continue to be autonomous as far as possible. It is recognized that the timing for implementation of new systems is determined by individual Members in the light of their available resources and relative priorities, but it is also understood that new WIS functionality is expected to be achieved mostly via TCP/IP protocols.

This Attachment does not cover fundamentals of TCP/IP but focuses on those aspects that are essential for successful application on the GTS. Such aspects include appropriate use of the GTS compared with the Internet, co-existence of the GTS and the Internet, IP and X.25 and Autonomous System addressing, router management, TCP/IP application services (such as FTP) and fault management. 
Information Technology Security (ITS) is an important consideration in the design and operation of networks today.  A comprehensive discussion on security can be found in the “WMO Guide on Information Technology Security”, which is available on the WMO website at: http://www.wmo.int/pages/prog/www/manuals.html.
Relationship of the Internet and GTS

The Internet has grown rapidly in capacity, penetration and diversity of applications. Its bandwidth greatly exceeds that of the GTS and it could potentially take over some functions of the GTS. Although day to day performance of the Internet used to be a recognized weakness, recent experience shows that in many countries, its reliability has reached acceptable levels.  It should be noted however that the very nature of the Internet will always mean that no one can build a system using the Internet for which specific service levels can be guaranteed.  The Internet is the result of the amalgamation of numerous telecommunication systems, for which no operator has complete responsibility.

It is therefore recognized that the internet can be used as:

      
An underlying technology for some components of the GTS in special conditions,

As a backup to the GTS; and

As a complement to the GTS.

Table 1.  Usage of GTS and Internet

	Communication Component
	Underlying Technologies
	Function

	GTS
	Dedicated links, high availability network clouds, VPN via Internet for backup or when no other technology is available

	Delivery of time critical communication for weather, water and climate operations

	Internet
	As provided by supplier
	Communication for less critical requirements and possibly for large volumes of data


Coexistence with the Internet also brings some special security problems that must be addressed to ensure the GTS can fulfill its function. In particular, the networks must be engineered in such a way that the GTS is protected from general Internet traffic and is secured against inappropriate use and unauthorized access. For example, the use of IP and dynamic routing protocols such as BGP4 (Border Gateway Protocol) on the GTS will have to be managed in such a way as to allow communication between non-adjacent Centres only with the knowledge and concurrence of all intermediate Centres. Otherwise there is a danger that large amounts of GTS capacity could be consumed by non-routine traffic, to the detriment of real time operational data exchange.

Evolution of the GTS

The use of the ISO/ITU standard X.25 was adopted by WMO in the early 1980's to facilitate the exchange of data and products encoded in WMO binary code forms (GRIB, BUFR etc) and to act as a base for higher level OSI applications. Although OSI was regarded at the time as the strategic direction for the evolution of data communications, this has changed.  Today, there is no doubt TCP/IP protocols are the most accepted and widespread protocols for exchange of data.

TCP/IP is still appropriate because:

(a) it is the dominant protocol suite in everyday use being now packaged with virtually all implementations of Unix and many PC operating systems such as Windows 95 and NT;

(b) it offers a wide range of standard applications (file transfer, electronic mail, remote logon, World Wide Web, etc.) that will greatly reduce the need for the WMO community to develop special procedures and protocols as it has had to do in the past.

(c) it provides useful features such as automatic alternate routing (in a meshed network) which could improve the reliability of the GTS.


This Attachment however takes account of the fact that centres have based plans and developed systems in line with the OSI standards, particularly X.25, as endorsed by WMO and specified in the Manual on the GTS. The transition to TCP/IP based services must continue in an orderly fashion from the X.25 based links in such a way that operation of the GTS is not disrupted or put at risk.

The Attachment provides for this by defining procedures for:

(a) an interim hybrid based on:

(i) carrying TCP/IP based services over an X.25 network service; or

(ii) carrying X.25 data over IP based network service via directly connected routers;

(b) subsequent transition to pure IP utilising directly connected routers, together with TCP/IP based application services, such as TCP sockets or File Transfer Protocol (FTP).

The transition to the second step (pure IP) is desirable because:
(a) Operating TCP/IP over X.25 may not provide expected throughput because of router processing overheads involved in packet encapsulation of IP frames within X.25 packets. This appears to become worse as line speeds increases. Limited tests which have been done between Centres in Region VI indicate efficiency less than 70% at 64Kbps.

(b) The management and maintenance activities required for the X.25 network and associated packet switches can be avoided.

(c) Carrying X.25 over IP requires use of proprietary features of specific router brands.

In order to move to pure IP, it is necessary to modify MSSs at each Centre to make use of TCP/IP services such as FTP and Sockets. This is covered in some detail in Chapter 4.

Other related issues

Many Centres now have experience of TCP/IP on the GTS.  Experience has shown that the main technical issues, which need to be addressed to establish widespread use of TCP/IP on the GTS, are:

(a) agreed methods for the message switching application to use TCP/IP either directly or via higher level applications e.g. FTP;

(b) an agreed  file naming convention and standard for metadata associated with files;

(c) a community wide Naming and Addressing agreement.

It is the aim of this Attachment to make some progress with these issues, some of which lie in the domain of Data Management as much as Telecommunications. It must also be recognised that overall, the existing GTS is not a homogenous network in the true sense of the word, but a collection of regional networks and discrete point-to-point links. Also managed networks using Frame Relay and MPLS (Multi Protocol Label Switching) technology are now part of the GTS. These developments introduce new issues regarding multi lateral co-operation in operating the GTS. While these issues are raised, they are beyond the scope of this Attachment.

Principles governing the use of TCP/IP on the GTS

Basic Concepts
The exchange of information using the standards proposed by the WMO uses a layer model for telecommunication.  These layers can be divided in 2 groups:

(a) The lowest layers are more or less the 7 layers of the OSI Model (http://en.wikipedia.org/wiki/OSI_model).  These layers are the standard TCP/IP protocol stack;

(b) The top layers are the WMO Message Switching System (MSS) applications.  


[image: image1.emf]OSI LAYER 7

APPLICATION

OSI LAYER 1

PHYSICAL

OSI LAYER 2

DATA LINK

OSI LAYER 3

NETWORK

OSI LAYER 4

TRANSPORT

OSI LAYER 5

SESSION

OSI LAYER 6

PRESENTATION

TCP/IP APPLICATIONS

Telnet, FTP, NFS, NIS

TCP/IP PHYSICAL

Coax, Fiver, 10baseT, etc.

TCP/IP DATA LINK

Ethernet, FDDI, PPP, etc.

TCP/IP NETWORK

IP, ARP, RARP, ICMP

TCP/IP TRANSPORT

Sockets/Streams

TCP/IP SESSION

RPC

No specific implementation 

of this layer in TCP/IP stack

MSS APPLICATIONS

Various implementations

WMO 

APPLICATION 

LAYER

TOP LAYERS

LOWER LAYERS


Figure 2.1.1 Layer Model for Telecommunications
The introduction of TCP/IP does not remove the need for the top MSS layers.  They are still required to properly route the weather and environmental data based on standard T1T2A1A2ii data designators (the WMO standard data designators are given in Attachment II-5) or based on standard file naming (described later in this attachment).

The bottom layers (TCP/IP protocol stack) are used to actually deliver the messages to a given location in the world.  When a message is transmitted, the MSS applications prepare the message and decide where the information should be sent.  The information is then encapsulated in the TCP/IP protocol stack layers and it is the bottom layers that actually deliver the messages to their destination.

The TCP/IP protocol suite is an enabler to:

(a) Simplify interconnectivity between computer systems by allowing several telecommunication technologies to be integrated into a coherent network which may include automatic redundant backup routes,

(b) Lower costs by providing standard telecommunication solutions,

(c) Build modern applications not just limited to strict, fixed store and forward traffic rules.

It should be noted that both the top layers (MSS applications) and bottom layers (TCP/IP protocol stack) use addresses and routing.  These addresses are different from layer to layer.  Also the routing is different.  The MSS layers use T1T2A1A2ii data designators and country codes for addresses.  The routing is a manual configuration based on the particular data needs of each Centre.  The TCP/IP layers use IP addresses and routing.  There is no correlation between the MSS addresses and the TCP/IP addresses.  TCP/IP addressing and routing is disuceed later in this attachment.

IPv6 words here?



Overall topology of interconnection

A general view of the possible interconnectivity between Centres is given in Figure 2.2.1.
[image: image2.wmf]BROADCAST

NETWORK

GTS

INTERNET

CENTER B

 OTHER NON

-

GTS LEGACY AND BACKUP LINKS 

CENTER A


Figure 2.2.1. Possible interconnectivity between Centres

This picture shows that there are many ways to interconnect Centres.  The functions carried out by a particular Centre will dictate which telecommunication systems and technologies that Centre will need to support.

The MDCN, GTS & Internet are separate networks.  Each provides different levels of security, service and redundancy.  They should therefore be used for different purposes and different traffic types.  They should also be kept as separate networks.  This in particular is discussed further later on in this attachment.


[image: image3.emf]BROADCAST

NETWORK

GTS

INTERNET

CENTER B

 OTHER NON-GTS LEGACY AND BACKUP LINKS 

CENTER A

INTERNET


Figure 2.2.2.  Using the Internet as a GTS link

In Figure 2.2.2, the Internet is also used in a second manner.  CBS has expressed the view that the use of Internet for GTS links can be considered in circumstances where they are cost effective, offer an acceptable level of service and where adequate security measures are implemented. In general, the same principles for routing and security apply where Internet links are used instead of dedicated links. This special configuration requires special devices and protocols and is a particular configuration of Virtual Private Networks (VPN).  Further details applying to the use of Internet based links, especially related to small GTS Centres, are given in Appendix 4.  
Since most Centres and most telecommunication systems already use TCP/IP, the interconnection using the various networks becomes a fairly simple task.  However, some care must be taken to address the counter effects of these benefits and in particular, more flexibility in interconnection and in applications comes at the price of less control on where traffic can go.  For example, a general purpose link to a GTS cloud network might get flooded with less critical traffic requested by a site that doesn’t normally request data through a given link.  It may also mean that traffic has trouble reaching its destination because there are several ill-defined routes (through both the GTS and the Internet) to get there.

This care can be achieved through traffic control and segregation, which would address three basic issues:

(a) traffic management (ensuring timely delivery of critical data, controlling limited bandwidth availability in some areas)

(b) security (protecting centres from unwanted threatening events)

(c) routing coherence (ensuring that the overall resulting network can deliver traffic without difficulty to any given location)

In order to properly manage the interconnections of Centres and networks, the following elelemtns are essential responsibilities of all Centres:

(a) Ensure that proper TCP/IP addressing is used and properly configured to maintain network integrity and to uniquely identify all components;

(b) Ensure that proper TCP/IP routing is used and properly configured to direct traffic on the correct network and to prevent traffic from going where it shouldn’t.

(c) Ensure that networks are separated from each other.  Networks can also be divided in various security zones.  Different networks and zones must not allow unfiltered routing and traffic to traverse their boundaries.  Firewall devices are used to control the borders if networks must be interconnected.

(d) Ensure that only proper traffic is allowed on any given network to control the volume of data and prevent link flooding.

The following sections discuss these elements in detail.

TCP/IP addressing
Centres must use officially registered IP addresses issued by the Internet Assigned Numbers Authority (IANA) or the relevant Regional Internet Registry.  Official IP addresses are required for all systems which communicate through any inter-organizational network, including the GTS, MDCN and Internet.
Since it is recognized that official IP addresses sometimes difficult to obtain in certain areas of the world, some compromise options have been developed to mitigate this problem.

Appendix 7 describes IP addresses in further detail and the recommended options for the use of IP addresses over the GTS.
If Centres use private IP addresses or non-official addresses on their internal networks, then Network Address Translation (NAT) must be adopted for any hosts requiring to communicate over the GTS or the Internet. 

A sufficient number of official addresses must be obtained to correspond to the number of hosts required to communicate externally, and the type of NAT supported by the Centre's access router. If static NAT is adopted, then a one to one correspondence of internal and official addresses is required. If dynamic NAT is used, then there can be more internal addresses than official addresses, with the router allocating the pool of official addresses dynamically as necessary. The documentation for the Centre's access router should be consulted to ascertain the NAT support provided.

Private addresses must not be visible on the GTS or Internet. Figure 2.3.1 shows simplified examples of allowable and non allowable arrangements.

[image: image4.emf]
Figure 2.3.1 (a)    (Allowed)
[image: image5.emf]
Figure 2.3.1 (b)    (Allowed)

[image: image6.emf]
Figure 2.3.1 (c)    (Not Allowed )

Summary of tasks to ensure proper use of IP on the GTS

 
Use only official IP addresses for external communication on the GTS.


Establish an IP connection with one or more Centres. This connection will be pure IP using PPP as a level 2 protocol on the link, (or a proprietary protocol such as Cisco HDLC by bilateral agreement) or IP over X.25 (RFC 1356). In this case use X.121 addresses as defined in Chapter 3.  Configure dynamic routing with BGP (unless you are a Centre with only one GTS connection and have agreed with your neighbouring Centre to use static routing).

Check the barrier between Internet and the GTS (prevent routing from the Internet to the GTS.)

Filter incoming and outgoing traffic in accordance with the requirements described above.

Routing and traffic management

Routing algorithms


In order to be able to send a packet, every host, router or equipment connected on an IP network must have a routing table. The table tells the system where to send the packet. This may be achieved by:

(a) Static routing; or

(b) Dynamic routing.

Static routing

With static routing, every required destination and next hop must be entered in the routing tables by the system administrator. Alternatively, a default route can be declared, although this option is mainly applicable to sites with only one connection to the outside world. If a default route is set up, filters must be established to ensure that only authorised hosts can access the GTS.

Whenever a new Centre is connected to the GTS with IP protocol, the site managers of all other IP capable Centres must add the new address to their routing tables. This might become a major task as IP connectivity spreads over the GTS.

Dynamic routing

With dynamic routing, the routing information is automatically exchanged between routers. This enables the network to learn new addresses and to use alternative paths under fault conditions in a partially meshed network topology. The initial set-up of dynamic routing may be somewhat more complex, but the ongoing management task is greatly reduced.

Use of dynamic routing requires selection of an appropriate routing protocol to operate over the links of the GTS. The protocol must be an exterior gateway protocol (e.g. EGP, BGP) as opposed to an interior gateway protocol (such as IGRP, RIP, OSPF) because interior gateway protocols are intended for use within a single management domain. The GTS is an aggregation of many separate management domains. As such, it is necessary to select a gateway protocol that can be autonomously managed by each Centre to implement routing and hence traffic flow, consistent with its particular requirements.

Two exterior gateway protocols are defined by RFCs - EGP and BGP (now release 4 - RFC 1771). As the GTS is not a tree structure, setting up routing with EGP may be difficult.  BGP 4 does not suffer topological constraints. It is more powerful, but a little more difficult to configure.

BGP can distribute subnetted routes. This feature might be very useful for the GTS. Instead of propagating host-based routes or full network routes, routing can be based on subnetted networks. Instead of declaring hosts eligible to use the GTS, a Centre could declare a full subnet of eligible hosts. In that case, the routing information consists of just an IP address and a subnet mask. For example, if a Centre has a class C addresses 193.168.1.0, by declaring that the subnet 193.168.1.16 with mask 255.255.255.248 is allowed to use the GTS, all hosts with IP address 193.168.1.17 to 193.168.1.22 will be routable on the GTS.

Recommended routing method

Based on consideration of the above factors the BGP4 routing protocol should be used between Centres on the GTS, unless an alternative is bilaterally agreed on individual links. Examples of BGP4 set-up for the Cisco router family are given in Appendix 2.

Network Segregation and Zoning
Any Centre which has a TCP/IP based GTS connection and a connection to another TCP/IP network is a potential weak point where the GTS could be exposed to deliberate or inadvertent interference through unwanted traffic or unauthorised connection to GTS hosts.

Centres are strongly encouraged to implement protective barriers such as firewall systems on the connection of their Centre with the Internet. It is important that every practical step is taken to prevent accidental or deliberate use of GTS links or unauthorised access to GTS Centres, by Internet users.

When setting up IP on the GTS, it is vital to ensure that the GTS does NOT become part of the Internet or an unintended transmission path for Internet traffic. Each Centre must consider the GTS and other TCP/IP networks (such as the Internet) as separate networks and ensure that inappropriate flow of traffic from one to the other cannot occur. This will ensure that the GTS is used only for transferring bona fide meteorological data between authorised hosts. 

To achieve traffic control and segregation, there are several important aspects to consider:

(a) IP addressing: using universally recognizable and coherent network addresses so that all systems only have one unique reference number, which is valid not only within the GTS but across the Internet and any other network which may eventually be interconnected to the GTS

(b) IP network routing rules: using a common set of routing protocols and rules to ensure that any traffic can be consistently sent to its destination without delay or confusion

(c) Zoning of each Centre’s network elements: creating different network zones with different security levels, to isolate a Centre’s critical elements from publicly available areas and ensuring that data can still flow between zones of differing security levels

Figure 2.2.1 illustrates in a general way, how a Centre with TCP/IP connection to the GTS and an Internet connection might be set up. This setup also infers that certain security function must be implemented.  Functions to be implemented include:

(a) allowing only GTS designated hosts to communicate through the GTS router;

(b) blocking access to GTS designated hosts through the firewall and Internet router;

(c) firewall allows only approved hosts on the Internet to access B hosts and then, only for approved applications such as FTP;

(d) prevention of access to A hosts from Internet via B hosts.

The actual choice of routers and firewall and the setting up of these will require expertise in the design and configuration of networking and security systems.  In addition to network security measures, it is vital that good security practices are followed in the management of all hosts in a Centre. Computer security is a complex subject in itself and Centres are encouraged to study this in depth and apply appropriate practices.  The Information Technology (IT) Security Guide available at 

http://www.wmo.int/pages/prog/www/documents.html

provides information on basic essential security practices.

Traffic management

Traffic management is an area which is unfortunately not limited to networks, but also involves data management and application configurations.  Several groups are therefore involved in this matter.

In general, it can be said that some applications such as file transfer, World Wide Web have potential to place heavy loads on the limited bandwidth circuits that comprise the GTS. Limits need to be applied to ensure that the GTS carries only important traffic such as the real time data and products currently exchanged on the GTS plus data to be carried to fulfil new requirements such as Distributed Data Bases (DDBs), and routinely exchanged large data files such as satellite imagery.

Less important traffic such as ad hoc file exchange, e-mail, general World Wide Web and suchlike should be carried on the Internet.  To protect the GTS, the full capabilities of TCP/IP connectivity and information exchange must be restricted. In practical terms, TCP/IP traffic carried on the GTS could be restricted on the basis of

(a) protocol type (e.g. FTP, HTTP, SMTP etc);

(b) originating and destination IP address; 

(c) a combination of the above.

If the measures adopted are to be successful, it is necessary that they be:

(a) not confined to a single router brand since it cannot be assumed that all centres will have the same brand of router; and

(b) be reasonably straightforward to configure, so that there is minimum risk that configuration errors or omissions will endanger the GTS.

Implementation Guidelines 

Addressing details
IP based services on the GTS may be implemented with direct IP connections.  In order to maintain communication with some legacy systems, a mixture of X.25 and IP is also possible.  However, Centres should strive to replace X.25 systems with TCP/IP services (FTP, Sockets) superseding IP over X.25, or X.25 over IP as the case may be.
If X.25 must be used, IP services may be carried over an X.25 network by encapsulating IP packets within X.25 packets. An appropriately configured router at each GTS Centre carries out this function. Alternatively, where routers of the same brand are used in adjacent Centres, X.25 data may be carried on an IP link using X.25 switching capability of the routers.

It is therefore still necessary to have an addressing framework for:

(a) X.25 packet switching between Centres;

(b) IP over X.25;

(c) Direct IP (including X.25 over IP).

Addressing for X.25 packet switching between Centres

Many centres have adopted X.25 for point to point connections between Message Switching Systems (MSS). A number of Centres have installed, or plan to install packet switches to provide capability for connections between non adjacent Centres. An addressing scheme has been developed for this purpose and has been adopted by WG-TEL at its thirteenth session in 1994.  It is a 14-digit scheme of the form:

0101xxxiiyyyzz where:

0101 
is a pseudo DNIC which does not correspond to any actual DNIC and therefore will ensure calls cannot be mistakenly switched to any network other than the GTS;

xxx
is the X.121 Country Code of the Centre;

ii 
is a Protocol indicator, being 00 for MSS, 11 for TCP/IP, 22 for 
OSI CONS, 33 for OSI CLNS;

yyy
is the nationally assigned port number;

zz
is the nationally assigned sub address number;

This addressing scheme is to be used for setting up Virtual Calls (VCs) for MSS applications and for any other GTS applications including carriage of IP traffic over X.25.

Addressing for IP over X.25

In order to carry IP traffic over X.25, two globally co-ordinated address schemes are necessary:

An X.25 scheme as described above; and

An IP address scheme to apply to the interface between the router and packet switch to enable the router to encapsulate the IP packets into X.25 packets.

The general arrangement is shown in figure 3.1.

For IP over X.25 to function correctly, it is necessary for the underlying X.25 network to be allocated a single IP network address and for each Centre to have an address within this network for the connection point between its router and its packet switch.  Each IP node on the network can be assigned a sequential host address within a single Class C IP address as illustrated in Figure 3.1. The Class C address can provide for 254 Centres to be connected using a subnet mask of 255.255.255.0.

[image: image21.png]Y AD 2 Eadl

| Leseuschen_Jy Adresss [ cvd de/mnaiareiematonalfotreadingz il =] @) Vetwendte Obisk

DDV Of-EZMW Reading (2 Mb/s)

The statistics were last updated Monday, 6 September 1999 at 12:11,
at which time 'tiofswegO1-a' had been up for 480 days, 21:15:49

*Daily’ Graph (5 Minute Average)

2000 k
1800 k
1200 k

0.0 k

Bits per Second

0.0k

4 6 8 10 12 14 16 18 20 22 0 2 4 6 & 10

Maxln: 2260 kbfs  Aversgeln 184kbfs  Curment In: 38320 bis
MaxOut: 633kbfs  Average Out:127 kbfs Current Out: 486 kbis

‘Weekly' Graph (30 Minute Average)

1640 k
123.0 k
X

a0k

Bits per Second

0.0k
Sn Mon  Tue  Ued  Thu  Fri  Sat S

Maxln: 1621 kbfs  Aversgeln 220kbis  Curment - 14560 bis
MaxOut: 7.2kbfs  Average Out: 137 kbis Current Out:67200 bis

*Monthly' Graph (2 Hour Average)

208.0

156.0 k
1040 k

2.0k

Bits per Second

0.0
Veek 35

Week 32 Week 34

Week 33

Maxln: 1965kbis  Aversgeln 247 kbis  CurmentIn: 17200 bis
MaxOut:2041 kbfs Average Out: 153 kbfs Curment Out: 13.5 kbis

== [Dokument Ubamitial EEAE T A

(i






Figure 3.1 - IP implemented over X.25 Network 

The Routers at each centre have to be set up so that they issue an X.25 call request to the X.25 port of the final destination Centre. This means that IP traffic passes through the packet switch only, and not the router of the intermediate Centre.

Addressing for Direct IP

The preferred connection for the future is to use direct IP links.  Centres already using IP over X.25 should consider updating the links to use direct IP.  This transition should be considered in the near future.  Figure 3.2 illustrates how a pair of Centres have agreed to implement a direct IP connection using the first available pair of ‘host’ numbers using the 193.105.178.0 network as an example. 




Figure 3.2. Direct IP link between centres B and C

Allocation of class C addresses for direct IP links
Routers have to be connected by links having unique subnet numbers. To achieve this, a Class C address is used (for example 193.105.178.0) with a mask of 255.255.255.252. This provides 62 subnets each with 2 hosts. These two host numbers are allocated to the ends of the link connecting the routers between the two Centres. The lowest useable network number is 193.105.178.4, with host addresses of 193.105.178.5 and 6. The next network number is 193.105.178.8, with host addresses of 193.105.178.9 and 10, followed by:
193.105.178.12, with host addresses of 193.105.178.13 and 14, followed by

193.105.178.16, with host addresses of 193.105.178.17 and 18, followed by

193.105.178.20, with host addresses of 193.105.178.21 and 22, and so on, up to

193.105.178.248, with host addresses of 193.105.178.249 and 250.

Addressing for X.25 over IP

Where two Centres have a common brand of Router (e.g. Cisco), and the traffic is mostly IP with some X.25, it may be appropriate to carry the X.25 over the directly connected routers as shown for the link between Centre B and Centre C in figure 3.3. The X.25 packets are carried within IP packets over the serial link between the routers, which may be a proprietary HDLC protocol, or a standard protocol such as PPP. This functionality requires that routers in each Centre contain X.25 packet switching software and that the X.25 route details are included in the router configuration. Examples of typical configurations are given in Appendix 2.




Figure 3.3, Combination of IP over X.25 and X.25 over IP
TCP/IP Routing

Autonomous System Numbers

The use of BGP4 as the recommended dynamic routing protocol for the GTS (Chapter 2) requires allocation of Autonomous System (AS) numbers to each GTS Centre.


The use of BGP requires introduction of the concept of the Autonomous System (AS)
.  Each GTS Centre manages an AS number to enable the Centre to adopt BGP with neighbouring centres.  In addition to addressing, this chapter shows allocation scheme of AS numbers.

The Internet Assigned Numbers Authority (IANA), through RFC1930, has reserved the block of AS numbers 64512 through 65535 for private use (not to be advertised on the global Internet). This provides 8 groups of 128 AS numbers to be assigned to GTS Centres, satisfying the current and foreseeable future needs of the GTS. The AS numbers will be assigned as follows:

MTN centres and reserve 

64512 to 64639
Centres within RA I 

64640 to 64767
Centres within RA II 

64768 to 64895

Centres within RA III
 
64896 to 65023
Centres within RA IV 

65024 to 65151
Centres within RA V 

65152 to 65279

Centres within RA VI
 
65280 to 65407

Antarctic
 


65408 to 65471

*Private use by GTS Centres
65472 to 65535

* These AS numbers are for national use and are not to be advertised on the GTS.

Implementation details

In order to implement IP services Centres need to know certain details of IP and X.25 addressing at other Centres on the GTS. The following diagrams and associated tables explain in detail, the information required at various Centres:                                                          


[image: image7.wmf] Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

CENTRE A

CENTRE C

CENTRE B

CENTRE D

IP address:

Ai

X.121: Ax

X.121: Ax

’

IP address: A

IP address: C

IP address: D

IP address: B

Packet

Switch

Packet

Switch

Packet

Switch

Packet

Switch

IP address:

Ci

X.121:

Cx

X.121:

Cx

’

IP address:

Bi

X.121:

Bx

IP address:

Di

X.121:

Dx

X.121:

Bx

’

X.121:

Dx

’


         Figure 3.4  IP over X.25 Network 

Table 3.4a.  IP and X.121 addresses to be known at Centre A

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE B

(Host to host)
	IP address : B
	IP address : Bi

X.121 : Bx
	CENTRE A – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ci

X.121 : Cx
	CENTRE A – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Di

X.121 : Dx
	CENTRE A – CENTRE C – CENTRE D

( Host [A] – Router [A] – Packet Switch [A]  

 – Packet Switch [C] – Packet Switch [D] –

 Router [D] – Host [D] )

                      [x] : CENTRE x

	CENTRE B

(MSS to MSS)
	X.121 : Bx’

(X.25 traffic)
	
	CENTRE A – CENTRE B

	CENTRE C

(MSS to MSS)
	X.121 : Cx’

(X.25 traffic)
	
	CENTRE A – CENTRE C

	CENTRE D

(MSS to MSS)
	X.121 : Dx’

(X.25 traffic)
	
	CENTRE A – CENTRE C – CENTRE D

( MSS [A] – Packet Switch [A] –

 Packet Switch [C] – Packet Switch [D] –

 MSS [D] )


      Table 3.4b.  IP and X.121 addresses to be known at CENTRE B

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE B – CENTRE A

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ci

X.121 : Cx
	CENTRE B – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Di

X.121 : Dx
	CENTRE B – CENTRE C – CENTRE D

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE B – CENTRE A

	CENTRE C

(MSS to MSS)
	X.121 : Cx’

(X.25 traffic)
	
	CENTRE B – CENTRE C

	CENTRE D

(MSS to MSS)
	X.121 : Dx’

(X.25 traffic)
	
	CENTRE B – CENTRE C – CENTRE D


      Table  3.4c.  IP and X.121 addresses to be known at CENTRE C

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Bi

X.121 : Bx
	CENTRE C – CENTRE B

	CENTRE D

(Host to host)
	IP address : D
	IP address : Di

X.121 : Dx
	CENTRE C – CENTRE D

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	X.121 : Bx’

(X.25 traffic)
	
	CENTRE C – CENTRE B

	CENTRE D

(MSS to MSS)
	X.121 : Dx’

(X.25 traffic)
	
	CENTRE C – CENTRE D


      Table 3.4d.  IP and X.121 addresses to be known at CENTRE D

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Bi

X.121 : Bx
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ci

X.121 : Cx
	CENTRE D – CENTRE C

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	X.121 : Bx’

(X.25 traffic)
	
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(MSS to MSS)
	X.121 : Cx’

(X.25 traffic)
	
	CENTRE D – CENTRE C



[image: image8.wmf] Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

CENTRE A

CENTRE C

CENTRE B

CENTRE D

IP address: Ac

IP address:

Ab

IP address: Ca

IP address:

Ba

IP address:

Cb

IP address:

Bc

IP address:

Cd

IP address: Dc

IP address: A

IP address: C

IP address: D

IP address: B

IP address: A

’

IP address: C

’

IP address: B

’

IP address: D

’


          Figure 3.5.  Direct IP Network 

Table 3.5a.  IP address to be known at CENTRE A

	
	       IP addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route

	CENTRE B

(Host to host)
	IP address : B
	IP address : Ba
	CENTRE A – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ca
	CENTRE A – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Ca
	CENTRE A – CENTRE C – CENTRE D

( Host [A] – Router [A] – Router [C]

  – Router [D] – Host [D] )

                    [x] : CENTRE x

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Ba
	CENTRE A – CENTRE B

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Ca
	CENTRE A – CENTRE C

	CENTRE D

(MSS to MSS)
	IP address : D’
	IP address : Ca 
	CENTRE A – CENTRE C – CENTRE D


Table 3.5b.  IP address to be known at CENTRE B

	
	       IP addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route

	CENTRE A

(Host to host)
	IP address : A
	IP address : Ab
	CENTRE B – CENTRE A

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cb
	CENTRE B – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Cb
	CENTRE B – CENTRE C – CENTRE D

	CENTRE A

(MSS to MSS)
	IP address : A’
	IP address : Ab
	CENTRE B – CENTRE A

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Cb
	CENTRE B – CENTRE C

	CENTRE D

(MSS to MSS)
	IP address : D’
	IP address : Cb
	CENTRE B – CENTRE C – CENTRE D


      Table 3.5c.  IP address to be known at CENTRE C

	
	       IP addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route

	CENTRE A

(Host to host)
	IP address : A
	IP address : Ac
	CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(Host to host)
	IP address : D
	IP address : Dc
	CENTRE C – CENTRE D

	CENTRE A

(MSS to MSS)
	IP address : A’
	IP address : Ac
	CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(MSS to MSS)
	IP address : D’
	IP address : Dc
	CENTRE C – CENTRE D


     Table 3.5d.  IP address to be known at CENTRE D

	
	       IP addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route

	CENTRE A

(Host to host)
	IP address : A
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cd
	CENTRE D – CENTRE C

	CENTRE A

(MSS to MSS)
	IP address : A’
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Cd
	CENTRE D – CENTRE C



[image: image9.wmf] Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

CENTRE A

CENTRE C

CENTRE B

CENTRE D

IP address:

Ai

X.121: Ax

X.121: Ax

’

IP address: A

IP address: C

IP address: D

IP address: B

Packet

Switch

Packet

Switch

Packet

Switch

IP address:

Ci

X.121:

Cx

X.121:

Cx

’

IP address:

Bi

X.121:

Bx

IP address: Dc

X.121:

Bx

’

IP address: B

’

IP address: C

’

IP address:

Bc

IP address:

Cb

IP address:

Cd

IP address: D

’


         Figure 3.6.  Coexistence of direct IP with IP over X.25
      Table 3.6a.  IP and X.121 addresses to be known at CENTRE A

	
	      Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE B

(Host to host)
	IP address : B
	IP address : Bi

X.121 : Bx
	CENTRE A – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ci

X.121 : Cx
	CENTRE A – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Ci

X.121 : Cx
	CENTRE A – CENTRE C – CENTRE D



	CENTRE B

(MSS to MSS)
	X.121 : Bx’

(X.25 traffic)
	
	CENTRE A – CENTRE B

	CENTRE C

(MSS to MSS)
	X.121 : Cx’

(X.25 traffic)
	
	CENTRE A – CENTRE C

	CENTRE D

(MSS to MSS)
	Possible only by store and forward via MSS at Centre C

(X.25 traffic)
	
	


 Table 3.6b.  IP and X.121 addresses to be known at CENTRE B

	
	       Addresses to be known
	
	

	Destination
	for communication

 between end
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE B – CENTRE A

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cb
	CENTRE B – CENTRE C

	CENTRE D

(Host to host)
	IP address : D
	IP address : Cb
	CENTRE B – CENTRE C – CENTRE D

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE B – CENTRE A

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Cb
	CENTRE B – CENTRE C

	CENTRE D

(MSS to MSS)
	IP address : D’
	IP address : Cb
	CENTRE B – CENTRE C – CENTRE D


      Table 3.6c.  IP and X.121 addresses to be known at CENTRE C

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(Host to host)
	IP address : D
	IP address : Dc
	CENTRE C – CENTRE D



	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(MSS to MSS)
	IP address : D’
	IP address : Dc
	CENTRE C – CENTRE D


     Table 3.6d.  IP and X.121 addresses to be known at CENTRE D

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cd
	CENTRE D – CENTRE C



	CENTRE A

(MSS to MSS)
	Possible only by store and forward via MSS at Centre C

(X.25 traffic) 
	
	

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Cd
	CENTRE D – CENTRE C



[image: image10.wmf] Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

CENTRE A

CENTRE C

CENTRE B

CENTRE D

IP address:

Ai

X.121: Ax

X.121: Ax

’

IP address: A

IP address: C

IP address: D

IP address: B

Packet

Switch

Packet

Switch

Packet

Switch

IP address:

Ci

X.121:

Cx

X.121:

Cx

’

IP address:

Bi

X.121:

Bx

IP address: Dc

X.121:

Bx

’

IP address: B

’

IP address: C

’

IP address:

Bc

IP address:

Cb

IP address:

Cd

X.121: 

Dx’

X.25 link

X.25 link

IP Link

IP Link


         Figure 3.7.  Coexistence of direct IP, IP over X.25 and X.25 over IP
 Table 3.7a.  IP and X.121 addresses to be known at CENTRE A

	
	      Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE B

(Host to host)
	IP address : B
	IP address : Bi

X.121 : Bx
	CENTRE A – CENTRE B

( Host [A] - Router [A] - "IP over X.25" - Packet Switch [A] - "IP over X.25" - Packet Switch [B] - "IP over X.25" - Router [B] - Host [B] )

	CENTRE C

(Host to host)
	IP address : C
	IP address : Ci

X.121 : Cx
	CENTRE A – CENTRE C

( Host [A] - Router [A] - "IP over X.25" - Packet Switch [A] - "IP over X.25" - Packet Switch [C] - "IP over X.25" - Router [C] - Host [C] )

	CENTRE D

(Host to host)
	IP address : D
	IP address : Ci

X.121 : Cx
	CENTRE A – CENTRE C – CENTRE D

( Host [A] - Router [A] - "IP over X.25" - Packet Switch [A] - "IP over X.25" - Packet Switch [C] - "IP over X.25" - Router [C] - "Direct IP" - Router [D] - Host [D] )

	CENTRE B

(MSS to MSS)
	X.121 : Bx’

(X.25 traffic)
	
	CENTRE A – CENTRE B

( MSS [A] - Packet Switch [A] - Packet Switch [B] - MSS [B] )

	CENTRE C

(MSS to MSS)
	X.121 : Cx’

(X.25 traffic)
	
	CENTRE A – CENTRE C

( MSS [A] - Packet Switch [A] - Packet Switch [C] - MSS [C] )

	CENTRE D

(MSS to MSS)
	X.121: Dx’

(X.25 traffic)
	
	
	CENTRE A – CENTRE C – CENTRE D

( MSS [A] - Packet Switch [A] - Packet Switch [C] - Router [C] - "X.25 over IP" - Router [D] - MSS [B] )


      Table 3.7b.  IP and X.121 addresses to be known at CENTRE B

	
	       Addresses to be known
	
	

	Destination
	for communication

 between end
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE B – CENTRE A

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cb
	CENTRE B – CENTRE C

( Host [B] - Router [B] -  "Direct IP" - Router [C] - Host [C] )

	CENTRE D

(Host to host)
	IP address : D
	IP address : Cb
	CENTRE B – CENTRE C – CENTRE D

( Host [B] - Router [B] - "Direct IP" -  Router [C] - "Direct IP" - Router [D] - Host [D] )

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE B – CENTRE A

	CENTRE C

(MSS to MSS)
	IP address : C’
	IP address : Cb
	CENTRE B – CENTRE C

( MSS [B] - Router [B] - "Direct IP" - Router [C] - MSS [C] )

	CENTRE D

(MSS to MSS)
	X.121: Dx’

(X.25 traffic)
	CENTRE B – CENTRE C – CENTRE D

( MSS [B] - Packet Switch [B] - Router [B] - "X.25 over IP" - Router [C] - "X.25 over IP" - Router [D] - MSS [D] )


      Table 3.7c.  IP and X.121 addresses to be known at CENTRE C

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Ai

X.121 : Ax
	CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(Host to host)
	IP address : D
	IP address : Dc
	CENTRE C – CENTRE D

( Host [C] - Router [C] -  "Direct IP" - Router [D] - Host [D] )

	CENTRE A

(MSS to MSS)
	X.121 : Ax’

(X.25 traffic)
	
	CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	IP address : B’
	IP address : Bc
	CENTRE C – CENTRE B

	CENTRE D

(MSS to MSS)
	X.121: Dx’

(X.25 traffic)
	CENTRE C – CENTRE D

( MSS [C] - Packet Switch [C] - Router [C] - "X.25 over IP" - Router [D] - MSS [D] )


      Table 3.7d.  IP and X.121 addresses to be known at CENTRE D

	
	       Addresses to be known
	
	

	Destination
	for communication

 between ends
	for communication

 between Routers
	      Suitable  route



	CENTRE A

(Host to host)
	IP address : A
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(Host to host)
	IP address : B
	IP address : Cd
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(Host to host)
	IP address : C
	IP address : Cd
	CENTRE D – CENTRE C



	CENTRE A

(MSS to MSS)
	X.121: Ax’

(X.25 traffic) 
	
	CENTRE D – CENTRE C – CENTRE A

	CENTRE B

(MSS to MSS)
	X.121: Bx’

(X.25 traffic)
	CENTRE D – CENTRE C – CENTRE B

	CENTRE C

(MSS to MSS)
	X.121: Cx’

(X.25 traffic)
	CENTRE D – CENTRE C


Management and allocation of addresses and AS numbers

X.25 addresses

The framework described above allows Centres full autonomy in allocating X.25 numbers. The WMO Secretariat will maintain a current list of X.25 addresses which Centres have allocated for use on the GTS.  Centres are requested to notify the Chief of Telecommunications and Monitoring Unit, WWW Basic Systems Department, WMO Secretariat by E-mail or fax of X.25 addresses allocated.
IP addresses

IP addresses should be acquired or agreed on as per the instructions in Appendix 7.
GTS nominated host/network addresses

Host and subnet IP addresses for use with GTS nominated Centres should be notified to WMO as described above.

AS numbers

AS numbers for use on the GTS will be co-ordinated and issued by the WMO Secretariat as required. Centres should direct their requests for AS numbers to WMO as described above.

Publication of addresses and AS numbers


The WMO will publish updated lists of addresses and AS numbers in the monthly WWW Newsletter and will also make these lists available in ASCII text form for access by FTP on the WMO web server and in World Wide Web format at http://www.wmo.int .
Adapting Message Switching Systems to TCP/IP 

Introduction

Although there are new requirements emerging, GTS usage is dominated by the traditional Message Switching application, which has been developed to use X.25 packet switching.  We now need to consider how best to migrate the message switching task to use TCP/IP to satisfy the new requirements by providing "Internet like" facilities on the GTS, and to stay aligned with IT industry trends. Additionally, migration of Message Switching Systems (MSS) to use TCP/IP means that X.25 infrastructure can be removed, greatly simplifying the technology of the GTS by moving to a pure IP network rather than a mixture of IP and X.25.

There are two possible technical approaches to this problem, one using TCP Sockets and the other FTP. In the long term the FTP approach is thought to be the most strategically attractive but may require more work to implement in operational Message Switching Systems.  Many Centres have chosen  to adopt an approach based on TCP Sockets as the first step towards a TCP/IP based GTS.

The transition of the MSSs to use TCP/IP protocols does not imply any change in the basic store and forward architecture of the GTS, as explained earlier in “section 2.1 Basic Concepts”. It is envisaged that the store and forward architecture, with automatic on forwarding based on routing tables, will remain. However, the adoption of FTP means there is an additional option for data exchange to be achieved through bilateral arrangements, by the use of FTP retrieve initiated by the receiving centre.

TCP Sockets based MSS


TCP Socket is an approach which is highly suitable for a programmatic implementation to provide regular exchange of messages. As such it should simply be regarded as an alternative protocol to X.25. A centre will be required to produce MSS application programs capable of transmitting and receiving via a TCP socket. Centres with current applications capable of driving an X.25 virtual circuit should be able to very quickly and simply produce a sockets version by changing a few system calls (see Appendix 3 for sample programs). The programming work involved is minimal and more importantly all other areas of the MSS such as queuing, routing, data management, operator interfaces etc. remain unchanged because the communication exchange is still based on the traditional message. 


The protocol defined here, is based on the assumption that the physical circuit over which the data is to be transmitted has low error rate and is subject to interruptions rarely. On such circuits, the TCP protocol can be expected to deliver error free data. However, some GTS circuits may not be of sufficient quality for the standard TCP socket to function reliably. The development of special protocols for use on low quality circuits may be studied further.


Loss of data may occur if the TCP session is lost. This may be due to MSS hardware, application or communications failure. A special case of this is when a Centre with more than one MSS switches from the primary to the backup systems. Recommendations to avoid this problem are given below.


One useful feature of the X.25 based communication that is not available using TCP sockets, is the ability to detect start and end of message by reference to the M bit in the X.25 packet header. No such bit or any equivalent feature exists in TCP. Therefore, to enable receiving centres to detect end of message, each message is preceded with an 8 character string giving the message length, plus two characters indicating message type (binary, alphanumeric or fax). Thereafter the message is structured within an SOH/ETX envelope as for exchange via X.25. The complete structure is illustrated in figure 4.1. Note that the message length does not include itself or the type indicator. It should always be eight characters long and include leading zeroes as required. The message type indicator should be encoded using ASCII characters BI for binary, AN for alphanumeric,and FX for facsimile. All new connections established must begin with a message length and type structure. 


[image: image11.wmf]Message

Message

nnn

length           type        SOH CR

CR LF  or

CR

CR LF  Heading

CR CR LF ETX

 (8 characters)   (2 characters)

nnnnn

Message length

Message length : Length from SOH to ETX  (e.g. 00001826 = 1826bytes)

  Message type  AN: Alphanumeric,  BI: Binary,  FX: facsimile


Figure 4.1 Message structure for Socket exchange applications
The rules for use of TCP/IP socket exchange can be summarised as:
1.  All new connections must start from a new message.

2.  Each message is preceded by a message length field of eight ASCII characters and a message type field of two ASCII characters.

3.  Message length is counted from SOH to ETX inclusive and must contain leading zeroes as necessary. 
4.  Message type must be encoded as BI for binary, AN for alphanumeric or FX for facsimile.

5.  Receiving centres will check synchronisation as follows:

*  Check that the first 8 characters are ASCII numeric

*  Check that the 9th and 10th characters are BI, AN or FX

*  Check that the 11th character is SOH

*  Check that the last character is ETX.

6. If synchronisation is lost the receiver shall break the connection using the following sequence of TCP user primitives:

· shutdown (to make sure that all data in the TCP send buffer has been transferred)

· close.

7. It is recommended to use separate sockets for ASCII and binary messages, and separate connections for sending and receiving. The sender should always be responsible for establishing the connection.

8. Once a connection is established, it should be maintained. 

9. If there should be a need to close a socket, the procedure should be as follows:

· shutdown  (to make sure that all data in the TCP send buffer has been transferred)

· close.

10. This procedure should also be used when a MSS is being shutdown.

11. If the receiving side receives a new unexpected connection request on a port for which it has an established socket, the old socket should be closed and the new socket accepted.

12. TCP/IP Service/Port numbers for these connections will be decided by bilateral agreement. The use of reserved ports (1 to 1023) should be avoided. The use of ports above 10000 is recommended.

13. To reduce the amount of data lost if an established connection fails, the TCP send and receive buffer sizes can be adjusted. The recommended value for the buffer size is 4KByte, however this value may be agreed on a bilateral basis.

14. To enable detection of message loss, the use of the channel sequence number, (CSN) is mandatory. When using the CSN to check for missing messages, the WMO request/repeat procedures should be used to recover these. It may be useful to automate this mechanism to avoid delays caused by manual interaction. In order to minimise data loss it is strongly recommended that Centres implement a 5 character long CSN in the future.

15. The channel sequence number 000 (or 00000 respectively) should indicate an initialisation, and should not cause retransmission requests.

FTP Procedures and File Naming Convention
Introduction

FTP (File Transfer Protocol) is a convenient and reliable method for exchanging files, especially large files. The protocol is defined in RFC 959.

The main issues to be considered are:

1.  Procedures for accumulating messages into files so as to minimise FTP overheads with short messages (applies only to existing message types);

2.  file naming conventions for existing message types (existing AHL);

3.  General filenaming conventions;

4.  file renaming;

5.  use of directories;

6.  account names and passwords;

7.  FTP sessions;

8.  Local FTP requirements;

9.  File compression.

Accumulating messages into files

One of the problems with using FTP to send traditional GTS messages is the overhead if each message is sent in a separate file. To overcome this problem, multiple messages in the standard GTS message envelope should be placed in the same file according to the rules set out below. This method of accumulating multiple messages applies only to messages for which AHLs have been assigned.

Centres have the option of including or deleting the Starting Line and End of Message strings and indicating which option they are using via the format identifier (refer points 2 and 4 below).
1.  Each message should be preceded by an 8 octet message length field (8 ASCII characters). The length includes the Starting Line (if present), AHL, text and End of Message (if present).

2.  Each message should start with the currently defined Starting Line and AHL as shown in figure 4.2.

3. Messages should be accumulated in files thus:

(a) length indicator, message 1 (8 characters);

(b) format identifier (2 characters);

(c) message 1;

(d) length indicator, message 2 (8 characters);

(e) format identifier (2 characters);

(f) message 2;

(g) and so on, until the last message; 

(h) If necessary, and subject to bilateral agreement, a 'dummy' message of zero length may be inserted after the last real message, to assist with end of file detection in certain MSS systems. This requirement does not exist in most cases and need only be implemented where necessary, and agreed between centres.

4.  Format identifier (2 ASCII characters) has the following values:-

(a) 00 if Starting Line and End of Message strings present;

(b) 01 if Starting Line and End of Message strings absent (not preferred, to be discontinued).

5.  The sending centre should combine messages in the file for no more than 60 seconds to minimise transmission delays; this limit should be set to a value depending upon the characteristics of the link.
6.  The sending centre should limit the number of messages in a file to a maximum of 100; this limit should be set to a value depending upon the characteristics of the link.
7.  The format applies regardless of the number of messages, i.e. it applies even if there is only one message in the file.


[image: image12.wmf]Message 2

length

(8 chacs)

Message  1

Format

Identifier

nnn

length                       SOH CR

CR LF  or

CR

CR LF Heading Text

CR CRLF ETX

 (8 characters)          

 00

nnnnn

Message length

 Starting Line and End of Message present

Message length : Length from SOH to ETX  (e.g. 00001826 = 1826bytes)



[image: image13.wmf]Message 2

 length

(8 characters)

Message 1

 length

(8 characters)

Format

Identifier

     01

Format

Identifier

     01

CR CR LF  Heading      Text

Message length

Option 

(not prefered, to be discontinued)

: Starting Line and End of Message absent

Message length : Length from first CR to end of Text (e.g. 00001826 = 1826bytes)

Figure 4.2 Structure of a typical message in a file

File naming conventions for existing message types (existing AHL)

The file naming convention is:

CCCCNNNNNNNN.ext

where:


CCCC is the international four letter location identifier of the sending Centre, as defined in WMO publication No. 9, Volume C;

NNNNNNNN is a sequential number from 1 to 99999999 generated by the sending Centre for each data type determined by ext; 0 is used for (re-) initialisation; Through bilateral agreement, Centres may use NNNN instead of NNNNNNNN in case of limitation on filename length.

ext is

‘ua’ for urgent alpha numeric information

‘ub’ for urgent binary information

‘a’ for normal alpha numeric information

‘b’ for normal binary information

‘f’ for facsimile information

Note: Where, through bilateral agreement, Centres allow alphanumeric and binary data in the one file, the b or ub extent shall be used.

General file naming conventions

The following file naming convention should be implemented with a transition period not exceeding 2008.  The implementation date is subject to review by CBS.

The procedure is based on transmission of file pairs, one file being the information file and the other being the associated metadata file. The concept of file pairs allows the communications function to be implemented independently of data management requirements for structure of metadata, yet provides for the carriage of whatever metadata is required. It is not compulsory to always have a .met file, such as when the information file itself is self-specifying or when a single .met file can describe several information files (for example as in the case of same data type for different times).  There is always however a clear relation between the Information File Name and the Metadata File Name, which should only differ from their Extension field and possible wildcards.  File names for new message types (no existing AHL) shall follow the following format.  It should be noted that file names for existing message types (existing AHL) can also follow the following format.

The File Name format is a predetermined combination of fields, delimited by the _ (underscore) character except for the last 2 fields, which are delimited by the . (period) character.

Each field can be of variable length, except for the Date/time stamp field which is predetermined.

The order of the fields is mandatory.

The File Name fields are as follows: 

	pflag_productidentifier_oflag_originator_yyyyMMddhhmmss[_freeformat].type[.compression]


where the mandatory fields are:

pflag
is a character or combination of characters indicating how to decode the productidentifier field.  At this time, the pflag field has only the following acceptable value:

Table 4.1 Accepted pflag values

	pflag
	Meaning

	T
	The productidentifier field will be decoded as a standard T1T2A1A2ii data designator (The WMO standard data designators are given in Attachment II-5)

	A
	The productidentifier field will be decoded as a standard Abbreviated Heading, including BBB as appropriate, space characters being discarded, e.g. T1T2A1A2iiCCCCYYGGgg[BBB]

	W
	Planned WMO Product Identifier

	Z
	Originating centre’s local product identifier


productidentifier  is a variable length field containing information that describes the nature of the data in the file.  The productidentifier field should be decoded according to the pflag.

The WMO Product Identifier to be used with pflag = W shall be decoded as follows:

<location indicator>,<data designator>,<free description>,<International date-timegroup>,<BBB

modification header>

The WMO Product Identifier is composed of two parts: the “static part” for description of

the product and the “optional part” to define the time stamp and status of the product (correction,

amendment).

The WMO Product Identifier is not case sensitive. These two parts are defined as follows:

Static part: <location indicator>,<data designator>,<free description>

<location indicator> defines the producer: Country, organization and the production centre;

The country shall be represented by the official ISO 3166 standard 2 letter code. Example:

<gb-metoffice-exeter>. Each field shall be separated by “-” symbol.

The ISO 3166 standard 2-letter code xx shall be used for international organizations and shall

therefore be the two first characters of the location indicator of international organizations, e.g.

“xx-eumetsat-darmstadt”, “xx-ecmwf-reading”.

<data designator> specifies the type of data with reference to the categories and subcategories defined

in the Common Table C-13 of the Manual on Codes, e.g. <SYNOP>, <TAF>, <MODEL>, <RADAR>,

<SATELLITE>, etc. When the type of data is a composite type, use the sign ”+” for concatenation.

<free description> is determined by the production centre to characterize the product.

Optional part: [,<International date-time group>,<BBB modification header>]

<International date-time group> is a YYYYMMDDHHMMSS time stamp of the product, full format

without substitution characters (only decimal digits). This field is optional because it can be

recovered from the file name field: yyyyMMddhhmmss

<BBB modification header> is a complementary group with a similar purpose as the current BBB

group of AHL

Note: In order to facilitate the identification of each field of the product identifier, the static part, as well as the optional part if used, shall comprise two symbols “,” separating the fields. Each field shall not contain any symbol “,”. If a field is empty, no character shall be inserted between the relevant field delimiters “_” or “,”.

oflag
is a character or combination of characters indicating how to decode the originator field.  At this time, the oflag field has only the following acceptable value:

Table 4.2 Accepted oflag values

	oflag
	Meaning

	C
	The originator field will be decoded as a standard CCCC country code


originator is a variable length field containing information that states where the file originated from.  The originator field should be decoded according to the oflag

yyyyMMddhhmmss is a fixed length date and time stamp field.  The interpretation of this field should be in accordance with the standard rules set for specific data description and types.  Therefore it may have various significance such as date of creation or the file, or date of collection of data. If a particular date and time stamp field is not specified, it should be replaced by a `-` (minus) character.  For example:  ------311500-- represents a stamp that specifies only the day (31st), hours (15) and minutes (00). If there are no rules for a specific data type, this field should represent the date and time of creation of the file by the originator.

Type is a variable length field that describes the general format type of the file.  Although this information could be considered somewhat redundant to the productidentifier field, it is kept as such for industry accepted standard compatibility.  It should be noted that the delimiter before the type field is a . (period).  This is to help parse the file name for fields, since the freeformat field could make use of further _ (underscore) to delimit subfields.
Table 4.3 Accepted type values

	type
	Meaning

	met
	The file is a metadata file pair which describes the content and format of the corresponding information file with the same name

	tif
	TIFF file

	gif
	GIF file

	png
	PNG file

	ps
	Postscript file

	mpg
	MPEG file

	jpg
	JPEG file

	txt
	text file

	htm
	HTML file

	bin
	a file containing data encoded in a WMO binary code form such as GRIB or BUFR

	doc
	a Microsoft Word file

	wpd
	a Corel WordPerfect file

	Hdf
	HDF file

	Nc
	NetCDF file

	Pdf
	Portable Document Format file


And the non mandatory fields are:

(a)
freeformat is a variable length field containing further descriptors as required by a given originator.  This field can be further divided in sub-fields.  Originating countries should strive to make their freeformat descriptions available to others.

(b)
compression is a field that specifies if the file uses industry standard compression techniques

Table 4.4 Accepted compression values

	compression
	Meaning

	Z
	The file has been compressed using the Unix COMPRESS technique

	zip
	The file has been compressed using the PKWare zip technique

	gz
	The file has been compressed using the Unix gzip technique

	bz2
	The file has been compressed using the Unix bzip2 technique


Maximum file name length:  Although no maximum length is specified for the entire file name, the mandatory fields shall not exceed 63 characters (including all delimiters) to allow processing by all international systems.

Character set:  The filenames shall be composed of any combination of the standard character set (ITU-T Rec. X.4) with the exceptions noted in Table 4.5.  Case insensitivity shall be used as it is widely accepted and implemented in the industry (for example email addresses and URLs). However, it is recommended to use the “canonical form” of file names when files are being processed in a system. In this manner it would be expected that:

(a) File names be saved in their original form as received (with any combination of upper-lower case characters or any character set);

(b) Files would be saved with lower case characters only for internal processing, comparison, name searches, etc.;

(c) Files would be retransmitted with the original saved name to preserve character set and the upper lower case differences.

This keeps the benefits of readability of upper lower case throughout the systems, but provides

case independence for processing and reference.
Table 4.5 Symbols for filenames

	Symbol
	Allowed
	Meaning

	_
	yes
	The underscore symbol is used has a delimiter symbol.  To be used only as a delimiter of fields.  The underscore is also accepted in the freeformat field, but not in other fields.

	-
	Yes
	The minus symbol shall be used only as a field delimiter inside the “location indicator” and “free description” fields of the WMO Product Identifier in the productidentifier field. For example, in the case of location indicator: gb-metoffice-exeter. This symbol shall not appear in the “data designator” field.

	+
	Yes
	The plus symbol shall be used to concatenate several words in a field of the WMO Product Identifier in the productidentifier field. For example, in the “data designator” field: TEMP+MOBIL or CLIMAT +TEMP+SHIP.

	.
	yes
	The period symbol is used has a delimiter symbol.  To be used only before the type and compression fields.

	/
	no
	Forward stroke often has special meaning for the full path specification of a filename in some operating systems

	\
	no
	Backward stroke often has special meaning for the full path specification of a filename in some operating systems

	>
	no
	Greater than symbol shall not be used since it often represents special file manipulation in some operating systems

	<
	no
	Less than symbol shall not be used since it often represents special file manipulation in some operating systems

	|
	no
	Vertical bar (pipe) symbol shall not be used since it often represents special file manipulation in some operating systems

	?
	no
	Question mark symbol shall not be used

	‘
	no
	Single quote shall not be used.

	“
	no
	double quotes shall not be used

	*
	no
	The star symbol is often used for wildcard specification in procedures that process filenames.

	Space
	no
	The space symbol shall not be used

	,
	yes
	The comma symbol shall be used as a field delimiter in the WMO

Product Identifier of the productidentifier field. For example, in the

static part: <location indicator>,<data designator>,<free description>.

The comma symbol can be also used in the freeformat field.

	A-Z a-z 0-9
	yes
	


The structure of the ‘.met’ file, related to the WMO Metadata standard, is not defined in this guide. 

Examples

· A possible imagery file (Sig Weather Chart) that would have originated from the United States:

T_PGBE07_C_KWBC_20020610180000_D241_SIG_WEATHER_250-600_VT_06Z.tif

· A possible model output file from France:

A_HPWZ89LFPW131200RRA_C_LFPW_20020913160300.bin

· A possible synoptic surface observations file from France:

W_fr-meteofrance-

Toulouse,SYNOP,MAIN+HOURS,,RRA_C_LFPW_20060913030000.txt

· A possible model output file from France:

W_fr-meteofrance-toulouse,GRIB,ARPEGE-75N10N-

60W65E_C_LFPW_200610000000.bin

· A possible image from Australia:

Z_IDN60000_C_AMMC_20020617000000.gif

Note that this shows that the date and time stamp is to be interpreted to be 00 hours,

00 minutes and 00 seconds.

· A possible compressed TOVS satellite data file from the United Kingdom:

Z_LWDA_C_EGRR_20020617000000_LWDA16_0000.BIN.Z

· A possible image (radar) from Canada:

T_SDCN50_C_CWAO_200204201530--_WKR_ECHOTOP,2-0,100M,AGL,78,N.gif

· A possible single-record GRIB file from Canada:

Z__C_CWAO_2002032812----_CMC_reg_TMP_ISBL_500_ps60km_2002032812_P036.bin

· A possible multiple record batch file from China:

Z_SM_C_BABJ_20020520101502.TXT 
File renaming

The method used by receiving centres to detect the presence of a new file may depend on the type of machine used. However most centres will do this by scanning a directory for new files.

To avoid problems with the receiving centre processing a file before it has completely arrived, all sending centres must remotely rename the files they send.

The file shall be sent with the added extent ‘.tmp’ and then renamed to the appropriate extent defined above when the transfer is completed. e.g.

(a)
put xxxxx RJTD00220401.a.tmp  
(xxxxx = local file name)

rename RJTD00220401.a.tmp  RJTD00220401.a

(b)
put xxxxx AMMC09871234.ub.tmp


rename AMMC09871234.ub.tmp AMMC09871234.ub

Use of directories

Some receiving centres may wish the files to be placed in specific sub-directories. This should be limited to require only that all files of the same type be delivered to the same directory. It is recommended that a separate directory be used for each host system which is initiating FTP sessions to avoid the possibility of filename duplication.

Account names and passwords

Using FTP the sender "logs in" to a remote machine using a specific account name and password. The receiving centre defines the account name and the password. There are potential security implications for centres so care needs to be taken.

The following general rules should however apply.

1.
The receiving centre defines the user account and password for the sending centre.

2. 
Anonymous FTP may be used or a specific account may be created. (If anonymous FTP is used, each sending Centre must have its own sub directory on the FTP server).

FTP Sessions

To limit the load on both the sending and receiving systems, no more than one FTP session per file type should exist at the same time. If for example, Centre A wishes to send two files to Centre B of the same type (say .ua), the second file must not be sent until the first is finished. Centres should limit the number of concurrent sessions with a particular Centre to five maximum.

The idle timer for closing the FTP session should be set to a value between the cut-off time for accumulating messages (max. 60 seconds) and a maximum of 3 minutes.

Local FTP requirements

All sending centres will need to allow for additional "static" FTP commands to be included in the FTP commands that they issue. For example some MVS centres may require the inclusion of "SITE" commands to define record and block lengths. Centres should support FTP commands as specified in RFC 959 unless some are excluded by bilateral agreement. There may also need to be bilaterally agreed procedures and commands.

It is the responsibility of receiving Centres to delete files after they have been processed.

Use of file compression

If large files are to be sent then it is often desirable to compress them first.

Centres should only use compression by bilateral agreement. 

Backup with an IP based GTS 

A final consideration is that of MSS backup. The new GTS will use IP addresses, where an individual address is usually associated with only one system. Should a system fail and an alternative be used there are implementation issues to be considered by transmitting centres. Ideally a transmitting centre should be unaffected by a receiving Centre's backup arrangements. This is a good principle, which all Centres should seek to adhere to. However it may not always be possible to achieve complete IP transparency. If this cannot be done sending Centres must be prepared to try an alternate IP address. Once using such an alternate address it must periodically try the primary address. It is suggested that such periodicity be established by bilateral agreement between centres because it will be heavily influenced by each centres backup strategy. 

Trouble shooting and problem resolution

IP Layer Tools 

In a large IP network, every router involved in the path between two hosts must know the next hop to be used to reach the destination address. As every router and/or link might be a point of failure, it is very important to determine rapidly where the problem is, and then how to solve it.

Suggested steps in resolving problems (not necessarily in the order given) are:

(a) check the remote centre (if the security policy of the remote centre allows it).

(b) check if the link to the “outside” network is reachable

(c) check the local network by trying to reach the next/default gateway

(d) check the local IP stack and configuration

Some basic tools that can be used such as Ping, Traceroute and Netstat are described below. PING and TRACEROUTE provide information on paths between hosts. They  both use ICMP  (traceroute also need UDP), but it should be noted that many sites block ICMP packets as part of their firewall security measures. To be able to locate problems in a network, it is necessary to have an exact documentation of the network.

PING

PING will check if the destination IP address can be reached. This tool is standard in almost every operating system with TCP/IP. On a Unix host the output looks like:

zinder# ping -s cadillac

PING cadillac : 56 data bytes

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=0. time=3. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=1. time=2. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=2. time=3. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=3. time=3. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=4. time=5. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=5. time=3. ms

64 bytes from cadillac ( 193.168.1.17 ) : icmp_seq=6. time=3. ms

----cadillac PING statistics----

7 packets transmitted, 7 packets received, 0% packet loss

round-trip (ms) min/avg/max = 2/3/5

A useful test could be to ping the MSS of the neighbouring Centre. If this ping succeeds with an acceptable time delay, it would indicate that the network is operating correctly. If the ping fails, it could mean that the circuit is down or the ICMP ping packets are being blocked by the neighbouring Centre's router or firewall. In this event, it could be useful to ping the serial interface of the neighbouring Centre's router. If this succeeds, then the communications link to the neighbouring Centre is working. Any malfunction would then be within the neighbouring Centre.

Ping can be used to check whether the network performance is reasonable. The time is the delay between sending and receiving back the packet. It is not really possible to give an average value of the delay, but it is more important to notice any variation.

Finally, it might happen that packets are lost. In this case, there are missing numbers in the icmp_seq number. Either packet loss or variation in delays will badly degrade the performance.

TRACEROUTE

This tool is used to show which routers are transited on the network between A and B. As said above, traceroute need UDP and ICMP packets to work. Firewalls or packet filter on router may block such traffic as part of local security policy. It is not available on all systems, but is rather easy to compile. It is a free tool available on the Internet.

Traceroute output looks like:

cadillac 22: traceroute ftp.inria.fr

traceroute to ftp.inria.fr (192.93.2.54), 30 hops max, 40 byte packets

 1  antonio.meteo.fr (137.129.1.5)  3 ms  2 ms  2 ms

 2  clara.meteo.fr (137.129.14.249)  1 ms  2 ms  2 ms

 3  andrea.meteo.fr (193.105.190.253)  4 ms  3 ms  2 ms

 4  octares1.octares.ft.net (193.48.63.5)  30 ms  35 ms  10 ms

 5  192.70.80.97 (192.70.80.97)  9 ms  15 ms  27 ms

 6  stamand1.renater.ft.net (195.220.180.21)  40 ms  96 ms  29 ms

 7  stamand3.renater.ft.net (195.220.180.41)  56 ms  100 ms  108 ms

 8  stlambert.rerif.ft.net (195.220.180.10)  63 ms  56 ms  34 ms

 9  193.55.250.34 (193.55.250.34)  46 ms  28 ms  26 ms

10  rocq-gwr.inria.fr (192.93.122.2)  21 ms  147 ms  85 ms

11  ftp.inria.fr (192.93.2.54)  86 ms  58 ms  128 ms

When a router does not know where to send the packet, the result may be like the following:

cadillac 22: traceroute 193.105.178.5

traceroute to 193.105.178.5 (193.105.178.5), 30 hops max, 40 byte packets

 1  antonio.meteo.fr (137.129.1.5)  2 ms  1 ms  1 ms

 2  clara.meteo.fr (137.129.14.249)  1 ms  4 ms  1 ms

 3  andrea.meteo.fr (193.105.190.253)  4 ms  11 ms  4 ms

 4  octares1.octares.ft.net (193.48.63.5)  42 ms  39 ms  42 ms

 5  192.70.80.97 (192.70.80.97)  8 ms  7 ms  7 ms

 6  stamand1.renater.ft.net (195.220.180.5)  48 ms  86 ms  113 ms

 7  rbs1.renater.ft.net (195.220.180.50)  63 ms  107 ms  154 ms

 8  Paris-EBS2.Ebone.net (192.121.156.105)  146 ms  167 ms  140 ms

 9  stockholm-ebs-s5-2.ebone.net (192.121.154.21)  100 ms  80 ms  92 ms

10  Amsterdam-ebs.Ebone.NET (192.121.155.13)  249 ms  227 ms  205 ms

11  amsterdam1.NL.EU.net (193.0.15.131)  257 ms  249 ms  316 ms

12  * Amsterdam5.NL.EU.net (134.222.228.81)  300 ms  297 ms

13  Amsterdam6.NL.EU.net (134.222.186.6)  359 ms  218 ms  304 ms

14  Paris1.FR.EU.net (134.222.228.50)  308 ms  311 ms  388 ms

15  * Etoile0.FR.EU.net (134.222.30.2)  177 ms *

16  Etoile0.FR.EU.net (134.222.30.2)  * * *

In the second case, cadillac would not be able to reach 193.105.178.5 because the router Etoile0.fr.eu.net failed to send the packet. With traceroute, it is not possible to know if it is a router failure or a link failure.

NETSTAT

This is a command available on most computing platforms. It gives information about the set up of the host's IP stack.

Netstat can be used to find out if the local IP address and subnet mask are configured correctly as well as if the routing information is still correct. There are many other options but is it not the intention of this guide to describe them all.

A sample output looks like:

$ netstat -rn

Routing tables

Internet:

Destination 
     Gateway         
   Netmask       
 Flags
Refs
Use  
   
Interface

default       
      141.38.48.2               
              
 UG 
12
4014211  
ec0

127.0.0.1        
      127.0.0.1                           
              UH      
9        
2321       
lo0

141.38.48     
      141.38.48.12     
  0xffffff00    
 U         3         
68981     
ec0

141.38.48.12  
      127.0.0.1                             

 UGH   
10        
253410   
lo0

195.37.164.100
      141.38.48.5
        


 UGH  
2
345
  
lo0

224                          141.38.48.12             0xf0000000       
 U         1         
19848     
ec0

$

The output shows that this particular host has the IP address 141.38.48.12 with a subnet mask of 24 bit (0Xffffff00 or 255.255.255.0). It also shows that the host 195.37.164.100 can be reached via the gateway 141.38.48.5, and the flags indicate that the route is up (U), that it is a route to a gateway (G) and that it is a host route (H). The first line indicates that all other destinations are reachable via the hosts default gateway 141.38.48.2.

In the next output:

$ netstat -rn

Routing tables

Internet:

Destination 
     Gateway       
   Netmask       
 Flags   
 Refs    
Use           Interface

default       
      141.38.48.2               
              UG 
  12      
4014211  
ec0

127.0.0.1        
      127.0.0.1                           
 UH        9          2321       
lo0

141.38.48     
      141.38.48.12     0xffffff00    
 U           3          68981     
ec0

141.38.48.12  
      127.0.0.1               
              UGH     10          253410   
lo0

195.37.164.100
      141.38.48.2
        

 UGHM  2
345
  
lo0

224                          141.38.48.12      0xf0000000      U            1         19848     
ec0

$

The only difference to the first sample output is, that the host route to 195.37.164.100 is now flagged with a M, which means that this route was modified by an ICMP redirect message from the old gateway 141.38.48.5. This usually means that the router with the IP address 141.38.48.5 has lost it’s route to  195.37.164.100 and may indicate a problem with the link to the remote network.

Other monitoring tools

Verifying correct IP connectivity is a necessary first step. Other tools can be used to provide more information on what is happening. There are many options. It is possible to use protocol analysers and SNMP based software tools. For example, Sun Microsystems bundles with Solaris a tool called snoop who can replace in most cases a local area network analyser. Others tools such as TCPDUMP are available free on the Internet and can be installed on various systems. TCPDUMP is often bundled in various Linux distributions. These tools require a rather good knowledge of IP protocol. But, for example, TCPDUMP might be used to diagnose application level problems. 

The following is a simple example on the host 'pontiac', of the capture of ICMP exchanges between zinder and cadillac. 

pontiac# /usr/local/bin/tcpdump -i nf0 host cadillac and zinder and proto icmp

15:28:06.68 cadillac.meteo.fr > zinder.meteo.fr: icmp: echo request

15:28:06.68 zinder.meteo.fr > cadillac.meteo.fr: icmp: echo reply

15:28:19.45 cadillac.meteo.fr > zinder.meteo.fr: icmp: echo request

15:28:19.45 zinder.meteo.fr > cadillac.meteo.fr: icmp: echo reply

15:28:29.44 cadillac.meteo.fr > zinder.meteo.fr: icmp: echo request

15:28:29.45 zinder.meteo.fr > cadillac.meteo.fr: icmp: echo reply

SNMP

Simple Network Management protocol was developed in the late 80’s in order to offer to network manager a standard tool for controlling networks. In most case SNMP could be used to replace more crude tools describe above. Unfortunately, good SNMP software is not cheap. SNMP is a client-server protocol. In order to be able to gather information with SNMP, the equipment connected on the network must have Management Information Base (MIB). These bases are catalogues of integer, counters, strings, etc.... The manager asks the agents to send it some values. These values might be for example, IP routing table. The example below is obtained by requesting with HP Open View (a commercial package) the routeing table on the host monica.meteo.fr.

Title:    : monica.meteo.fr

Name or IP Address: monica.meteo.fr

ipRouteDest      ipRouteMask      ipRouteNextHop   ipRouteProto  ipRouteMetric1

0.0.0.0              0.0.0.0               137.129.1.5        local                      0

136.156.0.0      255.255.0.0        137.129.1.5        ciscoIgrp               8786

137.129.1.0      255.255.255.0    137.129.1.6        local                      0

137.129.2.0      255.255.255.0    137.129.1.5        ciscoIgrp               1110

137.129.3.0      255.255.255.0    137.129.3.254    local                      0

137.129.4.0      255.255.255.0    137.129.4.254    local                      0

137.129.5.0      255.255.255.0    137.129.5.254    local                      0

137.129.6.0      255.255.255.0    137.129.1.62      local                      0

137.129.7.0      255.255.255.0    137.129.7.254    local                      0

137.129.8.0      255.255.255.0    137.129.8.254    local                      0

137.129.9.0      255.255.255.0    137.129.1.5        ciscoIgrp               1110

Information given above with TCPDUMP might be obtained with SNMP but to do so, probes running the Remote Monitoring MIB must be connected on the network.

On a bilateral basis, it might be useful for Centres to allow SNMP access to their router from the other NMC.  However, regular polling of other Centres’ routers should be avoided to avoid overloading of circuits.

MRTG

Another public domain package, called MRTG, is a very helpful tool to gather information about  the local network and about connected links. The Multi Router Traffic Grapher (MRTG) is a tool to monitor the traffic load on networks and links. It generates HTML pages containing images which provide a live visual representation of this traffic. It can also be implemented to indicate failures of  network links. MRTG consists of a Perl script which uses SNMP to read the traffic counters of your router(s) and a fast C program which logs the traffic data and creates graphs representing the traffic on the monitored network connection(s). Below is a sample output. It shows traffic statistics for a dedicated link and gives information about the traffic pattern on the link. This is just one of many other graphs one can create with MRTG. 

SYSLOG

Many of the possible problems can be located if one not only looks at the SYSLOG files on the hosts, but uses a SYSLOG server as well and lets the router(s) send their messages to it. This file can then be checked regularly e.g. for messages that indicate high CPU load, processes that use up much memory or CPU cycles, lines going up and down, and messages about events regarding the used routing protocol.

There are 8 different levels of messages the router will log to the syslog server.

They are:

Emergencies

0

System unusable

Alerts


1

Immediate action needed

Critical


2

Critical conditions

Errors


3

Error conditions

Warnings

4

Warning conditions

Notifications

5

Normal but significant condition

Informational

6

Informational messages only

Debugging

7

Debugging messages

The default logging facility on a cisco router is set to local7, this is important to know when configuring a host to be a syslog server and will be explained there.

The configuration commands on a Cisco router to activate logging are:

cisco-gts-1(config)#logging trap level-of-messages-to-log

cisco-gts-1(config)#logging 141.38.48.12

and can be checked with the command “show logging”:

cisco-gts-1#sho logging

Syslog logging: enabled (0 messages dropped, 0 flushes, 0 overruns)

Console logging: level debugging, 117892 messages logged

Monitor logging: level debugging, 8317 messages logged

Trap logging: level debugging, 117150 message lines logged

Logging to 141.38.48.12, 117150 message lines logged

Buffer logging: disabled

cisco-gts-1#

In this example, logging is set to the level debugging (“logging trap debugging”), and all messages from level 7 up to level 0 will be sent to the syslog server with the IP address 141.38.48.12.

To activate the SYSLOG  server on for instance a SGI UNIX machine, the following entries should be there:

In the file /etc/services: syslog          514/udp

In the file /etc/syslog.conf: local7.debug            /usr/people/cisco/logs/cisco.log

The local7.debug relates to the default facility of logging that is defined on a cisco router as mentioned (local7). The file above will be the file to which the syslog daemon writes all incoming syslog messages for local7.

The last action on the host is to have the syslog daemon reread it’s config file (kill –1 pid-of-syslogd).

Bandwidth Management

On an IP network, all packets will be routed over the links without any prioritisation mechanism. Therefore an FTP transfer can occupy all the bandwidth available starving all others applications. When traffic increases, it might therefore be needed to introduce some bandwidth management in the network configuration. Further information may be available on the online reference (http://www.wmo.ch/).

Appendix 1 - High Level TCP/IP Topology and TCP/IP Data Flows

The following diagrams show a high level view of the topology of a simple Center and the main data flows regarding GTS and Internet telecommunication.  More detailed X.25 over IP configurations can be found in the following appendices.
Figure A1.1 – General interconnectivity between Centres

[image: image14.wmf]BROADCAST

NETWORK

GTS

INTERNET

CENTER B

 OTHER NON

-

GTS LEGACY AND BACKUP LINKS 

CENTER A


Figure A1.2 – Topology of TCP/IP network in a simple Centre

[image: image15.wmf]CENTER A

PUBLIC 

SUBNET

GTS

INTERNET

WORKSTATION 

1

WORKSTATION 

2

WAFS RECEIVER

DIGITAL VIDEO 

BROADCAST  

RECEIVER

INTERNAL 

ROUTER 

/ 

FIREWALL

MESSAGE 

SWITCHING SERVER 

1

MESSAGE 

SWITCHING SERVER 

2

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

VPN 

INTERFACE

DMZ 

SUBNET

INTERNAL 

PROTECTED 

SUBNET

OTHER SYSTEMS

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

FIREWALL

LINK PROVIDED BY 

TELECOM SUPPLIER

LINK PROVIDED BY 

INTERNET SUPPLIER

WEB PORTAL 

/ 

SERVER 

1

WEB PORTAL 

/ 

SERVER 

2

FIREWALLS BLOCK 

ALL TRAFFIC IN 

BOTH DIRECTIONS BY 

DEFAULT

, 

ALLOWS 

ONLY KNOWN 

TRAFFIC


Figure A1.3 – Data flow of traffic over the GTS – IP only

[image: image16.wmf]CENTER A

PUBLIC 

SUBNET

GTS

INTERNET

WORKSTATION 

1

WORKSTATION 

2

WAFS RECEIVER

DIGITAL VIDEO 

BROADCAST  

RECEIVER

INTERNAL 

ROUTER 

/ 

FIREWALL

MESSAGE 

SWITCHING SERVER 

1

MESSAGE 

SWITCHING SERVER 

2

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

VPN 

INTERFACE

DMZ 

SUBNET

INTERNAL 

PROTECTED 

SUBNET

OTHER SYSTEMS

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

FIREWALL

LINK PROVIDED BY 

TELECOM SUPPLIER

LINK PROVIDED BY 

INTERNET SUPPLIER

WEB PORTAL 

/ 

SERVER 

1

WEB PORTAL 

/ 

SERVER 

2

TYPICAL GTS CONNECTION


Figure A1.4 – Data flow of traffic over the GTS – X.25 over IP

[image: image17.wmf]CENTER A

PUBLIC 

SUBNET

GTS

INTERNET

WORKSTATION 

1

WORKSTATION 

2

WAFS RECEIVER

DIGITAL VIDEO 

BROADCAST  

RECEIVER

INTERNAL 

ROUTER 

/ 

FIREWALL

MESSAGE 

SWITCHING 

SERVER 

1

MESSAGE 

SWITCHING SERVER 

2

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

VPN 

INTERFACE

DMZ 

SUBNET

INTERNAL 

PROTECTED 

SUBNET

OTHER SYSTEMS

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

FIREWALL

LINK PROVIDED BY 

TELECOM SUPPLIER

LINK PROVIDED BY 

INTERNET SUPPLIER

WEB PORTAL 

/ 

SERVER 

1

WEB PORTAL 

/ 

SERVER 

2

X

.

25 

ENCAPSULATING 

ROUTER

TYPICAL GTS CONNECTION WITH X

.

25 

over IP 

(

also requires an X

.

25 

capable MSS at other Center

)

SERIAL LINK 

WITH X

.

25


Figure A1.5 – Data flow of traffic using VPN over the Internet

[image: image18.wmf]CENTER A

PUBLIC 

SUBNET

GTS

INTERNET

WORKSTATION 

1

WORKSTATION 

2

WAFS RECEIVER

DIGITAL VIDEO 

BROADCAST  

RECEIVER

INTERNAL 

ROUTER 

/ 

FIREWALL

MESSAGE 

SWITCHING SERVER 

1

MESSAGE 

SWITCHING SERVER 

2

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

VPN 

INTERFACE

DMZ 

SUBNET

INTERNAL 

PROTECTED 

SUBNET

OTHER SYSTEMS

ACCESS DEVICE 

ROUTER 

/ 

FIREWALL

FIREWALL

LINK PROVIDED BY 

TELECOM SUPPLIER

LINK PROVIDED BY 

INTERNET SUPPLIER

WEB PORTAL 

/ 

SERVER 

1

WEB PORTAL 

/ 

SERVER 

2

TYPICAL VPN OVER INTERNET CONNECTION

AND


APPENDIX 2 - Cisco Router Configurations

This appendix is not intended to be a complete description of all available commands in a Cisco, nor a full course on this equipment, but it is useful to describe more precisely the configuration tasks in order to comply with the policy outlined in Chapter 2.

The configuration described below respects what is available in release 11.1 of Cisco IOS software. Some features are not available in previous releases, and some will be modified in the future.

We are going to describe different steps:


1. Establishing IP connection

· IP over PPP

· IP over X25

· X25 over IP (in fact it is X25 over TCP, the XOT protocol)


2. Routing configuration

· leaf node with static routes (Centre A)

· leaf node with dynamic routing (Centre C)

· configuration in a non-leaf node (in our case two different GTS connections, Centre B)


3. Security configuration

· filtering traffic based on declared IP addresses

· controlling routing exchanges between GTS and the Internet

In our example A is connected to B with IP over X25 link, B is connected to C with IP over PPP. There is also the option for the MSSs at B and C to communicate using X.25 over TCP/IP. A is a leaf node, B and C are non-leaf nodes. B and C are also connected to the Internet. B and its Internet provider use static routes
, C and its Internet provider use RIP
. 


[image: image19.wmf] Host

MSS

Router

 Host

MSS

Router

 Host

MSS

Router

CENTRE A

CENTRE C

CENTRE B

IP address:

Ai

X.121: Ax

X.121: Ax

’

IP address: A

IP address: C

IP address: B

Packet

Switch

Packet

Switch

IP address:

Bi

X.121:

Bx

X.121:

Bx

’

IP address: B

’

IP address: C

’

IP address:

Bc

IP address:

Cb

Internet

Internet

Packet

Switch

X.121: Cx

S

0

S

1

S

1

S

0

S

0


The following will be used along this appendix:

	
	X121 router address
	IP router address
	IP hosts address for GTS
	Autonomous-System

	Centre A
	01016661166666
	193.105.177.1
	194.168.1.16/255.255.255.248
	N.A.

	Centre B
	01017771177777
	193.105.177.2

193.105.178.5
	137.129.9.0/255.255.255.0
	65001

	Centre C
	01018881188888
	193.105.178.6
	195.1.1.0/255.255.255.0
	65200


Centres A and B use serial interface 0 to connect to the packet switches. Centres B and C use serial interfaces 1 for the PPP link.

Step 1: Establishing connections

Centre A:


interface serial 0


 encapsulation X25

 ! depending on local set-up (virtual channels, 

 ! windows... ) extra configuration might be necessary

 x25 address 01016661166666


 ip address 193.105.177.1 255.255.255.0


!

x25 map ip 193.105.177.2 01017771177777

Centre B:


interface serial 0


 encapsulation X25


 x25 address 01017771177777

 ! depending on local set-up (virtual channels, 

 ! windows... ) extra configuration might be necessary

 ip address 193.105.177.2 255.255.255.0


!

x25 map ip 193.105.177.1 01016661166666


!


interface serial 1


 encapsulation PPP


 ip address 193.105.178.5 255.255.255.252


!

! X25 over TCP commands

x25 routing


 x25 route 010188811* ip 193.105.178.6


 x25 route 010177711* interface serial 0

Centre C:


interface serial 0


 encapsulation X25


 x25 address 01018881188888


 ! depending on local set-up (virtual channels, 

 ! windows... ) extra configuration might be necessary

interface serial 1


 encapsulation PPP


 ip address 193.105.178.6 255.255.255.252

!

! X25 over TCP commands


x25 routing


 x25 route 010177711* ip 193.105.178.5


 x25 route 010188811* interface serial 0

After this first step, IP configuration between the routers is complete. The router in Centre A can then ping router in B. B can ping A and C, but A and C can’t communicate because there is no routing set up.

MSS at B and C can communicate with IP (once end-to-end routing is established) or with X25 over TCP. Experience has shown that all the X25 parameters on router in centres B and C MUST be the same (packet size, window size) to avoid any strange behaviour.

Step 2: Routing

Centre A:


! Simply define a default route with a metric 10 (the 

! price) via B

ip route 0.0.0.0 255.255.255.255 193.105.177.2 10

Centre B:

! First define static route with A



ip route 194.168.1.16 255.255.255.248 193.105.177.1 10


ip route 0.0.0.0 ip_provider_address 10


! BGP routing 


router bgp 65001


 network 137.129.9.0 mask 255.255.255.0


 neighbour 193.105.178.6 remote-as 65200


 ! Route to A is static, force sending to C


 redistribute static

Centre C:


! BGP routing 

router bgp 65200


 network 195.1.1.0


 neighbour 193.105.178.5 remote-as 65001

! 196.1.1.0 is network address for non-GTS hosts in C

router rip

 version 2

 network 195.1.1.0

 no auto-summary

A defines a default route. So, when A wants to communicate with C, the router knows where to send packets. C is going to receive routing information from B, so A is also reachable from C. It is also important to note that if A tries to reach an Internet site, attempts will be made through B’s Internet connection. It will fail because the Internet site A tries to reach can not return packets to A (only B’s address is reachable on the internet via B’s Internet connection). The link A to B link will thus carry some inappropriate data. Also note that we use RIP version 2 .  

Step 3: Security
Centre A:

! Declare which hosts can use GTS

access-list 1 permit 194.168.1.16 0.0.0.7

! Declare which hosts can come from GTS

access-list 2 permit 195.1.1.0 0.0.0.255

access-list 2 permit 137.129.9.0 0.0.0.255

!

interface serial 0

 ip access-group 1 out

 ip access-group 2 in

Centre B:

! Declare which hosts can use GTS

access-list 1 permit 137.129.9.0 0.0.0.255

! Declare which hosts can come from GTS

access-list 2 permit 195.1.1.0 0.0.0.255

access-list 2 permit 194.168.1.16 0.0.0.7

! Only accept BGP updates from AS neighbour

ip as-path access-list 3 permit ^$ 

ip as-path access-list 3 permit ^65200

!
interface serial 0

 ip access-group 1 out

 ip access-group 2 in

!

interface serial 1

 ip access-group 1 out

 ip access-group 2 in

! Restrict BGP updates
router bgp 65001

 network 137.129.9.0 mask 255.255.255.0

 neighbour 193.105.178.6 remote-as 65200

 neighbour 193.105.178.6 filter-list 3 in

 neighbour 193.105.178.6 filter-list 3 out

 redistribute static

Centre C:

! Declare which hosts can use GTS

access-list 1 permit 195.1.1.0 0.0.0.255

! Declare which hosts can come from GTS

access-list 2 permit 137.129.9.0 0.0.0.255

access-list 2 permit 194.168.1.16 0.0.0.7

! Only accept BGP updates from AS neighbour

ip as-path access-list 3 permit ^$

ip as-path access-list 3 permit ^65001

!

interface serial 0

 ip access-group 1 out

 ip access-group 2 in

! Restrict BGP updates
router bgp 65200

 network 195.1.1.0 mask 255.255.255.0

 neighbour 193.105.178.5 remote-as 65001

 neighbour 193.105.178.5 filter-list 3 in

 neighbour 193.105.178.5 filter-list 3 out

In these configurations, there are two important features used:

(a) BGP filtering


The access-list 3 in both B and C checks the autonomous system number sent by its neighbour. By filtering in and out in the BGP process this guarantees that all known routes must be issued from one of these ASs.

(b) IP filtering


The access-list 1 list allows IP addresses issued from within each Centre. This list should be quite stable. The access-list 2 checks the incoming IP addresses. As new Centres are added to the IP network, the corresponding addresses must be added to these access-lists.

It must also be noted that despite Internet connections in B and C no extra attention is required to control routing exchange. A static default route is not sent even if « redistribute static » is enabled. RIP and BGP ignore routing information known via the other protocol. 

APPENDIX 3 - Sample Socket Send and Receive Routines

/********************************************************************

* Sample TCP/IP Socket program that SENDS a single message

********************************************************************/

#include <stdio.h>

#include <unistd.h>

#include <stdlib.h>

#include <signal.h>

#include <string.h>

#include  <memory.h>

#include  <sys/socket.h>

#include  <netinet/in.h>

#include  <netdb.h>

/* TCP/IP DESTINATION and SERVICE ARE DEFINED BY THE RECEIVING CENTRE */

#define DESTINATION
"localhost"

#define SERVICE 
39000

#define GTS_LENFIELD 8

#define MAX_MSGSIZE
15000  /*  value of the send buffer size, recommended: 4096 */

static void GetDestinationInfo();

static void SetupSocket();

static void SendData();

static void MakeConnection();

static struct sockaddr_in dest;

static int

pr_sock;

/********************************************************************

*


MAINLINE

* 1. Ignore SIGPIPE signals. These are generated if a connection 

*    is lost. By default they cause a program to terminate.

* 2. Get information about the destination (GetDestinationInfo):

*
- IP number (and name)

*
- Service/Port number

* 3. Create a TCP/IP Socket (SetupSocket)

* 4. Connect to the destination centre (MakeConnection)

* 5. Send the message (SendData)

* 6. Close the socket (shutdown + close)

********************************************************************/

main(int argc, char *argv[])

{

signal (SIGPIPE,SIG_IGN);

GetDestinationInfo();

SetupSocket();

MakeConnection();

SendData();

/* shutdown(pr_sock,1) */

close(pr_sock);

}

/********************************************************************

*


GET DESTINATION INFO

* Store the destination IP number and service number in a socket

* structure (dest).

* 1. Convert the destination name to an IP number (gethostbyname)

* 2. Store the IP number and service number in the "dest" structure.

********************************************************************/

static void GetDestinationInfo()

{

struct hostent *hp;

hp = gethostbyname (DESTINATION);

if ( hp == NULL ) {

   printf("host error\n");

   exit(1);

   }

memset ((char *)&dest, 0, sizeof dest);

memcpy (&dest.sin_addr.s_addr, hp->h_addr, hp->h_length);

dest.sin_family = AF_INET;

dest.sin_port = SERVICE;

}

/********************************************************************

*


SETUP SOCKET

* Setup a TCP/IP Socket

* 1. Create the socket

* 2. Set the socket KEEPALIVE option.

*    This enables the automatic periodic transmission of "check" 

*    messages to be sent on the connection. If the destination 

*    does not respond then it is considered broken and this process

*    is notified (by SIGPIPE or end-of-file)

*3. Set the socket REUSEADDR option. Enable quicker restarting of

*    terminated processes.

*4. Reduce the size of the Socket send buffer to reduce the amount of data lost

*    if the connection fails.

********************************************************************/

static void SetupSocket()

{

int
on = 1;

int
rc;

int
buffsize = MAX_MSGSIZE;

pr_sock = socket (AF_INET, SOCK_STREAM, 0);

if (pr_sock < 0) {

   printf("sock error\n");

   exit(1);

   }

 rc = setsockopt(pr_sock,SOL_SOCKET,SO_KEEPALIVE,(char *)&on,sizeof(on));

 if (rc != 0) {

    printf("keepalive error\n");

     }

 rc = setsockopt(pr_sock,SOL_SOCKET,SO_REUSEADDR,(char *)&on,sizeof(on));

 if (rc != 0) {

    printf("reuse error\n");

     }

rc = setsockopt(pr_sock,SOL_SOCKET,SO_SNDBUF,(char *)&buffsize,sizeof(buffsize));

 if (rc != 0) {

    printf("unable to set send buffer size\n");

     }

}

/********************************************************************

*


MAKE CONNECTION

* Attempt to make a TCP/IP Socket connection to the destination on

* the agreed service/port number.

********************************************************************/

static void MakeConnection()

{

int
length;

length = sizeof (dest);

if ( connect (pr_sock,(struct sockaddr *)&dest,length) == -1 ) {

   printf("connection error\n");

   exit(1);

   }

printf("connected\n");

}

/********************************************************************

*



SEND DATA

* Send a message on the socket (5 times actually).

* 

* NOTE: A real program would check the return code from the write

* and if the write failed it would close the socket, raise an operator

* alarm, and then try to re-send from the start of the message

********************************************************************/

static void SendData()

{

char
msg[MAX_MSGSIZE+1], buffer[MAX_MSGSIZE+GTS_LENFIELD+3];

int
buflen, i, rc = 0;

strcpy(msg,"\001\r\r\n001\r\r\nTTAA01 AMMC 000000\r\r\n");

for (i=0;i<60;i++) 

    strcat(msg,"THE QUICK BROWN FOX JUMPS OVER THE LAZY DOG 0123456789\r\r\n");

strcat(msg,"\r\r\n\003");

sprintf(buffer,"%0*dAN%s",GTS_LENFIELD,strlen(msg),msg);

buflen = strlen(buffer);

for (i=0; i<5; i++) {

   rc = write(pr_sock,buffer,buflen);

   printf("write. rc = %d\n",rc);

   }

}

/********************************************************************

* TEST TCP/IP SOCKET RECEIVING PROGRAM.

* Program is designed to give some ideas as to how to receive GTS

* style messages on a TCP/IP Socket connection.

********************************************************************/

#include <stdio.h>

#include <unistd.h>

#include <stdlib.h>

#include <signal.h>

#include <string.h>

#include  <memory.h>

#include  <sys/socket.h>

#include  <netinet/in.h>

#include  <netdb.h>

#define SERVICE 

39000

#define MAX_MSGSIZE 

15000 

#define MAX_BUFLEN  

MAX_MSGSIZE + 100

#define  SOH     

'\001'

#define  ETX     

'\003'

#define  GTS_LENFIELD  

8

#define  GTS_SOCKET_HEADER  
10

static void SetupService();

static void RecvData();

static void AcceptConnection();

static int  ExtractMsg(char *buffer, int *buflen);

static int  CheckMsgBoundaries (char *, int);

static int  FindMessage (char *, int, int *);

static void ShiftBuffer (char *, int *, int);

static struct sockaddr_in dest;

static int
pr_sock, msgsock;

static char
buffer[MAX_BUFLEN+1];

static int
buflen = 0;

/********************************************************************

*



MAIN

* Listen for incoming IP calls and read any incoming messages on 

* the first call established.

*

* 1. Ignore SIGPIPE signals. These are generated if a connection

*    is lost. By default they cause a program to terminate.

* 2. Set-up a listening socket for incoming msgs (SetupService)

* 3. Accept the first call received (AcceptConnection)

* 4. Read any messages on this connection (RecvData)

* 5. Close the call and close the listening socket.

********************************************************************/

main(int argc, char *argv[])

{

signal (SIGPIPE,SIG_IGN);

SetupService();

AcceptConnection();

RecvData();

close(msgsock);

/* shutdown(pr_sock,1) */

close(pr_sock);

}

/********************************************************************

*


SETUP SERVICE

* Listen for calls on a given Service/Port.

* 1. Create a socket

* 2. Set the socket KEEPALIVE option.

*    This enables the automatic periodic transmission of "check" 

*    messages to be sent on the connection. If the destination 

*    does not respond then it is considered broken and this process

*    is notified (by SIGPIPE or end-of-file)

* 3. Set the socket REUSEADDR option. Enable quicker restarting of

*    terminated processes.

* 4. Bind the socket to the required Service/Port

* 5. Start listening for calls.

********************************************************************/

static void SetupService()

{

int
on = 1;

int
rc;

/* adjust the TCP receive buffer size 

int
buffsize = MAX_MSGSIZE; */

memset ((char *)&dest, 0, sizeof dest);

dest.sin_addr.s_addr = INADDR_ANY;

dest.sin_family = AF_INET;

dest.sin_port = SERVICE;

pr_sock = socket (AF_INET, SOCK_STREAM, 0);

if (pr_sock < 0) {

   printf("sock error\n");

   exit(1);

   }

 rc = setsockopt(pr_sock,SOL_SOCKET,SO_KEEPALIVE,(char *)&on,sizeof(on));

 if (rc != 0) {

    printf("keepalive error\n");

    exit(1);

    }

 rc = setsockopt(pr_sock,SOL_SOCKET,SO_REUSEADDR,(char *)&on,sizeof(on));

 if (rc != 0) {

    printf("reuse error\n");

    exit(1);

    }

/* adjust the TCP receive buffer size

rc = setsockopt(pr_sock,SOL_SOCKET,SO_RCVBUF,(char *)&buffsize,sizeof(buffsize));

 if (rc != 0) {

    printf("unable to set send receive size\n");

    }

*/

rc = bind(pr_sock,(struct sockaddr *)&dest,sizeof dest);

if ( rc < 0) { 

   printf("bind error\n");

   exit(1);

   }

rc = listen(pr_sock,1);

if ( rc < 0) { 

   printf("listen error\n");

   exit(1);

   }

printf("listening\n");

}

/********************************************************************

*


ACCEPT CONNECTION

* Wait for an incoming call (accept).

* Return the socket of the call established.

********************************************************************/

static void AcceptConnection()

{

int
addrlen;

printf("waiting connection\n");

addrlen = sizeof(sockaddr_in);

msgsock = accept (pr_sock,&dest,&addrlen);

if ( msgsock < 0) { 

   printf("accept error\n");

   exit(1);

   }

printf("connected\n");

}

/********************************************************************

*


RECV DATA

* Read data from the message/call socket.

* Extract GTS messages from this data.

* Keep reading until the sender drops the call or there is an error.

********************************************************************/

static void RecvData()

{

int
numr = 1;

int
rc = 0;

while (numr > 0 && rc >= 0) {

    numr = read(msgsock,buffer+buflen, MAX_BUFLEN-buflen);

    if (numr > 0) {

       buflen += numr;

       buffer[buflen] = '\0';

       printf("buffer = %s\n",buffer);

       rc = ExtractMsg(buffer,&buflen);

       }

   }

}

/********************************************************************

*


EXTRACT MSG

*   DESCRIPTION

*   This function accepts a buffer of data on input, along with the 

*   amount of data in the buffer, and extracts GTS messages from this

*   buffer.  

* 

*   Messages that are in the buffer are identified as follows...

* 

*   - The first 8 bytes of the message buffer HAVE to be a message 

*     length in character format. 

*     If the length exceeds the GTS defined maximum message size, or

*     does not consist of numeric characters, then an error is returned

*     (lost synchronisation).

*

*   - Immediately following the message length is a 2 character

*
Message Type: "AN" = Alphanumeric, "BI" = binary, “FX” = Fax

* 

*   - The GTS message begins with a SOH character, and is terminated

*     with a ETX character, if this does not occur, then an error is

*     returned (lost synchronization).

* 

*   - If a GTS message is identified, then it is extracted and the

*     message is shifted out of the buffer.

* 

*   - As there may be more than 1 message in the buffer, this function

*     will loop (extracting messages) until either and 

*     error or incomplete message is detected.

*  

*  

*   RETURNS     = 0  - Not a complete message in the buffer.

* 

< 0  - Fatal error in the format of the buffer.

*  

> 0  - Success, the message(s) have been extracted

********************************************************************/

static int ExtractMsg(char *buffer, int *buflen)

{

int   
rc, msglen;

char
msg[MAX_MSGSIZE+1];

/* FIND THE FIRST MESSAGE IN THE BUFFER */

rc = FindMessage (buffer, *buflen, &msglen);

/* WHILE A VALID MESSAGE LENGTH IS FOUND IN THE MESSAGE BUFFER... */

while ( rc > 0 ) {

  /* ENSURE THAT THE FIRST CHARACTER AFTER THE MESSAGE LENGTH IS

     A 'SOH' CHARACTER, AND THE LAST CHARACTER AS INDICATED BY 

     THE MESSAGE LENGTH IS AN 'ETX' CHARACTER. */

  if ( (rc = CheckMsgBoundaries (buffer, msglen)) < 0 ) 

     continue;

  /* PRINT THE EXTRACTED MESSAGE */

  memcpy(msg,buffer+GTS_SOCKET_HEADER,msglen);

  msg[msglen] = '\0';

  printf("GTS MSG = \n%s\n",msg);

  /* SHIFT THE JUST INJECTED MESSAGE OUT OF THE MESSAGE BUFFER,

     AND LOOP BACK TO LOOK FOR A NEW MESSAGE.  */

  ShiftBuffer (buffer, buflen, msglen);

  /* FIND THE FIRST MESSAGE IN THE SHIFTED BUFFER */

  rc = FindMessage (buffer, *buflen, &msglen);

  }

return (rc);

}

/********************************************************************

*


FIND MESSAGE

* Check that the complete message is at the start of the buffer.

* 1. Check the first 8 characters which are the message length

* 2. Check the next 2 characters - Message Type

* 3. Check that the complete message, as defined by the "message length"

*    field, is in the buffer.

* Return codes:

*     0 = message incomplete

*     1 = message complete

*    -1 = error

********************************************************************/

static int  FindMessage (char *buffer, int buflen, int *mlen)

{

   char  charlen[GTS_LENFIELD+1];

   int   intlen;

   *mlen = 0;

   /* IF THE LENGTH OF THE PASSED MESSAGE BUFFER IS NOT GREATER THAN

      10 CHARACTERS THEN RETURN 'INCOMPLETE'.   */

   if ( buflen < GTS_SOCKET_HEADER ) {

      return (0);

      }

   /* CHECK THAT THE MESSAGE TYPE IS VALID */

   if (strncmp(buffer+GTS_LENFIELD,"AN",2) && strncmp(buffer+GTS_LENFIELD,"BI",2) && strncmp(buffer+GTS_LENFIELD,"FX",2)) {

      printf("ERROR: Message Type field invalid");

      return (-1);

      }

   /* EXTRACT THE MESSAGE LENGTH */

   strncpy (charlen, buffer, GTS_LENFIELD);

   charlen[GTS_LENFIELD] = '\0';

   /* CHECK THAT THE MESSAGE LENGTH CHARACTER STRING COMPRISES 

      ENTIRELY OF DIGITS.  RETURN AN ERROR IF THIS IS NOT THE CASE. */

   if ( strspn (charlen, "0123456789") != strlen (charlen) ) {

      printf("ERROR: length not numeric");

      return (-1);

      }

   /* CONVERT THE MESSAGE LENGTH CHARACTER STRING TO AN INTEGER. */

   intlen = atoi (charlen);

   /* CHECK THAT THE LENGTH EXTRACTED FROM THE BUFFER IS NOT GREATER

      THAN THE GTS DEFINED MAXIMUM MESSAGE SIZE - RETURN AN ERROR IF

      THIS IS THE CASE. */

   if ( intlen > MAX_MSGSIZE ) {

      printf("ERROR: message overlength");

      return (-1);

      }

   /* CHECK IF THE ENTIRE MESSAGE HAS BEEN RECEIVED. RETURN IF NOT */

   if ( buflen < intlen + GTS_SOCKET_HEADER ) {

      return (0);

      }

   *mlen = intlen;

   return (1);

}

/********************************************************************

*


   CHECK MSG BOUNDARIES

* Confirm the first character after the Socket Header is

* a SOH, and the last character in the message (given by the message

* length) is an ETX.

********************************************************************/

static int  CheckMsgBoundaries (char *buffer, int msglen)

{

   /*  CHECK THAT THE FIRST CHARACTER (AFTER THE MESSAGE LENGTH

       FIELD) IS AN SOH CHARACTER - RETURN AN ERROR IF IT ISN'T. */

   if ( buffer[GTS_SOCKET_HEADER] != SOH ) {

      printf("ERROR: SOH not found\n");

      return (-1);

      }

   /*  CHECK THAT THE LAST CHARACTER (ACCORDING TO THE MESSAGE LENGTH

       FIELD) IS AN ETX CHARACTER - RETURN AN ERROR IF IT ISN'T. */ 

   if ( buffer[msglen+GTS_SOCKET_HEADER-1] != ETX ) {

      printf("ERROR: ETX not found\n");

      return (-1);

      }

   return (1);

}

/********************************************************************

*


   SHIFT BUFFER


* Shift the leading message in the buffer out of the buffer.  This may

* either empty the buffer, or move all or part of a new message to the

* start of the buffer.

********************************************************************/

static void  ShiftBuffer (char *buffer, int *buflen, int msglen)

{

   int  shiftlen;

   /* CALCULATE THE AMOUNT OF DATA TO BE SHIFTED OUT OF THE BUFFER. */

   shiftlen = msglen + GTS_SOCKET_HEADER;

   /* SHIFT THE 'PROCESSED' DATA  OUT OF THE BUFFER BY MOVING THE

      UNPROCESSED DATA OVER THE TOP OF IT. 

      CALCULATE THE NEW AMOUNT OF DATA IN THE BUFFER. */

   *buflen = *buflen - shiftlen;

   memcpy (buffer, buffer + shiftlen, *buflen);

}

APPENDIX 4 - Some security arrangements for small GTS Centres
Appendix 4 has been removed from this Attachment.  All IT Security material can now be found in the “Guide on Infromation technology Security” which is at 

http://www.wmo.int/pages/prog/www/documents.html
This Appendix provides information on low-cost measures to secure GTS centres, when they are connected to the Internet. The traditional GTS with Message Switching Systems passing bulletins over point-to-point circuits is inherently secure, while the Internet is inherently insecure. So, it is important to prevent Internet users from being able to traverse GTS links, where they may be able to cause damage to neighbouring centres. 

Security policy

In a mixed Internet/GTS environment, a security hole at a GTS centre may compromise other GTS segments. It is very likely that sooner or later, most of the GTS centres will be connected to the Internet, so a solution for the security aspects must be found, which are practical for all Centres and especially, small centres. Different levels of network security can be achieved with the tools available today at costs that vary from a few dollars (freeware) to expensive proprietary solutions. 

There are, however, other important aspects besides costs. Technical expertise and strong management support are indispensable elements to implement and enforce an effective security policy. Without them, no protection will be achieved, even with the best and most expensive firewall systems.

Coexistence of Internet and dedicated GTS links

There are many options for the general configuration of systems in small Centres. Figure 1 shows an arrangement where separate routers are used to provide access to Internet and GTS. 


Figure A4.1. Coexistence of GTS and Internet – separate access routers

To achieve cost reduction, GTS centres, may wish to consolidate the GTS and Internet networks, whilst still providing a level of security for their GTS systems. Figure 2 depicts a low cost configuration that may meet this objective. 


Figure A4.2. Coexistence of GTS and Internet – common access router

Protecting the GTS links from the Internet

It is important that the exterior router(s), connected to both GTS and Internet links be securely passworded, and protected so that it may not be configured via the Internet. Additionally, no Internet traffic should be allowed to propagate down GTS links, nor GTS traffic be sent to the Internet unless specifically intended to do so. This can be achieved by carefully filtering routing updates.


There should be a definite separation between general Internet services (www/http, e-mail access) and the GTS system (e. g. Message Switch). They should be in separate machines. Additionally, use of firewalling technology should be undertaken to limit general Internet access to the GTS Centre internal network, possibly restricting incoming connections to SMTP on the mail server, HTTP on the web server and DNS on the Domain Name servers.

In between the exterior router and critical systems, a firewall should be deployed. This firewall must have the capacity to limit, proxy or redirect access to internal hosts in order to protect them. Several brands of firewall are on the market, with ranges of capabilities. In most cases because of the simple nature of the network in small centres, a simple firewall may be deployed.

When connecting to the Internet, deploying some sort of firewall is virtually mandatory. The risks for internal data and systems would justify this. In order to allow the access control some low-cost options are available:

Linux computers


Linux Operating system is free, and runs on a variety of hardware platforms, notably on PCs. The newest versions of Linux (Kernel version 2.2) come with firewalling software called ipchains. Additionally, they support routing protocols through a routing program called gated. Centres  with some experience with UNIX will be able to get a working firewall setting up linux from scratch.

Windows NT


A variety of commercial packages exist. The familiarity with the Windows and relatively low cost of PC hardware is seen as main advantage.

Free Toolkits


A company called TIS (Trusted Information Systems) has released a set of source code, mainly for UNIX/LINUX hosts, which is freely available. This requires access to UNIX/LINUX machines, compilers, and requires good knowledge of Internet security issues.

Routers


Many routers have packet filtering capabilities. It is possible to deploy one of these as firewall, although they aren't very flexible. Smaller centres may want to consider this.

Desirable solution

Some firewall vendors have been providing firewall solutions, based on their hardware. Cisco's IOS firewall is a notable example. This type of solution is depicted in figure 3.


Figure A4.3. Coexistence of GTS and Internet – separate access routers plus firewall

GTS using the Internet 

There will be situations where GTS centres will use Internet to transport data and products. Security concerns are also applicable here. The arrangement shown below in Figure 4 represents a simple and safe way to use the Internet to connect neighbouring GTS centres that may become popular in small centres in future. Firewalling is done using access lists. 


Figure A4.4. The use of Internet between neighbouring GTS centres.

Virtual Private Network – secure GTS connections over the Internet

A virtual private network (VPN) is a private network implementation among organizations to communicate over a publicly accessible network. VPN message traffic can be carried over the Internet on top of the standard TCP/IP protocols. Secure VPNs use cryptographic tunneling protocols to provide the sender authentication, message integrity and confidentiality to achieve privacy. This is considered suitable for use for the transmission and exchange of meteorological data among centres.   
The most common Secure VPN protocol is IPSec. IPSec is designed to provide interoperable high quality, cryptographically-based security for IP. The set of security services offered includes access control, connectionless integrity, data origin authentication, protection against replays, as well as confidentiality.
IPSec is an end-to-end security protocol: all the functionality and intelligence of the VPN connection reside at the end points, either in a gateway or in the end-host. Through IPSec, it is possible to establish a tunnel between two gateways. An IPSec gateway would typically consist of an access router, a firewall or a VPN gateway on which the IPSec protocol is implemented. The IPSec gateway locates between the user’s private network and the carrier’s shared network.

IPSec tunnels are established dynamically and released automatically when they are not in use. To establish an IPSec tunnel, two gateways must authenticate themselves and define which security algorithms and keys they will use for the tunnel. The entire original IP packet is encrypted and wrapped inside IPSec authentication and encryption headers. Figure 5 shows an implementation of a VPN connection using IPSec between two GTS centres.

[image: image20.jpg]VPN Gateway

VPN Gateway GTS Host





Figure A4.5. The use of VPN over the Internet to connect two GTS centres.

The VPN gateway is connected to a firewall and a router. The VPN gateway can establish a VPN tunnel with other VPN gateways through the interface of the router. The flow of traffic into the internal network will be through the firewall and will be controlled by an access list defined by the user. 

IPSec makes use of the Authentication Header (AH) and the Encapsulation Security Payload (ESP) to achieve data integrity and confidentiality.

The most common encryption algorithm used in ESP is Triple Data Encryption Standard (3DES) and Advanced Encryption Standard (AES). They have encryption key sizes from 128 bits to 256 bits, providing sufficient protection for data traffic along the trunk.

Further details on VPN implementation can be found in the Guide on Virtual Private Network

(VPN) via the Internet between GTS centres. This Guide is available on the WMO web page at http://www.wmo.int/web/pages/prog/manuals.html.



APPENDIX 5 -  Reference material
General references on TCP/IP

1.  Internetworking TCP/IP Vol. 1 (2/E) - Douglas Comer - Prentice Hall 

2.  TCP/IP Illustrated Vol. 1. - Stevens - Addison-Wesley 

3.  TCP/IP Architecture, Protocols and Implementation - Feit - McGraw Hill 

4.  TCP/IP and Related Protocols - Black - McGraw Hill 

5.  TCP/IP Running a Successful Network - Washburn and Evans - Addison-Wesley 

6.  TCP/IP and ONC/NFS (2/E) - Santifaller - Addison-Wesley 

7.  Inside TCP/IP - Arnett et. al. - New Riders Publishing 

8.  Teach Yourself TCP/IP in 14 days - Parker - SAMS 

9.  Introduction to TCP/IP - Davidson - Springer

References on Security

1.  Firewalls and Internet Security- Cheswick & Bellovin - Addison-Wesley

2.  Building Internet Firewall – Chapman – O’Reilly

3.  Practical Unix Security - Garfinkel & Spafford - O'Reilly

4.  Internet RFC 2196 (Site security Handbook)
5.  http://www.computersecuritynow.com: a website with a lot of reference documents on implementing security
APPENDIX 6 - Suggested password management practices
Password management is a topic included in the IT Security discussion.  All IT Security material can now be found in the “Guide on Infromation technology Security” which is at 

http://www.wmo.int/pages/prog/www/documents.html
Passwords are the system’s first line of defence against unauthorised intrusion. While it is possible to violate system security without logging in, a poorly protected or chosen password can make a hacker’s task a lot easier. 

GOOD PASSWORDS: 

1.  Have both upper-case and lower case letters, and/or

2.  Have digits and/or non-alphanumeric characters. 

3.  Are 6 to 8 characters long. 

4.  Should consist of at least 2 words or groups of characters. 

5.  Should not be shared or used by more than one user. 

6.  Should not be used on more than one computer. 

7.  Should be changed regularly, eg monthly. 

8.  Can be typed quickly and easily, so that an observer cannot follow the keystrokes.

9.  Are easy to remember - so that they should not have to be written down. (e.g. use first letter of words in a well known phrase )

BAD PASSWORDS: 

1.  The name of: yourself, your spouse, your children, your parents, your pet, your friends, your favourite film stars/characters, anyone associated with you, your workstation or its host.

2.  The number of: your telephone, your car’s license plate, your user ID, any part of your credit cards’ numbers, or any number associated with you. 

3.  The birthday of yourself or anyone associated with you. 

4.  Any word from any dictionary, any place name, any proper noun. 

5.  The name of a well known public identity such as a sporting hero, entertainer or well known fictional character.

6.  Simple patterns: aaaaaaa, qwerty. 

7.  Any of 1 - 6 spelled backwards. 

8.  Any of 1 - 6 preceded or followed by a digit. 

9.  Any password that has been written down and left in an unlocked drawer or unsecured computer file. 

10.  Any password that has been on a machine that may have been successfully hacked (except as part of authorised exercises). 

11.  Any password on a machine that has been left unattended when any user is logged on.



Appendix 7 - IP addresses for use on the GTS

INTRODUCTION
The current “Recommended practices and procedures for the implementation, use and application of the Transmission Control Protocol/Internet Protocol (TCP/IP) on the GTS” (also known as “Guide on Use of TCP/IP on the GTS”), as given in this Attachment, describe guidelines and a procedure for assigning IP addresses to GTS links which are no longer adequate. In particular, it is stated that a number of official class C IP addresses were available through the WMO Secretariat to be assigned for GTS links.  These sets of IP addresses are no longer officially available, as a consequence of a strict application of Internet standards (RFCs) by Internet Authorities and Services Providers, and unfortunately cannot be used on the GTS, as they may now be assigned to other organizations on the Internet. The WMO Secretariat has therefore been instructed to discontinue the assignment of such IP addresses.

The Expert Team on Communication Techniques and Structure (ET-CTS) has been tasked to provide alternate solutions to solve this issue.

This document is a provisional description of the available options and related guidance to mitigate this problem and assist Members in their implementation. The included guidelines only concern the IP addressing. They do not change the existing recommendations on how IP relates to X.25 or other IP functionality.

The ET-CTS will proceed with developing the proposed amendments to this Attachment to reflect the new recommended practices for allocating IP addresses.

WHO CAN PROVIDE OFFICIAL IP ADDRESSES?

In order to build a network that interconnects many organizations from various countries in the world, it is essential to maintain a standard in the addressing scheme, and to maintain uniqueness in the allocation of addresses to the various organizations. The Internet community has identified this basic principle and created some official bodies to coordinate the distribution of official IP addresses. Today, this responsibility belongs to the Internet Assigned Numbers Authority (IANA), and its regional delegates, the relevant Regional Internet Registries:

AfriNIC (African Network Information Centre) – Africa region

APNIC (Asia Pacific Network Information Centre) – Asia Pacific region

ARIN (American Registry for Internet Numbers) – Americas and Southern Africa

LACNIC (Regional Latin-American and Caribbean IP Address Registry) – Latin America and some Caribbean islands

RIPE NCC (Réseaux IP Européens Network Coordination Centre) – Europe and surrounding areas

These organizations further delegate the allocation of addresses to their regional Internet and telecommunications suppliers through national Internet registries.

In this scheme, it is not the WMO’s responsibility to allocate IP addresses. Since the GTS is not built as a unique network under the complete authority of a single organization, the allocation of addresses must therefore go through the respective national Internet registry or the appropriate Regional Internet Registry.

However, several countries now face the issue of the restriction of allocation of IP version 4 (IPv4) addresses and may have difficulty obtaining official addresses. This problem is not an easy one to solve in the short term and provisional measures may have to be taken to allow the further development of the GTS. The following guidelines explain how to interconnect networks with and without the use of official IP addresses.

CONNECTING NETWORKS WITH OFFICIAL IP ADDRESSES

Using official IP addresses assigned directly to an organization (e.g. the NMS)

This remains the preferred option if it is feasible. It is basically the main procedure described in the existing “Guide on Use of TCP/IP on the GTS”. It follows all the Internet rules and allows an organization to build a coherent network with interconnections to the Internet, GTS and possibly other partner organizations. It is also the easiest configuration to maintain.

In interconnecting two countries to form a GTS link, the two National Meteorological Services should decide which one actually provides the address to the interconnecting link. The decision remains one of practicality for the countries. There are no general rules that would favour one set of addresses over another one.

Using official IP addresses provided by a telecommunications supplier

This option is very similar to the previous one. The addresses supplied would be official and all the rules would of course be followed.

It may require that a common telecommunications supplier be used between the two interconnecting organizations.

This option however has the drawback that a change in telecommunication suppliers may require a change in IP addressing as original incumbent reclaims “his” addresses. Each organization should plan for this possibility ahead of time and evaluate its impact on future operations. If these addresses are only used for link purposes and not for an organization’s internal purposes, then this drawback may be of minimal impact.

Using IP version 6 (IPv6) addresses

The new IP version 6 (IPv6) protocol standard was designed in great part to address the shortage of IPv4 addresses. Although the IPv6 protocol is available and supported in many telecommunication equipments available today, its implementation requires much planning. In particular, IPv4 and IPv6 are not compatible without the use of gateways and there are several operational tools still missing to make IPv6 usable for the GTS at this time. Converting to IPv6 would be a major task that can not be imposed on our members until the industry is ready to take this step as a whole.

This option is therefore not available today. It is only mentioned here for completeness and will be further studied over the next years.

CONNECTING NETWORKS WITHOUT OFFICIAL IP ADDRESSES

Using the “ip unnumbered” feature

Several network equipment suppliers (Cisco, 3Com, Juniper) have now introduced a feature in their configurations which allows the implementation of links without the need for allocation of IP addresses. This feature is usually called the “ip unnumbered“ feature. For example, Cisco provides a document on “Understanding and Configuring the ip unnumbered Command“ (see http://www.cisco. com/warp/public/701/20.html for details).

This feature is not a standard IP protocol feature, so it requires compatible equipment at both ends of the link to work (most frequent situation anyhow).

Routing between the two networks can be accomplished by binding the unnumbered interface to another existing interface in the router (either a real LAN or virtual loopback interface). The use of this feature may introduce limitations in routing flexibility.

Using RFC1918 – Addresses for private internets

The Internet Engineering Task Force (IETF) document “RFC1918 – Addresses for private internets“ describes a set of addresses reserved for use by organizations for sole intra-enterprise communications, without any intention to ever directly connect to other enterprises or the Internet itself.  Private internet addresses are sometimes referred to as “non-official IP addresses” or “non-registered IP addresses”.  Addresses for private internets are in fact official, but they are not unique.
Therefore the use of these addresses does not require official registration. The main purpose of this scheme is to allow a big organization to make use of a larger address space for its internal operations. As soon as the organization needs to exchange with others, a gateway must be traversed to enter an area of officially assigned addresses to maintain overall network coherence. This gateway must translate the internal RFC1918 addresses into official external IP addresses, which must be obtained via the official bodies. The function (usually performed by a router or firewall) that does this translation is called Network Address Translation (NAT). This address translation will also have the effect of concentrating several RFC1918 internal addresses into a very small number of official addresses, thus preserving official address space.

Although this scheme might seem attractive at first for our issue, the GTS is not the network of a single enterprise. At this time, any number of the WMO Member NMHSs and related organizations may already make use of the RFC1918 in their own networks, which may result in conflicting address allocations if the networks interconnect. A recommendation from WMO for the use of RFC1918 is almost an impossible task, as the NMHSs may already be under guidelines of their own government, which might conflict with a directive of WMO. However, interconnecting countries may find adequate address space within RFC1918 in a bilateral agreement.

This option is therefore feasible as long as the following points are carefully considered, planned, maintained and monitored:

1. 
Great care should be taken in selecting a proper RFC1918 set of addresses for links between organizations. It is important that the selected addresses are not already in use by any of the involved organizations.

2. 
Great care should be taken to ensure that routing configurations do not allow the leaking of RFC1918 addresses into other organization’s network or worse, into the Internet.

3. 
Although this solution will work quite satisfactorily between a few countries, it cannot be expanded to many directly interconnected countries, as the choice of RFC1918 addresses will get more and more complicated.

4. 
The IANA has reserved the following blocks in RFC1918.

10.0.0.0 - 10.255.255.255 (10/8 prefix)

172.16.0.0 - 172.31.255.255 (172.16/12 prefix)

192.168.0.0 - 192.168.255.255 (192.168/16 prefix)

Since many organizations already use the 10.0.0.0/8 block internally and since the 192.168.0.0/16
block is often used as default addresses by several equipment manufacturers, it is recommended that GTS links be used out of the 172.16.0.0/12 block only if possible.

5. 
Furthermore, it is also recommended that the 172.16.0.0/12 be subnetted in a way to maximizethe usage of the address space. To that effect, GTS links can be subnetted to /30 bits. This allows 4 hosts per link (leaving the hosts addresses 1 and 2 available to designate the 2 ends of a given link).

6. 
NMHS that consider using the RFC1918 addresses should consult with all potential NMHS with whom they might establish a link in order to coordinate and plan the use of these subnets ahead of time. In the case of address conflicts, other address schemes within RFC1918 might be used by bilateral agreement. The ET-CTS would like to be informed of such issues if they arise to further develop this recommendation.

The use of RFC1918 addresses should not introduce security problems as long as the above points are well managed.

RECOMMENDATION
All the options described above can be used in the GTS. The order of preference is as follows:

1. Using official IP addresses assigned directly to an organization, e.g. the NMHS (preferred).

2. Using official IP addresses provided by a telecommunications supplier.

3. Using the “ip unnumbered” feature.

4. Using RFC1918 – Addresses for private internets.

The use of IPv6 on the GTS is not recommended at this time.

It should be understood that all options that do not require official IP addresses are workarounds to mitigate the shortage of addresses and must be used with care.

CONFIGURATION EXAMPLES

Option 1 – Using existing organization (NMHS) official IP addresses or Option 2 – Using Telecommunication Supplier official IP addresses

This is the standard way to configure an interface between two networks.

Router A:

!

interface Ethernet0

ip address 131.238.17.11 255.255.255.0

!

interface Serial0

description 64Kbps leased line to router B

ip address 131.238.18.01 255.255.255.252

encapsulation ppp

bandwidth 64

!

ip route 142.47.43.0 255.255.255.0 131.238.18.2

!

Router B:

!

interface Ethernet0

ip address 142.47.43.201 255.255.255.0

!

interface Serial0

description 64Kbps leased line to router A

ip address 131.238.18.02 255.255.255.252

encapsulation ppp

bandwidth 64

!

ip route 131.238.17.0 255.255.255.0 131.238.18.1

CENTRE A





X.121: 0101xxx11yyyzz


IP: 193.105.177.3


Mask: 255.255.255.0








X.121: 0101xxx00yyyzz





CENTRE C





Host





X.121: 0101xxx00yyyzz





Router





X.121: 0101xxx00yyyzz





MSS





Host





MSS





Host





MSS





 CENTRE  B





X.121: 0101xxx11yyyzz


IP: 193.105.177.1


Mask: 255.255.255.0








X.121: 0101xxx11yyyzz


IP: 193.105.177.2


Mask: 255.255.255.0





Packet


Switch





Router





Router





Packet


Switch





Packet


Switch





CENTRE A





X.121: 0101xxx00yyyzz





CENTRE C





X.121: 0101xxx00yyyzz





Host





Router





Router





IP: 193.105.178.5


Mask: 255.255.255.252





MSS





Host





MSS





MSS





Host





X.121: 0101xxx11yyyzz


IP: 193.105.177.1


Mask: 255.255.255.0








X.121: 0101xxx11yyyzz


IP: 193.105.177.2


Mask: 255.255.255.0





IP: 193.105.178.6


Mask: 255.255.255.252





 CENTRE  B





Packet


Switch





Router





Packet


Switch





X.121: 0101xxx00yyyzz





CENTRE A





X.121: 0101xxx00yyyzz





CENTRE C





X.121: 0101xxx00yyyzz





Host





X.25 Interfaces





IP: 193.105.178.5


Mask: 255.255.255.252





MSS





Router





Packet


Switch





Host





MSS





Host





MSS





Router





IP: 193.105.178.6


Mask: 255.255.255.252





X.121: 0101xxx11yyyzz


IP: 193.105.177.1


Mask: 255.255.255.0








X.121: 0101xxx11yyyzz


IP: 193.105.177.2


Mask: 255.255.255.0





 CENTRE  B





Router





Packet


Switch





WWW


Server





Protected


Network





Internet





B Hosts





Router/


Firewall





Router/


Firewall





GTS





A Hosts


(GTS)





Firewall





WWW


Server








Router





SMTP


DNS





Protected


Network





Internet





GTS





GTS


Host





A Hosts allowed,


B Hosts blocked





A Hosts allowed by explicit permissions





Firewall





Open networks





Protected networks





Internet





B Hosts





Access


Router





Access Router





WWW


Server





GTS





A Hosts


(GTS)





Internet





'Open' access





Access Router





Neighbouring Centre





WWW


Server





Only neighbouring Centre GTS host allowed.





B Hosts





A Hosts


(GTS)








� An Autonomous System is defined in RFC1630 as “a set of routers under a single technical administration, using an interior gateway protocol and common metrics to route packets within the AS, and using an exterior gateway protocol to route packets to other ASs.”


� B cannot use EGP and BGP on the same router; one router cannot belong to more than one AS


� RIP is NOT a good choice for this type of configuration. But as RIP is the most basic protocol, it is used in this case too.





�Should we add IPv6 words in this documents, i.e. not yet supported although monitored. Etc…


�Are these still valid cisco configs?


�Verify this material is in the ITS Guide and then deleted from here.  Leave only the first few lines in Appendix 4.


�Verify this is in the Guide on ITS and then delete from here except first few lines.





_950352510.doc


IP Link







IP Link







X.25 link







X.25 link







X.121: Dx’







Router







IP address: Dc







Cd







IP address: 







Cb







IP address: 







Bc







IP address: 







’







IP address: C







’







IP address: B







’







Bx







X.121: 







Bx







X.121: 







Bi







IP address: 







’







Cx







X.121: 







Cx







X.121: 







Ci







IP address: 







Switch







Packet







Switch







Packet







Switch







Packet







IP address: B







IP address: D







IP address: C







IP address: A







’







X.121: Ax







X.121: Ax







Ai







IP address: 







CENTRE D







CENTRE B







CENTRE C







CENTRE A







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host












_1341658854.vsd
MSS APPLICATIONS
Various implementations


OSI LAYER 7
APPLICATION


OSI LAYER 1
PHYSICAL


OSI LAYER 2
DATA LINK


OSI LAYER 3
NETWORK


OSI LAYER 4
TRANSPORT


OSI LAYER 5
SESSION


OSI LAYER 6
PRESENTATION


TCP/IP APPLICATIONS
Telnet, FTP, NFS, NIS


TCP/IP PHYSICAL
Coax, Fiver, 10baseT, etc.


TCP/IP DATA LINK
Ethernet, FDDI, PPP, etc.


TCP/IP NETWORK
IP, ARP, RARP, ICMP


TCP/IP TRANSPORT
Sockets/Streams


TCP/IP SESSION
RPC


No specific implementation of this layer in TCP/IP stack


WMO APPLICATION LAYER


TOP LAYERS


LOWER LAYERS



_1341659431.vsd

_1094022675.doc


Message 2



length



(8 chacs)    







CR CRLF ETX







CR 







CR LF Heading Text          







nnn







nnnnn







CR LF  or 







Format



Identifier               







Message  1    







length                       SOH CR 







 







 (8 characters)           00







  







 







Message length







 







 Starting Line and End of Message present



Message length : Length from SOH to ETX  (e.g. 00001826 = 1826bytes)












_1094022860.doc


Message 2



 length



(8 characters)    







Format 



Identifier



     01               







Message length







Option (not prefered, to be discontinued): Starting Line and End of Message absent



Message length : Length from first CR to end of Text (e.g. 00001826 = 1826bytes)







Message 1



 length



(8 characters)    







Format 



Identifier



     01               







CR CR LF  Heading      Text          












_960641790.doc


Message length : Length from SOH to ETX  (e.g. 00001826 = 1826bytes)







CR CR LF ETX







 (8 characters)   (2 characters)







nnn







  Message type  AN: Alphanumeric,  BI: Binary,  FX: facsimile







length           type        SOH CR 







 







 







Message length







nnnnn







 







  







CR LF  Heading          







CR 







 







CR LF  or 







Message                







Message     







 












_950352520.doc


S0







S1







S1







S0







S0







X.121: Cx







Packet







Switch







Internet







Internet







Cb







IP address: 







Bc







IP address: 







’







IP address: C







’







IP address: B







’







Bx







X.121: 







Bx







X.121: 







Bi







IP address: 







Switch







Packet







Switch







Packet







IP address: B







IP address: C







IP address: A







’







X.121: Ax







X.121: Ax







Ai







IP address: 







CENTRE B







CENTRE C







CENTRE A







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host












_950352504.doc


’







IP address: D







’







IP address: B







’







IP address: C







’







IP address: A







IP address: B







IP address: D







IP address: C







IP address: A







IP address: Dc







Cd







IP address: 







Bc







IP address: 







Cb







IP address: 







Ba







IP address: 







IP address: Ca







Ab







IP address: 







IP address: Ac







CENTRE D







CENTRE B







CENTRE C







CENTRE A







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host












_950352507.doc


’







IP address: D







Cd







IP address: 







Cb







IP address: 







Bc







IP address: 







’







IP address: C







’







IP address: B







’







Bx







X.121: 







IP address: Dc







Bx







X.121: 







Bi







IP address: 







’







Cx







X.121: 







Cx







X.121: 







Ci







IP address: 







Switch







Packet







Switch







Packet







Switch







Packet







IP address: B







IP address: D







IP address: C







IP address: A







’







X.121: Ax







X.121: Ax







Ai







IP address: 







CENTRE D







CENTRE B







CENTRE C







CENTRE A







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host












_950352501.doc


’







Dx







X.121: 







’







Bx







X.121: 







Dx







X.121: 







Di







IP address: 







Bx







X.121: 







Bi







IP address: 







’







Cx







X.121: 







Cx







X.121: 







Ci







IP address: 







Switch







Packet







Switch







Packet







Switch







Packet







Switch







Packet







IP address: B







IP address: D







IP address: C







IP address: A







’







X.121: Ax







X.121: Ax







Ai







IP address: 







CENTRE D







CENTRE B







CENTRE C







CENTRE A







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host







Router







MSS







 Host












