### TT-GISC/10 Harmonization of file format of 24h cache

ET-WISC 2013 final report mention under item 4.11.4 that the GISCs should harmonize the Cache file format.

## Progress

At ET-WISC meeting, 2013, DWD and JMA reported about investigation of differences of the file format of each 24h cache. The result is:

- There are only minor differences, but these can result in serious inconvenience to the users, especially in case of active GISC backup procedures.

And the meeting encouraged all GISCs to use the full “Meteorological bulletin” as default representation of GTS-bulletin data instances. (See Final Report 77, 78, 79)

Additionally, JMA investigated file format of alphanumeric of  Exeter and Tehran and binary format of operational GISCs. The result is same as previous report. Four type of file format can be seen at 24h cache of GISCs

## Investigation results

## Alphanumeric format

- GISCs	 Beijing, Exeter, Melbourne, Offenbach, Seoul, Tokyo, Toulouse, Tehran (alphabetical order, the result of Beijing, Melbourne, Offenbach, Seoul, Tokyo, Toulouse, See Doc 56 of ET WISC MEETING [15-18 JULY 2013])

- Target	GTS bulletin SMJP01 RJTD

### Exeter

The file format of Exeter includes:

- Text part

| File Name | A_SMJP01RJTD090000_C_RJTD_20131209000000_d53fbd35ecc97ed94aa6bbf7a4e42a0e.txt |
| --- | --- |
| Head | AAXX 09004 47401 11/75 62608 10011 21072 30131 40146 53001 69902 72682 86200 |
| (Hex View) |  |
| Foot | 47991 16/// /0110 10241 20169 30133 40143 50008 60082 333 20195= |
| (Hex View) |  |

### Tehran

The file format of Tehran includes:

- Message Length, Format Identifier

- Starting Line

- Abbreviated heading

- Text part

- End of message signals

| File Name | 2013_12_050600_SMJP01_RJTD_.txt |
| --- | --- |
| Head | 0000141300 853 SMJP01 RJTD 050600 AAXX 05064 47401 11/65 72607 10052 21031 30063 40077 55003 69901 72582 8227/= |
| (Hex View) |  |
| Foot | 47991 16/// /3212 10244 20230 30076 40086 53007 60051= [ETX] |
| (Hex View) |  |

File format (alphanumeric) of GISCs

### Binary format

- GISCs	 Beijing, Exeter, Melbourne, Offenbach, Seoul, Tokyo, Toulouse, Tehran (alphabetical order)

- Target	GTS bulletin ISIC01 RJTD

### Beijing

The file format of Beijing includes:

- Message Length, Format Identifier

- Starting Line

- Abbreviated heading

- Text part (BUFR)

- End of message signals

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204211919.bin |
| --- | --- |
| Head | 0000780900 48123 ISIC01 RJTD 042100 BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Exeter

The file format of Exeter includes:

- Text part (BUFR)

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204210000_1594db83ed973d9108b636b6ac3f38e8.bin |
| --- | --- |
| Head | BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Melbourne

The file format of Melbourne includes:

- Text part (BUFR)

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204210000_1594db83ed973d9108b636b6ac3f38e8.bin |
| --- | --- |
| Head | BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Offenbach

The file format of Offenbach includes:

- Starting Line

- Abbreviated heading

- Text part (BUFR)

- End of message signals

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204212200.bin |
| --- | --- |
| Head | ISIC01 RJTD 042100 BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Seoul

The file format of Seoul includes:

- Abbreviated heading

- Text part (BUFR)

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204211945_80_99b48c2f7ac06afbfc0efc92141e9c88.bin |
| --- | --- |
| Head | ISIC01 RJTD 042100 BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Tokyo

The file format of Tokyo includes:

- Abbreviated heading

- Text part (BUFR)

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204211945_80.bufr |
| --- | --- |
| Head | ISIC01 RJTD 042100 BUFR |
| (Hex View) |  |
| (Hex View) |  |

### Toulouse

The file format of Toulouse includes:

- Text part (BUFR)

| File Name | A_ISIC01RJTD042100_C_RJTD_20131204210000_1594db83ed973d9108b636b6ac3f38e8.bin |
| --- | --- |
| Head | BUFR |
| (Hex View) |  |
| (Hex View) |  |

Tehran
The file format of Tehran includes:

- Message Length, Format Identifier

- Starting Line

- Abbreviated heading

- Text part (BUFR)

- End of message signals

| File Name | 041500_12_041500_ISIC01_RJTD_.bufr |
| --- | --- |
| Head | 0000780700 249 ISIC01 RJTD 041500 BUFR |
| (Hex View) |  |
| (Hex View) |  |

File format (binary) of GISCs

## Interoperability with the GTS

The message from GTS can be stored to 24h cache, also the file of 24h cache should be able to send to GTS. For re-transmission to the GTS, it should be not drop necessary parts (Abbreviated heading) from original message. Regarding 'Starting Line' part, It will be added when the message is created and sent for GTS transmission in the MSS with 'Message Length' and 'Format identifier' part .

## Filename (is just a filename)

The data files stored in the 24h cache have regular filenames with file name convention. So users can recognize what kind of data from file name. The other side, the data file might be downloaded with another file name such as temp1, temp2. In that sense, GISCs must keep ‘Abbreviated heading’ part in file.

## Inconvenience for users.

According to the investigation result above, four patterns of file format of 24h cache were confirmed. While a user downloads the data files from one GISC, there will be no problem for reading/processing data files. But when users switch data provider to another GISC, there will be the possibility that user can not read/process data files because of such a difference of file format.

Current status of 24h cache of GISCs

Sharing information of 24h cache is GISC’s task. WIS manual say “3.5.3.1 Each GISC shall collect from its area information that is intended for global exchange and shall share such information with other GISCs so that all GISCs have a common holding of information available for global exchange.” There is some ways to share data files, but when GISCs share data files from 24h cache directly, then several type of file format will be seen in 24h cache of one GISC.

After Synchronization data files from 24h cache directly

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

Convert!

GISCs must convert data file format.

Ideal status

## References:

[1] Report of the sixth session of ET-WISC (2013) (Annex 11) - http://wis.wmo.int/doc=2799

[2] Manual on GTS Attachment II-15 para 5.2 - http://library.wmo.int/opac/index.php?lvl=notice_display&id=47

## Recommended Text

Mr. Shigeharu NISHIKAWA reported additional investigation for file format of 24h cache to complement DWD and JMA’s report of ET-WISC meeting, 2013. He emphasized decision of previous ET-WISC meeting about file format of 24h cache, ‘the meeting encouraged all GISCs to use the full “Meteorological bulletin” as default representation of GTS-bulletin data instances.’. He also mentioned interoperability with the GTS, about filename and inconvenience for users.