| ## WORLD WEATHER WATCH ## COMMISSION FOR BASIC SYSTEMS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Steering Group on Radio Frequency Coordination (SG-RFC) Boulder, Colorado. USA. 11-13 March 2014. |  | Document SG-RFC/2014-1-10 |  |  |
|  |  | 25 February 2014 |  |  |
|  |  | English only |  |  |
|  |  | Restricted access required? (Y/N)* | N |  |
| STEERING GROUP ON RADIO FREQUENCY COORDINATION (SG-RFC) |  |  |  |  |
| J. LAFEUILLE (Secretariat) AND T. ONSAGER (USA) |  |  |  |  |
| RADIO-FREQUENCY SPECTRUM FOR SPACE WEATHER ACTIVITIES |  |  |  |  |

1. Introduction

The sixteenth Meteorological Congress (Cg-16) has recognized the potential socio-economic impact of space weather events that affect for instance international air navigation, spacecraft operations, power grid operations, radio-communications and GNSS localization and time signals. This is of specific relevance to WMO as meteorological operations are relying to a large extent on satellite observations and radio-communications, and GNSS time signals. Furthermore, there is an expectation from WMO Members that meteorological and space weather support to aviation can be ultimately delivered in an integrated manner.  Congress therefore agreed that support to the international coordination of space weather operations should be one of the areas of activities of the WMO Space Programme  (See: http://www.wmo.int/pages/prog/sat/spaceweather-intro_en.php) This activity is being pursued through the Inter-Programme Coordination Team on Space Weather (ICTSW) which reports to both the Commission for Basic Systems and the Commission for Aeronautical Meteorology.

The present paper provides initial considerations on the radio-frequency spectrum needed for ground-based space weather observations.

2. Overview of Ground-based Space Weather Observations

A review of space weather observation status and needs is provided in the “Statement of Guidance for Space Weather Observations” completed by ICTSW in May 2012.  Radio-frequencies are used in particular for ground-based solar and ionosphere monitoring. Space-based observations for space weather are not addressed in the present paper.

Ground-based solar radio observations are principally performed by either solar radio spectrographs operating in a continuous frequency range from HF to UHF, or by solar radio telescopes operating in discrete bands ranging from VHF to SHF.

Ionospheric observations are performed by the following active or passive techniques:

- GNSS receiver: Total electron content (TEC) along a given propagation path can be measured by tracking the time delay and phase shift of radio signals of a Global Navigation Satellite System by a ground receiver (so called ground based GNSS). As the satellites move over the observing site the TEC along many propagation paths can be used to reconstruct the distribution of ionospheric electron density. Complemented by a constellation of space-based radio-occultation receivers, the GNSS receiver network provides vertical profiles of electron density on a global scale.

- Radio absorption: Ground-based measurements of (~MHz) radio signals from known sources above the atmosphere (usually cosmic radio sources) can be used to map the absorption by the lower ionosphere (e.g. riometers). As the Earth turns, each radio source moves across the observer, and this information can be used to map the distribution of absorbing ionisation in the lower ionosphere.

- Ionosonde: An ionosonde transmits a sweep of (~MHz) radio signals into the ionosphere. Echoes are returned where the local plasma frequency equals the radio frequency. The peak frequency returned from each ionospheric layer is therefore a direct measure of the electron concentration. In contrast, the height of each layer is estimated from the time of flight of the radio signal assuming propagation through free space. Modern research ionosondes can also measure additional variables, most notably plasma velocities and the spectrum of gravity waves. Ionosondes can also detect absorption through the minimum frequency (fmin) observed on an ionogram, although lower values of fmin are limited by terrestrial radio noise and instrument sensitivity.

- Incoherent Scatter Radar: Incoherent Scatter Radars (ISRs) transmit powerful VHF or UHF (typically hundreds of MHz) radio pulses into the ionosphere and use the backscatter from electrons whose motion is controlled by the dynamics of the much slower and heavier ion population to determine the distribution of ionisation. While the height of ionospheric layers can be determined very accurately with ISRs, cross-calibration with ionosondes or the use of specific features in the ISR spectra are required to calibrate the electron concentration, which is inferred from the backscattered power. Careful fitting of the ISR spectrum can generate a host of parameters including line-of-sight velocities, electron and ion temperatures, and ion composition. Estimates of the horizontal distribution of ionisation can be obtained by moving the radar dish or by altering the phase of an antenna array. Vector velocities can be calculated by combining data from different antennas or from different beam directions. These research data are not yet available in real time.

- Coherent radar: Coherent radars broadcast ~MHz radio pulses and receive coherent echoes from local plasma instabilities in the ionosphere. In this way, the returned power and line-of-sight velocity can be determined. Vector velocities can be calculated by combining data from two or more radar stations.

- Ionospheric scintillation receivers, measure the random fluctuations of intensity and phase of radio waves resulting of variations in the refractive index of the ionosphere. Scintillation is typically observed in GPS signals operating at 1.58 and 1.23 GHz and is relevant for all trans-ionospheric transmissions.

3. Summary of frequencies used for Space Weather Activities

Table 1 summarizes the outcome of a preliminary survey of frequencies used by ICTSW members.

TABLE 1: SUMMARY OF RADIO-FREQUENCIES USED FOR SPACE WEATHER OBSERVATIONS

| Frequency range | Application | T/R | Country | Location | Comments |
| --- | --- | --- | --- | --- | --- |
| I. Ionospheric observations |  |  |  |  |  |
| 1-20 MHz | Digital Ionospheric Sounder | T/R | Belgium | Bourbes | http://ionosphere.meteo.be |
| 1-30 MHz | Ionosphere sounder (Ionosonde) | T/R | China 6 CMA sites: | Xinjiang Atushi Fujian Xiamen Shanxi Changan Guangxi Hengxian Qinghai Dulan Hubei Wuhan | Protection: -no radio emission within 1km in these frequencies - no high voltage power line within 100m |
| 2 + 0.02 MHz | Middle frequency radar | T/R | China | Shanxi Wuzai | protection |
| 3-30 MHz | Vertical ionosondes (VIS) | T/R | Australia |  | TX licence and quiet Rx Risk of interference with remote sources http://www.ips.gov.au/HF_Systems/1/3 |
| 3 to 100 MHz (?) | Oblique ionosphere sounding (OIS) | T/R | Australia |  | Extends to “Lower VHF”. |
| 7-300 MHz (?) | Ionospheric research radars | T/R | Australia | See SUPERDARN consortium | “HF-VHF” |
| 38.2 MHz | Ionosphere D-Region absorber | R | China | Heilongjiang Mohe Heilongjiang Jia Musi Beijing Lingsan Hainan Tunchan | Protection: -no radio emission within 1km in these frequencies - no high voltage power line within 100m |
| 1000-2000 MHz | GNSS receivers | R | Australia |  | See the GNSS frequencies: http://www.positim.com/gnss_freqs.jpg |
| 1.1 to 1.6 GHz mainly 1.22-1.57 GHz | GNSS ionospheric monitoring | R | Belgium | ROB, Brussels for an European network | http://gnss.be |
| 1227.6 MHz 1246.0 MHz 1575.42 MHz 1603.0 MHz 2491.75 MHz | Ionospheric scintillation receiver | R | China | Guangdong Maoming Guangdong Guangzhou Fujian Xiamen Guangdong Shaoguan Hubei Wuhan | Protection: - no mobile phone base station within 1km - no high voltage power line within 100m |
| II. Solar Observations |  |  |  |  |  |
| 10-80 MHz | Decameter Array (including HF spectrograph) | R | France | Nançay (47°N, 2°E) | Used for astronomic and solar corona observations http://secchirh.obspm.fr/instruments.php |
| 18-1800 MHz | Solar Radio Spectrograph | R | Australia | Culgoora observatory (Narrabri) | Need quiet environment within at least 30 km, preferably 200-300 km. Risk of interference with aircrafts, ground repeaters, or remote ground sources in case of temperature inversion over sea. http://www.ips.gov.au/Solar/2/2 |
| 25-180 MHz | Solar Radio Spectrograph | R | Australia | Learmonth | Same comment as Culgoora http://www.ips.gov.au/Solar/3/2 |
| 45-450 MHz | e-Callisto spectrograph | R | Belgium | Humain | Monitoring solar flares and CMEs |
| 20-650 MHz | Solar Radio Spectrograph | R | Greece | Thermopyle (38.47°N, 22.41°E) | http://secchirh.obspm.fr/instruments.php |
| 300-3000 MHz | Solar Radio Spectrograph | R | Belgium | Humain | In commissioning http://sidc.be/humain |
| 150-450 MHz | Radio Heliograph operating at 150, 236, 327, 410 and 432 MHz | R | France | Nançay (47°N, 2°E) | http://secchirh.obspm.fr/instruments.php (Operates typically 8:00-16:00 UTC) |
| 245 MHz – 15.4 GHz | Solar radio flux monitoring at 245, 410, 610, 1415, 2695, 4995, 8800, 15400 MHz | R | Australia | Learmonth | http://www.ips.gov.au/Solar/3/4 Solar Electro-Optical Network (SEON) |
| 245 MHz – 15.4 GHz | Solar radio flux monitoring at 245, 410, 610, 1415, 2695, 4995, 8800, 15400 MHz | R | USA | Sagamore Hill, Mass. | Solar Electro-Optical Network (SEON) |
| 245 MHz – 15.4 GHz | Solar radio flux monitoring at 245, 410, 610, 1415, 2695, 4995, 8800, 15400 MHz | R | USA | Palehua, Hawaï | Solar Electro-Optical Network (SEON) |
| 245 MHz – 15.4 GHz | Solar radio flux monitoring at 245, 410, 610, 1415, 2695, 4995, 8800, 15400 MHz | R | Italy | San Vito | Solar Electro-Optical Network (SEON) |
| 1400-1427 MHz (*) 1660-1670 MHz (*) 2750-2850 MHz (**) 4.990-5.000 GHz (*) 8.275-8.375 GHz 10.600-10.700 GHz(*) | Solar radio flux monitoring | R | Canada | Dominion Radio Astrophysical Observatory (DRAO) near Penticton, 49.32° N 119°62° W | (*): Radio-astronomy band (**): 10.7 cm solar radio flux. This frequency lies in a band allocated by ITU to the Radiolocation Service (radar) http://www.nrc-cnrc.gc.ca/eng/solutions/facilities/drao.html |
| 1400-1427 MHz (*) 1660-1670 MHz (*) 2750-2850 MHz (**) 4.990-5.000 GHz (*) 8.275-8.375 GHz 10.600-10.700 GHz(*) | Solar radio flux monitoring | R | Belgium | Humain (Planned) | Similar to Canada / DRAO |
| 2801 ±5% MHz (**) 4542 ±5% MHz 9084 ±5% MHz | Solar radio telescope | R | China | Shandong Shidao | Protection: no radio emission within 1km in these frequencies. (**) 10.7 cm solar radio flux, see above. |

Note: The table above summarizes the outcome of a preliminary survey responded by several ICTSW members. It is thought to be a representative indication of the main frequencies used for space weather but does not pretend to be exhaustive.

4.  Discussion

While many of the frequencies used for solar observations lie in bands allocated to Radio Astronomy, it is not the case of the 2750-2850 MHz frequency band, providing the 10.7 cm solar radio flux, which is used by several solar observatories around the world as an internationally agreed solar activity index since more than sixty years.  It is of utmost importance to preserve continuous observations in this data series at those solar observatories.

This 2 750-2 850 MHz band is part of the 2 700 - 2 900 MHz band allocated on a primary basis to the aeronautical radio-navigation service, meteorological ground based radars, and on a secondary basis to the radiolocation service.  This band is now under consideration within ITU by the Joint Task Group 4-5-6-7, looking for additional spectrum for the mobile service for IMT applications, and will be addressed by WRC-15 under agenda item 1.1.

5.  Proposed Action (by SG-RFC)

SG-RFC is invited to note the outcome of a preliminary survey of the use of radio-frequencies for ground-based observations of the Sun and the ionosphere in support of space weather activities.

SG-RFC is invited to include in the WMO position for WRC15 item 1.1 the use of the 2750-2850 MHz (10.7 cm) frequency range by solar observatories.

6. Draft Text for Inclusion in the SG-RFC Meeting Reports

(For meeting report)

SG-RFC noted the important use of radio-frequencies for ground-based observations of the Sun and the ionosphere in support of space weather activities.  It invited the ICTSW to keep under review the current and planned use of the radio-frequency spectrum for space weather and to keep the SG-RFC informed accordingly.

The session highlighted the critical need to protect the use of the 2750-2850 MHz frequency range for 10.7 cm solar radio flux measurement at solar observatories, and agreed to amend the WMO position for WRC accordingly.

(For the WMO position paper Doc.04, about agenda item 1.1)

-	2 700-2 900 MHz used for meteorological radars and for solar activity monitoring.

____________________

* If restricted access is selected the WMO Document will only be accessible to the WMO WIKI registered users.