| ## WORLD WEATHER WATCH ## COMMISSION FOR BASIC SYSTEMS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Steering Group on Radio Frequency Coordination (SG-RFC) Boulder, Colorado. USA. 11-13 March 2014. |  | Document SG-RFC/2014-1-25 |  |  |
|  |  | 02 March 2014 |  |  |
|  |  | English only |  |  |
|  |  | Restricted access required? (Y/N)* | N |  |
| STEERING GROUP ON RADIO FREQUENCY COORDINATION (SG-RFC) |  |  |  |  |
| Chair sg-rfc |  |  |  |  |
| Short Report on the WRC-15 Preparation Workshop (Dec 2013) |  |  |  |  |

Introduction

The first ITU Inter-regional Workshop on WRC-15 Preparation was held in Geneva from 4-5 December 2013. The main objective was the presentation of the on-going ITU-R preparatory studies for WRC-15 by the responsible groups, as well as on up-to-date information regarding the Bureau and regional preparations for CPM15-2, RA-15 and WRC-15, providing participants with the opportunity to exchange views and have a better understanding of the draft preliminary common views, positions and/or proposals of the concerned entities.

The document on the WMO Preliminary Position on WRC-15 Agenda was submitted and presented or discussed during the presentation of concerned agenda items. It was a good opportunity to repeat WMO concerns and maintain its tradition of active participation in ITU radio frequency coordination process.

The presentations from the regional groups showed that they were still discussing most of the agenda items and that some more time would be needed for the consolidation of most regional positions. In general, they supported the current studies, to some extent indirectly supporting WMO positions.

Two additional WRC-15 Preparation Workshop are planned for Q4 2014 and Q3 2015, when new interactions with regional groups will be more specific as their positions will be consolidated on most agenda items. WMO publicly stated the importance of the Workshop, as an excellent opportunity to have all regional groups together as well as the presentation of on-going studies by ITU-R groups. As the preparations for WRC-15 involves so many meetings, this type of workshop allows better vision of the whole situation.

In the Annexes, a timeline of main milestones leading to WRC-15, a table with bands of WMO concern and responsible ITU-R groups and tentative dates of scheduled meetings. Also a table a summary of regional positions to be updated with new reports during the meeting. The documentation plan for this meeting provide a link to the regional groups presentations during the preparation Workshop. As the information is very dynamic, tables and documents must be constantly reviewed and updated. Some meetings of concerned ITU-R groups were held after the Workshop.

Action (by SG-RFC) Proposed (for example, note the information, study the matter, etc.)

For Information

Draft Text for Inclusion in the SG-RFC Meeting Reports or Other Documents (indicate the appropriate document)

None

JTG4567

4A

4C

5B

7B

7C

CPM15-2

WRC-15

Deadline for

Submission of

Texts to CPM

Chapter Rap-

porteurs

15 Aug 2014

Draft CPM Report

Sept 2014

CPM Report

Aug 2015

2-27 Nov 2015

23 Mar-2 Apr 2015

1.17

1.6

1.9.2

1.10

9.1.1

1.11

1.12

1.1

Two WRC-15 Preparation Workshops are planned for

Q4 2014 and Q3 2015

| Agenda Item | Brief Description | WMO Concern | Responsible Group/Next meetings | Status |
| --- | --- | --- | --- | --- |
| 1.1 | Mobile Broadband & IMT | 1 400-1 427 MHz 1 675-1 710 MHz 2 025–2 110 MHz & 2 200–2 290 MHz 2 700-2 900 MHz 3 400-4 200 MHz 5 350-5 470 MHz | JTG4567 20-28 Feb 2014 | Estimated additional total spectrum requirements for IMT by the year 2020 ranges from 1340 MHz to 1960 MHz From WP 5A - The minimum spectrum requirement for non-IMT mobile broadband applications using the 5 GHz frequency range in the year 2018 is estimated at 880 MHz Some of the bands of WMO Concern are in the list of Potential Candidates bands |
| 1.6 | FSS @ 10-17 GHz | 10.6-10.7 GHz EESS (passive) 13.25-13.75 GHz EESS (active) | WP 4A 5-13 Feb 2014 | Draft CPM text describes testing and results to date No methods have been determined yet. |
| 1.9.2 | MMSS @ 7/8 GHz | 7 450-7 550 MHz (space-to-Earth) 8 025-8 400 MHz (space-to-Earth) | WP 4C 13-19 Feb 2014 | Latest draft CPM text: Doc. 4C/239, Annex 10 • Method 1: There would be no allocation to the MMSS within the 7 375-7 750 MHz and 8 025-8 400 MHz bands and therefore no change to the Radio Regulations. • Method 2: to be defined |
| 1.10 | MSS @ 22-26 GHz | 23.6-24 GHz (EESS passive) 25.5 – 27.0 GHz (First 500 MHz EESS/SRS space-to-Earth) | WP 4C 13-19 Feb 2014 |  |
| 1.11 | EESS @ 7-8 GHz | 7 450-7 550 MHz 7 750-7 900 MHz | WP 7B 6-13 May 2014 | Article 21 is modified to extend the range 7 145-7 235 MHz to 7 145-7 250 MHz Appendix 7 is modified to include the new EESS allocation. |
| 1.12 | EESS +600 MHz @ 9GHz | 9 300-9 500 MHz (Met radars) 10.6-10.7 GHz (Passive sensors) | WP 7C 6-13 May 2014 | Methods A and B are proposed to satisfy agenda item |
| 1.17 | UAIC aeronautical | 2 700-2 900 MHz 5 350-5 460 MHz 13.25-13.4 GHz EESS (active) | WP 5B 19-30 May 2014 | Method A: Allocation to AM(R)S in the frequency band 4200-4400 MHz |
| 9.1.1 | Protection of 406-406.1 MHz | 406-406.1 MHz (COSPAS/SARSAT) | WP 4C 13-19 May 2014 | The current version of the Draft CPM text also mentions that a WRC Resolution may be developed with a view of having an adequate protection of the MSS in the band 406-406.1 MHz in order to detect and successfully process 406 MHz distress signals taking into account the current and future deployment of services in adjacent bands. |
| 9.1.5 | FSS earth stations @ 3400-4200 MHz | 3 400-4 200 MHz (FSS Region 1 - VSAT) | SC 5-13 Feb 2014 | Draft CPM text: Doc. 4A/343, Annex 38 Current Draft CPM text includes specific suggestions to address this issue |
| 10 | Items for next WRC | Agenda items & positions as appropriate | CPM15-2 23 Mar-2 Apr 2015 |  |

SUMMARY OF REGIONAL POSITIONS (To be completed)

| Agenda Item | APT | ASMG | ATU | CEPT | CITEL | RCC |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  |  |  |  |  |  |
| 1.6 |  |  |  |  |  |  |
| 1.9.2 |  |  |  |  |  |  |
| 1.10 |  |  |  |  |  |  |
| 1.11 |  |  |  |  |  |  |
| 1.12 |  |  |  |  |  |  |
| 1.17 |  |  |  |  |  |  |
| 9.1.1 |  |  |  |  |  |  |
| 9.1.5 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |

* If restricted access is selected the WMO Document will only be accessible to the WMO WIKI registered users.