| ## WORLD WEATHER WATCH ## COMMISSION FOR BASIC SYSTEMS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Steering Group on Radio Frequency Coordination (SG-RFC) Boulder, Colorado. USA. 11-13 March 2014. |  | Document SG-RFC/2014-1-17 |  |  |
|  |  | 02 March 2014 |  |  |
|  |  | English only |  |  |
|  |  | Restricted access required? (Y/N)* | N |  |
| STEERING GROUP ON RADIO FREQUENCY COORDINATION (SG-RFC) |  |  |  |  |
| chair sg-rfc |  |  |  |  |
| Inter-American Telecommunication Commission (CITEL) |  |  |  |  |

Introduction

In its working structure, CITEL has a Working Group for the Preparation of CITEL for Regional and World Radiocommunication Conferences.  It is part of the Permanent Consultative Committee II (PCC.II): Radiocommunication including Broadcasting.

The positions of CITEL have not changed much since its presentation in the ITU Inter-regional Workshop on WRC-15 Preparation, in December 2013 (See Annex).

It is expected that CITEL will consolidate positions on many items during the coming XXIII Meeting of PCC.II that will be held from 17-21March 2014 in Cartagena de Indias, Colombia.

Action (by SG-RFC) Proposed (for example, note the information, study the matter, etc.)

For information

Draft Text for Inclusion in the SG-RFC Meeting Reports or Other Documents (indicate the appropriate document)

None

ANNEX

INTER–AMERICAN PROPOSALS FOR WORLD RADIOCOMMUNICATION CONFERENCES: DEFINITIONS

•PRELIMINARY VIEWS (PV): an informal statement that the Administration is considering possible Preliminary Proposals on specific themes.

•PRELIMINARY PROPOSAL (PP): a proposal that a CITEL Member State presents to PCC II with the purpose to turning it into an Inter-American Proposal and that has not been supported by another Member State.

•DRAFT INTER-AMERICAN PROPOSAL (DIAP): PRELIMINARY PROPOSAL that has been supported by at least one other Member State.

•INTER-AMERICAN PROPOSAL (IAP): DRAFT INTER-AMERICAN PROPOSAL (DIAP) for which the PCC II has ended its consideration and discussion, has been supported by at least six Members States and is not opposed by more than 50% of the number of supports obtained.

Positions by CITEL during the ITU Workshop (December 2013) on items of main WMO concerns

Agenda Item 1.1: IMT/terrestrial mobile broadband Preliminary Views

| Frequency Range (MHz) | Support Studies/Identification | Oppose |  |
| --- | --- | --- | --- |
| General CAN: Studies |  |  |  |
| 410-430 | CLM: Studies B: Identification | ARG |  |
| 470-698 | CLM: Studies CAN/USA: Identification | ARG/B |  |
| 1164-1215, 1215-1300, 1559-1610 |  |  | CAN/USA |
| 1300-1525 | B/CLM: Studies/Possible Identification (B: only in 1350-1525 MHz) CAN: Studies/Possible Identification, 1427-1525 MHz MEX: Studies/Possible Identification, 1452-1492 MHz | USA: for 1435-1525 MHz |  |

| 1695-1710 | CAN/USA: Studies/Possible Identification, 1695-1710 MHz |  |
| --- | --- | --- |
| 2700-2900 | B: Studies |  |
| 3400-3600 | B: Identification with technical conditions for sharing with FSS above 3600 MHz CLM: Studies/Possible Identification |  |
| 3600-4200 | CLM: Studies | ARG: for 3700-4200 B |
| 5350-5470 | USA: Identification for RLANs if studies show feasibility |  |
| 5925-6425 | B/NCG |  |

Agenda Item 1.1: IMT/terrestrial mobile broadband Status of the Preliminary Proposals, Draft Inter-American Proposals and Inter-American Proposals

| Frequency Range (MHz) |  | MOD Mobile allocation in Article 5 | NOC |
| --- | --- | --- | --- |
| 420-450 | Preliminary Proposal USA |  |  |
| 470-698 |  | Draft – Inter-American Proposal CAN/CTR | Inter-American Proposal1 ARG/B/DOM/EQA/NCG/PNR |
| 3400-3600 |  | Draft – Inter-American Proposal B/CTR: with IMT identification | Preliminary Proposal NCG |
| 3600-4200 | Draft – Inter-American Proposal B/NCG/MEX |  |  |
| 4500-4800 | Preliminary Proposal NCG |  |  |

Agenda Item 1.6.2: FSS (E-s) 250 MHz in Region 2 and 300 MHz in Region 3 in 13-17 GHz

Preliminary Views

Brazil/Canada/United States/Mexico

- Support studies towards consideration of possible additional primary allocations to the fixed-satellite service (FSS) (Earth-to-space) of 250 MHz in Region 2 and 300 MHz in Region 3 within the range 13-17 GHz and review the regulatory provisions on the current allocations to the fixed-satellite service within this range, in accordance with Resolution 152 (WRC-12), while protecting existing primary services in the band(s).

Agenda Item 1.9.2: MMSS 7 375-7 750 MHz and 8 025-8 400 MHz

Preliminary Views

Canada/United States

- If ITU-R studies demonstrate compatibility with incumbent services, including the adjacent SRS (space-to-Earth) allocation in the band 8 400-8 450 MHz, which is limited to deep space, these administrations will consider supporting allocations to the MMSS in the bands 7 375-7 750 MHz and 8 025-8 400 MHz, or portions thereof;

- ITU-R studies should determine appropriate measures (e.g., exclusion zones, radiation limits etc.), that will be needed to ensure protection of existing services in the bands 7375 – 7750 MHz and 8025 - 8400 MHz.

Agenda Item 1.10: MSS, including the satellite component for broadband applications, including IMT 22 GHz to 26 GHz

Draft Inter-American Proposals

Canada/United States

• NOC to Article 5

• SUP Resolution 234 (WRC-12)

Agenda Item 1.11: EESS (E-s) 7-8 GHz

Draft Inter-American Proposals

Canada/United States

- MOD Article 5 to add EESS (Earth-to-space) in the 7 190-7 250 MHz band and divide the Table of Frequency Allocation at 7 190 MHz to clarify the allocation of services within the Table.

- MOD No. 5.460 - consequential to dividing the Table at 7 190 MHz

- MOD Article 21 Tables 21-2 and 21-3 - consequential

- MOD Appendix 7 Table 7b - consequential

- SUP Resolution 650 (WRC-12)

Agenda Item 1.12: EESS (active) up to 600 MHz extension within 8 700-9 300 MHz and/or 9 900-10 500 MHz Preliminary Views

Brazil/Canada/United States

- Support studies that would lead to the potential extension of the current EESS (active) allocation in the frequency band 9 300-9 900 MHz by 600 MHz;

- Support the ITU-R study results on the EESS spectrum requirements which demonstrate that 1 200 MHz of contiguous spectrum is necessary;.

- Compatibility with existing services will have to be ensured, in accordance with the appropriate protection criteria and taking into account any available mitigation techniques that would reduce the level of unwanted emissions into adjacent band.

United States

- Only if studies prove that existing services cannot be protected and/or sufficient spectrum cannot be made available in the 9 900 MHz – 10.5 GHz range, does the United States support consideration of the 8 700-9 300 MHz range.

Agenda Item 1.17: Wireless avionics intra-communications

Preliminary Views (Brazil/Canada/United States)

- Support regulatory actions, including appropriate allocations to the AM(R)S limited to WAIC systems within existing worldwide AMS, AM(R)S and/or ARNS allocations below 15.7 GHz, providing ITU-R studies show compatibility with existing services in accordance with Resolution 423 (WRC-12). Those studies should consider frequency bands above 15.7 GHz if spectrum requirements cannot be met in existing worldwide AMS, AM(R)S and/or ARNS allocations below 15.7 GHz.

Preliminary Proposals (Brazil)

- MOD Article 5 to add AM(R)S allocation at 4200-4400 MHz and new footnote restricting use to WAIC

- MOD No. 5.438 to remove passive EESS and SRS secondary use

- SUP No. 5.440 to remove the use of 4202 MHz for standard frequency and time-signal satellite service

Agenda Item 9.1.1: Protection of the systems operating in the MSS in 406-406.1 MHz Preliminary Views

Canada/United States

- Supports the ongoing ITU-R studies with a view of having an adequate protection of the MSS band 406-406.1 MHz in order to detect and successfully process 406 MHz distress signals, which is vital to search and rescue missions.

Agenda Item 9.1.5 - ????

Agenda Item 10: Agenda Items for Future Conferences

Preliminary Proposals

Canada

- Suppress existing Resolution 808 (WRC-12) and replace it with a new Resolution for the WRC-18 agenda.

* If restricted access is selected the WMO Document will only be accessible to the WMO WIKI registered users.