| ## WORLD WEATHER WATCH ## COMMISSION FOR BASIC SYSTEMS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Steering Group on Radio Frequency Coordination (SG-RFC) Payerne, Switzerland. 22-25 September 2015. |  | Document SG-RFC/2015-1-08 |  |  |
|  |  | 31 August 2015 |  |  |
|  |  | English only |  |  |
|  |  | Restricted access required? (Y/N)* | N |  |
| STEERING GROUP ON RADIO FREQUENCY COORDINATION (SG-RFC) |  |  |  |  |
| David FRanc (USA) |  |  |  |  |
| CITEL IAP – WRC -15 Agenda Item 10 Proposal for WRC-19 Conference Agenda Item – Primary Allocation for the Meteorological Satellite Service in 460-470 MHz |  |  |  |  |

Introduction

To protect the recent significant investment and expansion of the ADCS systems, this proposal advocates studying sharing between the existing meteorological-satellite (space-to-Earth) service and incumbent services in the 460-470 MHz band with a view to upgrading the meteorological-satellite service to primary and potentially adopting a pfd limit on the meteorological-satellite (space-to-Earth) service to protect the incumbent services.  A co-primary allocation status would protect the ADCS from any new services entering the band.  Attached is a CITEL Inter-American proposal (IAP) that proses to place this issue on the agenda of WRC-19.

X. Action (by SG-RFC) Proposed

This proposal is presented to the September 2015 SG-RFC meeting for discussion.  SG-RFC members are encouraged to coordinate support of this proposal within their administrations and regional groups.

Y. Draft Text for Inclusion in the SG-RFC Meeting Reports or Other Documents

Dependent upon meeting discussion.

| World Radiocommunication Conference (WRC-15) Geneva, 2–27 November 2015 |  |
| --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |
| PLENARY MEETING | Addendum 24 to Document 4843-E |
|  | 11 August 2015 |
|  | Original: English |
| Member States of the Inter-American Telecommunication Commission (CITEL) |  |
| Proposals for the work of the conference |  |
| Agenda item 10 |  |

10	to recommend to the Council items for inclusion in the agenda for the next WRC, and to give its views on the preliminary agenda for the subsequent conference and on possible agenda items for future conferences, in accordance with Article 7 of the Convention,

Introduction

# BAND 460-470 MHz

The 460-470 MHz band is allocated on a primary basis to the fixed and mobile services. The meteorological-satellite service currently has a secondary allocation in this band.  Within this band, the Argos Data Collection System (ADCS) is used to monitor over 21,000 individual platforms around the globe for 1,900 operators in 118 countries.  Critical applications of the ADCS include atmospheric & ocean monitoring/research, tropical cyclone forecasting, fishery management, oil spill tracking, fishing vessel tracking, search & rescue modeling (at sea), anti-piracy alerting, import/export & hazardous materials tracking, endangered species studies, migration mapping, and wildlife tracking and management.

RF Central Station Alarm (CSA) systems operate on the same frequency as the ADCS downlink. Due to the potential for interference to the CSA systems, the operator turned off the ADCS on the NOAA-19 satellite. To provide additional  protection to the existing primary services in the band, the next generation of ADCS transmitters will implement a direct sequence spread spectrum in the satellite downlink to reduce the power flux density (pfd)  in the 460-470 MHz band to flux < -152 dBW/m2/4kHz.

To protect the recent significant investment and expansion of the ADCS systems, this proposal advocates studying sharing between the existing meteorological-satellite (space-to-Earth) service and incumbent services in the 460-470 MHz band with a view to upgrading the meteorological-satellite service to primary and potentially adopting a pfd limit on the meteorological-satellite (space-to-Earth) service to protect the incumbent services.  A co-primary allocation status would protect the ADCS from any new services entering the band.

ADD	IAP/4843A24/1

Draft New Resolution [IAP-A10-AI-Title-1.XX] (WRC-15)

Agenda for the 2019 World Radiocommunication Conference

The World Radiocommunication Conference (Geneva, 2015),

…

resolves

…

1 X.X	to review the allocations to the meteorological-satellite service in the 460-470 MHz band with a view to upgrade the secondary meteorological-satellite service allocation to primary status while protecting the existing primary services in the band, and without imposing any additional constraints on these primary services in accordance with Resolution [IAP-B10-Res-for-AI-1.XX] (WRC-15);

Reasons:

To allow meteorological-satellite service to operate on a co-primary status with fixed and mobile.

ADD	IAP/4843A24/2

Draft New Resolution [IAP-B10-Res-for-AI-1.XX] (WRC-15)

Primary Allocation to the Meteorological Satellite Service in the 460 – 470 MHz Band

The World Radiocommunication Conference (Geneva, 2015),

considering

a)	that the Argos Data Collection System (ADCS) is used to monitor over 21,000 individual platforms around the globe for 1,900 operators in 118 countries;

b)	that the use of ADCS can provide spectrum efficiency by interrogating collection platforms prior to their transmission;

c)	that ADCS may be authorized to operate on a secondary basis with respect to the fixed and mobile services,

recognizing

a)	that technology has been developed to provide more efficient use of the spectrum;

b)	that a digital spread spectrum scheme will be used to increase mitigation of potential interference to incumbents in the band;

c)	that the satellite-to-platform down-link at 465.9875 MHz significantly improves platform and system performance, including data latency and battery life;

d)	that due to the significant investment and expansion of ADCS, future conflicts or interference issues must be avoided,

e) 	that the 460-470 MHz band is allocated to the fixed and mobile services on a primary basis in all Regions;

f)	that No. 5.286AA identifies the frequency band 460-470 MHz for the use by administrations wishing to implement International Mobile Telecommunications (IMT);

g) 	that the meteorological-satellite service ground receivers will not claim protection from stations in the primary fixed and mobile services.

h)	that at least one administration has adopted national regulatory provisions providing a pfd limit at the Earth’s surface of -152 dBW/m2/4 kHz for protecting the fixed and mobile services in the band,

resolves

that taking into account the results of ITU-R studies, WRC-19 consider upgrading the current secondary allocation of the meteorological-satellite service (space-to-Earth) to primary in the 460-470 MHz frequency band without imposing any additional constraints on existing primary  services in the band and establish a pfd limit that shall not exceed -152 dBW/m2/4kHz at the Earth’s surface to protect these primary services.

resolves to invite the ITU-R

1.	to conduct in time for WRC-19, sharing and compatibility studies between the meteorological-satellite service (space-to-Earth) and the primary fixed and mobile services in the band 460-470 MHz and in adjacent bands as appropriate;

2	to complete the studies, taking into account the present use of the allocated band to determine the appropriate power flux density limit to be placed on the meteorological-satellite service (space-to-Earth) to protect the existing primary services in the band that shall not exceed -152 dBW/m2/4kHz at the Earth’s surface;

invites administrations

to participate actively in the studies and provide the technical and operational characteristics of the systems involved by submitting contributions to the ITU-R,

instructs the Secretary General

to bring this resolution to the attention of the Space Frequency Coordination Group (SFCG) and other international and regional organizations concerned.

Reasons:  A resolution will support the ITU-R studies needed under the relevant WRC-19 agenda item.

SUP	IAP/4843A24/3

RESOLUTION 806 (WRC-15)

Preliminary agenda for the 2018 World Radiocommunication Conference

Reasons:

Annex 1

PROPOSAL FOR ADDITIONAL AGENDA ITEM STUDYING THE ALLOCATION OF THE METEOROLOGICAL-SATELLITE SERVICE IN THE 460-470 MHZ BAND

Subject: Proposed Future WRC Agenda Item for WRC-2018 studying meteorological-satellite service in the 460-470 MHz band

Proposal: To review the services in the 460-470 MHz band with a view to upgrading the meteorological satellite service to primary status while protecting the existing primary services in the band, and without imposing any additional constraints on these primary services in accordance with Resolution AAA (WRC-15).

Background/reason:

The 460-470 MHz band is allocated on a primary basis to the fixed and mobile services. The meteorological-satellite service currently has a secondary allocation in this band.  Within this band, Argos Data Collection System (ADCS) equipment on meteorological-satellites is used to monitor over 21,000 individual data collection platforms around the globe for 1,900 operators in 118 countries.  Critical ADCS supported applications include atmospheric & ocean monitoring/research, tropical cyclone forecasting, fishery management, oil spill tracking, fishing vessel tracking, search & rescue modeling (at sea), anti-piracy alerting, import/export & hazardous materials tracking, endangered species studies, migration mapping, and wildlife tracking and management.

Radiocommunication services concerned: Meteorological Satellite Service, Fixed Service, Mobile Service, Earth Exploration Satellite Service

Indication of possible difficulties: None foreseen

Previous/ongoing studies on the issue: None to date

| Studies to be carried out by: SG7 | with the participation of: |
| --- | --- |

ITU-R Study Groups concerned: SG5

ITU resource implications, including financial implications (refer to CV126): Minimal

Common regional proposal:  Yes/No	Multicountry proposal:  Yes/No

Number of countries:

Remarks

* If restricted access is selected the WMO Document will only be accessible to the WMO WIKI registered users.