WORLD METEOROLOGICAL ORGANIZATION

[image: image4.png]



Manual on 
WMO Information System (WIS)
Volume I 
(Annex VII to the WMO Technical Regulations)
2012 edition


Basic Documents No. 2

Secretariat of the World Meteorological Organization-Geneva-Switzerland

CONTENTS

Manual on the WMO INFORMATION SYSTEM (WIS)

41
Introduction

1.1
Purpose of this Manual
4
2
Organization and Responsibilities
4
2.1
Organization of WIS
4
2.2
Compliance with Required WIS Functions
4
2.3
Interaction among WIS Centres
4
2.4
Implementation of WIS
4
2.5
Discovery, Access and Retrieval Function
4
2.6
Robustness and Reliability of Components
5
2.7
Collection and Dissemination Services
5
3
Designation Procedures for WIS Centres
5
3.1
General
5
3.2
Procedure for a Global Information System Centre (GISC)
5
3.3
Procedure for a Data Collection or Production Centre (DCPC)
6
3.4
Procedure for a National Centre (NC)
7
4
Functions of WIS
7
4.1
Roles in and Review of WIS Functions
7
4.2
List of WIS Functions
7
4.3
Functional Architecture of WIS
7
4.4
Data Flow among WIS Functions
7
4.5
Functional Requirements of a GISC
7
4.6
Functional Requirements of a DCPC
9
4.7
Functional Requirements of an NC
10
5
Technical Specifications of WIS
11
5.1
General
11
5.2
WIS-TechSpec-1: Uploading of Metadata for Data and Products
11
5.3
WIS-TechSpec-2: Uploading of Data and Products
12
5.4
WIS-TechSpec-3: Centralization of Globally Distributed Data
12
5.5
WIS-TechSpec-4: Maintenance of User Identification and Role Information
12
5.6
WIS-TechSpec-5: Consolidated View of Distributed Identification and Role Information
13
5.7
WIS-TechSpec-6: Authentication of a User
13
5.8
WIS-TechSpec-7: Authorization of a User Role
13
5.9
WIS-TechSpec-8: DAR Catalogue Search and Retrieval
13
5.10
WIS-TechSpec-9: Consolidated View of Distributed DAR Metadata Catalogues
13
5.11
WIS-TechSpec-10: Downloading Files via Dedicated Networks
14
5.12
WIS-TechSpec-11: Downloading Files via Non-dedicated Networks
14
5.13
WIS-TechSpec-12: Downloading Files via Other Methods
14
5.14
WIS-TechSpec-13: Maintenance of Dissemination Metadata
14
5.15
WIS-TechSpec-14: Consolidated View of Distributed Dissemination Metadata Catalogues
14
5.16
WIS-TechSpec-15: Reporting of Quality of Service
14
APPENDIX A – List of Abbreviations
16
APPENDIX B – Selected WMO Documents Relevant to WIS
17
APPENDIX C – Approved WIS Centres
19
APPENDIX C.1 Global Information System Centres (GISCs)
19
APPENDIX C.2 Data Collection or Production Centres (DCPCs)
19
APPENDIX C.3 National Centres (NCs)
19


1 Introduction
1.1 Purpose of this Manual
1.1.1 The Manual on WIS is designed to ensure adequate uniformity and standardization in the data, information, and communications practices, procedures and specifications employed among WMO Members in the operation of WIS as it supports the mission of WMO. The Manual on WIS sets out standard practices, procedures and specifications (distinguished by the term "shall") having the status of requirements in a technical resolution, which it is necessary that WMO Members follow or implement. The Manual on WIS also sets out recommended practices, procedures and specifications (distinguished by the term "should") which Members are urged to comply with. 
1.1.2 The Manual on WIS is Annex VII to the Technical Regulations (WMO‑No. 49), Volume I (General meteorological standards and recommended practices), where it is stated that WIS is established and operated in accordance with practices, procedures and specifications in the Manual on WIS. 
1.1.3 Because WIS cross cuts all WMO related discipline areas, many other WMO practices, procedures and specifications intersect WIS. Such practices, procedures and specifications are primarily defined in their specific publications, for example, the Manual on Global Data Processing and Forecast System (WMO‑No. 485), and the Manual on Global Observing Systems (WMO‑No. 488), among others.
2 Organization and Responsibilities
2.1 Organization of WIS

2.1.1 In keeping with WMO‑No. 49, Vol I, A.3, Centres operated by WMO Members and their collaborating organizations shall be categorized as one of the three types of WIS Centres forming the core infrastructure of WIS:
  -
Global Information System Centres (GISCs), 

  -
Data Collection or Production Centres (DCPCs), 

-
National Centres (NCs).

Refer to 4, Functions of WIS, for the distinct functions of the three types of WIS Centre (GISC, DCPC, NC).

2.1.2 Each Permanent Representative with WMO shall have the responsibility to authorize users of WIS.  Authority to manage the process may be delegated.

2.2  Compliance with Required WIS Functions

WIS Centres shall maintain compliance with required WIS functions. The Manual on WIS serves as instruction on practices, procedures and specifications for WIS functions, supplemented by additional information concerning practices, procedures and specifications for WIS functions set out in the Guide to WIS (WMO‑No. 1061). 
2.3 Interaction among WIS Centres

GISCs shall connect to other GISCs by the WIS Core Network, based on the Main Telecommunication Network (MTN). Data, products and metadata shall flow to a GISC from DCPCs and from NCs within its area of responsibility. An Area Meteorological Data Communication Network (AMDCN) shall connect each GISC to DCPCs and NCs in its area of responsibility. An AMDCN may span multiple Regional Meteorological Telecommunication Networks (RMTNs) and parts of RMTNs.
2.4 Implementation of WIS

WIS shall be implemented in two parallel parts. One part of WIS is the continued evolution of the WMO Global Telecommunication System (GTS). This focuses on further improving the delivery of time-critical and mission-critical data, products, and services including warnings. The other part of WIS extends WMO services through Discovery, Access and Retrieval (DAR) facilities, as well as flexible timely delivery.

2.5 Discovery, Access and Retrieval Function
As required per WMO‑No. 49, Vol. I, A.3, WIS shall be based on catalogues that contain metadata describing data and products available across WMO, plus metadata describing dissemination and access options. The Discovery, Access and Retrieval (DAR) function of WIS shall be the primary realization of the WIS comprehensive catalogue, maintained collaboratively by all WIS Centres.
2.6 Robustness and Reliability of Components
High robustness and reliability of WIS components are essential to the operation of WIS. Indicators of performance shall be evaluated in the designation procedure for WIS Centres. That evaluation shall include assurance that data content flowing via WIS network technologies fully satisfies requirements for security, authenticity, and reliability. Some aspects of service levels are identified within this Manual on WIS. 

2.7 Collection and Dissemination Services
2.7.1 WIS shall provide three types of collection and dissemination services:

  -
Routine collection and dissemination service for time-critical and operation-critical data and products: This service is based on real-time “push” mechanism including multicast and broadcast; it should be implemented through dedicated telecommunication means providing a guaranteed quality of service.

 -
Discovery, Access and Retrieval (DAR) service: This service is based on request/reply “pull” mechanism with relevant data management functions; it is implemented through the Internet.

 -
Timely delivery service for data and products: This service is based on delayed mode “push” mechanism; it is implemented through a combination of dedicated telecommunication means and of public data telecommunication networks, especially the Internet.
2.7.2 WIS shall support the WMO virtual all-hazards network ensuring fast, secure and reliable exchange of alert and warning information, including ITU (International Telecommunication Union) Recommendation X.1303 (Common Alerting Protocol).
Note: The virtual all-hazards network encompasses all of the technical and operational arrangements necessary for the timely handling and delivery of alert and warning information involving WMO.
2.7.3 The goal of the WMO Integrated Global Data Dissemination Service (IGDDS) is to ensure the definition and operational implementation of an efficient circulation scheme of space-based observation data and products meeting the needs of WMO programmes, in the context of WIS. IGDDS shall remain an important component of WIS, mainly for the exchange and dissemination of data and products generated by space-based observing systems.
3 Designation Procedures for WIS Centres 
3.1 General

3.1.1 The establishment and operation of WIS depends on WMO Member and related organizations taking on the functional roles of GISCs, DCPCs and NCs. Procedures for designation as a WIS Centre rely on the agreed WIS Functional Architecture and the WIS Compliance Specifications. 

3.1.2 As required per WMO‑No. 49, Vol. I, A.3, Congress and Executive Council shall consider the designation of GISCs and DCPCs on recommendations of the Commission for Basic Systems (CBS). The development of the CBS recommendations includes consultation and coordination with the relevant Technical Commissions responsible for the WMO and related international programmes concerned and with Regional Associations as appropriate. Note: The relevant groups established by Executive Council have a role in the GISC and DCPCs designation process, in accordance with their mandate.

3.2 Procedure for a Global Information System Centre (GISC)

The procedure for the designation of a GISC shall consist of four steps:
 (1) Statement of WIS Requirements

 (2) Service Offer by a Member for a Potential GISC
 (3) Demonstration of GISC Capabilities

 (4) Designation of a GISC.

3.2.1 Statement of WIS Requirements

The WMO Technical Commissions and other bodies representing the participating programmes, including regional bodies, shall state their requirements for WIS services and review them periodically. The list of all relevant requirements shall be compiled and regularly reviewed by the Commission for Basic Systems (CBS), and reported to Executive Council.

3.2.2 Service Offer by a Member for a Potential GISC

3.2.2.1 A WMO Member can apply for designation of a centre as one of the GISCs forming the core infrastructure of WIS. The service offer shall include:

  -
A statement of compliance with the required WIS functions,

  -
A proposal for the area of responsibility for WIS services, and 

  -
A formal commitment by the PR of the Member that such services shall be provided on a routine basis and sustained over time.

3.2.2.2 The service offer shall be addressed to WMO. CBS, in consultation with the Regional Association(s) concerned; shall analyse the proposed service offer with regard to WIS requirements and compliance with GISC functions and specifications, and shall formulate a recommendation.

3.2.3 Demonstration of GISC Capabilities

3.2.3.1 The Member offering a GISC shall demonstrate to CBS the capabilities of the proposed centre to provide WIS services to the accredited users with the required reliability and quality. Compliance shall be demonstrated for: real-time functions of data and product collection and dissemination: non real-time services for requests: storage functions for the required set of data and products and relevant up-to-date metadata catalogues: coordination functions with other GISCs and the planning of mutual back-up services: adherence to WIS standards and relevant data exchange policies and access rights. 
3.2.3.2 A formal commitment and time schedule to implement the GISC and to provide GISC services in accordance with the offer shall be given by the PR of the Member operating the candidate GISC. 
3.2.3.3 Upon the demonstration of capabilities of the candidate GISC, CBS shall submit its recommendation on the GISC designation to the Congress or Executive Council.

3.2.4 Designation of a GISC

The list of GISCs as approved by Congress or Executive Council is included in Appendix C of this Manual on WIS.

3.3 Procedure for a Data Collection or Production Centre (DCPC)

3.3.1 WMO has determined that all WMO and related international programmes shall be served by WIS and therefore each established centre shall implement required WIS functions. CBS shall recommend how these centres are categorized as DCPCs within WIS. 
3.3.2 The procedure for the designation of a DCPC shall consist of three steps:
 (1) Service Offer by a Potential DCPC

 (2) Demonstration of DCPC Capabilities
 (3) Designation of a DCPC.

3.3.3 Service Offer by a Potential DCPC

3.3.3.1 Required DCPC functions should be undertaken by a centre that has been established under a WMO or related international programme, and/or a Regional Association. Accordingly, the relevant Technical Commission and/or Regional Association shall consider the service offers by Members for potential DCPCs, and shall endorse candidate DCPCs.

3.3.3.2 The service offer of a candidate DCPCs shall then be submitted to CBS. CBS shall analyse the compliance to the required DCPC functions and specifications, and formulate a recommendation.

3.3.4 Demonstration of DCPC Capabilities

3.3.4.1 The Member offering a DCPC will be invited to demonstrate to CBS the capabilities of the proposed centre to provide WIS services in compliance with the DCPCs functions and responsibilities, including proper synchronisation and communications with its associated GISC. Compliance shall be demonstrated, where applicable, for: real-time functions of data and product dissemination; non real-time services for requests: provision of relevant up-to-date metadata catalogues; coordination and synchronization functions with the associated GISC; adherence to WIS standards and relevant data exchange policies and access rights. 
3.3.4.2 Upon acceptance of the demonstration of capabilities of the candidate DCPC, CBS shall submit its recommendation for the DCPC designation to the Congress or Executive Council.

3.3.5 Designation of a DCPC

The list of DCPCs as approved by Congress or Executive Council is included in Appendix C of this Manual on WIS. Each DCPC entry includes the name of the associated GISC.
3.4 Procedure for a National Centre (NC)
3.4.1 As required per WMO No. 49, Vol. I, A.3, each NC shall use WIS to provide data and products, in accord with its programme responsibilities. These shall be provided with associated metadata, in accordance with WIS practices, procedures and specifications.  Each NC shall participate as appropriate in the relevant monitoring of the performance of WIS.
3.4.2 Each WMO Member shall notify WMO of the current name and location of each of its centres to be designated as an NC.  CBS, with involvement of relevant Regional Associations and with the assistance of the WMO Secretariat, shall review the Member designations, to ensure support of each NC by a GISC, DCPC, or other NC. The NCs designated by Members shall be included in the list of WIS Centres in Appendix C of this Manual on WIS. Each NC entry includes the name of the associated GISC.
4 Functions of WIS
4.1 Roles in and Review of WIS Functions 
An ongoing process for understanding user requirements, including quality of service, shall determine the functional scope and physical sizing of WIS, thereby to ensure that WIS continues to be responsive to the current and future needs of the supported programmes. All supported programmes and Technical Commissions shall participate in this process, which shall be part of general WMO requirements reviews. 
4.2 List of WIS Functions

4.2.1 WIS Centres collectively support the major WIS functions listed here:
1
Collect Observations, Generate Products, Create Metadata and Archive Information

2
Assign User Role

3
Maintain and Expose Catalogue of Services and Information

4
Authorise Access to Information by Users

5
Deliver Information to Users (Internal and External)

6
Manage System Performance

Note: WIS is concerned with data management and telecommunications aspects but the actual content of data and products is out of scope for WIS itself. Such content is a matter for the specific programme supported.
4.2.2 The required standard interfaces to these functions are described in the WIS Technical Specifications (section 5 in this Manual on WIS).

4.3 Functional Architecture of WIS

Note: Guide to WIS 4.3 references the Functional Architecture of WIS, provided as supplementary guidance for WIS Centres in a technical document. 
4.4 Data Flow among WIS Functions

Note: Guide to WIS 4.4 provides, as supplementary guidance for WIS Centres, a data flow model of the WIS functional architecture for the required WIS functions, illustrating a possible implementation of major WIS functions. 
4.5 Functional Requirements of a GISC

4.5.1 General
Note: The phrase "information intended for global exchange" encompasses time-critical and operation-critical information (data and products). This includes "essential data" and part of the "additional data", as specified in WMO Resolution 25 (Cg-XIII) and Resolution 40 (Cg-XII). 
4.5.2 Receive Information from GISC Area

4.5.2.1 Each GISC shall receive information intended for global exchange from NCs and DCPCs within its area of responsibility. This requirement also intersects the WIS Discovery Access and Retrieval (DAR) requirement, noted here following.

4.5.2.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.5.3 Exchange Information with Other GISCs

4.5.3.1 Each GISC shall exchange information intended for global exchange with other GISCs.  Each GISC shall collect such information from its area and exchange with other GISCs so that each GISC has such information from all GISCs. 
4.5.3.2 GISCs should employ the MTN and associated collaborative mechanisms to achieve the exchange efficiently, and without detriment to the performance of any GISC.
4.5.3.3 See also section 5.4, WIS-TechSpec-3 (Centralization of Globally Distributed Data).

4.5.4 Disseminate Information to GISC Area

4.5.4.1 Each GISC shall disseminate information to NCs and DCPCs within its area of responsibility, including but not limited to the information intended for global exchange. 
4.5.4.2 See also sections: 5.11, WIS-TechSpec-10 (Downloading Files via Dedicated Networks); 5.12, WIS-TechSpec-11 (Downloading Files via Non-dedicated Networks); 5.13, WIS-TechSpec-11 (Downloading Files via Other Methods).
4.5.5 Maintain 24 hour Cache

4.5.5.1 Each GISC shall hold the information intended for global exchange for at least 24 hours and make it available via WMO request/reply ("Pull") mechanisms. This requirement intersects the WIS Discovery Access and Retrieval (DAR) requirement, noted here following. 
4.5.5.2 See also sections: 5.4, WIS-TechSpec-3 (Centralization of Globally Distributed Data); 5.5, WIS-TechSpec-4 (Maintenance of User Identification and Role Information); 5.6, WIS-TechSpec-5 (Consolidated View of Distributed Identification and Role Information).

4.5.6 Discovery Access and Retrieval

4.5.6.1 In support of the Discovery, Access and Retrieval (DAR) function, each GISC shall maintain and provide access to a comprehensive catalogue of information across all WMO programmes encompassed by WIS.  This includes, but is not limited to, information intended for global exchange. To satisfy the DAR functional requirement, GISCs are required to support, in interactive and batch modes: upload, change and delete of metadata; user discovery of metadata; user access to metadata; and synchronization of the WIS comprehensive metadata catalogue with other GISCs. 
4.5.6.2 See also sections 5.9, WIS-TechSpec-8 (DAR Catalogue Search and Retrieval), and 5.10, WIS-TechSpec-9 (Consolidated View of Distributed DAR Metadata Catalogues).

4.5.7 Data Network Connectivity of GISC
Each GISC shall provide around-the-clock connectivity to the public and dedicated communication networks at a capacity that is sufficient to meet its global and regional responsibilities. Each GISC should assure the appropriate level of availability and capacity for every telecommunications facility employed in support of WIS, to include as necessary  routing and backup arrangements.   Each GISC should maintain service level agreements with suppliers of its communication links and associated hardware.  

4.5.8 Coordinate Telecommunications in GISC Area

Each GISC shall co-ordinate with the Centres within its area of responsibility a WIS telecommunications infrastructure that can meet the WIS requirements for information exchange within the area. In the case of particular global and/or regional agreements, a GISC could also support the exchange of agreed WIS time critical and operational critical information with other areas. The telecommunications infrastructure is implemented through various technologies and services (for example, Internet, satellite-based data distribution, dedicated data network) in accordance with capacity and reliability requirements. 
4.5.9 Recovery Arrangements of a GISC
Each GISC shall implement and operate proper procedures and arrangements to provide swift recovery or backup of its essential services in the event of an outage. Each GISC shall maintain arrangements for its essential services to be taken up by another GISC in case of an incapacitating system failure. Each GISC should maintain arrangements for system back up in case of total site failure (for example, an offsite Disaster Recovery Centre) and for partial back up in cases otherwise affecting WIS functions within the GISC. 
4.5.10 Performance Monitoring of a GISC
4.5.10.1 Each GISC shall participate in monitoring the performance of WIS, including monitoring the collection and distribution of data and products intended for global exchanges. Each GISC shall report routinely to other GISCs and to WMO Secretariat information concerning the status and performance of connectivity to WIS Centres in its area, including capacity as well as technology used (for example, Internet, satellite-based data distribution, dedicated data network). CBS shall review and report on the status and performance of GISCs with the assistance of the WMO Secretariat.
4.5.10.2 Monitoring of the collection and dissemination of WIS information (data and products) should include, as appropriate, Integrated World Weather Watch Monitoring and other programme-related monitoring.
4.5.10.3 See also sections 5.16, WIS-TechSpec-15 (Reporting of Quality of Service).

4.6 Functional Requirements of a DCPC

4.6.1 General

Note: The term information is used in a general sense and includes data and products. 

The specific performance and functional requirements of a particular DCPC shall be determined by the programme it supports. DCPCs that support programmes with mission-critical responsibilities, and especially programmes with safety-of-life missions, shall maintain a high level of operational reliability, including required telecommunications. Each DCPC shall provide metadata describing the information it makes available through the WIS comprehensive catalogue, shall provide access to that information, and shall participate in monitoring the overall performance of WIS.
4.6.2 Collect Information from DCPC Area

4.6.2.1 As appropriate to its programme role, a DCPC shall collect information intended for dissemination to NCs within its area of responsibility (that is, regional collections).
4.6.2.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.6.3 Collect Programme-Related Information

4.6.3.1 As appropriate to its programme role, a DCPC shall collect the specific programme-related data and products.
4.6.3.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.6.4 Production Support of Programme-Related Information

4.6.4.1 As appropriate to its programme role, a DCPC shall provide data management and data communications adequate to support the production of regional or specialized data and products.
4.6.4.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.6.5 Provide Information Intended for Global Exchange

4.6.5.1 As appropriate to its programme role, each DCPC shall provide information intended for global exchange to its responsible GISC.
4.6.5.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.6.6 Disseminate Information

4.6.6.1 As appropriate to its programme role, each DCPC shall disseminate information other than information intended for global exchange. 
4.6.6.2 See also sections: 5.11, WIS-TechSpec-10 (Downloading Files via Dedicated Networks); 5.12, WIS-TechSpec-11 (Downloading Files via Non-dedicated Networks); 5.13, WIS-TechSpec-11 (Downloading Files via Other Methods).

4.6.7 Provide Access to Information

4.6.7.1 Each DCPC shall support access to its products via WMO request/reply (”Pull”) mechanisms in an appropriate manner.
4.6.7.2 See also sections: 5.5, WIS-TechSpec-4 (Maintenance of User Identification and Role Information); 5.7, WIS-TechSpec-6 (Authentication of a User); 5.8, WIS-TechSpec-7 (Authorization of a User Role).

4.6.8 Describe Information with Metadata

4.6.8.1 Each DCPC shall describe its data and products according to an agreed WMO metadata standard, provide access to this catalogue of data and products, and provide this metadata as appropriate to other centres, in particular a GISC. 
4.6.8.2 See also sections: 5.9, WIS-TechSpec-8 (DAR Catalogue Search and Retrieval); 5.10, WIS-TechSpec-9 (Consolidated View of Distributed DAR Metadata Catalogues).

4.6.9 Recovery Arrangements of a DCPC

As appropriate to its programme role, each DCPC shall implement and operate proper procedures and arrangements to provide swift recovery or backup of their essential services in the event of an outage.
4.6.10 Performance Monitoring of a DCPC

4.6.10.1 Each DCPC shall participate in monitoring the performance of WIS.
4.6.10.2 See also sections 5.16, WIS-TechSpec-15 (Reporting of Quality of Service).

4.7 Functional Requirements of an NC
4.7.1 Provide Data, Products and Metadata
4.7.1.1 As required per WMO No. 49, Vol. I, A.3, each NC shall use WIS to provide data and products, in accord with its programme responsibilities. These shall be provided with associated metadata, in accordance with WIS practices, procedures and specifications.

4.7.1.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.7.2 Collect Programme-Related Information

4.7.2.1 As appropriate to its programme role, each NC shall collect programme-related data and products.

4.7.2.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.7.3 Production Support of Programme-Related Information

4.7.3.1 As appropriate to its programme role, each NC shall provide data management and data communications adequate to support the production of data and products.

4.7.3.2 See also sections 5.2, WIS-TechSpec-1 (Uploading of Metadata for Data and Products), and 5.3, WIS-TechSpec-2 (Uploading of Data and Products).

4.7.4 Describe Information with Metadata

4.7.4.1 Each NC shall describe its data and products according to an agreed WMO standard and provide this information as appropriate to other centres. 

4.7.4.2 See also sections: 5.9, WIS-TechSpec-8 (DAR Catalogue Search and Retrieval).

4.7.5 Performance Monitoring of an NC
4.7.5.1 As required per WMO No. 49, Vol. I, A.3, each NC shall participate in monitoring the performance of WIS.

4.7.5.2 See also sections 5.16, WIS-TechSpec-15 (Reporting of Quality of Service).

5 Technical Specifications of WIS
5.1 General

5.1.1 Fifteen Technical Specifications (WIS‑TechSpecs) define the interfaces to the major WIS functions. The specifications for these interfaces are named and numbered as follows:

1. Uploading of Metadata for Data and Products

2. Uploading of Data and Products

3. Centralization of Globally Distributed Data

4. Maintenance of User Identification and Role Information

5. Consolidated View of Distributed Identification and Role Information

6. Authentication of a User

7. Authorization of a User Role

8. DAR Catalogue Search and Retrieval

9. Consolidated View of Distributed DAR Metadata Catalogues

10. Downloading Files via Dedicated Networks

11. Downloading Files via Non-dedicated Networks

12. Downloading Files via Other Methods

13. Maintenance of Dissemination Metadata

14. Consolidated View of Distributed Dissemination Metadata Catalogues

15. Reporting of Quality of Service
5.1.2 NCs shall support seven of the 15 Technical Specifications, specifically: WIS-TechSpec-1, -2, -4, -10, -11, -12, and -15. An NC can arrange through bilateral agreements for another NC, a DCPC or a GISC to perform functions on its behalf. 
5.1.3 According to the particular requirements of a DCPC in its programme role, DCPCs shall support up to 13 of the 15 Technical Specifications. DCPCs are not required to support WIS-TechSpec-3 and WIS-TechSpec-9.

5.1.4 WIS GISCs shall support all 15 Technical Specifications.

5.1.5 Any DCPC or NC is welcome to implement interfaces beyond the minimum required. Accordingly, the Technical Specification is mandatory wherever the interface is required to be applied.
5.1.6 The GTS file naming convention shall be used for files and the associated metadata record whenever it is necessary. The GTS file naming convention is documented in the Manual on GTS (WMO‑No. 386), Vol. 1, Part II, Attachment II-15. 
Note: Guide to WIS 5.1 references "WIS Compliance Specifications for GISCs, DCPCs, and NCs", provided as supplementary guidance for WIS Centres. 
5.2 WIS-TechSpec-1: Uploading of Metadata for Data and Products

5.2.1 This specification requires that each metadata record uploaded shall be represented in compliance with the WMO Core Metadata Profile of ISO 19115 version 1.2, with a unique identifier. Note: Guide to WIS 5.2 references "Metadata Representation", which defines the WMO Core Metadata Profile. 
5.2.2 Uploading shall use methods prescribed by the receiver, which is typically the host of a WIS DAR Metadata Catalogue.
5.2.3 DAR Metadata should be provided prior to the files or messages associated with the metadata.

5.2.4 For updating the DAR Metadata Catalogue, GISCs should support two kinds of maintenance facilities: a file upload facility for batch updating (add, replace, or delete metadata records treated as separate files); and an online form for changing metadata entries in the DAR Metadata Catalogue (add, change, or delete of elements in a record as well as whole records).

5.2.5 GISCs shall maintain the updated DAR Metadata Catalogue as a searchable resource (see WIS-TechSpec-8).

5.2.6 See also sections: 4.5.2 (Receive Information from GISC Area); 4.6.2 (Collect Information from DCPC Area); 4.6.3 (Collect Programme-Related Information); 4.6.4 (Produce Programme-Related Information).
5.3 WIS-TechSpec-2: Uploading of Data and Products

5.3.1 This specification requires that data or products uploaded shall be represented in the manner prescribed by the relevant programme, including where appropriate the Manual on GTS (WMO No. 386), Vol. 1, Part II, Attachment II-2, the Manual on Codes, and other WMO manual(s) and the GTS file naming convention as noted in 5.1.6.

5.3.2 Data and products should be handled as specified in the Manual on GTS, Vol. I, Part I, 1.3 Design principles of the GTS, and other WMO manual(s), specific to the relevant programme.
5.3.3 See also sections: 4.5.2 (Receive Information from GISC Area); 4.6.2 (Collect Information from DCPC Area); 4.6.3 (Collect Programme-Related Information); 4.6.4 (Produce Programme-Related Information).

5.4 WIS-TechSpec-3: Centralization of Globally Distributed Data

5.4.1 This specification requires that the Manual on GTS (WMO No. 386), Vol. 1, Part I, Attachment I-3 is applied as appropriate to the centralized copies of information intended for global exchange (described at 4.5.1). 

5.4.2 Warnings shall be transmitted end-to-end within WIS within two minutes. 

5.4.3 See also sections 4.5.3 (Exchange Information with Other GISCs), and 4.5.5 (Maintain 24 hour Cache).

5.5 WIS-TechSpec-4: Maintenance of User Identification and Role Information

5.5.1 User identification and role information shall be represented and communicated using methods prescribed by the receiver, which is typically the host of an identification and role information database.
Note: The term "user identification" in this context does not imply that a user is personally identifiable. Administrators of authentication and authorization at WIS Centres need to share updated identification and role information as a resource available across WIS Centres.  Yet, it is necessary to prevent the inappropriate disclosure of any personally identifiable information.

5.5.2 User identification and role information maintenance should satisfy timeliness requirements of the application and host centre.

5.5.3 See also sections: 4.5.5 (Maintain 24 hour Cache), and 4.6.7 (Provide Access to Information).

5.6 WIS-TechSpec-5: Consolidated View of Distributed Identification and Role Information

5.6.1 This interface for a consolidated view of distributed identification and role information is not yet required. (see also Note at 5.5.1)
5.6.2 WIS Centres that do exchange identification and role information should do so using data encryption technologies.
5.6.3 See also sections: 4.5.5 (Maintain 24 hour Cache), and 4.6.7 (Provide Access to Information).

5.7 WIS-TechSpec-6: Authentication of a User

5.7.1 WIS Centres should employ authentication standards, which may include public key infrastructure techniques.  
Note: Commercial, off-the-shelf authentication software based on industry and/or international standards should be preferred.
5.7.2 User authentication should satisfy application-specific and host centre processing constraints, and shall provide a quality of service that meets user requirements.

5.7.3 See also sections: 4.5.5 (Maintain 24 hour Cache), and 4.6.7 (Provide Access to Information).

5.8 WIS-TechSpec-7: Authorization of a User Role

5.8.1 WIS Centres should employ government-endorsed standards for user authorization software, techniques and procedures.
5.8.2 User authorization should satisfy application-specific and host centre processing constraints and shall provide a quality of service that meets user requirements.
5.8.3 See also sections: 4.5.5 (Maintain 24 hour Cache), and 4.6.7 (Provide Access to Information).

5.9 WIS-TechSpec-8: DAR Catalogue Search and Retrieval

5.9.1 This specification requires that each metadata catalogue host shall support the SRU (Search and Retrieve via URL) specification of the ISO 23950 Information Search and Retrieval Protocol. A WIS compliant SRU server shall support SRU version 1.1, the SRU searchRetrieve operation, the SRU Explain operation, the diagnostic schema for returning errors, and SRU Contextual Query Language (CQL) level 2. 

5.9.2 In addition to full text search, a WIS compliant SRU server shall search: at least eight indexes as character strings (abstract, title, author, keywords, format, identifier, type, crs (Coordinate Reference System); at least five indexes as ordered dates (creationDate, modificationDate, publicationDate, beginningDate, endingDate); and the index "bounds" as geographic coordinates (decimal degrees, space delimited, in the order north, west, south, east). 
Note: Guide to WIS 5.9 references the "WIS SRU Implementors Note". 
5.9.3 The search service shall provide a quality of service that meets user requirements.

5.9.4 See also sections: 4.5.6 (Discovery Access and Retrieval), and 4.6.8 (Describe Information with Metadata).

5.10 WIS-TechSpec-9: Consolidated View of Distributed DAR Metadata Catalogues

5.10.1 GISCs should exchange metadata catalogue updates using version 2 of Open Archives Initiative - Protocol for Metadata Harvesting (OAI-PMH). 
5.10.2 The exchange of metadata catalogue updates should satisfy that distributed instances of DAR Metadata do not diverge in content by more than one day. A mechanism for rapid update on an emergency basis should also be provided.
5.10.3 See also section 4.5.6 (Discovery Access and Retrieval).

5.11 WIS-TechSpec-10: Downloading Files via Dedicated Networks

5.11.1 This specification requires that data or products downloaded shall be represented in the manner prescribed by the relevant programme, including where appropriate the Manual on GTS (WMO No. 386), Vol. 1, Part II, Attachment II-2 and other WMO manual(s) and the GTS file naming convention as noted in 5.1.6. 

5.11.2 Data and products should be handled as specified in the Manual on GTS, Vol. I, Part I, 1.3 Design principles of the GTS, and other WMO manual(s), specific to the relevant programme. 
5.11.3 See also sections: 4.5.4 (Disseminate Information to GISC Area), and 4.6.6 (Provide Information Intended for Global Exchange).

5.12 WIS-TechSpec-11: Downloading Files via Non-dedicated Networks

5.12.1 This specification requires that data or products downloaded shall be represented and communicated in a manner appropriate to the relevant programme.  

5.12.2 Data and products should be handled as specified in the Manual on GTS, Vol. I, Part I, 1.3 Design principles of the GTS, and other WMO manual(s), specific to the relevant programme. 
5.12.3 See also sections: 4.5.4 (Disseminate Information to GISC Area), and 4.6.6 (Provide Information Intended for Global Exchange).

5.13 WIS-TechSpec-12: Downloading Files via Other Methods

5.13.1 This specification requires that data or products downloaded shall be represented and communicated in a manner appropriate to the relevant programme.
5.13.2 Data and products should be handled as specified in the Manual on GTS, Vol. I, Part I, 1.3 Design principles of the GTS, and other WMO manual(s), specific to the relevant programme. 
5.13.3 See also sections: 4.5.4 (Disseminate Information to GISC Area), and 4.6.6 (Provide Information Intended for Global Exchange).

5.14 WIS-TechSpec-13: Maintenance of Dissemination Metadata

5.14.1 This specification requires that dissemination metadata (including subscription information such as accounts and delivery particulars) shall be represented and communicated as prescribed by the host of the database of dissemination metadata.
5.14.2 Requests for changes to dissemination for information not part of the routine global exchange may be subject to the notification period for changes specified in GTS. Otherwise, changes to dissemination should apply within one day.
5.14.3 See also sections: 4.5.6 (Discovery Access and Retrieval), and 4.6.6 (Provide Information Intended for Global Exchange).

5.15 WIS-TechSpec-14: Consolidated View of Distributed Dissemination Metadata Catalogues

5.15.1 This interface is not yet required. This interface may be needed as part of a backup arrangement between centres.

5.15.2 See also section 4.5.6 (Discovery Access and Retrieval).

5.16 WIS-TechSpec-15: Reporting of Quality of Service

5.16.1 This specification requires that the reporting of quality of service shall be represented and communicated as prescribed by the host of the centralized reporting database.
5.16.2 Reports should be sent on a schedule determined by the centralized reporting manager based on the needs of WIS Centres.
5.16.3 See also sections: 4.5.7 (Network Connectivity of GISC); 4.5.8 (Coordinate Telecommunications in GISC Area); 4.5.9 (Recovery Arrangements of GISC), 4.5.10 (Performance Monitoring of a GISC); 4.6.9 (Recovery Arrangements of a DCPC); 4.6.10 (Performance Monitoring of a DCPC).

APPENDIX A – List of Abbreviations

Cg
World Meteorological Congress

CBS
Commission for Basic Systems

DAR
Discovery, Access and Retrieval

DCPC
Data Collection or Production Centre

GDPFS
Global Data Processing and Forecast System

GISC
Global Information System Centre

GOS
Global Observing Systems

GTS
Global Telecommunication System

IGDDS
Integrated Global Data Distribution Service
MTN
Main Telecommunication Network

NC
National Centre

NMC
National Meteorological Centre

WIS
WMO Information System

WMO
World Meteorological Organization

APPENDIX B – Selected WMO Documents Relevant to WIS

Policy Documents 
WMO‑No. 15
Basic Documents No. 1 (2007 edition)
WMO‑No. 49 
Technical Regulations: 
Volume I - General Meteorological Standards and Recommended Practices
Volume II - Meteorological Services for Air Navigation
Volume III - Meteorological Services for Hydrology
Volume IV – Quality Management
WMO‑No. 60
Agreements and Working Arrangements
WMO‑No. 508
Resolutions of Congress and the Executive Council

Executive Council Reports 
International exchange of data and products
WMO facilitates the free and unrestricted exchange of data and information, products and services in real- or near-real time on matters relating to safety and security of society, economic welfare and the protection of the environment.

WMO‑No. 837
Exchanging Meteorological Data — Guidelines on Relationships in Commercial Meteorological Activities. WMO Policy and Practice. 

Cg-XII
Resolution 40—WMO policy and practice for the exchange of meteorological and related data and products including guidelines on the relationships in commercial meteorological activities. 

Cg XIII
Resolution 25—Exchange of Hydrological Data and Products
Annex IV - Geneva Declaration of Thirteenth World Meteorological Congress
Manuals

WMO‑No. 9
Weather reporting:


Volume A - Observing stations

Volume C1 - Catalogue of Meteorological Bulletins

Volume C2 - Transmission Programmes 

Volume D - Information for Shipping
WMO‑No. 306
Manual on Codes

WMO‑No. 386
Manual on the Global Telecommunication System (GTS), Volumes I & II

WMO‑No. 485
Manual on the Global Data Processing and Forecasting Systems (GDPFS), Parts 1, 2 & 3

WMO‑No. 544
Manual on Global Observing Systems (GOS)

Guides
WMO‑No. 8
Guide to meteorological instruments and methods of observation

WMO‑No. 100
Guide to climatological practices

WMO‑No. 134
Guide to agricultural meteorological practices

WMO‑No. 168
Guide to hydrological practices

WMO‑No. 305
Guide on the Global Data-processing System

WMO‑No. 471
Guide to marine meteorological services

WMO‑No. 488
Guide on the Global Observing System

WMO‑No. 611
Guide to WMO Binary Code Form GRIB 1  - Technical Report No.17 - May 1994
***
Guide to WMO Table-Driven Code Forms: FM 94 BUFR and FM 95 CREX
***
Guide to FM 92 GRIB Edition 2
WMO‑No. 636
Guide on the automation of data-processing centres

WMO‑No. 702
Guide to wave analysis and forecasting

WMO‑No. 731
Guide on meteorological observation and information distribution systems at aerodromes

WMO‑No. 732
Guide to practices for meteorological offices serving aviation

WMO‑No. 750
Guide to moored buoys and other ocean data acquisition systems

WMO‑No. 788
Guide on World Weather Watch Data Management
WMO‑No. 834
Guide to public weather services practices

 GTS
Guide on Internet Practices

 GTS
Guide on VPN via the Internet between GTS centres

 GTS
Guide on the use of TCP/IP on the GTS

 GTS
Guide on Provisional Arrangements for the use of IP Addresses over the GTS

 GTS
Guide on IT Security

APPENDIX C – Approved WIS Centres
APPENDIX C.1 Global Information System Centres (GISCs)
 (none listed at this time)
APPENDIX C.2 Data Collection or Production Centres (DCPCs)

 (none listed at this time)

APPENDIX C.3 National Centres (NCs)

 (none listed at this time)

[image: image1][image: image2][image: image3]
WMO�No. 1060

















PAGE  
Manual on WMO Information System (WIS)
Page 2 of 19
Recommended Draft (version 1.0)  6 Oct 2010 


